|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.780/Y.1351 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (07/2004) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital terminal equipments – Principal characteristics of multiplexing equipment for the synchronous digital hierarchy SERIES Y: GLOBAL INFORMATION INFRASTRUCTURE, INTERNET PROTOCOL ASPECTS AND NEXT GENERATION NETWORKS Internet protocol aspects – Transport |  |  |  |
|  | Terms and definitions for synchronous digital hierarchy (SDH) networks |  |  |  |
|  | ITU-T Recommendation G.780/Y.1351 |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| General | G.700–G.709 |
| Coding of analogue signals by pulse code modulation | G.710–G.719 |
| Coding of analogue signals by methods other than PCM | G.720–G.729 |
| Principal characteristics of primary multiplex equipment | G.730–G.739 |
| Principal characteristics of second order multiplex equipment | G.740–G.749 |
| Principal characteristics of higher order multiplex equipment | G.750–G.759 |
| Principal characteristics of transcoder and digital multiplication equipment | G.760–G.769 |
| Operations, administration and maintenance features of transmission equipment | G.770–G.779 |
| Principal characteristics of multiplexing equipment for the synchronous digital hierarchy | G.780–G.789 |
| Other terminal equipment | G.790–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| QUALITY OF SERVICE AND PERFORMANCE - GENERIC AND USER-RELATED ASPECTS | G.1000–G.1999 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.6000–G.6999 |
| DIGITAL TERMINAL EQUIPMENTS | G.7000–G.7999 |
| DIGITAL NETWORKS | G.8000–G.8999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.780/Y.1351 Terms and definitions for synchronous digital hierarchy (SDH) networks

Summary This Recommendation provides terms, definitions, and abbreviations used in synchronous digital hierarchy (SDH) Recommendations. Physical layer terminology, synchronization-related terminology, and terms applicable to multiple technologies in addition to SDH are not included. This Recommendation contains terminology from many of the same source SDH Recommendation, but the changes were so exhaustive that one should not consider this as a simple revision of the 1999 version of ITU-T Rec. G.780. The goal of this Recommendation is to be a single normative source for terms in this subject area.

Source ITU-T Recommendation G.780/Y.1351 was approved on 22 July 2004 by ITU-T Study Group 15 (2001-2004) under the ITU-T Recommendation A.8 procedure.

Keywords Definitions, SDH, Synchronous Digital Hierarchy, terminology.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g. interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met.  The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2004

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Referenced definitions		2

3.1	Terms defined in other Recommendations		2

3.2	Terms defined in the present Recommendation		2

4	Abbreviations		13

5	Conventions		15

Appendix I – List of source Recommendations		15

ITU-T Recommendation G.780/Y.1351

Terms and definitions for synchronous digital hierarchy (SDH) networks

# 1	Scope

This Recommendation contains a list of terms, definitions, and abbreviations used in Recommendations dealing with Synchronous Digital Hierarchy (SDH) networks. SDH terms specific to synchronization or physical layer or characteristics are not a part of this Recommendation.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

–	ITU-T Recommendation G.702 (1988), Digital hierarchy bit rates.

–	ITU-T Recommendation G.703 (2001), Physical/electrical characteristics of hierarchical digital interfaces.

–	ITU-T Recommendation G.707/Y.1322 (2003), Network node interface for the synchronous digital hierarchy (SDH).

–	ITU-T Recommendation G.783 (2004), Characteristics of synchronous digital hierarchy (SDH) equipment functional blocks.

–	ITU-T Recommendation G.784 (1999), Synchronous digital hierarchy (SDH) management.

–	ITU-T Recommendation G.803 (2000), Architecture of transport networks based on the synchronous digital hierarchy (SDH).

–	ITU-T Recommendation G.805 (2000), Generic functional architecture of transport networks.

–	ITU-T Recommendation G.806 (2004), Characteristics of transport equipment – Description methodology and generic functionality.

–	ITU-T Recommendation G.826 (2002), End-to-end error performance parameters and objectives for international, constant bit-rate digital paths and connections.

–	ITU-T Recommendation G.828 (2000), Error performance parameters and objectives for international, constant bit-rate synchronous digital paths.

–	ITU-T Recommendation G.829 (2002), Error performance events for SDH multiplex and regenerator sections.

–	ITU-T Recommendation G.841 (1998), Types and characteristics of SDH network protection architectures.

–	ITU-T Recommendation G.842 (1997), Interworking of SDH network protection architectures.

–	ITU-T Recommendation M.3010 (2000), Principles for a telecommunications management network.

–	ITU-T Recommendation X.710 (1997) | ISO/IEC 9595:1998, Information technology – Open Systems Interconnection – Common Management Information service.

# 3	Referenced definitions

## 3.1	Terms defined in other Recommendations

This Recommendation uses the following terms defined in ITU-T Rec. G.805.

–	access point (AP)

–	adaptation

–	alarm

–	all-ones

–	characteristic information (CI)

–	connection

–	connection point (CP)

–	defect

–	failure

–	function

–	management information (MI)

–	network connection (NC)

–	path

–	process

–	signal degrade (SD)

–	signal fail (SF)

–	subnetwork connection (SNC)

–	trail

–	trail termination function (TT)

This Recommendation uses the following term defined in ITU-T Rec. G.7712/Y.1703.

–	embedded control channel (ECC)

This Recommendation uses the following terms defined in ITU-T Rec. G.870/Y.1352.

–	bridge

–	head-end

–	null signal

–	switch

–	tail-end

This Recommendation uses the following term defined in ITU-T Rec. M.3010.

–	telecommunications management network (TMN)

## 3.2	Terms defined in the present Recommendation

This Recommendation defines the following terms:

3.2.1	active trail/path/section/SNC/NC: The trail/path/section/SNC from which the signal is selected by the protection selector.

3.2.2	adapted Information (AI): The information passing across an AP.

3.2.3	add traffic: Normal or extra traffic inserted into working, protection, or non-pre-emptible unprotected channels on the ring at a ring node.

3.2.4	add/drop multiplexer (ADM): Network element that provides access to all, or some subset of the constituent signals contained within an STM-N signal. The constituent signals are added to (inserted), and/or dropped from (extracted) the STM-N signal as it passes through the ADM.

3.2.5	administrative unit group (AUG): One or more Administrative Units occupying fixed, defined positions in an STM payload are termed an Administrative Unit Group (AUG).

An AUG-1 consists of a homogeneous assembly of AU-3s or an AU-4.

3.2.6	administrative unit-n (AU-n): An Administrative Unit is the information structure which provides adaptation between the higher order path layer and the multiplex section layer. It consists of an information payload (the higher order Virtual Container) and an Administrative Unit pointer which indicates the offset of the payload frame start relative to the multiplex section frame start.

Two Administrative Units are defined. The AU-4 consists of a VC-4 plus an Administrative Unit pointer which indicates the phase alignment of the VC-4 with respect to the STM-N frame. The AU-3 consists of a VC-3 plus an Administrative Unit pointer which indicates the phase alignment of the VC-3 with respect to the STM-N frame. In each case, the Administrative Unit pointer location is fixed with respect to the STM-N frame.

3.2.7	administrative unit-n (AU-n) pointer: Administrative Unit pointer which indicates the offset of the payload frame start relative to the multiplex section frame start. See ITU-T Rec. G.707/Y.1322.

3.2.8	agent: Part of the MAF which is capable of responding to network management operations issued by a manager and may perform operations on managed objects, issuing events on behalf of managed objects. The managed objects can reside within the entity or in another open system. Managed objects from other open systems are controlled by a distant agent via a local manager. All SDH NEs will support at least an agent. Some SDH NEs will provide managers and agents (being managed). Some NEs (e.g., regenerators) will only support an agent.

3.2.9	alarm indication signal (AIS): A code sent downstream in a digital network as an indication that an upstream failure has been detected and alarmed. It is associated with multiple transport layers.

NOTE – See ITU-T Rec. G.707/Y.1322 for specific AIS signals.

3.2.10	asynchronous interface: These interfaces provide an output signal with frequency that is not traceable to a PRC and that meets the frequency offset requirements given in ITU-T Rec. G.703.

3.2.11	automatic protection switching (APS) controller: That part of a node that is responsible for generating and terminating information carried in the APS protocol and implementing the APS algorithm.

3.2.12	automatic protection switching (APS) request: That set of signals into an APS controller that determines its behaviour. An APS request can be either an externally initiated command or an automatically initiated command.

3.2.13	automatic protection switching (APS): Autonomous switching of a signal between and including two MSn_TT, Sn_TT, or Sm_TT functions, from a failed working trail/SNC to a protection trail/SNC and subsequent restoration using control signals carried by the K-bytes in the MSOH, HO POH, or LO POH.

3.2.14	automatically initiated command: An APS request that is initiated by any one of the following:

1)	multiplex section performance criteria;

2)	local equipment performance criteria; or

3)	received bridge requests.

3.2.15	auto-provisioning: The assignment of values to parameters within a network element, without those values being specifically entered externally by a user.

3.2.16	background block error (BBE): An errored block not occurring as part of an SES.

3.2.17	background block error ratio (BBER): The ratio of BBE in available time to total blocks in available time during a fixed measurement interval. The count of total blocks excludes all blocks during SESs.

3.2.18	bidirectional protection switching: A protection switching architecture in which, for a unidirectional failure (i.e., a failure affecting only one direction of transmission), both directions (of the "trail", "subnetwork connection", etc.) including the affected direction and the unaffected direction, are switched to protection.

3.2.19	bidirectional ring: In a bidirectional ring, normal routing of the normal traffic signals is such that both directions of a bidirectional connection travel along the ring through the same nodes, but in opposite directions.

3.2.20	bit interleaved parity-X (BIP-X): BIP-X code is defined as a method of error monitoring. With even parity an X-bit code is generated by the transmitting equipment over a specified portion of the signal in such a manner that the first bit of the code provides even parity over the first bit of all X-bit sequences in the covered portion of the signal, the second bit provides even parity over the second bit of all X-bit sequences within the specified portion, etc. Even parity is generated by setting the BIP-X bits so that there is an even number of 1s in each monitored partition of the signal. A monitored partition comprises all bits which are in the same bit position within the X-bit sequences in the covered portion of the signal. The covered portion includes the BIP-X.

3.2.21	block (generic definition): A block is a set of consecutive bits associated with the path; each bit belongs to one, and only one, block. Consecutive bits may not be contiguous in time.

3.2.22	bridge request: A message sent from a tail-end node to the head-end node requesting that the head-end perform a bridge of the normal traffic signals onto the protection channels.

3.2.23	bridge request status: A message sent from a tail-end node to all other nodes within the protection system indicating that the tail-end has requested a bridge.

3.2.24	concatenation: The process of summing the bandwidth of a number of smaller containers into a larger bandwidth container. Two versions exist:

–	Contiguous concatenation: maintains the contiguous bandwidth throughout the whole transport. Contiguous concatenation requires concatenation functionality at each network element.

–	Virtual concatenation: breaks the contiguous bandwidth into individual VCs, transports the individual VCs and recombines these VCs to a contiguous bandwidth at the end point of the transmission. Virtual concatenation requires concatenation functionality only at the path termination equipment.

NOTE – See ITU-T Rec. G.707/Y.1322.

3.2.25	container-n (n = 1-4): A container is the information structure which forms the network synchronous information payload for a Virtual Container. For each of the defined Virtual Containers there is a corresponding container. Adaptation functions have been defined for many common network rates into a limited number of standard containers. These include those rates already defined in ITU-T Rec. G.702. Further adaptation functions will be defined in the future for new broadband rates.

3.2.26	controller failure: The condition during which a node is no longer able to correctly operate the APS protocol, but still generates a correctly formatted SDH frame.

3.2.27	crossing K-bytes: When a node sees ring bridge requests of equal priority on both "sides" (this includes a switching node receiving a ring bridge request from the other end).

3.2.28	data communications channel (DCC): Within an STM-N signal there are three DCC channels, comprising bytes D1-D3, giving a 192 kbit/s channel, bytes D4-D12, giving a 576 kbit/s channel, and (in STM-256 only) D13-D156, giving a 9216 kbit/s channel. D1-D3 (DCCR) are accessible by all SDH NEs whereas D4-D12 (DCCM) and D13-D156 (DCCMX), not being part of the regenerator section overhead, are not accessible at regenerators. It is recommended to have both DCCM and DCCR available in backbone STM-16 (and higher order) network sections and DCCMX in STM-256 network sections. DCCM and DCCMX are used to forward data over the multiplex sections (using the OSI-routing protocols), and DCCR is used to forward data to the regenerators within the destination MS span. DCCM and DCCMX can be regarded as the backbone, while DCCR and LAN are used to interconnect this backbone to equipment that cannot be accessed through DCCM/DCCMX e.g., regenerators and non-SDH equipment.

DCCM/DCCMX and DCCR can be used to carry two independent, possibly proprietary management applications. An NE can choose to throughconnect DCCM/DCCMX on the physical level, or to terminate the DCCM/DCCMX and route the PDUs, while using the DCCR for interconnection within a subnetwork.

3.2.29	dedicated protection: A protection architecture that provides capacity dedicated to the protection of traffic-carrying capacity.

3.2.30	default APS code: This term refers to the APS bytes transmitted with the source node ID equal to the destination node ID.

3.2.31	desynchronizer: The desynchronizer function smoothes out the timing gaps resulting from decoded pointer adjustments and VC payload de-mapping in the time domain.

3.2.32	diverse routing of [trail/SNC] protection pair: Diverse routing of a working trail/SNC and its associated protection trail/SNC, where the working trail/SNC (in both directions of transmission) takes one (physical) route, and the protection trail/SNC (in both directions of transmission) takes another.

3.2.33	diverse routing of go and return: Bidirectional transport entity/signal (i.e., go and return) is set up on/routed over different physical facilities. Such routing may apply to individual trails, subnetwork connections, or signals.

3.2.34	drop traffic: Normal or extra traffic extracted from working, protection, or non-pre-emptible unprotected channels on the ring at a ring node.

3.2.35	drop-and-continue: A function within a ring node where traffic is both extracted from the working channels on the ring (drop), and transmitted onwards on the ring (continue).

3.2.36	dual hubbed: Dual hubbed traffic can be routed to either or both of two central offices (or similar sites). Dual hubbed traffic is survivable under a failure of one of the two hubs.

3.2.37	dual node interconnection: An architecture between two rings where two nodes in each ring are interconnected.

3.2.38	errored block (EB): A block in which one or more bits are in error.

3.2.39	errored second (ES): A one-second period with one or more errored blocks or at least one defect.

NOTE – See ITU-T Rec. G.828 for list of defects.

3.2.40	errored second ratio (ESR): The ratio of ES in available time to total seconds in available time during a fixed measurement interval.

3.2.41	externally initiated command: An APS request that is initiated by either an OS or a craftsperson.

3.2.42	extra traffic: Traffic that is carried over the protection channels when that capacity is not used for the protection of normal traffic. Extra traffic is not protected. Whenever the protection channels are required to protect the normal traffic, the extra traffic is pre-empted.

3.2.43	full pass-through: The action of transmitting the same K1, K2, and protection channels that are being received. Full pass-through may be either unidirectional or bidirectional as specified in the text. When a node enters unidirectional full pass-through, it shall continue sourcing the previously sourced K-bytes in the opposite direction, with the exception that K2 bits 6-8 shall reflect the appropriate status code.

3.2.44	generator polynomial: The polynomial that is used for encoding of any cyclic codes. The remainder after division of the information polynomial by generating polynomial is the redundancy part of the encoded code word.

3.2.45	hypothetical reference path: A Hypothetical Reference Path (HRP) is defined as the whole means of digital transmission of a digital signal of a specified rate, including the path overhead, between equipment at which the signal originates and terminates. An end-to-end Hypothetical Reference Path spans a distance of 27 500 km.

3.2.46	idle: A node that is not generating, detecting, or passing-through bridge requests or bridge request status information.

3.2.47	isolated node: A single node that is isolated from a traffic perspective by ring switches on each of its two spans by its adjacent nodes.

3.2.48	K-byte pass-through: The action of transmitting the same K1 and K2 bytes that are being received. Protection channels are not passed through. K-byte pass-through is bidirectional.

3.2.49	layer: A concept used to allow the transport network functionality to be described hierarchically as successive levels; each layer being solely concerned with the generation and transfer of its characteristic information.

3.2.50	long path: The path segment away from the span for which the bridge request is initiated. Typically there are other intermediate nodes along this path segment.

3.2.51	loss of frame (LOF): An LOF state of an STM-N signal is considered to have occurred when an OOF state persists for a defined period of time.

NOTE – See ITU-T Rec. G.783.

3.2.52	loss of signal (LOS): The LOS state is considered to have occurred when the amplitude of the relevant signal has dropped below prescribed limits for a prescribed period.

NOTE – See ITU-T Rec. G.783.

3.2.53	lower order VC access: The termination of a higher order VC for the purpose of adding, dropping, or cross-connecting any individual lower order VC or VC group.

3.2.54	managed object (MO): The management view of a resource within the telecommunication environment that may be managed via the agent. Examples of SDH managed objects are: equipment, receive port, transmit port, power supply, plug-in card, virtual container, multiplex section and regenerator section.

3.2.55	managed object class (MOC): An identified family of managed objects that share the same characteristics, e.g., "equipment" may share the same characteristics as "plug-in card".

3.2.56	management application function (MAF): An application process participating in system management. The management application function includes an agent (being managed) and/or manager. Each network element (NE) and operations system or mediation device (OS/MD) must support a management application function that includes at least an agent. A management application function is the origin and termination for all TMN messages.

3.2.57	manager: Part of the MAF which is capable of issuing network management operations (i.e., retrieve alarm records, set thresholds) and receiving events (i.e., alarms, performance). SDH NEs may or may not include a manager while SDH OS/MDs will include at least one manager.

3.2.58	message communications function (MCF): The message communications function provides facilities for the transport of TMN messages to and from the MAF, as well as facilities for the transit of messages. The message communications function does not originate or terminate messages (in the sense of the upper protocol layers).

3.2.59	misconnection: A condition in which traffic destined for a given node is incorrectly routed to another node and no corrective action has been taken.

3.2.60	most significant bit: The "leftmost" bit position, or first transmitted bit position in a byte.

3.2.61	multiplex section (MS): A multiplex section is the trail between and including two multiplex section trail termination functions.

3.2.62	multiplex section overhead (MSOH): See: section overhead.

3.2.63	network connection protection: A scheme that protects the largest possible subnetwork connection of a trail.

3.2.64	network element (NE): A stand-alone physical entity that supports at least network element functions (NEFs) and may also support outgoing signal fail (OSF) and/or mediation function (MF). It contains managed objects, a management communications function (MCF) and a management application function (MAF).

3.2.65	network element function (NEF): A function within an SDH entity that supports the SDH based network transport services, e.g., multiplexing, cross-connection, regeneration. The network element function is modelled by managed objects.

3.2.66	network node interface (NNI): The interface at a network node which is used to interconnect with another network node.

3.2.67	non-pre-emptible unprotected channel: A channel in a MS shared protection ring provisioned bidirectionally to provide transport without MS shared protection ring automatic protection switching. Non-pre-emptible unprotected channels are provisioned from (corresponding) working and protection channel pairs.

3.2.68	non-pre-emptible unprotected traffic: Unprotected traffic carried on protection locked-out channel which may not be pre-empted (e.g., by protection switches).

3.2.69	normal traffic: Traffic that is normally carried in the working channels/sections, except in the event of a protection switch, in which case it is restored on the protection channels/sections. Normal traffic is protected.

3.2.70	operations system function or mediation function (OSF/MF): A telecommunications management network (TMN) entity that processes management information to monitor and control the SDH network. In the SDH sub-portion of the TMN, no distinction is made between the operations system function and the mediation function; this entity being a MAF containing at least a manager.

3.2.71	operations system or mediation device (OS/MD): A stand-alone physical entity that supports OSF/MFs but does not support NEFs. It contains a message communications function (MCF) and a MAF.

3.2.72	outgoing signal fail (OSF): A signal fail indication output at the AP of a tandem connection termination function.

3.2.73	overhead access (OHA): The OHA function provides access to transmission overhead functions.

3.2.74	pass-through: The action of transmitting the same information that is being received for any given direction of transmission.

3.2.75	path overhead (POH): (See definition of "section overhead".)

3.2.76	path selector: Within an SNCP architecture, the node function that selects a tributary which is extracted from the working channels arriving from one side of the node or from the other side of the node, according to path level criteria.

3.2.77	pointer justification event (PJE): A PJE is an inversion of the I- or D-bits of the pointer, together with an increment or decrement of the pointer value to signify a frequency justification.

3.2.78	pointer: An indicator whose value defines the frame offset of a Virtual Container with respect to the frame reference of the transport entity on which it is supported.

3.2.79	primary node: Within an MS-shared protection ring interworking architecture, the node which provides the service selection and drop-and-continue functions for a tributary. Different tributaries may have different designated primary nodes.

3.2.80	propagation of switching: One protection switch leading to another. Propagation of switching is often, though not always, undesirable from a maintenance point of view.

3.2.81	protection channels: The channels allocated to transport the normal traffic during a switch event. Protection channels may be used to carry extra traffic in the absence of a switch event. When there is a switch event, normal traffic on the affected working channels is bridged onto on the protection channels.

3.2.82	reference point: The delimiter of a function.

3.2.83	regenerator section (RS): A regenerator section is the trail between and including two regenerator section terminations.

3.2.84	regenerator section overhead (RSOH): (See definition of "section overhead.")

3.2.85	restoral threshold: For automatically initiated commands, a hysteresis method is used when switching normal traffic from the protection channels back to the working channels. This method specifies a BER threshold for the multiplex section that is carrying the working channels. This threshold is commonly referred to as "restoral threshold". The restoral threshold is set to a lower BER than the signal degrade threshold.

3.2.86	ring failure: A failure for which restoration can only be accomplished by a ring switch.

3.2.87	ring interconnection: An architecture between two rings where one or more nodes in each ring are interconnected.

3.2.88	ring interworking: A network topology where two rings are connected at two points and operate such that failure at either of these two points will not cause loss of any traffic, except possibly that dropped or inserted at the point of failure.

3.2.89	ring interworking: A network topology whereby two rings are interconnected at two nodes on each ring, and the topology operates such that a failure at either of these two nodes will not cause loss of any working traffic.

3.2.90	ring switching: Protection mechanism that applies to both two-fibre and four-fibre rings. During a ring switch, the traffic from the affected span is carried over the protection channels on the long path.

3.2.91	ring: A collection of nodes forming a closed loop whereby each node is connected to two adjacent nodes via a duplex communications facility. A ring provides redundant bandwidth or redundant network equipment, or both, so distributed services can be automatically restored following a failure or degradation in the network. Thus, a ring can be self-healing.

3.2.92	SDH aligning: A procedure by which the frame offset information is incorporated into the Tributary Unit or the Administrative Unit when adapting to the frame reference of the supporting layer.

3.2.93	SDH digital path: An SDH digital path is a trail carrying an SDH payload and associated overhead through the layered transport network between the path terminating equipment. A digital path may be bidirectional or unidirectional and may comprise both customer owned portions and network operator owned portions.

3.2.94	SDH higher-order path layer networks: Those layer networks with characteristic information of VC-3 (see Note), VC-3-Xv (X = 1 … 256), VC-4, VC-4-Xc (X = 4, 16, 64, 256) or VC-4-Xv (X = 1 … 256).

NOTE – The VC-3 is considered to be a higher-order path if it is supported directly by an AU-3 in a multiplex section layer network; it is considered a lower-order path if it is supported by a TU-3 in a VC-4 layer network.

3.2.95	SDH lower-order path layer networks: Those layer networks with characteristic information of VC-11, VC-11-Xv (X = 1 … 64), VC-12, VC-12-Xv (X = 1 … 64), VC-2, VC-2-Xc (X = 2 … 7) (see Note), VC-2-Xv (X = 1 … 64) or VC-3-Xv (X = 1 … 256).

NOTE – Transported in one higher order VC-3.

3.2.96	SDH management network (SMN): An SDH management network is a subset of a TMN, responsible for managing SDH NEs. An SMN may be subdivided into a set of SDH management subnetworks.

3.2.97	SDH management subnetwork (SMS): An SDH management subnetwork (SMS) consists of a set of separate SDH ECCs and associated intra-site data communication links which have been interconnected to form an operations data communications control network within any given SDH transport topology. An SMS represents an SDH specific local communications network (LCN) portion of a network operator's overall operations data network or TMN.

3.2.98	SDH mapping: A procedure by which tributaries are adapted into Virtual Containers at the boundary of an SDH network.

3.2.99	SDH multiplex section layer: A layer network with characteristic information of STM-N, i.e., with a bit rate of STM-N and the multiplex section overhead as defined in ITU-T Rec. G.707/Y.1322.

3.2.100	SDH multiplexing: A procedure by which multiple lower-order path layer signals are adapted into a higher-order path or the multiple higher-order path layer signals are adapted into a multiplex section.

3.2.101	SDH path layer: A transport assembly composed of the SDH higher-order path layer network and lower-order path layer network together with the associated adaptation functions.

3.2.102	SDH regenerator section layer: A layer network with characteristic information of STM-N, i.e., with a bit rate of STM-N and the regenerator section overhead as defined in ITU-T Rec. G.707/Y.1322.

3.2.103	SDH section layer: A transport assembly composed of the SDH multiplex section layer network and regenerator section layer network together with the associated adaptation functions.

3.2.104	secondary circuit: Within an MS-shared protection ring interworking architecture, this is the alternate routing that traffic travelling from one ring to another follows. This alternate additional routing is used when the service circuit is interrupted.

3.2.105	secondary node: Within an MS-shared protection ring interworking architecture, the node which provides the alternate interworking route for a tributary.

3.2.106	section overhead: SOH information is added to the information payload to create an STM-N. It includes block framing information and information for maintenance, performance monitoring and other operational functions. The SOH information is further classified into Regenerator Section Overhead (RSOH) which is terminated at regenerator functions and Multiplex Section Overhead (MSOH) which passes transparently through regenerators and is terminated where the AUGs are assembled and disassembled. The rows 1-3 of the SOH are designated as RSOH while rows 5-9 are designated to be MSOH.

NOTE – See ITU-T Rec. G.707/Y.1322.

3.2.107	section: A trail in a section layer.

3.2.108	segmented ring: A ring that is separated into two or more segments, either externally using Forced Switches (FS-R), or automatically as a result of Signal Failed – Ring switches (SF-R).

3.2.109	service circuit: Within an MS-shared protection ring interworking architecture, this is the preferred original routing that traffic travelling from one ring to another normally follows.

3.2.110	service selector: Within an MS-shared protection ring architecture, the node function used for ring interworking. It selects traffic from either the channels arriving from one side of the node, or from traffic entering the ring, according to some criteria.

3.2.111	severely errored period (SEP): A sequence of between 3 to 9 consecutive SES. The sequence is terminated by a second which is not a SES.

NOTE – The Severely Errored Period (SEP) event is identical to the CSES event contained in ITU-T Rec. G.784 on condition that the lower threshold is fixed at three consecutive SESs.

3.2.112	severely errored period intensity (SEPI): The number of SEP events in available time, divided by the total available time in seconds. (See Notes 1, 2 and 3.)

NOTE 1 – The SEPI parameter has a unit of (1/s). This is to enable the SEPI objective to be easily translated to the equivalent number of SEP events over a specific measurement interval. It should be noted that the SEP event has no significance over a time interval of less than three seconds.

NOTE 2 – Ongoing studies of the SEP event and the SEPI parameter shall prove their usefulness in complementing the SESR parameter. Any objectives for the SEPI parameter (presently under study) shall empirically demonstrate this value.

NOTE 3 – The impact of SEP/SEPI on customer services has to be investigated.

3.2.113	severely errored second (SES): A one-second period which contains X% errored blocks or at least one defect. SES is a subset of ES.

NOTE – The value of X is specified for SDH in ITU-T Recs G.826, G.828, and G.829.

3.2.114	severely errored second ratio (SESR): The ratio of SES to total seconds in available time during a fixed measurement interval.

3.2.115	short path: The path segment over the span for which the bridge request is initiated. This span is always the one to which both the head-end and tail-end are connected. The short path bridge request is the bridge request sent over the span for which the bridge request is initiated.

3.2.116	single node interconnection: An architecture between two rings where one node in each ring is interconnected.

3.2.117	single point failure: Failure located at a single physical point in a ring. The failure may affect one or more fibres. A single point failure may be detected by any number of NEs.

3.2.118	span switching: Protection mechanism similar to 1:1 linear APS that applies only to four-fibre rings where working and protection channels are contained in separate fibres and the failure only affects the working channels. During a span switch, the normal traffic is carried over the protection channels on the same span as the failure.

3.2.119	span: The set of multiplex sections between two adjacent nodes on a ring.

3.2.120	squelched traffic: An all "1"s signal resulting from the squelching process.

3.2.121	squelching: The process of inserting AU-AIS in order to prevent misconnections.

3.2.122	subnetwork connection protection (SNCP): A working subnetwork connection is replaced by a protection subnetwork connection if the working subnetwork connection fails, or if its performance falls below a required level.

3.2.123	survivable network: A network that is capable of restoring traffic in the event of a failure. The degree of survivability is determined by the network's ability to survive single line system failures, multiple line system failures, and equipment failures.

3.2.124	switch completion time: The interval from the decision to switch to the completion of the bridge and switch operation at a switching node initiating the bridge request.

3.2.125	switching node: The node that performs the bridge or switch function for a protection event. In the case of a multiplex section switched ring network architecture, this node also performs any necessary squelching of misconnected traffic for VC-3/4 or higher rate paths.

3.2.126	synchronous digital hierarchy (SDH): The SDH is a hierarchical set of digital transport structures, standardized for the transport of suitably adapted payloads over physical transmission networks.

3.2.127	synchronous transport module (STM): An STM is the information structure used to support section layer connections in the SDH. It consists of information payload and Section Overhead (SOH) information fields organized in a block frame structure which repeats every 125 s. The information is suitably conditioned for serial transmission on the selected media at a rate which is synchronized to the network. A basic STM is defined at 155 520 kbit/s. This is termed STM-1. Higher capacity STMs are formed at rates equivalent to N times this basic rate. STM capacities for N = 4, N = 16, N = 64 and N = 256 are defined; higher values are under consideration.

The STM-0 comprises a single Administrative Unit of level 3. The STM-N, N  1, comprises a single Administrative Unit Group of level N (AUG-N) together with the SOH. The STM-N hierarchical bit rates are given in ITU-T Rec. G.707/Y.1322.

3.2.128	synchronous: The essential characteristic of time-scales or signals such that their corresponding significant instants occur at precisely the same average rate.

3.2.129	systematic code: The original data bits for binary codes are unchanged by the encoding procedure. Redundant bits or symbols (parity) are added separately to each code block.

3.2.130	termination node: The node (other than a primary or secondary node) where a tributary enters or exits the ring.

3.2.131	time slot interchange (TSI): For purposes of this Recommendation, TSI is the capability of changing the time slot position of through-connected traffic (i.e., traffic that is not added or dropped from the node).

3.2.132	trail protection: Normal traffic is carried over/selected from a protection trail instead of a working trail if the working trail fails, or if its performance falls below a required level.

3.2.133	trail segment: A segment for which one end is a trail termination.

3.2.134	transport: Facilities associated with the carriage of STM-1 or higher level signals.

3.2.135	tributary unit-n (TU-n): A Tributary Unit is an information structure which provides adaptation between the lower-order path layer and the higher-order path layer. It consists of an information payload (the lower-order Virtual Container) and a Tributary Unit pointer which indicates the offset of the payload frame start relative to the higher-order Virtual Container frame start.

The TU-n (n = 11, 12, 2, 3) consists of a VC-n together with a Tributary Unit pointer.

3.2.136	tributary unit group (TUG): One or more Tributary Units, occupying fixed, defined positions in a higher-order VC-n payload is termed a Tributary Unit Group (TUG). TUGs are defined in such a way that mixed capacity payloads made up of different size Tributary Units can be constructed to increase flexibility of the transport network.

A TUG-2 consists of a homogeneous assembly of identical TU-11s, TU-12s or a TU-2.

A TUG-3 consists of a homogeneous assembly of TUG-2s or a TU-3.

3.2.137	undefined bit: If a bit is undefined, its value is set to a logical "0" or a logical "1".

3.2.138	undefined byte: If a byte is undefined, it contains eight undefined bits.

3.2.139	undetected failure: Any equipment defect which is not detected by equipment maintenance functions, hence does not initiate a protection switch or provide the appropriate OA&M notification. These types of failures do not manifest themselves until a protection switch is attempted.

3.2.140	unidirectional protection switching: A protection switching architecture in which, for a unidirectional failure (i.e., a failure affecting only one direction of transmission), only the affected direction (of the "trail", "subnetwork connection", etc.) is switched to protection.

3.2.141	unidirectional ring: In a unidirectional ring (path switched or multiplex section switched), normal routing of the normal traffic is such that both directions of a bidirectional connection travel around the ring in the same direction (e.g., clockwise). Specifically, each bidirectional connection uses capacity along the entire circumference of the ring.

3.2.142	uniform routing of go and return: Bidirectional transport entity/signal (i.e., go and return) is set up on/routed over the same physical facilities. Such routing may apply to individual trails, subnetwork connections, or signals.

3.2.143	virtual container-n (VC-n): A Virtual Container is the information structure used to support path layer connections in the SDH. It consists of information payload and Path Overhead (POH) information fields organized in a block frame structure which repeats every 125 or 500 s. Alignment information to identify VC-n frame start is provided by the server network layer.

Two types of Virtual Containers have been identified.

–	Lower order Virtual Container-n: VC-n (n = 11, 12, 2, 3)

This element comprises a single Container-n (n = 11, 12, 2, 3) plus the lower order Virtual Container POH appropriate to that level.

–	Higher order Virtual Container-n: VC-n (n = 3, 4)

This element comprises either a single Container-n (n = 3, 4) or an assembly of Tributary Unit Groups (TUG-2s or TUG-3s), together with Virtual Container POH appropriate to that level.

3.2.144	working channels: The channels over which normal traffic is transported when there are no switch events.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

ADM	Add-Drop Multiplexer

AI	Adapted Information

AIS	Alarm Indication Signal

AP	Access Point

APS	Automatic Protection Switching

AUG	Administrative Unit Group

AU-n	Administrative Unit, level n

BBE	Background Block Error

BBER	Background Block Error Ratio

BIP-X	Bit Interleaved Parity-X

CI	Characteristic Information

CP	Connection Point

DCC	Data Communications Channel

EB	Errored Block

ECC	Embedded Control Channel

ES	Errored Second

ESR	Errored Second Ratio

FS-R	Forced Switched normal traffic to Protection-Ring

HO	Higher Order

HRP	Hypothetical Reference Path

LO	Lower Order

LOF	Loss of Frame

LOS	Loss of Signal

MAF	Management Application Function

MCF	Message Communications Function

MD	Mediation Device

MF	Mediation Function

MI	Management Information

MO	Managed Object

MOC	Managed Object Class

MS	Multiplex Section

MSn_TT	Multiplex Section layer, level n_Trail Termination

MSOH	Multiplex Section Overhead

NC	Network Connection

NEF	Network Element Function

NNI	Network Node Interface

OHA	Overhead Access

OS	Operations System

OSF	Operations System Function

OSF	Outgoing Signal Fail

OSI	Open Systems Interconnection

PJE	Pointer Justification Event

POH	Path Overhead

RS	Regenerator Section

RSOH	Regenerator Section Overhead

SD	Signal Degrade

SDH	Synchronous Digital Hierarchy

SEP	Severely Errored Period

SEPI	Severely Errored Period Intensity

SES	Severely Errored Second

SESR	Severely Errored Second Ratio

SF	Signal Fail

SF-R	Signal Fail-Ring

Sm	Lower order VC-m layer (m = 11, 12, 2, 3)

Sm_TT	Lower order VC-m layer (m = 11, 12, 2, 3)_Trail Termination

SMN	SDH Management Network

SMS	SDH Management Subnetwork

Sn	higher order VC-n layer (n = 3, 4)

Sn_TT	higher order VC-n layer (n = 3, 4)_Trail Trace

SNC	Subnetwork Connection

SNCP	Subnetwork Connection Protection

SOH	Section Overhead

STM	Synchronous Transport Module

STM-N	Synchronous Transport Module-N

TMN	Telecommunications Management Network

TSI	TimeSlot Interchange

TT	Trail Termination function

TUG	Tributary Unit Group

TU-n	Tributary Unit-n

VC	Virtual Container

VC-n	Virtual Container-n

# 5	Conventions

All abbreviated references in the Definitions clause of this Recommendation are to ITU-T Recommendations.

Appendix I

List of source Recommendations

This text is an updated version of ITU-T Recommendation G.780 – "Vocabulary of Terms for Synchronous Digital Hierarchy (SDH) Networks and Equipment". The abbreviations and terms were taken from the Recommendations listed below. Where the definitions were not a part of an explicit Definitions clause of the source Recommendation, the source Recommendation is referenced in a Note following the definition. After this Recommendation is finally approved, Corrigenda or revisions to the original sources of these terms will be proposed to replace the definitions in those documents by references to this one (except where the definition is part of the source Recommendation text and not in a definitions clause). The end result should be a single normative definition for each term in this subject area, contained in this Recommendation.

| Recommendation | Latest version |
| --- | --- |
| G.707/Y.1322 | 12/03 |
| G.783 | 02/04 |
| G.784 | 07/99 |
| G.803 | 03/00 |
| G.826 | 12/02 |
| G.828 | 03/00 |
| G.841 | 10/98 |
| G.842 | 04/97 |

ITU-T Y-SERIES  RECOMMENDATIONS

GLOBAL INFORMATION INFRASTRUCTURE, INTERNET PROTOCOL ASPECTS AND NEXT GENERATION NETWORKS

| GLOBAL INFORMATION INFRASTRUCTURE |  |
| --- | --- |
| General | Y.100–Y.199 |
| Services, applications and middleware | Y.200–Y.299 |
| Network aspects | Y.300–Y.399 |
| Interfaces and protocols | Y.400–Y.499 |
| Numbering, addressing and naming | Y.500–Y.599 |
| Operation, administration and maintenance | Y.600–Y.699 |
| Security | Y.700–Y.799 |
| Performances | Y.800–Y.899 |
| INTERNET PROTOCOL ASPECTS |  |
| General | Y.1000–Y.1099 |
| Services and applications | Y.1100–Y.1199 |
| Architecture, access, network capabilities and resource management | Y.1200–Y.1299 |
| Transport | Y.1300–Y.1399 |
| Interworking | Y.1400–Y.1499 |
| Quality of service and network performance | Y.1500–Y.1599 |
| Signalling | Y.1600–Y.1699 |
| Operation, administration and maintenance | Y.1700–Y.1799 |
| Charging | Y.1800–Y.1899 |
| NEXT GENERATION NETWORKS |  |
| Frameworks and functional architecture models | Y.2000–Y.2099 |
| Quality of Service and performance | Y.2100–Y.2199 |
| Service aspects: Service capabilities and service architecture | Y.2200–Y.2249 |
| Service aspects: Interoperability of services and networks in NGN | Y.2250–Y.2299 |
| Numbering, naming and addressing | Y.2300–Y.2399 |
| Network management | Y.2400–Y.2499 |
| Network control architectures and protocols | Y.2500–Y.2599 |
| Security | Y.2700–Y.2799 |
| Generalized mobility | Y.2800–Y.2899 |

For further details, please refer to the list of ITU-T Recommendations.

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure, Internet protocol aspects and Next Generation Networks |
| Series Z | Languages and general software aspects for telecommunication systems |