|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.8011.1/Y.1307.1 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (08/2004) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital networks – General aspects |  |  |  |
|  | Ethernet private line service |  |  |  |
|  | CAUTION ! PREPUBLISHED RECOMMENDATION This prepublication is an unedited version of a recently approved Recommendation. It will be replaced by the published version after editing. Therefore, there will be differences between this prepublication and the published version. |  |  |  |

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g. interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met.  The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU [had/had not] received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2004

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

ITU-T Recommendation G.8011.1/Y.1307.1 Ethernet Private Line Service

Summary This Recommendation defines the service attributes and parameters for carrying Ethernet characteristic information over dedicated bandwidth, point-to-point connections, provided by SDH, PDH, ATM, MPLS, or OTH server layer networks. This type of service is referred to as Ethernet Private Line (EPL) service. The recommendation is base on the Ethernet service framework as defined in G.8011/Y.1307.

Source Draft Recommendation G.8011.1/Y.1307.1 has been developed in Q.12/15.

ITU-T Recommendation G.8011.1/Y.1307.1

Ethernet Private Line Service

(Geneva, 2004)

# Scope

This Recommendation defines the service attributes and parameters for carrying Ethernet characteristic information over dedicated-bandwidth, point-to-point connections, provided by SDH, ATM, MPLS, PDH, ETY or OTH server layer networks.  This type of services is referred to as Ethernet Private Line (EPL) service.  The recommendation is base on the Ethernet service framework as defined in G.8011/Y.1307.

# References

The following Recommendations and other references contain provisions that, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

- ITU-T G.707 (2003), Network node interface for the Synchronous Digital Hierarchy (SDH).

- ITU-T G.709 (2001), Interfaces for the optical transport network (OTN).

- ITU-T G.809 (2003), Functional architecture of connectionless layer networks.

- ITU-T G.8010/Y.1306 (2004), Ethernet Layer Network Architecture

- ITU-T G.8011/Y.1307 (2004), Ethernet Services Framework

- ITU-T G.7043/Y.1343 (2004), Virtual concatenation of Plesiochronous Digital Hierarchy (PDH) signals

- ITU-T G.8012/Y.1308 (2004), Ethernet UNI and Ethernet NNI

- ITU-T G.8021/Y.1341 (2004), Characteristics of Ethernet transport network equipment functional blocks

- IEEE 802.3-2002: IEEE Standard for Information technology - Telecommunications and information exchange between systems - IEEE standard for local and metropolitan area networks - Specific requirements – Part 3: Carrier Sense Multiple Access with Collision Detection (CSMA/CD) Access Method and Physical Layer Specifications.

- IEEE 802.3ae2002: Information technology—Telecommunications and information exchange between systems—Local and metropolitan area networks—Specific requirements—Part 3: Carrier sense multiple access with collision detection (CSMA/CD) access method and physical layer specifications.

- IEEE 802.3ah-2004:  Information technology—Telecommunications and information exchange between systems—Local and metropolitan area networks—Specific requirements—Part 3: Carrier sense multiple access with collision detection (CSMA/CD) access method and physical layer specifications – Media Access Control Parameters.  Physical Layers and Management Parameters for Subscriber Access Networks.

- IEEE 802.1X-2001:  IEEE Standard for Local and metropolitan area networks – Port-based Network Access Control.

# Terms and definitions

This Recommendation uses the following terms defined in Recommendation G.8010:

  - ETH link

This Recommendation uses the following terms defined in Recommendation G.8011/Y.1307:

  - Access link

  - Block

  - Committed Information Rate (CIR)

  - Customer

  - Dedicated

  - Ethernet service

  - Network access point

  - Pass

  - Process (with respect to L2 control protocol frames)

  - Service Instance

  - Spatial

This Recommendation uses the following terms defined in Recommendation G.809:

  - Flow domain

  - Flow domain flow

  - Flow Point

  - Flow termination

  - Link flow

  - Network flow

  - Termination flow point

  - Traffic Conditioning function

This Recommendation uses the following terms defined in Recommendation G.8012:

  - Ety-NNI

  - Ety-UNI

This Recommendation defines the following terms:

  - EPL Type 1 – A EPL Type 1 service carries the ETH_CI traffic units between two Ethernet UNIs.

  - EPL Type 2 – A EPL Type 2 service carries the information from the 8B/10B symbol stream between two Ethernet UNIs.

  - N/R = not relevant: clause/subclause, which is not relevant to this Recommendation

# Acronyms and abbreviations

This Recommendation uses the following abbreviations:

ATM	Asynchronous Transfer Mode

CBR	Constant Bit Rate

CBS	Committed Burst Size

CI 	Characteristic Information

CIR	Committed Information Rate

CO-CS	Connection Oriented circuit service

CO-PS	Connection Oriented packet service

CLPS	Connectionless packet service

DA	Destination Address

EC	Ethernet Connection

EIR	Excess information rate

EPL	Ethernet Private Line

ETH	Ethernet MAC layer network

ETH_CI 	Ethernet MAC Characteristic Information

ETY	Ethernet physical layer network

Ety-NNI	Ethernet over Transport NNI

Ety-UNI	Ethernet UNI ETYn	Ethernet physical layer network or order n

EVC	Ethernet Virtual Circuit

FCS	Frame Check Sequence

FD	Flow domain

GFP	Generic Framing Procedure

LACP	Link Aggregation Control Protocol

LCAS	Link Capacity Adjustment Scheme

MAC	Media Access Control

MEF	Metro Ethernet Forum

MPLS	Multi-Protocol Label Switching

NNI	Network to Network Interface

OAM	Operations, Administration, Maintenance

OTH	Optical Transport Hierarchy

OTN	Optical Transport Network

PA	(Ethernet) Preamble

PDH	Plesiochronous Digital Hierarchy

PHY	Physical device

SA	Source Address

SDH	Synchronous Digital Hierarchy

SDU	Service Data Unit

SFD	Start of Frame Delimiter

SNCP	Sub-Network Connection Protection

STP	Spanning Tree Protocol

UNI	User Network Interface

# Conventions

None.

# Ethernet Private Lines

## Description

An EPL service is a point-to-point service between two demarc points as illustrated in Figure 6-1. The service is provided over connection oriented server layer networks with a committed information rate (CIR). Note that if a CO-PS server layer is used traffic management is required to ensure that the CIR is maintained. The level of transparency of an EPL can be either:

Type 1 – Frame-based characteristic information (see example in Annex A)

Type 2 – Character-based characteristic information (see example in Annex B)

Figure 6-1/G.8011.1/Y.1307.1 – Ethernet Private Line

### EPL Type 1

The generic method to support the transport of Ethernet MAC frames (ETH service) between two Ethernet UNIs is to terminate the Ethernet physical section layer (ETY), extract the Ethernet MAC frames (ETH_CI), and transport them over the SDH, PDH, ETY, ATM, MPLS, or OTH network. This is referred to as EPL Type 1 and described below.  An example of EPL Type 1 is shown in Annex A

### EPL Type 2

A second type of service with lower latency characteristics is defined for the 8B/10B encoded 1 Gbit/s Ethernet interface signal. The 8B/10B symbol stream (ETC_CI) within the interface signal is encoded and transported through the transport network, instead of the Ethernet MAC frames. This is referred to as EPL Type 2, and is described in Annex B.

Note that the Ethernet MAC frame is not extracted in this type of service. It essentially provides transparency at the ETY layer.

## EPL Type 1 service architecture

The components used to support an EPL Type 1 service are shown in Figure 6-1:

- the Ety-UNIs (UNI-N, UNI-C)

- the Ety-NNI

- the Ethernet connection

- the Access link

The EPL Type 1 service uses a Ety-UNI and is supported over Ethernet-over-PDH, Ethernet-over-SDH, and Ethernet-over-OTH Ety-NNIs.  EPL Type 1 service can also use Ethernet-over-ATM with CIR and Ethernet-over-MPLS with CIR NNIs.  The UNI and NNI are specified in G.8012.

Figure 6-2 shows the basic architecture of the EPL Type 1 service. The ETY layer is terminated at the UNI-N and the ETH frames are forwarded over a single ETH_FP to the server layer.

Figure 6-2/G.8011.1/Y.1307.1 – Ethernet Private Line Type 1 architecture

# EPL Type 1 Service Characteristics

G.8011/Y.1307 defines connection, UNI and NNI attributes for Ethernet services. In the case of an EPL service some of these attributes have fixed values or a limited allowed range of parameters.

## 7.1	Ethernet Connection (EC) attributes

The Ethernet connection attributes are described in the following subclauses and are summarized in Table 7-1.  Interworking with bridge control frames and MAC control frames is described in Tables 8-3, 8-4, 8-5, 9-2, and 9-3 of G.8011.1/Y.1307.1.

Table 7-1/G.8011.1/Y.1307.1 – EC service attributes

| EC Service Attribute | Service Attribute Parameters and Values |
| --- | --- |
| Network connectivity | Point-to-point |
| Transfer characteristics | Address – deliver unconditionally Drop Precedence – not applicable |
| Link type | Dedicated |
| Customer separation | Spatial |
| Service instance separation | Spatial |
| Connectivity monitoring | Sub-layer monitoring: On demand, proactive, none Inherent monitoring: Proactive |
| Bandwidth profile | CIR, CBS |
| UNI list | Arbitrary text string to identify associated UNIs |
| Preservation | VLAN – yes CoS – yes |
| Survivability | None, Server specific |

### 7.1.1	Network Connectivity

The connectivity of EPL is point-to-point.

### 7.1.2	Transfer characteristics of ETH_CI

All Ethernet MAC data frames are transported regardless of their Destination Address.

### 7.1.3	Link Type

This type of link is referred to as dedicated as defined in G.8011/Y.1307 section 7.3.1.

### 7.1.4	Customer separation

EPL uses spatial separation between customer traffic as defined in G.8011/Y.1307 section 7.4.1.

### 7.1.5	Service instance separation

EPL uses spatial separation as defined in G.8011/Y.1307 section 7.5.1.

### 7.1.6	Connectivity Monitoring

Connectivity monitoring can either be proactive (sub-layer monitoring, inherent monitoring) or on-demand using tools to be specified in future Recommendations under study in SG13.  In some network implementations, the connectivity monitoring can rely on the server layer connectivity monitoring (inherent monitoring).  It is also optional to perform no monitoring.

### 7.1.7	Bandwidth Profile

The EPL service is characterised by two rate parameters, i.e. CIR and CBS.  The EPL traffic conditioning function requirement is minimal.  A circuit will be allocated inside the network based on the flow CIR.  An Ethernet flow may exceed its assigned rate at its own risk.  A customer may implement shaping in order to avoid frame loss due to statistical variations in traffic.  See Appendix II for more details.  As noted in 7.1.10, the possibility of reduced bandwidth during LCAS-based restoration is for further study.

### 7.1.8	UNI List

The UNI list is an arbitrary string administered by the Service Provider that is used to identify the UNIs connected to the EC. It is intended for management and control purposes.

### 7.1.9	Preservation

This attribute indicates the preservation of specific components of the ETH_CI provided by the ETH layer network that is used to transport the Ethernet service.  Preservation means that the parameter value will be the same on ingress and egress to the EC.  The two parameters are the ingress VLAN ID and class of service (priority) of the ETH_CI.  Both are preserved in EPL.

### 7.1.10	Survivability

The transport network can provide survivability for the EPL.  The survivability alternatives for the ETH link are, for example:

- No protection

- Protection by means of SDH or OTH or ATM or MPLS protection schemes

- Restoration by means of SDH or OTH or ATM or MPLS restoration schemes.

The applicability of survivability by means of LCAS (in which the ETH link operates at reduced bandwidth during the defect condition) is for further study.

# 8	EPL Type 1 UNI Attributes

## 8.1	ETH_UNI Attributes

This section describes Service UNI Attributes that modify the behaviour of a particular instance of an Ethernet service at the demarc of the UNI to characterize the service. There is a UNI defined at each of the ETH and ETY layers.  These are summarized in Table 8-1.

Table 8-1/G.8011.1/Y.1307.1 – UNI service attributes

|  | UNI Service Attribute | Service Attribute Parameters and Values |
| --- | --- | --- |
| ETH | MAC service | IEEE 802.3-2002 Frame format |
|  | Multiplexed Access | no |
|  | UNI ID | Arbitrary text string to identify each UNI instance |
|  | UNI EC ID | Arbitrary text string to identify each EC instance |
|  | VLAN ID mapping | no |
|  | Bundling | All-to-one |
|  | Bandwidth profile | CIR, CBS |
|  | Layer 2 Control Protocol Processing | Specified in Tables 8-3, 8-4, and 8-5 |
| ETY | PHY Speed | 10 Mbit/s, 100 Mbit/s, 1 Gbit/s or 10 Gbit/s |
|  | PHY Mode | Full duplex |
|  | PHY Medium | IEEE 802.3-2002 Physical Interface |

### 8.1.1	MAC Service

The EPL Type 1 UNI supports all 802.3 MAC frames.  From a service viewpoint, the FCS is passed through at the ETY_UNI.  If the FCS is bad (i.e., the frame is errored) at the ETY_UNI, the frame is dropped.

### 8.1.2	Multiplexed Access

This attribute indicates if the access to the Ethernet transport service is multiplexed (i.e., contains multiple service instances) or not.  EPL does not use multiplexed access.

### 8.1.3	UNI ID

The UNI ID is an arbitrary string administered by the Service Provider that is used to identify the UNI. It is intended for management and control purposes.

### 8.1.4	UNI EC ID

The UNI EC ID is an arbitrary string administered by the Service Provider that is used to identify an EC at the UNI. It is intended for management and control purposes.

### 8.1.5	VLAN ID Mapping

At the UNI there is a mapping of each customer VLAN ID to at most one EC.  For EPL, all VLAN IDs are mapped into the same EC.  VLAN ID mapping is not supported.

### 8.1.6	Bundling

When a UNI has the Bundling attribute, it is configurable so that more than one VLAN ID can map to an EC at the UNI. For EPL, bundling is all-to-one.

### 8.1.7	Bandwidth Profile

The bandwidth profile at the ETH_UNI is specified in 7.1.7.

### 8.1.8	Layer 2 Control Protocol Processing

L2 control frames may be passed, processed, generated, or blocked as specified in Tables 8-2, 8-3, and 8-4.  G.8011/Y.1307 describes these actions.

Table 8-2/G.8011.1/Y.1307.1 - Ingress (sink) 802 L2 Control protocols UNI processing

| MAC Address | Valid Actions | L2 Control Protocol |
| --- | --- | --- |
| 01-80-C2-00-00-00 | pass | STP, MSTP, RSTP |
| 01-80-C2-00-00-01 | See table 8-3 | MAC Control (PAUSE) |
| 01-80-C2-00-00-02 | See table 8-3 | Slow protocols |
| 01-80-C2-00-00-03 | pass | 802.1X Port Authentication |
| 01-80-C2-00-00-04 | pass | reserved address |
| 01-80-C2-00-00-05 | pass | reserved address |
| 01-80-C2-00-00-06 | pass | reserved address |
| 01-80-C2-00-00-07 | pass | reserved address |
| 01-80-C2-00-00-08 | pass | reserved address |
| 01-80-C2-00-00-09 | pass | reserved address |
| 01-80-C2-00-00-0A | pass | reserved address |
| 01-80-C2-00-00-0B | pass | reserved address |
| 01-80-C2-00-00-0C | pass | reserved address |
| 01-80-C2-00-00-0D | pass | reserved address |
| 01-80-C2-00-00-0E | pass | reserved address |
| 01-80-C2-00-00-0F | pass | reserved address |
| 01-80-C2-00-00-10 | pass | Bridge management |
| 01-80-C2-00-00-20 | pass | GARP – GMRP address |
| 01-80-C2-00-00-21 | pass | GARP – GVRP address |
| 01-80-C2-00-00-22 | pass | GARP – reserved address |
| 01-80-C2-00-00-23 | pass | GARP – reserved address |
| 01-80-C2-00-00-24 | pass | GARP – reserved address |
| 01-80-C2-00-00-25 | pass | GARP – reserved address |
| 01-80-C2-00-00-26 | pass | GARP – reserved address |
| 01-80-C2-00-00-27 | pass | GARP – reserved address |
| 01-80-C2-00-00-28 | pass | GARP – reserved address |
| 01-80-C2-00-00-29 | pass | GARP – reserved address |
| 01-80-C2-00-00-2A | pass | GARP – reserved address |
| 01-80-C2-00-00-2B | pass | GARP – reserved address |
| 01-80-C2-00-00-2C | pass | GARP – reserved address |
| 01-80-C2-00-00-2D | pass | GARP – reserved address |
| 01-80-C2-00-00-2E | pass | GARP – reserved address |
| 01-80-C2-00-00-2F | pass | GARP – reserved address |

Table 8-3/G.8011.1/Y.1307.1 - Ingress (sink) 802.3 L2 Control protocols UNI processing

| MAC Address | Ethertype | Subtype | Valid Actions | L2 Control Protocol |
| --- | --- | --- | --- | --- |
| 01-80-C2-00-00-01 or unicast | 88-08 | 0x0001 | block | MAC Control (PAUSE) |
| 01-80-C2-00-00-02 | 88-09 | 0x01, 0x02 | pass | Slow protocols – LACP, LAMP |

Table 8-4/G.8011.1/Y.1307.1 - Egress (source) 802.3 L2 Control protocols UNI processing

| MAC Address | Ethertype | Subtype | Valid Actions | L2 Control Protocol |
| --- | --- | --- | --- | --- |
| 01-80-C2-00-00-01 or unicast | 88-08 | 0x0001 | none or generate | MAC Control (PAUSE) |
| 01-80-C2-00-00-02 | 88-09 | 0x01, 0x02 | none | Slow protocols – LACP, LAMP |

## 8.2	ETY UNI Attributes

The ETY_UNI is a PHY characterized by speed, mode, and medium.  These attributes are described in G.8011/Y.1307.  The attributes that apply to EPL are specified as follows:

### 8.2.1	Speed

This attribute indicates the speed of Ethernet PHY layer that is used to transport the Ethernet service.  There are four values defined by G.8012: 10 Mbit/s, 100 Mbit/s, 1 Gbit/s or 10 Gbit/s.

### 8.2.2	Mode

This attribute indicates the mode of Ethernet PHY layer that is used to transport the Ethernet service.  EPL uses the full duplex mode.

### 8.2.3	Medium

This attribute indicates the medium of Ethernet PHY layer that is used to transport the Ethernet service.  The values are defined by G.8012.

# 9	EPL Type 1 NNI Attributes

## 9.1	ETH_NNI Attributes

Table 9-1/G.8011.1/Y.1307.1 – NNI service attributes

|  | NNI Service Attribute | Service Attribute Parameters and Values |
| --- | --- | --- |
| ETH | MAC service | IEEE 802.3-2002 Frame format |
|  | NNI ID | Arbitrary text string to identify each NNI instance |
|  | NNI EC ID | Arbitrary text string to identify each EC instance |
|  | VLAN mapping | Not applicable |
|  | Bundling | Not applicable |
|  | Bandwidth profile | CIR, CBS |
|  | Layer 2 Control Protocol Processing | Specified in Tables 9-2 and 9-3 |
| Server | Server layer | SDH, PDH, OTH, ETY, ATM, MPLS |

### 9.1.1	MAC Service

The EPL Type 1 NNI supports all 802.3 MAC frames.  All ETH_CI is passed.

### 9.1.2	NNI Identification

The NNI ID is an arbitrary string administered by the Service Provider that is used to identify the NNI. It is intended for management and control purposes.

### 9.1.3	NNI EC Identification

As per section 8.1.4

### 9.1.4	VLAN Mapping

Not applicable.

### 9.1.5	Bundling

Not applicable.

### 9.1.6	Bandwidth Profile

The bandwidth profile at the ETH_NNI is specified in 7.1.7.

### 9.1.7	Layer 2 Control Protocol Processing

L2 protocols are only visible at the NNI if it is an ETY.  In this case, the L2 control protocols can be passed, processed, blocked, or none (per G.8011/Y.1307).  All 802.1 L2 protocols as listed in G.8011/Y.1307 Table 8-2 are passed.  The 802.3 L2 protocols processing are shown in Tables 9-2 and 9-3.  Note that the action taken at the NNI should be consistent with action taken at the UNI.

Table 9-2/G.8011.1/Y.1307.1 - Ingress (sink) 802.3 L2 Control protocols NNI processing

| MAC Address | Ethertype | Subtype | Valid Actions | L2 Control Protocol |
| --- | --- | --- | --- | --- |
| 01-80-C2-00-00-01 or unicast | 88-08 | 0x0001 | pass | MAC Control (PAUSE) |
| 01-80-C2-00-00-02 | 88-09 | 0x01, 0x02 | pass | Slow protocols – LACP, LAMP |

Table 9-3/G.8011.1/Y.1307.1 - Egress (source) 802.3 L2 Control protocols NNI processing

| MAC Address | Ethertype | Subtype | Valid Actions | L2 Control Protocol |
| --- | --- | --- | --- | --- |
| 01-80-C2-00-00-01 or unicast | 88-08 | 0x0001 | none | MAC Control (PAUSE) |
| 01-80-C2-00-00-02 | 88-09 | 0x01, 0x02 | none | Slow protocols – LACP, LAMP |

## 9.2	Server Layer Adaptation

The server layers for EPL Type 1 are specified in Table 9-4.

Table 9-4/G.8011.1/Y.1307.1 – 
EPL Type 1 server layers

Server layer technology

SDH

OTH

PDH

MPLS

ATM

ETY

ANNEX A

EPL Type 1 Network Models

Figure A-1: EPL Access Link With Type 1 Mapping Into SDH

A G.8010 model of an EPL Type 1 is provided in Figure A-1. It shows:

- the adaptation of a MAC client to the ETH layer

- the ETH trail termination (TT) function

- an ETH flow domain

- the adaptation of ETH_CI to the ETY layer

- the ETY TT function

- an 802.3 access link to a provider edge device

- the corresponding ETY TT function

- the ETY/ETH adaptation function (including the component functions)

- a traffic conditioning (TC) function

- the adaptation of ETH_CI into an SDH path via GFP-F encapsulation

- the trail termination functions of a VC-n subnetwork

- the adaptation into virtually concatenated and LCAS controllable VC-n’s

- a VC-n subnetwork

Note that the replicate and filter blocks shown in Figure A-1 may be null functions for an EPL.

ANNEX B

EPL Type 2

## B.1	EPL Type 2 Service Characteristics

The EPL Type 2 service is essentially an ETY layer service and does not process the Ethernet MAC frames (ETH layer). The service attributes defined in G.8011/Y.1307 therefore do not apply to this service. The service is at the full rate of the specific Ethernet physical signal. Sub rate services are not supported.

Survivability can be provided using the server layer protection/restoration mechanisms.

## B.2	1 Gbit/s EPL Type 2 service

Figure B-1 shows the architectural model of this service. The Ethernet coding Sublayer is terminated and the decoded signal is mapped into a VC-4-7v using GFP-T mapping.

Figure B-1/G.8011.1/Y.1307.1 –EPL Type 2 architecture for a 1 Gbit/s Ethernet signal

ANNEX C

Special case of bit transport for 10 Gbit/s Ethernet

10 Gbit/s Ethernet is defined in:

- IEEE 802.3ae-2002: Information technology—Telecommunications and information exchange between systems—Local and metropolitan area networks—Specific requirements—Part 3: Carrier sense multiple access with collision detection (CSMA/CD) access method and physical layer:  Amendment: Media Access Control (MAC) Parameters, Physical Layers, and Management Parameters for 10 Gb/s Operation

The 10GBASE-W signal is basically a STM-64 with a VC-4-64c and the Ethernet MAC mapped into the VC-4-64c using a 64B/66B coding.  (NOTE – The transport and path overhead used by the 10GBASE-W is compatible with that specified in G.707 , however, the 10GBASE-W signal uses only a subset of this overhead.)

Figure C-1/G.8011.1/Y.1307.1 – Ethernet Private Line architecture for the case of an OTH network (10G WAN case)

The 10GBASE-W signal is specified to use a clock accuracy of 20 ppm.   With this client signal accuracy, the 10GBASE-W binary signal can be transported as a CBR10G signal via an OTH network as specified in Recommendation G.709  (Figure C-1).  ODU2 path monitoring is deployed to assess the quality of service. The functional architecture is shown in Figure C-1.

Figure C-2/G.8011.1/Y.1307.1 – Ethernet Private Line architecture for the case of a SDH network (10GBASE-W signal's clock accuracy complies with the SDH clock accuracy requirements)

The VC-4-64c of the 10 GbE WAN signal can also be transported over a SDH network in case the clock requirements are met (see Appendix XII/G.707). VC-4-64c tandem connection monitoring can be deployed to assess the quality of service.  The functional architecture is shown in Figure C-2.

NOTE:  If interface signal clock accuracy is 20 ppm, the 10GBASE-W signal can be transported through an SDH network, but excessive pointer adjustments may occur.

ANNEX D

Processing of l2 Control Frames at the ETY/ETH Adaptation

Table D-1/G.8011.1/Y.1307.1 - Processing of L2 control frames at the ETY/ETH_A_Sk adaptation 802.3 Protocols process for UNI or NNI application

| MAC Address | Ethertype | Subtype | Valid Actions | L2 Control Protocol |
| --- | --- | --- | --- | --- |
| 01-80-C2-00-00-02 | 88-09 | 0x03 | Process, block (See note) | Slow protocols – 802.3ah OAM |

G.8011/Y.1307 describes the actions “Process” and “block”.

Note: If there is no UNI/NNI process implementing the 802.3ah OAM then the valid action becomes pass for the ingress table. For the egress table it becomes none. As a consequence if the customer is sending OAM frames across the Access Link these frames will be received by the customer equipment at the other end of the private line. For example, such a situation may occur when there is no support for 802.3ah OAM on the provider edge devices, while there is support for 802.3ah OAM on the two end customer devices (i.e. at the end of each access link).

Table D-2/G.8011.1/Y.1307.1 - Processing of L2 control frames at the ETY/ETH_A_So adaptation 802.3 Protocols process for UNI or NNI application

| MAC Address | Ethertype | Subtype | Valid Actions | L2 Control Protocol |
| --- | --- | --- | --- | --- |
| 01-80-C2-00-00-02 | 88-09 | 0x03 | Generate, none (see note) | Slow protocols - 802.3ah OAM |

G.8011/Y.1307 describes the actions “Generate” and “none”.

appendix I

Customer View and Network View of Ethernet Services

## Introduction

This Recommendation describes Ethernet services from the network perspective.  Ethernet services may also be described from the customer perspective.

The network viewpoint may be used by a carrier to define its network and manage the services and facilities within that network.  A carrier may choose to expose these services to its customers in SLAs or choose to use them internally.

The customer viewpoint of a service is simply viewing the carrier network from the customer side.  None of the network configuration, topology or management is visible to the customer.  However, performance measurements may be used to infer carrier network specifics.

It is important to note that both views are valid for all Ethernet services, though they both need not be used.  In the majority of cases, where customer and network view services are both used it is important to understand that they are complementary.

## MEF – G.8011.1/Y.1307.1 comparison

Using the MEF as an example, the MEF E-Line Ethernet service type that is defined in the MEF Phase I documents can be implemented using the EPL infrastructure Ethernet services defined in G.8011.1/Y.1307.1.

This can be further refined with the review of the many possible service definitions that the MEF suggests is possible.  Only the first such service definition is explored in this Appendix.

## Ethernet private line implementation

The G.8011.1/Y.1307.1 EPL service can be viewed as a subset of the MEF Ethernet private line service of the E-line service type and show the mapping of G.8011.1/Y.1307.1attributes to MEF attributes.  That is, the EPL described in this Recommendation can be used to implement the MEF service.

The attributes of the MEF Ethernet private line service are shown in the following tables along with the matched ETH_CI attribute from the G.8011.1/Y.1307.1 EPL definition.

Table I.1/G.8011.1/Y.1307.1: E-Line Service type EVC service attribute requirements for MEF Ethernet Private Line implemented with G.8011.1/Y.1307.1 EPL

| MEF EVC Service Attribute | MEF Service Attribute Parameters and Values | G.8011/Y.1307 EC attribute | G.8011.1/Y.1307.1 Value |
| --- | --- | --- | --- |
| EVC Type | MUST be Point-to-Point | Network Connectivity | point-to-point |
|  |  | Link type | dedicated |
| UNI List | MUST list the two UNIs associated with the EVC. | UNI list | Arbitrary text string to identify the UNIs |
| CE-VLAN ID Preservation | MUST be Yes | Preservation - VLAN | Yes |
| CE-VLAN CoS Preservation | MUST be Yes | Preservation - CoS | Yes |
| Unicast Service Frame Delivery | MUST Deliver Unconditionally | Transfer characteristics - address | Deliver Unconditionally |
| Multicast Service Frame Delivery | MUST Deliver Unconditionally |  |  |
| Broadcast Service Frame Delivery | MUST Deliver Unconditionally |  |  |
| (Note a) | - | Transfer characteristics – drop precedence | Not applicable |
| Layer 2 Control Protocol Processing (only applies for L2CP passed to the EVC) | SHOULD discard PAUSE SHOULD tunnel LACP, LAMP, 802.1x MUST tunnel STP, RSTP, MSTP, All LANs Bridge Management Group, GARP | UNI L2 Control protocol processing | PAUSE – block LACP/LAMP – pass 33 reserved addresses – pass |
| EVC Performance | Only one CoS is REQUIRED. A CoS ID of <EVC> MUST be specified. Frame Delay, Frame Delay Variation and Frame Loss Ratio MUST be specified. | (Note 1) |  |
| (Note b) | - | Customer separation | spatial |
| (Note b) | - | Service instance separation | spatial |
| (Note c) | - | Connectivity monitoring | proactive, on demand |
| (Note c) | - | Survivability | None, Server specific |
| MEF notes: a. Not specified by MEF, but handled implicitly by EVC performance. b. Handled implicitly by EVC performance parameters that prevent the sharing of resources. c. No equivalent G.8011 notes: 1. Not defined in G.8011. Depends on server layer. |  |  |  |

Table I.2/G.8011.1/Y.1307.1: E-Line Service type UNI service attribute requirements for MEF Ethernet Private Line implemented with G.8011.1/Y.1307.1EPL

| MEFUNI Service Attribute | MEF Service Attribute Parameters and Values | G.8011/Y.1307 UNI attribute | G.8011.1/Y.1307.1 Value |
| --- | --- | --- | --- |
| UNI Identifier | Arbitrary text string to identify the UNI | UNI ID | Arbitrary text string to identify the UNI |
| Physical Medium | IEEE 802.3-2002 Physical Interface | PHY Medium | Defined in G.8012 |
| Speed | 10 Mbit/s, 100 Mbit/s, 1 Gbps or 10 Gbps | PHY Speed | 10 Mbit/s, 100 Mbit/s, 1 Gbit/s or 10 Gbit/s |
| Mode | MUST be Full Duplex | PHY Mode | Full Duplex |
| MAC Layer | IEEE 802.3-2002 | MAC Service | IEEE 802.3-2002 |
| Service Multiplexing | MUST be No | Multiplexed access | No |
| UNI EVC ID | Arbitrary text string to identify each EVC instance | UNI EC ID | Arbitrary text string to identify each EC |
| CE-VLAN ID / EVC Map | All CE-VLAN IDs at the UNI MUST map to the E-Line Service type EVC. | VLAN Mapping | No |
| Maximum number of EVCs | MUST be 1 | (Note 1) | - |
| Bundling | MUST be No | Bundling | all-to-one |
| All to One Bundling | MUST be Yes | Bundling | all-to-one |
| Ingress Bandwidth Profile Per Ingress UNI | CIR: MUST be <= UNI Speed CBS: MUST be > largest Service Frame size | EC Bandwidth Profile | CIR and CBS |
| Layer 2 Control Protocol Processing | SHOULD discard PAUSE SHOULD pass LACP, LAMP, 802.1x MUST pass STP, RSTP, MSTP, All LANs Bridge Management Group, GARP | L2 Control protocol processing (Note 2) | PAUSE – block LACP/LAMP – pass 33 reserved addresses – pass |
| NOTES: 1. EPL is defined as point-to-point service. 2. These are the ingress actions. Valid actions per protocol on ingress and egress are defined in Tables 8-2, 8-3, 8-4, and 8-5/G.8011.1/Y.1307.1. |  |  |  |

- appendix II

Traffic Conditioning

- II.1	Introduction

A complete Ethernet traffic conditioning Recommendation is under study in SG13.  In the interim, a description is provided in this Appendix.

- II.2	Traffic Conditioning

The objective of traffic conditioning is to determine the conformance of the incoming Ethernet frames. The level of conformance is expressed as one of two colors; Green or Red.

Compliance for a Bandwidth Profile is described by 2 parameters that are associated with two token bucket algorithms. The parameters are:

1. Committed Information Rate (CIR) expressed as bytes per second. CIR must be ≥ 0.

2. Committed Burst Size (CBS) expressed as bytes. CBS must be ≥ Maximum Ethernet frame allowed to enter the network.

For a sequence of ingress Ethernet frames, {tj,lj}j≥0, with arrival times tj and lengths lj, the color assigned to each frame during traffic conditioning is defined by using the algorithm shown in Figure I-1. For this algorithm, Bc = CBS and Tc = CBS/CIR.

Figure II-1/G.8011.1/Y.1307.1/Y.1307.1 - Traffic Conditioning Algorithm

Note that the algorithm does not define an implementation of any network equipment. Any implementation that results in the same conditioning results meets the requirement of this recommendation.

___________________