|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.8012/Y.1308 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (08/2004) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital networks – General aspects SERIES Y: GLOBAL INFORMATION INFRASTRUCTURE, INTERNET PROTOCOL ASPECTS AND NEXT GENERATION NETWORKS Internet protocol aspects – Transport |  |  |  |
|  | Ethernet UNI and Ethernet NNI |  |  |  |
|  | ITU-T Recommendation G.8012/Y.1308 |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| QUALITY OF SERVICE AND PERFORMANCE - GENERIC AND USER-RELATED ASPECTS | G.1000–G.1999 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.6000–G.6999 |
| DIGITAL TERMINAL EQUIPMENTS | G.7000–G.7999 |
| DIGITAL NETWORKS | G.8000–G.8999 |
| General aspects | G.8000–G.8099 |
| Design objectives for digital networks | G.8100–G.8199 |
| Quality and availability targets | G.8200–G.8299 |
| Network capabilities and functions | G.8300–G.8399 |
| SDH network characteristics | G.8400–G.8499 |
| Management of transport network | G.8500–G.8599 |
| SDH radio and satellite systems integration | G.8600–G.8699 |
| Optical transport networks | G.8700–G.8799 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.8012/Y.1308 Ethernet UNI and Ethernet NNI

Summary This Recommendation specifies the Ethernet UNI and the Ethernet NNI. A set of physical Ethernet interfaces is defined for the Ethernet UNI and the Ethernet NNI. Further, a set of Ethernet over Transport interfaces are defined for the Ethernet NNI. The Ethernet over Transport NNI uses various server layer networks like ATM, OTH, PDH and SDH.

Source ITU-T Recommendation G.8012/Y.1308 was approved on 22 August 2004 by ITU-T Study Group 15 (2001-2004) under the ITU-T Recommendation A.8 procedure.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g. interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met.  The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2004

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Terms and definitions		2

4	Acronyms and abbreviations		2

5	Conventions		5

6	Ethernet transport network interface structure		5

6.1	UNI and NNI basic signal structure		9

6.2	Information structure for the ETH interfaces		11

6.3	MAC address		19

6.4	Tag		20

7	Multiplexing/mapping principles		20

7.1	Mapping		21

7.2	ETH single level VID multiplex		21

7.3	ETH two level VID multiplex		21

8	Physical specification of the Ethernet interfaces		22

Annex A – ETC point-to-point connection		23

Appendix I – Ethernet multiplexing		24

I.1		ETH two level VLAN Multiplex		24

Appendix II – 64B/66B link frames over 10GBASE-W and STM-64		25

II.1	64B/66B link frame over STM-64		25

II.2	64B/66B link frame over 10GBASE-W		25

II.3	Differences between 64B/66B link frame over STM-64 and 10GBASE-W		25

BIBLIOGRAPHY		26

ITU-T Recommendation G.8012/Y.1308

Ethernet UNI and Ethernet NNI

# 1	Scope

This Recommendation specifies the Ethernet UNI and the Ethernet NNI, where the Ethernet UNI is formed by an Ethernet interface and the Ethernet NNI by an Ethernet interface or an Ethernet over Transport interface. The Ethernet over Transport NNI uses various server layer networks like ATM, OTH, PDH and SDH. The detailed requirements are specified in a number of ITU-T Recommendations, ANSI Standards, IEEE Standards and IETF RFC, which are referred to.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

–	ITU-T Recommendation G.691 (2003), Optical interfaces for single channel STM-64 and other SDH systems with optical amplifiers.

–	ITU-T Recommendation G.704 (1998), Synchronous frame structures used at 1544, 6312, 2048, 8448 and 44 736 kbit/s hierarchical levels.

–	ITU-T Recommendation G.707/Y.1322 (2003), Network node interface for the synchronous digital hierarchy (SDH).

–	ITU-T Recommendation G.709/Y.1331 (2003), Interfaces for the optical transport network (OTN).

–	ITU-T Recommendation G.832 (1998), Transport of SDH elements on PDH networks – Frame and multiplexing structures, plus Amendment 1 (2004), Payload type code for virtual concatenation of 34 368 kbit/s signals.

–	ITU-T Recommendation G.7041/Y.1303 (2003), Generic framing procedure (GFP).

–	ITU-T Recommendation G.7042/Y.1305 (2004), Link capacity adjustment scheme (LCAS) for virtual concatenated signals.

–	ITU-T Recommendation G.7043/Y.1343 (2004), Virtual concatenation of plesiochronous digital hierarchy (PDH) signals.

–	ITU-T Recommendation G.8010/Y.1306 (2004), Architecture of Ethernet layer networks.

–	ITU-T Recommendation G.8011/Y.1307 (2004), Ethernet over transport – Ethernet services framework.

–	ITU-T Recommendation G.8040/Y.1340 (2004), GFP frame mapping into plesiochronous digital hierarchy (PDH).

–	ITU-T Recommendation I.363.5 (1996), B-ISDN ATM Adaptation Layer specification: Type 5 AAL.

–	ITU-T Recommendation X.85/Y.1321 (2001), IP over SDH using LAPS, plus Amendment 1 (2004), Bit-oriented method for LAPS.

–	ITU-T Recommendation X.86/Y.1323 (2001), Ethernet over LAPS.

–	ITU-T Recommendation Y.1730 (2004), Requirements for OAM functions in Ethernet-based networks and Ethernet services.

–	IEEE Std 802-2001, IEEE Standard for Local and Metropolitan Area Networks: Overview and Architecture.

–	IEEE Std 802.1D-1998, Information Technology – Telecommunications and information exchange between systems – Local and metropolitan area networks – Common specifications – Media Access Control (MAC) Bridges.

–	IEEE Std 802.1Q-2003, IEEE standard for local and metropolitan area networks: Virtual Bridged Local Area Networks.

–	IEEE Std 802.2-1998, Information technology – Telecommunications and information exchange between systems – Local and metropolitan area networks – Specific requirements – Part 2: Logical Link Control.

–	IEEE Std 802.3-2002, Information technology – Telecommunication and Information Exchange Between Systems – LAN/MAN – Specific requirements – Part 3: Carrier Sense Multiple Access with Collision Detection (CSMA/CD) Access Method and Physical Layer Specifications.

–	IEEE Std 802.3ae-2002, IEEE Standard for Carrier Sense Multiple Access with Collision Detection (CSMA/CD) Access Method and Physical Layer Specifications – Media Access Control (MAC) Parameters, Physical Layer and Management Parameters for 10 Gb/s Operation.

–	IETF RFC 2684 (1999), Multiprotocol Encapsulation over ATM Adaptation Layer 5.

–	ANSI T1.107 (2002), Digital Hierarchy – Formats Specifications.

# 3	Terms and definitions

This Recommendation defines the following terms:

3.1	ETH_CI traffic unit: See ITU-T Rec. G.8010/Y.1306.

3.2	network termination: The network element in the transport network, which is connected to the customer edge equipment.

3.3	UNI: An interface that is used for the interconnection of customer equipment with a network element of the transport network.

3.4	Ety-UNI: An UNI for the transfer of ETH_CI traffic unit over a physical Ethernet interface.

3.5	NNI: An interface that is used for the interconnection of networks elements within a transport network.

3.6	EoT-NNI: An NNI for the transfer of ETH_CI traffic unit over a transport layer network referred to in this Recommendation.

3.7	Ety-NNI: An NNI for the transfer of ETH_CI traffic unit over a physical Ethernet interface.

# 4	Acronyms and abbreviations

This Recommendation uses the following abbreviations:

ANSI	American National Standards Institute

ATM	Asynchronous Transfer Mode

C-Tag	Customer Tag

C-VID	Customer VID

CE	Customer Edge

CI	Characteristic Information

DA	Destination Address

EoA	Ethernet over ATM

EoM	Ethernet over MPLS

EoO	Ethernet over OTH

EoP	Ethernet over PDH

EoR	Ethernet over RPR

EoS	Ethernet over SDH

EoT	Ethernet over Transport

EoT-NNI	Ethernet over Transport NNI

ETH	Ethernet MAC layer network

ETH_CI 	Ethernet MAC Characteristic Information

ETHP	ETH path layer

ETHS	ETH segment sublayer

ETY	Ethernet PHY layer

ETYn	Ethernet PHY layer network of type n

Ety-NNI	Ethernet NNI

Ety-UNI	Ethernet UNI

Ety-UNI-C	Customer side of the Ety-UNI

Ety-UNI-N	Network side of the Ety-UNI

EUG	Ethernet Unit Group

EUGn	EUG level n

FCS	Frame Check Sequence

GFP	Generic Framing Procedure

GFP-F	Generic Framing Procedure – Frame Mapped

IaDI	Intra-Domain Interface

IEEE	Institute of Electrical and Electronics Engineers

IETF	Internet Engineering Task Force

IrDI	Inter-Domain Interface

LAN	Local Area Network

LAPS	Link Access Procedure – SDH

LCAS	Link Capacity Adjustment Scheme

LLC	Logical Link Control

MAC	Media Access Control

M_SDU	MAC Service Data Unit

MPLS	Multi-Protocol Label Switching

MS64	Multiplex Section – level 64

NNI	Network Node Interface

NT	Network Termination

ODU	Optical Channel Data Unit

ODUj	Optical Channel Data Unit – order j

ODUj-Xv	Virtual concatenated Optical Channel Data Unit – order j

ODUk	Optical Channel Data Unit – order k

ODUk-Xv	Virtual concatenated Optical Channel Data Unit – order k

OTH	Optical Transport Hierarchy

P11s	1544 kbit/s PDH path layer with synchronous 125 s frame structure according to ITU-T Rec. G.704

P12s	2048 kbit/s PDH path layer with synchronous 125 s frame structure according to ITU-T Rec. G.704

P31s	34 368 kbit/s PDH path layer with synchronous 125 s frame structure according to ITU-T Rec. G.832

P4s	139 264 kbit/s PDH path layer with synchronous 125 s frame structure according to ITU-T Rec. G.832

PA	(Ethernet) Preamble

PDH	Plesiochronous Digital Hierarchy

PHY	Physical

PMA	Physical Medium Attachment sublayer

PMD	Physical Medium Dependent sublayer

RFC	Request for Comments

RPR	Resilient Packet Ring

RS64	Regenerator Section – level 64

S-Tag	Service Provider Tag

S-VID	Service Provider VID

SA	Source Address

SDH	Synchronous Digital Hierarchy

SFD	Start of Frame Delimiter

SNAP	Sub-Network Access Protocol

STM-N	Synchronous Transport Module – level N

TCI	Tag Control Information

UNI	User Network Interface

VC	Virtual Channel (ATM)

VC	Virtual Container (SDH)

VC-m	Lower Order VC – order m

VC-n	Higher Order VC – order n

VC-n-Xc	Contiguous concatenated VC – order n

VC-n-Xv	Virtual concatenated VC – order n

VID	VLAN Identifier

VLAN	Virtual LAN

VLAN ID	VLAN Identifier

# 5	Conventions

None.

# 6	Ethernet transport network interface structure

The Ethernet transport network as specified in ITU-T Rec. G.8010/Y.1306 implies two interface classes:

•	Ethernet interface as specified in IEEE 802.3 series;

•	Ethernet-over-Transport (EoT) interface as specified in this Recommendation.

The Ethernet interface can be deployed as an Ethernet User-to-Network Interface (Ety-UNI) at the edge of the transport network and as an Ethernet Network Node Interface (Ety-NNI) within the transport network. The Ethernet NNI may be deployed as an Intra-Domain Interface (IaDI) within a single administrative domain and as an Inter-Domain Interface (IrDI) between two administrative domains.

The Ethernet-over-Transport (EoT) interface can be deployed as an Ethernet IaDI and IrDI NNI within the transport network.

NOTE – The deployment of EoT interface as an Ethernet UNI is for further study.

The Ethernet UNI could be used for the provision of Ethernet services as described in ITU-T Rec. G.8011/Y.1307. Two or more Ety-UNIs are used for such a service as illustrated in Figure 6-1.

Figure 6-1/G.8012/Y.1308 – Locations of Ethernet Ety-UNI and NNI

The Ethernet UNI and NNI encompass multiple layer networks, each with its dedicated UNI and NNI (Figure 6-2).

Figure 6-2/G.8012/Y.1308 – Example of layer networks in Ethernet UNI and
NNI for case of pt-pt ETH layer connection

The Ethernet UNI could be used for the provision of an access link to a Service Node (SN), such as IP-Router, ASON switch, etc. In this case the Ety-UNI-C is terminated at the customer edge (CE) and the Ety-UNI-N is terminated at the NT. See Figure 6-3. It should be noted that a SN requires the support of SN specific protocols and may require the support of further layer networks. These SN related protocols and layer networks are outside the scope of this Recommendation and, therefore, not shown in the figure.

Figure 6-3/G.8012/Y.1308 – Locations of Ethernet Ety-UNI and
NNI of an access link to an SN

The Ethernet UNI and NNI encompass multiple layer networks, each with its dedicated UNI and NNI (Figure 6-4).

Figure 6-4/G.8012/Y.1308 – Example of layer networks in Ethernet UNI and NNI 
for case of access link to a client layer service node

The Ethernet connection may also be between two service nodes as shown in Figure 6-5.

Figure 6-5/G.8012/Y.1308 – EoT-NNI between service nodes

The Ethernet NNI encompasses multiple layer networks, each with its dedicated NNI (Figure 6-6).

Figure 6-6/G.8012/Y.1308 – Example of layer networks in Ethernet NNI 
for case of interconnecting two ETH layer service nodes

Figure 6-7 shows the use of the Ethernet NNI (Ety-NNI or EoT-NNI) as an IaDI and an IrDI.

Figure 6-7/G.8012/Y.1308 – Locations of Ethernet UNI and NNI in a multi-operator network

This Recommendation specifies the ETH UNI, ETH NNI including the encapsulation into the ETH's server layer (SrvA in Figures 6-2, 6-4 and 6-6).

The Ethernet UNI and NNI may carry informational elements of three planes (Figure 6-8):

–	Data (or User) Plane, optionally including a Data Communication Network (DCN) supporting management plane and control plane communications;

–	Control Plane (e.g., related to BPDUs, ASON etc.);

–	Management Plane.

Figure 6-8/G.8012/Y.1308 – Three planes of Ethernet UNI and NNI

Each UNI or NNI is divided into three plane specific UNI/NNIs:

–	UNID and NNID for the data plane informational elements, including OAM which is terminated in the layer's termination, adaptation and connection/flow forwarding functions;

–	UNIC and NNIC for the control plane informational elements;

–	UNIM and NNIM for the management plane informational elements.

UNIC, UNIM and NNIC are optional elements within a UNI and NNI.

This Recommendation specifies the UNID and NNID.

## 6.1	UNI and NNI basic signal structure

The basic structure is shown in Figure 6-9. A client signal of the ETH layer network is mapped into the M_SDU by one of the four different encapsulations illustrated by the vertical line at the top of Figure 6-9.

Figure 6-9/G.8012/Y.1308 – Structure of the ETH interfaces

### 6.1.1	ETH substructure

The ETH layer as defined in ITU-T Rec. G.8010/Y.1306 is further structured in sublayers in order to support the network management and supervision functionalities defined in ITU-T Recs G.8010/Y.1306 and Y.1730:

–	segment monitoring (ETHS);

–	end-to-end path supervision (ETHP);

–	optional adaptation of client signals via Logical Link Control (LLC) and Sub-Network Access Protocol (SNAP).

### 6.1.2	Ethernet structure

The Ethernet interface consists of the Ethernet Physical (ETY) layer.

### 6.1.3	Ethernet-over-Transport structure

The Ethernet-over-Transport interface consists of the multiple layers, of which only the first one is illustrated in Figure 6-9. The next layers are outside the scope of this Recommendation; the reader is referred to the appropriate technology Recommendations (e.g., ITU-T Rec. G.707/Y.1322 for SDH).

There are a number of such EoT interfaces defined under this Recommendation as depicted in Figure 6-9:

•	Ethernet-over-PDH (EoP);

•	Ethernet-over-SDH (EoS);

•	Ethernet-over-OTH (EoO);

•	Ethernet-over-ATM (EoA);

•	Ethernet-over-MPLS (EoM);

•	Ethernet-over-RPR (EoR).

In the case of circuit switched transport, the payload bandwidths available are shown in Tables 6-1, 6-2 and 6-3, respectively, for PDH, SDH and OTH.

Table 6-1/G.8012/Y.1308  Bandwidth of the payload of PDH path signals

| PDH type | PDH payload (kbit/s) | In steps of (kbit/s) |
| --- | --- | --- |
| P11s | 1536 – (64/24)  1533 |  |
| P12s | 1980 |  |
| P31s | 33 856 |  |
| P32e | 4696/4760 * 44 736  44 134 |  |
| P11s-Xv, X = 1 to 16 |  1533 to  24 528 |  1533 |
| P12s-Xv, X = 1 to 16 | 1980 to 31 680 | 1980 |
| P31s-Xv, X = 1 to 8 | 33 856 to 270 848 | 33 856 |
| P32e-Xv, X = 1 to 8 |  44 134 to  353 072 |  44 134 |

Table 6-2/G.8012/Y.1308  Bandwidth of the payload of SDH VCs

| VC type | VC payload (kbit/s) | In steps of (kbit/s) |
| --- | --- | --- |
| VC-11 | 1600 |  |
| VC-12 | 2176 |  |
| VC-2 | 6784 |  |
| VC-3 | 48 384 |  |
| VC-4 | 149 760 |  |
| VC-4-4c | 599 040 |  |
| VC-4-16c | 2 396 160 |  |
| VC-4-64c | 9 584 640 |  |
| VC-4-256c | 38 338 560 |  |
| VC-11-Xv, X = 1 to 64 | 1600 to 102 400 | 1600 |
| VC-12-Xv, X = 1 to 64 | 2176 to 139 264 | 2176 |
| VC-2-Xv, X = 1 to 64 | 6784 to 434 176 | 6784 |
| VC-3-Xv, X = 1 to 256 | 48 384 to 12 386 304 | 48 384 |
| VC-4-Xv, X = 1 to 256 | 149 760 to 38 338 560 | 149 760 |

Table 6-3/G.8012/Y.1308  Bandwidth of the OTH ODUs

| ODU type | OPU payload (kbit/s) | In steps of (kbit/s) |
| --- | --- | --- |
| ODU1 | 2 488 320 |  |
| ODU2 | 238/237  9 953 280  9 995 277 |  |
| ODU3 | 238/236  39 813 120  40 150 519 |  |
| ODU1-Xv, X = 1 to 256 | 2 488 320 to 637 009 920 | 2 488 320 |
| ODU2-Xv, X = 1 to 256 |  9 995 277 to  2 558 709 902 |  9 995 277 |
| ODU3-Xv, X = 1 to 256 |  40 150 519 to  10 278 532 946 |  40 150 519 |

## 6.2	Information structure for the ETH interfaces

The information structure for the ETH interfaces is represented by information containment relationships and flows. The principal information containment relationships are described in Figure 6-10.

### 6.2.1	ETH principle information containment relationship

The ETH_CI consists of a MAC Destination Address (DA), a MAC Source Address (SA), a MAC Service Data Unit (M_SDU) (see ITU-T Rec. G.8010/Y.1306 and IEEE 802.3ae, clause 2). The M_SDU may optionally include a Tag (see IEEE 802.1Q). A client signal of the ETH layer network is mapped into the M_SDU via one of four different encapsulations (see Figures 6-10 and 7-1):

–	type encapsulation;

–	length/LLC encapsulation;

–	length/LLC/SNAP encapsulation;

–	length/LLC/SNAP/Type encapsulation.

Table 6-4/G.8012/Y.1308  Overview of encapsulated units

| Encapsulation type | Reference |
| --- | --- |
| LLC/SNAP/EtherType client encapsulation | IEEE 802, subclause 10.5 RFC 2684, subclause 5.1 |
| LLC/SNAP/PID client encapsulation | IEEE 802, subclause 10.3, RFC 2684, subclause 5.2 |
| LLC encapsulated SNAP client | IEEE 802.2, clause 3, RFC 2684, subclause 5.1 |
| LLC encapsulated non-SNAP client | IEEE 802.2, clause 3 |
| Length encapsulated LLC client | IEEE 802.3, clause 3 |
| EtherType encapsulated client | IEEE 802.3, clause 3 |
| Tag | IEEE 802.1Q, subclause 9.3.2 |
| ETH encapsulated MAC SDU | IEEE 802.3, clause 3 IEEE 802.3ae, clause 2 |
| MAC encapsulated ETH | IEEE 802.3, clause 3 |

|  |  |  |  |  |  |  |  | 00-00-00 | Type | Client data | LLC/SNAP/EtherType encapsulated client |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | SNAP OUI | SNAP Pid | Client data | LLC/SNAP/PID encapsulated client |  |
|  |  |  |  |  | 0xAA | 0xAA | 0x03 | SNAP data |  |  | LLC encapsulated SNAP client |  |
|  |  |  |  |  | DSAP | SSAP | Ctrl | Client data |  |  | LLC encapsulated non-SNAP client |  |
|  |  |  |  | Length | Logical Link Control data |  |  |  |  |  | Length encapsulated LLC client |  |
|  |  |  |  | Type | Client data |  |  |  |  |  | EtherType encapsulated client |  |
|  |  |  | TAG | MAC Service Data Unit |  |  |  |  |  |  | Tag (optional) |  |
|  | DA | SA | MAC Service Data Unit |  |  |  |  |  |  |  | ETH_CI |  |
| Link header | ETH_CI |  |  |  |  |  |  |  |  |  | Link trailer | Link frame |

Figure 6-10/G.8012/Y.1308 – Ethernet principle information containment

#### 6.2.1.1	ETY link frame

The ETH_CI is extended with a MAC FCS field and a Preamble (PA) and Start of Frame Delimiter (SFD). See Figure 6-11.

|  |  | DA | SA | MAC Service Data Unit | ETH_CI |  |
| --- | --- | --- | --- | --- | --- | --- |
| PA | SFD | ETH_CI |  |  | MAC FCS | MAC frame |

Figure 6-11/G.8012/Y.1308 – ETY link frame

#### 6.2.1.2	GFP-F link frame

The ETH_CI is extended with a MAC FCS field and then mapped as specified in ITU-T Rec. G.7041/Y.1303 in the GFP Payload Information Field. A core Header with PLI and cHEC fields and a Payload Header field with subfield is prepended. The PTI subfield has value 000, the PFI subfield has value 0, the EXI subfield has value 0000 and the UPI subfield has value 0x01. See Figure 6-12. The maximum size of the GFP payload information field is specified in 6.1.2/G.7041/Y.1303.

|  |  | DA |  | SA | MAC Service Data Unit | ETH_CI |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ETH_CI |  |  |  | MAC FCS |  |
| Core header | Payload header | GFP payload information field |  |  |  |  | GFP-F |
| PLI cHEC | PTI = 000 PFI = 0 EXI = 0000 UPI = 0x01 tHEC |  |  |  |  |  |  |

Figure 6-12/G.8012/Y.1308 – GFP-F link frame

#### 6.2.1.3	64B/66B link frame

The ETH_CI is extended with a MAC FCS field and a Preamble (PA) and Start of Frame Delimiter (SFD) as shown in Figure 6-13. For encapsulation the MAC frame is further extended with an S, T and idles. The coding of S, T and idles and the minimum number of idles is specified in IEEE 802.3ae.

|  |  |  | DA | SA | MAC Service Data Unit | ETH_CI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PA | SFD | ETH_CI |  |  | MAC FCS | MAC frame |  |  |
| S | MAC frame |  |  |  |  |  | T | Idle | 64B/66B |

Figure 6-13/G.8012/Y.1308 – 64B/66B link frame

#### 6.2.1.4	LAPS link frame

The ETH_CI is extended with a MAC FCS field and then mapped as specified in ITU-T Rec. X.86/Y.1323 in the LAPS Information Field. An Address field with value 0x04, a Control field with value 0x03, a SAPI field with value 0xfe01 are prepended and a LAPS FCS field is appended. See Figure 6-14.

|  |  |  | DA | SA | MAC Service Data Unit | ETH_CI |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ETH_CI |  |  | MAC FCS |  |  |
| Addr (0x04) | Ctrl 0x03 | SAPI (0xfe01) | LAPS information field |  |  |  | FCS | LAPS |

Figure 6-14/G.8012/Y.1308 – LAPS link frame

#### 6.2.1.5	AAL5 CPCS-PDU link frame

##### 6.2.1.5.1	LLC encapsulation based without MAC FCS

The ETH_CI is mapped as specified in RFC 2684 in the LLC encapsulation based AAL5 CPCS-PDU Payload field after Padding and SNAP and LLC encapsulation, optionally padded with an AAL5 CPCS-PDU PAD field. The LLC header value is fixed to 0xAA-AA-03. The SNAP header is fixed to 0x00-80-C2-00-07. A CPCS-UU field with undefined value, a CPI field with value 0x00, Length field and a CRC-32 field are added to complete the AAL5 CPCS-PDU as specified in ITU-T Rec. I.363.5. See Figure 6-15.

|  |  |  |  |  |  | DA | SA | MAC Service Data Unit | ETH_CI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SNAP OUI (0x00-80-C2) | SNAP Pid (0x00-07) | PAD (0x00-00) | Information |  |  | SNAP encapsulation |  |  |  |  |  |
| DSAP (0xAA) | SSAP (0xAA) | Control (0x03) | Information |  |  |  |  |  | LLC encapsulation |  |  |  |  |  |
| CPCS-PDU payload |  |  |  |  |  |  |  |  | PAD | CPCS UU | CPI | Length | CRC-32 | AAL5 CPCS-PDU |

Figure 6-15/G.8012/Y.1308 – LLC encapsulation based AAL5 CPCS-PDU
link frame without MAC FCS preservation

##### 6.2.1.5.2 	LLC encapsulation based with MAC FCS

The ETH_CI is extended with a MAC FCS field and then mapped as specified in RFC 2684 in the LLC encapsulation based AAL5 CPCS-PDU Payload field after Padding and SNAP and LLC encapsulation, optionally padded with an AAL5 CPCS-PDU PAD field. The LLC header value is fixed to 0xAA-AA-03. The SNAP header is fixed to 0x00-80-C2-00-01. A CPCS-UU field with undefined value, a CPI field with value 0x00, Length field and a CRC-32 field are added to complete the AAL5 CPCS-PDU as specified in ITU-T Rec. I.363.5. See Figure 6-16.

|  |  |  |  |  |  | DA | SA | MAC Service Data Unit |  | ETH_CI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | ETH_CI |  |  |  | MAC FCS |  |  |  |  |  |  |  |  |  |
|  |  |  | SNAP OUI (0x00-80-C2) | SNAP Pid (0x00-01) | PAD (0x00-00) | Information |  |  |  |  | SNAP encapsulation |  |  |  |  |  |  |  |  |
| DSAP (0xAA) | SSAP (0xAA) | Control (0x03) | Information |  |  |  |  |  |  |  | LLC encapsulation |  |  |  |  |  |  |  |  |
| CPCS-PDU payload |  |  |  |  |  |  |  |  |  |  | PAD | CPCS UU |  | CPI |  | Length |  | CRC-32 | AAL5 CPCS-PDU |

Figure 6-16/G.8012/Y.1308 – LLC encapsulation based AAL5 CPCS-PDU 
link frame with MAC FCS preservation

##### 6.2.1.5.3	VC-multiplexing based without MAC FCS

The ETH_CI is mapped as specified in RFC 2684 in the VC-multiplexing based AAL5 CPCS-PDU Payload field optionally padded with an AAL5 CPCS-PDU PAD field. A CPCS-UU field with undefined value, a CPI field with value 0x00, Length field and a CRC-32 field are added to complete the AAL5 CPCS-PDU as specified in ITU-T Rec. I.363.5. See Figure 6-17.

| DA | SA | MAC Service Data Unit |  | ETH_CI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CPCS-PDU payload |  |  | PAD |  | CPCS UU | CPI | Length | CRC-32 | AAL5 CPCS-PDU |

Figure 6-17/G.8012/Y.1308 – VC-multiplexing based AAL5CPCS-PDU
link frame without MAC FCS preservation

##### 6.2.1.5.4	VC-multiplexing based with MAC FCS

The ETH_CI is extended with a MAC FCS field and then mapped as specified in RFC 2684 in the VC-multiplexing based AAL5 CPCS-PDU Payload field optionally padded with an AAL5 CPCS-PDU PAD field. A CPCS-UU field with undefined value, a CPI field with value 0x00, Length field and a CRC-32 field are added to complete the AAL5 CPCS-PDU as specified in ITU-T Rec. I.363.5. See Figure 6-18.

| DA | SA | MAC Service Data Unit | ETH_CI |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ETH_CI |  |  | MAC FCS |  |  |  |  |  |  |
| CPCS-PDU payload |  |  |  | PAD | CPCS UU | CPI | Length | CRC-32 | AAL5 CPCS-PDU |

Figure 6-18/G.8012/Y.1308 – VC-multiplexing based AAL5 CPCS-PDU link frame 
with MAC FCS preservation

#### 6.2.1.6	MPLS link frame

For further study.

#### 6.2.1.7	RPR link frame

For further study.

### 6.2.2	Ethernet UNI

The Ethernet UNI is supported by the interfaces listed below.

NOTE – The Ethernet UNI may also be supported by a 10GBase-W interface using the link frame specified in 6.2.1.3.

#### 6.2.2.1	Ety-UNI

The Ety-UNI deploys the ETY link frame as specified in 6.2.1.1.

#### 6.2.2.2	EoS UNI

The Ethernet over SDH (EoS) UNI deploys the 64B/66B link frame as specified in 6.2.1.3. The mapping of the 64B/66B encoded bit stream is specified in Annex F/G.707/Y.1322. Path overhead of the VC-4-64c is specified in ITU-T Rec. G.707/Y.1322. The components of the Ethernet over SDH UNI are illustrated in Figure 6-22.

NOTE 1 – A 10GBASE-W interface can be deployed as an EoS UNI when its clock accuracy is ±4.6 ppm or better. Refer to Appendix II.

NOTE 2 – The deployment of other EoT interfaces as an Ethernet UNI is for further study.

### 6.2.3	Ethernet NNI

The Ethernet NNI is supported by the interfaces listed below.

#### 6.2.3.1	Ety-NNI

The Ethernet NNI deploys the ETY link frame as specified in 6.2.1.1.

#### 6.2.3.2	EoP NNI

The Ethernet over PDH NNI deploys either the GFP-F link frame as specified in 6.2.1.2, or the LAPS link frame as specified in 6.2.1.4. The mapping of the GFP-F link frames into P11s/P11s-Xv, P12s/P12s-Xv, P31s/P31s-Xv and P32e/P32-Xv is specified in ITU-T Rec. G.8040/Y.1340. The mapping of the LAPS link frames into P11s, P12s, P31s and P4s is specified in ITU-T Rec. X.85/Y.1321.

The frame structure of P11s, P12s, and P32e is specified in ITU-T Rec. G.704 and the frame structure of P12s is specified in ITU-T Rec. G.832. Virtual concatenation of these PDH signals is specified in ITU-T Rec. G.7043/Y.1343.

For the channelized P32e the direct multiplexing of P11s into P32 is specified in ANSI T1.107, subclause 9.3.

For interworking at administrative domain boundaries between GFP-F and LAPS link frames the following rule applies:

–	At international boundaries, or at the boundaries between the networks of different operators, the GFP-F encapsulation defined in 6.2.1.2 shall be used unless otherwise mutually agreed by the operators providing the transport. Within a national network or within the domain of a single operator, LAPS encapsulation as defined in 6.2.1.4 may be used.

Figure 6-19 illustrates the relation of the components of Ethernet over PDH NNI using GFP-F encapsulation and Figure 6-20 the ones using LAPS.

Figure 6-19/G.8012/Y.1308 – Components of the Ethernet over PDH NNI using GFP-F

Figure 6-20/G.8012/Y.1308 – Components of the Ethernet over PDH NNI using LAPS

#### 6.2.3.3	EoS NNI

The Ethernet over SDH (EoS) NNI deploys either the GFP-F link frame as specified in 6.2.1.2, or the LAPS link frame as specified in 6.2.1.4, or for the case of a VC-4-64c supported ETH topological link the 64B/66B link frame as specified in 6.2.1.3. The mapping of the GFP-F link frames and the mapping of LAPS link frames into V-C11/VC-11-Xv, VC-12/VC-12-Xv, VC-3/VC-3-Xv, VC-4/VC-4-Xv and VC-4-Xc is specified in 10.6 and 10.3 respectively of ITU-T Rec. G.707/Y.1322. The mapping of the 64B/66B encoded bit stream is specified in Annex F/G.707/Y.1322.

NOTE 1 – A 10GBASE-W interface can be deployed as an EoS NNI when its clock accuracy is ±4.6 ppm or better. Refer to Appendix II.

NOTE 2 – The special case of using a GFP-T link for 1 Gbit/s Ethernet (i.e., Type 2 Ethernet private line service as described in ITU-T Rec. G.8011.1/Y.1307.1) is specified in Annex A.

Path overhead and virtual concatenation of the VCs is specified in ITU-T Rec. G.707/Y.1322.

For interworking at administrative domain boundaries between GFP-F and LAPS and between GFP-F and 64B/66B link frames the following rules apply:

–	GFP-F  LAPS: At international boundaries, or at the boundaries between the networks of different operators, the GFP-F encapsulation defined in 6.2.1.2 shall be used unless otherwise mutually agreed by the operators providing the transport. Within a national network or within the domain of a single operator, LAPS encapsulation as defined in 6.2.1.4 may be used.

–	GFP-F  64B/66B: At international boundaries, or at the boundaries between the networks of different operators, the 64B/66B encapsulation defined in 6.2.1.3 shall be used unless otherwise mutually agreed by the operators providing the transport. Within a national network or within the domain of a single operator, GFP-F encapsulation as defined in 6.2.1.2 may be used.

–	LAPS  64B/66B: At international boundaries, or at the boundaries between the networks of different operators, the 64B/66B encapsulation defined in 6.2.1.3 shall be used unless otherwise mutually agreed by the operators providing the transport. Within a national network or within the domain of a single operator, LAPS encapsulation as defined in 6.2.1.4 may be used.

The components of the Ethernet over SDH NNI using the default encapsulation are illustrated in Figures 6-21 and 6-22.

Figure 6-21/G.8012/Y.1308 – Components of the Ethernet over SDH NNI 
using GFP-F or LAPS encapsulation

Figure 6-22/G.8012/Y.1308 – Components of the Ethernet over SDH UNI and NNI 
when using 64B/66B encapsulation

#### 6.2.3.4	EoO NNI

The Ethernet over OTH NNI deploys the GFP-F link frame as specified in 6.2.1.2 and its components are illustrated in Figure 6-23. The mapping of the GFP-F link frame into ODUj/ODUk and ODUj-Xv is specified in 17.3 and 18.2.4 respectively of ITU-T Rec. G.709/Y.1331.

Path overhead and virtual concatenation of the ODUs is specified in ITU-T Rec. G.709/Y.1331.

Figure 6-23/G.8012/Y.1308 – Components of the Ethernet over OTH NNI

#### 6.2.3.5	EoA NNI

The Ethernet over ATM NNI deploys either the LLC-encapsulation, or VC-multiplexing based AAL5 CPCS-CPU link frame as specified in 6.2.1.5. The mapping of AAL5 CPCS-CPU link frame into a VC is specified in ITU-T Rec. I.363.5.

The components of the Ethernet over ATM NNI are illustrated in Figure 6-24.

Figure 6-24/G.8012/Y.1308 – Components of the Ethernet over ATM NNI

#### 6.2.3.6	EoM NNI

For further study.

#### 6.2.3.7	EoR NNI

For further study.

## 6.3	MAC address

The Ethernet frame includes two 48-bit MAC addresses: MAC Destination Address (DA) and MAC Source Address (SA) (as specified in IEEE 802.3). Any of these addresses could occur at the Ety-UNI and NNI.

The set of 248 MAC addresses (Figure 6-25) is divided into two main subsets (IEEE 802, clause 9):

–	247 individual MAC addresses (referred to as unicast).

–	247 group MAC addresses (referred to as multicast).

One of the group MAC addresses is defined as a:

–	broadcast MAC address (FF-FF-FF-FF-FF-FF).

Thirty-three of the group MAC addresses are defined as control frames (IEEE 802.1D, IEEE 802.1Q):

–	All bridges address (01-80-C2-00-00-10).

–	Reserved addresses (01-80-C2-00-00-00 to 01-80-C2-00-00-0F).

–	GARP Application addresses (01-80-C2-00-00-20 to 01-80-C2-00-00-2F).

Figure 6-25/G.8012/Y.1308 – MAC address structure

## 6.4	Tag

The Ethernet frame optionally includes a Tag as specified in IEEE 802.1Q, clause 9. The Tag control information (TCI) contains a 3-bit user priority field that can take 8 values 0 to 7.

The TCI also includes a VLAN ID as specified in IEEE 802.1Q, clause 9. Note that VLAN ID is defined to be between 1 and 4094. A VLAN ID = 0, indicates the Tag that only contains user priority. The CFI bit of the TCI field is ignored.

These tags are present at the Ety-UNI, the Ety-NNI and the EoT-NNI.

# 7	Multiplexing/mapping principles

Figure 7-1 shows the relationship between various information structure elements and illustrates the multiplexing structure and mappings for the ETH from client signal to link frames.

Figure 7-1/G.8012/Y.1308 – Ethernet mapping and multiplexing

## 7.1	Mapping

The client signal is mapped into the ETH signal (frame) directly via TYPE encapsulation, or indirectly via LENGTH encapsulation. With LENGTH encapsulation one or more additional encapsulation steps are present: a Logical Link Control (LLC) with SAP encapsulation, a LLC with Sub-Network Access Protocol (SNAP) and PID encapsulation, or a LLC/SNAP with TYPE encapsulation. Refer to 6.2.1.

The ETH signal (frame) is then mapped into the applicable link frame and those link frames are transported over an ETH topological link.

## 7.2	ETH single level VID multiplex

Figure 7-2 illustrates a single level multiplexing of up to 4094 ETH signals into an ETH topological link. The ETH_CI traffic unit is for that purpose extended with a C-Tag including a VLAN Identifier (C-VID) (Figure 7-3), as specified in IEEE 802.1Q and then multiplexed into an Ethernet Unit Group level 1. This multiplexing structure is applicable for the Ethernet UNI and the NNI.

Figure 7-2/G.8012/Y.1308 – Ethernet single level multiplexing

Figure 7-3/G.8012/Y.1308 – Ethernet single level multiplexing method

## 7.3	ETH two level VID multiplex

The specification of a two-level multiplexing is currently under development in project IEEE P802.1ad. See Appendix I.

# 8	Physical specification of the Ethernet interfaces

References for the physical characteristics of the Ety-UNI and Ety-NNI are given in Table 8-1 and for EoT-UNI and EoT-NNI in Table 8-2.

Table 8-1/G.8012/Y.1308  Ety interfaces for Ethernet over Transport

| Ethernet interface | Reference |
| --- | --- |
| 10BASE-T | IEEE 802.3, clause 14 |
| 100BASE-T | IEEE 802.3, clause 25 |
| 1000BASE-SX | IEEE 802.3, clause 38 |
| 1000BASE-LX | IEEE 802.3, clause 38 |
| 10GBASE-SR | IEEE 802.3ae, clauses 49 and 52 |
| 10GBASE-LR | IEEE 802.3ae, clauses 49 and 52 |
| 10GBASE-ER | IEEE 802.3ae, clauses 49 and 52 |

Table 8-2/G.8012/Y.1308  EoT interfaces for Ethernet over Transport

| Ethernet interface | Reference |
| --- | --- |
| 10GBASE-SW (Note) | IEEE 802.3ae, clauses 50 and 52 |
| 10GBASE-LW (Note) | IEEE 802.3ae, clauses 50 and 52 |
| 10GBASE-EW (Note) | IEEE 802.3ae, clauses 50 and 52 |
| NOTE – The differences between the 10GBASE-W based Ety-UNI, a 10GBase-W based Ety-NNI and 10G EoS NNI are described in Appendix II. |  |

Annex A

ETC point-to-point connection

An ETC point-to-point connection is provided by means of a Type 2 Ethernet private line service specified in ITU-T Rec. G.8011.1/Y.1307.1, 1 Gbit/s Ethernet, the CI of the individual 8B/10B line code characters (ETC_CI) is carried by mapping them into GFP-T, as specified in ITU-T Rec. G.7041/Y.1303. As shown in Figure A.1, the information from eight 8B/10B characters is mapped into a 64B/65B block code, eight of which are mapped into a superblock. At least 95 superblocks are then mapped into a GFP-T frame. The GFP PTI subfield has value 000, the PFI subfield has value 0, the EXI subfield has value 0000 and the UPI subfield has value 0x06. The maximum size of the GFP payload information field is specified in 6.1.2/G.7041/Y.1303.

Figure A.1/G.8012/Y.1308 – GFP-T mapping for 8B/10B encoded 1 Gbit/s Ethernet

Appendix I

Ethernet multiplexing

## I.1	ETH two level VLAN Multiplex

Figure I.1 illustrates a two-level multiplexing of up to M × 4094 ETH signals into an ETH topological link. The ETH_CI traffic unit is for that purpose extended with a first level C-Tag including a C-VID as specified in IEEE 802.1Q and then multiplexed into an Ethernet Unit Group level 1. The EUG1 is extended with a second level S-Tag including also an S-VID (Figure I.2) and then multiplexed into an Ethernet Unit Group level 2.

The structure of the S-Tag and the value of M are currently being defined as part of the provider bridgework currently under way in the IEEE P802.1ad task force. It is anticipated that the structure of the S-Tag will be similar to the structure of the existing C-Tag.

Figure I.1/G.8012/Y.1308 – Ethernet two-level multiplexing

Figure I.2/G.8012/Y.1308 – Ethernet two-level multiplexing method

Appendix II

64B/66B link frames over 10GBASE-W and STM-64

## II.1	64B/66B link frame over STM-64

Annex F/G.707/Y.1322 specifies the Mapping of a 64B/66B-encoded signal into VC-4-64c. Any optical STM-N interface specified in ITU-T Rec. G.691 could be used for the Ethernet NNI.

## II.2	64B/66B link frame over 10GBASE-W

IEEE 803.3ae specifies the 64B/66B coding, the use of overhead of the VC-4-64c, MS64 and RS64, the timing of the interface and the optical characteristics of the 10GBASE-W type Ethernet interfaces.

## II.3	Differences between 64B/66B link frame over STM-64 and 10GBASE-W

The specification for the 10GBASE-W type Ety and 10G EoS differ in respect of:

–	coding of the Overhead of the RS64;

–	timing of the SDH signal;

–	set of optical interfaces.

BIBLIOGRAPHY

–	IEEE Standards Association Project Authorization Request, Project P802.1ad (C/LM) Standard for Local and Metropolitan Area Networks – Virtual Bridged Local Area Networks – Amendment 4: Provider Bridges. http://standards.ieee.org/board/nes/1-999.html

ITU-T Y-SERIES  RECOMMENDATIONS

GLOBAL INFORMATION INFRASTRUCTURE, INTERNET PROTOCOL ASPECTS AND NEXT GENERATION NETWORKS

| GLOBAL INFORMATION INFRASTRUCTURE |  |
| --- | --- |
| General | Y.100–Y.199 |
| Services, applications and middleware | Y.200–Y.299 |
| Network aspects | Y.300–Y.399 |
| Interfaces and protocols | Y.400–Y.499 |
| Numbering, addressing and naming | Y.500–Y.599 |
| Operation, administration and maintenance | Y.600–Y.699 |
| Security | Y.700–Y.799 |
| Performances | Y.800–Y.899 |
| INTERNET PROTOCOL ASPECTS |  |
| General | Y.1000–Y.1099 |
| Services and applications | Y.1100–Y.1199 |
| Architecture, access, network capabilities and resource management | Y.1200–Y.1299 |
| Transport | Y.1300–Y.1399 |
| Interworking | Y.1400–Y.1499 |
| Quality of service and network performance | Y.1500–Y.1599 |
| Signalling | Y.1600–Y.1699 |
| Operation, administration and maintenance | Y.1700–Y.1799 |
| Charging | Y.1800–Y.1899 |
| NEXT GENERATION NETWORKS |  |
| Frameworks and functional architecture models | Y.2000–Y.2099 |
| Quality of Service and performance | Y.2100–Y.2199 |
| Service aspects: Service capabilities and service architecture | Y.2200–Y.2249 |
| Service aspects: Interoperability of services and networks in NGN | Y.2250–Y.2299 |
| Numbering, naming and addressing | Y.2300–Y.2399 |
| Network management | Y.2400–Y.2499 |
| Network control architectures and protocols | Y.2500–Y.2599 |
| Security | Y.2700–Y.2799 |
| Generalized mobility | Y.2800–Y.2899 |

For further details, please refer to the list of ITU-T Recommendations.

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure, Internet protocol aspects and Next Generation Networks |
| Series Z | Languages and general software aspects for telecommunication systems |