|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.8040/Y.1340 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (06/2004) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital networks – General aspects SERIES Y: GLOBAL INFORMATION INFRASTRUCTURE, INTERNET PROTOCOL ASPECTS AND NEXT GENERATION NETWORKS Internet protocol aspects – Transport |  |  |  |
|  | GFP frame mapping into Plesiochronous Digital Hierarchy (PDH) |  |  |  |
|  | ITU-T Recommendation G.8040/Y.1340 |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| QUALITY OF SERVICE AND PERFORMANCE - GENERIC AND USER-RELATED ASPECTS | G.1000–G.1999 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.6000–G.6999 |
| DIGITAL TERMINAL EQUIPMENTS | G.7000–G.7999 |
| DIGITAL NETWORKS | G.8000–G.8999 |
| General aspects | G.8000–G.8099 |
| Design objectives for digital networks | G.8100–G.8199 |
| Quality and availability targets | G.8200–G.8299 |
| Network capabilities and functions | G.8300–G.8399 |
| SDH network characteristics | G.8400–G.8499 |
| Management of transport network | G.8500–G.8599 |
| SDH radio and satellite systems integration | G.8600–G.8699 |
| Optical transport networks | G.8700–G.8799 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.8040/Y.1340 GFP frame mapping into Plesiochronous Digital Hierarchy (PDH)

Summary This Recommendation provides the mapping to be used for the transport of GFP frames over PDH at the various hierarchical bit rates defined in ITU-T Rec. G.702. This mapping covers the 44 736 kbit/s signals and is used in conjunction with the frame structures defined in ITU-T Recs G.704 and G.7043/Y.1343.

Source ITU-T Recommendation G.8040/Y.1340 was approved on 13 June 2004 by ITU-T Study Group 15 (2001-2004) under the ITU-T Recommendation A.8 procedure.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g. interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met.  The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2004

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Definitions		1

4	Abbreviations		1

5	Conventions		2

6	Mapping GFP frames into PDH signals		2

6.1	Mapping into N × 1544 kbit/s		2

6.2	Mapping into N × 2048 kbit/s		3

6.3	Mapping into N × 44 736 kbit/s		3

6.4	Mapping into N × 34 368 kbit/s		4

Introduction

The generic framing procedure (GFP), as defined in ITU-T Rec. G.7041/Y.1303, was developed for transport of data clients over synchronous digital hierarchy (SDH) networks of ITU-T Rec. G.707/Y.1322 and optical transport networks (OTN) of ITU-T Rec. G.709/Y.1331. Interfaces from the plesiochronous digital hierarchy (PDH) are ubiquitous, especially in the access networks where there is a desire to carry client data signals. GFP has been identified as a suitable technique for mapping data frames into PDH signals of the hierarchies of ITU-T Rec. G.704 as well as the virtually concatenated PDH signals as defined in ITU-T Rec. G.7043/Y.1343.

ITU-T Recommendation G.8040/Y.1340

GFP frame mapping into Plesiochronous Digital Hierarchy (PDH)

# 1	Scope

This Recommendation provides the mapping of GFP-encapsulated data into PDH signals for transport over PDH trails. GFP was originally defined for an octet-wise mapping into octet-oriented transport containers. This octet-oriented mapping characteristic is maintained except that a nibble alignment is used for the 44 736 kbit/s signal similar to that specified for ATM in ITU-T Rec. G.804.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

–	ITU-T Recommendation G.702 (1988), Digital hierarchy bit rates.

–	ITU-T Recommendation G.704 (1998), Synchronous frame structures used at 1544, 6312, 2048, 8448 and 44 736 kbit/s hierarchical levels.

–	ITU-T Recommendation G.804 (2004), ATM cell mapping into plesiochronous digital hierarchy (PDH).

–	ITU-T Recommendation G.832 (1998), Transport of SDH elements on PDH networks – Frame and multiplexing structures.

–	ITU-T Recommendation G.7041/Y.1303 (2003), Generic framing procedure (GFP).

–	ITU-T Recommendation G.7043/Y.1343 (2004), Virtual concatenation of plesiochronous digital hierarchy (PDH) signals.

# 3	Definitions

This Recommendation defines the following term:

3.1	nibble: A group of four bits.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

GFP	Generic Framing Procedure

LCAS	Link Capacity Adjustment Scheme

PDH	Plesiochronous Digital Hierarchy

VCG	Virtual Concatenation Group

# 5	Conventions

The octets of the GFP frames are mapped into the PDH octets or nibbles in transmission bit order. Specifically, bit 1 of a GFP octet is the first bit to be transmitted in the PDH octet or nibble into which it is mapped.

# 6	Mapping GFP frames into PDH signals

## 6.1	Mapping into N × 1544 kbit/s

### 6.1.1	Frame and multiframe format

The 1544 kbit/s signal payload is typically arranged as 24 octets (i.e., time slots 1 to 24), as illustrated in Figure 6-1a.

The multiframe structure for the 24-frame multiframe as described in ITU-T Rec. G.704 shall be used. The first octet following the first framing bit of the multiframe is used to carry the concatenation overhead, as defined in ITU-T Rec. G.7043/Y.1343 and illustrated in Figure 6-1b. This octet is reserved for all values of N, (N = 1…16).

Figure 6-1/G.8040/Y.1340 – Octet-aligned mapping for GFP into the 1544 kbit/s signal

### 6.1.2	GFP frame rate adaptation

When GFP client data frames are not available from the GFP source adaptation process, GFP idle frames (or Client Management frames) shall be inserted in order to perform rate adaptation as described in ITU-T Rec. G.7041/Y.1303.

### 6.1.3	Scrambling of the GFP payload

GFP frames are scrambled consistent with ITU-T Rec. G.7041/Y.1303.

### 6.1.4	GFP frame delineation

GFP frame delineation is performed in the manner described in ITU-T Rec. G.7041/Y.1303.

## 6.2	Mapping into N × 2048 kbit/s

### 6.2.1	Frame and multiframe format

The basic frame structure at 2048 kbit/s as described in ITU-T Rec. G.704 shall be used. Time slots 1-31 are used to carry the GFP octets.

The multiframe structure for the 16-frame multiframe as described in ITU-T Rec. G.704 shall be used. Time slot 1 of the first frame of the multiframe is used to carry the concatenation overhead, as defined in ITU-T Rec. G.7043/Y.1343 and illustrated in Figure 6-2. This octet is reserved for all values of N, (N = 1...16).

Figure 6-2/G.8040/Y.1340 – Octet-aligned mapping for GFP into the 2048 kbit/s signal

### 6.2.2	GFP frame rate adaptation

When GFP client data frames are not available from the GFP source adaptation process, GFP idle frames (or Client Management frames) shall be inserted in order to perform rate adaptation as described in ITU-T Rec. G.7041/Y.1303.

### 6.2.3	Scrambling of the GFP payload

GFP frames are scrambled consistent with ITU-T Rec. G.7041/Y.1303.

### 6.2.4	GFP frame delineation

GFP frame delineation is performed in the manner described in ITU-T Rec. G.7041/Y.1303.

## 6.3	Mapping into N × 44 736 kbit/s

### 6.3.1	Frame format

The multiframe format at 44 736 kbit/s, as described in ITU-T Rec. G.704, shall be used. As illustrated in Figure 6-3, each 44 736 kbit/s subframe (M-subframe) contains 672 bits, which may be regarded as 168 nibbles, with 21 nibbles between each frame overhead bit position. The first octet (two nibbles) following the first framing bit (X1) of the multiframe is used to carry the concatenation overhead, as defined in ITU-T Rec. G.7043/Y.1343 and illustrated in Figure 6-4. This octet is reserved for all values of N, (N = 1…16).

GFP octets are mapped into the nibbles of the subframe with the GFP octet boundaries corresponding to a nibble boundary. At the receiver, the GFP frame delineation must be performed for each of the two possible nibble alignments of the octets in order to identify the proper alignment. Individual GFP frames can cross subframe boundaries, as illustrated in Figure 6-4. This mapping is similar to the HEC-based mapping of ATM into 44 736 kbit/s signals described in ITU-T Rec. G.804, which also uses an octet to nibble mapping.

Figure 6-3/G.8040/Y.1340 – Nibble structure for the 44 736 kbit/s signal subframe

Figure 6-4/G.8040/Y.1340 – GFP mapping into a 44 736 kbit/s signal frame

### 6.3.2	GFP frame rate adaptation

When GFP client data frames are not available from the GFP source adaptation process, GFP idle frames shall be inserted in order to perform rate adaptation as described in ITU-T Rec. G.7041/Y.1303.

### 6.3.3	Scrambling of the GFP payload

GFP frames are scrambled consistent with ITU-T Rec. G.7041/Y.1303.

### 6.3.4	GFP frame delineation

GFP frame delineation is performed in the manner described in ITU-T Rec. G.7041/Y.1303.

## 6.4	Mapping into N × 34 368 kbit/s

### 6.4.1	Frame format

The basic frame and multiframe structure at 34 368 kbit/s as described in ITU-T Rec. G.832 shall be used. As defined in ITU-T Rec. G.7043/Y.1343 and illustrated in Figure 6-5, the first octet following the FA2 octet of the multiframe is used to carry the concatenation overhead. This octet is reserved for all values of N, (N = 1…16).

Figure 6-5/G.8040/Y.1340 – Octet-aligned mapping for GFP into the 34 368 kbit/s signal

### GFP frame rate adaptation

When GFP client data frames are not available from the GFP source adaptation process, GFP idle frames (or Client Management frames) shall be inserted in order to perform rate adaptation as described in ITU-T Rec. G.7041/Y.1303.

### Scrambling of the GFP payload

GFP frames are scrambled consistent with ITU-T Rec. G.7041/Y.1303.

### GFP frame delineation

GFP frame delineation is performed in the manner described in ITU-T Rec. G.7041/Y.1303.

ITU-T Y-SERIES  RECOMMENDATIONS

GLOBAL INFORMATION INFRASTRUCTURE, INTERNET PROTOCOL ASPECTS AND NEXT GENERATION NETWORKS

| GLOBAL INFORMATION INFRASTRUCTURE |  |
| --- | --- |
| General | Y.100–Y.199 |
| Services, applications and middleware | Y.200–Y.299 |
| Network aspects | Y.300–Y.399 |
| Interfaces and protocols | Y.400–Y.499 |
| Numbering, addressing and naming | Y.500–Y.599 |
| Operation, administration and maintenance | Y.600–Y.699 |
| Security | Y.700–Y.799 |
| Performances | Y.800–Y.899 |
| INTERNET PROTOCOL ASPECTS |  |
| General | Y.1000–Y.1099 |
| Services and applications | Y.1100–Y.1199 |
| Architecture, access, network capabilities and resource management | Y.1200–Y.1299 |
| Transport | Y.1300–Y.1399 |
| Interworking | Y.1400–Y.1499 |
| Quality of service and network performance | Y.1500–Y.1599 |
| Signalling | Y.1600–Y.1699 |
| Operation, administration and maintenance | Y.1700–Y.1799 |
| Charging | Y.1800–Y.1899 |
| NEXT GENERATION NETWORKS |  |
| Frameworks and functional architecture models | Y.2000–Y.2099 |
| Quality of Service and performance | Y.2100–Y.2199 |
| Service aspects: Service capabilities and service architecture | Y.2200–Y.2249 |
| Service aspects: Interoperability of services and networks in NGN | Y.2250–Y.2299 |
| Numbering, naming and addressing | Y.2300–Y.2399 |
| Network management | Y.2400–Y.2499 |
| Network control architectures and protocols | Y.2500–Y.2599 |
| Security | Y.2700–Y.2799 |
| Generalized mobility | Y.2800–Y.2899 |

For further details, please refer to the list of ITU-T Recommendations.

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure, Internet protocol aspects and Next Generation Networks |
| Series Z | Languages and general software aspects for telecommunication systems |