|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.806 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | Amendment 1 (06/2004) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital networks – General aspects |  |  |  |
|  | Characteristics of transport equipment – Description methodology and generic functionality Amendment 1 |  |  |  |
|  | ITU-T Recommendation G.806 (2004) – Amendment 1 |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| General aspects | G.800–G.809 |
| Design objectives for digital networks | G.810–G.819 |
| Quality and availability targets | G.820–G.829 |
| Network capabilities and functions | G.830–G.839 |
| SDH network characteristics | G.840–G.849 |
| Management of transport network | G.850–G.859 |
| SDH radio and satellite systems integration | G.860–G.869 |
| Optical transport networks | G.870–G.879 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| QUALITY OF SERVICE AND PERFORMANCE - GENERIC AND USER-RELATED ASPECTS | G.1000–G.1999 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.6000–G.6999 |
| DIGITAL TERMINAL EQUIPMENTS | G.7000–G.7999 |
| DIGITAL NETWORKS | G.8000–G.8999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.806 Characteristics of transport equipment – Description methodology and generic functionality Amendment 1

Summary This amendment contains editorial and technical additions to the 02/2004 edition of ITU-T Rec. G.806.

Source Amendment 1 to ITU-T Recommendation G.806 (2004) was approved on 13 June 2004 by ITU-T Study Group 15 (2001-2004) under the ITU-T Recommendation A.8 procedure.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g. interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met.  The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2005

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS

Page

1)	Introduction		1

2)	Clause 6.2.3.1.1		1

3)	Clause 8.5.4.1.1		2

4)	Clause 8.5.4.1.2		3

5)	Clause 10.1.1.2		4

6)	Clause VII.2.1		4

7)	Clause VII.2.2		4

8)	Clause VII.2.3		4

9)	New Appendix VIII		5

ITU-T Recommendation G.806

Characteristics of transport equipment – Description methodology
and generic functionality

Amendment 1

# 1)	Introduction

This amendment contains editorial and technical additions to the 02/2004 edition of ITU-T Rec. G.806.

# 2)	Clause 6.2.3.1.1

Replace Table 6-6 as follows:

Table 6-6/G.806  Clearing time requirements

| Detector threshold | Set/Clear values associated with detector threshold | STM-N Multiplex section VC-4-Xc VC-4 VC-3 | VC-2 VC-12 VC-11 |
| --- | --- | --- | --- |
| 10–3 | 10–3/10–4 | 10 ms | 40 ms |
| 10–4 | 10–4/10–5 | 100 ms | 400 ms |
| 10–5 | 10–5/10–6 | 1 s | 4 s |
| 10–6 | 10–6/10–7 | 10 s | 40 s |
| 10–7 | 10–7/10–8 | 100 s | 400 s |
| 10–8 | 10–8/10–9 | 1000 s | 4000 s |
| 10–9 | 10–9/10–10 | 10 000 s |  |
| NOTE – The values in this table for the clearing times are upper bounds. For STM-N and VC-4-Xc it is possible to reduce the maximum clearing times in column 3 by a factor between 1 and N (for STM-N multiplex section) or between 1 and X (for VC-4-Xc), respectively (but note that clearing times below 10 ms are not recommended). |  |  |  |

# 3)	Clause 8.5.4.1.1

Replace Figure 8-15 as follows:

Figure 8-15/G.806  Client-specific GFP-F source processes

Replace the "PTI and UPI generation" process with the following:

PTI and UPI generation: The PTI field of the GFP type header of an incoming GFP client data frame is set to "000". The UPI field of the GFP data frame type header is set according to the specific client signal and mapping. The UPI codes are defined in Table 6-3/G.7041/Y.1303.

In the case where Client_SF and CSFEnable are true, GFP client management frames are inserted instead of GFP client data frames. The PTI field of the GFP type header of the GFP client management frame is set to "100". The UPI field is set to "0000 0010". These UPI codes are defined in Table 6-4/G.7041/Y.1303. These GFP client management frames have no payload information field. They are generated as defined in 6.3.3/G.7041/Y.1303.

# 4)	Clause 8.5.4.1.2

Replace Figure 8-16 as follows:

Figure 8-16/G.806  Client-specific GFP-F sink processes

Replace the second paragraph of the "PTI and UPI supervision" process

"GFP frames with an accepted PTI (AcPTI, see 8.5.1.1) value of "100" are Client Management Frames (CMF). Received CMFs are forwarded to the application-specific CMF processing function. Application specific CMF processes are currently not defined for GFP-F, they are for further study."

with:

GFP frames with an accepted PTI (AcPTI, see 8.5.1.1) value of "100" are Client Management Frames (CMF). Received CMFs are checked for a UPI value of  "0000 0010" for the dCSF defect detection and then discarded.

Replace the "Demapping of client data process" as follows:

Demapping of client data: The client data frame is extracted from the client payload information field of the GFP frame. One GFP frame results in one client frame. The mapping for the different client signals is defined in 7/G.7041/Y.1303.

Add the following defect to the "Defects" process after the dUPM defect:

dCSF: see 6.2.6.4

Replace the "Consequent Actions"

aClient_SF		GFP_SF or dUPM

with:

aClient_SF		GFP_SF or dUPM or dCSF

Add the following defect correlation in the "Defect correlations" process after the cUPM defect correlation:

cCSF		dCSF and (not dUPM) and (not GFP_SF) and CSF_Reported

# 5)	Clause 10.1.1.2

Add the following notes at the end of the "Received RI selection" description before the "Defects" description and renumber accordingly:

NOTE 12 – The purpose of the buffer is to provide the last-known received MST/RS_Ack to the source in the absence of any better information (e.g., in case all the members in the received direction have failed). This mechanism allows the source to continue to transmit without interruption during transient failures affecting the complete set of received path signal(s).

NOTE 13 – The buffer is set to the initial status at the startup of the adaptation function operation.

# 6)	Clause VII.2.1

Replace the second paragraph

"These three steps can be taken in any order. This scenario will illustrate the order as mentioned above."

with:

These three steps can be taken in any order and the operation will be hitless for the client signal, irrespective of the order. This scenario will illustrate the order as mentioned above.

# 7)	Clause VII.2.2

Replace the first paragraph

"In order to add a member, three items need to be provisioned: the source and the sink MI_ProvM and the path connectivity in between. These three operations can be done in any order."

with:

In order to add a member, three items need to be provisioned: the source and the sink MI_ProvM and the path connectivity inbetween. These three operations can be done in any order and the operation will be hitless for the client signal, irrespective of the order.

# 8)	Clause VII.2.3

Replace the first paragraph of this clause as follows:

In order to delete a member, three items need to be provisioned: the source and the sink MI_ProvM and the path connectivity inbetween. These three operations can be done in any order, but only the removal of the member at the source end first will be hitless to the client signal. Both the taking down of the connection for the member, and the removal of the member at the sink end first will result in a temporary disruption of the client signal until the resulting MST=FAIL status for the member reaches the source end, and the member is removed from the active multiplex there (and CTRL=DNU is inserted). The subsequent removal of the member at the source will have no effect on the client signal.

# 9)	New Appendix VIII

Add the following new Appendix VIII:

Appendix VIII

Consequent actions for signals with no AIS/FDI defined

In transport networks in the case of server layer defects, normally an AIS/FDI is generated for the downstream client signal at the Server/Client adaptation sink function. The AIS/FDI signal is an indication to the downstream network elements that the Client signal is missing due to a server layer defect. AIS/FDI suppresses downstream alarms and initiates protection/restoration actions at the client layer, if applicable.

Some client signals may not have an AIS/FDI signal defined due to various reasons (e.g., not needed in the original application of the client signal, the signal was originally at the bottom of the layer stack and it was not expected to be transported over a server layer).

In the case where no client signal AIS/FDI is available, but a downstream indication of the defect condition is needed (e.g., to trigger protection switching) the following actions can be considered at the output port for the client signal:

•	Turn off the output transmitting device;

•	Insert Error code (e.g., /V/, 10B_ERR for 1 GbE).

These actions are only possible in the case where a single instance of a client signal is transported over the output port, as all the client signal instances at the output port are affected by such actions. Note that these actions will result in server layer defects at the downstream input port which may lead to the wrong assumption that the server layer trail has a problem, which is not the case (see Figure VIII.1).

Other options are:

•	Insert signal fail message (if defined for that particular client signal and a client management channel exists between the network elements, e.g., CSF in GFP).

•	No action.

Figure VIII.1/G.806  Example of "turn off output" action

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure, Internet protocol aspects and Next Generation Networks |
| Series Z | Languages and general software aspects for telecommunication systems |