|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

| ITU-T | G.811 |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU | (09/97) |

SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

Digital transmission systems – Digital networks – Design objectives for digital networks

Timing characteristics of primary reference clocks

ITU-T  Recommendation  G.811

(Previously  CCITT  Recommendation)

ITU-T  G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| For further details, please refer to ITU-T List of Recommendations. |  |
| --- | --- |
| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| INTERNATIONAL ANALOGUE CARRIER SYSTEM |  |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TRANSMISSION MEDIA CHARACTERISTICS |  |
| DIGITAL TRANSMISSION SYSTEMS |  |
| TERMINAL EQUIPMENTS | G.700–G.799 |
| General | G.700–G.709 |
| Coding of analogue signals by pulse code modulation | G.710–G.719 |
| Coding of analogue signals by methods other than PCM | G.720–G.729 |
| Principal characteristics of primary multiplex equipment | G.730–G.739 |
| Principal characteristics of second order multiplex equipment | G.740–G.749 |
| Principal characteristics of higher order multiplex equipment | G.750–G.759 |
| Principal characteristics of transcoder and digital multiplication equipment | G.760–G.769 |
| Operations, administration and maintenance features of transmission equipment | G.770–G.779 |
| Principal characteristics of multiplexing equipment for the synchronous digital hierarchy | G.780–G.789 |
| Other terminal equipment | G.790–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| General aspects | G.800–G.809 |
| Design objectives for digital networks | G.810–G.819 |
| Quality and availability targets | G.820–G.829 |
| Network capabilities and functions | G.830–G.839 |
| SDH network characteristics | G.840–G.849 |
| Telecommunications management network | G.850–G.859 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| General | G.900–G.909 |
| Parameters for optical fibre cable systems | G.910–G.919 |
| Digital sections at hierarchical bit rates based on a bit rate of 2048 kbit/s | G.920–G.929 |
| Digital line transmission systems on cable at non-hierarchical bit rates | G.930–G.939 |
| Digital line systems provided by FDM transmission bearers | G.940–G.949 |
| Digital line systems | G.950–G.959 |
| Digital section and digital transmission systems for customer access to ISDN | G.960–G.969 |
| Optical fibre submarine cable systems | G.970–G.979 |
| Optical line systems for local and access networks | G.980–G.999 |

ITU-T  RECOMMENDATION  G.811

TIMING  CHARACTERISTICS  OF  PRIMARY  REFERENCE  CLOCKS

Summary

This Recommendation outlines minimum requirements for timing devices used as primary reference clocks in synchronization networks. These networks include Public Switched Telephone Networks (PSTNs) and Synchronous Digital Hierarchy (SDH) networks.

Source

ITU-T Recommendation G.811 was revised by ITU-T Study Group 13 (1997-2000) and was approved under the WTSC Resolution No. 1 procedure on the 19th of September 1997.

Keywords

Clock, Clock Performance Objectives, Clock Performance Parameters, Jitter Performance, Wander Performance.

FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL  PROPERTY  RIGHTS

The ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. The ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, the ITU had/had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  1998

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Definitions		2

4	Abbreviations		2

5	Frequency accuracy		2

6	Noise generation		2

6.1	Wander		2

6.2	Jitter		5

7	Phase discontinuity		5

8	Degradation of the performance of a PRC		5

9	Interfaces		5

Recommendation G.811

Recommendation G.811     (09/97)

TIMING  CHARACTERISTICS  OF  PRIMARY  REFERENCE  CLOCKS

(revised in 1997)

# 1	Scope

This Recommendation outlines the requirements for Primary Reference Clocks (PRCs) suitable for synchronization supply to digital networks. These requirements apply under the normal environmental conditions specified for digital equipment.

A typical PRC provides the reference signal for the timing or synchronization of other clocks within a network or section of a network. In particular, the PRC can also provide the reference signal to the slave clock specified in Recommendation G.812 within the network nodes where the PRC is located. This Recommendation defines the PRC output but it does not apply to the output of slave clock directly fed by a PRC. The long-term accuracy of the PRC should be maintained at 1 part in 1011 or better with verification to Coordinated Universal Time (UTC). A PRC may be realized as an autonomous clock, operating independently of other sources. Alternatively, the PRC may be realized as a non-autonomous clock which is disciplined by UTC-derived precision signals received from a radio or satellite system. In either case, the requirements for long-term accuracy and short-term stability, as specified in this Recommendation, apply.

The long-term accuracy of 1 part in 1011 or better is adequate when a single PRC provides the reference synchronization signal to all other clocks within a network (synchronous mode of operation). In the pseudo-synchronous mode, i.e. not all clocks in the network have timing traceable to the same PRC, the long-term accuracy is dependent on the number of PRCs in that network.

When more than one PRC is used in a network, a statistical approach is needed to determine the long-term accuracy of each PRC in that network.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[1]	CCITT Recommendation G.703 (1991), Physical/electrical characteristics of hierarchical digital interfaces.

[2]	ITU-T Recommendation G.783 (1997), Characteristics of Synchronous Digital Hierarchy (SDH) equipment functional blocks.

[3]	ITU-T Recommendation G.810 (1996), Definitions and terminology for synchronization networks.

[4]	CCITT Recommendation G.812 (1988), Timing requirements at the outputs of slave clocks suitable for plesiochronous operation of international digital links.

[5]	CCITT Recommendation G.822 (1988), Controlled slip rate objectives on an international digital connection.

[6]	ITU-T Recommendation G.823 (1993), The control of jitter and wander within digital networks which are based on the 2048 kbit/s hierarchy.

[7]	ITU-T Recommendation G.824 (1993), The control of jitter and wander within digital networks which are based on the 1544 kbit/s hierarchy.

[8]	ITU-T Recommendation G.825 (1993), The control of jitter and wander within digital networks which are based on the Synchronous Digital Hierarchy (SDH).

# 3	Definitions

The terms and definitions used in this Recommendation are contained in Recommendation G.810.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

CMI	Coded Mark Inversion

FPM	Flicker Phase Modulation

MTIE	Maximum Time Interval Error

NE	Network Element

OAM	Operations and Maintenance

PLL	Phase Locked Loop

PRC	Primary Reference Clock

SDH	Synchronous Digital Hierarchy

SEC	SDH Equipment Clock

SSMB	Synchronization Status Message Byte

STM	Synchronous Transport Module

TDEV	Time Deviation

UI	Unit Interval

UTC	Coordinated Universal Time

WFM	White Frequency Modulation

# 5	Frequency accuracy

The maximum allowable fractional frequency offset for observation times greater than one week is 1 part in 1011, over all applicable operational conditions.

# 6	Noise generation

The noise generation of a PRC clock represents the amount of phase noise produced at its output. A suitable reference, for practical testing purposes, implies a performance level that is more stable than the output requirements. The ability of the clock to limit this noise is described by its frequency stability. The measures MTIE and Time Deviation (TDEV) are useful for characterization of noise generation performance.

MTIE and TDEV are measured through an equivalent 10 Hz, first-order, low-pass measurement filter, at a maximum sampling time t0 of 1/30 seconds. The minimum measurement period for TDEV is twelve times the integration period (T = 12t). For measurements over longer observation periods, alternative filter bandwidth and sampling time may be required for practical considerations.

## 6.1	Wander

The wander, expressed in MTIE, measured using the independent clock configuration defined in Figure 2a/G.810 should have the following limits:

| MTIE: | 0.275 × 10–3t + 0.025 ms | for | 0.1 < t  1000 s |
| --- | --- | --- | --- |
|  | 10–5t + 0.29 ms | for | t > 1000 s |

The resultant requirements are shown in Figure 1.

FIGURE 1/G.811...[D01] =

The wander, expressed in TDEV, measured using the independent clock configuration defined in Figure 2a/G.810 should have the following limits:

| TDEV: | 3 ns | for | 0.1 < t  100 s |
| --- | --- | --- | --- |
|  | 0.03 t ns | for | 100 < t  1000 s |
|  | 30 ns | for | 1000 < t < 10 000 s |

The resultant requirements are shown in Figure 2.

FIGURE 2/G.811...[D02] =

## 6.2	Jitter

While most specifications in this Recommendation are independent of the output interface at which they are measured, this is not the case for jitter production; jitter generation specifications must utilize existing specifications that are currently specified differently for different interface rates. These requirements are stated separately for the interfaces identified in clause 8. To be consistent with other jitter requirements, the specifications are in UIpp, where the UI corresponds to the reciprocal of the bit rate of the interface.

The intrinsic jitter at 2048 kHz and 2048 kbit/s output interfaces as measured over a 60-second interval shall not exceed 0.05 UIpp when measured through a single pole band-pass filter with corner frequencies at 20 Hz and 100 kHz.

The intrinsic jitter at 1544 kbit/s output interfaces as measured over a 60-second interval shall not exceed 0.015 UIpp when measured through a single pole band-pass filter with corner frequencies at 10 Hz and 40 kHz.

# 7	Phase discontinuity

Primary reference clocks need a very high reliability and are likely to include replication of the equipment in order to ensure the continuity of the output. However, any phase discontinuity, due to internal operations within the clock, should only result in a lengthening or shortening of the timing signal interval and must not, at the clock output, cause a phase discontinuity in excess of 1/8 UI (this refers to output signals at 1544 kbit/s, 2048 kbit/s or 2048 kHz).

# 8	Degradation of the performance of a PRC

If redundancy is applied and a clock frequency departs significantly from its nominal value, this should be detected and switching to an undegraded oscillator should then be effected. This switching should be accomplished before the MTIE or TDEV specification is exceeded.

# 9	Interfaces

The requirements in this Recommendation are related to reference points which may be internal to the equipment or NE in which the PRC is embedded and are therefore not necessarily available for measurement or analysis by the user. Consequently, the performance of the PRC is not specified at these internal reference points, but rather at the external interfaces of the equipment. The output interfaces specified for the equipment in which the PRC clock may be contained are:

–	2048 kHz interfaces according to clause 10/G.703 with additional jitter and wander requirements as specified herein;

–	1544 kbit/s interfaces according to clause 2/G.703 with additional jitter and wander requirements as specified herein;

–	2048 kbit/s interfaces according to clause 6/G.703 with additional jitter and wander requirements as specified herein;

–	other interfaces (such as sine wave 8 kHz to 5 MHz) are for further study.

Note that not all of the above interfaces may be implemented on all equipment.

| ITU-T RECOMMENDATIONS SERIES |  |
| --- | --- |
| Series A | Organization of the work of the ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communication |
| Series Z | Programming languages |