|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

| ITU-T | G.832 |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU | (10/98) |

SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

Digital transmission systems – Digital networks – Network capabilities and functions

Transport of SDH elements on PDH networks – Frame and multiplexing structures

ITU-T  Recommendation  G.832

(Previously  CCITT  Recommendations)

ITU-T  G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| For further details, please refer to ITU-T List of Recommendations. |  |
| --- | --- |
| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| INTERNATIONAL ANALOGUE CARRIER SYSTEM |  |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS |  |
| TRANSMISSION MEDIA CHARACTERISTICS |  |
| DIGITAL TRANSMISSION SYSTEMS |  |
| TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| General aspects | G.800–G.809 |
| Design objectives for digital networks | G.810–G.819 |
| Quality and availability targets | G.820–G.829 |
| Network capabilities and functions | G.830–G.839 |
| SDH network characteristics | G.840–G.849 |
| Telecommunications management network | G.850–G.859 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |

ITU-T RECOMMENDATION G.832 Transport of sdh elements on pdh networks – Frame and multiplexing structures

Summary This Recommendation defines frame structures and multiplexing arrangements for the transport of SDH elements over existing PDH transport networks operating at G.702 hierarchical rates of 34 368 kbit/s, 44 736 kbit/s, 97 728 kbit/s and 139 264 kbit/s. It is envisaged that these frame structures could, unless otherwise indicated, be used for the transport of other signals (e.g. ATM cells).

History

| Issue | Notes |
| --- | --- |
| 02/98 | Second revision: Modifications are made to subclauses 2.1.2, 2.4.2, 3.1.1 and 3.4.3 to support the Synchronization Status Message (SSM) instead of the timing marker in bit 8 of byte MA in the 34 368 and 139 264 kbit/s signals. The NR byte definition is modified and now aligned with the definition of byte N1 in Annex D/G.707. |
| 11/95 | First revision: Terms FERF and FEBE are replaced by RDI and REI. References to Recommendations G.708 and G.709 are replaced by references to Recommendation G.707. The trail trace identifier specification has been modified and refers to Recommendations G.707 and G.831. |
| 11/93 | Initial version |

Source ITU-T Recommendation G.832 was revised by ITU-T Study Group 15 (1997-2000) and was approved under the WTSC Resolution No. 1 procedure on the 13th of October 1998.

FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation the term recognized operating agency (ROA) includes any individual, company, corporation or governmental organization that operates a public correspondence service. The terms Administration, ROA and public correspondence are defined in the Constitution of the ITU (Geneva, 1992).

INTELLECTUAL PROPERTY RIGHTS

The ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. The ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, the ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  1999

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Page

1	Introduction		1

1.1	Scope		1

1.2	References		1

1.3	Abbreviations		1

2	Frame structures		2

2.1	Frame structure at 34 368 kbit/s		2

2.1.1	General		2

2.1.2	Overhead allocation		3

2.2	Frame structure at 44 736 kbit/s		4

2.2.1	Intermediate 500 ms frame structure		4

2.2.2	Overhead allocation		5

2.3	Frame structure at 97 728 kbit/s		5

2.3.1	General		5

2.3.2	Overhead allocation		6

2.4	Frame structure at 139 264 kbit/s		7

2.4.1	General		7

2.4.2	Overhead allocation		7

3	Multiplexing structures		9

3.1	Support of SDH elements in the 34 368 kbit/s frame structure		9

3.1.1	Coding of multiframe indicator for Tributary Unit multiframe indication		10

3.2	Support of SDH elements in the 44 736 kbit/s frame structure		11

3.3	Support of SDH elements in the 97 728 kbit/s frame structure		12

3.4	Support of SDH elements in the 139 264 kbit/s frame structure		13

3.4.1	Multiplexing of 20 ´ TUG-2		13

3.4.2	Multiplexing of 2 ´ TUG-3 and 5 ´ TUG-2		14

3.4.3	Coding of multiframe indicator for Tributary Unit multiframe indication		15

Annex A – Description of the 16-byte frame and CRC-7 calculation		16

Recommendation G.832

TRANSPORT OF SDH ELEMENTS ON PDH NETWORKS – FRAME AND MULTIPLEXING STRUCTURES

(revised in 1998)

# 1	Introduction

## 1.1	Scope

The purpose of this Recommendation is to provide frame structures and multiplexing arrangements for the transport of SDH elements over existing PDH transport networks operating at various G.702 hierarchical rates. It is also envisaged that these frame structures could, unless otherwise indicated, be used for the transport of other signals (e.g. ATM cells).

## 1.2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

–	ITU-T Recommendation G.707 (1996), Network node interface for the Synchronous Digital Hierarchy (SDH).

–	ITU-T Recommendation G.804 (1998), ATM cell mapping into Plesiochronous Digital Hierarchy (PDH).

–	CCITT Recommendation T.50 (1992), International Reference Alphabet (IRA) (Formerly International Alphabet No. 5 or IA5) – Information technology – 7-bit coded character set for information  interchange.

## 1.3	Abbreviations

This Recommendation uses the following abbreviations:

BIP-8	Bit Interleaved Parity-8

C-n	Container of level n

IEC	Incoming Error Count

LSB	Least Significant Bit

MFI	MultiFrame Indicator

MSB	Most Significant Bit

PDH	Plesiochronous Digital Hierarchy

POH	Path Overhead

PTR	PoinTeR

RDI	Remote Defect Indication

REI	Remote Error Indication

SDH	Synchronous Digital Hierarchy

SOH	Section Overhead

TTI	Trail Trace Identifier

TU-n	Tributary Unit of level n

TUG-n	Tributary Unit Group of level n

VC-n	Virtual Container of level n

NOTE 1 – The order of transmission of information in all diagrams in this Recommendation is first from left to right and then top to bottom. Within each byte the most significant bit is transmitted first. The most significant bit (bit 1) is illustrated at the left of all diagrams.

NOTE 2 – In this Recommendation the term "SDH elements" indicates the various VCs with their associated pointers.

# 2	Frame structures

The frame structures defined in this Recommendation are intended to be used in a generic way. When implementing these frame structures, care should be taken to ensure that the performance of the frame alignment mechanism is not compromised by the payload content.

## 2.1	Frame structure at 34 368 kbit/s

### 2.1.1	General

The basic frame structure at 34 368 kbit/s comprises 7 octets of overhead and 530 octets of payload capacity per 125 ms as shown in Figure 2-1.

Figure 2-1/G.832 – Frame structure at 34 368 kbit/s

### 2.1.2	Overhead allocation

The values and allocation of the overhead bytes are shown in Figure 2-2 and are described below.

Figure 2-2/G.832 – Overhead allocation at 34 368 kbit/s

FA1/FA2

Frame Alignment signal. This has the same pattern as A1/A2 defined in Recommendation G.707.

EM

Error Monitoring, BIP-8. One byte is allocated for error monitoring. This function shall be a BIP-8 code using even parity. The BIP-8 is calculated over all bits, including the overhead bits, of the previous 125 ms frame. The computed BIP-8 is placed in the EM byte of the current 125 ms frame.

TR

Trail Trace. This byte is used to repetitively transmit a trail access point identifier so that a trail receiving terminal can verify its continued connection to the intended transmitter. The trail access point identifier shall use the access point identifier format as defined in clause 3/G.831.

A 16-byte frame is defined for the transmission of the access point identifier and this is described in Annex A.

MA	Maintenance and Adaptation byte

Bit 1		RDI

Bit 2		REI – This bit is set to "1" and sent back to the remote trail termination if one or more 	errors were detected by the BIP-8, and is otherwise set to zero.

Bits 3 to 5	Payload type

Code 	Signal

000		Unequipped

001		Equipped, non-specific

010		ATM

011		SDH TU-12s

Bits 6-7		Multiframe indicator

Bit 8		This bit is used in a four-frame multiframe. The phase of the multiframe is determined by the value of MA bits 6 and 7, according to:

Bit 6		Bit 7	Bit 8

0			0	SSM bit 1 (MSB)

0			1	SSM bit 2

1			0	SSM bit 3

1			1	SSM bit 4 (LSB)

The four bits of the multiframe are allocated to the Synchronization Status Message (SSM). The coding of the SSM is given in Table 5/G.707.

When interworking with "old" equipment which used bit 8 as a timing marker (non-multiframe), the "new" equipment implementing the above requirement should be capable of being configured to transmit the "old" requirement as given below:

Bit 8, Timing marker. This bit is set to "0" to indicate that the timing source is traceable to a Primary Reference Clock, and is otherwise set to "1".

NR

Network Operator byte. This byte is allocated for maintenance purposes specific to individual Network Operators. Its transparency from Trail termination to Trail termination is not guaranteed. In the case where this byte is modified at an intermediate point in the trail, the EM byte must be appropriately corrected to ensure performance monitoring integrity. For Tandem Connection Maintenance, the byte is used in accordance with Annex D/G.707.

GC

General purpose communications channel (e.g. to provide data/voice channel connection for maintenance purposes).

## 2.2	Frame structure at 44 736 kbit/s

The basic frame structure at 44 736 kbit/s is described in Recommendation G.704.

### 2.2.1	Intermediate 500 ms frame structure

Since the 44 736 kbit/s frame structure is an asynchronous format with a nominal frame time of 106.402 ms, an intermediate 500 ms frame structure is defined below. This frame consists of 2736 bytes of payload, 4 framing bytes and 19 bytes of fixed stuff as shown in Figure 2-3.

Figure 2-3/G.832 – Intermediate 500 ms frame structure
for transport of SDH elements in the 
44 736 kbit/s frame structure

When this intermediate frame is combined along with the 44 736 kbit/s frame level stuffing and the 56 frame overhead bits of the C-bit parity format, the resultant signal will be 44 736 095 bit/s (within the range of the nominal 44 736 kbit/s ± 20 ppm).

### 2.2.2	Overhead allocation

The framing bytes and fixed stuff bytes are defined below.

•	F1 – 11110110

•	F2 – 00101000

•	F3 – 11110110

•	F4 – 00101000

•	Fixed stuff – 11001100

## 2.3	Frame structure at 97 728 kbit/s

### 2.3.1	General

Using the full 97 728 kbit/s rate, there are 1537 bytes available every 125 ms. As the net capacity of the Container-3 needs 756 bytes every 125 ms, two Container-3s can be allocated leaving 15 bytes every 125 ms for the overhead for the 97 728 kbit/s signal and VC-3 POH functionality and for pointers as shown in Figure 2-4.

Figure 2-4/G.832 – Frame structure and overhead allocation for the 97 728 kbit/s

### 2.3.2	Overhead allocation

The overheads and pointers are described below.

A1, A2	Frame alignment signal:

A1	11110110

A2	001010MM,

where MM  (= 00/01/10/11) is multiframe indicator for Ha and Hb bytes.

D	A 64 kbit/s data communication channel.

Ha, Hb	These multiframe structured bytes have the following functionality:

H1, H2, H3	Reduced AU-3 pointer, which has the same function as defined in Recommendation G.707.

B2	Error monitoring, BIP-8. One byte is allocated for error monitoring of the 97 728 kbit/s signal. This function shall be a BIP-8 code using even parity. The BIP-8 is calculated over all bits of the previous 500 ms multiframe. The computed BIP-8 is placed in the B2 byte of the current 500 ms multiframe.

K1	Bit 8 is used for RDI. Bits 1-7 are reserved for future use.

J1		VC-3 path trace as defined in Recommendation G.707.

B3		VC-3 path BIP-8 as defined in Recommendation G.707.

C2/G1		Bits 1-4 are for VC-3 path REI, bit 5 is for VC-3 RDI and bits 6-8 are for VC-3 signal 	label.

F2		VC-3 path user channel as defined in Recommendation G.707.

H4		VC-3 position indicator as defined in Recommendation G.707.

## 2.4	Frame structure at 139 264 kbit/s

### 2.4.1	General

The basic frame structure at 139 264 kbit/s comprises 16 octets of overhead and 2160 octets of payload capacity per 125 ms as shown in Figure 2-5.

Figure 2-5/G.832 – Frame structure at 139 264 kbit/s

### 2.4.2	Overhead allocation

The values and allocation of the overhead bytes are shown in Figure 2-6 and are described below.

Figure 2-6/G.832 – Overhead allocation at 139 264 kbit/s

FA1/FA2

Frame Alignment signal. This has the same pattern as A1/A2 defined in Recommendation G.707.

EM

Error Monitoring, BIP-8. One byte is allocated for error monitoring. This function shall be a BIP-8 code using even parity. The BIP-8 is calculated over all bits of the previous 125 ms frame. The computed BIP-8 is placed in the EM byte of the current 125 ms frame.

TR

Trail Trace. This byte is used to repetitively transmit a trail access point identifier so that a trail receiving terminal can verify its continued connection to the intended transmitter. The trail access point identifier shall use the access point identifier format as defined in clause 3/G.831.

A 16-byte frame is defined for the transmission of the access point identifier and this is described in Annex A.

MA	Maintenance and Adaptation byte

Bit 1		RDI

Bit 2		REI – This bit is set to "1" and sent back to the remote trail termination if one or more errors were detected by the BIP-8, and is otherwise set to zero.

Bits 3 to 5	Payload type

Code 	Signal

000		Unequipped

001		Equipped, non-specific

010		ATM

011		SDH elements mapping I	20 ´ TUG-2

100		SDH elements mapping II	2 ´ TUG-3 and 5 ´ TUG-2

Bits 6-7	Multiframe indicator

Bit 8	This bit is used in a four-frame multiframe. The phase of the multiframe is determined by the value of MA bits 6 and 7, according to:

Bit 6		Bit 7	Bit 8

0			0	SSM bit 1 (MSB)

0			1	SSM bit 2

1			0	SSM bit 3

1			1	SSM bit 4 (LSB)

The four bits of the multiframe are allocated to the Synchronization Status Message (SSM). The coding of the SSM is given in Table 5/G.707.

When interworking with "old" equipment which used bit 8 as a timing marker (non-multiframe), the "new" equipment implementing the above requirement should be capable of being configured to transmit the "old" requirement as given below:

Bit 8, Timing marker. This bit is set to "0" to indicate that the timing source is traceable to a Primary Reference Clock, and is otherwise set to "1".

NR

Network Operator byte. This byte is allocated for maintenance purposes specific to individual Network Operators. Its transparency from Trail termination to Trail termination is not guaranteed. In the case where this byte is modified at an intermediate point in the trail, the EM byte must be appropriately corrected to ensure performance monitoring integrity. For Tandem Connection Maintenance, the byte is used in accordance with Annex D/G.707.

GC

General purpose communications channel (e.g. to provide data/voice channel connection for maintenance purposes).

P1/P2

Automatic Protection Switching.

# 3	Multiplexing structures

## 3.1	Support of SDH elements in the 34 368 kbit/s frame structure

14 ´ TU-12s are arranged in the 530 octets payload area as shown in Figure 3-1.

Columns one (except the first octet), thirty and thirty-one are occupied by fixed stuff, the 14 ´ TU-12s are 1-column interleaved into this structure and have a fixed phase relationship with respect to the frame structure. The Tributary Unit pointers occupy the octets in the first row from columns 2 to 15.

Figure 3-1/G.832 – Support of 14 TU-12s in the 34 368 kbit/s frame structure

The TU-12 elements are defined in Recommendation G.707.

Figure 3-2 shows the multiplexing structure.

Figure 3-2/G.832 – Multiplexing route for the 34 368 kbit/s frame structure

### 3.1.1	Coding of multiframe indicator for Tributary Unit multiframe indication

Table 3-1 shows the coding of the multiframe indicator (bits 6 and 7 of the MA octet) in case of TU-12s mapping.

| Table 3-1/G.832 |  |  |  |
| --- | --- | --- | --- |
| Bit 6 | Bit 7 | TU-PTR content in the following frame |  |
| 0 | 0 | V1 |  |
| 0 | 1 | V2 |  |
| 1 | 0 | V3 |  |
| 1 | 1 | V4 | 500 ms Tributary Unit multiframe |

The relation between the TU-PTR content and the coding of the payload dependent bits is shown in Figures 8-13/G.707 and 8-14/G.707.

## 3.2	Support of SDH elements in the 44 736 kbit/s frame structure

A TU-12 consists of 144 bytes per 500 ms. 19 ´ TU-12s are arranged in the 2736 octets payload area of the intermediate 500 ms frame as shown in Figure 3-3.

The Tributary Unit pointers occupy the octets in the first row from columns 2 to 20.

Figure 3-4 shows the multiplexing structure.

Figure 3-3/G.832 – Multiplexing of 19 TU-12s into the
intermediate 500 ms frame structure for transport
within the 44 736 kbit/s frame structure

Figure 3-4/G.832 – Multiplexing route for the transport
of SDH elements in the 44 736 kbit/s frame structure

## 3.3	Support of SDH elements in the 97 728 kbit/s frame structure

The 1522 octets payload capacity can be used to support two reduced VC-3s (VC-3Rs). The VC-3R can contain seven TUG-2s or one C-3 defined in Recommendation G.707.

Figure 3-5 shows the multiplexing structure with the relevant lower level options.

Figure 3-5/G.832 – Multiplexing routes for the transport of SDH elements
in the 97 728 kbit/s frame structure

## 3.4	Support of SDH elements in the 139 264 kbit/s frame structure

The 2160 octets payload capacity can be used to support the following multiplexing options:

•	Option I – 20 ´ TUG-2;

•	Option II – 2 ´ TUG-3 and 5 ´ TUG-2.

The TUG-2 and the TUG-3 are defined in Recommendation G.707.

Figure 3-6 shows the two TUG multiplexing opportunities with the relevant lower level structure.

Figure 3-6/G.832 – Multiplexing routes for the transport
of SDH elements into the 139 264 kbit/s frame structure

### 3.4.1	Multiplexing of 20 ´ TUG-2

The arrangement of 20 TUG-2s multiplexed into the 9 rows by 240 columns payload is shown in Figure 3-7. The 20 TUG-2s are one-byte interleaved into this structure and have a fixed phase relationship with respect to the frame overhead.

Figure 3-7/G.832 – Multiplexing of 20 ´ TUG-2 into the 139 264 kbit/s payload capacity

### 3.4.2	Multiplexing of 2 ´ TUG-3 and 5 ´ TUG-2

The arrangement of 2 TUG-3s and 5 TUG-2s multiplexed into the 9 rows by 240 columns payload is shown in Figure 3-8.

As a first step, four fixed stuff columns are added to each TUG-3 structure in the leading positions, resulting in two 90 column structures ("A" and "B"). The 5 TUG-2s are one-byte interleaved to a 60 columns by 9 rows structure ("C").

These three intermediate structures are then byte interleaved in the following sequence:

[ABACBABC]1        [ABACBABC]2........................[ABACBABC]30

If further interworking flexibility is required, this arrangement could also be further demultiplexed into one TUG-3 and 12 (7 + 5) TUG-2s or all TUG-2s in which case a maximum of 19 TUG-2s can be carried. If only TUG-2s are required, this last situation is unlikely as the arrangement described in 3.4.1 provides for 20 TUG-2s.

Figure 3-8/G.832 – Multiplexing of 2 ´ TUG-3 and 5 ´ TUG-2
into the 139 264 kbit/s payload capacity

### 3.4.3	Coding of multiframe indicator for Tributary Unit multiframe indication

Table 3-2 shows the coding of the multiframe indicator (bits 6 and 7 of the MA octet) in the case of TU-1xs mapping.

| Table 3-2/G.832 |  |  |  |
| --- | --- | --- | --- |
| Bit 6 | Bit 7 | TU-PTR content in the following frame |  |
| 0 | 0 | V1 |  |
| 0 | 1 | V2 |  |
| 1 | 0 | V3 |  |
| 1 | 1 | V4 | 500 ms Tributary Unit multiframe |

The relation between the TU-PTR content and the coding of the payload dependent bits is shown in Figures 8-13/G.707 and 8-14/G.707.

Annex A

Description of the 16-byte frame and CRC-7 calculation

The first byte of the string is a frame start marker and includes the result of a CRC-7 calculation over the previous frame. The following 15 bytes are used for the transport of 15 T.50 characters (international reference version) required for the access point identifier. The 16-byte frame is given below:

| 1 | C1 | C | C | C | C | C | C7 | Frame start marker |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | X | X | X | X | X | X | X | byte 2 |
|  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |
| 0 | X | X | X | X | X | X | X | byte 16 |
| X | X | X | X | X | X | X | T.50 character |  |
| C1 | C | C | C | C | C | C7 | Result of the CRC-7 calculation over the previous frame |  |

The description of the CRC-7 calculation is given below:

Multiplication/division process

A particular CRC-7 word is the remainder after multiplication by x7 and the division (modulo 2) by the generator polynomial x7 + x3 + 1, of the polynomial representation of the previous Trail Trace Identifier (TTI) multiframe.

When representing the contents of the block as a polynomial, the first bit in the block, i.e. byte 1, bit 1 should be taken as being the most significant bit. Similarly, C1 is defined to be the most significant bit of the remainder and C7 to be the least significant bit of the remainder.

Encoding procedure

The CRC-7 word is static because the data is static (the TTI represents the source address). This means that the CRC-7 checksum can be calculated either over the previous multiframe or a priori. In the last case this means that the 16-byte string that is loaded in a device for repetitive transmission should have the calculated checksum in the correct position.

The encoding procedure is as follows:

i)	The CRC-7 bits in the TTI are replaced by binary 0s.

ii)	The TTI is then acted upon by the multiplication/division process referred to above.

iii)	The remainder resulting from the multiplication/division process is inserted into the CRC-7 location.

The CRC-7 bits generated do not affect the result of the multiplication process because, as indicated in i) above, the CRC-7 bit positions are initially set to 0 during the multiplication/division process.

Decoding procedure

i)	A received TTI is acted upon by the multiplication/division process referred to above after having its CRC-7 bits extracted and replaced by 0s.

ii)	The remainder resulting from the division process is then compared on a bit-by-bit basis with the CRC-7 bits received.

iii)	If the remainder calculated in the decoder exactly corresponds to the CRC-7 bits received, it is assumed that the received TTI is error free.

| ITU-T RECOMMENDATIONS SERIES |  |
| --- | --- |
| Series A | Organization of the work of the ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure |
| Series Z | Programming languages |