|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.841 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | Corrigendum 1 (08/2002) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital networks – SDH network characteristics |  |  |  |
|  | Types and characteristics of SDH network protection architectures Corrigendum 1 |  |  |  |
|  | ITU-T Recommendation G.841 (1998) – Corrigendum 1 |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| General aspects | G.800–G.809 |
| Design objectives for digital networks | G.810–G.819 |
| Quality and availability targets | G.820–G.829 |
| Network capabilities and functions | G.830–G.839 |
| SDH network characteristics | G.840–G.849 |
| Management of transport network | G.850–G.859 |
| SDH radio and satellite systems integration | G.860–G.869 |
| Optical transport networks | G.870–G.879 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| QUALITY OF SERVICE AND PERFORMANCE | G.1000–G.1999 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.6000–G.6999 |
| DIGITAL TERMINAL EQUIPMENTS | G.7000–G.7999 |
| DIGITAL NETWORKS | G.8000–G.8999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.841 Types and characteristics of SDH network protection architectures Corrigendum 1

Source Corrigendum 1 to ITU-T Recommendation G.841 (1998) was prepared by ITU-T Study Group 15 (2001-2004) and approved under the WTSA Resolution 1 procedure on 16 August 2002.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2002

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS

Page

1	Introduction		1

2	Corrections to Table 7-4		1

3	Note to clause 7.2.4.2 paragraph 3		3

4	Text replacing part of 7.1.2.1		3

5	Correction to Table B.1		4

6	Editorial changes to Table B.5		4

ITU-T Recommendation G.841

Types and characteristics of SDH network protection architectures

Corrigendum 1

# 1	Introduction

This corrigendum contains editorial and technical corrections to the 1998 revision of ITU-T Rec. G.841.

# 2	Corrections to Table 7-4

Changes to Table 7-4 are indicated by revision marks.

| Table 7-4/G.841  1:n bidirectional protection switching without extra traffic example |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Failure condition or controller state |  | APS bytes |  |  |  |  |  |  |  | Action |  |  |
|  |  | C  A |  |  |  | A  C |  |  |  |  |  |  |
|  |  | byte K1 |  | byte K2 |  | byte K1 | byte K2 |  |  | at C |  | at A |
| No failures (protection section not in use) |  | 0000 0000 |  | 0000 1000 |  | 0000 0000 | 0000 1000 |  |  | Null signal is bridged to protection. Selector is released. |  | Null signal is bridged to protection. Selector is released. |
| Working section 2 degraded in direction A  C |  | 1010 0010 |  | 0000 1000 |  | 0000 0000 | 0000 1000 |  |  | Failure detected. Request normal traffic signal 2 bridge – SD. |  |  |
|  |  | 1010 0010 |  | 0000 1000 |  | 0010 0010 | 0010 1000 |  |  |  |  | Bridge normal traffic signal 2. Reverse request normal traffic signal 2 bridge. |
|  |  | 1010 0010 |  | 0010 1000 |  | 0010 0010 | 0010 1000 |  |  | Switch normal traffic signal 2 from protection section. Bridge normal traffic signal 2 to protection. |  |  |
|  |  | 1010 0010 |  | 0010 1000 |  | 0010 0010 | 0010 1000 |  |  |  |  | Switch normal traffic signal 2 from protection. Bidirectional switch completed. |
| Working section 1 failed in direction C  A |  | 1010 0010 |  | 0010 1000 |  | 1100 0001 | 0000 1000 |  |  |  |  | Failure detected. Request normal traffic signal 1 bridge – SF. Release normal traffic signal 2 switch and bridge. |
| (This pre-empts the normal traffic signal 2 switch) |  | 0010 0001 |  | 0001 1000 |  | 1100 0001 | 0000 1000 |  |  | Bridge normal traffic signal 1 to protection. Reverse request normal traffic signal 1 bridge. Release normal traffic signal 2 switch. |  |  |
|  |  | 0010 0001 |  | 0001 1000 |  | 1100 0001 | 0001 1000 |  |  |  |  | Switch normal traffic signal 1. Bridge normal traffic signal 1. |
|  |  | 0010 0001 |  | 0001 1000 |  | 1100 0001 | 0001 1000 |  |  | Switch normal traffic signal 1. Bidirectional switch completed. |  |  |
| Working section 1 repaired |  | 0010 0001 |  | 0001 1000 |  | 0110 0001 | 0001 1000 |  |  |  |  | Wait to restore. |
| (Working section 2 still degraded) |  | 1010 0010 |  | 0000 1000 |  | 0110 0001 | 0001 1000 |  |  | Request normal traffic signal 2 bridge. Release normal traffic signal 1 switch and bridge. |  |  |
|  |  | 1010 0010 |  | 0000 1000 |  | 0010 0010 | 0010 1000 |  |  |  |  | Bridge normal traffic signal 2. Reverse request normal traffic signal 2 bridge. Release normal traffic signal 1 switch. |
|  |  | 1010 0010 |  | 0010 1000 |  | 0010 0010 | 0010 1000 |  |  | Bridge normal traffic signal 2. Switch normal traffic signal 2. |  |  |
| Working section 1 repaired (Working section 2 still degraded) |  | 1010 0010 |  | 0010 1000 |  | 0010 0010 | 0010 1000 |  |  |  |  | Switch normal traffic signal 2. Bidirectional switch completed. |
| Working section 2 repaired |  | 0110 0010 |  | 0010 1000 |  | 0010 0010 | 0010 1000 |  |  | Wait to restore normal traffic signal 2. |  |  |
| Wait to restore expired (no failures) | 0000 0000 |  | 0010 1000 |  | 0010 0010 |  |  | 0010 1000 | Drop normal traffic signal 2 bridge order. Release normal traffic signal 2 switch. |  |  |  |
|  | 0000 0000 |  | 0010 1000 |  | 0000 0000 |  |  | 0000 1000 |  |  | Drop normal traffic signal 2 bridge Drop. Drop normal traffic signal 2 bridge request. Release normal traffic signal 2 switch. |  |
|  | 0000 0000 |  | 0000 1000 |  | 0000 0000 |  |  | 0000 1000 | Drop normal traffic signal 2 bridge. Null signal is bridged to protection. |  | Null signal is bridged to protection. |  |

# 3	Note to clause 7.2.4.2 paragraph 3

A Note is added in clause 7.2.4.2, after paragraph 3:

NOTE – There is an issue with interworking the protection scheme with WDM systems. The issue occurs when four-fibre spring traffic is carried over a WDM system and, depending upon the order of repair of a failed WDM link, it may cause the protection scheme to "flap" between worker and protection. This issue is currently under study.

# 4	Text replacing part of 7.1.2.1

The manual switch null signal should no longer be used for 1:n protection without extra traffic. The second paragraph of item 4) in clause 7.1.2.1 should be revised as follows:

For 1  1 systems, manual switch null signal transfers the working section back from protection to the working section, unless an equal or higher priority request is in effect. Since manual switch has lower priority than SF or SD on a working section, this command will be carried out only if the working section is not in SF or SD condition. "Manual Switch Null Signal" has higher priority than "Manual Switch – Normal Traffic Signal 1" when both commands are detected at the same time.

# 5	Correction to Table B.1

Note 2 to Table B.1 is revised as follows:

NOTE 2 – In the case of Signal Degrade (SD) on both working sections, no protection switching should take place.

# 6	Editorial changes to Table B.5

Changes to Table B.5 are indicated by removing underlining revision marks.

| Table B.5/G.841 – Example of 1  1 bidirectional switching optimized for a network using predominantly 1  1 bidirectional switching – Forced Switch from Working Section 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Failure condition or controller state | APS bytes |  |  |  | Action |  |
|  | C  A |  | A  C |  |  |  |
|  | byte K1 | byte K2 | byte K1 | byte K2 | at C | at A |
| No fault condition traffic on channel 2 | 0000 0000 | 0010 0000 | 0000 0000 | 0010 0000 |  |  |
| Forced Switch from section 2 at side C | 1110 0010 | 0010 0000 | 0000 0000 | 0010 0000 | Detect local request. Update K1. |  |
|  | 1110 0010 | 0010 0000 | 0010 0010 | 0010 0000 |  | Detect remote request. Switch from channel 2 (to channel 1). Issue Reverse Request. |
|  | 1110 0010 | 0010 0000 | 0010 0010 | 0010 0000 | Detect reverse request. Switch from channel 2 (to channel 1).. |  |
| Clear Forced Switch at side C | 0000 0000 | 0001 0000 | 0010 0010 | 0010 0000 | Send no request. Update K1, K2. |  |
| No Switch active. Traffic on section 1 | 0000 0000 | 0001 0000 | 0000 0000 | 0001 0000 |  | Send no request. Update K1, K2. |

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |