|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

| ITU-T | G.853.10 |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU | (03/99) |

SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

Digital transmission systems – Digital networks – Management of transport network

Information viewpoint for pre-provisioned link connection management

ITU-T  Recommendation  G.853.10

(Previously  CCITT  Recommendation)

ITU-T  G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| For further details, please refer to ITU-T List of Recommendations. |  |
| --- | --- |
| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| INTERNATIONAL ANALOGUE CARRIER SYSTEM |  |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS |  |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TRANSMISSION SYSTEMS |  |
| TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| General aspects | G.800–G.809 |
| Design objectives for digital networks | G.810–G.819 |
| Quality and availability targets | G.820–G.829 |
| Network capabilities and functions | G.830–G.839 |
| SDH network characteristics | G.840–G.849 |
| Management of transport network | G.850–G.859 |
| SDH radio and satellite systems integration | G.860–G.869 |
| Optical transport networks | G.870–G.879 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |

ITU-T RECOMMENDATION G.853.10 information viewpoint for pre-provisioned link connection management

Summary The pre-provisioned link connection management community is used to assign transport entities (link connections or connection termination points) to a caller and to control and update the available capacity of the linking entity (link or link end) that contains the transport entities. The available capacity of the linking entity contains all the unassigned transport entities. This community should be used in the case where transport entities have already been provisioned inside the linking entity using the pre-provisioned adaptation management service (G.85x.8 series of Recommendations). The capability of having pre-provisioned transport entities is available in technologies such as SDH or WDM.

Source ITU-T Recommendation G.853.10 was prepared by ITU-T Study Group 4 (1997-2000) and was approved under the WTSC Resolution No. 1 procedure on the 26th of March 1999.

FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation the term recognized operating agency (ROA) includes any individual, company, corporation or governmental organization that operates a public correspondence service. The terms Administration, ROA and public correspondence are defined in the Constitution of the ITU (Geneva, 1992).

INTELLECTUAL PROPERTY RIGHTS

The ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. The ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, the ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  1999

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Definitions		1

4	Abbreviations		1

5	Conventions		2

6	Class diagrams		2

6.1	UML class diagram representing relationships between classes		2

6.2	UML class diagram representing the Inheritance Hierarchy		3

7	Label references		3

8	Information object class definitions		4

8.1	plcmLayerNetworkDomain		4

8.2	plcmLink		4

8.3	plcmLinkConnection		4

8.4	plcmLinkEnd		5

8.5	plcmNetworkCTP		5

9	Information relationship definitions		5

10	Static schemas		5

11	Dynamic schemas		5

12	Attributes		6

12.1	plcmCallerId		6

Recommendation G.853.10

INFORMATION VIEWPOINT FOR PRE-PROVISIONED LINK
CONNECTION MANAGEMENT

(Geneva, 1999)

## 1	Scope

This information viewpoint specification is related to the pre-provisioned linkconnection management enterprise specification defined in Recommendation G.852.10.

## 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[1]	ITU-T Recommendation G.851.1 (1996), Management of the transport network – Application of the RM-ODP framework.

[2]	ITU-T Recommendation G.853.1 (1999), Common elements of the information viewpoint for the management of a transport network.

[3]	ITU-T Recommendation G.852.10 (1999), Enterprise viewpoint for pre-provisioned link connection management.

[4]	ITU-T Recommendation G.854.10 (1999), Computational viewpoint for pre-provisioned link connection management.

## 3	Definitions

None.

## 4	Abbreviations

This Recommendation uses the following abbreviations.

CTP		Connection Termination Point

Id		Identifier

imp		imported

LC		Link Connection

LE		Link End

LND		Layer Network Domain

pam		pre-provisioned adaptation management

plcm		pre-provisioned link connection management

RM-ODP	Reference Model for Open Distributed Processing

UML		Unified Modelling Language

# 5	Conventions

None.

# 6	Class diagrams

## 6.1	UML class diagram representing relationships between classes

Figure 1/G.853.10 – UML class diagram with relationships between classes

## 6.2	UML class diagram representing the Inheritance Hierarchy

Figure 2/G.853.10 – UML class diagram representing the Inheritance Hierarchy

# 7	Label references

| Full label reference | Local label reference |
| --- | --- |
| <"Rec. G.853.1", INFORMATION_OBJECT: layerNetworkDomain> | layerNetworkDomain |
| <"Rec. G.853.1", INFORMATION_OBJECT: link> | link |
| <"Rec. G.853.1", INFORMATION_OBJECT: linkConnection> | linkConnection |
| <"Rec. G.853.1", INFORMATION_OBJECT: linkEnd> | linkEnd |
| <"Rec. G.853.1", INFORMATION_OBJECT: networkCTP> | networkCTP |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: layerNetworkDomainIsMadeOf> | layerNetworkDomainIsMadeOf |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: linkHasLinkConnections> | linkHasLinkConnections |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: linkEndHasNetworkCTPs> | linkEndHasNetworkCTPs |
| <"Rec. G.853.8", ATTRIBUTE: pamAvailableLinkCapacity> | pamAvailableLinkCapacity |
| <"Rec. G.853.1", ATTRIBUTE: userLabel> | userLabel |

# 8	Information object class definitions

## 8.1	plcmLayerNetworkDomain

<COMMUNITY: Pre-provisioned LinkConnection Management, ROLE: layer network domain>

DEFINITION

"This object class is derived from <layerNetworkDomain>."

ATTRIBUTE

-- none additional

RELATIONSHIP

<layerNetworkDomainIsMadeOf>

## 8.2	plcmLink

<COMMUNITY: Pre-provisioned LinkConnection Management, ROLE: linking entity>

DEFINITION

"This object class is derived from <link>."

ATTRIBUTE

<pamAvailableLinkCapacity>

"pamAvailableLinkCapacity provides the number of free linkconnections in the plcmLink available for a client usage. This attribute is imported from "Rec. G.853.8", COMMUNITY: pre-provisioned adaptation management."

RELATIONSHIP

<layerNetworkDomainIsMadeOf>

<linkHasLinkConnections>

## 8.3	plcmLinkConnection

<COMMUNITY: Pre-provisioned LinkConnection Management, ROLE: transport entity>

DEFINITION

"This object class is derived from <linkConnection>."

ATTRIBUTE

<userLabel>

"<COMMUNITY: Pre-provisioned LinkConnection Management, COMMUNITY POLICY: useUserLabel and notifyUserLabelChange>

This attribute is imported from Recommendation G.853.1 and it is used to hold a user friendly name of the plcmLinkConnection."

<plcmCallerId>

"This attribute is used to indicate to which caller the plcmLinkConnection has been assigned. The initial value for this attribute is NULL."

RELATIONSHIP

<layerNetworkDomainIsMadeOf>

<linkHasLinkConnections>

## 8.4	plcmLinkEnd

<COMMUNITY: Pre-provisioned LinkConnection Management, ROLE: linking entity>

DEFINITION

"This object class is derived from <linkEnd>."

ATTRIBUTE

<pamAvailableLinkCapacity>

"pamAvailableLinkCapacity provides the number of free networkCTPs in the plcmLinkEnd available for a client usage. This attribute is imported from "Rec. G.853.8", COMMUNITY: pre-provisioned adaptation management."

RELATIONSHIP

<layerNetworkDomainIsMadeOf>

<linkEndHasNetworkCTPs>

## 8.5	plcmNetworkCTP

<COMMUNITY: Pre-provisioned LinkConnection Management, ROLE: transport entity>

DEFINITION

"This object class is derived from <networkCTP>."

ATTRIBUTE

<userLabel>

"<COMMUNITY: Pre-provisioned LinkConnection Management, COMMUNITY POLICY: useUserLabel and notifyUserLabelChange>

This attribute is imported from Recommendation G.853.1 and it is used to hold a user friendly name of the plcmNetworkCTP."

<plcmCallerId>

"This attribute is used to indicate to which caller the plcmNetworkCTP has been assigned. The initial value for this attribute is NULL."

RELATIONSHIP

<layerNetworkDomainIsMadeOf>

<linkEndHasNetworkCTPs>

# 9	Information relationship definitions

None additional.

# 10	Static schemas

None.

# 11	Dynamic schemas

None.

# 12	Attributes

## 12.1	plcmCallerId

<COMMUNITY: Pre-provisioned LinkConnection Management, ACTION: assign transport entities, OBLIGATION: callerId>

<COMMUNITY: Pre-provisioned LinkConnection Management, ACTION: de-assign transport entities, OBLIGATION: supplyCallerId>

<COMMUNITY: Pre-provisioned LinkConnection Management, ACTION: report transport entities change, OBLIGATION: informCallerId>

DEFINITION

"This attribute is used to indicate to which caller the transport entity has been assigned. When the transport entity is not assigned, the value has to be NULL. When the transport entity is assigned, the value has to be different from NULL."

| ITU-T RECOMMENDATIONS SERIES |  |
| --- | --- |
| Series A | Organization of the work of the ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |