|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

| ITU-T | G.853.8 |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU | (03/99) |

SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

Digital transmission systems – Digital networks – Management of transport network

Information viewpoint for pre-provisioned adaptation management

ITU-T  Recommendation  G.853.8

(Previously  CCITT  Recommendation)

ITU-T  G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| For further details, please refer to ITU-T List of Recommendations. |  |
| --- | --- |
| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| INTERNATIONAL ANALOGUE CARRIER SYSTEM |  |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS |  |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.609 |
| DIGITAL TRANSMISSION SYSTEMS |  |
| TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| General aspects | G.800–G.809 |
| Design objectives for digital networks | G.810–G.819 |
| Quality and availability targets | G.820–G.829 |
| Network capabilities and functions | G.830–G.839 |
| SDH network characteristics | G.840–G.849 |
| Management of transport network | G.850–G.859 |
| SDH radio and satellite systems integration | G.860–G.869 |
| Optical transport networks | G.870–G.879 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |

ITU-T RECOMMENDATION G.853.8 Information viewpoint for pre-provisioned adaptation management

Summary The objective of pre-provisioned adaptation management community is to provide link capacity to client-layer(s) from a server layer. This community should be used in the case where client transport entities can be provisioned inside the link during the adaptation management. This capability of having pre-provisioned client transport entities is available in technologies such as SDH or WDM.

Source ITU-T Recommendation G.853.8 was prepared by ITU-T Study Group 4 (1997-2000) and was approved under the WTSC Resolution No. 1 procedure on the 26th of March 1999.

FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation the term recognized operating agency (ROA) includes any individual, company, corporation or governmental organization that operates a public correspondence service. The terms Administration, ROA and public correspondence are defined in the Constitution of the ITU (Geneva, 1992).

INTELLECTUAL PROPERTY RIGHTS

The ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. The ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, the ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  1999

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Definitions		1

4	Abbreviations		1

5	Conventions		1

6	UML class diagram representing relationships between classes		2

7	UML class diagram representing the inheritance hierarchy		4

8	Label references		6

9	Information object class definitions		7

9.1	pamClientLayerNetworkDomain		7

9.2	pamLinkConnection		7

9.3	pamNetworkCTP		7

9.4	pamServerLayerNetworkDomain		7

9.5	pamTopologicalLink		7

9.6	pamTopologicalLinkEnd		8

9.7	pamTrail		8

9.8	pamNetworkTTP		8

9.9	pamSubnetwork		9

9.10	pamSubnetworkTP		9

10	Information relationship definitions		9

10.1	pamClientLayerNetworkDomainIsMadeOf		9

10.2	pamLayerNetworkDomainCanServeLnds		9

10.3	pamLinkEndHasNetworkCTPs		10

10.4	pamTopologicalLinkEndIsSupportedByNetworkTTP		10

10.5	pamLinkHasLinkConnections		10

10.6	pamTopologicalLinkIsSupportedByTrail		10

10.7	pamServerLayerNetworkDomainIsMadeOf		11

11	Static schemas		11

12	Dynamic schemas		11

13	Attributes		11

13.1	pamAvailableLinkCapacity		11

Page

13.2	pamMaxProvisionableCapacity		12

13.3	pamPotentialLinkCapacity		12

13.4	pamProvisionedLinkCapacity		12

Appendix A – Example on the use of the attributes: pamAvailableLinkCapacity, pamMaxProvisionableCapacity, pamPotentialLinkCapacity,  pamProvisionedLinkCapacity		12

Recommendation G.853.8

information viewpoint for pre-provisioned 
adaptation management

(Geneva, 1999)

# 1	Scope

This information viewpoint specification is related to the pre-provisioned adaptation management enterprise specification defined in Recommendation G.852.8.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[1]	ITU-T Recommendation G.851.1 (1996), Management of the transport network – Application of the RM-ODP framework.

[2]	ITU-T Recommendation G.852.2 (1999), Enterprise viewpoint description of transport resource model.

[3]	ITU-T Recommendation G.852.8 (1999), Enterprise viewpoint for pre-provisioned adaptation management.

# 3	Definitions

None.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

LC	Link Connection

pam	pre-provisioned adaptation management

SDH	Synchronous Digital Hierarchy

TEM	transport enterprise model

tu	tributary unit

UML	Unified Modelling Language

# 5	Conventions

None.

# 6	UML class diagram representing relationships between classes

Figure 1/G.853.8 – pam link, link connection, trail and subnetworkTP class diagram

Figure 2/G.853.8 – pam networkTTP, network CTP, subnetworkTP and
linkEnd class diagram

# 7	UML class diagram representing the inheritance hierarchy

Figure 3/G.853.8 – Information object class inheritance diagram

Figure 4/G.853.8 – Information relationship class inheritance diagram

# 8	Label references

| Table 1/G.853.8 – Correspondence between local label references and full label references |  |
| --- | --- |
| Full label reference | Local label reference |
| <"Rec. G.853.1", INFORMATION_OBJECT: layerNetworkDomain> | layerNetworkDomain |
| <"Rec. G.853.1", INFORMATION_OBJECT: linkConnection> | linkConnection |
| <"Rec. G.853.1", INFORMATION_OBJECT: linkEnd> | linkEnd |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: linkEndHasNetworkCTPs> | linkEndHasNetworkCTPs |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: linkIsTerminatedByLinkEnd> | linkIsTerminatedByLinkEnd |
| <"Rec. G.853.1", INFORMATION_OBJECT: networkCTP> | networkCTP |
| <"Rec. G.853.1", INFORMATION_OBJECT: networkTTP> | networkTTP |
| <"Rec. G.853.1", INFORMATION_OBJECT: subnetwork> | subnetwork |
| <"Rec. G.853.1", INFORMATION_OBJECT: subnetworkTP | subnetworkTP |
| <"Rec. G.853.1", INFORMATION_OBJECT: topologicalLink> | topologicalLink |
| <"Rec. G.853.1", INFORMATION_OBJECT: topologicalLinkEnd> | topologicalLinkEnd |
| <"Rec. G.853.1", INFORMATION_OBJECT:trail> | trail |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: topologicalLinkIsSupportedByTrail> | topologicalLinkIsSupportedByTrail |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: topologicalLinkEndIsSupportedByNetworkTTP> | topologicalLinkEndIsSupportedByNetworkTTP |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: layerNetworkDomainIsMadeOf > | layerNetworkDomainIsMadeOf |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: linkHasLinkConnections> | linkHasLinkConnections |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: linkEndHasNetworkCTPs> | linkEndHasNetworkCTPs |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: networkTTPAdaptsNetworkCTP> | networkTTPAdaptsNetworkCTP |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: subnetworkIsDelimitedBy> | subnetworkIsDelimitedBy |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: subnetworkTPIsRelatedToExtremity> | subnetworkTPIsRelatedToExtremity |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: linkConnectionIsSupportedByTrail> | linkConnectionIsSupportedByTrail |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: linkBinds> | linkBinds |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: linkEndIsBoundTo> | linkEndIsBoundTo |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: LayerNetworkDomainCanServeLnds> | LayerNetworkDomainCanServeLnds |

# 9	Information object class definitions

## 9.1	pamClientLayerNetworkDomain

<COMMUNITY:pre-provisioned adaptation management, COMMUNITY POLICY, OBLIGATION: consistentClientAndServerSignalId>

DEFINITION

"This object class is derived from <layerNetworkDomain>."

RELATIONSHIP

< pamLayerNetworkDomainCanServeLnds >,

<pamClientLayerNetworkDomainIsMadeOf>.

## 9.2	pamLinkConnection

<COMMUNITY:pre-provisioned adaptation management,ROLE:client transport entity>

DEFINITION

"This object class is derived from <linkConnection>."

RELATIONSHIP

<subnetworkTPIsRelatedToExtremity>,

<linkConnectionIsSupportedByTrail>,

<linkHasLinkConnections>,

<pamClientLayerNetworkDomainIsMadeOf>.

## 9.3	pamNetworkCTP

<COMMUNITY:pre-provisioned adaptation management,ROLE:client transport entity>

DEFINITION

"This object class is derived from <networkCTP>."

RELATIONSHIP

<subnetworkTPIsRelatedToExtremity>,

<networkTTPAdaptsNetworkCTP>,

<linkEndHasNetworkCTPs>,

<pamClientLayerNetworkDomainIsMadeOf>.

## 9.4	pamServerLayerNetworkDomain

<COMMUNITY:pre-provisioned adaptation management, COMMUNITY POLICY, OBLIGATION: consistentClientAndServerSignalId >

DEFINITION

"This object class is derived <layerNetworkDomain>."

RELATIONSHIP

< pamLayerNetworkDomainCanServeLnds>,

<pamServerLayerNetworkDomainIsMadeOf>.

## 9.5	pamTopologicalLink

<COMMUNITY:pre-provisioned adaptation management,ROLE:client linking entity>

DEFINITION

"This object class represents a link served by only one trail.

This object class is derived from <topologicalLink>."

ATTRIBUTE

<pamAvailableLinkCapacity>,

<pamMaxProvisionableCapacity>,

<pamPotentialLinkCapacity>,

<pamProvisionedLinkCapacity>.

INVARIANT

inv_consistentAvailableLinkCapacity

"The <pamAvailableLinkCapacity> is lower than or equal to the <pamProvisionedLinkCapacity>."

inv_consistentProvisionedLinkCapacity

"The <pamProvisionedLinkCapacity> is lower than or equal to the <pamMaxProvisionableCapacity>."

inv_consistentPotentialLinkCapacity

"The <pamPotentialLinkCapacity> is lower than or equal to the <pamMaxProvisionableCapacity>."

RELATIONSHIP

<pamTopologicalLinkIsSupportedByTrail>,

<pamLinkHasLinkConnections>,

<pamCompoundLinkHasLinks>,

<pamClientLayerNetworkDomainIsMadeOf>,

<pamLinkBinds>.

## 9.6	pamTopologicalLinkEnd

<COMMUNITY:pre-provisioned adaptation management,ROLE:client linking entity>

DEFINITION

"This object class represents a linkEnd served by only one networkTTP."

"This object class is derived from <topologicalLinkEnd>."

ATTRIBUTES

<pamAvailableLinkCapacity>,

<pamMaxProvisionableCapacity>,

<pamPotentialLinkCapacity>,

<pamProvisionedLinkCapacity>.

INVARIANT

inv_consistentAvailableLinkCapacity

"The <pamAvailableLinkCapacity> is lower than or equal to the <pamProvisionedLinkCapacity>."

inv_consistentProvisionedLinkCapacity

"The <pamProvisionedLinkCapacity> is lower than or equal to the <pamMaxProvisionableCapacity>."

inv_consistentPotentialLinkCapacity

"The <pamPotentialLinkCapacity> is lower than or equal to the <pamMaxProvisionableCapacity>."

RELATIONSHIP

<pamLinkEndHasNetworkCTP>,

<pamTopologicalLinkEndIsSupportedByNetworkTTP>,

<pamClientLayerNetworkDomainIsMadeOf>,

<pamClientLayerNetworkDomainIsMadeOf>,

<pamlinkEndIsBoundTo>.

## 9.7	pamTrail

<COMMUNITY:pre-provisioned adaptation management,ROLE:server transport entity>

DEFINITION

"This object class is derived from <trail>."

RELATIONSHIP

<pamTopologicalLinkIsSupportedByTrail>,

<linkConnectionIsSupportedByTrail>,

<pamServerLayerNetworkDomainIsMadeOf>.

## 9.8	pamNetworkTTP

<COMMUNITY:pre-provisioned adaptation management,ROLE:server transport entity>

DEFINITION

"This object class is derived from <networkTTP>."

RELATIONSHIP

<pamTopologicalLinkEndIsSupportedByNetworkTTP>,

<networkTTPAdaptsNetworkCTP>,

<pamServerLayerNetworkDomainIsMadeOf>.

## 9.9	pamSubnetwork

<COMMUNITY:pre-provisioned adaptation management, COMMUNITY POLICY, OBLIGATION: architecturalConstraint>

DEFINITION

"This object class is derived from <subnetwork>."

RELATIONSHIP

<subnetworkIsDelimitedBy>,

<LinkBinds>,

<LinkEndIsBoundTo>.

## 9.10	pamSubnetworkTP

<COMMUNITY:pre-provisioned adaptation management, COMMUNITY POLICY, OBLIGATION: architecturalConstraint>

DEFINITION

"This object class is derived from <subnetworkTP>."

RELATIONSHIP

<subnetworkIsDelimitedBy>,

<subnetworkTPIsRelatedToExtremity>.

# 10	Information relationship definitions

## 10.1	pamClientLayerNetworkDomainIsMadeOf

DEFINITION

"The pamClientLayerNetworkDomainIsMadeOf relationship class describes the relationship that exists between the client layer network and all its components.

This relationship type is a subtype of <layerNetworkDomainIsMadeOf>."

ROLE

container

"Played by instances of information <pamClientLayerNetworkDomain> object type or subtype."

element

"Played by instances of the following information object type or subtype:

<pamTopologicalLink>,

<pamTopologicalLinkEnd>,

<pamLinkConnection>,

<pamNetworkCTP>,

<pamSubnetworkTP>."

## 10.2	pamLayerNetworkDomainCanServeLnds

<COMMUNITY:pre-provisioned adaptation management,COMMUNITY POLICY:consistentClientAndServerSignalId>

DEFINITION

"The pamLayerNetworkDomainCanServeLnds relationship class describes the relationship that exists between the server layer network and all its client layer network domains.

This relationship type is a subtype of <layerNetworkDomainIsServedByLayerNetworkDomain >."

ROLE

client

"Played by instances of <pamClientLayerNetworkDomain> information object type or subtype."

server

"Played by an instance of <pamServerLayerNetworkDomain> information object type or subtype."

INVARIANT

inv_clientRoleCardinality

"One and only one client per signal identification supported by the server may participate in the 			relationship."

## 10.3	pamLinkEndHasNetworkCTPs

DEFINITION

"The pamLinkEndHasNetworkCTPs relationship class describes the relationship that exists between <pamTopologicalLinkEnd> and its components.

This relationship type is a subtype of <LinkEndHasNetworkCTPs>."

ROLE

container

"Played by an instance of <pamTopologicalLinkEnd> information object type or subtype."

element

"Played by instances of <pamNetworkCTP> information object type or subtype."

INVARIANT

inv_consistentProvisionedCapacitys

"The element role cardinality is equal to the <pamProvisionedLinkCapacity> of the container."

## 10.4	pamTopologicalLinkEndIsSupportedByNetworkTTP

DEFINITION

"The pamTopologicalLinkEndIsSupportedByNetworkTTP relationship class describes the relationship that exists between the total capacity available from a <pamNetworkTTP> and the provisioned number of <pamNetworkCTP> of all the supported <linkEnd>.

This relationship type is a subtype of < topologicalLinkEndIsSupportedByNetworkTTP >."

ROLE

client

"Played by instances of <linkEnd> information object type or subtype."

server

"Played by an instance of <pamNetworkTTP> information object type or subtype."

INVARIANT

inv_consistentCapacity

"The <pamMaxProvisionableCapacity> is greater than or equal to the sum of the <pamProvisionedLinkCapacity> of all the clients of the considered <pamNetworkTTP>."

inv_consistentPotentialLinkCapacity

"The <pamMaxProvisionableCapacity> is greater than or equal to the sum of the <pamProvisionedLinkCapacity> of all the clients of the considered <pamNetworkTTP>."

## 10.5	pamLinkHasLinkConnections

DEFINITION

"The pamLinkHasLinkConnections relationship class describes the relationship that exists between <pamTopologicalLink> and its components <pamLinkConnection>.

This relationship type is a subtype of <linkHasLinkConnections>."

ROLE

container

"Played by an instance of <pamTopologicalLink> information object type or subtype."

element

"Played by instances of <pamLinkConnections> information object type or subtype."

INVARIANT

inv_consistentProvisionedCapacity

"The element role cardinality is equal to the <pamProvisionedLinkCapacity> of the container."

## 10.6	pamTopologicalLinkIsSupportedByTrail

DEFINITION

"The pamTopologicalLinkIsSupportedByTrail relationship class describes the relationship that exists between the total capacity available from a trail and the provisioned link capacity of all the supported links.

This relationship type is a subtype of <topologicalLinkIsSupportedByTrail>."

ROLE

client

"Played by instances of <pamTopologicalLink> information object type or subtype."

server

"Played by an instance of <pamTrail> information object type or subtype."

INVARIANT

inv_consistentCapacity

"The <pamMaxProvisionableCapacity> is greater than or equal to the sum of the <pamProvisionedLinkCapacity> of all the clients of the trail."

inv_consistentPotentialLinkCapacity

"The <pamMaxProvisionableCapacity> for each client is equal to the <pamPotentialLinkCapacity> plus the sum of the <pamProvisionedLinkCapacity> of all the clients."

## 10.7	pamServerLayerNetworkDomainIsMadeOf

DEFINITION

"The pamServerLayerNetworkDomainIsMadeOf relationship class describes the relationship that exists between the server layer network and all its components.

This relationship type is a subtype of <layerNetworkDomainIsMadeOf>."

ROLE

container

"Played by instances of information <pamServerLayerNetworkDomain> object type or subtype."

element

"Played by instances of the following information object type or subtype:

<pamTrail>,

<pamNetworkTTP>."

# 11	Static schemas

None.

# 12	Dynamic schemas

None.

# 13	Attributes

## 13.1	pamAvailableLinkCapacity

<COMMUNITY:pre-provisioned adaptation management,ACTION:provision capacity to client linking entity,OBLIGATION:deduceAvailableCapacity>

<COMMUNITY:pre-provisioned adaptation management,ACTION: remove capacity from client linking entity OBLIGATION:increaseAvailableCapacity>

DEFINITION

"The pamAvailableLinkCapacity attribute represents the provisioned capacity that is not assigned."

INVARIANT

inv_1

"The pamAvailableLinkCapacity can never be less than zero."

## 13.2	pamMaxProvisionableCapacity

<COMMUNITY:pre-provisioned adaptation management,ACTION:provision capacity to client linking entity >

<COMMUNITY:pre-provisioned adaptation management,ACTION: remove capacity from client linking entity>

DEFINITION

"The pamMaxProvisionableCapacity attribute provides the value that represents the maximum capacity that could be provisioned for a link from the server trail of this link. However, this maximum may not be provisioned for this link if several links are supported by the same server trail: in this case, they are sharing the maximum capacity."

INVARIANT

inv_1

"The pamMaxProvisionableCapacity can never be less than zero."

## 13.3	pamPotentialLinkCapacity

This information concept is related to the following enterprise entities:

<"G852-02R",COMMUNITY:pre-provisioned adaptation management,ACTION:provision capacity to client linking entity>

<"G852-02R",COMMUNITY:pre-provisioned adaptation management,ACTION: remove capacity from client linking entity>

DEFINITION

"The pamPotentialLinkCapacity attribute provides a value that represents the potential capacity that could be provisioned on a link from the capacity that has not already been provisioned from the server trail of this link."

INVARIANT

inv_1

"The pamPotentialLinkCapacity can never be less than zero."

## 13.4	pamProvisionedLinkCapacity

This information concept is related to the following enterprise entities:

<"G852-02R",COMMUNITY:pre-provisioned adaptation management,ACTION:provision capacity to client linking entity,OBLIGATION:deduceAvailableCapacity>

<"G852-02R",COMMUNITY:pre-provisioned adaptation management,ACTION: remove capacity from client linking entity OBLIGATION:increaseAvailableCapacity>

DEFINITION

"The pamProvisionedLinkCapacity attribute provides the capacity provisioned on a link."

INVARIANT

inv_1

"The pamProvisionedLinkCapacity can never be less than zero."

Appendix A

Example on the use of the attributes: pamAvailableLinkCapacity, pamMaxProvisionableCapacity, pamPotentialLinkCapacity, 
pamProvisionedLinkCapacity

Scenario description: a SDH VC4 layer is server for a VC12 and a VC3 layer.

Initial state: VC12 and VC3 links (respectively linkEnds) exist but are not associated with trail (respectively networktTTP).

| VC4 trail or networkTTP | VC12 link or link end | VC3 link or link end |
| --- | --- | --- |
| not associated | pamMaxProvisionableCapacity: 0 | pamMaxProvisionableCapacity: 0 |
|  | pamPotentialLinkCapacity: 0 | pamPotentialLinkCapacity: 0 |
|  | pamProvisionedLinkCapacity: 0 | pamProvisionedLinkCapacity: 0 |
|  | pamAvailableLinkCapacity: 0 | pamAvailableLinkCapacity: 0 |

State 1: a VC4 trail has been associated with client links or link ends.

| VC4 trail or networkTTP | VC12 link or link end | VC3 link or link end |
| --- | --- | --- |
| set up | pamMaxProvisionableCapacity: 63 | pamMaxProvisionableCapacity: 3 |
|  | pamPotentialLinkCapacity: 63 | pamPotentialLinkCapacity: 3 |
|  | pamProvisionedLinkCapacity: 0 | pamProvisionedLinkCapacity: 0 |
|  | pamAvailableLinkCapacity: 0 | pamAvailableLinkCapacity: 0 |

State 2: a VC3 link connection has been provisioned.

| VC4 trail or networkTTP | VC12 link or link end | VC3 link or link end |
| --- | --- | --- |
| set up | pamMaxProvisionableCapacity: 63 | pamMaxProvisionableCapacity: 3 |
|  | pamPotentialLinkCapacity: 42 | pamPotentialLinkCapacity: 2 |
|  | pamProvisionedLinkCapacity: 0 | pamProvisionedLinkCapacity: 1 |
|  | pamAvailableLinkCapacity: 0 | pamAvailableLinkCapacity: 1 |

State 3: 21 VC12 link connections have been provisioned.

| VC4 trail or networkTTP | VC12 link or link end | VC3 link or link end |
| --- | --- | --- |
| set up | pamMaxProvisionableCapacity: 63 | pamMaxProvisionableCapacity: 3 |
|  | pamPotentialLinkCapacity: 21 | pamPotentialLinkCapacity: 1 |
|  | pamProvisionedLinkCapacity: 21 | pamProvisionedLinkCapacity: 1 |
|  | pamAvailableLinkCapacity: 21 | pamAvailableLinkCapacity: 1 |

State 4: 5 VC12 link connections have been assigned or involved in subnetwork connections.

| VC4 trail or networkTTP | VC12 link or link end | VC3 link or link end |
| --- | --- | --- |
| set up | pamMaxProvisionableCapacity: 63 | pamMaxProvisionableCapacity: 3 |
|  | pamPotentialLinkCapacity: 21 | pamPotentialLinkCapacity: 1 |
|  | pamProvisionedLinkCapacity: 21 | pamProvisionedLinkCapacity: 1 |
|  | pamAvailableLinkCapacity: 16 | pamAvailableLinkCapacity: 1 |

| ITU-T RECOMMENDATIONS SERIES |  |
| --- | --- |
| Series A | Organization of the work of the ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |

This state is managed in the pre-provisioned linkConnection management community (plcm).