|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

| ITU-T | G.854.10 |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU | (03/99) |

SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

Digital transmission systems – Digital networks – Management of transport network

Computational viewpoint for pre-provisioned link connection management

ITU-T  Recommendation  G.854.10

(Previously  CCITT  Recommendation)

ITU-T  G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| For further details, please refer to ITU-T List of Recommendations. |  |
| --- | --- |
| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| INTERNATIONAL ANALOGUE CARRIER SYSTEM |  |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS |  |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TRANSMISSION SYSTEMS |  |
| TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| General aspects | G.800–G.809 |
| Design objectives for digital networks | G.810–G.819 |
| Quality and availability targets | G.820–G.829 |
| Network capabilities and functions | G.830–G.839 |
| SDH network characteristics | G.840–G.849 |
| Management of transport network | G.850–G.859 |
| SDH radio and satellite systems integration | G.860–G.869 |
| Optical transport networks | G.870–G.879 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |

ITU-T RECOMMENDATION G.854.10 computational viewpoint for pre-provisioned link connection management

Summary The pre-provisioned link connection management community is used to assign transport entities (link connections or connection termination points) to a caller and to control and update the available capacity of the linking entity (link or link end) that contains the transport entities. The available capacity of the linking entity contains all the unassigned transport entities. This community should be used in the case where transport entities have already been provisioned inside the linking entity using the pre-provisioned adaptation management service (See Recommendation G.853.8). The capability of having pre-provisioned transport entities is available in technologies such as SDH or WDM. Enterprise community actions covered by this specification: – COMMUNITY pre-provisioned linkConnection management; – assign transport entities; – de-assign transport entities; – report transport entities change.

Source ITU-T Recommendation G.854.10 was prepared by ITU-T Study Group 4 (1997-2000) and was approved under the WTSC Resolution No. 1 procedure on the 26th of March 1999.

FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation the term recognized operating agency (ROA) includes any individual, company, corporation or governmental organization that operates a public correspondence service. The terms Administration, ROA and public correspondence are defined in the Constitution of the ITU (Geneva, 1992).

INTELLECTUAL PROPERTY RIGHTS

The ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. The ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, the ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  1999

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Definitions		1

4	Abbreviations		1

5	Conventions		2

6	Label references		2

7	Interfaces		2

7.1	Query interfaces		2

7.2	Operational interfaces		3

7.2.1	PLCM Link Connection Assignment Interface		3

7.2.2	PLCM NetworkCTP Assignment Interface		6

7.3	Report interfaces		9

7.3.1	PLCM Reporting Arcview Interface		9

7.3.2	PLCM Reporting Pointview Interface		11

7.4	ASN.1 supporting productions		14

Recommendation G.854.10

COMPUTATIONAL VIEWPOINT FOR PRE-PROVISIONED LINK 
CONNECTION MANAGEMENT

(Geneva, 1999)

# 1	Scope

This computational viewpoint specification is related to the pre-provisioned linkConnection management enterprise specification defined in Recommendation G.852.10 and the pre-provisioned linkConnection management information specification defined in Recommendation G.853.10.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[1]	ITU-T Recommendation G.851.1 (1996), Management of the transport network – Application of the RM-ODP framework.

[2]	ITU-T Recommendation G.853.1 (1999), Common elements of the information viewpoint for the management of a transport network.

[3]	ITU-T Recommendation G.852.10 (1999), Enterprise viewpoint for pre-provisioned link connection management.

[4]	ITU-T Recommendation G.853.10 (1999), Information viewpoint for pre-provisioned link connection management.

# 3	Definitions

None.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

ASN.1	Abstract Syntax Notation One

CTP	Connection Termination Point

Id	Identifier

Ifce	Interface

inv	invariant

ITU	International Telecommunication Union

layerND	layer Network Domain

LC	Link Connection

LE	Link End

LND	Layer Network Domain

pam	pre-provisioned adaptation management

plcm	pre-provisioned link connection management

RM-ODP	Reference Model for Open Distributed Processing

# 5	Conventions

In order to increase the readability of the behaviour in the operations:

•	parameters are written in bold;

•	elements defined in the information specification are written in italic.

# 6	Label references

| Full label reference | Local label reference |
| --- | --- |
| <"Rec. G.853.10", INFORMATION_OBJECT:plcmLayerNetworkDomain> | plcmLayerNetworkDomain |
| <"Rec. G.853.10", INFORMATION_OBJECT:plcmLink> | plcmLink |
| <"Rec. G.853.10", INFORMATION_OBJECT:plcmLinkConnection> | plcmLinkConnection |
| <"Rec. G.853.10", INFORMATION_OBJECT:plcmLinkEnd> | plcmLinkEnd |
| <"Rec. G.853.10", INFORMATION_OBJECT:plcmNetworkCTP> | plcmNetworkCTP |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP:layerNetworkDomainIsMadeOf> | layerNetworkDomainIsMadeOf |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP:linkHasLinkConnections> | linkHasLinkConnections |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP:link?EndHasNetworkCTPs> | linkEndHasNetworkCTPs |
| <"Rec. G.853.8", ATTRIBUTE:pamAvailableLinkCapacity> | pamAvailableLinkCapacity |
| <"Rec. G.853.10", ATTRIBUTE:plcmCallerId> | plcmCallerId |

| Full ASN.1 production reference | Local label reference |
| --- | --- |
| <"Rec. X.721:1992:Attribute-ASN1Module": SimpleNameType> | SimpleNameType |
| <"Rec. X.680:1997":INTEGER> | INTEGER |

# 7	Interfaces

This specification do not define operations which change the userLabel of the resources or report this change to the notification receiver because they do not change the state of the system. There exact signature will be developed as part of the engineering viewpoint specification with the concerned technology.

## 7.1	Query interfaces

This specification refers to interfaces that allow get access identification and properties of resources involved in the "pre-provisioned linkConnection management" community. As the invocation of contained operations do not modify any state, there is no interest to develop them explicitly. Their exact signature will be developed as part of the engineering viewpoint, with the concerned technology. These interfaces are listed in the following table with the information they allow access to.

| Interface name | Information object | Attributes and relationships |
| --- | --- | --- |
| plcmLayerNetworkDomainQueryIfce | <plcmLayerNetworkDomain> | <resourceId> <signalIdentification> <layerNetworkDomainIsMadeOf, ROLE: element> |
| plcmLinkQueryIfce | <plcmLink> | <resourceId> <signalIdentification> <linkDirectionality> <pamAvailableLinkCapacity> <layerNetworkDomainIsMadeOf, ROLE: containerLND> <linkHasLinkConnections, ROLE: elementLC> |
| plcmLinkConnectionQueryIfce | <plcmLinkConnection> | <resourceId> <signalIdentification> <directionality> <userLabel> <plcmCallerId> <layerNetworkDomainIsMadeOf, ROLE: containerLND> <linkHasLinkConnections, ROLE: containerLink> |
| plcmLinkEndQueryIfce | <plcmLinkEnd> | <resourceId> <signalIdentification> <groupDirectionality> <pamAvailableLinkCapacity> <layerNetworkDomainIsMadeOf, ROLE: containerLND> <linkEndHasNetworkCTPs, ROLE: elementCTP> |
| plcmNetworkCTPQueryIfce | <plcmNetworkCTP> | <resourceId> <pointDirectionality> <userLabel> <plcmCallerId> <layerNetworkDomainIsMadeOf, ROLE: containerLND> <linkEndHasNetworkCTPs, ROLE: containerLE> |

## 7.2	Operational interfaces

### 7.2.1	PLCM Link Connection Assignment Interface

The plcm link connection assignment interface provides functionality for assigning link connections within links to callers. It satisfies the enterprise requirements stated in <"Rec. G.852.10", COMMUNITY: Pre-provisioned LinkConnection Management, ACTION: assign transport entities and ACTION: de-assign transport entities>.

COMPUTATIONAL_INTERFACE plcmAssignmentIfce {

OPERATION	<assignLinkConnectionOnLink>;

OPERATION	<de-assignLinkConnectionOnLink>; }

#### 7.2.1.1	assign Link Connection on Link

<COMMUNITY:  Pre-provisioned LinkConnection Management, ACTION: assign transport entities>

OPERATION assignLinkConnectionOnLink {

INPUT_PARAMETERS

layerND: layerNetworkDomainIfce ::= PlcmLayerNetworkDomainQueryIfce

requestingCaller: plcmCallerId

involvedLink: PlcmLinkId

requestedLinkConnections: PlcmRequestedLinkConnectionChoice;

OUTPUT_PARAMETERS

providedLinkConnections: SetOfLinkConnections;

RAISED_EXCEPTIONS

linkAndLinkConnectionNotCompatible: involvedLink;

invalidLinkConnection: linkConnection;

notEnoughLinkConnections: numberOfLC ::= INTEGER;

linkConnectionAlreadyAssigned: linkConnection;

inconsistentSignalIdentification: NULL;

inconsistentDirectionality: NULL;

failureToSetLinkConnectionCallerId: NULL;

failureToDecreaseCapacity: NULL;

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT plcmLayerNetworkDomain>;

requestingCaller: <INFORMATION ATTRIBUTE plcmCallerId>;

involvedLink: <INFORMATION OBJECT plcmLink>;

requestedLinkConnections ELEMENTS: <INFORMATION OBJECT plcmLinkConnection>;

providedLinkConnections ELEMENTS: <INFORMATION OBJECT plcmLinkConnection>;

linkConnection: <INFORMATION OBJECT plcmLinkConnection>;

PRE_CONDITIONS

inv_linkAndLinkConnectionCompatible

"involvedLink and requestedLinkConnections shall refer to element of the same <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_existingRequestedLinkConnections

"requestedLinkConnections refers to elementLC in a <linkHasLinkConnections> relationship where involvedLink refers to containerLink."

--	NOTE – The following invariant is only needed when the numberOfLC choice is used for the requestedLinkConnections by the caller.

inv_requestedLinkConnectionsAvailable

"the number of available linkConnections in <pamAvailableLinkCapacity> of involvedLink has to be greater than or equal to the number of requested linkConnections referred to by requestedLinkConnections."

inv_freeLinkConnections

"<plcmCallerId> of <plcmLinkConnections> referred to by requestedLinkConnections must be equal to NULL.";

POST_CONDITIONS

inv_assignedLinkConnections

"<plcmCallerId> of <plcmLinkConnections> referred to by requestedLinkConnections must be set to the identifier of the caller referred to by requestingCaller."

inv_capacityDecrease

"<pamAvailableLinkCapacity> of <plcmLink> referred to by involvedLink has been decreased by the number (size of 'SET OF' in requestedLinkConnections) of <plcmLinkConnections> that have been requested to be assigned.";

EXCEPTIONS

IF PRE_CONDITION inv_linkAndLinkConnectionCompatible NOT_VERIFIED RAISE_EXCEPTION linkAndLinkConnectionNotCompatible;

IF PRE_CONDITION inv_existingRequestedLinkConnections NOT_VERIFIED RAISE_EXCEPTION invalidLinkConnection;

IF PRE_CONDITION inv_requestedLinkConnectionsAvailable NOT_VERIFIED RAISE_EXCEPTION notEnoughLinkConnections;

IF PRE_CONDITION inv_freeLinkConnections NOT_VERIFIED RAISE_EXCEPTION linkConnectionAlreadyAssigned;

IF PRE_CONDITION <INFORMATION_RELATIONSHIP: linkHasLinkConnections, INVARIANT: inv_signalIdentification> NOT_VERIFIED RAISE_EXCEPTION inconsistentSignalIdentification;

IF PRE_CONDITION <INFORMATION_RELATIONSHIP: linkHasLinkConnections, INVARIANT: inv_directionality> NOT_VERIFIED RAISE_EXCEPTION inconsistentDirectionality;

IF POST_CONDITION inv_assignedLinkConnections NOT_VERIFIED RAISE_EXCEPTION failureToSetLinkConnectionCallerId;

IF POST_CONDITION inv_capacityDecrease NOT_VERIFIED RAISE_EXCEPTION failureToDecreaseCapacity;

; }

#### 7.2.1.2	de-assign Link Connection on Link

<COMMUNITY: Pre-provisioned LinkConnection Management, ACTION: de-assign transport entities>

OPERATION de-assignLinkConnectionOnLink {

INPUT_PARAMETERS

layerND: layerNetworkDomainIfce ::= PlcmLayerNetworkDomainQueryIfce

requestingCaller: plcmCallerId

involvedLink: PlcmLinkId

involvedLinkConnections: SetOfLinkConnections;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

linkAndLinkConnectionNotCompatible: involvedLink;

invalidLinkConnection: linkConnection;

notAssignedToCaller: linkConnection;

failureToDe-assignLinkConnection: NULL;

failureToIncreaseCapacity: NULL;

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT plcmLayerNetworkDomain>;

requestingCaller: <INFORMATION ATTRIBUTE plcmCallerId>;

involvedLink: <INFORMATION OBJECT plcmLink>;

involvedLinkConnections ELEMENTS: <INFORMATION OBJECT plcmLinkConnection>;

linkConnection: <INFORMATION OBJECT plcmLinkConnection>;

PRE_CONDITIONS

inv_linkAndLinkConnectionCompatible

"involvedLink and involvedLinkConnections shall refer to element of the same <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_existingDe-assignedLinkConnections

"involvedLinkConnections refers to elementLC in a <linkHasLinkConnections> relationship where involvedLink refers to containerLink."

inv_callerAssignedLinkConnections

"<plcmCallerId> of <plcmLinkConnections> referred to by involvedLinkConnections must be equal to the identifier of requestingCaller."

POST_CONDITIONS

inv_de-assignedLinkConnections

"<plcmCallerId> of <plcmLinkConnections> referred to by involvedLinkConnections is equal to NULL."

inv_capacityIncrease

"<pamAvailableLinkCapacity> of <plcmLink> referred to by involvedLink has been increased by the number (size of 'SET OF' in involvedLinkConnections) of <plcmLinkConnections> that have been requested to be de-assigned.";

EXCEPTIONS

IF PRE_CONDITION inv_linkAndLinkConnectionCompatible NOT_VERIFIED RAISE_EXCEPTION linkAndLinkConnectionNotCompatible;

IF PRE_CONDITION inv_existingDe-assignedLinkConnections NOT_VERIFIED RAISE_EXCEPTION invalidLinkConnection;

IF PRE_CONDITION inv_callerAssignedLinkConnections NOT_VERIFIED RAISE_EXCEPTION notAssignedToCaller;

IF POST_CONDITION inv_de-assignedLinkConnections NOT_VERIFIED RAISE_EXCEPTION failureToDe-assignLinkConnection;

IF POST_CONDITION inv_capacityIncrease NOT_VERIFIED RAISE_EXCEPTION failureToIncreaseCapacity;

;}

### 7.2.2	PLCM NetworkCTP Assignment Interface

The plcm networkCTP assignment interface provides functionality for assigning network CTPs within link ends to callers. It satisfies the enterprise requirements stated in <"Rec. G.852.10", COMMUNITY: Pre-provisioned LinkConnection Management, ACTION: assign transport entities and ACTION: de-assign transport entities>.

COMPUTATIONAL_INTERFACE plcmNetworkCTPAssignmentIfce {

OPERATION	<assignNetworkCTPOnLinkEnd>;

OPERATION	<de-assignNetworkCTPOnLinkEnd>; }

#### 7.2.2.1	assign NetworkCTP on LinkEnd

<COMMUNITY: Pre-provisioned LinkConnection Management, ACTION: assign transport entities>

OPERATION assignNetworkCTPOnLinkEnd {

INPUT_PARAMETERS

layerND: layerNetworkDomainIfce ::= PlcmLayerNetworkDomainQueryIfce

requestingCaller: plcmCallerId

involvedLinkEnd: PlcmLinkEndId

requestedNetworkCTPs: PlcmRequestedNetworkCTPChoice;

OUTPUT_PARAMETERS

providedNetworkCTPs: SetOfNetworkCTPs;

RAISED_EXCEPTIONS

linkEndAndNetworkCTPNotCompatible: involvedLinkEnd;

invalidNetworkCTP: networkCTP;

notEnoughNetworkCTPs: numberOfNetworkCTPs ::= INTEGER;

networkCTPAlreadyAssigned: networkCTP;

inconsistentSignalIdentification: NULL;

inconsistentDirectionality: NULL;

failureToSetNetworkCTPCallerId: NULL;

failureToDecreaseCapacity: NULL;

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT plcmLayerNetworkDomain>;

requestingCaller: <INFORMATION ATTRIBUTE plcmCallerId>;

involvedLinkEnd: <INFORMATION OBJECT plcmLinkEnd>;

requestedNetworkCTPs ELEMENTS: <INFORMATION OBJECT plcmNetworkCTP>;

providedNetworkCTPs ELEMENTS: <INFORMATION OBJECT plcmNetworkCTP>;

networkCTP: <INFORMATION OBJECT plcmNetworkCTP>;

PRE_CONDITIONS

inv_linkEndAndNetworkCTPCompatible

"involvedLinkEnd and requestedNetworkCTPs shall refer to element of the same <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_existingRequestedNetworkCTPs

"requestedNetworkCTPs refers to elementCTP in a <linkEndHasNetworkCTPs> relationship where 	involvedLinkEnd refers to containerLE."

-- 	NOTE – The following invariant is only needed when the numberOfNetworkCTPs choice is used for the requestedNetworkCTPs by the caller.

inv_requestedNetworkCTPsAvailable

"the number of available NetworkCTPs in <pamAvailableLinkCapacity> of involvedLinkEnd has to be greater than or equal to the number of requested NetworkCTPs referred to by requestedNetworkCTPs."

inv_freeNetworkCTPs

"<plcmCallerId> of <plcmNetworkCTPs> referred to by requestedNetworkCTPs must be equal to NULL.";

POST_CONDITIONS

inv_assignedNetworkCTPs

"<plcmCallerId> of <plcmLinkConnection> referred to by requestedNetworkCTPs must be set to the identifier of the caller referred to by requestingCaller."

inv_capacityDecrease

"<pamAvailableLinkCapacity> of <plcmLinkEnd> referred to by involvedLinkEnd has been decreased by the number (size of 'SET OF' in requestedNetworkCTPs) of plcmNetworkCTPs that have been requested to be assigned.";

EXCEPTIONS

IF PRE_CONDITION inv_linkEndAndNetworkCTPCompatible NOT_VERIFIED RAISE_EXCEPTION linkEndAndNetworkCTPNotCompatible;

IF PRE_CONDITION inv_existingRequestedNetworkCTPs NOT_VERIFIED RAISE_EXCEPTION invalidNetworkCTP;

IF PRE_CONDITION inv_requestedNetworkCTPsAvailable NOT_VERIFIED RAISE_EXCEPTION notEnoughNetworkCTPs;

IF PRE_CONDITION inv_freeNetworkCTPs NOT_VERIFIED RAISE_EXCEPTION networkCTPAlreadyAssigned;

IF PRE_CONDITION <INFORMATION_RELATIONSHIP: linkEndHasNetworkCTPs, INVARIANT: inv_signalIdentification> NOT_VERIFIED RAISE_EXCEPTION inconsistentSignalIdentification;

IF PRE_CONDITION <INFORMATION_RELATIONSHIP: linkEndHasNetworkCTPs, INVARIANT: inv_directionality> NOT_VERIFIED RAISE_EXCEPTION inconsistentDirectionality;

IF POST_CONDITION inv_assignedNetworkCTPs NOT_VERIFIED RAISE_EXCEPTION failureToSetNetworkCTPCallerId;

IF POST_CONDITION inv_capacityDecrease NOT_VERIFIED RAISE_EXCEPTION failureToDecreaseCapacity;

; }

#### 7.2.2.2	de-assign NetworkCTP on LinkEnd

<COMMUNITY: Pre-provisioned LinkConnection Management, ACTION: de-assign transport entities>

OPERATION de-assignNetworkCTPOnLinkEnd {

INPUT_PARAMETERS

layerND: layerNetworkDomainIfce ::= PlcmLayerNetworkDomainQueryIfce

requestingCaller: plcmCallerId

involvedLinkEnd: PlcmLinkEndId

involvedNetworkCTPs: SetOfNetworkCTPs;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

linkEndAndNetworkCTPNotCompatible: involvedLinkEnd;

invalidNetworkCTP: networkCTP;

notAssignedToCaller: networkCTP;

failureToDe-assignNetworkCTP: NULL;

failureToIncreaseCapacity: NULL;

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT plcmLayerNetworkDomain>;

requestingCaller: <INFORMATION ATTRIBUTE plcmCallerId>;

involvedLinkEnd: <INFORMATION OBJECT plcmLinkEnd>;

involvedNetworkCTPs ELEMENTS: <INFORMATION OBJECT plcmNetworkCTP>;

networkCTP: <INFORMATION OBJECT plcmNetworkCTP>;

PRE_CONDITIONS

inv_linkEndAndNetworkCTPCompatible

"involvedLinkEnd and involvedNetworkCTPs shall refer to element of the same <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_existingDe-assignedNetworkCTPs

"involvedNetworkCTPs refers to elementCTP in a <linkEndHasNetworkCTPs> relationship where involvedLinkEnd refers to containerLE."

inv_callerAssignedNetworkCTPs

"<plcmCallerId> of <plcmNetworkCTP> referred to by involvedNetworkCTPs must be equal to the identifier of requestingCaller."

POST_CONDITIONS

inv_de-assignedNetworkCTPs

"<plcmCallerId> of <plcmNetworkCTP> referred to by involvedNetworkCTPs is equal to NULL."

inv_capacityIncrease

"<pamAvailableLinkCapacity> of <plcmLinkEnd> referred to by involvedLinkEnd has been increased by the number (size of 'SET OF' in involvedNetworkCTPs) of plcmNetworkCTPs that have been requested to be de-assigned.";

EXCEPTIONS

IF PRE_CONDITION inv_linkEndAndNetworkCTPCompatible NOT_VERIFIED RAISE_EXCEPTION linkEndAndNetworkCTPNotCompatible;

IF PRE_CONDITION inv_existingDe-assignedNetworkCTPs NOT_VERIFIED RAISE_EXCEPTION invalidNetworkCTP;

IF PRE_CONDITION inv_callerAssignedNetworkCTPs NOT_VERIFIED RAISE_EXCEPTION notAssignedToCaller;

IF POST_CONDITION inv_de-assignedNetworkCTPs NOT_VERIFIED RAISE_EXCEPTION failureToDe-assignNetworkCTP;

IF POST_CONDITION inv_capacityIncrease NOT_VERIFIED RAISE_EXCEPTION failureToIncreaseCapacity;

;}

## 7.3	Report interfaces

### 7.3.1	PLCM Reporting Arcview Interface

The plcm reporting arcview interface provides functionality for the reporting of the assignment of link connections to callers and for the de-assignment from callers. It satisfies the enterprise requirements stated in <"Rec. G.852.10", COMMUNITY: Pre-provisioned LinkConnection Management, ACTION: report transport entities change>.

COMPUTATIONAL_INTERFACE plcmReportingArcviewIfce {

OPERATION	<reportLinkConnectionAssignedOnLink>;

OPERATION	<reportLinkConnectionDesssignOnLink>; }

#### 7.3.1.1	Report Link Connection Assigned On Link

<COMMUNITY: Pre-provisioned LinkConnection Management, ACTION: report transport entities change>

OPERATION reportLinkConnectionAssignedOnLink {

INPUT_PARAMETERS

layerND: layerNetworkDomainIfce ::= PlcmLayerNetworkDomainQueryIfce

involvedCaller: plcmCallerId

involvedLink: PlcmLinkId

assignedLinkConnections: SetOfLinkConnections;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

-- none

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT plcmLayerNetworkDomain>;

involvedCaller: <INFORMATION ATTRIBUTE plcmCallerId>;

involvedLink: <INFORMATION OBJECT plcmLink;

assignedLinkConnections ELEMENTS: <INFORMATION OBJECT plcmLinkConnection>;

TRIGGERING_CONDITIONS

PRE-CONDITIONS

inv_linkAndLinkConnectionExistingAndCompatible

"involvedLink and assignedLinkConnections shall refer to element of the same <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_involvedLinkAndAssignedLinkConnectionsCompatible

"assignedLinkConnections refers to elementLC in a <linkHasLinkConnections> relationship where involvedLink refers to containerLink."

inv_freeLinkConnections

"<plcmCallerId> of <plcmLinkConnection> referred to by assignedLinkConnections must be equal to NULL.";

POST_CONDITIONS

inv_assignedLinkConnection

"<plcmCallerId> of <plcmLinkConnection> referred to by assignedLinkConnections must be set to the identifier of the caller referred to by involvedCaller."

inv_capacityDecrease

"<pamAvailableLinkCapacity> of <plcmLink> referred to by involvedLink has been decreased by the number of plcmLinkConnections that have been requested to be assigned.";

EXCEPTIONS

-- none

;}

#### 7.3.1.2	Report Link Connection De-Assigned On Link

<COMMUNITY: Pre-provisioned LinkConnection Management, ACTION: report transport entities change>

OPERATION reportLinkConnectionDe-assignOnLink {

INPUT_PARAMETERS

layerND: layerNetworkDomainIfce ::= PlcmLayerNetworkDomainQueryIfce

involvedLink: PlcmLinkId

de-assignedLinkConnections: SetOfLinkConnections;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

-- none

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT plcmLayerNetworkDomain>;

involvedLink: <INFORMATION OBJECT plcmLink>;

de-assignedLinkConnections ELEMENTS: <INFORMATION OBJECT plcmLinkConnection>;

TRIGGERING_CONDITIONS

PRE-CONDITIONS

inv_linkAndLinkConnectionExistingAndCompatible

"involvedLink and de-assignedLinkConnections shall refer to element of the same <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_involvedLinkAndDe-assignedLinkConnectionsCompatible

"de-assignedLinkConnections refers to elementLC in a <linkHasLinkConnections> relationship where involvedLink refers to containerLink."

inv_assignedLinkConnections

"<plcmCallerId> of <plcmLinkConnection> referred to by de-assignedLinkConnections is different from NULL.";

POST_CONDITIONS

inv_de-assignedLinkConnections

"<plcmCallerId> of <plcmLinkConnection> referred to by de-assignedLinkConnections is equal to NULL."

inv_capacityIncrease

"<pamAvailableLinkCapacity> of <plcmLink> referred to by involvedLink has been increased by the number of plcmLinkConnections that have been requested to be de-assigned.";

EXCEPTIONS

-- none

;}

### 7.3.2	PLCM Reporting Pointview Interface

The plcm reporting pointview interface provides functionality for the reporting of the assignment of network CTPs to callers and for the de-assignment from callers. It satisfies the enterprise requirements stated in <"Rec. G.852.10", COMMUNITY: Pre-provisioned LinkConnection Management, ACTION: report transport entities change>

COMPUTATIONAL_INTERFACE plcmReportingPointviewIfce {

OPERATION	<reportNetworkCTPAssignedOnLink>;

OPERATION	<reportNetworkCTPDesssignOnLink>; }

#### 7.3.2.1	Report Network CTP Assigned On Link End

<COMMUNITY: Pre-provisioned LinkConnection Management, ACTION: report transport entities change>

OPERATION reportNetworkCTPAssignedOnLinkEnd {

INPUT_PARAMETERS

layerND: layerNetworkDomainIfce ::= PlcmLayerNetworkDomainQueryIfce

involvedCaller: plcmCallerId

involvedLinkEnd: PlcmLinkEndId

assignedNetworkCTPs: SetOfNetworkCTPs;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

-- none

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerNDId: <INFORMATION OBJECT plcmLayerNetworkDomain>;

involvedCaller: <INFORMATION ATTRIBUTE plcmCallerId>;

involvedLinkEnd: <INFORMATION OBJECT plcmLinkEnd>;

assignedNetworkCTPs ELEMENTS: <INFORMATION OBJECT plcmNetworkCTP>;

TRIGGERING_CONDITIONS

PRE-CONDITIONS

inv_linkAndLinkConnectionExistingAndCompatible

"involvedLinkEnd and requestedNetworkCTPs shall refer to element of the same <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_involvedLinkEndAndAssignedNetworkCTPsCompatible

"assignedNetworkCTPs refers to elementCTP in a <linkEndHasNetworkCTPs> relationship where involvedLinkEnd refers to containerLE."

inv_freeNetworkCTPs

"<plcmCallerId> of <plcmNetworkCTP> referred to by assignedNetworkCTPs must be equal to NULL.";

POST_CONDITIONS

inv_assignedNetworkCTP

"<plcmCallerId> of <plcmNetworkCTP> referred to by assignedNetworkCTPs must be set to the identifier of the caller referred to by involvedCaller."

inv_capacityDecrease

"<pamAvailableLinkCapacity> of <plcmLinkEnd> referred to by involvedLinkEnd has been decreased by the number of plcmNetworkCTPs that have been requested to be assigned.";

EXCEPTIONS

-- none

;}

#### 7.3.2.2	Report  network CTP de-assigned on link end

<COMMUNITY: Pre-provisioned LinkConnection Management, ACTION: report transport entities change>

OPERATION reportNetworkCTPDe-assignOnLinkEnd {

INPUT_PARAMETERS

layerND: layerNetworkDomainIfce ::= PlcmLayerNetworkDomainQueryIfce

involvedLinkEnd: PlcmLinkEndId

de-assignedNetworkCTPs: SetOfNetworkCTPs;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

-- none

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerNDId: <INFORMATION OBJECT plcmLayerNetworkDomain>;

involvedLinkEnd: <INFORMATION OBJECT plcmLinkEnd>;

de-assignedNetworkCTPs ELEMENTS: <INFORMATION OBJECT plcmNetworkCTP>;

TRIGGERING_CONDITIONS

PRE-CONDITIONS

inv_linkAndLinkConnectionExistingAndCompatible

"involvedLinkEnd and de-assignedNetworkCTPs shall refer to element of the same <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_involvedLinkEndAndDe-assignedNetworkCTPsCompatible

"de-assignedNetworkCTPs refers to elementCTP in a <linkEndHasNetworkCTPs> relationship where involvedLinkEnd refers to containerLE."

inv_assignedNetworkCTPs

"<plcmCallerId> of <plcmNetworkCTP> referred to by de-assignedNetworkCTPs is different from NULL.";

POST_CONDITIONS

inv_de-assignedNetworkCTPs

"<plcmCallerId> of <plcmNetworkCTP> referred to by de-assignedNetworkCTPs is equal to NULL."

inv_capacityIncrease

"<pamAvailableLinkCapacity> of <plcmLinkEnd> referred to by involvedLinkEnd has been increased by the number of plcmNetworkCTPs that have been requested to be de-assigned.";

EXCEPTIONS

-- none

;}

## 7.4	ASN.1 supporting productions

This subclause defines the ASN.1 productions which are used within other ASN.1 productions in the operations.

In this Recommendation, when an interface name is used within an ASN.1 production, the same label will be used, starting with a capital letter. The complete ASN.1 type definition for this query interface (e.g. use of ObjectIdentifier, INTEGER, ...) will be developed as part of the engineering viewpoint, with the concerned technology.

PlcmLinkEndId ::= CHOICE {

linkEndQueryInterface	PlcmLinkEndQueryInterface,
	userIdentifier	SimpleNameType };

PlcmLinkId ::= CHOICE {

linkQueryInterface	PlcmLinkQueryInterface,
	userIdentifier	SimpleNameType };

PlcmRequestedLinkConnectionChoice ::= CHOICE {

plcmLinkConnections	SetOfLinkConnections,
	numberOfLC	INTEGER };

PlcmRequestedNetworkCTPChoice ::= CHOICE {

plcmNetworkCTPs	SetOfNetworkCTPs,
	numberOfNetworkCTPs	INTEGER };

SetOfLinkConnections ::= SET OF CHOICE {

linkConnectionId	PlcmLinkConnectionQueryIfce,
	userIdentifier	SimpleNameType }

SetOfNetworkCTPs ::= SET OF CHOICE {

networkCTPId	PlcmNetworkCTPQueryIfce,
	userIdentifier	SimpleNameType}

| ITU-T RECOMMENDATIONS SERIES |  |
| --- | --- |
| Series A | Organization of the work of the ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |