|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

| ITU-T | G.854.12 |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU | (03/99) |

SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

Digital transmission systems – Digital networks – Management of transport network

Computational viewpoint for pre-provisioned link management

ITU-T  Recommendation  G.854.12

(Previously  CCITT  Recommendation)

ITU-T  G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| For further details, please refer to ITU-T List of Recommendations |  |
| --- | --- |
| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| INTERNATIONAL ANALOGUE CARRIER SYSTEM |  |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS |  |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TRANSMISSION SYSTEMS |  |
| TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| General aspects | G.800–G.809 |
| Design objectives for digital networks | G.810–G.819 |
| Quality and availability targets | G.820–G.829 |
| Network capabilities and functions | G.830–G.839 |
| SDH network characteristics | G.840–G.849 |
| Management of transport network | G.850–G.859 |
| SDH radio and satellite systems integration | G.860–G.869 |
| Optical transport networks | G.870–G.879 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |

ITU-T RECOMMENDATION G.854.12 computational viewpoint for pre-provisioned link management

Summary The pre-provisioned link management service provides functionality to add/remove transport entities (link connections or connection termination points) to/from client linking entities (links/link ends). The client linking entities have to be created using the topology management community (see G.85x.3 series of Recommendations). The transport entities that could be potentially added to the client linking entity have to be provided using either the pre-provisioned adaptation management (see G.85x.8 series of Recommendations) within topological linking entities or using the link management within client linking entities. Enterprise community actions covered by this Recommendation: – COMMUNITY plm "pre-provisioned link management"; – "add transport entities to client linking entity"; – "remove transport entities from client linking entity"; – "report client linking entity capacity change".

Source ITU-T Recommendation G.854.12 was prepared by ITU-T Study Group 4 (1997-2000) and was approved under the WTSC Resolution No. 1 procedure on the 26th of March 1999.

FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation the term recognized operating agency (ROA) includes any individual, company, corporation or governmental organization that operates a public correspondence service. The terms Administration, ROA and public correspondence are defined in the Constitution of the ITU (Geneva, 1992).

INTELLECTUAL PROPERTY RIGHTS

The ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. The ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, the ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  1999

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Definitions		1

4	Abbreviations		1

5	Conventions		2

6	Label references		2

7	Interfaces		3

7.1	Query interfaces		3

7.2	Operational interfaces		4

7.2.1	Pre-provisioned link management interface		4

7.2.2	Pre-provisioned link end management interface		6

7.3	Report interfaces		9

7.3.1	Pre-provisioned link management reporting interface		9

7.3.2	Pre-provisioned link end management reporting interface		10

7.4	ASN.1 supporting productions		12

Recommendation G.854.12

Computational viewpoint FOR PRE-PROVISIONED
LINK MANAGEMENT

(Geneva, 1999)

# 1	Scope

This computational viewpoint specification is related to the pre-provisioned link management enterprise specification defined in Recommendation G.852.12 and to the pre-provisioned link management information specification defined in Recommendation G.853.12.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[1]	ITU-T Recommendation G.851.1 (1996), Management of the transport network – Application of the RM-ODP framework.

[2]	ITU-T Recommendation G.853.1 (1999), Common elements of the information viewpoint for the management of a transport network.

[3]	ITU-T Recommendation G.852.12 (1999), Enterprise viewpoint for pre-provisioned link management.

[4]	ITU-T Recommendation G.853.12 (1999), Information viewpoint for pre-provisioned link management.

[5]	ITU-T Recommendation G.852.8 (1999), Enterprise viewpoint for pre-provisioned adaptation management.

[6]	ITU-T Recommendation G.853.8 (1999), Information viewpoint for pre-provisioned adaptation management.

# 3	Definitions

None.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

ASN.1		Abstract Syntax Notation One

CTP		Connection Termination Point

Id		Identifier

Ifce		Interface

layerND	Layer Network Domain

LC		Link Connection

LE		LinkEnd

LND		Layer Network Domain

ND		Network Domain

pam		Pre-provisioned Adaptation Management

plm		Pre-provisioned Link Management

# 5	Conventions

In order to increase the readability of the behaviour in the operations:

–	parameters are written in bold;

–	elements defined in the information specification are written in italic.

In this Recommendation, when an interface is used in an ASN.1 production, the same label will be used starting with a capital letter. This complete ASN.1 production for this Query Interface (e.g. use of OBJECT IDENTIFIER, INTEGER, etc.) will be developed as part of the Engineering Viewpoint within the concerned technology.

# 6	Label references

| Table 1/G.854.12 – Label references |  |
| --- | --- |
| Full label reference | Local label reference |
| <"Rec. G.853.12", INFORMATION_OBJECT:plmClientLink> | plmClientLink |
| <"Rec. G.853.12", INFORMATION_OBJECT:plmClientLinkEnd> | plmClientLinkEnd |
| <"Rec. G.853.12", INFORMATION_OBJECT:plmLayerNetworkDomain> | plmLayerNetworkDomain |
| <"Rec. G.853.12", INFORMATION_OBJECT:plmLinkConnection> | plmLinkConnection |
| <"Rec. G.853.12", INFORMATION_OBJECT:plmNetworkCTP> | plmNetworkCTP |
| <"Rec. G.853.12", INFORMATION_OBJECT:plmProviderLink> | plmProviderLink |
| <"Rec. G.853.12", INFORMATION_OBJECT:plmProviderLinkEnd> | plmProviderLinkEnd |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP:linkHasLink Connections> | linkHasLinkConnections |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP:linkEndHas NetworkCTPs> | linkEndHasNetworkCTPs |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP:representSame ResourceAs> | representSameResourceAs |
| <"Rec. G.853.8", ATTRIBUTE:pamAvailableLinkCapacity> | pamAvailableLinkCapacity |

| Full ASN.1 production reference | Local label reference |
| --- | --- |
| <"Rec. X.721 : 1992 : Attribute-ASN1Module" : SimpleNameType> | SimpleNameType |
| <"Rec. X.680 : 1997" : INTEGER> | INTEGER |

# 7	Interfaces

## 7.1	Query interfaces

This specification refers to interfaces that allow get access identification and properties of resources involved in the "pre-provisioned link management" community. As the invocation of contained operations do not modify any state, there is no interest to develop them explicitly. Their exact signature will be developed as part of the engineering viewpoint, with the concerned technology. These interfaces are listed in Table 2 with the information to which they allow access to.

Table 2/G.854.12 – Correspondence between interface names, information objects,
attributes and relationships

| Interface name | Information object | Attributes and relationships |
| --- | --- | --- |
| plmClientLinkQueryIfce | <plmClientLink> | <resourceId> <signalIdentification> <linkDirectionality> <layerNetworkDomainIsMadeOf, ROLE: containerLND> <linkHasLinkConnections, ROLE: elementLC> |
| plmClientLinkEndQueryIfce | <plmClientLinkEnd> | <resourceId> <signalIdentification> <topologicalEndDirection> <layerNetworkDomainIsMadeOf, ROLE: containerLND> <linkEndHasNetworkCTPs, ROLE: elementCTP> |
| plmLayerNetworkDomainQueryIfce | <plmLayerNetworkDomain> | <resourceId> <signalIdentification> <layerNetworkDomainIsMadeOf, ROLE: element> |
| plmLinkConnectionQueryIfce | <plmLinkConnection> | <resourceId> <signalIdentification> <directionality> <layerNetworkDomainIsMadeOf, ROLE: containerLND> <linkHasLinkConnections, ROLE: containerLink> |
| plmNetworkCTPQueryIfce | <plmNetworkCTP> | <resourceId> <layerNetworkDomainIsMadeOf, ROLE: containerLND> <linkEndHasNetworkCTPs, ROLE: containerLE> |

Table 2/G.854.12 – Correspondence between interface names, information objects,
attributes and relationships (concluded)

| Interface name | Information object | Attributes and relationships |
| --- | --- | --- |
| plmProviderLinkQueryIfce | <plmProviderLink> | <resourceId> <signalIdentification> <linkDirectionality> <pamAvailableLinkCapacity> <layerNetworkDomainIsMadeOf, ROLE: containerLND> <linkHasLinkConnections, ROLE: elementLC> |
| plmProviderLinkEndQueryIfce | <plmProviderLinkEnd> | <resourceId> <signalIdentification> <topologicalEndDirection> <pamAvailableLinkCapacity> <layerNetworkDomainIsMadeOf, ROLE: containerLND> <linkEndHasNetworkCTPs, ROLE: elementCTP> |

## 7.2	Operational interfaces

### 7.2.1	Pre-provisioned link management interface

The pre-provisioned link management interface provides functionality for adding and removing link connections within links (arc-oriented view). It satisfies the enterprise requirements stated in <"Rec. G.852.12", COMMUNITY: Pre-provisioned Link Management, ACTION: add transport entities to client linking entity and ACTION: remove transport entities from client linking entity>.

COMPUTATIONAL_INTERFACE plmLinkManagementIfce {

OPERATION  <addLinkConnectionToLink>;

OPERATION  <removeLinkConnectionFromLink>; }

#### 7.2.1.1	add linkConnection to link

<COMMUNITY:Pre Provisioned Link Management, ACTION: add transport entities to client linking entity>

OPERATION addLinkConnectionToLink {

INPUT_PARAMETERS

layerND: layerNetworkDomainIfce ::= PlmLayerNetworkDomainQueryIfce

involvedClientLink: PlmClientLinkId;

involvedProviderLink: PlmProviderLinkId;

requestedLinkConnections: PlmRequestedLinkConnectionChoice;

OUTPUT_PARAMETERS

providedLinkConnections: SetOfPLMLinkConnections;

RAISED_EXCEPTIONS

clientLinkDoNotExist: plmClientLink;

providerLinkDoNotExist: plmProviderLink;

linkAndLinkConnectionNotCompatible: plmLinkConnection;

notEnoughLinkConnections: number ::= INTEGER;

noLinkCapacity: NULL;

failureToAddLinkConnections: NULL;

failureToIncreaseCapacity: NULL;

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT plmLayerNetworkDomain>;

involvedClientLink: <INFORMATION OBJECT plmClientLink>;

involvedProviderLink: <INFORMATION OBJECT plmProviderLink>;

setOfPLMLinkConnections ELEMENTS: <INFORMATION OBJECT

plmLinkConnection>;

PRE_CONDITIONS

inv_clientLinkAvailable

"involvedClientLink shall refer to element of the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_providerLinkAvailable

"involvedProviderLink shall refer to element of the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_linkAndLinkConnectionCompatible

"requestedLinkConnections shall refer to elementLC of the <linkHasLinkConnections> relationship where involvedProviderLink refers to containerLink."

inv_requestedNumberOfLinkConnectionsAvailable

"the number of available (available means: the requested link connections shall not participate in any <representSameResourceAs> relationship) linkConnections in <pamAvailableLinkCapacity> of involvedProviderLink has to be greater than or equal to the number of requested linkConnections referred to by requestedLinkConnections."

inv_linkCapacityNotEmpty

"the number of available link connections in <pamAvailableLinkCapacity> of involvedProviderLink has to be greater than zero."

POST_CONDITIONS

inv_addLinkConnections

"every link connection of requestedLinkConnections is an elementLC in a <linkHasLinkConnections> relationship where involvedClientLink refers to containerLink."

inv_capacityIncrease

"<pamAvailableLinkCapacity> of <plmLink> referred to by involvedClientLink has been increased by the number of <plmLinkConnections> that have been requested to be added in requestedLinkConnections.";

EXCEPTIONS

IF PRE_CONDITION inv_clientLinkAvailable NOT_VERIFIED

RAISE_EXCEPTION clientLinkDoNotExist;

IF PRE_CONDITION inv_providerLinkAvailable NOT_VERIFIED

RAISE_EXCEPTION providerLinkDoNotExist;

IF PRE_CONDITION inv_linkAndLinkConnectionCompatible NOT_VERIFIED

RAISE_EXCEPTION linkAndLinkConnectionNotCompatible;

IF PRE_CONDITION inv_requestedNumberOfLinkConnectionsAvailable

NOT_VERIFIED RAISE_EXCEPTION notEnoughLinkConnections;

IF PRE_CONDITION inv_ linkCapacityNotEmpty NOT_VERIFIED

RAISE_EXCEPTION noLinkCapacity;

IF POST_CONDITION inv_addLinkConnections NOT_VERIFIED

RAISE_EXCEPTION failureToAddLinkConnections;

IF POST_CONDITION inv_capacityIncrease NOT_VERIFIED RAISE_EXCEPTION

failureToIncreaseCapacity;

; }

#### 7.2.1.2	remove LinkConnection from Link

<COMMUNITY: Pre Provisioned Link Management, ACTION: remove transport entities from client linking entity>

OPERATION removeLinkConnectionFromLink {

INPUT_PARAMETERS

layerND: layerNetworkDomainIfce ::= PlmLayerNetworkDomainQueryIfce

involvedClientLink: PlmClientLinkId;

involvedLinkConnections: SetOfPlmLinkConnections;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

linkDoNotExist: plmClientLink;

invalidLinkConnection: plmLinkConnection;

failureToRemoveLCs: NULL;

failureToDecreaseCapacity: NULL;

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT plmLayerNetworkDomain>;

involvedClientLink: <INFORMATION OBJECT plmClientLink>;

involvedLinkConnections ELEMENTS: <INFORMATION OBJECT plmLinkConnection>;

PRE_CONDITIONS

inv_linkAvailable

"involvedClientLink shall refer to element of the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_existingLinkConnections

"involvedLinkConnections refers to elementLC in a <linkHasLinkConnections> relationship where involvedClientLink refers to containerLink."

POST_CONDITIONS

inv_LinkConnectionsRemoved

"None of the involvedLinkConnections refer to elementLC in a <linkHasLinkConnections> relationship where involvedClientLink refers to containerLink."

inv_capacityDecrease

"<pamAvailableLinkCapacity> of <plmLink> referred to by involvedClientLink has been decreased by the number of <plmLinkConnections> in involvedLinkConnections that have been requested to be removed.";

EXCEPTIONS

IF PRE_CONDITION inv_linkAvailable NOT_VERIFIED RAISE_EXCEPTION linkDoNotExist;

IF PRE_CONDITION inv_existingLinkConnections NOT_VERIFIED RAISE_EXCEPTION invalidLinkConnection;

IF POST_CONDITION inv_LinkConnectionsRemoved NOT_VERIFIED RAISE_EXCEPTION failureToRemoveLCs;

IF POST_CONDITION inv_capacityDecrease NOT_VERIFIED RAISE_EXCEPTION 	failureToDecreaseCapacity;

}

### 7.2.2	Pre-provisioned link end management interface

The pre-provisioned link end management interface provides functionality for adding and removing CTPs within link ends (point-oriented view). It satisfies the enterprise requirements stated in <"Rec. G.852.12", COMMUNITY: Pre-provisioned Link Management, ACTION: add transport entities to client linking entity and ACTION: remove transport entities from client linking entity>.

COMPUTATIONAL_INTERFACE plmLinkEndManagementIfce {

OPERATION <addNetworkCTPToLinkEnd>;

OPERATION <removeNetworkCTPFromLinkEnd>;

#### 7.2.2.1	add NetworkCTP to Link End

<COMMUNITY:Pre Provisioned Link Management, ACTION: add transport entities to client linking entity>

OPERATION addNetworkCTPToLinkEnd {

INPUT_PARAMETERS

layerND: layerNetworkDomainIfce ::= PlmLayerNetworkDomainQueryIfce

involvedClientLinkEnd: PlmClientLinkEndId;

involvedProviderLinkEnd: PlmProviderLinkEndId;

requestedNetworkCTPs: PlmRequestedNetworkCTPChoice;

OUTPUT_PARAMETERS

providedNetworkCTPs: SetOfPLMNetworkCTPs;

RAISED_EXCEPTIONS

clientLinkEndDoNotExist: plmClientLinkEnd;

providerLinkEndDoNotExist: plmProviderLinkEnd;

linkEndAndNetworkCTPNotCompatible: plmNetworkCTP;

notEnoughNetworkCTPs: number ::= INTEGER;

noLinkEndCapacity: NULL;

failureToAddNetworkCTPs: NULL;

failureToIncreaseCapacity: NULL;

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT plmLayerNetworkDomain>;

involvedClientLinkEnd: <INFORMATION OBJECT plmClientLinkEnd>;

involvedProviderLinkEnd: <INFORMATION OBJECT plmProviderLinkEnd>;

setOfPLMNetworkCTPs ELEMENTS: <INFORMATION OBJECT plmNetworkCTP>;

PRE_CONDITIONS

inv_clientLinkEndAvailable

"involvedClientLinkEnd shall refer to element of the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_providerLinkEndAvailable

"involvedProviderLinkEnd shall refer to element of the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_linkEndAndNetworkCTPCompatible

"requestedNetworkCTPs shall refer to elementCTP of the <linkEndHasNetworkCTPs> relationship where involvedProviderLinkEnd refers to containerLE."

inv_requestedNumberOfNetworkCTPsAvailable

"the number of available (available means: the requested networkCTPs shall not participate in any <representSameResourceAs> relationship) networkCTPs in <pamAvailableLinkCapacity> of involvedProviderLinkEnd has to be greater than or equal to the number of requested networkCTPs referred to by requestedNetworkCTPs."

inv_linkEndCapacityNotEmpty

"the number of available networkCTPs in <pamAvailableLinkCapacity> of involvedProviderLinkEnd has to be greater than zero."

POST_CONDITIONS

inv_addNetworkCTPs

"every networkCTP of requestedNetworkCTPs refers to elementCTP in a <linkEndHasNetworkCTPs> relationship where involvedClientLinkEnd refers to containerLE."

inv_capacityIncrease

"<pamAvailableLinkCapacity> of <plmLinkEnd> referred to by involvedClientLinkEnd has been increased by the number of <plmNetworkCTPs> that have been requested to be added in requestedNetworkCTPs.";

EXCEPTIONS

IF PRE_CONDITION inv_clientLinkEndAvailable NOT_VERIFIED RAISE_EXCEPTION clientLinkEndDoNotExist;

IF PRE_CONDITION inv_providerLinkEndAvailable NOT_VERIFIED RAISE_EXCEPTION providerLinkEndDoNotExist;

IF PRE_CONDITION inv_linkEndAndNetworkCTPCompatible NOT_VERIFIED RAISE_EXCEPTION linkEndAndNetworkCTPNotCompatible;

IF PRE_CONDITION inv_requestedNumberOfNetworkCTPsAvailable NOT_VERIFIED RAISE_EXCEPTION notEnoughNetworkCTPs;

IF PRE_CONDITION inv_ linkEndCapacityNotEmpty NOT_VERIFIED RAISE_EXCEPTION noLinkEndCapacity;

IF POST_CONDITION inv_addNetworkCTPs NOT_VERIFIED RAISE_EXCEPTION failureToAddNetworkCTPs;

IF POST_CONDITION inv_capacityIncrease NOT_VERIFIED RAISE_EXCEPTION failureToIncreaseCapacity;

; }

#### 7.2.2.2	remove NetworkCTP from Link End

<COMMUNITY: Pre Provisioned Link Management, ACTION: remove transport entities from client linking entity>

OPERATION removeNetworkCTPFromLinkEnd {

INPUT_PARAMETERS

layerND: layerNetworkDomainIfce ::= PlmLayerNetworkDomainQueryIfce

involvedClientLinkEnd: PlmClientLinkEndId;

involvedNetworkCTPs: SetOfPlmNetworkCTPs;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

linkEndDoNotExist: plmClientLinkEnd;

invalidNetworkCTP: plmNetworkCTP;

failureToRemoveNetworkCTPs: NULL;

failureToDecreaseCapacity: NULL;

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT plmLayerNetworkDomain>;

involvedClientLinkEnd: <INFORMATION OBJECT plmClientLinkEnd>;

involvedNetworkCTPs ELEMENTS: <INFORMATION OBJECT plmNetworkCTP>;

PRE_CONDITIONS

inv_linkEndAvailable

"involvedClientLinkEnd shall refer to element of the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_existingNetworkCTPs

"involvedNetworkCTPs refers to elementCTP in a <linkEndHasNetworkCTPs> relationship where involvedLinkEnd refers to containerLE."

POST_CONDITIONS

inv_networkCTPsRemoved

"None of the involvedNetworkCTPs shall refer to elementCTP in a <linkEndHasNetworkCTPs> relationship where involvedLinkEnd refers to containerLE."

inv_capacityDecrease

"<pamAvailableLinkCapacity> of <plmLinkEnd> referred to by involvedLinkEnd has been decreased by the number of <plmNetworkCTPs> in involvedNetworkCTPs that have been requested to be removed.";

EXCEPTIONS

IF PRE_CONDITION inv_linkEndAvailable NOT_VERIFIED RAISE_EXCEPTION

linkEndDoNotExist;

IF PRE_CONDITION inv_existingNetworkCTPs NOT_VERIFIED

RAISE_EXCEPTION invalidNetworkCTP;

IF POST_CONDITION inv_networkCTPsRemoved NOT_VERIFIED

RAISE_EXCEPTION failureToRemoveNetworkCTPs;

IF POST_CONDITION inv_capacityDecrease NOT_VERIFIED RAISE_EXCEPTION

failureToDecreaseCapacity;

}

## 7.3	Report interfaces

### 7.3.1	Pre-provisioned link management reporting interface

The pre-provisioned link management reporting interface provides functionality for the reporting of the adding/removing of link connections to/from links (arc-oriented view). It satisfies the enterprise requirements stated in <"Rec. G.852.10", COMMUNITY: Pre-provisioned Link Management, ACTION: report client linking entity capacity change>.

COMPUTATIONAL_INTERFACE plmLinkManagementReportingIfce {

OPERATION <reportLinkConnectionAddedToLink>;

OPERATION <reportLinkConnectionRemovedFromLink>; }

#### 7.3.1.1	Report Link Connection Added To Link

<COMMUNITY: Pre Provisioned Link Connection Management, ACTION: report client linking entity capacity change>

OPERATION reportLinkConnectionAddedToLink {

INPUT PARAMETERS

layerND: layerNetworkDomainIfce ::= PlmLayerNetworkDomainQueryIfce

involvedClientLink: PlmClientLinkId;

addedLinkConnections: SetOfPlmLinkConnections;

OUTPUT PARAMETERS

-- none

RAISED EXCEPTIONS

-- none

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT plmLayerNetworkDomain>;

involvedClientLink: <INFORMATION OBJECT plmClientLink>;

addedLinkConnections ELEMENTS: <INFORMATION OBJECT plmLinkConnection>;

TRIGGERING CONDITIONS

PRE-CONDITIONS

inv_linkAndLinkConnectionExistingAndCompatible

"involvedClientLink and addedLinkConnections shall refer to element of the same <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_notInLink

"None of the link connections in addedLinkConnections shall refer to elementLC in a <linkHasLinkConnections> relationship where involvedClientLink refers to containerLink."

POST_CONDITIONS

inv_inLink

"Every link connection in addedLinkConnections shall refer to elementLC in a <linkHasLinkConnections> relationship where involvedClientLink refers to containerLink."

EXCEPTIONS

-- none

;}

#### 7.3.1.2	Report Link Connection Removed From Link

<COMMUNITY: Pre Provisioned Link Connection Management, ACTION: report client linking entity capacity change>

OPERATION  reportLinkConnectionRemovedFromLink {

INPUT PARAMETERS

layerND: layerNetworkDomainIfce ::= PlmLayerNetworkDomainQueryIfce

involvedClientLink: PlmClientLinkId;

removedLinkConnections: SetOfPlmLinkConnections;

OUTPUT PARAMETERS

-- none

RAISED EXCEPTIONS

-- none

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT plmLayerNetworkDomain>;

involvedClientLink: <INFORMATION OBJECT plmClientLink>

removedLinkConnections ELEMENTS: <INFORMATION OBJECT plmLinkConnection>

TRIGGERING CONDITIONS

PRE-CONDITIONS

inv_linkAndLinkConnectionExistingAndCompatible

"involvedClientLink and removedLinkConnections shall refer to element of the same <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_inLink

"Every link connection in removedLinkConnections shall refer to elementLC in a <linkHasLinkConnections> relationship where involvedClientLink refers to containerLink."

POST_CONDITIONS

inv_notInLink

"None of the link connections in removedLinkConnections shall refer to elementLC in a <linkHasLinkConnections> relationship where involvedClientLink refers to containerLink."

EXCEPTIONS

-- none

;}

### 7.3.2	Pre-provisioned link end management reporting interface

The pre-provisioned link end management reporting interface provides functionality for the reporting of the adding/removing of networkCTPs to/from link ends (point-oriented view). It satisfies the enterprise requirements stated in <"Rec. G.852.10", COMMUNITY: Pre-provisioned Link Management, ACTION: report client linking entity capacity change>.

COMPUTATIONAL_INTERFACE plmLinkEndManagementReportingIfce {

OPERATION <reportNetworkCTPAddedToLinkEnd>;

OPERATION <reportNetworkCTPRemovedFromLinkEnd>; }

#### 7.3.2.1	Report NetworkCTP Added To Link End

<COMMUNITY: Pre Provisioned Link Connection Management, ACTION: report client linking entity capacity change>

OPERATION reportNetworkCTPAddedToLinkEnd {

INPUT PARAMETERS

layerND: layerNetworkDomainIfce ::= PlmLayerNetworkDomainQueryIfce

involvedClientLinkEnd: PlmClientLinkEndId;

addedNetworkCTPs: SetOfPlmNetworkCTPs;

OUTPUT PARAMETERS

-- none

RAISED EXCEPTIONS

-- none

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT plmLayerNetworkDomain>;

involvedClientLinkEnd: <INFORMATION OBJECT plmClientLinkEnd>;

addedNetworkCTPs ELEMENTS: <INFORMATION OBJECT plmNetworkCTP>;

TRIGGERING CONDITIONS

PRE-CONDITIONS

inv_linkEndAndNetworkCTPExistingAndCompatible

"involvedClientLinkEnd and addedNetworkCTPs shall refer to element of the same <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_notInLinkEnd

"None of the networkCTPs in addedNetworkCTPs shall refer to elementCTP of a 	<linkEndHasNetworkCTPs> relationship where involvedClientLinkEnd refers to containerLE."

POST_CONDITIONS

inv_inLinkEnd

"Every networkCTP in addedNetworkCTPs shall refer to elementCTP in a <linkEndHasNetworkCTPs> relationship where involvedClientLinkEnd refers to an containerLE."

EXCEPTIONS

-- none

;}

#### 7.3.2.2	Report NetworkCTP Removed From Link End

<COMMUNITY: Pre Provisioned Link Connection Management, ACTION: report client linking entity capacity change>

OPERATION reportNetworkCTPRemovedFromLinkEnd {

INPUT PARAMETERS

layerND: layerNetworkDomainIfce ::= PlmLayerNetworkDomainQueryIfce

involvedClientLinkEnd: PlmClientLinkEndId;

removedNetworkCTPs: SetOfPlmNetworkCTPs;

OUTPUT PARAMETERS

-- none

RAISED EXCEPTIONS

-- none

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT plmLayerNetworkDomain>;

involvedClientLinkEnd: <INFORMATION OBJECT plmClientLinkEnd>;

removedNetworkCTPs ELEMENTS: <INFORMATION OBJECT plmNetworkCTP>;

TRIGGERING CONDITIONS

PRE-CONDITIONS

inv_linkEndAndNetworkCTPExistingAndCompatible

"involvedClientLinkEnd and removedNetworkCTPs shall refer to element of the same <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_inLinkEnd

"Every networkCTP in removedNetworkCTPs shall refer to elementCTP in a <linkEndHasNetworkCTPs> relationship where involvedClientLinkEnd refers to containerLE."

POST_CONDITIONS

inv_notInLinkEnd

"None of the networkCTPs in removedNetworkCTPs refer to elementCTP of a <linkEndHasNetworkCTPs> relationship where involvedClientLinkEnd refers to containerLE."

EXCEPTIONS

-- none

;}

## 7.4	ASN.1 supporting productions

This subclause defines the ASN.1 productions which are used within other ASN.1 productions in the operations.

In this Recommendation, when an interface name is used within an ASN.1 production, the same label will be used, starting with a capital letter. The complete ASN.1 type definition for this query interface (e.g. use of ObjectIdentifier, INTEGER, ...) will be developed as part of the engineering viewpoint, with the concerned technology.

PlmClientLinkEndId ::= CHOICE {
	clientLinkEndQueryInterface			PlmClientLinkEndQueryInterface,
	userIdentifier					SimpleNameType }

PlmClientLinkId ::= CHOICE {
	clientLinkQueryInterface				PlmClientLinkQueryInterface,
	userIdentifier					SimpleNameType }

PlmProviderLinkEndId ::= CHOICE {
	providerLinkEndQueryInterface			PlmProviderLinkEndQueryInterface,
	userIdentifier					SimpleNameType }

PlmProviderLinkId ::= CHOICE {
	providerLinkQueryInterface			PlmProviderLinkQueryInterface,
	userIdentifier					SimpleNameType }

PlmRequestedLinkConnectionChoice ::= CHOICE {
	plmLinkConnectionIds	SetOfPlmLinkConnections,
	numberOfLinkConnections	INTEGER,
	allAvailableLinkConnections	NULL}

PlmRequestedNetworkCTPChoice  ::= CHOICE {
	plmNetworkCTPIds	SetOfPlmNetworkCTPs,
	numberOfNetworkCTPs	INTEGER,
	allAvailableNetworkCTPs	NULL}

SetOfPlmLinkConnections ::= SET OF CHOICE {
	linkConnectionId	PLMLinkConnectionQueryIfce,
	userIdentifier			SimpleNameType}

SetOfPlmNetworkCTPs ::= SET OF CHOICE {
	networkCTPId	PlmNetworkCTPQueryIfce,
	userIdentifier		SimpleNameType}

| ITU-T RECOMMENDATIONS SERIES |  |
| --- | --- |
| Series A | Organization of the work of the ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |