|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.854.16 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (01/2001) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital networks – Management of transport network |  |  |  |
|  | Computational viewpoint for pre-provisioned route discovery |  |  |  |
|  | ITU-T Recommendation G.854.16 (Formerly CCITT Recommendation) |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| General aspects | G.800–G.809 |
| Design objectives for digital networks | G.810–G.819 |
| Quality and availability targets | G.820–G.829 |
| Network capabilities and functions | G.830–G.839 |
| SDH network characteristics | G.840–G.849 |
| Management of transport network | G.850–G.859 |
| SDH radio and satellite systems integration | G.860–G.869 |
| Optical transport networks | G.870–G.879 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.854.16 Computational viewpoint for pre-provisioned route discovery

- Summary This computational viewpoint specification is related to the pre-provisioned route discovery enterprise and information specification. The objective of the community is to identify appropriate routes for setting up, or reserving the route components for (by using other communities), a trail, a tandem connection or a subnetwork connection. It is possible to request conditions that have to be met by the identified route. In addition to proposing routes, information is provided to help in the selection among the candidate route on the basis of a set of properties. In case of protection, the number of routes corresponds to the protection scheme chosen.

- Source ITU-T Recommendation G.854.16 was prepared by ITU-T Study Group 4 (2001-2004) and approved under the WTSA Resolution 1 procedure on 19 January 2001.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2001

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Definitions		1

4	Abbreviations		1

5	Conventions		2

6	Label references		2

7	Interfaces		3

7.1	Query interfaces		3

7.2	Operational interfaces		5

7.2.1	Route discovery interface		5

7.3	Reporting interfaces		7

7.3.1	Route discovery reporting interface		7

7.4	ASN.1 supporting productions		8

ITU-T Recommendation G.854.16

Computational viewpoint for pre-provisioned route discovery

# 1	Scope

This computational viewpoint specification is related to the pre-provisioned route discovery enterprise specification defined in ITU-T G.852.16 and the pre-provisioned route discovery information specification defined in ITU-T G.853.16.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[1]	ITU-T G.851.1 (1996), Management of the transport network – Application of the RM-ODP framework.

[2]	ITU-T G.853.1 (1999), Common elements of the information viewpoint for the management of a transport network.

[3]	ITU-T G.852.16 (2001), Enterprise viewpoint for pre-provisioned route discovery.

[4]	ITU-T G.853.16 (2001), Information viewpoint for pre-provisioned route discovery.

[5]	ITU-T G.853.10 (1999), Information viewpoint for pre-provisioned link connection management.

[6]	ITU-T G.854.3 (1999), Computational viewpoint for topology management.

# 3	Definitions

No new term is defined in this Recommendation.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

ASN.1		Abstract Syntax Notation One

CTP		Connection Termination Point

Id		Identifier

Ifce		Interface

inv		invariant

ITU-T		International Telecommunication Union – Telecommunication Standardization Sector

layerND	layerNetwork Domain

LC		LinkConnection

plcm		pre-provisioned link connection management

prd		pre-provisioned route discovery

Rec.		Recommendation

RM-ODP	Reference Model for Open Distributed Processing

TTP		Trail Termination Point

# 5	Conventions

In order to increase the readability of the behaviour in the operations:

•	parameters are written in bold;

•	elements defined in the information viewpoint specification are written in italic.

# 6	Label references

| Full label reference | Local label reference |
| --- | --- |
| <"Rec. G.853.16", INFORMATION_OBJECT: prdAccessGroup> | prdAccessGroup |
| <"Rec. G.853.16", INFORMATION_OBJECT: prdLayerNetworkDomain> | prdLayerNetworkDomain |
| <"Rec. G.853.16", INFORMATION_OBJECT: prdLink> | prdLink |
| <"Rec. G.853.16", INFORMATION_OBJECT: prdLinkConnection> | prdLinkConnection |
| <"Rec. G.853.16", INFORMATION_OBJECT: prdLinkEnd> | prdLinkEnd |
| <"Rec. G.853.16", INFORMATION_OBJECT: prdNetworkTTP> | prdNetworkTTP |
| <"Rec. G.853.16", INFORMATION_OBJECT: prdNetworkCTP> | prdNetworkCTP |
| <"Rec. G.853.16", INFORMATION_OBJECT: prdRoutingConditions> | prdRoutingConditions |
| <"Rec. G.853.16", INFORMATION_OBJECT: prdRoute> | prdRoute |
| <"Rec. G.853.16", INFORMATION_OBJECT: prdRouteSet> | prdRouteSet |
| <"Rec. G.853.16", INFORMATION_OBJECT: prdSubnetwork> | prdSubnetwork |
| <"Rec. G.853.16", INFORMATION_OBJECT: prdTopologicalLink> | prdTopologicalLink |
| <"Rec. G.853.16", INFORMATION_OBJECT: prdTopologicalLinkEnd> | prdTopologicalLinkEnd |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: layerNetworkDomainIsMadeOf> | layerNetworkDomainIs MadeOf |
| <"Rec. G.853.16", INFORMATION_RELATIONSHIP: prdRouteFulfillsRoutingConditions> | prdRouteFulfillsRouting Conditions |
| <"Rec. G.853.16", INFORMATION_RELATIONSHIP: prdRouteSetHasRoutes> | prdRouteSetHasRoutes |
| <"Rec. G.853.1", INFORMATION_ATTRIBUTE: directionality> | directionality |
| <"Rec. G.853.1", INFORMATION_ATTRIBUTE: linkDirectionality> | linkDirectionality |
| <"Rec.G.853.10", INFORMATION_ATTRIBUTE: plcmCallerId> | plcmCallerId |
| <"Rec. G.853.1", INFORMATION_ATTRIBUTE: pointDirectionality> | pointDirectionality |
| <"Rec.G.853.16", INFORMATION_ATTRIBUTE: prdArc-Point-orientedRouteComponents> | prdArc-Point-orientedRouteComponents |

| Full label reference | Local label reference |
| --- | --- |
| <"Rec.G.853.16", INFORMATION_ATTRIBUTE: prdReturnedProperties> | prdReturnedProperties |
| <"Rec.G.853.16", INFORMATION_ATTRIBUTE: prdRouteEnds> | prdRouteEnds |
| <"Rec.G.853.16", INFORMATION_ATTRIBUTE: prdIndividualRouteComponentsList> | prdIndividualRouteComponentsList |
| <"Rec. G.853.1", INFORMATION_ATTRIBUTE: resourceId> | resourceId |
| <"Rec.G.853.1", INFORMATION_ATTRIBUTE: signalIdentification> | signalIdentification |
| <"Rec.G.853.1", INFORMATION_ATTRIBUTE: topologicalEndDirection> | topologicalEndDirection |
| <"Rec. G.853.1", INFORMATION_ATTRIBUTE: userLabel> | userLabel |
| <"Rec. G.854.3", INTERFACE: commonReportResourceIdChangeIfce> | commonReportResourceId ChangeIfce |
| <"Rec. G.854.3", INTERFACE: commonResourceIfce> | commonResourceIfce |

| Full ASN.1 production reference | Local label reference |
| --- | --- |
| <"Rec. X.721:1992: Attribute-ASN.1 Module": SimpleNameType> | SimpleNameType |
| <"Rec. X.680:1997: GraphicString"> | GraphicString |

# 7	Interfaces

This Recommendation does not define operations which change the userLabel of the resources or report this change to the notification receiver (these operations are defined in the superclass interfaces defined in ITU-T G.854.3 and used in this Recommendation) because they do not change the state of the system. There exact signature will be developed as part of the engineering viewpoint specification with the concerned technology.

## 7.1	Query interfaces

This Recommendation refers to interfaces that allow access to identification and properties of resources involved in the "pre-provisioned route discovery" community. As the invocation of contained operations does not modify any state, there is no interest in developing them explicitly. Their exact signature will be developed as part of the engineering viewpoint, with the concerned technology. These interfaces are listed in the following table with the information they allow access to.

| Interface name | Information object | Attributes and relationships |
| --- | --- | --- |
| prdAccessGroupQueryIfce | <prdAccessGroup> | <resourceId> <signalIdentification> <topologicalEndDirection> <layerNetworkDomainIsMadeOf, ROLE: containerLND> |
| prdLayerNetworkDomainQueryIfce | <prdLayerNetworkDomain> | <resourceId> <signalIdentification> <layerNetworkDomainIsMadeOf, ROLE: element> |
| prdLinkConnectionQueryIfce | <prdLinkConnection> | <resourceId> <signalIdentification> <directionality> <userLabel> <plcmCallerId> <layerNetworkDomainIsMadeOf, ROLE: containerLND> |
| prdLinkEndQueryIfce | <prdLinkEnd> | <resourceId> <signalIdentification> <topologicalEndDirection> <layerNetworkDomainIsMadeOf, ROLE: containerLND> |
| prdLinkQueryIfce | <prdLink> | <resourceId> <signalIdentification> <linkDirectionality> <layerNetworkDomainIsMadeOf, ROLE: containerLND> |
| prdNetworkCTPQueryIfce | <prdNetworkCTP> | <resourceId> <signalIdentification> <pointDirectionality> <userLabel> <plcmCallerId> <layerNetworkDomainIsMadeOf, ROLE: containerLND> |
| prdNetworkTTPQueryIfce | <prdNetworkTTP> | <resourceId> <signalIdentification> <pointDirectionality> <layerNetworkDomainIsMadeOf, ROLE: containerLND> |
| prdRouteSetQueryIfce | <prdRouteSet> | <resourceId> <prdRouteEnds> <layerNetworkDomainIsMadeOf, ROLE: containerLND> <prdRouteSetFulfillsRouting Conditions, ROLE: fulfilledConditions> <prdRouteSetHasRoutes, ROLE: route> |

| Interface name | Information object | Attributes and relationships |
| --- | --- | --- |
| prdRouteQueryIfce | <prdRoute> | <resourceId> <prdIndividualRouteComponents List> <prdReturnedProperties> <layerNetworkDomainIsMadeOf, ROLE: containerLND> <prdRouteSetHaseRoutes, ROLE: routeSet> |
| prdRoutingConditionsQueryIfce | <prdRoutingConditions> | <resourceId> <prdArc-Point-orientedRouteComponents> <prdRouteSetFulfillsRouting Conditions, ROLE: routeSet> |
| prdSubnetworkQueryIfce | <prdSubnetwork> | <resourceId> <signalIdentification> <layerNetworkDomainIsMadeOf, ROLE: containerLND> |
| prdTopologicalLinkEndQueryIfce | <prdTopologicalLinkEnd> | <resourceId> <signalIdentification> <topologicalEndDirection> <layerNetworkDomainIsMadeOf, ROLE: containerLND> |
| prdTopologicalLinkQueryIfce | <prdTopologicalLink> | <resourceId> <signalIdentification> <linkDirectionality> <layerNetworkDomainIsMadeOf, ROLE: containerLND> |

## 7.2	Operational interfaces

### 7.2.1	Route discovery interface

The route discovery interface provides functionality for retrieving potential routes that can be used when setting up trails, tandem connections or subnetwork connections as defined in <"Rec. G.852.16", COMMUNITY: pre-provisioned route discovery>.

COMPUTATIONAL INTERFACE routeDiscoveryIfce{

DERIVED FROM <commonResourceIfce>

OPERATION {

<discoverRoutes>;

}

}

Discover routes

<COMMUNITY: pre-provisioned route discovery, ACTION: discover routes>

OPERATION discoverRoutes {

INPUT_PARAMETERS

routeEnds: RouteEnds ::= SEQUENCE OF {

aEnd	AEndChoice,

zEnd	ZEndChoice};

-- one aEnd/zEnd pair is sufficient to represent the ends of all the routes

suppliedRoutingConditions: SuppliedRoutingConditions

-- the ASN.1 type for suppliedRoutingConditions will be developed in technology

-- specific extensions of the route discovery community

arcOrPointOrientedRouteComponents: ArcPointOriented;

layerND: LayerNetworkDomainChoice;

routeDir: RouteDirChoice ::= CHOICE {

dir			Directionality,

linkDir		LinkDirectionality,

pointDir		PointDirectionality,

topologicalEndDir	TopologicalEndDirectionality};

suppliedUserIdentifier: UserIdentifier;

OUTPUT_PARAMETERS

routeSet: ReturnedRouteSet ::= SET OF ReturnedRoute;

routeEnds: RouteEnds

-- the ASN.1 type for ReturnedRoute will be developed in technology

-- specific extensions of the pre-provisioned route discovery community

RAISED_EXCEPTIONS

invalidRouteEnds: routeEnds;

routingConditionsNotAssociated: SuppliedRoutingConditions;

newRouteComponentsNotPartOfLND: SET OF RouteComponents;

newRouteComponentsNotFree:   SET OF RouteComponents;

routeComponentsNotConsistent: NULL;

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

accessGroup: < INFORMATION OBJECT: prdAccessGroup>;

link: < INFORMATION OBJECT: prdLink>;

linkConnection: < INFORMATION OBJECT: prdLinkConnection>;

linkEnd : < INFORMATION OBJECT: prdLinkEnd>;

networkCTP: < INFORMATION OBJECT: prdNetworkCTP>;

networkTTP: < INFORMATION OBJECT: prdNetworkTTP>;

subnetwork: <INFORMATION OBJECT: prdSubnetwork>;

topologicalLink: < INFORMATION OBJECT: prdTopologicalLink>;

topologicalLinkEnd: < INFORMATION OBJECT: prdTopologicalLinkEnd>;

suppliedRoutingConditions: < INFORMATION OBJECT: prdRoutingConditions>;

arcOrPointOrientedRouteComponents: <INFORMATION ATTRIBUTE: prdArc-Point-orientedRouteComponents>;

layerND: <INFOMATION OBJECT: prdLayerNetworkDomain>;

suppliedUserIdentifier: <INFORMATION ATTRIBUTE: resourceId>;

routeSet: <INFORMATION OBJECT: prdRouteSet>;

routeEnds: <INFORMATION ATTRIBUTE: prdRouteEnds>;

dir : < INFORMATION ATTRIBUTE: directionality > ;

linkDir: < INFORMATION ATTRIBUTE: linkDirectionality>;

topologicalEndDir: <INFORMATION ATTRIBUTE: topologicalEndDirectionality>;

pointDir : < INFORMATION ATTRIBUTE: pointDirectionality > ;

PRE_CONDITIONS

inv_routeEndsLNDContainment

"The elements representing the aEnd and zEnd of the routeEnds are referring to element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

POST_CONDITIONS

inv_associatedWithSuppliedRoutingConditions

"The suppliedRoutingConditions have to participate in the <prdRouteSetFulfillRoutingConditions>  relationship taking the fulfilledConditions role while routeSet is taking on the routeSet role."

inv_newRouteComponentsLNDContainment

"All route components of  routeSet must refer to element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_freeNewRouteComponents

"<plcmCallerId> of all route components contained in routeSet must be equal to NULL."

inv_consistentRouteComponents

"Only linkConnections are allowed as route components in routeSet when arcOrPointOrientedRouteComponents is equal to arc-oriented. Only networkCTPs are allowed as route components in routeSet when arcOrPointOrientedRouteComponents is equal to point-oriented."

EXCEPTIONS

IF PRE_CONDITION inv_routeEndsLNDContainment NOT_VERIFIED RAISE_EXCEPTION invalidRouteEnds;

IF POST_CONDITION inv_associatedWithSuppliedRoutingConditions NOT_VERIFIED RAISE_EXCEPTION routingConditionsNotAssociated;

IF POST_CONDITION inv_newRouteComponentsLNDContainment NOT_VERIFIED RAISE_EXCEPTION newRouteComponentsNotPartOfLND;

IF POST_CONDITION inv_freeNewRouteComponents NOT_VERIFIED RAISE_EXCEPTION newRouteComponentsNotFree;

IF POST_CONDITION inv_consistentRouteComponents NOT_VERIFIED RAISE_EXCEPTION routeComponentsNotConsistent;

;}

## 7.3	Reporting interfaces

### 7.3.1	Route discovery reporting interface

The route discovery reporting interface provides functionality for the reporting of the retrieving of potential routes that can be used for setting up trails, tandem connections or subnetwork connections as defined in <"Rec. G.852.16", COMMUNITY: pre-provisioned route discovery>.

COMPUTATIONAL INTERFACE routeDiscoveryReportingIfce{

DERIVED FROM <commonResourceIdChangeIfce>

OPERATION {

<reportRouteDiscovery>;

}

}

Route discovery reporting

<COMMUNITY: pre-provisioned route discovery, ACTION: report route discovery>

OPERATION reportRouteDiscovery {

INPUT_PARAMETERS

layerND: LayerNetworkDomainChoice;

routeSet: ReturnedRouteSet ::= SET OF ReturnedRoute;

routeEnds: RouteEnds

-- the ASN.1 type for ReturnedRoute will be developed in technology

-- specific extensions of the pre-provisioned route discovery community

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

-- none

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

layerND: <INFOMATION OBJECT: prdLayerNetworkDomain>;

routeSet: <INFORMATION OBJECT: prdRouteSet>;

routeEnds: <INFORMATION ATTRIBUTE: prdRouteEnds>;

TRIGGERING_CONDITIONS

PRE_CONDITIONS

inv_noRouteSet

" routeSet does not refer to any element in the the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

POST_CONDITIONS

inv_existingRouteSet

" routeSet does refer to element in the the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

EXCEPTIONS

-- none

;}

## 7.4	ASN.1 supporting productions

In this Recommendation, when an interface name is used within an ASN.1 production, the same label will be used, starting with a capital letter. The complete ASN.1 type definition for this query interface (e.g. use of ObjectIdentifier, INTEGER, ...) will be developed as part of the engineering viewpoint, with the concerned technology.

AccessGroupChoice ::= CHOICE {

prdAccessGroupQueryIfce	PrdAccessGroupQueryIfce,

userIdentifier	UserIdentifier };

AEndChoice ::= CHOICE {

networkTTPChoice		NetworkTTPChoice,

networkCTPChoice		NetworkCTPChoice,

linkConnectionChoice		LinkConnectionChoice,

accessGroupChoice		AccessGroupChoice,

linkChoice		LinkChoice,

linkEndChoice		LinkEndChoice,

topologicalLinkChoice		TopologicalLinkChoice,

topologicalLinkEndChoice		TopologicalLinkEndChoice,

subnetworkChoice		SubnetworkChoice};

ArcPointOriented ::= CHOICE {

arc		NULL,

point		NULL};

LayerNetworkDomainChoice ::= CHOICE {

prdLayerNetworkDomainQueryIfce		PrdLayerNetworkDomainQueryIfce,

userIdentifier		UserIdentifier};

LinkChoice ::= CHOICE {

prdLinkQueryIfce		PrdLinkQueryIfce,

userIdentifier		UserIdentifier};

LinkConnectionChoice ::= CHOICE {

prdLinkConnectionQueryIfce	PrdLinkConnectionQueryIfce,

userIdentifier		UserIdentifier};

LinkEndChoice ::= CHOICE {

prdLinkEndQueryIfce		PrdLinkEndQueryIfce,

userIdentifier		UserIdentifier};

NetworkCTPChoice ::= CHOICE {

prdNetworkCTPQueryIfce		PrdNetworkCTPQueryIfce,

userIdentifier		UserIdentifier};

NetworkTTPChoice ::= CHOICE {

prdNetworkTTPQueryIfce		PrdNetworkTTPQueryIfce,

userIdentifier		UserIdentifier};

ProtectionScheme ::= CHOICE {

noProtection		NULL,

1+1_Protection		NULL,

1:n_Protection		NULL,

m:n_Protection		NULL};

ReturnedProperties :: SET OF {

property1	Property1

....

propertyN	PropertyN};

-- the ASN.1 type for ReturnedProperties will be developed in technology

-- specific extensions of the pre-provisioned route discovery community

ReturnedRoute ::= SEQUENCE OF {

route				PrdRouteQueryIfce;

routeProperties		ReturnedProperties,

-- 0 implies none supplied

routeComponents		SEQUENCE OF RouteComponents};

-- the number of the SEQUENCE OF in routeComponents corresponds to the protection scheme requested

-- the final ASN.1 type for ReturnedRoute will be developed in technology specific extensions

RouteComponents ::= CHOICE {

arcView		SEQUENCE OF LinkConnectionChoice,

pointView		SEQUENCE OF NetworkCTPChoice;

SubnetworkChoice ::= CHOICE {

prdSubnetworkQueryIfce		PrdSubnetworkQueryIfce

userIdentifier		UserIdentifier};

SuppliedRoutingConditions ::= SET OF {

topologyCriteria			TopologyComponentCriteria,

maximumNumberOfNodes			INTEGER,

bandwidthConstraints			INTEGER,

protectionScheme			ProtectionScheme,

.....

};

-- banwidth constraints requests the maximum average spare capacity and shall be expressed in %

-- the ASN.1 type for suppliedRoutingConditions will be developed in technology specific extensions

TopologicalLinkChoice ::= CHOICE {

prdTopologicalLinkQueryIfce		PrdTopologicalLinkQueryIfce,

userIdentifier			UserIdentifier};

TopologicalLinkEndChoice ::= CHOICE {

prdTopologicalLinkEndQueryIfce			PrdTopologicalLinkEndQueryIfce,

userIdentifier			UserIdentifier};

TopologyComponentCriteria :: SET OF {

criteria1			Criteria1

....

criteriaN			CriteriaN};

-- the ASN.1 type for TopologyComponentCriteria will be developed in technology

-- specific extensions of the pre-provisioned route discovery community

UserIdentifier ::= SimpleNameType;

ZEndChoice ::= CHOICE {

networkTTPChoice		NetworkTTPChoice,

networkCTPChoice		NetworkCTPChoice,

linkConnectionChoice		LinkConnectionChoice,

accessGroupChoice		AccessGroupChoice,

linkChoice		LinkChoice,

linkEndChoice		LinkEndChoice,

topologicalLinkChoice		TopologicalLinkChoice,

topologicalLinkEndChoice		TopologicalLinkEndChoice,

subnetworkChoice		SubnetworkChoice };

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |