|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

| ITU-T | G.854.3 |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU | (03/99) |

SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

Digital transmission systems – Digital networks – Management of transport network

Computational viewpoint for topology management

ITU-T  Recommendation  G.854.3

(Previously  CCITT  Recommendation)

ITU-T  G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| For further details, please refer to ITU-T List of Recommendations. |  |
| --- | --- |
| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| INTERNATIONAL ANALOGUE CARRIER SYSTEM |  |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS |  |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TRANSMISSION SYSTEMS |  |
| TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| General aspects | G.800–G.809 |
| Design objectives for digital networks | G.810–G.819 |
| Quality and availability targets | G.820–G.829 |
| Network capabilities and functions | G.830–G.839 |
| SDH network characteristics | G.840–G.849 |
| Management of transport network | G.850–G.859 |
| SDH radio and satellite systems integration | G.860–G.869 |
| Optical transport networks | G.870–G.879 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |

ITU-T RECOMMENDATION G.854.3 computational viewpoint for topology management

Summary The topology management community is used to managing the topology of a layer network domain and the relationships between the resources of the layer network domain being managed. The service provided by the community allows for the creation and deletion of the following resources inside a layer network domain: subnetwork, link, topological link, link end, topological link end and access group. The service also provides a set of reporting actions to advise potential notification receivers of creation and deletion of resources in the community. The service is available between one single caller and one single provider. The partitioning of subnetworks and links is not addressed in this community. Enterprise community actions covered by this specification: • create access group; • delete access group; • create link; • delete link; • create link end; • delete link end; • create subnetwork; • delete subnetwork; • create topological link; • delete topological link; • create topological link end; • delete topological link end; • report access group creation; • report access group deletion; • report link creation; • report link deletion; • report link end creation; • report link end deletion; • report subnetwork creation; • report subnetwork deletion; • report topological link creation; • report topological link deletion; • report topological link end creation; • report topological link end deletion.

Source ITU-T Recommendation G.854.3 was prepared by ITU-T Study Group 4 (1997-2000) and was approved under the WTSC Resolution No. 1 procedure on the 26th of March 1999.

FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation the term recognized operating agency (ROA) includes any individual, company, corporation or governmental organization that operates a public correspondence service. The terms Administration, ROA and public correspondence are defined in the Constitution of the ITU (Geneva, 1992).

INTELLECTUAL PROPERTY RIGHTS

The ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. The ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, the ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  1999

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Definitions		1

4	Abbreviations		1

5	Conventions		2

6	Label references		2

7	Interfaces		3

7.1	Query interfaces		3

7.2	Operational interfaces		6

7.2.1	Common resource interface		6

7.2.2	Topology resources provisioning interface		7

7.3	Report interfaces		20

7.3.1	Common report resourceId change interface		20

7.3.2	Topology resources reporting interface		21

7.4	ASN.1 supporting productions		30

Recommendation G.854.3

computational viewpoint for topology management

(Geneva, 1999)

# 1	Scope

This computational viewpoint specification is related to the topology management enterprise specification defined in Recommendation G.852.3 and the topology management information specification defined in Recommendation G.853.3.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[1]	ITU-T Recommendation G.851.1 (1996), Management of the transport network – Application of the RM-ODP framework.

[2]	ITU-T Recommendation G.853.1 (1999), Common elements of the information viewpoint for the management of a transport network.

[3]	ITU-T Recommendation G.852.3 (1999), Enterprise viewpoint fort topology management.

[4]	ITU-T Recommendation G.853.3 (1999), Information viewpoint for topology management.

# 3	Definitions

None.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

AG	Access Group

ASN.1	Abstract Syntax Notation One

CTP	Connection Termination Point

Id	Identifier

Ifce	Interface

inv	invariant

ITU	International Telecommunication Union

layerND	layer Network Domain

LE	Link End

LND	Layer Network Domain

NTTP	Network Trail Termination Point

RM-ODP	Reference Model for Open Distributed Processing

SN	Subnetwork

SNC	Subnetwork Connection

SNTP	Subnetwork Termination Point

TL	Topological Link

TLE	Topological Link End

topman	topology management

TP	Termination Point

TTP	Trail Termination Point

# 5	Conventions

In order to increase the readability of the behaviour in the operations:

•	parameters are written in bold;

•	elements defined in the information specification are written in italic.

# 6	Label references

| Full label reference | Local label reference |
| --- | --- |
| <"Rec. G.853.1", INFORMATION_OBJECT:linkConnection> | linkConnection |
| <"Rec. G.853.1", INFORMATION_OBJECT:networkCTP> | networkCTP |
| <"Rec. G.853.1", INFORMATION_OBJECT:subnetworkConnection> | subnetworkConnection |
| <"Rec. G.853.3", INFORMATION_OBJECT: topmanAccessGroup> | topmanAccessGroup |
| <"Rec. G.853.3", INFORMATION_OBJECT: topmanLayerNetworkDomain> | topmanLayerNetworkDomain |
| <"Rec. G.853.3", INFORMATION_OBJECT: topmanLink> | topmanLink |
| <"Rec. G.853.3", INFORMATION_OBJECT: topmanLinkEnd> | topmanLinkEnd |
| <"Rec. G.853.3", INFORMATION_OBJECT: topmanNetworkTTP> | topmanNetworkTTP |
| <"Rec. G.853.3", INFORMATION_OBJECT: topmanSubnetwork> | topmanSubnetwork |
| <"Rec. G.853.3", INFORMATION_OBJECT: topmanSubnetworkTP> | topmanSubnetworkTP |
| <"Rec. G.853.3", INFORMATION_OBJECT: topmanTopologicalLink> | topmanTopologicalLink |
| <"Rec. G.853.3", INFORMATION_OBJECT: topmanTopologicalLinkEnd> | topmanTopologicalLinkEnd |
| <"Rec. G.853.1", ATTRIBUTE: linkDirectionality> | linkDirectionality |
| <"Rec. G.853.1", ATTRIBUTE: pointDirectionality> | pointDirectionality |
| <"Rec. G.853.1", ATTRIBUTE: resourceId> | resourceId |
| <"Rec. G.853.1", ATTRIBUTE: signalIdentification> | signalIdentification |
| <"Rec. G.853.1", ATTRIBUTE: topologicalEndDirection> | topologicalEndDirection |
| <"Rec. G.853.1", ATTRIBUTE: userLabel> | userLabel |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: accessGroupIsMadeOfNetworkTTPs> | accessGroupIsMadeOfNetworkTTPs |

| Full label reference | Local label reference |
| --- | --- |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: layerNetworkDomainIsMadeOf> | layerNetworkDomainIsMadeOf |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: linkBinds> | linkBinds |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: linkConnectionIsTerminatedByTopologicalEntities> | linkConnectionIsTerminatedBy Topological Entities |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: linkEndHasNetworkCTPs> | linkEndHasNetworkCTPs |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: linkEndIsBoundTo> | linkEndIsBoundTo |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: linkHasLinkConnections> | linkHasLinkConnections |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: subnetworkHasSubnetworkConnections> | subnetworkHasSubnetwork Connections |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: subnetworkIsDelimitedBy> | subnetworkIsDelimitedBy |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: subnetworkTPIsRelatedToExtremity> | subnetworkTPIsRrelatedToExtremity |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: topologicalLinkEndIsSupportedByNetworkTTP> | topologicalLinkEndIsSupportedBy NetworkTTP |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: topologicalLinkIsSupportedByTrail> | topologicalLinkIsSupportedByTrail |

| Full ASN.1 production reference | Local label reference |
| --- | --- |
| <"Rec. X.721:1992: Attribute-ASN1Module" : SimpleNameType> | SimpleNameType |
| <"Rec. X.680:1997": GraphicString> | GraphicString |

# 7	Interfaces

This specification does not define operations which change the userLabel of the resources or report this change to the notification receiver because they do not change the state of the system. Their exact signature will be developed as part of the engineering viewpoint, with the concerned technology.

## 7.1	Query interfaces

This specification refers to interfaces that allow get access to identification and properties of resources involved in the "topology management" community. As the invocation of contained operations do not modify any state, there is no interest to develop them explicitly. Their exact signature will be developed as part of the engineering viewpoint, with the concerned technology. These interfaces are listed in the following table with the information they allow access to.

| Interface name | Information object | Attributes and relationships |
| --- | --- | --- |
| topmanAccessGroup QueryIfce | <topmanAccessGroup> | <resourceId> <signalIdentification> <topologicalEndDirection> <userLabel> <accessGroupIsMadeOfNetworkTTPs, ROLE: elementTTP> <linkBinds, ROLE: transferCapacityLink> <layerNetworkDomainIsMadeOf, ROLE: containerLND> |
| topmanLayerNetwork DomainQueryIfce | <topmanLayerNetworkDomain> | <resourceId> <signalIdentification> <layerNetworkDomainIsMadeOf, ROLE: element> |
| topmanLinkEndQuery Ifce | <topmanLinkEnd> | <resourceId> <signalIdentification> <topologicalEndDirection> <userLabel> <layerNetworkDomainIsMadeOf, ROLE: containerLND> <linkEndIsBoundTo, ROLE: topologicalEntity> <linkEndHasNetworkCTPs, ROLE: elementCTP> |
| topmanLinkQueryIfce | <topmanLink> | <resourceId> <signalIdentification> <linkDirectionality> <userLabel> <linkBinds, ROLE: a_endTopological, ROLE: z_endTopological> <layerNetworkDomainIsMadeOf, ROLE: containerLND> <linkHasLinkConnections, ROLE: elementLC> |
| topmanNetworkTTP QueryIfce | <topmanNetworkTTP> | <resourceId> <signalIdentification> <subnetworkTPIsRelatedToExtremity, ROLE: abstractionSNTP> <layerNetworkDomainIsMadeOf, ROLE: containerLND> <accessGroupIsMadeOfNetworkTTPs, ROLE: containerAG> |

| Interface name | Information object | Attributes and relationships |
| --- | --- | --- |
| topmanSubnetwork QueryIfce | <topmanSubnetwork> | <resourceId> <signalIdentification> <userLabel> <linkBinds, ROLE: transferCapacityLink> <subnetworkIsDelimitedBy, ROLE: elementSNTP> <linkEndIsBoundTo, ROLE: transferCapacityLE> <subnetworkHasSubnetworkConnections, ROLE: elementSNC> <linkConnectionIsTerminatedByTopological Entities, ROLE: transportEntityLC> <layerNetworkDomainIsMadeOf, ROLE: containerLND> |
| topmanSubnetworkTP QueryIfce | <topmanSubnetworkTP> | <resourceId> <signalIdentification> <pointDirectionality> <subnetworkIsDelimitedBy, ROLE: containerSN> <subnetworkTPIsRelatedToExtremity, ROLE: extremity> <layerNetworkDomainIsMadeOf, ROLE: containerLND> |
| topmanTopological LinkEndQueryIfce | <topmanTopologicalLinkEnd> | <resourceId> <signalIdentification> <topologicalEndDirection> <userLabel> <layerNetworkDomainIsMadeOf, ROLE: containerLND> <linkEndIsBoundTo, ROLE: topologicalEntity> <topologicalLinkEndIsSupportedByNetwork TTP, ROLE: serverTTP> |
| topmanTopological LinkQueryIfce | <topmanTopologicalLink> | <resourceId> <signalIdentification> <linkDirectionality> <userLabel> <linkBinds, ROLE: a_endTopological, ROLE: z_endTopological> <layerNetworkDomainIsMadeOf, ROLE: containerLND> <topologicalLinkIsSupportedByTrail, ROLE: serverTrail> |

## 7.2	Operational interfaces

### 7.2.1	Common resource interface

The common resource interface provides functionality for changing the resource identifier of the resources by using the operation "change resource identifier". It satisfies the enterprise requirements stated in <"Rec. G.852.2", COMMUNITY: Transport Network Enterprise Model, General policy>.

COMPUTATIONAL_INTERFACE commonResourceIfce {

OPERATION	<changeResourceIdentifier>; }

#### 7.2.1.1	Change resource identifier

<"Rec. G.852.2", COMMUNITY: Transport Network Enterprise Model, General policy>

OPERATION changeResourceId {

INPUT_PARAMETERS

layerND: LayerNetworkDomainId ::= TopmanLayerNetworkDomainQueryIfce;

involvedResource: ResourceChoice

newResourceIdentifier: UserIdentifier ::= SimpleNameType;

OUTPUT_PARAMETERS

agreedResourceIdentifier: UserIdentifier ::= SimpleNameType;

RAISED_EXCEPTIONS

invalidResource: NULL;

newResourceIdentifierNotUnique: newResourceIdentifier;

failureToChangeId: NULL;

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: topmanLayerNetworkDomain>;

involvedResource: <INFORMATION OBJECT networkInformationTop>;

newResourceIdentifier: <INFORMATION ATTRIBUTE: resourceId>;

agreedResourceIdentifier: <INFORMATION ATTRIBUTE: resourceId>;

PRE_CONDITIONS

inv_existingInvolvedResource

"involvedResource shall refer to element in the same <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_uniqueResourceId

"newResourceIdentifier must not be equal to <resourceId> of any element in the same <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

POST_CONDITIONS

inv_idChanged

"<resourceId> of the resource referred to by involvedResource has to be equal to newResourceIdentifier and agreedResourceIdentifier.";

EXCEPTIONS

IF PRE_CONDITION inv_existingInvolvedResource NOT_VERIFIED RAISE_EXCEPTION invalidResource;

IF PRE_CONDITION inv_uniqueResourceId NOT_VERIFIED RAISE_EXCEPTION newResourceIdentifierNotUnique;

IF POST_CONDITION inv_idChanged NOT_VERIFIED RAISE_EXCEPTION failureToChangeId;

;}

### 7.2.2	Topology resources provisioning interface

The Topology Resources Provisioning Interface provides functionality for the provisioning of the resources defined in <"Rec. G.852.2", COMMUNITY: Transport Network Enterprise Model>.

COMPUTATIONAL_INTERFACE topologyResourcesProvisioningIfce {

OPERATION	<createAccessGroup>;

<deleteAccessGroup>;

<createLink>;

<deleteLink>;

<createLinkEnd>;

<deleteLinkEnd>;

<createSubnetwork>;

<deleteSubnetwork>;

<createTopologicalLink>;

<deleteTopologicalLink>;

<createTopologicalLinkEnd>;

<deleteTopologicalLinkEnd>; }

#### 7.2.2.1	Create access group

<COMMUNITY: Topology Management, ACTION: create access group>

OPERATION createAccessGroup {

INPUT_PARAMETERS

layerND: LayerNetworkDomainId;

suppliedUserIdentifier: UserIdentifier;

-- zero length string or 0 implies none supplied.

suppliedUserLabel: GraphicString;

-- zero length string implies none supplied.

suppliedDirection: topologicalEndDirection;

OUTPUT_PARAMETERS

accessGroup: AccessGroupId;

RAISED_EXCEPTIONS

userIdentifierNotUnique: suppliedUserIdentifier;

failureToSetUserIdentifier: NULL;

failureToSetUserLabel: NULL;

failureToCreateAccessGroup: NULL;

failureToSetDirection: NULL;

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: topmanLayerNetworkDomain>;

suppliedUserIdentifier: <INFORMATION ATTRIBUTE: resourceId>;

suppliedUserLabel: <INFORMATION ATTRIBUTE: userLabel>;

accessGroup: <INFORMATION OBJECT: topmanAccessGroup>;

suppliedDirection: <INFORMATION ATTRIBUTE: topologicalEndDirection>;

PRE_CONDITIONS

inv_uniqueUserIdentifier

"The suppliedUserIdentifier value shall not be equal to the <resourceId> value of any element in the 		<layerNetworkDomainIsMadeOf> relationship where the layerND refers to containerLND."

POST_CONDITIONS

inv_agreedUserIdentifier

"The <resourceId> value of <topmanAccessGroup> referred to by accessGroup is equal to the suppliedUserIdentifier value, if it is supplied."

inv_agreedUserLabel

"The <userLabel> value of <topmanAccessGroup> referred to by accessGroup is equal to the suppliedUserLabel value, if it is supplied."

inv_existingAccessGroup

"accesssGroup refers to element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_agreedDirection

"The <topologicalEndDirection> value of <topmanAccessGroup> referred to by accessGroup is equal to the suppliedDirection value, if it is supplied, or set to 'undefined' if not supplied."

EXCEPTIONS

IF PRE_CONDITION inv_uniqueUserIdentifier NOT_VERIFIED RAISE_EXCEPTION userIdentifierNotUnique;

IF POST_CONDITION inv_agreedUserIdentifier NOT_VERIFIED RAISE_EXCEPTION failureToSetUserIdentifier;

IF POST_CONDITION inv_agreedUserLabel NOT_VERIFIED RAISE_EXCEPTION failureToSetUserLabel;

IF POST_CONDITION inv_existingAccessGroup NOT_VERIFIED RAISE_EXCEPTION failureToCreateAccessGroup;

IF POST_CONDITION inv_agreedDirection NOT_VERIFIED RAISE_EXCEPTION
failureToSetDirection;

;}

#### 7.2.2.2	Delete access group

<COMMUNITY: Topology Management, ACTION: delete access group>

OPERATION deleteAccessGroup {

INPUT_PARAMETERS

layerND: LayerNetworkDomainId;

accessGroup: AccessGroupId;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

incorrectAccessGroup: accessGroup;

networkTTPsExisting: NULL;

failureToDeleteAccessGroup: NULL;

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: topmanLayerNetworkDomain>;

accessGroup: <INFORMATION OBJECT: topmanAccessGroup>;

PRE_CONDITIONS

inv_existingAccessGroup

"accesssGroup refers to element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_noNetworkTTPs

"accessGroup shall not refer to any containerAG in a <accessGroupIsMadeOfNetworkTTPs> relationship."

POST_CONDITIONS

inv_noAccessGroup

"accessGroup does not refer to any element in the <layerNetworkDomainIsMadeOf> relationship."

EXCEPTIONS

IF PRE_CONDITION inv_existingAccessGroup NOT_VERIFIED RAISE_EXCEPTION incorrectAccessGroup;

IF PRE_CONDITION inv_noNetworkTTPs NOT_VERIFIED RAISE_EXCEPTION networkTTPsExisting;

IF POST_CONDITION inv_noAccessGroup NOT_VERIFIED RAISE_EXCEPTION failureToDeleteAccessGroup;

;}

#### 7.2.2.3	Create link

<COMMUNITY: Topology Management, ACTION: create link>

OPERATION createLink {

INPUT_PARAMETERS

layerND: LayerNetworkDomainId;

a_End: LinkEndChoice;

z_End: LinkEndChoice;

suppliedUserIdentifier: UserIdentifier;

-- zero length string or 0 implies none supplied.

suppliedUserLabel: GraphicString;

-- zero length string implies none supplied.

suppliedDirectionality: linkDirectionality;

OUTPUT_PARAMETERS

link: LinkId;

RAISED_EXCEPTIONS

incorrectLinkEnds: LinkEndChoice;

userIdentifierNotUnique: suppliedUserIdentifier;

failureToSetUserIdentifier: NULL;

failureToCreateLink: NULL;

failureToBindLink: NULL;

failureToSetUserLabel: NULL;

failureToSetDirectionality: NULL;

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: topmanLayerNetworkDomain>;

subnetwork: <INFORMATION OBJECT: topmanSubnetwork>;

accessGroup: <INFORMATION OBJECT: topmanAccessGroup>;

suppliedUserIdentifier: <INFORMATION ATTRIBUTE: resourceId>;

suppliedUserLabel: <INFORMATION ATTRIBUTE: userLabel>;

suppliedDirectionality: <INFORMATION ATTRIBUTE: linkDirectionality>;

link: <INFORMATION OBJECT: topmanLink>;

PRE_CONDITIONS

inv_sameLayerND

"a_End and z_End shall both refer to element in the same <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_uniqueUserIdentifier

"The suppliedUserIdentifier value shall not be equal to the <resourceId> value of any element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

POST_CONDITIONS

inv_existingLink

"link, a_End and z_End refer to element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_linkBinds

"a_End and z_End respectively refer to a_endTopological and z_endTopological in the <linkBinds> relationship where link refers to transferCapacityLink."

inv_agreedUserIdentifier

"The <resourceId> value of <topmanLink> referred to by link is equal to the suppliedUserIdentifier value, if it is supplied.

inv_agreedUserLabel

"The <userLabel> value of <topmanLink> referred to by link is equal to the suppliedUserLabel value, if it is supplied.

"inv_agreedDirectionality

"The <linkDirectionality> value of <topmanLink> referred to by link is equal to the suppliedDirectionality value, if it is supplied, or set to 'undefined' if not supplied."

EXCEPTIONS

IF PRE_CONDITION inv_sameLayerND NOT_VERIFIED RAISE_EXCEPTION incorrectLinkEnds;

IF PRE_CONDITION inv_uniqueUserIdentifier NOT_VERIFIED RAISE_EXCEPTION userIdentifierNotUnique;

IF POST_CONDITION inv_existingLink NOT_VERIFIED RAISE_EXCEPTION failureToCreateLink;

IF POST_CONDITION inv_linkBinds NOT_VERIFIED RAISE_EXCEPTION failureToBindLink;

IF POST_CONDITION inv_agreedUserIdentifier NOT_VERIFIED RAISE_EXCEPTION failureToSetUserIdentifier;

IF POST_CONDITION inv_agreedUserLabel NOT_VERIFIED RAISE_EXCEPTION failureToSetUserLabel;

IF POST_CONDITION inv_agreedDirectionality NOT_VERIFIED RAISE_EXCEPTION failureToSetDirectionality;

;}

#### 7.2.2.4	Delete link

<COMMUNITY: Topology Management, ACTION: delete link>

OPERATION deleteLink {

INPUT_PARAMETERS

layerND: LayerNetworkDomainId;

link: LinkId;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

incorrectLink: link;

linkConnectionExisting: NULL;

failureToDeleteLink: NULL;

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: topmanLayerNetworkDomain>;

link: <INFORMATION OBJECT: topmanLink>;

PRE_CONDITIONS;

inv_existingLink

"link refers to the element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_noLinkConnection

"link shall not refer to any containerLink of a <linkHasLinkConnections> relationship."

POST_CONDITIONS

inv_noLink

"link does not participate in any <layerNetworkDomainIsMadeOf> AND <linkBinds> relationships."

EXCEPTIONS

IF PRE_CONDITION inv_existingLink NOT_VERIFIED RAISE_EXCEPTION incorrectLink;

IF PRE_CONDITION inv_noLinkConnection NOT_VERIFIED RAISE_EXCEPTION linkConnectionExisting;

IF POST_CONDITION inv_noLink NOT_VERIFIED RAISE_EXCEPTION failureToDeleteLink;

;}

#### 7.2.2.5	Create link end

This operation creates a link end bound to a subnetwork.

<COMMUNITY: Topology Management, ACTION: create link end>

OPERATION createLinkEnd {

INPUT_PARAMETERS

layerND: LayerNetworkDomainId;

subnetwork: SubnetworkId;

suppliedUserIdentifier: UserIdentifier;

-- zero length string or 0 implies none supplied.

suppliedUserLabel: GraphicString;

-- zero length string implies none supplied.

suppliedDirection: topologicalEndDirection;

OUTPUT_PARAMETERS

linkEnd: LinkEndId;

RAISED_EXCEPTIONS

incorrectSubnetwork: subnetwork;

userIdentifierNotUnique: suppliedUserIdentifier;

failureToCreateLinkEnd: NULL;

failureToBindLinkEnd: NULL;

failureToSetUserIdentifier: NULL;

failureToSetUserLabel: NULL;

failureToSetDirection: NULL;

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: topmanLayerNetworkDomain>;

subnetwork: <INFORMATION OBJECT: topmanSubnetwork>;

suppliedUserIdentifier: <INFORMATION ATTRIBUTE: resourceId>;

suppliedUserLabel: <INFORMATION ATTRIBUTE: userLabel>;

suppliedDirection: <INFORMATION ATTRIBUTE: topologicalEndDirection>;

linkEnd: <INFORMATION OBJECT: topmanLinkEnd>;

PRE_CONDITIONS

inv_subnetworkExisting

"subnetwork shall refer to element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_uniqueUserIdentifier

"The suppliedUserIdentifier value shall not be equal to the <resourceId> value of any element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND.";

POST_CONDITIONS

inv_existingLinkEnd

"linkEnd and subnetwork refer to element in the same <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_boundLinkEnd

"linkEnd refers to transferCapacityLE in the <linkEndIsBoundTo> relationship where subnetwork refers to topologicalEntity."

inv_agreedUserIdentifier

"The <resourceId> value of <topmanLinkEnd> referred to by linkEnd is equal to the suppliedUserIdentifier value, if it is supplied."

inv_agreedUserLabel

"The <userLabel> value of <topmanLinkEnd> referred to by linkEnd is equal to the suppliedUserLabel value, if it is supplied."

inv_agreedDirection

"The <topologicalEndDirection> value of <topmanLinkEnd> referred to by linkEnd is equal to the suppliedDirection value, if it is supplied, or set to 'undefined' if not supplied."

EXCEPTIONS

IF PRE_CONDITION inv_subnetworkExisting NOT_VERIFIED RAISE_EXCEPTION incorrectSubnetwork;

IF PRE_CONDITION inv_uniqueUserIdentifier NOT_VERIFIED RAISE_EXCEPTION userIdentifierNotUnique;

IF POST_CONDITION inv_existingLinkEnd NOT_VERIFIED RAISE_EXCEPTION failureToCreateLinkEnd;

IF POST_CONDITION inv_boundLinkEnd NOT_VERIFIED RAISE_EXCEPTION failureToBindLinkEnd;

IF POST_CONDITION inv_agreedUserIdentifier NOT_VERIFIED RAISE_EXCEPTION failureToSetUserIdentifier;

IF POST_CONDITION inv_agreedUserLabel NOT_VERIFIED RAISE_EXCEPTION failureToSetUserLabel;

IF POST_CONDITION inv_agreedDirection NOT_VERIFIED RAISE_EXCEPTION failureToSetDirection;

;}

#### 7.2.2.6	Delete link end

<COMMUNITY: Topology Management, ACTION: delete link end>

OPERATION deleteLinkEnd {

INPUT_PARAMETERS

layerND: LayerNetworkDomainId;

linkEnd: LinkEndId;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

incorrectLinkEnd: linkEnd;

networkCTPExisting: NULL;

failureToDeleteLinkEnd: NULL;

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: topmanLayerNetworkDomain>;

linkEnd: <INFORMATION OBJECT: topmanLinkEnd>;

PRE_CONDITIONS

inv_existingLinkEnd

"linkEnd refers to element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_noNetworkCTPs

"linkEnd shall not refer to any containerLE in a <linkEndHasNetworkCTPs> relationship."

POST_CONDITIONS

inv_noLinkEnd

"linkEnd does not refer to any element in the <layerNetworkDomainIsMadeOf> relationship."

EXCEPTIONS

IF PRE_CONDITION inv_existingLinkEnd NOT_VERIFIED RAISE_EXCEPTION incorrectLinkEnd;

IF PRE_CONDITION inv_noNetworkCTPs NOT_VERIFIED RAISE_EXCEPTION networkCTPExisting;

IF POST_CONDITION inv_noLinkEnd NOT_VERIFIED RAISE_EXCEPTION failureToDeleteLinkEnd;

;}

#### 7.2.2.7	Create subnetwork

<COMMUNITY: Topology Management, ACTION: create subnetwork>

OPERATION createSubnetwork {

INPUT_PARAMETERS

layerND: LayerNetworkDomainId;

suppliedUserIdentifier: UserIdentifier;

-- zero length string or 0 implies none supplied.

suppliedUserLabel: GraphicString;

-- zero length string implies none supplied.

OUTPUT_PARAMETERS

subnetwork: SubnetworkId;

networkTTPs: SetOfNetworkTTPs;

-- the networkTTPs parameter value is provided when <PERMISSION: successReturnTTPs> is
-- supported.

RAISED_EXCEPTIONS

userIdentifierNotUnique: suppliedUserIdentifier;

failureToSetUserIdentifier: NULL;

failureToSetUserLabel: NULL;

failureToCreateSubnetwork: NULL;

failureToCreateNTTP: NULL;

failureToAssociateNTTP: NULL;

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: topmanLayerNetworkDomain>;

suppliedUserIdentifier: <INFORMATION ATTRIBUTE: resourceId>;

suppliedUserLabel: <INFORMATION ATTRIBUTE: userLabel>;

subnetwork: <INFORMATION OBJECT: topmanSubnetwork>;

networkTTPs ELEMENTS: <INFORMATION OBJECT: topmanNetworkTTP>;

PRE_CONDITIONS

inv_uniqueUserIdentifier

"The suppliedUserIdentifier value shall not be equal to the <resourceId> value of any element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

POST_CONDITIONS

inv_agreedUserIdentifier

"The <resourceId> value of the <topmanSubnetwork> referred to by subnetwork is equal to the suppliedUserIdentifier value, if it is supplied."

inv_agreedUserLabel

"The <userLabel> value of the <topmanSubnetwork> referred to by subnetwork is equal to the suppliedUserIdentifier value, if it is supplied."

inv_existingSubnetwork

"subnetwork refers to element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

-- NOTE – The following invariants are only needed if the networkTTPs appear automatically when the subnetwork is created.

inv_existingNetworkTTP

"networkTTPs refers to element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_nttpAssociated

"networkTTPs refers to extremity of the <subnetworkTPIsRelatedToExtremity> relationship where the abstractionSNTP (which is a subnetworkTP) is also elementSNTP of a <subnetworkIsDelimitedBy> relationship where subnetwork refers to containerSN.

";

EXCEPTIONS

IF PRE_CONDITION inv_uniqueUserIdentifier NOT_VERIFIED RAISE_EXCEPTION userIdentifierNotUnique;

IF POST_CONDITION inv_agreedUserIdentifier NOT_VERIFIED RAISE_EXCEPTION failureToSetUserIdentifier;

IF POST_CONDITION inv_agreedUserLabel NOT_VERIFIED RAISE_EXCEPTION failureToSetUserLabel;

IF POST_CONDITION inv_existingSubnetwork NOT_VERIFIED RAISE_EXCEPTION failureToCreateSubnetwork;

IF POST_CONDITION inv_existingNetworkTTP NOT_VERIFIED RAISE_EXCEPTION failureToCreateNTTP;

IF POST_CONDITION inv_nttpAssociated NOT_VERIFIED RAISE_EXCEPTION failureToAssociateNTTP;

;}

#### 7.2.2.8	Delete subnetwork

<COMMUNITY: Topology Management, ACTION: delete subnetwork>

OPERATION deleteSubnetwork {

INPUT_PARAMETERS

layerND: LayerNetworkDomainId;

subnetwork: SubnetworkId;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

incorrectSubnetwork: subnetwork;

subnetworkInUse: NULL;

subnetworkContainsSubnetworkConnections: NULL;

boundSubnetwork: NULL;

failureToDeleteSubnetwork: NULL;

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: topmanLayerNetworkDomain>;

subnetwork: <INFORMATION OBJECT: topmanSubnetwork>;

PRE_CONDITIONS

inv_existingSubnetwork

"subnetwork refers to element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_noSubnetworkTP

"subnetwork shall not refer to any containerSN in a <subnetworkIsDelimitedBy> relationship."

inv_noSubnetworkConnections

"subnetwork shall not refer to any containerSN in a <subnetworkHasSubnetworkConnections> relationship."

inv_noBinding

"subnetwork shall not refer to any a_endTopological or z_endTopological in a <linkBinds> or <linkConnectionIsTerminatedByTopologicalEntities> relationship or to any topologicalEntity in a <linkEndIsBoundTo> relationship."

POST_CONDITIONS

inv_noSubnetwork

"subnetwork must not refer to any element in a <layerNetworkDomainIsMadeOf> relationship."

EXCEPTIONS

IF PRE_CONDITION inv_existingSubnetwork NOT_VERIFIED RAISE_EXCEPTION incorrectSubnetwork;

IF PRE_CONDITION inv_noSubnetworkTP NOT_VERIFIED RAISE_EXCEPTION subnetworkInUse;

IF PRE_CONDITION inv_noSubnetworkConnection NOT_VERIFIED RAISE_EXCEPTION subnetworkContainsSubnetworkConnections;

IF PRE_CONDITION inv_noBinding NOT_VERIFIED RAISE_EXCEPTION boundSubnetwork;

IF POST_CONDITION inv_noSubnetwork NOT_VERIFIED RAISE_EXCEPTION failureToDeleteSubnetwork;

;}

#### 7.2.2.9	Create topological link

<COMMUNITY: Topology Management, ACTION: create topological link>

OPERATION createTopologicalLink {

INPUT_PARAMETERS

layerND: LayerNetworkDomainId;

a_End: LinkEndChoice;

z_End: LinkEndChoice;

suppliedUserIdentifier: UserIdentifier;

-- zero length string or 0 implies none supplied.

suppliedUserLabel: GraphicString;

-- zero length string implies none supplied.

suppliedDirectionality: linkDirectionality;

OUTPUT_PARAMETERS

topologicalLink: TopologicalLinkId;

RAISED_EXCEPTIONS

incorrectLinkEnds: LinkEndChoice;

userIdentifierNotUnique: suppliedUserIdentifier;

failureToSetUserIdentifier: NULL;

failureToCreateTopologicalLink: NULL;

failureToBindTopologicalLink: NULL;

failureToSetUserLabel: NULL;

failureToSetDirectionality: NULL;

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: topmanLayerNetworkDomain>;

subnetwork: <INFORMATION OBJECT: topmanSubnetwork>;

accessGroup: <INFORMATION OBJECT: topmanAccessGroup>;

suppliedUserIdentifier: <INFORMATION ATTRIBUTE: resourceId>;

suppliedUserLabel: <INFORMATION ATTRIBUTE: userLabel>;

suppliedDirectionality: <INFORMATION ATTRIBUTE: linkDirectionality>;

topologicalLink: <INFORMATION OBJECT: topmanLink>;

PRE_CONDITIONS

inv_sameLayerND

"a-End and z-End shall both refer to element in the same <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_uniqueUserIdentifier

"The suppliedUserIdentifier value shall not be equal to the <resourceId> value of any element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

POST_CONDITIONS

inv_existingTopologicalLink

"link, a_End and z_End refer to element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_linkBinds

"a_End and z_End respectively refer to a_endTopological and z_endTopological in the <linkBinds> relationship where topologicalLink refers to transferCapacityLink."

inv_agreedUserIdentifier

"The <resourceId> value of <topmanTopologicalLink> referred to by topologicalLink is equal to the suppliedUserIdentifier value, if it is supplied.

inv_agreedUserLabel

"The <userLabel> value of <topmanTopologicalLink> referred to by topologicalLink is equal to the suppliedUserLabel value, if it is supplied.

"inv_agreedDirectionality

"The <linkDirectionality> value of <topmanTopologicalLink> referred to by topologicalLink is equal to the suppliedDirectionality value, if it is supplied, or set to 'undefined' if not supplied."

EXCEPTIONS

IF PRE_CONDITION inv_sameLayerND NOT_VERIFIED RAISE_EXCEPTION incorrectLinkEnds;

IF PRE_CONDITION inv_uniqueUserIdentifier NOT_VERIFIED RAISE_EXCEPTION userIdentifierNotUnique;

IF POST_CONDITION inv_existingTopologicalLink NOT_VERIFIED RAISE_EXCEPTION failureToCreateTopologicalLink;

IF POST_CONDITION inv_linkBinds NOT_VERIFIED RAISE_EXCEPTION failureToBindTopologicalLink;

IF POST_CONDITION inv_agreedUserIdentifier NOT_VERIFIED RAISE_EXCEPTION failureToSetUserIdentifier;

IF POST_CONDITION inv_agreedUserLabel NOT_VERIFIED RAISE_EXCEPTION failureToSetUserLabel;

IF POST_CONDITION inv_agreedDirectionality NOT_VERIFIED RAISE_EXCEPTION failureToSetDirectionality;

;}

#### 7.2.2.10	Delete topological link

<COMMUNITY: Topology Management, ACTION: delete topological link>

OPERATION deleteTopologicalLink {

INPUT_PARAMETERS

layerND: LayerNetworkDomainId;

topologicalLink: TopologicalLinkId;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

incorrectTopologicalLink: topologicalLink;

serverTrailExisting: NULL;

failureToDeleteTopologicalLink: NULL;

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: topmanLayerNetworkDomain>;

topologicalLink: <INFORMATION OBJECT: topmanTopologicalLink>;

PRE_CONDITIONS;

inv_existingTopologicalLink

"topologicalLink refers to the element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_noServerTrail

"topologicalLink shall not refer to any clientTL of a <topologicalLinkIsSupportedByTrail> relationship."

POST_CONDITIONS

inv_noTopologicalLink

"topologicalLink does not participate in any <layerNetworkDomainIsMadeOf> AND <linkBinds> relationships."

EXCEPTIONS

IF PRE_CONDITION inv_existingTopologicalLink NOT_VERIFIED RAISE_EXCEPTION incorrectTopologicalLink;

IF PRE_CONDITION inv_noServerTrail NOT_VERIFIED RAISE_EXCEPTION serverTrailExisting;

IF POST_CONDITION inv_noTopologicalLink NOT_VERIFIED RAISE_EXCEPTION failureToDeleteTopologicalLink;

;}

#### 7.2.2.11	Create topological link end

This operation creates a topological link end bound to a subnetwork.

<COMMUNITY: Topology Management, ACTION: create topological link end>

OPERATION createTopologicalLinkEnd {

INPUT_PARAMETERS

layerND: LayerNetworkDomainId;

subnetwork: SubnetworkId;

suppliedUserIdentifier: UserIdentifier;

-- zero length string or 0 implies none supplied.

suppliedUserLabel: GraphicString;

-- zero length string implies none supplied.

suppliedDirection: topologicalEndDirection;

OUTPUT_PARAMETERS

topologicalLinkEnd: TopologicalLinkEndId;

RAISED_EXCEPTIONS

incorrectSubnetwork: subnetwork;

userIdentifierNotUnique: suppliedUserIdentifier;

failureToCreateTopologicalLinkEnd: NULL;

failureToBindTopologicalLinkEnd: NULL;

failureToSetUserIdentifier: NULL;

failureToSetUserLabel: NULL;

failureToSetDirection: NULL;

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: topmanLayerNetworkDomain>;

subnetwork: <INFORMATION OBJECT: topmanSubnetwork>;

suppliedUserIdentifier: <INFORMATION ATTRIBUTE: resourceId>;

suppliedUserLabel: <INFORMATION ATTRIBUTE: userLabel>;

suppliedDirection: <INFORMATION ATTRIBUTE: topologicalEndDirection>;

topologicalLinkEnd: <INFORMATION OBJECT: topmanTopologicalLinkEnd>;

PRE_CONDITIONS

inv_subnetworkExisting

"subnetwork shall refer to element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_uniqueUserIdentifier

"The suppliedUserIdentifier value shall not be equal to the <resourceId> value of any element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND.";

POST_CONDITIONS

inv_existingTopologicalLinkEnd

"topologicalLinkEnd and subnetwork refer to element in the same <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_boundTopologicalLinkEnd

"topologicalLinkEnd refers to transferCapacityLE in the <linkEndIsBoundTo> relationship where subnetwork refers to topologicalEntity."

inv_agreedUserIdentifier

"The <resourceId> value of <topmanTopologicalLinkEnd> referred to by topologicalLinkEnd is equal to the suppliedUserIdentifier value, if it is supplied."

inv_agreedUserLabel

"The <userLabel> value of <topmanTopologicalLinkEnd> referred to by topologicalLinkEnd is equal to the suppliedUserLabel value, if it is supplied."

inv_agreedDirection

"The <topologicalEndDirection> value of <topmanTopologicalLinkEnd> referred to by topologicalLinkEnd is equal to the suppliedDirection value, if it is supplied, or set to 'undefined' if not supplied."

EXCEPTIONS

IF PRE_CONDITION inv_subnetworkExisting NOT_VERIFIED RAISE_EXCEPTION 
incorrectSubnetwork;

IF PRE_CONDITION inv_uniqueUserIdentifier NOT_VERIFIED RAISE_EXCEPTION userIdentifierNotUnique;

IF POST_CONDITION inv_existingTopologicalLinkEnd NOT_VERIFIED RAISE_EXCEPTION failureToCreateTopologicalLinkEnd;

IF POST_CONDITION inv_boundTopologicalLinkEnd NOT_VERIFIED RAISE_EXCEPTION failureToBindTopologicalLinkEnd;

IF POST_CONDITION inv_agreedUserIdentifier NOT_VERIFIED RAISE_EXCEPTION failureToSetUserIdentifier;

IF POST_CONDITION inv_agreedUserLabel NOT_VERIFIED RAISE_EXCEPTION failureToSetUserLabel;

IF POST_CONDITION inv_agreedDirection NOT_VERIFIED RAISE_EXCEPTION 
failureToSetDirection;

;}

#### 7.2.2.12	Delete topological link end

<COMMUNITY: Topology Management, ACTION: delete topological link end>

OPERATION deleteTopologicalLinkEnd {

INPUT_PARAMETERS

layerND: LayerNetworkDomainId;

topologicalLinkEnd: TopologicalLinkEndId;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

incorrectTopologicalLinkEnd: topologicalLinkEnd;

serverTTPExisting: NULL;

failureToDeleteTopologicalLinkEnd: NULL;

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: topmanLayerNetworkDomain>;

topologicalLinkEnd: <INFORMATION OBJECT: topmanTopologicalLinkEnd>;

PRE_CONDITIONS

inv_existingTopologicalLinkEnd

"topologicalLinkEnd refers to element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_noServerTTP

"topologicalLinkEnd shall not refer to any clientTLE in a <topologicalLinkEndIsSupportedByNetworkTTP> relationship."

POST_CONDITIONS

inv_noTopologicalLinkEnd

"topologicalLinkEnd does not refer to any element in the <layerNetworkDomainIsMadeOf> relationship."

EXCEPTIONS

IF PRE_CONDITION inv_existingTopologicalLinkEnd NOT_VERIFIED RAISE_EXCEPTION incorrectTopologicalLinkEnd;

IF PRE_CONDITION inv_noServerTTP NOT_VERIFIED RAISE_EXCEPTION serverTTPExisting;

IF POST_CONDITION inv_noTopologicalLinkEnd NOT_VERIFIED RAISE_EXCEPTION failureToDeleteTopologicalLinkEnd;

;}

## 7.3	Report interfaces

### 7.3.1	Common report resourceId change interface

The common report resourceId change interface provides functionality for reporting the change of the resource identifier of the resources by using the operation "report resourceId change". It satisfies the enterprise requirements stated in <"Rec. G.852.2", COMMUNITY: Transport Network Enterprise Model, General policy>.

COMPUTATIONAL_INTERFACE commonReportResourceIdChangeIfce {

OPERATION	<reportResourceIdChange>; }

#### 7.3.1.1	Report resourceId change

<"Rec. G.852.2", COMMUNITY: Transport Network Enterprise Model, General policy>

OPERATION reportResourceIdChange {

INPUT_PARAMETERS

layerND: LayerNetworkDomainId;

involvedResource: ResourceChoice

oldResourceIdentifier: UserIdentifier ::= SimpleNameType;

newResourceIdentifier: UserIdentifier ::= SimpleNameType;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

-- none

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: topmanLayerNetworkDomain>;

involvedResource: <INFORMATION OBJECT networkInformationTop>;

oldResourceIdentifier: <INFORMATION ATTRIBUTE: resourceId>;

newResourceIdentifier: <INFORMATION ATTRIBUTE: resourceId>;

TRIGGERING_CONDITIONS

PRE_CONDITIONS

inv_existingInvolvedResource

"involvedResource shall refer to element in the same <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_oldResourceIdentifier

"<resourceId> of the resource referred to by involvedResource must be equal to oldResourceIdentifier.";

inv_differentNewResourceIdentifier

"<resourceId> of the resource referred to by involvedResource must be different from newResourceIdentifier.";

POST_CONDITIONS

inv_newResourceIdentifier

"<resourceId> of the resource referred to by involvedResource must be equal to newResourceIdentifier.";

EXCEPTIONS

-- none

;}

### 7.3.2	Topology resources reporting interface

The Topology Resources Reporting Interface provides functionality for the reporting of the creation and deletion of the resources defined in <"Rec. G.852.2", COMMUNITY: Transport Network Enterprise Model>.

COMPUTATIONAL_INTERFACE topologyResourcesReportingIfce {

OPERATION	<reportAccessGroupCreation>;

<reportAccessGroupDeletion>;

<reportLinkCreation>;

<reportLinkDeletion>;

<reportLinkEndCreation>;

<reportLinkEndDeletion>;

<reportSubnetworkTPCreation>;

<reportSubnetworkTPDeletion>;

<reportSubnetworkCreation>;

<reportSubnetworkDeletion>; }

<reportTopologicalLinkCreation>;

<reportTopologicalLinkDeletion>;

<reportTopologicalLinkEndCreation>;

<reportTopologicalLinkEndDeletion>; }

#### 7.3.2.1	Report access group creation

<COMMUNITY: Topology Management, ACTION: report access group creation>

OPERATION reportAccessGroupCreation {

INPUT_PARAMETERS

layerND: LayerNetworkDomainId;

accessGroup: AccessGroupId;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

-- none

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: topmanLayerNetworkDomain>;

accessGroup: <INFORMATION OBJECT: topmanAccessGroup>;

TRIGGERING_CONDITIONS

PRE_CONDITIONS

-- none

POST_CONDITIONS

inv_existingAccessGroup

"accessGroup refers to element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

EXCEPTIONS

-- none

;}

#### 7.3.2.2	Report access group deletion

<COMMUNITY: Topology Management, ACTION: report access group deletion>

OPERATION reportAccessGroupDeletion {

INPUT_PARAMETERS

layerND: LayerNetworkDomainId;

accessGroup: AccessGroupId;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

-- none

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: topmanLayerNetworkDomain>;

accessGroup: <INFORMATION OBJECT: topmanAccessGroup>;

TRIGGERING_CONDITIONS

PRE_CONDITIONS

inv_existingAccessGroup

"accessGroup refers to element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_noNetworkTTPs

"accessGroup shall not refer to any containerAG in an <accessGroupIsMadeOfNetworkTTPs> relationship."

POST_CONDITIONS

inv_noAccessGroup

"accessGroup shall not refer to any element in the <layerNetworkDomainIsMadeOf> relationship."

EXCEPTIONS

-- none

;}

#### 7.3.2.3	Report link creation

<COMMUNITY: Topology Management, ACTION: report link creation>

OPERATION reportLinkCreation {

INPUT_PARAMETERS

layerND: LayerNetworkDomainId;

link: LinkId;

a_End: LinkEndChoice;

z_End: LinkEndChoice;

OUTPUT PARAMETERS

-- none

RAISED EXCEPTIONS

-- none

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: topmanLayerNetworkDomain>; link: <INFORMATION OBJECT: topmanLink>;

subnetwork: <INFORMATION OBJECT: topmanSubnetwork>;

accessGroup: <INFORMATION OBJECT: topmanAccessGroup>;

TRIGGERING_CONDITIONS

PRE_CONDITIONS

-- none

POST_CONDITIONS

inv_existingLink

"link refers to element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_linkBinds

"a_End and z_End respectively refer to a_endTopological and z_endTopological in the <linkBinds> relationship where link refers to transferCapacityLink."

EXCEPTIONS

-- none

;}

#### 7.3.2.4	Report link deletion

<COMMUNITY: Topology Management, ACTION: report link deletion>

OPERATION reportLinkDeletion {

INPUT_PARAMETERS

layerND: LayerNetworkDomainId;

link: LinkId;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

-- none

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: topmanLayerNetworkDomain>;

link: <INFORMATION OBJECT: topmanLink>;

TRIGGERING_CONDITIONS

PRE_CONDITIONS

inv_existingLink

"link refers to element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_noLinkConnection

"link shall not refer to any containerLink of a <linkHasLinkConnections> relationship."

POST_CONDITIONS

inv_noLink

"link does not refer to any element in the <layerNetworkDomainIsMadeOf> relationship."

EXCEPTIONS

-- none

;}

#### 7.3.2.5	Report link end creation

<COMMUNITY: Topology Management, ACTION: report link end creation>

OPERATION reportLinkEndCreation {

INPUT_PARAMETERS

layerND: LayerNetworkDomainId;

linkEnd: LinkEndId;

subnetwork: SubnetworkId;

OUTPUT PARAMETERS

-- none

RAISED EXCEPTIONS

-- none

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: topmanLayerNetworkDomain>;

linkEnd: <INFORMATION OBJECT: topmanLinkEnd>;

subnetwork: <INFORMATION OBJECT: topmanSubnetwork>;

TRIGGERING_CONDITIONS

PRE_CONDITIONS

-- none

POST_CONDITIONS

inv_existingLinkEnd

"linkEnd and subnetwork refer to element in the same <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_boundLinkEnd

"linkEnd refers to transferCapacityLE in the <linkEndIsBoundTo> relationship where subnetwork refers to topologicalEntity."

EXCEPTIONS

-- none

;}

#### 7.3.2.6	Report link end deletion

<COMMUNITY: Topology Management, ACTION: report link end deletion>

OPERATION reportLinkEndDeletion {

INPUT_PARAMETERS

layerND: LayerNetworkDomainId;

linkEnd: LinkEndId;

OUTPUT PARAMETERS

-- none

RAISED EXCEPTIONS

-- none

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: topmanLayerNetworkDomain>;

linkEnd: <INFORMATION OBJECT: topmanLinkEnd>;

TRIGGERING_CONDITIONS

PRE_CONDITIONS

inv_existingLinkEnd

"linkEnd refers to element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_existingLinkEndBinding

"linkEnd refers to topologicalEntity in the <linkEndIsBoundTo> relationship."

inv_noNetworkCTPs

"linkEnd shall not refer to any containerLE in a <linkEndHasNetworkCTPs> relationship."

POST_CONDITIONS

inv_noLinkEnd

"linkEnd does not refer to any element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

EXCEPTIONS

-- none

;}

#### 7.3.2.7	Report subnetwork creation

<COMMUNITY: Topology Management, ACTION: report subnetwork creation>

OPERATION reportSubnetworkCreation {

INPUT_PARAMETERS

layerND: LayerNetworkDomainId;

subnetwork: SubnetworkId;

networkTTPs: SetOfNnetworkTTPs;

-- the networkTTPs parameter value is provided when <PERMISSION: informTTPs> is supported.

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

-- none

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: topmanLayerNetworkDomain>;

subnetwork: <INFORMATION OBJECT: topmanSubnetwork>;

networkTTPs ELEMENTS: <INFORMATION OBJECT: topmanNetworkTTP>;

TRIGGERING_CONDITIONS

PRE_CONDITIONS

-- none

POST_CONDITIONS

inv_existingSubnetwork

"subnetwork refer to the element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

;

EXCEPTIONS

-- none

;}

#### 7.3.2.8	Report subnetwork deletion

<COMMUNITY: Topology Management, ACTION: report subnetwork deletion>

OPERATION reportSubnetworkDeletion {

INPUT_PARAMETERS

layerND: LayerNetworkDomainId;

subnetwork: SubnetworkId;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

-- none

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: topmanLayerNetworkDomain>;

subnetwork: <INFORMATION OBJECT: topmanSubnetwork>;

TRIGGERING_CONDITIONS

PRE_CONDITIONS

inv_existingSubnetwork

"subnetwork must refer to element in a <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_noSubnetworkTP

"subnetwork shall not refer to any containerSN in a <subnetworkIsDelimitedBy> relationship."

inv_noBinding

"subnetwork shall not refer to any a_endTopological or z_endTopological in a <linkBinds> relationship or to any topologicalEntity in a <linkEndIsBoundTo> relationship."

POST_CONDITIONS

inv_noSubnetwork

"subnetwork does not participate in the <layerNetworkDomainIsMadeOf> relationship."

EXCEPTIONS

-- none

;}

#### 7.3.2.9	Report subnetworkTP creation

<COMMUNITY: Topology Management, COMMUNITY POLICY: architecturalConstraints>

OPERATION reportSubnetworkTPCreation {

INPUT PARAMETERS

layerND: LayerNetworkDomainId;

subnetworkTP: SubnetworkTPId ::= SimpleNameType;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

-- none

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: topmanLayerNetworkDomain>;

subnetworkTP: <INFORMATION OBJECT: topmanSubnetworkTP>;

TRIGGER CONDITION

PRE_CONDITIONS

-- none

POST_CONDITIONS

inv_existingSubnetworkTP

"subnetworkTP refer to the element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

;

EXCEPTIONS

-- none

;}

#### 7.3.2.10	Report subnetworkTP deletion

<COMMUNITY: Topology Management, COMMUNITY POLICY: architecturalConstraints>

OPERATION reportSubnetworkTPDeletion {

INPUT_PARAMETERS

layerND: LayerNetworkDomainId;

subnetworkTP: SubnetworkTPId ::= SimpleNameType;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

-- none

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: topmanLayerNetworkDomain>;

subnetworkTP: <INFORMATION OBJECT: topmanSubnetworkTP>;

TRIGGER CONDITION

PRE_CONDITIONS

inv_existingSubnetworkTP

"subnetworkTP must refer to element in a <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

POST_CONDITIONS

inv_noSubnetworkTP

"subnetworkTP does not participate in the <layerNetworkDomainIsMadeOf> relationship."

EXCEPTIONS

-- none

;}

#### 7.3.2.11	Report topological link creation

<COMMUNITY: Topology Management, ACTION: report topological link creation>

OPERATION reportTopologicalLinkCreation {

INPUT_PARAMETERS

layerND: LayerNetworkDomainId;

topologicalLink: TopologicalLinkId;

a_End: LinkEndChoice;

z_End: LinkEndChoice;

OUTPUT PARAMETERS

-- none

RAISED EXCEPTIONS

-- none

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: topmanLayerNetworkDomain>;

topologicalLink: <INFORMATION OBJECT: topmanTopologicalLink>;

subnetwork: <INFORMATION OBJECT: topmanSubnetwork>;

accessGroup: <INFORMATION OBJECT: topmanAccessGroup>;

TRIGGERING_CONDITIONS

PRE_CONDITIONS

-- none

POST_CONDITIONS

inv_existingTopologicalLink

"topologicalLink refers to element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_linkBinds

"a_End and z_End respectively refer to a_endTopological and z_endTopological in the <linkBinds> relationship where link refers to transferCapacityLink."

EXCEPTIONS

-- none

;}

#### 7.3.2.12	Report topological link deletion

<COMMUNITY: Topology Management, ACTION: report topological link deletion>

OPERATION reportTopologicalLinkDeletion {

INPUT_PARAMETERS

layerND: LayerNetworkDomainId;

topologicalLink: TopologicalLinkId;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

-- none

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: topmanLayerNetworkDomain>; topologicalLink: <INFORMATION OBJECT: topmanTopologicalLink>;

TRIGGERING_CONDITIONS

PRE_CONDITIONS

inv_existingTopologicalLink

"topologicalLink refers to element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_noLinkConnection

"topologicalLink shall not refer to any containerLink of a <linkHasLinkConnections> relationship."

POST_CONDITIONS

inv_noLink

"topologicalLink does not refer to any element in the <layerNetworkDomainIsMadeOf> relationship."

EXCEPTIONS

-- none

;}

#### 7.3.2.13	Report topological link end creation

<COMMUNITY: Topology Management, ACTION: report topological link end creation>

OPERATION reportTopologicalLinkEndCreation {

INPUT_PARAMETERS

layerND: LayerNetworkDomainId;

topologicalLinkEnd: TopologicalLinkEndId;

subnetwork: SubnetworkId;

OUTPUT PARAMETERS

-- none

RAISED EXCEPTIONS

-- none

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: topmanLayerNetworkDomain>;

topologicalLinkEnd: <INFORMATION OBJECT: topmanTopologicalLinkEnd>;

subnetwork: <INFORMATION OBJECT: topmanSubnetwork>;

TRIGGERING_CONDITIONS

PRE_CONDITIONS

-- none

POST_CONDITIONS

inv_existingLinkEnd

"topologicalLinkEnd and subnetwork refer to element in the same <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_boundTopologicalLinkEnd

"topologicalLinkEnd refers to transferCapacityLE in the <linkEndIsBoundTo> relationship where subnetwork refers to topologicalEntity."

EXCEPTIONS

-- none

;}

#### 7.3.2.14	Report topological link end deletion

<COMMUNITY: Topology Management, ACTION: report topological link end deletion>

OPERATION reportTopologicalLinkEndDeletion {

INPUT_PARAMETERS

layerND: LayerNetworkDomainId;

topologicalLinkEnd: TopologicalLinkEndId;

OUTPUT PARAMETERS

-- none

RAISED EXCEPTIONS

-- none

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: topmanLayerNetworkDomain>; topologicalLinkEnd: <INFORMATION OBJECT: topmanTopologicalLinkEnd>;

TRIGGERING_CONDITIONS

PRE_CONDITIONS

inv_existingTopologicalLinkEnd

"topologicalLinkEnd refers to element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_existingTopologicalLinkEndBinding

"topologicalLinkEnd refers to topologicalEntity in the <linkEndIsBoundTo> relationship."

inv_noNetworkCTPs

"topologicalLinkEnd shall not refer to any containerLE in a <linkEndHasNetworkCTPs> relationship."

POST_CONDITIONS

inv_noTopologicalLinkEnd

"topologicalLinkEnd does not refer to any element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

EXCEPTIONS

-- none

;}

## 7.4	ASN.1 supporting productions

In this specification, when an interface name is used within an ASN.1 production, the same label will be used, starting with a capital letter. The complete ASN.1 type definition for this query interface (e.g. use of ObjectIdentifier, INTEGER, ...) will be developed as part of the engineering viewpoint, with the concerned technology.

AccessGroupId ::= CHOICE {
	accessGroupQueryIfce		TopmanAccessGroupQueryIfce
	userIdentifier				UserIdentifier};

LayerNetworkDomainId ::= CHOICE {
	layerNetworkDomainQueryIfce	TopmanLayerNetworkDomainQueryIfce,
	userIdentifier				UserIdentifier };

LinkEndChoice ::= CHOICE {
	subnetwork				SubnetworkId,
	accessGroup				AccessGroupId };

LinkEndId ::= CHOICE {
	linkEndQueryIfce			TopmanLinkEndQueryIfce,
	userIdentifier				UserIdentifier };

LinkId ::= CHOICE {
	linkQueryIfce				TopmanLinkQueryIfce,
	userIdentifier				UserIdentifier };

ResourceChoice ::= CHOICE {
	resourceId				ResourceIfce,
	userIdentifier				SimpleNameType};

SetOfNetworkTTPs ::= SET OF CHOICE {
	networkTTPId			TopmanNetworkTTPQueryIfce,
	userIdentifier				SimpleNameType}

SubnetworkId ::= CHOICE {
	subnetworkQueryIfce		TopmanSubnetworkQueryIfce,
	userIdentifier				UserIdentifier};

TopologicalLinkEndId ::= CHOICE {
	topologicalLinkEndQueryIfce	TopmanTopologicalLinkEndQueryIfce,
	userIdentifier				UserIdentifier };

TopologicalLinkId ::= CHOICE {
	topologicalLinkQueryIfce		TopmanTopologicalLinkQueryIfce,
	userIdentifier				UserIdentifier };

UserIdentifier ::= SimpleNameType;

| ITU-T RECOMMENDATIONS SERIES |  |
| --- | --- |
| Series A | Organization of the work of the ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |