|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

| ITU-T | G.854.6 |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU | (03/99) |

SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

Digital transmission systems – Digital networks –Management of transport network

Computational viewpoint for trail management

ITU-T  Recommendation  G.854.6

(Previously  CCITT  Recommendation)

ITU-T  G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| For further details, please refer to ITU-T List of Recommendations. |  |
| --- | --- |
| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| INTERNATIONAL ANALOGUE CARRIER SYSTEM |  |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS |  |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TRANSMISSION SYSTEMS |  |
| TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| General aspects | G.800–G.809 |
| Design objectives for digital networks | G.810–G.819 |
| Quality and availability targets | G.820–G.829 |
| Network capabilities and functions | G.830–G.839 |
| SDH network characteristics | G.840–G.849 |
| Management of transport network | G.850–G.859 |
| SDH radio and satellite systems integration | G.860–G.869 |
| Optical transport networks | G.870–G.879 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |

ITU-T RECOMMENDATION G.854.6 computational viewpoint for trail management

Summary Enterprise community actions covered by this specification: • set-up point-to-point trail; • modify trail; • release trail; • create trail termination point; • delete trail termination point; • associate trail termination point with access group; • disassociate trail termination point from access group; • associate trail termination point with subnetwork; • disassociate trail termination point from subnetwork; • report trail set-up; • report trail release; • report trail modification; • report trail termination point creation; • report trail termination point deletion; • report association of trail termination point with access group; • report disassociation of trail termination point from access group; • report association of trail termination point with subnetwork; • report disassociation of trail termination point from subnetwork.

Source ITU-T Recommendation G.854.6 was prepared by ITU-T Study Group 4 (1997-2000) and was approved under the WTSC Resolution No. 1 procedure on the 26th of March 1999.

FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation the term recognized operating agency (ROA) includes any individual, company, corporation or governmental organization that operates a public correspondence service. The terms Administration, ROA and public correspondence are defined in the Constitution of the ITU (Geneva, 1992).

INTELLECTUAL PROPERTY RIGHTS

The ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. The ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, the ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  1999

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Definitions		1

4	Abbreviations		1

5	Conventions		2

6	Label references		2

7	Interfaces		3

7.1	Query interfaces		3

7.2	Operational interfaces		4

7.2.1	Trail provisioning interface		4

7.3	Reporting interfaces		16

7.3.1	Trail provisioning reporting interface		16

7.4	ASN.1 supporting productions		25

Recommendation G.854.6

COMPUTATIONAL VIEWPOINT FOR TRAIL MANAGEMENT

(Geneva, 1999)

# 1	Scope

This computational viewpoint specification is related to the trail management enterprise specification defined in Recommendation G.852.6 and the trail management information specification defined in Recommendation G.853.6.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[1]	ITU-T Recommendation G.851.1 (1996), Management of the transport network – Application of the RM-ODP framework.

[2]	ITU-T Recommendation G.853.1 (1999), Common elements of the information viewpoint for the management of a transport network.

[3]	ITU-T Recommendation G.852.6 (1999), Enterprise viewpoint for trail management.

[4]	ITU-T Recommendation G.853.6 (1999), Information viewpoint for trail management.

# 3	Definitions

None.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

ASN.1	Abstract Syntax Notation One

Id	Identifier

Ifce	Interface

inv	invariant

ITU	International Telecommunication Union

layerND	layerNetwork Domain

LC	LinkConnection

ND	Network Domain

RM-ODP	Reference Model for Open Distributed Processing

tm	trail management

TTP	Trail Termination Point

# 5	Conventions

In order to increase the readability of the behaviour in the operations:

•	parameters are written in bold;

•	elements defined in the information viewpoint specification are written in italic.

# 6	Label references

| Full label reference | Local label reference |
| --- | --- |
| <"Rec. G.853.6", INFORMATION_OBJECT: tmAccessGroup> | tmAccessGroup |
| <"Rec. G.853.6", INFORMATION_OBJECT: tmLayerNetworkDomain> | tmLayerNetworkDomain |
| <"Rec. G.853.6", INFORMATION_OBJECT: tmNetworkTTP> | tmNetworkTTP |
| <"Rec. G.853.6", INFORMATION_OBJECT: tmSubnetwork> | tmSubnetwork |
| <"Rec. G.853.6", INFORMATION_OBJECT: tmSubnetworkConnection> | tmSubnetworkConnection |
| <"Rec. G.853.6", INFORMATION_OBJECT: tmSubnetworkTP> | tmSubnetworkTP |
| <"Rec. G.853.6", INFORMATION_OBJECT: tmTrafficDescriptor> | tmTrafficDescriptor |
| <"Rec. G.853.6", INFORMATION_OBJECT: tmTrail> | tmTrail |
| <"Rec. G.853.6", INFORMATION_OBJECT: tmTrailServiceCharacteristics> | tmTrailServiceCharacteristics |
| <"Rec.G.853.1", INFORMATION_RELATIONSHIP: accessGroupIsMadeOfNetworkTTPs> | accessGroupIsMadeOfNetworkTTPs |
| <"Rec. G.853.1", INFORMATION_RELATIONSHIP: layerNetworkDomainIsMadeOf> | layerNetworkDomainIs MadeOf |
| <"Rec.G.853.1", INFORMATION_RELATIONSHIP: subnetworkConnectionIsTerminatedByPointToPoint> | subnetworkConnectionIsTerminatedByPointToPoint |
| <"Rec.G.853.1", INFORMATION_RELATIONSHIP: subnetworkIsDelimitedBy> | subnetworkIsDelimitedBy |
| <"Rec.G.853.1", INFORMATION_RELATIONSHIP: subnetworkTPIsRelatedToExtremity> | subnetworkTPIsRelatedToExtremity |
| <"Rec. G.853.6", INFORMATION_RELATIONSHIP: tmTrailHasImmediateTrafficDescriptor> | tmTrailHasImmediateTrafficDescriptor |
| <"Rec.G.853.6", INFORMATION_RELATIONSHIP: tmTrailHasTSC> | tmTrailHasTSC |
| <"Rec.G.853.1", INFORMATION_RELATIONSHIP: trailIsTerminatedByPointToPoint> | trailIsTerminatedByPointToPoint |
| <"Rec. G.853.1", INFORMATION_ATTRIBUTE: directionality> | directionality |
| <"Rec. G.853.1", INFORMATION_ATTRIBUTE: resourceId> | resourceId |
| <"Rec.G.853.1", INFORMATION_ATTRIBUTE: signalIdentification> | signalIdentification |
| <"Rec.G.853.1", INFORMATION_ATTRIBUTE: userLabel> | userLabel |
| <"Rec. G.854.3", INTERFACE: commonReportResourceIdChangeIfce> | commonReportResourceIdChangeIfce |
| <"Rec. G.854.3", INTERFACE: commonResourceIfce> | commonResourceIfce |

| Full ASN.1 production reference | Local label reference |
| --- | --- |
| <"Rec. X.721:1992: Attribute-ASN.1 Module": SimpleNameType> | SimpleNameType |
| <"Rec. X.680:1997: GraphicString"> | GraphicString |

# 7	Interfaces

This specification does not define operations which change the userLabel of the resources or report this change to the notification receiver because they do not change the state of the system. Their exact signature will be developed as part of the engineering viewpoint specification with the concerned technology.

## 7.1	Query interfaces

This specification refers to interfaces that allow get access to identification and properties of resources involved in the "trail management" community. As the invocation of the contained operations do not modify any state, there is no interest to develop them explicitly. Their exact signature will be developed as part of the engineering viewpoint, with the concerned technology. These interfaces are listed in Table 1 with the information they allow access to.

| Table 1/G.854.6 – Query interfaces |  |  |
| --- | --- | --- |
| Interface name | Information object | Attributes and relationships |
| tmAccessGroupQueryIfce | <tmAccessGroup> | <resourceId> <signalIdentification> <topologicalEndDirection> <layerNetworkDomainIsMadeOf, ROLE: containerLND> <accessGroupIsMadeOfNetworkTTPs, ROLE: elementTTP> |
| tmLayerNetworkDomainQueryIfce | <tmLayerNetworkDomain> | <resourceId> <signalIdentification> <layerNetworkDomainIsMadeOf, ROLE: element> |
| tmNetworkTTPQueryIfce | <tmNetworkTTP> | <resourceId> <signalIdentification> <userLabel> <pointDirectionality> <accessGroupIsMadeOfNetworkTTPs, ROLE: containerAG> <trailIsTerminatedByPointToPoint, ROLE: transportEntityTrail> <subnetworkTPIsRelatedToExtremity, ROLE: abstractionSNTP> <layerNetworkDomainIsMadeOf, ROLE: containerLND> |
| tmSubnetworkConnectionQueryIfce | <tmSubnetworkConnection> | <resourceId> <signalIdentification> <directionality> <subnetworkConnectionIsTerminatedBy PointToPoint, ROLE: a_endSNTP, ROLE: z_endSNTP> <layerNetworkDomainIsMadeOf, ROLE: containerLND> |
| tmSubnetworkQueryIfce | <tmSubnetwork> | <resourceId> <signalIdentification> <layerNetworkDomainIsMadeOf, ROLE: containerLND> <subnetworkIsDelimitedBy, ROLE: elementSNTP> |

Table 1/G.854.6 – Query interfaces (concluded)

| Interface name | Information object | Attributes and relationships |
| --- | --- | --- |
| tmSubnetworkTPQueryIfce | <tmSubnetworkTP> | <resourceId> <signalIdentification> <pointDirectionality> <subnetworkTPIsRelatedToExtremity, ROLE: extremity> <layerNetworkDomainIsMadeOf, ROLE: ContainerLND> <subnetworkIsDelimitedBy, ROLE: containerSN> <subnetworkConnectionIsTerminatedBy PointToPoint, ROLE: transportEntitySNC> |
| tmTrafficDescriptorQueryIfce | <tmTrafficDescriptor> | <signalIdentification> <tmTrailHasImmediateTrafficDescriptor, ROLE: trailTD> |
| tmTrailQueryIfce | <tmTrail> | <resourceId> <signalIdentification> <userLabel> <directionality> <trailIsTerminatedByPointToPoint, ROLE: a_endNTTP, ROLE: z_endNTTP> <tmTrailHasTSC, ROLE: transportQualifierTSC> <tmTrailHasImmediateTrafficDescriptor, ROLE: immediateTrafficDescriptorTrail> <layerNetworkDomainIsMadeOf, ROLE: containerLND> |
| tmTrailServiceCharacteristicsQueryIfce | <tmTrailServiceCharacteristics> | <resourceId> <signalIdentification> <tmTrailHasTSC, ROLE: transportQualifiedTrail> |

## 7.2	Operational interfaces

### 7.2.1	Trail provisioning interface

The trail provisioning interface manages the immediate set-up, release and modification of point-to-point trails as well as the creation and deletion of network TTPs and the association and disassociation of network TTPs with/from access groups and subnetworks. The interface is required to satisfy the enterprise requirements stated in:

<"Recommendation G.852.6, "COMMUNITY trail management, ACTION: setup point-to-point trail>,

<"Recommendation G.852.6, "COMMUNITY trail management, ACTION: release trail>,

<"Recommendation G.852.6, "COMMUNITY trail management, ACTION: modify trail>,

<"Recommendation G.852.6, "COMMUNITY trail management, ACTION: create trail termination point>,

<"Recommendation G.852.6, "COMMUNITY trail management, ACTION: delete trail termination point>,

<"Recommendation G.852.6, "COMMUNITY trail management, ACTION: associate trail termination point with access group>,

<"Recommendation G.852.6, "COMMUNITY trail management, ACTION: disassociate trail termination point from access group>,

<"Recommendation G.852.6, "COMMUNITY trail management, ACTION: associate trail termination point with subnetwork>,

<"Recommendation G.852.6, "COMMUNITY trail management, ACTION: disassociate trail termination point from subnetwork>

By inheriting the properties of the commonResourceIfce, the trailProvisioningIfce provides the capability to change the resource identifier of the resources involved.

COMPUTATIONAL INTERFACE trailProvisioning Ifce{

DERIVED FROM <commonResourceIfce>

OPERATION {

<setupPointToPointTrail>;

<releaseTrail>;

<modifyTrail>;

<createNetworkTTP>;

<deleteNetworkTTP>

<associateNetworkTTPWithAccessGroup>;

<disassociateNetworkTTPFromAccessGroup>;

<associateNetworkTTPWithSubnetwork>;

<disassociateNetworkTTPFromSubnetwork>;

}

}

#### 7.2.1.1	Set-up point-to-point trail

<COMMUNITY: trail management, ACTION: setup point-to-point trail>

OPERATION setupPointToPointTrail {

INPUT_PARAMETERS

aEnd: AEndChoice ::= CHOICE {

networkTTP NetworkTTPChoice,

accessGroup AccessGroupChoice};

zEnd: ZEndChoice ::= CHOICE {

networkTTP NetworkTTPChoice,

accessGroup AccessGroupChoice};

layerND: LayerNetworkDomainChoice;

dir: Directionality;

suppliedUserIdentifier: UserIdentifier;

-- zero length string or 0 implies none supplied.

serviceCharacteristics: TrailServiceCharacteristicsId;

-- reference can be used to determine any QOS or routing characteristics;

trafficDescriptor: ImmediateTrafficDescriptorId;

suppliedUserLabel: GraphicString;

-- zero length string implies none supplied

OUTPUT_PARAMETERS

newTrail: TrailChoice;

connectedAEndNetworkTTP: NetworkTTPChoice;

connectedZEndNetwrkTTP: NetworkTTPChoice;

RAISED_EXCEPTIONS

networkTTPsNotPartOfLayerND: SequenceOf  NetworkTTPChoice;

-- the list contains one element when only one point is incorrect, i.e. is not part of the
-- layerNetworkDomain.

aEndNetworkTTPConnected:NetworkTTPChoice;

networkTTPsInAEndAccessGroupConnected: AccessGroupChoice;

zEndNetworkTTPConnected: NetworkTTPChoice;

networkTTPsInZEndAccessGroupConnected: AccessGroupChoice;

userIdentifierNotUnique: UserIdentifier;

failureToSetUserIdentifier: NULL;

failureToCreateTrail: NULL;

invalidTransportServiceCharacteristics: NULL;

invalidTrafficDescriptor: NULL;

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

networkTTP: <INFORMATION OBJECT: tmNetworkTTP>;

accessGroup: <INFORMATION OBJECT: tmAccessGroup>;

layerND: <INFOMATION OBJECT: tmLayerNetworkDomain>;

dir: <INFORMATION ATTRIBUTE: Directionality>;

suppliedUserIdentifier: <INFORMATION ATTRIBUTE: resourceId>;

serviceCharacteristics: <INFORMATION OBJECT: tmTrailServiceCharacteristics>;

trafficDescriptor: <INFORMATION OBJECT: tmTrafficDescriptor>;

suppliedUserLabel: <INFORMATION ATTRIBUTE: userLabel>;

newTrail: <INFORMATION OBJECT: tmTrail>;

connectedAEndNetworkTTP: <INFORMATION OBJECT: tmNetworkTTP>;

connectedZEndNetworkTTP: <INFORMATION OBJECT: tmNetworkTTP>;

PRE_CONDITIONS

inv_layerNDContainment

"aEnd and zEnd are referring to element in a <layerNetworkDomainIsMadeOf> relationship
where layerND refers to containerLND."

inv_freeAEndNetworkTTP

"If aEnd is a networkTTP, this networkTTP must not refer to a_endNTTP or z_endNTTP in any 
<trailIsTerminatedByPointToPoint> relationship."

inv_freeAEndAccessGroupMember

"If aEnd is an accessGroup, at least one networkTTP referring to elementTTP in an <accessGroupIsMadeOfNetworkTTPs> relationship where this accessGroup refers to containerAG, must not refer to a_endNTTP or z_endNTTP in any <trailIsTerminatedByPointToPoint> relationship."

inv_freeZEndNetworkTTP

"If zEnd is a networkTTP, this networkTTP must not refer to a_endNTTP or z_endNTTP in any 
<trailIsTerminatedByPointToPoint> relationship."

inv_freeZEndAccessGroupMember

"If zEnd is an accessGroup, at least one networkTTP referring to elementTTP in an <accessGroupIsMadeOfNetworkTTPs> relationship where this accessGroup refers to containerAG, must not refer to a_endNTTP or z_endNTTP in any <trailIsTerminatedByPointToPoint> relationship."

inv_uniqueUserIdentifier

"suppliedUserIdentifier shall not be equal to <resourceId> of any element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

POST_CONDITIONS

inv_agreedUserIdentifier

"<resourceId> of <tmTrail> referred to by newTrail is equal to suppliedUserIdentifier, if it is supplied."

inv_connectedTrail

"connectedAEndNetworkTTP, connectedZEndNetworkTTP and newTrail must respectively refer to a_endNTTP, z_endNTTP and transportEntityTrail in a <trailIsTerminatedByPointToPoint> relationship."

inv_transportServiceCharacteristics

"newTrail and serviceCharacteristics must refer to transportQualifiedTrail and transportQualifierTSC in a <tmTrailHasTSC> relationship."

inv_trafficDescriptor

"newTrail and trafficDescriptor must refer to trailTD and immediateTrafficDescriptorTrail in a <tmTrailHasImmediateTrafficDescriptor> relationship."

EXCEPTIONS

IF PRE_CONDITION inv_layerNDContainment NOT_VERIFIED RAISE_EXCEPTION
			networkTTPsNotPartOfLayerND;

IF PRE_CONDITION inv_freeAEndNetworkTTP NOT_VERIFIED RAISE_EXCEPTION 
			aEndNetworkTTPConnected ;

IF PRE_CONDITION inv_freeAEndAccessGroupMember NOT_VERIFIED 
			RAISE_EXCEPTION networkTTPsInAEndAccessGroupConnected ;

IF PRE_CONDITION inv_freeZEndNetworkTTP NOT_VERIFIED RAISE_EXCEPTION 
			zEndNetworkTTPConnected ;

IF PRE_CONDITION inv_freeZEndAccessGroupMember NOT_VERIFIED 
			RAISE_EXCEPTION networkTTPsInZEndAccessGroupConnected ;

IF PRE_CONDITION inv_uniqueUserIdentifier NOT_VERIFIED RAISE_EXCEPTION 
			userIdentifierNotUnique;

IF POST_CONDITION inv_agreedUserIdentifier NOT VERIFIED RAISE_EXCEPTION 
			failureToSetUserIdentifier;

IF POST_CONDITION inv_connectedTrail NOT_VERIFIED RAISE_EXCEPTION 
			failureToCreateTrail;

IF POST_CONDITION inv_transportServiceCharacteristics NOT_VERIFIED 
			RAISE_EXCEPTION invalidTransportServiceCharacteristics;

IF POST_CONDITION inv_trafficDescriptor NOT_VERIFIED RAISE_EXCEPTION 
			invalidTrafficDescriptor;

}

#### 7.2.1.2	Release trail

<COMMUNITY: trail management, ACTION: release trail>

OPERATION	releaseTrail {

INPUT_PARAMETERS

trail: TrailChoice;

layerND: LayerNetworkDomainChoice;

OUTPUT_PARAMETERS

trail: TrailChoice;

RAISED_EXCEPTIONS

unknownTrail: TrailChoice;

trailConnected: NULL;

failureToReleaseTrail: NULL;

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

trail: <INFORMATION OBJECT: tmTrail>;

layerND: <INFOMATION OBJECT: tmLayerNetworkDomain>;

PRE_CONDITIONS

inv_existingTrail

"trail must refer to element in the <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_trailConnected

"trail must not refer to transportEntityTrail in a <trailIsTerminatedByPointToPoint> relationship."

POST_CONDITIONS

inv_trailReleased

"trail must not refer to any element in a <layerNetworkDomainIsMadeOf> relationship."

EXCEPTIONS

IF PRE_CONDITION inv_existingTrail NOT_VERIFIED RAISE_EXCEPTION

unknownTrail;

IF PRE_CONDITION inv_trailConnected NOT_VERIFIED RAISE_EXCEPTION

trailConnected;

IF POST_CONDITION inv_trailReleased NOT_VERIFIED RAISE_EXCEPTIONfailureToReleaseTrail;

}

#### 7.2.1.3	Modify trail

<COMMUNITY: trail management, ACTION: modify trail>

OPERATION modifyTrail {

INPUT_PARAMETERS

trail: TrailChoice;

layerND: LayerNetworkDomainChoice;

newTrafficDescriptor: ImmediateTrafficDescriptorId;

newServiceCharacteristics: TrailServiceCharacteristicsId;

OUTPUT_PARAMETERS

agreedTrafficDescriptor: ImmediateTrafficDescriptorId;

-- The old trafficDescriptor is maintained if the new one cannot be supported

agreedServiceCharacteristics: TrailServiceCharacteristicsId;

-- The old serviceCharacteristics is maintained if the new one cannot be supported

RAISED_EXCEPTIONS

unknownTrail: TrailChoice;

invalidServiceCharacteristicsRequested: NULL;

invalidTrafficDescriptorRequested: NULL;

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

trail: <INFORMATION OBJECT: tmTrail>;

layerND: <INFOMATION OBJECT: tmLayerNetworkDomain>;

newTrafficDescriptor: <INFORMATION OBJECT: tmTrafficDescriptor>;

newServiceCharacteristics: <INFORMATION OBJECT: tmTrailServiceCharacteristics>;

agreedTrafficDescriptor: <INFORMATION OBJECT: tmTrafficDescriptor>;

agreedServiceCharacteristics: <INFORMATION OBJECT: tmTrailServiceCharacteristics>;

PRE_CONDITIONS

inv_existingTrail

"trail must refer to element in a <layerNetworkDomainIsMadeOf> relationship where layerND being the containerLND."

POST_CONDITIONS

inv_agreedServiceCharacteristics

"trail and agreedServiceCharacteristics must be involved as transportQualifiedTrail and transportQualifierTSC in a <tmTrailHasTSC> relationship."

inv_agreedTrafficDescriptor

"trail and agreedTrafficDescriptor must be involved as trailTD and 
immediateTrafficDescriptorTrail in a <tmTrailHasImmediateTrafficDescriptor> relationship."

EXCEPTIONS

IF PRE_CONDITION inv_existingTrail NOT_VERIFIED RAISE_EXCEPTION

unknownTrail;

IF POST_CONDITION inv_agreedServiceCharacteristics NOT_VERIFIED RAISE_EXCEPTION 
			invalidServiceCharacteristicsRequested;

IF POST_CONDITION inv_agreedTrafficDescriptor NOT_VERIFIED RAISE_EXCEPTION 
			invalidTrafficDescriptorRequested;

}

#### 7.2.1.4	Create networkTTP

<COMMUNITY: trail management, ACTION: create trail termination point>

OPERATION createNetworkTTP {

INPUT_PARAMETERS

layerND: LayerNetworkDomainChoice;

pointDir: PointDirectionality;

suppliedUserIdentifier: UserIdentifier;

-- zero length string or 0 implies none supplied.

suppliedUserLabel: GraphicString;

-- zero length implies none supplied.

OUTPUT_PARAMETERS

networkTTP: NetworkTTPChoice;

RAISED_EXCEPTIONS

userIdentifierNotUnique: UserIdentifier;

failureToCreateNetworkTTP: NULL;

failureToSetUserIdentifier:NULL;

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

layerND: INFORMATION OBJECT: tmLayerNetworkDomain>;

suppliedUserIdentifier: <INFORMATION ATTRIBUTE: resourceId>;

pointDir: <INFORMATION ATTRIBUTE: pointDirectionality>;

networkTTP: <INFORMATION OBJECT: tmNetworkTTP>;

suppliedUserLabel: <INFORMATION ATTRIBUTE: userLabel>;

PRE_CONDITIONS

inv_uniqueUserIdentifier

"suppliedUserIdentifier shall not be equal to resourceId of any element in a
		<layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

POST_CONDITIONS

inv_existingNetworkTTP

"networkTTP and layerND must respectively refer to element and containerLND in a <layerNetworkDomainIsMadeOf> relationship."

inv_agreedUserIdentifier

"resourceId of tmNetworkTTP referenced by networkTTP is equal to suppliedUserIdentifier, if it is supplied."

EXCEPTIONS

IF PRE_CONDITION inv_uniqueUserIdentifier NOT_VERIFIED RAISE_EXCEPTION 
			userIdentifierNotUnique;

IF POST_CONDITION inv_existingNetworkTTP NOT_VERIFIED RAISE_EXCEPTION 
			failureToCreateNetworkTTP;

IF POST_CONDITION inv_agreedUserIdentifier NOT_VERIFIED RAISE_EXCEPTION 
			failureToSetUserIdentifier;

}

#### 7.2.1.5	Delete networkTTP

<COMMUNITY: trail management, ACTION: delete trail termination point>

OPERATION deleteNetworkTTP {

INPUT_PARAMETERS

layerND:LayerNetworkDomainChoice;

networkTTP: NetworkTTPChoice;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

invalidNetworkTTP: NetworkTTPChoice;

networkTTPTerminatesTrail: NetworkTTPChoice;

networkTTPAssociatedWithSubnetwork: NetworkTTPChoice;

networkTTPAssociatedWithAccessGroup: NetworkTTPChoice;

failureToDeleteNetworkTTP: NULL;

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: tmLayerNetworkDomain>;

networkTTP: <INFORMATION OBJECT: tmNetworkTTP>;

PRE_CONDITIONS

inv_existingNetworkTTP

"networkTTP must refer to element in a <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_noTrailTermination

"networkTTP may not refer to a_endNTTP or z_endNTTP in any 
<trailIsTerminatedByPointToPoint> relationship."

inv_noSubnetworkAssociation

"The networkTTP shall not refer to extremity of a <subnetworkTPIsRelatedToExtremity> relationship where abstractionSNTP references elementSNTP of a <subnetworkIsDelimitedBy> relationship.

"

inv_noAccessGroupAssociation

"networkTTP may not refer to elementTTP in an <accessGroupIsMadeOfNetworkTTPs> relationship."

POST_CONDITIONS

inv_noNetworkTTP

"networkTTP does not refer to element in any <layerNetworkDomainIsMadeOf> relationship ."

EXCEPTIONS

IF PRE_CONDITION inv_existingNetworkTTP NOT_VERIFIED RAISE_EXCEPTION 	invalidNetworkTTP;

IF PRE_CONDITION inv_noTrailTermination NOT_VERIFIED RAISE_EXCEPTION 	networkTTPTerminatesTrail;

IF PRE_CONDITIONinv_noSubnetworkAssociation NOT_VERIFIED RAISE_EXCEPTION 	networkTTPAssociatedWithSubnetwork;

IF PRE_CONDITION inv_noAccessGroupAssociation NOT_VERIFIED RAISE_EXCEPTION 	networkTTPAssociatedWithAccessGroup;

IF POST_CONDITION inv_noNetworkTTP NOT_VERIFIED RAISE_EXCEPTION 	failureToDeleteNetworkTTP;

}

#### 7.2.1.6	Associate networkTTP with access group

<COMMUNITY: trail management, ACTION: associate trail termination point with access group>

OPERATION associateNetworkTTPWithAccessGroup{

INPUT_PARAMETERS

layerND: LayerNetworkDomainChoice;

accessGroup: AccessGroupChoice;

networkTTP: NetworkTTPChoice;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

networkTTPAndAccessGroupNotCompatible: NULL;

networkTTPAlreadyAssociated: NULL;

failureToAssociateNetworkTTP: NULL;

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: tmLayerNetworkDomain>;

accessGroup: <INFORMATION OBJECT: tmAccessGroup>;

networkTTP:<INFORMATION OBJECT: tmNetworkTTP>;

PRE_CONDITIONS

inv_networkTTPAndAccessGroupExistingAndCompatible

"networkTTP and accessGroup shall refer to element of the same <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_networkTTPNotAlreadyAssociated

"networkTTP shall not refer to elementTTP in any <accessGroupIsMadeOfNetworkTTPs> relationship."

POST_CONDITIONS

inv_networkTTPAssociated

"networkTTP refers to elementTTP in an <accessGroupIsMadeOfNetworkTTPs> relationship where accessGroup refers to containerAG."

EXCEPTIONS

IF PRE_CONDITION inv_networkTTPAndAccessGroupExistingAndCompatible NOT_VERIFIED RAISE_EXCEPTION

networkTTPAndAccessGroupNotCompatible;

IF PRE_CONDITION inv_networkTTPNotAlreadyAssociated NOT_VERIFIED_RAISE_EXCEPTION

networkTTPAlreadyAssociated;

IF POST_CONDITION inv_networkTTPAssociated NOT_VERIFIED RAISE_EXCEPTION

failureToAssociateNetworkTTP;

}

#### 7.2.1.7	Disassociate networkTTP from access group

<COMMUNITY: trail management, ACTION: disassociate trail termination point from access group>

OPERATION disassociateNetworkTTPfromAccessGroup {

INPUT_PARAMETERS

layerND: LayerNetworkDomainChoice;

accessGroup: AccessGroupChoice;

networkTTP: NetworkTTPChoice;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

networkTTPAndAccessGroupNotCompatible: NULL;

networkTTPNotAssociated: NULL;

failureToDisassociate: NULL;

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: tmLayerNetworkDomain>;

accessGroup: <INFORMATION OBJECT: tmAccessGroup>;

networkTTP:	<INFORMATION OBJECT: tmNetworkTTP>;

PRE_CONDITIONS

inv_networkTTPAssociated

"networkTTP refers to elementTTP of <accessGroupIsMadeOfNetworkTTPs> relationship where accessGroup refers to containerAG."

POST_CONDITIONS

inv_networkTTPDisassociated

"networkTTP shall not refer to elementTTP in an <accessGroupIsMadeOfNetworkTTPs> relationship where accessGroup refers to containerTTP."

EXCEPTIONS

IF PRE_CONDITION inv_networkTTPAndAccessGroupExistingAndCompatible NOT_VERIFIED RAISE_EXCEPTION

networkTTPAndAccessGroupNotCompatible;

IF PRE_CONDITION inv_networkTTPAssociated NOT_VERIFIED RAISE_EXCEPTION

networkTTPNotAssociated;

IF POST_CONDITION inv_networkTTPDisassociated NOT_VERIFIED RAISE_EXCEPTION

failureToDisassociate;

}

#### 7.2.1.8	Associate networkTTP with subnetwork

<COMMUNITY: trail management, ACTION: associate trail termination point with subnetwork>

OPERATION associateNetworkTTPWithSubnetwork {

INPUT_PARAMETERS

layerND: LayerNetworkDomainChoice;

subnetwork: SubnetworkChoice;

networkTTP: NetworkTTPChoice;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

networkTTPAndSubnetworkNotCompatible: NULL;

networkTTPAssociated: NetworkTTPChoice;

failureToAssociateNetworkTTP: NULL;

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: tmLayerNetworkDomain>;

subnetwork: <INFORMATION OBJECT: tmSubnetwork>;

networkTTP: <INFORMATION OBJECT: tmNetworkTTP>;

PRE_CONDITIONS

inv_networkTTPAndSubnetworkExistingAndCompatible

"networkTTP and subnetwork shall refer to element of the same <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_networkTTPNotAssociated

"The networkTTP shall not refer to extremity of a <subnetworkTPIsRelatedToExtremity> relationship where abstractionSNTP references elementSNTP of a <subnetworkIsDelimitedBy> relationship."

POST_CONDITIONS

inv_networkTTPAssociated

"networkTTP refers to extremity of a <subnetworkTPIsRelatedToExtremity> relationship where abstractionSNTP, which is a subnetworkTP, is also referencing elementSNTP of a <subnetworkIsDelimitedBy> relationship where subnetwork refers to containerSN.

"

EXCEPTIONS

IF PRE_CONDITION inv_networkTTPAndSubnetworkExistingAndCompatible NOT_VERIFIED 
		RAISE_EXCEPTION

networkTTPAndSubnetworkNotCompatible;

IF PRE_CONDITIONinv_networkTTPNotAssociated NOT_VERIFIED_RAISE_EXCEPTION

networkTTPAssociated;

IF POST_CONDITION inv_networkTTPAssociated NOT_VERIFIED RAISE_EXCEPTION

failureToAssociateNetworkTTP;

}

#### 7.2.1.9	Disassociate networkTTP from subnetwork

<COMMUNITY: trail management, ACTION: disassociate trail termination point from subnetwork>

OPERATION disassociateNetworkTTPfromSubnetwork {

INPUT_PARAMETERS

layerND: LayerNetworkDomainChoice;

subnetwork: SubnetworkChoice;

networkTTP: NetworkTTPChoice;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

networkTTPAndSubnetworkNotCompatible: NULL;

networkTTPNotAssociated: NTTPId

networkTTPConnected: NULL;

failureToDisassociate: NULL;

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: tmLayerNetworkDomain>;

subnetwork: <INFORMATION OBJECT: tmSubnetwork>

networkTTP: 	<INFORMATION OBJECT: tmNetworkTTP>;

PRE_CONDITIONS

inv_networkTTPAssociated

"networkTTP refers to extremity of a <subnetworkTPIsRelatedToExtremity> relationship where abstractionSNTP, which is a subnetworkTP, is also elementSNTP of a <subnetworkIsDelimitedBy> relationship where subnetwork refers to containerSN.

"

inv_networkTTPNotAssociated

"abstractionSNTP (which is a subnetworkTP) of a <subnetworkTPIsRelatedToExtremity> relationship where networkTTP refers to extremity, shall not reference a_endSNTP or z_endSNTP of a <subnetworkConnectionIsTerminatedByPointToPoint> relationship.

"

POST_CONDITIONS

inv_networkTTPNotAssociated

<"networkTTP shall not refer to extremity of a <subnetworkTPIsRelatedToExtremity> relationship where abstractionSNTP references elementSNTP of a <subnetworkIsDelimitedBy> relationship. "

EXCEPTIONS

IF PRE_CONDITION inv_networkTTPAndSubnetworkExistingAndCompatible NOT_VERIFIED RAISE_EXCEPTION

networkTTPAndSubnetworkNotCompatible;

IF PRE_CONDITIONinv_networkTTPAssociated NOT_VERIFIED RAISE_EXCEPTION

networkTTPNotAssociated;

IF PRE_CONDITION inv_networkTTPNotConnected NOT_VERIFIED RAISE_EXCEPTION

networkTTPConnected;

IF POST_CONDITION inv_networkTTPNotAssociated NOT_VERIFIED RAISE_EXCEPTION

failureToDisassociate;

}

## 7.3	Reporting interfaces

### 7.3.1	Trail provisioning reporting interface

The trail provisioning reporting interface reports the immediate set-up, release and modification of point-to-point trails as well as the creation and deletion of network TTPs and the association and disassociation of network TTPs with access groups and subnetworks. The interface is required to satisfy the enterprise requirements stated in:

<"Recommendation G.852.6,"COMMUNITY trail management, ACTION: report trail setup>,

<"Recommendation G.852.6,"COMMUNITY trail management, ACTION: report trail release>,

<"Recommendation G.852.6,"COMMUNITY trail management, ACTION: report trail modification>,

<"Recommendation G.852.6,"COMMUNITY trail management, ACTION: report trail termination point creation>,

<"Recommendation G.852.6,"COMMUNITY trail management, ACTION: report trail termination point deletion>,

<"Recommendation G.852.6,"COMMUNITY trail management, ACTION: report association of trail termination point with access group>,

<"Recommendation G.852.6,"COMMUNITY trail management, ACTION: report disassociation of trail termination point from access group>,

<"Recommendation G.852.6,"COMMUNITY trail management, ACTION: report association of trail termination point with subnetwork>,

<"Recommendation G.852.6,"COMMUNITY trail management, ACTION: report disassociation of trail termination point from subnetwork>.

By inheriting the properties of the <commonReportResourceIdChangeIfce> the trailProvisioningReportingIfce provides the capability of reporting the change of the resource identifier of the resources involved.

COMPUTATIONAL INTERFACEtrailProvisioningReportingIfce {

DERIVED FROM		<commonReportResourceIdChangeIfce>

OPERATION {

<reportPointToPointTrailSet_up>;

<reportTrailRelease>;

<reportTrailModification>;

<reportNetworkTTPCreation>;

<report networkTTPDeletion>;

<reportAssociationOfNetworkTTPWithAccessGroup>;

<reportDisassociationOfNetworkTTPFromAccessGroup>;

<reportAssociationOfNetworkTTPWithSubnetwork>;

<reportDisassociationOfNetworkTTPFromSubnetwork>;

}

}

#### 7.3.1.1	Report point-to-point trail set-up

<COMMUNITY: trail management, ACTION: report trail setup>

OPERATION reportPointToPointTrailSet_up{

INPUT_PARAMETERS

newTrail: TrailChoice;

connectedAEndNetworkTTP: NetworkTTPChoice;

connectedZEndNetworkTTP: NetworkTTPChoice;

layerND: LayerNetworkDomainChoice;

OUTPUT_PARAMETERS

-- none

RAISED EXCEPTIONS

-- none

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

newTrail: <INFORMATION OBJECT: tmTrail>;

connectedAEndNetworkTTP: <INFORMATION OBJECT: tmNetworkTTP>;

connectedZEndNetworkTTP: <INFORMATION OBJECT: tmNetworkTTP>;

layerND: <INFORMATION OBJECT: tmLayerNetworkDomain>;

TRIGGERING CONDITIONS

PRE_CONDITIONS

inv_layerNDContainment

"connectedAEndNetworkTTP and connectedZEndNetworkTTP are referencing element in a <layerNetworkDomainIsMadeOf> relationship where layerND is referencing containerLND."

POST_CONDITIONS

inv_connectedTrail

"connectedAEndNetworkTTP, connectedZEndNetworkTTP and newTrail respectively reference a_endNTTP, z_endNTTP and transportEntityTrail in a <trailIsTerminatedByPointToPoint> relationship."

EXCEPTIONS

-- none

}

#### 7.3.1.2	Report trail release

<COMMUNITY: trail management, ACTION: report trail release>

OPERATION reportTrailRelease {

INPUT_PARAMETERS

trail: TrailChoice;

layerND: LayerNetworkDomainChoice;

OUTPUT_PARAMETERS

-- none

RAISED EXCEPTIONS

-- none

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

trail: <INFORMATION OBJECT: tmTrail>;

layerND: <INFORMATION OBJECT: tmLayerNetworkDomain>;

TRIGGERING CONDITIONS

PRE_ CONDITIONS

inv_connectedTrail

"trail must refer to transportEntityTrail in a <trailIsTerminatedByPointToPoint> relationship."

POST_CONDITIONS

inv_disconnectedTrail

"trail is not allowed participating in a <trailIsTerminatedByPointToPoint> relationship referencing transportEntityTrail. "

EXCEPTIONS

-- none

}

#### 7.3.1.3	Report trail modification

<COMMUNITY: trail management, ACTION: report trail modification>

OPERATION reportTrailModification {

INPUT_PARAMETERS

trail: TrailChoice;

layerND: LayerNetworkDomainChoice;

newTrafficDescriptor: ImmediateTrafficDescriptorId ;

newServiceCharacteristics: TrailServiceCharacteristicsId;

agreedTrafficDescriptor: ImmediateTrafficDescriptorId ;

agreedServiceCharacteristics: TrailServiceCharacteristicsId;

OUTPUT_PARAMETERS

-- none

RAISED EXCEPTIONS

-- none

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

trail: <INFORMATION OBJECT: tmTrail>;

layerND: <INFORMATION OBJECT: tmLayerNetworkDomain>;

newTrafficDescriptor: <INFORMATION OBJECT: tmTrafficDescriptor>;

newServiceCharacteristics: <INFORMATION OBJECT: tmTrailServiceCharacteristics>;

agreedTrafficDescriptor: <INFORMATION OBJECT: tmTrafficDescriptor>;

agreedServiceCharacteristics: <INFORMATION OBJECT: tmTrailServiceCharacteristics>;

TRIGGERING CONDITIONS

PRE_ CONDITIONS

inv_newServiceCharacteristics

"trail and newServiceCharacteristics must not be involved as transportQualifiedTrail and transportQualifierTSC in a <tmTrailHasTSC> relationship."

inv_newTrafficDescriptor

"trail and newTrafficDescriptor must not be involved as trailTD and immediateTrafficDescriptorTrail in a <tmTrailHasImmediateTrafficDescriptor> relationship."

POST_CONDITIONS

inv_agreedServiceCharacteristics

"trail and agreedServiceCharacteristics must be involved as transportQualifiedTrail and transportQualifierTSC in a <tmTrailHasTSC> relationship."

inv_agreedTrafficDescriptor

"trail and agreedTrafficDescriptor must be involved as trailTD and immediateTrafficDescriptorTrail in a <tmTrailHasImmediateTrafficDescriptor> relationship."

EXCEPTIONS

-- none

}

#### 7.3.1.4	Report networkTTP creation

<COMMUNITY: trail management , ACTION: report trail termination point creation>

OPERATION reportNetworkTTPCreation {

INPUT_PARAMETERS

networkTTP: NetworkTTPChoice;

layerND: LayerNetworkDomainChoice;

pointDir: PointDirectionality;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

-- none

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

networkTTP: <INFORMATION OBJECT: tmNetworkTTP>;

layerND: <INFORMATION OBJECT: tmLayerNetworkDomain>;

pointDir: <INFORMATION ATTRIBUTE: pointDirectionality>;

TRIGGERING_CONDITIONS

PRE_CONDITIONS

-- none

POST_CONDITIONS

inv_existingNetworkTTP

"networkTTP must refer to element in a <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

EXCEPTIONS

-- none

}

#### 7.3.1.5	Report networkTTP deletion

<COMMUNITY: trail management , ACTION: report trail termination point deletion>

OPERATION reportNetworkTTPDeletion {

INPUT_PARAMETERS

networkTTP: NetworkTTPChoice;

layerND: LayerNetworkDomainChoice;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

-- none

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

networkTTP: <INFORMATION OBJECT: tmNetworkTTP>;

layerND: <INFORMATION OBJECT: tmLayerNetworkDomain>;

TRIGGERING_CONDITIONS

PRE_CONDITIONS

inv_existingNetworkTTP

"networkTTP must refer to element in a <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_noTrailTermination

"networkTTP may not refer to a_endNTTP or z_endNTTP in any

<trailIsTerminatedByPointToPoint> relationship."

inv_noSubnetworkAssociation

"networkTTP shall not refer to extremity of a <subnetworkTPIsRelatedToExtremity> relationship where abstractionSNTP references elementSNTP of a <subnetworkIsDelimitedBy> relationship.

"

inv_noAccessGroupAssociation

"networkTTP may not refer to elementTTP in an <accessGroupIsMadeOfNetworkTTPs> relationship."

POST_CONDITIONS

inv_noNetworkTTP

"networkTTP does not refer to element in a <layerNetworkDomainIsMadeOf> relationship."

EXCEPTIONS

-- none

}

#### 7.3.1.6	Report association of networkTTP with access group

<COMMUNITY: trail management , ACTION: report association of trail termination point with access group>

OPERATION reportAssociationOfNetworkTTPWithAccessGroup {

INPUT_PARAMETERS

layerND: LayerNetworkDomainChoice;

accessGroup: AccessGroupChoice;

networkTTP: NetworkTTPChoice;

OUTPUT PARAMETERS

-- none

RAISED EXCEPTIONS

-- none

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: tmLayerNetworkDomain>;

accessGroup: <INFORMATION OBJECT: tmAccessGroup>;

networkTTP: <INFORMATION OBJECT: tmNetworkTTP>;

TRIGGERING_CONDITIONS

PRE_CONDITIONS

inv_networkTTPAndAccessGroupExistingAndCompatible

"networkTTP and accessGroup shall refer to element of the same <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_networkTTPNotAlreadyAssociated

"networkTTP shall not refer to elementTTP in any <accessGroupIsMadeOfNetworkTTPs> relationship."

POST_CONDITIONS

inv_networkTTPAssociated

"networkTTP refers to elementTTP of an <accessGroupIsMadeOfNetworkTTPs> relationship where accessGroup refers to containerAG."

EXCEPTIONS

-- none

}

#### 7.3.1.7	Report disassociation of networkTTP from access group

<COMMUNITY: trail management , ACTION: report disassociation of trail termination point from accessGroup>

OPERATION reportDisassociationOfNetworkTTPFromAccessGroup {

INPUT_PARAMETERS

layerND: LayerNetworkDomainChoice;

accessGroup: AccessGroupChoice;

networkTTP: NetworkTTPChoice;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

-- none

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT: tmLayerNetworkDomain>;

accessGroup: <INFORMATION OBJECT: tmAccessGroup>;

networkTTP: <INFORMATION OBJECT: tmNetworkTTP>;

TRIGGERING_CONDITIONS

PRE_CONDITIONS

inv_networkTTPAssociated

"networkTTP refers to elementTTP of an <accessGroupIsMadeOfNetworkTTPs> relationship where accessGroup refers to containerAG."

POST_CONDITIONS

inv_networkTTPDisassociated

"networkTTP shall not refer to elementTTP in an <accessGroupIsMadeOfNetworkTTPs> relationship where accessGroup refers to containerAG."

EXCEPTIONS

-- none

}

#### 7.3.1.8	Report association of networkTTP with subnetwork

<COMMUNITY: trail management , ACTION: report association of trail termination point with subnetwork>

OPERATION reportAssociationOfNetworkTTPWithSubnetwork {

INPUT_PARAMETERS

layerND: LayerNetworkDomainChoice;

subnetwork: SubnetworkChoice;

networkTTP: NetworkTTPChoice;

OUTPUT PARAMETERS

-- none

RAISED EXCEPTIONS

-- none

BEHAVIOUR

SEMI_FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT tmLayerNetworkDomain>;

subnetwork: <INFORMATION OBJECT tmSubnetwork>;

networkTTP: <INFORMATION OBJECT tmNetworkTTP>;

TRIGGERING_CONDITIONS

PRE_CONDITIONS

inv_networkTTPAndSubnetworkExistingAndCompatible

"networkTTP and subnetwork shall refer to element of the same <layerNetworkDomainIsMadeOf> relationship where layerND refers to containerLND."

inv_networkTTPNotAssociated

"networkTTP shall not refer to extremity of a <subnetworkTPIsRelatedToExtremity> relationship where abstractionSNTP references elementSNTP of a <subnetworkIsDelimitedBy> relationship.

"

POST_CONDITIONS

inv_networkTTPAssociated

"networkTTP refers to extremity of a <subnetworkTPIsRelatedToExtremity> relationship where abstractionSNTP, which is a subnetworkTP, is also elementSNTP of a <subnetworkIsDelimitedBy> relationship where subnetwork refers to containerSN."

EXCEPTIONS

-- none

}

#### 7.3.1.9	Report disassociation of networkTTP from subnetwork

<COMMUNITY: trail management , ACTION: report disassociation of trail termination point from subnetwork>

OPERATION reportDisassociationOfNetworkTTPFromSubnetwork {

INPUT_PARAMETERS

layerND: LayerNetworkDomainChoice;

subnetwork: SubnetworkChoice;

networkTTP: NetworkTTPChoice;

OUTPUT_PARAMETERS

-- none

RAISED_EXCEPTIONS

-- none

BEHAVIOUR

SEMI-FORMAL

PARAMETER_MATCHING

layerND: <INFORMATION OBJECT tmLayerNetworkDomain>;

subnetwork: <INFORMATION OBJECT tmSubnetwork>;

networkTTP: 	<INFORMATION OBJECT tmNetworkTTP>;

TRIGGERING_CONDITIONS

PRE_CONDITIONS

inv_networkTTPAssociated

"networkTTP refers to extremity of a <subnetworkTPIsRelatedToExtremity> relationship where abstractionSNTP, which is a subnetworkTP, is also elementSNTPof a <subnetworkIsDelimitedBy> relationship where subnetwork refers to containerSN.

"

inv_networkTTPNotConnected

"abstractionSNTP (which is a subnetworkTP) of a <subnetworkTPIsRelatedToExtremity> relationship where networkTTP refers to extremity, shall not reference a_endSNTP or z_endSNTP of a <subnetworkConnectionIsTerminatedByPointToPoint> relationship.

"

POST_CONDITIONS

inv_networkTTPNotAssociated

"networkTTP shall not refer to extremity of a <subnetworkTPIsRelatedToExtremity> relationship where abstractionSNTP references elementSNTP of a <subnetworkIsDelimitedBy> relationship."

EXCEPTIONS

-- none

}

## 7.4	ASN.1 supporting productions

In this specification, when an interface name is used within an ASN.1 production, the same label will be used, starting with a capital letter. The complete ASN.1 type definition for this query interface (e.g. use of ObjectIdentifier, INTEGER, ...) will be developed as part of the engineering viewpoint, with the concerned technology.

AccessGroupChoice ::= CHOICE {
	tmAccessGroupQueryifce		TmAccessGroupQueryIfce,
	userIdentifier				UserIdentifier } ;

ImmediateTrafficDescriptorId ::= TmTrafficDescriptorQueryIfce;

LayerNetworkDomainChoice ::= CHOICE {
	tmLayerNetworkDomainQueryIfce	TmLayerNetworkDomainQueryIfce, 
	userIdentifier					UserIdentifier };

NetworkTTPChoice ::= CHOICE {
	tmNetworkTTPQueryIfce		TmNetworkTTPQueryIfce,
	userIdentifier				UserIdentifier};

SequenceOfNetworkTTPChoice ::=	SEQUENCE OF {
	a_endNetworkTTP		NetworkTTPChoice,
	z_endNetworkTTP		NetworkTTPChoice };

SubnetworkChoice ::= CHOICE {
	tmSubnetworkQueryIfce		TmSubnetworkQueryIfce,
	userIdentifier				UserIdentifier};

TrailChoice ::= CHOICE {
	tmTrailQueryIfce TmTrailQueryIfce,
	userIdentifier			UserIdentifier};

TrailServiceCharacteristicsId ::= TmTrailServiceCharcteristisQueryIfce;

UserIdentifier ::= SimpleNameType;

| ITU-T RECOMMENDATIONS SERIES |  |
| --- | --- |
| Series A | Organization of the work of the ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure |
| Series Z | Languages and general software aspects for telecommunication systems |