|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

| ITU-T | G.854.8 |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU | (03/99) |

SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

Digital transmission systems – Digital networks – Management of transport network

Computational viewpoint for pre-provisioned adaptation management

ITU-T  Recommendation  G.854.8

(Previously  CCITT  Recommendation)

ITU-T  G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| For further details, please refer to ITU-T List of Recommendations. |  |
| --- | --- |
| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| INTERNATIONAL ANALOGUE CARRIER SYSTEM |  |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS |  |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TRANSMISSION SYSTEMS |  |
| TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| General aspects | G.800–G.809 |
| Design objectives for digital networks | G.810–G.819 |
| Quality and availability targets | G.820–G.829 |
| Network capabilities and functions | G.830–G.839 |
| SDH network characteristics | G.840–G.849 |
| Management of transport network | G.850–G.859 |
| SDH radio and satellite systems integration | G.860–G.869 |
| Optical transport networks | G.870–G.879 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |

ITU-T RECOMMENDATION G.854.8 COMPUTATIONAL viewpoint for pre-PROVISIONED adaptation management

Summary The objective of pre-provisioned adaptation management community is to provide link capacity to client-layer(s) from a server layer. This community should be used in the case where client transport entities can be provisioned inside the link during the adaptation management. This capability of having pre-provisioned client transport entities is available in technologies such as SDH or WDM.

Source ITU-T Recommendation G.854.8 was prepared by ITU-T Study Group 4 (1997-2000) and was approved under the WTSC Resolution No. 1 procedure on the 26th of March 1999.

FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation the term recognized operating agency (ROA) includes any individual, company, corporation or governmental organization that operates a public correspondence service. The terms Administration, ROA and public correspondence are defined in the Constitution of the ITU (Geneva, 1992).

INTELLECTUAL PROPERTY RIGHTS

The ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. The ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, the ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  1999

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Definitions		1

4	Abbreviations		2

5	Conventions		2

6	Label references		2

7	Interfaces		4

7.1	Query interfaces		4

7.2	Operational interfaces		5

7.2.1	Associate trail with topologicalLink		6

7.2.2	Disassociate trail from topologicalLink		8

7.2.3	Add capacity to link		10

7.2.4	Remove capacity from Link		12

7.2.5	Associate networkTTP with topologicalLinkEnd		14

7.2.6	Disassociate networkTTP from topologicalLinkEnd		16

7.2.7	Add capacity to LinkEnd		18

7.2.8	Remove capacity from LinkEnd		21

7.3	Report interfaces		22

7.3.1	Report associate trail with topologicalLink		23

7.3.2	Report disassociate trail with topologicalLink		23

7.3.3	Report add capacity to Link		24

7.3.4	Report remove capacity from Link		25

7.3.5	Report associate networkTTP with topologicalLinkEnd		25

7.3.6	Report disassociate networkTTP with topologicalLinkEnd		26

7.3.7	Report add capacity to LinkEnd		27

7.3.8	Report remove capacity from LinkEnd		27

7.4	ASN.1 supporting productions		28

Recommendation G.854.8

COMPUTATIONAL viewpoint for pre-PROVISIONED
adaptation management

(Geneva, 1999)

# 1	Scope

This computational viewpoint specification is related to the pre-provisioned adaptation management enterprise specification defined in Recommendation G.852.8 and the pre-provisioned adaptation management information specification defined in Recommendation G.853.8.

The computational design in clause 2 of this preliminary computational viewpoint specification covers the following enterprise communities and associated actions:

COMMUNITY pam "pre-provisioned adaptation management"

pam "assign server transport entity to client linking entity"

pam "deassign server transport entity from client linking entity"

pam "report assignment of server transport entity"

pam "report deassignment of server transport entity"

pam "provision capacity to client linking entity"

pam "remove capacity from client linking entity"

pam "report client linking entity capacity provisioning"

pam "report client linking entity capacity removal"

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[1]	ITU-T Recommendation G.851.1 (1996), Management of the transport network – Application of the RM-ODP framework.

[2]	ITU-T Recommendation G.853.1 (1999), Common elements of the information viewpoint for the management of a transport network.

[3]	ITU-T Recommendation G.852.8 (1999), Enterprise viewpoint for pre-provisioned adaptation management.

[4]	ITU-T Recommendation G.853.8 (1999), Information viewpoint for pre-provisioned adaptation management.

# 3	Definitions

None.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

ASN.1	Abstract Syntax Notation One

CTP	Connection Termination Point

Id	Identifier

Ifce	Interface

inv	invariant

layerND	layerNetwork Domain

LC	LinkConnection

ND	Network Domain

RM-ODP	Reference Model for Open Distributed Processing

# 5	Conventions

In order to increase the readability of the behaviour in the operations:

•	parameters are written in bold;

•	elements defined in the information specification are written in italic.

In this Recommendation, when an interface is used in an ASN.1 production, the same label will be used starting with a capital letter. The complete ASN.1 production for this Query interface (e.g. use of OBJECT IDENTIFIER, INTEGER, etc.) will be developed as part of the Engineering Viewpoint with the concerned technology.

# 6	Label references

Table 1/G.854.8 – Label references

| Full label reference | Local label reference |
| --- | --- |
| <"Rec. G.854.3", INTERFACE: commonReportResourceIfce> | commonReportResourceIfce |
| <"Rec. G.854.3", INTERFACE: commonResourceIfce> | commonResourceIfce |
| <"Rec. G.853.8", ATTRIBUTE: pamAvailableLinkCapacity> | pamAvailableLinkCapacity |
| <"Rec. G.853.8", INFORMATION_OBJECT: pamClientLayerNetworkDomain> | pamClientLayerNetworkDomain |
| <"Rec. G.853.8", INFORMATION_OBJECT: pamLinkConnection> | pamLinkConnection |
| <"Rec. G.853.8", INFORMATION_OBJECT: pamLinkEnd> | pamLinkEnd |
| <"Rec. G.853.8", INFORMATION_RELATIONSHIP: PamLinkEndHasNetworkCTPs> | pamLinkEndHasNetworkCTPs |
| <"Rec. G.853.8", INFORMATION_RELATIONSHIP: pamLinkIsTerminatedByLinkEnd> | pamLinkIsTerminatedByLinkEnd |
| <"Rec. G.853.8", ATTRIBUTE: pamMaxProvisionableCapacity> | pamMaxProvisionableCapacity |
| <"Rec. G.853.8", INFORMATION_OBJECT: pamNetworkCTP> | pamNetworkCTP |

Table 1/G.854.8 – Label references (concluded)

| Full label reference | Local label reference |
| --- | --- |
| <"Rec. G.853.8", INFORMATION_OBJECT: pamNetworkTTP> | pamNetworkTTP |
| <"Rec. G.853.8", ATTRIBUTE: pamPotentialLinkCapacity> | pamPotentialLinkCapacity |
| <"Rec. G.853.8", ATTRIBUTE: pamProvisionedLinkCapacity> | pamProvisionedLinkCapacity |
| <"Rec. G.853.8-xx", INFORMATION_OBJECT: pamServerLayerNetworkDomain> | pamServerLayerNetworkDomain |
| <"Rec. G.853.8", INFORMATION_OBJECT: pamSubnetwork> | pamSubnetwork |
| "Rec. G.853.8", INFORMATION_OBJECT: pamSubnetworkTP | pamSubnetworkTP |
| <"Rec. G.853.8", INFORMATION_OBJECT: pamTopologicalLink> | pamTopologicalLink |
| <"Rec. G.853.8", INFORMATION_OBJECT: pamTopologicalLinkEnd> | pamTopologicalLinkEnd |
| <"Rec. G.853.8", INFORMATION_OBJECT: pamTrail> | pamTrail |
| <"Rec. G.853.8", INFORMATION_RELATIONSHIP: pamTopologicalLinkIsSupportedByTrail> | pamTopologicalLinkIsSupportedByTrail |
| <"Rec. G.853.8", INFORMATION_RELATIONSHIP: pamTopologicalLinkEndIsSupportedByNetworkTTP> | pamTopologicalLinkEndIsSupportedByNetworkTTP |
| <"Rec. G.853.8", INFORMATION_RELATIONSHIP: pamLayerNetworkDomainIsMadeOf> | pamLayerNetworkDomainIsMadeOf |
| <"Rec. G.853.8", INFORMATION_RELATIONSHIP: pamLinkHasLinkConnections> | pamLinkHasLinkConnections |
| <"Rec. G.853.8", INFORMATION_RELATIONSHIP: pamLinkEndHasNetworkCTPs> | pamLinkEndHasNetworkCTPs |
| <"Rec. G.853.8", INFORMATION_RELATIONSHIP: pamNetworkTTPAdaptsNetworkCTP> | pamNetworkTTPAdaptsNetworkCTP |
| <"Rec. G.853.8", INFORMATION_RELATIONSHIP: pamSubnetworkIsDelimitedBy> | pamSubnetworkIsDelimitedBy |
| <"Rec. G.853.8", INFORMATION_RELATIONSHIP: pamSubnetworkTPIsRelatedToExtremity> | pamSubnetworkTPIsRelatedToExtremity |
| <"Rec. G.853.8", INFORMATION_RELATIONSHIP: pamLinkConnectionIsSupportedByTrail> | pamLinkConnectionIsSupportedByTrail |
| <"Rec. G.853.8", INFORMATION_RELATIONSHIP: pamLinkBinds> | pamLinkBinds |
| <"Rec. G.853.8", INFORMATION_RELATIONSHIP: pamLayerNetworkDomainCanServeLnds> | pamLayerNetworkDomainCanServeLnds |

| Full ASN.1 production reference | Local label reference |
| --- | --- |
| <"Rec. X.721: 1992: Attribute-ASN1Module": SimpleNameType> | SimpleNameType |

# 7	Interfaces

## 7.1	Query interfaces

This Recommendation refers to interfaces that allow get access to identification and properties of resources involved in the "pre-provisioned adaptation management community". As the invocation of contained operations does not modify any state, there is no interest to develop them explicitly. There exact signature will be developed as part of the engineering viewpoint, with the concerned technology. These interfaces are listed in Table 2 with the information they allow to access.

Table 2/G.854.8 – Correspondence between Interface names, 
Information objects, Attributes and relationships

| Interface name | Information object | Attributes and relationships |
| --- | --- | --- |
| pamLayerNetworkDomainQueryIfce | <pamlayerNetworkDomain> | <resourceId> <signalIdentification> <pamLayerNetworkDomainIsMadeOf, ROLE : element> <pamLayerNetworkDomainCanServeLnds, ROLE : Client> <pamLayerNetworkDomainCanServeLnds, ROLE : Server> |
| pamLinkConnectionQueryIfce | <pamLinkConnection> | <resourceId> <signalIdentification> <pamLayerNetworkDomainIsMadeOf, ROLE : containerND> <linkHasLinkConnections, ROLE : containerLink> <linkConnectionIsSupportedByTrail, ROLE : serverTrail> |
| pamLinkEndlQueryIfce | <pamLinkEnd> | <resourceId> <signalIdentification> <pamLinkEndHasNetworkCTPs, ROLE : elementCTP> <pamLinkIsTerminatedByLinkEnd, ROLE : transferCapacityLink> <pamTopologicalLinkEndIsSupportedByNetworkTTP, ROLE : server> <pamLayerNetworkDomainIsMadeOf, ROLE : containerND> <pamAvailableLinkCapacity> <pamMaxProvisionableCapacity> <pamPotentialLinkCapacity> <pamProvisionedLinkCapacity> |
| pamNetworkCTPQueryIfce | <pamNetworkCTP> | <resourceId> <signalIdentification> <pamLinkEndHasNetworkCTPs, ROLE : containerLE> <networkTTPAdaptsNetworkCTP, ROLE : serverTP> |
| pamNetworkTTPQueryfce | <pamNetworkTTP> | <resourceId> <signalIdentification> <pamTopologicalLinkEndIsSupportedByNetworkTTP, ROLE : client> <pamLayerNetworkDomainIsMadeOf, ROLE : containerND> <networkTTPAdaptsNetworkCTP, ROLE : clientTP > |

Table 2/G.854.8 – Correspondence between Interface names, 
Information objects, Attributes and relationships (concluded)

| Interface name | Information object | Attributes and relationships |
| --- | --- | --- |
| pamSubnetworkQueryIfce | <pamSubnetwork> | <resourceId> <signalIdentification> <pamSubnetworkIsDelimitedBy, ROLE : elementSNTP> |
| pamSubnetworkTPQueryIfce | <pamSubnetworkTP> | <resourceId> <signalIdentification> <pamSubnetworkIsDelimitedBy, ROLE : containerSN> <subnetworkTPIsRelatedToExtremity, ROLE : extremity> |
| pamTopologicalLinkQueryIfce | <pamTopologicalLink> | <resourceId> <signalIdentification> <pamTopologicalLinkIsSupportedByTrail, ROLE : serverTrail> <pamLayerNetworkDomainIsMadeOf, ROLE : element> <linkHasLinkConnections, ROLE : elementLC> <linkBinds, ROLE : a_endTopological> <linkBinds, ROLE : z_endTopological> <pamAvailableLinkCapacity> <pamMaxProvisionableCapacity> <pamPotentialLinkCapacity> <pamProvisionedLinkCapacity> |
| pamTrailQueryIfce | <pamTrail> | <resourceId> <signalIdentification> <pamTopologicalLinkIsSupportedByTrail,ROLE : clientTL> <pamLayerNetworkDomainIsMadeOf, ROLE : element> <linkConnectionIsSupportedByTrail, ROLE : clientLC> |

## 7.2	Operational interfaces

COMPUTATIONAL_INTERFACE <commonResourceIfce>

COMPUTATIONAL_INTERFACE preProvisionedAdaptationManagementArcIfce

OPERATION {

associateTrailWithTopologicalLink;

disassociateTrailFromTopologicalLink;

addCapacityToLink;

removeCapacityFromLink

}

COMPUTATIONAL_INTERFACE preProvisionedAdaptationManagementPointIfce

OPERATION {

associateNetworkTTPWithTopologicalLinkEnd;

disassociateNetworkTTPFromTopologicalLinkEnd;

addCapacityToLinkEnd;

removeCapacityFromLinkEnd

}

### 7.2.1	Associate trail with topologicalLink

<COMMUNITY: pre-provisioned adaptation management, ACTION: assign server transport entity to client linking entity>

OPERATION associateTrailWithTopologicalLink {

INPUT_PARAMETERS

link: LinkId;

clientLayerNetworkDomain: LayerNetworkDomainId

trail: TrailId;

OUTPUT_PARAMETERS

potentialCapacity: Capacity;

resultingLinkConnections: LinkConnectionList;

--   The resultingLinkConnections parameter value is provided when the

--   <PERMISSION:returnClientTransportEntities> is supported.

RAISED_EXCEPTIONS

incorrectLink: LinkId

incorrectTrail: TrailId

linkAndTrailsNotCompatible: NULL;

initialCapacitiesFailure: SEQUENCE {

availableLinkCapacity		Capacity;

maxProvisionableCapacity	Capacity;

potentialLinkCapacity		Capacity;

ProvisionedLinkCapacity 		Capacity};

trailAlreadyAssociated: NULL;

finalCapacitiesFailure: SEQUENCE {

availableLinkCapacity		Capacity;

maxProvisionableCapacity	Capacity;

potentialLinkCapacity		Capacity;

ProvisionedLinkCapacity		Capacity};

consistencyFailure: NULL;

failureToAssociate: NULL;

BEHAVIOUR

PARAMETER_MATCHING

link: <INFORMATION OBJECT:pamTopologicalLink>;

clientLayerNetworkDomain: <INFORMATION OBJECT:pamClientLayerNetworkDomain>;

trail: <INFORMATION OBJECT:pamTrail>;

potentialCapacity: <ATTRIBUTE:pamPotentialLinkCapacity>;

resultingLinkConnections ELEMENTS: <INFORMATION OBJECT:pamLinkConnection>

incorrectLink: <INFORMATION OBJECT:pamTopologicalLink>;

incorrectTrail: <INFORMATION OBJECT:pamTrail>;

availableLinkCapacity: <ATTRIBUTE:pamAvailableLinkCapacity>;

maxProvisionableCapacity: <ATTRIBUTE:pamMaxProvisionableCapacity>;

potentialLinkCapacity: <ATTRIBUTE:pamPotentialLinkCapacity>;

ProvisionedLinkCapacity: <ATTRIBUTE:pamProvisionedLinkCapacity>;

PRE_CONDITIONS

inv_LinkExists

"The link refers to element of a <pamClientlayerNetworkDomainIsMadeOf> where the container refers to clientLayerNetworkDomain."

inv_TrailExists

"The trail refers to element of a <pamServerlayerNetworkDomainIsMadeOf> where the container is <pamServerLayerNetworkDomain>."

inv_LNDConsistency

"The trail refers to element of a <pamServerlayerNetworkDomainIsMadeOf> where the container is server regarding a <layerNetworkDomainIsServedByLayerNetworkDomain> relationship. The client of this latter relationship is referred by Link."

inv_nullCapacityValues

"The attributes of <pamTopologicalLink> referred by link have the following values:

•	pamAvailableLinkCapacity equal to 0;

•	pamMaxProvisionableCapacity equal to 0;

•	pamPotentialLinkCapacity equal to 0;

•	pamProvisionedLinkCapacity equal to 0."

inv_trailNotAssociated

"The trail shall not refer to any server in any <pamTopologicalLinkIsSupportedByTrail> relationship where link refers to client."

POST_CONDITIONS

inv_finalCapacityValues

"The attributes of <pamTopologicalLink> referred by link have the following values:

•	pamPotentialLinkCapacity is equal to pamMaxProvisionableCapacity of the 
<pamTopologicalLink> minus pamProvisionedLinkCapacity of each other client in the <pamTopologicalLinkIsSupportedByTrail> relationship where trail refers to server;

•	pamMaxProvisionableCapacity is updated to its nominal value in accordance to the trail;

•	pamAvailableLinkCapacity equal to 0;

•	pamProvisionedLinkCapacity equal to 0."

inv_potentialCapacity

"The potentialCapacity value is equal to the <pamPotentialLinkCapacity> attribute value of the <pamTopologicalLink> referred by link."

inv_LNDConsistency

"The trail refers to element of a <pamServerlayerNetworkDomainIsMadeOf> where the container is server regarding a <layerNetworkDomainIsServedByLayerNetworkDomain> relationship. The client of this latter relationship is also container of a <pamClientlayerNetworkDomainIsMadeOf> relationship where link refers to element."

inv_trailAssociated

"The trail shall refer to a server in a <pamTopologicalLinkIsSupportedByTrail> relationship where link refers to client.";

EXCEPTIONS

IF PRE_CONDITION inv_LinkExists NOT_VERIFIED RAISE_EXCEPTION incorrectLink;

IF PRE_CONDITION inv_TrailExists NOT_VERIFIED RAISE_EXCEPTION incorrectTrail;

IF PRE_CONDITION inv_LNDConsistency NOT_VERIFIED RAISE_EXCEPTION linkAndTrailsNotCompatible;

IF PRE_CONDITION inv_trailNotAssociated NOT_VERIFIED RAISE_EXCEPTION trailAlreadyAssociated;

IF PRE_CONDITION inv_nullCapacityValues NOT_VERIFIED RAISE_EXCEPTION initialCapacitiesFailure;

IF POST_CONDITION inv_finalCapacityValues NOT_VERIFIED RAISE_EXCEPTION finalCapacitiesFailure;

IF POST_CONDITION inv_LNDConsistency NOT_VERIFIED RAISE_EXCEPTION consistencyFailure;

IF POST_CONDITION inv_trailAssociated NOT_VERIFIED RAISE_EXCEPTION failureToAssociate;

}

### 7.2.2	Disassociate trail from topologicalLink

<COMMUNITY: pre-provisioned adaptation management, ACTION: deassign server transport entity from client linking entity>

OPERATION disassociateTrailFromTopologicalLink {

INPUT_PARAMETERS

link: LinkId;

clientLayerNetworkDomain: LayerNetworkDomainId;

trail: TrailId;

OUTPUT_PARAMETERS

--   none

RAISED_EXCEPTIONS

incorrectLink: LinkId

incorrectTrail: TrailId

trailNotAssociated: NULL;

capacityProvisioned: Capacity;

finalCapacitiesFailure: SEQUENCE {

availableLinkCapacity		Capacity;

maxProvisionableCapacity	Capacity;

potentialLinkCapacity		Capacity;

provisionedLinkCapacity		Capacity};

failureToDisassociate: NULL;

BEHAVIOUR

PARAMETER_MATCHING

link: <INFORMATION OBJECT:pamTopologicalLink>;

clientLayerNetworkDomain: <INFORMATION OBJECT:pamClientLayerNetworkDomain>;

trail: <INFORMATION OBJECT:pamTrail>;

incorrectLink: <INFORMATION OBJECT:pamTopologicalLink>;

incorrectTrail: <INFORMATION OBJECT:pamTrail>;

capacityProvisioned: <ATTRIBUTE:pamProvisionedLinkCapacity>;

availableLinkCapacity: <ATTRIBUTE:pamAvailableLinkCapacity>;,

maxProvisionableCapacity: <ATTRIBUTE:pamMaxProvisionableCapacity>;

potentialLinkCapacity:<ATTRIBUTE:pamPotentialLinkCapacity>;

provisionedLinkCapacity:<ATTRIBUTE:pamProvisionedLinkCapacity>;

PRE_CONDITIONS

inv_LinkExists

"The link refers to element of a <pamClientlayerNetworkDomainIsMadeOf> where the container refers to clientLayerNetworkDomain."

inv_TrailExists

"The trail refers to element of a <pamServerlayerNetworkDomainIsMadeOf> where the container is <pamServerLayerNetworkDomain>."

inv_LNDConsistency

"The trail refers to element of a <pamServerlayerNetworkDomainIsMadeOf> where the container is server regarding a <layerNetworkDomainIsServedByLayerNetworkDomain> relationship. The client of this latter relationship is referred by Link."

inv_trailAssociated

"The trail shall refer to a server in a <pamTopologicalLinkIsSupportedByTrail> relationship where link refers to client."

inv_ pamProvisionedLinkCapacity

"The pamProvisionedLinkCapacity attribute value of the <pamTopologicalLink> referred by link is equal to 0.";

POST_CONDITIONS

inv_nullCapacityValues

The attributes of <pamTopologicalLink> referred by link have the following values:

•	pamAvailableLinkCapacity equal to 0;

•	pamMaxProvisionableCapacity equal to 0;

•	pamPotentialLinkCapacity equal to 0;

•	pamProvisionedLinkCapacity equal to 0."

inv_LNDConsistency

"The trail shall refer to element of a <pamServerlayerNetworkDomainIsMadeOf> where the container is server regarding a <layerNetworkDomainIsServedByLayerNetworkDomain>. The client of this latter relationship is also container of a <pamClientlayerNetworkDomainIsMadeOf> where link refers to element."

inv_trailNotAssociated

"The trail shall not refer to any server in any <pamTopologicalLinkIsSupportedByTrail> where link refers to client.";

EXCEPTIONS

IF PRE_CONDITION inv_LinkExists NOT_VERIFIED RAISE_EXCEPTION incorrectLink;

IF PRE_CONDITION inv_TrailExists NOT_VERIFIED RAISE_EXCEPTION incorrectTrail;

IF PRE_CONDITION inv_LNDConsistency NOT_VERIFIED RAISE_EXCEPTION linkAndTrailsNotCompatible;

IF PRE_CONDITION inv_trailAssociated NOT_VERIFIED RAISE_EXCEPTION trailAlreadyAssociated;

IF PRE_CONDITION inv_pamProvisionedLinkCapacity NOT_VERIFIED RAISE_EXCEPTION capacityProvisioned;

IF POST_CONDITION inv_trailNotAssociated NOT_VERIFIED RAISE_EXCEPTION failureToDissociate;

IF POST_CONDITION inv_nullCapacityValues NOT_VERIFIED RAISE_EXCEPTION finalCapacitiesFailure;

IF POST_CONDITION inv_LNDConsistency NOT_VERIFIED RAISE_EXCEPTION failureToDissociate;

### 7.2.3	Add capacity to link

<COMMUNITY: pre-provisioned adaptation management, ACTION: provision capacity to client linking entity>

OPERATION addCapacityToLink {

INPUT_PARAMETERS

link: LinkId;

clientLayerNetworkDomain: LayerNetworkDomainId

capacity: RequestedCapacity::= CHOICE{

requestedChannels	SEQUENCE OF {Channel};

requestedNumberOfLinkConnections	Capacity}

--   The requestedChannels sub-parameter value is provided when the

--   <PERMISSION: selectClientTransportEntities> is supported.

--   Channel indicates the channel number such as timeslot in SDH

OUTPUT_PARAMETERS

numberOfLinkConnections: Capacity;

resultingLinkConnections: LinkConnectionList;

RAISED_EXCEPTIONS

incorrectLink: LinkId

insufficientCapacity: Capacity;

invalidChannelsNumber: SEQUENCE OF {Channel};

channelsAlreadyProvisioned: SEQUENCE OF {Channel};

failureToCreateLCs: NULL;

failureToAssociateLCs: NULL;

failureToSupportLCs: NULL;

failureToIncreaseCapacity: SEQUENCE {

availableLinkCapacity		Capacity;

maxProvisionableCapacity	Capacity;

potentialLinkCapacity		Capacity;

provisionedLinkCapacity		Capacity};

BEHAVIOUR

PARAMETER_MATCHING

link: <INFORMATION OBJECT:pamTopologicalLink>;

clientLayerNetworkDomain: <INFORMATION OBJECT:pamClientLayerNetworkDomain>;

numberOfLinkConnections: <ATTRIBUTE:pamProvisionedLinkCapacity>;

resultingLinkConnections ELEMENTS: <INFORMATION OBJECT:pamLinkConnection>;

Channel: <INFORMATION OBJECT:pamLinkConnection>;

incorrectLink: <INFORMATION OBJECT:pamTopologicalLink>;

insufficientCapacity: <ATTRIBUTE:pamPotentialLinkCapacity>;

availableLinkCapacity: <ATTRIBUTE:pamAvailableLinkCapacity>;

maxProvisionableCapacity: <ATTRIBUTE:pamMaxProvisionableCapacity>;

potentialLinkCapacity:<ATTRIBUTE:pamPotentialLinkCapacity>;

provisionedLinkCapacity:<ATTRIBUTE:pamProvisionedLinkCapacity>;

PRE_CONDITIONS

inv_LinkExists

"The link refers to element of a <pamClientlayerNetworkDomainIsMadeOf> where the container refers to clientLayerNetworkDomain".

inv ChannelsNumber

"The Channel shall refer to a valid value with regard to the server of the client link."

inv_ChannelsDoNotExist

"The Channel shall not refer to any element in any <linkHasLinkConnections> where link refers to container."

inv_capacityExists

"The pamPotentialLinkCapacity attribute value of the <pamTopologicalLink> referred by link shall be greater or equal to the requestedNumberOfLinkConnections."

POST_CONDITIONS

inv_provisionedLinkCapacityIncrease

"The pamProvisionedLinkCapacity attribute value of the <pamTopologicalLink> referred by link is increased by the value of requestedNumberOfLinkConnections."

inv_availableLinkCapacityIncrease

"The pamAvailableLinkCapacity value of the <pamTopologicalLink> referred by link is increased by the value of requestedNumberOfLinkConnections."

inv_PotentialLinkCapacityDecrease

"The pamPotentialLinkCapacity value of all <pamTopologicalLink> involved in the same <pamTopologicalLinkIsSupportedByTrail> as the link is decreased by the value of requestedNumberOfLinkConnections (down to each associated characteristic information)."

inv_createdLC

"The resultingLinkConnections refer to element of the <linkHasLinkConnections> relationship where link refers to container."

inv_LCAssociated

"The resultingLinkConnections shall refer to extremity in two <subnetworkTPIsRelatedToExtremity> relationships. The associated abstraction are participating in two <subnetworkIsDelimitedBy> relationships where each container is the A_end and Z_end of a <linkBinds> relationship for which link refers to transferCapacity.

"

inv_LCSupported

"The resultingLinkConnections refer to client in a <linkConnectionIsSupportedByTrail> relationship where the trail refers to the server. This trail refers also to a server in a <TopologicalLinkIsSupportedByTrail> relationship where the link refers to client.

"

inv_mapping_requestedCapacityToCapacity

"The numberOfLinkConnections value is equal to the pamProvisionedLinkCapacity value of the <pamTopologicalLink> referred by link.";

EXCEPTIONS

IF PRE_CONDITION inv_LinkExists NOT_VERIFIED RAISE_EXCEPTION incorrectLink;IF PRE_CONDITION inv_capacityExists NOT_VERIFIED RAISE_EXCEPTION insufficientCapacity;

IF PRE_CONDITION inv ChannelNumber NOT_VERIFIED RAISE_EXCEPTION invalidChannelNumber;

IF PRE_CONDITION inv_ChannelsDoNotExist NOT_VERIFIED RAISE_EXCEPTION channelAlreadyProvisioned;

IF POST_CONDITION inv_createdLC NOT_VERIFIED RAISE_EXCEPTION failureToCreateLCs;

IF POST_CONDITION inv_LCAssociated NOT_VERIFIED RAISE_EXCEPTION failureToAssociateLCs;

IF POST_CONDITION inv_LCSupported NOT_VERIFIED RAISE_EXCEPTION failureToSupportLCs;

IF POST_CONDITION inv_provisonnedLinkCapacityIncrease NOT_VERIFIED RAISE_EXCEPTION failureToIncreaseCapacity;

IF POST_CONDITION inv_availableLinkCapacityIncrease NOT_VERIFIED RAISE_EXCEPTION failureToIncreaseCapacity;

IF POST_CONDITION inv_PotentialLinkCapacityDecrease NOT_VERIFIED RAISE_EXCEPTION failureToIncreaseCapacity;

}

### 7.2.4	Remove capacity from Link

<COMMUNITY: pre-provisioned adaptation management, ACTION: remove capacity from client linking entity>

OPERATION removeCapacityFromLink {

INPUT_PARAMETERS

link: LinkId;

clientLayerNetworkDomain: LayerNetworkDomainId

capacity: RequestedCapacity::= CHOICE{

requestedChannels	SEQUENCE OF {Channel};requestedNumberOfLinkConnections	Capacity}

--   The requestedChannels sub-parameter value is provided when the

--   <PERMISSION: selectClientTransportEntities> is supported.

--   Channel indicates the channel number such as timeslot in SDH

OUTPUT_PARAMETERS

provisionedLinkConnections: Capacity;

RAISED_EXCEPTIONS

incorrectLink: LinkId

insufficientCapacity: Capacity;

invalidChannelsNumber: SEQUENCE OF {channel};

failureToDecreaseCapacity: SEQUENCE {

availableLinkCapacity		Capacity,

maxProvisionableCapacity	Capacity,

potentialLinkCapacity		Capacity,

ProvisionedLinkCapacity		Capacity};

failureToRemoveLC: NULL;

BEHAVIOUR

PARAMETER_MATCHING

link: <INFORMATION OBJECT:pamTopologicalLink>;

clientLayerNetworkDomain: <INFORMATION OBJECT:pamClientLayerNetworkDomain>;

Channel: <INFORMATION OBJECT:pamLinkConnection>;

incorrectLink: <INFORMATION OBJECT:pamTopologicalLink>;

provisionedLinkConnections: <ATTRIBUTE:pamProvisionedLinkCapacity>;

insufficientCapacity: <ATTRIBUTE:pamProvisionedLinkCapacity>;

availableLinkCapacity: <ATTRIBUTE:pamAvailableLinkCapacity>;

maxProvisionableCapacity: <ATTRIBUTE:pamMaxProvisionableCapacity>;

potentialLinkCapacity:<ATTRIBUTE:pamPotentialLinkCapacity>;

provisionedLinkCapacity:<ATTRIBUTE:pamProvisionedLinkCapacity>;

PRE_CONDITIONS

inv_LinkExists

"The link refers to element of a <pamClientlayerNetworkDomainIsMadeOf> where the container refers to clientLayerNetworkDomain."

inv_capacityExists

"The pamAvailableLinkCapacity attribute value of the <pamTopologicalLink> referred by link is greater or equal to the requestedNumberOfLinkConnections value."

inv_linkConnectionPartOfLink

"Each Channel refers to element of <linkHasLinkConnections> where the container is referred by link."

POST_CONDITIONS

inv_LCRemoved

"The Channel shall not refer to any element in a <linkHasLinkConnections> relationship where the link refers to container."

inv_PotentialLinkCapacityIncrease

"The pamPotentialLinkCapacity value of all <pamTopologicalLink> involved in the same <pamTopologicalLinkIsSupportedByTrail> as the link are increased by the value of requestedNumberOfLinkConnections (down to each associated characteristic information)."

inv_provCapacityDecrease

"The pamProvisionedLinkCapacity attribute value of the <topologicalLink> referred by link has been decreased by the requestedNumberOfLinkConnections value."

inv_availCapacityDecrease

"The pamAvailableLinkCapacity attribute value of the <topologicalLink> referred by link has been decreased by the requestedNumberOfLinkConnections value."

inv_mappingToReturnedCapacity

"The provisionedLinkConnections value is equal to the new pamProvisionedLinkCapacity attribute value of the <topologicalLink> referred by link."

EXCEPTIONS

IF PRE_CONDITION inv_LinkExists NOT_VERIFIED RAISE_EXCEPTION incorrectLink;

IF PRE_CONDITION inv_capacityExists NOT_VERIFIED RAISE_EXCEPTION insufficientCapacity;

IF PRE_CONDITION inv_linkConnectionPartOfLink NOT_VERIFIED RAISE_EXCEPTION invalidChannelsNumber;

IF PRE_CONDITION inv_LCRemoved NOT_VERIFIED RAISE_EXCEPTION failureToRemoveLC;

IF POST_CONDITION inv_PotentialLinkCapacityIncrease NOT_VERIFIED RAISE_EXCEPTION failureToDecreaseCapacity;

IF POST_CONDITION inv_provCapacityDecrease NOT_VERIFIED RAISE_EXCEPTION failureToDecreaseCapacity;

IF POST_CONDITION inv_availCapacityDecrease NOT_VERIFIED RAISE_EXCEPTION failureToDecreaseCapacity;

}

### 7.2.5	Associate networkTTP with topologicalLinkEnd

<COMMUNITY: pre-provisioned adaptation management, ACTION: assign server transport entity to client linking entity>

OPERATION associatenetworkTTPWithTopologicalLinkEnd {

INPUT_PARAMETERS

link: LinkEndId;

clientLayerNetworkDomain: LayerNetworkDomainId;

networkTTP: NetworkTTPId;

OUTPUT_PARAMETERS

potentialCapacity: Capacity;

resultingNetworkCTPs: networkCTPList;

RAISED_EXCEPTIONS

incorrectLinkEnd: LinkEndId

incorrectNetworkTTP: NetworkTTPId

linkEndAndNetworkTTPsNotCompatible: NULL;

initialCapacitiesFailure: SEQUENCE {

availableLinkEndCapacity		Capacity,

maxProvisionableCapacity	Capacity,

potentialLinkEndCapacity:	Capacity,

provisionedLinkEndCapacity	Capacity};

networkTTPAlreadyAssociated: NULL;

finalCapacitiesFailure: SEQUENCE {

availableLinkEndCapacity 	Capacity,

maxProvisionableCapacity	Capacity,

potentialLinkEndCapacity		Capacity,

provisionedLinkEndCapacity	Capacity};

consistencyFailure: NULL;

failureToAssociate: NULL;

BEHAVIOUR

PARAMETER_MATCHING

linkEnd: <INFORMATION OBJECT:pamTopologicalLinkEnd>;

clientLayerNetworkDomain: <INFORMATION OBJECT:pamClientLayerNetworkDomain>;

networkTTP: <INFORMATION OBJECT:pamNetworkTTP>;

potentialCapacity: <ATTRIBUTE:pamPotentialLinkCapacity>;

resultingNetworkCTPs ELEMENTS: <INFORMATION OBJECT:pamNetworkCTP>

incorrectLinkEnd: <INFORMATION OBJECT:pamTopologicalLinkEnd>;

incorrectNetworkTTP: <INFORMATION OBJECT:pamNetworkTTP>;

availableLinkEndCapacity: <ATTRIBUTE:pamAvailableLinkCapacity>;

maxProvisionableCapacity: <ATTRIBUTE:pamMaxProvisionableCapacity>;

potentialLinkEndCapacity: <ATTRIBUTE:pamPotentialLinkCapacity>;

ProvisionedLinkEndCapacity:<ATTRIBUTE:pamProvisionedLinkCapacity>;

PRE_CONDITIONS

inv_LinkEndExists

"The linkEnd refers to element of a <pamClientlayerNetworkDomainIsMadeOf> where the container refers to clientLayerNetworkDomain."

inv_NetworkTTPExists

"The networkTTP refers to element of a <pamServerlayerNetworkDomainIsMadeOf> where the container is <pamServerLayerNetworkDomain>."

inv_LNDConsistency

"The networkTTP refers to element of a <pamServerlayerNetworkDomainIsMadeOf> where the container is server regarding a <layerNetworkDomainIsServedByLayerNetworkDomain> relationship. The client of this latter relationship is referred by LinkEnd."

inv_nullCapacityValues

"The attributes of <pamTopologicalLinkEnd> referred by linkEnd have the following values:

•	pamAvailableLinkCapacity equal to 0;

•	pamMaxProvisionableCapacity equal to 0;

•	pamPotentialLinkCapacity equal to 0;

•	pamProvisionedLinkCapacity equal to 0."

inv_networkTTPNotAssociated

"The networkTTP shall not refer to any server in any <pamTopologicalLinkEndIsSupportedByNetworkTTP> relationship where linkEnd refers to client."

POST_CONDITIONS

inv_finalCapacityValues

"The attributes of <pamTopologicalLinkEnd> referred by linkEnd have the following values:

•	pamPotentialLinkCapacity is equal to pamMaxProvisionableCapacity of the 
<pamTopologicalLinkEnd> minus pamProvisionedLinkCapacity of each other client in the <pamTopologicalLinkEndIsSupportedByNetworkTTP> relationship where networkTTP refers to server;

•	pamMaxProvisionableCapacity is updated to its nominal value in accordance to the networkTTP;

•	pamAvailableLinkCapacity equal to 0;

•	pamProvisionedLinkCapacity equal to 0."

inv_potentialCapacity

"The potentialCapacity value is equal to the <pamPotentialLinkCapacity> attribute value of the <pamTopologicalLinkEnd> referred by linkEnd."

inv_LNDConsistency

"The networkTTP refers to element of a <pamServerlayerNetworkDomainIsMadeOf> where the container is server regarding a <layerNetworkDomainIsServedByLayerNetworkDomain> relationship. The client of this latter relationship is also container of a <pamClientlayerNetworkDomainIsMadeOf> relationship where linkEnd refers to element."

inv_networkTTPAssociated

"The networkTTP shall refer to a server in a <pamTopologicalLinkEndIsSupportedByNetworkTTP> relationship where linkEnd refers to client."

EXCEPTIONS

IF PRE_CONDITION inv_LinkEndExists NOT_VERIFIED RAISE_EXCEPTION incorrectLinkEnd;

IF PRE_CONDITION inv_NetworkTTPExists NOT_VERIFIED RAISE_EXCEPTION incorrectNetworkTTP;

IF PRE_CONDITION inv_LNDConsistency NOT_VERIFIED RAISE_EXCEPTION linkAndNetworkTTPsNotCompatible;

IF PRE_CONDITION inv_networkTTPNotAssociated NOT_VERIFIED RAISE_EXCEPTION networkTTPAlreadyAssociated;

IF PRE_CONDITION inv_nullCapacityValues NOT_VERIFIED RAISE_EXCEPTION initialCapacitiesFailure;

IF POST_CONDITION inv_finalCapacityValue> NOT_VERIFIED RAISE_EXCEPTION finalCapacitiesFailure;

IF POST_CONDITION inv_LNDConsistency NOT_VERIFIED RAISE_EXCEPTION consistencyFailure;

IF POST_CONDITION inv_networkTTPAssociated NOT_VERIFIED RAISE_EXCEPTION failureToAssociate;

}

### 7.2.6	Disassociate networkTTP from topologicalLinkEnd

<COMMUNITY: pre-provisioned adaptation management, ACTION: deassign server transport entity from client linking entity>

OPERATION disassociateNetworkTTPFromTopologicalLinkEnd {

INPUT_PARAMETERS

linkEnd: LinkEndId;

clientLayerNetworkDomain: LayerNetworkDomainId;

networkTTP: NetworkTTPId;

OUTPUT_PARAMETERS

--   none

RAISED_EXCEPTIONS

incorrectLinkEnd: LinkEndId

incorrectNetworkTTP: NetworkTTPId

networkTTPNotAssociated: NULL;

capacityProvisioned: Capacity;

finalCapacitiesFailure: SEQUENCE {

availableLinkEndCapacity		Capacity,

maxProvisionableCapacity	Capacity,

potentialLinkEndCapacity		Capacity,

provisionedLinkEndCapacity	Capacity};

failureToDisassociate: NULL;

BEHAVIOUR

PARAMETER_MATCHING

linkEnd: <INFORMATION OBJECT:pamTopologicalLinkEnd>;

clientLayerNetworkDomain: <INFORMATION OBJECT:pamClientLayerNetworkDomain>;

networkTTP: <INFORMATION OBJECT:pamNetworkTTP>;

incorrectLinkEnd: <INFORMATION OBJECT:pamTopologicalLinkEnd>;

incorrectNetworkTTP: <INFORMATION OBJECT:pamNetworkTTP>;

capacityProvisioned: <ATTRIBUTE:pamProvisionedLinkCapacity>;

availableLinkEndCapacity: <ATTRIBUTE:pamAvailableLinkCapacity>;,

maxProvisionableCapacity: <ATTRIBUTE:pamMaxProvisionableCapacity>;

potentialLinkEndCapacity:<ATTRIBUTE:pamPotentialLinkCapacity>;

provisionedLinkEndCapacity:<ATTRIBUTE:pamProvisionedLinkCapacity>;

PRE_CONDITIONS

inv_LinkEndExists

"The linkEnd refers to element of a <pamClientlayerNetworkDomainIsMadeOf> where the container refers to clientLayerNetworkDomain."

inv_NetworkTTPExists

"The networkTTP refers to element of a <pamServerlayerNetworkDomainIsMadeOf> where the container is <pamServerLayerNetworkDomain>."

inv_LNDConsistency

"The networkTTP refers to element of a <pamServerlayerNetworkDomainIsMadeOf> where the container is server regarding a <layerNetworkDomainIsServedByLayerNetworkDomain> relationship. The client of this latter relationship is referred by LinkEnd."

inv_networkTTPAssociated

"The networkTTP shall refer to a server in a <pamTopologicalLinkEndIsSupportedByNetworkTTP> relationship where linkEnd refers to client."

inv_ pamProvisionedLinkCapacity

"The pamProvisionedLinkCapacity attribute value of the <pamTopologicalLinkEnd> referred by linkEnd is equal to 0."

POST_CONDITIONS

inv_nullCapacityValues

"The attributes of <pamTopologicalLinkEnd> referred by linkEnd have the following values:

•	pamAvailableLinkCapacity equal to 0;

•	pamMaxProvisionableCapacity equal to 0;

•	pamPotentialLinkCapacity equal to 0;

•	pamProvisionedLinkCapacity equal to 0."

inv_LNDConsistency

"The networkTTP shall refer to element of a <pamServerlayerNetworkDomainIsMadeOf> where the container is server regarding a <layerNetworkDomainIsServedByLayerNetworkDomain>. The client of this latter relationship is also container of a <pamClientlayerNetworkDomainIsMadeOf> where linkEnd refers to element."

inv_networkTTPNotAssociated

"The networkTTP shall not refer to any server in any <pamTopologicalLinkEndIsSupportedByNetworkTTP> where linkEnd refers to client.";

EXCEPTIONS

IF PRE_CONDITION inv_LinkEndExists NOT_VERIFIED RAISE_EXCEPTION incorrectLinkEnd;

IF PRE_CONDITION inv_NetworkTTPExists NOT_VERIFIED RAISE_EXCEPTION incorrectNetworkTTP;

IF PRE_CONDITION inv_LNDConsistency NOT_VERIFIED RAISE_EXCEPTION linkAndNetworkTTPsNotCompatible;

IF PRE_CONDITION inv_networkTTPAssociated NOT_VERIFIED RAISE_EXCEPTION networkTTPAlreadyAssociated;

IF PRE_CONDITION inv_pamProvisionedLinkCapacity NOT_VERIFIED RAISE_EXCEPTION capacityProvisioned;

IF POST_CONDITION inv_networkTTPNotAssociated NOT_VERIFIED RAISE_EXCEPTION failureToDissociate;

IF POST_CONDITION inv_nullCapacityValues NOT_VERIFIED RAISE_EXCEPTION finalCapacitiesFailure;

IF POST_CONDITION inv_LNDConsistency NOT_VERIFIED RAISE_EXCEPTION failureToDissociate;

}

### 7.2.7	Add capacity to LinkEnd

<COMMUNITY: pre-provisioned adaptation management, ACTION: provision capacity to client linking entity>

OPERATION addCapacityToLinkEnd {

INPUT_PARAMETERS

linkEnd: LinkEndId;

clientLayerNetworkDomain: LayerNetworkDomainId

capacity: ResquestedCapacity::= CHOICE{

requestedChannels	SEQUENCE OF {Channel};

requestedNumberOfNetworkCTPs	Capacity}

--   The requestedChannels sub-parameter value is provided when the

--   <PERMISSION: selectClientTransportEntities> is supported.

--   Channel indicates the channel number such as timeslot in SDH

OUTPUT_PARAMETERS

numberOfnetworkCTPs: Capacity;

resultingnetworkCTPs: networkCTPList

RAISED_EXCEPTIONS

incorrectLinkEnd: LinkEndId

insufficientCapacity: Capacity;

invalidChannelsNumber: SEQUENCE OF {Channel};

channelsAlreadyProvisioned: SEQUENCE OF {Channel};

failureToCreateLCs: NULL;

failureToAssociateLCs: NULL;

failureToSupportLCs: NULL;

failureToIncreaseCapacity: SEQUENCE {

availableLinkEndCapacity		Capacity,

maxProvisionableCapacity	Capacity,

potentialLinkEndCapacity		Capacity,

ProvisionedLinkEndCapacity	Capacity};

BEHAVIOUR

PARAMETER_MATCHING

linkEnd: <INFORMATION OBJECT:pamTopologicalLinkEnd>;

clientLayerNetworkDomain: <INFORMATION OBJECT:pamClientLayerNetworkDomain>;

numberOfNetworkCTP: <ATTRIBUTE:pamProvisionedLinkCapacity>;

resultingNetworkCTPs ELEMENTS: <INFORMATION OBJECT:pamLinkEndConnection>;

Channel: <INFORMATION OBJECT:pamLinkEndConnection>;

incorrectLinkEnd: <INFORMATION OBJECT:pamTopologicalLinkEnd>;

insufficientCapacity: <ATTRIBUTE:pamPotentialLinkCapacity>;

availableLinkEndCapacity: <ATTRIBUTE:pamAvailableLinkCapacity>;

maxProvisionableCapacity: <ATTRIBUTE:pamMaxProvisionableCapacity>;

potentialLinkEndCapacity:<ATTRIBUTE:pamPotentialLinkCapacity>;

provisionedLinkEndCapacity:<ATTRIBUTE:pamProvisionedLinkCapacity>;

PRE_CONDITIONS

inv_LinkEndExists

"The linkEnd refers to element of a <pamClientlayerNetworkDomainIsMadeOf> where the container refers to clientLayerNetworkDomain."

inv ChannelsNumber

"The Channel shall refer to a valid value with regard to the server of the client linkEnd."

inv_ChannelsDoNotExist

"The Channel shall not refer to any element in any <linkHasNetworkCTP> where linkEnd refers to container."

inv_capacityExists

"The pamPotentialLinkCapacity attribute value of the <pamTopologicalLinkEnd> referred by linkEnd shall be greater or equal to the requestedNumberOfNetworkCTPs."

POST_CONDITIONS

inv_provisionedLinkEndCapacityIncrease

"The pamProvisionedLinkCapacity attribute value of the <pamTopologicalLinkEnd> referred by linkEnd is increased by the value of requestedNumberOfNetworkCTPs."

inv_availableLinkEndCapacityIncrease

"The pamAvailableLinkCapacity value of the <pamTopologicalLinkEnd> referred by linkEnd is increased by the value of requestedNumberOfNetworkCTsP."

inv_PotentialLinkEndCapacityDecrease

"The pamPotentialLinkCapacity value of all <pamTopologicalLinkEnd> involved in the same <pamLinkEndIsSupportedByTrail > as the linkEnd is decreased by the value of requestedNumberOfNetworkCTPs (down to each associated characteristic information)."

inv_createdLC

"The resultingNetworkCTPs refers to element of the <linkHasNetworkCTP> relationship where linkEnd refers to container."

inv_LCAssociated

"The resultingNetworkCTPs shall refer to extremity in two <subnetworkTPIsRelatedToExtremity> relationships. The associated abstraction are participating in two <subnetworkIsDelimitedBy> relationships where each container is the A_end and Z_end of a <linkBinds> relationship for which linkEnd refers to transferCapacity.

"

inv_LCSupported

"The resultingNetworkCTPs refer to client in a <networkCTPIsSupportedByNetworkTTP> relationship where the networkTTP refers to the server. This networkTTP refers also to a server in a <linkIsSupportedByNetworkTTP> relationship where the linkEnd refers to client.

"

inv_mapping_requestedCapacityToCapacity

"The numberOfNetworkCTP value is equal to the pamProvisionedLinkCapacity value of the <pamTopologicalLinkEnd> referred by linkEnd."

EXCEPTIONS

IF PRE_CONDITION inv_LinkEndExists NOT_VERIFIED RAISE_EXCEPTION incorrectLinkEnd;

IF PRE_CONDITION inv_capacityExists NOT_VERIFIED RAISE_EXCEPTION insufficientCapacity;

IF PRE_CONDITION inv_ChannelNumber NOT_VERIFIED RAISE_EXCEPTION invalidChannelNumber;

IF PRE_CONDITION inv_ChannelsDoNotExist NOT_VERIFIED RAISE_EXCEPTION channelAlreadyProvisioned;

IF POST_CONDITION inv_createdLC NOT_VERIFIED RAISE_EXCEPTION failureToCreateLCs;

IF POST_CONDITION inv_LCAssociated NOT_VERIFIED RAISE_EXCEPTION failureToAssociateLCs;

IF POST_CONDITION inv_LCSupported NOT_VERIFIED RAISE_EXCEPTION failureToSupportLCs;

IF POST_CONDITION inv_provisonnedLinkEndCapacityIncrease NOT_VERIFIED RAISE_EXCEPTION failureToIncreaseCapacity;

IF POST_CONDITION inv_availableLinkEndCapacityIncrease NOT_VERIFIED RAISE_EXCEPTION failureToIncreaseCapacity;

IF POST_CONDITION inv_PotentialLinkEndCapacityDecrease NOT_VERIFIED RAISE_EXCEPTION failureToIncreaseCapacity;

}

### 7.2.8	Remove capacity from LinkEnd

<COMMUNITY: pre-provisioned adaptation management, ACTION: remove capacity from client linking entity>

OPERATION removeCapacityFromLinkEnd {

INPUT_PARAMETERS

linkEnd: LinkEndId;

clientLayerNetworkDomain: LayerNetworkDomainId

capacity: RequestedCapacity::= CHOICE{

requestedChannels	SEQUENCE OF {Channel};

requestedNumberOfNetworkCTP	Capacity}

--   The requestedChannels sub-parameter value is provided when the

--   <PERMISSION: selectClientTransportEntities> is supported.

--   Channel indicates the channel number such as timeslot in SDH

OUTPUT_PARAMETERS

provisionedNetworkCTPs: Capacity;

RAISED_EXCEPTIONS

incorrectLinkEnd: LinkEndId

insufficientCapacity: Capacity;

invalidChannelsNumber: SEQUENCE OF {Channel};

failureToDecreaseCapacity: SEQUENCE {

availableLinkEndCapacity		Capacity,

maxProvisionableCapacity	Capacity,

potentialLinkEndCapacity		Capacity,

provisionedLinkEndCapacity	Capacity};

failureToRemoveLC: NULL;

BEHAVIOUR

PARAMETER_MATCHING

linkEnd: <INFORMATION OBJECT:pamTopologicalLinkEnd>;

clientLayerNetworkDomain: <INFORMATION OBJECT:pamClientLayerNetworkDomain>;

Channel: <INFORMATION OBJECT:pamNetworkCTP>;

provisionedNetworkCTPs: <ATTRIBUTE:pamProvisionedLinkCapacity>;

incorrectLinkEnd: <INFORMATION OBJECT:pamTopologicalLinkEnd>;

insufficientCapacity: <ATTRIBUTE:pamProvisionedLinkCapacity>;

availableLinkEndCapacity: <ATTRIBUTE:pamAvailableLinkCapacity>;

maxProvisionableCapacity: <ATTRIBUTE:pamMaxProvisionableCapacity>;

potentialLinkEndCapacity:<ATTRIBUTE:pamPotentialLinkCapacity>;

provisionedLinkEndCapacity:<ATTRIBUTE:pamProvisionedLinkCapacity>;

PRE_CONDITIONS

inv_LinkEndExists

"The linkEnd refers to element of a <pamClientlayerNetworkDomainIsMadeOf> where the container refers to clientLayerNetworkDomain."

inv_capacityExists

"The pamAvailableLinkCapacity attribute value of the <pamTopologicalLinkEnd> referred by linkEnd is greater or equal to the requestedNumberOfNetworkCTPs value."

inv_networkCTPPartOfLinkEnd

"Each Channel refers to element of <linkHasNetworkCTP> where the container is referred by linkEnd."

POST_CONDITIONS

inv_LCRemoved

"The Channel shall not refer to any element in a <linkHasNetworkCTP> relationship where the linkEnd refers to container."

inv_PotentialLinkEndCapacityIncrease

"The pamPotentialLinkCapacity value of all <pamTopologicalLinkEnd> involved in the same <pamTopologicalLinkEndIsSupportedByNetworkTTP> as the linkEnd is increased by the value of requestedNumberOfNetworkCTPs (down to each associated characteristic information)."

inv_provCapacityDecrease

"The pamProvisionedLinkCapacity attribute value of the <topologicalLinkEnd> referred by linkEnd has been decreased by the requestedNumberOfNetworkCTPs value."

inv_availCapacityDecrease

"The pamAvailableLinkCapacity attribute value of the <topologicalLinkEnd> referred by linkEnd has been decreased by the requestedNumberOfNetworkCTPs value."

inv_mappingToReturnedCapacity

"The provisionedNetworkCTPs value is equal to the new pamProvisionedLinkCapacity attribute value of the <topologicalLinkEnd> referred by linkEnd."

EXCEPTIONS

IF PRE_CONDITION inv_LinkEndExists NOT_VERIFIED RAISE_EXCEPTION incorrectLinkEnd;

IF PRE_CONDITION inv_capacityExists NOT_VERIFIED RAISE_EXCEPTION insufficientCapacity;

IF PRE_CONDITION inv_networkCTPPartOfLinkEnd NOT_VERIFIED RAISE_EXCEPTION invalidChannelsNumber;

IF PRE_CONDITION inv_LCRemoved NOT_VERIFIED RAISE_EXCEPTION failureToRemoveLC;

IF POST_CONDITION inv_PotentialLinkEndCapacityIncrease NOT_VERIFIED RAISE_EXCEPTION failureToDecreaseCapacity;

IF POST_CONDITION inv_provCapacityDecrease NOT_VERIFIED RAISE_EXCEPTION failureToDecreaseCapacity;

IF POST_CONDITION inv_availCapacityDecrease NOT_VERIFIED RAISE_EXCEPTION failureToDecreaseCapacity;

}

## 7.3	Report interfaces

COMPUTATIONAL_INTERFACE <commonReportResourceIfce>

COMPUTATIONAL_INTERFACE preProvisionedAdaptationManagementArcReportIfce

OPERATION {

ReportAssociateTrailWithTopologicalLink;

reportDisassociateTrailFromTopologicalLink;

reportAddCapacityToLink;

reportRemoveCapacityFromLink

}

COMPUTATIONAL_INTERFACE preProvisionedAdaptationManagementPointReportIfce

OPERATION {

ReportAssociateNetworkTTPWithTopologicalLinkEnd;

ReportDisassociateNetworkTTPFromTopologicalLinkEnd;

ReportAddCapacityToLinkEnd;

reportRemoveCapacityFromLinkEnd

}

### 7.3.1	Report associate trail with topologicalLink

<COMMUNITY: pre-provisioned adaptation management, ACTION: report assignment of server transport entity>

OPERATION reportAssociateTrailWithTopologicalLink {

INPUT_PARAMETERS

link: LinkId;

clientLayerNetworkDomain: LayerNetworkDomainId;

trail: TrailId;

OUTPUT_PARAMETERS

--   none

RAISED EXCEPTIONS

--   none

BEHAVIOUR

PARAMETER_MATCHING

link: <INFORMATION OBJECT:pamTopologicalLink>;

clientLayerNetworkDomain: <INFORMATION OBJECT:pamClientLayerNetworkDomain>;

trail: <INFORMATION OBJECT:pamTrail>;

TRIGGER CONDITION

PRE_CONDITIONS

inv_trailNotAssociated

"The trail shall not refer to server in any <pamTopologicalLinkIsSupportedByTrail> where link refers to client."

POST_CONDITIONS

inv_trailAssociated

"The trail refers to server in a <pamTopologicalLinkIsSupportedByTrail> relationship where link refers to client."

}

### 7.3.2	Report disassociate trail with topologicalLink

<COMMUNITY: pre-provisioned adaptation management, ACTION: report deassignment of server transport entity>

OPERATION reportDisassociateTrailWithTopologicalLink {

INPUT_PARAMETERS

link: LinkId;

clientLayerNetworkDomain: LayerNetworkDomainId;

trail: TrailId;

OUTPUT_PARAMETERS

--   none

RAISED EXCEPTIONS

--   none

BEHAVIOUR

PARAMETER_MATCHING

link: <INFORMATION OBJECT:pamTopologicalLink>;

clientLayerNetworkDomain: <INFORMATION OBJECT:pamClientLayerNetworkDomain>;

trail: <INFORMATION OBJECT:pamTrail>;

TRIGGER CONDITION

PRE_CONDITIONS

inv_trailAssociated

"The trail refers a server in a <pamTopologicalLinkIsSupportedByTrail> relationship where link refers to client."

POST_CONDITIONS

inv_trailNotAssociated

"The trail shall not refer to server in any <pamTopologicalLinkIsSupportedByTrail> where link refers to client."

}

### 7.3.3	Report add capacity to Link

<COMMUNITY: pre-provisioned adaptation management, ACTION: report client linking entity capacity provisioning>

OPERATION reportAddCapacityToLink {

INPUT_PARAMETERS

link: LinkId;

clientLayerNetworkDomain: LayerNetworkDomainId;

numberOfLinkConnections: Capacity;

resultingLinkConnections: LinkConnectionList;

OUTPUT_PARAMETERS

--   none

RAISED EXCEPTIONS

--   none

BEHAVIOUR

PARAMETER_MATCHING

link: <INFORMATION OBJECT:pamTopologicalLink>;

clientLayerNetworkDomain: <INFORMATION OBJECT:pamClientLayerNetworkDomain>;

numberOfLinkConnections: <ATTRIBUTE:pamProvisionedLinkCapacity>;

resultingLinkConnections ELEMENTS: <INFORMATION OBJECT:pamLinkConnection>;

TRIGGERING CONDITION

PRE_CONDITIONS

inv_provisionedLinkCapacity

"The pamProvisionedLinkCapacity attribute value of the <pamTopologicalLink> referred by involvedLink is a valid value>."

POST_CONDITIONS

inv_provisionedLinkCapacityIncrease

"The pamProvisionedLinkCapacity attribute value of the <pamTopologicalLink> referred by involvedLink has increased comparing to its value in the PRE_CONDITION>."

}

### 7.3.4	Report remove capacity from Link

<COMMUNITY: pre-provisioned adaptation management, ACTION: report client linking entity capacity removal>

OPERATION reportRemoveCapacityFromLink {

INPUT_PARAMETERS

link: LinkId;

clientLayerNetworkDomain: LayerNetworkDomainId;

capacity: RequestedCapacity::= CHOICE{

requestedChannels	SEQUENCE OF {Channel};

--  channel indicates the channel number such as timeslot in SDH

requestedNumberOfLinkConnections	Capacity}

OUTPUT_PARAMETERS

--   none

RAISED EXCEPTIONS

--   none

BEHAVIOUR

PARAMETER_MATCHING

link: <INFORMATION OBJECT:pamTopologicalLink>;

clientLayerNetworkDomain: <INFORMATION OBJECT:pamClientLayerNetworkDomain>;

requestedChannels ELEMENTS: <INFORMATION OBJECT:pamLinkConnection>;

requestedNumberOfLinkConnections: <ATTRIBUTE:pamProvisionedLinkCapacity>;

TRIGGERING CONDITION

PRE_CONDITIONS

--   none

POST_CONDITIONS

inv_provisionedLinkCapacityDecrease

"The pamProvisionedLinkCapacity attribute value of the pamTopologicalLink referred by link has decreased."

}

### 7.3.5	Report associate networkTTP with topologicalLinkEnd

<COMMUNITY: pre-provisioned adaptation management, ACTION: report assignment of server transport entity>

OPERATION reportAssociatenetworkTTPWithTopologicalLinkEnd {

INPUT_PARAMETERS

linkEnd: LinkEndId;

clientLayerNetworkDomain: LayerNetworkDomainId;

networkTTP: NetworkTTPId;

OUTPUT_PARAMETERS

--   none

RAISED EXCEPTIONS

--   none

BEHAVIOUR

PARAMETER_MATCHING

linkEnd: <INFORMATION OBJECT:pamTopologicalLinkEnd>;

clientLayerNetworkDomain: <INFORMATION OBJECT:pamClientLayerNetworkDomain>;

networkTTP: <INFORMATION OBJECT:pamNetworkTTP>;

TRIGGER CONDITION

PRE_CONDITIONS

inv_networkTTPNotAssociated

"The networkTTP shall not refer to server in any <pamTopologicalLinkEndIsSupportedByNetworkTTP> where linkEnd refers to client."

POST_CONDITIONS

inv_networkTTPAssociated

"The networkTTP refers to server in a <pamTopologicalLinkEndIsSupportedByNetworkTTP> relationship where linkEnd refers to client."

}

### 7.3.6	Report disassociate networkTTP with topologicalLinkEnd

<COMMUNITY: pre-provisioned adaptation management, ACTION: report deassignment of server transport entity>

OPERATION reportDisassociatenetworkTTPWithTopologicalLinkEnd {

INPUT_PARAMETERS

linkEnd: LinkEndId;

clientLayerNetworkDomain: LayerNetworkDomainId;

networkTTP: NetworkTTPId;

OUTPUT_PARAMETERS

--   none

RAISED EXCEPTIONS

--   none

BEHAVIOUR

PARAMETER_MATCHING

linkEnd: <INFORMATION OBJECT:pamTopologicalLinkEnd>;

clientLayerNetworkDomain: <INFORMATION OBJECT:pamClientLayerNetworkDomain>;

networkTTP: <INFORMATION OBJECT:pamNetworkTTP>;

TRIGGER CONDITION

PRE_CONDITIONS

inv_networkTTPAssociated

"The networkTTP refers a server in a <pamTopologicalLinkEndIsSupportedByNetworkTTP> relationship where linkEnd refers to client."

POST_CONDITIONS

inv_networkTTPNotAssociated

"The networkTTP shall not refer to server in any <pamTopologicalLinkEndIsSupportedByNetworkTTP> where linkEnd refers to client."

}

### 7.3.7	Report add capacity to LinkEnd

<COMMUNITY: pre-provisioned adaptation management, ACTION: report client linking entity capacity provisioning>

OPERATION reportAddCapacityToLinkEnd {

INPUT_PARAMETERS

linkEnd: LinkEndId;

clientLayerNetworkDomain: LayerNetworkDomainId

numberOfNetworkCTPs: Capacity;

resultingNetworkCTPs: NetworkCTPsList;

OUTPUT_PARAMETERS

--   none

RAISED EXCEPTIONS

--   none

BEHAVIOUR

PARAMETER_MATCHING

linkEnd: <INFORMATION OBJECT:pamTopologicalLinkEnd>;

clientLayerNetworkDomain: <INFORMATION OBJECT:pamClientLayerNetworkDomain>;

numberOfNetworkCTPs: <ATTRIBUTE:pamProvisionedLinkCapacity>;

resultingNetworkCTPs ELEMENTS: <INFORMATION OBJECT:pamNetworkCTP>;

TRIGGERING CONDITION

PRE_CONDITIONS

inv_provisionedLinkEndCapacity

"The pamProvisionedLinkCapacity attribute value of the <pamTopologicalLinkEnd> referred by involvedLinkEnd is a valid value>."

POST_CONDITIONS

inv_provisionedLinkEndCapacityIncrease

"The pamProvisionedLinkCapacity attribute value of the <pamTopologicalLinkEnd> referred by involvedLinkEnd has increased comparing to its value in the PRE_CONDITION>."

}

### 7.3.8	Report remove capacity from LinkEnd

<COMMUNITY: pre-provisioned adaptation management, ACTION: report client linking entity capacity removal>

OPERATION reportRemoveCapacityFromLinkEnd {

INPUT_PARAMETERS

linkEnd: LinkEndId;

clientLayerNetworkDomain: LayerNetworkDomainId

capacity: RequestedCapacity::= CHOICE{

requestedChannels	SEQUENCE OF {Channel};

--  channel indicates the channel number such as timeslot in SDH

requestedNumberOfNetworkCTPs	Capacity}

OUTPUT_PARAMETERS

--   none

RAISED EXCEPTIONS

--   none

BEHAVIOUR

PARAMETER_MATCHING

linkEnd: <INFORMATION OBJECT:pamTopologicalLinkEnd>;

clientLayerNetworkDomain: <INFORMATION OBJECT:pamClientLayerNetworkDomain>;

requestedChannels ELEMENTS: <INFORMATION OBJECT:pamNetworkCTP>;

requestedNumberOfNetworkCTPs: <ATTRIBUTE:pamProvisionedLinkCapacity>;

TRIGGERING CONDITION

PRE_CONDITIONS

--   none

POST_CONDITIONS

inv_provisionedLinkEndCapacityDecrease

"The pamProvisionedLinkCapacity attribute value of the pamTopologicalLinkEnd referred by linkEnd has decreased>."

}

## 7.4	ASN.1 supporting productions

In this Recommendation, when an interface name is used within an ASN.1 production, the same label will be used starting with a capital letter. The complete ASN.1 type definition for this query interface (e.g. OBJECT IDENTIFIER, INTEGER, ...) will be developed as part of engineering viewpoint with the concerned technology.

Capacity::= INTEGER;

Channel::= SimpleNameType

LayerNetworkDomain::= CHOICE {

layerNetworkDomainIfce LayerNetworkDomainIfce,

userIdentifier SimpleNameType };

LinkConnectionList::= CHOICE {

linkConnectionQueryIfceList 		SEQUENCE OF {LinkConnectionQueryIfce},

channelList					SEQUENCE OF {Channel}};

LinkEndId::= CHOICE {

linkEndQueryIfce LinkEndQueryIfce,

userIdentifier SimpleNameType };

LinkId::= CHOICE {

linkQueryIfce LinkQueryIfce,

userIdentifier SimpleNameType };

networkCTPList::= CHOICE {

networkCTPQueryIfceList	SEQUENCE OF{NetworkCTPQueryIfce};

channelList				SEQUENCE OF{Channel}

NetworkTTPId::= CHOICE {

networkTTPQueryIfce NetworkTTPQueryIfce,

userIdentifier SimpleNameType };

TrailId::= CHOICE {

trailQueryIfce TrailQueryIfce,

userIdentifier SimpleNameType };

| ITU-T RECOMMENDATIONS SERIES |  |
| --- | --- |
| Series A | Organization of the work of the ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |