|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

| ITU-T | G.962 |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU | Amendment 1 (06/97) |

SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

Digital transmission systems – Digital sections and digital line system – Digital section and digital transmission systems for customer access to ISDN

Access digital section for ISDN primary rate at 2048 kbit/s

Amendment 1: Maintenance channel

ITU-T  Recommendation  G.962  –  Amendment 1

(Previously  CCITT  Recommendation)

ITU-T  G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| For further details, please refer to ITU-T List of Recommendations. |  |
| --- | --- |
| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| INTERNATIONAL ANALOGUE CARRIER SYSTEM |  |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TRANSMISSION SYSTEMS |  |
| TERMINAL EQUIPMENTS | G.700–G.799 |
| General | G.700–G.709 |
| Coding of analogue signals by pulse code modulation | G.710–G.719 |
| Coding of analogue signals by methods other than PCM | G.720–G.729 |
| Principal characteristics of primary multiplex equipment | G.730–G.739 |
| Principal characteristics of second order multiplex equipment | G.740–G.749 |
| Principal characteristics of higher order multiplex equipment | G.750–G.759 |
| Principal characteristics of transcoder and digital multiplication equipment | G.760–G.769 |
| Operations, administration and maintenance features of transmission equipment | G.770–G.779 |
| Principal characteristics of multiplexing equipment for the synchronous digital hierarchy | G.780–G.789 |
| Other terminal equipment | G.790–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| General aspects | G.800–G.809 |
| Design objectives for digital networks | G.810–G.819 |
| Quality and availability targets | G.820–G.829 |
| Network capabilities and functions | G.830–G.839 |
| SDH network characteristics | G.840–G.849 |
| Telecommunications management network | G.850–G.859 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| General | G.900–G.909 |
| Parameters for optical fibre cable systems | G.910–G.919 |
| Digital sections at hierarchical bit rates based on a bit rate of 2048 kbit/s | G.920–G.929 |
| Digital line transmission systems on cable at non-hierarchical bit rates | G.930–G.939 |
| Digital line systems provided by FDM transmission bearers | G.940–G.949 |
| Digital line systems | G.950–G.959 |
| Digital section and digital transmission systems for customer access to ISDN | G.960–G.969 |
| Optical fibre submarine cable systems | G.970–G.979 |
| Optical line systems for local and access networks | G.980–G.999 |

ITU-T RECOMMENDATION G.962 access digital section for isdn primary rate at 2048 kbit/s AMENDMENT 1 Maintenance channel

Source Amendment 1 to ITU-T Recommendation G.962, was prepared by ITU-T Study Group 13 (1997-2000) and was approved under the WTSC Resolution No. 1 procedure on the 20th of June 1997.

FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

The ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. The ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, the ITU had/had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  1997

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Page

C.7	Maintenance channel		1

C.7.1	Introduction		1

C.7.2	Maintenance channel layered structure		1

Appendix I		6

Recommendation G.962

access digital section for isdn primary rate at 2048 kbit/s

AMENDMENT 1

Maintenance channel

(Geneva, 1997)

Introduce a new subclause C.7 into Recommendation G.962, Annex C.

## C.7	Maintenance channel

### C.7.1	Introduction

The Maintenance Channel (MC) consists of a 4 kbit/s data channel using the international Sa4 bit in the non-frame alignment TS 0 word. The MC shall be used for transporting reports and sending commands between the ET and the other FUs. See Figure C.1.

Figure C.1/G.962 – Maintenance channel outline for PRA

The maintenance channel includes a data protocol which is capable of a maximum of 70 messages/second and should function as specified and be secure with a maximum BER of  for the PRA digital section.

Each PRA digital section shall use its own MC for message signals pertaining to its own operation and maintenance facilities.

### C.7.2	Maintenance channel layered structure

The structure of the MC is based on a three-layer model. These are:

1)	physical layer;

2)	digital section control layer;

3)	message layer.

#### C.7.2.1	Physical layer

The data for the MC are inserted and removed in the TS 0 non-frame alignment word bit 4. This bit is specified in Recommendation G.704 as Sa4. Each FU shall generate a data stream toward its counterpart as well as monitoring it. The Sa4 bit occurs every 250 microseconds giving an effective data rate of 4 kbit/s.

#### C.7.2.2	Digital section data link control layer

The protocol used to convey the messages between the FU shall be LAPD which is a protocol that operates at the data link layer of the OSI architecture. This protocol is specified in Recommendations Q.920 and Q.921. Because the uncomplex function that the protocol is put to, only the unacknowledged operation is required in this case. With this type of operation, the layer 3 messages are transmitted in Unnumbered Information (UI) frames. At the control layer, the UI frames are not acknowledged as this is covered in acknowledgment messages at the layer 3 level. Even if transmission and format errors are detected, no error recovery mechanism is defined at this layer.

When the MC is in an idle state, each control layer generator shall continuously send the idle code which is 01111110.

The ET shall act as the network provider and the NT1, LT as the user. The format of the LAPD frame is shown in Figure C.2 below.

Data link layer peer-to-peer exchanges are in frames conforming to Figure C.2.

| 8 7 6 5 4 3 2 1 | Bit Octet |
| --- | --- |
| FLAG 0 1 1 1 1 1 1 0 | 1 |
| ADDRESS high-order octet | 2 |
| ADDRESS low-order octet | 3 |
| CONTROL | 4 |
| MESSAGE LAYER | 5 to n-3 |
| FCS | n-2 |
| FCS | n-1 |
| FLAG 0 1 1 1 1 1 1 0 | n |

Figure C.2/G.962 – Data link frame structure

##### C.7.2.2.1	Flag sequence

All frames shall start and end with the flag 01111110 sequence. The closing flag sequence of a frame cannot be used as the opening flag for the following frame.

##### C.7.2.2.2	Address field

This field shall consist of two octets as shown in Figure C.3.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Bit Octet |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SAPI |  |  | C/R | EA 0 | 2 |
| 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | Setting |
|  |  |  | TEI |  |  |  | EA 1 | 3 |
|  |  |  | See below |  |  |  | 0 | Setting |

TEI = 0000000 for NT2

TEI = 0001000 for NT1

TEI = 0001011 for LT

TEI = 0001110 for ET

Figure C.3/G.962 – Address field format

##### C.7.2.2.3	Control field

The control field is a single octet shown in Figure C.4. As no dialogue occurs at layer 2, the P/F bit is always set to 1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | P/F |  |  |  |  | 4 |
| 0 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | Setting |

Figure C.4/G.962 – Control field format

#### C.7.2.3	Message Layer (ML)

The structure of a LAPD frame consists of a four-byte LAPD header and a two-byte LAPD trailer framing the maintenance channel header and data as shown below.

##### C.7.2.3.1	Message layer header

The message layer header section follows the standard LAPD header and has the following format:

| Byte | Description |
| --- | --- |
| 1 | Transmission direction |
| 2 | Pad byte – not used |
| 3 | Primary destination SAPI |
| 4 | Pad byte – not used |
| 5-6 | Destination descriptor |
| 7-8 | Source descriptor |
| 9 | Length |
| 10 | Pad byte – not used |

#### C.7.2.3.1.1	Transmission direction

The transmission medium byte is set to 02H for Upstream (NT1 to ET) messages and 03H for Downstream (ET to NT1) messages.

##### C.7.2.3.1.2	Primary destination SAPI

The primary destination Service Access Point Identifier (SAPI) is used as a router address. If the SAPI address is corresponding to one shown in Table C.5, then the destination descriptors are not read and the complete message layer information is passed on as per the routing address. If however the SAPI is not one listed in Table C.5 but is one belonging to Table I.4, then the destination descriptor information is used.

| Table C.5/G.962 – Primary destination service access point identifiers |  |
| --- | --- |
| Routing SAPI | Routing address |
| RELAY_MIU | 21H |
| RELAY_ET | 20H |
| RELAY_LT | 1EH |
| RELAY_NT1 | 1CH |
| RELAY_HALF_LT | 1FH |
| RELAY_HALF_NT1 | 1DH |

C.7.2.3.1.3	Destination descriptors

The destination descriptor is encoded as a sequence of bit fields in 2 contiguous bytes.

| Bit | Description |
| --- | --- |
| 15-12 | Card type |
| 11-6 | Secondary destination SAPI |
| 5-2 | Position |

The destination card types are listed in Table C.6 called Card types.

| Table C.6/G.962 – Card types |  |
| --- | --- |
| Card | Value |
| MIU | 00H |
| ASU | 01H |
| LINE_ASU | 02H |
| LT_ASU | 03H |
| NT1_ASU | 04H |
| ET | 05H |
| LT | 06H |
| NT1 | 07H |
| HALF_LT | 08H |
| HALT_NT1 | 09H |
| LOCAL | 0AH |
| ALL_CARDS | 0BH |

The secondary destination service access point identifiers can be directed to particular applications service access points. An example is given in Appendix I, Table I.4 called Secondary destination service access point identifiers, and can have a range of 00H to 3FH.

The destination position is the slot number within a particular type of functional unit, such as LTs or NT1s and can take any value from 01 to 0EH with 0FH for undefined position.

##### C.7.2.3.1.4	Source descriptor

The source descriptor is encoded as a sequence of bit fields in 2 contiguous bytes.

| Bit | Description |
| --- | --- |
| 15-12 | Source card type (src. card) |
| 11-8 | Source position (src. pos) |
| 7-4 | Bearer |

The source card types and positions have the same definition as in C.7.2.3.1.3.

Bearer is the bearer number along which the message was transmitted and can have any value between 01 and 0FH.

##### C.7.2.3.1.5	Length

This byte indicates the length of the data section. The minimum length is 2 while the maximum is 64 bytes. The values are given hexadecimal format.

##### C.7.2.3.2	Message layer data

The message layer data consists of a signal definition followed by a parameter and data field. The parameter and data type fields are optional. A message layer signal may, in its own right, convey all the information required; however, in some cases, additional information may be required. In this case, parameter data is attached to a signal. If a message has no parameter data attached, the signal consists only of the signal number as 2 consecutive bytes. In a signal with attached parameters, the parameter data follows the signal definition number.

##### C.7.2.3.2.1	Signal definitions

The signal definition number and associated parameters go together to make a complete message. A number of examples are given in Appendix I, Table I.1 called Signal definitions.

Currently signal definition numbers have been allocated from 0001H to 00C1H. All other signal numbers have not been allocated and are available for further enhancements or other applications.

##### C.7.2.3.2.2	Parameter descriptor

The parameter descriptor describes every byte of the parameter data. A number of examples are given in Appendix I, Table I.2 called Parameter descriptors. Every byte can be interpreted by looking at the data type (type entry) definition in Appendix I, Table I.3 called Data types.

appendix I

Examples of message layer data are given in Tables I.1 to I.4.

| Table I.1/G.962 – Signal definitions |  |  |
| --- | --- | --- |
| Signal name | Decimal number | Hexadecimal number |
| CONFIGURE_CHECK | 1 | 01 |
| LT_POSITION_REQUEST | 2 | 02 |
| NT1_POSITION_REQUEST | 3 | 03 |
| HALF_BEARER_SUBRACK_CHECK | 4 | 04 |
| HALF_STANDBY_SUBRACK_CHECK | 5 | 05 |
| ET_CONFIGURE_STANDBY | 6 | 06 |
| LT_CONFIGURE_STANDBY | 7 | 07 |
| ET_CONFIGURE_NORMAL | 8 | 08 |
| LT_CONFIGURE_NORMAL | 9 | 09 |
| NT1_CONFIGURE | 10 | 0A |
| ALARM_PARAMETERS | 11 | 0B |
| CARD_PRESENT | 12 | 0C |
| ET_CARD_POWERING_UP_CONFIGURE_REQUEST | 13 | 0D |
| ET_CARD_POWERING_UP_CONNECT_REQUEST | 14 | 0E |
| ET_CONFIRM_CONFIGURE | 15 | 0F |
| LT_FULL_PATCH_INDICATION_OFF | 16 | 10 |
| LT_FULL_PATCH_INDICATION_ON | 17 | 11 |
| PATCHED_BEARER_OK | 18 | 12 |
| PATCH_TO | 19 | 13 |
| RELEASE_PATCH | 20 | 14 |
| REQUEST_PATCH | 21 | 15 |
| STANDBY_AVAILABLE | 22 | 16 |

| Table I.1/G.962 – Signal definitions (concluded) |  |  |
| --- | --- | --- |
| Signal name | Decimal number | Hexadecimal number |
| STANDBY_NOT_PATCHING_THIS_BEARER | 23 | 17 |
| STANDBY_PATCHING_THIS_BEARER | 24 | 18 |
| CONFIGURED_HALF_STANDBY | 25 | 19 |
| HALF_BEARER_POWERING_UP_CONFIGURE_REQUEST | 26 | 1A |

| Table I.2/G.962 – Parameter descriptions |  |  |  |
| --- | --- | --- | --- |
| Parameter descriptor | Parameter byte No. | Parameter name | Data type |
| CONFIG_REPLY_Par | 1 | Reply | Config_Response |
| CONFIGURE_CHECK_Par | 1 | Standby_Position | Position8 |
| CONFIGURED_HALF_STANDBY_Par | 1 | Config | Configuration_Type |
| HALF_STANDBY_SUBRACK_CHECK_Par | 1 | NT1_Protected_Position | Position8 |
|  | 2 | Standby_Position | Position8 |
| LT_BEARER_REPORT_Par | 1 | LTtoNT1 | LinkAlarmReport |
|  | 2 | NT1toLT | LinkAlarmReport |
|  | 3 | ETtoLT | LinkAlarmReport |
|  | 4 | NT2toNT1 | LinkAlarmReport |
|  | 5 | NT1toNT2 | LinkAlarmReprot |
|  | 6 | LTPosition | Position8 |
|  | 7 | NT1Position | Position8 |

| Table I.3/G.9962 – Data types |  |  |
| --- | --- | --- |
| Data type | Value | Description |
| Alarm_Class | Bit 0 | Status |
|  | Bit 1-2 | Class |
|  | Bit 3-7 | Delay |
| Boot | 00 | FALSE |
|  | 01 | TRUE |
| Byte | 00-FF |  |
| CardType | 00 | MIU |
|  | 01 | ASU |
|  | 02 | LINE_ASU |
|  | 03 | LT_ASU |
|  | 04 | NT1_ASU |

| Table I.3/G.9962 – Data types (concluded) |  |  |
| --- | --- | --- |
| Data type | Value | Description |
|  | 05 | ET |
|  | 06 | LT |
|  | 07 | NT1 |
|  | 08 | HALF_LT |
|  | 09 | HALT_NT1 |
|  | 0A | LOCAL |
|  | 0B | ALL_CARDS |
| Config_Response | 80 | POSITIVE_RESPONSE |
| Configuration_Type | 00 | UNCONFIGURED |
|  | 01 | UNPROTECTED |
|  | 02 | PROTECTED |
|  | 03 | HALF_STANDBY |
|  | 04 | FULL_STANDBY |
|  | 05 | OTHER |
| LinkAlarmReport | Bit 0 | REPORT_OK |
|  | Bit 1 | LRX |
|  | Bit 2 | AIS |
|  | Bit 3 | LFA |
|  | Bit 4 | SES |
|  | Bit 5 | ES |
|  | Bit 6 | REPORT_NONE |
|  | Bit 7 | DM |

| Table I.4/G.962 – Secondary destination service access point identifiers |  |
| --- | --- |
| SAPI | Number |
| MIU_CONFIGURATION | 0EH |
| ET_PATCH_CONTROL | 10H |
| LT_PATCH_CONTROL | 10H |
| NT1_PATCH_CONTROL | 10H |
| ET_PATCH_MONITOR | 11H |
| LT_PATCH_MONITOR | 11H |

| ITU-T RECOMMENDATIONS SERIES |  |
| --- | --- |
| Series A | Organization of the work of the ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communication |
| Series Z | Programming languages |