|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.971 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (06/2004) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital sections and digital line system – Optical fibre submarine cable systems |  |  |  |
|  | General features of optical fibre submarine cable systems |  |  |  |
|  | CAUTION ! PREPUBLISHED RECOMMENDATION This prepublication is an unedited version of a recently approved Recommendation. It will be replaced by the published version after editing. Therefore, there will be differences between this prepublication and the published version. |  |  |  |

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g. interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met.  The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU [had/had not] received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2004

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

ITU-T Recommendation G.971 General features of optical fibre submarine cable systems (revised in 2004)

- Summary - This Recommendation applies to optical fibre submarine cable systems. The purpose of this Recommendation is to identify the main features of optical fibre submarine cable systems, and to provide generic information on relevant Recommendations in the field of optical fibre submarine cable systems. A common implementation relevant to all the optical fibre submarine cable systems is described in Annex A. Specific information relevant to each optical fibre submarine cable systems is included in Annexes of other Recommendations. The updated data on cable ships and submersible equipment of various countries are also described in the Appendix I.

- Source and history 1993 ITU-T Recommendation G.971 was created by ITU-T Study Group 15 (1993-1996). 1996 ITU-T Recommendation G.971 was revised by ITU-T Study Group 15 (1993-1996). 2000 ITU-T Recommendation G.971 was revised by ITU-T Study Group 15 (1997-2000) and approved under the WTSC Resolution 1 procedure on 4 April 2000. 200y ITU-T Recommendation G.971 was revised by ITU-T Study Group 15 (2001-2004) and was approved as the 4th version, under the AAP procedure on the XX of MM 200Y. This revision updated Appendix 1, data on cable ships and submarine equipment of various countries. Moreover, any specific information commonly used in relevant Recommendations were included in Annex A. As seen above, this Recommendation has evolved considerably over the years; therefore the reader is warned to consider the appropriate version to determine the characteristics of already deployed product, taking into account the year of production. In fact, products are expected to comply with the Recommendation that was in force at the time of their manufacture, but may not fully comply with subsequent versions of the Recommendation.

ITU-T Recommendation G.971

General features of optical fibre submarine cable systems

# 1	Scope

This Recommendation applies to optical fibre submarine cable systems.

The purpose of this Recommendation is to identify the main features of optical fibre submarine cable systems, and to provide generic information on relevant Recommendations in the field of optical fibre submarine cable systems. The Annex A contains common implementation aspects of all optical submarine cable systems.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation dose not give it, as a stand-alone document, the status of a Recommendation.

# 2.1	Normative references

The following ITU-T Recommendation contains provisions which, through reference in this text, constitute provisions of this Recommendation.

-	ITU-T Recommendation G.972 (2000), Definition of terms relevant to optical fibre submarine cable systems.

-	ITU-T Recommendation G.973 (19962003), Characteristics of repeaterless optical fibre submarine cable systems.

-	ITU-T Recommendation G.974 (1993), Characteristics of regenerative optical fibre submarine cable systems.

-	ITU-T Recommendation G.975 (2000), Forward error correction for submarine systems.

-	ITU-T Recommendation G.975.1 (2004), Forward error correction for high bit rate DWDM submarine systems.

-	ITU-T Recommendation G.976 (2000), Test methods applicable to optical fibre submarine cable systems.

-	ITU-T Recommendation G.977 (2004), Characteristics of optically amplified optical fibre submarine cable systems.

# 3	Terms and definitions

Terms used in this Recommendation are defined in ITU-T Recommendation G.972.

# 4	Abbreviations

This Recommendation uses the following abbreviations.

BU	Branching Unit

CTE	Cable Terminating Equipment

PFE	Power Feeding Equipment

TTE	Terminal Transmission Equipment

# 5	Features of optical fibre submarine cable systems

An optical fibre submarine cable system has specific technical features:

a)	A submarine cable system should achieve a long lifetime and a high reliability; the main reason is that, due to the difficulty in accessing the submerged plant, the construction and maintenance of a link are long and expensive; moreover most of submarine links are of strategic importance in the transmission network and the interruption of a link usually results in significant loss of traffic and revenue.

b)	A submarine cable system should possess mechanical characteristics which enable it:

1)	to be installed accurately with correct slack and with due safety consideration on the sea bed; deep water installations may reach 8000 metres. (In general, submarine cable systems shall be installed, buried or inspected by specially designed cable ships and submerged equipments. Detailed information of such cable ships and submerged equipments (i.e. ploughs, ROVs, etc.) is contained in Appendix I.);

2)	to resist the sea bottom environment condition at the installation depth, and particularly hydrostatic pressure, temperature, abrasion, corrosion, and marine life;

3)	to be adequately protected (i.e. by armoring or burying) against aggression, due for example to trawlers or anchors;

4)	to survive recovery from such a depth, and subsequent repair and relay, with due safety consideration.

c)	The material characteristics of a submarine cable system should enable the optical fibre:

1)	to achieve its desired reliability over its design lifetime;

2)	to tolerate stated loss and aging mechanisms, especially bending, strain, hydrogen, stress, corrosion and radiation.

d)	The transmission quality of a submarine cable system should follow as a minimum ITU-T Recommendation G.821.

Figure 1 shows the basic concept of optical fibre submarine cable systems and boundaries. Optical submarine repeaters or optical submarine branching units could be included, depending on each system requirement.

In Figure 1, A denotes the system interfaces at the terminal station (where the system can be interfaced to terrestrial digital links or to other submarine cable systems), and B denotes beach joints or landing points. Numbers in brackets in the Figure refer to ITU-T Recommendation G.972.

# 6	Relationship among Recommendations relevant to optical submarine cable systems

Relationship among the various Recommendations pertaining to optical fibre submarine cable systems are shown in the flow chart presented in Figure 2.

Figure 1/G.971 – Example of optical fibre submarine cable systems

Figure 2/G.971 – Relationship between Recommendations
relevant to optical submarine cable systems

ANNEX A

Common implementation aspects of optical submarine cable systems for manufacturing, installing and maintenance

A.1	Introduction

This annex outlines the common aspects of submarine cable systems, which are specified in ITU-T Recommendations G. 973, G.974 and G. 977, such as manufacturing, installation, and maintenance.

The information provided in this ANNEX is intended as a guide to current practice and is not intended as a Recommendation relating to existing or future systems.

# A.2	Manufacturing

## A.2.1	Quality in optical fibre submarine cable systems

The high performance and reliability requirement established for an optical fibre submarine cable system can be fulfilled only if stringent quality procedures are applied during designing, manufacturing, and laying a system. Although quality procedures are particular to each optical fibre submarine cable supplier, the following basic principles generally apply.

#### A.2.1.1	Qualification of the designs and technologies

This activity, part of the development process, is intended to demonstrate that the performance of a technology, a component or an assembly is compatible with meeting the overall system performance and gives reasonable confidence that the reliability target can be met. Qualification includes high stress testing, intended to estimate the ruggedness of the technology, component or sub-assembly and to determine the screening procedure, and long term life testing (some of which might be accelerated for instance by the temperature), the purpose of which is to confirm the validity of the screening procedure and to evaluate the life time and/or the reliability of the technology, component or assembly. Qualification of a cable or submarine equipment may also includes sea trials.

#### A.2.1.2	Certification of components and sub-assemblies

This activity, part of the manufacturing process, is intended to assure the ability of each component or assembly to comply with its performance and reliability specifications once installed. For submarine equipment, each component is individually certified.

The certification is based on the results of screening tests, intended to remove any unsatisfactory item or component, and particularly those likely to exhibit early failures.

#### A.2.1.3	Manufacturing inspection

This activity, during the manufacturing process, is intended to verify that the Quality Plan is respected, that each operation is accomplished along the agreed procedure, and that the result is satisfactory.

The responsibility for manufacturing inspection can be shared between the manufacturer and the purchasers of an optical fibre submarine cable system.

#### A.2.1.4	Factory Acceptance Tests

After completion of the manufacturing of each item (TSE and Submerged Equipment), functional and performance tests must be carried out in order to release the equipment from the Factory.

This activity, conducted in Factory, should comprise all tests necessary to confirm that TSE (including final software) and Submerged Equipment (Repeater and Cable sections) are ready for installation or assembly. The tests should demonstrate that the requirements of the Technical Specification will be met by the Segments and the full Network once installed or assembled if no discrepancy occurs during installation or assembly period.

On completion of factory testing, Equipment may be tested during a confidence trial period to control its stability.

## A.2.2	Assembly and loading procedure

Link assembly consists of jointing the cable sections, the repeaters and the branching units, together with monitoring that the guaranteed margin is present for each fibre in each cable section, so as to constitute the submarine portion. Link assembly is usually performed in the cable factory prior to loading.

Ship loading consists of installing the submarine portion, or fractions of it, on board the cable ship, prior to laying. Ship loading is generally performed with the link unpowered. Tests are made periodically during loading to confirm that the performance of the assembled equipment has not been affected by the loading process.

# A.3	System installation

## A.3.1	Submarine route survey

Route survey is performed prior to cable laying so as to select the cable route and means of cable protection (lightweight protection, armour, burial). The route survey consists in studying the sea depth profile, the sea bottom temperature and seasonal variations, the morphology and nature of the sea bottom, the position of existing cables and pipes, the cable fault history, fishing and mining activities, sea current, seismic activity, laws, etc.

A cable route study should normally be carried out prior to the start of a route survey to determine all environmental, political, economic and practical aspects related to the route. Discussions should be held with local authorities and fishing bodies for this purpose, together with inspection of landing sites and access points as necessary.

An assessment of burial feasibility can also be carried out as part of the route survey, either through direct continuous measurement (Burial Assessment Survey, BAS) or discrete periodic measurement (Cone Penetrometer Testing, CPT).

## A.3.2	Submarine cable installation

Cable laying is normally performed using a recognized cable-ship after any necessary route clearance in shallow water has been carried out (e.g. pre-lay grapnel run, PLGR).

Laying is normally undertaken only when weather and sea conditions do not create severe risk of damage to the submarine portion, cable ship and laying equipment, or of injury to the personnel.

The cable may be buried in the seabed to increase cable protection. Burial can be undertaken during laying using a sea plow towed by the laying cable ship, or after laying using a self-propelled submersible robot or other means.

During laying, a predetermined cable overlength (slack) is laid, so as to ensure that the cable is properly laid on the sea bottom.

The system should be tested during the laying and at the end of laying, so as to ensure that no significant system degradation has been induced. Laying testing includes transmission and functional tests, and may include tests on redundant subassemblies. To permit test during cable laying, the link may be powered, provided that safety regulations are respected.

## A.3.3	Land cable installation and testing

Land Cable tests will be performed after the completion of Land Cable installation at each site to confirm performances.

Especially, the Return Earth System shall be tested after its installation.

## A.3.4	Terminal station equipment installation and testing

After completion of Terminal Station Equipment installation activities in the Cable Terminal Station, a Site Acceptance Testing program should be conducted based on the Factory Acceptance Test program already performed. Results of both periods should be compared. In the event of an unfavourable comparison between the two sets of results the cause of the irregularities should be determined.

All equipment units provided as spares shall be tested for correct operation by substitution with working units.

On completion of the suite tests, the equipment shall be subject to a continuous confidence trial period to be defined depending of the equipment type.

Following the Site Acceptance Testing period for each item, interconnection of Equipment should be carried out to control their interoperability. A specific integration test plan should then be conducted. The results obtained could be compared with previous results (including Technology Demonstration). In the event of an unfavourable comparison between the two sets of results, the cause of the irregularities should be determined.

# A.4	System commissioning

Commissioning testing is performed prior to installing traffic on the system to ensure that the system meets its overall transmission performance contractual requirement, and that all functionalities with respect to the Network Management are operating. When extra margins are available at BOL, it is recommended to assess those one in order to track the ageing of the system.

If redundancy is used in the design to meet the reliability performance, redundant component could be used for correcting faults occurring during lay or prior to commissioning. However, the objective is to ensure that the number of redundant devices remaining available is sufficient to meet, with a high probability, the target for the number of ship repairs.

On completion of the System commissioning period, a continuous Transmission Segment out of service Confidence Trial should followed. Carefully controlled procedures should be established to prevent introduction of errors through human action. Any irregularity, variation alarm or non-routine event observed should be investigated.

# A.5	Maintenance

## A.5.1	Routine maintenance

Routine maintenance is performed from the terminal stations using the supervisory system. It consists in periodic monitoring of the system parameters and, when required, in preventive redundancy switching.

## A.5.2	Maintenance at sea

Optical fibre submarine cable systems can be subject to faults due in particular to external aggression and to component failure. It is important to define and develop well-established and efficient repair procedures and equipment, to facilitate repair and limit loss of traffic.

Maintenance at sea is usually performed using dedicated repair cable ships.

#### A.5.2.1	Fault localization

For systems equipped with optical submarine repeaters, a first localization to within one supervisory section is obtained using the supervisory system.

For the end cable sections, cable fault localization may be achieved from the terminal stations, using adequate electrical measurement (resistance, capacitance, insulation, etc.) and optical reflectometry.

Similarly, cable fault localization may be achieved from the cable ship after cable recovery, using the same methods.

Electroding can be used to locate the cable route.

#### A.5.2.2	Cable recovery

During cable recovery it may be necessary, in order to limit the mechanical tension applied to the cable, to cut the cable on the sea bottom prior to recovering both ends separately.

#### A.5.2.3	Sea repair

Several methods can be used for sea repair according to the sea depth:

–	the shallow water repair may necessitate the addition of a cable length, but not that of a repeater; a repair margin is generally included in the shallow water optical power budget since the shallow water sections are the most exposed at risk from external aggression, even though precautions are taken;

–	the deep sea repair usually necessitates the addition of a cable length and sometimes of a repeater to compensate for the extra attenuation, if the extra attenuation incurred cannot be accommodated in the available margin; generally, a very low repair margin is included in the deep water optical power budget since deep sea repairs are not frequent.

When a fault is identified to within one supervisory section, the section may be replaced by a mini-system, without further localization. This method may save time, but requires more spare equipment.

Repair safety procedures are applied on board the cable ship and in the terminal station, so as to ensure the safety of the personnel operating on board the cable ship. In particular, power safety procedures involve earthing the cable in the terminal station, on board the cable ship and at branching unit.

APPENDIX I

Data on cable ships and submersible equipments of various countries

(Mar del Plata, 1968, amended at Geneva, 1972, 1976, 1980, 1984, 1988, 1955, 2000 and 2004)

Section 1 – Cable ships

| Name of ship | Year of con- struc- tion | Dis- place- ment (tons) | Overall length (m) | Draft (m) | Normal speed (knots) | Range (auto- nomy) (nautical miles) | Number of tanks | Cable capacity |  |  | Cable gear |  |  |  | Maximum operating depth (m) | Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Cable |  | Re- peaters | Cable engine |  | Unwinding pulley |  |  |  |
|  |  |  |  |  |  |  |  | Cubic metres (m3) | Weight (tons) |  | Drum (diameter) (m) | Linear (pairs of wheels) | Bow sheave (diameter) (m) | Stern sheave (diameter) (m) |  |  |
|  |  |  |  |  |  |  | DENMARK Ships belonging to Tele Denmark |  |  |  |  |  |  |  |  |  |
| Peter Faber | 1982 | 3680 | 78.35 | Ice 3.8 Summer 5.0 | 13.0 | 7000 | 1 tank 1 hold | 310 230 | 600 400 | App. 10 | 3.0 |  | 2 x 3.0 | – | 4000 | Reinforced for operation in icefilled waters. A-frame for ROV. Two hydraulic double-drum warping winches. |
| Maersk Fighter | 1992/94 | 2961 | 82.5 | 6.24 | 15.7 Max | 7700 | 2 | 1263 | 2400 | 24 | 4.0 (25t) | 65 (4t) | 4000 | – | – | Laying/burying and repair of all types of cables (coaxial, optical fibre and power cables). Ploughs and ROV capability. |
| Heimdal | 1982 / 2000 | 11493 | 136.7 | 6.60 | 15 Max | 16000 | 4 | 4670 | 6224 | >120 | 4.0 (25t) | 20 (20t) | – | 2 x 3.0 | All | Laying/burying and repair of all types of cables (coaxial, optical fibre and power cables). Ploughs and ROV capability. |
| Maersk Defender | 1996 | 11980 | 96.0 | 8.70 | 12 | 17000 | 4 | 3158 | 6000 | 12 | 4.0 (25t) | 20 (20t) |  | 2 x 3.0 | All | Laying/burying and repair of all types of cables (coaxial, optical fibre and power cables). Ploughs and ROV capability. |
| Lodbrog | 1985 / 2002 | 12503 | 143.4 | 8.50 | 16.0 | 10000 | 6 | 2940 | 5040 | 84 | 2 x 4.0 (25t) | 2 x 6 (6t) | - | 2 x 3.0 | All | Laying/burying and repair of all types of cables (coaxial, optical fibre and power cables). ROV capability, SWL 8 ton. |
|  |  |  |  |  |  |  | FINLAND 1) Ship belonging to Sonera LTD |  |  |  |  |  |  |  |  |  |
| M/S Telepaatti | 1978 (modification) | 450 | 42.6 | 3.0 | 12 | – | 1 | – | 350 | – | 2 linear engines with 3 caterpillar tracks on each | 3.0 |  | 300 |  | Laying of all types of telecom cables. Specially equipped for cable route survey and cable repair. Fully automatic autopilot and DP-system. |
|  |  |  |  |  |  |  | 2) Ship belonging to YIT Primatel |  |  |  |  |  |  |  |  |  |
| c/s Telepaatti | 1978 modification 1999 | 450 | 42.6 | 3.0 | 10.5 | - | 1 | 250 | 260 | - | - | 2 linear engines with 3 caterpillar tracks on each | 3.0 | - | 300 | Laying of all types of telecom cables and <150 mm power cables. Specially equipped for cable route survey and cable repair. Fully automatic autopilot and DP-system. |

| Name of ship | Year of con- struc- tion | Dis- place- ment (tons) | Overall length (m) | Draft (m) | Normal speed (knots) | Range (auto- nomy) (nautical miles) | Number of tanks | Cable capacity |  |  | Cable gear |  |  |  | Maximum operating depth (m) | Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Cable |  | Re- peaters | Cable engine |  | Unwinding pulley |  |  |  |
|  |  |  |  |  |  |  |  | Cubic metres (m3) | Weight (tons) |  | Drum (diameter) (m) | Linear (pairs of wheels) | Bow sheave (diameter) (m) | Stern sheave (diameter) (m) |  |  |
|  |  |  |  |  |  |  | FRANCE 1) Ships belonging to France Telecom |  |  |  |  |  |  |  |  |  |
| Vercors | 1974 | 11 000 | 136 | 7.2 | 16.6 | 12 000 | 3 | 2425 | 4900 | 144 | 3.0 | 24 | 3.0 | Chute | All | Laying and repair of all types of telecom cables. Burying of cables with plough. |
| Leon Thevenin | 1983 | 6800 | 107 | 6.24 | 15.0 | 10 000 | 2 + 1 | 1420 | 2000 | 11 | 3.4 | 12 | 3.0 | Chute | All | Laying and repair of all types of telecom cables. Burying of cables using Scarab. |
| Raymond Croze | 1983 | 6800 | 107 | 6.24 | 15.0 | 10 000 | 2 + 1 | 1420 | 2000 | 11 | 3.4 | 12 | 3.0 | Chute | All | Laying and repair of all types of telecom cables. Burying of cables using Scorpio 2000. |
|  |  |  |  |  |  |  | 2) Ships belonging to Alda Marine |  |  |  |  |  |  |  |  |  |
| Ile de Sein Ile de Batz Ile de Brehat | 2002 | 18006 | 140.4 | 8.016 | 15.0 | 15000 | 2 + 2 | 3000 | 5500 | 202 | 4.0 | 21 | NA | 3.0 | All | Laying and repair of all types of telecom cables. Burying of cables with. 2/3m Rock plough. Sea state 7 A frame |
| Ile de Ré | 1983 rebuilt 2002 | 12687 | 143.4 | 7.23 | 16.0 | 11000 | 3 + 3 | 2900 | 4500 | 84 | 2 x 4.0 | NA | NA | 3.0 | All | Laying and repair of types of cable. No plough ROV to 2500m |
|  |  |  |  |  |  |  | ITALY 1) Ship belonging to Elettra TLC S.p.A |  |  |  |  |  |  |  |  |  |
| Teliri | 1996 | 6500 | 111.5 | 6.5 | 14.0 | 10 000 | 3 | 2000 | 2600 | 70 | N.2  3.5 | N.1  18wp | 3 | 4 | All | Laying and repair optical fibre systems |
| Gertament (ex John Cabot) | 1995 rebuilt 1998 | 5000 | 96.6 | 7.3 | 12.0 | 8000 | 3 | 600 | 1900 | 24 | N 1  3 | 1  18+ 1  6 | 3 | 3 | All | Laying, survey and repair optical fibre systems |
| Pertinacia | 2002 | 12650 | 129.9 | 8.4 | 14 | 10 000 | 2 | 4000 | 6400 | 2 x 100 | N 1 x 4 | 1 x 20 1 x 6 | – | 3 | All | Lay and repair optical fibre systems |
|  |  |  |  |  |  |  | 2) Ships belonging to Pirelli Cavi |  |  |  |  |  |  |  |  |  |
| Arabella | 1975 | 2620 | 76.66 | 5.18 | 11 | 2000 | 2 | 1100 | 2000 | – |  | – | – | 3 | All | Lay/repair. |
| G.Verne | 1984 | 16 900 | 128.5 | 8.5 | 10 | 8000 | 2 | 2600 | 8000 | 20 | 6.0 (50t) | 1 (Pads type 10t) | – | 6.0 | All | Stern only. |
|  |  |  |  |  |  |  | SPAIN Ships belonging to Tyco Submarine Systems Ltd. |  |  |  |  |  |  |  |  |  |
| Teneo | 1992 | 4000 | 81 | 5.7 | 14.5 | 4200 | 2 | 500 | 1000 | 20 | 2  3.5 | 1  9 | 2  3 | 1  3 | All | Lays and repairs of all types of telephone cables. |
| Atlantida | 1987 | 7853 | 114 | 6.5 | 15.7 | 6800 | 3 | 1500 | 2500 | 33 | 2  3.5 | 1  12 | 2  3 | 1  3 | All | Lays and repairs of all types of telephone cables. |
| Iberus | 1978 | 10 000 | 136.03 | 6.6 | 12.5 | 13 500 | 3 | 2580 | 4000 | 108 | 1  3 | 1  20 | – | 2  3 | All | Lays and repairs of all types of telephone cables. |

| Name of ship | Year of con- struc- tion | Dis- place- ment (tons) | Overall length (m) | Draft (m) | Normal speed (knots) | Range (auto- nomy) (nautical miles) | Number of tanks | Cable capacity |  |  | Cable gear |  |  |  | Maximum operating depth (m) | Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Cable |  | Re- peaters | Cable engine |  | Unwinding pulley |  |  |  |
|  |  |  |  |  |  |  |  | Cubic metres (m3) | Weight (tons) |  | Drum (diameter) (m) | Linear (pairs of wheels) | Bow sheave (diameter) (m) | Stern sheave (diameter) (m) |  |  |
|  |  |  |  |  |  |  | JAPAN 1) Ships belonging to Kokusai Cable Ship (KCS) |  |  |  |  |  |  |  |  |  |
| KDD Ocean Link | 1992 | 11 700 | 133.2 | 7.0 | 15 | 10 000 | Main 3 Spare 4 | 2320 | 4500 | 100 | 3.6 | 21 | 3.2 | 4.0 | All | Laying by linear engine. Lays and repairs all types of submarine cables. |
| KDD Pacific Link | 1997 | – | 109.0 | 7.5 | 12 | – | Main 2 Spare 2 | 2720 | 4500 | – | 3.6 | 20 | – | 3.0 | All | Laying by linear engine. Lays and repairs all types of submarine cables. |
|  |  |  |  |  |  |  | 2) Ships belonging to NTT World Engineering Marine Corporation (NTT-WE Marine) |  |  |  |  |  |  |  |  |  |
| NTT Kuroshio Maru | 1974 | 5656 | 119.3 | 5.60 | 16.5 | 6883 | 3 | 1429 | 1900 | 95 | 3.8 | 8 (24 inch) | 3.0 | 3.0 | All | Lays and repairs all types of telephone cables. |
| Subaru | 1999 | 9557 | 123.3 | 7.0 | 13.2 | 8800 | Main 2 Spare 2 | 2770 | 4000 | 50 | 4.0 | 21 | – | 3.2 | All | Lays and repairs all types of telephone cables. |
|  |  |  |  |  |  |  | UNITED KINGDOM 1) Ship belonging to British Telecommunications plc |  |  |  |  |  |  |  |  |  |
| Sovereign | 1991 | 13 018 | 131 | 7.0 | 13.5 | 14 000 | 4 | 2800 | 6200 | 90 | 3.50 |  | 3.00 | 3.50 | All | Lays, repairs all types of coaxial and optical fibre cable. (Operated by C&W marine.) |
|  |  |  |  |  |  |  | 2) Ships belonging to Global Marine Systems Ltd |  |  |  |  |  |  |  |  |  |
| Alert | 1961 | 9477 | 130 | 7.1 | 14 | 10 000 | 3 | 1509 | 3100 | 48 | 2.98 |  | 2.98 | 2.98 | All | Laying by linear engine and sea-bed burial by plow. Lays/repairs all types of coaxial and optical fibre cables. |
| Cable Venture | 1962 | 16 983 | 153 | 8.97 | 12.5 | 10 000 | 4 + 1 (spare) | 5086 | 9000 | 400 | 2.80 |  | 3.00 | 3.39 | All | Laying by linear cable engine. Ploughs, lays and repairs armoured and lightweight cables. |
| Mercury | 1962 | 11 683 | 144 | 7.5 | 14.5 | 8000 | 3 | 2970 | 3500 | 144 | 3.05 |  | 3.50 | Chute 3.05 | All | Ditto (no plough). |
| Cable Enterprise | 1964 | 5759 | 113 | 5.84 | 13 | 8000 | 3 | 887 | 2150 | 30 | 2.8 |  | 3.00 | Chute 3.05 | All | Lays/repairs armoured cables. Repairs lightweight cables. (Note) |
| Monarch | 1975 | 4639 | 97 | 5.5 | 14 | 7000 | 4 | 417 | 850 | 12 | 3.00 |  | 3.00 | None | All | Lays/repairs armoured coaxial and optical fibre cables. Repairs lightweight coaxial and optical fibre cables. Detrenching/reburial by submersible jetting. |

| Name of ship | Year of con- struc- tion | Dis- place- ment (tons) | Overall length (m) | Draft (m) | Normal speed (knots) | Range (auto- nomy) (nautical miles) | Number of tanks | Cable capacity |  |  | Cable gear |  |  |  | Maximum operating depth (m) | Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Cable |  | Re- peaters | Cable engine |  | Unwinding pulley |  |  |  |
|  |  |  |  |  |  |  |  | Cubic metres (m3) | Weight (tons) |  | Drum (diameter) (m) | Linear (pairs of wheels) | Bow sheave (diameter) (m) | Stern sheave (diameter) (m) |  |  |
|  |  |  |  |  |  |  | UNITED KINGDOM 2) Ships belonging to Global Marine Systems Ltd |  |  |  |  |  |  |  |  |  |
| Iris | 1976 | 4639 | 97 | 5.5 | 14 | 7000 | 4 | 417 | 850 | 12 | 3.00 |  | 3.00 | None | All | Lays/repairs armoured coaxial and optical fibre cables. Repairs lightweight coaxial and optical fibre cables. |
| MV Cable Installer | 1980 | 6065 | 89.42 | 5 | 12 | 42 days | 4 | 840 | 1600 | None | 3.0 | 4-track pair | – | 3.0 | – | Repeaterless installation vessel fully DP Cegelec 901 system. |
| Seaspread | 1980 | 10 887 | 116 | 6.8 | 13 | 65 days | 2 | 1010 | 1701 | – | 2  3 | – | – | 3 | All | Lays/repairs by aft drums. Burial by plough. Lays/repairs armoured and lightweight cables. |
| Pacific Guardian | 1984 | 7526 | 116 | 6.32 | 14.0 | 8000 | 3 | 1416 | 3470 | 96 | 3.5 |  | 3.00 | 3.00 | All | Laying by linear cable engine. Lays and repairs armoured and lightweight cables. |
| Sir Elic Sharp | 1988 | 7526 | 115 | 6.3 | 13.5 | 9600 | 3 | 1416 | 1700 | 96 | 2  3.5 | – | 3 | 3 | All | Laying by linear cable engine. Repairs and lays armoured and lightweight cables. Post lay/repair burial by integral ROV. |
| MV Cable Innovator | 1995 | – | 142 | 8.3 | 14.5 | 42 days | 4 | 4900 | 7500 | 180 | 4.0 | 21 pair (min) | – | 4.0 | – | Simplex D/P system. Lays/repairs cables. |
|  |  |  |  |  |  |  | MARSHALL ISLANDS Ship belonging to Tyco Submarine Systems Ltd. |  |  |  |  |  |  |  |  |  |
| CS Coastal Connector | 1997 Converted in 1996 | 6761 | 92.47 | 7.1 | 12.5 | 25 000 | 3 main 1 spare | 675 (main, total) 70 (spare) | 1600 | 30 | 2  3 | N/A | N/A | 2  3 | – | The CS Coastal Connector is stern laying design. She is capable of deploying the SCARAB II, SCARAB IV, and Pacific SCARAB I ROV's, as well as the Seabed Tractor |
| CS Tyco Provider | 1978, Converted in 1999 | 14 500 | 139.4 | 7.6 | 14.5 | 20 000 | 5 | 3349 | 6000 | 100+ | 2  4 | – | – | 2  3 | – | The CS Tyco Provider is a stern laying design. She is capable of deploying Sea Plow VIII. |
|  |  |  |  |  |  |  | NETHERLANDS ANTILLES Ship belonging to Tyco Submarine Systems Ltd. (charterer) |  |  |  |  |  |  |  |  |  |
| Dock Express 20 | 1983 | 21 731 | 169.52 | 8.79 | 12.5 | 20 500 | 3 main 2 spare | 4050 (main, total) 640 (spare, total) | 10 000 | 100+ | 1  3.0 | 1  3 module belt type | N/A | 2  3 |  | The Dock Express 20 is a stern laying design. She is capable of deploying the SCARAB II ROV, as well as the Seabed Tractor and Sea Plow VI. |

| Name of ship | Year of con- struc- tion | Dis- place- ment (tons) | Overall length (m) | Draft (m) | Normal speed (knots) | Range (auto- nomy) (nautical miles) | Number of tanks | Cable capacity |  |  | Cable gear |  |  |  | Maximum operating depth (m) | Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Cable |  | Re- peaters | Cable engine |  | Unwinding pulley |  |  |  |
|  |  |  |  |  |  |  |  | Cubic metres (m3) | Weight (tons) |  | Drum (diameter) (m) | Linear (pairs of wheels) | Bow sheave (diameter) (m) | Stern sheave (diameter) (m) |  |  |
|  |  |  |  |  |  |  | UNITED STATES OF AMERICA Ships belonging to AT&T |  |  |  |  |  |  |  |  |  |
| CS Charles L. Brown | 1954, Reflag-ged in 1985 | 4298 | 99.94 | 5.6 | 13 | 7550 | 3 | 660 | 1186 | 30+ | 2  3 | N/A | 2  3 | N/A | – | The Charles L. Brown is primarily a repair ship. She is not fitted with any stern laying equipment. She is capable of deploying the SCARAB II ROV. |
| CS Global Link | 1990 | 16 375 | 145.7 | 8.08 | 15 | 10 000 | 3 main, 4 spare | 3258 (main, total) 164 (spare, total) | 6098 | 100+ | 2  3.7 | 1 Western Gear Tractor Type | 2  3 | 1 trough/ Chute type | – | The Global Link is capable of deploying the SCARAB II ROV's. |
| CS Global Mariner | 1993 | 15 638 | 151.5 | 7.8 | 13.8 | 10 000 | 2 main, 3 spare | 2172 (main, total) 447 (spare, total) | 4999 | 80+ | 2  3.7 | 1 Dowty 21 pair | 2  3 | 1 trough/ Chute type | – | The Global Mariner is capable of deploying the SCARAB II and SCARAB IV ROV's, as well as Sea Plow VII, Sea Plow VIII, and the Seabed Tractor. |
| CS Global Sentinel | 1991 | 16 375 | 145.7 | 8.08 | 15 | 10 000 | 3 main, 4 spare | 3258 (main, total) 164 (spare, total) | 6098 | 100+ | 2  3.7 | 1 Dowty 21 pair | 2  3 | 1 trough/ Chute type | – | The Global Sentinel is capable of deploying the SCARAB II and SCARAB IV, and Pacific SCARAB I ROV's, as well as Sea Plow VII and Sea Plow VIII. |
| NOTE – Only relatively short cables are laid and only shore-end. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Section 2 – Submersible equipments

| Type of submersible | Weight (tons) | Overall length (m) | Width (m) | Height (m) | Trenching system | Trenching | Propulsion | Max.operating depth (m) | Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | FRANCE Submersibles belonging to France Telecom (FTRSI) |  |  |  |  |  |
| ELISE2 Submersible Plough system | 17 | 7.60 | 2.90 | 2.95 | Ploughshare | Immediate burial up to 1.1 m | Towed by support ship | 1500 | Lay and bury all types of cables. |
| ELISE3 Submersible Plough system | 17 | 7.60 | 2.90 | 2.95 | Ploughshare | Immediate burial up to 1.1 m | Towed by support ship | 1500 | Lay and bury all types of cables. |
| Self-advancing buried system CASTOR2 | 12 | 7.0 | 2.40 | 3.00 | Trenching wheel or chain | Burial of existing cables down to 2 m | Tracked vehicle | 1000 | Burial of cables and pipes. Visual inspection. |
| Scarab 3 | 9 | 4.0 | 3.50 | 2.10 | High pressure water jets | Up to 60 cm depth | Thrusters (inspection) Back drive (burial) | 1000 (burial) 2000 (inspect) | Visual inspection, post lay burial, cable location, cable manipulation, cable cutting. |
| Remote control submersible Scorpio 2000 | 3.4 | 2.9 | 1.5 | 2.11 | High pressure water jets | Up to 60 cm depth | Thrusters | 1000 | Visual inspection, post lay burial, cable location/ manipulation/cutting. |
|  |  |  |  | ITALY 1) Submersibles belonging to Elettra TLC SpA |  |  |  |  |  |
| Plough Taurus 1 | 14 | 9 | 4.6 | 4.5 | Plough share | Up to 1 mt | Plough Taurus 1 | 14 | 9 |
| Plough Taurus 2 | 16 | 9.5 | 4.5 | 5.1 | Plough share | Up to 1.5 mt | Plough Taurus 2 | 16 | 9.5 |
| ROV – Phoenix 2 | 6.8 | 4.8 | 2 | 2.6 | High/low pressure jetting | Up to 1.2 mt | ROV – Phoenix 2 | 6.8 | 4.8 |
|  |  |  |  | ITALY 2) Submersibles belonging to Pirelli Cavi |  |  |  |  |  |
| Plough 1 | 10 | 7 | 2.7 | 3 | Plough share | Up to 1 m | Plough 1 | 10 | 7 |
| Plough 2 | 9 | 8.5 | 3.8 | 3.5 | Plough share | Up to 1.2 m | Plough 2 | 9 | 8.5 |
|  |  |  |  | UNITED KINGDOM Submersibles belonging to Global Marine Systems Ltd |  |  |  |  |  |
| Submersible trencher | 17.0 | 6.6 | 4 | 3.4 | Fluidization and cutting jets and dredge pump | Up to 1 m depth with cutting and fluidization jets | Three vertical and four horizontal thrusters, track drive differential steering | 274 | Trench in existing cable and pipe. |
| Submersible Plough system | 9.75 | 6.1 | 2.6 | 2.6 | Ploughshare proceeded by disc | Immediate burial of cable on ploughing | Towed by support ship | 900 | Lay and bury cable, umbilical and pipe in one action giving full cable protection. |
| Remote control submersible 2 off Cirus A&B | 3.2 | 3.5 | 2.1 | 2.3 | Water jets | Trenching capability 0.3 m | Thrusters (7) | 1000 | Visual inspection, cable location/inspection/deburial, manipulation. Tools include cable cutter, cable gripper and 2 manipulators with line cutters. |

| Type of submersible | Weight (tons) | Overall length (m) | Width (m) | Height (m) | Trenching system | Trenching | Propulsion | Max.operating depth (m) | Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | UNITED KINGDOM Submersibles belonging to Global Marine Systems Ltd |  |  |  |  |  |
| Plough 2 off A&B | 14.5 | 9 | 4.1 | 4 | Passive blade | Trenching capability 1.0 m | Towed | 1000 | Steerable, repeater burial. |
| Remote control submersible ROV 128 | 7.5 | 2.9 | 1.8 | 2.0 | Jetting tool | Trenching capability 0.6 m | Tracked burial Thrusters survey | 1000 (burial) 2000 (survey) | Tools include cable cutter, cable gripper and 2 manipulators with line cutters. |
| Underwater vehicle- MARLIN | 7.8 | 4.191 | 2.438 | 3.175 | Burial skid | To 1.0 m (Optimized for 0-30 kPa soil) | Hydraulic driven thrusters | 2500 | Burial, deburial, inspection. Maintenance and repair. Tools include cable cutter, cable gripper. |
| Scarab I – Umbilically tethered ROV | 3.2 | 2.74 | 1.82 | 1.52 | Jetting tool | Up to 0.6 m | Thrusters: 2 vertical 4 vectored | 2000 | Cable detection and inspection. Visual survey. Cable manipulation and cutting Debris elimination. Cable and repeater burial/ deburial. |
| Subtrack - ROV | 10.0 | 8.0 (Max) | 3.7 | 3.8 | Jetting tool | Burial to 1.0 m | Electro hydraulic track drives | 1000 | Cable burial and deburial. Inspection. Maintenance and repair. |
| EUREKA: Deepwater burial + trenching system | 17 (Max) | 5.5 | 4.2 | 3.85 | Jetting tool Rock wheel cutter Mechanical chain excavator | 1 m 1.2 m 2.2 m | Electro hydraulic track drives | 1500 | Capable of burying cable, small flexible flowlines and also rigid pipes. Can also debury cable and restore. Visual and electronic inspections. |
| Plough 5 | 14.0 | 9.0 | 4.6 | 3.7 | Passive blade | Variable from 0-1100 mm (600-900 mm in all conditions) | Towed | 1000 | Simultaneously lay and bury cables and umbilicals at varying depths. |
| Plough 6 and 7 | 14.0 | 9.0 | 4.6 | 3.7 | Passive blade | Max burial depth: 1100 mm | Towed | 1000 | Simultaneously lay and bury cables and umbilicals at varying depths. |
| Cable Plough 1000 mm | 14.4 | 9.75 | 4.1 | 3.9 | Passive blade | 1000 mm (Good conditions: 1100 mm; Repeaters/Joints: 500 mm) | Towed | 1000 | Simultaneously lay and bury cables and umbilicals at varying depths. |

| Type of submersible | Weight (tons) | Overall length (m) | Width (m) | Height (m) | Trenching system | Trenching | Propulsion | Max.operating depth (m) | Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | DENMARK Submersibles belonging to Telecom Denmark |  |  |  |  |  |
| Plough D | 13.5 | 9.0 | 4.6 | 3.7 | Plough share | Variable from 0-1100 mm (600-900 mm in all conditions) | Towed by host vessel | 1500 | Lay and bury telecom cables, power cables and umbilicals. Cables: Up to 120 mm (bary). Joints and repeaters: Up to 400 mm (pass). |
| Plough 7 | 13.5 | 9.0 | 4.6 | 3.7 | Plough share | Variable from 0-1100 mm (600-900 mm in all conditions) | Towed by surface vessel | 1000 | Lay and bury fibre optic cables, power cables and umbilicals. |
| Subtrack- Subsea tractor | 10.0 | 8.0 (Max) | 3.7 | 3.8 | Jetting tool | Burial to 1.0 m | Electro hydraulic track drives | 1000 | Cable burial and deburial. Inspection. Maintenance and repair. |
| Super Phantom S4-ROV | 0.09 | 1.5 | 0.75 | 0.6 | – | – | Thrusters 4 prop fwd/aft 2 prop vertical 2 prop transvers | 300 | Inspect cables and other underwater objects. Can also be used to inspect seabed conditions. |
|  |  |  |  | JAPAN 1) Submersibles belonging to KCS |  |  |  |  |  |
| MARCAS-II-ROV | Jet tool mode: 8.0 Track base mode: 7.5 | Jet tool mode: 2.9 Track base mode: 5.3 | Jet tool mode: 2.3 Track base mode: 4.0 | Jet tool mode: 3.2 Track base mode: 3.8 | Water jet tool | – | 4 horizontal, 2 vertical and 2 balance thrusters | Jet tool mode: 2500 Track base mode: 2000 | Post-lay burial, maintenance of cable. Can survey seabed. |
| MARCAS-SBT-ROV | 15 (minimum) 23 (maximum) | Jet tool mode: 9.5 Chain cutter mode: 13.0 Wheel cutter mode: 12.0 | Jet tool mode: 5.5 Chain cutter mode: 5.5 Wheel cutter mode: 5.5 | Jet tool mode: 4.4 Chain cutter mode: 4.4 Wheel cutter mode: 4.4 | 2.1 m Rear jet tool and 1 m Forward jet tool 1.2 m Wheel cutter 3 m Chain cutter |  | One single hydraulic thruster | 1500 | Lay and burial, post-lay burial, maintenance of cable, and survey of seabed. |

| Type of submersible | Weight (tons) | Overall length (m) | Width (m) | Height (m) | Trenching system | Trenching | Propulsion | Max.operating depth (m) | Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | JAPAN 2) Submersibles belonging to NTT-WE Marine |  |  |  |  |  |
| Plough-type MARK-5 submarine cable burying system | 19.0 | 9.1 | 4.0 | 4.0 | – | Up to 1.5 m depth immediate burial of cable on ploughing | Towed by support ship | 600 | Simultaneous or post-lay burial of cable. |
| Submarine cable repair burial and inspection system | 6.2 | 3.8 | 2.1 | 2.3 | Fluidization jets | Fluidization jets | Vertical and horizontal thrusters | 1000 | Post-lay burial maintenance of cable and survey of seabed. |
| Plough-type MARK-6 submarine cable burying system | 18 | 9.3 | 5.1 | 4.4 | – | Up to 2.0 m depth immediate burial of cable on ploughing | Towed by support ship | 1500 | Simultaneous or post-lay burial of cable. |
| Submarine cable repair burial and inspection system | 8.0 | 3.2 | 2.1 | 2.8 | Fluidization jets | Trenching capability 1.0 m | Vertical and horizontal thrusters | 2500 | Cable detection & inspection visual survey. Cable manipulation & cutting. Debris stockage. Cable & repeater burial/deburial. |
| SEA MOLE Tractor type Submarine cable Burial system | 23 | 8 | 6 | 3.5 | 3 types attachment (jetting tool, wheel cutter & chain cutter) | Trenching capability 2.0 m (max.) | – | 1000 |  |
|  |  |  |  | SPAIN 1) Submersible belonging to Tyco Submarine Systems Ltd. |  |  |  |  |  |
| ARADO I | 12 | 9 | 4.6 | 4 | Plow-share | 1100 mm | Towed | 1500 | Bury cable from 19 to 40 mm. Bury repeaters until 380 mm. Velocity 1 m/s. |
| ARADO II | 12 | 9 | 4.6 | 4 | Plow-share | 1500 mm | Towed | 1500 | Bury cable from 17 to 150 mm. Bury repeaters until 380 mm. Velocity 1 m/s. |
| ARDI | 3.6 | 6.1 | 3 | 2.6 | Plow-share | 900 mm | Towed | 1500 | System to evaluate if land is arable. |
| NEREUS | 8.5 | 3.2 | 3.4 | 2.9 |  | 1 m | 150 KW | 2000 | Repair, inspect and bury all types of telephone cable 2  manipulating 7 functions. Velocity 3 knots. |

| Type of submersible | Weight (tons) | Overall length (m) | Width (m) | Height (m) | Trenching system | Trenching | Propulsion | Max.operating depth (m) | Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | SPAIN 1) Submersible belonging to Tyco Submarine Systems Ltd. |  |  |  |  |  |
| SCARAB III | 8.5 | 4 | 3.9 | 2.1 |  | 0.6 | 180 KW | 2000 | Repair, inspect and bury all types of telephone cable 2 x manipulating 7 functions. Velocity 3.1 knots. |
|  |  |  |  | 2) Submersible belonging to Consorcio ESCARAB |  |  |  |  |  |
| ROV | 8.5 | 4.0 | 3.9 | 2.1 |  | Up to 1 m |  | 2000 1000 |  |
|  |  |  |  | UNITED STATES OF AMERICA Submersibles belonging to Tyco Submarine Systems Ltd. |  |  |  |  |  |
| PACIFIC SCARAB I | 5.48 | 4.27 | 1.83 | 3.05 | Jetting Modules | 560 meters/hour. Soil hardness to 100 kPa. | 150HP Electro Hydraulically Powered using 8 thrusters | 2500 | PACIFIC SCARAB I Submersible Craft Assisting Repair and Burial is a tethered, swimming ROV capable of operating at depths of 2500 meters. It can locate, inspect, retrieve, and bury submarine cables. |
| SCARAB II | 3.45 | 3.7 | 2.1 | 2.3 | 35 HP Cable Jetter | 255m/hr depending on soil conditions. Soil hardness to 60 kPa. | Horizontal: 4×5 HP Electric Thrusters Vertical: 2×5 HP Electric Thrusters Aft Lateral: 1×10 HP Hydraulic Thruster Bow: 2×2.5 HP Hydraulic Thrusters | 1850 | SCARAB II Submersible Craft Assisting Repair and Burial is a tethered, swimming ROV capable of operating at depths of 1850 meters. It can locate, inspect, retrieve, and bury submarine cables. |

| Type of submersible | Weight (tons) | Overall length (m) | Width (m) | Height (m) | Trenching system | Trenching | Propulsion | Max.operating depth (m) | Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | UNITED STATES OF AMERICA Submersibles belonging to Tyco Submarine Systems Ltd. |  |  |  |  |  |
| SCARAB IV | 4.6 | 3.4 | 2.02 | 1.96 | Jetting Modules | 530 meters/hour Soil hardness to 100 kPa | 150 HP Electro-Hydraulically Powered using 8 thrusters | 1850 | SCARAB IV Submersible Craft Assisting Repair and Burial is a tethered, swimming ROV capable of operating at depths of 1850 meters. It can locate, inspect, retrieve, and bury submarine cables. SCARAB IV is part of the ACMA SCARAB Agreement. |
| Sea Plow VI | 25.5 | 10.5 | 6.0 | 4.3 | Towed Plow System | 1.2 meter burial | Towed by ship | 1000 | Sea Plow VI is a towed burial tool employing state-of-the-art burial features. It can achieve 1.2 meter burial depth in up to 1000 meter water depth. |
| Sea Plow VII | 14.0 | 10.5 | 6.0 | 4.3 | Towed Plow System | 1.0 meter burial | Towed by ship. 1 Thruster for Launches and Recoveries | 1400 | Sea Plow VII is a towed burial tool employing state-of-the-art burial features. It can achieve 1.0 meter burial depth in up to 1400 meter water depth. |
| Sea Plow VIII | 19.3 | 9.2 | 5.5 | 3.6 | Towed Plow System with water jet assist | 1.5 meter burial | Towed by ship | 1500 | Sea Plow VIII is a towed burial tool employing state of the art burial features. It can achieve 1.5 meter burial depth in up to 1500 meter water depth. |