|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.972 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (06/2004) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital sections and digital line system – Optical fibre submarine cable systems |  |  |  |
|  | Definition of terms relevant to optical fibre submarine cable systems |  |  |  |
|  | ITU-T Recommendation G.972 |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| General | G.900–G.909 |
| Parameters for optical fibre cable systems | G.910–G.919 |
| Digital sections at hierarchical bit rates based on a bit rate of 2048 kbit/s | G.920–G.929 |
| Digital line transmission systems on cable at non-hierarchical bit rates | G.930–G.939 |
| Digital line systems provided by FDM transmission bearers | G.940–G.949 |
| Digital line systems | G.950–G.959 |
| Digital section and digital transmission systems for customer access to ISDN | G.960–G.969 |
| Optical fibre submarine cable systems | G.970–G.979 |
| Optical line systems for local and access networks | G.980–G.989 |
| Access networks | G.990–G.999 |
| QUALITY OF SERVICE AND PERFORMANCE - GENERIC AND USER-RELATED ASPECTS | G.1000–G.1999 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.6000–G.6999 |
| DIGITAL TERMINAL EQUIPMENTS | G.7000–G.7999 |
| DIGITAL NETWORKS | G.8000–G.8999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.972 Definition of terms relevant to optical fibre submarine cable systems

Summary This Recommendation applies to optical fibre submarine cable systems. The purpose of this Recommendation is to provide definitions of terms relevant to optical fibre submarine cable systems, including terms relevant to system configuration, system aspects, terminal equipment, optical submarine repeaters and branching units, optical fibre submarine cable, manufacturing and installation, and the maintenance of the submarine portion. Annex A is the alphabetical list of terms defined in this Recommendation.

Source ITU-T Recommendation G.972 was approved on 13 June 2004 by ITU-T Study Group 15 (2001-2004) under the ITU-T Recommendation A.8 procedure.

History 1.0 G.972 1993-03-12 2.0 G.972 1997-04-08 3.0 G.972 2000-10-06 4.0 G.972 2004-06-13

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g. interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met.  The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2004

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS

Page

1	Scope		1

2	References		1

2.1	Normative reference		1

3	Terms and definitions		2

3.1	Terms relevant to elements of system configuration		2

3.2	Terms relevant to system aspects		5

3.3	Terms relevant to terminal equipment		8

3.4	Terms relevant to optical submarine repeaters and branching units		9

3.5	Terms relevant to optical fibre submarine cable		9

3.6	Terms relevant to manufacturing and installation		11

3.7	Terms relevant to the maintenance of the submarine portion		12

4	Abbreviations		12

Annex A – Alphabetical list of terms defined in this Recommendation		14

ITU-T Recommendation G.972

Definition of terms relevant to optical fibre submarine cable systems

# 1	Scope

The purpose of this Recommendation is to provide definitions of terms relevant to optical fibre submarine cable systems.

A small number of terms defined in this Recommendation are associated with definitions in ITU-T Recs G.601, G.602 and G.701. Reference to these definitions are given in parenthesis as an aid to ensure consistency between the different Recommendations in the event of future amendments.

Figure 1/G.971 shows the basic concept of optical fibre submarine cable systems and boundaries. Optical submarine repeaters or optical submarine branching units could be included, depending on each system requirement.

In Figure 1/G.971, A denotes the system interfaces at the terminal station (where the system can be interfaced to terrestrial digital links or to other submarine cable systems), and B denotes beach joints or landing points. Letters in brackets in the definitions of this Recommendation refer to the above figure.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

## 2.1	Normative reference

–	ITU-T Recommendation G.601 (1988), Terminology for cables.

–	ITU-T Recommendation G.602 (1988), Reliability and availability of analogue cable transmission systems and associated equipments.

–	ITU-T Recommendation G.701 (1993), Vocabulary of digital transmission and multiplexing, and  pulse code modulation (PCM) terms.

–	ITU-T Recommendation G.971 (2004), General features of optical fibre submarine cable systems.

–	ITU-T Recommendation G.976 (2004), Test methods applicable to optical fibre submarine cable systems.

# 3	Terms and definitions

This Recommendation defines the following terms:

## 3.1	Terms relevant to elements of system configuration

1001	optical fibre submarine cable system: A set of equipment designed to permit the interconnection of two or more terminal stations.

The optical fibre submarine cable system is usually composed of terminal equipment (Terminal Transmission Equipment, Power Feeding Equipment, Maintenance Controller, etc.), and submersible equipment [Cable, Repeater(s), Branching Unit(s), etc.].

1002	optical fibre submarine cable link: A link which interconnects two terminal stations using a single optical fibre submarine cable system or an integrated system using system portions supplied by different suppliers.

1003	optical fibre submarine cable network: A network which interconnects three or more terminal stations using a single optical fibre submarine cable system or an integrated system made of system portions supplied by different suppliers.

1004	land portion: The portion between the system interface in the terminal station (A), and the beach joint or landing point (B) when it exists. It includes the optical fibre land cable, land joints, and the system terminal equipment.

1005	submarine portion: The portion of the system laying on the seabed, between the beach joints or landing points (B), which includes the optical fibre submarine cable and the submarine equipment (e.g., optical submarine repeater(s), optical submarine branching unit(s), and optical submarine cable jointing box(es)).

1006	beach joint: The cable joint made between the optical fibre submarine cable and the optical fibre land cable.

1007	terminal station: The telecommunication station usually located in the vicinity of the landing point and housing the optical fibre submarine cable system terminal equipment and that of associated terrestrial systems.

1008	Terminal Station Equipment (TSE): The TSE comprises the STE and the ancillary equipment used for the operation of the optical fibre submarine cable system.

1009	System Terminal Equipment (STE): The STE comprises all equipments specific to a given optical fibre submarine cable system.

1010	Terminal Transmission Equipment (TTE): The equipment terminating the optical submarine transmission line at the optical interface, and connected to the system interface.

1011	supervisory system: The whole of equipment and subassemblies commonly providing one or more of the following functions:

–	monitoring the performance of the submarine equipment and subassemblies;

–	monitoring the performance of the system terminal equipment;

–	monitoring the end-to-end performance of the digital line sections;

–	enabling fault location inside the submarine plant, to within one repeater section, where possible;

–	controlling redundancy switching, if provided inside the submarine portion;

–	providing interfaces to other management facilities.

1012	maintenance controller: A computer with an interface to the supervision and remote maintenance equipment, which is commonly used during the supervision and remote maintenance activity.

1013	Power Feeding Equipment (PFE): The equipment providing, through a power conductor in the optical fibre submarine cable, a stabilized constant electrical current for powering optical submarine repeaters and/or optical submarine branching units.

1014	Cable Terminating Equipment (CTE): The equipment providing the interface between the optical fibre from the TTE and the optical fibre cable, and the interface between the power feeding line from the PFE and the power feeding conductor from the optical fibre cable.

NOTE – The CTE is usually part of the PFE.

1015	power feed earth: The earth provided to establish the return path of the repeater power feeding current circuit.

1016	power feed earth electrode cable: The cable installed between the power feed earth and the PFE.

1017	optical fibre station cable: The optical fibre cable installed inside the cable station.

1018	optical fibre land cable: The optical fibre cable installed between the CTE and the beach joint.

1019	optical fibre submarine cable: The submarine cable using optical fibres as transmission line.

1020	optical submarine repeater: (Related to definition 1001 provided in ITU-T Rec. G.601.) An equipment essentially including one or more regenerators or amplifiers and associated devices, in the submarine portion.

1021	bipolar repeater: An optical submarine repeater which can be powered by a power feeding current flowing in either direction.

1022	Branching Unit (BU): An equipment connecting more than two optical fibre submarine cable sections.

NOTE – Applications for the optical submarine branching may include the following additional functions:

–	optical signal path switching;

–	signal regeneration or amplification;

–	power path switching.

1023	Undersea Branching Multiplexer (UBM): A branching unit that separates lower level digital signals from incoming optical line signals and recombines them differently in outgoing optical line signals.

1024	integration specification: A set of specifications established to permit the interconnection of two or more optical fibre submarine cable systems designed by different suppliers.

1025	integration line optical interface: The optical interface between fractions of the submarine cable sections provided by different suppliers.

1026	shallow water: Water depths down to a given limit, corresponding to the depths of fishing activity, or more generally of marine activity, creating a risk of cable fault.

NOTE – The limit of shallow water is of the order of 1000 metres.

1027	deep water: Water depths exceeding the limit of shallow water.

1028	elementary cable section: The whole length of optical fibre cable between two pieces of equipment (repeaters, branching units or terminal transmission equipment).

1029	repeatered submarine cable: An electrically powered underwater optical fibre cable, designed for repeatered applications, and suitable for shallow and deep water use, which has been extensively tested to show it can be installed and repaired in situ, even in worst weather conditions, without any impairment of optical, electrical or mechanical performance or reliability.

NOTE – For appropriate test methods, please refer to ITU-T Rec. G.976.

1030	repeaterless submarine cable: An underwater optical fibre cable, designed for unrepeatered applications, and suitable for shallow and deep water use, which has been extensively tested to show it can be installed and repaired in situ, even in worst weather conditions, without any impairment of optical, electrical or mechanical performance or reliability.

NOTE 1 – For appropriate test methods, please refer to ITU-T Rec. G.976.

NOTE 2 – The term "repeaterless" is also known as "unrepeatered" or "non-repeatered".

1031	Marinized Terrestrial Cable (MTC): An underwater optical fibre cable construction, based on a conventional multi-fibre terrestrial cable core protected to withstand the marine environment, designed for unrepeatered applications and tested for use in non-aggressive shallow waters with varying repair capability.

NOTE 1 – Sometimes known in Europe as an "Underwater" cable.

NOTE 2 – Appropriate test methods are under study.

1032	Single Wavelength Systems (SWS): A bidirectional optical system that carries on only one LOC.

1033	Wavelength Division Multiplexing (WDM): An aggregate of several LOCs to be carried through part or the whole of submarine line on the same line fibre.

1034	Wavelength Division Multiplexing System (WDMS): A bidirectional optical system that carries on several LOCs.

1035	Dense Wavelength Division Multiplexing (DWDM): An aggregate of large number of LOCs to be carried through part or the whole of the submarine line on the same line fibre.

1036	Dense Wavelength Division Multiplexing System (DWDMS): A bidirectional optical system that carry on a large number of LOCs.

1037	gain equalizer: Gain equalizer is the means used to adapt the submerged plant gain profile characteristics suitable for transmission.

1038	tilt equalizer: Tilt equalizer is the means for WDMS to provide the equalization of the residual gain/wavelength tilt which accumulates when the signal is transmitted through a chain of submerged repeaters.

1039	slope equalizer: Slope equalizer is the means for WDMS to provide the equalization of the residual gain/wavelength slope which accumulates when the signal is transmitted through a chain of submerged repeaters.

1040	shunt fault: A shunt fault is a current leakage path between the power conductor and the sea water without a break in the power conductor.

1041	Remotely Pumped Optical Amplifier (RPOA): An OFA consisting of a section of erbium doped fibre that is activated by a pump beam sent from the terminal station.

1042	Distributed Raman Amplifier (DRA): An OFA using the transmission fibre, as an amplifier medium, that is pumped from the terminal station. The gain is obtained all along the fibre (therefore distributed) using the Raman properties of the fibre until sufficient pump power is available.

1043	Raman gain coefficient: For further study.

## 3.2	Terms relevant to system aspects

2001	system design life: The period of time over which the optical fibre submarine cable system is designed to operate in conformance with its performance specification.

2002	optical power budget: The allocation of the available optical power in an optical section.

2003	digital line section: (Related to definitions 3007 and 3012 provided in ITU-T Rec. G.701.) The whole of the means of transmission of a digital signal at a specified bit rate between the corresponding input and output system interfaces at the digital distribution frame or equivalent. A digital line section forms a part of a digital link and includes terminal equipment, repeaters and branching units. The definition normally applies to the combination of "go" and "return" directions of transmission.

NOTE – This definition differs from the definition of ITU-T Rec. G.701 as the latter excludes multiplexers. In an optical fibre submarine cable system, a digital line section is meant to include optical submarine branching multiplexers, if any, and also terminal multiplexing equipment included in the Terminal Transmission Equipment (TTE) which may include multiplexing.

2004	optical section: The whole of the optical path as physical transmission medium between the optical source output (point O) of one equipment and the optical detector input (point I) of the following equipment.

2005	optical source output: The point where the launched power is measured (point O).

2006	optical detector input: The point where the received optical power is measured (point I).

2007	supervisory section: The fraction of the submarine portion extending from one specified point in a repeater to a similar point in the adjacent repeater which can be identified using the supervisory system for fault location.

2008	system interface: (Related to definition 1008 provided in ITU-T Rec. G.701.) The point (A) on the specified equipment, such as the digital distribution frame, at which each digital line section terminates. The interfaces are usually designated as Ii, pertaining to the incoming tributaries, and Io, pertaining to the outgoing tributaries.

2009	optical interface: (Related to definition 1008 provided in ITU-T Rec. G.701.) A common boundary between two associated parts of an optical section.

2010	transmit terminal optical interface: The optical interface at the output port (point S) of a transmit terminal equipment.

2011	receive terminal optical interface: The optical interface at the input port (point R) of a receive terminal equipment.

2012	repeater (or BU) optical output interface: The optical interface at the output port (point S) of a repeater (or BU).

NOTE – The output port is usually at the splice between the repeater fibre and the cable fibre.

2013	repeater (or BU) optical input interface: The optical interface at the input port (point R) of a repeater (or BU).

NOTE – The input port usually is at the splice between the repeater fibre and the cable fibre.

2014	optical line signal: The optical signal transmitted on one optical fibre in the submarine portion.

2015	line frame: (Related to definition 4007 provided in ITU-T Rec. G.701.) A cyclic set of consecutive time slots in the optical line signal, in which the relative position of each time slot can be identified.

NOTE – The line frame results from the multiplexing and coding operations performed by the TTE, taking into account the inclusion of the service and supervisory channels.

2016	system interface bit rate: (Related to definition 2013 provided in ITU-T Rec. G.701.) The number of bits (binary digits) per unit time in the digital signal at the system interface.

NOTE – Several interface bit rates may coexist for one single optical fibre submarine cable system.

2017	line error ratio: The error ratio as could be observed using appropriate means at a given location in the submarine portion.

2018	apparent line error ratio: The value of the line error ratio obtained using the supervisory equipment of the submarine optical fibre cable system.

2019	electroding: The use of undersea equipment for the detection of a low frequency modulation of the power feeding current for cable location purposes.

2020	power feeding current: The stabilized electric current provided by the PFE through the cable power conductor with return through the power feed earth.

2021	line bit rate: (Related to definition 2014 provided in ITU-T Rec. G.701.) The number of signal elements of the optical line signal per unit time.

NOTE – The line bit rate results from the multiplexing and coding operations performed by the TTE, taking into account the inclusion of the service and supervisory channel.

2022	line code: (Related to definition 9002 provided in ITU-T Rec. G.701.) A code chosen to suit the characteristics of the submarine portion, that defines the equivalence between sets of digits presented for transmission and the corresponding sequence of signal elements transmitted over that channel.

NOTE – The line code and violations of the line code can be used for system monitoring and supervisory.

2023	system performance penalty: A parameter included in the optical power budget to take into account phenomena such as chromatic dispersion, partition noise, etc.

2024	cable section margin: A parameter included in the optical power budget, to take into account the variation of the fibre attenuation due to ageing during the system design life.

2025	equipment margin: A parameter included in the optical power budget to take into account the variation of the optical power at both ends of a cable section due to equipment component ageing during the system design life.

2026	repair allowance: A parameter included in the optical power budget to take into account the possible increase of attenuation of the cable fibre due to cable repair during the system design life.

2027	unassigned margin: A parameter included in the power budget as a provision for unforeseen phenomena.

2028	guaranteed margin: The minimum margin in the power budget.

2029	overload margin: The minimum difference between the received power and the input power above which the bit error ratio would exceed a specified level.

2030	expected ship repair number: The mean number of repairs by cable ship (statistical expectation) due to system failures during the system design life.

NOTE – This excludes faults due to external aggression.

2031	reliability budget: A reliability model indicating the reliability of each of the components of the submarine portion to be met so as to keep the expected ship repair number below a given limit.

2032	availability: (See the definitions provided in ITU-T Rec. G.602.) The ability of the system to be in a state to perform adequately at a given instant of time within a given time interval. The availability of an optical fibre submarine cable system is quantified by the ratio of the time during which the system is operating to a specified total time.

NOTE – System interruptions due to faults requiring ship repair are excluded for availability calculation.

2033	end-to-end performance monitoring: The evaluation of the end-to-end transmission quality performance of each digital line section in the submarine optical fibre cable transmission system.

2034	supervisory of an optical fibre submarine cable system: The function provided by the supervisory system of the optical fibre submarine cable system so as to permit fault localization, repeater performance monitoring and remote-controlled redundancy switching.

2035	Forward Error Correction (FEC): A technique which consists in transmitting the data in an encoded form such that the redundancy added by the coding allows the decoding to detect and correct errors.

2036	FEC frame: In frame-structured coding algorithms, the cyclic set of consecutive time slots delivered by the FEC encoder which, at a minimum, contains a frame alignment word, the data at system interface (A) and the redundant information computed with the FEC algorithm.

2037	Q factor: A measurement of the quality of the received signal (see ITU-T Rec. G.976).

2038	Line Optical Channel (LOC): A bidirectional optical data channel carried on a specific optical frequency/wavelength for each transmission direction.

2039	Submarine Cable Optical Interface (SCOI): The bidirectional optical interface between the submarine cable including the terrestrial cable section and the TTE.

2040	LOC-TTE: A TTE whose SCOI is composed of only one LOC.

2041	WDM-TTE: A TTE equipped with WM and WD, whose SCOI is a WDM.

2042	Submarine Electro-Optic Interface (SEOI): The bidirectional interface inside the TTE where an electro-optic conversion and an electrical generation are performed between a LOC and an electrical channel.

2043	Submarine Digital Line Section (SDLS): A bidirectional continuous optical path along which one LOC links two TTE at the SEOI level.

2044	Terrestrial Interface (TI): The interface between the submarine system and the terrestrial network.

2045	Intermediate Terrestrial Interface (ITI): It is to be noted that the TTE can be composed of two distinct pieces of equipment interfaced together, the first piece called Submarine Cable Transmission Terminal Equipment (SCTTE), facing with submarine cable, and the second piece, called Terrestrial Network Transmission Terminal Equipment (TNTTE), facing the terrestrial network. In this case, an intermediate interface is required, which links the two pieces of equipment. This interface is composed of bidirectional data interfaces and, where applicable, of an extra link used to exchange information between the two pieces of TTE equipment.

2046	dispersion map: For further study.

## 3.3	Terms relevant to terminal equipment

3001	service channel: A communication channel established between associated terminal stations through the optical fibre submarine cable system for the purpose of operating and maintaining the system.

3002	order wire channel: A voice service channel.

3003	PFE earth protection equipment: A protective device that automatically routes the power feeding current to the station earth under abnormal PFE earth conditions.

3004	PFE personnel protection equipment: Protective equipment installed to prevent personnel from gaining access to dangerous potentials.

3005	PFE mutual protection: The ability of a power feeding unit installed at one end of a link to provide in an emergency situation the whole of the power necessary for a given link, when in normal operating condition that total power is shared between the power feeding equipment installed at both ends of the link.

3006	PFE nominal current: The nominal value of the power feeding current.

3007	maximum PFE current: The value of the power feeding current above which the PFE switches off in order to protect the system.

3008	maximum PFE voltage: The value of the voltage at the PFE output above which the PFE switches off in order to protect the system.

3009	PFE current stability: The limitation of the variation of the PFE current in time.

3010	FEC encoder: A unit included in the transmit TTE which accomplishes all the digital operations needed to translate the digital data at system interface (A) into the desired coded bit sequence according to the employed FEC algorithm.

3011	FEC decoder: A unit included in the receive TTE which accomplishes the correction of the data transmitted through the submarine portion into symbol decisions that reproduce, as accurately as possible, the data that was encoded by the FEC encoder.

3012	Wavelength Multiplexer (WM): The equipment required to combine several LOCs and/or WDM coming from different fibres into a common WDM composed of all the combined LOCs.

3013	Wavelength Demultiplexer (WD): The equipment required to split a WDM into several LOCs and/or WDM to be carried on different fibres.

3014	Submarine Cable Transmission Terminal Equipment (SCTTE): For further study.

3015	Terrestrial Network Transmission Terminal Equipment (TNTTE): For further study.

3016	umbilic: The extra link used at ITI to exchange information between the two pieces of TTE which are the SCTTE and the TNTTE.

3017	outer code: For further study.

3018	inner code: For further study.

3019	hard decision decoding: Hard decision decoding refers to the use of a single quantization level in bit sampling, resulting in binary bit information provided to the decoding process.

3020	soft decision decoding: For the same received waveform, soft decision decoders make use of multiple levels of quantization (typically three or more).

3021	coding gain: Coding gain means the improvement of received optical sensitivity by FEC, without considering penalty by bit rate increasing.

3022	net coding gain: Net coding gain means the improvement of received optical sensitivity by FEC, with considering penalty by bit rate increasing.

3023	Q limit: Q limit means the minimum required allowable Q factor of the input signal for the receiver decision circuit in order to achieve a reference BER.

3024	redundancy ratio: Redundancy ratio describes the ratio between the number of information bits covered by the FEC and the number of bits after encoding comprising the same information plus the added redundancy information.

3025	latency: Latency means transmission delay for FEC encoding, decoding, interleaving and de-interleaving.

## 3.4	Terms relevant to optical submarine repeaters and branching units

4001	repeater (or BU) housing: The mechanical piecepart of a repeater (or branching unit).

4002	repeater (or BU) optoelectronic unit: The optoelectronic piecepart of a repeater (or branching unit).

4003	repeater (or BU) supervisory circuit: Electronic circuits installed in a repeater (or branching unit) to perform, in association with the supervisory equipment installed in the terminal station, system supervision and fault localization.

4004	repeater (or BU) power supply and protection circuit: The electronic circuits installed in a repeater (or BU) to power in association with the PFE the repeater (or BU) electronic unit with a regulated voltage, and to provide a protection against electrical discharges due to cable discharge or lightning.

4005	branching unit sea electrode: An electrode provided at a BU to establish the return path of the power feeding current through the sea.

4006	Full Fibre Drop BU (FFD-BU): A BU where the optical interconnection between the three submarine cables is made by physically connecting fibre pairs between any two cables.

4007	WDM-BU: A BU where the optical interconnection between the three submarine cables made through WM and WD, that is adding and dropping one or more LOCs out the N-WDM.

## 3.5	Terms relevant to optical fibre submarine cable

5001	lightweight cable: Cable suitable for laying, recovery and operation, where no special protection is required.

5002	lightweight protected cable: Lightweight cable with an additional protective layer.

NOTE – This cable is suitable for laying, recovery and operation in areas where cable erosion or fishbite risk is significant.

5003	single armoured cable: A cable with a single layer of protective armour.

NOTE – This cable is suitable for laying, burial, recovery and operation and is suitably protected for specific area in shallow water.

5004	double armoured cable: A cable with a double layer of protective armour.

NOTE – This cable is suitable for laying, burial, recovery and operation and is suitably protected for specific area in shallow water.

5005	rock armoured cable: A cable with a multiple layer of protective armour (usually two), the outer layer being wound with a small pitch.

NOTE – This cable is suitable for laying, recovery and operation and is suitably protected for specific area in shallow water.

5006	land cable: Cable with suitable protection for installation in duct or direct burying in ground.

5007	cable breaking load (CBL): The minimum guaranteed breaking strength of the cable, with ends fixed, taking into account material and dimensional tolerances of the cable components.

5008	fibre-breaking cable load: The load which when applied longitudinally to the cable with fixed ends results in an instantaneous fibre break.

5009	Nominal Permanent Tensile Strength (NPTS): The maximum permanent cable tension that does not significantly reduce the system performance, lifetime and reliability. NPTS represents the maximum residual tension that may be permanently applied to the cable on the seabed after installation.

NOTE – NPTS is also known elsewhere as the maximum permanent cable load.

5010	Nominal Operating Tensile Strength (NOTS): The maximum average operational tension which the cable can support for the period required for marine operations (typically 48 hours) without significantly reducing the system performance, lifetime and reliability. NOTS represents the maximum average operational tension during installation or repair.

NOTE – NOTS is also known elsewhere as the operational cable load.

5011	Nominal Transient Tensile Strength (NTTS): The maximum short-term tension that can be applied to the cable during an at-sea recovery operation over a cumulative period of approximately one hour without significantly reducing the system performance, lifetime and reliability. NTTS represents the maximum transient or unexpected tension that may be applied to the cable, and is normally limited to a percentage of the CBL from a mechanical safety point of view.

NOTE – NTTS is also known elsewhere as the transitory cable load.

5012	longitudinal water propagation: The cable length being penetrated by water as a function of water depth or pressure and of duration of submission for an open-ended cable.

5013	cable jointing box: A mechanical assembly used to join two optical fibre submarine cables.

5014	cable coupler (or cable termination): A mechanical assembly used to join an optical fibre submarine cable to a repeater or a branching unit.

5015	cable transition: A transition between different types of cables, which can be achieved using either a transition cable or a transition joint.

5016	cable gyration: The rotation of one end of a cable length relative to the other end.

5017	cable elongation with fixed end: The cable elongation under a load, when both ends are prevented from rotating.

5018	cable elongation with free gyration: The cable elongation under a load, when one end is free to rotate.

5019	cable modulus: The length of cable, measured in km, the weight of which in water is equal to the cable breaking load.

5020	cable operational modulus: The length of cable, measured in km, the weight of which in water is equal to the Nominal Operation Tensile Strength (NOTS).

5021	cable full safe modulus: The length of cable, measured in km, the weight of which in water is equal to the Nominal Permanent Tensile Strength (NPTS).

5022	cable transitory modulus: The length of cable, measured in km, the weight of which in water is equal to the Nominal Transient Tensile Strength (NTTS).

5023	cabled fibre attenuation: The optical attenuation of the cable, including any fibre overlength and fibre splices present, as measured in dB per length of cable.

5024	cabled fibre strain: The strain in an optical fibre in a cable after manufacture.

5025	cable safety margin: The cable safety margin is the difference in tensile load between the measured load at failure and the maximum installation/recovery load proposed by the installer. The cable safety margin may typically be expressed as a percentage of measured load at failure.

5026	duct shielded cable: A cable with a circumferential safety shield which may be the fish-bite protection shield, and which is suitable to be pulled into ducts.

5027	tight cable structure: A cable structure where the fibres are strongly maintained in the cable, so that the fibre elongation is essentially equal to that of the cable.

5028	loose cable structure: A cable structure where the fibres are free to move inside the cable, so that the fibre elongation is lower than that of the cable, staying zero until the cable elongation reaches a given value.

5029	transitory cable load: The load, which could be accidentally encountered, particularly during recovery operations.

5030	operational cable load: The load, which could be encountered during repairs.

5031	permanent cable load: The load, which characterizes the status of cable after lay.

5032	minimum cable bending radius: The bending radius which is a guideline for cable handling.

## 3.6	Terms relevant to manufacturing and installation

6001	qualification: The activity, part of the development process, intended to demonstrate that a technology, a component or an assembly is able to comply with its performance and reliability specification.

6002	sea trial: A test made at sea during the development process, as part of the qualification of the submarine portion.

6003	certification: The activity, part of the manufacturing process, intended to eliminate all components or subassemblies which present a risk of not complying with their performance and reliability specification.

6004	batch qualification: The activity, part of the manufacturing process, intended to eliminate batches of components suspect to present a generalized failure mechanism.

6005	route survey: The activity performed prior to cable laying so as to select the cable route and cable protection (fishbite protection, armour, burying).

6006	link assembly: The activity of jointing the cable sections, the repeaters and the branching units, together with monitoring the performance of each cable section.

6007	ship loading: The operation of installing the submarine portion or fractions of it on board the cable ship prior to laying.

6008	cable laying: The operation of laying on the sea bottom the submarine portion or fractions of it.

6009	laying limit condition: Weather and sea condition above which laying should not be undertaken or should be interrupted.

6010	slack control: Operation consisting in controlling that a predetermined cable overlength (slack) is layed.

6011	shore end: The end of the submarine cable, to be laid from the cableship toward the beach so as to be connected to the land cable.

6012	initial splice: The splice made on board the cableship between parts of the submarine portion at the beginning of a lay.

6013	final splice: The splice made on board the cableship between parts of the submarine portion at the end of a lay.

6014	cable burial: The operation consisting in burying the cable in the seabed so as to provide better cable protection.

NOTE – The burial operation may be carried out either during installation or as a post-lay activity.

6015	laying angle: The angle between the cable being laid (which is theoretically along a straight line) and the sea surface.

6016	manufacturing inspection: The activity during the manufacturing process intended to verify that the Quality Plan is respected, that each operation is accomplished along the agreed procedure, and that the result is satisfactory.

6017	commissioning testing: A test prior to installing traffic on the system to ensure that the system meets its overall transmission performance contractual requirement, and that all functionalities with respect to the network management are operating.

## 3.7	Terms relevant to the maintenance of the submarine portion

7001	cable recovery: The operation of recovering a cable from the sea bottom.

7002	deep sea repair: The operation of repairing a submarine cable system installed in deep sea.

7003	shallow water repair: The operation of repairing a submarine cable system installed in shallow water.

7004	mini system: A length of cable, pre-equipped with repeaters, to be used for replacing one or more optical cable sections and the associated repeaters during a sea repair.

7005	spare repeater (or BU): An additional repeater (or BU) stored in a marine depot or on board a cableship to be used during a repair operation.

7006	spare cable: An additional cable length, stored in a marine depot or on board a cableship to be used during a repair operation.

7007	spare storage condition: The environment condition for storing the spare repeaters and cable.

7008	repair safety procedure: Procedures to be applied on board the cable ship and in the terminal station, so as to ensure the safety of the personnel during a repair.

7009	submersible robot: A remote-controlled submersible vehicle which can be used for locating, observing, post-lay burying, or recovering a cable installed in shallow water.

7010	routine maintenance: The periodic monitoring of the system parameters and preventive redundancy switching performed from the terminal station using the supervisory system.

7011	fault localization: For further study.

7012	fault location: For further study.

# 4	Abbreviations

This Recommendation uses the following abbreviations.

BU	Optical Submarine Branching Unit

CBL	Cable Breaking Load

COTDR	Coherent Optical Time Domain Reflectometry

CTE	Cable Terminating Equipment

DRA	Distributed Raman Amplifier

DWDM	Dense Wavelength Division Multiplexing

DWDMS	Dense Wavelength Division Multiplexing System

FEC	Forward Error Correction

FFD-BU	Full Fibre Drop Branching Unit

ITI	Intermediate Terrestrial Interface

LOC	Line Optical Channel

MTC	Marinized Terrestrial Cable

NOTS	Nominal Operating Tensile Strength

NPTS	Nominal Permanent Tensile Strength

NTTS	Nominal Transient Tensile Strength

OFA	Optical Fibre Amplifier

PFE	Power Feeding Equipment

RPOA	Remotely Pumped Optical Amplifier

SCOI	Submarine Cable Optical Interface

SCS	Single Channel System

SCTTE	Submarine Cable Transmission Terminal Equipment

SDLS	Submarine Digital Line Section

SEOI	Submarine Electro-Optic Interface

STE	System Terminal Equipment

SWS	Single Wavelength Systems

TI	Terrestrial Interface

TNTTE	Terrestrial Network Transmission Terminal Equipment

TSE	Terminal Station Equipment

TTE	Terminal Transmission Equipment

UBM	Undersea Branching Multiplexer

WD	Wavelength Demultiplexer

WDM	Wavelength Division Multiplexing

WDM-BU	Wavelength Division Multiplex-Branching Unit

WDMS	Wavelength Division Multiplexing System

WDM-TTE	Wavelength Division Multiplex-Terminal Transmission Equipment

WM	Wavelength Multiplexer

Annex A

Alphabetical list of terms defined in this Recommendation

2018	Apparent line error ratio

2032	Availability

6004	Batch qualification

1006	Beach joint

1021	Bipolar repeater

1022	Branching unit (BU)

4005	Branching unit sea electrode

5007	Cable breaking load (CBL)

6014	Cable burial

5014	Cable coupler (or cable termination)

5017	Cable elongation with fixed end

5018	Cable elongation with free gyration

5021	Cable full safe modulus

5016	Cable gyration

5013	Cable jointing box

6008	Cable laying

5019	Cable modulus

5020	Cable operational modulus

7001	Cable recovery

5025	Cable safety margin

2024	Cable section margin

5024	Cabled fibre strain

1014	Cable terminating equipment (CTE)

5015	Cable transition

5022	Cable transitory modulus

5023	Cabled fibre attenuation

5024	Cabled fibre strain

6003	Certification

3021	Coding gain

6017	Commissioning testing

7002	Deep sea repair

1027	Deep water

1035	Dense wavelength division multiplexing (DWDM)

1036	Dense wavelength division multiplexing system (DWDMS)

2003	Digital line section

2046	Dispersion map

1042	Distributed Raman Amplifier

5004	Double armoured cable

5026	Duct shielded cable

2019	Electroding

1028	Elementary cable section

2033	End-to-end performance monitoring

2025	Equipment margin

2030	Expected ship repair number

7011	Fault localization

7012	Fault location

3011	FEC decoder

3010	FEC encoder

2036	FEC frame

5008	Fibre-breaking cable load

6013	Final splice

2035	Forward error correction (FEC)

4006	Full fibre drop BU (FFD-BU)

1037	Gain equalizer

2028	Guaranteed margin

3019	Hard decision decoding

6012	Initial splice

3018	Inner code

1025	Integration line optical interface

1024	Integration specification

2045	Intermediate terrestrial interface (ITI)

5006	Land cable

1004	Land portion

3025	Latency

6015	Laying angle

6009	Laying limit condition

5001	Lightweight cable

5002	Lightweight protected cable

2021	Line bit rate

2022	Line code

2017	Line error ratio

2015	Line frame

2038	Line optical channel (LOC)

6006	Link assembly

2040	LOC-TTE

5012	Longitudinal water propagation

5028	Loose cable structure

1012	Maintenance controller

6016	Manufacturing inspection

1031	Marinized terrestrial cable (MTC)

3007	Maximum PFE current

3008	Maximum PFE voltage

7004	Mini system

5032	Minimum cable bending radius

3022	Net coding gain

5010	Nominal operating tensile strength (NOTS)

5009	Nominal permanent tensile strength (NPTS)

5011	Nominal transient tensile strength (NTTS)

5030	Operational cable load

2006	Optical detector input

1018	Optical fibre land cable

1017	Optical fibre station cable

1019	Optical fibre submarine cable

1002	Optical fibre submarine cable link

1003	Optical fibre submarine cable network

1001	Optical fibre submarine cable system

2009	Optical interface

2014	Optical line signal

2002	Optical power budget

2004	Optical section

2005	Optical source output

1020	Optical submarine repeater

3002	Order wire channel

3017	Outer code

2029	Overload margin

5031	Permanent cable load

3009	PFE current stability

3003	PFE earth protection equipment

3005	PFE mutual protection

3006	PFE nominal current

3004	PFE personnel protection equipment

1015	Power feed earth

1016	Power feed earth electrode cable

2020	Power feeding current

1013	Power feeding equipment (PFE)

2037	Q factor

3023	Q limit

6001	Qualification

1043	Raman gain coefficient

2011	Receive terminal optical interface

3024	Redundancy ratio

2031	Reliability budget

1041	Remotely pumped optical amplifier (RPOA)

2026	Repair allowance

7008	Repair safety procedure

4001	Repeater (or BU) housing

4002	Repeater (or BU) optoelectronic unit

2013	Repeater (or BU) optical input interface

2012	Repeater (or BU) optical output interface

4004	Repeater (or BU) power supply and protection circuit

4003	Repeater (or BU) supervisory circuit

1029	Repeatered submarine cable

1030	Repeaterless submarine cable

5005	Rock armoured cable

6005	Route survey

7010	Routine maintenance

6002	Sea trial

3001	Service channel

1026	Shallow water

7003	Shallow water repair

6007	Ship loading

6011	Shore end

1040	Shunt fault

5003	Single armoured cable

1032	Single wavelength systems (SWS)

6010	Slack control

1039	Slope equalizer

3020	Soft decision decoding

7006	Spare cable

7005	Spare repeater (or BU)

7007	Spare storage condition

2039	Submarine cable optical interface (SCOI)

3014	Submarine cable transmission terminal equipment (SCTTE)

2043	Submarine digital line section (SDLS)

2042	Submarine electro-optic interface (SEOI)

1005	Submarine portion

2039	Submarine cable optical interface (SCOI)

3014	Submarine cable transmission terminal equipment (SCTTE)

7009	Submersible robot

2034	Supervisory of an optical fibre submarine cable system

2007	Supervisory section

1011	Supervisory system

2001	System design life

2008	System interface

2016	System interface bit rate

2023	System performance penalty

1009	System terminal equipment (STE)

1007	Terminal station

1008	Terminal station equipment (TSE)

1010	Terminal transmission equipment (TTE)

2044	Terrestrial interface (TI)

3015	Terrestrial network transmission terminal equipment (TNTTE)

5027	Tight cable structure

1038	Tilt equalizer

5029	Transitory cable load

2010	Transmit terminal optical interface

3016	Umbilic

2027	Unassigned margin

1023	Undersea branching multiplexer (UBM)

3013	Wavelength demultiplexer (WD)

1033	Wavelength division multiplexing (WDM)

1034	Wavelength division multiplexing system (WDMS)

3012	Wavelength multiplexer (WM)

4007	WDM-BU

2041	WDM-TTE

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure, Internet protocol aspects and Next Generation Networks |
| Series Z | Languages and general software aspects for telecommunication systems |