|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.983.3 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (03/2001) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital sections and digital line system – Optical line systems for local and access networks |  |  |  |
|  | A broadband optical access system with increased service capability by wavelength allocation |  |  |  |
|  | ITU-T Recommendation G.983.3 (Formerly CCITT Recommendation) |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| General | G.900–G.909 |
| Parameters for optical fibre cable systems | G.910–G.919 |
| Digital sections at hierarchical bit rates based on a bit rate of 2048 kbit/s | G.920–G.929 |
| Digital line transmission systems on cable at non-hierarchical bit rates | G.930–G.939 |
| Digital line systems provided by FDM transmission bearers | G.940–G.949 |
| Digital line systems | G.950–G.959 |
| Digital section and digital transmission systems for customer access to ISDN | G.960–G.969 |
| Optical fibre submarine cable systems | G.970–G.979 |
| Optical line systems for local and access networks | G.980–G.989 |
| Access networks | G.990–G.999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.983.3 A broadband optical access system with increased service capability by wavelength allocation

- Summary This Recommendation defines new wavelength allocations to distribute ATM-PON signals and additional service signals simultaneously. New wavelength bands for additional services are made available by constraining the current ATM-PON downstream wavelength to a portion of downstream optical spectrum originally specified in ITU-T G.983.1. The new bands have potential to provide unidirectional and bidirectional services. The wavelength allocation defined in this Recommendation enables the distribution of video broadcast services or data services. The general optical characteristics of these services are taken into account. However, the detailed specifications of these services, such as modulation scheme, signal format, and so on are beyond the scope of this Recommendation.

- Source ITU-T Recommendation G.983.3 was prepared by ITU-T Study Group 15 (2001-2004) and approved under the WTSA Resolution 1 procedure on 15 March 2001.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2001

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Abbreviations		2

4	Definitions		4

5	Architecture of the optical access network		5

5.1	Network architecture		5

5.1.1	FTTCab/C/B scenario		6

5.1.2	FTTH scenario		6

5.2	Reference configuration		6

5.2.1	Service Node Interface		8

5.2.2	Interface at the reference points S/R and R/S		8

5.3	Functional blocks		8

5.4	ONU functional block		9

5.4.1	Optical Distribution Network interface		9

5.4.2	Multiplexing		9

5.4.3	User Port		9

5.4.4	ONU powering		9

5.5	Optical Line Termination functional block		9

5.6	Optical Distribution Network functional block		10

6	Services		10

7	User Network Interface and Service Node Interface		11

8	Optical network requirements		11

8.1	Layered structure of optical network		11

8.2	Physical medium dependent layer requirements for the wavelength allocation		11

8.3	Physical medium dependent layer requirements for the ATM-PON		13

8.3.1	Digital signal nominal bit rate		13

8.3.2	Physical media and transmission method		18

8.3.3	Bit rate		18

8.3.4	Line code		19

8.3.5	Operating wavelength		19

8.3.6	Transmitter at Old and Oru		19

8.3.7	Optical path between Old/Oru and Ord/Olu		19

8.3.8	Receiver at Ord and Olu		20

8.4	Physical medium dependent layer requirements for the Enhancement Band		20

Page

9	Operations Administration and Maintenance (OAM) functionality		21

10	Performance		21

11	Environmental condition		21

12	Safety		21

Appendix I – Optical interface parameters examples for ODN		22

I.1	Introduction		22

I.2	Examples		22

Appendix II – Deployment scenarios		23

II.1	Introduction		23

II.2	Configuration examples		23

II.2.1	Case 1: Reception of Basic Band only		23

II.2.2	Case 2: Reception of Basic Band and Enhancement Band signals by separate ONUs		25

II.2.3	Case 3: Reception of Basic Band and Enhancement Band signals by integrated ONUs		25

II.2.4	Case 4: Arrangement of WDMs to minimize upstream path loss		26

II.2.5	Case 5: ODNs with reduced attenuation range for Enhancement Band		27

II.2.6	Example in case of insufficiency of loss budget between E-ONU and E-OLT		28

Appendix III – Isolation between the Basic Band and the Enhancement Band		28

III.1	Introduction		28

III.2	Example for required isolation at Basic Band receiver from Enhancement Band signal		30

III.2.1	Video services		30

III.2.2	DWDM services		33

III.3	Example of isolation classes		34

III.4	Deployment examples		35

III.4.1	Model to be considered		35

III.4.2	In case of 64-QAM		37

III.4.3	Filter characteristics		39

Appendix IV – Effect of optical return loss of ODN		40

IV.1	Introduction		40

IV.2	Effect of ODN optical return loss in the Basic Band		41

IV.2.1	Reflectance model to be considered		41

IV.2.2	Influence of reflectance into ONU receiver		42

IV.2.3	Influence of reflectance into OLT receiver (in the signal region)		42

Page

IV.2.4	Influence of reflectance into OLT receiver (in the no signal region)		44

IV.2.5	Summary in effects of ODN optical return loss in the Basic Band		44

IV.3	Influence of reflectance from Enhancement Band to Basic Band		47

IV.3.1	Influence of reflectance into OLT receiver from Enhancement Band		47

Appendix V – Optical parameters for reference point (c) and (e) and  two example deployment cases		50

V.1	Introduction		50

V.2	Optical level diagram examples		50

V.2.1	Case 1: ODN centric view of optical parameters		50

V.2.2	Case 2: Equipment centric view of optical parameters		50

NOTE  This Recommendation uses the term "ONU" to refer to both ONTs and ONUs. Any reference to ONUs includes ONTs as well within this Recommendation.

ITU-T Recommendation G.983.3

A broadband optical access system with increased service 
capability by wavelength allocation

# 1	Scope

This Recommendation describes some extended functions for the ATM-PON system defined in ITU-T G.983.1.

This Recommendation describes the Broadband Passive Optical Network (BPON) system that uses wavelength division multiplexing techniques (WDM). These WDM techniques allow operators to provide additional services without disturbing the basic ATM-PON system.

This Recommendation defines new wavelength allocations to distribute ATM-PON signals and additional service signals simultaneously. New wavelength bands for additional services are made available by constraining the current ATM-PON downstream wavelength to a portion of downstream optical spectrum originally specified in ITU-T G.983.1. The new bands have potential to provide unidirectional and bidirectional services. The wavelength allocation defined in this Recommendation enables the distribution of video broadcast services or data services, and the general optical characteristics of these services are taken into account. However, the detailed specifications of these services, such as modulation scheme, signal format, and so on are beyond the scope of this Recommendation.

This Recommendation defines new reference points and optical interface parameters because of the new WDM and/or optical power splitter/combining functions needed at OLT and/or ONU sites.

However, the reference points and Optical Access Network (OAN) described in this Recommendation will remain compatible with the existing ITU-T G.982 and G.983.1.

This Recommendation also defines new environmental conditions required for the ONU equipment. These conditions are required for outside plant applications of the ATM-PON system.

This Recommendation describes the relevant differences from ITU-T G.983.1. However, sections that specify the TC layer are not included in this Recommendation, because they are not relevant to WDM enhancements. The specifications of the TC layer are given by other Recommendations, such as ITU-T G.983.1.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. References cited in ITU-T G.983.1 also apply to this Recommendation but are not reproduced here to avoid redundancy.

[1]	ITU-T G.692 (1998), Optical interfaces for multichannel systems with optical amplifiers.

[2]	ITU-T G.983.1 (1998), Broadband optical access systems based on Passive Optical Networks (PON), plus Corrigendum 1 (1999).

[3]	ITU-T G.983.2 (2000), ONT management and control interface specification for ATM PON.

[4]	ITU-T G.959.1 (2001), Optical transport network physical layer interfaces.

# 3	Abbreviations

This Recommendation uses the following abbreviations:

AF		Adaptation Function

AM-VSB	Amplitude Modulation-Vestigial Side Band

APS		Automatic Protection Switching

ATM		Asynchronous Transfer Mode

BER		Bit Error Ratio

BIP		Bit Interleaved Parity

B-ISDN	Broadband Integrated Services Digital Network

BPON		Broadband Passive Optical Networks

CID		Consecutive Identical Digit

CNR		Carrier-to-Noise Ratio

CPE		Cell Phase Error

CRC		Cyclic Redundancy Check

DSL		Digital Subscriber Line

DWDM		Dense Wavelength Division Multiplexing

E/O		Electrical/Optical

E-OLT		Enhancement Band-Optical Line Termination

E-ONU		Enhancement Band-Optical Network Unit

FP-LD		Fabry-Perot Laser Diode

FTTB/C	Fibre to the Building/Curb

FTTCab	Fibre to the Cabinet

FTTH		Fibre to the Home

HEC		Header Error Control

IEC		International Electrotechnical Commission

ISDN		Integrated Services Digital Network

LAN		Local Area Network

LCD		Loss of Cell Delineation

LCF		Laser Control Field

LSB		Least Significant Bit

LT		Line Terminal

MAC		Media Access Control

MLM		Multi-Longitudinal Mode

MSB		Most Significant Bit

NRZ		Non Return to Zero

NT		Network Termination

O/E		Optical/Electrical

OAM		Operation, Administration and Maintenance

OAN		Optical Access Network

ODF		Optical Distribution Frame

ODN		Optical Distribution Network

OLT		Optical Line Termination

ONT		Optical Network Termination

ONU		Optical Network Unit

OpS		Operations System

ORL		Optical Return Loss

PLOAM	Physical Layer OAM

PON		Passive Optical Network

PRBS		Pseudo-Random Bit Sequence

PST		PON Section Trace

PSTN		Public Switched Telephone Network

QAM		Quadrature Amplitude Modulation

QoS		Quality of Service

QPSK		Quadrature Phase Shift Keying

RAU		Request Access Unit

RIN		Relative Intensity Noise

RMS		Root Mean Square

RXCF		Receiver Control Field

SCM		Sub-Carrier Mulitiplexing

SDH		Synchronous Digital Hierarchy

SLM		Single-Longitudinal Mode

SN		Serial Number

SNI		Service Node Interface

TC		Transmission Convergence

TDMA		Time Division Multiple Access

UI		Unit Interval

UNI		User Network Interface

UPC		Usage Parameter Control

VC		Virtual Channel

VP		Virtual Path

VPI		Virtual Path Identifier

WDM		Wavelength Division Multiplexing

WF1		Wavelength division multiplexing and/or optical combining functions 1

WF2		Wavelength division multiplexing and/or optical splitting functions 2

# 4	Definitions

This Recommendation defines the following terms:

4.1	diplex working: Bidirectional communication using a different wavelength for each direction of transmission over a single fibre.

4.2	duplex working: Bidirectional communication using the same wavelength for both directions of transmission over a single fibre.

4.3	logical reach: The logical reach is defined as the maximum length that can be achieved for a particular transmission system independent of optical budget.

4.4	Optical Access Network (OAN): The set of access links sharing the same network-side interfaces and supported by optical access transmission systems. The OAN may include a number of ODNs connected to the same OLT.

4.5	Optical Distribution Network (ODN): An ODN provides the optical transmission means from the OLT towards the users, and vice versa. It utilizes passive optical components.

4.6	Optical Line Termination (OLT): An OLT provides the network-side interface of the OAN, and is connected to one or more ODNs.

4.7	Optical Network Termination (ONT): An ONU used for FTTH and includes the User Port function.

4.8	Optical Network Unit (ONU): An ONU provides (directly or remotely) the user-side interface of the OAN, and is connected to the ODN.

4.9	user port function: The User Port Function (UPF) adapts the specific UNI requirements to the core and management functions. The AN may support a number of different accesses and user network interfaces which require specific functions according to the relevant interface specification and the access bearer capability requirements, i.e. bearers for information transfer and protocols.

4.10	verification: It is possible for a malicious user to masquerade as other ONU and use the network if the user knows that the ONU is power-off. Verification is a function to check whether the connected ONU is masqueraded by a malicious user.

4.11	Wavelength Division Multiplexing (WDM): Bidirectional multiplexing using different optical wavelength for up and downstream signals and multiplexing using different optical wavelength for Basic Band, Enhancement Band and Future Band signals.

4.12	basic band: Wavelength region allocated for the ATM-PON downstream capabilities.

4.13	enhancement band: Wavelength region allocated for new additional service capabilities, which include at least video services and Dense Wavelength Division Multiplexing (DWDM) services.

4.14	future band: Reserved wavelength region for future use.

4.15	E-OLT: OLT, which provides additional service capabilities including video services and DWDM services in Enhancement Band.

4.16	E-ONU: ONU, which provides additional service capabilities including video services and DWDM services in Enhancement Band.

4.17	WF1: WDM and/or optical combining/splitting functions, which separate/combine wavelength and/or split/combine optical power for ATM-PON transport service and additional services. It is located between ODN and the OLT.

4.18	WF2: WDM and/or optical combining/splitting functions, which separate/combine wavelength and/or split/combine optical power for ATM-PON transport service and additional services. It is located between ODN and the ONU.

4.19	WDM-L: WDM, which separate/combine 1.3 µm region and 1.5 µm region, and is located in OLT side.

4.20	WDM-N: WDM, which separate/combine 1.3 µm region and 1.5 µm region, and is located in ONU side.

4.21	BPON: Broadband Passive Optical Network (BPON). BPONs are one-to-n broadband optical transmission systems. BPONs can transparently transport any type of data, for example voice, video, IP data, etc. The BPON is able to carry data regardless of the type of data link frame (i.e. not only native ATM but also HDL Ethernet frame, etc.).

# 5	Architecture of the optical access network

## 5.1	Network architecture

The optical section of a local access network system could be either a point-to-point, active, or passive point-to-multipoint architecture. Figure 1 shows the architectures considered, which range from Fibre to the Home (FTTH), through Fibre to the Building/Curb (FTTB/C) to Fibre to the Cabinet (FTTCab). The Optical Access Network (OAN) is common to all architectures shown in Figure 1, hence commonality in this system has the potential to generate large worldwide volumes. The FTTB/C and FTTCab network options are predominantly different only as a result of implementation, as a result they can be treated the same in this Recommendation.

Figure 1/G.983.3 – Network architecture

### 5.1.1	FTTCab/C/B scenario

See 5.1.1/G.983.1.

### 5.1.2	FTTH scenario

FTTH; Fibre to the Home service drivers are similar to those of the previous scenarios and are determined by:

	Indoor ONUs can be considered, resulting in more favourable environmental conditions.

	Some changes of intermediate ONU may be required to upgrade access network capabilities to accommodate future evolution of broadband and multimedia services.

	Maintenance is easy, because it requires maintenance only for fibre systems, and all fibre systems are regarded as more reliable than hybrid fibre-metallic ones.

	FTTH is a driver for the development of advanced optoelectronics technologies. The greater volume in production of optical modules will also accelerate the reduction in cost.

When these factors can be fully exploited they may counterbalance a slightly higher per line cost. In that situation the FTTH scenario may be regarded as economically feasible even in the short term.

## 5.2	Reference configuration

The general reference configuration from ITU-T G.983.1 is shown in Figure 2a.

The specific reference configuration for this Recommendation is shown in Figure 2b which provides clarification of the new specification based on the revised wavelength allocation.

The ODN offers one or more optical paths between one OLT and one or more ONUs. Each optical path is defined between reference points S and R in a specific wavelength window. The two directions for optical transmission in the ODN are identified as follows:

–	downstream direction for signals travelling from the OLT to the ONU(s);

	upstream direction for signals travelling from the ONU(s) to the OLT.

This clause describes the reference architecture for supporting Broadband transmission over a PON. This system consists of Optical Line Termination (OLT), Optical Network Unit (ONU), Wavelength division multiplexing and/or optical combining/splitting functions (WF1 and WF2) for the new wavelength allocation plan defined in this Recommendation and fibre cable which has a Passive Optical Network (PON) configuration with a passive optical splitter.

One fibre is passively split between multiple ONUs who share the capacity of one fibre. Because of the passive splitting, special actions are required with respect to privacy and security. Moreover, in the upstream direction a TDMA protocol is required.

WF1 and WF2 are used for separating/combining wavelengths and/or splitting/combining optical power for ATM-PON transport service and additional services such as video broadcast. These may be integrated into or provided as external optical component parts connected to the OLT and the ONU. Therefore, WF1 and WF2 could be regarded as a part of the OLT and the ONU. In the case of integration into the OLT or the ONU, reference points (c) and (e) in Figure 2b are regarded as internal reference points of the OLT and the ONU respectively.

Functionality required for WF1 and WF2 will vary depending upon the particular system configuration, regulations, service strategy, and geographic conditions for each telecom operator. These configurations based on scenarios by operators are described in Appendix II.

Figure 2a/G.983.3 – Reference configuration for an ATM based PON

Figure 2b/G.983.3 – Reference configuration for a Broadband PON

### 5.2.1	Service Node Interface

See ITU-T G.902.

### 5.2.2	Interface at the reference points S/R and R/S

This interface at the reference points S/R and R/S is defined as IFPON This is a PON-specific interface that supports all the protocol elements necessary to allow the transmission between OLT and ONUs.

## 5.3	Functional blocks

See 5.3/G.983.1.

## 5.4	ONU functional block

As an example, the FTTH ONU is active and decouples the access network delivery mechanism from the in-house distribution. The ONU consists of ODN interface, User Port, Transmission, Services, Customers Multiplexing (MUX)/demultiplexing functions and powering, which is described in Figure 3.

Figure 3/G.983.3 – Example of ONU functional blocks

### 5.4.1	Optical Distribution Network interface

The ODN interface handles the optoelectronic conversion process. The ODN interface extracts ATM cells from the downstream PON payload and inserts ATM cells into the upstream PON payload based on synchronization acquired from the downstream frame timing.

The ODN interface may handle the separation/combination of designated wavelengths and/or the optical splitter/combiner.

### 5.4.2	Multiplexing

Multiplexer (MUX) multiplexes service interfaces to ODN interface. Only valid ATM cells can be passed through the MUX, so many VPs can share the assigned upstream bandwidth effectively.

### 5.4.3	User Port

The User Port interfaces over UNI to a terminal. The User Port may handle inserting ATM cells into the upstream payload and extracting ATM cells from the downstream payload. In addition, the User Port may interface for video delivery and/or additional service from the OLT to customers.

### 5.4.4	ONU powering

ONU powering may be implementation dependent.

## 5.5	Optical Line Termination functional block

The OLT is connected to the switched networks via standardized interfaces (VB5.x, V5.x, NNI's). At the distribution side, it presents optical accesses according to the agreed requirements, in terms of bit rate, power budget, etc.

The OLT consists of three parts: the service port function; ODN interface; and MUX for VP grooming (see Figure 4). This combination is not intended to preclude the Virtual Channel (VC) layer function in the OLT. VC layer function is for further study.

1)	Service port function

The function interfaces to service nodes. The service port function may handle inserting ATM cells into the upstream SDH payload and extracting ATM cells from the downstream SDH payload. The function may be duplicated, then a protection switching function is necessary.

2)	MUX

The MUX provides VP connections between the service port function and the ODN interface and different VPs are assigned to different services at IFPON. Various information such as main contents, signalling, and OAM flows are exchanged by using VCs of the VP.

3)	ODN interface

The ODN interface handles the optoelectronic conversion process. The ODN interface handles inserting ATM cells into the downstream PON payload and extracting ATM cells from the upstream PON payload. In addition, the ODN interface may handle the separation/combination of designated wavelengths and/or the optical splitter/combiner.

Figure 4/G.983.3 – Example of OLT functional blocks

## 5.6	Optical Distribution Network functional block

See 5.6/G.983.1.

# 6	Services

Such a high speed access system could provide the complete range of all currently known and new services being discussed for the residential subscribers and business customers. Here transmission system's service independence has to be regarded.

These services cover a broad range of network requirements like bit rate, symmetry/asymmetry or delay, and range from video distribution, with varying degrees of interactivity, to electronic data transfer, LAN interconnection, transparent Virtual Paths, and so on.

What specific services should be provided is clearer to some operators than to others and depends heavily on the particular regulatory conditions of each operators' markets, as well as on the own markets potential. How these services are delivered in a cost effective way is a function not only of legal conditions, but also of factors including existing telecommunications infrastructure, dwelling distribution and residential and business customers mix.

In spite of this varied market background there are some features which have been perceived as common to all parties. They can be summarized as follows:

–	some services require bit rates higher than those supported by PSTN and basic ISDN. These services are best delivered on fibre-based or fibre hybrid networks;

	as the services evolve and newer services are introduced, the bandwidth and management requirements will increase. This requires the access network to be flexible and easily upgradable;

	additional services, such as video distribution service overlaid ATM-PON, can be provided in an appropriate manner using the WDM scheme, where necessary. The choice of video modulation type will have an impact on the system requirements, but the video modulation system is beyond the scope of this Recommendation.

# 7	User Network Interface and Service Node Interface

The location of UNI and SNI is preliminary given in the reference configuration (see Table 1).

Other types of services such as video broadcast are for further study.

Table 1/G.983.3 – UNI and SNI

| Service type | UNI standard | SNI standard |
| --- | --- | --- |
| Defined in ITU-T G.982 | ITU-T G.902 | ITU-T G.902 |
| – Digital broadband video system – Multimedia services – VP leased line – ATM SVC | ITU-T I.432 IEEE 802.3 | ITU-T G.967.1 ITU-T G.967.2 |

# 8	Optical network requirements

## 8.1	Layered structure of optical network

See 8.1/G.983.1.

## 8.2	Physical medium dependent layer requirements for the wavelength allocation

The wavelength of the ATM-PON downstream signal (single fibre system) is in the 1.5 µm band and that of the ATM-PON upstream signal is in the 1.3 µm band in ITU-T G.983.1. This Recommendation defines the new wavelength allocation to enable additional new services using the wavelength band defined in ITU-T G.983.1. The ATM-PON downstream wavelength band shall be compressed within the original 1.5 µm band and new wavelength bands shall be assigned for new additional services.

Figure 5 and Table 2 define the new wavelength allocation. This reduces the ATM-PON downstream allocation currently defined in ITU-T G.983.1 to a portion of the available downstream optical spectrum. This portion for the ATM-PON shall be referred to as the "Basic Band". An additional wavelength band will then be made available and be referred to as the "Enhancement Band." Uses for the Enhancement Band include at least video services and Wavelength Division Multiplexing (WDM) services. A guardband separates the Basic Band from the Enhancement Band. The interference between signals in these two bands causes signal degradation to each other, which shall be kept negligible. Required isolation between these bands depends on optical power level, modulation scheme and so on. An example of WDM filters is described in Appendix III.

The specifications for Future L Band in the 1.5 µm band, and the "Reserved for Allocation by ITU-T" band in the Intermediate region, are for further study.

NOTE  Wavelengths in Enhancement Band may be used not only for downstream but also for upstream signal transmission in the WDM scheme.

Figure 5/G.983.3 – Wavelength allocation

Table 2/G.983.3 – Parameters for wavelength allocation in Figure 5

| Items | Notation | Unit | Nominal value | Application examples |
| --- | --- | --- | --- | --- |
| 1.3 μm wavelength band |  |  |  | For use in ATM-PON upstream. |
| Lower limit |  | nm | 1260 |  |
| Upper limit |  | nm | 1360 |  |
| Intermediate wavelength band |  |  |  | For future use  Reserved band including guardbands for allocation by ITU-T. |
| Lower limit |  | nm | 1360 |  |
| Upper limit | λ1 | nm | 1480 |  |
| Basic Band |  |  |  | For use in ATM-PON downstream. |
| Lower limit | λ1 | nm | 1480 |  |
| Upper limit | λ2 | nm | 1500 |  |
| Enhancement Band (Option 1) |  |  |  | For additional digital service use. |
| Lower limit | λ3 | nm | 1539 |  |
| Upper limit | λ4 | nm | 1565 |  |
| Enhancement Band (Option 2) |  |  |  | For video distribution service. |
| Lower limit | λ3 | nm | 1550 |  |
| Upper limit | λ4 | nm | 1560 |  |
| Future L Band |  |  |  | For future use  Reserved band for allocation by ITU-T. |
| Lower limit | λ5 | nm | For further study |  |
| Upper limit | λ6 | nm |  |  |
| NOTE 1  The central frequencies in the Enhancement Band for DWDM application shall be based on the frequency grid given in ITU-T G.959.1 and G.692. NOTE 2  The value of isolation between the Basic Band signal and the Enhancement Band signal is not decided uniquely, because system configuration varies by regulations, service strategy, and geographic conditions as well as offered services in the Enhancement Band. Here, isolation should be interpreted as both the ratio of leaked Basic Band signal to E-ONU or E-OLT from Basic Band signal and the ratio of leaked Enhancement Band signal to ONU or OLT from Enhancement Band signal at WF1 or WF2. Both types of isolation should be defined separately. Common use of WDM filters is expected for cost reduction through large volume production. The isolation examples required in typical cases are described in Appendix III. NOTE 3  Isolation value recommended for between the Basic Band signal and the Enhancement Band signals (Option 1) is for further study. NOTE 4  Isolation value recommended for between the Basic Band signal and the Enhancement Band signals (Option 2) is for further study. NOTE 5  Applied filter has appropriate loss characteristics to achieve required isolation. However, loss characteristics outside the 1480-1580 nm range (e.g. in the future L Band) is for further study. |  |  |  |  |

## 8.3	Physical medium dependent layer requirements for the ATM-PON

### 8.3.1	Digital signal nominal bit rate

The transmission line rate should be a multiple of 8 kHz. The target standardized system will have the following nominal line rates:

–	Option 1: Symmetric 155.52 Mbit/s for FTTCab/C/B/H.

–	Option 2: Asymmetric 155.52 Mbit/s upstream/622.08 Mbit/s downstream for FTTCab/C/B.

Parameters to be defined are categorized by downstream and upstream, and nominal bit rate as shown in Table 3.

Table 3/G.983.3 – Relation between parameter categories and tables

| Transmission direction | Nominal bit rate | Table |
| --- | --- | --- |
| Downstream | 155.52 Mbit/s | Table 4b (downstream, 155 Mbit/s) |
|  | 622.08 Mbit/s | Table 4c (downstream, 622 Mbit/s) |
| Upstream | 155.52 Mbit/s | Table 4d (upstream, 155 Mbit/s) |

All parameters are specified as follows, and shall be in accordance with Table 4a (ODN), Table 4b (downstream, 155 Mbit/s), Table 4c (downstream, 622 Mbit/s) and Table 4d (upstream, 155 Mbit/s). These tables will be globally referred to as Table 4 in this Recommendation unless more precision is needed. There are 6 kinds of ONUs and they are distinguished by bit rates of 155.52 Mbit/s and 622.08 Mbit/s and by optical path loss of Class A, Class B, and Class C (defined in ITU-T G.982). Some parameters described in Table 4 are described in Appendix I as examples for implementation.

The specification in Table 4 does not include the Maximum reflectance of equipment measured at transmitter wavelength and the Maximum reflectance of equipment measured at receiver wavelength for the E-ONU and E-OLT. Reflection from E-ONU and E-OLT may cause signal degradation in the ATM-PON transmission. The specifications for E-ONU and E-OLT reflectances at the 1.3 μm upstream and at the Basic Band downstream wavelengths are for further study.

All parameter values specified are worst-case values, assumed to be met over the range of standard operating conditions (e.g. temperature and humidity ranges), and they include ageing effects. The parameters are specified relative to an optical section design objective of a Bit Error Ratio (BER) not worse than 1  1010 for the extreme case of optical path attenuation and dispersion conditions.

All parameters are specified for IFPON and relevant optical parameters for reference points of (c) and (e) in Figure 2b are described in Appendix V.

Attenuation range in the ODN may be reduced for some application area such as new-built field or limited area. In this case, required optical parameters will be relaxed and enhance applicability of optical modules compliant to ITU-T G.983.1. Appendix V also describes the example case of reduced attenuation range of the ODN.

Table 4a/G.983.3 – Physical medium dependant layer parameters of ODN

| Items | Unit | Specification |
| --- | --- | --- |
| Fibre type | – | ITU-T G.652 |
| Attenuation range (ITU-T G.982) | dB | Class A: 5-20 Class B: 10-25 Class C: 15-30 |
| Differential optical path loss | dB | 15 |
| Maximum optical path penalty | dB | 1 |
| Max differential logical reach | km | 20 |
| Maximum fibre distance between S/R and R/S points | km | 20 |
| Minimum supported split ratio | – | Restricted by path loss and ONU addressing limits. PON with passive splitters (16 or 32 way split) |
| Bidirectional transmission | – | 1-fibre WDM (2-fibre system is beyond the scope of this Recommendation) |
| Maintenance wavelength | nm | to be defined |

Table 4b/G.983.3 – Optical interface parameters of 155 Mbit/s 
downstream direction for the Basic Band

| Items | Unit | Specifications |  |  |
| --- | --- | --- | --- | --- |
| OLT Transmitter (optical interface Old) |  |  |  |  |
| Nominal bit rate | Mbit/s | 155.52 |  |  |
| Operating wavelength | nm | 1480-1500 |  |  |
| Line code | – | Scrambled NRZ |  |  |
| Mask of the transmitter eye diagram | – | See Figure 6/G.983.1 |  |  |
| Maximum reflectance of equipment, measured at transmitter wavelength | dB | NA |  |  |
| Minimum ORL of ODN at Old and Olu (Notes 1 and 2) | dB | more than 32 |  |  |
| ODN Class |  | Class A | Class B | Class C |
| Mean launched power MIN | dBm | Note 4 | Note 4 | Note 4 |
| Mean launched power MAX | dBm | Note 4 | Note 4 | Note 4 |
| Launched optical power without input to the transmitter | dBm | NA |  |  |
| Extinction ratio | dB | more than 10 |  |  |
| Tolerance to the transmitter incident light power | dB | more than 15 |  |  |
| If MLM Laser – Maximum RMS width | nm | 1.8 |  |  |
| If SLM Laser – Maximum 20 dB width (Note 3) | nm | 1 |  |  |
| If SLM Laser – Minimum side mode suppression ratio | dB | 30 |  |  |

Table 4b/G.983.3 – Optical interface parameters of 155 Mbit/s 
downstream direction for the Basic Band (concluded)

| Items | Unit | Specifications |  |  |
| --- | --- | --- | --- | --- |
| ONU Receiver (optical interface Ord) |  |  |  |  |
| Maximum reflectance of equipment, measured at receiver wavelength | dB | less than 20 |  |  |
| Bit error ratio | – | less than 1010 |  |  |
| ODN Class |  | Class A | Class B | Class C |
| Minimum sensitivity | dBm | Note 4 | Note 4 | Note 4 |
| Minimum overload | dBm | Note 4 | Note 4 | Note 4 |
| Consecutive identical digit immunity | bit | more than 72 |  |  |
| Jitter tolerance | – | See Figure 9/G.983.1 |  |  |
| Tolerance to the reflected optical power | dB | less than 10 |  |  |
| NOTE 1  The value of "minimum ORL of ODN at point Oru and Ord, and Olu and Old" should be more than 20 dB in optional cases which are described in Appendix I/G.983.1. NOTE 2  The values on ONU transmitter reflectance for the case that the value of "minimum ORL of ODN at point Oru and Ord, and Olu and Old" is 20 dB are described in Appendix IV. NOTE 3  Values of maximum 20 dB width, and minimum side mode suppression ratio are referred to in ITU-T G.957. NOTE 4  Tentative parameters are described in Appendix I. |  |  |  |  |

Table 4c/G.983.3 – Optical interface parameters of 622 Mbit/s 
downstream direction for the Basic Band

| Items | Unit | Specifications |  |  |
| --- | --- | --- | --- | --- |
| OLT Transmitter (optical interface Old) |  |  |  |  |
| Nominal bit rate | Mbit/s | 622.08 |  |  |
| Operating wavelength | nm | 1480-1500 |  |  |
| Line code | – | Scrambled NRZ |  |  |
| Mask of the transmitter eye diagram | – | See Figure 6/G.983.1 |  |  |
| Maximum reflectance of equipment, measured at transmitter wavelength | dB | NA |  |  |
| Minimum ORL of ODN at Olu and Old (Notes 1 and 2) | dB | more than 32 |  |  |
| ODN Class |  | Class A | Class B | Class C |
| Mean launched power MIN | dBm | Note 4 | Note 4 | Note 4 |
| Mean launched power MAX | dBm | Note 4 | Note 4 | Note 4 |
| Launched optical power without input to the transmitter | dBm | NA |  |  |
| Extinction ratio | dB | more than 10 |  |  |
| Tolerance to the transmitter incident light power | dB | more than 15 |  |  |
| If MLM Laser – Maximum RMS width | nm | NA |  |  |

Table 4c/G.983.3 – Optical interface parameters of 622 Mbit/s 
downstream direction for the Basic Band (concluded)

| Items | Unit | Specifications |  |  |
| --- | --- | --- | --- | --- |
| If SLM Laser – Maximum 20 dB width (Note 3) | nm | 1 |  |  |
| If SLM Laser – Minimum side mode suppression ratio | dB | 30 |  |  |
| ONU Receiver (optical interface Ord) |  |  |  |  |
| Maximum reflectance of equipment, measured at receiver wavelength | dB | less than 20 |  |  |
| Bit error ratio | – | less than 1010 |  |  |
| ODN Class |  | Class A | Class B | Class C |
| Minimum sensitivity | dBm | Note 4 | Note 4 | Note 4 |
| Minimum overload | dBm | Note 4 | Note 4 | Note 4 |
| Consecutive identical digit immunity | bit | more than 72 |  |  |
| Jitter tolerance | – | See Figure 9/G.983.1 |  |  |
| Tolerance to the reflected optical power | dB | less than 10 |  |  |
| NOTE 1  The value of "minimum ORL of ODN at point Oru and Ord, and Olu and Old" should be more than 20 dB in optional cases which are described in Appendix I/G.983.1. NOTE 2  The values on ONU transmitter reflectance for the case that the value of "minimum ORL of ODN at point Oru and Ord, and Olu and Old" is 20 dB are described in Appendix IV. NOTE 3  Values of maximum 20 dB width, and minimum side mode suppression ratio are referred to in ITU-T G.957. NOTE 4  Tentative parameters are described in Appendix I. |  |  |  |  |

Table 4d/G.983.3 – Optical interface parameters of 155 Mbit/s upstream direction

| Items | Unit | Specifications |  |  |
| --- | --- | --- | --- | --- |
| ONU Transmitter (optical interface Oru) |  |  |  |  |
| Nominal bit rate | Mbit/s | 155.52 |  |  |
| Operating wavelength | nm | 1260-1360 |  |  |
| Line code | – | Scrambled NRZ |  |  |
| Mask of the transmitter eye diagram | – | See Figure 7/G.983.1 |  |  |
| Maximum reflectance of equipment, measured at transmitter wavelength | dB | less than 6 |  |  |
| Minimum ORL of ODN at Oru and Ord (Notes 1 and 2) | dB | more than 32 |  |  |
| ODN Class |  | Class A | Class B | Class C |
| Mean launched power MIN | dBm | Note 4 | Note 4 | Note 4 |
| Mean launched power MAX | dBm | Note 4 | Note 4 | Note 4 |
| Launched optical power without input to the transmitter | dBm | less than Min sensitivity 10 |  |  |
| Extinction ratio | dB | more than 10 |  |  |

Table 4d/G.983.3 – Optical interface parameters of 155 Mbit/s upstream direction (concluded)

| Items | Unit | Specifications |  |  |
| --- | --- | --- | --- | --- |
| Tolerance to the transmitter incident light power | dB | more than 15 |  |  |
| If MLM Laser – Maximum RMS width | nm | 5.8 |  |  |
| If SLM Laser – Maximum 20 dB width (Note 3) | nm | 1 |  |  |
| If SLM Laser – Minimum side mode suppression ratio | dB | 30 |  |  |
| Jitter transfer | – | See Figure 8/G.983.1 |  |  |
| Jitter generation from 0.5 kHz to 1.3 MHz | UI p-p | 0.2 |  |  |
| OLT Receiver (optical interface Olu) |  |  |  |  |
| Maximum reflectance of equipment, measured at receiver wavelength | dB | less than 20 |  |  |
| Bit error ratio | – | less than 1010 |  |  |
| ODN Class |  | Class A | Class B | Class C |
| Minimum sensitivity | dBm | Note 4 | Note 4 | Note 4 |
| Minimum overload | dBm | Note 4 | Note 4 | Note 4 |
| Consecutive identical digit immunity | bit | more than 72 |  |  |
| Jitter tolerance | – | NA |  |  |
| Tolerance to the reflected optical power | dB | less than 10 |  |  |
| NOTE 1  The value of "minimum ORL of ODN at point Oru and Ord, and Olu and Old" should be more than 20 dB in optional cases which are described in Appendix I/G.983.1. NOTE 2  The values of ONU transmitter reflectance for the case that the value of "minimum ORL of ODN at point Oru and Ord, and Olu and Old" is 20 dB are described in Appendix IV. NOTE 3  Values of maximum 20 dB width, and minimum side mode suppression ratio are referred to in ITU-T G.957. NOTE 4  Tentative parameters are described in Appendix I. |  |  |  |  |

### 8.3.2	Physical media and transmission method

#### 8.3.2.1	Transmission medium

This specification is based on the fibre described in ITU-T G.652.

#### 8.3.2.2	Transmission direction

The signal is transmitted both upstream and downstream through the transmission medium.

#### 8.3.2.3	Transmission methodology

Bidirectional transmission is accomplished by use of Wavelength Division Multiplexing (WDM) technique of 1310 nm region and Basic Band (1480-1500 nm) region wavelengths on a single fibre.

### 8.3.3	Bit rate

See 8.2.3/G.983.1.

### 8.3.4	Line code

See 8.2.4/G.983.1.

### 8.3.5	Operating wavelength

#### 8.3.5.1	Downstream direction

The operating wavelength range for the downstream direction on single fibre systems shall be 1480-1500 nm.

#### 8.3.5.2	Upstream direction

The operating wavelength range for the upstream direction shall be 1260-1360 nm.

### 8.3.6	Transmitter at Old and Oru

All parameters are specified as follows, and shall be in accordance with Table 4.

See the following clauses in ITU-T G.983.1 for each item:

8.2.6.1		for Source type;

8.2.6.2		for Spectral characteristics;

8.2.6.3		for Mean launched power;

8.2.6.3.1	for Launched optical power without input to the transmitter;

8.2.6.4		for Minimum extinction ratio;

8.2.6.6 		for Mask of transmitter eye diagram;

8.2.6.6.1 	for OLT transmitter; and

8.2.6.6.2 	for ONU transmitter.

#### 8.3.6.1	Maximum reflectance of equipment, measured at transmitter wavelength

Reflections from equipment (ONU/OLT) back to the cable plant are specified by the maximum permissible reflectance of equipment measured at Old/Oru. It shall be in accordance with Table 4.

#### 8.3.6.2	Tolerance to the reflected optical power

The specified transmitter performance must be met in the presence at S of the optical reflection level specified in Table 4.

### 8.3.7	Optical path between Old/Oru and Ord/Olu

See the following clauses in ITU-T G.983.1 for each item:

8.2.7.2 		for Minimum optical return loss of the cable plant at point R/S, including any connector; 	and

8.2.7.3 		for Maximum discrete reflectance between points S and R.

#### 8.3.7.1	Attenuation range

Three classes of attenuation ranges are being specified as defined in ITU-T G.982:

–	5-20 dB: Class A;

–	10-25 dB: Class B;

–	15-30 dB: Class C.

Attenuation specifications are assumed to be worst-case values including losses due to splices, connectors, optical attenuators (if used) or other passive optical devices, and any additional cable margin to cover allowances for:

1)	future modifications to the cable configuration (additional splices, increased cable lengths, etc.);

2)	fibre cable performance variations due to environmental factors; and

3)	degradation of any connector, optical attenuators (if used) or other passive optical devices between points S and R, when provided.

#### 8.3.7.2	Dispersion

Systems considered limited by dispersion have maximum values of dispersion (ps/nm) specified in Table 4. These values are consistent with the maximum optical path penalties specified. They take into account the specified transmitter type, and the fibre dispersion coefficient over the operating wavelength range.

Systems considered limited by attenuation do not have maximum dispersion values specified and are indicated in Table 4 with the entry "NA" (not applicable).

### 8.3.8	Receiver at Ord and Olu

All parameters are specified as follows, and shall be in accordance with Table 4.

See the following clauses in ITU-T G.983.1 for each item:

8.2.8.1 		for Minimum sensitivity;

8.2.8.2 		for Minimum overload;

8.2.8.3 		for Maximum optical path penalty;

8.2.8.5 		for Differential optical path loss;

8.2.8.6 		for Clock extraction capability;

8.2.8.7 		for Jitter performance;

8.2.8.7.1	for Jitter transfer;

8.2.8.7.2 	for Jitter tolerance;

8.2.8.7.3	for Jitter generation;

8.2.8.8 		for Consecutive identical digit (CID) immunity;

8.2.8.9 		for Tolerance to reflected power; and

8.2.8.10 	for Transmission quality and error performance.

#### 8.3.8.1	Maximum reflectance of receiver equipment, measured at receiver wavelength

Reflections from equipment (ONU/OLT) back to the cable plant are specified by the maximum permissible reflectance of equipment measured at Ord and Olu. It shall be in accordance with Table 4.

## 8.4	Physical medium dependent layer requirements for the Enhancement Band

Optical parameters, line code, modulation scheme and so on for the Enhancement Band are out of scope of this Recommendation.

However, reflection from ONU and OLT may cause signal degradation in the Enhancement Band services. Therefore the Maximum reflectance of equipment measured at transmitter wavelength and the Maximum reflectance of equipment measured at receiver wavelength for the ONU and the OLT should be specified in the Enhancement Band wavelength. The specification for reflectance of the ONU and OLT are for further study.

# 9	Operations Administration and Maintenance (OAM) functionality

See clause 9/G.983.1.

# 10	Performance

See clause 10/G.983.1.

# 11	Environmental condition

For the OLT and indoor ONU, the conditions of IEC 60721-3-3 are recommended. For the outdoor ONU, the conditions of IEC 60721-3-4, with the expanded environmental range (4.1E) specified in ETS 300 019-1-4, are recommended.

The conditions of IEC 60801-2 and 60801-3 for electro magnetic compatibility are recommended.

Examples of the applied environmental conditions for the OLT and ONU temperature and relative humidity are described in Table 5. The other environmental conditions such as environmental pollutants and chemicals are for further study.

Table 5/G.983.3 – Examples of environmental conditions

| Applied example | Temperature (C) |  | Relative humidity (%) |  | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | Normal | Short term | Normal | Short term |  |
| OLT | 5 to 40 | 0 to 50 (Note 1) | 5 to 85 | 5 to 90 (Note 1) | IEC 60721-3-3 class 3k3 |
| Indoor ONU | 5 to 45 | – | 5 to 95 | – | IEC 60721-3-3 class 3k5 |
| Outdoor ONU (Note 2) | 45 to 45 (Note 3) | – | 8 to 100 | – | ETS 300 019-1-4 class 4.1E |
| NOTE 1  Option 1: short term refers to a period of not more than 72 consecutive hours and a total of not more than 15 days in one year. Option 2: short term refers to a period of not more than 12 consecutive hours and a total of not more than four days in one year. NOTE 2  The environmental conditions for the ONU are environmental conditions outside the ONU enclosure, and assume that the ONU (including housing/cabinet) is provided by a single supplier and is located in a conventional indoor or outdoor setting. Environmental conditions for ONUs that are located in other settings (e.g. inside an operator-provided cabinet or a non-conditioned indoor environment such as an attic or garage) are for future study. NOTE 3  In many instances, network operators are willing to relax the low temperature requirement to 40 degrees. |  |  |  |  |  |

# 12	Safety

Safety is defined in ITU-T G.983.1.

Appendix I

Optical interface parameters examples for ODN

## I.1	Introduction

Tentative optical parameters for ODN are described below (see Table 4) and are given in this Appendix I as examples for implementation. If appropriate parameters are approved and entered into Table 4, this Appendix will be removed.

## I.2	Examples

Specified numerical values described in the following table indicate tentative values for items described in Table 4.

Table I.1/G.983.3 – Optical interface parameters of 155 Mbit/s 
downstream direction for the Basic Band

| Items | Unit | Specifications |  |  |
| --- | --- | --- | --- | --- |
| OLT Transmitter (optical interface Old) |  |  |  |  |
| ODN Class |  | Class A | Class B | Class C |
| Mean launched power MIN | dBm | 7.5 | 2.5 | 0.5 |
| Mean launched power MAX | dBm | 3 | +2 | +4 |
| ONU Receiver (optical interface Ord) |  |  |  |  |
| ODN Class |  | Class A | Class B | Class C |
| Minimum sensitivity | dBm | 28.5 | 28.5 | 31.5 |
| Minimum overload | dBm | 8 | 8 | 11 |

Table I.2/G.983.3 – Optical interface parameters of 622 Mbit/s 
downstream direction for the Basic Band

| Items | Unit | Specifications |  |  |
| --- | --- | --- | --- | --- |
| OLT Transmitter (optical interface Old) |  |  |  |  |
| ODN Class |  | Class A | Class B | Class C |
| Mean launched power MIN | dBm | 5.5 | 0.5 | –0.5 |
| Mean launched power MAX | dBm | 1 | +4 | +4 |
| ONU Receiver (optical interface Ord) |  |  |  |  |
| ODN Class |  | Class A | Class B | Class C |
| Minimum sensitivity | dBm | 26.5 | 26.5 | 31.5 |
| Minimum overload | dBm | 6 | 6 | 11 |

Table I.3/G.983.3 – Optical interface parameters of 155 Mbit/s 
upstream direction

| Items | Unit | Specifications |  |  |
| --- | --- | --- | --- | --- |
| ONU Transmitter (optical interface Oru) |  |  |  |  |
| ODN Class |  | Class A | Class B | Class C (Note) |
| Mean launched power MIN | dBm | 7.5 | 5.5 | 3.5 |
| Mean launched power MAX | dBm | 0 | +2 | +4 |
| OLT Receiver (optical interface Olu) |  |  |  |  |
| ODN Class |  | Class A | Class B | Class C (Note) |
| Minimum sensitivity | dBm | 28.5 | 31.5 | 34.5 |
| Minimum overload | dBm | 5 | 8 | 11 |
| NOTE  The values proposed for the upstream Class C are best estimates. They are therefore subject to change in the future. |  |  |  |  |

Appendix II

Deployment scenarios

## II.1	Introduction

It is assumed that various system configurations or scenarios will be introduced by telecom operators. These system configurations or scenarios are influenced by regulations, service strategy, and the geographic conditions for each telecom operator. These configurations include pure WDM-based overlays and those which rely on FDM techniques to separate the Basic Band and Enhancement Band signals.

Configuration examples are described below.

## II.2	Configuration examples

### II.2.1	Case 1: Reception of Basic Band only

If customers do not require additional services such as video service etc. over the Enhancement Band under the wavelength allocation of Figure 5, then two system configurations would be as shown in Figures II.1 and II.2. In the case of Figure II.1, the WDM blocking filter may be needed to prevent the signal in the Enhancement Band from reaching the ONU. In the case of Figure II.2, an FDM-based approach, newer version ONUs, which are equipped with an electrical bandsplit filter, are required.

Figure II.1/G.983.3 – Reception of Basic Band only by blocking filter

Figure II.2/G.983.3 – Reception of Basic Band only by FDM-based approach

### II.2.2	Case 2: Reception of Basic Band and Enhancement Band signals by separate ONUs

If operators provide both ATM-PON and additional services such as video service etc. in separate equipment modules, the WDM-based system configuration would be Figure II.3.

Figure II.3/G.983.3 – Reception of Basic Band and Enhancement 
Band signals by separate ONUs

### II.2.3	Case 3: Reception of Basic Band and Enhancement Band signals by integrated ONUs

If operators provide both ATM-PON and additional services such as video service etc. in the same equipment, then two system configurations would be as shown in Figures II.4 and II.5. Figure II.4 shows a WDM-based approach. This type's ONUs are equipped with WDM filter. Figure II.5 shows FDM-based approach. This type's ONUs are equipped with electrical bandsplit filter.

Figure II.4/G.983.3 – Reception of Basic Band and Enhancement Band 
signals by integrated ONUs (WDM-based approach)

Figure II.5/G.983.3  Reception of Basic Band and Enhancement Band 
signals by integrated ONUs (FDM-based approach)

### II.2.4	Case 4: Arrangement of WDMs to minimize upstream path loss

This arrangement is aimed at minimizing the upstream path loss (at 1300 nm) when an Enhancement Band is added. In this case WDM-L and WDM-N separate the 1300 nm and 1500 nm bands and WF1 and WF2 split the 1500 nm band. This allows existing upstream transmitters and receivers to be used without a change in the end-to-end loss. The performance of WF1 and WF2 needs only to be specified in the 1500 nm band since 1300 nm signals are eliminated. It avoids the problem of near-end crosstalk when the Enhancement Band carries a downstream-only service. It avoids problems of reflections in the Basic Band being a problem to the Enhancement Band receivers. A disadvantage of this approach is that it is unsuitable for use with Basic Band electro-optic modules that incorporate G.983.1 WDMs.

Figure II.6/G.983.3 – Arrangement of WDMs to minimize upstream path loss

### II.2.5	Case 5: ODNs with reduced attenuation range for Enhancement Band

A problem for some operators arises when an existing Class C ODN is deployed. This is specified to have 30 dB path loss in both the 1300 and 1500 nm windows. Existing Basic Band transceivers will not have sufficient power budget when WDM components are added externally.

Case 4 could avoid this problem for the upstream path when the upstream and downstream transmitters and receivers are available as separate modules.

The downstream path will have additional losses for the WDMs in the 1500 nm band which could be as high as 1.5 dB at each end. This means that the 1500 nm transmitters and receivers need 3 dB extra power budget. One way of compensating this 3 dB is to apply a modified Class C, which takes into account the differential losses of the fibre in the 1300 and 1500 nm windows. The maximum loss at 1300 nm is 30 dB and the maximum loss at 1500 nm is 27 dB. Many existing Class C ODNs and all Class C with reduced attenuation range could meet a specification having less than 27 dB loss in the 1480-1580 nm band and less than 30 dB loss in the 1260-1360 nm band.

### II.2.6	Example in case of insufficiency of loss budget between E-ONU and E-OLT

In case of insufficiency of loss budget between E-ONU and E-OLT, it is possible to shorten the distance between E-ONU and E-OLT. Figure II.7 shows an example. In that case, output from E-OLT is inputted to second stage of splitter.

Figure II.7/G.983.3  Arrangement in case of insufficiency of loss 
budget between E-ONU and E-OLT

Appendix III

Isolation between the Basic Band and the Enhancement Band

## III.1	Introduction

The Enhancement Band can be used for many different types of services, such as video distribution and digital high bit-rate services.

Each service (characterized by modulation format or bit rate, the amount of carriers or amount of wavelengths) will require its own Enhancement Band receiver sensitivity and allowable cross-talk from the Basic Band signal.

These parameters, together with the total optical power in the Enhancement Band, will in turn determine the necessary isolation value at the Enhancement Band receiver from the Basic Band signal, and the necessary isolation value at the Basic Band receiver from the Enhancement Band signal.

In general, "Isolation" is defined as the loss difference between two wavelengths measured at the same port. Required Isolation for WDM filter such as WF1 and WF2 should be estimated considering reflections of equipment and/or ODN. The effect of reflections is described in Appendix IV.

This appendix provides some initial calculations of the optical characteristics of the Enhancement Band services. These are used to find example isolation values of the Basic Band from the Enhancement Band. Such examples may be useful to vendors in guiding their development efforts. However, the exact requirements of the isolation values, and the isolation required by the enhancement signals from the Basic Band depend on many factors that are out of the scope of this Recommendation. Any system design will need to do a complete accounting of all these factors to arrive at a complete optical isolation specification.

Isolation to be considered is schematically shown in Figure III.1.

The type of service and its characteristics will determine isolation performances of WF1 and WF2. As the optical power of the Basic Band signal will vary with the PON system class (A, B, C), the isolation values will also be different for the different ODN classes.

Out of the many different service and class scenarios, several isolation classes could then be defined.

The following tables illustrate several service scenarios. The scenarios have to be analysed for each PON Class.

The following notations are used in this appendix:

LODN-BB 	Optical loss of Basic Band in the ODN

LODN-EB 	Optical loss of Enhancement Band in the ODN

PBB		Optical power of Basic Band signal at the input to the ONU

PEB		Optical Power of Enhancement Band signal at the input to the E-ONU

F.F.S		for further study

The following assumptions are taken for the examples:

	LODN-BB = LODN-EB.

	The allowable cross-talk at the Basic Band receiver [at reference points (c) and (e)] from the Enhancement Band signal is assumed to be at least 10 dB lower than the lowest Basic Band signal.

NOTE  This assumption should be modified to some higher value than 10 dB, for example to 13 dB, because the total optical noise to the receiver consists of multiple reflections as well as leak optical power. Permissible total noise may be 10 dB lower than the lowest signal level, but permissible noise power for each noise component is for further study. Therefore, the following are tentatively described on the assumption that each noise component be at least 10 dB lower than the signal.

	Assuming following WF1 and WF2 insertion losses: 0 dB – 1.5 dB.

## III.2	Example for required isolation at Basic Band receiver from Enhancement Band signal

### III.2.1	Video services

Required isolation depends on the minimum power required by the receiver to produce a usable signal plus the total amount of system tolerance (transmitter, fibre, and optical component loss variations). For subcarrier multiplexed video systems, the carrier-to-noise ratio (CNR) is given by Equation (III-1).

CNR=12B⋅m2RIN+2ei+4kTi2R	(III-1)

Here B is the channel bandwidth, m is the per-channel modulation index, RIN is the relative intensity noise of the transmitter, e is the charge of the electron, i is the detected photocurrent, kT is the Boltzman factor, and R is the receiver impedance. The assumed values for all these parameters are given in the following table:

| RIN (dB/Hz) | 150.0 |
| --- | --- |
| Receiver Temperature (K) | 300.0 |
| Receiver Impedance (Ω) | 75.0 |
| Receiver Responsivity (A/W) | 0.85 |
| RMS OMI | 25% |

The required SNR depends on the signal format being considered. The values assumed here are given in the following table:

| Channel Format | (dB) |
| --- | --- |
| QPSK | 16 |
| 16-QAM | 22 |
| 64-QAM | 28 |
| 256-QAM | 34 |
| AM-VSB | 44 (Note) |
| NOTE  The value given for AM-VSB is the CNR. |  |

The RMS optical modulation index (OMI), along with N, the number of channels, can be used to compute m, by using Equation (III-2). The responsivity is used to convert the photocurrent to optical power.

m=OMI2N	(III-2)

Equation (III-1) can be re-arranged as shown in Equation (III-3). This gives a quadratic in photocurrent, and the solution to this equation then can be converted into the required optical power, PEB, at the point between the E-ONU and WF2, shown in Table III.1.

RIN−m22B⋅CNRi2+2ei+4kTR=0	(III-3)

Table III.1/G.983.3  PEB, minimal power of Enhancement Band 
signal at the input to the E-ONU

| Transmission System | Carrier Format | Number of carriers | Carrier BW (MHz) | Required Power (dBm) |
| --- | --- | --- | --- | --- |
| SCM | AM-VSB | 40 | 4.5 | –7.7 |
|  |  |  | 6.5 | –6.7 |
|  | QPSK | 60 | 18 | –18.3 |
|  | 64-QAM | 110 | 5.2 | –13.6 |
|  |  |  | 7.0 | –13.0 |
|  | 256-QAM | 110 | 5.2 | –10.5 |
|  |  |  | 7.0 | –9.8 |
| FM Conversion | AM-VSB | 40 | 4.5 | –14.1 |
|  |  |  | 6.5 | –11.5 |
|  | 64-QAM | 110 | 5.2 | –16.7 |
|  |  |  | 7.0 | –14.3 |
|  | 256-QAM | 110 | 5.2 | –13.3 |
|  |  |  | 7.0 | –10.1 |

The isolation values at WF2 (the ratio of leaked Enhancement Band signal to ONU from Enhancement Band signal at common port of WF2) are obtained from:

Isolation of WF2 = PEB req + Rx∆ + Tx∆ – PBB min + 13 dB,

where Rx∆ and Tx∆ are the dynamic range tolerances of the receiver and transmitter. We assume values of Rx∆ = 2 dB, and Tx∆ = 1 dB.

For example, if the minimum required enhancement power at the E-ONU receiver, PEB req = 10 dBm and PBB min = 30 dBm, then required isolation = 36 dB.

The resulting isolation at WF2 is specified in Table III.2. These are assuming that PBB min = 30 dBm, which is true for class B equipment. Note that the above calculation assumes that the enhancement and Basic Band signals suffer the same insertion loss through WF2.

Table III.2/G.983.3 – Isolation requirements at Basic Band 
receiver from Enhancement Band

| Transmission System | Carrier Format | Number of carriers | Carrier BW (MHz) | Required Isolation (dB) |
| --- | --- | --- | --- | --- |
| SCM | AM-VSB | 40 | 4.5 | 38.3 |
|  |  |  | 6.5 | 39.3 |
|  | QPSK | 60 | 18 | 27.7 |
|  | 64-QAM | 110 | 5.2 | 32.4 |
|  |  |  | 7.0 | 33.0 |
|  | 256-QAM | 110 | 5.2 | 35.5 |
|  |  |  | 7.0 | 36.2 |
| FM Conversion | AM-VSB | 40 | 4.5 | 31.9 |
|  |  |  | 6.5 | 34.5 |
|  | 64-QAM | 110 | 5.2 | 29.3 |
|  |  |  | 7.0 | 31.7 |
|  | 256-QAM | 110 | 5.2 | 32.7 |
|  |  |  | 7.0 | 35.9 |

The isolation required at WF1 is driven by the reflections in the PON, which can be either –32 dB or –20 dB. The video transmitter will most likely be operating near the SBS limit of the fibre, implying a maximum launch power of +17 dBm. With a reflection loss of –20 dB, the reflection can be as high as –3 dBm. If the signal must be 13 dB higher than the interference, then the combined isolation of the WF1 and diplexer must be 40 dB. If PON reflectance of –32 dB is assumed, then the isolation of WF1 and diplexer must be 28 dB.

Note that the above isolations assume that the video and Basic Band signals overlap in electrical frequency. This is not necessarily true, and in some notable cases, such as the QPSK video, the video signal is entirely out of band. In these cases, the video signal only impairs the Basic Band signal via increased shot and relative intensity noise. This can reduce the required isolation greatly, in some cases to the point where no optical isolation is required at all.

### III.2.2	DWDM services

The provision of DWDM services over the PON will also place requirements on the isolation of filters WF1 and WF2. The starting point again is the sensitivities of the receivers that would be used for the DWDM upgrade. Table III.3 below gives the typical values for optical receivers, and also for the filters that are required to demultiplex the individual channels.

Table III.3/G.983.3 – The typical values for optical receivers and filters

| Table a  Data Rate | SDH-1 | SDH-4 | GbE | SDH-16 |
| --- | --- | --- | --- | --- |
| Maximum overload (PIN photodiode) (dBm) | –8 | –7 | –6 | –4 |
| Minimum sensitivity (PIN photodiode) (dBm) | –36 | –31 | –25 | –20 |
| Maximum overload (APD) (dBm) | NA | –10 | –9 | –9 |
| Minimum sensitivity (APD) (dBm) | NA | –38 | –33 | –29 |

| Table b  Number of Channels | 1 | 8 | 16 | 32 |
| --- | --- | --- | --- | --- |
| 200 GHz Filter max loss (dB) | 2 | 3.5 | 5 | NA |
| 100 GHz Filter Max loss (dB) | 2.5 | 4.5 | 6.5 | 8.5 |
| Basic/Enhancement Filter max loss (dB) | 1 | NA | NA | NA |

By building hypothetical loss budgets using these numbers, one can derive the maximum launched powers (per channel) at both ends of the PON. The result depends on the speed and number of channels, as shown in Table III.4. Note that the upstream launch powers are higher than the downstream powers because the upstream channels must pass through the lossy multichannel WDM demultiplexer, while the downstream channels only pass through a single WDM filter.

Table III.4/G.983.3 – The maximal Tx power of Enhancement Band signal at IFPON

a  just after the WF1 going downstream

|  | PIN (dBm) |  | APD (dBm) |  |
| --- | --- | --- | --- | --- |
| Channels | SDH-1 | SDH-4 | GbE | SDH-16 |
| 4 | –4.5 | 0.5 | –1.5 | 2.5 |
| 8 | –4.5 | 0.5 | –1.5 | 2.5 |
| 16 | –4.0 | 1.0 | –1.0 | 3.0 |

b  just after WF2 going upstream

|  | PIN (dBm) |  | APD (dBm) |  |
| --- | --- | --- | --- | --- |
| Channels | SDH-1 | SDH-4 | GbE | SDH-16 |
| 4 | –2.0 | 3.0 | 1.0 | 4.8 |
| 8 | –1.5 | 3.5 | 1.5 | 5.0 |
| 16 | 2.0 | 7.0 | 5.0 | 6.8 |

The isolation values are obtained by first computing the maximum Enhancement Band power entering either WF1 or WF2. The two sources of this power are the forward path power (upstream in the case of WF1) and the reflection path power (downstream in the case of WF1). These powers are summed arithmetically, including the multiplier for the number of channels involved, and produce the interfering power. The worst case of this interfering power is then compared with the worst case of the Basic Band power. Their difference plus 13 dB is then the required isolation. The isolations for WF1 and WF2 are specified in Table III.5 assuming a class B PON. Note that the isolation stated for WF1 also includes whatever isolation that the diplexer provides inside the OLT, hence the actual isolation required of WF1 is actually less.

Table III.5/G.983.3 – Isolation requirements at Basic Band receiver 
from Enhancement Band

a  for WF1

|  | PIN (dB) |  | APD (dB) |  |
| --- | --- | --- | --- | --- |
| Channels | SDH-1 | SDH-4 | GbE | SDH-16 |
| 4 | 37.3 | 42.3 | 40.3 | 44.0 |
| 8 | 40.7 | 45.7 | 43.7 | 47.3 |
| 16 | 47.1 | 52.1 | 50.1 | 52.0 |

b  for WF2

|  | PIN (dB) |  | APD (dB) |  |
| --- | --- | --- | --- | --- |
| Channels | SDH-1 | SDH-4 | GbE | SDH-16 |
| 4 | 20.8 | 25.8 | 23.8 | 27.8 |
| 8 | 24.0 | 29.0 | 27.0 | 30.8 |
| 16 | 28.6 | 33.6 | 31.6 | 34.6 |

## III.3	Example of isolation classes

Possible isolation classes for the ONU WF2 filter are shown in Table III.6. It would appear that 30 dB of isolation is sufficient for most all of the DWDM upgrades, and some of the video upgrades. Hence, this could define the first class of isolation. For exceptional cases of DWDM upgrade, or for many of the video upgrades, 40 dB of isolation is required. This could define the second class of isolation.

Table III.6/G.983.3 – Isolation Classes

|  |  | Guaranteed Isolation |  |
| --- | --- | --- | --- |
|  |  | at EB from BB (dB) | at BB from EB (dB) |
| Isolation Class | Case 1 | F.F.S | 30 |
|  | Case 2 | F.F.S | 40 |
|  | Case 3 | F.F.S | F.F.S |

## III.4	Deployment examples

This clause shows examples of WDM filters' characteristics such as power loss and isolation in deploying this system.

### III.4.1	Model to be considered

Figure III.2 shows an example of model.

In order to calculate the value of isolation, the SNR of three receivers such as OLT-Rx, ONU-Rx and E-Rx should be considered. For that, each amount of interference optical power and the power of signal should be considered.

Figure III.2/G.983.3 – Model for calculation

#### III.4.1.1	SNR of OLT-Rx

Figure III.3 shows the noise signals received at OLT-Rx receiver.

Figure III.3/G.983.3 – Noise signals at OLT-Rx

Then, amount of Interference optical power is given by Equation (III-4).

XL[dBm] = 10  log(XL[mW])	(III-4)

where:

XL[mW] = XLdl[mW] + XLdr[mW] + XLvl[mW] + XLvr[mW]

So, if the reflected optical power is at least 10 dB lower than the signal power, then Equation (III-5) is satisfied.

Su[dBm]  XL[dBm]  10[dB] 	(III-5)

#### III.4.1.2	SNR of ONU-Rx

Figure III.4 shows the noise signals received at ONU-Rx receiver.

Figure III.4/G.983.3 – Noise signals at ONU-Rx

Then, amount of Interference optical power is given by Equation (III-6).

XN[dBm] = 10  log(XN[mW])	(III-6)

where:

XN[mW] = XNul[mW] + XNur[mW] + XNvl[mW] + XNvr[mW]

So, if the reflected optical power is at least 10 dB lower than the signal power, then Equation (III-7) is satisfied.

Sd[dBm]  XN[dBm]  10[dB] 	(III-7)

#### III.4.1.3	SNR of E-Rx

Figure III.5 shows the noise signals received at E-Rx receiver.

Figure III.5/G.983.3 – Noise signals at E-Rx

Then, amount of Interference optical power (XE) is given by Equation (III-8).

XE[dBm] = 10 × log(XE[mW])	(III-8)

where:

XE[mW] = XEul[mW] + XEur[mW] + XEdl[mW] + XEdr[mW]

So, Equation (III-9) is satisfied.

Sv[dBm]  XE[dBm]  (*)[dB]	(III-9)

(*)	It depends on the specification of video signal. Refer to III.4.2.3.

### III.4.2	In case of 64-QAM

	Assumption of the Basic Band signal

•	155 Mbit/s Class B

Pout(ATM-OLT) = 2.5 to +2 [dBm] @ODN-I/F

Pin(ATM-OLT) = 8 to –31.5 [dBm] @ODN-I/F

Pout(ATM-ONU) = 5.5 to +2 [dBm] @ODN-I/F

Pin(ATM-ONU) = 8 to –28.5 [dBm] @ODN-I/F

•	All WDM insertion loss of through port for all wavelengths is 0 to 1.5 dB

•	ORL of ODN is 32 dB, ORL of Equipment at receiver wavelength is 20 dB

	Assumption of the 64-QAM signal:

•	64-QAM modulation and 40 carriers for video (27.4 dB SNR is required)

Pout(video) = +7 to +10 [dBm] @ODN I/F

Pin(video) = 0 to 18 [dBm] @ODN I/F

•	Frequency Range: 54 MHz ~ 750 MHz

•	Carrier Spacing: 6 MHz

•	Required SNR: 28 dB

#### III.4.2.1	Calculation of OLT-Rx

Su [dBm] = –31.5 [dBm]  1.5 [dB]  1.5 [dB]

XLdl [dBm] = +2 [dBm]  LLTRd [dB]

XLdr [dBm] = +2 [dBm]  32 [dB]  0 [dB]  LLARd [dB]

XLvl [dBm] = +10 [dBm]  LWF1EAv [dB]  LLARv [dB]

XLvr [dBm] = +10 [dBm]  32 [dB]  LWF1CAv [dB]  LLARv [dB]

From Equation III-4 and Equation III-5, an example of WDM characters are as follows:

LLTRd = 50 [dB] (Loss of downstream in WDM-L from Port T to port R)

LLARd = 20 [dB] (Loss of downstream in WDM-L from Port A to port R)

LWF1EAv = 50 [dB] (Loss of video in WDM-WF1 from Port E to port A)

LLARv = 20 [dB] (Loss of video in WDM-L from Port A to port R)

LWF1CAv = 20 [dB] (Loss of video in WDM-WF1 from Port C to port A)

#### III.4.2.2	Calculation of ONU-Rx

Sd [dBm] = –28.5 [dBm]  1.5 [dB]  1.5 [dB]

XNul [dBm] = +2 [dBm]  LNTRu [dB]

XNur [dBm] = +2 [dBm]  32 [dB]  0 [dB]  LNARu [dB]

XNvl [dBm] = (18 [dBm] + 3 [dB])  LWF2CAv [dB]  0 [dB]

XNvr[dBm] = (18 [dBm] + 3 [dB])  0 [dB]  20 [dB]  LWF2EAv [dB]  0 [dB]

3 [dB] = +10 [dBm]  (+7 [dBm])

From Equation III-6 and Equation III-7, examples of WDM characteristics are as follows:

LNTRu = 50 [dB] (Loss of upstream in WDM-N from Port T to port R)

LNARu = 20 [dB] (Loss of upstream in WDM-N from Port A to port R)

LWF2CAv = 29 [dB] (Loss of video in WDM-WF2 from Port C to port A)

LWF2EAv = 20 [dB] (Loss of video in WDM-WF2 from Port E to port A)

#### III.4.2.3	Calculation of E-Rx

S/X for video receiver requires 27.4 dB to transmit 40 carriers with –18 dBm received power. So, Equation (III-9 bis) is satisfied.

Sv[dBm] – XE[dBm]  27.4 dB	(III-9 bis)

Sv [dBm] = 18 [dBm]  1.5 [dB]

XEul [dBm] = +2 [dBm]  LWF2AEu [dB]

XEur [dBm] = +2 [dBm]  32 [dB]  LWF2CEu [dB]

XEdl [dBm] = (–27.5 [dBm] + 4.5 [dB])  LWF2CEd [dB]

XEdr [dBm] = (–27.5 [dBm] + 4.5 [dB])  0 [dB]  20 [dB]  LWF2AEd [dB]

4.5 [dB] = +2 [dBm]  (–2.5 [dBm])

From Equation (III-8) and Equation (III-9 bis), examples of WDM characters are as follows:

LWF2AEu = 50 [dB] (Loss of upstream in WDM-WF2 from Port A to port E)

LWF2CEu = 31.5 [dB] (Loss of upstream in WDM-WF2 from Port C to port E)

LWF2CEd = 31.5 [dB] (Loss of downstream in WDM-WF2 from Port C to port E)

LWF2AEd = 20 [dB] (Loss of downstream in WDM-WF2 from Port A to port E)

### III.4.3	Filter characteristics

From an estimation of III.4.2.1, III.4.2.2 and III.4.2.3, required filter characteristics are shown in Tables III.7 and III.8.

Table III.7/G.983.3 – Required filter characteristics (OLT side)

| WF1 |  |  |  |  | WDM-L |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Port | Wavelength | Min | Max |  | Port | Wavelength | Min | Max |
| LWF1Acd | A→C | Down | 0 | 1.5 | LLTAd | T→A | Down | 0 | 1.5 |
| LWF1CAu | C→A | Up | 0 | 1.5 | LLARu | A→R | Up |  | 1.5 |
| LWF1CEv | C→E | Video | 0 | 1.5 | LLATv | A→T | Video |  |  |
| LWF1CEd | C→E | Down |  |  | LLARd | A→R | Down | 20 |  |
| LWF1CEu | C→E | Up |  |  | LLATu | A→T | Up |  |  |
| LWF1CAv | C→A | Video | 20 |  | LLARv | A→R | Video | 20 |  |
| LWF1AEd | A→E | Down |  |  | LLTRd | T→R | Down | 50 |  |
| LWF1AEu | A→E | Up |  |  | LLRTu | R→T | Up |  |  |
| LWF1EAv | E→A | Video | 50 |  | LLTRv | T→R | Video |  |  |

Table III.8/G.983.3 – Required filter characteristics (ONU side)

| WF2 |  |  |  |  | WDM-N |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Port | Wavelength | Min | Max |  | Port | Wavelength | Min | Max |
| LWF2Acu | A→C | Up | 0 | 1.5 | LNTAu | T→A | Up | 0 | 1.5 |
| LWF2CAd | C→A | Down | 0 | 1.5 | LNARd | A→R | Down |  | 1.5 |
| LWF2CEv | C→E | Video | 0 | 1.5 | LNARv | A→T | Video | 0 |  |
| LWF2CEu | C→E | Up | 31.5 |  | LNARu | A→R | Up | 20 |  |

Table III.8/G.983.3 – Required filter characteristics (ONU side) (concluded)

| WF2 |  |  |  |  | WDM-N |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Port | Wavelength | Min | Max |  | Port | Wavelength | Min | Max |
| LWF2CEd | C→E | Down | 31.5 |  | LNATd | A→T | Down |  |  |
| LWF2CAv | C→A | Video | 29 |  | LNATv | A→R | Video |  |  |
| LWF2AEu | A→E | Up | 50 |  | LNTRu | T→R | Up | 50 |  |
| LWF2AEd | A→E | Down | 20 |  | LNRTd | R→T | Down |  |  |
| LWF2EAv | E→A | Video | 20 |  | LNTTv | T→R | Video |  |  |

Isolation is defined as the loss difference between two wavelengths measured at the same port. Therefore, we can calculate "Isolation" using Tables III.7 and III.8.

Isolation at WDM-WF1: Isolate Basic Band upstream signal from Enhancement band signal.

LWF1CAv(min) [dB] – LWF1CAu(max) [dB] = 20 [dB] – 1.5 [dB] = 18.5 [dB]

Isolation at WDM-WF2: Isolate Enhancement Band signal from Basic band upstream signal.

LWF2CEu(min) [dB]  LWF2CEv(max) [dB] = 31.5 [dB]  1.5 [dB] = 30 [dB]

Isolation at WDM-WF2: Isolate Enhancement Band signal from Basic Band downstream signal.

LWF2CEd(min) [dB]  LWF2CEv(max) [dB] = 31.5 [dB]  1.5 [dB] = 30 [dB]

Isolation at WDM-WF2: Isolate Basic Band downstream signal from Enhancement Band signal.

LWF2CAv(min) [dB]  LWF2CAd(max) [dB] = 29 [dB]  1.5 [dB] = 27.5 [dB]

Appendix IV

Effect of optical return loss of ODN

## IV.1	Introduction

Each network model has its own optical return loss (ORL) of ODN and BPON is sensitive to the ORL of ODN. This appendix describes the relationship among some types of reflectance to be considered, WDM isolation of ONU and OLT, and ONU equipment reflectance for transmitter and receiver for each case that ORL of ODN is 32 dB and 20 dB.

The fundamental consideration has already been described in Appendix II/G.983.1. Similar consideration is applicable and extended for BPON ODN including effects by Enhancement Band, E-ONU, and E-OLT.

In the calculation of optical parameters, we assume that ONU and E-ONU equipment reflectance for receiver is 20 dB and OLT and E-OLT equipment reflectance for receiver is 20 dB. This appendix describes condition equations and calculation results for the reflectance, which restrict the parameters.

NOTE  Permissible total noise to the receiver, which consists of multiple reflections as well as leaked optical power, may be 10 dB lower than the lowest signal level. Since the permissible noise power for each noise component is for further study, thus the following clauses IV.2 and IV.3 are tentatively described on the assumption that each noise component be at least 10 dB lower than the signal. This assumption should be modified and interpreted to some higher value than 10 dB, for example to 13 dB, according to the permissible noise power assigned to each noise component such as reflections and leaked power.

## IV.2	Effect of ODN optical return loss in the Basic Band

### IV.2.1	Reflectance model to be considered

Figure IV.1/G.983.3 – Reflectance model to be considered

The following notations are used in this appendix:

POru_n			Optical output power of ONU #n transmitter at Oru

POld			Optical output power of OLT transmitter at Old

PEOld			Optical output power of E-OLT transmitter at Old

Ronu_t			ONU transmitter equipment reflectance

Rolt_t			OLT transmitter equipment reflectance

Reonu_r		E-ONU receiver equipment reflectance

Rupper			ORL of ODN at Oru and Ord

Rdown			ORL of ODN at Old and Olu

LODN-BB		Optical loss of Basic Band in the ODN

LODN-EB		Optical loss of Enhancement Band in the ODN

LWF1			Insertion loss of WF1

LWF2			Insertion loss of WF2

Iolt_t			WDM isolation for OLT transmitter

Iolt_r			WDM isolation for OLT receiver

Ionu_r			WDM isolation for ONU receiver

IWF1			WF1 isolation for Basic Band

IWF2			WF2 isolation for Basic Band

These values are all treated as positive in this appendix.

The following assumptions are taken for the examples:

	LODN-BB = LODN-EB

	Minimum insertion loss for WF1 and WF2 is equal to 0 dB.

	Insertion loss between the port at reference point (c) and the port at IFPON side of WF1 is identical for upstream/downstream 1.3 µm, and upstream/downstream 1.5 µm Basic Band wavelengths.

	Insertion loss between the port at reference point (e) and the port at IFPON side of WF2 is identical for upstream/downstream 1.3 µm, and upstream/downstream 1.5µm Basic Band wavelengths.

### IV.2.2	Influence of reflectance into ONU receiver

Figure IV.2 shows the path of reflected signal to be considered. Equation (IV-1) must be satisfied:

POru_1 – Rupper – Ionu_r < (permissible interference optical power)	(IV-1)

Assuming permissible interference optical power is equal to (minimum sensitivity –10 dB), permissible interference optical power = Pmin(A,B,C) – 10 dB. Pmin(A,B,C) means minimum sensitivity at reference point (e) for Class A, B, C described in Table V.1.

Then Ionu_r is estimated and summarized in Table IV.1.

Figure IV.2/G.983.3 – Model for incidence into ONU receiver

### IV.2.3	Influence of reflectance into OLT receiver (in the signal region)

The analysis for the influence of reflectance into OLT receiver is performed in two conditions; one is that the reflected signal overlaps the upstream burst signal region and the other is that the reflected signal is in the delay measurement window where there is no signal.

In the signal region, the following three cases are to be considered.

#### IV.2.3.1	Case 1

Figure IV.3 shows the path of reflectance signals. Equation (IV-2) must be satisfied:

(maximum difference of burst signal optical levels) – Rupper  2LWF2  Ronu_t	(IV-2)

< (permissible interference optical power ratio)

Assuming permissible interference optical power ratio is 10 dB, we obtain the results (see Table IV.1). Here, loss of LWF2 is assumed as 0 dB to estimate the worst case.

Figure IV.3/G.983.3 – Model 1 for incidence into OLT receiver

#### IV.2.3.2	Case 2

Figure IV.4 shows the path of reflectance signals. Equation (IV-3) must be satisfied:

(maximum difference of burst signal optical levels)  Rolt_t – Rdown  2 LWF1  2Iolt_t	(IV-3)

< (permissible interference optical power )

Assuming permissible interference optical power is equal to 10 dB, we obtain the results (see Table IV.1). To estimate the worst case of Iolt_t, loss of LWF1 is assumed as 0 dB.

Figure IV.4/G.983.3 – Model 2 for incidence into OLT receiver

#### IV.2.3.3	Case 3

Figure IV.5 shows the path of reflectance signals. Equation (IV-4) must be satisfied:

POld  Rdown  Iolt_r < (permissible interference optical power)	(IV-4)

Assuming permissible interference optical power is equal to minimum sensitivity 10 dB, we obtain the results (see Table IV.1).

Figure IV.5/G.983.3 – Model 3 for incidence into OLT receiver

### IV.2.4	Influence of reflectance into OLT receiver (in the no signal region)

In the no signal region, main cases of reflectance to be considered are the following two cases.

#### IV.2.4.1	Case 1

Figure IV.3 shows the path of reflectance signals. Equation (IV-5) must be satisfied:

POru_n – Rupper – 2LWF2 – Ronu_t – (minimum optical path attenuation)	(IV-5)

< (determination level as no signal)

Assuming determination level as no signal is equal to minimum sensitivity 10 dB, we obtain the results (see Table IV.1). To estimate the worst case of Ronu_t, loss of LWF2 is assumed as 0 dB.

#### IV.2.4.2	Case 2

Figure IV.5 shows the path of reflectance signals. Equation (IV-6) must be satisfied:

POld – Rdown – Iolt_r < (determination level as no signal)	(IV-6)

Assuming determination level as no signal is equal to minimum sensitivity 10 dB, we obtain the results (see Table IV.1).

### IV.2.5	Summary in effects of ODN optical return loss in the Basic Band

The calculation method mentioned above is available for the case that ODN reflectance is 20 dB. Table IV.1 shows the requirement for optical parameters when minimum ORL of ODN is 32 dB and 20 dB.

WDM isolation parameter is implementation matter, and values concerning WDM isolation parameters in Table IV.1 are just informative. This appendix includes ONU and OLT equipment reflectance. Considering the characteristic of the WDM, Ronu_t is equal to the reflectance of ONU measured at transmitter wavelength.

When ORL of ODN is 32 dB, ONU transmitter equipment reflectance must be less than incident optical power. It shall be 6 dB, which is available in an ordinary FP-LD module.

In the case that ORL of ODN is 20 dB, ONU transmitter equipment reflectance must be less than 13.5 dB.

As mentioned above, maximum ONU transmitter equipment reflectance is sensitive to the value of ORL of ODN, which depends on the network built by the common carrier. In the case that ORL of ODN is 32 dB and 20 dB, values of equipment reflectance for ONU transmitter in Table IV.1 are applicable. In the other case, the appropriate value is derived by means of the calculation method mentioned above.

Table IV.1/G.983.3 – Values for ONU transmitter equipment reflectance

| Min ORL of ODN | Class | Optical parameters | Required characteristics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Aa) | Ba) | Ca) | Da) | Ea) | Fa) |
| 32 dB | A | WDM isolation for ONU receiver | 6.5 |  |  |  |  |  |
|  |  | WDM isolation for ONU transmitter |  |  |  |  |  |  |
|  |  | WDM isolation for OLT receiver |  |  |  | 3.5 |  | 3.5 |
|  |  | WDM isolation for OLT transmitter |  |  | NA |  |  |  |
|  |  | Equipment reflectance for ONU transmitter |  | 0.5 |  |  | 1.5 |  |
|  | B | WDM isolation for ONU receiver | 8.5 |  |  |  |  |  |
|  |  | WDM isolation for ONU transmitter |  |  |  |  |  |  |
|  |  | WDM isolation for OLT receiver |  |  |  | 11.5 |  | 11.5 |
|  |  | WDM isolation for OLT transmitter |  |  | NA |  |  |  |
|  |  | Equipment reflectance for ONU transmitter |  | 0.5 |  |  | 1.5 |  |
|  | C | WDM isolation for ONU receiver | 13.5 |  |  |  |  |  |
|  |  | WDM isolation for ONU transmitter |  |  |  |  |  |  |
|  |  | WDM isolation for OLT receiver |  |  |  | 16.5 |  | 16.5 |
|  |  | WDM isolation for OLT transmitter |  |  | NA |  |  |  |
|  |  | Equipment reflectance for ONU transmitter |  | 0.5 |  |  | 1.5 |  |
| 20 dB | A | WDM isolation for ONU receiver | 18.5 |  |  |  |  |  |
|  |  | WDM isolation for ONU transmitter |  |  |  |  |  |  |
|  |  | WDM isolation for OLT receiver |  |  |  | 15.5 |  | 15.5 |
|  |  | WDM isolation for OLT transmitter |  |  | 3.3 |  |  |  |
|  |  | Equipment reflectance for ONU transmitter |  | 12.5 |  |  | 13.5 |  |
|  | B | WDM isolation for ONU receiver | 20.5 |  |  |  |  |  |
|  |  | WDM isolation for ONU transmitter |  |  |  |  |  |  |
|  |  | WDM isolation for OLT receiver |  |  |  | 23.5 |  | 23.5 |
|  |  | WDM isolation for OLT transmitter |  |  | 3.3 |  |  |  |
|  |  | Equipment reflectance for ONU transmitter |  | 12.5 |  |  | 13.5 |  |
|  | C | WDM isolation for ONU receiver | 25.5 |  |  |  |  |  |
|  |  | WDM isolation for ONU transmitter |  |  |  |  |  |  |
|  |  | WDM isolation for OLT receiver |  |  |  | 28.5 |  | 28.5 |
|  |  | WDM isolation for OLT transmitter |  |  | 3.3 |  |  |  |
|  |  | Equipment reflectance for ONU transmitter |  | 12.5 |  |  | 13.5 |  |
| a) A, B, C, D, E and F represent Equation (IV-1), Equation (IV-2), Equation (IV-3), Equation (IV-4), Equation (IV-5) and Equation (IV-6), respectively. |  |  |  |  |  |  |  |  |

## IV.3	Influence of reflectance from Enhancement Band to Basic Band

### IV.3.1	Influence of reflectance into OLT receiver from Enhancement Band

Service using Enhancement Band signals may degrade quality of the Basic Band (ATM-PON) service, and vice versa. Three types of reflection of Enhancement Band signals are considered. One is the reflection from the ODN, and the others are those from the E-ONU and the blocking filter.

#### IV.3.1.1	Case 1

Figure IV.6 shows the path of reflectance signals from ODN. Equation (IV-7) must be satisfied:

PEOld – Rdown  Iolt_r – IWF1 < (determination level as no signal)	(IV-7)

Assuming determination level as no signal is equal to minimum sensitivity 10 dB, we obtain the condition for IWF1. Example for required isolation of IWF1 is shown in Table IV.2. Example conditions refer to Table IV.1 and Table V.1.

Figure IV.6/G.983.3 – Model 4 for incidence into OLT receiver

Table IV.2/G.983.3 – ORL versus Isolation at WF1

| Min ORL of ODN | Class | Example conditions |  |  | Required IWF1 (dB) |
| --- | --- | --- | --- | --- | --- |
|  |  | PEold (dBm) | Iolt_r (dB) | Pmin (dBm) |  |
| 32 dB | A | +16 | 3.5 | –28.5 | 19 |
|  | B |  | 11.5 | –31.5 | 14 |
|  | C |  | 16.5 | –34.5 | 12 |
| 20 dB | A |  | 15.5 | –28.5 | 19 |
|  | B |  | 23.5 | –31.5 | 14 |
|  | C |  | 28.5 | –34.5 | 12 |

#### IV.3.1.2	Case 2

Figure IV.7 shows the path of reflectance signals from E-ONU. Equation (IV-8) must be satisfied:

Σ (PEOld – 2 LODN-EB – Reonu_r )  Iolt_r – IWF1 < (determination level as no signal)	(IV-8)

Here, the first Σ (summation) denotes reflection from all connected E-ONUs. In other words,

PEOld – 2 LODN-EB  Reonu_r + 10 × LogN  Iolt_r – IWF1 < (determination level as no signal).

Here, N shows the number of connected E-ONUs.

Assuming determination level as no signal is equal to minimum sensitivity 10 dB, we obtain the condition for IWF1. Example for required isolation of IWF1 is shown in Table IV.3. Minimum loss of ODN and appropriate number of N is assumed. Example conditions refer to Tables IV.1 and V.1.

Figure IV.7/G.983.3 – Model 5 for incidence into OLT receiver

Table IV.3/G.983.3 – ORL versus Isolation at WF1

| Min ORL of ODN | Assumption conditions |  |  |  |  |  |  | Required IWF1 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ODN Class | Loss of ODN (dB) | No. of E-ONU N | PEold (dBm) | Iolt_r (dB) | Pmin (dBm) | Reonu_r (dB) |  |
| 32 dB | A | 5 | 2 | +16 | 3.5 | –28.5 | 20 | 24 |
|  | B | 10 | 8 |  | 11.5 | –31.5 |  | 15 |
|  | C | 15 | 32 |  | 16.5 | –34.5 |  | 9 |
| 20 dB | A | 5 | 2 |  | 15.5 | –28.5 |  | 12 |
|  | B | 10 | 8 |  | 23.5 | –31.5 |  | 3 |
|  | C | 15 | 32 |  | 28.5 | –34.5 |  | NA |

#### IV.3.1.3	Case 3

Deployment scenarios described in Appendix II demonstrate configuration example of mixture in ONUs served by Basic Band only and ONUs served both by Basic Band and Enhancement Band. In this case, Blocking filters for Basic Band only users may be applied. Some type of Blocking filter may cause reflections to degrade signal quality.

Figure IV.8 shows the path of reflection signals from Blocking filters. Equation (IV-9) must be satisfied:

Σ (PEOld – 2 LODN-EB – RWF2_r )  Iolt_r – IWF1 < (determination level as no signal)	(IV-9)

Here, RWF2_r means the reflectance of the Blocking filter and the first Σ (summation) denotes reflection from all connected Blocking Filters. In other words,

PEOld – 2 LODN-EB  RWF2_r + 10 × LogN  Iolt_r – IWF1 < (determination level as no signal).

Here, N shows the number of connected blocking filters.

Assuming determination level as no signal is equal to minimum sensitivity 10 dB, we obtain the condition for RWF2_r. An example for required reflectance of RWF2_r is shown in Table IV.4. Minimum loss of ODN and appropriate number of N is assumed. Isolation of IWF1 is assumed to be larger value among those in Tables IV.2 and IV.3. Example conditions refer to Tables IV.1 and V.1.

Figure IV.8/G.983.3 – Model 6 for incidence into OLT receiver

Table IV.4  ORL versus Isolation at WF1

| Min ORL of ODN | Assumption conditions |  |  |  |  |  |  | Required RWF2_r (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ODN Class | Loss of ODN (dB) | No. of E-ONU N | PEold (dBm) | Iolt_r (dB) | Pmin (dBm) | IWF1 (dB) |  |
| 32 dB | A | 5 | 2 | +16 | 3.5 | –28.5 | 24 | 20 |
|  | B | 10 | 8 |  | 11.5 | –31.5 | 15 | 20 |
|  | C | 15 | 32 |  | 16.5 | –34.5 | 12 | 17 |
| 20 dB | A | 5 | 2 |  | 15.5 | –28.5 | 19 | 13 |
|  | B | 10 | 8 |  | 23.5 | –31.5 | 14 | 9 |
|  | C | 15 | 32 |  | 28.5 | –34.5 | 12 | 5 |

Appendix V

Optical parameters for reference point (c) and (e) and 
two example deployment cases

## V.1	Introduction

The optical power parameters for reference points of (c) and (e) shown here are examples to guide implementation.

## V.2	Optical level diagram examples

### V.2.1	Case 1: ODN centric view of optical parameters

Optical parameters examples for IFPON are assumed to be the same as those described in Appendix I and the attenuation range of the ODN is compliant to ITU-T G.982.

In this case, example optical power level diagrams are shown below:

WDM loss of WF1 = 0 dB to 1.5 dB, WDM loss of WF2 = 0 dB to 1.5 dB.

Optical path penalty of 1 dB is included at the reference points of R/S and S/R at IFPON.

Table V.1/G.983.3 – Optical power level diagram (example)

|  | Ref.(e) |  | IFPON (Oru, Ord) |  | ODN loss |  | IFPON (Olu, Old) |  | Ref.(c) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | dBm |  | dBm |  | dB |  | dBm |  | dBm |  |
| Range | Min | Max | Min | Max | Min | Max | Min | Max | Min | Max |
| Downstream |  |  |  |  |  |  |  |  |  |  |
| 155M Class A | –30 | –8 | –28.5 | –8 | 5 | 20 | –7.5 | –3 | –6 | –3 |
| 155M Class B | –30 | –8 | –28.5 | –8 | 10 | 25 | –2.5 | 2 | –1 | 2 |
| 155M Class C | –33 | –11 | –31.5 | –11 | 15 | 30 | –0.5 | 4 | 1 | 4 |
| 622M Class A | –28 | –6 | –26.5 | –6 | 5 | 20 | –5.5 | –1 | –4 | –1 |
| 622M Class B | –28 | –6 | –26.5 | –6 | 10 | 25 | –0.5 | 4 | 1 | 4 |
| 622M Class C | –33 | –11 | –31.5 | –11 | 15 | 30 | –0.5 | 4 | 1 | 4 |
| Upstream |  |  |  |  |  |  |  |  |  |  |
| 155M Class A | –6 | 0 | –7.5 | 0 | 5 | 20 | –28.5 | –5 | –30 | –5 |
| 155M Class B | –4 | 2 | –5.5 | 2 | 10 | 25 | –31.5 | –8 | –33 | –8 |
| 155M Class C | –2 | 4 | –3.5 | 4 | 15 | 30 | –34.5 | –11 | –36 | –11 |

### V.2.2	Case 2: Equipment centric view of optical parameters

Optical parameters for reference points of (c) and (e) are same as those specified in ITU-T G.983.1 and the attenuation range of the ODN is reduced from that in ITU-T G.982.

The purpose of this clause is to survey the BPON applicability of optical modules compliant to ITU-T G.983.1. Maximum variation of the attenuation range in the ODN is reduced in order to

compensate for the extra dynamic range induced by WF1 and WF2. Applied ODN parameters are shown in Table V.2.

With regards to ODN Class A, optical parameters are not specified in ITU-T G.983.1, thus the case of Class A is not described.

Table V.2/G.983.3 – Physical medium dependant layer parameters of ODN

| Items | Unit | Specification |
| --- | --- | --- |
| Fibre type | – | ITU-T G.652 |
| Attenuation range | dB | Reduced Class B with attenuation range: 10-22 Reduced Class C with attenuation range: 15-27 |
| Differential optical path loss | dB | 12 |
| Maximum optical path penalty | dB | 1 |
| Max differential logical reach | km | 20 |
| Maximum fibre distance between S/R and R/S points | km | 20 |
| Minimum supported split ratio | – | Restricted by path loss and ONU addressing limits. PON with passive splitters (16 or 32 way split) |
| Bidirectional transmission | – | 1-fibre WDM (2-fibre system is beyond the scope of this Recommendation) |
| Maintenance wavelength | nm | To be defined |

In this case, example optical power level diagrams are shown below:

WDM loss of WF1 = 0 dB to 1.5 dB, WDM loss of WF2 = 0 dB to 1.5 dB.

Optical path penalty of 1 dB is included at the reference points of R/S and S/R at IFPON.

Table V.3/G.983.3 – Optical power level diagram (example)

|  | Ref.(e) |  | IFPON (Oru, Ord) |  |  | ODN loss |  |  | IFPON (Olu, Old) |  | Ref.(c) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | dBm |  | dBm |  |  | dB |  |  | dBm |  | dBm |  |
| Range | Min | Max | Min | Max |  | Min | Max |  | Min | Max | Min | Max |
| Downstream |  |  |  |  |  |  |  |  |  |  |  |  |
| 155M reduced Class B | –30 | –8 | –28.5 |  | –8 | 10 |  | 22 | –5.5 | 2 | –4 | 2 |
| 155M reduced Class C | –33 | –11 | –31.5 |  | –11 | 15 |  | 27 | –3.5 | 4 | –2 | 4 |
| 622M reduced Class B | –28 | –6 | –26.5 |  | –6 | 10 |  | 22 | –3.5 | 4 | –2 | 4 |
| 622M reduced Class C | –33 | –11 | –31.5 |  | –11 | 15 |  | 27 | –3.5 | 4 | –2 | 4 |
| Upstream |  |  |  |  |  |  |  |  |  |  |  |  |
| 155M reduced Class B | –4 | 2 | –5.5 |  | 2 | 10 |  | 22 | –28.5 | –8 | –30 | –8 |
| 155M reduced Class C | –2 | 4 | –3.5 |  | 4 | 15 |  | 27 | –31.5 | –11 | –33 | –11 |

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |