|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.983.3 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | Amendment 1 (06/2002) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital sections and digital line system – Optical line systems for local and access networks |  |  |  |
|  | A broadband optical access system with increased service capability by wavelength allocation Amendment 1 |  |  |  |
|  | ITU-T Recommendation G.983.3 (2001) – Amendment 1 |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| General | G.900–G.909 |
| Parameters for optical fibre cable systems | G.910–G.919 |
| Digital sections at hierarchical bit rates based on a bit rate of 2048 kbit/s | G.920–G.929 |
| Digital line transmission systems on cable at non-hierarchical bit rates | G.930–G.939 |
| Digital line systems provided by FDM transmission bearers | G.940–G.949 |
| Digital line systems | G.950–G.959 |
| Digital section and digital transmission systems for customer access to ISDN | G.960–G.969 |
| Optical fibre submarine cable systems | G.970–G.979 |
| Optical line systems for local and access networks | G.980–G.989 |
| Access networks | G.990–G.999 |
| QUALITY OF SERVICE AND PERFORMANCE | G.1000–G.1999 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.6000–G.6999 |
| DIGITAL TERMINAL EQUIPMENTS | G.7000–G.7999 |
| DIGITAL NETWORKS | G.8000–G.8999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.983.3 A broadband optical access system with increased service capability by wavelength allocation Amendment 1

Summary ITU-T Rec. G.983.1 on "Broadband optical access systems based on passive optical networks (PON)" describes systems with transmission rates of 155.52 or 622.08 Mbit/s downstream and 155.52 Mbit/s upstream. G.983.1 Amendment 1 allows enhancement to include 622.08 Mbit/s upstream. ITU-T Rec. G.983.3 which describes "Broadband Passive Optical Network (B-PON) systems with increased service capability by wavelength allocation" could not take account of Amendment 1/G.983.1. This Amendment describes modifications to the requirements for isolation and return loss in G.983.3 made necessary because of the power budget changes needed for 622.08 Mbit/s upstream rate.

Source Amendment 1 to ITU-T Recommendation G.983.3 was prepared by ITU-T Study Group 15 (2001-2004) and approved under the WTSA Resolution 1 procedure on 13 June 2002.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2002

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS

Page

1)	Introduction		1

2)	Modifications to ITU-T Rec. G.983.3		1

2.1)	Scope		1

2.2)	References		1

2.3)	Clause 8.3.1		1

2.4)	Clause I.2		3

2.5)	Clause IV.2.5		4

2.6)	Clause IV.3.1		5

2.7)	Subclause IV.3.1.1		5

2.8)	Subclause IV.3.1.2		6

2.9)	Sublause IV.3.1.3		6

2.10)	Clause V.2.1		7

2.11)	Clause V.2.2		8

ITU-T Recommendation G.983.3

A broadband optical access sytem with increased service capability by wavelength allocation

AMENDMENT  1

# 1)	Introduction

This amendment describes modifications to the requirements for isolation and return loss in ITU-T Rec. G.983.3 to align with ITU-T Rec. G.983.1 Amemdment 1 on 622.08 Mbit/s upstream bit rate operation.

# 2)	Modifications to ITU-T Rec. G.983.3

## 2.1)	Scope

All words of  "G.983.1" in Scope should be replaced by  "G.983.1 and G.983.1 Amendment 1".

## 2.2)	References

Change the reference [2] as follows:

[2] 	ITU-T Recommendation G.983.1 (1998), Broadband optical access systems based on Passive Optical Networks (PON). Also refer to the Corrigendum 1 (July, 1999) and Amendment 1 (Nov., 2001).

## 2.3)	Clause 8.3.1

The list of options in 8.3.1 should be updated to include the following items:

–	Option 1: Symmetric 155.52 Mbit/s.

–	Option 2: Asymmetric 155.52 Mbit/s upstream / 622.08 Mbit/s downstream.

–	Option 3: Symmetric 622.08 Mbit/s.

Table 3 and its associated texts are modified to include the following items:

Table 3/G.983.3 – Relation between parameter categories and tables

| Transmission direction | Nominal bit rate | Table |
| --- | --- | --- |
| Downstream | 155.52 Mbit/s | Table 4b (downstream, 155 Mbit/s) |
|  | 622.08 Mbit/s | Table 4c (downstream, 622 Mbit/s) |
| Upstream | 155.52 Mbit/s | Table 4d (upstream, 155 Mbit/s) |
|  | 622.08 Mbit/s | Table 4e (upstream, 622 Mbit/s) |

All parameters are specified as follows, and shall be in accordance with Table 4a (ODN), Table 4b (downstream, 155 Mbit/s), Table 4c (downstream, 622 Mbit/s), Table 4d (upstream, 155 Mbit/s), and Table 4e (upstream 622 Mbit/s). These tables are generally called Table 4 in this Recommendation, if no confusion is possible. There are 9 kinds of ONUs. They are distinguished by downstream and upstream bit rates of 155.52 Mbit/s and 622.08 Mbit/s and by optical path loss of Class A, Class B, and Class C (defined in ITU-T Rec. G.982). Some parameters described in Table 4 are described in Appendix I as examples for implementation.

Add Table 4e/G.983.3 as follows:

| Table 4e/G.983.3 – Optical interface parameters of 622 Mbit/s upstream direction |  |  |  |  |
| --- | --- | --- | --- | --- |
| Items | Unit | Specifications |  |  |
| ONU Transmitter (optical interface Oru) |  |  |  |  |
| Nominal bit rate | Mbit/s | 622.08 |  |  |
| Operating wavelength | nm | 1260-1360 |  |  |
| Line code | – | Scrambled NRZ |  |  |
| Mask of the transmitter eye diagram | – | See Figure 7/G.983.1 |  |  |
| Maximum reflectance of equipment, measured at transmitter wavelength | dB | Less than 6 |  |  |
| Minimum ORL of ODN at Oru and Ord (Notes 1 and 2) | dB | More than 32 |  |  |
| ODN Class |  | Class A | Class B | Class C |
| Mean launched power MIN | dBm | (Note 5) | (Note 5) | (Note 5) |
| Mean launched power MAX | dBm | (Note 5) | (Note 5) | (Note 5) |
| Launched optical power without input to the transmitter | dBm | less than Min sensitivity 10 |  |  |
| Extinction ratio | dB | More than 10 |  |  |
| Tolerance to the transmitter incident light power | dB | More than 15 |  |  |
| If MLM Laser – Maximum RMS width (Note 3) | nm | MLM type 1: 1.4 MLM type 2: 2.1 MLM type 3: 2.7 |  |  |
| If SLM Laser – Maximum 20 dB width (Note 4) | nm | 1 |  |  |
| If SLM Laser – Minimum side mode suppression ratio | dB | 30 |  |  |
| Jitter transfer | – | See Figure 8/G.983.1 |  |  |
| Jitter generation from 0.5 kHz to 1.3 MHz | UI p-p | 0.2 |  |  |
| OLT Receiver (optical interface Olu) |  |  |  |  |
| Maximum reflectance of equipment, measured at receiver wavelength | dB | Less than 20 |  |  |
| Bit error ratio | – | Less than 1010 |  |  |
| ODN Class |  | Class A | Class B | Class C |
| Minimum sensitivity | dBm | (Note 5) | (Note 5) | (Note 5) |
| Minimum overload | dBm | (Note 5) | (Note 5) | (Note 5) |
| Consecutive identical digit immunity | bit | More than 72 |  |  |
| Jitter tolerance | – | NA |  |  |
| Tolerance to the reflected optical power | dB | Less than 10 |  |  |
| NOTE 1 – The value of "minimum ORL of ODN at point Oru and Ord, and Olu and Old" should be more than 20 dB in optional cases which are described in Appendix I/G.983.1. NOTE 2 – The values of ONU transmitter reflectance for the case that the value of "minimum ORL of ODN at point Oru and Ord, and Olu and Old" is 20 dB are described in Appendix IV. NOTE 3 – Transmitter types meeting narrower spectral width specifications are allowed wider central wavelength ranges. The specified laser types produce less than 1 dB of optical path penalty over the ODN. Lasers with different optical parameters may be substituted provided that: 1) the total wavelength range does not exceed 1260 nm to 1360 nm; and 2) any increase in optical path penalty over 1 dB is compensated by an increase of the minimum transmitted launch power or a decrease of the minimum receiver sensitivity. For interoperability, the specified laser types with less than 1 dB optical path penalty are recommended. NOTE 4 – Values of maximum 20 dB width, and minimum side mode suppression ratio are referred to in ITU-T Rec. G.957. NOTE 5 – Tentative parameters are described in Appendix I |  |  |  |  |

## 2.4)	Clause I.2

Change the description as follows:

Specified numerical values described in the following tables indicate tentative values for items described in Table 4 of Recommendation G.983.3. Tables I.1 to I.4 are based on case 2 described in Appendix II of Recommendation G.983.3. Table I.5 is based on case 4 because the upstream signals pass through fewer WDM filters than case 2 and can be relaxed on their ODN attenuation.

Add Table I.4/G.983.3 and Table I.5/G.983.3 as follows:

Table I.4/G.983.3 – Optical interface parameters of 622 Mbit/s upstream direction

| Items | Unit | ODN Class |  |  |
| --- | --- | --- | --- | --- |
|  |  | Class A | Class B | Class Ca) |
| ONU Transmitter (optical interface Oru) |  |  |  |  |
| Mean launched power MIN | dBm | –7.5 | –2.5 | –2.5 |
| Mean launched power MAX | dBm | –1 | +4 | +4 |
| OLT Receiver (optical interface Olu) |  |  |  |  |
| Minimum sensitivity | dBm | –28.5 | –28.5 | –33.5 |
| Minimum overload | dBm | –6 | –6 | –11 |
| a) The values proposed for upstream Class C are best estimates. They are therefore subject to change in the future. |  |  |  |  |

Table I.5/G.983.3 – Optical interface parameters of 622 Mbit/s downstream and upstream directions based on case 4 described in Appendix II

| Items | Unit | ODN Class |  |  |
| --- | --- | --- | --- | --- |
|  |  | Class A | Class B | Class C |
| OLT Transmitter |  |  |  |  |
| Mean Launched Power MIN | dBm | 5.5 | 0.5 | 0.5 |
| Mean Launched Power MAX | dBm | 1 | +4 | +4 |
| ONU Receiver |  |  |  |  |
| Minimum Sensitivity | dBm | 26.5 | 26.5 | 31.5 |
| Minimum Overload | dBm | 6 | 6 | 11 |
| ONU Transmitter |  |  |  |  |
| Mean Launched Power MIN | dBm | 6 | 1 | 1 |
| Mean Launched Power MAX | dBm | 1 | 4 | 4 |
| OLT Receiver |  |  |  |  |
| Minimum Sensitivity | dBm | 27 | 27 | 32 |
| Minimum Overload | dBm | 6 | 6 | 11 |

## 2.5)	Clause IV.2.5

Modify Table IV.1/G.983.3 as follows:

| Table IV.1/G.983.3 – Values for ONU transmitter equipment reflectance |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Min ORL |  | Class |  | Optical parameters |  | Required characteristics |  |  |  |  |  |  |
| of ODN |  |  |  |  | Aa) |  | Ba) | Ca) | Da) | Ea) | Fa) | Option |
|  |  |  | WDM isolation for ONU receiver |  | 6.5 |  |  |  |  |  |  | 1 |
|  |  |  | WDM isolation for ONU transmitter |  |  |  |  |  |  |  |  |  |
|  | A |  | WDM isolation for OLT receiver |  |  |  |  |  | 5.5 |  | 5.5 | 2, 3 |
|  |  |  | WDM isolation for OLT transmitter |  |  |  |  | NA |  |  |  | 1 |
|  |  |  | Equipment reflectance for ONU transmitter |  |  |  | 0.5 |  |  | 1.5 |  | 1, 2 |
|  |  |  | WDM isolation for ONU receiver |  | 8.5 |  |  |  |  |  |  | 1, 3 |
|  |  |  | WDM isolation for ONU transmitter |  |  |  |  |  |  |  |  |  |
| 32 dB | B |  | WDM isolation for OLT receiver |  |  |  |  |  | 13.5 |  | 13.5 | 2 |
|  |  |  | WDM isolation for OLT transmitter |  |  |  |  | NA |  |  |  | 1 |
|  |  |  | Equipment reflectance for ONU transmitter |  |  |  | 0.5 |  |  | 1.5 |  | 1, 2 |
|  |  |  | WDM isolation for ONU receiver |  | 13.5 |  |  |  |  |  |  | 1, 2, 3 |
|  |  |  | WDM isolation for ONU transmitter |  |  |  |  |  |  |  |  |  |
|  | C |  | WDM isolation for OLT receiver |  |  |  |  |  | 16.5 |  | 16.5 | 1, 2 |
|  |  |  | WDM isolation for OLT transmitter |  |  |  |  | NA |  |  |  | 1 |
|  |  |  | Equipment reflectance for ONU transmitter |  |  |  | 0.5 |  |  | 1.5 |  | 1, 2 |
|  |  |  | WDM isolation for ONU receiver |  | 18.5 |  |  |  |  |  |  | 1 |
|  |  |  | WDM isolation for ONU transmitter |  |  |  |  |  |  |  |  |  |
|  | A |  | WDM isolation for OLT receiver |  |  |  |  |  | 17.5 |  | 17.5 | 2, 3 |
|  |  |  | WDM isolation for OLT transmitter |  |  |  |  | 3.3 |  |  |  | 1 |
|  |  |  | Equipment reflectance for ONU transmitter |  |  |  | 12.5 |  |  | 13.5 |  | 1, 2 |
|  |  |  | WDM isolation for ONU receiver |  | 20.5 |  |  |  |  |  |  | 1, 3 |
|  |  |  | WDM isolation for ONU transmitter |  |  |  |  |  |  |  |  |  |
| 20 dB | B |  | WDM isolation for OLT receiver |  |  |  |  |  | 25.5 |  | 25.5 | 2 |
|  |  |  | WDM isolation for OLT transmitter |  |  |  |  | 3.3 |  |  |  | 1 |
|  |  |  | Equipment reflectance for ONU transmitter |  |  |  | 12.5 |  |  | 13.5 |  | 1, 2 |
|  |  |  | WDM isolation for ONU receiver |  | 25.5 |  |  |  |  |  |  | 1, 2, 3 |
|  |  |  | WDM isolation for ONU transmitter |  |  |  |  |  |  |  |  |  |
|  | C |  | WDM isolation for OLT receiver |  |  |  |  |  | 28.5 |  | 28.5 | 1, 2 |
|  |  |  | WDM isolation for OLT transmitter |  |  |  |  | 3.3 |  |  |  | 1 |
|  |  |  | Equipment reflectance for ONU transmitter |  |  |  | 12.5 |  |  | 13.5 |  | 1, 2 |
| a) A, B, C, D, E and F represent Equation A, Equation B, Equation C, Equation D, Equation E and Equation F, respectively. |  |  |  |  |  |  |  |  |  |  |  |  |

## 2.6)	Clause IV.3.1

Add the following sentence at the end of the clause:

IWF1 is defined by the worst value of options described in 8.3.1.

## 2.7)	Subclause IV.3.1.1

Modify Table IV.2/G.983.3 as follows:

Table IV.2/G.983.3 – ORL versus Isolation at WF1

| Min ORL of ODN | Class | Example conditions |  |  | Required IWF1 (dB) | Option |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | PEold (dBm) | Iolt_r (dB) | Pmin (dBm) |  |  |
|  | A | +16 | 3.5 | –28.5 | 19 | 1 |
| 32 dB | B |  | 11.5 | –31.5 | 14 | 1 |
|  | C |  | 16.5 | –34.5 | 12 | 1, 2, 3 |
|  | A |  | 15.5 | –28.5 | 19 | 1 |
| 20 dB | B |  | 23.5 | –31.5 | 14 | 1 |
|  | C |  | 28.5 | –34.5 | 12 | 1, 2, 3 |

## 2.8)	Subclause IV.3.1.2

Modify Table IV.3/G.983.3 as follows:

Table IV.3/G.983.3 – ORL versus Isolation at WF1

| Min ORL of ODN | Assumption conditions |  |  |  |  |  |  | RequiredIWF1 (dB) | Option |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ODNClass | Loss of ODN (dB) | No. of E-ONU N | PEold (dBm) | Iolt_r(dB) | Pmin (dBm) | Reonu_r (dB) |  |  |
|  | A | 5 | 2 | +16 | 3.5 | –28.5 | 20 | 24 | 1 |
| 32 dB | B | 10 | 8 |  | 11.5 | –31.5 |  | 15 | 1 |
|  | C | 15 | 32 |  | 16.5 | –34.5 |  | 9 | 1, 2, 3 |
|  | A | 5 | 2 |  | 15.5 | –28.5 |  | 12 | 1 |
| 20 dB | B | 10 | 8 |  | 23.5 | –31.5 |  | 3 | 1 |
|  | C | 15 | 32 |  | 28.5 | –34.5 |  | NA | 1, 2, 3 |

## 2.9)	Sublause IV.3.1.3

Modify Table IV.4/G.983.3 as follows:

Table IV.4/G.983.3 – ORL versus Isolation at WF1

| Min ORL of ODN | Assumption conditions |  |  |  |  |  |  | Required RWF2_r (dB) | Option |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ODNClass | Loss of ODN (dB) | No. of E-ONU N | PEold (dBm) | Iolt_r(dB) | Pmin (dBm) | IWF1 (dB) |  |  |
|  | A | 5 | 2 | +16 | 3.5 | –28.5 | 24 | 20 | 1, 2, 3 |
| 32 dB | B | 10 | 8 |  | 11.5 | –31.5 | 15 | 20 | 1, 2, 3 |
|  | C | 15 | 32 |  | 16.5 | –34.5 | 12 | 17 | 1, 2, 3 |
|  | A | 5 | 2 |  | 15.5 | –28.5 | 19 | 13 | 1, 2, 3 |
| 20 dB | B | 10 | 8 |  | 23.5 | –31.5 | 14 | 9 | 1, 2, 3 |
|  | C | 15 | 32 |  | 28.5 | –34.5 | 12 | 5 | 1, 2, 3 |

## 2.10)	Clause V.2.1

Modify Table V.1/G.983.3 as follows:

Table V.1/G.983.3 – Optical power level diagram (example)

|  | Ref. (e) |  | IFPON (Oru, Ord) |  | ODN loss |  | IFPON (Olu, Old) |  | Ref.(c) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | dBm |  | dBm |  | dB |  | dBm |  | dBm |  |
| Range | Min Max |  | Min Max |  | Min Max |  | Min Max |  | Min Max |  |
| Downstream |  |  |  |  |  |  |  |  |  |  |
| 155M Class A | –30 | –8 | –28.5 | –8 | 5 | 20 | –7.5 | –3 | –6 | –3 |
| 155M Class B | –30 | –8 | –28.5 | –8 | 10 | 25 | –2.5 | 2 | –1 | 2 |
| 155M Class C | –33 | –11 | –31.5 | –11 | 15 | 30 | –0.5 | 4 | 1 | 4 |
| 622M Class A | –28 | –6 | –26.5 | –6 | 5 | 20 | –5.5 | –1 | –4 | –1 |
| 622M Class B | –28 | –6 | –26.5 | –6 | 10 | 25 | –0.5 | 4 | 1 | 4 |
| 622M Class C | –33 | –11 | –31.5 | –11 | 15 | 30 | –0.5 | 4 | 1 | 4 |
| Upstream |  |  |  |  |  |  |  |  |  |  |
| 155M Class A | –6 | 0 | –7.5 | 0 | 5 | 20 | –28.5 | –5 | –30 | –5 |
| 155M Class B | –4 | 2 | –5.5 | 2 | 10 | 25 | –31.5 | –8 | –33 | –8 |
| 155M Class C | –2 | 4 | –3.5 | 4 | 15 | 30 | –34.5 | –11 | –36 | –11 |
| 622M Class A | –6 | –1 | –7.5 | –1 | 5 | 20 | –28.5 | –6 | –30 | –6 |
| 622M Class B | –1 | 4 | –2.5 | 4 | 10 | 25 | –28.5 | –6 | –30 | –6 |
| 622M Class C | –1 | 4 | –2.5 | 4 | 15 | 30 | –33.5 | –11 | –35 | –11 |

## 2.11)	Clause V.2.2

Modify the third paragraph as follows:

With regards to ODN Class A of 155M, optical parameters are not specified in ITU-T Rec. G.983.1, thus the case of Class A is not described for 155M.

Modify Table V.3/G.983.3 as follows:

Table V.3/G.983.3 –  Optical power level diagram (example)

|  | Ref. (e) |  | IFPON (Oru, Ord) |  | ODN loss |  | IFPON (Olu, Old) |  | Ref.(c) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | dBm |  | dBm |  | dB |  | dBm |  | dBm |  |
| Range | Min Max |  | Min Max |  | Min Max |  | Min Max |  | Min Max |  |
| Downstream |  |  |  |  |  |  |  |  |  |  |
| 155M reduced Class B | –30 | –8 | –28.5 | –8 | 10 | 22 | –5.5 | 2 | –4 | 2 |
| 155M reduced Class C | –33 | –11 | –31.5 | –11 | 15 | 27 | –3.5 | 4 | –2 | 4 |
| 622M reduced Class A | –28 | –6 | –26.5 | –6 | 5 | 17 | –8.5 | –1 | –7 | –1 |
| 622M reduced Class B | –28 | –6 | –26.5 | –6 | 10 | 22 | –3.5 | 4 | –2 | 4 |
| 622M reduced Class C | –33 | –11 | –31.5 | –11 | 15 | 27 | –3.5 | 4 | –2 | 4 |
| Upstream |  |  |  |  |  |  |  |  |  |  |
| 155M reduced Class B | –4 | 2 | –5.5 | 2 | 10 | 22 | –28.5 | –8 | –30 | –8 |
| 155M reduced Class C | –2 | 4 | –3.5 | 4 | 15 | 27 | –31.5 | –11 | –33 | –11 |
| 622M reduced Class A | –6 | –1 | –7.5 | –1 | 5 | 17 | –25.5 | –6 | –27 | –6 |
| 622M reduced Class B | –1 | 4 | –2.5 | 4 | 10 | 22 | –25.5 | –6 | –27 | –6 |
| 622M reduced Class C | –1 | 4 | –2.5 | 4 | 15 | 27 | –30.5 | –11 | –32 | –11 |

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |