|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.992.2 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | Amendment 1 (03/2003) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital sections and digital line system – Access networks |  |  |  |
|  | Splitterless asymmetric digital subscriber line (ADSL) transceivers Amendment 1: Revised Annex C |  |  |  |
|  | ITU-T Recommendation G.992.2 (1999) – Amendment 1 |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| General | G.900–G.909 |
| Parameters for optical fibre cable systems | G.910–G.919 |
| Digital sections at hierarchical bit rates based on a bit rate of 2048 kbit/s | G.920–G.929 |
| Digital line transmission systems on cable at non-hierarchical bit rates | G.930–G.939 |
| Digital line systems provided by FDM transmission bearers | G.940–G.949 |
| Digital line systems | G.950–G.959 |
| Digital section and digital transmission systems for customer access to ISDN | G.960–G.969 |
| Optical fibre submarine cable systems | G.970–G.979 |
| Optical line systems for local and access networks | G.980–G.989 |
| Access networks | G.990–G.999 |
| QUALITY OF SERVICE AND PERFORMANCE | G.1000–G.1999 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.6000–G.6999 |
| DIGITAL TERMINAL EQUIPMENTS | G.7000–G.7999 |
| DIGITAL NETWORKS | G.8000–G.8999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.992.2 Splitterless asymmetric digital subscriber line (ADSL) transceivers Amendment 1 Revised Annex C

Summary This Amendment 1 to ITU-T Rec. G.992.2 revises Annex C for improved robustness and extended reach. It includes the changes introduced by G.992.2 (1999) Corrigendum 1 (07/2002).

Source Amendment 1 to ITU-T Recommendation G.992.2 (1999) was prepared by ITU-T Study Group 15 (2001-2004) and approved under the WTSA Resolution 1 procedure on 16 March 2003.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2003

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS

Page

C.1	Scope		1

C.2	Definitions and abbreviations		1

C.3	Reference models		2

C.4	Operating modes (new)		6

C.5	ATU interfaces		7

C.6	ATU functional characteristics (pertains to clause 7)		7

C.7	aoc on-line adaptation and reconfiguration (pertains to clause 9)		14

C.8	In-service performance monitoring and surveillance (pertains to clause 10)		15

C.9	Initialization		16

C.10	Fast retraining (supplements clause 12)		36

C.11	Power management (pertains to clause 13)		41

Appendix IV – Example Overlapped PSD Masks for use in a TCM-ISDN crosstalk environment		41

ITU-T Recommendation G.992.2

Splitterless asymmetric digital subscriber line (ADSL) transceivers

Amendment 1

Revised Annex C

ADSL above POTS co-existing in the same binder as TCM-ISDN DSL

## C.1	Scope

This annex describes those specifications that are unique to an ADSL system co-existing in the same binder as TCM-ISDN as defined in Appendix III/G.961. The subclauses in this annex provide the additions and modifications to the corresponding clauses in the main body. The modifications described in this annex provide a performance improvement over the Splitterless ADSL system specified in main body in an environment co-existing with TCM-ISDN. It is preferred that ADSL system implementing this annex also implements the main body.

This annex defines several optional operating modes or "profiles", negotiable through G.994.1, to allow limited independent control of:

•	FEXT and NEXT period transmission in both upstream and downstream directions

•	overlapped and non-overlapped spectrum downstream during FEXT and NEXT periods

These new optional profiles (defined in C.4 as Profiles 1 to 6) offer improved robustness and extended reach compared to the previously defined operating modes.

NOTE – References to figures or tables of Appendix IV will be defined when Appendix IV is made available.

## C.2	Definitions and abbreviations

### C.2.1	Definitions

C.2.1.1	Dual Bitmap: The Dual Bitmap method has dual bit rates under the FEXT and NEXT noise from TCM-ISDN.

C.2.1.2	FEXT Bitmap: Similar to the Dual Bitmap method; however, transmission only occurs during FEXT noise from TCM-ISDN.

C.2.1.3	Hyperframe: Five Superframes structure which synchronized TTR.

C.2.1.4	Bitmap-FR: ATU-C transmitter bitmap under TCM-ISDN FEXT noise generated at ATU-R.

C.2.1.5	Bitmap-NR: ATU-C transmitter bitmap under TCM-ISDN NEXT noise generated at ATU-R.

C.2.1.6	Bitmap-FC: ATU-R transmitter bitmap under TCM-ISDN FEXT noise generated at ATU-C.

C.2.1.7	Bitmap-NC: ATU-R transmitter bitmap under TCM-ISDN NEXT noise generated at ATU-C.

C.2.1.8	FEXTR duration: TCM-ISDN FEXT duration at ATU-R estimated by the ATU-C.

C.2.1.9	NEXTR duration: TCM-ISDN NEXT duration at ATU-R estimated by the ATU-C.

C.2.1.10	FEXTC duration: TCM-ISDN FEXT duration at ATU-C estimated by the ATU-R.

C.2.1.11	NEXTC duration: TCM-SDN NEXT duration at ATU-C estimated by the ATU-R.

C.2.1.12	FEXTR symbol: DMT symbol transmitted by ATU-C during TCM-ISDN FEXT.

C.2.1.13	NEXTR symbol: DMT symbol transmitted by ATU-C during TCM-ISDN NEXT.

C.2.1.14	FEXTC symbol: DMT symbol transmitted by ATU-R during TCM-ISDN FEXT.

C.2.1.15	NEXTC symbol: DMT symbol transmitted by ATU-R during TCM-ISDN NEXT.

C.2.1.16	NSWF: Sliding window frame counter.

### C.2.2	Abbreviations

TTR	TCM- ISDN Timing Reference

TTRC	Timing Reference used in ATU-C

TTRR	Timing Reference used in ATU-R

UI	Unit Interval

## C.3	Reference models

### C.3.1	ATU-C transmitter reference model (replaces figure in 4.2)

Figure C.1/G.992.2 – ATU-C transmitter reference model for ATM transport

### C.3.2	ATU-R transmitter reference model (replaces figure in 4.2)

Figure C.2/G.992.2 – ATU-R transmitter reference model for ATM transport

### C.3.3	ATU transmitter timing model (new)

#### C.3.3.1	TCM-ISDN crosstalk timing model (new)

Figure C.3 shows the timing chart of the crosstalk from TCM-ISDN.

Figure C.3/G.992.2 – Timing chart of the TCM-ISDN cross-talk

The data stream of TCM-ISDN transmitted in a TTR period. The TCM-ISDN CO transmits the symbols in the first half of the TTR period and the TCM-ISDN RT transmits in the second half of the TTR period. The ATU-C receives NEXT noise from ISDN in the first half of the TTR period and FEXT noise from ISDN in the second half of the TTR period. On the other hand, the ATU-R receives FEXT noise from the TCM-ISDN in the first half of the TTR period and NEXT noise from the ISDN in the second half of the TTR period.

As defined in C.6.2.2 and in C.9, the ATU-C shall estimate the FEXTR and NEXTR duration at ATU-R, and ATU-R shall estimate FEXTC and NEXTC duration at ATU-C taking propagation delay on the subscriber line into consideration.

The ATU-C shall transmit any symbol by synchronizing with the TTRC. The ATU-R shall transmit any symbol by synchronizing with the TTRR generated from received TTRC.

#### C.3.3.2	Sliding Window (new)

Figure C.4 shows the timing chart of the downstream transmission.

Figure C.4/G.992.2 – Sliding Window

The "Sliding Window" operation defines the procedures to transmit symbols under the crosstalk noise environment synchronized to the period of the TTR. The FEXTC/R symbol represents the symbol completely inside the FEXTC/R duration. The NEXTC/R symbol represents the symbol containing any NEXTC/R duration. Thus, there are more NEXTC/R symbols than FEXTC/R symbols.

The ATU-C decides which transmission symbol is a FEXTR or NEXTR symbol according to the Sliding Window and transmits it with the corresponding bit table. Similarly, ATU-R decides which transmission symbol is a FEXTC or NEXTC and transmits it with the corresponding bit table. Although the phase of the Sliding Window is asynchronous with the TTRC/R, the pattern is fixed to the 345 frames of the hyperframe (see C.6.2.2).

#### C.3.3.3	ATU-C symbol synchronization to TTRC (new)

The time duration of 345 symbols is equal to 34 cycles of TTRC (or 32 cycles of TTRC for symbols without a cyclic prefix). This implies a PLL lock at the ATU-R.

#### C.3.3.4	Dual Bitmap switching (new)

The ATU-C transmits FEXTR symbols using Bitmap-FR (in FEXTR duration), and transmits NEXTR symbols using Bitmap-NR (in NEXTR duration) according to the result of initialization. The ATU-R transmits FEXTC symbols using Bitmap-FC (in FEXTC duration), and transmits NEXTC symbols using Bitmap-NC (in NEXTC duration) in the same manner.

The ATU-C shall have the capability to disable Bitmap-NC and Bitmap-NR (see- C.6.7)--

-As an option, an ATU-C may  have the ability to enable or disable Bitmap-NC independently of Bitmap-NR. This is controlled by way of the profiles negotiated through G.994.1.

#### C.3.3.5	Loop timing at ATU-R (new)

The phase relation between received symbol and transmitted symbol of ATU-R at reference point U-R shall meet the phase tolerances as shown in Figure C.5.

Figure C.5/G.992.2 – Loop timing for ATU-R

## C.4	Operating modes (new)

The following profiles are defined to support independent control of FEXT and NEXT bitmaps in the upstream and downstream direction, as well as independent control of the downstream spectrum for each downstream bitmap:

Profile 1

For Profile 1, upstream transmission only uses Bitmap-FC, and downstream transmission only uses Bitmap-FR with non-overlapped spectrum.

Profile 2

For Profile 2, upstream transmission uses both Bitmap-FC and Bitmap-NC, and downstream transmission uses both Bitmap-FR and Bitmap-NR. Non-overlapped spectrum is used with both downstream bitmaps.

Profile 3

For Profile 3, upstream transmission only uses Bitmap-FC, and downstream transmission only uses Bitmap-FR with overlapped spectrum. An example of a downstream PSD mask for this operating mode is shown in Figure IV.3.

Profile 4

For Profile 4, upstream transmission uses both Bitmap-FC and Bitmap-NC, and downstream transmission uses both Bitmap-FR and Bitmap-NR. Overlapped spectrum is used with both downstream bitmaps.

Profile 5

For Profile 5, upstream transmission only uses Bitmap-FC, and downstream transmission uses both Bitmap-FR and Bitmap-NR. Non-overlapped spectrum is used with Bitmap-NR, and overlapped spectrum is used with Bitmap-FR. An example of a downstream PSD mask for use with Bitmap-NR is shown in Figure IV.1 and described in Table IV.1 in Appendix IV. An example of a downstream PSD mask for use with Bitmap-FR is shown in Figure IV.2 and described in Table IV.2 in Appendix IV.

Profile 6

For Profile 6, upstream transmission uses both Bitmap-FC and Bitmap-NC, and downstream transmission uses both Bitmap-FR and Bitmap-NR. Non-overlapped spectrum is used with Bitmap-NR, and overlapped spectrum is used with Bitmap-FR. An example of a downstream PSD mask for use with Bitmap-NR is shown in Figure IV.1 and described in Table IV.1 in Appendix IV. An example of a downstream PSD mask for use with Bitmap-FR is shown in Figure IV.2 and described in Table IV.2 in Appendix IV.

Table 11.9.1/G.994.1 contains the code points to support these profiles.

## C.5	ATU interfaces

### C.5.1	ATM interface for ATM transport (replaces figure in 6.1)

The functional data interface at the ATU for ATM transport are shown in Figure C.6.

Figure C.6/G.992.2 – ATU functional interfaces to the ATM layer at the V or T reference point

## C.6	ATU functional characteristics (pertains to clause 7)

### C.6.1	Payload transfer delay (supplements 7.1.1)

The one-way transfer delay (excluding cell specific functionalities) for payload bits from the V reference point at the central office end (V-C) to the T reference point at remote the end (T-R) shall be as specified in 7.1.1 with an additional 5 ms for transfer delay attributed to the rate converter. The same requirement applies in the opposite direction, from the T-R reference point to the V-C reference point.

The maximum rate converter delay specified in the above text shall not apply to bit rates of 32 kbit/s and 64 kbit/s.

### C.6.2	Framing (pertains to 7.3)

#### C.6.2.1	Superframe structure (supplements 7.3.3.1)

Since the rate converter reorders the user data and overhead bit-level data to create hyperframes, the input data frames to the constellation encoder are different than those defined in 7.3.3.1.

#### C.6.2.2	Hyperframe structure (new)

This annex uses the hyperframe structure shown in Figures C.7 and C.8. Both figures show the phase relationship between the TTRC/R and the hyperframe at the point U-C and U-R. Each hyperframe is composed of five superframes, which are numbered from 0 to 4. In order to indicate the boundary of the hyperframe, the inverse synch symbol is used for the Ninv-th superframe, which is generated from a tone-by-tone 180-degree phase reversal of the synchronization symbol (see C.6.3.1) except for the pilot tone. Ninv-th is defined as:

Ninv-th =3 (SPF#3) for downstream0 (SPF#0) for upstream

The FEC Output Frame from the interleaver is put into the rate-converter. The bit-level data stream from the rate-converter is extracted according to the size of Bitmap-FR/C and Bitmap-NR/C using the Sliding Window (see C.3.3.2 and C.3.3.4).

In order to make the bit rate to be a multiple of 32 kbit/s, the dummy bits are inserted at the end of hyperframe by the rate converter (see C.6.6). The hyperframe is composed of 345 DMT symbols, numbered from 0 to 344. Each symbol is assigned as a FEXTR/C or NEXTR/C symbol in a FEXTR/C or NEXTR/C duration (see C.3.3.1). The following numerical formula gives the information which duration Ndmt-th DMT symbol belongs to at ATU transmitter.

Downstream data (see Figure C.9):

For (Ndmt = 0, 1, ..., 344)

S = 272 × Ndmt mod 2760

if { (S + 271 < a) or (S > a + b) }	then FEXTR symbol

else 		then NEXTR symbol

where a = 1243, b = 1461

Upstream data (see Figure C.10):

For (Ndmt = 0, 1, ..., 344)

S = 272 × Ndmt mod 2760

if { (S > a) and (S + 271 < a + b) }	then FEXTC symbol

else 		then NEXTC symbol

where a = 1315, b = 1293

Thus, 128 DMT symbols are allocated in the FEXTR/C duration (FEXTC/R symbols), and 217 DMT symbols are allocated in the NEXTR/C duration (NEXTC/R symbols). The symbols are composed of:

FEXTC/R symbol:

–	Number of symbols using Bitmap-FR/C =	126

–	Number of synch symbols =	1

–	Number of inverse synch symbols =	1

NEXTC/R symbol:

–	Number of symbols using Bitmap-NR/C =	214

–	Number of synch symbols =	3

For modems not using any of the profiles defined in C.4, and modems using Profile 1, during FEXT Bitmap mode, the ATU-C shall transmit only the pilot tone as NEXTR symbol. For Profile 3, the ATU-C shall not transmit any signal in NEXTR symbols. The remaining Profiles, i.e. Profiles 2, 4, 5, and 6 use the dual bit map technique.

During FEXT Bitmapping mode, the ATU-R shall not transmit any signal as NEXTC symbol.

Figure C.7/G.992.2 – Hyperframe structure for downstream

Figure C.8/G.992.2 – Hyperframe structure for upstream

Figure C.9/G.992.2 – Symbol pattern in a hyperframe with cyclic prefix – Downstream

Figure C.10/G.992.2 – Symbol pattern in a hyperframe with cyclic prefix – Upstream

### C.6.3	Modulation (pertains to 7.10)

#### C.6.3.1	Inverse Synchronization Symbol (new)

Except for the pilot tone, the Inverse Synchronization symbol shall be generated from a tone-by-tone 180-degree phase reversal of Synchronization symbol (i.e. + maps to –, and – maps to +, for each of the 4-QAM signal constellation).

#### C.6.3.2	Gain scaling of synchronization symbol

At Initialization, the sync symbol reference transmit PSD level shall be set at the reference PSD level + 10log(gsync2) dBm/Hz, with gsync2 defined as the average gi2 value over the used (i.e. bi > 0) subcarriers in the NEXT or FEXT bitmap, whichever results in the highest average gain. The sync symbol reference transmit PSD shall not be updated with used subcarrier gain changes during SHOWTIME.

#### C.6.3.3	Downstream Pilot subcarrier (supplements 7.10.1.2)

In the downstream direction, subcarrier nC-PILOT1 (f = nC-PILOT1  f kHz) shall be reserved for a pilot; that is b(nC-PILOT1) = 0 and g(nC-PILOT1) = gsync.

#### C.6.3.4	Downstream data pattern (supplements 7.10.5)

Bits d2i+1 and d2i+2, which modulate the pilot carrier that has tone index i, shall be overwritten by {0,0}, generating the (+,+) constellation point.

### C.6.4	Transmitter spectral mask (replaces 7.12.3)

The spectral mask(s) of this annex shall fall within the masks defined in Annex A or Annex B. For modems not using any of the profiles defined in C.4, when C-MSG1 bit 16 is 0b, the PSD mask specified in Annex A shall be used. When C-MSG1 bit 16 is 1, the PSD mask specified in Annex B shall be used.

For modems complying with Profiles 1 and 2, C-MSG1 bit 16 shall be set to 0. For modems complying with Profiles 3 to 6, C-MSG1 bit 16 shall be set to 1.

The ATU-C may use different PSD masks during FEXTR symbols and NEXTR symbols. These masks may differ from, but shall fall within, the masks defined in Annex A or Annex B. Example PSD masks can be found in Appendix IV.

### C.6.5	Dual Bitmap (new)

The Dual Bitmap method has individual bit rates under the FEXT and NEXT noise, and this needs an additional bit and gain table, {bi, gi}. The dual bitmaps are switched synchronized with the sliding window pattern of NEXT/FEXT symbols.

### C.6.6	Rate Converter (new)

The output of the interleaver is input to the rate converter. The rate converter buffering changes the data frame boundaries between the reference points B and C according to Bitmap-FR/C, Bitmap-NR/C and the Sliding Window. However for the difference of the data rates between the reference points B and C, and to make the bit rate to be a multiple of 32 kbit/s, the dummy bits are inserted at the end of the hyperframe. The number of the dummy bits shall be:

# dummyR = (fR × 126 + nR × 214) – (tR × 340)	for downstream data

#dummyC  = (fC × 126 + nC × 214) – (tC × 340)	for upstream data

where tR is the number of allocated bits in one frame at the reference point B in ATU-C transmitter, fR and nR are the numbers of bits in Bitmap-FR and Bitmap-NR, respectively. Similarly, where tC is the number of allocated bits in one frame at the reference point B in ATU-R transmitter, fC and nC are the numbers of bits in Bitmap-FC and Bitmap-NC, respectively. During FEXT Bitmap mode, nR and nC are zero.

At the receiver, the inserted dummy bits shall be removed.

The receiver shall determine Bitmap-FR/C and Bitmap-NR/C so that the number of dummy bits is less than 126 in initialization sequence.

### C.6.7	FEXT Bitmap (new)

The FEXT Bitmap mode uses the Dual Bitmap technique (see C.6.5) to transmit data only during FEXT. As an option, modems may have the ability to enable or disable Bitmap-NR independently of Bitmap-NC  in order to control the FEXT Bitmap mode upstream and downstream independently.

For modems not using any of the profiles defined in C.4 and modems using Profile 1, the ATU-C shall transmit only the pilot tone as NEXTR symbol. For Profile 3, the ATU-C shall not transmit any signal in NEXTR symbols.

The ATU-R disables Bitmap-NC and shall not transmit any signal as NEXTC symbol (see Figures C.7 and C.8).

For modems not using any of the profiles defined in C.4, the Dual versus FEXT bit mapping mode is selected during G.994.1 using bit "DBM" (see 11.2 and 11.3). For modems using any of the profiles defined in C.4, the bitmap mode is selected during G.994.1.

## C.7	aoc on-line adaptation and reconfiguration (pertains to clause 9)

### C.7.1	Bit swap request message encoding (replaces 9.2.4)

This message tells the transmitter which subcarriers are to be modified. The format of the request is shown in Table C.1.

Table C.1/G.992.2 – Format of the bit swap request message

| Message header | Message field 1-4 |  |  |
| --- | --- | --- | --- |
| {11111111b} (8 bits) | Bitmap index (1 bit) | Command (7 bits) | Subcarrier index (8 bits) |

The request shall comprise nine bytes as follows:

•	An aoc message header consisting of eight binary ones.

•	Message fields 1-4, each of which each consists of a one-bit bitmap index, a seven-bit command followed by a related eight-bit subcarrier index. One-bit bitmap index and valid seven-bit commands for the bit swap message shall be as shown in Table C.2. In Table C.2, the MSB for the bit swap request command represents the bitmap index. In the Bitmap index, 0 indicates Bitmap FR, and 1b indicates NR for downstream data. Similarly, 0 indicates Bitmap FC and 1b indicates NC for upstream data. The eight-bit subcarrier index is counted from low to high frequencies with the lowest frequency subcarrier having the number zero. The subcarrier index zero shall not be used.

•	The bit swap between FEXTC/R symbols and NEXTC/R symbols is not allowed.

Table C.2/G.992.2 – Bit swap request command

| Value (8 bit) | Interpretation |
| --- | --- |
| y0000000b | Do nothing |
| y0000001b | Increase the number of allocated bits by one |
| y0000010b | Decrease the number of allocated bits by one |
| y0000011b | Increase the transmitted power by 1 dB |
| y0000100b | Increase the transmitted power by 2 dB |
| y0000101b | Increase the transmitted power by 3 dB |
| y0000110b | Reduce the transmitted power by 1 dB |
| y0000111b | Reduce the transmitted power by 2 dB |
| y0001xxxb | Reserved for vendor discretionary commands |
| NOTE – y is "0b" for FEXTC/R symbols, and "1b" for NEXTC/R symbols of the Sliding Window. |  |

To avoid gi divergence between ATU-C and ATU-R after several bit swaps, for a gi update of  dB the new gi value should be given by:

gi = (1/512) × round(512 × gi × 10 exp(/20))

### C.7.2	Extended bit swap request message encoding (supplements 9.2.5)

The format of the extended bit swap request is shown in Table C.3.

Table C.3/G.992.2 – Format of the bit swap request message

| Message header | Message field 1-6 |  |  |
| --- | --- | --- | --- |
| {11111100b} (8 bits) | Bitmap index (1 bit) | Command (7 bits) | Subcarrier index (8 bits) |

In the same manner as the bit swap request, each of the massage fields of the extended bit swap request consists of a one-bit bitmap index, a seven-bit command followed by a related eight-bit subcarrier index.

### C.7.3	Bit swap acknowledge message encoding (supplements 9.2.6)

The bit swap superframe counter number shall only indicate the last superframe (SPF#4) of a hyperframe.

The new bit and/or transmit power table(s) shall then take effect starting from the first frame (frame 0) of SPF#0 of a hyperframe.

If the bit swap superframe counter number contained in the received bit swap acknowledge message does not indicate SPF#4, then the new table(s) shall take effect starting from frame 0 of SPF#0 of the next hyperframe.

## C.8	In-service performance monitoring and surveillance (pertains to clause 10)

### C.8.1	ADSL line related primitives (pertains to 10.1)

#### C.8.1.1	ADSL line related near-end defects (supplements 10.1.3)

Two near-end defects are defined:

•	Loss of signal (LOS): ADSL power shall be measured only in the FEXTC duration at ATU-C, or only in the FEXTR duration at ATU-R.

•	Severely Errored Frame (SEF): A SEF defect occurs when the content of two consecutively received ADSL synchronization symbols in the FEXTC duration at ATU-C, or in the FEXTR duration at ATU-R, does not correlate with the expected content over a subset of the tones. A SEF defect terminates when the content of two consecutively received ADSL synchronization symbols in the FEXTC duration at ATU-C, or in the FEXTR duration at ATU-R, correlate with the expected contents over the same subset. The correlation method, the selected subset of tones, and the threshold for declaring these defect conditions are implementation discretionary.

#### C.8.1.2	ADSL line related far-end defects (supplements 10.1.4)

•	Far-end Loss of signal (LOS): The ADSL power shall be measured only in the FEXTC duration at ATU-C, or only in the FEXTR duration at ATU-R.

### C.8.2	Test parameters (supplements 10.4)

#### C.8.2.1	Near-end test parameters (supplements 10.4.1)

The following near-end test parameters are defined:

•	Attenuation (ATN): The received signal power shall be measured only in the FEXTC duration at ATU-C, or only in the FEXTR duration at ATU-R.

•	Signal-to-Noise ratio (SNR) margin: During the FEXT Bitmap mode, this primitive represents the SNR margin in the FEXTC duration at ATU-C, or in the FEXTR duration at ATU-R.

#### C.8.2.2	Far-end test parameters (supplements 10.4.2)

The following far-end test parameters are defined:

•	Attenuation (ATN): The received signal power shall be measured only in the FEXTC duration at ATU-C, or only in the FEXTR duration at ATU-R.

•	Signal-to-Noise ratio (SNR) margin: During the FEXT Bitmap mode, this primitive represents the SNR margin in the FEXTC duration at ATU-C, or in the FEXTR duration at ATU-R.

## C.9	Initialization

### C.9.1	Initialization with Hyperframe (new)

The exchange of messages between ATU-C and ATU-R are performed using FEXTC and FEXTR symbols. The initialization sequence has two symbol rates. One is 4.3125 kbaud for the symbol without a cyclic prefix, and the other is 4  69/68 kbaud for the symbol with a cyclic prefix. 32 TTR cycles have the same period as 345 times 4.3125 kHz DMT symbols, and 34 TTR cycles have the same period as 345 times 4  69/68 kHz DMT symbols.

During FEXT Bitmap mode, the ATU-R shall not transmit any signal as the NEXTC symbols. For modems not using any of the profiles defined in C.4 and modems using Profile 1, the ATU-C shall only transmit the pilot tone as the NEXTR symbols except:

•	C-PILOT1 (C-PILOT1A): accompanied by a signal to allow the ATU-C to indicate the phase of TTRC to the ATU-R (see C.9.3.1);

•	C-QUIETn: not transmit any signal.

For Profile 3, the ATU-C shall not transmit any signal in NEXTR symbols.

For Profiles 2, 4, 5, and 6, the ATU-C may transmit data and pilot during the NEXTR symbols.

The ATU-C shall enter C-PILOT1 at the beginning of the hyperframe. The ATU-C transmits information regarding the phase of the TTRC to ATU-R during C-PILOT1. The ATU-R shall enter R-REVERB1 at the beginning of the hyperframe without cyclic prefix.

From C-PILOT1 to C-SEGUE1, the following numerical formula gives the information which duration Ndmt-th DMT symbol belongs to (see Figure C.11):

For (Ndmt = 0, 1, ..., 344)

S = 256 × Ndmt mod 2760

if { (S + 255 < a) or (S > a + b) }	then FEXTR symbol

else			then NEXTR symbol

where a = 1243, b = 1461

In order to enter C-RATES1 at the beginning of the hyperframe with cyclic prefix, the number of symbols from C-PILOT1 to C-SEGUE1 shall be a multiple of 345 DMT symbols.

From R-REVERB1 to R-SEGUE1, the following numerical formula gives the information which duration Ndmt-th DMT symbol belongs to (see Figure C.12):

For S = 256 × Ndmt mod 2760 (Ndmt = 0, 1, ..., 344)

if { (S > a) and (S + 255 < a + b) }	then FEXTC symbol

else 		then NEXTC symbol

where a = 1315, b = 1293

From C-RATES1 to C-SEGUE3, the number of symbols is a multiple of 345 DMT symbols. The following numerical formula gives the information which duration Ndmt-th DMT symbol belongs to. ATU-C transmits the message data in FEXTR symbols (see Figure C.9).

For (Ndmt = 0, 1, ..., 344)

S = 272 × Ndmt mod 2760

if { (S + 271 < a) or (S > a + b) }	then FEXTR symbol

else			then NEXTR symbol

where a = 1243, b = 1461

The ATU-R enters R-REVERB3 at the beginning of the hyperframe with cyclic prefix, which is extracted from received signal. From R-REVERB3 to R-SEGUE5 the number of symbols is a multiple of 345 DMT symbols. The following numerical formula gives the information which duration Ndmt-th DMT symbol belongs to. ATU-R transmits the message data in FEXTC symbols (see Figure C.10).

For (Ndmt = 0, 1, …, 344)

S = 272 × Ndmt mod 2760

if { (S > a) and (S + 271 < a + b) }	then FEXTC symbol

else			then NEXTC symbol

where a = 1315, b = 1293

Figure C.11/G.992.2 – Symbol pattern in a hyperframe without cyclic prefix – Downstream

Figure C.12/G.992.2 – Symbol pattern in a hyperframe without cyclic prefix – Upstream

### C.9.2	Handshake – ATU-C (supplements 11.2)

From C-SILENT1, the ATU-C may transition to either C-TONES or C-SYNC under instruction of the network operator.

#### C.9.2.1	CL messages (supplements 11.2)

Table C.4/G.992.1 – ATU-C CL message bit definitions for Annex C

| NPar(2) bit | Definition |
| --- | --- |
| DBM | If set to ZERO, this bit shall indicate Bitmap-NR and Bitmap-NC are enabled (Dual Bitmap mode) and are used to transmit data. If set to ONE, this bit shall indicate Bitmap-NR and Bitmap-NC are disabled (FEXT Bitmap mode), i.e. only Bitmap-FR and Bitmap-FC are used to transmit data by ATU-C and ATU-R respectively. This mode selection shall be only performed by the ATU-C. If it is set to ONE in a CL message, it must be set to ONE in subsequent MS messages from either the ATU-C or ATU-R (only applicable for G.992.2 Annex C) (Note). |
| Profile 1 | If set to ONE, this bit shall indicate that the ATU-C supports Profile 1. |
| Profile 2 | If set to ONE, this bit shall indicate that the ATU-C supports Profile 2. |
| Profile 3 | If set to ONE, this bit shall indicate that the ATU-C supports Profile 3. |
| Profile 4 | If set to ONE, this bit shall indicate that the ATU-C supports Profile 4. |
| Profile 5 | If set to ONE, this bit shall indicate that the ATU-C supports Profile 5. |
| Profile 6 | If set to ONE, this bit shall indicate that the ATU-C supports Profile 6. |
| SPar(2) bit | Definition |
| C-PILOT | If set to ONE, this bit shall indicate that the ATU-C supports negotiation of the optional pilot tones and TTR indication signals. This bit shall be set to ONE to indicate support for any of the profiles defined in C.4. |
| NPar(3) bit | Definition |
| nC-PILOT1=64 | If the C-PILOT bit is set to ONE, this bit shall also be set to ONE, indicating that the ATU-C supports transmission of pilot tone on subcarrier 64. |
| nC-PILOT1=48 | If the C-PILOT bit is set to ONE, this bit shall also be set to ONE, indicating that the ATU-C supports transmission of pilot tone on subcarrier 48. |
| nC-PILOT1=32 | If the C-PILOT bit is set to ONE, this bit shall also be set to ONE, indicating that the ATU-C supports transmission of pilot tone on subcarrier 32. |
| nC-PILOT1=16 | If the C-PILOT bit is set to ONE, this bit shall also be set to ONE, indicating that the ATU-C supports transmission of pilot tone on subcarrier 16. |
| A48 / B48 | If the C-PILOT bit is set to ONE, this bit shall also be set to ONE, indicating that the ATU-C supports transmission of TTR indication signal A48 / B48. |
| A24 / B24 | If the C-PILOT bit is set to ONE, this bit shall also be set to ONE, indicating that the ATU-C supports transmission of TTR indication signal A24 / B24. |
| C-REVERB33-63 | If the C-PILOT bit is set to ONE, this bit shall also be set to ONE, indicating that the ATU-C supports transmission of TTR indication signal C-REVERB33-63. |
| C-REVERB6-31 | If the C-PILOT bit is set to ONE, this bit shall also be set to ONE, indicating that the ATU-C supports transmission of TTR indication signal C-REVERB6-31. |
| NOTE – The DBM bit is only used to maintain backward compatibility with G.992.2 Annex C 1999. If any of the profile bits (Table 11.9.1/G.994.1) are set to ONE in a received CLR message, DBM shall be set to ONE in the CL message and shall be ignored by the ATU-R. |  |

#### C.9.2.2	MS messages (supplements 11.2)

Table C.5/G.992.1 – ATU-C MS message bit definitions for Annex C

| NPar(2) bit | Definition |
| --- | --- |
| DBM | If set to ZERO, this bit shall indicate Bitmap-NR and Bitmap-NC are enabled (Dual Bitmap mode) and are used to transmit data. If set to ONE, this bit shall indicate Bitmap-NR and Bitmap-NC are disabled (FEXT Bitmap mode), i.e. only Bitmap-FR and Bitmap-FC are used to transmit data by ATU-C and ATU-R respectively. This mode selection shall be only performed by ATU-C. This bit shall be set to ONE if it was set to ONE in a previous CL message (only applicable for G.992.2 Annex C) (Note 1). |
| Profile 1 | If set to ONE, this bit shall indicate that the ATU-C is selecting Profile 1. |
| Profile 2 | If set to ONE, this bit shall indicate that the ATU-C is selecting Profile 2. |
| Profile 3 | If set to ONE, this bit shall indicate that the ATU-C is selecting Profile 3. |
| Profile 4 | If set to ONE, this bit shall indicate that the ATU-C is selecting Profile 4. |
| Profile 5 | If set to ONE, this bit shall indicate that the ATU-C is selecting Profile 5. |
| Profile 6 | If set to ONE, this bit shall indicate that the ATU-C is selecting Profile 6. |
| SPar(2) bit | Definition |
| C-PILOT | If set to ONE, this bit shall indicate that the ATU-C wishes to select a pilot tone and TTR indication signal. This bit shall be set to ONE to select one of the profiles defined in C.4 |
| NPar(3) bit | Definition |
| nC-PILOT1=64 | If set to ONE, this bit shall indicate that the ATU-C is selecting the pilot tone on subcarrier 64 (Note 2). |
| nC-PILOT1=48 | If set to ONE, this bit shall indicate that the ATU-C is selecting the pilot tone on subcarrier 48 (Note 2). |
| nC-PILOT1=32 | If set to ONE, this bit shall indicate that the ATU-C is selecting the pilot tone on subcarrier 32 (Note 2). |
| nC-PILOT1=16 | If set to ONE, this bit shall indicate that the ATU-C is selecting the pilot tone on subcarrier 16 (Note 2). |
| A48 / B48 | If set to ONE, this bit shall indicate that the ATU-C is selecting TTR indication signal A48 / B48 (Note 2). |
| A24 / B24 | If set to ONE, this bit shall indicate that the ATU-C is selecting TTR indication signal A24 / B24 (Note 2). |
| C-REVERB33-63 | If set to ONE, this bit shall indicate that the ATU-C is selecting TTR indication signal C-REVERB33-63 (Note 2). |
| C-REVERB6-31 | If set to ONE, this bit shall indicate that the ATU-C is selecting TTR indication signal C-REVERB6-31 (Note 2). |
| NOTE 1 – The DBM bit is only used to maintain backward compatibility with G.992.2 Annex C 1999. NOTE 2 – One and only one pilot tone bit, and one and only one TTR indication signal bit shall be set in an MS message. |  |

### C.9.3	Handshake – ATU-R (supplements 11.3)

Upon command from the host controller, the ATU-R shall initiate handshaking by transitioning from the R-SILENT0 state to either the G.994.1 R-TONES-REQ state or the R-SYNC state.

#### C.9.3.1	CLR messages (supplements 11.3)

Table C.6/G.992.1 – ATU-R CLR message bit definitions for Annex C

| NPar(2) bit | Definition |
| --- | --- |
| DBM | This bit shall be set to ONE. |
| Profile 1 | If set to ONE, this bit shall indicate that the ATU-R supports Profile 1. |
| Profile 2 | If set to ONE, this bit shall indicate that the ATU-R supports Profile 2. |
| Profile 3 | If set to ONE, this bit shall indicate that the ATU-R supports Profile 3. |
| Profile 4 | If set to ONE, this bit shall indicate that the ATU-R supports Profile 4. |
| Profile 5 | If set to ONE, this bit shall indicate that the ATU-R supports Profile 5. |
| Profile 6 | If set to ONE, this bit shall indicate that the ATU-R supports Profile 6. |
| SPar(2) bit | Definition |
| C-PILOT | If set to ONE, this bit shall indicate that the ATU-R supports negotiation of the optional pilot tones and TTR indication signals. This bit shall be set to ONE to indicate support for any of the profiles defined in C.4. |
| NPar(3) bit | Definition |
| nC-PILOT1=64 | This bit shall be set to ONE, indicating that the ATU-R supports reception of pilot tone on subcarrier 64. |
| nC-PILOT1=48 | If set to ONE, this bit shall indicate that the ATU-R supports reception of pilot tone on subcarrier 48. |
| nC-PILOT1=32 | If set to ONE, this bit shall indicate that the ATU-R supports reception of pilot tone on subcarrier 32. |
| nC-PILOT1=16 | If set to ONE, this bit shall indicate that the ATU-R supports reception of pilot tone on subcarrier 16. |
| A48 / B48 | This bit shall be set to ONE, indicating that the ATU-R supports reception of either TTR indication signal A48 or B48 (Note). |
| A24 / B24 | If set to ONE, this bit shall indicate that the ATU-R supports reception of either TTR indication signal A24 or B24 (Note). |
| C-REVERB33-63 | If set to ONE, this bit shall indicate that the ATU-R supports reception of TTR indication signal C-REVERB33-63. |
| C-REVERB6-31 | If set to ONE, this bit shall indicate that the ATU-R supports reception of TTR indication signal C-REVERB6-31. |
| NOTE – A48 and A24 shall not be used for Profile 3. |  |

#### C.9.3.2	MS messages (supplements 11.3)

Table C.7/G.992.1 – ATU-R MS message NPar(2) bit definitions for Annex C

| NPar(2) bit | Definition |
| --- | --- |
| DBM | If set to ZERO, this bit shall indicate Bitmap-NR and Bitmap-NC are enabled (Dual Bitmap mode) and are used to transmit data. If set to ONE, this bit shall indicate Bitmap-NR and Bitmap-NC are disabled (FEXT Bitmap mode), i.e. only Bitmap-FR and Bitmap-FC are used to transmit data by ATU-C and ATU-R respectively. This mode selection shall be only performed by ATU-C. This bit shall be set to ONE if it was set to ONE in a previous CL message (only applicable for G.992.2 Annex C) (Note). |
| NOTE – The DBM bit is only used to maintain backward compatibility with G.992.2 Annex C 1999. |  |

#### C.9.3.3	MP messages (new)

Table C.8/G.992.1 – ATU-R MP message bit definitions for Annex C

| NPar(2) bit | Definition |
| --- | --- |
| DBM | This bit shall be set to ONE if it was set to ONE in a previous CL message (Note 1). |
| Profile 1 | If set to ONE, this bit shall indicate that the ATU-R is proposing to use Profile 1. |
| Profile 2 | If set to ONE, this bit shall indicate that the ATU-R is proposing to use Profile 2. |
| Profile 3 | If set to ONE, this bit shall indicate that the ATU-R is proposing to use Profile 3. |
| Profile 4 | If set to ONE, this bit shall indicate that the ATU-R is proposing to use Profile 4. |
| Profile 5 | If set to ONE, this bit shall indicate that the ATU-R is proposing to use Profile 5. |
| Profile 6 | If set to ONE, this bit shall indicate that the ATU-R is proposing to use Profile 6. |
| SPar(2) bit | Definition |
| C-PILOT | If set to ONE, this bit shall indicate that the ATU-R wishes to propose a pilot tone and TTR indication signal. This bit shall be set to ONE to propose one of the profiles defined in C.4. |
| NPar(3) bit | Definition |
| nC-PILOT1=64 | If set to ONE, this bit shall indicate that the ATU-R is proposing the use of pilot tone on subcarrier 64 (Note 2). |
| nC-PILOT1=48 | If set to ONE, this bit shall indicate that the ATU-R is proposing the use of pilot tone on subcarrier 48 (Note 2). |
| nC-PILOT1=32 | If set to ONE, this bit shall indicate that the ATU-R is proposing the use of pilot tone on subcarrier 32 (Note 2). |
| nC-PILOT1=16 | If set to ONE, this bit shall indicate that the ATU-R is proposing the use of pilot tone on subcarrier 16 (Note 2). |
| A48 / B48 | If set to ONE, this bit shall indicate that the ATU-R is proposing the use of TTR indication signal A48 / B48 (Note 2). |
| A24 / B24 | If set to ONE, this bit shall indicate that the ATU-R is proposing the use of TTR indication signal A24 / B24 (Note 2). |
| C-REVERB33-63 | If set to ONE, this bit shall indicate that the ATU-R is proposing the use of TTR indication signal C-REVERB33-63 (Note 2). |
| C-REVERB6-31 | If set to ONE, this bit shall indicate that the ATU-R is proposing the use of TTR indication signal C-REVERB6-31 (Note 2). |
| NOTE 1 – The DBM bit is only used to maintain backward compatibility with G.992.2 Annex C 1999. NOTE 2 – One and only one pilot tone bit, and one and only one TTR indication signal bit shall be set in an MP message. |  |

### C.9.4	Escape from Handshake to Fast Retrain (replaces 11.5)

See Figure C.13.

#### C.9.4.1	C-QUIET-EF1 (replaces 11.5.1)

C-QUIET-EF1 begins at the termination of G.994.1. The minimum duration of C-QUIET-EF1 is 128 symbols. The maximum duration of C-QUIET-EF1 is 2048 symbols. The ATU-C terminates C-QUIET-EF1 and enters C-RECOV at the beginning of the hyperframe.

#### C.9.4.2	R-QUIET-EF1 (replaces 11.5.2)

R-QUIET-EF1 begins at the termination of G.994.1. The minimum duration of R-QUIET-EF1 is 128 DMT symbols after the detection of C-RECOV. The ATU-R shall progress to R-RECOV only after it has detected any part of the C-RECOV signal that is needed for reliable detection. The ATU-R enters R-RECOV2 synchronized with the hyperframe after the synchronization of ADC clock with the received C-RECOV signal.

Time-outs C-TO2, R-TO1, R-TO3 are vendor discretionary. It is advisable to make these duration as short as possible.

Figure C.13/G.992.2 – Timing diagram of Escape to Fast Retrain

### C.9.5	Transceiver training – ATU-C (supplements 11.7)

During transceiver training from C-REVERB1 to C-SEGUE1 except C-PILOTn and C-QUIETn, the ATU-C shall transmit both FEXTR and NEXTR symbols when Bitmap-NR is enabled (Dual Bitmap mode). For modems not using any of the profiles defined in C.4 and modems using Profile 1, the ATU-C shall not transmit NEXTR symbols except the pilot tone when Bitmap-NR is disabled (FEXT Bitmap mode). For Profile 3, the ATU-C shall not transmit any signal in NEXTR symbols. The duration of each state is defined in Figure C.18.

#### C.9.5.1		C-PILOT1 (supplements 11.7.2)

The ATU-C shall start its NSWF counter immediately after entering C-PILOT1, and then increment the NSWF counter with modulo 345 from 0 when it transmits each DMT symbol. According to the sliding window function and this counter, the ATU-C decides to transmit all of the subsequent symbols in either FEXTR or NEXTR symbols (see Figures C.11 and C.9).

C-PILOT1 has two signals.

The first signal is the pilot tone as a single frequency sinusoid.

For Profiles 1 and 2, the frequency of the pilot tone shall be selected from one of the following choices:

1)	fC-PILOT1 = 276 kHz (nC-PILOT1 = 64);

2)	fC-PILOT1 = 207 kHz (nC-PILOT1 = 48).

For Profiles 3 to 6, the frequency of the pilot tone shall be selected from one of the following choices:

1)	fC-PILOT1 = 276 kHz (nC-PILOT1 = 64);

2)	fC-PILOT1 = 207 kHz (nC-PILOT1 = 48);

3)	fC-PILOT1 = 138 kHz (nC-PILOT1 = 32);

4)	fC-PILOT1 = 69 kHz (nC-PILOT1 = 16).

For modems not using any of the profiles defined in C.4, the frequency of the pilot tone shall be:

fC-PILOT1 = 276 kHz (nC-PILOT1 = 64)

Transmitters that use any of the profiles defined in C.4 shall support all of these pilot tones. For backwards compatibility, receivers shall support nC-PILOT1 = 64. Support of the other pilot tones by a receiver is optional. The pilot tone shall be selected during G.994.1.

The second signal is the TTR indication signal used to transmit NEXTR/FEXTR information. The ATU-R can detect the phase information of the TTRC from this signal.

For Profiles 1 and 2, the TTR indication signal shall be selected from one of the following choices:

1)	A48 signal – the constellation encoding of the 48-th carrier with 2-bit constellation as the follows:

(+ , +) to indicate a FEXTR symbol;

(+ , –) to indicate a NEXTR symbol.

2)	C-REVERB33-63 – subcarriers 33 through 63 of C-REVERB, transmitted only in the first four DMT symbols of each hyperframe in C-PILOT1 to indicate the beginning of the hyperframe.

For Profile 3, the TTR indication signal shall be selected from one of the following choices:

1)	B48 signal –the constellation encoding of the 48th carrier with 2-bit constellation as follows:

(+ , –) to indicate the first and the last symbol in consecutive FEXTR symbols;

(+ , +) to indicate the other symbols in consecutive FEXTR symbols.

2)	B24 signal – the constellation encoding of the 24th carrier with 2-bit constellation as follows:

(+ , –) to indicate the first and the last symbol in consecutive FEXTR symbols;

(+ , +) to indicate the other symbols in consecutive FEXTR symbols.

3)	C-REVERB6-31 – subcarriers 6 through 31 of C-REVERB, transmitted only in the first four DMT symbols of each hyperframe in C-PILOT1 to indicate the beginning of the hyperframe.

For Profiles 4 to 6, the TTR indication signal shall be selected from one of the following choices:

1)	A48 signal – the constellation encoding of the 48th carrier with 2-bit constellation as follows:

(+ , +) to indicate a FEXTR symbol;

(+ , –) to indicate a NEXTR symbol.

2)	A24 signal – the constellation encoding of the 24th carrier with 2-bit constellation as follows:

(+ , +) to indicate a FEXTR symbol;

(+ , –) to indicate a NEXTR symbol.

3)	C-REVERB6-31 – subcarriers 6 through 31 of C-REVERB, transmitted only in the first four DMT symbols of each hyperframe in C-PILOT1 to indicate the beginning of the hyperframe.

For modems not using any of the profiles defined in C.4, the TTR indication signal shall be:

A48 signal – the constellation encoding of the 48th carrier with 2-bit constellation as follows:

(+ , +) to indicate a FEXTR symbol;

(+ , –) to indicate a NEXTR symbol.

Transmitters that use any of the profiles defined in C.4 shall support all of these TTR indication signals. For backwards compatibility, receivers shall support TTR indication signal A48. Support for the other TTR indication signals by a receiver is optional. The TTR signal shall be selected during G.994.1.

#### C.9.5.2	C-PILOT1A (supplements 11.7.3)

C-PILOT1A has two signals and it is the same transmitted signal as C-PILOT1 (see C.9.3.1).

#### C.9.5.3	C-REVERB3 (supplements 11.7.11)

In order to synchronize the first symbol of C-RATES1 with the beginning of the hyperframe and to inform the entering timing of C-RATES1 to ATU-R, the first symbol of C-SEGUE1 shall be transmitted inside of the FEXTR duration. Therefore, the duration of C-REVERB3 is 3628 DMT symbols (see Figure C.14).

Figure C.14/G.992.2 – Timing diagram of C-SEGUE1 to C-RATES1

#### C.9.5.4		C-REVERB1 (supplements 11.7.5)

Bits d2i+1 and d2i+2, which modulate the pilot carrier that has tone index i, shall be overwritten by {0,0}, generating the (+,+) constellation point.

### C.9.6	Transceiver training – ATU-R (supplements 11.8)

During transceiver training from R-REVERB1 to R-SEGUE1 except R-QUIETn, the ATU-R shall transmit both FEXTC and NEXTC symbols when Bitmap-NC is enabled (Dual Bitmap mode) and shall not transmit NEXTC symbols when Bitmap-NC is disabled (FEXT Bitmap mode). The duration of each state is defined in Figure C.18.

#### C.9.6.1		R-QUIET2 (supplements 11.8.1)

The ATU-R enters R-REVERB1 after it completes timing recovery and Hyperframe synchronization from C-PILOT1/C-PILOT1A.

#### C.9.6.2		R-REVERB1 (supplements 11.8.2)

The ATU-R shall start its NSWF counter immediately after entering R-REVERB1, and then increment the NSWF counter modulo 345 from 0 when it transmits each DMT symbol. The ATU-C and ATU-R shall have the same value since hyperframe alignment between the ATU-C and ATU-R shall be maintained. According to the sliding window and this counter, the ATU-R decides to transmit all of the subsequent symbols in either the FEXTC or the NEXTC symbol.

#### C.9.6.3		R-QUIET3 (replaces 11.8.3)

The final symbol of R-QUIET3 accommodates the frame alignment of the transmitter to that of the receiver. It may be shortened by any number of samples. The maximum duration of R-QUIET3 is 6145 DMT symbols.

#### C.9.6.4		R-REVERB2 (supplements 11.8.5)

After ATU-R detects C-SEGUE1, the ATU-R enters R-SEGUE1. The maximum duration of R-REVERB2 is 3643 DMT symbols.

### C.9.7	Channel analysis (ATU-C) (supplements 11.9)

The ATU-C shall transmit the FEXTR symbols-- For modems not using any of the profiles defined in C.4 and modems using Profiles 1, 2, 4, 5 and 6, the ATU-C shall not transmit the NEXTR symbols except for the pilot tone. For Profile 3, the ATU-C shall not transmit any signal in NEXTR symbols. In C-MEDLEY, ATU-C shall transmit both FEXTR and NEXTR symbols, when Bitmap-NR is enabled (Dual Bitmap mode). For modems not using any of the profiles defined in C.4 and modems using Profile 1, the ATU-C shall not transmit NEXTR symbols except pilot tone, when Bitmap-NR is disabled (FEXT Bitmap mode). For Profile 3, the ATU-C shall not transmit any signal in NEXTR symbols. The duration of each state is defined in Figure C.18.

#### C.9.7.1	C-SEGUE1 (supplements 11.9.1)

The duration of C-SEGUE1 is 20 symbols in order that the first symbol of C-SEGUE1 shall be inside of the FEXTR duration (see Figure C.14).

#### C.9.7.2	C-MEDLEY (supplements 11.9.6)

The definition of C-MEDLEY is the same as in 11.9.6, except for the duration of the SNR estimation at ATU-R for the downstream. With the periodical noise of TCM- ISDN, the SNR also changes in the same cycle, as shown in Figure C.15. When Bitmap-NR is enabled, ATU-C transmits the signal in both of FEXTR and NEXTR symbols and ATU-R estimates two SNRs from the received NEXTR and FEXTR symbols, respectively, as defined in Figure C.16.

The following numerical formula gives the information that received Ndmt-th DMT symbol at ATU-R belongs to:

For (Ndmt = 0, 1, …, 344)

S = 272 × Ndmt mod 2760

if { (S + 271 < a) or (S > d) }	then symbol for estimation of FEXTR SNR

if { (S > b) and (S + 271 < c) }	then symbol for estimation of NEXTR SNR

where a = 1243, b = 1403, c = 2613, d = 2704

When Bitmap-NR is disabled (FEXT Bitmap mode), the ATU-C only transmits the signal in FEXTR symbols, and the ATU-R estimates the SNR from the received FEXTR symbols. For modems not using any of the profiles defined in C.4 and modems using Profile 1, the ATU-C shall transmit only the pilot tone as NEXTR symbol. For Profile 3, the ATU-C shall not transmit any signal in NEXTR symbols. The number of bits of NEXTR shall be no more than the number of bits of FEXTR.

NOTE – At the transmitter, the PRD sequence generator is either always updated or always stopped during NEXTR symbol when Bitmap-NR is disabled (FEXT Bitmap mode). The receiver should be able to support both modes of transmitter operation.

Figure C.15/G.992.2 – Estimation of periodic Signal-to-Noise Ratio

Figure C.16/G.992.2 – Symbol pattern in a hyperframe for S/N estimation – Downstream

### C.9.8	Channel analysis (ATU-R) (supplements 11.10)

From R-RATES1 to R-CRC2, the ATU-R shall transmit the FEXTC symbols and shall not transmit the NEXTC symbols. In R-SEGUE2 and R-MEDLEY, the ATU-R shall transmit both FEXTC and NEXTC symbols when Bitmap-NC is enabled (Dual Bitmap mode) and shall not transmit NEXTC symbols when Bitmap-NC is disabled (FEXT Bitmap mode). The duration of each state is defined in Figure C.18.

#### C.9.8.1		R-SEGUE1 (supplements 11.10.1)

The maximum duration of R-SEGUE1 is 14 symbols (see Figure C.14).

#### C.9.8.2		R-REVERB3 (supplements 11.10.2)

The ATU-R shall start R-REVERB3 aligned with the beginning of a Hyperframe.

#### C.9.8.3		R-SEGUE2 (supplements 11.10.3)

The duration of R-SEGUE2 is 13 symbols.

#### C.9.8.4		R-MEDLEY (supplements 11.10.8)

The definition of R-MEDLEY is the same as in 11.10.8, except for the duration of the SNR estimation at the ATU-C for the upstream channel. With the periodic noise of TCM- ISDN, the SNR also changes in the same cycle, as shown in Figure C.15. When Bitmap-NC is enabled, the ATU-R shall transmit the signal in both of FEXTC and NEXTC symbols, and ATU-C shall estimate two SNRs from the received NEXTC and FEXTC symbols, respectively, as defined in Figure C.17.

The following numerical formula gives the information that received Ndmt-th DMT symbol at ATU-C belongs to:

For Ndmt = 0, 1, …, 344

S = 272 × Ndmt mod 2760

if { (S > b) and (S + 271 < c) }	then symbol for estimation of FEXTC SNR

if { (S + 271< a) }	then symbol for estimation of NEXTC SNR

where a = 1148, b = 1315, c = 2608

When Bitmap-NC is disabled (FEXT Bitmap mode), the ATU-R only transmits the signal in FEXTC symbols, and the ATU-C estimates the SNR from the received FEXTC symbols. - The number of bits of NEXTC shall be no more than the number of bits of FEXTC.

NOTE – At the transmitter, the PRD sequence generator is either always updated or always stopped during NEXTR symbol when Bitmap-NR is disabled (FEXT Bitmap mode). The receiver should be able to support both modes of transmitter operation.

Figure C.17/G.992.2 – Symbol pattern in a hyperframe for S/N estimation – Upstream

### C.9.9	Exchange – ATU-C (supplements 11.11)

During C-RATESn, C-MSGn, C-B&G, and C-CRCn, the ATU-C shall transmit the FEXTR symbol. In the other signals, the ATU-C shall transmit both FEXTR and NEXTR symbols when Bitmap-NR is enabled (Dual Bitmap mode), and, for modems not using any of the profiles defined in C.4 and modems using Profile 1,  shall not transmit the NEXTR symbols except pilot tone when Bitmap-NR is disabled (FEXT Bitmap mode). For Profile 3, the ATU-C shall not transmit any signal in NEXTR symbols. The duration of each state is defined in Figure C.19.

Figure C.18/G.992.2 – Timing diagram of the initialization sequence (part 1)

Figure C.19/G.992.2 – Timing diagram of the initialization sequence (part 2)

Notes to Figures C.18 and C.19

NOTE 1 – The ATU-C shall transmit the FEXTR symbols. For modems not using any of the profiles defined in C.4 and modems using Profiles 1, 2, 4, 5 and 6, the ATU-C shall not transmit the NEXTR symbols except for the pilot tone. For Profile 3, the ATU-C shall not transmit any signal in NEXTR symbols.

NOTE 2 – The ATU-C shall transmit both FEXTR and NEXTR symbols, when Bitmap-NR is enabled (Dual Bitmap mode). For modems not using any of the profiles defined in C.4 and modems using Profile 1, the ATU-C shall not transmit the NEXTR symbols except pilot tone, when Bitmap-NR is disabled (FEXT Bitmap mode). For Profile 3, the ATU-C shall not transmit any signal in NEXTR symbols.

NOTE 3 – ATU-R shall transmit the FEXTC symbols, and shall not transmit the NEXTC symbols.

NOTE 4 – ATU-R shall transmit both FEXTC and NEXTC symbols, when Bitmap-NC is enabled (Dual Bitmap mode). ATU-R shall not transmit NEXTC symbols, when Bitmap-NC is disabled (FEXT Bitmap mode).

NOTE 5 – Except for Profile 3, for which the ATU-C shall only transmit FEXTR symbols, the ATU-C shall transmit both FEXTR and NEXTR symbols.

C.9.9.1		C-MSG2 (supplements to 11.11.9)

Two bits are encoded onto each of the subcarriers numbered n1C-MSG2 through (n1C-MSG2 + 3) using the 4-QAM constellation labelling given in 7.10.3 (for the synchronization symbol) and 11.7.5 (for C-REVERB1). The same two bits are also encoded in the same way onto a set of backup carriers, namely, subcarriers n2C-MSG2 through (n2C-MSG2 + 3). The least significant byte of the message is transmitted in the first symbol of C-MSG2, with the two least significant bits of each byte encoded onto carriers n1C-MSG2 and n2C-MSG2. In addition, the pilot, subcarrier nC-PILOT1, shall be modulated with (,). Following C-MSG2, the ATU-C shall enter signalling state C-CRC3.

For modems not using any of the profiles defined in C.4 and modems using Profiles 1, 2, 4, 5 or 6 :

n1C-MSG2 = 43

n2C-MSG2 = 91

For Profile 3:

n1C-MSG2 = 13

n2C-MSG2 = 25

C.9.9.1.1	Total number of bits per symbol supported (supplements 11.11.9.4)

The maximum number of bits per symbol is defined at the reference point B, that is calculated from the FEXTC and NEXTC upstream channel performance. (E.g. if the maximum numbers of bits that can be supported in FEXTC and NEXTC symbols are 111 and 88, {Total number of bits per symbol supported} = (111 × 126 + 88 × 214)/340 = 96).

The number of symbols per hyperframe is 340. The number of FEXT symbols per hyperframe is 126. The number of NEXT symbols per hyperframe is 214.

#### C.9.9.2		C-B&G (replaces 11.11.13)

C-B&G shall be used to transmit to the ATU-R the bits and gains information, Bitmap-FC {b1, g1, b2, g2, ... b31, g31}, and Bitmap-NC {b33, g33, b34, g34, ... b63, g63}, that are to be used on the upstream carriers. bi of Bitmap-FC indicates the number of bits to be coded by ATU-R transmitter onto the i-th upstream carrier in FEXTC symbols; gi of Bitmap-FC indicates the scale factor, relative to the gain that was used for that carrier during the transmission of R-MEDLEY, that shall be applied to the i-th upstream carrier in FEXTC symbols. Similarly, bi of Bitmap-NC indicates the number of bits onto the (i – 32)-th upstream carrier in NEXTC symbols; gi of Bitmap-NC indicates the scale factor that shall be applied to the (i – 32)-th upstream carrier in NEXTC symbols.

Because no bits or energy will be transmitted at dc or one-half the sampling rate, b0, g0, b32, g32, b64, and g64 are all presumed to be zero and shall not be transmitted.

The C-B&G information shall be mapped in a 992-bit (124 byte) message m defined by:

m = {m991, m990, …, m1, m0} = {g63, b63, … g33, b33, g31, b31, … g1, b1},

with the MSB of bi and gi in the higher m index and m0 being transmitted first. The message m shall be transmitted in 124 symbols, using the transmission method as described in 11.11.9.

When Bitmap-NC is disabled (FEXT Bitmap mode), bi and gi of Bitmap-NC shall be set to zero.

#### C.9.9.3		C-SEGUE3 (supplements 11.11.16)

The duration of C-SEGUE3 is 18 symbols. Following C-SEGUE-3, ATU-C completes the initialization and enters C-SHOWTIME. In C-SHOWTIME, ATU-C shall transmit the signal using Bitmap-FR and Bitmap-NR with the sliding window.

When Bitmap-NR is disabled (FEXT Bitmap mode), for modems not using any of the profiles defined in C.4 and modems using Profile 1, the ATU-C shall transmit only the pilot tone as NEXTR symbols. For Profile 3, the ATU-C shall not transmit any signal in NEXTR symbols.

### C.9.10	Exchange – ATU-R (supplements 11.12)

The ATU-R shall transmit only the FEXTC symbols in R-MSGn, R-RATESn, R-B&G, R-CRCn. In other signals, the ATU-R shall transmit both FEXTC and NEXTC symbols when Bitmap-NC is enabled (Dual Bitmap mode) and shall not transmit NEXTC symbols when Bitmap-NC is disabled (FEXT Bitmap mode).The duration of each state is defined in Figure C.19.

#### C.9.10.1	R-MSG-RA (related to 11.12.2)

##### C.9.10.1.1	Total number of bits supported (Bmax) (replaces 11.12.2.7)

This parameter shall be defined as in R-MSG2; see C.9.10.2.

#### C.9.10.2	R-MSG2 (supplements 11.12.8)

##### C.9.10.2.1	Total number of bits per symbol supported (supplements 11.12.8.4)

The maximum number of bits per symbol is defined at the reference point B, that is calculated from the FEXTR and NEXTR downstream channel performance.

#### C.9.10.3	R-B&G (replaces 11.12.14)

The purpose of R-B&G is to transmit to ATU-C the bits and gains information, Bitmap-FR {b1, g1, b2, g2, ... b255, g255}, and Bitmap-NR {b257, g257, b258, g258, ... b511, g511}, to be used on the downstream subcarriers. bi of Bitmap-FR indicates the number of bits to be coded by ATU-C transmitter onto the i-th downstream subcarrier in FEXTR symbols; gi of Bitmap-FR indicates the scale factor that shall be applied to the i-th downstream subcarrier in FEXTR symbols, relative to the gain that was used for that carrier during the transmission of C-MEDLEY. Similarly, bi of Bitmap-NR indicates the number of bits onto the (i – 256)-th downstream carrier in NEXTR symbols; gi of Bitmap-NR indicates the scale factor that shall be applied to the (i – 256)-th downstream carrier in NEXTR symbols. Because no bits or energy will be transmitted at DC or one-half the sampling rate, b0, g0, b256, g256, b512, and g512 are all presumed to be zero, and are not transmitted. When subcarrier 64 is reserved as the pilot tone, b64 and b320 shall be set to 0b, and, for modems not using any of the profiles defined in C.4 and modems using Profiles 1, 2, 4, 5 or 6, g64 and g320 shall be set to gsync. For Profile 3, g64 shall be set to gsync and g320 shall be set to 0. When subcarrier 48 is reserved as the pilot tone, b48 and b304, shall be set to 0, and, for modems not using any of the profiles defined in C.4 and modems using Profiles 1, 2, 4, 5 or 6, g48 and g304 shall be set to gsync. For Profile 3, g48 shall be set to gsync and g304 shall be set to 0. When subcarrier 32 is reserved as the pilot tone, b32 and b288, shall be set to 0, and, for modems not using any of the profiles defined in C.4 and modems using Profiles 1, 2, 4, 5 or 6, g32 and g288 shall be set to gsync. For Profile 3, g32 shall be set to gsync and g288 shall be set to 0. When subcarrier 16 is reserved as the pilot tone, b16 and b272, shall be set to 0, and, for modems not using any of the profiles defined in C.4 and modems using Profiles 1, 2, 4, 5 or 6, g16 and g272 shall be set to gsync. For Profile 3, g16 shall be set to gsync and g272 shall be set to 0. The value gsync represents the gain scaling applied to the sync symbol.

The R-B&G information shall be mapped in a 8160-bit (1020 byte) message m defined by:

m = {m8159, m8158, …, m1, m0} = {g511, b511, … g257, b257, g255, b255, … g1, b1},

with the MSB of bi and gi in the higher m index and m0 being transmitted first. The message m shall be transmitted in 1020 symbols, using the transmission method as described in 11.12.8.

Values of bi and gi shall be set to 0 for values of 127 < I < 256, and 383 < i < 512.

When Bitmap-NR is disabled (FEXT Bitmap mode), bi and gi of Bitmap-NR shall be set to zero.

#### C.9.10.4	R-SEGUE5 (replaces 11.12.17)

The duration of R-SEGUE5 is 13 symbols. Following R-SEGUE-5, ATU-R completes the initialization and enters R-SHOWTIME. In R-SHOWTIME, ATU-R shall transmit the signal using Bitmap-FC and Bitmap-NC with the sliding window.

## C.10	Fast retraining (supplements clause 12)

The definition of Fast Retrain procedure is the same as main body except for the definition of C-RECOV, the introduction of R-RECOV2, and duration of symbols which includes the Sliding Window operation. The definition of C-RECOV and R-RECOV2 are defined in C.10.2. The duration of each state is defined in Figure C.22.

### C.10.1		Fast Retrain overview (pertains to 12.1)

For modems using any of the profiles defined in C.4, the fast retrain procedure shall not be used. The use of fast retrain for these profiles is for further study.

#### C.10.1.1	Profile requirement (supplements 12.1.1)

B&G tables of profile contain both Bitmap-FR and Bitmap-NR in the ATU-C, and both Bitmap-FC and Bitmap-NC in the ATU-R.

### C.10.2		Definition of Fast Retrain signals (supplements 12.2)

C-RECOV signal is changed and the signal R-RECOV2 is added.

•	C-RECOV consists of a single tone signal corresponding to subcarrier 68 without cyclic prefix accompanied by the C-PILOT1 signal which contains subcarrier 64 and 48 (see C.9.5.1). ATU-C shall transmit this signal which includes 64, 68 and 48th subcarrier as FEXTR symbol, and shall transmit the signal which includes 64 and 48th subcarrier as NEXTR symbol. This signal allows the ATU-R to perform or maintain timing recovery and hyperframe synchronization.

•	R-RECOV2 is single tone signal corresponding to subcarrier 22 without cyclic prefix. The PSD level to be used for R-RECOV2 shall be the same as R-RECOV. ATU-R transmits R-RECOV2 both FEXTC and NEXTC symbols.

### C.10.3		Fast Retrain procedure (supplements 12.3)

#### C.10.3.1	ATU-C initiated from SHOWTIME (supplements 12.3.1)

Figures C.20 and C.21 show the timing diagrams for the Fast Retrain with the ATU-C initiating the procedure.

Time-outs C-TO2, C-TO3, R-TO1, R-TO3 are vendor discretionary. It is advisable to make these durations as short as possible.

The following issues are different from main body and transition state timing is similar to initialization (see C.9).

•	The ATU-C shall enter C-RECOV at the beginning of the hyperframe without cyclic prefix.

•	The ATU-C shall transmit the pilot tone as the NEXTR symbols in C-MSG-n and C-CRC-n.

•	When Bitmap-NR is disabled (FEXT Bitmap mode) in preceding showtime, the ATU-C shall transmit the pilot tone as NEXTR symbols, except C-RECOV and C-QUIET-FR.

•	The ATU-C enters C-MEDLEY-FR at the beginning of the hyperframe with cyclic prefix.

•	The ATU-R shall enter R-RECOV2 at the beginning of the hyperframe without cyclic prefix after the synchronization of ADC clock with the received C-RECOV signal. Last symbol of R-RECOV may be shortened.

•	The ATU-R shall not transmit any signal as the NEXTC symbols in R-MSG-n and R-CRC-n.

•	When Bitmap-NC is disabled (FEXT Bitmap mode) in preceding showtime, the ATU-R shall not transmit the NEXTC symbols, except R-RECOV and R-RECOV2.

•	The ATU-R enters R-MEDLEY-FR at the beginning of the hyperframe with cyclic prefix.

#### C.10.3.2	ATU-R initiated from SHOWTIME (supplements 12.3.2)

Figure C.22 shows the timing diagram for the Fast Retrain with the ATU-R initiating the procedure.

Time-outs C-TO2, R-TO2, R-TO3 are vendor discretionary. It is advisable to make these durations as short as possible.

Figure C.20/G.992.2 – Timing diagram of the Fast Retrain procedure, ATU-C initiated from SHOWTIME (part 1)

Figure C.21/G.992.2 – Timing diagram of the Fast Retrain procedure, ATU-C initiated from SHOWTIME (part 2)

Figure C.22/G.992.2 – Timing diagram of the Fast Retraining procedure, 
ATU-R initiated from SHOWTIME

Notes to Figures C.21 and C.22

NOTE 1 – ATU-C shall transmit the FEXTR symbols, and shall not transmit the NEXTR symbols except pilot tone.

NOTE 2 – The ATU-C shall transmit both FEXTR and NEXTR symbols, when Bitmap-NR is enabled (Dual Bitmap mode). ATU-C shall not transmit the NEXTR symbols except pilot tone, when Bitmap-NR is disabled (FEXT Bitmap mode).

NOTE 3 – ATU-R shall transmit the FEXTC symbols, and shall not transmit the NEXTC symbols.

NOTE 4 – ATU-R shall transmit both FEXTC and NEXTC symbols, when Bitmap-NC is enabled (Dual bitmap mode). ATU-R shall not transmit NEXTC symbols, when Bitmap-NC is disabled (FEXT Bitmap mode).

NOTE 5 – ATU-C shall transmit both FEXTR and NEXTR symbols. However, the transmission signal is different between FEXTR and NEXTR symbol (see C.10.2).

NOTE 6 – ATU-R shall transmit both FEXTC and NEXTC symbols.

### C.10.4	Initiated from L3 or ITU-T Rec. G.994.1 (replaces 12.5)

A Fast Retrain Procedure initiated from an idle link state or via an Escape from Handshake shall be according to C.10.3.1 or C.10.3.2 except for a longer time duration for R-TO1 or R-TO2 of the R-RECOV signal. This will allow for re-acquisition of loop timing and hyperframe synchronization with the received C-RECOV signal at the ATU-R.

## C.11	Power management (pertains to clause 13)

### C.11.1	Transition from L0 to L1 (T0d) (pertains to 13.4.2)

#### C.11.1.1	Exchange entry procedure (replaces 13.4.2.1)

This clause defines the Exchange entry procedure. This procedure reuses states, signals, and rules for determining the next state contained within C.9.6 as defined in the following steps.

1)	After successful termination of the eoc handshake procedure, the ATU-R shall start R-QUIET-PM at the Hyperframe boundary. R-QUIET-PM is defined as no transmitted signal onto the U-R interface. The ATU-R shall maintain loop timing and Hyperframe synchronization during the R-QUIET-PM signal.

2)	After the detection of R-QUIET-PM, the ATU-C shall start C-REVERB4 at next 287th symbol (frame 286) of the hyperframe. In this procedure, the duration of C-REVERB4 is 345n – 304 (3  n  9).

3)	After the detection of C-REVERB4, the ATU-R responds by sending R-REVERB4 at next 37th symbol (frame 36) of the hyperframe. The exchange portion of the initialization procedure defined in C.9.6 shall continue from this point (i.e. the ATU-R shall send R-SEGUE after 296 symbols of R-REVERB4). During R-MSG-RA, the ATU-R shall use the "no options selected" message. The ATU-C implementation shall include a means that blocks the loss of signal defect during exchange entry procedure.

Appendix IV

Example Overlapped PSD Masks for use in a TCM-ISDN crosstalk environment

(For further study. See Note in C.1, Scope.)

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |