|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.992.2 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | Amendment 2 (10/2003) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital sections and digital line system – Access networks |  |  |  |
|  | Splitterless asymmetric digital subscriber line (ADSL) transceivers Amendment 2: New Appendix IV – Example overlapped PSD masks for use in a TCM-ISDN crosstalk environment |  |  |  |
|  | ITU-T Recommendation G.992.2 (1999) – Amendment 2 |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| General | G.900–G.909 |
| Parameters for optical fibre cable systems | G.910–G.919 |
| Digital sections at hierarchical bit rates based on a bit rate of 2048 kbit/s | G.920–G.929 |
| Digital line transmission systems on cable at non-hierarchical bit rates | G.930–G.939 |
| Digital line systems provided by FDM transmission bearers | G.940–G.949 |
| Digital line systems | G.950–G.959 |
| Digital section and digital transmission systems for customer access to ISDN | G.960–G.969 |
| Optical fibre submarine cable systems | G.970–G.979 |
| Optical line systems for local and access networks | G.980–G.989 |
| Access networks | G.990–G.999 |
| QUALITY OF SERVICE AND PERFORMANCE - GENERIC AND USER-RELATED ASPECTS | G.1000–G.1999 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.6000–G.6999 |
| DIGITAL TERMINAL EQUIPMENTS | G.7000–G.7999 |
| DIGITAL NETWORKS | G.8000–G.8999 |
| General aspects | G.8000–G.8099 |
| Design objectives for digital networks | G.8100–G.8199 |
| Quality and availability targets | G.8200–G.8299 |
| Network capabilities and functions | G.8300–G.8399 |
| SDH network characteristics | G.8400–G.8499 |
| Management of transport network | G.8500–G.8599 |
| SDH radio and satellite systems integration | G.8600–G.8699 |
| Optical transport networks | G.8700–G.8799 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.992.2 Splitterless asymmetric digital subscriber line (ADSL) transceivers Amendment 2 New Appendix IV Example overlapped PSD masks for use in a TCM-ISDN crosstalk environment

Summary This amendment contains Appendix IV, titled "Example overlapped PSD masks for use in a TCM-ISDN crosstalk environment" to ITU-T Rec. G.992.2, giving an example of overlapped PSD masks for use in a TCM-ISDN crosstalk environment.

Source Amendment 2 to ITU-T Recommendation G.992.2 (1999) was agreed by ITU-T Study Group 15 (2001-2004) on 31 October 2003.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g. interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met.  The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2004

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

ITU-T Recommendation G.992.2

Splitterless asymmetric digital subscriber line (ADSL) transceivers

Amendment 2

New Appendix IV

Example overlapped PSD masks for use in
a TCM-ISDN crosstalk environment

This appendix defines example-shaped overlapped downstream PSD masks for use in a TCM-ISDN crosstalk environment. These masks may be used with Annex C modes of operation that use overlapped PSDs.

## IV.1	Example downstream PSD masks  for use with Profiles 5 and 6

In this clause, two example downstream PSD masks are described. They may be used for downstream dual bitmap modes with overlapped spectrum. In general, using overlapped spectrum downstream may result in NEXT to the upstream channel. To meet spectrum compatibility requirements, the frequency components overlapping the upstream channel are shaped to reduce the crosstalk. The first example is a spectrally shaped mask used during the NEXT phase of the TTR clock. The second PSD mask has an alternative spectral shaping and is designed for use during the FEXT phase of the TTR clock.

### IV.1.1	Downstream shaped overlapped PSD mask for use during NEXT periods

The shaped overlapped spectral mask for use during NEXT periods of the TTR clock is defined in Table IV.1 and shown plotted in Figure IV.1. Spectral shaping is provided in the frequency band overlapping the ADSL upstream channel. Adherence to this mask will result in spectral compatibility with other systems deployed in an access network in a TCM-ISDN crosstalk environment.

Note that the definitions given in Table IV.1 and Figure IV.1 are those of a PSD mask. The corresponding PSD template is 3.5 dB below the mask at all frequencies.

Table IV.1/G.992.2 – Tabulation of a shaped overlapped downstream PSD
mask for use during NEXT periods of the TTR clock

| Frequency f (kHz) | PSD (dBm/Hz) Peak values |
| --- | --- |
| 0 < f < 4 | –97.5, with 15 dBrn power in 0-4 kHz window |
| 4 < f < 32 | –94.5 |
| 32 < f < 109 | –94.5 + 20.65 × log2 (f /32) |
| 109 < f < 138 | –58 + 58 × log2 (f /109) |
| 138 < f < 200 | –38.3 + 3.36 × log2 (f /138) |
| 200 < f < 552 | –36.5 |
| 552 < f < 956 | –36.5 – 36 × log2 (f /552) |
| 956 < f < 1800 | –65 |
| 1800 < f < 2290 | –65 – 72 × log2 (f /1800) |
| 2290 < f < 3093 | –90 |
| 3093 < f < 4545 | –90, peak with max power in the [f, f  1 MHz] window of (–36.5 – 36 × log2 (f /1104)  60) dBm |
| 4545 < f < 11040 | –90 peak, with max power in the [f, f  1 MHz] window of –50 dBm |

Figure IV.1/G.992.2 – A shaped overlapped downstream PSD mask
for use during NEXT periods of the TTR clock

### IV.1.2	Downstream shaped overlapped PSD mask for use during FEXT periods

The shaped overlapped spectral mask for use during FEXT periods of the TTR clock is defined in Table IV.2 and shown plotted in Figure IV.2. Spectral shaping is provided in the frequency band overlapping the ADSL upstream channel. Adherence to this mask will result in spectral compatibility with other systems deployed in an access network in a TCM-ISDN crosstalk environment.

Note that the definitions given in Table IV.2 and Figure IV.2 are those of a PSD mask. The corresponding PSD template is 3.5 dB below the mask at all frequencies.

Table IV.2/G.992.2 – Tabulation of a shaped overlapped downstream PSD mask
for use during FEXT periods of the TTR clock

| Frequency f (kHz) | PSD (dBm/Hz) Peak values |
| --- | --- |
| 0 < f < 4 | –97.5, with 15 dBrn power in 0-4 kHz window |
| 4 < f < 4.8 | –94.5 |
| 4.8 < f < 50 | –94.5 + 11.0 × log2 (f/4.8) |
| 50 < f < 126 | –57.5 + 15.7 × log2 (f/50) |
| 126 < f < 552 | –36.5 |
| 552 < f < 956 | –36.5 – 36 × log2 (f/552) |
| 956 < f < 1800 | –65 |
| 1800 < f < 2290 | –65 – 72 × log2 (f/1800) |
| 2290 < f < 3093 | –90 |
| 3093 < f < 4545 | –90 peak, with max power in the [f, f  1 MHz] window of (–36.5 – 36 × log2 (f/1104)  60) dBm |
| 4545 < f < 11040 | –90 peak, with max power in the [f, f  1 MHz] window of –50 dBm |

Figure IV.2/G.992.2 – A shaped overlapped downstream PSD mask
for use during FEXT periods of the TTR clock

## IV.2	Example downstream PSD mask for use with Profile 3

An example shaped overlapped spectral mask for use with Profile 3 is defined in Table IV.3 and shown in Figure IV.3. Spectral shaping is provided in the frequency band overlapping the ADSL upstream channel. Adherence to this mask will result in spectral compatibility with other systems deployed in an access network in a TCM-ISDN crosstalk environment.

Note that the definitions given in Table IV.3 and Figure IV.3 are those of a PSD mask. The corresponding PSD template is 3.5 dB below the mask at all frequencies.

Table IV.3/G.992.2 – Tabulation of a shaped downstream PSD mask for Profile 3

| Frequency f (kHz) | PSD (dBm/Hz) Peak values |
| --- | --- |
| 0 < f < 4 | –97.5, with 15 dBrn power in 0-4 kHz window |
| 4 < f < 5 | –92.5 + 18.64 × log2 (f/4) |
| 5 < f < 5.25 | –86.5 |
| 5.25 < f < 16 | –86.5 + 15.25 × log2 (f/5.25) |
| 16 < f < 32 | –62 + 25.5 × log2 (f/16) |
| 32 < f < 552 | –36.5 |
| 552 < f < 956 | –36.5 – 36 × log2 (f/552) |
| 956 < f < 1800 | –65 |
| 1800 < f < 2290 | –65 – 72 × log2 (f/1800) |
| 2290 < f < 3093 | –90 |
| 3093 < f < 4545 | –90 peak, with max power in the [f, f  1 MHz] window of (–36.5 – 36  log2 (f/1104)  60) dBm |
| 4545 < f < 11040 | –90 peak, with max power in the [f, f  1 MHz] window of –50 dBm |

Figure IV.3/G.992.2 – A shaped downstream PSD mask for Profile 3

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure, Internet protocol aspects and Next Generation Networks |
| Series Z | Languages and general software aspects for telecommunication systems |