|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.992.5 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (01/2005) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital sections and digital line system – Access networks |  |  |  |
|  | Asymmetric Digital Subscriber Line (ADSL) transceivers – Extended bandwidth ADSL2 (ADSL2plus) |  |  |  |
|  | CAUTION ! PREPUBLISHED RECOMMENDATION This prepublication is an unedited version of a recently approved Recommendation. It will be replaced by the published version after editing. Therefore, there will be differences between this prepublication and the published version. |  |  |  |

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g. interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met.  The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU [had/had not] received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2005

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

ITU-T Recommendation G.992.5 Asymmetric Digital Subscriber Line (ADSL) transceivers – Extended bandwidth ADSL2 (ADSL2plus)

Summary This Recommendation describes Asymmetrical Digital Subscriber Line (ADSL) transceivers on a metallic twisted pair that allows high-speed data transmission between the network operator end (ATU-C) and the customer end (ATU-R), using extended bandwidth. This Recommendation defines a variety of frame bearers in conjunction with one of two other services or without underlying service, dependent on the environment: - ADSL transmission simultaneously on the same pair with voiceband service; - ADSL transmission simultaneously on the same pair with ISDN (see Appendix I or II/G.961 [1]) services; - ADSL transmission without underlying service, optimized for deployment with ADSL over voiceband service in the same binder cable; - ADSL transmission without underlying service, optimized for deployment with ADSL over ISDN service in the same binder cable; - ADSL transmission with extended upstream bandwidth, simultaneously on the same pair with voice band service. ADSL transmission on the same pair with voiceband services and operating in an environment with TCM-ISDN (see Appendix III/G.961 [1]) services in an adjacent pair, is for further study. This Recommendation specifies the physical layer characteristics of the extended bandwidth Asymmetrical Digital Subscriber Line (ADSL) interface to metallic loops. As compared to the ADSL2 transceiver defined in ITU-T Rec. G.992.3 [5], the operating modes use double the downstream bandwidth. When operating on the same pair with voice band services, an additional operating mode is defined, also using the double upstream bandwidth. This Recommendation has been written to help ensure the proper interfacing and interworking of ADSL transmission units at the customer end (ATU-R) and at the network operator end (ATU-C) and also to define the transport capability of the units. Proper operation shall be ensured when these two units are manufactured and provided independently. A single twisted pair of telephone wires is used to connect the ATU-C to the ATU-R. The ADSL transmission units must deal with a variety of wire pair characteristics and typical impairments (e.g., crosstalk and noise). An extended bandwidth ADSL transmission unit (ADSL2plus transceiver) can simultaneously convey all of the following: a number of downstream frame bearers, a number of upstream frame bearers, a baseband POTS/ISDN duplex channel, and ADSL line overhead for framing, error control, operations, and maintenance. Systems support a net data rate ranging up to a minimum of 16 Mbit/s downstream and 800 kbit/s upstream. Support of net data rates above 16 Mbit/s downstream and support of net data rates above 800 kbit/s upstream are optional.

This Recommendation includes mandatory requirements, recommendations and options; these are designated by the words shall", "should" and "may" respectively. The word "will" is used only to designate events that take place under some defined set of circumstances. This Recommendation is written as a delta Recommendation relative to ITU-T Rec. G.992.3. For the clauses which have been changed, this Recommendation contains complete replacement text (unless explicitly indicated). For the clauses which have not been changed, this Recommendation contains only the clause heading, with reference to ITU-T Rec. G.992.3. This Recommendation defines several optional capabilities and features: – transport of STM and/or ATM and/or packets; – transport of a network timing reference; – multiple latency paths; – multiple frame bearers; – short initialization procedure; – dynamic rate repartitioning; – seamless rate adaptation. It is the intention of this Recommendation to provide, by negotiation during initialization, for U-interface compatibility and interoperability between transceivers complying with this Recommendation and between transceivers that include different combinations of options. History This Recommendation describes extended bandwidth ADSL2 (ADSL2plus) transceivers, as a delta to the second generation ADSL (ADSL2 – ITU-T Rec. G.992.3). This Recommendation has been written to provide additional features, relative to ITU-T Rec. G.992.3. ITU-T Rec. G.992.3 was approved on the 29 of July 2002. Several potential improvements have been identified to better address higher data rates for shorter loops and longer reach for high data rates. This Recommendation provides a new ADSL U-interface specification, including the identified improvements, which the ITU-T believes will be most helpful to the ADSL industry. Relative to ITU-T Rec. G.992.3, the following application-related feature has been added: – Improved support for services requiring high downstream data rates (e.g., broadband entertainment services). Relative to ITU-T Rec. G.992.3, the following PMS-TC-related feature has been added: – Support of up to 3 Reed-Solomon codewords per symbol; Relative to ITU-T Rec. G.992.3, the following PMD-related features have been added: – Extended downstream bandwidth to 2.208 MHz (512 subcarriers) for all operation modes (POTS/ISDN/All Digital Mode); – Downstream spectrum control with individual maximum transmit PSD at U-C reference point per subcarrier, under operator control through CO-MIB, allows configuration per the regional requirements (e.g., North America, Europe or Japan) and deployment scenarios (e.g., CO or Remote). – Downstream spectrum shaping during Showtime (shaped transmit PSD in the passband, i.e., non-flat) for improved downstream transmit PSD flexibility. Through negotiation during initialization, the capability of equipment to support the G.992.5 and other G.992.x ADSL Recommendations is identified. For reasons of interoperability, equipment may choose to support multiple Recommendations, such that it is able to adapt to the operating mode supported by the far-end equipment.

ITU-T Recommendation G.992.5

Asymmetric Digital Subscriber Line (ADSL) transceivers –
Extended bandwidth ADSL2 (ADSL2plus)

# 1	Scope

For interrelationships of this Recommendation with other G.99x-series Recommendations, see Appendix VI – Bibliography [B1].

This Recommendation describes the interface between the telecommunications network and the customer installation in terms of their interaction and electrical characteristics. The requirements of this Recommendation apply to a single asymmetric digital subscriber line (ADSL).

ADSL provides a variety of frame bearers in conjunction with other services:

- ADSL service on the same pair with voiceband services (including POTS and voiceband data services). The ADSL service occupies a frequency band above the voiceband service, and is separated from it by filtering.

- ADSL service on the same pair as ISDN service, as defined in Appendices I and II/G.961 [1]. The ADSL service occupies a frequency band above the ISDN service, and is separated from it by filtering.

- ADSL service with extended upstream bandwidth, on the same pair with voiceband services (including POTS and voiceband data services). The ADSL service occupies a frequency band above the voiceband service, and is separated from it by filtering.

ADSL also provides a variety of frame bearers without baseband services (i.e., POTS or ISDN) being present on the same pair.

- ADSL service on a pair, with improved spectral compatibility with ADSL over POTS present on an adjacent pair.

- ADSL service on a pair, with improved spectral compatibility with ADSL over ISDN present on an adjacent pair.

In the direction from the network operator to the customer premises (i.e., the downstream direction), the frame bearers provided may include low-speed frame bearers and high-speed frame bearers; in the other direction from the customer premises to the central office (i.e., the upstream direction), only low-speed frame bearers are provided.

The transmission system is designed to operate on two-wire twisted metallic copper pairs with mixed gauges. This Recommendation is based on the use of copper pairs without loading coils, but bridged taps are acceptable in all but a few unusual situations.

Operation on the same pair with voiceband services (e.g., POTS and voiceband data services), and with TCM-ISDN service as defined in Appendix III/G.961 [1] on an adjacent pair, is for further study.

An overview of digital subscriber line transceivers can be found in Appendix VI – Bibliography [B1].

Specifically, this Recommendation:

–	defines the Transmission Protocol Specific Transmission Convergence Sub-layer for ATM, STM and Packet transport through the frame bearers provided;

–	defines the combined options and ranges of the frame bearers provided;

–	defines the line code and the spectral composition of the signals transmitted by both ATU-C and ATU-R;

–	defines the initialization procedure for both the ATU-C and the ATU-R;

–	specifies the transmit signals at both the ATU-C and ATU-R;

–	describes the organization of transmitted and received data into frames;

–	defines the functions of the OAM channel;

In separate annexes it also:

- describes the transmission technique used to support the simultaneous transport of voiceband services and frame bearers (ADSL over POTS, Annex A) on a single twisted-pair;

- describes the transmission technique used to support the simultaneous transport of ISDN services as defined in Appendices I and II/G.961 [1], and frame bearers (ADSL over ISDN, Annex B) on a single twisted-pair;

- describes the transmission technique used to support the transport of only frame bearers on a pair, with improved spectral compatibility with ADSL over POTS present on adjacent pair (All Digital Mode, Annex I).

- describes the transmission technique used to support the transport of only frame bearers on a pair, with improved spectral compatibility with ADSL over ISDN present on adjacent pair (All Digital Mode, Annex J).

- describes the transmission technique used to support the simultaneous transport of voice band services and frame bearers for extended upstream bandwidth operation (EUADSL2plus over POTS, Annex M) on a single twisted-pair.

This Recommendation defines the minimal set of requirements to provide satisfactory simultaneous transmission between the network and the customer interface of a variety of frame bearers and other services such as POTS or ISDN. This Recommendation permits network providers an expanded use of existing copper facilities. All required physical layer aspects to ensure compatibility between equipment in the network and equipment at a remote location are specified. Equipment may be implemented with additional functions and procedures.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[1]	ITU-T Recommendation G.961 (1993), Digital transmission system on metallic local lines for ISDN basic rate access.

[2]	ITU-T Recommendation G.994.1 (2003), Handshake procedures for digital subscriber line (DSL) transceivers.

[3]	ITU-T Recommendation G.996.1 (2001), Test procedures for digital subscriber line (DSL) transceivers.

[4]	ITU-T Recommendation G.997.1 (2003), Physical layer management for digital subscriber line (DSL) transceivers.

[5]	ITU-T Recommendation G.992.3 (2002), Asymmetric digital subscriber line (ADSL) transceivers 2 (ADSL2), plus Amendment 1 (2003).

For Annex B

[6]	ETSI TS 102 080 V1.3.2 (2000-05), Transmission and Multiplexing (TM); Integrated Services Digital Network (ISDN) basic rate access; Digital transmission system on metallic local lines.

# 3	Definitions

ITU-T Rec. G.992.3 defines terms applicable to this Recommendation.

# 4	Abbreviations

ITU-T Rec. G.992.3 defines abbreviations applicable to this Recommendation.

# 5	Reference Models

See clause 5/G.992.3.

This Recommendation provides tools for the operator of the access network to control the ADSL transmit PSD and aggregate power in the downstream and upstream directions. Depending on regional spectrum management guidelines, these tools may be needed to enable remote ADSL deployment. In this case, the ATU-C is located in a remote cabinet located between the central office and the customer premises, rather than the Access Node.

# 6	Transport Protocol Specific Transmission Convergence (TPS-TC) function

See clause 6/G.992.3.

# 7	Physical Media Specific Transmission Convergence (PMS-TC) function

See clause 7/G.992.3.

## 7.1	Transport capabilities

See 7.1/G.992.3.

## 7.2	Additional functions

See 7.2/G.992.3.

## 7.3	Block interface signals and primitives

See 7.3/G.992.3.

## 7.4	Block diagram and internal reference point signals

See 7.4/G.992.3.

## 7.5	Control parameters

See 7.5/G.992.3.

## 7.6	Frame structure

### 7.6.1	Derived definitions

See 7.6.1/G.992.3.

### 7.6.2	Valid framing configurations

Table 7-1 displays the allowable range of each PMS-TC control parameter. Additionally, the control parameters shall satisfy some relationships to one another for the set of control parameter values to be valid as displayed in Table 7-1. Some ranges of the valid control parameter values are expressed in terms of NSC, which is the number of subcarriers as defined in 8.8.1, Subcarriers.

An additional requirement is made on the value of the Bp,n. Each frame bearer shall be transported in one and only one latency path. This means that in any valid framing configuration, there shall be no more than one non-zero control parameter in each set { B0,n, B1,n, B2,n, B3,n }.

Table 7-1/G.992.5 – Valid framing configurations

| Parameter | Capability |
| --- | --- |
| MSGmin | 4000  MSGmin < 64000 |
| MSGmax | MSGmax = 64000 |
| NBC | 1  NBC  4 |
| NLP | 1  NLP  4 |
| MSGLP | 0  MSGLP  3 |
| MSGC | The valid values of MSGC are those required to support valid minimum and maximum overhead rates, MSGmin and MSGmax. |
| Bp,n | 0≤Bp,n≤254, nBp,n≤254 |
| Mp | 1, 2, 4, 8 or 16. If Rp = 0 then Mp = 1 |
| Tp | 1  TP  64 |
| Rp | 0, 2, 4, 6, 8, 10, 12, 14, or 16 |
| Dp | 1, 2, 4, 8, 16, 32, 64 if Rp = 0 then Dp = 1 |
| Lp | 1  Lp  15  (NSC – 1) and Lp shall be such that 8≤Lp≤15×NSC–1 |
| Relation of Sp and Mp | Configurations that satisfy the following relationship are valid: Mp / 3  Sp  32  Mp (see Note 1). |
| Overhead Rate Constraints | Configurations that satisfy the following relationship are valid: 0.1 kbit/s  ORp  64 kbit/s (see Note 2). |
| Delay Constraints | Configurations that satisfy the following relationship are valid: 1/3  Sp  64 (see Note 3). |
| Overhead Channel Period | At initialization, configurations that provide a period for each overhead channel PERp between 15 and 20 ms are valid. After an on-line reconfiguration of type 2 (DRR) or type 3 (SRA), configurations that provide a period for each overhead channel PERp between 1.875 and 160 ms are valid. |
| NOTE 1 – This condition is a bound on the number of Mux Data Frames per symbol. NOTE 2 – The 0.1 kbit/s overhead rate lower bound corresponds to an SEQp = 2 (see Table 7-14/G.992.3) and an overhead channel period of 160 ms. NOTE 3 – This condition puts bounds on the number of FEC codewords per symbol. |  |

NOTE – The G.992.5 PMS-TC function differs from the G.992.3 PMS-TC function only in the upper bound of the number of Mux Data Frames per symbol and the number of FEC codewords per symbol. ITU-T Rec. G.992.5 defines an upper bound of three, while ITU-T Rec. G.992.3 defines an upper bound of two.

### 7.6.3	Mandatory configurations

See 7.6.3/G.992.3.

## 7.7	Data plane procedures

See 7.7/G.992.3.

## 7.8	Control plane procedures

See 7.8/G.992.3.

## 7.9	Management plane procedures

See 7.9/G.992.3.

## 7.10	Initialization procedures

See 7.10/G.992.3.

For use in this Recommendation, the unsigned 12-bit net_max value in Table 7-18/G.992.3 is the data rate divided by 8000 (rather than 4000 for use in ITU-T Rec. G.992.3).

## 7.11	On-line reconfiguration

See 7.11/G.992.3.

## 7.12	Power management mode

See 7.12/G.992.3.

# 8	Physical Media Dependent function

See clause 8/G.992.3.

## 8.1	Transport capabilities

See 8.1/G.992.3.

## 8.2	Additional functions

See 8.2/G.992.3.

## 8.3	Block interface signals and primitives

See 8.3/G.992.3.

## 8.4	Block diagram and internal reference point signals

See 8.4/G.992.3.

## 8.5	Control parameters

### 8.5.1	Definition of control parameters

The configuration of the PMD function is controlled by a set of control parameters defined in 8.5.1/G.992.3.

The tssi values depend on CO-MIB settings (see ITU-T Rec. G.997.1 [4]) and on local capabilities and are exchanged in the G.994.1 Phase. The tssi values are determined by the ATU transmit function:

–	For the upstream direction, the CO-MIB settings consist of a per upstream subcarrier indication of which subcarriers may be in the upstream SUPPORTEDset and which subcarriers shall not be in the upstream SUPPORTEDset. This information is conveyed from ATU-C to ATU-R in the G.994.1 CL upstream spectrum shaping parameter block and is used by the ATU-R (in combination with local restrictions) to determine which subcarriers to include in the upstream SUPPORTEDset (see  8.13.2.4).

–	For the downstream direction, the CO-MIB settings consist of a per downstream subcarrier indication of which subcarriers may be in the downstream SUPPORTEDset and which subcarriers shall not be in the downstream SUPPORTEDset. This information is used by the ATU-C (in combination with local restrictions) to determine which subcarriers to include in the downstream SUPPORTEDset (see  8.13.2.4).

–	For the downstream direction, the CO-MIB settings also include the downstream PSD mask applicable at the U-C2 reference point (see clause 5). This MIB PSD mask may impose PSD restrictions in addition to the Limit PSD mask defined in the relevant Annex, as relevant for the chosen application option. This information is used by the ATU-C (in combination with local restrictions) to determine which subcarriers to include in the downstream SUPPORTEDset (see 8.13.2.4) and to determine the level of spectrum shaping (i.e., tssi value) to be applied to these subcarriers. The downstream PSD mask specified through the CO-MIB shall satisfy the requirements defined in the remainder of this clause.

The downstream PSD mask in the CO-MIB shall be specified through a set of breakpoints. Each breakpoint shall consist of a subcarrier index t and a MIB PSD mask level (expressed in dBm/Hz) at that subcarrier. The set of breakpoints can then be represented as [(t1, PSD1), (t2, PSD2), … , (tN, PSDN)]. In the CO-MIB, the subcarrier index shall be coded as an unsigned integer in the range from roundup(f_pb_start/f) to rounddown(f_pb_stop/f), where f_pb_start and f_pb_stop are the lower and higher edge, of the passband respectively and f is the subcarrier spacing defined in 8.8.1. The passband is defined in Annexes A, B or I, as relevant to the selected application option. The MIB PSD mask level shall be coded as an unsigned integer representing the MIB PSD mask levels 0 dBm/Hz (coded as 0) to –127.5 dBm/Hz (coded as 255), in steps of 0.5 dBm/Hz, with valid range 0 to –95 dBm/Hz. The maximum number of breakpoints is 32.

The set of breakpoints specified in the CO-MIB shall comply to the following restrictions, and the corresponding MIB PSD mask for each frequency f shall be defined as following:

1)	General

–	tn < tn + 1 for n = 1 to N – 1

–	fn = tn  f

2)	Low-frequency end and high-frequency end of MIB PSD mask (f)

–	t1 = roundup(f_pb_start/f) or (73 ≤ t1 ≤ 271)

–	tN = rounddown(f_pb_stop/f)

–	f_lm_start = frequency at which the flat extension below f1 intersects the Limit mask (0 Hz if no intersect).

–	f_lm_stop = frequency at which the flat extension above fN intersects the Limit mask.

–	At frequencies below f1 and at frequencies above fN, the MIB PSD mask shall be obtained as follows:

MIB PSD mask f=Limit mask ff<flmstartPSD1flmstart≤f≤f1PSDNfN<f≤flmstopLimit mask ff>flmstop

3)	MIB PSD stopband in lower frequency part

if (73 ≤ t1 ≤ 271) then:

–	PSD1 = –95 dBm/Hz

–	Set of valid t2 values is every 10th tone starting from tone 100 up until tone 280

–	The value t1 shall be:

t1=rounddown t2−PSD2−PSD12.2 dB/tone

–	At frequencies between f1 and f2, the MIB PSD mask is obtained by interpolation in dB on a logarithmic frequency scale as follows:

MIB PSD mask f=PSD1+PSD2–PSD1×log f/Δft1logt2/t1 f1<f≤f2

4)	MIB PSD inband shaping

if t1 = roundup(f_pb_start/f) then for n = 1 to N – 1:

if (73 ≤ t1 ≤ 271) then for n = 2 to N – 1:

–	The inband slope shall comply to:

PSDn+1−PSDntn+1−tn≤0.75dB/tone

–	MAX (PSDn) – MIN (PSDn) £ 20 dB

–	MAX PSD of the Limit mask –20 dB £ MAX (PSDn) £ MAX PSD of the Limit mask

–	The MIB PSD mask is obtained by interpolation in dB on a linear frequency scale as follows:

MIB PSD mask f=PSDn + PSDn+1−PSDn×f/Δf−tntn+1−tn fn<f≤fn+1

NOTE – If the first breakpoint has subcarrier index 73 ≤ t1 ≤ 271, then a stopband is created in the lower frequency part of the passband, with spectrum shaping applied to the remainder of the passband. If t1 = roundup(f_pb_start/f), then only spectrum shaping is applied over the whole passband.

5)	RFI band specification

–	A RFI band is specified in the CO-MIB PSD mask through a set of 4 breakpoints (t(i + 1), PSD(i + 1)) to (t(i + 4), PSD(i + 4)), as shown in Figure 8-1. In addition, the CO-MIB also contains an explicit indication that the pair (t(i + 2), t(i + 3)) represents an RFI band (see ITU-T Rec. G.997.1).

–	The restrictions on the breakpoints specifying an RFI band are:

PSDi+1−PSDi+2ti+1−ti+2≤1.5dB/tone

PSDi+2≥PSDLimitmaskfi+2–33.5 dB

PSDi+2=PSDi+3

PSDi+3≥PSDLimitmaskfi+3–33.5 dB

PSDi+4–PSDi+3ti+4–ti+3≤1.5 dB/tone

–	In the RFI band, the MIB PSD mask is given by following equations:

MIB PSD maskf=PSDi+1fi+1≤f≤fi+2PSDi+2=PSDi+3fi+2≤f≤fi+3PSDi+4fi+3≤f≤fi+4

Figure 8-1/G.992.5 – Restrictions on breakpoints and MIB PSD mask (f)

The (informative) MIB PSD Template is defined as the CO-MIB PSD mask –3.5 dB, for f_lm_start ≤ f ≤ f_lm_stop, except for the MIB PSD stopband in the lower frequency part, which remains at –95 dBm/Hz, and the stopband of the RFI bands which remains at PSD(i + 2).

The PSD mask to which the ATU-C transmitter shall comply at the U-C2 reference point shall be the minimum (at each frequency) of the Limit PSD mask (specified in Annexes A, B, or I) and the CO-MIB PSD mask specified through the CO-MIB.

The following figures give a number of examples of MIB PSD masks which can be constructed within the above restrictions. Figure 8-2 illustrates a number of PSD masks which introduce a stopband in the first part of the frequency band. Figure 8-3 illustrates the restrictions on the MIB PSD in-band shaping. The PSD toolbox techniques shown in these figures may be combined in practice.

Figure 8-2/G.992.5 – Illustration of a stopband in the first part of the frequency band

Figure 8-3/G.992.5 – Illustration of the restrictions on the MIB PSD in-band shaping

### 8.5.2	Mandatory and optional settings of control parameters

See 8.5.2/G.992.3.

### 8.5.3	Setting control parameters during initialization

See 8.5.3/G.992.3.

#### 8.5.3.1	During the G.994.1 Phase

See 8.5.3.1/G.992.3.

#### 8.5.3.2	During the channel analysis phase

The format of the PMD function control parameters involved in the MSG1 messages shall be as shown in Table 8-1.

Table 8-1/G.992.5 – Format of PMD function control parameters included in MSG1

| Parameter | Format |
| --- | --- |
| TARSNRM | Unsigned 9-bit integer, 0 to 310 (0 to 31 dB in 0.1-dB steps). |
| MINSNRM | Unsigned 9-bit integer, 0 to 310 (0 to 31 dB in 0.1-dB steps). |
| MAXSNRM | Unsigned 9-bit integer, 0 to 310 (0 to 31 dB in 0.1-dB steps). The value 511 is a special value, indicating that excess margin relative to MAXSNRM need not be minimized (see 8.6.4/G.992.3), i.e., that the MAXSNRM value is effectively infinite. |
| RA-MODE | Unsigned 2-bit integer, values 1 to 3. |
| PM-MODE | Binary 2-bit indication, each set to 0 or 1. |
| RA-USNRM | Unsigned 9-bit integer, 0 to 310 (0 to 31 dB in 0.1-dB steps). |
| RA-UTIME | Unsigned 14-bit integer, 0 to 16383 (in seconds). |
| RA-DSNRM | Unsigned 9-bit integer, 0 to 310 (0 to 31 dB in 0.1-dB steps). |
| RA-DTIME | Unsigned 14-bit integer, 0 to 16383 (in seconds). |
| BIMAX | Unsigned 4 bit integer, 8 to 15. |
| EXTGI | Unsigned 8-bit integer, 0 to 255 (0 to 25.5 dB in 0.1-dB steps). |
| CA-MEDLEY | Unsigned 6-bit integer, 0 to 63 (times 512 symbols). |
| WINDOW SAMPLES | Window samples are represented by NSCds/64 entries. Each entry is 16-bit unsigned integer, in multiple of 2–16 (see 8.8.4). |

The value CA-MEDLEY represents the minimum duration (in multiples of 512 symbols) of the MEDLEY state during the Initialization Channel Analysis Phase. It can be different for the ATU-C (CA-MEDLEYus indicates the minimum length of the R-MEDLEY state) and the ATU-R (CA-MEDLEYds indicates the minimum length of the C-MEDLEY state). See 8.13.5.1.4 and 8.13.5.2.4/G.992.3.

The PMD function control parameters exchanged in the C-MSG1 message are listed in Table 8-2. Window samples shall be included only if windowing is applied (which is indicated in C-MSG-FMT, see 8.13.3.1.10).

| Table 8-2/G.992.5 – PMD function control parameters included in C-MSG1 |  |  |
| --- | --- | --- |
| Octet Nr [i] | Parameter | Format PMD bits [8  i + 7 to 8  i + 0] |
| 0 | TARSNRMds (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 1 | TARSNRMds (msb) | [ 0000 00xx ], bit 8 |
| 2 | MINSNRMds (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 3 | MINSNRMds (msb) | [ 0000 000x ], bit 8 |
| 4 | MAXSNRMds (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 5 | MAXSNRMds (msb) | [ 0000 000x ], bit 8 |
| 6 | RA-MODEds | [ 0000 00xx ], bits 1 to 0 |
| 7 | PM-MODE | [ 0000 00xx ], bits 1 to 0 |
| 8 | RA-USNRMds (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 9 | RA-USNRMds (msb) | [ 0000 000x ], bit 8 |
| 10 | RA-UTIMEds (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 11 | RA-UTIMEds (msb) | [ 00xx xxxx ], bits 13 to 8 |
| 12 | RA-DSNRMds (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 13 | RA-DSNRMds (msb) | [ 0000 000x ], bit 8 |
| 14 | RA-DTIMEds (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 15 | RA-DTIMEds (msb) | [ 00xx xxxx ], bits 13 to 8 |
| 16 | BIMAXds | [ 0000 xxxx ], bits 3 to 0 |
| 17 | EXTGIds | [ xxxx xxxx ], bits 7 to 0 |
| 18 | CA-MEDLEYus | [ 00xx xxxx ], bits 5 to 0 |
| 19 | Reserved | [ 0000 0000 ] |
| 20 | w(0) (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 21 | w(0) (msb) | [ xxxx xxxx ], bits 15 to 8 |
| ... | ... | ... |
| 18 + NSCds/32 | w(NSCds/64 – 1) (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 19 + NSCds/32 | w(NSCds/64 – 1) (msb) | [ xxxx xxxx ], bits 15 to 8 |

The PMD function control parameters exchanged in the R-MSG1 message are listed in Table 8-3.

Table 8-3/G.992.5 – PMD function control parameters included in R-MSG1

| Octet Nr [i] | Parameter | Format PMD bits [8  i + 7 to 8  i + 0] |
| --- | --- | --- |
| 0 | BIMAXus | [ 0000 xxxx ], bits 3 to 0 |
| 1 | EXTGIus | [ xxxx xxxx ], bits 7 to 0 |
| 2 | CA-MEDLEYds | [ 00xx xxxx ], bits 5 to 0 |
| 3 |  | [ 0000 0000 ] |

The value EXTGI shall be in the [0..MAXNOMPSD – NOMPSD] range. The value may or may not depend on the transmit PMD function’s capabilities and the line characteristics identified during Channel Discovery Phase. The receive PMD function shall use gi values in the [_14.5 .. + 2.5 + EXTGI] range. Depending on its capabilities and the line characteristics identified during Channel Discovery Phase, the receive PMD function may or may not use gi values up to the allowed maximum value.

The ATU-C shall set the REFPSDds, the downstream tssi and the EXTGIds values such that the downstream transmit PSD mask is not violated at any of the subcarriers in the downstream MEDLEYset, even if the gi value requested by the ATU-R is as high as (2.5 + EXTGI) dB for one or more of those subcarriers.

NOTE – An extended range for gi values can only be used if the transmit PSD function chooses to use a nominal transmit PSD level that is below the maximum transmit PSD level allowed by the CO-MIB (see 8.5.1) and can only be used within the transmit PSD mask limitations set by the CO-MIB.

#### 8.5.3.3	During the Exchange Phase

The format of the PMD function control and test parameters involved in the PARAMS messages shall be as shown in Table 8-4.

Table 8-4/G.992.5 – Format of PMD function control parameters included in PARAMS

| Parameter | Format |
| --- | --- |
| LATN | Test parameter (see 8.12.3/G.992.3) |
| SATN | Test parameter (see 8.12.3/G.992.3) |
| SNRM | Test parameter (see 8.12.3/G.992.3) |
| ATTNDR | Test parameter (see 8.12.3/G.992.3) |
| ACTATP | Test parameter (see 8.12.3/G.992.3) |
| TRELLIS | Binary indication, set to 0 or 1. |
| Bits and Gains table | Bits and gains table is represented by NSC – 1 entries or 2  (NSC – 1) octets. Each entry is a 16-bit unsigned integer. Bits in 4 LSB, Gain in 12 MSB, linear scale. The gain value shall be represented with 3 bits before and 9 bits after the decimal point, i.e., a granularity of 1/512 in linear scale. |
| Tone ordering table | Tone ordering is represented by NSC – 1 entries. Each entry is an 11-bit unsigned integer, representing a subcarrier index in the 1 to NSC – 1 range. |

The test parameters are mapped into messages using an integer number of octets per parameter value. In case the parameter value as defined in 8.12.3/G.992.3  is represented with a number of bits that is not an integer number of octets, the parameter value shall be mapped into the least significant bits of the message octets. Unused more significant bits shall be set to 0 for unsigned parameter values and shall be set to the sign bit for signed parameter values.

The PMD function control parameters and test parameters exchanged in the C-PARAMS message are listed in Table 8-5.

| Table 8-5/G.992.5 – PMD function control parameters included in C-PARAMS |  |  |
| --- | --- | --- |
| Octet Nr [i] | Parameter | Format PMD bits [8  i + 7 to 8  i + 0] |
| 0 | LATNus (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 1 | LATNus (msb) | [ 0000 00xx ], bits 9 and 8 |
| 2 | SATNus (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 3 | SATNus (msb) | [ 0000 00xx ], bits 9 and 8 |
| 4 | SNRMus (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 5 | SNRMus (msb) | [ ssss sxxx ], bits 10 to 8 |
| 6 | ATTNDRus (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 7 | ATTNDRus | [ xxxx xxxx ], bits 15 to 8 |
| 8 | ATTNDRus | [ xxxx xxxx ], bits 23 to 16 |
| 9 | ATTNDRus (msb) | [ xxxx xxxx ], bits 31 to 24 |
| 10 | ACTATPus (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 11 | ACTATPus (msb) | [ ssss ssxx ], bits 9 and 8 |
| 12 | TRELLISus | [ 0000 000x], bit 0 |
| 13 | Reserved | [ 0000 0000 ] |
| 14 | Upstream Bits and Gains For subcarrier 1 (lsb) | [ gggg bbbb ], bits 7 to 0 |
| 15 | Upstream Bits and Gains For subcarrier 1 (msb) | [ gggg gggg ], bits 15 to 8 |
| .. | ... | ... |
| 10 + 2  NSCus | Upstream Bits and Gains Subcarrier NSCus – 1 (lsb) | [ gggg bbbb ], bits 7 to 0 |
| 11 + 2  NSCus | Upstream Bits and Gains Subcarrier NSCus – 1 (msb) | [ gggg gggg ], bits 15 to 8 |
| 12 + 2  NSCus | Reserved | [ 0000 0000 ] |
| 13 + 2  NSCus | Reserved | [ 0000 0000 ] |
| 14 + 2  NSCus | Upstream Tone ordering First subcarrier to map (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 15 + 2  NSCus | Upstream Tone ordering First subcarrier to map (msb) | [ 0000 0xxx ], bits 10 to 8 |
| ... | ... | ... |
| 10 + 4  NSCus | Upstream Tone ordering Last subcarrier to map (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 11 + 4  NSCus | Upstream Tone ordering Last subcarrier to map (msb) | [ 0000 0xxx ], bits 10 to 8 |

The PMD function control parameters exchanged in the R-PARAMS message are listed in Table 8-6.

| Table 8-6/G.992.5 – PMD function control parameters included in R-PARAMS |  |  |
| --- | --- | --- |
| Octet Nr [i] | Parameter | Format PMD bits [8  i + 7 to 8  i + 0] |
| 0 | LATNds (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 1 | LATNds (msb) | [ 0000 00xx ], bits 9 and 8 |
| 2 | SATNds (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 3 | SATNds (msb) | [ 0000 00xx ], bits 9 and 8 |
| 4 | SNRMds (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 5 | SNRMds (msb) | [ ssss sxxx ], bits 10 to 8 |
| 6 | ATTNDRds (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 7 | ATTNDRds | [ xxxx xxxx ], bits 15 to 8 |
| 8 | ATTNDRds | [ xxxx xxxx ], bits 23 to 16 |
| 9 | ATTNDRds (msb) | [ xxxx xxxx ], bits 31 to 24 |
| 10 | ACTATPds (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 11 | ACTATPds (msb) | [ ssss ssxx ], bits 9 and 8 |
| 12 | TRELLISds | [ 0000 000x ], bit 0 |
| 13 | Reserved | [ 0000 0000 ] |
| 14 | Downstream Bits and Gains For subcarrier 1 (lsb) | [ gggg bbbb ], bits 7 to 0 |
| 15 | Downstream Bits and Gains For subcarrier 1 (msb) | [ gggg gggg ], bits 15 to 8 |
| ... | ... | ... |
| 10 + 2  NSCds | Downstream Bits and Gains Subcarrier NSCds – 1 (lsb) | [ gggg bbbb ], bits 7 to 0 |
| 11 + 2  NSCds | Downstream Bits and Gains Subcarrier NSCds – 1 (msb) | [ gggg gggg ], bits 15 to 8 |
| 12 + 2  NSCds | Reserved | [ 0000 0000 ] |
| 13 + 2  NSCds | Reserved | [ 0000 0000 ] |
| 14 + 2  NSCds | Downstream Tone ordering First subcarrier to map (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 15 + 2  NSCds | Downstream Tone ordering First subcarrier to map | [ 0000 0xxx ], bits 10 to 8 |
| ... | ... | ... |
| 10 + 4  NSCds | Downstream Tone ordering Last subcarrier to map | [ xxxx xxxx ], bits 7 to 0 |
| 11 + 4  NSCds | Downstream Tone ordering Last subcarrier to map | [ 0000 0xxx ], bits 10 to 8 |

## 8.6	Constellation encoder for data symbols

See 8.6/G.992.3.

## 8.7	Constellation encoder for synchronization and L2 exit symbols

See 8.7/G.992.3.

## 8.8	Modulation

See 8.8/G.992.3.

### 8.8.1	Subcarriers

See 8.8.1/G.992.3.

### 8.8.2	Inverse Discrete Fourier Transform

See 8.8.2/G.992.3.

### 8.8.3	Cyclic prefix and cyclic suffix

With a data symbol rate of 4 kHz, a DMT subcarriers spacing of f = 4.3125 kHz and an IDFT size of 2  NSC, a cyclic prefix of (2  NSC  5/64) samples could be used. That is:

2×NSC+2×NSC×564 × 4.0 kHz=2×NSC × 4.3125 kHz=fs (the sample frequency)

The cyclic prefix shall, however, be shortened to (2  NSC  4/64 = NSC/8) samples, and a synchronization symbol (with a length of 2  NSC  68/64 samples) is inserted after every 68 data symbols. That is:

2×NSC×464+2×NSC×69=2×NSC×564+2×NSC×68

For symbols with cyclic prefix, the last NSC/8 samples of output of the IDFT (xn for n = 2  NSC _ NSC/8 to 2  NSC – 1) shall be prepended to the block of 2  NSC samples, to form a block of (2  NSC  17/16) samples. Symbols with cyclic prefix are transmitted at a symbol rate of 4.3125  16/17  4.059 kHz.

In the downstream direction, the ATU-C transmitter may apply windowing. If windowing is applied, symbols with cyclic prefix shall also have a cyclic suffix. If windowing is not applied, symbols with cyclic prefix shall not have a cyclic suffix. For symbols with a cyclic suffix, the first NSCds/32 samples of output of the IDFT (xn for n = 0 to NSCds/32 – 1) shall be appended to the block of (2  NSC  17/16) samples, to form a block of (2  NSC  69/64) samples. Symbols with cyclic suffix are transmitted at a symbol rate of 4.3125  16/17  4.059 kHz.

The cyclic prefix (and suffix if windowing is applied) shall be used for all symbols transmitted starting from the Channel Analysis Phase of the initialization sequence (see 8.13.5). Before the Channel Analysis Phase, all symbols shall be transmitted without cyclic prefix and without cyclic suffix. Symbols transmitted without cyclic prefix and without cyclic suffix are transmitted at a symbol rate of 4.3125 kHz.

If an oversampled IDFT is used (i.e., N > NSC, see 8.8.2), the number of cyclic prefix and cyclic suffix samples shall be adapted accordingly. For symbols with cyclic prefix, the last N/8 samples of output of the IDFT (xn for n = 2  N – N/8 to 2  N – 1) shall be prepended to the block of 2  N samples, to form a block of (2  N  17/16) samples. For symbols with cyclic suffix, the first N/32 samples of output of the IDFT (xn for n = 0 to N/32 – 1) shall be appended to the block of (2  N  17/16) samples, to form a block of (2  N  69/64) samples.

### 8.8.4	Parallel/Serial convertor

The block of xn samples (n = 0 to 2  NSC – 1) shall be read out to the digital-to-analog converter (DAC) in sequence.

If no cyclic prefix is used, the DAC samples yn in sequence are:

yn=xnfor n=0 to 2×NSC– 1

If a cyclic prefix is used, the DAC samples yn in sequence are (see Figure 8-5):

yn=xn–2×NSC–NSC8for n=0 to NSC8–1yn=xn–NSC8for n=NSC8 to 1716×2×NSC–1

If for the downstream direction, a cyclic prefix is used and a cyclic suffix is used (windowing), then the DAC samples in sequence shall be:

1–wi×prevxi+wi×x2×NSC–NSC8+i for i=0 to NSC32–1x2×NSC–NSC8+i for i=NSC32 to NSC8–1xi for i=0 to 2×NSC–1

where the prev_x(i) corresponds to the cyclic suffix of the previous symbol (see 8.3.3). The NSC/32 samples in the cyclic suffix of the previous symbol and the first NSC/32 samples of the cyclic prefix of the current symbol are transmitted as overlapping in time, i.e., a weighted sum is transmitted. The above defined DAC sample sequence corresponds to applying a (2  NSC  69/64) samples window. The window in discrete time representation (set of wi) shall defined as follows:

wi is vendor discretionaryfor i=0 to NSC64–1wi=1–wNSC32–1–ifor i=NSC64 to NSC32–1wi=1for i=NSC32 to 2×NSC×1716–1wi=w2×NSC×6964–1−ifor i=2×NSC×1716 to 2×NSC×6964–1

Each of the time discrete window samples shall be represented in a 16-bit unsigned integer as a multiple of 65536, in the 0 to 1 – 2–16 range.

The window in continuous time representation (w(t)) shall be defined as follows:

wt=i=02×NSC×6964–1sinct×fs–i ⋅ wi

The ATU-C shall indicate in the C-MSG-FMT message whether windowing is applied or not. In case windowing is applied, the C-MSG1 shall contain the window samples w(i) for i = 0 to NSC/64 – 1 (see 8.5.3.2). These NSC/64 samples define the complete window of 2  NSC  69/64 samples as defined above.

NOTE – The C-MSG1 message is transmitted in the Initialization procedures (see 8.13) and Short initialization procedures (see 8.14). It is not transmitted in the Loop diagnostics mode procedures (see 8.15).

If an oversampled IDFT is used (i.e., N > NSC, see 8.8.2), the number of window samples shall be adapted accordingly from the window in continuous time representation w(t) to a window of 2  N  69/64 samples. The ATU-C shall truncate in time and round samples to the same precision as the (non-oversampled) window samples communicated to the ATU-R in the C-MSG1 message. The ATU-R receiver shall take into account the error that the ATU-C transmitter may introduce through this time truncation and value rounding process.

Filtering may be applied to the sample sequence going into the DAC.

### 8.8.5	DAC and AFE

The DAC produces an analog signal that is passed through the analog front-end (AFE) and transmitted across the digital subscriber line (DSL). The analog front end may include filtering.

If the transmit PMD function is configured in the L3 idle state, then a zero output voltage shall be transmitted at the U-C2 (for ATU-C) and the U-R2 (for ATU-R) reference point (see reference model in 5.4). The analog front end may include filtering.

## 8.9	Transmitter dynamic range

See 8.9/G.992.3.

Due to the non-flat PSD used for the downstream transmitted signals, the MTPR requirements at the ATU-C are for further study.

## 8.10	Transmitter spectral masks

See 8.10/G.992.3.

In this Recommendation, Annex L is not defined.

## 8.11	Control plane procedures

See 8.11/G.992.3.

## 8.12	Management plane procedures

See 8.12/G.992.3.

## 8.13	Initialization procedures

See 8.13/G.992.3.

### 8.13.1	Overview

See 8.13.1/G.992.3.

### 8.13.2	G.994.1 phase

See 8.13.2/G.992.3.

#### 8.13.2.1	Handshake – ATU-C

See 8.13.2.1/G.992.3.

The G.992.5 handshake codepoints are defined in ITU-T Rec. G.994.1 [2].

#### 8.13.2.2	Handshake – ATU-R

See 8.13.2.2/G.992.3.

#### 8.13.2.3	G.994.1 transmit PSD levels

See 8.13.2.3/G.992.3.

#### 8.13.2.4	Spectral bounds and shaping parameters

The spectrum bounds and shaping parameters shall be applied as described in 8.13.2.4/G.992.3, with the following differences:

–	If windowing is applied in the downstream direction, the w2(f) used in Equation 8-1/G.992.3 is the Fourier transform of the autocorrelation function of the window w(t) (see 8.8.4), normalized such that the integral of W2(f) is equal to unity.

–	The downstream tssi values as indicated in the G.994.1 CL message shall be used in the Channel Discovery Phase.

–	Starting from the transceiver training phase, tssi values for subcarriers included the downstream SUPPORTEDset shall be ceiled according to the following relationship, before being applied relative to the REFPSDds level:

ceiled_log_tssi=MINlog_tssi+PCBds, 0 dB

The ceiled_log_tssi shall only be computed at beginning of transceiver training phase and shall not be adapted when PCBds changes during showtime (e.g., with entry into the L2 power management state or with L2 trim, see 9.4.1.7).

–	Starting from the transceiver training phase, tssi values for subcarriers not included in the downstream SUPPORTEDset shall be applied as indicated in the G.994.1 CL message, relative to the REFPSDds level.

NOTE 1 – This corresponds to a ceiling of the transmit PSD to the REFPSDds level over the subcarriers included in the SUPPORTEDset, and a lowering of the transmit PSD by PCBds dB over the subcarriers not included in the SUPPORTEDset. Depending on the spectrum shaping applied through the tssi values as indicated in the G.994.1 CL message, this may reduce the transmit PSD level only in a part or in the whole passband.

NOTE 2 – Because the ATU-C applies the downstream powercutback through a ceiling of the downstream tssi values before being applied relative to the REFPSDds level. This implies that the ATU-R receiver will have to take into account the downstream tssi values indicated in the G.994.1 CL message, when determining the downstream powercutback to be requested through the R-MSG-PCB message.

Figure 8-4 shows an example of the downstream tssi values as a function of the subcarrier index i, for the case that the SUPPORTEDset contains the subcarriers with index i = 100 to 399 and i = 484 to 500 and for N = 2  NSC = 1024 (oversampled IDFT). At frequencies i  f, with 100  i  399 and 484  i  500 and f = 4.3125 kHz, the tssi value is chosen such that for ideal filters, ideal DAC and ideal AFE, the spectrum on the U-interface corresponds with the transmit spectrum given in Annex A. At frequencies i  f, with 400  i  483 the carriers are not in the SUPPORTEDset to avoid the HAM-band [1.81, 2.0] MHz, taking into account a 20-tone transition band. In this case no windowing is assumed, and therefore some extra notch-filter need to be applied to reach a stopband of –80 dBm/Hz.

Figure 8-4/G.992.5 – Example of the downstream log_tssi values (in dB)
as function of the subcarrier index

NOTE 3 – For the downstream direction, the CO-MIB contains a per subcarrier indication of the maximum transmit PSD level at the U-C reference point, to apply at all times, except during the G.994.1 Phase. The CO-MIB also contains a per subcarrier indication whether the subcarrier is or is not allowed to be sent starting from the initialization Channel Analysis phase. From this information and taking into account its own capabilities, the ATU-C selects the downstream SUPPORTEDset of subcarriers and computes the CL downstream spectrum shaping parameter block information.

NOTE 4 – For the upstream direction, the CO-MIB contains a per subcarrier indication whether the subcarrier is or is not allowed to be sent starting from the initialization Channel Analysis phase. This information is conveyed to the ATU-R in the CL upstream spectrum shaping parameter block (through SUPPORTEDset indication and only using tssi values 0 and 1 in linear scale). From this information and taking into account its own capabilities, the ATU-R selects the upstream SUPPORTEDset of subcarriers and computes the CLR upstream spectrum shaping parameter block information.

### 8.13.3	Channel Discovery Phase

See 8.13.3/G.992.3.

#### 8.13.3.1	ATU-C channel discovery

See 8.13.3.1/G.992.3.

##### 8.13.3.1.1	C-QUIET1

See 8.13.3.1.1/G.992.3.

##### 8.13.3.1.2	C-COMB1

The duration of the C-COMB1 state is of fixed length. In the C-COMB1 state, the ATU-C shall transmit 128 C-COMB symbols.

During this state, the ATU-R performs timing recovery and measures some characteristics of the downstream channel for C-TREF pilot tone selection and for the estimation of the required ATU-R minimum Upstream Power Cutback and ATU-R minimum Downstream Power Cutback. These functions can be continued during C-COMB2.

The C-COMB symbol shall be defined as a wideband multi-tone symbol containing the 16 subcarriers with index 11, 23, 35, 47, 59, 64, 71, 83, 95, 107, 119, 143, 179, 203, 227 and 251 and all subsequent subcarriers with index 251 + k  24 (with k integer such that the subcarrier index is in the 256 to NSCds – 1 range). The subcarrier spacing has been selected to minimize audible interference into the POTS band prior to applying cutbacks that may be required in the presence of an off-Hook POTS terminal and to limit aggregate transmit power to 8.9 dBm (i.e., the 12 dB power cutback level).

The subcarriers contained in the C-COMB symbol shall modulate the same data bits that are used for the C-REVERB symbols, in such a way that same subcarrier indexes modulate the same data bits with the same 4-QAM constellation, as defined in 8.13.4.1.1. The subcarriers not contained in the C-COMB symbol shall be transmitted at no power (i.e., Xi = Yi = 0).

The C-COMB1 state shall be followed by the C-QUIET2 state.

##### 8.13.3.1.3	C-QUIET2

See 8.13.3.1.3/G.992.3.

##### 8.13.3.1.4	C-COMB2

See 8.13.3.1.4/G.992.3.

##### 8.13.3.1.5	C-ICOMB1

See 8.13.3.1.5/G.992.3.

##### 8.13.3.1.6	C-LINEPROBE

See 8.13.3.1.6/G.992.3.

##### 8.13.3.1.7	C-QUIET3

See 8.13.3.1.7/G.992.3.

##### 8.13.3.1.8	C-COMB3

See 8.13.3.1.8/G.992.3.

##### 8.13.3.1.9	C-ICOMB2

See 8.13.3.1.9/G.992.3.

##### 8.13.3.1.10	C-MSG-FMT

The C-MSG-FMT state is of fixed length. In the C-MSG-FMT state, the ATU-C shall transmit 96 symbols of C-COMB or C-ICOMB to modulate the C-MSG-FMT message and crc. The C-MSG-FMT message conveys information about the presence, format and length of subsequent ATU-C and ATU-R messages.

The C-MSG-FMT message, m, is defined by:

m = {m15, … , m0}

Bits shall be defined as shown in Table 8-7.

Table 8-7/G.992.5 – Bit definition for the C-MSG-FMT message

| Bit index | Parameter | Definition |
| --- | --- | --- |
| 0 | FMT_R-REVERB1 (value 0 or 1) | Set to 1 indicates that the ATU-C requests an extended duration of the R-REVERB1 state. Set to 0 indicates it does not. |
| 1 |  | Reserved, set to 0. |
| 2 | FMT_C-REVERB4 (value 0 or 1) | Set to 1 indicates that the ATU-C requests an extended duration of the C-REVERB4 state. Set to 0 indicates it does not. |
| 7..3 | FMT_R-QUIET4 (value 0 to 31) | The (0 to 31) value mapped in these bits indicates the duration of the R-QUIET4 state. The MSB shall be mapped on the higher message bit index. |
| 8 | FMT_C-MSG-PCB | Set to 1 indicates that the C-MSG-PCB message shall include the C-BLACKOUT bits. Set to 0 indicates it shall not. |
| 9 | FMT_C-MSG1 | Set to 1 indicates that windowing is applied with window samples included in the C-MSG1 message. Set to 0 indicates no windowing is applied and no window samples are included in C-MSG1 message. |
| 10 | FMT-C-MEDLEYPRBS | Set to 1 indicates that the ATU-C requests to use the higher order PRBS for the C-MEDLEY data pattern (see 8.13.5.1.4). |
| 15..11 |  | Reserved, set to 0. |

The 16 bits m0-m15 shall be transmitted in 48 symbol periods (m0 first and m15 last). A zero bit shall be transmitted as three consecutive C-COMB symbols. A one bit shall be transmitted as three consecutive C-ICOMB symbols.

After the C-MSG-FMT message has been transmitted, a crc shall be appended to the message. The 16 crc bits shall be computed from the 16 message m bits using the equation:

cD=aDD16modulo gD

where:

aD=m0D15+m1D14...+m15

is the message polynomial formed from the 16 bits of the C-MSG-FMT message, with m0 the least significant bit of the first octet of the C-MSG-FMT message:

gD=D16+D12+D5+1

is the crc generator polynomial, and

cD=c0D15+c1D14...+c14D+c15

is the crc check polynomial.

The 16 bits c0-c15 shall be transmitted in 48 symbol periods (c0 first and c15 last) using the same modulation as used to transmit the message m.

The C-MSG-FMT state shall be followed by the C-MSG-PCB state.

##### 8.13.3.1.11	C-MSG-PCB

See 8.13.3.1.11/G.992.3.

##### 8.13.3.1.12	C-QUIET4

See 8.13.3.1.12/G.992.3.

#### 8.13.3.2	ATU-R channel discovery

See 8.13.3.2/G.992.3.

##### 8.13.3.2.1	R-QUIET1

See 8.13.3.2.1/G.992.3.

##### 8.13.3.2.2	R-COMB1

See 8.13.3.2.2/G.992.3.

##### 8.13.3.2.3	R-QUIET2

See 8.13.3.2.3/G.992.3.

##### 8.13.3.2.4	R-COMB2

See 8.13.3.2.4/G.992.3.

##### 8.13.3.2.5	R-ICOMB1

See 8.13.3.2.5/G.992.3.

##### 8.13.3.2.6	R-LINEPROBE

See 8.13.3.2.6/G.992.3.

##### 8.13.3.2.7	R-QUIET3

See 8.13.3.2.7/G.992.3.

##### 8.13.3.2.8	R-COMB3

See 8.13.3.2.8/G.992.3.

##### 8.13.3.2.9	R-ICOMB2

See 8.13.3.2.9/G.992.3.

##### 8.13.3.2.10	R-MSG-FMT

The R-MSG-FMT state is of fixed length. In the R-MSG-FMT state, the ATU-R shall transmit 96 symbols of R-COMB or R-ICOMB to modulate the R-MSG-FMT message and crc. The R-MSG-FMT message conveys information about the presence, format and length of subsequent ATU-C and ATU-R messages.

The R-MSG-FMT message, m, is defined by:

m = {m15, … , m0}

Bits shall be defined as shown in Table 8-8.

| Table 8-8/G.992.5 – Bit definition for the R-MSG-FMT message |  |  |
| --- | --- | --- |
| Bit index | Parameter | Definition |
| 0 | FMT-R-REVERB1 (value 0 or 1) | Set to 1 indicates that the ATU-R requests an extended duration of the R-REVERB1 state. Set to 0 indicates it does not. |
| 1 |  | Reserved, set to 0. |
| 2 | FMT-C-REVERB4 (value 0 or 1) | Set to 1 indicates that the ATU-R requests an extended duration of the C-REVERB4 state. Set to 0 indicates it does not. |
| 6..3 | FMT-C-TREF1 (value 1 to 15) | The value mapped in these bits indicates the minimum duration of the C-TREF1 state. The MSB shall be mapped on the higher message bit index. |
| 7 | FMT-R-MSG-PCB (value 0 or 1) | Set to 1 indicates that the R-MSG-PCB message shall include the R-BLACKOUT bits. Set to 0 indicates it shall not. |
| 8 | FMT-C-TREF2 (value 0 or 1) | Indicates that the ATU-R requests the ATU-C to transmit C-TREF symbols (if set to 1) or C-QUIET symbols (if set to 0) during R-ECT. |
| 9 | FMT-C-PILOT (value 0 or 1) | Set to 1 indicates that the ATU-R requests the ATU-C to transmit a fixed 4-QAM constellation point on the C-TREF pilot tone. Set to 0 indicates it does not. |
| 10 | FMT-C-MEDLEYPRBS | Set to 1 indicates that the ATU-R requests to use the higher order PRBS for the C-MEDLEY data pattern (see 8.13.5.1.4). |
| 15..11 |  | Reserved, set to 0. |

The 16 bits m0-m15 shall be transmitted in 48 symbol periods (m0 first and m15 last). A zero bit shall be transmitted as three consecutive R-COMB symbols. A one bit shall be transmitted as three consecutive R-ICOMB symbols.

After the R-MSG-FMT message has been transmitted, a crc shall be appended to the message. The 16 crc bits shall be computed in the same way as for the C-MSG-FMT message. The 16 bits c0-c15 shall be transmitted in 48 symbol periods (c0 first and c15 last) using the same modulation as used to transmit the message m.

The R-MSG-FMT state shall be followed by the R-MSG-PCB state.

##### 8.13.3.2.11	R-MSG-PCB

In each direction the transmit power will be reduced by a power cutback which is the highest of the power cutback values determined by the ATU-R and the ATU-C. The ATU-R can consider its receiver dynamic range as determined by observing C-COMB1, and the local line conditions determined by the optional R-LINEPROBE when determining its cutback levels.

The R-MSG-PCB state is of fixed length. In the R-MSG-PCB state, the ATU-R shall transmit 144 or 144 + 3  NSCds symbols of R-COMB or R-ICOMB to modulate the R-MSG-PCB message and crc, depending on whether the R-BLACKOUT bits are included or not. The R-MSG-PCB message conveys the ATU-R determined power cutback levels for both the upstream and downstream directions, the hook status as known by the ATU-R, the signal used for timing recovery during different states and the downstream BLACKOUT bits.

The ATU-R shall indicate in the R-MSG-FMT message whether the R-MSG-PCB message includes the R-BLACKOUT bits or not. If the R-MSG-PCB does not include the R-BLACKOUT bits, the R-MSG-PCB message, m, is defined by:

m = {m31, … , m0}

If the R-MSG-PCB includes the R-BLACKOUT bits, the R-MSG-PCB message, m, is defined by:

m = {m31 + NSCds, … , m0}

Bits shall be defined as shown in Table 8-9.

Table 8-9/G.992.5 – Bit definition for the R-MSG-PCB message

| Bit index | Parameter | Definition |
| --- | --- | --- |
| 5..0 | R-MIN_PCB_DS | ATU-R Minimum Downstream Power Cutback (6-bit value with MSB in bit 5 and LSB in bit 0) |
| 11..6 | R-MIN_PCB_US | ATU-R Minimum Upstream Power Cutback (6-bit value with MSB in bit 11 and LSB in bit 6) |
| 13..12 | HOOK_STATUS | Hook Status (2-bit value with MSB in bit 13 and LSB in bit 12) |
| 15..14 |  | Reserved, set to 0. |
| 26..16 | C-PILOT | Subcarrier index of downstream pilot tone (11-bit value with MSB in bit 26 and LSB in bit 16) |
| 31..27 |  | Reserved, set to 0. |
| 31 + NSCds..32 | R-BLACKOUT | Blackout indication per subcarrier (subcarrier NSCds _ 1 in bit 31 + NSCds, subcarrier 0 in bit 32). Bit 32 shall be set to 0 (i.e., no blackout of DC subcarrier) |

The ATU-R Minimum Downstream Power Cutback level shall be coded as defined in Table 8-10.

Table 8-10/G.992.5 – ATU-R minimum downstream power cutback

| Value (6-bit) | ATU-R minimum downstream power cutback (dB) |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| … | … |
| 40 | 40 |
| 41-63 | Reserved |

The ATU-R Minimum Upstream Power Cutback level shall be coded as defined in Table 8-11.

Table 8-11/G.992.5 – ATU-R minimum upstream power cutback

| Value (6-bit) | ATU-R minimum upstream power cutback (dB) |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| … | … |
| 40 | 40 |
| 41-63 | Reserved |

The hook status shall be coded as defined in Table 8-12. The hook state "Unknown" is intended to be indicated by a device that normally indicates the on- or off- hook state. The state "Not capable to detect" is intended to be indicated by a device that never sets the on or on hook state (e.g., is not capable or disabled to detect the hook state).

Table 8-12/G.992.5 – Hook status

| Value (2-bit) | Hook status |
| --- | --- |
| 0 | Unknown |
| 1 | On-hook |
| 2 | Off-hook |
| 3 | Not capable to detect |

The C-PILOT value shall indicate the index of the C-TREF pilot subcarrier to be used by the ATU-C for the C-TREF timing reference and to be used by the ATU-R during C-TREF1/C-TREF2 for timing recovery. The spectral shaping information exchanged during the G.994.1 phase and the BLACKOUT information exchanged in R-MSG-PCB allows the ATU-R to determine the set of subcarriers the ATU-C will transmit in and after the Channel Analysis Phase (i.e., to determine the MEDLEYset, see 8.13.2.4). The ATU-R shall select a C-TREF pilot subcarrier from the MEDLEYset.

The R-BLACKOUT bits shall contain the R-BLACKOUT bit settings for each of the subcarriers 1 to NSCds – 1. The R-BLACKOUT bit set to 0 for a particular subcarrier indicates that the ATU-C shall transmit that subcarrier at the ATU-C reference transmit PSD level (REFPDSds) level, and including spectral shaping, for the remainder of initialization, starting from the Transceiver Training Phase. The R-BLACKOUT bit be set to 1 indicates that the ATU-C shall transmit no power ("blackout") on that subcarrier, for the remainder of initialization, starting from the Transceiver Training Phase.

An R-MSG-PCB message containing 32 bits m31-m0 shall be transmitted in 96 symbol periods (m0 first and m31 last). An R-MSG-PCB message containing 32 + NSCds bits m31 + NSCds – m0 shall be transmitted in 96 + 3  NSCds symbol periods (m0 first and m31 + NSCds last). A zero bit shall be transmitted as three consecutive R-COMB symbols. A one bit shall be transmitted as three consecutive R-ICOMB symbols.

After the R-MSG-PCB message has been transmitted, a crc shall be appended to the message. The 16 crc bits shall be computed from the 32 or 32 + NSCds message m bits in the same way as the crc bits are calculated for the C-MSG-FMT message.

The 16 bits c0-c15 shall be transmitted in 48 symbol periods (c0 first and c15 last) using the same modulation as used to transmit the message m.

The R-MSG-PCB state shall be followed by the R-REVERB1 state.

### 8.13.4	Transceiver training phase

See 8.13.4/G.992.3.

### 8.13.5	Channel analysis phase

See 8.13.5/G.992.3.

#### 8.13.5.1	ATU-C channel analysis

See 8.13.5.1/G.992.3.

##### 8.13.5.1.1	C-MSG1

The C-MSG1 state is of fixed length. In this state, the ATU-C shall transmit LEN_C-MSG1 C-REVERB or C-SEGUE symbols to modulate the C-MSG1 prefix, message and crc. The C-MSGS1 state shall be the first state in which the ATU-C transmits the cyclic prefix.

The C-MSG1 prefix, p, is defined by:

p = {p31, … , p0} = { 01010101 01010101 01010101 01010101 }

The 32 bits p0 to p31 shall be transmitted in 32 symbol periods (p0 first and p31 last). A zero bit shall be transmitted as a C-REVERB symbol. A one bit shall be transmitted as a C-SEGUE symbol.

The value LEN_C-MSG1 shall be defined as the length of the C-MSG1 prefix, message and crc in bits. Table 8-13 lists the length of the C-MSG1 message summed over TPC-TC, PMS-TC and PMD layers. The TPS-TC, PMS-TC and PMD bits each correspond to an even number of octets.

Table 8-13/G.992.5 – C-MSG1 prefix, message and crc length

| Part of message | Length (bits or symbols) |
| --- | --- |
| Prefix | 32 |
| Npmd (see Note) | 160 or 160 + NSCds/4 |
| Npms | 32 |
| Ntps | 0 |
| Nmsg | 192 or 192 + NSCds/4 |
| CRC | 16 |
| LEN_C-MSG1 (symbols) | 240 or 240 + NSCds/4 |
| NOTE – Length depends on whether windowing is applied or not (see 8.5.3.2). |  |

The C-MSG1 message, m, is defined by:

m=tpsNtps–1, ... ,tps0,pmsNpms–1, ... ,pms0,pmdNpmd–1, ... ,pmd0=mNmsg–1, ... ,m0

The C-MSG1 message conveys 3 sets of parameters, related to TPS-TC, PMS-TC and PMD configuration. TPS-TC parameters are conveyed in the bits tpsNtps–1 to tps0 and are defined in clause 6. PMS-TC parameters are conveyed in the bits pmsNpms–1 to pms0 and are defined in clause 7. PMD parameters are conveyed in the bits pmdNpmd–1 to pmd0 and are defined in clause 8.

The Nmsg bits m0–mNmsg–1 shall be transmitted in Nmsg symbol periods (m0 first and mNmsg–1 last), immediately following the prefix, and using the same modulation as used to transmit the prefix p.

After the C-MSG1 message has been transmitted, a crc shall be appended to the message. The 16 crc bits shall be computed from the Nmsg message m bits (thus not including the prefix) in the same way as the crc bits are calculated for the C-MSG-PCB message.

The 16 bits c0-c15 shall be transmitted in 16 symbol periods (c0 first and c15 last) using the same modulation as used to transmit the message m.

The C-MSG1 state shall be followed by the C-REVERB5 state.

##### 8.13.5.1.2	C-REVERB5

See 8.13.5.1.2/G.992.3.

##### 8.13.5.1.3	C-SEGUE2

See 8.13.5.1.3/G.992.3.

##### 8.13.5.1.4	C-MEDLEY

The C-MEDLEY state is of fixed length. In this state, the ATU-C shall transmit LEN-MEDLEY symbols. The value LEN-MEDLEY shall be the maximum of the CA-MEDLEYus and CA-MEDLEYds values indicated by the ATU-C and the ATU-R in the C-MSG1 and R-MSG1 messages respectively. The value LEN-MEDLEY shall be a multiple of 512 and shall be less than or equal to 32256. The number of symbols transmitted in the C-MEDLEY state shall be equal to the number of symbols transmitted by the ATU-R in the R-MEDLEY state.

A C-MEDLEY symbol shall be defined depending on its symbolcount within the C-MEDLEY state. The first symbol transmitted in the C-MEDLEY state shall have symbolcount equal to zero. For each symbol transmitted in the C-MEDLEY state, the symbolcount shall be incremented.

The data pattern modulated onto each C-MEDLEY symbol shall be taken from one of the pseudo-random binary sequence (PRBS) defined by:

dn=1 for n=1 to 9 and dn=dn−4⊕dn−9 for n>9

or

dn=1 for n=1 to 14 and dn=dn–5⊕dn–11⊕dn–12⊕dn–14 for n>14

Support of the 14th order PRBS is optional for ATU-C and ATU-R. The 14th order PRBS shall be used if and only if the FMT-C-MEDLEYPRBS bit is set to 1 in both the C-MSG-FMT and R-MSG-FMT message respectively. The 9th order polynomial shall be used otherwise.

The C-MEDLEY symbol with symbolcount i shall modulate the 2  NSCds bits d2  NSCds  i + 1 to d2  NSCds  (i + 1).

Bits shall be extracted from the PRBS in pairs. For each symbol transmitted in the C-MEDLEY state, NSCds pairs (2  NSCds bits) shall be extracted from the PRBS generator. The first extracted pair shall be modulated onto subcarrier 0 (so the bits are effectively ignored). The subsequent pairs are used to define the Xi and Yi components for the subcarriers i = 1 to NSCds – 1, as defined in Table 8-36/G.992.3 for C-REVERB symbols. For the subcarriers i = NSCds to 2  NSCds – 1, the Xi = Yi = 0.

While the ATU-C is in the C-MEDLEY state, the ATU-C and ATU-R may perform further training and SNR estimation.

The C-MEDLEY state shall be followed by the C-EXCHMARKER state.

##### 8.13.5.1.5	C-EXCHMARKER

See 8.13.5.1.5/G.992.3.

#### 8.13.5.2	ATU-R channel analysis

See 8.13.5.2/G.992.3.

### 8.13.6	Exchange phase

See 8.13.6/G.992.3.

#### 8.13.6.1	ATU-C exchange

See 8.13.6.1/G.992.3.

##### 8.13.6.1.1	C-MSG2

See 8.13.6.1.1/G.992.3.

##### 8.13.6.1.2	C-REVERB6

The C-REVERB6 state is of variable length. In this state, the ATU-C shall transmit a minimum of MAX(NSCds – NSCus – 10, 80) and a maximum of MAX(NSCds – NSCus + 1990, 2000) C-REVERB symbols.

This state is a filler state to allow the ATU-C to receive (and decode) the complete R-MSG2 message.

If the ATU-R has transmitted R-REVERB symbols during the R-EXCHMARKER state, the ATU-C shall continue to transmit C-REVERB symbols until after the ATU-R transitioning to the R-REVERB6 state. Within 80 to 2000 symbols after the ATU-R transitioning to the R-REVERB6 state, the ATU-C shall transition to the next state.

If the ATU-R has transmitted R-SEGUE symbols during the R-EXCHMARKER state, the ATU-C shall continue to transmit C-REVERB symbols until after the ATU-R transitioning to the R-REVERB7 state. Within 80 to 2000 symbols after the ATU-R transitioning to the R-REVERB7 state, the ATU-C shall transition to the next state.

The C-REVERB6 state shall be followed by the C-SEGUE3 state.

##### 8.13.6.1.3	C-SEGUE3

See 8.13.6.1.3/G.992.3.

##### 8.13.6.1.4	C-PARAMS

See 8.13.6.1.4/G.992.3, with changing Table 8-14 as follows:

Table 8-14/G.992.5 – C-PARAMS message and crc length

| Part of message | Length (bits or symbols) |
| --- | --- |
| Npmd | 96 + 32  NSCus |
| Npms | 224 |
| Ntps | 0 |
| Nmsg | 320 + 32  NSCus |
| CRC | 16 |
| LEN_C-PARAMS (state length in symbols) | 336+32×NSCus2×NSCC−PARAMS |
| NOTE – x denotes rounding to the higher integer. |  |

##### 8.13.6.1.5	C-REVERB7

See 8.13.6.1.5/G.992.3.

##### 8.13.6.1.6	C-SEGUE4

See 8.13.6.1.6/G.992.3.

#### 8.13.6.2	ATU-R exchange

See 8.13.6.2/G.992.3.

##### 8.13.6.2.1	R-MSGS2

The R-MSG2 state is of fixed length. In the R-MSG2 state, the ATU-R shall transmit NSCds + 16 R-REVERB or R-SEGUE symbols to modulate the R-MSG2 message and crc.

The R-MSG2 message, m, is defined by:

m = {mNSCds – 1, … , m0}

The bit mi shall be set to 1 to indicate that the ATU-C shall use subcarrier index i to modulate the C-PARAMS message. The bit mi shall be set to 0 to indicate that the ATU-C shall not use subcarrier index i to modulate the C-PARAMS message. At least 4 subcarriers shall be used for modulation of the C-PARAMS message. The C-PARAM message will be transmitted at about 8 kbit/s times the number of subcarriers used for modulation of the message.

If the ATU-R has set the R-MSG-FMT message bit FMT-C-PILOT to 1, then the ATU-C modulates the C-TREF pilot tone with a fixed constellation point. In this case, the ATU-R shall not use the C-TREF pilot tone for modulation of the C-PARAMS message.

The bits m0–mNSCds – 1 shall be transmitted in NSCds symbol periods (m0 first and mNSCds – 1 last). A zero bit shall be transmitted as an R-REVERB symbol. A one bit shall be transmitted as an R-SEGUE symbol.

After the R-MSG2 message has been transmitted, a crc shall be appended to the message. The 16 crc bits shall be computed from the NSCds message m bits in the same way as the crc bits are calculated for the C-MSG-PCB message.

The 16 bits c0-c15 shall be transmitted in 16 symbol periods (c0 first and c15 last) using the same modulation as used to transmit the message m.

If the ATU-R has transmitted R-REVERB symbols during the R-EXCHMARKER state, the R-MSG2 state shall be followed by the R-REVERB6 state. If the ATU-R has transmitted R-SEGUE symbols during the R-EXCHMARKER state, the R-MSG2 state shall be followed by the R-REVERB7 state.

##### 8.13.6.2.2	R-REVERB6

See 8.13.6.2.2/G.992.3.

##### 8.13.6.2.3	R-SEGUE3

See 8.13.6.2.3/G.992.3.

##### 8.13.6.2.4	R-PARAMS

See 8.13.6.2.4/G.992.3, changing Table 8-40/G.992.3 with Table 8-15:

Table 8-15/G.992.5 – R-PARAMS message and crc length

| Part of message | Length (bits or symbols) |
| --- | --- |
| Npmd | 96 + 32  NSCds |
| Npms | 224 |
| Ntps | 0 |
| Nmsg | 320 + 32  NSCds |
| CRC | 16 |
| LEN_R-PARAMS (state length in symbols) | 336+32×NSCds2×NSCR−PARAMS |
| NOTE – x denotes rounding to the higher integer. |  |

##### 8.13.6.2.5	R-REVERB7

See 8.13.6.2.5/G.992.3.

##### 8.13.6.2.6	R-SEGUE4

See 8.13.6.2.6/G.992.3.

### 8.13.7	Timing diagram of the initialization procedures

Figure 8-5 shows the timing diagram of the first part of the initialization procedures, from the G.994.1 phase up to the start of the Channel Analysis phase. Figures 8-6 to 8-9 show the second part of the Initialization procedures, from the end of the Channel Analysis Phase up to Showtime. These four timing diagrams represent the four cases resulting from whether the C-PARAMS and/or R-PARAMS states are included or not.

Figure 8-5/G.992.5 – Timing diagram of the initialization procedure (part 1)

Figure 8-6/G.992.5 – Timing diagram of the initialization procedure (part 2) 
with C-PARAMS and with R-PARAMS states

Figure 8-7/G.992.5 – Timing diagram of the initialization procedure (part 2) 
without C-PARAMS and without R-PARAMS states

Figure 8-8/G.992.5 – Timing diagram of the initialization procedure (part 2) 
with C-PARAMS and without R-PARAMS states

Figure 8-9/G.992.5 – Timing diagram of the initialization procedure (part 2) 
without C-PARAMS and with R-PARAMS states

## 8.14	Short initialization procedures

A short initialization sequence is defined to allow the ATUs to quickly enter Showtime from a L3 power management state or as a fast recovery procedure from changing of line conditions during Showtime. The short initialization sequence shall be optional for both ATU-C and ATU-R (with indication in ITU-T Rec. G.994.1, see 8.13.2). If the short initialization sequence is supported, the ATU should also support unbalanced bitswap (i.e., type 3 On-Line Reconfiguration with restriction to change bi, gi and Lp only, see 9.4.1.1).

The state diagram of the short sequence shall be the same as the one shown in Figures 8-5 to 8-9, with the exception of the entry procedures which shall be as depicted in Figures 8-10 and 8-11. Figure 8-10 shows the entry procedure for an ATU-C initiated short initialization. The ATU-C shall keep transmitting 128 symbols of C-COMB1 followed by 256 symbols of silence (C-QUIET2) until either the ATU-R responds with R-COMB1 during one of the C-QUIET2 states or a vendor discretionary timeout C-T1 is reached. If the short initialization is used as a fast recovery procedure from showtime, the ATU-R should reply to the first transmission of the C-COMB initialization signal.

Figure 8-10/G.992.5 – Timing diagram of the entry into the short initialization procedure, ATU-C initiated

Figure 8-11 shows the entry procedure for an ATU-R initiated short initialization. The ATU-R shall keep transmitting 128 symbols of R-COMB1 followed by 256 symbols of silence (R-QUIET2) until either the ATU-C responds with C-COMB2 during one of the R-QUIET2 states or a vendor discretionary timeout R-T1 is reached. If the short initialization is used as a fast recovery procedure from showtime, the ATU-C should reply to the first transmission of the R-COMB initialization signal.

Figure 8-11/G.992.5 – Timing diagram of the entry into the short initialization procedure, ATU-R initiated

The short initialization procedure may be used for the link state transition from the L3 state to the L0 state (see 9.5.3/G.992.3). Fast error recovery (during the L0 or L2 link state) is through the short initialization procedure. At the start of the short initialization procedure, the ADSL link state shall be changed to the L3 state. When the ATU reaches the Showtime state through the short initialization procedure, the ADSL link shall be in the L0 state (see Figure 9-5/G.992.3).

The short initialization procedure should be completed within 3 s. However, to meet this requirement, proper time budget balancing between ATU-C and ATU-R is required. Table 8-16 lists recommended time budgets for the variable portions of each ATU initialization sequence. Figures 8-12 and 8-13 show the recommended timing diagram for the short initialization procedure.

| Table 8-16/G.992.5 – Recommended duration for variable portions of the initialization sequence |  |  |
| --- | --- | --- |
| ATU state | Recommended duration (symbols) | Note |
| C-MSG-PCB | =96 | No C-BLACKOUT bits included (last previous exchanged BLACKOUT bits remain valid) |
| R-MSG-PCB | =144 | No R-BLACKOUT bits included (last previous exchanged BLACKOUT bits remain valid) |
| R-REVERB1 | =272 |  |
| R-QUIET4 | =0 | ATU-C hybrid fine tuning state is skipped |
| C-TREF1 | 1024 | Faster upstream channel estimation, less precise timing and no ATU-R hybrid fine tuning |
| R-QUIET5 | =1024 |  |
| C-REVERB3 | =512  64 | Faster downstream channel estimation and equalizer training |
| C-REVERB4 | =256 |  |
| C-MEDLEY | 1024 | Less accurate SNR estimation |
| R-MEDLEY | 1024 | Less accurate SNR estimation |
| C-REVERB6 | 120 | Limit through faster and simpler bit allocation algorithm |
| R-REVERB6 | 120 | Limit through faster and simpler bit allocation algorithm |

Figure 8-12/G.992.5 – Timing diagram of the short initialization procedure (part 1)

Figure 8-13/G.992.5 – Timing diagram of the short initialization procedure (part 2)

## 8.15	Loop diagnostics mode procedures

See 8.15/G.992.3.

### 8.15.1	Overview

See 8.15.1/G.992.3.

### 8.15.2	Channel discovery phase

See 8.15.2/G.992.3.

#### 8.15.2.1	ATU-C channel discovery phase

The sequence of states in the Loop Diagnostics Mode shall be the same as for the Initialization sequence (defined in 8.13.3.1). Each state shall have fixed duration in Loop Diagnostics Mode, as shown in the Loop Diagnostics Mode timing diagram in Figure 8-14.

The signals transmitted during each of the states in the Loop Diagnostics Mode shall be the same as for the Initialization sequence (defined in 8.13.3.1).

The states C-ICOMB1 and C-LINEPROBE states and the C-BLACKOUT bits shall be included during an Initialization in Loop Diagnostics Mode.

The C-MSG-FMT message shall be as defined in Table 8-17.

Table 8-17/G.992.5 – Bit definition for the C-MSG-FMT message

| Bit index | Parameter | Definition |
| --- | --- | --- |
| 9..0 |  | Reserved, set to 0 |
| 10 | FMT-C-MEDLEYPRBS | See Table 8-7 |
| 15..11 |  | Reserved, set to 0 |

The C-MSG-PCB message shall be as defined in Table 8- 18.

Table 8-18/G.992.5 – Bit definition for the C-MSG-PCB message

| Bit index | Parameter | Definition |
| --- | --- | --- |
| 5..0 | C-MIN_PCB_DS | See Table 8-27/G.992.3 |
| 11..6 | C-MIN_PCB_US | See Table 8-27/G.992.3 |
| 13..12 | HOOK_STATUS | See Table 8-27/G.992.3 |
| 15..14 |  | Reserved, set to 0 |
| NSCus + 15..16 | C-BLACKOUT | See Table 8-27/G.992.3 |
| NSCus + 23..NSCus + 16 | Pass/Fail | Success or Failure Cause indication of last previous initialization |
| NSCus + 31..NSCus + 24 | Last_TX_State | Last transmitted state of last previous initialization |

The Pass/Fail bits shall contain a Success or Failure Cause indication. The possible indications and their coding shall be as defined in Table 8-19. If the initialization in loop diagnostics mode is immediately following the ATU-C power-up, information about the last previous initialization may not be available. In that case, a successful last previous initialization shall be indicated.

Table 8-19/G.992.5 – Success and Failure Cause indications

| Value (higher bit index left) | Definition |
| --- | --- |
| 1111 1111 | Successful |
| 0001 0001 | Failed – Insufficient capacity |
| 0010 0010 | Failed – crc error in one of the received messages |
| 0100 0100 | Failed – Time out exceeded |
| 1000 1000 | Failed – Unexpected received message content |
| 0000 0000 | Failed – Cause unknown |
| Other | Reserved |

The Last_TX_State bits shall contain the index of the last ATU-C state that was successful transmitted during the last previous initialization. The index of the ATU-C state shall be represented by an 8-bit integer value from 0 (G.994.1 phase) and 1 (C-QUIET1) to 31 (C-SEGUE4) and 32 (C-SHOWTIME). The states shall be numbered in the order transmitted in time, as shown in the timing diagrams in Figures 8-14 and 8-15. The states that can be optionally omitted shall also be counted when calculating the index of a state. For example, the index of C-QUIET3 shall always be 7 regardless of whether the C-ICOMB1 and C-LINEPROBE states are included or not. In case the first octet of C-MSG-PCB indicates a successful initialization, this second octet shall encode the index of the last state, i.e. the index of C-SHOWTIME.

An addition of a crc and the bit transmission order for the C-MSG-FMT and C-MSG-PCB messages shall be as defined for the Initialization sequence in 8.13.3.1. However, the message and crc bits shall be transmitted with 8 symbols per bit modulation, where a zero bit shall be transmitted as 8 consecutive C-COMB symbols and a one bit shall be transmitted as 8 consecutive C-ICOMB symbols. This will make the transmission more robust against misdetection of the time marker transitions that precede these messages.

#### 8.15.2.2	ATU-R channel discovery phase

The sequence of states in the Loop Diagnostics Mode shall be the same as for the Initialization sequence (defined in 8.13.3.2). Each state shall have fixed duration in Loop Diagnostics Mode, as shown in the Loop Diagnostics Mode timing diagram in Figure 8-14.

The signals transmitted during each of the states in the Loop Diagnostics Mode shall be the same as for the Initialization sequence (defined in 8.13.3.2).

The states R-ICOMB1 and R-LINEPROBE states and the R-BLACKOUT bits shall be included during an Initialization in Loop Diagnostics Mode.

The R-MSG-FMT message shall be as defined in Table 8-20.

Table 8-20/G.992.5 – Bit definition for the R-MSG-FMT message

| Bit index | Parameter | Definition |
| --- | --- | --- |
| 7..0 |  | Reserved, set to 0 |
| 8 | FMT-C-TREF2 | See Table 8-31/G.992.3 |
| 9 | FMT-C-PILOT | See Table 8-31/G.992.3 |
| 10 | FMT-C-MEDLEYPRBS | See Table 8-31/G.992.3 |
| 15..11 |  | Reserved, set to 0 |

The R-MSG-PCB message shall be as defined in Table 8-21.

Table 8-21/G.992.5 – Bit definition for the R-MSG-PCB message

| Bit index | Parameter | Definition |
| --- | --- | --- |
| 5..0 | R-MIN_PCB_DS | See Table 8-32/G.992.3 |
| 11..6 | R-MIN_PCB_US | See Table 8-32/G.992.3 |
| 13..12 | HOOK_STATUS | See Table 8-32/G.992.3 |
| 15..14 |  | Reserved, set to 0 |
| 26..16 | C-PILOT | See Table 8-32/G.992.3 |
| 31..27 |  | Reserved, set to 0 |
| 31 + NSCds..32 | R-BLACKOUT | See Table 8-32/G.992.3 |
| 39 + NSCds..32 + NSCds | Pass/Fail | Success or Failure Cause indication of last previous initialization |
| 47 + NSCds..40 + NSCds | Last_TX_State | Last transmitted state of last previous initialization |

The Pass/Fail bits shall contain a Success or Failure Cause indication. The possible indications and their coding shall be as defined for the ATU-C in Table 8-19. If the initialization in loop diagnostics mode is immediately following the ATU-R power-up or self test, information about the last previous initialization may not be available. In that case, a successful last previous initialization shall be indicated.

The Last_TX_State bits shall contain the index of the last ATU-R state that was successfully transmitted during the last previous initialization. The index of the ATU-R state shall be represented by an 8-bit integer value from 0 (G.994.1 phase) and 1 (R-QUIET1) to 30 (R-SEGUE4) and 31 (R-SHOWTIME). The states shall be numbered in the order transmitted in time, as shown in the timing diagrams in Figures 8-14 and 8-15. The states that can be optionally omitted shall also be counted when calculating the index of a state. For example, the index of R-QUIET3 shall always be 7 regardless of whether the R-ICOMB1 and R-LINEPROBE states are included or not. In case the first octet of the C-MSG-PCB message indicates a successful initialization, this second octet shall encode the index of the last state, i.e., the index of R-SHOWTIME.

The addition of a 16-bit crc and the bit transmission order for the R-MSG-FMT and R-MSG-PCB messages shall be as defined for the Initialization sequence in 8.13.3.2. However, the bits shall be transmitted with 8 symbols per bit modulation, where a zero bit shall be transmitted as 8 consecutive R-COMB symbols and a one bit shall be transmitted as 8 consecutive R-ICOMB symbols. This will make the transmission more robust against misdetection of the time marker transitions that precede these messages.

### 8.15.3	Transceiver training phase

See 8.15.3/G.992.3.

### 8.15.4	Channel analysis phase

See 8.15.4/G.992.3.

### 8.15.5	Exchange phase

See 8.15.5/G.992.3.

#### 8.15.5.1	ATU-C exchange phase

See 8.15.5.1/G.992.3.

#### 8.15.5.2	ATU-R exchange phase

See 8.15.5.2/G.992.3.

##### 8.15.5.2.1	Channel information bearing messages

In the loop diagnostics mode, the ATU-R shall send (1 + NSCds/32) messages to the ATU-C: R-MSGx-LD, numbered from x = 1 to 1 + NSCds/32. These messages contain the downstream test parameters defined in 8.15.1.

The information fields of the different messages shall be as shown in Tables 8-22 to 8-26.

| Table 8-22/G.992.5 – Format of the R-MSG1-LD message |  |  |
| --- | --- | --- |
| Octet Nr [i] | Information | Format message bits [8  i + 7 to 8  i + 0] |
| 0 | Sequence number | [ 0000 0001 ] |
| 1 | Reserved | [ 0000 0000 ] |
| 2 | Hlin scale (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 3 | Hlin scale (msb) | [ xxxx xxxx ], bits 15 to 8 |
| 4 | LATN (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 5 | LATN (msb) | [ 0000 00xx ], bits 9 and 8 |
| 6 | SATN (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 7 | SATN (msb) | [ 0000 00xx ], bits 9 and 8 |
| 8 | SNRM (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 9 | SNRM (msb) | [ 0000 00xx ], bits 9 and 8 |
| 10 | ATTNDR (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 11 | ATTNDR | [ xxxx xxxx ], bits 15 to 8 |
| 12 | ATTNDR | [ xxxx xxxx ], bits 23 to 16 |
| 13 | ATTNDR (msb) | [ xxxx xxxx ], bits 31 to 24 |
| 14 | Far-end ACTATP (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 15 | Far-end ACTATP (msb) | [ ssss ssxx ], bits 9 and 8 |

Table 8-23/G.992.5 – Format of the Hlin(i) R-MSGx-LD message

| Octet Nr [i] | Information | Format message bits [8  i + 7 to 8  i + 0] |
| --- | --- | --- |
| 0 | Sequence number | [xxxx xxxx] (as 8-bit unsigned integer) |
| 1 | Reserved | [ 0000 0000 ] |
| 2 | Hlin(64  k) real (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 3 | Hlin(64  k) real (msb) | [ xxxx xxxx ], bits 15 to 8 |
| 4 | Hlin(64  k) imag (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 5 | Hlin(64  k) imag (msb) | [ xxxx xxxx ], bits 15 to 8 |
| .. | .. | .. |
| 254 | Hlin(64  k + 63) real (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 255 | Hlin(64  k + 63) real (msb) | [ xxxx xxxx ], bits 15 to 8 |
| 256 | Hlin(64  k + 63) imag (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 257 | Hlin(64  k + 63) imag (msb) | [ xxxx xxxx ], bits 15 to 8 |
| NOTE – For each of the values k = 0 to NSCds/64 – 1, a single R-MSGx-LD message shall be transmitted, with sequence number x = 2 + k. |  |  |

Table 8-24/G.992.5 – Format of the Hlog(i) R-MSGx-LD message

| Octet Nr [i] | Information | Format |
| --- | --- | --- |
| 0 | Sequence number | [xxxx xxxx] (as 8-bit unsigned integer) |
| 1 | Reserved | [ 0000 0000 ] |
| 2 | Hlog(128  k) (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 3 | Hlog(128  k) (msb) | [ 0000 00xx ], bits 9 and 8 |
| .. | .. | ….. |
| 256 | Hlog(128  k + 127) (lsb) | [ xxxx xxxx ], bits 7 to 0 |
| 257 | Hlog(128  k + 127) (msb) | [ 0000 00xx ], bits 9 and 8 |
| NOTE – For each of the values k = 0 to NSCds/128 – 1, a single R-MSGx-LD message shall be transmitted, with sequence number x = NSCds/64 + 2 + k. |  |  |

Table 8-25/G.992.5 – Format of the QLN(i) R-MSGx-LD message

| Octet Nr [i] | Information | Format message bits [8  i + 7 to 8  i + 0] |
| --- | --- | --- |
| 0 | Sequence number | [xxxx xxxx] (as 8-bit unsigned integer) |
| 1 | Reserved | [ 0000 0000 ] |
| 2 | QLN(256  k) | [ xxxx xxxx ], bits 7 to 0 |
| .. | .. | .. |
| 257 | QLN(256  k + 255) | [ xxxx xxxx ], bits 7 to 0 |
| NOTE – For each of the values k = 0 to NSCds/256 – 1, a single R-MSGx-LD message shall be transmitted, with sequence number x = 3  NSCds/128 + 2 + k. |  |  |

Table 8-26/G.992.5 – Format of the SNR(i) R-MSGx-LD message

| Octet Nr [i] | Information | Format message bits [8  i + 7 to 8  i + 0] |
| --- | --- | --- |
| 0 | Sequence number | [xxxx xxxx] (as 8-bit unsigned integer) |
| 1 | Reserved | [ 0000 0000 ] |
| 2 | SNR(256  k) | [ xxxx xxxx ], bits 7 to 0 |
| .. | .. | .. |
| 257 | SNR(256  k + 255) | [ xxxx xxxx ], bit 7 to 0 |
| NOTE – For each of the values k = 0 to NSCds/256 – 1, a single R-MSGx-LD message shall be transmitted, with sequence number x = 7  NSCds/256 + 2 + k. |  |  |

The messages shall be transmitted in order of ascending octet number (i.e., the sequence number shall be transmitted first) and each octet shall be transmitted LSB first.

The addition of a 16-bit crc and the bit transmission order for the R-MSGx-LD messages shall be as defined for the Initialization sequence in 8.13. However, the message and crc bits shall be transmitted with 8 symbols per bit modulation, where a zero bit shall be transmitted as eight consecutive R-REVERB symbols and a one bit shall be transmitted as eight consecutive R-SEGUE symbols. The resulting state duration (needed to transmit the message and crc) is shown in Table 8-27.

Table 8-27/G.992.5 – ATU-R loop diagnostics state durations

| State | Duration (symbols) |
| --- | --- |
| R-MSG1-LD | 1152 |
| R-MSGx-LD with x > 1 | 16640 |

The resulting number of symbols needed to transmit each of the messages and crc is shown in the Loop Diagnostics timing diagrams in Figures 8-14 and 8-15.

##### 8.15.5.2.2	Message flow, acknowledgement and retransmission

See 8.15.2.2/G.992.3.

### 8.15.6	Timing diagram of the loop diagnostics procedures

Figure 8-14/G.992.5 – Loop diagnostics timing diagram (part 1)

Figure 8-15/G.992.5 – Loop diagnostics timing diagram (part 2)

## 8.16	On-line reconfiguration of the PMD function

See 8.16/G.992.3.

## 8.17	Power management in the PMD function

See 8.17/G.992.3.

# 9		Management Protocol Specific Transmission Convergence (MPS-TC) functions

See clause 9/G.992.3.

## 9.1	Transport functions

See 9.1/G.992.3.

## 9.2	Additional functions

See 9.2/G.992.3.

## 9.3	Block interface signals and primitives

See 9.3/G.992.3.

## 9.4	Management plane procedures

See 9.4/G.992.3.

### 9.4.1	Commands

See 9.4.1/G.992.3.

#### 9.4.1.1	On-line reconfiguration command

The on-line reconfiguration commands shall be used to control certain on-line dynamic behaviour defined in this clause. Additional information is provided on this dynamic behaviour in clause 10. On-line reconfiguration commands may be initiated by either ATU as shown in Table 9-1. However, the initiator is only provided with means to effect changes in its receiver and the corresponding transmitter. The responding ATU may use the on-line reconfiguration commands shown in Table 9-2 or may positively acknowledge the initiator’s request by transmitting a line signal corresponding to the PMD.Synchflag primitive. The on-line reconfiguration commands shall consist of multiple octets. The first octet shall be the on-line reconfiguration command designator shown in Table 9-2/G.992.3. The remaining octets shall be as shown in Tables 9-1, 9-2 and 9-3. The octets shall be sent using the format described in 7.8.2.3/G.992.3  and using the protocol described in 7.8.2.4.

Table 9-1/G.992.5 – On-line reconfiguration commands transmitted
by the initiating receiver

| Message length (Octets) | Element name (Command) |
| --- | --- |
| 4+4×Nf | 0416 Request Type 1 followed by: 2 octets for the number of subcarriers Nf 4  Nf octets describing subcarrier parameter field for each subcarrier |
| 4+2×NLP+NBC+4×Nf | 0516 Request Type 2 followed by: 2  NLP octets containing new Lp values for the NLP enabled latency paths, NBC octets containing new Bp, n values for the NBC enabled frame bearers, 2 octets for the number of carriers Nf 4  Nf octets describing subcarrier parameter field for each subcarrier |
| 4+2×NLP+NBC+4×Nf | 0616 Request Type 3 followed by: 2  NLP octets containing new Lp values for the NLP enabled latency paths, NBC octets containing new Bp, n values for the NBC enabled frame bearers, 2 octets for the number of carriers Nf 4  Nf octets describing subcarrier parameter field for each subcarrier |
|  | All other octet values are reserved by the ITU-T. |

Table 9-2/G.992.5 – On-line reconfiguration commands transmitted
by the responding transmitter

| Message length (Octets) | Element name (Command) |
| --- | --- |
| 3 | 8116 Defer Type 1 Request followed by: 1 octet for reason code |
| 3 | 8216 Reject Type 2 Request followed by: 1 octet for reason code |
| 3 | 8316 Reject Type 3 Request followed by: 1 octet for reason code |
|  | All other octet values are reserved by the ITU-T. |

An ATU may request only changes in its receiver operation. Changes may be requested concurrently by both ATUs; each transaction shall follow the procedures described in this clause. An ATU-R shall not initiate an OLR command if it has transmitted an L2 Grant command and is awaiting a response.

A subcarrier parameter field contains 4 octets formatted as [ 0000 0ccc cccc cccc gggg gggg gggg bbbb ]. The carrier index i (11 bits), the gi (12 bits) and the bi (4 bits). The carrier index shall be the three least significant bits of the first octet and the second octet of the subcarrier field. The least significant bits of the carrier index i shall be contained in the second octet. The gi shall be contained in the third octet and the four most significant bits of the fourth octet. The least significant bits of gi shall be contained in the fourth octet. The bi shall be the least significant 4 bits of the fourth octet.

Type 1 and Type 2 shall be sent such that the PMD parameter L is unchanged. If an ATU implements the optional short PMD initialization sequence, then the ATU should also implement Type 3 OLR operations changing bi, gi and Lp.

Reason codes associated with the OLR commands are shown in Table 9-3.

Table 9-3/G.992.5 – Reason codes for OLR commands

| Reason | Octet value | Applicable to defer Type 1 | Applicable to reject Type 2 | Applicable to reject Type 3 |
| --- | --- | --- | --- | --- |
| Busy | 0116 | X | X | X |
| Invalid parameters | 0216 | X | X | X |
| Not enabled | 0316 |  | X | X |
| Not supported | 0416 |  | X | X |

Upon transmitting an on-line reconfiguration command, the initiator shall await a response to the command, either an on-line reconfiguration command for defer or reject or the line signal corresponding to the PMD.Synchflag primitive. If the response is not received within the timeout of the high priority overhead messages displayed in Table 7-17/G.992.3, the initiator shall abandon the current on-line reconfiguration command. A new command may be initiated immediately, including an identical request.

Upon receipt of an on-line reconfiguration command, the responder shall respond with either an on-line reconfiguration command for defer or reject or the line signal corresponding to the PMD.Synchflag primitive. In the case of sending the line signal corresponding to the PMD.Synchflag primitive, the ATU shall reconfigure the effected PMD, PMS-TC, and TPS-TC functions as described in the reconfiguration clauses describing those functions. In the case of defer or reject, the receiver shall supply a reason code from the following: 0116 for busy, 0216 for invalid parameters, 0316 for not enabled, and 0416 for not supported. The reason codes 0116 and 0216 shall be the only codes used in an on-line reconfiguration command for defer type 1 request.

Upon receipt of a line signal corresponding to the PMD.Synchflag primitive, the initiator shall reconfigure the effected PMD, PMS-TC, and TPS-TC functions as described in the reconfiguration clauses describing those functions. If an on-line reconfiguration command for defer or reject is received, the initiator shall abandon the current on-line reconfiguration command. A new command may be initiated immediately, including an identical request.

#### 9.4.1.2	eoc commands

See 9.4.1.2/G.992.3.

#### 9.4.1.3	Time commands

See 9.4.1.3/G.992.3.

#### 9.4.1.4	Inventory command

See 9.4.1.4/G.992.3.

#### 9.4.1.5	Control value read commands

See 9.4.1.5/G.992.3.

#### 9.4.1.6	Management counter read commands

See 9.4.1.6/G.992.3.

#### 9.4.1.7	Power management commands

The power management command shall be used to propose power management transitions from one link state to another as described in the Power Management clause. The power management command may be initiated by either ATU as prescribed in the Power Management clause as shown in Table 9-4. The responses shall be using the command shown in Table 9-5. The power management command is variable in length. The first octet shall be power management command designator shown in Table 9-3/G.992.3. The remaining octets shall be as shown in Table 9-4. The power management response commands are variable in length. The first octet shall be power management command designator shown in Table 9-3/G.992.3. The second shall be as shown in Table 9-5. The octets shall be sent using the format described in 7.8.2.3/G.992.3 and using the protocol described in 7.8.2.4/G.992.3.

Table 9-4/G.992.5 – Power management commands transmitted
by the initiating ATU

| Message length (octets) | Element name (Command) |
| --- | --- |
| 3 | 0116 Simple Request followed by: 1 octet for the new proposed link state |
| 4 + 4  NLP | 0216 L2 Request followed by: 1 octet for minimum PCBds value (dB) 1 octet for maximum PCBds value (dB); 2  NLP octets containing maximum Lp values for the NLP enabled latency paths; 2  NLP octets containing minimum Lp values for the NLP enabled latency paths. |
| 3 | 0316 L2 Trim followed by: 1 octet for the proposed new value of PCBds (dB) |
|  | All other octet values are reserved by the ITU-T. |

Table 9-5/G.992.5 – Power management command transmitted
by the responding ATU

| Message length (octets) | Element name (Command) |
| --- | --- |
| 2 | 8016 Grant |
| 3 | 8116 Reject followed by: 1 octet for reason code |
| 3 | 8316 L2 Reject followed by: 1 octet for reason code |
| 3 | 8416 L2 Trim Grant followed by: 1 octet containing the exit symbol PCBds value |
| 3 | 8516 L2 Trim Reject followed by: 1 octet for reason code |
| 7+2×NLP+4×Nf | 8616 L2 Grant followed by: 2  NLP octets containing new Lp values for the NLP enabled latency paths, 1 octet containing the actual PCBds value 1 octet containing the exit symbol PCBds value, 1 octet containing the exit symbol bi/gi table flag, 2 octets for the number of carriers Nf 4  Nf octets describing subcarrier parameter field for each subcarrier |
|  | All other octet values are reserved by the ITU-T. |

In the L2 Request, L2 Grant, and L2 Trim Request, and L2 Trim Grant messages, power cutback values shall be expressed as an absolute power cutback in the range of 0 to 40 dB in steps of 1 dB. The cutback is defined in terms of PCBds. The minimum and maximum requested values are defined in absolute terms and not relative to the current PCBds value. Values not inclusively within the range of the PCBds determined during initialization to 40 dB shall not be encoded. It is intended that up to 40 dB of absolute power cutback can be performed for the L2 link state using the PCBds control parameter and that the gain values can be used to additionally adjust the gain per carrier as required. The extra power cutback applied during the L2 state (i.e., PCBds(L2) – PCBds(init)) shall be applied as a flat cutback (i.e., each subcarrier is reduced by the same amount) relative to the L0 transmit PSD level (i.e., relative to the REFPSDds(init) transmit PSD level, adjusted by the ceiled_log_tssi values as determined and applied during transceiver training).

Reason codes associated with the power management commands are shown in Table 9-6.

Table 9-6/G.992.5 – Reason codes for power management commands

| Reason | Octet value | Applicable to Reject | Applicable to L2 Reject | Applicable to L2 Trim Reject |
| --- | --- | --- | --- | --- |
| Busy | 0116 | X | X |  |
| Invalid | 0216 | X | X | X |
| State not desired | 0316 | X |  |  |
| Infeasible parameters | 0416 |  | X | X |

##### 9.4.1.7.1	Simple request by ATU-R

See 9.4.1.7.1/G.992.3.

##### 9.4.1.7.2	Simple request by ATU-C

See 9.4.1.7.2/G.992.3.

##### 9.4.1.7.3	L2 Request by ATU-C

See 9.4.1.7.3/G.992.3.

##### 9.4.1.7.4	L2 Trim Request by ATU-C

See 9.4.1.7.4/G.992.3.

#### 9.4.1.8	Clear eoc messages

See 9.4.1.8/G.992.3.

#### 9.4.1.9	Non-standard facility overhead commands

See 9.4.1.9/G.992.3.

#### 9.4.1.10	Test parameter messages

The PMD test parameters read commands shall be used to access the value of certain PMD test parameters maintained by the far ATU in accordance with the description of the PMD function. The local parameter values shall be retrieved as described in this clause. The PMD test parameter read command may be initiated by either ATU as shown in Table 9-7. The responses shall be using the command shown in Table 9-8. The PMD test parameter read command shall consist of two to six octets. The first octet shall be the PMD test parameter command designator shown in Table 9-4. The remaining octets shall be as shown in Table 9-7. The PMD test parameter read response command shall be multiple octets. The first octet shall be PMD test parameter read command designator shown in Table 9-4/G.992.3. The remaining octets shall be as shown in Table 9-8. The octets shall be sent using the format described in 7.8.2.3/G.992.3  and using the protocol described in 7.8.2.4/G.992.3.

Table 9-7/G.992.5 – PMD test parameter read commands transmitted by the initiator

| Message length (octets) | Element name (Command) |
| --- | --- |
| 3 | 0116 Single Read followed by: 1 octet describing the test parameter ID |
| 2 | 0316 Next Multiple Read |
| 4 | 0416 Multiple Read Block followed by: 2 octets describing the subcarrier index |
| 6 | 0516 Block Read followed by 2 octets describing the start sub-carrier index 2 octets describing the stop sub-carrier index |
|  | All other octet values are reserved by the ITU-T |

Table 9-8/G.992.5 – PMD test parameter read command transmitted by the responder

| Message length (octets) | Element name (Command) |
| --- | --- |
| Variable (see Note) | 8116 followed by: octets for the test parameter arranged for the single read format |
| 12 | 8216 followed by: octets for the test parameters arranged for the multiple read format |
| 2 | 8016 NACK |
| Variable (see Note) | 8416 followed by octets for the test parameter arranged for the block read format |
|  | All other octet values are reserved by the ITU-T. |
| NOTE – Variable length equals 2 plus length shown in Table 9-9. |  |

| Table 9-9/G.992.5 – PMD test parameter ID values |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test parameter ID | Test parameter name | Length for single read | Length for multiple read | Length for Multiple Read |
| 0116 | Channel Transfer Function Hlog(f) per subcarrier | 2 + NSC  2 octets | 4 octets | 2 + (stop sub-carrier – start sub_carrier + 1)  2 octets |
| 0216 | Reserved by ITU-T |  |  |  |
| 0316 | Quiet Line Noise PSD QLN(f) per subcarrier | 2 + NSC octets | 3 octets | 2 + (stop sub-carrier – start sub_carrier + 1) octets |
| 0416 | Signal-to-noise ratio SNR(f) per subcarrier | 2 + NSC octets | 3 octets | 2 + (stop sub-carrier – start sub_carrier + 1) octets |
| 0516 | Reserved by ITU-T |  |  |  |
| 2116 | Line Attenuation LATN | 2 octets | N/A | N/a |
| 2216 | Signal Attenuation SATN | 2 octets | N/A | N/a |
| 2316 | Signal-to-Noise Margin SNRM | 2 octets | N/A | N/a |
| 2416 | Attainable Net Data Rate ATTNDR | 4 octets | N/A | N/a |
| 2516 | Near-end Actual Aggregate Transmit Power ACTATP | 2 octets | N/A | N/a |
| 2616 | Far-end Actual Aggregate Transmit Power ACTATP | 2 octets | N/A | N/a |

Upon receipt of one of the PMD test parameter read commands, the receiving ATU shall transmit the corresponding response message. If an unrecognized test parameter is requested, the response shall be a PMD test parameter command for NACK. The function of the receiving or transmitting ATUs is not otherwise affected.

The PMD test parameters are all derived according to the procedures in the PMD function clause of this Recommendation. Following initialization, the PMD shall maintain training test parameters until the overhead command for update test parameters is received.

The parameters are transferred in the order and format defined in Table 9-9. During a test parameter read command for single read, all information for the test parameter is transferred. If the test parameter is an aggregate parameter, only one value is transferred. If the test parameter has a value per subcarrier, then all values are transferred from subcarrier index #0 to subcarrier index #NSC – 1 in a single message. The format of the octets is as described in the PMD clause. Values that are formatted as multiple octets shall be inserted in the response message in most significant to least significant octet order.

During a test parameter read command for multiple read or next, information for all test parameters associated with a particular subcarrier is transferred. Aggregate test parameters are not transferred with the PMD test parameter read command for multiple read or next. The subcarrier used for a PMD test parameter read command for multiple read shall be the subcarrier contained within the command. This subcarrier index shall be saved. Each subsequent PMD test parameter command for next shall increment and use the saved subcarrier index. If the subcarrier index reaches NSC, the response shall be a PMD test parameter command for NACK. The per subcarrier values are inserted into the message according to the numeric order of the octets designators show in Table 9-9. The format of the octets is as described in PMD clause of this Recommendation. Values that are formatted as multiple octets shall be inserted in the response message in most significant to least significant octet order.

During a test parameter read command for a block read, information for all test parameters associated with the specified block of subcarriers is transferred. Aggregate test parameters are not transferred with the PMD test parameter block read command. If the test parameter has a value per subcarrier, then all values are transferred from subcarrier index #start subcarrier to subcarrier index #stop subcarrier in a single message. The format of the octets is as described in the PMD subclause. Values that are formatted as multiple octets shall be inserted in the response message most significant to least significant octet order.

In transferring the value of the channel transfer function Hlog(f), the measurement time shall be inserted into the message, followed by the value m (see 8.12.3.1/G.992.3). The measurement time is included only once in a PMD test parameter response for single read or block read. The measurement time is included in each response for multiple read or next multiple read.

In transferring the value of the quiet line noise QLN(f), the measurement time shall be inserted into the message, followed by the n value (see 8.12.3.2/G.992.3). The measurement time is included only once in a PMD test parameter response for single read or block read. The measurement time is included in each response for multiple read or next multiple read.

In transferring the value of the signal-to-noise ratio SNR(f), the measurement time shall be inserted into the message, followed by the SNR value (see 8.12.3.3/G.992.3). The measurement time is included only once in a PMD test parameter response for single read or block read. The measurement time is included in each response for multiple read or next multiple read.

The values for test parameters defined with fewer bits than shown in Table 9-9 shall be inserted into the message using the least significant bits of the two octets. Unused more significant bits shall be set to 0 for unsigned quantities and to the value of the sign bit for signed quantities.

##### 9.4.1.10.1	Single read command

See 9.4.1.10.1/G.992.3.

##### 9.4.4.10.2	Multiple read protocol with next

See 9.4.1.10.2/G.992.3.

##### 9.4.1.10.3	Block Read Command

See 9.4.1.10.3/G.992.3.

## 9.5	Power management

See 9.5/G.992.3.

# 10	Dynamic behaviour

See 10/G.992.3.

Annex A

Specific requirements for an ADSL system operating
in the frequency band above POTS

This annex defines those parameters of the ADSL system that have been left undefined in the body of this Recommendation because they are unique to an ADSL service that is frequency-division duplexed with POTS.

## A.1	ATU-C functional characteristics (pertains to clause 8)

### A.1.1	ATU-C control parameter settings

The ATU-C control parameter settings to be used in the parameterized parts of the main body and/or to be used in this annex are listed in Table A.1. Control parameters are defined in 8.5.

Table A.1/G.992.5 – ATU-C control parameter settings

| Parameter | Default setting | Characteristics |
| --- | --- | --- |
| NSCds | 512 |  |
| NOMPSDds | – 40 dBm/Hz | Settings may be changed relative to this value during G.994.1 phase; see 8.13.2. |
| MAXNOMPSDds | – 40 dBm/Hz | Settings may be changed relative to this value during G.994.1 phase; see 8.13.2. |
| MAXNOMATPds | 20.4 dBm | Settings may be changed relative to this value during G.994.1 phase; see 8.13.2. |

### A.1.2	ATU-C downstream transmit spectral mask for overlapped spectrum operation (supplements 8.10)

The passband is defined as the band from 25.875 to 2208 kHz and is the widest possible band used (i.e., for ADSL over POTS implemented with overlapped spectrum). Limits defined within the passband apply also to any narrower bands used.

Figure A.1 defines the limit spectral mask for the transmit signal. The low-frequency stopband is defined as frequencies below 25.875 kHz and includes the POTS band, the high-frequency stopband is defined as frequencies greater than 2208 kHz.

| Frequency (kHz) | PSD level (dBm/Hz) | MBW |
| --- | --- | --- |
| 0 | – 97.5 | 100 Hz |
| 4 | – 97.5 | 100 Hz |
| 4 | – 92.5 | 100 Hz |
| 10 | interpolated | 10 kHz |
| 25.875 | – 36.5 | 10 kHz |
| 1104 | – 36.5 | 10 kHz |
| 1622 | – 46.5 | 10 kHz |
| 2208 | – 47.8 | 10 kHz |
| 2500 | – 59.4 | 10 kHz |
| 3001.5 | – 80 | 10 kHz |
| 3175 | – 100 | 10 kHz |
| 12000 | – 100 | 10 kHz |

Additionally, the PSD mask shall be satisfying the following requirements:

| Frequency (kHz) | PSD level (dBm/Hz) | MBW |
| --- | --- | --- |
| 3750 | – 100 | 1 MHz |
| 4545 | – 110 | 1 MHz |
| 7225 | – 112 | 1 MHz |
| 12000 | – 112 | 1 MHz |

NOTE 1 – All PSD measurements are in 100 ; the POTS band total power measurement is in 600 .

NOTE 2 – The breakpoint frequencies and PSD values are exact; the indicated slopes are approximate. The breakpoints in the tables shall be connected by linear straight lines on a dB/log(f) plot.

NOTE 3 – MBW specifies the measurement bandwidth. The MBW specified for a certain breakpoint with frequency fi is applicable for all frequencies satisfying fi  f  fj, where fj is the frequency of the next specified breakpoint.

NOTE 4 – The power in a 1-MHz sliding window is measured in a 1-MHz bandwidth, starting at the measurement frequency, i.e., power in the [f, f  1 MHz] window shall conform to the specification at frequency f.

NOTE 5 – The step in the PSD mask at 4 kHz is to protect V.90 performance. Originally, the PSD mask continued the 21-dB/octave slope below 4 kHz hitting a floor of – 97.5 dBm/Hz at 3400 Hz. It was recognized that this might impact V.90 performance, and so the floor was extended to 4 kHz.

NOTE 6 – All PSD and power measurements shall be made at the U-C interface.

Figure A.1/G.992.5 – ATU-C transmitter PSD mask for overlapped spectrum operation

#### A.1.2.1	Passband PSD and response

There are three different PSD masks for the ATU-C transmit signal, depending on the type of signal sent. Across the whole passband, the transmit PSD level shall not exceed the maximum passband transmit PSD level, defined as:

–	NOMPSDds + 1 dB, for initialization signals up to and including the Channel Discovery Phase;

–	REFPSDds + 1 dB, during the remainder of initialization, starting with the Transceiver Training Phase;

–	MAXNOMPSDds – PCBds + 3.5 dB, during showtime.

The group delay variation over the passband shall not exceed 50 µs.

The maximum passband transmit PSD level allows for 1 dB of non-ideal transmit filter effects (e.g., passband ripple and transition band rolloff).

For spectrum management purposes, the ATU-C transmitter PSD template for overlapped spectrum operation is defined in Table A.2 (informative):

Table A.2/G.992.5 – ATU-C transmitter PSD template for
overlapped spectrum operation

| Frequency (kHz) | PSD level (dBm/Hz) |
| --- | --- |
| 0 | – 101 |
| 4 | – 101 |
| 4 | – 96 |
| 25.875 | – 40 |
| 1104 | – 40 |
| 1622 | – 50 |
| 2208 | – 51.3 |
| 2500 | – 62.9 |
| 3001.5 | – 83.5 |
| 3175 | – 100 |
| 3750 | – 100 |
| 4545 | – 110 |
| 7225 | – 112 |
| 12000 | – 112 |

#### A.1.2.2	Aggregate transmit power

There are three different PSD masks for the ATU-C transmit signal, depending on the type of signal sent (see A.1.2.1). In all cases,

–	the aggregate transmit power in the voiceband, measured at the U-C interface, and that is delivered to the Public Switched Telephone Network (PSTN) interface shall not exceed + 15 dBrn (see ITU-T Rec. G.996.1 [3] for method of measurement);

–	the aggregate transmit power across the whole passband, shall not exceed (MAXNOMATPds – PCBds) by more than 0.5 dB, in order to accommodate implementational tolerances, and shall not exceed 20.9 dBm.

–	the aggregate transmit power over the 0 to 12 MHz band, shall not exceed (MAXNOMATPds – PCBds) by more than 0.9 dB, in order to account for residual transmit power in the stopbands and implementational tolerances.

The power emitted by the ATU-C is limited by the requirements in this clause. Notwithstanding these requirements, it is assumed that the ADSL will comply with applicable national requirements on emission of electromagnetic energy.

For spectrum management purposes, the PSD template nominal passband aggregate transmit power is 20.4 dBm.

### A.1.3	ATU-C transmitter PSD mask for non-overlapped spectrum operation (supplements 8.10)

Figure A.2 defines the limit spectral mask for the ATU-C transmitted signal, which results in reduced NEXT into the ADSL upstream band, relative to the mask in A.1.2. Adherence to this mask will in many cases result in improved upstream performance of the other ADSL systems in the same or adjacent binder group, with the improvement dependent upon the other interferers. This mask differs from the mask in A.1.2 only in the band from 4 kHz to 138 kHz.

The passband is defined as the band from 138 to 2208 kHz. Limits defined within the passband apply also to any narrower bands used.

The low-frequency stopband is defined as frequencies below 138 kHz and includes the POTS band, the high-frequency stopband is defined as frequencies greater than 2208 kHz.

| Frequency (kHz) | PSD level (dBm/Hz) | MBW |
| --- | --- | --- |
| 0 | – 97.5 | 100 Hz |
| 4 | – 97.5 | 100 Hz |
| 4 | – 92.5 | 100 Hz |
| 10 | interpolated | 10 kHz |
| 80 | – 72.5 | 10 kHz |
| 138 | – 44.2 | 10 kHz |
| 138 | – 36.5 | 10 kHz |
| 1104 | – 36.5 | 10 kHz |
| 1622 | – 46.5 | 10 kHz |
| 2208 | – 47.8 | 10 kHz |
| 2500 | – 59.4 | 10 kHz |
| 3001.5 | – 80 | 10 kHz |
| 3175 | – 100 | 10 kHz |
| 12000 | – 100 | 10 kHz |

Additionally, the PSD mask shall be satisfying the following requirements:

| Frequency (kHz) | PSD level (dBm/Hz) | MBW |
| --- | --- | --- |
| 3750 | – 100 | 1 MHz |
| 4545 | – 110 | 1 MHz |
| 7225 | – 112 | 1 MHz |
| 12000 | – 112 | 1 MHz |

NOTE 1 – All PSD measurements are in 100 ; the POTS band total power measurement is in 600 .

NOTE 2 – The breakpoint frequencies and PSD values are exact; the indicated slopes are approximate. The breakpoints in the tables shall be connected by linear straight lines on a dB/log(f) plot.

NOTE 3 – MBW specifies the measurement bandwidth. The MBW specified for a certain breakpoint with frequency fi is applicable for all frequencies satisfying  fi  f  fj, where fj is the frequency of the next specified breakpoint.

NOTE 4 – The power in a 1-MHz sliding window is measured in a 1-MHz bandwidth, starting at the measurement frequency, i.e., power in the [f, f  1 MHz] window shall conform to the specification at frequency f.

NOTE 5 – The step in the PSD mask at 4 kHz is to protect V.90 performance. Originally, the PSD mask continued the 21-dB/octave slope below 4 kHz hitting a floor of –97.5 dBm/Hz at 3400 Hz. It was recognized that this might impact V.90 performance, and so the floor was extended to 4 kHz.

NOTE 6 – All PSD and power measurements shall be made at the U-C interface.

Figure A.2/G.992.5 – ATU-C transmitter PSD mask for non-overlapped spectrum operation

#### A.1.3.1	Passband PSD and response

See A.1.2.1. For spectrum management purposes, the PSD template for non-overlapped spectrum operation is defined in Table A.3 (informative):

Table A.3/G.992.5 – ATU-C transmitter PSD template
for non-overlapped spectrum operation

| Frequency (kHz) | PSD level (dBm/Hz) |
| --- | --- |
| 0 | – 101 |
| 4 | – 101 |
| 4 | – 96 |
| 80 | – 76 |
| 138 | – 47.7 |
| 138 | – 40 |
| 1104 | – 40 |
| 1622 | – 50 |
| 2208 | – 51.3 |
| 2500 | – 62.9 |
| 3001.5 | – 83.5 |
| 3175 | – 100 |
| 3750 | – 100 |
| 4545 | – 110 |
| 7225 | – 112 |
| 12000 | – 112 |

#### A.1.3.2	Aggregate transmit power

See A.1.2.2. In addition, for non-overlapped spectrum operation, the aggregate transmit power across the whole passband shall not exceed 20.4 dBm.

For spectrum management purposes, the PSD template nominal passband aggregate transmit power is 19.9 dBm.

## A.2	ATU-R functional characteristics (pertains to clause 8)

### A.2.1	ATU-R control parameter settings

The ATU-R control parameter settings to be used in the parameterized parts of the main body and/or to be used in this annex are listed in Table A.4. Control parameters are defined in 8.5.

Table A.4/G.992.5 – ATU-R control parameter settings

| Parameter | Default setting | Characteristics |
| --- | --- | --- |
| NSCus | 32 |  |
| NOMPSDus | –38 dBm/Hz | Settings may be changed relative to this value during G.994.1 phase; see 8.13.2. |
| MAXNOMPSDus | –38 dBm/Hz | Settings may be changed relative to this value during G.994.1 phase; see 8.13.2. |
| MAXNOMATPus | 12.5 dBm | Settings may be changed relative to this value during G.994.1 phase; see 8.13.2. |

### A.2.2	ATU-R upstream transmit spectral mask (supplements 8.10)

The passband is defined as the band from 25.875 to 138 kHz and is the widest possible band used. Limits defined within the passband apply also to any narrower bands used.

Figure A.3 defines the spectral mask for the transmit signal. The low-frequency stopband is defined as frequencies below 25.875 kHz and includes the POTS band (see also Figure A.1), the high-frequency stopband is defined as frequencies greater than 138 kHz.

| Frequency (kHz) | PSD level (dBm/Hz) | MBW |
| --- | --- | --- |
| 0 | – 97.5 | 100 Hz |
| 4 | – 97.5 | 100 Hz |
| 4 | – 92.5 | 100 Hz |
| 10 | interpolated | 10 kHz |
| 25.875 | – 34.5 | 10 kHz |
| 138 | – 34.5 | 10 kHz |
| 243 | – 93.2 | 10 kHz |
| 686 | – 100 | 10 kHz |
| 5275 | – 100 | 10 kHz |
| 12000 | – 100 | 10 kHz |

Additionally the PSD mask shall be satisfying following requirements:

| Frequency (kHz) | PSD level (dBm/Hz) | MBW |
| --- | --- | --- |
| 1411 | – 100 | 1 MHz |
| 1630 | – 110 | 1 MHz |
| 5275 | – 112 | 1 MHz |
| 12000 | – 112 | 1 MHz |

NOTE 1 – All PSD measurements are in 100 ; the POTS band total power measurement is in 600 .

NOTE 2 – The breakpoint frequencies and PSD values are exact; the indicated slopes are approximate. The breakpoints in the tables shall be connected by linear straight lines on a dB/log(f) plot.

NOTE 3 – MBW specifies the measurement bandwidth. The MBW specified for a certain breakpoint with frequency fi is applicable for all frequencies satisfying fi  f  fj, where fj is the frequency of the next specified breakpoint.

NOTE 4 – The power in a 1-MHz sliding window is measured in a 1-MHz bandwidth, starting at the measurement frequency, i.e., power in the [f, f  1 MHz] window shall conform to the specification at frequency f.

NOTE 5 – The step in the PSD mask at 4 kHz is to protect V.90 performance. Originally, the PSD mask continued the 21-dB/octave slope below 4 kHz hitting a floor of – 97.5 dBm/Hz at 3400 Hz. It was recognized that this might impact V.90 performance, and so the floor was extended to 4 kHz.

NOTE 6 – All PSD and power measurements shall be made at the U-R interface.

Figure A.3/G.992.5 – ATU-R transmitter PSD mask

#### A.2.2.1	Passband PSD and response

There are three different PSD masks for the ATU-R transmit signal, depending on the type of signal sent. Across the whole passband, the transmit PSD level shall not exceed the maximum passband PSD level, defined as:

–	NOMPSDus + 1 dB, for initialization signals up to and including the Channel Discovery Phase;

–	REFPSDus + 1 dB, during the remainder of initialization, starting with the Transceiver Training Phase;

–	MAXNOMPSDus – PCBus + 3.5 dB, during showtime.

The group delay variation over the passband shall not exceed 50 µs.

The maximum transmit PSD level allows for a 1 dB of non-ideal transmit filter effects (e.g. passband ripple and transition band rolloff).

For spectrum management purposes, the PSD template is defined in Table A.5 (informative):

Table A.5/G.992.5 – ATU-R transmitter PSD template

| Frequency (kHz) | PSD level (dBm/Hz) |
| --- | --- |
| 0 | – 101 |
| 4 | – 101 |
| 4 | – 96 |
| 25.875 | – 38 |
| 138 | – 38 |
| 229.6 | – 92.9 |
| 686 | – 100 |
| 1411 | – 100 |
| 1630 | – 110 |
| 5275 | – 112 |
| 12000 | – 112 |

#### A.2.2.2	Aggregate transmit power

There are three different PSD masks for the ATU-R transmit signal, depending on the type of signal sent (see A.2.2.1). In all cases:

–	the aggregate transmit power in the voiceband, measured at the U-R interface, and that is delivered to the Plain Old Telephone Service (POTS) interface shall not exceed + 15 dBrn (see ITU-T Rec. G.996.1 [3] for method of measurement);

–	the aggregate transmit power across the whole passband, shall not exceed (MAXNOMATPus – PCBus) by more than 0.5 dB, in order to accommodate implementational tolerances, and shall not exceed 13.0 dBm.

–	the aggregate transmit power over the 0 to 12 MHz band, shall not exceed (MAXNOMATPus – PCBus) by more than 0.8 dB, in order to account for residual transmit power in the stopbands and implementational tolerances.

The power emitted by the ATU-R is limited by the requirements in this clause. Notwithstanding these requirements, it is assumed that the ADSL will comply with applicable national requirements on emission of electromagnetic energy.

For spectrum management purposes, the PSD template nominal passband aggregate transmit power is 12.5 dBm.

## A.3	Initialization

For this annex, no additional requirements apply (relative to the main body of this Recommendation).

## A.4	Electrical characteristics

See A.4/G.992.3.

The G.992.3 requirements applying over a frequency band up to 1104 kHz shall be met over a frequency band up to 2208 kHz.

Annex B

Specific requirements for an ADSL system operating in the frequency band above ISDN as defined in ITU-T Rec. G.961 Appendices I and II

This annex defines those parameters of the ADSL system that have been left undefined in the body of this Recommendation because they are unique to an ADSL service that is frequency-division duplexed with ISDN-BA on the same digital subscriber line. The scope is to establish viable ways enabling the simultaneous deployment of ADSL and 160 kbit/s (2B  D) Basic Rate Access with the constraint to use existing transmission technologies as those specified in Appendices I and II/G.961 [1].

### B.1	ATU-C functional characteristics (pertains to clause 8)

### B.1.1	ATU-C control parameter settings

The ATU-C control parameter settings to be used in the parameterized parts of the main body and/or to be used in this annex are listed in Table B.1. Control parameters are defined in 8.5.

Table B.1/G.992.5 – ATU-C control parameter settings

| Parameter | Default setting | Characteristics |
| --- | --- | --- |
| NSCds | 512 |  |
| NOMPSDds | –40 dBm/Hz | Settings may be changed relative to this value during G.994.1 phase; see 8.13.2. |
| MAXNOMPSDds | –40 dBm/Hz | Settings may be changed relative to this value during G.994.1 phase; see 8.13.2. |
| MAXNOMATPds | 19.9 dBm | Settings may be changed relative to this value during G.994.1 phase; see 8.13.2. |

### B.1.2	ATU-C downstream transmit spectral mask for overlapped spectrum operation (supplements 8.10)

The passband is defined as the band from 120 kHz (see Figure B.1) to 2208 kHz and is the widest possible band used (i.e., for ADSL over ISDN implemented with overlapped spectrum). Limits defined within the passband apply also to any narrower bands used.

Figure B.1 defines the limit spectral mask for the transmit signal. The low-frequency stopband is the ISDN band and is defined as frequencies below 120 kHz (see Figure B.1); the high-frequency stopband is defined as frequencies greater than 2208 kHz.

| Frequency (kHz) | PSD level (dBm/Hz) | MBW |
| --- | --- | --- |
| 0 | – 90 | 10 kHz |
| 50 | – 90 | 10 kHz |
| 80 | – 81.8 | 10 kHz |
| 120 | – 36.5 | 10 kHz |
| 1104 | – 36.5 | 10 kHz |
| 1622 | – 46.5 | 10 kHz |
| 2208 | – 47.8 | 10 kHz |
| 2500 | – 59.4 | 10 kHz |
| 3001.5 | – 80 | 10 kHz |
| 3175 | – 100 | 10 kHz |
| 12000 | – 100 | 10 kHz |

Additionally, the PSD mask shall be satisfying following requirements:

| Frequency (kHz) | PSD level (dBm/Hz) | MBW |
| --- | --- | --- |
| 3750 | – 100 | 1 MHz |
| 4545 | – 110 | 1 MHz |
| 7225 | – 112 | 1 MHz |
| 12000 | – 112 | 1 MHz |

NOTE 1 – All PSD measurements shall measure the spectral power into a resistive load having a value of 100 .

NOTE 2 – The breakpoint frequencies and PSD values are exact; the indicated slopes are approximate. The breakpoints in the tables shall be connected by linear straight lines on a dB/log(f) plot.

NOTE 3 – MBW specifies the measurement bandwidth. The MBW specified for a certain breakpoint with frequency fi is applicable for all frequencies satisfying fi  f  fj, where fj is the frequency of the next specified breakpoint.

NOTE 4 – The power in a 1-MHz sliding window is measured in a 1-MHz bandwidth, starting at the measurement frequency, i.e., power in the [f, f  1 MHz] window shall conform to the specification at frequency f.

NOTE 5 – All PSD and power measurements shall be made at the U-C interface.

Figure B.1/G.992.5 – ATU-C transmitter PSD mask for overlapped spectrum operation

The ISDN port of the ISDN splitter shall be terminated with the appropriate 2B1Q or 4B3T design impedance for ISDN-BA as defined in ETSI TS 102 080 [6].

It is intended that the degradation impact on the ISDN-BA line system performance be no more than 4.5 dB and 4 dB, for 2B1Q and 4B3T line codes respectively, at the insertion loss reference frequency.

#### B.1.2.1	Passband PSD and response

There are three different PSD masks for the ATU-C transmit signal, depending on the type of signal sent. Across the whole passband, the transmit PSD level shall not exceed the maximum passband transmit PSD level, defined as:

–	NOMPSDds + 1 dB, for initialization signals up to and including the Channel Discovery Phase;

–	REFPSDds + 1 dB, during the remainder of initialization, starting with the Transceiver Training Phase;

–	MAXNOMPSDds – PCBds + 3.5 dB, during showtime.

The group delay variation over the passband shall not exceed 50 µs.

The maximum transmit PSD allows for a 1 dB of non-ideal transmit filter effects (e.g., passband ripple and transition band rolloff).

For spectrum management purposes, the PSD template is defined in Table B.2 (informative):

Table B.2/G.992.5 – ATU-C transmitter PSD template
for overlapped spectrum operation

| Frequency (kHz) | PSD level (dBm/Hz) |
| --- | --- |
| 0 | – 90 |
| 50 | – 90 |
| 80 | – 85.3 |
| 120 | – 40 |
| 1104 | – 40 |
| 1622 | – 50 |
| 2208 | – 51.3 |
| 2500 | – 62.9 |
| 3001.5 | – 83.5 |
| 3175 | – 100 |
| 3750 | – 100 |
| 4545 | – 110 |
| 7225 | – 112 |
| 12000 | – 112 |

#### B.1.2.2	Aggregate transmit power

There are three different PSD masks for the ATU-C transmit signal, depending on the type of signal sent (see B.1.2.1). In all cases:

–	the aggregate transmit power across the whole passband shall not exceed (MAXNOMATPds – PCBds) by more than 0.5 dB, in order to accommodate implementational tolerances, and shall not exceed 20.4 dBm;

–	the aggregate transmit power over the 0 to 11.040 MHz band, shall not exceed (MAXNOMATPds – PCBds) by more than 0.9 dB, in order to account for residual transmit power in the stopbands and implementational tolerances.

The power emitted by the ATU-C is limited by the requirements in this clause. Notwithstanding these requirements, it is assumed that the ADSL will comply with applicable national requirements on emission of electromagnetic energy.

For spectrum management purposes, the PSD template nominal passband aggregate transmit power is 19.9 dBm.

### B.1.3	ATU-C transmitter PSD mask for non-overlapped spectrum operation (supplements 8.10)

Figure B.2 defines the limit spectral mask for the ATU-C transmitted signal, which results in reduced NEXT into the ADSL upstream band, relative to the mask in B.1.2. Adherence to this mask will in many cases result in improved upstream performance of the other ADSL systems in the same or adjacent binder group, with the improvement dependent upon the other interferers. This mask differs from the mask in B.1.2 only in the band from 50 kHz to 254 kHz.

The passband is defined as the band from 254 to 2208 kHz. Limits defined within the passband apply also to any narrower bands used.

The low-frequency stopband is defined as frequencies below 254 kHz and includes the ISDN band, the high-frequency stopband is defined as frequencies greater than 2208 kHz.

| Frequency (kHz) | PSD level (dBm/Hz) | MBW |
| --- | --- | --- |
| 0 | – 90 | 10 kHz |
| 93.1 | – 90 | 10 kHz |
| 209 | – 62 | 10 kHz |
| 254 | – 48.5 | 10 kHz |
| 254 | – 36.5 | 10 kHz |
| 1104 | – 36.5 | 10 kHz |
| 1622 | – 46.5 | 10 kHz |
| 2208 | – 47.8 | 10 kHz |
| 2500 | – 59.4 | 10 kHz |
| 3001.5 | – 80 | 10 kHz |
| 3175 | – 100 | 10 kHz |
| 12000 | – 100 | 10 kHz |

Additionally, the PSD mask shall be satisfying following requirements:

| Frequency (kHz) | PSD level (dBm/Hz) | MBW |
| --- | --- | --- |
| 3750 | – 100 | 1 MHz |
| 4545 | – 110 | 1 MHz |
| 7225 | – 112 | 1 MHz |
| 12000 | – 112 | 1 MHz |

NOTE 1 – All PSD measurements shall measure the spectral power into a resistive load having a value of 100 .

NOTE 2 – The breakpoint frequencies and PSD values are exact; the indicated slopes are approximate. The breakpoints in the tables shall be connected by linear straight lines on a dB/log(f) plot.

NOTE 3 – MBW specifies the measurement bandwidth. The MBW specified for a certain breakpoint with frequency fi is applicable for all frequencies satisfying fi  f  fj, where fj is the frequency of the next specified breakpoint.

NOTE 4 – The power in a 1-MHz sliding window is measured in a 1-MHz bandwidth, starting at the measurement frequency, i.e., power in the [f, f  1 MHz] window shall conform to the specification at frequency f.

NOTE 5 – All PSD and power measurements shall be made at the U-C interface.

Figure B.2/G.992.5 – ATU-C transmitter PSD mask for non-overlapped spectrum operation

The ISDN port of the ISDN splitter shall be terminated with the appropriate 2B1Q or 4B3T design impedance for ISDN-BA as defined in ETSI TS 102 080 [6].

It is intended that the degradation impact on the ISDN-BA line system performance be no more than 4.5 dB and 4 dB, for 2B1Q and 4B3T line codes respectively, at the insertion loss reference frequency.

#### B.1.3.1	Passband PSD and response

See § B.1.2.1. For spectrum management purposes, the PSD template is defined in Table B.3 (informative):

Table B.3/G.992.5 – ATU-C transmitter PSD template
for non-overlapped spectrum operation

| Frequency (kHz) | PSD level (dBm/Hz) |
| --- | --- |
| 0 | – 90 |
| 93.1 | – 90 |
| 209 | – 65.5 |
| 254 | – 52 |
| 254 | – 40 |
| 1104 | – 40 |
| 1622 | – 50 |
| 2208 | – 51.3 |
| 2500 | – 62.9 |
| 3001.5 | – 83.5 |
| 3175 | – 100 |
| 3750 | – 100 |
| 4545 | – 110 |
| 7225 | – 112 |
| 12000 | – 112 |

#### B.1.3.2	Aggregate transmit power

See B.1.2.2. In addition, for non-overlapped spectrum operation, the aggregate transmit power across the whole passband shall not exceed 19.8 dBm.

For spectrum management purposes, the PSD template nominal passband aggregate transmit power is 19.3 dBm.

## B.2	ATU-R functional characteristics (pertains to clause 8)

### B.2.1	ATU-R control parameter settings

The ATU-R control parameter settings to be used in the parameterized parts of the main body and/or to be used in this annex are listed in Table B.4. Control parameters are defined in 8.5.

Table B.4/G.992.5 – ATU-R control parameter settings

| Parameter | Default setting | Characteristics |
| --- | --- | --- |
| NSCus | 64 |  |
| NOMPSDus | –38 dBm/Hz | Setting may be changed relative to this value during G.994.1 phase; see 8.13.2. |
| MAXNOMPSDus | –38 dBm/Hz | Setting may be changed relative to this value during G.994.1 phase; see 8.13.2. |
| MAXNOMATPus | 13.3 dBm | Setting may be changed relative to this value during G.994.1 phase; see 8.13.2. |
| Tones 1 to 32 | Enabled/Disabled | Signifies that the transmission of upstream tones 1 to 32 (or a subset thereof) is enabled/disabled. Negotiated in the G.994.1 phase (see B.3). |

### B.2.2	ATU-R upstream transmit spectral mask (supplements 8.10)

The passband is defined as the band from 120 kHz (see Figure B.1) to 276 kHz and is the widest possible band used. Limits defined within the passband apply also to any narrower bands used.

Figure B.3 defines the spectral mask for the transmit signal. The low-frequency stopband is the ISDN band and is defined as frequencies below 120 kHz (see Figure B.1). The high-frequency stopband is defined as frequencies greater than 276 kHz.

| Frequency (kHz) | PSD level (dBm/Hz) | MBW |
| --- | --- | --- |
| 0 | – 90 | 10 kHz |
| 50 | – 90 | 10 kHz |
| 80 | – 81.8 | 10 kHz |
| 120 | – 34.5 | 10 kHz |
| 276 | – 34.5 | 10 kHz |
| 508.8 | – 98.0 | 10 kHz |
| 686 | – 100 | 10 kHz |
| 5275 | – 100 | 10 kHz |
| 12000 | – 100 | 10 kHz |

Additionally the PSD mask shall be satisfying following requirements:

| Frequency (kHz) | PSD level (dBm/Hz) | MBW |
| --- | --- | --- |
| 1411 | – 100 | 1 MHz |
| 1630 | – 110 | 1 MHz |
| 5275 | – 112 | 1 MHz |
| 12000 | – 112 | 1 MHz |

NOTE 1 – All PSD measurements shall measure the spectral power into a resistive load having a value of 100 .

NOTE 2 – The breakpoint frequencies and PSD values are exact; the indicated slopes are approximate. The breakpoints in the tables shall be connected by linear straight lines on a dB/log(f) plot.

NOTE 3 – MBW specifies the measurement bandwidth. The MBW specified for a certain breakpoint with frequency fi is applicable for all frequencies satisfying fi  f  fj, where fj is the frequency of the next specified breakpoint.

NOTE 4 – The power in a 1-MHz sliding window is measured in a 1-MHz bandwidth, starting at the measurement frequency, i.e., power in the [f, f  1 MHz] window shall conform to the specification at frequency f.

NOTE 5 – All PSD and power measurements shall be made at the U-R interface.

NOTE 6 – The upstream PSD mask is intended for use with ISDN 2B1Q and ISDN 4B3T. However, some deployments have reported field issues with ISDN 4B3T NT activation when operating with ADSL overlay. ISDN passband versus ADSL passband tradeoff and ISDN splitter characteristics need further study. A result thereof could be a limitation of the ADSL transmit power below 138 kHz when operating over ISDN 4B3T. Such transmit power limitation can be achieved through frequency domain shaping or masking of the tones below tone index 33 (if the ATU-R transmitter supports tones 1 to 32) or through time-domain filtering with filter rolloff from 138 kHz (if the ATU-R transmitter does not support tones 1 to 32).

Figure B.3/G.992.5 – ATU-R transmitter PSD mask

The ISDN port of the ISDN splitter shall be terminated with the appropriate 2B1Q or 4B3T design impedance for ISDN-BA as defined in ETSI TS 102 080 [6].

It is intended that the degradation impact on the ISDN-BA line system performance be no more than 4.5 dB and 4 dB, for 2B1Q and 4B3T line codes respectively, at the insertion loss reference frequency.

#### B.2.2.1	Passband PSD and response

There are three different PSD masks for the ATU-R transmit signal, depending on the type of signal sent. Across the whole passband, the transmit PSD level shall not exceed the maximum passband transmit PSDlevel, defined as:

–	NOMPSDus + 1 dB, for initialization signals up to and including the Channel Discovery Phase;

–	REFPSDus + 1 dB, during the remainder of initialization, starting with the Transceiver Training Phase;

–	MAXNOMPSDus – PCBus + 3.5 dB, during showtime.

The group delay variation over the passband shall not exceed 50 µs.

The maximum transmit PSD allows for a 1 dB of non-ideal transmit filter effects (e.g., passband ripple and transition band rolloff).

For spectrum management purposes, the PSD template is defined in Table B.5 (informative):

Table B.5/G.992.5 – ATU-R transmitter PSD template

| Frequency (kHz) | PSD level (dBm/Hz) |
| --- | --- |
| 0 | – 90 |
| 50 | – 90 |
| 80 | – 85.3 |
| 120 | – 38 |
| 276 | – 38 |
| 491 | – 97.8 |
| 686 | – 100 |
| 1411 | – 100 |
| 1630 | – 110 |
| 5275 | – 112 |
| 12000 | – 112 |

#### B.2.2.2	Aggregate transmit power

There are three different PSD masks for the ATU-R transmit signal, depending on the type of signal sent (see B.2.2.1). In all cases:

–	the aggregate transmit power across the whole passband, shall not exceed (MAXNOMATPus – PCBus) by more than 0.5 dB, in order to accommodate implementational tolerances, and shall not exceed 13.8 dBm;

–	the aggregate transmit power over the 0 to 11.040 MHz band shall not exceed (MAXNOMATPus – PCBus) by more than 0.8 dB, in order to account for residual transmit power in the stopbands and implementational tolerances.

The power emitted by the ATU-R is limited by the requirements in this clause. Notwithstanding these requirements, it is assumed that the ADSL will comply with applicable national requirements on emission of electromagnetic energy.

For spectrum management purposes, the PSD template nominal passband aggregate transmit power is 13.3 dBm.

## B.3	Initialization

See B.3/G.992.3.

## B.4	Electrical characteristics

See B.4/G.992.3.

The G.992.3 requirements applying over a frequency band up to 1104 kHz shall be met over a frequency band up to 2208 kHz.

Annex C

Specific requirements for an ADSL system operating in the same cable
as ISDN as defined in ITU-T Rec. G.961 Appendix III

For further study.

Annex D

ATU-C and ATU-R state diagrams

See Annex D/G.992.3.

Annex E

POTS and ISDN-BA splitters

See Annex E/G.992.3.

For operation according to Annexes A, B and I, the G.992.3 requirements applying over a frequency band up to 1104 kHz shall be met over a frequency band up to 2208 kHz.

Annex F

ATU-x performance requirements for Region A (North America)

## F.1	Performance requirements for operation of ADSL over POTS (Annex A)

For further study.

## F.2	Performance requirements for operation of All Digital Mode ADSL (Annex I)

For further study.

## F.3	Performance requirements for operation of ADSL over POTS, with extended upstream bandwidth (Annex L)

For further study.

Annex G

ATU-x performance requirements for Region B (Europe)

## G.1	Performance requirements for operation of ADSL over POTS (Annex A)

For further study.

## G.2	Performance requirements for operation of ADSL over ISDN (Annex B)

For further study.

## G.3	Performance requirements for operation of All Digital Mode ADSL (Annex I)

For further study.

## G.4	Performance requirements for operation of All Digital Mode ADSL (Annex J)

For further study.

## G.5	Performance requirements for operation of ADSL over POTS, with extended upstream bandwidth (Annex L)

For further study.

Annex H

Specific requirements for a synchronized symmetrical DSL (SSDSL)
system operating in the same cable binder as ISDN as defined
in ITU-T Rec. G.961 Appendix III

For further study.

Annex I

All Digital Mode ADSL with improved spectral compatibility
with ADSL over POTS

This annex defines those parameters of the ADSL system that have been left undefined in the body of this Recommendation because they are unique to an all digital ADSL service with improved spectral compatibility with ADSL over POTS.

## I.1	ATU-C functional characteristics (pertains to clause 8)

### I.1.1	ATU-C control parameter settings

The ATU-C control parameter settings to be used in the parameterized parts of the main body and/or to be used in this annex are listed in Table I.1. Control parameters are defined in 8.5.

Table I.1/G.992.5 – ATU-C control parameter settings

| Parameter | Default setting | Characteristics |
| --- | --- | --- |
| NSCds | 512 |  |
| NOMPSDds | –40 dBm/Hz | Settings may be changed relative to this value during G.994.1 phase; see 8.13.2. |
| MAXNOMPSDds | –40 dBm/Hz | Settings may be changed relative to this value during G.994.1 phase; see 8.13.2. |
| MAXNOMATPds | 20.4 dBm | Settings may be changed relative to this value during G.994.1 phase; see 8.13.2. |

### I.1.2	ATU-C downstream transmit spectral mask for overlapped spectrum operation (supplements 8.10)

The passband is defined as the band from 3 to 2208 kHz and is the widest possible band used (i.e., implemented with overlapped spectrum). Limits defined within the passband apply also to any narrower bands used.

Figure I.1 defines the limit spectral mask for the transmit signal. The low-frequency stopband is defined as frequencies below 3 kHz, the high-frequency stopband is defined as frequencies greater than 2208 kHz.

| Frequency (kHz) | PSD level (dBm/Hz) | MBW |
| --- | --- | --- |
| 0 | – 48.5 | 100 Hz |
| 1.5 | – 48.5 | 100 Hz |
| 3 | – 36.5 | 100 Hz |
| 10 | – 36.5 | 10 kHz |
| 25.875 | – 36.5 | 10 kHz |
| 1104 | – 36.5 | 10 kHz |
| 1622 | – 46.5 | 10 kHz |
| 2208 | – 47.8 | 10 kHz |
| 2500 | – 59.4 | 10 kHz |
| 3001.5 | – 80 | 10 kHz |
| 3175 | – 100 | 10 kHz |
| 12000 | – 100 | 10 kHz |

Additionally, the PSD mask shall be satisfying following requirements:

| Frequency (kHz) | PSD level (dBm/Hz) | MBW |
| --- | --- | --- |
| 3750 | – 100 | 1 MHz |
| 4545 | – 110 | 1 MHz |
| 7225 | – 112 | 1 MHz |
| 12000 | – 112 | 1 MHz |

NOTE 1 – All PSD measurements are in 100 .

NOTE 2 – The breakpoint frequencies and PSD values are exact; the indicated slopes are approximate. The breakpoints in the tables shall be connected by linear straight lines on a dB/log(f) plot.

NOTE 3 – MBW specifies the measurement bandwidth. The MBW specified for a certain breakpoint with frequency fi is applicable for all frequencies satisfying fi  f  fj, where fj is the frequency of the next specified breakpoint.

NOTE 4 – The power in a 1-MHz sliding window is measured in a 1-MHz bandwidth, starting at the measurement frequency, i.e., power in the [f, f  1 MHz] window shall conform to the specification at frequency f.

NOTE 5 – All PSD and power measurements shall be made at the U-C interface.

NOTE – When deployed in the same cable as ADSL-over-POTS (Annex A/G.992.1, Annexes A and B/G.992.2, Annex A/G.992.3 and Annex A/G.992.4), there may be a spectral compatibility issue between the two systems due to the overlap of the All-Digital Loop downstream channel with the ADSL-over-POTS upstream channel at frequencies below 138 kHz. Detailed study of spectrum compatibility is referred to regional bodies. Deployment restrictions for systems using the downstream PSD masks defined in this annex may be imposed (e.g., by the regional regulatory authority).

Figure I.1/G.992.5 – ATU-C transmitter PSD mask for overlapped spectrum operation

#### I.1.2.1	Passband PSD and response

There are three different PSD masks for the ATU-C transmit signal, depending on the type of signal sent. Across the whole passband, the transmit PSD level shall not exceed the maximum passband transmit PSD level, defined as:

–	NOMPSDds + 1 dB, for initialization signals up to and including the Channel Discovery Phase;

–	REFPSDds + 1 dB, during the remainder of initialization, starting with the Transceiver Training Phase;

–	MAXNOMPSDds – PCBds + 3.5 dB, during showtime.

The group delay variation over the passband shall not exceed 50 s.

The maximum passband transmit PSD level allows for 1 dB of non-ideal transmit filter effects (e.g. passband ripple and transition band rolloff).

For spectrum management purposes, the ATU-C transmitter PSD template for overlapped spectrum operation is defined in Table I.2 (informative):

Table I.2/G.992.5 – ATU-C transmitter PSD template
for overlapped spectrum operation

| Frequency (kHz) | PSD level (dBm/Hz) |
| --- | --- |
| 0 | – 52 |
| 1.5 | – 52 |
| 3 | – 40 |
| 1104 | – 40 |
| 1622 | – 50 |
| 2208 | – 51.3 |
| 2500 | – 62.9 |
| 3001.5 | – 83.5 |
| 3175 | – 100 |
| 3750 | – 100 |
| 4545 | – 110 |
| 7225 | – 112 |
| 12000 | – 112 |

#### I.1.2.2	Aggregate transmit power

There are three different PSD masks for the ATU-C transmit signal, depending on the type of signal sent (see I.1.2.1). In all cases:

–	the aggregate transmit power across the whole passband shall not exceed (MAXNOMATPds – PCBds) by more than 0.5 dB, in order to accommodate implementational tolerances, and shall not exceed 20.9 dBm.

–	the aggregate transmit power over the 0 to 12 MHz band shall not exceed (MAXNOMATPds – PCBds) by more than 0.9 dB, in order to account for residual transmit power in the stopbands and implementational tolerances.

The power emitted by the ATU-C is limited by the requirements in this clause. Notwithstanding these requirements, it is assumed that the ADSL will comply with applicable national requirements on emission of electromagnetic energy.

For spectrum management purposes, the PSD template nominal passband aggregate transmit power is 20.4 dBm.

### I.1.3	ATU-C transmitter PSD mask for non-overlapped spectrum operation (supplements 8.10)

The ATU-C transmit spectral mask shall be identical to the ATU-C transmit spectral mask for non-overlapped spectrum operation over POTS, as defined in Figure A.2 in A.1.3, with the following modification:

–	For 0  f  4, the PSD shall be below – 97.5 dBm/Hz (no extra limitation of max power in 0- 4 kHz band).

Adherence to this mask will in many cases result in improved upstream performance of the other ADSL systems in the same or adjacent binder group, with the improvement dependent upon the other interferers. This mask differs from the mask in I.1.2 only in the band below 138 kHz.

The passband is defined as the band from 138 to 2208 kHz. Limits defined within the passband apply also to any narrower bands used.

The low-frequency stopband is defined as frequencies below 138 kHz; the high-frequency stopband is defined as frequencies greater than 2208 kHz.

#### I.1.3.1	Passband PSD and response

See A.1.3.1.

#### I.1.3.2	Aggregate transmit power

See A.1.3.2.

## I.2	ATU-R functional characteristics (pertains to clause 8)

### I.2.1	ATU-R control parameter settings

The ATU-R control parameter settings to be used in the parameterized parts of the main body of this Recommendation and/or to be used in this annex are listed in Table I.3. Control parameters are defined in 8.5.

Table I.3/G.992.5 – ATU-R control parameter settings

| Parameter | Default setting | Characteristics |
| --- | --- | --- |
| NSCus | 32 |  |
| NOMPSDus | –38 dBm/Hz | Settings may be changed relative to this value during G.994.1 phase; see 8.13.2. |
| MAXNOMPSDus | –38 dBm/Hz | Settings may be changed relative to this value during G.994.1 phase; see 8.13.2. |
| MAXNOMATPus | 13.3 dBm | Settings may be changed relative to this value during G.994.1 phase; see 8.13.2. |

### I.2.2	ATU-R upstream transmit spectral mask (supplements 8.10)

The passband is defined as the band from 3 to 138 kHz and is the widest possible band used. Limits defined within the passband apply also to any narrower bands used.

Figure I.2 defines the spectral mask for the transmit signal. The low-frequency stopband is defined as frequencies below 3 kHz, the high-frequency stopband is defined as frequencies greater than 138 kHz.

| Frequency (kHz) | PSD level (dBm/Hz) | MBW |
| --- | --- | --- |
| 0 | – 46.5 | 100 Hz |
| 1.5 | – 46.5 | 100 Hz |
| 3 | – 34.5 | 100 Hz |
| 10 | – 34.5 | 10 kHz |
| 138 | – 34.5 | 10 kHz |
| 243 | – 93.2 | 10 kHz |
| 686 | – 100 | 10 kHz |
| 5275 | – 100 | 10 kHz |
| 12000 | – 100 | 10 kHz |

Additionally, the PSD mask shall be satisfying following requirements:

| Frequency (kHz) | PSD level (dBm/Hz) | MBW |
| --- | --- | --- |
| 1411 | – 100 | 1 MHz |
| 1630 | – 110 | 1 MHz |
| 5275 | – 112 | 1 MHz |
| 12000 | – 112 | 1 MHz |

NOTE 1 – All PSD measurements are in 100 .

NOTE 2 – The breakpoint frequencies and PSD values are exact; the indicated slopes are approximate. The breakpoints in the tables shall be connected by linear straight lines on a dB/log(f) plot.

NOTE 3 – MBW specifies the measurement bandwidth. The MBW specified for a certain breakpoint with frequency fi is applicable for all frequencies satisfying fi  f  fj, where fj is the frequency of the next specified breakpoint.

NOTE 4 – The power in a 1-MHz sliding window is measured in a 1-MHz bandwidth, starting at the measurement frequency, i.e., power in the [f, f  1 MHz] window shall conform to the specification at frequency f.

NOTE 5 – All PSD and power measurements shall be made at the U-R interface.

Figure I.2/G.992.5 – ATU-R transmitter PSD mask

#### I.2.2.1	Passband PSD and response

There are three different PSD masks for the ATU-R transmit signal, depending on the type of signal sent. Across the whole passband, the transmit PSD level shall not exceed the maximum passband PSD level, defined as:

–	NOMPSDus + 1 dB, for initialization signals up to and including the Channel Discovery Phase;

–	REFPSDus + 1 dB, during the remainder of initialization, starting with the Transceiver Training Phase;

–	MAXNOMPSDus – PCBus + 3.5 dB, during showtime.

The group delay variation over the passband shall not exceed 50 µs.

The maximum transmit PSD level allows for 1 dB of non-ideal transmit filter effects (e.g., passband ripple and transition band rolloff).

For spectrum management purposes, the PSD template is defined in Table I.4 (informative):

Table I.4/G.992.5 – ATU-R transmitter PSD template

| Frequency (kHz) | PSD level (dBm/Hz) |
| --- | --- |
| 0 | – 50 |
| 1.5 | – 50 |
| 3 | – 38 |
| 138 | – 38 |
| 229.6 | – 92.9 |
| 686 | – 100 |
| 1411 | – 100 |
| 1630 | – 110 |
| 5275 | – 112 |
| 12000 | – 112 |

#### I.2.2.2	Aggregate transmit power

There are three different PSD masks for the ATU-R transmit signal, depending on the type of signal sent (see I.2.2.1). In all cases:

–	the aggregate transmit power across the whole passband shall not exceed (MAXNOMATPus – PCBus) by more than 0.5 dB, in order to accommodate implementational tolerances, and shall not exceed 13.8 dBm.

–	the aggregate transmit power over the 0 to 12 MHz band shall not exceed (MAXNOMATPus – PCBus) by more than 0.8 dB, in order to account for residual transmit power in the stopbands and implementational tolerances.

The power emitted by the ATU-R is limited by the requirements in this clause. Notwithstanding these requirements, it is assumed that the ADSL will comply with applicable national requirements on emission of electromagnetic energy.

For spectrum management purposes, the PSD template nominal passband aggregate transmit power is 13.3 dBm.

## I.3	Initialization

For this annex, no additional requirements apply (relative to the main body of this Recommendation).

## I.4	Electrical characteristics

See I.4/G.992.3.

The G.992.3 requirements applying over a frequency band up to 1104 kHz shall be met over a frequency band up to 2208 kHz.

Annex J

All Digital Mode ADSL with improved spectral compatibility
with ADSL over ISDN

## J.1	ATU-C Functional Characteristics (pertains to § 8)

### J.1.1	ATU-C Control Parameter Settings

The ATU-C Control Parameter Settings to be used in the parameterized parts of the main body and/or to be used in this Annex are listed in Table J.1. Control Parameters are defined in § 8.5.

Table J.1/G.992.5 – ATU-C Control Parameter Settings

| Parameter | Default Setting | Characteristics |
| --- | --- | --- |
| NSCds | 512 |  |
| NOMPSDds | -40 dBm/Hz | Setting may be changed relative to this value during G.994.1 phase, see § 8.13.2. |
| MAXNOMPSDds | -40 dBm/Hz | Setting may be changed relative to this value during G.994.1 phase, see § 8.13.2. |
| MAXNOMATPds (operation per § J.1.2) | 20.4 dBm | Setting may be changed relative to this value during G.994.1 phase, see § 8.13.2. |

### J.1.2	ATU-C downstream transmit spectral mask for overlapped spectrum operation (supplements § 8.10)

The ATU-C transmit spectral mask shall be identical to the ATU-C transmit spectral mask for overlapped spectrum operation, as defined in Figure I.1 in § I.1.2.

The passband is defined as the band from 3 to 2208 kHz and is the widest possible band used (i.e., implemented with overlapped spectrum). Limits defined within the passband apply also to any narrower bands used.

The low-frequency stop-band is defined as frequencies below 3 kHz, the high-frequency stop-band is defined as frequencies greater than 2208 kHz.

NOTE - 	When deployed in the same cable as ADSL-over-POTS (G.992.1 Annex A, G.992.2 Annexes A & B, G.992.3 Annex A and G.992.4 Annex A), there may be a spectral compatibility issue between the two systems due to the overlap of the All-Digital Loop downstream channel with the ADSL-over-POTS upstream channel at frequencies below 138 kHz. Detailed study of spectrum compatibility is referred to regional bodies. Deployment restrictions for systems using the downstream PSD masks defined in this Annex may be imposed (e.g., by the regional regulatory authority).

#### J.1.2.1	Passband PSD and response

See  I.1.2.1.

#### J.1.2.2	Aggregate transmit power

See § I.1.2.2.

### J.1.3	ATU-C downstream transmit spectral mask for non-overlapped spectrum operation (supplements § 8.10)

The ATU-C transmit spectral mask shall be identical to the ATU-C transmit spectral mask for non-overlapped spectrum operation over ISDN, as defined in Figure B.2 in § B.1.3.

Adherence to this mask will in many cases result in improved upstream performance of the other ADSL systems in the same or adjacent binder group, with the improvement dependent upon the other interferers. This mask differs from the mask in § J.1.2 only in the band below 254 kHz.

The passband is defined as the band from 254 to 2208 kHz. Limits defined within the passband apply also to any narrower bands used.

The low-frequency stop-band is defined as frequencies below 254 kHz, the high-frequency stop-band is defined as frequencies greater than 2208 kHz.

#### J.1.3.1	Passband PSD and response

See  B.1.3.1.

#### J.1.3.2	Aggregate transmit power

See § B.1.3.2.

## J.2	ATU-R Functional Characteristics (pertains to § 8)

### J.2.1	ATU-R Control Parameter Settings

The ATU-R Control Parameter Settings to be used in the parameterized parts of the main body and/or to be used in this Annex are listed in Table J.2. Control Parameters are defined in § 8.5.

Table J.2/G.992.5 – ATU-R Control Parameter Settings

| Parameter | Setting | Characteristics |
| --- | --- | --- |
| NSCus | 64 |  |
| NOMPSDus | –38 dBm/Hz. | Setting may be changed relative to this value during G.994.1 phase, see § 8.13.2. |
| MAXNOMPSDus | –38 dBm/Hz. | Setting may be changed relative to this value during G.994.1 phase, see § 8.13.2. |
| MAXNOMATPus | 13.4 dBm | Setting may be changed relative to this value during G.994.1 phase, see § 8.13.2. |

### J.2.2	ATU-R upstream transmit spectral mask (supplements § 8.10)

The ATU-R transmit PSD shall comply to one of the allowed family of spectral masks ADLU-32, ADLU-36,… ADLU-64 (see NOTE 1). Each of the spectral masks shall be as defined in Figure J.1 and Table J.3.

The passband is defined as the band from 3 kHz to an upperbound frequency f1, defined in Table J.3. It is the widest possible band used. Limits defined within the passband apply also to any narrower bands used.

Figure J.1 defines the family of ATU-R spectral masks for the transmit signal. The low-frequency stop-band is defined as frequencies below 3 kHz, the high-frequency stop-band is defined as frequencies greater than the passband upperbound frequency f1 defined in Table J.3. The Inband_peak_PSD, PSD_int and the frequencies f1 and f_int shall be as defined in Table J.3.

NOTE 1 - 	The ATU-R selects a transmit PSD mask from the family of upstream transmit PSD masks specified in Table J.3, based on the limitations imposed by the CO-MIB (which are exchanged during the G.994.1 Phase of initialization, see § 8.13.2.4) and based on the capabilities of its transmit PMD function.

NOTE 2 - 	When deployed in the same cable as ADSL-over-POTS (G.992.1 Annex A, G.992.2 Annexes A & B, G.992.3 Annex A, G.992.4 Annex A and G.992.5 Annex A), there may be a spectral compatibility issue between the two systems due to the overlap of the All Digital Mode upstream channel with the ADSL over POTS downstream channel at frequencies above 138 kHz. Detailed study of spectrum compatibility is referred to regional bodies. Deployment restrictions for systems using the upstream PSD masks defined in this Annex may be imposed (e.g., by the regional regulatory authority).

| Frequency (kHz) | PSD level (dBm/Hz) | Measurement BW |
| --- | --- | --- |
| 0 | -46.5 | 100 Hz |
| 1.5 | -46.5 | 100 Hz |
| 3 | Inband_peak_PSD | 100 Hz |
| 10 | Inband_peak_PSD | 10 kHz |
| f1 | Inband_peak_PSD | 10 kHz |
| f_int | PSD_int | 10 kHz |
| 686 | -100 | 10 kHz |
| 5275 | -100 | 10 kHz |
| 12000 | -100 | 10 kHz |

Additionally the PSD mask shall be satisfying following requirements:

| Frequency (kHz) | PSD level (dBm/Hz) |  | Measurement BW |
| --- | --- | --- | --- |
| 1411 | -100 | 1 MHz |  |
| 1630 | -110 | 1 MHz |  |
| 5275 | -112 | 1 MHz |  |
| 12000 | -112 | 1 MHz |  |

NOTE 1 - 	All PSD measurements are in 100 ; the POTS band total power measurement is in 600 .

NOTE 2 - 	The breakpoint frequencies and PSD values are exact; the indicated slopes are approximate. The breakpoints in the tables shall be connected by linear straight lines on a dB/log(f) plot.

NOTE 3 - 	MBW specifies the Measurement Bandwidth. The MBW specified for a certain breakpoint with frequency fi is applicable for all frequencies satisfying fi f  fj, where fj is the frequency of the next specified breakpoint.

NOTE 4 - 	The power in a 1 MHz sliding window is measured in a 1 MHz bandwidth, starting at the measurement frequency i.e. power in the [f, f  1 MHz] window shall conform to the specification at frequency f.

NOTE 5 - 	All PSD and power measurements shall be made at the U-R interface

Figure J.1/G.992.5 - ATU-R transmitter PSD mask

Table J.3/G.992.5 - Inband peak PSD, PSD_int and the frequencies f1 and f_int

| Upstream Mask- Number | Designator | Template Nominal PSD (dBm/Hz) | Template Maximum Aggregate Transmit Power (dBm) | Inband Peak PSD (dBm/Hz) | Frequency f1 (kHz) | Intercept Frequency f_int (kHz) | Intercept PSD Level PSD_int (dBm/Hz) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | ADLU-32 | -38.0 | 13.4 | -34.5 | 138.00 | 242.92 | -93.2 |
| 2 | ADLU-36 | -38.5 | 13.4 | -35.0 | 155.25 | 274.00 | -94.0 |
| 3 | ADLU-40 | -39.0 | 13.4 | -35.5 | 172.50 | 305.16 | -94.7 |
| 4 | ADLU-44 | -39.4 | 13.4 | -35.9 | 189.75 | 336.40 | -95.4 |
| 5 | ADLU-48 | -39.8 | 13.4 | -36.3 | 207.00 | 367.69 | -95.9 |
| 6 | ADLU-52 | -40.1 | 13.4 | -36.6 | 224.25 | 399.04 | -96.5 |
| 7 | ADLU-56 | -40.4 | 13.4 | -36.9 | 241.50 | 430.45 | -97.0 |
| 8 | ADLU-60 | -40.7 | 13.4 | -37.2 | 258.75 | 461.90 | -97.4 |
| 9 | ADLU-64 | -41.0 | 13.4 | -37.5 | 276.00 | 493.41 | -97.9 |

#### J.2.2.1	Passband PSD and response

See § I.2.2.1.

For spectrum management purposes, the PSD template is defined in Table J.4.and Table J.5 (informative).

Table J.4/G.992.5 – ATU-R transmit PSD template definition

| Frequency (kHz) | PSD level (dBm/Hz) |
| --- | --- |
| 0 | -50 |
| 1.5 | -50 |
| 3 | Inband_peak_PSD -3.5 dB |
| f1 | Inband_peak_PSD -3.5 dB |
| f_int_templ | PSD_int_templ |
| 686 | -100 |
| 1411 | -100 |
| 1630 | -110 |
| 5275 | -112 |
| 12000 | -112 |

Table J.5/G.992.5 – The f_int_templ and PSD_int_templ values 
for the ATU-R transmit PSD template

| Upstream Mask- Number | Designator | Template Intercept Frequency f_int_templ (kHz) | Template Intercept PSD Level PSD_int_templ (dBm/Hz) |
| --- | --- | --- | --- |
| 1 | ADLU-32 | 234.34 | -93.0 |
| 2 | ADLU-36 | 264.33 | -93.8 |
| 3 | ADLU-40 | 294.39 | -94.5 |
| 4 | ADLU-44 | 324.52 | -95.1 |
| 5 | ADLU-48 | 354.71 | -95.7 |
| 6 | ADLU-52 | 384.95 | -96.2 |
| 7 | ADLU-56 | 415.25 | -96.7 |
| 8 | ADLU-60 | 445.59 | -97.2 |
| 9 | ADLU-64 | 475.99 | -97.6 |

#### J.2.2.2	Aggregate transmit power

There are three different PSD masks for the ATU-R transmit signal, depending on the type of signal sent (see § J.2.2.1). In all cases,

- the aggregate transmit power across the whole passband, shall not exceed (MAXNOMATPus-PCBus) by more than 0.5 dB, in order to accommodate implementational tolerances, and shall not exceed 13.9 dBm.

- the aggregate transmit power over the 0 to 12 MHz band, shall not exceed (MAXNOMATPus - PCBus) by more than 0.8 dB, in order to account for residual transmit power in the stop bands and implementational tolerances.

The power emitted by the ATU-R is limited by the requirements in this section. Notwithstanding these requirements, it is assumed that the ADSL will comply with applicable national requirements on emission of electromagnetic energy.

For spectrum management purposes, the PSD template nominal passband aggregate transmit power is 13.4 dBm.

## J.3	Initialization

The ATU-C and ATU-R shall support all upstream PSD masks listed in Table J.3.

### J.3.1	Handshake – ATU-C (supplements § 8.13.2.1)

The G.994.1 codepoints required for the initialization of ATU-C and ATU-R shall be contained in an “Annex J Submode PSD Masks” SPAR(2) parameter block. This parameter block shall be added to the G.994.1 codetree defined for G.992.5 Annex J (All Digital Mode ADSL with improved Spectral Compatibility with ADSL over ISDN).

#### J.3.1.1	CL messages (supplements § 8.13.2.1.1)

The CL message {Par(2)} fields are defined in Table 8-20. Additional  G.994.1 CL message {Par(2)} fields for all digital mode operation are defined in Table J.4.

Table J.4/G.992.5 – ATU-C CL message additional Par(2) PMD bit definitions

| Spar(2) bit | Definition of related Npar(3) bits |
| --- | --- |
| Submode PSD masks | This parameter block indicates to the ATU-R which PSD masks are supported. The Submode PSD Masks field indicates which upstream PSD masks are supported. Its value will depend on CO-MIB element settings and local capabilities of the ATU-C. This field shall be coded in PSD Mask NPar(3) Octet 1 and 2. The coding shall be as follows: the bit associated to an upstream PSD mask shall be set to ONE to indicate that this mask is supported. The ATU-C shall set to ONE one of the upstream PSD mask bits to indicate to the ATU-R the selection of one of the PSD mask listed in Table J.3. |

#### J.3.1.2	MS messages (supplements § 8.13.2.1.2)

The MS message {Par(2)} fields are defined in Table 8-21. Additional G.994.1 MS message {Par(2)} fields for all digital mode operation are defined in Table J.5.

Table J.5/G.992.5 – ATU-C MS message additional Par(2) PMD bit definitions

| SPar(2) bit | Definition of related Npar(3) bits |
| --- | --- |
| Submode PSD masks | This parameter block indicates to the ATU-R which PSD masks are selected. The Submode PSD Masks field indicates which upstream PSD masks is selected. This field shall be coded in PSD Mask NPar(3) Octet 1 and 2. The coding shall be as follows: the bit associated to an upstream PSD mask shall be set to ONE to indicate that this mask is selected. Each of these bits may be set to ONE only if that bit was set to ONE in the last previous CL message and the last previous CLR message. The ATU-C shall set to ONE one of the upstream PSD mask bits to indicate to the ATU-R the selection of one of the PSD mask listed in Table J.3. |

### J.3.2	Handshake – ATU-R (supplements § 8.13.2.2)

The G.994.1 codepoints required for the initialization of ATU-C and ATU-R shall be contained in an “Annex J Submode PSD Masks” SPAR(2) parameter block. This parameter block shall be added to the G.994.1 codetree defined for G.992.5 Annex J (All Digital Mode ADSL with improved Spectral Compatibility with ADSL over ISDN).

#### J.3.2.1	CLR messages (supplements § 8.13.2.2.1)

The CLR message {Par(2)} fields are defined in Table 8-22. Additional  G.994.1 CLR message {Par(2)} fields are defined in Table J.6.

Table J.6/G.992.5 – ATU-R CLR message additional Par(2) PMD bit definitions

| SPar(2) bit | Definition of related Npar(3) bits |
| --- | --- |
| Submode PSD masks | This parameter block indicates to the ATU-C which PSD masks are supported. This field shall be coded in PSD Mask NPar(3) Octet 1 and 2. The coding shall be as follows: the bit associated to an upstream PSD mask shall be set to ONE to indicate that this mask is supported. As the ATU-R shall support all PSD mask configurations, it shall set all mask bits to ONE (1). |

#### J.3.2.2	MS messages (supplements § 8.13.2.2.2)

The MS message {Par(2)} fields are defined in Table 8-23. Additional  G.994.1 CLR message {Par(2)} fields are defined in Table J.7.

Table J.7/G.992.5 – ATU-R MS message additional Par(2) PMD bit definitions

| SPar(2) bit | Definition of related Npar(3) bits |
| --- | --- |
| Submode PSD masks | This parameter block indicates to the ATU-C which PSD masks are selected. This field shall be coded in PSD Mask NPar(3) Octet 1 and 2. The coding shall be as follows: the bit associated to an upstream PSD mask shall be set to ONE to indicate that this mask is selected. Each of these bits may be set to ONE only if that bit was set to ONE in the last previous CL message and the last previous CLR message. The ATU-R shall set to ONE one of the PSD mask bits to indicate to the ATU-C the selection of one PSD mask listed in Table J.3. |

### J.3.3	Spectral bounds and shaping parameters (supplements § 8.13.2.4)

See M.3.3.

J.3.4	Upstream spectrum shaping

The upstream spectrum shaping for G.992.5 Annex J is defined in the same way as for G.992.3 Annex J (see ITU-T Rec. G.992.3). The difference with J.3.4/G.992.3 are the handshake codepoints being identically defined but added under the G.992.5 Annex J Spar(1) codetree.

## J.4	Electrical characteristics

The ATU shall meet the electrical characteristics defined in § I.4.

Annex K

TPS-TC functional descriptions

See Annex K/G.992.3, with the following changes:

- The G.994.1 codepoints shall represent the data rate divided by 8000 bit/s. The last row of Table K.6/G.992.3 shall show "8000 bit/s" instead of "4000 bit/s".

- The ATU shall support a net data rate of at least 16 Mbit/s. Net_minn, Net_maxn and Net_reserven entry in Table K.4/G.992.3, Table K.11/G.992.3 and Table K.20/G.992.3 shall show "16 Mbit/s" instead of "8 Mbit/s".

- Replace Table K.3a and Table K.3b with Table K.3a and Table K.3b.

- The referenced Appendix V is as contained in this Recommendation.

Table K.3a/G.992.5 - INP_min and delay_max related Downstream Net Datarates limits 
(in kbit/s).

|  |  |  |  | INP_min |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 0 | ½ | 1 | 2 | 4 | 8 | 16 |
|  |  | delay_max [ms] | 1 (*) | 24432 | 0 | 0 | 0 | 0 | 0 | 0 |
|  |  |  | 2 | 24432 | 7104 | 3008 | 960 | 0 | 0 | 0 |
|  |  |  | 4 | 24432 | 15232 | 7104 | 3008 | 960 | 0 | 0 |
|  |  |  | 8 | 24432 | 22896 | 15232 | 7104 | 3008 | 960 | 0 |
|  |  |  | 16 | 24432 | 22896 | 15232 | 7552 | 3520 | 1472 | 448 |
|  |  |  | 32 | 24432 | 22896 | 15232 | 7552 | 3712 | 1728 | 704 |
|  |  |  | 63 | 24432 | 22896 | 15232 | 7552 | 3712 | 1728 | 704 |
|  |  | (*) In G.997.1, a 1 ms delay is reserved to mean that Sp ≤ 1 and Dp= 1. |  |  |  |  |  |  |  |  |

Table K.3b/G.992.5 - INP_min and delay_max related Upstream Net Datarates limits 
(in kbit/s).

|  |  |  |  | INP_min |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 0 | ½ | 1 | 2 | 4 | 8 | 16 |
|  |  | delay_max [ms] | 1 (*) | 3520 | 0 | 0 | 0 | 0 | 0 | 0 |
|  |  |  | 2 | 3520 | 3072 | 1472 | 448 | 0 | 0 | 0 |
|  |  |  | 4 | 3520 | 3264 | 1728 | 704 | 192 | 0 | 0 |
|  |  |  | 8 | 3520 | 3264 | 1792 | 832 | 320 | 64 | 0 |
|  |  |  | 16 | 3520 | 3264 | 1792 | 832 | 384 | 128 | 0 |
|  |  |  | 32 | 3520 | 3264 | 1792 | 832 | 384 | 128 | 0 |
|  |  |  | 63 | 3520 | 3264 | 1792 | 832 | 384 | 128 | 0 |
|  |  | (*) In G.997.1, a 1 ms delay is reserved to mean that Sp ≤ 1 and Dp= 1. |  |  |  |  |  |  |  |  |

Annex L

Intentionally left blank

This annex is intentionally left blank to facilitate alignment of the G.992.3 and G.992.5 table of contents.

Annex M

Specific requirements for an ADSL system with extended upstream bandwidth, operating in the frequency band above POTS

This annex defines those parameters of the ADSL system with extended upstream bandwidth that have been left undefined in the body of this Recommendation because they are unique to an ADSL service that is frequency-division duplexed with POTS.

## M.1	ATU-C Functional Characteristics (pertains to § 8)

### M.1.1	ATU-C Control Parameter Settings

The ATU-C Control Parameter Settings to be used in the parameterized parts of the main body and/or to be used in this Annex are listed in Table M.1. Control Parameters are defined in § 8.5.

Table M.1/G.992.5 – ATU-C Control Parameter Settings

| Parameter | Default Setting | Characteristics |
| --- | --- | --- |
| NSCds | 512 |  |
| NOMPSDds | -40 dBm/Hz | Setting may be changed relative to this value during G.994.1 phase, see § 8.13.2. |
| MAXNOMPSDds | -40 dBm/Hz | Setting may be changed relative to this value during G.994.1 phase, see § 8.13.2. |
| MAXNOMATPds (operation per § M.1.2) | 20.4 dBm | Setting may be changed relative to this value during G.994.1 phase, see § 8.13.2. |

### M.1.2	ATU-C downstream transmit spectral mask for overlapped spectrum operation (supplements § 8.10)

The ATU-C transmit spectral mask shall be identical to the ATU-C transmit spectral mask for overlapped spectrum operation over POTS, as defined in Figure A.1 in § A.1.2.

The passband is defined as the band from 25.875 to 2208 kHz and is the widest possible band used (i.e., implemented with overlapped spectrum). Limits defined within the passband apply also to any narrower bands used.

The low-frequency stop-band is defined as frequencies below 25.875 kHz, the high-frequency stop-band is defined as frequencies greater than 2208 kHz.

#### M.1.2.1	Passband PSD and response

See  A.1.2.1.

#### M.1.2.2	Aggregate transmit power

See § A.1.2.2.

### M.1.3	ATU-C downstream transmit spectral mask for non-overlapped spectrum operation (supplements § 8.10)

The ATU-C transmit spectral mask shall be identical to the ATU-C transmit spectral mask for non-overlapped spectrum operation over ISDN, as defined in Figure B.2 in § B.1.3.

Adherence to this mask will in many cases result in improved upstream performance of the other ADSL systems in the same or adjacent binder group, with the improvement dependent upon the other interferers. This mask differs from the mask in § M.1.2 only in the band below 254 kHz.

The passband is defined as the band from 254 to 2208 kHz. Limits defined within the passband apply also to any narrower bands used.

The low-frequency stop-band is defined as frequencies below 254 kHz, the high-frequency stop-band is defined as frequencies greater than 2208 kHz.

In addition, the maximum PSD level in the in 0-4 kHz band shall not exceed  -97.5 dBm/Hz measured in a reference impedance of 100 Ohm, and  the aggregate transmit power in the in 0-4 kHz band shall not exceed  +15 dBrn measured in a reference impedance of 600 Ohm.

#### M.1.3.1	Passband PSD and response

See  B.1.3.1.

#### M.1.3.2	Aggregate transmit power

See § B.1.3.2.

## M.2	ATU-R Functional Characteristics (pertains to § 8)

### M.2.1	ATU-R Control Parameter Settings

The ATU-R Control Parameter Settings to be used in the parameterized parts of the main body and/or to be used in this Annex are listed in Table M.2. Control Parameters are defined in § 8.5.

Table M.2/G.992.5 – ATU-R Control Parameter Settings

| Parameter | Setting | Characteristics |
| --- | --- | --- |
| NSCus | 64 |  |
| NOMPSDus | –38 dBm/Hz. | Setting may be changed relative to this value during G.994.1 phase, see § 8.13.2. |
| MAXNOMPSDus | –38 dBm/Hz. | Setting may be changed relative to this value during G.994.1 phase, see § 8.13.2. |
| MAXNOMATPus | 12.5 dBm | Setting may be changed relative to this value during G.994.1 phase, see § 8.13.2. |

### M.2.2	ATU-R upstream transmit spectral mask (supplements § 8.10)

The ATU-R transmit PSD shall comply to one of the allowed family of spectral masks EU-32, EU-36,… EU-64 (see NOTE 1). Each of the spectral masks shall be as defined in Figure M.1 and Table M.3.

The passband is defined as the band from 25.875 kHz to an upperbound frequency f1, defined in Table M.3. It is the widest possible band used. Limits defined within the passband apply also to any narrower bands used.

Figure M.1 defines the family of ATU-R spectral masks for the transmit signal. The low-frequency stop-band is defined as frequencies below 25.875 kHz, the high-frequency stop-band is defined as frequencies greater than the passband upperbound frequency f1 defined in Table M.3. The Inband_peak_PSD, PSD_int and the frequencies f1 and f_int shall be as defined in Table M.3.

NOTE 1 - 	The ATU-R selects a transmit PSD mask from the family of upstream transmit PSD masks specified in Table M.3, based on the limitations imposed by the CO-MIB (which are exchanged during the G.994.1 Phase of initialization, see § 8.13.2.4) and based on the capabilities of its transmit PMD function.

NOTE 2 - 	When deployed in the same cable as ADSL-over-POTS (G.992.1 Annex A, G.992.2 Annexes A & B, G.992.3 Annex A, G.992.4 Annex A and G.992.5 Annex A), there may be a spectral compatibility issue between the two systems due to the overlap of the Annex M upstream channel with the ADSL-over-POTS downstream channel at frequencies above 138 kHz. Detailed study of spectrum compatibility is referred to regional bodies. Deployment restrictions for systems using the upstream PSD masks defined in this Annex may be imposed (e.g., by the regional regulatory authority).

| Frequency (kHz) | PSD level (dBm/Hz) | Measurement BW |
| --- | --- | --- |
| 0 | -97.5 | 100 Hz |
| 4 | -97.5 | 100 Hz |
| 4 | -92.5 | 100 Hz |
| 10 | interpolated | 10 kHz |
| 25.875 | Inband_peak_PSD | 10 kHz |
| f1 | Inband_peak_PSD | 10 kHz |
| f_int | PSD_int | 10 kHz |
| 686 | -100 | 10 kHz |
| 5275 | -100 | 10 kHz |
| 12000 | -100 | 10 kHz |

Additionally the PSD mask shall be satisfying following requirements:

| Frequency (kHz) | PSD level (dBm/Hz) |  | Measurement BW |
| --- | --- | --- | --- |
| 1411 | -100 | 1 MHz |  |
| 1630 | -110 | 1 MHz |  |
| 5275 | -112 | 1 MHz |  |
| 12000 | -112 | 1 MHz |  |

NOTE 1 - 	All PSD measurements are in 100 ; the POTS band total power measurement is in 600 .

NOTE 2 - 	The breakpoint frequencies and PSD values are exact; the indicated slopes are approximate. The breakpoints in the tables shall be connected by linear straight lines on a dB/log(f) plot.

NOTE 3 - 	MBW specifies the Measurement Bandwidth. The MBW specified for a certain breakpoint with frequency fi is applicable for all frequencies satisfying fi f  fj, where fj is the frequency of the next specified breakpoint.

NOTE 4 - 	The power in a 1 MHz sliding window is measured in a 1 MHz bandwidth, starting at the measurement frequency i.e. power in the [f, f  1 MHz] window shall conform to the specification at frequency f.

NOTE 5 - 	The step in the PSD mask at 4 kHz is to protect V.90 performance. Originally, the PSD mask continued the 21 dB/octave slope below 4 kHz hitting a floor of -97.5 dBm/Hz at 3400 Hz. It was recognized that this might impact V.90 performance, and so the floor was extended to 4 kHz.

NOTE 6 - 	All PSD and power measurements shall be made at the U-R interface

Figure M.1/G.992.5 - ATU-R transmitter PSD mask

Table M.3/G.992.5 - Inband peak PSD, PSD_int and the frequencies f1 and f_int

| Upstream Mask- Number | ## Designator | Template Nominal PSD (dBm/Hz) | Template Maximum Aggregate Transmit Power (dBm) | Inband Peak PSD (dBm/Hz) | Frequency f1 (kHz) | Intercept Frequency f_int (kHz) | Intercept PSD Level PSD_int (dBm/Hz) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | EU-32 | -38.0 | 12.5 | -34.5 | 138.00 | 242.92 | -93.2 |
| 2 | EU-36 | -38.5 | 12.62 | -35.0 | 155.25 | 274.00 | -94.0 |
| 3 | EU-40 | -39.0 | 12.66 | -35.5 | 172.50 | 305.16 | -94.7 |
| 4 | EU-44 | -39.4 | 12.75 | -35.9 | 189.75 | 336.40 | -95.4 |
| 5 | EU-48 | -39.8 | 12.78 | -36.3 | 207.00 | 367.69 | -95.9 |
| 6 | EU-52 | -40.1 | 12.87 | -36.6 | 224.25 | 399.04 | -96.5 |
| 7 | EU-56 | -40.4 | 12.94 | -36.9 | 241.50 | 430.45 | -97.0 |
| 8 | EU-60 | -40.7 | 12.97 | -37.2 | 258.75 | 461.90 | -97.4 |
| 9 | EU-64 | -41.0 | 12.98 | -37.5 | 276.00 | 493.41 | -97.9 |

#### M.2.2.1	Passband PSD and response

See § I.2.2.1.

For spectrum management purposes, the PSD template is defined in Table M.4 and Table M.5 (informative).

Table M.4/G.992.5 – ATU-R transmit PSD template definition

| Frequency (kHz) | PSD level (dBm/Hz) |
| --- | --- |
| 0 | -101 |
| 4 | -101 |
| 4 | -96 |
| 25.875 | Inband_peak_PSD -3.5 dB |
| F1 | Inband_peak_PSD -3.5 dB |
| F_int _templ | PSD_int_templ |
| 686 | -100 |
| 1411 | -100 |
| 1630 | -110 |
| 5275 | -112 |
| 12000 | -112 |

Table M.5/G.992.5 – The f_int_templ and PSD_int_templ values 
for the ATU-R transmit PSD template

| Upstream Mask- Number | Designator | Template Intercept Frequency f_int_templ (kHz) | Template Intercept PSD Level PSD_int_templ (dBm/Hz) |
| --- | --- | --- | --- |
| 1 | EU-32 | 234.34 | -93.0 |
| 2 | EU-36 | 264.33 | -93.8 |
| 3 | EU-40 | 294.39 | -94.5 |
| 4 | EU-44 | 324.52 | -95.1 |
| 5 | EU-48 | 354.71 | -95.7 |
| 6 | EU-52 | 384.95 | -96.2 |
| 7 | EU-56 | 415.25 | -96.7 |
| 8 | EU-60 | 445.59 | -97.2 |
| 9 | EU-64 | 475.99 | -97.6 |

#### M.2.2.2	Aggregate transmit power

There are three different PSD masks for the ATU-R transmit signal, depending on the type of signal sent (see § M.2.2.1). In all cases,

	the aggregate transmit power in the voiceband, measured at the U-R interface, and that is delivered to the Plain Old Telephone Service (POTS) interface shall not exceed +15dBrn (see ITU-T Recommendation G.996.1 [03] for method of measurement);

	the aggregate transmit power across the whole passband, shall not exceed (MAXNOMATPus-PCBus) by more than 0.5 dB, in order to accommodate implementational tolerances, and shall not exceed 13.0 dBm.

	the aggregate transmit power over the 0 to 12 MHz band, shall not exceed (MAXNOMATPus - PCBus) by more than 0.8 dB, in order to account for residual transmit power in the stop bands and implementational tolerances.

The power emitted by the ATU-R is limited by the requirements in this section. Notwithstanding these requirements, it is assumed that the ADSL will comply with applicable national requirements on emission of electromagnetic energy.

For spectrum management purposes, the PSD template nominal passband aggregate transmit power is 12.5 dBm.

## M.3	Initialization

The ATU-C and ATU-R shall support all upstream PSD masks listed in Table M.3.

### M.3.1	Handshake – ATU-C (supplements § 8.13.2.1)

The G.994.1 codepoints required for the initialization of ATU-C and ATU-R shall be contained in an “Annex M Submode PSD Masks” SPAR(2) parameter block. This parameter block shall be added to the G.994.1 codetree defined for G.992.5 Annex M (Specific requirements for an ADSL system with extended upstream bandwidth, operating in the frequency band above POTS).

#### M.3.1.1	CL messages (supplements § 8.13.2.1.1)

The CL message {Par(2)} fields are defined in Table 8-20. Additional  G.994.1 CL message {Par(2)} fields for extended upstream operation over POTS are defined in Table M.4.

Table M.4/G.992.5 – ATU-C CL message additional Par(2) PMD bit definitions

| Spar(2) bit | Definition of related Npar(3) bits |
| --- | --- |
| Submode PSD masks | This parameter block indicates to the ATU-R which PSD masks are supported. The Submode PSD Masks field indicates which upstream PSD masks are supported. Its value will depend on CO-MIB element settings and local capabilities of the ATU-C. This field shall be coded in PSD Mask NPar(3) Octet 1 and 2. The coding shall be as follows: the bit associated to an upstream PSD mask shall be set to ONE to indicate that this mask is supported. The ATU-C shall set to ONE one of the upstream PSD mask bits to indicate to the ATU-R the selection of one of the PSD mask listed in Table M.3. |

#### M.3.1.2	MS messages (supplements § 8.13.2.1.2)

The MS message {Par(2)} fields are defined in Table 8-21. Additional G.994.1 MS message {Par(2)} fields for extended upstream operation over POTS are defined in Table M.5.

Table M.5/G.992.5 – ATU-C MS message additional Par(2) PMD bit definitions

| SPar(2) bit | Definition of related Npar(3) bits |
| --- | --- |
| Submode PSD masks | This parameter block indicates to the ATU-R which PSD masks are selected. The Submode PSD Masks field indicates which upstream PSD masks is selected. This field shall be coded in PSD Mask NPar(3) Octet 1 and 2. The coding shall be as follows: the bit associated to an upstream PSD mask shall be set to ONE to indicate that this mask is selected. Each of these bits may be set to ONE only if that bit was set to ONE in the last previous CL message and the last previous CLR message. The ATU-C shall set to ONE one of the upstream PSD mask bits to indicate to the ATU-R the selection of one of the PSD mask listed in Table M.3. |

### M.3.2	Handshake – ATU-R (supplements § 8.13.2.2)

The G.994.1 codepoints required for the initialization of ATU-C and ATU-R shall be contained in an “Annex M Submode PSD Masks” SPAR(2) parameter block. This parameter block shall be added to the G.994.1 codetree defined for G.992.5 Annex M (Specific requirements for an ADSL system with extended upstream bandwidth, operating in the frequency band above POTS).

#### M.3.2.1	CLR messages (supplements § 8.13.2.2.1)

The CLR message {Par(2)} fields are defined in Table 8-22. Additional  G.994.1 CLR message {Par(2)} fields are defined in Table M.6.

Table M.6/G.992.5 – ATU-R CLR message additional Par(2) PMD bit definitions

| SPar(2) bit | Definition of related Npar(3) bits |
| --- | --- |
| Submode PSD masks | This parameter block indicates to the ATU-C which PSD masks are supported. This field shall be coded in PSD Mask NPar(3) Octet 1 and 2. The coding shall be as follows: the bit associated to an upstream PSD mask shall be set to ONE to indicate that this mask is supported. As the ATU-R shall support all PSD mask configurations, it shall set all mask bits to ONE (1). |

#### M.3.2.2	MS messages (supplements § 8.13.2.2.2)

The MS message {Par(2)} fields are defined in Table 8-23. Additional  G.994.1 CLR message {Par(2)} fields are defined in Table M.7.

Table M.7/G.992.5 – ATU-R MS message additional Par(2) PMD bit definitions

| SPar(2) bit | Definition of related Npar(3) bits |
| --- | --- |
| Submode PSD masks | This parameter block indicates to the ATU-C which PSD masks are selected. This field shall be coded in PSD Mask NPar(3) Octet 1 and 2. The coding shall be as follows: the bit associated to an upstream PSD mask shall be set to ONE to indicate that this mask is selected. Each of these bits may be set to ONE only if that bit was set to ONE in the last previous CL message and the last previous CLR message. The ATU-R shall set to ONE one of the PSD mask bits to indicate to the ATU-C the selection of one PSD mask listed in Table M.3. |

### M.3.3	Spectral bounds and shaping parameters (supplements § 8.13.2.4)

In the CLR message, the ATU-R shall indicate all supported PSD masks. The CLR message may include the upstream spectral shaping (tssi) and upstream spectrum bounds information of the preferred upstream PSD mask.

In the CL message, the ATU-C shall indicate the selected mode. The CL message may include the upstream spectral shaping (tssi) and spectrum bounds information of the selected mode.

If the upstream spectrum bounds and shaping parameters of the CLR message, and the PSD mask selection in the CL message are found to be inconsistent, then the ATU-R shall do either of the following:

- The ATU-R sends an MS message indicating that it is not prepared to select a mode at this time (according to § 10.1.1/G.994.1). After termination of the G.994.1 session, the ATU-R calculates new upstream spectrum bounds and shaping parameters offline, taking into account the upstream spectrum bounds, shaping parameters, and PSD mask specified by the ATU-C in the CL message of previous G.994.1 session. In a subsequent G.994.1 session, the ATU-R sends a CLR message including the new spectrum bounds and shaping parameters corresponding to the selected PSD mask;

- The ATU-R calculates new upstream spectrum bounds and shaping parameters online, taking into account the upstream spectrum bounds, shaping parameters, and PSD mask specified by the ATU-C in the CL message. In the same G.994.1 session, the ATU-R repeats the CLR/CL exchange transaction with a CLR message including the new spectrum bounds and shaping parameters corresponding to the selected PSD mask.

M.3.4	Upstream spectrum shaping

The upstream spectrum shaping for G.992.5 Annex M is defined in the same way as for G.992.3 Annex M (see ITU-T Rec. G.992.3). The difference with M.3.4/G.992.3 are the handshake codepoints being identically defined but added under the G.992.5 Annex M Spar(1) codetree.

## M.4	Electrical characteristics

The ATU shall meet the electrical characteristics defined in § A.4.

Appendix I

ATM Layer to Physical Layer logical interface

See Appendix I/G.992.3.

Appendix II

Compatibility with other customer premises equipment

See Appendix II/G.992.3.

Appendix III

The impact of primary protection devices on line balance

See Appendix III/G.992.3.

Appendix IV

PSD template to be used in capacity calculations with
in-band transmit spectrum shaping

This appendix informs on the PSD template to be used in capacity calculations for cases where in-band transmit spectrum shaping (tssi) is applied.

This Recommendation supports the possibility for downstream spectrum control with individual maximum transmit PSD at U-C reference point per subcarrier, under operator control through the CO-MIB (see 8.5.1), to allow configuration per the regional requirements (e.g., North America, Europe or Japan) and deployment scenarios (e.g., CO or Remote). The downstream spectrum is controlled via the control parameter MIB_PSD_mask(f) (see 8.5.1). The parameter MIB_PSD_mask(f) defines the PSD mask at the U-C reference point. The average PSD at the U-C reference point is given by MIB_PSD_template(f) (see 8.5.1). The inband part of the CO-MIB specified PSD mask (part with the MAXINSLOPE) will typically be realized by adjusting the gain values of individual subcarriers using the tssi (transmit spectrum shaping) values.

In capacity calculations, capacity depends on the transmit power on each individual subcarrier. This power is proportional to the squared tssi value.

For a flat PSD template, the tssi gain values are equal to 1 (assuming ideal flat time-domain filtering, DAC and AFE) and therefore the transmit power on each subcarrier can be directly calculated from the MIB_PSD_template.

However, it should be brought to attention that, if the inband part is non-flat shaped, it cannot be assumed that the tssi gain values exactly follow the shape of the MIB_PSD_template. In other words, it cannot be assumed that the tssi values equal the value MIB_PSD_template(i) – NOMPSD. This is because the side lobes of higher power subcarriers will increase the PSD of lower power carriers as measured at the U-C reference point.

For this reason, an equivalent PSD template shall be defined for the purpose of capacity calculations, incorporating the transmit spectrum shaping tssi:

Capacity_PSD_template(i) = powergain_DAC&AFE  tssi2(i)

The tssi values can be calculated using:

MIB_PSD_template_dB(f) = MIB_PSD_mask_dB(f) –3.5 dB

MIB_PSD_template_(i) = 10^(MIB_PSD_template_dB(i.f)/10)

for n_IB_low_MIB  i  n_IB_high_MIB

tssi2 = A–1  MIB_PSD_template/powergain_DAC&AFE

Alternatively, one can directly calculate:

Capacity_PSD_template(i) = A–1  MIB_PSD_template

where:

–	tssi2 is the vector of the squared values of tssi, i.e., tssi(i)^2

–	A–1 is the inverse of matrix A

–	A is the matrix:

Am,n=1K×1716×sinc1716×m–n2

for n_IB_low_MIB  m  n_IB_high_MIB, n_IB_low_MIB  n  n_IB_high_MIB

With K=1716×sin c1716×i2=1.1162=0.48 dB

–	n_IB_low_MIB is the first tone of the inband part of the CO-MIB PSD mask.

Using the definitions in 8.5.1:

n_IB_low_MIB = t1     if t1 = roundup(f_pb_start/f)

n_IB_low_MIB = t2     if 100  t1  256

–	n_IB_high_MIB is the last tone of the inband part of the CO-MIB PSD mask.

Using the definitions in 8.5.1:

n_IB_high_MIB = tN

Capacity calculations should use the equivalent PSD template on each individual subcarrier as calculated by Capacity_PSD_template.

Appendix V

Constraints on Delay, Impulse Noise Protection, Overhead Rate, and Net Data Rate when Bonding

This appendix considers the case when multiple transceivers form a bonding group and the differential delay among members of the group is controlled through the delay_min parameter derived from G.994.1. This appendix outlines a set of simple rules that allow the construction of a valid set of configuration parameters involving the minimum delay (delay_min), the minimum impulse noise protection (INP_min), the minimum overhead message rate (MSGmin), the minimum net data rate (net_min) and the data rate granularity. These rules restrict the framing parameters and may lead to a reduction in the attainable data rates.

The rules are as follows:

- Set delay_min = delay_max. In either the upstream or downstream direction, all transceivers in a bonding group should use the same delay.   In the downstream direction, a value for delay can be selected from either Table V.1 or Table V.2. When using delays from Table V.2, since the internal representation of delay_min and delay_max are restricted to be integers, delay_min should be set to floor(delay_min) and delay_max should be set to ceil(delay_max) where floor(·) and ceil(·) are the ‘greatest integer less than’ and ‘smallest integer greater than’ respectively. In the upstream direction, delay_min and delay_max should be selected from Table V.3.

- Set the minimum net data rate below the values shown in Table V.1 or Table V.2 and Table V.3 for downstream and upstream respectively. Depending on the downstream PSD mask and value of BIMAX, the actual maximum net data rate might be lower than those shown in these tables.

- Depending on the delay, the valid range of MSGmin and the corresponding data rate granularity (minimum value of net_max – net_min) are listed in Tables V.4 and Table V.5.

Table V.1/G.992.5 - Maximum downstream net data rate (kbit/s) 
for various values of delay_min = delay_max and INP_min.

|  |  | INP_min (**) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | ½ | 1 | 2 | 4 | 8 | 16 |
| delay_min delay_max (ms) | 1 (*) | 24432 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 2 | 16256 | 7104 | 3008 | 960 | 0 | 0 | 0 |
|  | 4 | 16256 | 15232 | 7104 | 3008 | 960 | 0 | 0 |
|  | 8 | 16256 | 15232 | 15232 | 7104 | 3008 | 960 | 0 |
|  | 16 | 8064 | 7552 | 7552 | 7552 | 3520 | 1472 | 448 |
|  | 32 | 3968 | 3712 | 3712 | 3712 | 3712 | 1728 | 704 |
| (*) In G.997.1, a 1ms delay is reserved to mean that Sp ≤ 1 and Dp= 1. (**)Values of INP_min in gray are optional. |  |  |  |  |  |  |  |  |

Table V.2/G.992.5 - Maximum downstream net data rate (kbit/s) 
for various values of delay_min = delay_max and INP_min.

|  |  | INP_min (**) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | ½ | 1 | 2 | 4 | 8 | 16 |
| delay_min delay_max (ms) | 1.33 (*) | 24432 | 6576 | 2448 | 432 | 0 | 0 | 0 |
|  | 2.67 (*) | 24432 | 14736 | 6576 | 2448 | 432 | 0 | 0 |
|  | 5.33 (*) | 24432 | 22896 | 14736 | 6576 | 2448 | 432 | 0 |
| (*) Set delay_max = ceil(delay) and delay_min = floor(delay). (**) Values of INP_min in gray are optional. |  |  |  |  |  |  |  |  |

Table V.3/G.992.5 - Maximum upstream net data rate (kbit/s) 
for various values of delay_min = delay_max and INP_min.

|  |  | INP_min (**) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | ½ | 1 | 2 | 4 | 8 | 16 |
| delay_min delay_max (ms) | 1 (*) | 3520 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 2 | 3520 | 3072 | 1472 | 448 | 0 | 0 | 0 |
|  | 4 | 3520 | 3264 | 1728 | 704 | 192 | 0 | 0 |
|  | 8 | 1920 | 1792 | 1792 | 832 | 320 | 64 | 0 |
|  | 16 | 896 | 832 | 832 | 832 | 384 | 128 | 0 |
|  | 32 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| (*) In G.997.1, a 1ms delay is reserved to mean that Sp ≤ 1 and Dp= 1. (**) Values of INP_min in gray are optional. |  |  |  |  |  |  |  |  |

Table V.4/G.992.5 - Range of MSGmin and minimum data rate granularity 
(net_max – net_min) when delay is selected from Tables V.1 or Table V.3.

| MSGmin (kbit/s) | Datarate granularity (kbit/s) |
| --- | --- |
| 60-64 | not supported |
| 29-60 | 64 |
| 14-28 | 32 |
| 6-13 | 16 |
| 4-5 | 8 |

Table V.5/G.992.5 - Range of MSGmin and minimum data rate granularity 
(net_max – net_min) when delay is selected from Table V.2.

| MSGmin (kbit/s) | data rate granularity (kbit/s) |
| --- | --- |
| 45-64 | not supported |
| 21-44 | 48 |
| 9-20 | 24 |
| 4-8 | 12 |

Appendix VI

Bibliography

[B1]	ITU-T Recommendation G.995.1 (2001), Overview of digital subscriber line (DSL) Recommendations.

_______________