|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.994.1 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (05/2003) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital sections and digital line system – Access networks |  |  |  |
|  | Handshake procedures for digital subscriber line (DSL) transceivers |  |  |  |
|  | ITU-T Recommendation G.994.1 |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| General | G.900–G.909 |
| Parameters for optical fibre cable systems | G.910–G.919 |
| Digital sections at hierarchical bit rates based on a bit rate of 2048 kbit/s | G.920–G.929 |
| Digital line transmission systems on cable at non-hierarchical bit rates | G.930–G.939 |
| Digital line systems provided by FDM transmission bearers | G.940–G.949 |
| Digital line systems | G.950–G.959 |
| Digital section and digital transmission systems for customer access to ISDN | G.960–G.969 |
| Optical fibre submarine cable systems | G.970–G.979 |
| Optical line systems for local and access networks | G.980–G.989 |
| Access networks | G.990–G.999 |
| QUALITY OF SERVICE AND PERFORMANCE – GENERIC AND USER-RELATED ASPECTS | G.1000–G.1999 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.6000–G.6999 |
| DIGITAL TERMINAL EQUIPMENTS | G.7000–G.7999 |
| DIGITAL NETWORKS | G.8000–G.8999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.994.1 Handshake procedures for digital subscriber line (DSL) transceivers

Summary This Recommendation provides a flexible mechanism for digital subscriber line (DSL) transceivers to exchange capabilities and to select a common mode of operation. It includes parameters relating to service and application requirements as well as parameters pertinent to various DSL transceivers. This Recommendation is currently an integral part of the start-up procedure for ITU-T Recs G.991.2, G.992.1, G.992.2, G.992.3, G.992.4 and G.992.5. It is anticipated that future DSL Recommendations will also be able to make use of this Recommendation. Provisions are also included for exchanging non-standard information.

Source ITU-T Recommendation G.994.1 was approved by ITU-T Study Group 15 (2001-2004) under the ITU-T Recommendation A.8 procedure on 22 May 2003.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g., interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met.  The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2004

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Definitions		2

4	Abbreviations		2

5	System reference diagram		3

6	Signals and modulation		4

6.1	Description of signals		4

6.2	Modulation		6

6.3	Transmit filter characteristics		7

7	Description of messages		7

7.1	CL – Capabilities List		7

7.2	CLR – Capabilities List + Request		7

7.3	MR – Mode Request		7

7.4	MS – Mode Select		7

7.5	MP – Mode Proposal		7

7.6	ACK(1) – Acknowledge, Type 1		7

7.7	ACK(2) – Acknowledge, Type 2		8

7.8	NAK-EF – Negative Acknowledge, Errored Frame		8

7.9	NAK-NR – Negative Acknowledge, Not Ready		8

7.10	NAK-NS – Negative Acknowledge, Not Supported		8

7.11	NAK-CD – Negative Acknowledge, Clear Down		8

7.12	REQ-MS – Request MS Message		8

7.13	REQ-MR – Request MR Message		8

7.14	REQ-CLR – Request CLR Message		8

7.15	REQ-RTX – Retransmission Message		9

8	Structure of messages		9

8.1	Format convention		9

8.2	Frame structure		9

8.3	Frame check sequence field		10

8.4	Octet transparency		10

8.5	Inter-frame time fill		11

9	Message coding format		11

9.1	General		11

9.2	Coding format for parameters in the I and S fields		11

9.3	Identification field (I)		14

9.4	Standard information field (S)		23

Page

9.5	Non-standard information field (NS)		128

9.6	Overall message composition		129

10	G.994.1 transactions		130

10.1	Basic transactions		130

10.2	Extended transactions		131

10.3	Message segmentation		133

10.4	State transition diagrams		133

10.5	Retransmission transactions		140

11	Start-up/cleardown procedures		141

11.1	Duplex start-up procedures		141

11.2	Half-duplex start-up procedures		142

11.3	Cleardown procedure		143

12	Error recovery procedures		145

Annex A – Support for legacy non-G.994.1 devices		146

Annex B – Operation over multiple wire pairs		146

Annex C – An operating mode for use in the same cable as TCM-ISDN		147

C.0	Scope (supplements 1)		147

C.1	Signals and modulation (pertains to 6)		147

C.2	Start-up/cleardown procedures (pertains to 11)		150

Appendix I – G.994.1 sample sessions		152

Appendix II – Provider code contact information		153

Appendix III – Support for legacy DMT-based devices		154

Appendix IV – Procedure for the assignment of additional G.994.1 parameters		154

IV.1	Introduction		154

IV.2	Procedure		154

Appendix V – Rules for code point table numbering		155

Appendix VI – Bibliography		155

ITU-T Recommendation G.994.1

Handshake procedures for digital subscriber line (DSL) transceivers

# 1	Scope

This Recommendation defines signals, messages, and procedures for exchanging these between digital subscriber line (DSL) equipment, when the modes of operation of the equipment need to be automatically established and selected, but before signals are exchanged which are specific to a particular DSL Recommendation.

For interrelationships of this Recommendation with other G.99x-series Recommendations, see ITU-T Rec. G.995.1.

The principal characteristics of this Recommendation are as follows:

a)	use over metallic local loops;

b)	provisions to exchange capabilities information between DSL equipment to identify common modes of operation;

c)	provisions for DSL equipment at either end of the loop to select a common mode of operation or to request the other end to select the mode;

d)	provisions for exchanging non-standard information between DSL equipment;

e)	provisions to exchange and request service and application related information;

f)	support for both duplex and half-duplex transmission modes;

g)	support for multi-pair operation;

h)	provisions for DSL equipment at the remote end of the loop (xTU-R) to propose a common mode of operation (new in Version 2 of this Recommendation through the use of new message type MP and its associated transactions);

i)	support for a retransmission mechanism (new in Version 3 of this Recommendation through the use of new message type REQ-RTX).

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

–	ITU-T Recommendation G.991.2 (2001), Single-pair high-speed digital subscriber line (SHDSL) transceivers.

–	ITU-T Recommendation G.992.1 (1999), Asymmetric digital subscriber line (ADSL) transceivers, plus Amendment 1 (2003).

–	ITU-T Recommendation G.992.2 (1999), Splitterless asymmetric digital subscriber line (ADSL) transceivers.

–	ITU-T Recommendation G.992.3 (2002), Asymmetric digital subscriber line transceivers 2 (ADSL2).

–	ITU-T Recommendation G.992.4 (2002), Splitterless asymmetric digital subscriber line transceivers 2 (Splitterless ADSL2).

–	ITU-T Recommendation G.992.5 (2003), Asymmetric digital subscriber line (ADSL) transceivers – Extended bandwidth ADSL2 (ADSL2+).

–	ITU-T Recommendation G.997.1 (2003), Physical layer management for digital subscriber line (DSL) transceivers.

–	ITU-T Recommendation T.35 (2000), Procedure for the allocation of ITU-T defined codes for non-standard facilities.

–	ISO/IEC 3309:1993, Information technology – Telecommunications and information exchange between systems – High-level data link control (HDLC) procedures – Frame structure.

# 3	Definitions

This Recommendation defines the following terms:

3.1	carrier set: A set of one or more frequencies associated with the PSD mask of a particular xDSL Recommendation.

3.2	downstream: The direction of transmission from the xTU-C to the xTU-R.

3.3	errored frame: A frame that contains a frame check sequence (FCS) error.

3.4	Galf: An octet of value 8116, i.e., the ones complement of an HDLC flag.

3.5	initiating signal: A signal that initiates the start-up procedure of a G.994.1 session.

3.6	initiating station: The station that initiates the start-up procedure of a G.994.1 session.

3.7	invalid frame: A frame that has fewer than four octets between flags, excluding transparency octets.

3.8	message: Framed information conveyed via modulated transmission.

3.9	responding signal: A signal that is sent in response to an initiating signal.

3.10	responding station: The station that responds to the initiation of the start-up procedure of a G.994.1 session.

3.11	session: A G.994.1 session comprises a start-up procedure, one or more transactions, and a cleardown procedure (except as noted in clause 12).

3.12	signalling family: A group of carrier sets which are integral multiples of a given carrier spacing frequency.

3.13	subcarrier: Refer to the associated xDSL Recommendation for the definition of this term.

3.14	transaction: A sequence of G.994.1 messages, ending with either a positive acknowledgement [ACK(1) (except as noted in 7.6)], a negative acknowledgement (NAK), or a time-out (see clause 12).

3.15	upstream: The direction of transmission from the xTU-R to the xTU-C.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

ACK	Acknowledge Message

ADSL	Asymmetric Digital Subscriber Line

CL	Capabilities List

CLR	Capabilities List Request

FCS	Frame Check Sequence

HSTU	Handshake Transceiver Unit

ISO	International Organization for Standardization

ITU-T	International Telecommunication Union – Telecommunication Standardization Sector

LSB	Least Significant Bit

MP	Mode Proposal Message

MR	Mode Request Message

MS	Mode Select Message

MSB	Most Significant Bit

NAK		Negative Acknowledge Message

PSTN	Public Switched Telephone Network

REQ	Request Message Type Message

xDSL	Any of the various types of Digital Subscriber Lines (DSL)

xTU-C	xDSL central site terminal unit

xTU-R	xDSL remote terminal unit

# 5	System reference diagram

Figure 1 illustrates the system reference model for this Recommendation.

Figure 1/G.994.1 – System reference model

The system reference model identifies the necessary functional blocks and the reference points and/or interface points that may be used or have implications in this Recommendation.

The system reference model comprises the following blocks:

•	transceivers: HSTU-C and HSTU-R;

•	splitter function;

•	loop plant;

•	user terminal;

•	POTS set;

•	CO line circuit.

This Recommendation defines the signals, messages and procedures for common mode selection, and as such only relates to the functions associated with the HSTU-C and HSTU-R blocks. HSTU is used to signify that the signals, messages and procedures defined in this Recommendation are different from the G.99x Recommendations that use this Recommendation as a common start-up procedure. The remaining blocks are included to illustrate a reference system.

Splitters may or may not be present at the central and remote sites and are therefore signified by dotted boxes. If splitters are present, the HSTU-C (HSTU-R) may be connected to either or to both the U-C (U-R) and U-C2 (U-R2) interface points, signified by the dotted connecting lines.

The reference points V, U, T/S may not need to be exposed. This system reference model does not imply any particular implementation of the signals, messages, and procedures defined in this Recommendation.

The HSTU negotiates modes of operation on behalf of one or more xDSL terminal units that are hereafter referred to as xTU.

# 6	Signals and modulation

## 6.1	Description of signals

The signalling families used in this Recommendation, and the carrier sets defined within each family, are described in this clause.

Each xDSL mode of operation has a mandatory carrier set associated with it. For each xDSL mode of operation implemented by a G.994.1 station, the initial G.994.1 transmission from the station shall include the associated mandatory carrier set for that mode.

NOTE 1 – In the interest of explicitly indicating the presence of HSTU-x that might not have common modes, the initial transmission should use as many carriers as possible, and HSTU-x are encouraged to detect all carriers from all signalling families.

Both duplex and half-duplex transmission modes are defined for use within this Recommendation. The transmission mode supported is a function of carrier set and is specified in Tables 1 and 3.

Frequencies other than those specified in Tables 1 and 3 shall not be transmitted simultaneously with G.994.1 signals.

The tolerance of the symbol rate and carrier frequencies for an HSTU-C shall be 50 ppm. The tolerance of the symbol rate and carrier frequencies for an HSTU-R shall be 200 ppm during R-TONES-REQ and 50 ppm during and after R-TONE1 (in duplex transmission mode) or R-FLAG1 (in half-duplex transmission mode). The HSTU-R may perform partial or full loop timing acquisition during the silence period preceding R-TONE1 or R-FLAG1. For both the HSTU-C and HSTU-R transmitter, the symbol rate and carrier frequencies shall be time locked.

NOTE 2 – National regulations may limit signal transmission to those carriers that fall within the PSD mask of the xDSL mode(s) supported by the station.

NOTE 3 – It is advised to monitor for existing services prior to transmitting signals to avoid interfering with them.

NOTE 4 – New xDSL services that wish to use this Recommendation are strongly encouraged to use currently defined carrier sets rather than defining new carrier sets.

NOTE 5 – The need for any additional carrier set(s) to support the VDSL specifications identified in Table 11.0.1 is for further study.

### 6.1.1	4.3125 kHz signalling family

Carrier frequencies within this signalling family are given by N × 4.3125 kHz, where N is a positive integer. The symbol rate shall be 4312.5/8  539.0625 symbols per second.

Within this family, there are three upstream carrier sets, designated A43, B43 and C43. Each upstream carrier set has an associated downstream carrier set that carries the same designation. The carrier set frequencies and the maximum transmit power level per carrier for each carrier set are defined in Table 1 where frequency = N × 4.3125 kHz.

The carrier sets in this family are mandatory for the xDSL modes listed in Table 2. One or more carriers listed in Tables 1 or 3 may be transmitted in addition to the mandatory carrier set listed in Table 2. Carriers not listed in Tables 1 or 3 shall not be transmitted.

Table 1/G.994.1 – Carrier sets for the 4.3125 kHz signalling family

| Carrier set designation | Upstream carrier sets |  | Downstream carrier sets |  | Transmission mode |
| --- | --- | --- | --- | --- | --- |
|  | Frequency indices (N) | Maximum power level/carrier (dBm) | Frequency indices (N) | Maximum power level/carrier (dBm) |  |
| A43 | 9 17 25 | –1.65 | 40 56 64 | –3.65 | Duplex only |
| B43 | 37 45 53 | –1.65 | 72 88 96 | –3.65 | Duplex only |
| C43 | 7 9 | –1.65 | 12 14 64 | –3.65 | Duplex only |
| J43 | 9 17 25 | –1.65 | 72 88 96 | –3.65 | Duplex only |

Table 2/G.994.1 – Mandatory carrier sets

| xDSL Recommendation(s) | Carrier set designation |
| --- | --- |
| G.992.1 – Annex A, G.992.2 – Annexes A/B, G.992.3 – Annexes A/I, G.992.4 – Annexes A/B | A43 |
| G.992.1 – Annex B, G.992.3 – Annex B | B43 |
| G.992.1 – Annex C, G.992.2 – Annex C, G.992.1 – Annex H | C43 |
| G.992.3 – Annex J | J43 |

### 6.1.2	4 kHz signalling family

Carrier frequencies within this signalling family are given by N  4 kHz, where N is a positive integer. The symbol rate shall be 4000/5  800 symbols per second.

Within this family there is one upstream carrier set, designated A4. The associated downstream carrier set has the same designation. The carrier frequencies and the maximum transmit power level per carrier are defined in Table 3, where frequency  N  4 kHz.

The carrier sets in this family are mandatory for the xDSL modes listed in Table 4. One or more carriers listed in Tables 1 or 3 may be transmitted in addition to the mandatory carrier set listed in Table 4. Carriers not listed in Tables 1 or 3 shall not be transmitted.

Table 3/G.994.1 – Carrier sets for the 4 kHz signalling family

| Carrier set designation | Upstream carrier sets |  | Downstream carrier sets |  | Transmission mode |
| --- | --- | --- | --- | --- | --- |
|  | Frequency indices (N) | Maximum power level/carrier (dBm) | Frequency indices (N) | Maximum power level/carrier (dBm) |  |
| A4 | 3 | +5 | 5 | +5 | Half-duplex only |

Table 4/G.994.1 – Mandatory carrier sets

| xDSL Recommendation(s) | Carrier set designation |
| --- | --- |
| G.991.2 | A4 |

## 6.2	Modulation

All messages in G.994.1 are sent with one or more carrier sets. All carrier frequencies within a carrier set, and all carrier sets are simultaneously modulated with the same data bits using Differentially encoded binary Phase Shift Keying (DPSK). The transmit point is rotated 180° from the previous point if the transmit bit is a 1, and the transmit point is rotated 0° from the previous point if the transmit bit is a 0.

For each signal transmitted, the transmit signal shall have a rectangular pulse shaping, defined as:

where:

	means signal multiplication

	means signal convolution

ƒi	are the G.994.1 carrier frequencies (defined in 6.1)

i	are the G.994.1 carrier phases (discretionary constants)

T	is the symbol period

T =	(8/4312.5) seconds for the 4.3125 kHz signalling family and (5/4000) seconds for the 4 kHz signalling family

An =	1 or –1 and is differentially encoded according to:

An = A n–1 if bn  0;

An = –An–1 if bn  1.

bn	 represents the bit transmitted in symbol n

rect(t)	is a rectangular pulse shape defined by:

rect(t) 	1 if |t| < T/2;

	0 otherwise;

htx	represents the impulse response of the transmit filter.

## 6.3	Transmit filter characteristics

### 6.3.1	4.3125 kHz signalling family

For the 4.3125 kHz signalling family, the transmit filter shall have a bandwidth such that all of the  points of the filter shall have frequencies which differ at least 4.3125 kHz from any G.994.1 carrier frequency used.

### 6.3.2	4 kHz signalling family

The transmit spectrum is shaped by the rectangular pulse filtering of the modulation; no additional shaping is needed to be introduced by the transmit filter. Therefore, for the 4 kHz signalling family, the transmit filter shall not introduce significant distortion to the rectangular pulse shaping. Specifically, at frequencies less than 4 kHz from the carrier frequency, the magnitude response of the transmit filter shall have less than 0.5 dB variation from the magnitude response at the carrier frequency. At other frequencies the magnitude response of the transmit filter shall not exceed the magnitude response at the carrier frequency. The magnitude response at the carrier frequency is such that the maximum power requirements in Table 3 are satisfied. Variation in the group delay of the transmit filter should not be excessive at frequencies that are less than 4 kHz from the carrier frequency.

# 7	Description of messages

## 7.1	CL – Capabilities List

This message may be sent by an HSTU-C in response to the reception of either a complete CLR message, or an intermediate frame of a segmented CLR message. It conveys a list of possible modes of operation of the xTU-C.

## 7.2	CLR – Capabilities List + Request

This message may be sent by an HSTU-R. It conveys a list of possible modes of operation of the xTU-R and requests the transmission of a CL message by the HSTU-C.

## 7.3	MR – Mode Request

This message may be sent by an HSTU-R. It requests the transmission of an MS message by the HSTU-C.

## 7.4	MS – Mode Select

This message may be sent by an HSTU-C or an HSTU-R. It requests the initiation of a particular mode of operation.

## 7.5	MP – Mode Proposal

This message may be sent by an HSTU-R. It proposes a particular mode of operation and requests the transmission of an MS message by the HSTU-C.

## 7.6	ACK(1) – Acknowledge, Type 1

This message either:

•	acknowledges receipt of a complete CL message or an intermediate frame of a segmented CL message and ends a G.994.1 transaction; or

•	acknowledges receipt of a complete MS message or an intermediate frame of a segmented MS message and initiates the G.994.1 session cleardown procedure specified in 11.3.

## 7.7	ACK(2) – Acknowledge, Type 2

This message acknowledges receipt of an intermediate frame of a segmented CL, CLR, MP or MS message and requests the transmission of the next frame of the message.

## 7.8	NAK-EF – Negative Acknowledge, Errored Frame

This message may be sent by an HSTU-C or an HSTU-R in response to the reception of an errored frame. It aborts a G.994.1 session in accordance with the error recovery procedure specified in clause 12.

## 7.9	NAK-NR – Negative Acknowledge, Not Ready

This message may be sent by an HSTU-C or an HSTU-R to acknowledge receipt of a complete MS message or an intermediate frame of a segmented MS message. It ends a G.994.1 transaction. It indicates that the receiving station is temporarily unable (i.e., Not Ready) to invoke the mode requested by the transmitting station but wishes to continue the G.994.1 session.

## 7.10	NAK-NS – Negative Acknowledge, Not Supported

This message may be sent by an HSTU-C or an HSTU-R to acknowledge receipt of a complete MP or MS message or an intermediate frame of a segmented MP or MS message. It ends a G.994.1 transaction. For a received message with the same or lower G.994.1 version number, transmission of NAK-NS indicates that the receiving station either does not support or has disabled the mode requested by the transmitting station. For a received message with a higher G.994.1 version number, NAK-NS indicates either reception of an unknown message type or reception of an unexpected message according to the transactions defined in this version of this Recommendation.

## 7.11	NAK-CD – Negative Acknowledge, Clear Down

This message may be sent in response to any frame of a message other than one of the NAK messages. It indicates that the received information was not understood. This may be due to an unknown message type (same or lower G.994.1 version level), an unexpected message type according to the defined transactions (same or lower G.994.1 version level), or a syntax error while parsing the message. Since a NAK-CD normally indicated a lack of compliance with this Recommendation, it initiates the G.994.1 session cleardown procedure specified in 11.3.

## 7.12	REQ-MS – Request MS Message

This message may be sent by an HSTU-C in response to the reception of an MR message. It requests the transmission of an MS message by the HSTU-R. It indicates that the HSTU-C does not wish to select a mode and is deferring the mode selection to the HSTU-R.

## 7.13	REQ-MR – Request MR Message

This message may be sent by an HSTU-C in response to the reception of a complete MS message or an intermediate frame of a segmented MS message. It requests the transmission of an MR message by the HSTU-R. It indicates that the HSTU-C wishes to select the mode.

## 7.14	REQ-CLR – Request CLR Message

This message may be sent by an HSTU-C in response to the reception of either an MR message, a complete MP or MS message, or an intermediate frame of a segmented MP or MS message. It requests the transmission of a CLR message by the HSTU-R. It indicates that the HSTU-C wishes to perform a capabilities exchange.

## 7.15	REQ-RTX – Retransmission Message

This message may be sent instead of the NAK-EF message by an HSTU-C or an HSTU-R in response to the reception of an errored frame. It requests the retransmission of the errored frame.

# 8	Structure of messages

A message consists of one or more segments. Each segment is encapsulated in a frame.

## 8.1	Format convention

The basic format convention used for messages is illustrated in Figure 2. Bits are grouped into octets. The bits of each octet are shown horizontally and are numbered from 1 to 8. Octets are displayed vertically and are numbered from 1 to N.

The octets are transmitted in ascending numerical order. Within an octet, bit 1 is the first bit to be transmitted.

For fields that are contained within a single octet, the lowest numbered bit of the field represents the least significant bit (20). When a field spans multiple octets, the lowest numbered bit of the field in the highest numbered octet containing the field represents the least significant bit (20). The order of bit values within each octet increases as the bit number increases. The order of bit values from octet to octet increases as the octet number decreases. Figure 3 illustrates a field that spans two octets.

An exception to this convention is the Frame Check Sequence (FCS) field, which spans two octets. In this case, the order of bit values within the octets is reversed. Bit 1 of the first octet is the MSB and bit 8 of the second octet is the LSB (Figure 4).

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | 1 |
|  |  |  |  |  |  |  |  | 2 |
|  |  |  |  |  |  |  |  | ... |
|  |  |  |  |  |  |  |  | N |

Figure 2/G.994.1 – Format convention

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 24 | 23 | 22 | 1st octet of field |
| 21 | 20 |  |  |  |  |  |  | 2nd octet of field |

Figure 3/G.994.1 – Field mapping convention

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28 |  |  |  |  |  |  | 215 | 1st octet of field |
| 20 |  |  |  |  |  |  | 27 | 2nd octet of field |

Figure 4/G.994.1 – FCS mapping convention

## 8.2	Frame structure

The frame structure is shown in Figure 5. The contents of a frame shall consist of an integer number of octets.

Frames shall begin and end with standard HDLC flag octets (01111110) as defined in ISO/IEC 3309. At least three but not more than five flags shall be sent to begin a frame. At least two but not more than three flags shall follow the FCS of each frame.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Flag |  |  |  |  |  |  |  | 1 |
| Flag |  |  |  |  |  |  |  | 2 |
| Flag |  |  |  |  |  |  |  |  |
| Flag (optional) |  |  |  |  |  |  |  |  |
| Flag (optional) |  |  |  |  |  |  |  |  |
| Message segment |  |  |  |  |  |  |  |  |
| FCS (first octet) |  |  |  |  |  |  |  |  |
| FCS (second octet) |  |  |  |  |  |  |  |  |
| Flag |  |  |  |  |  |  |  | N – 2 |
| Flag |  |  |  |  |  |  |  | N – 1 |
| Flag (optional) |  |  |  |  |  |  |  | N |

Figure 5/G.994.1 – Frame structure

## 8.3	Frame check sequence field

The FCS field is 16 bits (2 octets) in length. As defined in ISO/IEC 3309, it shall be the ones complement of the sum (modulo 2) of:

a)	the remainder of xk (x15  x14  x13  x12  x11  x10  x9  x8  x7  x6  x5  x4  x3  x2  x  1) divided (modulo 2) by the generator polynomial x16  x12  x5  1, where k is the number of bits in the frame existing between, but not including, the last bit of the final opening flag and the first bit of the FCS, excluding octets inserted for transparency; and

b)	the remainder of the division (modulo 2) by the generator polynomial x16  x12  x5  1, of the product of x16 by the content of the frame existing between, but not including, the last bit of the final opening flag and the first bit of the FCS, excluding octets inserted for transparency.

As a typical implementation at the transmitter, the initial content of the register of the device computing the remainder of the division is preset to all binary ONEs and is then modified by division by the generator polynomial (as described above) on the information field. The ones complement of the resulting remainder is transmitted as the 16-bit FCS.

As a typical implementation at the receiver, the initial content of the register of the device computing the remainder of the division is preset to all binary ONEs. The final remainder, after multiplication by x16 and then division (modulo 2) by the generator polynomial x16  x12  x5  1 of the serial incoming protected bits and the FCS after removal of the transparency octets, will be 00011101000011112 (x15 through x0, respectively) in the absence of transmission errors.

## 8.4	Octet transparency

G.994.1 messages use the octet transparency method defined in ISO/IEC 3309. In this approach, any data that is equal to 7E16 (the Flag Sequence) or 7D16 (the Control Escape) are escaped as described below.

After Frame Check Sequence (FCS) computation, the transmitter examines the entire frame between the two Flag Sequences. Any data octets which are equal to the Flag Sequence or the Control Escape are replaced by a two-octet sequence consisting of the Control Escape octet followed by the original octet Exclusive-OR'ed with hexadecimal 2016. In summary, the following substitutions are made:

•	a data octet of 7E16 is encoded as two octets {7D16, 5E16};

•	a data octet of 7D16 is encoded as two octets {7D16, 5D16}.

On reception, prior to FCS computation, each Control Escape octet (7D16) is removed, and the following octet is exclusive-OR'ed with hexadecimal 2016 (unless the following octet is 7E16, which is the flag, and indicates the end of frame, and therefore an abort has occurred). In summary, the following substitutions are made:

•	a sequence of 7D16, 5E16 is replaced by the data octet 7E16;

•	a sequence of 7D16, 5D16 is replaced by the data octet 7D16;

•	a sequence of 7D16, 7E16 aborts the frame.

Since octet stuffing is used, the frame is guaranteed to have an integer number of octets.

## 8.5	Inter-frame time fill

In duplex mode, an integer number of flags shall be transmitted between frames. In half-duplex mode, silence shall be transmitted between frames.

# 9	Message coding format

## 9.1	General

The message information field consists of three components:

a)	an identification field (I); followed by

b)	a standard information field (S); and

c)	an optional non-standard information field (NS).

This general structure is shown in Figure 6.

| Identification (I) field | Standard information (S) field | Non-standard information (NS) field |
| --- | --- | --- |

Figure 6/G.994.1 – Information field structure

## 9.2	Coding format for parameters in the I and S fields

In both the I and S fields most of the information to be conveyed consists of parameters relating to particular modes, features or capabilities associated with the two stations.

In order to:

a)	encode these parameters in accordance with a consistent set of rules; and

b)	allow future extension to the parameter list in a way that permits present and future G.994.1 implementations to correctly parse the information field,

the parameters are linked together in a predefined tree structure. The order in which the parameters in the tree are transmitted and the use of delimiting bits that enable the tree to be reconstructed at the receiver are described in the rules set out below.

### 9.2.1	Parameter classification

Parameters (Pars) are classified as:

•	NPars – Parameters which have no subparameters associated with them; and

•	SPars – Parameters that have subparameters associated with them.

The general structure of this tree is as shown in Figure 7.

At level 1, the highest level of the tree, each SPar has associated with it a series of Pars (NPars and possibly SPars) at level 2 in the tree. At level 2 in the tree, each SPar has associated with it a series of NPars at level 3 in the tree. Level 3 is the lowest level in the tree. Therefore, there are no SPars at this level.

Figure 7/G.994.1 – Tree structure linking parameters in the I and S fields

### 9.2.2	Order of transmission of parameters

Parameters are binary encoded and transmitted serially. Parameters of the same type (i.e., level, classification and association) are transmitted sequentially as a parameter block consisting of an integral number of octets.

The order of transmission of NPars and SPars is specified in Figure 8.

Figure 8/G.994.1 – Order of transmission of NPars and SPars

Par(2)n indicates a set of level 2 parameters associated with the nth level 1 SPar, and consists of NPar(2)n parameters and possibly SPar(2)n parameters.

NPar(3)n,m indicates a set of level 3 NPars associated with the mth level 2 SPar which in turn is associated with the nth level 1 SPar.

Transmission of parameters begins with the first octet of NPar(1) and ends with the last octet of Par(2)N.

The order of transmission of the Par(2) blocks is the same as the order of transmission of the corresponding SPar(1) bits. Similarly, the order of transmission of the NPar(3)n blocks is the same as the order of transmission of the corresponding SPar(2)n bits.

### 9.2.3	Delimiting and parsing of parameter blocks

The use of delimiting bits is illustrated in Figure 8. Within each octet of a parameter block, at least one bit is defined as a delimiting bit. This is used to define the last octet in the block to be transmitted. A binary ZERO in this bit position indicates that there is at least one additional octet in the block to be transmitted. A binary ONE in this bit position indicates the last octet in the block to be transmitted.

Bit 8 is used to delimit the NPar(1) block, the SPar(1) block, and each of the Par(2) blocks. There are N of these Par(2) blocks, one for each of the capabilities in the SPar(1) block that is enabled (set to binary ONE).

In order for this parsing rule to function correctly, both the identification field (I) and the standard information field (S) shall include at least one octet of NPar(1) and at least one octet of SPar(1).

Bit 7 is used to delimit each NPar(2) block, each SPar(2) block, and each of the associated NPar(3) blocks. Figure 8 indicates that there are M of these NPar(3) blocks, one for each of the capabilities in the SPar(2)n block that is enabled (set to binary ONE). M may be different for each of the Par(2) blocks.

A Par(2) block may either contain both NPar(2) and SPar(2) octets, or NPar(2) octets alone. To indicate that a Par(2) block contains only NPar(2) octets, bits 7 and 8 are both set to binary ONE in the last NPar(2) octet to be transmitted.

Bits 1 through 7 at level 1 of the tree and bits 1 through 6 at levels 2 and 3 of the tree may be used to encode parameters.

Octets at the end of any Par block that would have contained all ZEROs except for delimiting bits may be omitted from transmission, provided that terminating bits are correctly set for the transmitted octets.

To allow for compatibility with future versions of this Recommendation, receivers shall parse all parameter blocks and ignore information that is not understood. However, to be able to correctly parse the parameter blocks, it is necessary to pay attention to the number of SPar(1) and SPar(2) bits that are set, even if the meaning of one or more of these bits is not understood.

## 9.3	Identification field (I)

The identification field consists of four components:

a)	a one-octet message type field; followed by

b)	a one-octet version number field;

c)	an additional information field; and

d)	a bit-encoded parameter field.

This general structure is shown in Figure 9.

| Message type field | Version number field | Additional information field | Bit-encoded parameter field |
| --- | --- | --- | --- |

Figure 9/G.994.1 – Identification field structure

### 9.3.1	Message type field

The purpose of the message type field is to identify the message type of the frame. The field is one octet in length and occupies the first octet in the identification field. The tree structure encoding rules specified in 9.2 are not applicable to this field. The encoding shall be as shown in Table 5. An "X" indicates that the message type is supported for the stated version number (see 9.3.2) whilst a "–" indicates that it is not.

NOTE – Message types other than those specified in Table 5 are reserved for allocation by the ITU-T.

Table 5/G.994.1 – Message type field format

| Bits |  |  |  |  |  |  |  |  | Message type | G.994.1 Version 1 support | G.994.1 Version 2 support | G.994.1 Version 3 support |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |
|  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | MS | X | X | X |
|  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | MR | X | X | X |
|  | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | CL | X | X | X |
|  | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | CLR | X | X | X |
|  | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | MP | – | X | X |
|  | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | ACK(1) | X | X | X |
|  | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | ACK(2) | X | X | X |
|  | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | NAK-EF | X | X | X |
|  | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | NAK-NR | X | X | X |
|  | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | NAK-NS | X | X | X |
|  | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | NAK-CD | X | X | X |
|  | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | REQ-MS | X | X | X |
|  | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | REQ-MR | X | X | X |
|  | 0 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | REQ-CLR | X | X | X |
|  | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | REQ-RTX | – | – | X |

### 9.3.2	Version number field

The purpose of the version number field is to identify the version number of G.994.1 to which the equipment conforms.

This Recommendation includes the functionality of G.994.1 – 2002 (Version 2) in its entirety and therefore replaces it. It also specifies new functionality by way of a new message type and new transactions, resulting in Version 3.

G.994.1 – 2002 (Version 2), in turn, included the functionality of G.994.1 – 1999 (Version 1) in its entirety and therefore replaced it. It also specified new functionality by way of a new message type and new transactions, resulting in Version 2.

Tables 5, 13 and 14 indicate the message types, basic transactions and extended transactions respectively that are supported by each version. All transmitted messages shall indicate the highest version number supported by the transmitting equipment, independent of the version number received in any message.

NOTE 1 – The version number of G.994.1 is not updated with the addition of new codepoints. The version number is updated when a structural change is made. Examples of structural changes are the addition of new message types and new transactions. To ensure backward compatibility, future versions with higher version numbers will include, without change, all of the existing transactions, messages and information from previous versions. There is no relationship between the G.994.1 version number and the actual version in which particular parameters are first defined. For example, parameters defined for the first time in this version, such as those for Annex A/G.992.5, may be used by a Version 1 or 2 implementation of G.994.1.

If the received message is an MS, it is the expected message according to the defined transactions, and it can be parsed correctly; it shall be ACKed if the features are supported, independent of the version number of the message.

If the received message is of an unknown type or is not the expected message according to the defined transactions, and the version number field indicates a higher version, a NAK-NS shall be sent.

The field is one octet in length and occupies the second octet of the identification field. The tree structure encoding rules specified in 9.2 are not applicable to this field. The encoding shall be in accordance with Table 6.

NOTE 2 – Version numbers other than those specified in Table 6 are reserved for allocation by the ITU-T.

Table 6/G.994.1 – Version number field format

|  | Bits |  |  |  |  |  |  |  | Version number |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | Version 1 |
|  | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | Version 2 |
|  | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | Version 3 |

### 9.3.3	Additional information field

The additional information field consists of either a vendor ID information block or a retransmission information block according to Table 12.

#### 9.3.3.1	Vendor ID information block

Coding for the Vendor ID information block is shown in Table 7. The tree structure encoding rules specified in 9.2 are not applicable to this field. For MP, MR, MS, ACK, NAK and REQ messages, the vendor ID field is not used and is therefore of zero length.

NOTE – This field should typically identify the vendor of the G.994.1 functionality, whether implemented in hardware or software. It is not intended to indicate the system integrator.

Table 7/G.994.1 – Vendor ID information block

T.35 country code (2 octets – Note 1)

Provider code (vendor identification) (4 octets – Note 2)

Vendor-specific information (2 octets)

NOTE 1 – If the bits in the first octet are not all set to binary ONE, the bits in the second octet shall be set to binary ZERO by the transmitter and ignored by the receiver. The only purpose of the country code is to identify the country of registry of the provider code. NOTE 2 – Specification of the coding and order of transmission of this field is the responsibility of the regional standards body allocating the provider code. See Appendix II for provider code contact information.

#### 9.3.3.2	Retransmission information block

Coding of the Retransmission information block is shown in Table 7.1. The tree structure encoding rules specified in 9.2 are not applicable to this field. The Retransmission information block shall only be used for REQ-RTX messages. For all other message types, the Retransmission information block is not used and is therefore of zero length.

Table 7.1/G.994.1 – Retransmission information block

Last Correctly Received Message (LCRM) (1 octet – Note 1)

Multi Segment Frame Number (MSFN) (1 octet – Note 2)

NOTE 1 – This octet contains the message type code of the last correctly received frame. The NULL message code (FF16) shall be used if an error free frame has not been received in the current G.994.1 session. NOTE 2 – This octet contains the segment index number of a message that has been segmented into more than one frame. The first segment or message contained in one frame shall have an MSFN value of 0. The second segment shall have an MSFN value of 1, and so on. Although the segmented frames are not explicitly numbered, the HSTU-R and HSTU-C shall maintain internal counters.

### 9.3.4	Parameter field

This field contains parameters that are independent of the mode to be selected and are typically either service or application related.

The parameter field of CL, CLR, MP and MS messages is encoded in accordance with the rules described in 9.2. For MR, ACK, NAK and REQ messages, the parameter field is not used and is therefore of zero length.

The parameter field consists of a set of octets in which each parameter is assigned a unique bit position. A binary ONE in the assigned bit position indicates that the parameter is valid. The validity of multiple parameters may be conveyed by transmitting a binary ONE in each bit position corresponding to a valid parameter.

The NPars and SPars are listed below, beginning with Table 8 until Table 9.31.

Table 8/G.994.1 – Identification field – NPar(1) coding

| Bits |  |  |  |  |  |  |  |  | NPar(1)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Reserved for allocation by the ITU-T |
|  | x | x | x | x | x | x | 1 | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | 1 | x | x | x | x | x | x | Non-standard field |
|  | x | 0 | 0 | 0 | 0 | 0 | 0 | 0 | No parameters set in this octet |

Table 9/G.994.1 – Identification field – SPar(1) coding – Octet 1

| Bits |  |  |  |  |  |  |  |  | SPar(1)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Net data rate upstream (Note 1) |
|  | x | x | x | x | x | x | 1 | x | Net data rate downstream (Note 1) |
|  | x | x | x | x | x | 1 | x | x | Data flow characteristics upstream (Note 2) |
|  | x | x | x | x | 1 | x | x | x | Data flow characteristics downstream (Note 2) |
|  | x | x | x | 1 | x | x | x | x | xTU-R splitter information (Note 3) |
|  | x | x | 1 | x | x | x | x | x | xTU-C splitter information (Note 3) |
|  | x | 1 | x | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | 0 | 0 | 0 | 0 | 0 | 0 | 0 | No parameters set in this octet |
| NOTE 1 – In a CLR, CL, or MP message, the data rates indicated in this Recommendation are of informative nature and do not imply any requirements on the net data rate during data mode. The data rate values are set and used by layers higher than the xTU-x although an xTU-x may monitor the information. The data rate values are useful to help the higher layers to select between the various G.99x.x transceivers based upon information indicated by an application layer. In an MS message: • If the xTU-x is capable of supporting the information, it shall respond with an ACK. • If the xTU-x is either not capable of supporting the information or the negotiation of this information, it shall respond with a NAK-NS. NOTE 2 – In a CLR, CL, or MP message, the latencies indicated in this Recommendation are of informative nature and do not imply any requirements on the latency during data mode. The latency values are set and used by layers higher than the xTU-x although an xTU-x may monitor the information. The latency values are useful to help the higher layers to select between the various G.99x.x transceivers and coding parameters based upon information indicated by an application layer. In an MS message: • If the xTU-x is capable of supporting the information, it shall respond with an ACK. • If the xTU-x is either not capable of supporting the information or the negotiation of this information, it shall respond with a NAK-NS. NOTE 3 – Splitter information is only conveyed by an xTU-x if it has the capability of determining the local splitter information. Splitter information is only a capabilities indication and shall not be included in an MP or MS message. |  |  |  |  |  |  |  |  |  |

Table 9.0.1/G.994.1 – Identification field – SPar(1) coding – Octet 2

| Bits |  |  |  |  |  |  |  |  | SPar(1)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Relative power level/carrier for upstream carrier set A43 (Note) |
|  | x | x | x | x | x | x | 1 | x | Relative power level/carrier for downstream carrier set A43 (Note) |
|  | x | x | x | x | x | 1 | x | x | Relative power level/carrier for upstream carrier set B43 (Note) |
|  | x | x | x | x | 1 | x | x | x | Relative power level/carrier for downstream carrier set B43 (Note) |
|  | x | x | x | 1 | x | x | x | x | Relative power level/carrier for upstream carrier set C43 (Note) |
|  | x | x | 1 | x | x | x | x | x | Relative power level/carrier for downstream carrier set C43 (Note) |
|  | x | 1 | x | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | 0 | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |
| NOTE – The relative power level/carrier reported in a CLR, CL, MP, or MS message indicates the level used during the current G.994.1 session, including the start-up and cleardown procedures. It does not imply any requirements on the transmit power in this or future sessions. |  |  |  |  |  |  |  |  |  |

Table 9.0.2/G.994.1 – Identification field – SPar(1) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | SPar(1)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Relative power level/carrier for upstream carrier set A4 (Note) |
|  | x | x | x | x | x | x | 1 | x | Relative power level/carrier for downstream carrier set A4 (Note) |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | 1 | x | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | 0 | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |
| NOTE – The relative power level/carrier reported in a CLR, CL, MP, or MS message indicates the level used during the current G.994.1 session, including the start-up and cleardown procedures. It does not imply any requirements on the transmit power in this or future sessions. |  |  |  |  |  |  |  |  |  |

Table 9.1/G.994.1 – Identification field – Net data rate upstream NPar(2) coding – Octet 1

| Bits |  |  |  |  |  |  |  |  | Net data rate upstream NPar(2)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 1 | 1 | 1 | 1 | 1 | 1 | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Unspecified by terminal |
|  | x | x | 1 | x | x | x | x | x | Maximum net data rate upstream (bits 5-1 × 2 Mbit/s) |
|  | x | x | 0 | x | x | x | x | x | Maximum net data rate upstream (bits 5-1 × 64 kbit/s) |

Table 9.1.1/G.994.1 – Identification field – Net data rate upstream NPar(2) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | Net data rate upstream NPar(2)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 1 | 1 | 1 | 1 | 1 | 1 | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Unspecified by terminal |
|  | x | x | 1 | x | x | x | x | x | Minimum net data rate upstream (bits 5-1 × 2 Mbit/s) |
|  | x | x | 0 | x | x | x | x | x | Minimum net data rate upstream (bits 5-1 × 64 kbit/s) |

Table 9.1.2/G.994.1 – Identification field – Net data rate upstream NPar(2) coding – Octet 3

| Bits |  |  |  |  |  |  |  |  | Net data rate upstream NPar(2)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 1 | 1 | 1 | 1 | 1 | 1 | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Unspecified by terminal |
|  | x | x | 1 | x | x | x | x | x | Average net data rate upstream (bits 5-1 × 2 Mbit/s) |
|  | x | x | 0 | x | x | x | x | x | Average net data rate upstream (bits 5-1 × 64 kbit/s) |

Table 9.3/G.994.1 – Identification field – Net data rate downstream NPar(2) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | Net data rate downstream NPar(2)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 1 | 1 | 1 | 1 | 1 | 1 | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Unspecified by terminal |
|  | x | x | 1 | x | x | x | x | x | Maximum net data rate downstream (bits 5-1 × 2 Mbit/s) |
|  | x | x | 0 | x | x | x | x | x | Maximum net data rate downstream (bits 5-1 × 64 kbit/s) |

Table 9.3.1/G.994.1 – Identification field – Net data rate downstream NPar(2) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | Net data rate downstream NPar(2)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 1 | 1 | 1 | 1 | 1 | 1 | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Unspecified by terminal |
|  | x | x | 1 | x | x | x | x | x | Minimum net data rate downstream (bits 5-1 × 2 Mbit/s) |
|  | x | x | 0 | x | x | x | x | x | Minimum net data rate downstream (bits 5-1 × 64 kbit/s) |

Table 9.3.2/G.994.1 – Identification field – Net data rate downstream
NPar(2) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | Net data rate downstream NPar(2)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 1 | 1 | 1 | 1 | 1 | 1 | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Unspecified by terminal |
|  | x | x | 1 | x | x | x | x | x | Average net data rate downstream (bits 5-1 × 2 Mbit/s) |
|  | x | x | 0 | x | x | x | x | x | Average net data rate downstream (bits 5-1 × 64 kbit/s) |

Table 9.5/G.994.1 – Identification field – Data flow characteristics upstream
NPar(2) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | Data flow characteristics upstream NPar(2)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 1 | 1 | 1 | 1 | 1 | 1 | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Unspecified by terminal |
|  | x | x | 0 | x | x | x | x | x | Maximum latency upstream (bits 5 to 1) × 1 ms |
|  | x | x | 1 | x | x | x | x | x | Maximum latency upstream (4 + bits 5 to 1) × 10 ms |

Table 9.5.1/G.994.1 – Identification field – Data flow characteristics upstream
NPar(2) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | Data flow characteristics upstream Npar(2)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 1 | 1 | 1 | 1 | 1 | 1 | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Unspecified by terminal |
|  | x | x | 0 | x | x | x | x | x | Average latency upstream (bits 5 to 1) × 1 ms |
|  | x | x | 1 | x | x | x | x | x | Average latency upstream (4 + bits 5 to 1) × 10 ms |

Table 9.7/G.994.1 – Identification field – Data flow characteristics downstream
NPar(2) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | Data flow characteristics downstream NPar(2)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 1 | 1 | 1 | 1 | 1 | 1 | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Unspecified by terminal |
|  | x | x | 0 | x | x | x | x | x | Maximum latency downstream (bits 5 to 1) × 1 ms |
|  | x | x | 1 | x | x | x | x | x | Maximum latency downstream (4 + bits 5 to 1) × 10 ms |

Table 9.7.1/G.994.1 – Identification field – Data flow characteristics downstream
NPar(2) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | Data flow characteristics downstream NPar(2)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 1 | 1 | 1 | 1 | 1 | 1 | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Unspecified by terminal |
|  | x | x | 0 | x | x | x | x | x | Average latency downstream (bits 5 to 1) × 1 ms |
|  | x | x | 1 | x | x | x | x | x | Average latency downstream (4 + bits 5 to 1) × 10 ms |

Table 9.9/G.994.1 – Identification field – xTU-R splitter information NPar(2) coding

|  | Bits |  |  |  |  |  |  |  | xTU-R splitter information NPar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | LPF is voice |
|  | x | x | x | x | x | x | 1 | x | LPF is USA ISDN |
|  | x | x | x | x | x | 1 | x | x | LPF is European ISDN |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Non-standard LPF |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 9.11/G.994.1 – Identification field – xTU-C splitter information NPar(2) coding

|  | Bits |  |  |  |  |  |  |  | xTU-C splitter information NPar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | HPF is 25 kHz (voice) |
|  | x | x | x | x | x | x | 1 | x | HPF is 90 kHz USA ISDN |
|  | x | x | x | x | x | 1 | x | x | HPF is 150 kHz (ADSL with European ISDN) |
|  | x | x | x | x | 1 | x | x | x | HPF is 300 kHz (VDSL) |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Non-standard HPF |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 9.15/G.994.1 – Identification field – Relative power level/carrier for
upstream carrier set A43 – NPar(2) coding

|  | Bits |  |  |  |  |  |  |  | Relative power level/carrier for upstream carrier set A43 Npar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Attenuation in G.994.1 Transmit Power per Carrier Relative to Maximum Power (bits 6-1 × 0.5 dB) for upstream carrier set A43 (Note). |
| NOTE – All carriers in the carrier set shall be transmitted at the same power level. |  |  |  |  |  |  |  |  |  |

Table 9.17/G.994.1 – Identification field – Relative power level/carrier for
downstream carrier set A43 – NPar(2) coding

|  | Bits |  |  |  |  |  |  |  | Relative power level/carrier for downstream carrier set A43 Npar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Attenuation in G.994.1 Transmit Power per Carrier Relative to Maximum Power (bits 6-1 × 0.5 dB) for downstream carrier set A43 (Note). |
| NOTE – All carriers in the carrier set shall be transmitted at the same power level. |  |  |  |  |  |  |  |  |  |

Table 9.19/G.994.1 – Identification field – Relative power level/carrier
for upstream carrier set B43 – NPar(2) coding

|  | Bits |  |  |  |  |  |  |  | Relative power level/carrier for upstream carrier set B43 Npar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Attenuation in G.994.1 Transmit Power per Carrier Relative to Maximum Power (bits 6-1 × 0.5 dB) for upstream carrier set B43 (Note). |
| NOTE – All carriers in the carrier set shall be transmitted at the same power level. |  |  |  |  |  |  |  |  |  |

Table 9.21/G.994.1 – Identification field – Relative power level/carrier for
downstream carrier set B43 – NPar(2) coding

|  | Bits |  |  |  |  |  |  |  | Relative power level/carrier for downstream carrier set B43 Npar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Attenuation in G.994.1 Transmit Power per Carrier Relative to Maximum Power (bits 6-1 × 0.5 dB) for downstream carrier set B43 (Note). |
| NOTE – All carriers in the carrier set shall be transmitted at the same power level. |  |  |  |  |  |  |  |  |  |

Table 9.23/G.994.1 – Identification field – Relative power level/carrier for
upstream carrier set C43 – NPar(2) coding

|  | Bits |  |  |  |  |  |  |  | Relative power level/carrier for upstream carrier set C43 Npar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Attenuation in G.994.1 Transmit Power per Carrier Relative to Maximum Power (bits 6-1 × 0.5 dB) for upstream carrier set C43 (Note). |
| NOTE – All carriers in the carrier set shall be transmitted at the same power level. |  |  |  |  |  |  |  |  |  |

Table 9.25/G.994.1 – Identification field – Relative power level/carrier for
downstream carrier set C43 – NPar(2) coding

|  | Bits |  |  |  |  |  |  |  | Relative power level/carrier for downstream carrier set C43 Npar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Attenuation in G.994.1 Transmit Power per Carrier Relative to Maximum Power (bits 6-1 × 0.5 dB) for downstream carrier set C43 (Note). |
| NOTE – All carriers in the carrier set shall be transmitted at the same power level. |  |  |  |  |  |  |  |  |  |

Table 9.29/G.994.1 – Identification field – Relative power level/carrier for
upstream carrier set A4 – NPar(2) coding

|  | Bits |  |  |  |  |  |  |  | Relative power level/carrier for upstream carrier set A4 Npar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Attenuation in G.994.1 Transmit Power per Carrier Relative to Maximum Power (bits 6-1 × 0.5 dB) for upstream carrier set A4 (Note). |
| NOTE – All carriers in the carrier set shall be transmitted at the same power level. |  |  |  |  |  |  |  |  |  |

Table 9.31/G.994.1 – Identification field – Relative power level/carrier for
downstream carrier set A4 – NPar(2) coding

|  | Bits |  |  |  |  |  |  |  | Relative power level/carrier for downstream carrier set A4 Npar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Attenuation in G.994.1 Transmit Power per Carrier Relative to Maximum Power (bits 6-1 × 0.5 dB) for downstream carrier set A4 (Note). |
| NOTE – All carriers in the carrier set shall be transmitted at the same power level. |  |  |  |  |  |  |  |  |  |

## 9.4	Standard information field (S)

In the standard information field the parameters represent modes of working or capabilities relating to the xTU-R or xTU-C.

The standard information field of CL, CLR, MP and MS messages is encoded in accordance with the rules described in 9.2. For MR, ACK, NAK and REQ messages, the standard information field is not used and is therefore of zero length.

The standard information field consists of a set of octets in which each capability is assigned a unique bit position. A binary ONE in the assigned bit position indicates that the capability is valid.

For messages CL and CLR, the validity of multiple capabilities may be conveyed by transmitting a binary ONE in each bit position corresponding to a valid capability. For messages MP and MS, multiple capabilities may be selected only if they can all be supported simultaneously within the xTU concerned.

The level 1 Pars for MP, MS, CL and CLR messages are listed below, beginning with Table 10. Lower level Pars follow, beginning with Table 11.1. The interpretation and use of these lower level Pars is defined in the respective xDSL Recommendations or other technical specifications.

Table 10/G.994.1 – Standard information field – NPar(1) coding

|  | Bits |  |  |  |  |  |  |  |  | NPar(1)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  |  | x | x | x | x | x | x | x | 1 | Voiceband: V.8 (Note 1) |
|  |  | x | x | x | x | x | x | 1 | x | Voiceband: V.8 bis (Note 1) |
|  |  | x | x | x | x | x | 1 | x | x | Silent period (Note 2) |
|  |  | x | x | x | x | 1 | x | x | x | G.997.1 (Note 3) |
|  |  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  |  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  |  | x | 1 | x | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |
| NOTE 1 – Setting this bit to binary ONE in an MS message initiates the G.994.1 session cleardown procedure specified in 11.3, and requests a V.8 or V.8 bis handshake in the voiceband, with the xTU-R taking on the role of a calling station and the xTU-C taking on the role of an answering station. NOTE 2 – This bit shall be set to binary ONE in a CLR or CL message. Setting this bit to binary ONE in an MS message initiates the G.994.1 session cleardown procedure specified in 11.3, and requests a silence period at the other transmitter of approximately 1 minute. The station that invoked the silent period by transmitting MS may terminate the silent period prior to the 1 minute by restarting a G.994.1 session. NOTE 3 – The use of this bit is for further study and shall be set to binary ZERO in CLR, CL and MS. |  |  |  |  |  |  |  |  |  |  |

Table 11/G.994.1 – Standard information field – SPar(1) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | SPar(1)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | G.992.1 – Annex A (Note) |
|  | x | x | x | x | x | x | 1 | x | G.992.1 – Annex B (Note) |
|  | x | x | x | x | x | 1 | x | x | G.992.1 – Annex C (Note) |
|  | x | x | x | x | 1 | x | x | x | G.992.2 – Annexes A/B (Note) |
|  | x | x | x | 1 | x | x | x | x | G.992.2 – Annex C (Note) |
|  | x | x | 1 | x | x | x | x | x | G.992.1 – Annex H (Note) |
|  | x | 1 | x | x | x | x | x | x | G.992.1 – Annex I (Note) |
|  | x | 0 | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |
| NOTE – The spectrum information indicated in the NPar(3) fields associated with these Recommendations is of informative nature and does not imply any requirements on the transmit spectrum used during initialization and data mode. Regardless of the spectrum information, the transmit spectrum shall comply with their respective Recommendations. Spectrum information may only be included in a CLR or CL message, not in an MP or MS message. The spectrum information is coded in 8 bits (across 2 octets) as a binary representation of the subcarrier index. – Maximum frequencies: up to and including the subcarrier index; – Minimum frequencies: above and including the subcarrier index. |  |  |  |  |  |  |  |  |  |

Table 11.0.1/G.994.1 – Standard information field – SPar(1) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | SPar(1)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | G.991.2 – Annex A |
|  | x | x | x | x | x | x | 1 | x | G.991.2 – Annex B |
|  | x | x | x | x | x | 1 | x | x | Committee T1 MCM VDSL (Note 1) |
|  | x | x | x | x | 1 | x | x | x | Committee T1 SCM VDSL (Note 2) |
|  | x | x | x | 1 | x | x | x | x | ETSI MCM VDSL (Note 3) |
|  | x | x | 1 | x | x | x | x | x | ETSI SCM VDSL (Note 3) |
|  | x | 1 | x | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | 0 | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |
| NOTE 1 – Use of this bit is defined in "Draft Trial-Use Standard For Telecommunication – Interface Between Networks and Customer Installation – Very High Bit-rate Digital Subscriber Line (VDSL) Metallic Interface – Part 3: Technical Specification for Multi-Carrier Modulation (MCM) Transceivers". NOTE 2 – Use of this bit is defined in "Draft Trial-Use Standard For Telecommunication – Interface Between Networks and Customer Installation – Very High Bit-rate Digital Subscriber Line (VDSL) Metallic Interface – Part 2: Technical Specification for Single-Carrier Modulation (SCM) Transceivers". NOTE 3 – Use of this bit is defined in ETSI TS 101270-2. |  |  |  |  |  |  |  |  |  |

Table 11.0.2/G.994.1 – Standard information field – SPar(1) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | SPar(1)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | G.992.3 – Annex A |
|  | x | x | x | x | x | x | 1 | x | G.992.3 – Annex B |
|  | x | x | x | x | x | 1 | x | x | G.992.3 – Annex I |
|  | x | x | x | x | 1 | x | x | x | G.992.3 – Annex J |
|  | x | x | x | 1 | x | x | x | x | G.992.4 – Annex A |
|  | x | x | 1 | x | x | x | x | x | G.992.4 – Annex I |
|  | x | 1 | x | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | 0 | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.0.3/G.994.1 – Standard information field – SPar(1) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | SPar(1)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | G.992.5 – Annex A |
|  | x | x | x | x | x | x | 1 | x | G.992.5 – Annex B |
|  | x | x | x | x | x | 1 | x | x | G.992.5 – Annex I |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | 1 | x | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | 0 | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.1/G.994.1 – Standard information field – G.992.1 Annex A NPar(2) coding

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex A NPar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | R-ACK1 |
|  | x | x | x | x | x | x | 1 | x | R-ACK2 |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | STM |
|  | x | x | x | 1 | x | x | x | x | ATM |
|  | x | x | 1 | x | x | x | x | x | G.997.1 – Clear EOC OAM |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.2/G.994.1 – Standard information field – G.992.1 Annex A SPar(2) coding

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex A SPar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Sub-channel information |
|  | x | x | x | x | x | x | 1 | x | Spectrum frequency upstream |
|  | x | x | x | x | x | 1 | x | x | Spectrum frequency downstream |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.2.1/G.994.1 – Standard information field – G.992.1 Annex A
Sub-channel information NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex A Sub-channel information NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | AS0 downstream |
|  | x | x | x | x | x | x | 1 | x | AS1 downstream |
|  | x | x | x | x | x | 1 | x | x | AS2 downstream |
|  | x | x | x | x | 1 | x | x | x | AS3 downstream |
|  | x | x | x | 1 | x | x | x | x | LS0 downstream |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.2.1.1/G.994.1 – Standard information field – G.992.1 Annex A
Sub-channel information NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex A Sub-channel information NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | LS1 downstream |
|  | x | x | x | x | x | x | 1 | x | LS2 downstream |
|  | x | x | x | x | x | 1 | x | x | LS0 upstream |
|  | x | x | x | x | 1 | x | x | x | LS1 upstream |
|  | x | x | x | 1 | x | x | x | x | LS2 upstream |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.2.2/G.994.1 – Standard information field – G.992.1 Annex A
Spectrum frequency upstream NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex A Spectrum frequency upstream NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | x | x | Spectrum minimum frequency upstream (bits 7 and 8) |

Table 11.2.2.1/G.994.1 – Standard information field – G.992.1 Annex A
Spectrum frequency upstream NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex A Spectrum frequency upstream NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Spectrum minimum frequency upstream (bits 1 to 6) |

Table 11.2.2.2/G.994.1 – Standard information field – G.992.1 Annex A
Spectrum frequency upstream NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex A Spectrum frequency upstream NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | x | x | Spectrum maximum frequency upstream (bits 7 and 8) |

Table 11.2.2.3/G.994.1 – Standard information field – G.992.1 Annex A
Spectrum frequency upstream NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex A Spectrum frequency upstream NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Spectrum maximum frequency upstream (bits 1 to 6) |

Table 11.2.3/G.994.1 – Standard information field – G.992.1 Annex A
Spectrum frequency downstream NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex A Spectrum frequency downstream NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | x | x | Spectrum minimum frequency downstream (bits 7 and 8) |

Table 11.2.3.1/G.994.1 – Standard information field – G.992.1 Annex A
Spectrum frequency downstream NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex A Spectrum frequency downstream NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Spectrum minimum frequency downstream (bits 1 to 6) |

Table 11.2.3.2/G.994.1 – Standard information field – G.992.1 Annex A
Spectrum frequency downstream NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex A Spectrum frequency downstream NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | x | x | Spectrum maximum frequency downstream (bits 7 and 8) |

Table 11.2.3.3/G.994.1 – Standard information field – G.992.1 Annex A
Spectrum frequency downstream NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex A Spectrum frequency downstream NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Spectrum maximum frequency downstream (bits 1 to 6) |

Table 11.3/G.994.1 – Standard information field – G.992.1 Annex B NPar(2) coding

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex B NPar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | R-ACK1 |
|  | x | x | x | x | x | x | 1 | x | R-ACK2 |
|  | x | x | x | x | x | 1 | x | x | Upstream tones 1 to 32 |
|  | x | x | x | x | 1 | x | x | x | STM |
|  | x | x | x | 1 | x | x | x | x | ATM |
|  | x | x | 1 | x | x | x | x | x | G.997.1 – Clear EOC OAM |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.4/G.994.1 – Standard information field – G.992.1 Annex B SPar(2) coding

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex B SPar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Sub-channel information |
|  | x | x | x | x | x | x | 1 | x | Spectrum frequency upstream |
|  | x | x | x | x | x | 1 | x | x | Spectrum frequency downstream |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.4.1/G.994.1 – Standard information field – G.992.1 Annex B
Sub-channel information NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex B Sub-channel information NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | AS0 downstream |
|  | x | x | x | x | x | x | 1 | x | AS1 downstream |
|  | x | x | x | x | x | 1 | x | x | AS2 downstream |
|  | x | x | x | x | 1 | x | x | x | AS3 downstream |
|  | x | x | x | 1 | x | x | x | x | LS0 downstream |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.4.1.1/G.994.1 – Standard information field – G.992.1 Annex B
Sub-channel information NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex B Sub-channel information NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | LS1 downstream |
|  | x | x | x | x | x | x | 1 | x | LS2 downstream |
|  | x | x | x | x | x | 1 | x | x | LS0 upstream |
|  | x | x | x | x | 1 | x | x | x | LS1 upstream |
|  | x | x | x | 1 | x | x | x | x | LS2 upstream |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.4.2/G.994.1 – Standard information field – G.992.1 Annex B
Spectrum frequency upstream NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex B Spectrum frequency upstream NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | x | x | Spectrum minimum frequency upstream (bits 7 and 8) |

Table 11.4.2.1/G.994.1 – Standard information field – G.992.1 Annex B
Spectrum frequency upstream NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex B Spectrum frequency upstream NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Spectrum minimum frequency upstream (bits 1 to 6) |

Table 11.4.2.2/G.994.1 – Standard information field – G.992.1 Annex B
Spectrum frequency upstream NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex B Spectrum frequency upstream NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | x | x | Spectrum maximum frequency upstream (bits 7 and 8) |

Table 11.4.2.3/G.994.1 – Standard information field – G.992.1 Annex B
Spectrum frequency upstream NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex B Spectrum frequency upstream NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Spectrum maximum frequency upstream (bits 1 to 6) |

Table 11.4.3/G.994.1 – Standard information field – G.992.1 Annex B
Spectrum frequency downstream NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex B Spectrum frequency downstream NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | x | x | Spectrum minimum frequency downstream (bits 7 and 8) |

Table 11.4.3.1/G.994.1 – Standard information field – G.992.1 Annex B
Spectrum frequency downstream NPar(3) coding – Octet 2

| Bits |  |  |  |  |  |  |  |  | G.992.1 Annex B Spectrum frequency downstream NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Spectrum minimum frequency downstream (bits 1 to 6) |

Table 11.4.3.2/G.994.1 – Standard information field – G.992.1 Annex B
Spectrum frequency downstream NPar(3) coding – Octet 3

| Bits |  |  |  |  |  |  |  |  | G.992.1 Annex B Spectrum frequency downstream Npar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | x | x | Spectrum maximum frequency downstream (bits 7 and 8) |

Table 11.4.3.3/G.994.1 – Standard information field – G.992.1 Annex B
Spectrum frequency downstream NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex B Spectrum frequency downstream Npar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Spectrum maximum frequency downstream (bits 1 to 6) |

Table 11.5/G.994.1 – Standard information field – G.992.1 Annex C NPar(2) coding – Octet 1

| Bits |  |  |  |  |  |  |  |  | G.992.1 Annex C NPar(2)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | R-ACK1 |
|  | x | x | x | x | x | x | 1 | x | R-ACK2 |
|  | x | x | x | x | x | 1 | x | x | DBM |
|  | x | x | x | x | 1 | x | x | x | STM |
|  | x | x | x | 1 | x | x | x | x | ATM |
|  | x | x | 1 | x | x | x | x | x | G.997.1 – Clear EOC OAM |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.5.1/G.994.1 – Standard information field – G.992.1 Annex C
NPar(2) coding – Octet 2

| Bits |  |  |  |  |  |  |  |  | G.992.1 Annex C NPar(2)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Profile 1 |
|  | x | x | x | x | x | x | 1 | x | Profile 2 |
|  | x | x | x | x | x | 1 | x | x | Profile 3 |
|  | x | x | x | x | 1 | x | x | x | Profile 4 |
|  | x | x | x | 1 | x | x | x | x | Profile 5 |
|  | x | x | 1 | x | x | x | x | x | Profile 6 |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.6/G.994.1 – Standard information field – G.992.1 Annex C SPar(2) coding

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex C SPar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Sub-channel information |
|  | x | x | x | x | x | x | 1 | x | Spectrum frequency upstream |
|  | x | x | x | x | x | 1 | x | x | Spectrum frequency downstream |
|  | x | x | x | x | 1 | x | x | x | C-PILOT |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.6.1/G.994.1 – Standard information field – G.992.1 Annex C
Sub-channel information NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex C Sub-channel information NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | AS0 downstream |
|  | x | x | x | x | x | x | 1 | x | AS1 downstream |
|  | x | x | x | x | x | 1 | x | x | AS2 downstream |
|  | x | x | x | x | 1 | x | x | x | AS3 downstream |
|  | x | x | x | 1 | x | x | x | x | LS0 downstream |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.6.1.1/G.994.1 – Standard information field – G.992.1 Annex C
Sub-channel information NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex C Sub-channel information NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | LS1 downstream |
|  | x | x | x | x | x | x | 1 | x | LS2 downstream |
|  | x | x | x | x | x | 1 | x | x | LS0 upstream |
|  | x | x | x | x | 1 | x | x | x | LS1 upstream |
|  | x | x | x | 1 | x | x | x | x | LS2 upstream |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.6.2/G.994.1 – Standard information field – G.992.1 Annex C
Spectrum frequency upstream NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex C Spectrum frequency upstream NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | x | x | Spectrum minimum frequency upstream (bits 7 and 8) |

Table 11.6.2.1/G.994.1 – Standard information field – G.992.1 Annex C
Spectrum frequency upstream NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex C Spectrum frequency upstream NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Spectrum minimum frequency upstream (bits 1 to 6) |

Table 11.6.2.2/G.994.1 – Standard information field – G.992.1 Annex C
Spectrum frequency upstream NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex C Spectrum frequency upstream NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | x | x | Spectrum maximum frequency upstream (bits 7 and 8) |

Table 11.6.2.3/G.994.1 – Standard information field – G.992.1 Annex C
Spectrum frequency upstream NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex C Spectrum frequency upstream NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Spectrum maximum frequency upstream (bits 1 to 6) |

Table 11.6.3/G.994.1 – Standard information field – G.992.1 Annex C
Spectrum frequency downstream NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex C Spectrum frequency downstream NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | x | x | Spectrum minimum frequency downstream (bits 7 and 8) |

Table 11.6.3.1/G.994.1 – Standard information field – G.992.1 Annex C
Spectrum frequency downstream NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex C Spectrum frequency downstream NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Spectrum minimum frequency downstream (bits 1 to 6) |

Table 11.6.3.2/G.994.1 – Standard information field – G.992.1 Annex C
Spectrum frequency downstream NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex C Spectrum frequency downstream NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | x | x | Spectrum maximum frequency downstream (bits 7 and 8) |

Table 11.6.3.3/G.994.1 – Standard information field – G.992.1 Annex C
Spectrum frequency downstream NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex C Spectrum frequency downstream NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Spectrum maximum frequency downstream (bits 1 to 6) |

Table 11.6.4/G.994.1 – Standard information field – G.992.1 Annex C
C-PILOT NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex C C-PILOT NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | nC-PILOT1 = 64 |
|  | x | x | x | x | x | x | 1 | x | nC-PILOT1 = 48 |
|  | x | x | x | x | x | 1 | x | x | nC-PILOT1 = 32 |
|  | x | x | x | x | 1 | x | x | x | nC-PILOT1 = 16 |
|  | x | x | x | 1 | x | x | x | x | A48/B48 |
|  | x | x | 1 | x | x | x | x | x | C-REVERB33-63 |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.6.4.1/G.994.1 – Standard information field – G.992.1 Annex C
C-PILOT NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex C C-PILOT NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | A24/B24 |
|  | x | x | x | x | x | x | 1 | x | C-REVERB6-31 |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.7/G.994.1 – Standard information field – G.992.2 Annexes A/B NPar(2) coding

|  | Bits |  |  |  |  |  |  |  | G.992.2 Annexes A/B NPar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | R-ACK1 |
|  | x | x | x | x | x | x | 1 | x | R-ACK2 |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Fast retrain |
|  | x | x | x | 1 | x | x | x | x | RS16 |
|  | x | x | 1 | x | x | x | x | x | G.997.1 – Clear EOC OAM |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.8/G.994.1 – Standard information field – G.992.2 Annexes A/B SPar(2) coding

|  | Bits |  |  |  |  |  |  |  | G.992.2 Annexes A/B SPar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Reserved for allocation by the ITU-T |
|  | x | x | x | x | x | x | 1 | x | Spectrum frequency upstream |
|  | x | x | x | x | x | 1 | x | x | Spectrum frequency downstream |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.8.2/G.994.1 – Standard information field – G.992.2 Annexes A/B
Spectrum frequency upstream NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.2 Annexes A/B Spectrum frequency upstream NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | x | x | Spectrum minimum frequency upstream (bits 7 and 8) |

Table 11.8.2.1/G.994.1 – Standard information field – G.992.2 Annexes A/B
Spectrum frequency upstream NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.2 Annexes A/B Spectrum frequency upstream NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Spectrum minimum frequency upstream (bits 1 to 6) |

Table 11.8.2.2/G.994.1 – Standard information field – G.992.2 Annexes A/B
Spectrum frequency upstream NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.2 Annexes A/B Spectrum frequency upstream NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | x | x | Spectrum maximum frequency upstream (bits 7 and 8) |

Table 11.8.2.3/G.994.1 – Standard information field – G.992.2 Annexes A/B
Spectrum frequency upstream NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.2 Annexes A/B Spectrum frequency upstream NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Spectrum maximum frequency upstream (bits 1 to 6) |

Table 11.8.3/G.994.1 – Standard information field – G.992.2 Annexes A/B
Spectrum frequency downstream NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.2 Annexes A/B Spectrum frequency downstream NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | x | x | Spectrum minimum frequency downstream (bits 7 and 8) |

Table 11.8.3.1/G.994.1 – Standard information field – G.992.2 Annexes A/B
Spectrum frequency downstream NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.2 Annexes A/B Spectrum frequency downstream NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Spectrum minimum frequency downstream (bits 1 to 6) |

Table 11.8.3.2/G.994.1 – Standard information field – G.992.2 Annexes A/B
Spectrum frequency downstream NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.2 Annexes A/B Spectrum frequency downstream NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | x | x | Spectrum maximum frequency downstream (bits 7 and 8) |

Table 11.8.3.3/G.994.1 – Standard information field – G.992.2 Annexes A/B
Spectrum frequency downstream NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.2 Annexes A/B Spectrum frequency downstream NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Spectrum maximum frequency downstream (bits 1 to 6) |

Table 11.9/G.994.1 – Standard information field – G.992.2 Annex C NPar(2) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.2 Annex C NPar(2)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | R-ACK1 |
|  | x | x | x | x | x | x | 1 | x | R-ACK2 |
|  | x | x | x | x | x | 1 | x | x | DBM |
|  | x | x | x | x | 1 | x | x | x | Fast retrain |
|  | x | x | x | 1 | x | x | x | x | RS16 |
|  | x | x | 1 | x | x | x | x | x | G.997.1 – Clear EOC OAM |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.9.1/G.994.1 – Standard information field – G.992.2 Annex C
NPar(2) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.2 Annex C NPar(2)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Profile 1 |
|  | x | x | x | x | x | x | 1 | x | Profile 2 |
|  | x | x | x | x | x | 1 | x | x | Profile 3 |
|  | x | x | x | x | 1 | x | x | x | Profile 4 |
|  | x | x | x | 1 | x | x | x | x | Profile 5 |
|  | x | x | 1 | x | x | x | x | x | Profile 6 |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.10/G.994.1 – Standard information field – G.992.2 Annex C SPar(2) coding

|  | Bits |  |  |  |  |  |  |  | G.992.2 Annex C SPar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Reserved for allocation by the ITU-T |
|  | x | x | x | x | x | x | 1 | x | Spectrum frequency upstream |
|  | x | x | x | x | x | 1 | x | x | Spectrum frequency downstream |
|  | x | x | x | x | 1 | x | x | x | C-PILOT |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.10.2/G.994.1 – Standard information field – G.992.2 Annex C
Spectrum frequency upstream NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.2 Annex C Spectrum frequency upstream NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | x | x | Spectrum minimum frequency upstream (bits 7 and 8) |

Table 11.10.2.1/G.994.1 – Standard information field – G.992.2 Annex C
Spectrum frequency upstream NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.2 Annex C Spectrum frequency upstream NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Spectrum minimum frequency upstream (bits 1 to 6) |

Table 11.10.2.2/G.994.1 – Standard information field – G.992.2 Annex C
Spectrum frequency upstream NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.2 Annex C Spectrum frequency upstream NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | x | x | Spectrum maximum frequency upstream (bits 7 and 8) |

Table 11.10.2.3/G.994.1 – Standard information field – G.992.2 Annex C
Spectrum frequency upstream NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.2 Annex C Spectrum frequency upstream NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Spectrum maximum frequency upstream (bits 1 to 6) |

Table 11.10.3/G.994.1 – Standard information field – G.992.2 Annex C
Spectrum frequency downstream NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.2 Annex C Spectrum frequency downstream NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | x | x | Spectrum minimum frequency downstream (bits 7 and 8) |

Table 11.10.3.1/G.994.1 – Standard information field – G.992.2 Annex C
Spectrum frequency downstream NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.2 Annex C Spectrum frequency downstream NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Spectrum minimum frequency downstream (bits 1 to 6) |

Table 11.10.3.2/G.994.1 – Standard information field – G.992.2 Annex C
Spectrum frequency downstream NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.2 Annex C Spectrum frequency downstream NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | x | x | Spectrum maximum frequency downstream (bits 7 and 8) |

Table 11.10.3.3/G.994.1 – Standard information field – G.992.2 Annex C
Spectrum frequency downstream NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.2 Annex C Spectrum frequency downstream NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Spectrum maximum frequency downstream (bits 1 to 6) |

Table 11.10.4/G.994.1 – Standard information field – G.992.2 Annex C
C-PILOT NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.2 Annex C C-PILOT NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | nC-PILOT1 = 64 |
|  | x | x | x | x | x | x | 1 | x | nC-PILOT1 = 48 |
|  | x | x | x | x | x | 1 | x | x | nC-PILOT1 = 32 |
|  | x | x | x | x | 1 | x | x | x | nC-PILOT1 = 16 |
|  | x | x | x | 1 | x | x | x | x | A48/B48 |
|  | x | x | 1 | x | x | x | x | x | C-REVERB33-63 |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.10.4.1/G.994.1 – Standard information field – G.992.2 Annex C
C-PILOT NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.2 Annex C C-PILOT NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | A24/B24 |
|  | x | x | x | x | x | x | 1 | x | C-REVERB6-31 |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.11/G.994.1 – Standard information field – G.992.1 Annex H NPar(2) coding

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex H NPar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | EFT |
|  | x | x | x | x | x | x | 1 | x | Fast path |
|  | x | x | x | x | x | 1 | x | x | 1.544 Mbit/s |
|  | x | x | x | x | 1 | x | x | x | STM |
|  | x | x | x | 1 | x | x | x | x | ATM |
|  | x | x | 1 | x | x | x | x | x | G.997.1 – Clear EOC OAM |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.12/G.994.1 – Standard information field – G.992.1 Annex H SPar(2) coding

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex H SPar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Reserved for allocation by the ITU-T |
|  | x | x | x | x | x | x | 1 | x | Spectrum frequency upstream |
|  | x | x | x | x | x | 1 | x | x | Spectrum frequency downstream |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.12.2/G.994.1 – Standard information field – G.992.1 Annex H
Spectrum frequency upstream NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex H Spectrum frequency upstream NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | x | x | Spectrum minimum frequency upstream (bits 7 and 8) |

Table 11.12.2.1/G.994.1 – Standard information field – G.992.1 Annex H
Spectrum frequency upstream NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex H Spectrum frequency upstream NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Spectrum minimum frequency upstream (bits 1 to 6) |

Table 11.12.2.2/G.994.1 – Standard information field – G.992.1 Annex H
Spectrum frequency upstream NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex H Spectrum frequency upstream NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | x | x | Spectrum maximum frequency upstream (bits 7 and 8) |

Table 11.12.2.3/G.994.1 – Standard information field – G.992.1 Annex H
Spectrum frequency upstream NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex H Spectrum frequency upstream NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Spectrum maximum frequency upstream (bits 1 to 6) |

Table 11.12.3/G.994.1 – Standard information field – G.992.1 Annex H
Spectrum frequency downstream NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex H Spectrum frequency downstream NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | x | x | Spectrum minimum frequency downstream (bits 7 and 8) |

Table 11.12.3.1/G.994.1 – Standard information field – G.992.1 Annex H
Spectrum frequency downstream NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex H Spectrum frequency downstream NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Spectrum minimum frequency downstream (bits 1 to 6) |

Table 11.12.3.2/G.994.1 – Standard information field – G.992.1 Annex H
Spectrum frequency downstream NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex H Spectrum frequency downstream NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | x | x | Spectrum maximum frequency downstream (bits 7 and 8) |

Table 11.12.3.3/G.994.1 – Standard information field – G.992.1 Annex H
Spectrum frequency downstream NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex H Spectrum frequency downstream NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Spectrum maximum frequency downstream (bits 1 to 6) |

Table 11.13/G.994.1 – Standard information field – G.992.1 Annex I NPar(2) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex I NPar(2)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | R-ACK1 |
|  | x | x | x | x | x | x | 1 | x | R-ACK2 |
|  | x | x | x | x | x | 1 | x | x | DBM |
|  | x | x | x | x | 1 | x | x | x | STM |
|  | x | x | x | 1 | x | x | x | x | ATM |
|  | x | x | 1 | x | x | x | x | x | G.997.1 – Clear EOC OAM |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.13.1/G.994.1 – Standard information field – G.992.1 Annex I
NPar(2) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex I NPar(2)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Spectral shaping downstream #1 (shaped ssvi) |
|  | x | x | x | x | x | x | 1 | x | Spectral shaping downstream #2 (flat ssvi) |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.14/G.994.1 – Standard information field – G.992.1 Annex I SPar(2) coding

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex I SPar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Sub-channel information |
|  | x | x | x | x | x | x | 1 | x | Spectrum frequency upstream |
|  | x | x | x | x | x | 1 | x | x | Spectrum frequency downstream |
|  | x | x | x | x | 1 | x | x | x | C-PILOT |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.14.1/G.994.1 – Standard information field – G.992.1 Annex I
Sub-channel information NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex I Sub-channel information NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | AS0 downstream |
|  | x | x | x | x | x | x | 1 | x | AS1 downstream |
|  | x | x | x | x | x | 1 | x | x | AS2 downstream |
|  | x | x | x | x | 1 | x | x | x | AS3 downstream |
|  | x | x | x | 1 | x | x | x | x | LS0 downstream |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.14.1.1/G.994.1 – Standard information field – G.992.1 Annex I
Sub-channel information NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex I Sub-channel information NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | LS1 downstream |
|  | x | x | x | x | x | x | 1 | x | LS2 downstream |
|  | x | x | x | x | x | 1 | x | x | LS0 upstream |
|  | x | x | x | x | 1 | x | x | x | LS1 upstream |
|  | x | x | x | 1 | x | x | x | x | LS2 upstream |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.14.2/G.994.1 – Standard information field – G.992.1 Annex I
Spectrum frequency upstream NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex I Spectrum frequency upstream NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | x | x | Spectrum minimum frequency upstream (bits 7 and 8) |

Table 11.14.2.1/G.994.1 – Standard information field – G.992.1 Annex I
Spectrum frequency upstream NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex I Spectrum frequency upstream NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Spectrum minimum frequency upstream (bits 1 to 6) |

Table 11.14.2.2/G.994.1 – Standard information field – G.992.1 Annex I
Spectrum frequency upstream NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex I Spectrum frequency upstream NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | x | x | Spectrum maximum frequency upstream (bits 7 and 8) |

Table 11.14.2.3/G.994.1 – Standard information field – G.992.1 Annex I
Spectrum frequency upstream NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex I Spectrum frequency upstream NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Spectrum maximum frequency upstream (bits 1 to 6) |

Table 11.14.3/G.994.1 – Standard information field – G.992.1 Annex I
Spectrum frequency downstream NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex I Spectrum frequency downstream NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | x | x | Spectrum minimum frequency downstream (bits 7 and 8) |

Table 11.14.3.1/G.994.1 – Standard information field – G.992.1 Annex I
Spectrum frequency downstream NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex I Spectrum frequency downstream NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Spectrum minimum frequency downstream (bits 1 to 6) |

Table 11.14.3.2/G.994.1 – Standard information field – G.992.1 Annex I
Spectrum frequency downstream NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex I Spectrum frequency downstream NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | x | x | Spectrum maximum frequency downstream (bits 7 and 8) |

Table 11.14.3.3/G.994.1 – Standard information field – G.992.1 Annex I
Spectrum frequency downstream NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex I Spectrum frequency downstream NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Spectrum maximum frequency downstream (bits 1 to 6) |

Table 11.14.4/G.994.1 – Standard information field – G.992.1 Annex I
C-PILOT NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex I C-PILOT NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | nC-PILOT1 = 64 |
|  | x | x | x | x | x | x | 1 | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | A48/B48 |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.14.4.1/G.994.1 – Standard information field – G.992.1 Annex I
C-PILOT NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.1 Annex I C-PILOT NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Reserved for allocation by the ITU-T |
|  | x | x | x | x | x | x | 1 | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | x | 1 | x | x | nC-PILOT1 = 128 |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.15/G.994.1 – Standard information field – G.991.2 Annex A – NPar(2) coding

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A NPar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Training mode (Note) (see also Table 11.14) |
|  | x | x | x | x | x | x | 1 | x | PMMS mode (Note) (see also Table 11.14) |
|  | x | x | x | x | x | 1 | x | x | Regenerator silent period (Note) |
|  | x | x | x | x | 1 | x | x | x | 4-Wire |
|  | x | x | x | 1 | x | x | x | x | SRU |
|  | x | x | 1 | x | x | x | x | x | Diagnostic mode |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |
| NOTE – Only one of these bits shall be set at any given time. |  |  |  |  |  |  |  |  |  |

Table 11.16/G.994.1 – Standard information field –
G.991.2 Annex A – SPar(2) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A SPar(2)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream training parameters |
|  | x | x | x | x | x | x | 1 | x | Upstream training parameters |
|  | x | x | x | x | x | 1 | x | x | Downstream PMMS parameters |
|  | x | x | x | x | 1 | x | x | x | Upstream PMMS parameters |
|  | x | x | x | 1 | x | x | x | x | TPS-TC parameters |
|  | x | x | 1 | x | x | x | x | x | Downstream framing parameters |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.0.1/G.994.1 – Standard information field – G.991.2 Annex A –
SPar(2) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A SPar(2)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream framing parameters |
|  | x | x | x | x | x | x | 1 | x | Dual-Mode TPS-TC parameters |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.1/G.994.1 – Standard information field – G.991.2 Annex A
Downstream training parameters – NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A downstream training NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | x | x | x | x | x | Downstream PBO (dB) (bits 5-1 × 1.0 dB) |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |

Table 11.16.1.1/G.994.1 – Standard information field – G.991.2 Annex A
Downstream training parameters – NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A downstream training NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream base data rate unspecified by terminal |
|  | x | x | x | x | x | x | 1 | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Downstream base data rate = 192 kbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Downstream base data rate = 256 kbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.1.2/G.994.1 – Standard information field – G.991.2 Annex A
Downstream training parameters – NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A downstream training NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream base data rate = 320 kbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Downstream base data rate = 384 kbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Downstream base data rate = 448 kbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Downstream base data rate = 512 kbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Downstream base data rate = 576 kbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Downstream base data rate = 640 kbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.1.3/G.994.1 – Standard information field – G.991.2 Annex A
Downstream training parameters – NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A downstream training NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream base data rate = 704 kbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Downstream base data rate = 768 kbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Downstream base data rate = 832 kbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Downstream base data rate = 896 kbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Downstream base data rate = 960 kbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Downstream base data rate = 1.024 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.1.4/G.994.1 – Standard information field – G.991.2 Annex A
Downstream training parameters – NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A downstream training NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream base data rate = 1.088 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Downstream base data rate = 1.152 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Downstream base data rate = 1.216 Mbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Downstream base data rate = 1.280 Mbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Downstream base data rate = 1.344 Mbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Downstream base data rate = 1.408 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.1.5/G.994.1 – Standard information field – G.991.2 Annex A
Downstream training parameters – NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A downstream training NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream base data rate = 1.472 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Downstream base data rate = 1.536 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Downstream base data rate = 1.600 Mbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Downstream base data rate = 1.664 Mbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Downstream base data rate = 1.728 Mbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Downstream base data rate = 1.792 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.1.6/G.994.1 – Standard information field – G.991.2 Annex A
Downstream training parameters – NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A downstream training NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream base data rate = 1.856 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Downstream base data rate = 1.920 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Downstream base data rate = 1.984 Mbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Downstream base data rate = 2.048 Mbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Downstream base data rate = 2.112 Mbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Downstream base data rate = 2.176 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.1.7/G.994.1 – Standard information field – G.991.2 Annex A
Downstream training parameters – NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A downstream training NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream base data rate = 2.240 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Downstream base data rate = 2.304 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Downstream base data rate = 768 or 776 kbit/s, asymmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Downstream base data rate = 1.536 or 1.544 Mbit/s, asymmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.1.8/G.994.1 – Standard information field – G.991.2 Annex A
Downstream training parameters – NPar(3) coding – Octet 9

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A downstream training NPar(3)s – Octet 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream sub-data rate = 0 kbit/s |
|  | x | x | x | x | x | x | 1 | x | Downstream sub-data rate = 8 kbit/s |
|  | x | x | x | x | x | 1 | x | x | Downstream sub-data rate = 16 kbit/s |
|  | x | x | x | x | 1 | x | x | x | Downstream sub-data rate = 24 kbit/s |
|  | x | x | x | 1 | x | x | x | x | Downstream sub-data rate = 32 kbit/s |
|  | x | x | 1 | x | x | x | x | x | Downstream sub-data rate = 40 kbit/s |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.1.9/G.994.1 – Standard information field – G.991.2 Annex A
Downstream training parameters – NPar(3) coding – Octet 10

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A downstream training NPar(3)s – Octet 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream sub-data rate = 48 kbit/s |
|  | x | x | x | x | x | x | 1 | x | Downstream sub-data rate = 56 kbit/s |
|  | x | x | x | x | x | 1 | x | x | Downstream sub-data rate unspecified by terminal |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.2/G.994.1 – Standard information field – G.991.2 Annex A
Upstream training parameters – NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A upstream training NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | x | x | x | x | x | Upstream PBO (dB) (bits 5-1 × 1.0 dB) |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |

Table 11.16.2.1/G.994.1 – Standard information field – G.991.2 Annex A
Upstream training parameters – NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A upstream training NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream base data rate unspecified by terminal |
|  | x | x | x | x | x | x | 1 | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Upstream base data rate = 192 kbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Upstream base data rate = 256 kbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.2.2/G.994.1 – Standard information field – G.991.2 Annex A
Upstream training parameters – NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A upstream training NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream base data rate = 320 kbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Upstream base data rate = 384 kbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Upstream base data rate = 448 kbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Upstream base data rate = 512 kbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Upstream base data rate = 576 kbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Upstream base data rate = 640 kbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.2.3/G.994.1 – Standard information field – G.991.2 Annex A
Upstream training parameters – NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A upstream training NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream base data rate = 704 kbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Upstream base data rate = 768 kbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Upstream base data rate = 832 kbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Upstream base data rate = 896 kbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Upstream base data rate = 960 kbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Upstream base data rate = 1.024 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.2.4/G.994.1 – Standard information field – G.991.2 Annex A
Upstream training parameters – NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A upstream training NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream base data rate = 1.088 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Upstream base data rate = 1.152 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Upstream base data rate = 1.216 Mbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Upstream base data rate = 1.280 Mbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Upstream base data rate = 1.344 Mbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Upstream base data rate = 1.408 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.2.5/G.994.1 – Standard information field – G.991.2 Annex A
Upstream training parameters – NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A upstream training NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream base data rate = 1.472 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Upstream base data rate = 1.536 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Upstream base data rate = 1.600 Mbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Upstream base data rate = 1.664 Mbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Upstream base data rate = 1.728 Mbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Upstream base data rate = 1.792 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.2.6/G.994.1 – Standard information field – G.991.2 Annex A
Upstream training parameters – NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A upstream training NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream base data rate = 1.856 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Upstream base data rate = 1.920 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Upstream base data rate = 1.984 Mbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Upstream base data rate = 2.048 Mbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Upstream base data rate = 2.112 Mbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Upstream base data rate = 2.176 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.2.7/G.994.1 – Standard information field – G.991.2 Annex A
Upstream training parameters – NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A upstream training NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream base data rate = 2.240 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Upstream base data rate = 2.304 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Upstream base data rate = 768 or 776 kbit/s, asymmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Upstream base data rate = 1.536 or 1.544 Mbit/s, asymmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.2.8/G.994.1 – Standard information field – G.991.2 Annex A
Upstream training parameters – NPar(3) coding – Octet 9

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A upstream training NPar(3)s – Octet 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream sub-data rate = 0 kbit/s |
|  | x | x | x | x | x | x | 1 | x | Upstream sub-data rate = 8 kbit/s |
|  | x | x | x | x | x | 1 | x | x | Upstream sub-data rate = 16 kbit/s |
|  | x | x | x | x | 1 | x | x | x | Upstream sub-data rate = 24 kbit/s |
|  | x | x | x | 1 | x | x | x | x | Upstream sub-data rate = 32 kbit/s |
|  | x | x | 1 | x | x | x | x | x | Upstream sub-data rate = 40 kbit/s |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.2.9/G.994.1 – Standard information field – G.991.2 Annex A
Upstream training parameters – NPar(3) coding – Octet 10

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A upstream training NPar(3)s – Octet 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream sub-data rate = 48 kbit/s |
|  | x | x | x | x | x | x | 1 | x | Upstream sub-data rate = 56 kbit/s |
|  | x | x | x | x | x | 1 | x | x | Upstream sub-data rate unspecified by terminal |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.3/G.994.1 – Standard information field – G.991.2 Annex A
Downstream PMMS parameters – NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A downstream PMMS NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | x | x | x | x | x | Downstream PBO (dB) (bits 5-1 × 1.0 dB) |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |

Table 11.16.3.1/G.994.1 – Standard information field – G.991.2 Annex A
Downstream PMMS parameters – NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A downstream PMMS NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream base data rate unspecified by terminal |
|  | x | x | x | x | x | x | 1 | x | Transmit silence |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Downstream base data rate = 192 kbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Downstream base data rate = 256 kbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.3.2/G.994.1 – Standard information field – G.991.2 Annex A
Downstream PMMS parameters – NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A downstream PMMS NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream base data rate = 320 kbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Downstream base data rate = 384 kbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Downstream base data rate = 448 kbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Downstream base data rate = 512 kbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Downstream base data rate = 576 kbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Downstream base data rate = 640 kbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.3.3/G.994.1 – Standard information field – G.991.2 Annex A
Downstream PMMS parameters – NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A downstream PMMS NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream base data rate = 704 kbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Downstream base data rate = 768 kbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Downstream base data rate = 832 kbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Downstream base data rate = 896 kbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Downstream base data rate = 960 kbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Downstream base data rate = 1.024 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.3.4/G.994.1 – Standard information field – G.991.2 Annex A
Downstream PMMS parameters – NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A downstream PMMS NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream base data rate = 1.088 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Downstream base data rate = 1.152 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Downstream base data rate = 1.216 Mbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Downstream base data rate = 1.280 Mbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Downstream base data rate = 1.344 Mbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Downstream base data rate = 1.408 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.3.5/G.994.1 – Standard information field – G.991.2 Annex A
Downstream PMMS parameters – NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A downstream PMMS NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream base data rate = 1.472 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Downstream base data rate = 1.536 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Downstream base data rate = 1.600 Mbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Downstream base data rate = 1.664 Mbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Downstream base data rate = 1.728 Mbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Downstream base data rate = 1.792 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.3.6/G.994.1 – Standard information field – G.991.2 Annex A
Downstream PMMS parameters – NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A downstream PMMS NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream base data rate = 1.856 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Downstream base data rate = 1.920 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Downstream base data rate = 1.984 Mbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Downstream base data rate = 2.048 Mbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Downstream base data rate = 2.112 Mbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Downstream base data rate = 2.176 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.3.7/G.994.1 – Standard information field – G.991.2 Annex A
Downstream PMMS parameters – NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A downstream PMMS NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream base data rate = 2.240 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Downstream base data rate = 2.304 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Downstream base data rate = 768 kbit/s, asymmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Downstream base data rate = 1.544 Mbit/s, asymmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.3.8/G.994.1 – Standard information field – G.991.2 Annex A
Downstream PMMS parameters – NPar(3) coding – Octet 9

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A downstream PMMS NPar(3)s – Octet 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 1 | Fixed value during PMMS |

Table 11.16.3.9/G.994.1 – Standard information field – G.991.2 Annex A
Downstream PMMS parameters – NPar(3) coding – Octet 10

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A downstream PMMS NPar(3)s – Octet 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Fixed value during PMMS |

Table 11.16.3.10/G.994.1 – Standard information field – G.991.2 Annex A
Downstream PMMS parameters – NPar(3) coding – Octet 11

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A downstream PMMS NPar(3)s – Octet 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Downstream PMMS duration unspecified by terminal |
|  | x | x | x | x | x | x | x | x | Downstream PMMS duration (bits 6-1 × 50 ms) |
|  | x | x | 1 | 1 | 1 | 1 | 1 | 1 | Reserved for allocation by the ITU-T |

Table 11.16.3.11/G.994.1 – Standard information field – G.991.2 Annex A
Downstream PMMS parameters – NPar(3) coding – Octet 12

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A downstream PMMS NPar(3)s – Octet 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | x | x | x | Downstream PMMS scrambler polynomial Index (i2, i1, i0) |
|  | x | x | 1 | 1 | 1 | 1 | 1 | 1 | Reserved for allocation by the ITU-T |

Table 11.16.3.12/G.994.1 – Standard information field – G.991.2 Annex A
Downstream PMMS parameters – NPar(3) coding – Octet 13

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A downstream PMMS NPar(3)s – Octet 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 1 | x | x | x | x | x | Worst-case PMMS target margin (dB) (bits 5-1 × 1.0 dB – 10 dB) |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.3.13/G.994.1 – Standard information field – G.991.2 Annex A
Downstream PMMS parameters – NPar(3) coding – Octet 14

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A downstream PMMS NPar(3)s – Octet 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 1 | x | x | x | x | x | Current-condition PMMS target margin (dB) (bits 5-1 × 1.0 dB – 10 dB) |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.4/G.994.1 – Standard information field – G.991.2 Annex A
Upstream PMMS parameters – NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A upstream PMMS NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | x | x | x | x | x | Upstream PBO (dB) (bits 5-1 × 1.0 dB) |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |

Table 11.16.4.1/G.994.1 – Standard information field – G.991.2 Annex A
Upstream PMMS parameters – NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A upstream PMMS NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream base data rate unspecified by terminal |
|  | x | x | x | x | x | x | 1 | x | Transmit silence |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Upstream base data rate = 192 kbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Upstream base data rate = 256 kbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.4.2/G.994.1 – Standard information field – G.991.2 Annex A
Upstream PMMS parameters – NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A upstream PMMS NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream base data rate = 320 kbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Upstream base data rate = 384 kbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Upstream base data rate = 448 kbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Upstream base data rate = 512 kbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Upstream base data rate = 576 kbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Upstream base data rate = 640 kbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.4.3/G.994.1 – Standard information field – G.991.2 Annex A
Upstream PMMS parameters – NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.991.2 Annex A upstream PMMS NPar(3)s – Octet 4 |
|  | x | x | x | x | x | x | x | 1 | Upstream base data rate = 704 kbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Upstream base data rate = 768 kbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Upstream base data rate = 832 kbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Upstream base data rate = 896 kbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Upstream base data rate = 960 kbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Upstream base data rate = 1.024 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.4.4/G.994.1 – Standard information field – G.991.2 Annex A
Upstream PMMS parameters – NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A upstream PMMS NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream base data rate = 1.088 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Upstream base data rate = 1.152 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Upstream base data rate = 1.216 Mbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Upstream base data rate = 1.280 Mbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Upstream base data rate = 1.344 Mbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Upstream base data rate = 1.408 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.4.5/G.994.1 – Standard information field – G.991.2 Annex A
Upstream PMMS parameters – NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A upstream PMMS NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream base data rate = 1.472 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Upstream base data rate = 1.536 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Upstream base data rate = 1.600 Mbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Upstream base data rate = 1.664 Mbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Upstream base data rate = 1.728 Mbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Upstream base data rate = 1.792 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.4.6/G.994.1 – Standard information field – G.991.2 Annex A
Upstream PMMS parameters – NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A upstream PMMS NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream base data rate = 1.856 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Upstream base data rate = 1.920 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Upstream base data rate = 1.984 Mbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Upstream base data rate = 2.048 Mbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Upstream base data rate = 2.112 Mbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Upstream base data rate = 2.176 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.4.7/G.994.1 – Standard information field – G.991.2 Annex A
Upstream PMMS parameters – NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A upstream PMMS NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream base data rate = 2.240 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Upstream base data rate = 2.304 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Upstream base data rate = 768 kbit/s, asymmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Upstream base data rate = 1.544 Mbit/s, asymmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.4.8/G.994.1 – Standard information field – G.991.2 Annex A
Upstream PMMS parameters – NPar(3) coding – Octet 9

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A upstream PMMS NPar(3)s – Octet 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 1 | Fixed value during PMMS |

Table 11.16.4.9/G.994.1 – Standard information field – G.991.2 Annex A
Upstream PMMS parameters – NPar(3) coding – Octet 10

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A upstream PMMS NPar(3)s – Octet 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Fixed value during PMMS |

Table 11.16.4.10/G.994.1 – Standard information field – G.991.2 Annex A
Upstream PMMS parameters – NPar(3) coding – Octet 11

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A upstream PMMS NPar(3)s – Octet 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Upstream PMMS duration unspecified by terminal |
|  | x | x | x | x | x | x | x | x | Upstream PMMS duration (bits 6-1 × 50 ms) |
|  | x | x | 1 | 1 | 1 | 1 | 1 | 1 | Reserved for allocation by the ITU-T |

Table 11.16.4.11/G.994.1 – Standard information field – G.991.2 Annex A
Upstream PMMS parameters – NPar(3) coding – Octet 12

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A upstream PMMS NPar(3)s – Octet 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | x | x | x | Upstream PMMS scrambler polynomial Index (i2, i1, i0) |
|  | x | x | 1 | 1 | 1 | 1 | 1 | 1 | Reserved for allocation by the ITU-T |

Table 11.16.4.12/G.994.1 – Standard information field – G.991.2 Annex A
Upstream PMMS parameters – NPar(3) coding – Octet 13

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A upstream PMMS NPar(3)s – Octet 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 1 | x | x | x | x | x | Worst-case PMMS target margin (dB) (bits 5-1 × 1.0 dB – 10 dB) |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.4.13/G.994.1 – Standard information field – G.991.2 Annex A
Upstream PMMS parameters – NPar(3) coding – Octet 14

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A upstream PMMS NPar(3)s – Octet 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 1 | x | x | x | x | x | Current-condition PMMS target margin (dB) (bits 5-1 × 1.0 dB – 10 dB) |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.5/G.994.1 – Standard information field – G.991.2 Annex A
TPS-TC parameters – NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A TPS-TC parameter NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Clock Mode 1 |
|  | x | x | x | x | x | x | 1 | x | Clock Mode 2 |
|  | x | x | x | x | x | 1 | x | x | Clock Mode 3a |
|  | x | x | x | x | 1 | x | x | x | Clock Mode 3b |
|  | x | x | x | 1 | x | x | x | x | Low Latency |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.5.1/G.994.1 – Standard information field – G.991.2 Annex A
TPS-TC parameters – NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A TPS-TC parameter NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Clear Channel |
|  | x | x | x | x | x | x | 1 | x | Clear Channel Byte Oriented |
|  | x | x | x | x | x | 1 | x | x | Unaligned DS1 |
|  | x | x | x | x | 1 | x | x | x | Aligned DS1/Fractional DS1 |
|  | x | x | x | 1 | x | x | x | x | ATM |
|  | x | x | 1 | x | x | x | x | x | Synchronous ISDN-BRA |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.5.2/G.994.1 – Standard information field – G.991.2 Annex A
TPS-TC parameters – NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A TPS-TC parameter NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  | x | x | x | Number of ISDN BRA (0 to 6) |
|  |  |  | x | x | x |  |  |  | Z-bits used for ISDN BRA Signalling (0 to 7) |

Table 11.16.6/G.994.1 – Standard information field – G.991.2 Annex A
Downstream framing parameters NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A Downstream framing parameter NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | Sync Word (bits 14 and 13) |
|  |  |  | x | x | x | x |  |  | Stuff Bits (bits 1 to 4) |

Table 11.16.6.1/G.994.1 – Standard information field – G.991.2 Annex A
Downstream framing parameters NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A Downstream framing parameter NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Sync Word (bits 12 to 7) |

Table 11.16.6.2/G.994.1 – Standard information field – G.991.2 Annex A
Downstream framing parameters NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A Downstream framing parameter NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Sync Word (bits 6 to 1) |

Table 11.16.7/G.994.1 – Standard information field – G.991.2 Annex A
Upstream framing parameters NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A Upstream framing parameter NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | Sync Word (bits 14 and 13) |
|  |  |  | x | x | x | x |  |  | Stuff Bits (bits 1 to 4) |

Table 11.16.7.1/G.994.1 – Standard information field – G.991.2 Annex A
Upstream framing parameters NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A Upstream framing parameter NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Sync Word (bits 12 to 7) |

Table 11.16.7.2/G.994.1 – Standard information field – G.991.2 Annex A
Upstream framing parameters NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A Upstream framing parameter NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Sync Word (bits 6 to 1) |

Table 11.16.8/G.994.1 – Standard information field – G.991.2 Annex A
Dual Mode TPS-TC parameters – NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.991.2 Annex A Dual Mode TPS-TC parameter Npar(3)s – Octet 1 |
|  | x | x | x | x | x | x | x | x | TPS-TCa data rate – n  64 kbit/s (1 to 36) |
|  | x | x | 1 | 1 | 1 | 1 | 1 | 1 | Unspecified by terminal |

Table 11.16.8.1/G.994.1 – Standard information field – G.991.2 Annex A
Dual Mode TPS-TC parameters – NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A Dual Mode TPS-TC parameter Npar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | x | x | x | TPS-TCa sub-data rate – i  8 kbit/s (0 to 7) |
|  | x | x | 1 | 1 | 1 | 1 | 1 | 1 | Unspecified by terminal |

Table 11.16.8.2/G.994.1 – Standard information field – G.991.2 Annex A
Dual Mode TPS-TC parameters – NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A Dual Mode TPS-TC parameter Npar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Type 1 – TPS-TCb: Clear Channel |
|  | x | x | x | x | x | x | 1 | x | Type 1 – TPS-TCb: Clear Channel Byte-Oriented |
|  | x | x | x | x | x | 1 | x | x | Type 1 – TPS-TCb: Unaligned DS1 |
|  | x | x | x | x | 1 | x | x | x | Type 1 – TPS-TCb: Aligned DS1/Fractional DS1 |
|  | x | x | x | 1 | x | x | x | x | Type 1 – TPS-TCb: ATM |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.8.3/G.994.1 – Standard information field – G.991.2 Annex A
Dual Mode TPS-TC parameters – NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A Dual Mode TPS-TC parameter Npar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  | x | x | x | Number of ISDN BRA (0 to 6) |
|  |  |  | x | x | x |  |  |  | Z-bits used for ISDN BRA Signalling (0 to 7) |

Table 11.16.8.4/G.994.1 – Standard information field – G.991.2 Annex A
Dual Mode TPS-TC parameters – NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A Dual Mode TPS-TC parameter Npar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Type 2 – TPS-TCa: Unaligned DS1 |
|  | x | x | x | x | x | x | 1 | x | Type 2 – TPS-TCa: Aligned DS1/Fractional DS1 |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.8.5/G.994.1 – Standard information field – G.991.2 Annex A
Dual Mode TPS-TC parameters – NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex A Dual Mode TPS-TC parameter Npar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Type 3 – TPS-TCa: Unaligned DS1 |
|  | x | x | x | x | x | x | 1 | x | Type 3 – TPS-TCa: Aligned DS1/Fractional DS1 |
|  | x | x | x | x | x | 1 | x | x | Type 3 – TPS-TCb: Clear Channel |
|  | x | x | x | x | 1 | x | x | x | Type 3 – TPS-TCb: Clear Channel Byte-Oriented |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.17/G.994.1 – Standard information field – G.991.2
Annex B – NPar(2) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B NPar(2)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Training mode (Note) (see also Table 11.16) |
|  | x | x | x | x | x | x | 1 | x | PMMS mode (Note) (see also Table 11.16) |
|  | x | x | x | x | x | 1 | x | x | Regenerator silent period (Note) |
|  | x | x | x | x | 1 | x | x | x | 4-Wire |
|  | x | x | x | 1 | x | x | x | x | SRU |
|  | x | x | 1 | x | x | x | x | x | Diagnostic Mode |
| NOTE – Only one of these bits shall be set at any given time. |  |  |  |  |  |  |  |  |  |

Table 11.17.1/G.994.1 – Standard information field – G.991.2
Annex B – NPar(2) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B NPar(2)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Warm-start enable (Note) |
|  | x | x | x | x | x | x | 1 | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |
| NOTE – Reserved for ETSI TM6. |  |  |  |  |  |  |  |  |  |

Table 11.18/G.994.1 – Standard information field – G.991.2 Annex B –
SPar(2) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B SPar(2)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream training parameters |
|  | x | x | x | x | x | x | 1 | x | Upstream training parameters |
|  | x | x | x | x | x | 1 | x | x | Downstream PMMS parameters |
|  | x | x | x | x | 1 | x | x | x | Upstream PMMS parameters |
|  | x | x | x | 1 | x | x | x | x | TPS-TC Parameters |
|  | x | x | 1 | x | x | x | x | x | Downstream framing parameters |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.0.1/G.994.1 – Standard information field – G.991.2
Annex B – SPar(2) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B SPar(2)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream framing parameters |
|  | x | x | x | x | x | x | 1 | x | Dual-Mode TPS-TC Parameters |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.1/G.994.1 – Standard information field – G.991.2 Annex B
Downstream training parameters – NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B downstream training NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | x | x | x | x | x | Downstream PBO (dB) (bits 5-1 × 1.0 dB) |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |

Table 11.18.1.1/G.994.1 – Standard information field – G.991.2 Annex B
Downstream training parameters – NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B downstream training NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream base data rate unspecified by terminal |
|  | x | x | x | x | x | x | 1 | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Downstream base data rate = 192 kbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Downstream base data rate = 256 kbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.1.2/G.994.1 – Standard information field – G.991.2 Annex B
Downstream training parameters – NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B downstream training NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream base data rate = 320 kbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Downstream base data rate = 384 kbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Downstream base data rate = 448 kbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Downstream base data rate = 512 kbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Downstream base data rate = 576 kbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Downstream base data rate = 640 kbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.1.3/G.994.1 – Standard information field – G.991.2 Annex B
Downstream training parameters – NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B downstream training NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream base data rate = 704 kbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Downstream base data rate = 768 kbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Downstream base data rate = 832 kbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Downstream base data rate = 896 kbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Downstream base data rate = 960 kbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Downstream base data rate = 1.024 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.1.4/G.994.1 – Standard information field – G.991.2 Annex B
Downstream training parameters – NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B downstream training NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream base data rate = 1.088 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Downstream base data rate = 1.152 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Downstream base data rate = 1.216 Mbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Downstream base data rate = 1.280 Mbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Downstream base data rate = 1.344 Mbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Downstream base data rate = 1.408 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.1.5/G.994.1 – Standard information field – G.991.2 Annex B
Downstream training parameters – NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B downstream training NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream base data rate = 1.472 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Downstream base data rate = 1.536 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Downstream base data rate = 1.600 Mbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Downstream base data rate = 1.664 Mbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Downstream base data rate = 1.728 Mbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Downstream base data rate = 1.792 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.1.6/G.994.1 – Standard information field – G.991.2 Annex B
Downstream training parameters – NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B downstream training NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream base data rate = 1.856 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Downstream base data rate = 1.920 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Downstream base data rate = 1.984 Mbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Downstream base data rate = 2.048 Mbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Downstream base data rate = 2.112 Mbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Downstream base data rate = 2.176 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.1.7/G.994.1 – Standard information field – G.991.2 Annex B
Downstream training parameters – NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B downstream training NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream base data rate = 2.240 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Downstream base data rate = 2.304 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Downstream base data rate = 2.304 Mbit/s, asymmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Downstream base data rate = 2.048 Mbit/s, asymmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.1.8/G.994.1 – Standard information field – G.991.2 Annex B
Downstream training parameters – NPar(3) coding – Octet 9

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B downstream training NPar(3)s – Octet 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream sub-data rate = 0 kbit/s |
|  | x | x | x | x | x | x | 1 | x | Downstream sub-data rate = 8 kbit/s |
|  | x | x | x | x | x | 1 | x | x | Downstream sub-data rate = 16 kbit/s |
|  | x | x | x | x | 1 | x | x | x | Downstream sub-data rate = 24 kbit/s |
|  | x | x | x | 1 | x | x | x | x | Downstream sub-data rate = 32 kbit/s |
|  | x | x | 1 | x | x | x | x | x | Downstream sub-data rate = 40 kbit/s |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.1.9/G.994.1 – Standard information field – G.991.2 Annex B
Downstream training parameters – NPar(3) coding – Octet 10

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B downstream training NPar(3)s – Octet 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream sub-data rate = 48 kbit/s |
|  | x | x | x | x | x | x | 1 | x | Downstream sub-data rate = 56 kbit/s |
|  | x | x | x | x | x | 1 | x | x | Downstream sub-data rate unspecified by terminal |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.2/G.994.1 – Standard information field – G.991.2 Annex B
Upstream training parameters – NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B upstream training NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | x | x | x | x | x | Upstream PBO (dB) (bits 5-1 × 1.0 dB) |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |

Table 11.18.2.1/G.994.1 – Standard information field – G.991.2 Annex B
Upstream training parameters – NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B upstream training NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream base data rate unspecified by terminal |
|  | x | x | x | x | x | x | 1 | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Upstream base data rate = 192 kbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Upstream base data rate = 256 kbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.2.2/G.994.1 – Standard information field – G.991.2 Annex B
Upstream training parameters – NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B upstream training NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream base data rate = 320 kbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Upstream base data rate = 384 kbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Upstream base data rate = 448 kbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Upstream base data rate = 512 kbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Upstream base data rate = 576 kbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Upstream base data rate = 640 kbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.2.3/G.994.1 – Standard information field – G.991.2 Annex B
Upstream training parameters – NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B upstream training NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream base data rate = 704 kbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Upstream base data rate = 768 kbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Upstream base data rate = 832 kbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Upstream base data rate = 896 kbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Upstream base data rate = 960 kbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Upstream base data rate = 1.024 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.2.4/G.994.1 – Standard information field – G.991.2 Annex B
Upstream training parameters – NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B upstream training NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream base data rate = 1.088 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Upstream base data rate = 1.152 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Upstream base data rate = 1.216 Mbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Upstream base data rate = 1.280 Mbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Upstream base data rate = 1.344 Mbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Upstream base data rate = 1.408 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.2.5/G.994.1 – Standard information field – G.991.2 Annex B
Upstream training parameters – NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B upstream training NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream base data rate = 1.472 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Upstream base data rate = 1.536 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Upstream base data rate = 1.600 Mbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Upstream base data rate = 1.664 Mbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Upstream base data rate = 1.728 Mbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Upstream base data rate = 1.792 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.2.6/G.994.1 – Standard information field – G.991.2 Annex B
Upstream training parameters – NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B upstream training NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream base data rate = 1.856 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Upstream base data rate = 1.920 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Upstream base data rate = 1.984 Mbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Upstream base data rate = 2.048 Mbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Upstream base data rate = 2.112 Mbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Upstream base data rate = 2.176 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.2.7/G.994.1 – Standard information field – G.991.2 Annex B
Upstream training parameters – NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B upstream training NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream base data rate = 2.240 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Upstream base data rate = 2.304 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Upstream base data rate = 2.304 Mbit/s, asymmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Upstream base data rate = 2.048 Mbit/s, asymmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.2.8/G.994.1 – Standard information field – G.991.2 Annex B
Upstream training parameters – NPar(3) coding – Octet 9

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B upstream training NPar(3)s – Octet 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream sub-data rate = 0 kbit/s |
|  | x | x | x | x | x | x | 1 | x | Upstream sub-data rate = 8 kbit/s |
|  | x | x | x | x | x | 1 | x | x | Upstream sub-data rate = 16 kbit/s |
|  | x | x | x | x | 1 | x | x | x | Upstream sub-data rate = 24 kbit/s |
|  | x | x | x | 1 | x | x | x | x | Upstream sub-data rate = 32 kbit/s |
|  | x | x | 1 | x | x | x | x | x | Upstream sub-data rate = 40 kbit/s |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.2.9/G.994.1 – Standard information field – G.991.2 Annex B
Upstream training parameters – NPar(3) coding – Octet 10

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B upstream training NPar(3)s – Octet 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream sub-data rate = 48 kbit/s |
|  | x | x | x | x | x | x | 1 | x | Upstream sub-data rate = 56 kbit/s |
|  | x | x | x | x | x | 1 | x | x | Upstream sub-data rate unspecified by terminal |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.3/G.994.1 – Standard information field – G.991.2 Annex B
Downstream PMMS parameters – NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B downstream PMMS NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | x | x | x | x | x | Downstream PBO (dB) (bits 5-1 × 1.0 dB) |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |

Table 11.18.3.1/G.994.1 – Standard information field – G.991.2 Annex B
Downstream PMMS parameters – NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B downstream PMMS NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream base data rate unspecified by terminal |
|  | x | x | x | x | x | x | 1 | x | Transmit silence |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Downstream base data rate = 192 kbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Downstream base data rate = 256 kbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.3.2/G.994.1 – Standard information field – G.991.2 Annex B
Downstream PMMS parameters – NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B downstream PMMS NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream base data rate = 320 kbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Downstream base data rate = 384 kbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Downstream base data rate = 448 kbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Downstream base data rate = 512 kbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Downstream base data rate = 576 kbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Downstream base data rate = 640 kbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.3.3/G.994.1 – Standard information field – G.991.2 Annex B
Downstream PMMS parameters – NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B downstream PMMS NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream base data rate = 704 kbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Downstream base data rate = 768 kbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Downstream base data rate = 832 kbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Downstream base data rate = 896 kbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Downstream base data rate = 960 kbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Downstream base data rate = 1.024 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.3.4/G.994.1 – Standard information field – G.991.2 Annex B
Downstream PMMS parameters – NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B downstream PMMS NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream base data rate = 1.088 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Downstream base data rate = 1.152 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Downstream base data rate = 1.216 Mbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Downstream base data rate = 1.280 Mbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Downstream base data rate = 1.344 Mbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Downstream base data rate = 1.408 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.3.5/G.994.1 – Standard information field – G.991.2 Annex B
Downstream PMMS parameters – NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B downstream PMMS NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream base data rate = 1.472 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Downstream base data rate = 1.536 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Downstream base data rate = 1.600 Mbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Downstream base data rate = 1.664 Mbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Downstream base data rate = 1.728 Mbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Downstream base data rate = 1.792 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.3.6/G.994.1 – Standard information field – G.991.2 Annex B
Downstream PMMS parameters – NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B downstream PMMS NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream base data rate = 1.856 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Downstream base data rate = 1.920 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Downstream base data rate = 1.984 Mbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Downstream base data rate = 2.048 Mbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Downstream base data rate = 2.112 Mbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Downstream base data rate = 2.176 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.3.7/G.994.1 – Standard information field – G.991.2 Annex B
Downstream PMMS parameters – NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B downstream PMMS NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream base data rate = 2.240 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Downstream base data rate = 2.304 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Downstream base data rate = 2.304 Mbit/s, asymmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Downstream base data rate = 2.048 Mbit/s, asymmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.3.8/G.994.1 – Standard information field – G.991.2 Annex B
Downstream PMMS parameters – NPar(3) coding – Octet 9

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B downstream PMMS NPar(3)s – Octet 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 1 | Fixed value during PMMS |

Table 11.18.3.9/G.994.1 – Standard information field – G.991.2 Annex B
Downstream PMMS parameters – NPar(3) coding – Octet 10

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B downstream PMMS NPar(3)s – Octet 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Fixed value during PMMS |

Table 11.18.3.10/G.994.1 – Standard information field – G.991.2 Annex B
Downstream PMMS parameters – NPar(3) coding – Octet 11

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B downstream PMMS NPar(3)s – Octet 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Downstream PMMS duration unspecified by terminal |
|  | x | x | x | x | x | x | x | x | Downstream PMMS duration (bits 6-1 × 50 ms) |
|  | x | x | 1 | 1 | 1 | 1 | 1 | 1 | Reserved for allocation by the ITU-T |

Table 11.18.3.11/G.994.1 – Standard information field – G.991.2 Annex B
Downstream PMMS parameters – NPar(3) coding – Octet 12

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B downstream PMMS NPar(3)s – Octet 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | x | x | x | Downstream PMMS scrambler polynomial Index (i2, i1, i0) |
|  | x | x | 1 | 1 | 1 | 1 | 1 | 1 | Reserved for allocation by the ITU-T |

Table 11.18.3.12/G.994.1 – Standard information field – G.991.2 Annex B
Downstream PMMS parameters – NPar(3) coding – Octet 13

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B downstream PMMS NPar(3)s – Octet 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 1 | x | x | x | x | x | Worst-case PMMS target margin (dB) (bits 5-1 × 1.0 dB – 10 dB) |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.3.13/G.994.1 – Standard information field – G.991.2 Annex B
Downstream PMMS parameters – NPar(3) coding – Octet 14

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B downstream PMMS NPar(3)s – Octet 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 1 | x | x | x | x | x | Current-condition PMMS target margin (dB) (bits 5-1 × 1.0 dB – 10 dB) |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.4/G.994.1 – Standard information field – G.991.2 Annex B
Upstream PMMS parameters – NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B upstream PMMS NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | x | x | x | x | x | Upstream PBO (dB) (bits 5-1 × 1.0 dB) – Octet 1 |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |

Table 11.18.4.1/G.994.1 – Standard information field – G.991.2 Annex B
Upstream PMMS parameters – NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B upstream PMMS NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream base data rate unspecified by terminal |
|  | x | x | x | x | x | x | 1 | x | Transmit silence |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Upstream base data rate = 192 kbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Upstream base data rate = 256 kbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.4.2/G.994.1 – Standard information field – G.991.2 Annex B
Upstream PMMS parameters – NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B upstream PMMS NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream base data rate = 320 kbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Upstream base data rate = 384 kbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Upstream base data rate = 448 kbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Upstream base data rate = 512 kbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Upstream base data rate = 576 kbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Upstream base data rate = 640 kbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.4.3/G.994.1 – Standard information field – G.991.2 Annex B
Upstream PMMS parameters – NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B upstream PMMS NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream base data rate = 704 kbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Upstream base data rate = 768 kbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Upstream base data rate = 832 kbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Upstream base data rate = 896 kbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Upstream base data rate = 960 kbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Upstream base data rate = 1.024 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.4.4/G.994.1 – Standard information field – G.991.2 Annex B
Upstream PMMS parameters – NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B upstream PMMS NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream base data rate = 1.088 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Upstream base data rate = 1.152 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Upstream base data rate = 1.216 Mbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Upstream base data rate = 1.280 Mbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Upstream base data rate = 1.344 Mbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Upstream base data rate = 1.408 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.4.5/G.994.1 – Standard information field – G.991.2 Annex B
Upstream PMMS parameters – NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B upstream PMMS NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream base data rate = 1.472 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Upstream base data rate = 1.536 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Upstream base data rate = 1.600 Mbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Upstream base data rate = 1.664 Mbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Upstream base data rate = 1.728 Mbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Upstream base data rate = 1.792 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.4.6/G.994.1 – Standard information field – G.991.2 Annex B
Upstream PMMS parameters – NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B upstream PMMS NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream base data rate = 1.856 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Upstream base data rate = 1.920 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Upstream base data rate = 1.984 Mbit/s, symmetric PSD |
|  | x | x | x | x | 1 | x | x | x | Upstream base data rate = 2.048 Mbit/s, symmetric PSD |
|  | x | x | x | 1 | x | x | x | x | Upstream base data rate = 2.112 Mbit/s, symmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Upstream base data rate = 2.176 Mbit/s, symmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.4.7/G.994.1 – Standard information field – G.991.2 Annex B
Upstream PMMS parameters – NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B upstream PMMS NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream base data rate = 2.240 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | x | 1 | x | Upstream base data rate = 2.304 Mbit/s, symmetric PSD |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Upstream base data rate = 2.304 Mbit/s, asymmetric PSD |
|  | x | x | 1 | x | x | x | x | x | Upstream base data rate = 2.048 Mbit/s, asymmetric PSD |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.4.8/G.994.1 – Standard information field – G.991.2 Annex B
Upstream PMMS parameters – NPar(3) coding – Octet 9

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B upstream PMMS NPar(3)s – Octet 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 1 | Fixed value during PMMS |

Table 11.18.4.9/G.994.1 – Standard information field – G.991.2 Annex B
Upstream PMMS parameters – NPar(3) coding – Octet 10

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B upstream PMMS NPar(3)s – Octet 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Fixed value during PMMS |

Table 11.18.4.10/G.994.1 – Standard information field – G.991.2 Annex B
Upstream PMMS parameters – NPar(3) coding – Octet 11

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B upstream PMMS NPar(3)s – Octet 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Upstream PMMS duration unspecified by terminal |
|  | x | x | x | x | x | x | x | x | Upstream PMMS duration (bits 6-1 × 50 ms) |
|  | x | x | 1 | 1 | 1 | 1 | 1 | 1 | Reserved for allocation by the ITU-T |

Table 11.18.4.11/G.994.1 – Standard information field – G.991.2 Annex B
Upstream PMMS parameters – NPar(3) coding – Octet 12

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B upstream PMMS NPar(3)s – Octet 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | x | x | x | Upstream PMMS scrambler polynomial Index (i2, i1, i0) |
|  | x | x | 1 | 1 | 1 | 1 | 1 | 1 | Reserved for allocation by the ITU-T |

Table 11.18.4.12/G.994.1 – Standard information field – G.991.2 Annex B
Upstream PMMS parameters – NPar(3) coding – Octet 13

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B upstream PMMS NPar(3)s – Octet 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 1 | x | x | x | x | x | Worst-case PMMS target margin (dB) (bits 5-1 × 1.0 dB – 10 dB) |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.4.13/G.994.1 – Standard information field – G.991.2 Annex B
Upstream PMMS parameters – NPar(3) coding – Octet 14

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B upstream PMMS NPar(3)s – Octet 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 1 | x | x | x | x | x | Current-condition PMMS target margin (dB) (bits 5-1 × 1.0 dB – 10 dB) |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.5/G.994.1 – Standard information field – G.991.2 Annex B
TPS-TC parameters – NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B TPS-TC parameter NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Clock Mode 1 |
|  | x | x | x | x | x | x | 1 | x | Clock Mode 2 |
|  | x | x | x | x | x | 1 | x | x | Clock Mode 3a |
|  | x | x | x | x | 1 | x | x | x | Clock Mode 3b |
|  | x | x | x | 1 | x | x | x | x | Low Latency |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.5.1/G.994.1 – Standard information field – G.991.2 Annex B
TPS-TC parameters – NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B TPS-TC parameter NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Clear Channel |
|  | x | x | x | x | x | x | 1 | x | Clear Channel Byte Oriented |
|  | x | x | x | x | x | 1 | x | x | Unaligned D2048S |
|  | x | x | x | x | 1 | x | x | x | Aligned D2048S/Fractional D2048S |
|  | x | x | x | 1 | x | x | x | x | ATM |
|  | x | x | 1 | x | x | x | x | x | Unaligned D2048U |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.5.2/G.994.1 – Standard information field – G.991.2 Annex B
TPS-TC parameters – NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B TPS-TC parameter NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Synchronous ISDN-BRA |
|  | x | x | x | x | x | x | 1 | x | PTM (Note) |
|  | x | x | x | x | x | 1 | x | x | STM with DSC (Note) |
|  | x | x | x | x | 1 | x | x | x | LAPV5 Enveloped POTS or ISDN (Note) |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |
| NOTE – Reserved for ETSI TM6. |  |  |  |  |  |  |  |  |  |

Table 11.18.5.3/G.994.1 – Standard information field – G.991.2 Annex B
TPS-TC parameters – NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B TPS-TC parameter NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  | x | x | x | Number of ISDN BRA (0 to 6) |
|  |  |  | x | x | x |  |  |  | Z-bits used for ISDN BRA Signalling (0 to 7) |

Table 11.18.6/G.994.1 – Standard information field – G.991.2 Annex B
Downstream framing parameters NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B Downstream framing parameter NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | Sync Word (bits 14 and 13) |
|  |  |  | x | x | x | x |  |  | Stuff Bits (bits 1 to 4) |

Table 11.18.6.1/G.994.1 – Standard information field – G.991.2 Annex B
Downstream framing parameters NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B Downstream framing parameter NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Sync Word (bits 12 to 7) |

Table 11.18.6.2/G.994.1 – Standard information field – G.991.2 Annex B
Downstream framing parameters NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B Downstream framing parameter NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Sync Word (bits 6 to 1) |

Table 11.18.7/G.994.1 – Standard information field – G.991.2 Annex B
Upstream framing parameters NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B Upstream framing parameter NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | Sync Word (bits 14 and 13) |
|  |  |  | x | x | x | x |  |  | Stuff Bits (bits 1 to 4) |

Table 11.18.7.1/G.994.1 – Standard information field – G.991.2 Annex B
Upstream framing parameters NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B Upstream framing parameter NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Sync Word (bits 12 to 7) |

Table 11.18.7.2/G.994.1 – Standard information field – G.991.2 Annex B
Upstream framing parameters NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B Upstream framing parameter NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Sync Word (bits 6 to 1) |

Table 11.18.8/G.994.1 – Standard information field – G.991.2 Annex B
Dual Mode TPS-TC parameters – NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B Dual Mode TPS-TC parameter Npar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | TPS-TCa data rate – n  64 kbit/s (1 to 36) |
|  | x | x | 1 | 1 | 1 | 1 | 1 | 1 | Unspecified by terminal |

Table 11.18.8.1/G.994.1 – Standard information field – G.991.2 Annex B
Dual Mode TPS-TC parameters – NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B Dual Mode TPS-TC parameter Npar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | x | x | x | TPS-TCa sub-data rate – i  8 kbit/s (0 to 7) |
|  | x | x | 1 | 1 | 1 | 1 | 1 | 1 | Unspecified by terminal |

Table 11.18.8.2/G.994.1 – Standard information field – G.991.2 Annex B
Dual Mode TPS-TC parameters – NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B Dual Mode TPS-TC parameter NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Type 1 – TPS-TCb: Clear Channel |
|  | x | x | x | x | x | x | 1 | x | Type 1 – TPS-TCb: Clear Channel Byte-Oriented |
|  | x | x | x | x | x | 1 | x | x | Type 1 – TPS-TCb: Unaligned D2048U |
|  | x | x | x | x | 1 | x | x | x | Type 1 – TPS-TCb: Unaligned D2048S |
|  | x | x | x | 1 | x | x | x | x | Type 1 – TPS-TCb: Aligned D2048S/Fractional D2048S |
|  | x | x | 1 | x | x | x | x | x | Type 1 – TPS-TCb: ATM |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.8.3/G.994.1 – Standard information field – G.991.2 Annex B
Dual Mode TPS-TC parameters – NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B Dual Mode TPS-TC parameter Npar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  | x | x | x | Number of ISDN BRA (0 to 6) |
|  |  |  | x | x | x |  |  |  | Z-bits used for ISDN BRA Signalling (0 to 7) |

Table 11.18.8.4/G.994.1 – Standard information field – G.991.2 Annex B
Dual Mode TPS-TC parameters – NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B Dual Mode TPS-TC parameter NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Type 2 – TPS-TCa: Unaligned D2048U |
|  | x | x | x | x | x | x | 1 | x | Type 2 – TPS-TCa: Unaligned D2048S |
|  | x | x | x | x | x | 1 | x | x | Type 2 – TPS-TCa: Aligned D2048S/Fractional D2048S |
|  | x | x | x | x | 1 | x | x | x | Type 2 – TPS-TCa: Synchronous ISDN BRA |
|  | x | x | x | 1 | x | x | x | x | Type 2 – TPS-TCb: ATM (Note) |
|  | x | x | 1 | x | x | x | x | x | Type 2 – TPS-TCb: PTM (Note) |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |
| NOTE – Reserved for ETSI TM6. |  |  |  |  |  |  |  |  |  |

Table 11.18.8.5/G.994.1 – Standard information field – G.991.2 Annex B
Dual Mode TPS-TC parameters – NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B Dual Mode TPS-TC parameter NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Type 3 – TPS-TCa: Unaligned D2048U |
|  | x | x | x | x | x | x | 1 | x | Type 3 – TPS-TCa: Unaligned D2048S |
|  | x | x | x | x | x | 1 | x | x | Type 3 – TPS-TCa: Aligned D2048S/Fractional D2048S |
|  | x | x | x | x | 1 | x | x | x | Type 3 – TPS-TCa: Synchronous ISDN BRA |
|  | x | x | x | 1 | x | x | x | x | Type 3 – TPS-TCb: Clear Channel |
|  | x | x | 1 | x | x | x | x | x | Type 3 – TPS-TCb: Clear Channel Byte-Oriented |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.8.6/G.994.1 – Standard information field – G.991.2 Annex B
Dual Mode TPS-TC parameters – NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B Dual Mode TPS-TC parameter NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Type 1 – TPS-TCb: PTM (Note) |
|  | x | x | x | x | x | x | 1 | x | Type 1 – TPS-TCa: STM with DSC (Note) |
|  | x | x | x | x | x | 1 | x | x | Type 1 – TPS-TCa: LAPV5 Enveloped POTS or ISDN (Note) |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |
| NOTE – Reserved for ETSI TM6. |  |  |  |  |  |  |  |  |  |

Table 11.18.8.7/G.994.1 – Standard information field – G.991.2 Annex B
Dual Mode TPS-TC parameters – NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex B Dual Mode TPS-TC parameter NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | DRR Support (Note): 01 DRR not supported 10 DRR supported, STU-C DRR master 11 DRR supported, STU-R DRR master |
|  | x | x | x | x | x | x |  |  | Lead Time (in frames) (Note) count = bits 6-2 (supported values = 1 to 15) |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |
| NOTE – Reserved for ETSI TM6. |  |  |  |  |  |  |  |  |  |

Table 11.23/G.994.1 – Standard information field – ETSI MCM VDSL – NPar(2) coding

|  | Bits |  |  |  |  |  |  |  | ETSI MCM VDSL Npar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream use of lower band |
|  | x | x | x | x | x | x | 1 | x | Downstream use of lower band |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by ETSI |
|  | x | x | x | x | 1 | x | x | x | STM |
|  | x | x | x | 1 | x | x | x | x | ATM |
|  | x | x | 1 | x | x | x | x | x | G.997.1 – Clear EOC OAM |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.24/G.994.1 – Standard information field – ETSI MCM VDSL – SPar(2) coding

|  | Bits |  |  |  |  |  |  |  | ETSI MCM VDSL Spar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Sub-channel information (Note) |
|  | x | x | x | x | x | x | 1 | x | Reserved for allocation by ETSI |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by ETSI |
|  | x | x | x | x | 1 | x | x | x | IDFT/DFT size |
|  | x | x | x | 1 | x | x | x | x | Initial length of CE |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by ETSI |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |
| NOTE – The use of this bit is for further study and shall be set to ZERO in CLR, CL, MP and MS messages. This bit specifies the supported bearer channels for VDSL upstream/downstream transmissions in the TPS-TC sub-layer. The bearer channels are for further study. |  |  |  |  |  |  |  |  |  |

Table 11.24.4/G.994.1 – Standard information field – ETSI MCM VDSL
IDFT/DFT size – NPar(3) coding

|  | Bits |  |  |  |  |  |  |  | ETSI MCM VDSL IDFT/DFT size Npar(3)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | n5 | n4 | n3 | n2 | n1 | n0 | IDFT/DFT size (n × 256 points) |

Table 11.24.5/G.994.1 – Standard information field – ETSI MCM VDSL
Initial length of CE – NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | ETSI MCM VDSL Initial length of CE Npar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | ce9 | ce8 | ce7 | ce6 | Initial sample length of cyclic extension (high order bits) |

Table 11.24.5.1/G.994.1 – Standard information field – ETSI MCM VDSL
Initial length of CE – NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | ETSI MCM VDSL Initial length of CE Npar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | ce5 | ce4 | ce3 | ce2 | ce1 | ce0 | Initial sample length of cyclic extension (high order bits) |

Table 11.29/G.994.1 – Standard information field – G.992.3 Annex A NPar(2) coding

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A NPar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | NTR |
|  | x | x | x | x | x | x | 1 | x | Short initialization |
|  | x | x | x | x | x | 1 | x | x | Diagnostics mode |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.30/G.994.1 – Standard information field – G.992.3 Annex A
SPar(2) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A SPar(2)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Spectrum bounds upstream |
|  | x | x | x | x | x | x | 1 | x | Spectrum shaping upstream |
|  | x | x | x | x | x | 1 | x | x | Spectrum bounds downstream |
|  | x | x | x | x | 1 | x | x | x | Spectrum shaping downstream |
|  | x | x | x | 1 | x | x | x | x | Transmit signal images above the Nyquist frequency |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.30.0.1/G.994.1 – Standard information field – G.992.3 Annex A
SPar(2) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A SPar(2)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream overhead data rate |
|  | x | x | x | x | x | x | 1 | x | Upstream overhead data rate |
|  | x | x | x | x | x | 1 | x | x | Maximum number of downstream TPS-TC functions of each type |
|  | x | x | x | x | 1 | x | x | x | Maximum number of upstream TPS-TC functions of each type |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.30.0.2/G.994.1 – Standard information field – G.992.3 Annex A
SPar(2) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A SPar(2)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream STM TPS-TC #0 |
|  | x | x | x | x | x | x | 1 | x | Upstream STM TPS-TC #0 |
|  | x | x | x | x | x | 1 | x | x | Downstream ATM TPS-TC #0 |
|  | x | x | x | x | 1 | x | x | x | Upstream ATM TPS-TC #0 |
|  | x | x | x | 1 | x | x | x | x | Downstream PTM TPS-TC #0 |
|  | x | x | 1 | x | x | x | x | x | Upstream PTM TPS-TC #0 |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.30.0.3/G.994.1 – Standard information field – G.992.3 Annex A
SPar(2) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A SPar(2)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream PMS-TC latency path #0 supported |
|  | x | x | x | x | x | x | 1 | x | Upstream PMS-TC latency path #0 supported |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.30.0.4/G.994.1 – Standard information field – G.992.3 Annex A
SPar(2) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A SPar(2)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream STM TPS-TC #1 |
|  | x | x | x | x | x | x | 1 | x | Upstream STM TPS-TC #1 |
|  | x | x | x | x | x | 1 | x | x | Downstream ATM TPS-TC #1 |
|  | x | x | x | x | 1 | x | x | x | Upstream ATM TPS-TC #1 |
|  | x | x | x | 1 | x | x | x | x | Downstream PTM TPS-TC #1 |
|  | x | x | 1 | x | x | x | x | x | Upstream PTM TPS-TC #1 |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.30.0.5/G.994.1 – Standard information field – G.992.3 Annex A
SPar(2) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A SPar(2)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream PMS-TC latency path #1 supported |
|  | x | x | x | x | x | x | 1 | x | Upstream PMS-TC latency path #1 supported |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.30.0.6/G.994.1 – Standard information field – G.992.3 Annex A
SPar(2) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A SPar(2)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream STM TPS-TC #2 |
|  | x | x | x | x | x | x | 1 | x | Upstream STM TPS-TC #2 |
|  | x | x | x | x | x | 1 | x | x | Downstream ATM TPS-TC #2 |
|  | x | x | x | x | 1 | x | x | x | Upstream ATM TPS-TC #2 |
|  | x | x | x | 1 | x | x | x | x | Downstream PTM TPS-TC #2 |
|  | x | x | 1 | x | x | x | x | x | Upstream PTM TPS-TC #2 |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.30.0.7/G.994.1 – Standard information field – G.992.3 Annex A
SPar(2) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A SPar(2)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream PMS-TC latency path #2 supported |
|  | x | x | x | x | x | x | 1 | x | Upstream PMS-TC latency path #2 supported |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.30.0.8/G.994.1 – Standard information field – G.992.3 Annex A
SPar(2) coding – Octet 9

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A SPar(2)s – Octet 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream STM TPS-TC #3 |
|  | x | x | x | x | x | x | 1 | x | Upstream STM TPS-TC #3 |
|  | x | x | x | x | x | 1 | x | x | Downstream ATM TPS-TC #3 |
|  | x | x | x | x | 1 | x | x | x | Upstream ATM TPS-TC #3 |
|  | x | x | x | 1 | x | x | x | x | Downstream PTM TPS-TC #3 |
|  | x | x | 1 | x | x | x | x | x | Upstream PTM TPS-TC #3 |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.30.0.9/G.994.1 – Standard information field – G.992.3 Annex A
SPar(2) coding – Octet 10

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A SPar(2)s – Octet 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream PMS-TC latency path #3 supported |
|  | x | x | x | x | x | x | 1 | x | Upstream PMS-TC latency path #3 supported |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.30.1/G.994.1 – Standard information field – G.992.3 Annex A
Spectrum bounds upstream NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Spectrum bounds upstream NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | x | x | x | NOMPSDus (bits 9 to 7) |

Table 11.30.1.1/G.994.1 – Standard information field – G.992.3 Annex A
Spectrum bounds upstream NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Spectrum bounds upstream NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | NOMPSDus (bits 6 to 1) |

Table 11.30.1.2/G.994.1 – Standard information field – G.992.3 Annex A
Spectrum bounds upstream NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Spectrum bounds upstream NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | x | x | x | MAXNOMPSDus (bits 9 to 7) |

Table 11.30.1.3/G.994.1 – Standard information field – G.992.3 Annex A
Spectrum bounds upstream NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Spectrum bounds upstream NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | MAXNOMPSDus (bits 6 to 1) |

Table 11.30.1.4/G.994.1 – Standard information field – G.992.3 Annex A
Spectrum bounds upstream NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Spectrum bounds upstream NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | x | x | x | MAXNOMATPus (bits 9 to 7) |

Table 11.30.1.5/G.994.1 – Standard information field – G.992.3 Annex A
Spectrum bounds upstream NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Spectrum bounds upstream NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | MAXNOMATPus (bits 6 to 1) |

Table 11.30.2/G.994.1 – Standard information field – G.992.3 Annex A
Spectrum shaping upstream NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Spectrum shaping upstream NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | "First" subcarrier index i (bits 12 to 7) |

Table 11.30.2.1/G.994.1 – Standard information field – G.992.3 Annex A
Spectrum shaping upstream NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Spectrum shaping upstream NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | "First" subcarrier index i (bits 6 to 1) |

Table 11.30.2.2/G.994.1 – Standard information field – G.992.3 Annex A
Spectrum shaping upstream NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Spectrum shaping upstream NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  |  |  | x |  |  |  |  |  | "First" subcarrier in supported set |
|  | x | x |  | 0 | 0 | 0 | 0 | x | "First" log_tssi (bit 7) |

Table 11.30.2.3/G.994.1 – Standard information field – G.992.3 Annex A
Spectrum shaping upstream NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Spectrum shaping upstream NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | "First" log_tssi (bits 6 to 1) |

Table 11.30.2.4*(j – 1)/G.994.1 – Standard information field – G.992.3 Annex A
Spectrum shaping upstream NPar(3) coding – Octet 4*(j – 1) + 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Spectrum shaping upstream NPar(3)s – Octet 4*(j – 1) + 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | "Last" subcarrier index i (bits 12 to 7) |
| NOTE – j is the number of subcarrier indices used to specify the spectral shape. |  |  |  |  |  |  |  |  |  |

Table 11.30.2.4*(j – 1) + 1/G.994.1 – Standard information field – G.992.3 Annex A
Spectrum shaping upstream NPar(3) coding – Octet 4*(j – 1) + 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Spectrum shaping upstream NPar(3)s – Octet 4*(j – 1) + 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | "Last" subcarrier index i (bits 6 to 1) |
| NOTE – j is the number of subcarrier indices used to specify the spectral shape. |  |  |  |  |  |  |  |  |  |

Table 11.30.2.4*(j – 1) + 2/G.994.1 – Standard information field – G.992.3 Annex A
Spectrum shaping upstream NPar(3) coding – Octet 4*(j – 1) + 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Spectrum shaping upstream NPar(3)s – Octet 4*(j – 1) + 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  |  |  | x |  |  |  |  |  | "Last" subcarrier in supported set |
|  | x | x |  | 0 | 0 | 0 | 0 | x | "Last" log_tssi (bit 7) |
| NOTE – j is the number of subcarrier indices used to specify the spectral shape. |  |  |  |  |  |  |  |  |  |

Table 11.30.2.4*(j – 1) + 3/G.994.1 – Standard information field – G.992.3 Annex A
Spectrum shaping upstream NPar(3) coding – Octet 4*(j – 1) + 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Spectrum shaping upstream NPar(3)s – Octet 4*(j – 1) + 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | "Last" log_tssi (bits 6 to 1) |

Table 11.30.3/G.994.1 – Standard information field – G.992.3 Annex A
Spectrum bounds downstream NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Spectrum bounds downstream NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | x | x | x | NOMPSDds (bits 9 to 7) |

Table 11.30.3.1/G.994.1 – Standard information field – G.992.3 Annex A
Spectrum bounds downstream NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Spectrum bounds downstream NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | NOMPSDds (bits 6 to 1) |

Table 11.30.3.2/G.994.1 – Standard information field – G.992.3 Annex A
Spectrum bounds downstream NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Spectrum bounds downstream NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | x | x | x | MAXNOMPSDds (bits 9 to 7) |

Table 11.30.3.3/G.994.1 – Standard information field – G.992.3 Annex A
Spectrum bounds downstream NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Spectrum bounds downstream NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | MAXNOMPSDds (bits 6 to 1) |

Table 11.30.3.4/G.994.1 – Standard information field – G.992.3 Annex A
Spectrum bounds downstream NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Spectrum bounds downstream NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | x | x | x | MAXNOMATPds (bits 9 to 7) |

Table 11.30.3.5/G.994.1 – Standard information field – G.992.3 Annex A
Spectrum bounds downstream NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Spectrum bounds downstream NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | MAXNOMATPds (bits 6 to 1) |

Table 11.30.4/G.994.1 – Standard information field – G.992.3 Annex A
Spectrum shaping downstream NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Spectrum shaping downstream NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | "First" subcarrier index i (bits 12 to 7) |

Table 11.30.4.1/G.994.1 – Standard information field – G.992.3 Annex A
Spectrum shaping downstream NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Spectrum shaping downstream NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | "First" subcarrier index i (bits 6 to 1) |

Table 11.30.4.2/G.994.1 – Standard information field – G.992.3 Annex A
Spectrum shaping downstream NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Spectrum shaping downstream NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  |  |  | x |  |  |  |  |  | "First" subcarrier in supported set |
|  | x | x |  | 0 | 0 | 0 | 0 | x | "First" log_tssi (bit 7) |

Table 11.30.4.3/G.994.1 – Standard information field – G.992.3 Annex A
Spectrum shaping downstream NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Spectrum shaping downstream NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | "First" log_tssi (bits 6 to 1) |

Table 11.30.4.4*(j – 1)/G.994.1 – Standard information field – G.992.3 Annex A
Spectrum shaping downstream NPar(3) coding – Octet 4*(j – 1) + 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Spectrum shaping downstream NPar(3)s – Octet 4*(j – 1) + 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | "Last" subcarrier index i (bits 12 to 7) |
| NOTE – j is the number of subcarrier indices used to specify the spectral shape. |  |  |  |  |  |  |  |  |  |

Table 11.30.4.4*(j – 1) + 1/G.994.1 – Standard information field – G.992.3 Annex A
Spectrum shaping downstream NPar(3) coding – Octet 4*(j – 1) + 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Spectrum shaping downstream NPar(3)s – Octet 4*(j – 1) + 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | "Last" subcarrier index i (bits 6 to 1) |
| NOTE – j is the number of subcarrier indices used to specify the spectral shape. |  |  |  |  |  |  |  |  |  |

Table 11.30.4.4*(j – 1) + 2/G.994.1 – Standard information field – G.992.3 Annex A
Spectrum shaping downstream NPar(3) coding – Octet 4*(j – 1) + 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Spectrum shaping downstream NPar(3)s – Octet 4*(j – 1) + 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  |  |  | x |  |  |  |  |  | "Last" subcarrier in supported set |
|  | x | x |  | 0 | 0 | 0 | 0 | x | "Last" log_tssi (bit 7) |
| NOTE – j is the number of subcarrier indices used to specify the spectral shape. |  |  |  |  |  |  |  |  |  |

Table 11.30.4.4*(j – 1) + 3/G.994.1 – Standard information field – G.992.3 Annex A
Spectrum shaping downstream NPar(3) coding – Octet 4*(j – 1) + 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Spectrum shaping downstream NPar(3)s – Octet 4*(j – 1) + 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | "Last" log_tssi (bits 6 to 1) |

Table 11.30.5/G.994.1 – Standard information field – G.992.3 Annex A
Transmit signal images above the Nyquist frequency NPar(3) coding

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A transmit signal images above the Nyquist frequency NPar(3)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x |  |  | IDFT size N |
|  | x | x |  |  |  |  | x | x | IFFT fill |

Table 11.30.7/G.994.1 – Standard information field – G.992.3 Annex A
Downstream overhead data rate NPar(3) coding

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream overhead data rate NPar(3)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Minimum overhead data rate ((n + 1) × 1 kbit/s, n = 3 to 63) |

Table 11.30.8/G.994.1 – Standard information field – G.992.3 Annex A
Upstream overhead data rate NPar(3) coding

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream overhead data rate NPar(3)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Minimum overhead data rate ((n + 1) × 1 kbit/s, n = 3 to 63) |

Table 11.30.9/G.994.1 – Standard information field – G.992.3 Annex A
Maximum number of downstream TPS-TC functions of each type NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Maximum number of downstream TPS-TC functions of each type NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  | x | x | x | Maximum number of downstream STM TPS-TCs (n = 0 to 4) |
|  | x | x | x | x | x |  |  |  | Maximum number of downstream ATM TPS-TCs (n = 0 to 4) |

Table 11.30.9.1/G.994.1 – Standard information field – G.992.3 Annex A
Maximum number of downstream TPS-TC functions of each type NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Maximum number of downstream TPS-TC functions of each type NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  | x | x | x | Maximum number of downstream PTM TPS-TCs (n = 0 to 4) |
|  | x | x | x | x | x |  |  |  | Reserved for allocation by the ITU-T |

Table 11.30.10/G.994.1 – Standard information field – G.992.3 Annex A
Maximum number of upstream TPS-TC functions of each type NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Maximum number of upstream TPS-TC functions of each type NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  | x | x | x | Maximum number of upstream STM TPS-TCs (n = 0 to 4) |
|  | x | x | x | x | x |  |  |  | Maximum number of upstream ATM TPS-TCs (n = 0 to 4) |

Table 11.30.10.1/G.994.1 – Standard information field – G.992.3 Annex A
Maximum number of upstream TPS-TC functions of each type NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A Maximum number of upstream TPS-TC functions of each type NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  | x | x | x | Maximum number of upstream PTM TPS-TCs (n = 0 to 4) |
|  | x | x | x | x | x |  |  |  | Reserved for allocation by the ITU-T |

Table 11.30.13/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #0 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #0 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 12 to 7) |

Table 11.30.13.1/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #0 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #0 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 6 to 1) |

Table 11.30.13.2/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #0 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #0 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.13.3/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #0 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #0 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.13.4/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #0 NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #0 NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 12 to 7) |

Table 11.30.13.5/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #0 NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #0 NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 6 to 1) |

Table 11.30.13.6/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #0 NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #0 NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Delay_max (Maximum delay) (n milliseconds, n = 0 to 63) |

Table 11.30.13.7/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #0 NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #0 NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | Error_max (Maximum bit error ratio) |
|  | x | x |  |  | x | x |  |  | INP_min (Minimum Impulse Noise Protection) |
|  | x | x |  | x |  |  |  |  | Reserved for allocation by the ITU-T |
|  | x | x | x |  |  |  |  |  | Reserved for allocation by the ITU-T |

Table 11.30.14/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #0 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #0 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 12 to 7) |

Table 11.30.14.1/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #0 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #0 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 6 to 1) |

Table 11.30.14.2/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #0 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #0 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.14.3/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #0 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #0 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.14.4/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #0 NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #0 NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 12 to 7) |

Table 11.30.14.5/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #0 NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #0 NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 6 to 1) |

Table 11.30.14.6/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #0 NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #0 NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Delay_max (Maximum delay) (n milliseconds, n = 0 to 63) |

Table 11.30.14.7/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #0 NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #0 NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | Error_max (Maximum bit error ratio) |
|  | x | x |  |  | x | x |  |  | INP_min (Minimum Impulse Noise Protection) |
|  | x | x |  | x |  |  |  |  | Reserved for allocation by the ITU-T |
|  | x | x | x |  |  |  |  |  | Reserved for allocation by the ITU-T |

Table 11.30.15/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #0 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #0 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 12 to 7) |

Table 11.30.15.1/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #0 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #0 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 6 to 1) |

Table 11.30.15.2/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #0 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #0 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.15.3/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #0 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #0 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.15.4/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #0 NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #0 NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 12 to 7) |

Table 11.30.15.5/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #0 NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #0 NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 6 to 1) |

Table 11.30.15.6/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #0 NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #0 NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Delay_max (Maximum delay) (n milliseconds, n = 0 to 63) |

Table 11.30.15.7/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #0 NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #0 NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | Error_max (Maximum bit error ratio) |
|  | x | x |  |  | x | x |  |  | INP_min (Minimum Impulse Noise Protection) |
|  | x | x |  | x |  |  |  |  | Reserved for allocation by the ITU-T |
|  | x | x | x |  |  |  |  |  | IMA_flag |

Table 11.30.16/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #0 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #0 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 12 to 7) |

Table 11.30.16.1/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #0 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #0 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 6 to 1) |

Table 11.30.16.2/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #0 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #0 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.16.3/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #0 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #0 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.16.4/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #0 NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #0 NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 12 to 7) |

Table 11.30.16.5/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #0 NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #0 NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 6 to 1) |

Table 11.30.16.6/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #0 NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #0 NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Delay_max (Maximum delay) (n milliseconds, n = 0 to 63) |

Table 11.30.16.7/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #0 NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #0 NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | Error_max (Maximum bit error ratio) |
|  | x | x |  |  | x | x |  |  | INP_min (Minimum Impulse Noise Protection) |
|  | x | x |  | x |  |  |  |  | Reserved for allocation by the ITU-T |
|  | x | x | x |  |  |  |  |  | IMA_flag |

Table 11.30.17/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #0 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #0 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 12 to 7) |

Table 11.30.17.1/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #0 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #0 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 6 to 1) |

Table 11.30.17.2/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #0 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #0 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.17.3/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #0 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #0 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.17.4/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #0 NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #0 NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 12 to 7) |

Table 11.30.17.5/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #0 NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #0 NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 6 to 1) |

Table 11.30.17.6/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #0 NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #0 NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Delay_max (Maximum delay) (n milliseconds, n = 0 to 63) |

Table 11.30.17.7/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #0 NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #0 NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | Error_max (Maximum bit error ratio) |
|  | x | x |  |  | x | x |  |  | INP_min (Minimum Impulse Noise Protection) |
|  | x | x |  | x |  |  |  |  | Reserved for allocation by the ITU-T |
|  | x | x | x |  |  |  |  |  | Reserved for allocation by the ITU-T |

Table 11.30.18/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #0 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #0 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 12 to 7) |

Table 11.30.18.1/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #0 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #0 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 6 to 1) |

Table 11.30.18.2/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #0 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #0 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.18.3/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #0 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #0 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.18.4/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #0 NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #0 NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 12 to 7) |

Table 11.30.18.5/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #0 NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #0 NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 6 to 1) |

Table 11.30.18.6/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #0 NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #0 NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Delay_max (Maximum delay) (n milliseconds, n = 0 to 63) |

Table 11.30.18.7/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #0 NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #0 NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | Error_max (Maximum bit error ratio) |
|  | x | x |  |  | x | x |  |  | INP_min (Minimum Impulse Noise Protection) |
|  | x | x |  | x |  |  |  |  | Reserved for allocation by the ITU-T |
|  | x | x | x |  |  |  |  |  | Reserved for allocation by the ITU-T |

Table 11.30.19/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PMS-TC latency path #0 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PMS-TC latency path #0 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.19.1/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PMS-TC latency path #0 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PMS-TC latency path #0 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.20/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PMS-TC latency path #0 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PMS-TC latency path #0 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.20.1/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PMS-TC latency path #0 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PMS-TC latency path #0 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.21/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #1 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #1 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 12 to 7) |

Table 11.30.21.1/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #1 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #1 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 6 to 1) |

Table 11.30.21.2/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #1 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #1 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.21.3/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #1 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #1 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.21.4/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #1 NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #1 NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 12 to 7) |

Table 11.30.21.5/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #1 NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #1 NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 6 to 1) |

Table 11.30.21.6/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #1 NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #1 NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Delay_max (Maximum delay) (n milliseconds, n = 0 to 63) |

Table 11.30.21.7/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #1 NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #1 NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | Error_max (Maximum bit error ratio) |
|  | x | x |  |  | x | x |  |  | INP_min (Minimum Impulse Noise Protection) |
|  | x | x |  | x |  |  |  |  | Reserved for allocation by the ITU-T |
|  | x | x | x |  |  |  |  |  | Reserved for allocation by the ITU-T |

Table 11.30.22/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #1 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #1 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 12 to 7) |

Table 11.30.22.1/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #1 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #1 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 6 to 1) |

Table 11.30.22.2/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #1 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #1 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.22.3/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #1 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #1 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.22.4/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #1 NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #1 NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 12 to 7) |

Table 11.30.22.5/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #1 NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #1 NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 6 to 1) |

Table 11.30.22.6/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #1 NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #1 NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Delay_max (Maximum delay) (n milliseconds, n = 0 to 63) |

Table 11.30.22.7/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #1 NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #1 NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | Error_max (Maximum bit error ratio) |
|  | x | x |  |  | x | x |  |  | INP_min (Minimum Impulse Noise Protection) |
|  | x | x |  | x |  |  |  |  | Reserved for allocation by the ITU-T |
|  | x | x | x |  |  |  |  |  | Reserved for allocation by the ITU-T |

Table 11.30.23/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #1 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #1 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 12 to 7) |

Table 11.30.23.1/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #1 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #1 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 6 to 1) |

Table 11.30.23.2/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #1 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #1 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.23.3/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #1 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #1 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.23.4/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #1 NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #1 NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 12 to 7) |

Table 11.30.23.5/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #1 NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #1 NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 6 to 1) |

Table 11.30.23.6/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #1 NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #1 NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Delay_max (Maximum delay) (n milliseconds, n = 0 to 63) |

Table 11.30.23.7/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #1 NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #1 NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | Error_max (Maximum bit error ratio) |
|  | x | x |  |  | x | x |  |  | INP_min (Minimum Impulse Noise Protection) |
|  | x | x |  | x |  |  |  |  | Reserved for allocation by the ITU-T |
|  | x | x | x |  |  |  |  |  | IMA_flag |

Table 11.30.24/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #1 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #1 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 12 to 7) |

Table 11.30.24.1/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #1 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #1 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 6 to 1) |

Table 11.30.24.2/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #1 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #1 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.24.3/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #1 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #1 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.24.4/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #1 NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #1 NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 12 to 7) |

Table 11.30.24.5/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #1 NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #1 NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 6 to 1) |

Table 11.30.24.6/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #1 NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #1 NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Delay_max (Maximum delay) (n milliseconds, n = 0 to 63) |

Table 11.30.24.7/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #1 NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #1 NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | Error_max (Maximum bit error ratio) |
|  | x | x |  |  | x | x |  |  | INP_min (Minimum Impulse Noise Protection) |
|  | x | x |  | x |  |  |  |  | Reserved for allocation by the ITU-T |
|  | x | x | x |  |  |  |  |  | IMA_flag |

Table 11.30.25/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #1 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #1 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 12 to 7) |

Table 11.30.25.1/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #1 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #1 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 6 to 1) |

Table 11.30.25.2/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #1 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #1 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.25.3/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #1 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #1 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.25.4/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #1 NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #1 NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 12 to 7) |

Table 11.30.25.5/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #1 NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #1 NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 6 to 1) |

Table 11.30.25.6/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #1 NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #1 NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Delay_max (Maximum delay) (n milliseconds, n = 0 to 63) |

Table 11.30.25.7/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #1 NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #1 NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | Error_max (Maximum bit error ratio) |
|  | x | x |  |  | x | x |  |  | INP_min (Minimum Impulse Noise Protection) |
|  | x | x |  | x |  |  |  |  | Reserved for allocation by the ITU-T |
|  | x | x | x |  |  |  |  |  | Reserved for allocation by the ITU-T |

Table 11.30.26/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #1 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #1 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 12 to 7) |

Table 11.30.26.1/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #1 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #1 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 6 to 1) |

Table 11.30.26.2/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #1 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #1 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.26.3/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #1 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #1 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.26.4/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #1 NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #1 NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 12 to 7) |

Table 11.30.26.5/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #1 NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #1 NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 6 to 1) |

Table 11.30.26.6/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #1 NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #1 NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Delay_max (Maximum delay) (n milliseconds, n = 0 to 63) |

Table 11.30.26.7/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #1 NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #1 NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | Error_max (Maximum bit error ratio) |
|  | x | x |  |  | x | x |  |  | INP_min (Minimum Impulse Noise Protection) |
|  | x | x |  | x |  |  |  |  | Reserved for allocation by the ITU-T |
|  | x | x | x |  |  |  |  |  | Reserved for allocation by the ITU-T |

Table 11.30.27/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PMS-TC latency path #1 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PMS-TC latency path #1 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.27.1/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PMS-TC latency path #1 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PMS-TC latency path #1 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.27.2/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PMS-TC latency path #1 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PMS-TC latency path #1 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  | x | x | x | x | R1 max value (2 × n, n = 0 to 15) |
|  | x | x | x | x |  |  |  |  | Reserved for allocation by the ITU-T |

Table 11.30.27.3/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PMS-TC latency path #1 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PMS-TC latency path #1 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  | x | x | x | D1 max value (2n, n = 0 to 7) |
|  | x | x | x | x | x |  |  |  | Reserved for allocation by the ITU-T |

Table 11.30.28/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PMS-TC latency path #1 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PMS-TC latency path #1 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.28.1/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PMS-TC latency path #1 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PMS-TC latency path #1 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.28.2/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PMS-TC latency path #1 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PMS-TC latency path #1 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  | x | x | x | x | R1 max value (2 × n, n = 0 to 15) |
|  | x | x | x | x |  |  |  |  | Reserved for allocation by the ITU-T |

Table 11.30.28.3/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PMS-TC latency path #1 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PMS-TC latency path #1 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  | x | x | x | D1 max value (2n, n = 0 to 7) |
|  | x | x | x | x | x |  |  |  | Reserved for allocation by the ITU-T |

Table 11.30.29/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #2 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #2 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 12 to 7) |

Table 11.30.29.1/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #2 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #2 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 6 to 1) |

Table 11.30.29.2/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #2 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #2 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.29.3/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #2 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #2 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.29.4/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #2 NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #2 NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 12 to 7) |

Table 11.30.29.5/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #2 NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #2 NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 6 to 1) |

Table 11.30.29.6/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #2 NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #2 NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Delay_max (Maximum delay) (n milliseconds, n = 0 to 63) |

Table 11.30.29.7/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #2 NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #2 NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | Error_max (Maximum bit error ratio) |
|  | x | x |  |  | x | x |  |  | INP_min (Minimum Impulse Noise Protection) |
|  | x | x |  | x |  |  |  |  | Reserved for allocation by the ITU-T |
|  | x | x | x |  |  |  |  |  | Reserved for allocation by the ITU-T |

Table 11.30.30/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #2 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #2 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 12 to 7) |

Table 11.30.30.1/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #2 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #2 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 6 to 1) |

Table 11.30.30.2/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #2 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #2 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.30.3/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #2 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #2 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.30.4/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #2 NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #2 NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 12 to 7) |

Table 11.30.30.5/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #2 NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #2 NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 6 to 1) |

Table 11.30.30.6/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #2 NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #2 NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Delay_max (Maximum delay) (n milliseconds, n = 0 to 63) |

Table 11.30.30.7/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #2 NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #2 NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | Error_max (Maximum bit error ratio) |
|  | x | x |  |  | x | x |  |  | INP_min (Minimum Impulse Noise Protection) |
|  | x | x |  | x |  |  |  |  | Reserved for allocation by the ITU-T |
|  | x | x | x |  |  |  |  |  | Reserved for allocation by the ITU-T |

Table 11.30.31/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #2 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #2 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 12 to 7) |

Table 11.30.31.1/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #2 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #2 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 6 to 1) |

Table 11.30.31.2/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #2 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #2 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.31.3/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #2 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #2 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.31.4/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #2 NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #2 NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 12 to 7) |

Table 11.30.31.5/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #2 NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #2 NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 6 to 1) |

Table 11.30.31.6/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #2 NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #2 NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Delay_max (Maximum delay) (n milliseconds, n = 0 to 63) |

Table 11.30.31.7/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #2 NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #2 NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | Error_max (Maximum bit error ratio) |
|  | x | x |  |  | x | x |  |  | INP_min (Minimum Impulse Noise Protection) |
|  | x | x |  | x |  |  |  |  | Reserved for allocation by the ITU-T |
|  | x | x | x |  |  |  |  |  | IMA_flag |

Table 11.30.32/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #2 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #2 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 12 to 7) |

Table 11.30.32.1/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #2 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #2 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 6 to 1) |

Table 11.30.32.2/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #2 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #2 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.32.3/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #2 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #2 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.32.4/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #2 NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #2 NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 12 to 7) |

Table 11.30.32.5/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #2 NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #2 NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 6 to 1) |

Table 11.30.32.6/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #2 NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #2 NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Delay_max (Maximum delay) (n milliseconds, n = 0 to 63) |

Table 11.30.32.7/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #2 NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #2 NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | Error_max (Maximum bit error ratio) |
|  | x | x |  |  | x | x |  |  | INP_min (Minimum Impulse Noise Protection) |
|  | x | x |  | x |  |  |  |  | Reserved for allocation by the ITU-T |
|  | x | x | x |  |  |  |  |  | IMA_flag |

Table 11.30.33/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #2 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #2 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 12 to 7) |

Table 11.30.33.1/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #2 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #2 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 6 to 1) |

Table 11.30.33.2/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #2 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #2 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.33.3/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #2 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #2 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.33.4/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #2 NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #2 NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 12 to 7) |

Table 11.30.33.5/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #2 NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #2 NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 6 to 1) |

Table 11.30.33.6/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #2 NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #2 NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Delay_max (Maximum delay) (n milliseconds, n = 0 to 63) |

Table 11.30.33.7/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #2 NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #2 NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | Error_max (Maximum bit error ratio) |
|  | x | x |  |  | x | x |  |  | INP_min (Minimum Impulse Noise Protection) |
|  | x | x |  | x |  |  |  |  | Reserved for allocation by the ITU-T |
|  | x | x | x |  |  |  |  |  | Reserved for allocation by the ITU-T |

Table 11.30.34/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #2 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #2 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 12 to 7) |

Table 11.30.34.1/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #2 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #2 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 6 to 1) |

Table 11.30.34.2/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #2 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #2 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.34.3/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #2 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #2 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.34.4/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #2 NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #2 NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 12 to 7) |

Table 11.30.34.5/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #2 NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #2 NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 6 to 1) |

Table 11.30.34.6/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #2 NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #2 NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Delay_max (Maximum delay) (n milliseconds, n = 0 to 63) |

Table 11.30.34.7/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #2 NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #2 NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | Error_max (Maximum bit error ratio) |
|  | x | x |  |  | x | x |  |  | INP_min (Minimum Impulse Noise Protection) |
|  | x | x |  | x |  |  |  |  | Reserved for allocation by the ITU-T |
|  | x | x | x |  |  |  |  |  | Reserved for allocation by the ITU-T |

Table 11.30.35/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PMS-TC latency path #2 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PMS-TC latency path #2 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.35.1/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PMS-TC latency path #2 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PMS-TC latency path #2 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.35.2/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PMS-TC latency path #2 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PMS-TC latency path #2 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  | x | x | x | x | R2 max value (2 × n, n = 0 to 15) |
|  | x | x | x | x |  |  |  |  | Reserved for allocation by the ITU-T |

Table 11.30.35.3/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PMS-TC latency path #2 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PMS-TC latency path #2 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  | x | x | x | D2 max value (2n, n = 0 to 7) |
|  | x | x | x | x | x |  |  |  | Reserved for allocation by the ITU-T |

Table 11.30.36/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PMS-TC latency path #2 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PMS-TC latency path #2 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.36.1/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PMS-TC latency path #2 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PMS-TC latency path #2 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.36.2/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PMS-TC latency path #2 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PMS-TC latency path #2 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  | x | x | x | x | R2 max value (2 × n, n = 0 to 15) |
|  | x | x | x | x |  |  |  |  | Reserved for allocation by the ITU-T |

Table 11.30.36.3/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PMS-TC latency path #2 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PMS-TC latency path #2 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  | x | x | x | D2 max value (2n, n = 0 to 7) |
|  | x | x | x | x | x |  |  |  | Reserved for allocation by the ITU-T |

Table 11.30.37/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #3 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #3 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 12 to 7) |

Table 11.30.37.1/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #3 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #3 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 6 to 1) |

Table 11.30.37.2/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #3 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #3 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.37.3/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #3 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #3 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.37.4/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #3 NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #3 NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 12 to 7) |

Table 11.30.37.5/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #3 NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #3 NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 6 to 1) |

Table 11.30.37.6/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #3 NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #3 NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Delay_max (Maximum delay) (n milliseconds, n = 0 to 63) |

Table 11.30.37.7/G.994.1 – Standard information field – G.992.3 Annex A
Downstream STM TPS-TC #3 NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream STM TPS-TC #3 NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | Error_max (Maximum bit error ratio) |
|  | x | x |  |  | x | x |  |  | INP_min (Minimum Impulse Noise Protection) |
|  | x | x |  | x |  |  |  |  | Reserved for allocation by the ITU-T |
|  | x | x | x |  |  |  |  |  | Reserved for allocation by the ITU-T |

Table 11.30.38/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #3 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #3 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 12 to 7) |

Table 11.30.38.1/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #3 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #3 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 6 to 1) |

Table 11.30.38.2/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #3 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #3 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.38.3/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #3 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #3 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.38.4/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #3 NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #3 NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 12 to 7) |

Table 11.30.38.5/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #3 NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #3 NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 6 to 1) |

Table 11.30.38.6/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #3 NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #3 NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Delay_max (Maximum delay) (n milliseconds, n = 0 to 63) |

Table 11.30.38.7/G.994.1 – Standard information field – G.992.3 Annex A
Upstream STM TPS-TC #3 NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream STM TPS-TC #3 NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | Error_max (Maximum bit error ratio) |
|  | x | x |  |  | x | x |  |  | INP_min (Minimum Impulse Noise Protection) |
|  | x | x |  | x |  |  |  |  | Reserved for allocation by the ITU-T |
|  | x | x | x |  |  |  |  |  | Reserved for allocation by the ITU-T |

Table 11.30.39/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #3 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #3 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 12 to 7) |

Table 11.30.39.1/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #3 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #3 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 6 to 1) |

Table 11.30.39.2/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #3 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #3 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.39.3/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #3 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #3 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.39.4/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #3 NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #3 NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 12 to 7) |

Table 11.30.39.5/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #3 NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #3 NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 6 to 1) |

Table 11.30.39.6/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #3 NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #3 NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Delay_max (Maximum delay) (n milliseconds, n = 0 to 63) |

Table 11.30.39.7/G.994.1 – Standard information field – G.992.3 Annex A
Downstream ATM TPS-TC #3 NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream ATM TPS-TC #3 NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | Error_max (Maximum bit error ratio) |
|  | x | x |  |  | x | x |  |  | INP_min (Minimum Impulse Noise Protection) |
|  | x | x |  | x |  |  |  |  | Reserved for allocation by the ITU-T |
|  | x | x | x |  |  |  |  |  | IMA_flag |

Table 11.30.40/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #3 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #3 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 12 to 7) |

Table 11.30.40.1/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #3 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #3 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 6 to 1) |

Table 11.30.40.2/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #3 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #3 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.40.3/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #3 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #3 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.40.4/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #3 NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #3 NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 12 to 7) |

Table 11.30.40.5/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #3 NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #3 NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 6 to 1) |

Table 11.30.40.6/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #3 NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #3 NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Delay_max (Maximum delay) (n milliseconds, n = 0 to 63) |

Table 11.30.40.7/G.994.1 – Standard information field – G.992.3 Annex A
Upstream ATM TPS-TC #3 NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream ATM TPS-TC #3 NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | Error_max (Maximum bit error ratio) |
|  | x | x |  |  | x | x |  |  | INP_min (Minimum Impulse Noise Protection) |
|  | x | x |  | x |  |  |  |  | Reserved for allocation by the ITU-T |
|  | x | x | x |  |  |  |  |  | IMA_flag |

Table 11.30.41/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #3 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #3 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 12 to 7) |

Table 11.30.41.1/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #3 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #3 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 6 to 1) |

Table 11.30.41.2/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #3 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #3 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.41.3/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #3 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #3 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.41.4/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #3 NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #3 NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 12 to 7) |

Table 11.30.41.5/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #3 NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #3 NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 6 to 1) |

Table 11.30.41.6/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #3 NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #3 NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Delay_max (Maximum delay) (n milliseconds, n = 0 to 63) |

Table 11.30.41.7/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PTM TPS-TC #3 NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PTM TPS-TC #3 NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | Error_max (Maximum bit error ratio) |
|  | x | x |  |  | x | x |  |  | INP_min (Minimum Impulse Noise Protection) |
|  | x | x |  | x |  |  |  |  | Reserved for allocation by the ITU-T |
|  | x | x | x |  |  |  |  |  | Reserved for allocation by the ITU-T |

Table 11.30.42/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #3 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #3 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 12 to 7) |

Table 11.30.42.1/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #3 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #3 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_min (minimum net data rate, bits 6 to 1) |

Table 11.30.42.2/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #3 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #3 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.42.3/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #3 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #3 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.42.4/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #3 NPar(3) coding – Octet 5

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #3 NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 12 to 7) |

Table 11.30.42.5/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #3 NPar(3) coding – Octet 6

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #3 NPar(3)s – Octet 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_reserve (Minimum reserved net data rate, bits 6 to 1) |

Table 11.30.42.6/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #3 NPar(3) coding – Octet 7

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #3 NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Delay_max (Maximum delay) (n milliseconds, n = 0 to 63) |

Table 11.30.42.7/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PTM TPS-TC #3 NPar(3) coding – Octet 8

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PTM TPS-TC #3 NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | Error_max (Maximum bit error ratio) |
|  | x | x |  |  | x | x |  |  | INP_min (Minimum Impulse Noise Protection) |
|  | x | x |  | x |  |  |  |  | Reserved for allocation by the ITU-T |
|  | x | x | x |  |  |  |  |  | Reserved for allocation by the ITU-T |

Table 11.30.43/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PMS-TC latency path #3 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PMS-TC latency path #3 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.43.1/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PMS-TC latency path #3 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PMS-TC latency path #3 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.43.2/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PMS-TC latency path #3 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PMS-TC latency path #3 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  | x | x | x | x | R3 max value (2 × n, n = 0 to 15) |
|  | x | x | x | x |  |  |  |  | Reserved for allocation by the ITU-T |

Table 11.30.43.3/G.994.1 – Standard information field – G.992.3 Annex A
Downstream PMS-TC latency path #3 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A downstream PMS-TC latency path #3 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  | x | x | x | D3 max value (2n, n = 0 to 7) |
|  | x | x | x | x | x |  |  |  | Reserved for allocation by the ITU-T |

Table 11.30.44/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PMS-TC latency path #3 NPar(3) coding – Octet 1

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PMS-TC latency path #3 NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 12 to 7) |

Table 11.30.44.1/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PMS-TC latency path #3 NPar(3) coding – Octet 2

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PMS-TC latency path #3 NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Net_max (Maximum net data rate, bits 6 to 1) |

Table 11.30.44.2/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PMS-TC latency path #3 NPar(3) coding – Octet 3

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PMS-TC latency path #3 NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  | x | x | x | x | R3 max value (2 × n, n = 0 to 15) |
|  | x | x | x | x |  |  |  |  | Reserved for allocation by the ITU-T |

Table 11.30.44.3/G.994.1 – Standard information field – G.992.3 Annex A
Upstream PMS-TC latency path #3 NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex A upstream PMS-TC latency path #3 NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  | x | x | x | D3 max value (2n, n = 0 to 7) |
|  | x | x | x | x | x |  |  |  | Reserved for allocation by the ITU-T |

Table 11.31/G.994.1 – Standard information field – G.992.3 Annex B NPar(2) coding

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex B NPar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | NTR |
|  | x | x | x | x | x | x | 1 | x | Short initialization |
|  | x | x | x | x | x | 1 | x | x | Diagnostics mode |
|  | x | x | x | x | 1 | x | x | x | Tones 1 to 32 |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

The SPar(2) and NPar(3) parameters for G.992.3 Annex B are identical to those of G.992.3 Annex A. Therefore, Tables 11.30 through 11.30.44.3/G.994.1 shall be used for G.992.3 Annex B. Transmission of SPar(2) and NPar(3) octets for G.992.3 Annex B shall follow transmission of Table 11.31/G.994.1, and precede transmission of Table 11.33/G.994.1. Effectively, Tables 11.30 through 11.30.44.3/G.994.1 become renumbered to Tables 11.32 through 11.32.44.3 for G.992.3 Annex B.

Table 11.33/G.994.1 – Standard information field – G.992.3 Annex I NPar(2) coding

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex I NPar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | NTR |
|  | x | x | x | x | x | x | 1 | x | Short initialization |
|  | x | x | x | x | x | 1 | x | x | Diagnostics mode |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

The SPar(2) and NPar(3) parameters for G.992.3 Annex I are identical to those of G.992.3 Annex A. Therefore, Tables 11.30 through 11.30.44.3/G.994.1 shall be used for G.992.3 Annex I. Transmission of SPar(2) and NPar(3) octets for G.992.3 Annex I shall follow transmission of Table 11.33/G.994.1, and precede transmission of Table 11.35/G.994.1. Effectively, Tables 11.30 through 11.30.44.3/G.994.1 become renumbered to Tables 11.34 through 11.34.44.3 for G.992.3 Annex I.

Table 11.35/G.994.1 – Standard information field – G.992.3 Annex J NPar(2) coding

|  | Bits |  |  |  |  |  |  |  | G.992.3 Annex J NPar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | NTR |
|  | x | x | x | x | x | x | 1 | x | Short initialization |
|  | x | x | x | x | x | 1 | x | x | Diagnostics mode |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

The SPar(2) and NPar(3) parameters for G.992.3 Annex J are identical to those of G.992.3 Annex A. Therefore, Tables 11.30 through 11.30.44.3/G.994.1 shall be used for G.992.3 Annex J. Transmission of SPar(2) and NPar(3) octets for G.992.3 Annex J shall follow transmission of Table 11.35/G.994.1, and precede transmission of Table 11.37/G.994.1. Effectively, Tables 11.30 through 11.30.44.3/G.994.1 become renumbered to Tables 11.36 through 11.36.44.3 for G.992.3 Annex J.

Table 11.37/G.994.1 – Standard information field – G.992.4 Annex A NPar(2) coding

|  | Bits |  |  |  |  |  |  |  | G.992.4 Annex A NPar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | NTR |
|  | x | x | x | x | x | x | 1 | x | Short initialization |
|  | x | x | x | x | x | 1 | x | x | Diagnostics mode |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

The SPar(2) and NPar(3) parameters for G.992.4 Annex A are identical to those of G.992.3 Annex A. Therefore, Tables 11.30 through 11.30.44.3/G.994.1 shall be used for G.992.4 Annex A. Transmission of SPar(2) and NPar(3) octets for G.992.4 Annex A shall follow transmission of Table 11.37/G.994.1, and precede transmission of Table 11.39/G.994.1. Effectively, Tables 11.30 through 11.30.44.3/G.994.1 become renumbered to Tables 11.38 through 11.38.44.3 for G.992.4 Annex A.

Table 11.39/G.994.1 – Standard information field – G.992.4 Annex I NPar(2) coding

|  | Bits |  |  |  |  |  |  |  | G.992.4 Annex I NPar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | NTR |
|  | x | x | x | x | x | x | 1 | x | Short initialization |
|  | x | x | x | x | x | 1 | x | x | Diagnostics mode |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

The SPar(2) and NPar(3) parameters for G.992.4 Annex I are identical to those of G.992.3 Annex A. Therefore, Tables 11.30 through 11.30.44.3/G.994.1 shall be used for G.992.4 Annex I. Transmission of SPar(2) and NPar(3) octets for G.992.4 Annex I shall follow transmission of Table 11.39/G.994.1. Effectively, Tables 11.30 through 11.30.44.3/G.994.1 become renumbered to Tables 11.40 through 11.40.44.3 for G.992.4 Annex I.

Table 11.43/G.994.1 – Standard information field – G.992.5 Annex A NPar(2) coding

|  | Bits |  |  |  |  |  |  |  | G.992.5 Annex A NPar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | NTR |
|  | x | x | x | x | x | x | 1 | x | Short initialization |
|  | x | x | x | x | x | 1 | x | x | Diagnostics mode |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

The SPar(2) and NPar(3) parameters for G.992.5 Annex A are identical to those of G.992.3 Annex A. Therefore, Tables 11.30 through 11.30.44.3/G.994.1 shall be used for G.992.5 Annex A. Transmission of SPar(2) and NPar(3) octets for G.992.5 Annex A shall follow transmission of Table 11.43/G.994.1, and precede transmission of Table 11.45/G.994.1. Effectively, Tables 11.30 through 11.30.44.3/G.994.1 become renumbered to Tables 11.44 through 11.44.44.3 for G.992.5 Annex A.

Table 11.45/G.994.1 – Standard information field – G.992.5 Annex B NPar(2) coding

|  | Bits |  |  |  |  |  |  |  | G.992.5 Annex B NPar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | NTR |
|  | x | x | x | x | x | x | 1 | x | Short initialization |
|  | x | x | x | x | x | 1 | x | x | Diagnostics mode |
|  | x | x | x | x | 1 | x | x | x | Tones 1 to 32 |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

The SPar(2) and NPar(3) parameters for G.992.5 Annex B are identical to those of G.992.3 Annex A. Therefore, Tables 11.30 through 11.30.44.3/G.994.1 shall be used for G.992.5 Annex B. Transmission of SPar(2) and NPar(3) octets for G.992.5 Annex B shall follow transmission of Table 11.45/G.994.1, and precede transmission of Table 11.46/G.994.1. Effectively, Tables 11.30 through 11.30.44.3/G.994.1 become renumbered to Tables 11.46 through 11.46.44.3 for G.992.5 Annex B.

Table 11.47/G.994.1 – Standard information field – G.992.5 Annex I NPar(2) coding

|  | Bits |  |  |  |  |  |  |  | G.992.5 Annex I NPar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | NTR |
|  | x | x | x | x | x | x | 1 | x | Short initialization |
|  | x | x | x | x | x | 1 | x | x | Diagnostics mode |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

The SPar(2) and NPar(3) parameters for G.992.5 Annex I are identical to those of G.992.5 Annex A. Therefore, Tables 11.33 through 11.33.44.3/G.994.1 shall be used for G.992.5 Annex I. Transmission of SPar(2) and NPar(3) octets for G.992.5 Annex I shall follow transmission of Table 11.47/G.994.1. Effectively, Tables 11.33 through 11.33.44.3/G.994.1 become renumbered to Tables 11.48 through 11.48.44.3 for G.992.5 Annex I.

## 9.5	Non-standard information field (NS)

MP, MS, CL and CLR messages may optionally contain a non-standard information field to convey information beyond that defined in this Recommendation. For MR, ACK, NAK and REQ messages, the non-standard information field is not used and is therefore of zero length. When non-standard information is to be sent, the "Non-standard field" parameter shall be set to binary ONE in the identification field of the transmitted message (see Table 8).

The non-standard information field may be composed of one or more non-standard information blocks. The format of the non-standard information field is shown in Figure 10. The first octet of the non-standard information field shall indicate the number of non-standard information blocks to follow.

Number of non-standard information blocks = N (1 octet)

Non-standard information block 1

Non-standard information block 2

. . .

Non-standard information block N

Figure 10/G.994.1 – Non-standard information field (NS) format

Each non-standard information block (see Figure 11) is composed of:

•	a length indicator (one octet) specifying the length of the remainder of the block;

•	a country code (2 octets), as defined in ITU-T Rec. T.35;

•	a 4-octet provider code as specified by the country identified in ITU-T Rec. T.35; and

•	non-standard information (M octets).

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Non-standard information length  M  6 (1 octet) |  |  |  |  |  |  |  |
| T.35 country code (2 octets – Note 1) |  |  |  |  |  |  |  |
| Provider code (vendor identification) (4 octets – Note 2) |  |  |  |  |  |  |  |
| Vendor-specific information (M octets) |  |  |  |  |  |  |  |

NOTE 1 – If the bits in the first octet are not all set to binary ONE, the bits in the second octet
shall be set to binary ZERO by the transmitter and ignored by the receiver.

NOTE 2 – Specification of the coding and order of transmission of this field is the responsibility
of the regional standards body allocating the provider code. See Appendix II for provider code
contact information.

Figure 11/G.994.1 – Non-standard information block format

## 9.6	Overall message composition

Table 12 shows which fields are permitted in each message type. An "X" indicates that the field shall be included whilst a "–" indicates that the field shall not be included.

Once a Transaction C (see 10.1) has been completed, any subsequent MS message within the same G.994.1 session shall only contain those octets of the identification (I) and standard information (S) fields, and those blocks of non-standard information (NS) that were contained in both the CLR and CL messages of the previous Transaction C.

Table 12/G.994.1 – Overall message composition

| Messages | Identification |  |  |  | Standard information | Non-standard information |
| --- | --- | --- | --- | --- | --- | --- |
|  | Message type and version (2 octets) | Additional information |  | Service and channel parameters (Note 1) | Modulations and Protocols available (Note 2) | (Note 3) 1+i=1N7+Mi octets |
|  |  | Vendor ID (8 octets) | Retransmission (Note 4) |  |  |  |
| MR | X | – | – | – | – | – |
| CLR | X | X | – | X | X | As necessary |
| CL | X | X | – | X | X | As necessary |
| MS | X | – | – | X | X | As necessary |
| MP | X | – | – | X | X | As necessary |
| ACK | X | – | – | – | – | – |
| NAK | X | – | – | – | – | – |
| REQ | X | – | X | – | – | – |
| NOTE 1 – As defined in the tables in 9.3.4/G.994.1. NOTE 2 – As defined in the tables in 9.4/G.994.1. NOTE 3 – As defined in Figures 10 and 11 in 9.5/G.994.1. NOTE 4 – Only used for REQ-RTX, as defined in 9.3.3.2/G.994.1. |  |  |  |  |  |  |

# 10	G.994.1 transactions

All permitted G.994.1 transactions are described in this clause.

An overview of the set of basic transactions specified in this Recommendation and the use of ACK(1) is given in 10.1. The use of REQ-MS, REQ-MR and REQ-CLR to create extended transactions is described in 10.2. Message segmentation and the use of ACK(2) is discussed in 10.3. The complete specification of all state transitions permitted during a G.994.1 session is contained in 10.4. The use of REQ-RTX to recover from the reception of an errored frame is described in 10.5.

Error recovery procedures and the use of NAK-EF are discussed in clause 12.

## 10.1	Basic transactions

Basic transactions may be classified as one of two types:

•	those that exchange and negotiate capabilities between the HSTU-C and HSTU-R; and

•	those that select a mode of operation.

Table 13 shows the set of basic transactions specified in this Recommendation. An "X" indicates that the basic transaction is supported for the stated version number (see 9.3.2) whilst a "–" indicates that it is not. Each transaction is initiated by the HSTU-R, and ends with an ACK(1). With basic transactions, the HSTU-R controls the negotiation procedure. At the end of a basic G.994.1 transaction, the stations shall either terminate the G.994.1 session (applies to transactions A, B, C and D) as specified in 11.3, or shall go to the Initial HSTU-x Transaction state (applies to transaction C only) as shown in Figures 12 or 13.

NOTE – The continuation of G.994.1 modulation after the conclusion of the G.994.1 session, for the purpose of implementing other protocols (for example ITU-T Rec. G.997.1 – see Table 10), is for further study.

Table 13/G.994.1 – Basic G.994.1 transactions

| Transaction identifier | HSTU-R | HSTU-C | HSTU-R | G.994.1 Version 1 support | G.994.1 Version 2 and 3 support |
| --- | --- | --- | --- | --- | --- |
| A | MS  | ACK(1) |  | X | X |
| B | MR  | MS  | ACK(1) | X | X |
| C | CLR  | CL  | ACK(1) | X | X |
| D | MP  | MS  | ACK(1) | – | X |

### 10.1.1	Transaction A

In Transaction A, the HSTU-R selects a mode of operation and requests that the HSTU-C transition to the selected mode. When the HSTU-C responds with an ACK(1) message, both stations shall transition to the selected mode.

If the HSTU-R cannot determine a common mode of operation (either standard or non-standard) from previous capabilities exchanges, or is not prepared to select a mode at this time, it shall send an MS message with the Non-standard field bit in Table 8 and all coding points in Tables 10 and 11 set to binary ZERO. When the HSTU-C receives this message, it shall respond with an ACK(1) message. The HSTU-R shall then initiate the cleardown procedure specified in 11.3.

### 10.1.2	Transaction B

In Transaction B, the HSTU-R requests that the HSTU-C select the mode of operation. The HSTU-C selects the mode by transmitting an MS message. When the HSTU-R responds with an ACK(1) message, both stations shall transition to the selected mode.

If the HSTU-C cannot determine a common mode of operation (either standard or non-standard) from previous capabilities exchanges, or is not prepared to select a mode at this time, it shall send an MS message with the Non-standard field bit in Table 8 and all coding points in Tables 10 and 11 set to binary ZERO. When the HSTU-R receives this message, it shall respond with an ACK(1) message. The HSTU-C shall then initiate the cleardown procedure specified in 11.3.

### 10.1.3	Transaction C

In Transaction C, capabilities are exchanged and negotiated by the two stations. Transaction C shall be followed by either Transaction A, Transaction B, or Transaction D during the same session to select a common mode of operation identified during the capabilities exchange.

### 10.1.4	Transaction D

In Transaction D, the HSTU-R proposes a mode of operation and requests that the HSTU-C select the mode of operation. The HSTU-C selects the mode by transmitting an MS message. When the HSTU-R responds with an ACK(1) message, both stations shall transition to the selected mode.

If the HSTU-C cannot determine a common mode of operation (either standard or non-standard) from previous capabilities exchanges, or is not prepared to select a mode at this time, it shall send an MS message with the Non-standard field bit in Table 8 and all coding points in Tables 10 and 11 set to binary ZERO. When the HSTU-R receives the MS message, it shall respond with an ACK(1) message. The HSTU-C shall then initiate the cleardown procedure specified in 11.3.

## 10.2	Extended transactions

Table 14 shows the set of extended transactions specified in this Recommendation. An "X" indicates that the extended transaction is supported for the stated Version number (see 9.3.2) whilst a "–" indicates that it is not. Each transaction is initiated by the HSTU-R and ends with an ACK(1). Extended transactions are derived from a concatenation of two basic transactions. They are used when the HSTU-C wishes to control the negotiation procedure. At the end of an extended G.994.1 transaction, the stations shall either terminate the G.994.1 session as specified in 11.3, or shall go to the Initial HSTU-x Transaction state as shown in Figures 12 or 13.

NOTE – The continuation of G.994.1 modulation after the conclusion of the G.994.1 session, for the purpose of implementing other protocols (for example ITU-T Rec. G.997.1 – see Table 10), is for further study.

Table 14/G.994.1 – Extended G.994.1 transactions

| Transaction identifier | HSTU-R | HSTU-C | HSTU-R | HSTU-C | HSTU-R | G.994.1 Version 1 support | G.994.1 Version 2 and 3 support |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A:B | MS  | REQ-MR  | MR  | MS  | ACK(1) | X | X |
| B:A | MR  | REQ-MS  | MS  | ACK(1) |  | X | X |
| A:C | MS  | REQ-CLR  | CLR  | CL  | ACK(1) | X | X |
| B:C | MR  | REQ-CLR  | CLR  | CL  | ACK(1) | X | X |
| D:C | MP  | REQ-CLR  | CLR  | CL  | ACK(1) | – | X |

### 10.2.1	Transaction A:B

In Transaction A:B, the HSTU-R selects a mode of operation and requests that the HSTU-C transition to the selected mode. However, rather than responding to the MS message with an ACK(1) message as is the case for basic transaction A, the HSTU-C responds to the MS message with a REQ-MR message requesting the HSTU-R to proceed directly into basic transaction B without returning to the Initial Transaction state.

### 10.2.2	Transaction B:A

In Transaction B:A, the HSTU-R requests that the HSTU-C select the mode of operation. However, rather than responding to the MR message with an MS message as is the case for basic transaction B, the HSTU-C responds to the MR message with a REQ-MS message requesting the HSTU-R to proceed directly into basic transaction A without returning to the Initial Transaction state.

### 10.2.3	Transaction A:C

In Transaction A:C, the HSTU-R selects a mode of operation and requests that the HSTU-C transition to the selected mode. However, rather than responding to the MS message with an ACK(1) message as is the case for basic transaction A, the HSTU-C responds to the MS message with a REQ-CLR message requesting the HSTU-R to proceed directly into basic transaction C without returning to the Initial Transaction state.

### 10.2.4	Transaction B:C

In Transaction B:C, the HSTU-R requests that the HSTU-C select the mode of operation. However, rather than responding to the MR message with an MS message as is the case for basic transaction B, the HSTU-C responds to the MR message with a REQ-CLR message requesting the HSTU-R to proceed directly into a transaction C without returning to the Initial Transaction state.

### 10.2.5	Transaction D:C

In Transaction D:C, the HSTU-R proposes a mode of operation and requests that the HSTU-C select the mode of operation. However, rather than responding to the MP message with an MS message as is the case for basic transaction D, the HSTU-C responds to the MP message with a REQ-CLR message requesting the HSTU-R to proceed directly into basic transaction C without returning to the Initial Transaction state.

## 10.3	Message segmentation

Not including the two octets of FCS and any octets that have been inserted to achieve octet transparency (see 8.4), the maximum number of octets in any frame shall be 64. If the message exceeds this limit, the remainder of the message may be contained in subsequent frames. Whether or not the message exceeds 64 octets, it may be split into segments.

The receiving station shall parse the frame to determine if the message has been fully transmitted. If the message has not been fully transmitted, the receiving station may request transmission of the next segment by transmitting an ACK(2). Further segments shall only be sent in response to an ACK(2). Only CLR, CL, MP, and MS messages may be segmented.

Where non-standard information is present in the message, the standard and non-standard information may be conveyed in separate frames.

## 10.4	State transition diagrams

For equipment that conforms to G.994.1 Version 1 (see 9.3.2), Figures 12.1 and 13.1 specify all permitted state transitions for the HSTU-R and HSTU-C stations respectively. For equipment that conforms to G.994.1 Version 2 (see 9.3.2), Figures 12.2 and 13.2 specify all permitted state transitions for the HSTU-R and HSTU-C stations respectively. For equipment that conforms to G.994.1 Version 3 (see 9.3.2), Figures 12.3 and 13.3 specify all permitted state transitions for the HSTU-R and HSTU-C stations respectively.

The state transition diagrams show state information (the state name and current transmitted message) and transition information (received message that caused the state change).

Figure 12.1/G.994.1 – HSTU-R state transition diagram (Version 1)

Figure 12.2/G.994.1 – HSTU-R state transition diagram (Version 2)

Figure 12.3/G.994.1 – HSTU-R state transition diagram (Version 3)

Figure 13.1/G.994.1 – HSTU-C state transition diagram (Version 1)

Figure 13.2/G.994.1 – HSTU-C state transition diagram (Version 2)

Figure 13.3/G.994.1 – HSTU-C state transition diagram (Version 3)

Transmission of signals associated with a selected mode shall follow the transmission of ACK(1) and the G.994.1 cleardown procedure. The time delay between the end of a G.994.1 session and the start of the selected mode is specified in the associated Recommendation.

## 10.5	Retransmission transactions

Retransmission transactions occur whenever an HSTU receives an errored frame and wishes to transmit the REQ-RTX message to initiate a retransmission instead of transmitting the NAK-EF message.

Tables 15 and 16 show the set of retransmission transactions specified in this Recommendation. An "X" indicates that the transaction is supported for the stated Version number (see 9.3.2) whilst a "–" indicates that it is not.

Table 15/G.994.1 – Retransmission G.994.1 transactions

| Transaction identifier | HSTU-C | HSTU-R | G.994.1 Version 1 support | G.994.1 Version 2 support | G.994.1 Version 3 support |
| --- | --- | --- | --- | --- | --- |
| RA | REQ-RTX | (Note) | – | – | X |
| NOTE – Any frame that complies with the transaction rules that could follow the frame indicated in the LCRM and MSFN octets of the REQ-RTX message. |  |  |  |  |  |

Table 16/G.994.1 – Retransmission G.994.1 transactions

| Transaction identifier | HSTU-R | HSTU-C | G.994.1 Version 1 support | G.994.1 Version 2 support | G.994.1 Version 3 support |
| --- | --- | --- | --- | --- | --- |
| RB | REQ-RTX | (Note) | – | – | X |
| NOTE – Any frame that complies with the transaction rules that could follow the frame indicated in the LCRM and MSFN octets of the REQ-RTX message. |  |  |  |  |  |

### 10.5.1	Transaction RA

In Transaction RA, the HSTU-C receives a frame that contains an FCS error. The HSTU-C shall transmit a REQ-RTX message with the LCRM and MSFN octets encoded as described in 9.3.3.2.

Upon correctly receiving the REQ-RTX message, the HSTU-R shall transmit any frame that complies with the transaction rules that could follow the frame indicated in the LCRM and MSFN octets of the REQ-RTX message.

The HSTU-C shall send no more than three consecutive REQ-RTX messages. Instead of a fourth consecutive REQ-RTX message, the HSTU-C shall send a NAK-CD message to cleardown the session.

If the HSTU-R cannot determine the state of the session when it receives a REQ-RTX message, it shall transmit a NAK-CD message to cleardown the session.

### 10.5.2	Transaction RB

In Transaction RB, the HSTU-R receives a frame that contains an FCS error. The HSTU-R shall transmit a REQ-RTX message with the LCRM and MSFN octets encoded as described in 9.3.3.2.

Upon correctly receiving the REQ-RTX message, the HSTU-C shall transmit any frame that complies with the transaction rules that could follow the frame indicated in the LCRM and MSFN octets of the REQ-RTX message.

If the HSTU-C receives a REQ-RTX message with LCRM set to NULL, it shall respond with a NAK-CD message to cleardown the session.

The HSTU-R shall send no more than three consecutive REQ-RTX messages. Instead of a fourth consecutive REQ-RTX message, the HTU-R shall send a NAK-CD message to cleardown the session.

If the HSTU-C cannot determine the state of the session when it receives a REQ-RTX message, it shall transmit a NAK-CD message to cleardown the session.

# 11	Start-up/cleardown procedures

## 11.1	Duplex start-up procedures

### 11.1.1	HSTU-R initiated start-up procedure

Figure 14 displays the timing for the HSTU-R initiated duplex start-up procedure. Initially, the HSTU-R is in state R-SILENT0 transmitting silence, and the HSTU-C is in state C-SILENT1 transmitting silence. The HSTU-R shall initiate the start-up procedure by transmitting signals from one or both of its signalling families, with phase reversals every 16 ms (R-TONES-REQ). When this has been detected by the HSTU-C, the HSTU-C shall respond by transmitting signals from one or both of its signalling families (C-TONES). When this has been detected by the HSTU-R, the HSTU-R shall transmit silence (R-SILENT1) for 50 to 500 ms and shall then transmit signals from only one signalling family (R-TONE1). The minimum detection time for C-TONES shall be 50 ms. When the HSTU-C has detected R-TONE1, it shall respond by transmitting Galfs on modulated carriers (C-GALF1). When the HSTU-R has detected Galfs, it shall respond by transmitting Flags on modulated carriers (R-FLAG1). When the HSTU-C has detected Flags, it shall respond by transmitting Flags (C-FLAG1). When the HSTU-R has detected Flags, it shall begin the first transaction.

Figure 14 shows the timing requirements between events that shall be adhered to. 1 is the time period from detection of a signal (e.g., R-TONE1) to the transmission of the next signal (e.g., C-GALF1).

Figure 14/G.994.1 – HSTU-R initiated duplex start-up procedure

### 11.1.2	HSTU-C initiated start-up procedure

Figure 15 displays the timing for the HSTU-C initiated duplex start-up procedure. Initially, the HSTU-R is in state R-SILENT0 transmitting silence, and the HSTU-C is in state C-SILENT1 transmitting silence. The HSTU-C shall initiate the start-up procedure by transmitting signals from one or both of its signalling families (C-TONES). When this has been detected by the HSTU-R, the HSTU-R shall respond by transmitting signals from only one signalling family (R-TONE1). The minimum detection time for C-TONES shall be 50 ms. When the HSTU-C has detected R-TONE1, it shall respond by transmitting Galfs on modulated carriers (C-GALF1). When the HSTU-R has detected Galfs, it shall respond by transmitting Flags on modulated carriers (R-FLAG1). When the HSTU-C has detected Flags, it shall respond by transmitting Flags (C-FLAG1). When the HSTU-R has detected Flags, it shall begin the first transaction.

Figure 15 shows the timing requirements between events that shall be adhered to. 1 is the time period from detection of a signal (e.g., R-TONE1) to the transmission of the next signal (e.g., C-GALF1).

Figure 15/G.994.1 – HSTU-C initiated duplex start-up procedure

## 11.2	Half-duplex start-up procedures

### 11.2.1	HSTU-R initiated start-up procedure

Figure 16 displays the timing for the HSTU-R initiated half-duplex start-up procedure. Initially, the HSTU-R is in state R-SILENT0 transmitting silence, and the HSTU-C is in state C-SILENT1 transmitting silence. The HSTU-R shall initiate the start-up procedure by transmitting signals from one or both of its signalling families, with phase reversals every 16 ms (R-TONES-REQ). When this has been detected by the HSTU-C, the HSTU-C shall respond by transmitting signals from one or both of its signalling families (C-TONES). When this has been detected by the HSTU-R, the HSTU-R shall transmit silence (R-SILENT1) for 50 to 500 ms and shall then transmit flags modulated on carriers from only one signalling family (R-FLAG1). The minimum detection time for C-TONES shall be 50 ms. When the HSTU-C has detected R-FLAG1, it shall respond by transmitting silence. When the HSTU-R has detected the silence, it shall continue to transmit Flags for period 1 and shall then begin the first transaction.

Figure 16 shows the timing requirements between events that shall be adhered to. 1 is the time period from detection of a signal (e.g., R-TONE1) to the transmission of the next signal (e.g., C-GALF1).

Figure 16/G.994.1 – HSTU-R initiated half-duplex start-up procedure

### 11.2.2	HSTU-C initiated start-up procedure

Figure 17 displays the timing for the HSTU-C initiated half-duplex start-up procedure. Initially, the HSTU-R is in state R-SILENT0 transmitting silence, and the HSTU-C is in state C-SILENT1 transmitting silence. The HSTU-C shall initiate the start-up procedure by transmitting signals from one or both of its signalling families (C-TONES). When this has been detected by the HSTU-R, the HSTU-R shall respond by transmitting flags modulated on carriers from only one signalling family (R-FLAG1). The minimum detection time for C-TONES shall be 50 ms. When the HSTU-C has detected R-FLAG1, it shall respond by transmitting silence. When the HSTU-R has detected the silence, it shall continue to transmit Flags for period 1 and shall then begin the first transaction.

Figure 17 shows the timing requirements between events that shall be adhered to. 1 is the time period from detection of a signal (e.g., R-TONE1) to the transmission of the next signal (e.g., C-GALF1).

Figure 17/G.994.1 – HSTU-C initiated half-duplex start-up procedure

## 11.3	Cleardown procedure

Figure 18 displays the timing for cleardown of a duplex G.994.1 session (by either the HSTU-R or the HSTU-C). When an HSTU-R (HSTU-C) receives an ACK(1) message in response to an MS message, or receives a NAK-CD message, it shall initiate the cleardown procedure. After receiving the ACK(1) or NAK-CD message, the HSTU-R (HSTU-C) shall continue to transmit Flags for a period not to exceed 0.5 s. It shall then transmit 4 octets of Galf (referred to as R-GALF2 for an HSTU-R, C-GALF2 for an HSTU-C), followed by silence which terminates the G.994.1 session. When the HSTU-C (HSTU-R) detects either Galfs or silence, it shall continue to transmit Flags (referred to as C-FLAG2 for an HSTU-C, R-FLAG2 for an HSTU-R) for a period not to exceed 0.5 s, followed by silence which terminates the G.994.1 session.

If a received MS message indicates a common operating mode, both stations shall transition to the selected mode upon termination of the G.994.1 session. If a received MS message indicates that there is no common operating mode (see 10.1), or the cleardown was initiated by a NAK-CD, both stations shall return to the initial G.994.1 state (R-SILENT0 for an HSTU-R, C-SILENT1 for an HSTU-C).

Figure 18/G.994.1 – Duplex cleardown procedure

Figure 19 displays the timing for cleardown of a half-duplex G.994.1 session (by either the HSTU-R or the HSTU-C). When an HSTU-R (HSTU-C) receives an ACK(1) message in response to an MS message or receives NAK-CD messages, it shall initiate the cleardown procedure.

After receiving the ACK(1) or NAK-CD message, the HSTU-R (HSTU-C) shall continue to transmit silence for a period not to exceed 0.5 s. It shall then transmit 4 octets of Galf (referred to as R-GALF2 for an HSTU-R, C-GALF2 for an HSTU-C), followed by silence which terminates the G.994.1 session.

When the HSTU-C (HSTU-R) detects either Galfs or continued silence after a timeout of 0.5 s, it shall continue to transmit silence for a period not to exceed 0.5 s and then terminate the G.994.1 session.

If a received MS message indicates a common operating mode, both stations shall transition to the selected mode upon termination of the G.994.1 session. If a received MS message indicates that there is no common operating mode (see 10.1) or cleardown was initiated by a NAK-CD, both stations shall return to the initial G.994.1 state (R-SILENT0 for an HSTU-R, C-SILENT1 for an HSTU-C).

Figure 19/G.994.1 – Half-duplex cleardown procedure

# 12	Error recovery procedures

The timing requirements used for error recovery during the start-up procedure of a G.994.1 session are specified in Figures 14 through 17.

During a G.994.1 session, the maximum time between the reception of the end of any frame and the start of transmission of the next frame except the REQ-RTX message shall be 0.5 s.

During a G.994.1 session, for transactions where the HSTU-R transmits the final message, the maximum time between the end of transmission of the final frame of the transaction and the start of transmission of the first frame of the next transaction shall be 0.5 s.

During a G.994.1 session, the minimum time a station shall wait after transmitting a frame shall be 1.25 s. If either station times out, it shall immediately return to the initial G.994.1 state (R-SILENT0 for an HSTU-R, C-SILENT1 for an HSTU-C) and remain silent for a minimum period of 0.5 s. It may then initiate another G.994.1 session.

If an errored frame is received in any state (including the Initial HSTU-x Transaction State), the receiving station shall either:

1)	transmit a REQ-RTX message according to the transactions of 10.5; or

2)	abort the G.994.1 session by sending a NAK-EF message.

If NAK-EF is transmitted, it shall then return immediately to the initial G.994.1 state (R-SILENT0 for an HSTU-R, C-SILENT1 for an HSTU-C) and remain silent for a minimum period of 0.5 s. It may then initiate another G.994.1 session.

If a REQ-RTX message is to be transmitted, the minimum time a station shall wait after the reception of any non-invalid frame shall be .75 s. If a station is receiving non-flag octets that are longer than an invalid frame, it shall defer transmitting REQ-RTX, otherwise the maximum time a station shall wait after the reception of any non-invalid frame is 1.0 s.

If a NAK-EF message is received in any state, the receiving station shall return immediately to the initial G.994.1 state (R-SILENT0 for an HSTU-R, C-SILENT1 for an HSTU-C) and remain silent for a minimum period of 0.5 s. It may then initiate another G.994.1 session.

If an invalid frame is received in any state, it shall be ignored.

Annex A

Support for legacy non-G.994.1 devices

Systems that support interoperability with non-G.994.1-compliant devices shall perform the following procedure at initialization:

The HSTU-R shall alternately attempt G.994.1 and other non-G.994.1 initialization as follows:

–	transmit R-TONES-REQ for 2 s;

–	transmit silence for 100 ms;

–	transmit other non-G.994.1 initiating signal for 2 s;

–	transmit silence for 100 ms;

–	repeat above (starting with R-TONES-REQ).

Manufacturers are encouraged to implement multimode systems to cover this interoperability issue.

NOTE – Future handshake protocols may modify the sequence described in this annex. Implementations are encouraged to tolerate such differences.

Annex B

Operation over multiple wire pairs

Some HSTU-x devices may support operation over multiple wire pairs as either a primary or secondary mode of operation. For these devices, the following additions shall be made to the start-up procedures shown in 11.1 and 11.2:

•	R-Tones-Req (for HSTU-R initiated start-up) and C-Tones shall be transmitted simultaneously on all available wire pairs.

•	The HSTU-R shall select an active wire pair from the set of pairs on which it receives C-Tones.

NOTE – The criterion for active pair selection is implementation dependent; however, implementors are strongly encouraged to consider acceptable performance as a part of the selection process.

•	R-Tone1, R-Flag1, and all succeeding signals transmitted by the HSTU-R shall only be transmitted on the selected active wire pair. Silence shall be sent on any other pairs.

•	The HSTU-C shall designate the pair on which R-Tone1 and/or R-Flag1 is received as its active pair.

•	All succeeding signals transmitted by the HSTU-C shall only be transmitted on the designated active wire pair. Silence shall be sent on any other pairs.

Annex C

An operating mode for use in the same cable as TCM-ISDN

## C.0	Scope (supplements 1)

This annex defines an enhanced mode of operation for use by xDSL systems coexisting in the same binder as TCM-ISDN as defined in Appendix III/G.961. The clauses in this annex provide supplementary and replacement material to the clauses in the main body. The nature of the material is parenthetically indicated in the clause heading. The modifications described in this annex allow a performance improvement from the system specified in the main body in an environment coexisting with TCM-ISDN in the same cable. This enhanced mode of operation uses two new modulation methods:

•	TTR synchronized (TTRS) modulation – Signalling is synchronized to the TTR clock and transmission occurs during TCM-ISDN FEXT periods; and

•	Duplex ½ symbol rate (DHSR) modulation – The symbol rate used with carrier set C43 is 4312.5/16 = 269.53125 symbols per second as opposed to 4312.5/8 = 539.0625 symbols per second as currently defined in 6.1.1. Once the DHSR mode is entered, transmission occurs continuously as per the duplex mode.

## C.1	Signals and modulation (pertains to 6)

### C.1.1	Description of signals (modifies 6.1)

Four transmission modes, Duplex ½ symbol rate (DHSR), TTR synchronized (TTRS), duplex and half-duplex, are defined for use within this Recommendation. The two transmission modes, DHSR and TTRS are defined for use within Annex C. The transmission mode supported is a function of carrier set and is specified in Table C.1.

Frequencies other than those specified in Tables C.1 and 3 shall not be transmitted simultaneously with G.994.1 signals.

Two new signals C-SYNC1 and R-SYNC1 are specified as follows:

C-SYNC1: Unmodulated carriers that are synchronized with TTRC as shown in Figure C.1. Energy shall be transmitted during all shaded time slots (DMT symbols). Phase reversals shall be applied to subcarrier 12 of carrier set C43 between the 2nd and 3rd FEXT DMT symbols of subframe rows 3, 6, 11, and 18. Silence shall be transmitted in all unshaded time slots (DMT symbols). The start of signal shall be aligned with a TTR hyper frame.

R-SYNC1: Unmodulated carriers that are synchronized with TTRR as shown in Figure C.2. Energy shall be transmitted during all shaded time slots (DMT symbols). Phase reversals shall be applied to subcarrier 9 of carrier set C43 between the 2nd and 3rd FEXT DMT symbols of subframe rows 5, 9, 15, and 23. Silence shall be transmitted in all unshaded time slots (DMT symbols). The start of signal shall be aligned with a TTR hyper frame.

The tolerance of the symbol rate and carrier frequencies for an HSTU-C shall be 50 ppm. The tolerance of the symbol rate and carrier frequencies for an HSTU-R shall be 200 ppm during R-TONES-REQ and 50 ppm during and after R-SYNC1. The HSTU-R may perform partial or full loop timing acquisition during the silence period preceding R-SYNC1. For both the HSTU-C and HSTU-R transmitter, the symbol rate and carrier frequencies shall be time locked.

#### C.1.1.1	Signalling family (replaces 6.1.1)

Carrier frequencies within this signalling family are given by N × 4.3125 kHz, where N is a positive integer. The symbol rate shall be 4312.5*32/345  400 symbols per second when using the TTRS modulation and the symbol rate shall be 4312.5/16  269.53125 symbols per second when using the DHSR modulation.

Within this family, there are three upstream carrier sets, designated A43, B43, and C43. Each upstream carrier set has an associated downstream carrier set that carries the same designation. The carrier set frequencies and the maximum transmit power level per carrier for each carrier set are defined in Table C.1 where frequency = N × 4.3125 kHz.

The carrier sets in this family are mandatory for the xDSL modes listed in Table C.2. One or more carriers listed in Table C.1 may be transmitted in addition to the mandatory carrier set listed in Table C.2. Carriers not listed in Table C.1 or Table 3 shall not be transmitted.

Table C.1/G.994.1 – Carrier sets for the 4.3125 kHz signalling family

| Carrier set designation | Upstream carrier sets |  | Downstream carrier sets |  | Transmission mode |
| --- | --- | --- | --- | --- | --- |
|  | Frequency indices (N) | Maximum power level/carrier (dBm) | Frequency indices (N) | Maximum power level/carrier (dBm) |  |
| A43 | 9 17 25 | –1.65 | 40 56 64 | –3.65 | Duplex only |
| B43 | 37 45 53 | –1.65 | 72 88 96 | –3.65 | Duplex only |
| C43 (Note) | 7 9 | –1.65 | 12 14 64 (Note) | –3.65 | TTRS or DHRS |
| J43 | 9 17 25 | –1.65 | 72 88 96 | –3.65 | Duplex only |
| NOTE – For the TTRS mode only, a pilot tone using Subcarrier 16, modulated by data pattern {0,0}, generating the (+,+) constellation point is transmitted during FEXTR periods in conjunction with the modulated carriers. |  |  |  |  |  |

Table C.2/G.994.1 – Mandatory carrier sets

| xDSL Recommendation(s) | Carrier set designation |
| --- | --- |
| G.992.1 – Annex A, G.992.2 – Annexes A/B, G.992.3 – Annexes A/I, G.992.4 – Annexes A/B | A43 |
| G.992.1 – Annex B, G.992.3 – Annex B | B43 |
| G.992.1 – Annex C, G.992.2 – Annex C, G.992.1 – Annex H, [& subsequent Annexes for use in TCM-ISDN environment] | C43 |
| G.992.3 – Annex J | J43 |

### C.1.2	Modulation (replaces 6.2)

All messages in Annex C are sent with the C43 carrier set. All carrier frequencies within the carrier set are simultaneously modulated with the same data bits using Differentially encoded binary Phase Shift Keying (DPSK). The transmit point is rotated 180° from the previous point if the transmit bit is a 1, and the transmit point is rotated 0° from the previous point if the transmit bit is a 0. When using the TTRS modulation, the modulation shall be synchronized with TTRC and TTRR respectively. Energy shall only be transmitted during all shaded time slots (DMT symbols) as shown in Figure C.1 for downstream and Figure C.2 for upstream. Silence shall be transmitted in all unshaded time slots (DMT symbols).

Figure C.1/G.994.1 – Symbol pattern in a hyperframe – Downstream (from Figure C.15/G.992.1)

Figure C.2/G.994.1 – Symbol pattern in a hyperframe – Upstream (from Figure C.16/G.992.1)

## C.2	Start-up/cleardown procedures (pertains to 11)

### C.2.1	Start-up procedures (new)

#### C.2.1.1	HSTU-R initiated start-up procedure (new)

Figure C.3 displays the timing for the HSTU-R initiated start-up procedure. Initially, the HSTU-R is in state R-SILENT0 transmitting silence, and the HSTU-C is in state C-SILENT1 transmitting silence. The HSTU-R shall initiate the start-up procedure by transmitting signals from the C43 carrier set, with phase reversals every 16 ms (R-TONES-REQ). When this has been detected by the HSTU-C, the HSTU-C shall respond by transmitting the C-SYNC1 signal synchronized with the beginning of a TTR hyper-frame. When this has been detected by the HSTU-R, the HSTU-R shall transmit silence (R-SILENT1) for 160 to 500 ms and shall then transmit signals from only one signalling family (R-SYNC1). The minimum detection time for C-SYNC1 shall be 160 ms. When the HSTU-C has detected R-SYNC1, it shall respond by transmitting Galfs on modulated carriers (C-GALF1). If the HSTU-C wishes to use Duplex ½ symbol rate (DHSR) mode, it shall transmit C-GALF using the DHSR modulation continuously as per the duplex mode. If the HSTU-C wishes to use TTR Synchronized (TTRS) mode, it shall transmit C-GALF1 using the TTRS modulation. For the TTRS mode only, a pilot tone using Subcarrier 16, modulated by data pattern {0,0}, generating the (+,+) constellation point shall also be transmitted beginning with C-GALF1. The HSTU-C shall maintain the selected modulation throughout the subsequent handshake procedure. When the HSTU-R has detected Galfs, it shall respond by transmitting Flags on modulated carriers (R-FLAG1). The HSTU-R shall transmit R-FLAG1 and subsequent messages using the same modulation as the HSTU-C transmitted during C-GALF1. When the HSTU-C has detected Flags, it shall respond by transmitting Flags (C-FLAG1). When the HSTU-R has detected Flags, it shall begin the first transaction.

Figure C.3 shows the timing requirements between events that shall be adhered to. 1 is the time period from detection of a signal (e.g., R-SYNC1) to the transmission of the next signal (e.g., C-GALF1).

Figure C.3/G.994.1 – HSTU-R initiated start-up procedure

#### C.2.1.2	HSTU-C initiated start-up procedure (new)

Figure C.4 displays the timing for the HSTU-C initiated start-up procedure. Initially, the HSTU-R is in state R-SILENT0 transmitting silence, and the HSTU-C is in state C-SILENT1 transmitting silence. The HSTU-C shall initiate the start-up procedure by transmitting the C-SYNC1 signal synchronized with the beginning of a TTR hyper-frame. When this has been detected by the HSTU-R, the HSTU-R shall respond by transmitting signal (R-SYNC1). The minimum detection time for C-SYNC1 shall be 160 ms. When the HSTU-C has detected R-SYNC1, it shall respond by transmitting Galfs on modulated carriers (C-GALF1). If the HSTU-C wishes to use Duplex ½ symbol rate (DHSR) mode, it shall transmit C-GALF using the DHSR modulation continuously as per the duplex mode. If the HSTU-C wishes to use TTR Synchronized (TTRS) mode, it shall transmit C-GALF1 using the TTRS modulation. For the TTRS mode only, a pilot tone using Subcarrier 16, modulated by data pattern {0,0}, generating the (+,+) constellation point shall also be transmitted beginning with C-GALF1. The HSTU-C shall maintain the selected modulation throughout the subsequent handshake procedure. When the HSTU-R has detected Galfs, it shall respond by transmitting Flags on modulated carriers (R-FLAG1). The HSTU-R shall transmit R-FLAG1 and subsequent messages using the same modulation as the HSTU-C transmitted during C-GALF1. When the HSTU-C has detected Flags, it shall respond by transmitting Flags (C-FLAG1). When the HSTU-R has detected Flags, it shall begin the first transaction.

Figure C.4 shows the timing requirements between events that shall be adhered to. 1 is the time period from detection of a signal (e.g., R-SYNC1) to the transmission of the next signal (e.g., C-GALF1).

Figure C.4/G.994.1 – HSTU-C initiated duplex start-up procedure

### C.2.2	Interoperability procedure (new)

If an HSTU-R does not respond to C-SYNC1, the HSTU-C shall alternately attempt C-TONES initialization as follows:

–	transmit C-SYNC1 for 2 s;

–	transmit silence for 100 ms;

–	transmit C-TONES for 2 s;

–	transmit silence for 100 ms;

–	repeat above (starting with C-SYNC1).

Appendix I

G.994.1 sample sessions

| Session | msg 1 | msg 2 | msg 3 | msg 4 | msg 5 | msg 6 | msg 7 | msg 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | CLR | cl | ACK(1) | MS | ack(1) |  |  |  |
| 2 | MS | ack(1) |  |  |  |  |  |  |
| 3 | MS | req-mr | MR | ms | ACK(1) |  |  |  |
| 4 | MS | req-clr | CLR | cl | ACK(1) | MS | ack(1) |  |
| 5 | CLR | cl | ACK(1) | MR | ms | ACK(1) |  |  |
| 6 | MR | ms | ACK(1) |  |  |  |  |  |
| 7 | MR | req-ms | MS | ack(1) |  |  |  |  |
| 8 | MR | req-clr | CLR | cl | ACK(1) | MR | ms | ACK(1) |
| NOTE – HSTU-R transmitted messages are shown in uppercase, whilst HSTU-C transmitted messages are shown in lowercase. |  |  |  |  |  |  |  |  |

In the case when an FCS indicates an errored frame, a REQ-RTX transaction may become part of a basic or enhanced transaction. Some examples are given below. An "X" next to a message indicates the message was received with an FCS error. The parenthetical after the REQ-RTX indicates the contents of the Last Correctly Received Message (LCRM) octet.

| Session | msg1 | msg2 | msg3 | msg4 | msg5 | msg6 | msg7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 9 | CLR | cl | ACK(1) | MS X | req-rtx (ack(1)) | MS | ack(1) |
| 10 | CLR | cl X | REQ-RTX (NULL) | nak-cd |  |  |  |
| 11 | CLR0 | ack(2) | CLR1 | ack(2) | CLR2 X | req-rtx (clr1) | CLR2 |
| 12 | CLR | cl X | REQ-RTX (NULL) X | req-rtx (clr) | REQ-RTX (NULL) | nak-cd |  |
| 13 | CLR | cl X | REQ-RTX (NULL) X | req-rtx (clr) X | REQ-RTX (NULL) | nak-cd |  |
| 14 | MS | ack(1) X | REQ-RTX (NULL) | ack(1) |  |  |  |
| 15 | CLR X | req-rtx (null) X | REQ-RTX (NULL) | nak-cd |  |  |  |

Appendix II

Provider code contact information

It is often difficult for implementers to find information about obtaining provider codes since they are regional and not specified in ITU-T Rec. T.35. This appendix contains information about obtaining provider codes.

| Country | Relevant document | Contact telephone | Contact information |
| --- | --- | --- | --- |
| USA | T1.220 | +1 732 699 5577 | Telcordia Language Standards Department 444 Hoes Lane, Piscataway NJ 08854 USA |
| Japan | – | +81 3 3432 1551 | TTC, 1-2-11, Hamamatsu-cho, Minato-ku, Tokyo 105-0013, JAPAN |
| Belgium | – | Tel. +32 2 226 88 99 Fax. +32 2 223 11 28 | Belgian Institute of postal services and telecommunications Avenue de l'Astronomie, 14 Boîte 21 1210 Brussels, BELGIUM |

Appendix III

Support for legacy DMT-based devices

Systems that support interoperability with [1] (see Appendix VI – Bibliography) should perform the following functions at initialization:

–	The HSTU-C should monitor for R-ACK-REQ (as defined in [1]);

–	The HSTU-R should monitor for C-ACT1, C-ACT2, C-ACT3, C-ACT4, and C-TONE (as defined in [1]);

–	The HSTU-R should alternately attempt initiating signals from this Recommendation and [1] as follows:

•	transmit R-TONES-REQ for 2 s;

•	transmit silence for 100 ms;

•	transmit R-ACK-REQ for 2 s;

•	transmit silence for 100 ms;

•	repeat above (starting with R-TONES-REQ).

Upon detection of the above tones, the system that supports interoperability with [1] should follow the procedures and functions described therein.

Appendix IV

Procedure for the assignment of additional G.994.1 parameters

## IV.1	Introduction

This appendix defines the procedure for requesting the assignment of G.994.1 parameters that are indicated as being "Reserved for allocation by the ITU-T." This procedure is intended to allow for the expedient assignment of such parameters.

This procedure does not cover changes to the overall structure of this Recommendation (which requires application of the ITU-T Resolution 1 procedures).

## IV.2	Procedure

An ITU-T Working Party or Study Group that identifies a need for the assignment of a G.994.1 parameter is required to make that request to the Chairman of the Study Group with responsibility for this Recommendation with copies of the request to the editor of this Recommendation and the TSB counsellor for the Study Group concerned. After consultation with the editor of this Recommendation, the Chairman will either approve the request or provide an alternative solution to the request (if a change is necessary to achieve compliance with this Recommendation). Periodically, these allocated parameters will be included in the G.994.1 Implementor's Guide, which will finally be merged into a subsequent version of this Recommendation.

It is expected that requests using this procedure will obtain a response within one month.

Appendix V

Rules for code point table numbering

This appendix defines the rules that apply to the assignment of table numbering in 9.3.4 (parameter field), and 9.4 (standard information field).

| Digit position | Used for |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | Npar(1) | Spar(1) |  |  |  |
| examples  rules  | Table x (x is even) | Table x (x is odd) |  |  |  |
| 2 | Npar(1) extensions |  | Npar(2) | Spar(2) |  |
| examples  rules  | Table x.1 (x is even) |  | Table x.y (x is odd) (y is odd) | Table x.y (x is odd) (y is even) |  |
| 3 |  | Spar(1) extensions | Npar(2) extensions |  | Npar(3) |
| examples  rules  |  | Table x.y.1 (x is odd) (y ≡ 0) | Table x.y.1 (x is odd) (y is odd) |  | Table x.y.1 (x is odd) (y is even) |
| 4 |  |  |  | Spar(2) extensions | Npar(3) extensions |
| examples  rules  |  |  |  | Table x.y.z.1 (x is odd) (y is even) (z ≡ 0) | Table x.y.1.1 (x is odd) (y is even) |

Appendix VI

Bibliography

[1]	ANSI T1.413-1998, Network to Customer Installation Interfaces – Asymmetric Digital Subscriber Line (ADSL) Metallic Interface.

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |