|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.994.1 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | Amendment 1 (02/2004) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital sections and digital line system – Access networks |  |  |  |
|  | Handshake procedures for digital subscriber line (DSL) transceivers Amendment 1 |  |  |  |
|  | ITU-T Recommendation G.994.1 (2003) – Amendment 1 |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| General | G.900–G.909 |
| Parameters for optical fibre cable systems | G.910–G.919 |
| Digital sections at hierarchical bit rates based on a bit rate of 2048 kbit/s | G.920–G.929 |
| Digital line transmission systems on cable at non-hierarchical bit rates | G.930–G.939 |
| Digital line systems provided by FDM transmission bearers | G.940–G.949 |
| Digital line systems | G.950–G.959 |
| Digital section and digital transmission systems for customer access to ISDN | G.960–G.969 |
| Optical fibre submarine cable systems | G.970–G.979 |
| Optical line systems for local and access networks | G.980–G.989 |
| Access networks | G.990–G.999 |
| QUALITY OF SERVICE AND PERFORMANCE - GENERIC AND USER-RELATED ASPECTS | G.1000–G.1999 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.6000–G.6999 |
| DIGITAL TERMINAL EQUIPMENTS | G.7000–G.7999 |
| DIGITAL NETWORKS | G.8000–G.8999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.994.1 Handshake procedures for digital subscriber line (DSL) transceivers Amendment 1

Summary This amendment addresses the following issues: • Specification of the signalling family and carrier frequencies to support the IEEE EFM VDSL specification. • Parameters to support IEEE EFM. • Specification of the carrier set and parameters to support G.992.3 Annex M. • Specification of the carrier sets for G.992.1 Annexes H & I, G.992.3 Annex M, G.992.4 Annex I, and G.992.5 Annexes A, B, I, J & M. • Parameters to support ITU-T Rec. G.991.2. • Parameters to support G.992.3 Annex L – merged with Annex A. • A parameter to support T1 enhanced SHDSL. • Parameters to support G.992.3 Annex J extended upstream PSDs. • Parameters to support upstream mask selection in G.992.5 Annexes J and M. • Text to clarify the meaning of "capabilities". • Specification of an alternative carrier set for use where regulatory requirements prevent the use of A43.

Source Amendment 1 to ITU-T Recommendation G.994.1 (2003) was approved on 22 February 2004 by ITU-T Study Group 15 (2001-2004) under the ITU-T Recommendation A.8 procedure.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g. interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met.  The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2004

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS

Page

1)	Tables 1 and 2		1

2)	Tables 11.x.x		2

3)	Table 11.35		32

4)	Table 11.47		32

5)	Clause 3, Defitions		32

6)	New clause 13		33

7)	Table 11.31		33

8)	Table 11.33		34

9)	Table 11.35		34

10)	Table 11.37		34

11)	Table 11.39		34

12)	Table 11.43		35

13)	Table 11.45		35

14)	Table 9.xx		35

ITU-T Recommendation G.994.1

Handshake procedures for digital subscriber line (DSL) transceivers

Amendment 1

# 1)	Tables 1 and 2

Revise Tables 1 and 2 in 6.1.1 (4.3125 kHz signalling family) as follows:

Table 1/G.994.1 – Carrier sets for the 4.3125 kHz signalling family

| Carrier set designation | Upstream carrier sets |  | Downstream carrier sets |  | Transmission mode |
| --- | --- | --- | --- | --- | --- |
|  | Frequency indices (N) | Maximum power level/carrier (dBm) | Frequency indices (N) | Maximum power level/carrier (dBm) |  |
| A43 | 9 17 25 | –1.65 | 40 56 64 | –3.65 | Duplex only |
| B43 | 37 45 53 | –1.65 | 72 88 96 | –3.65 | Duplex only |
| C43 | 7 9 | –1.65 | 12 14 64 | –3.65 | Duplex only |
| J43 | 9 17 25 | –1.65 | 72 88 96 | –3.65 | Duplex only |
| V43 | 37 53 | –1.65 | 64 88 | –3.65 | Duplex only |
| A43c (Note) | 9 17 25 | –1.65 | 257 293 337 | –3.65 | Duplex only |
| NOTE – In some jurisdictions, it may be necessary to limit the maximum power level, for example –23.65 dBm/carrier where the PSD is limited to –60 dBm/Hz. |  |  |  |  |  |

Table 2/G.994.1 – Mandatory carrier sets

| xDSL Recommendation(s) | Carrier set designation |
| --- | --- |
| G.992.1 – Annex A, G.992.2 – Annexes A/B, G.992.3 – Annexes A/I/L, G.992.4 – Annexes A/I G.992.5 – Annexes A/I | A43 |
| G.992.5 – Annexes A/I (Note) | A43c |
| G.992.1 – Annex B, G.992.3 – Annex B G.992.5 – Annex B | B43 |
| G.992.1 – Annexes C/H/I, G.992.2 – Annex C | C43 |
| G.992.3 – Annexes J/M, G.992.5 – Annexes J/M | J43 |
| NOTE – To be used where spectrum management forbids use of tone-set A43, typically where G.992.5 is deployed from a cabinet. |  |

# 2)	Tables 11.x.x

Revise existing and add new Tables 11.x.x as follows:

Table 11.0.1/G.994.1 – Standard information field – SPar(1) coding – Octet 2

| Bits |  |  |  |  |  |  |  |  | SPar(1)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | G.991.2 – Annexes A/F |
|  | x | x | x | x | x | x | 1 | x | G.991.2 – Annex B |
|  | x | x | x | x | x | 1 | x | x | Committee T1* MCM VDSL (Note 1) |
|  | x | x | x | x | 1 | x | x | x | Committee T1 SCM VDSL (Note 2) |
|  | x | x | x | 1 | x | x | x | x | ETSI MCM VDSL (Note 3) |
|  | x | x | 1 | x | x | x | x | x | ETSI SCM VDSL (Note 3) |
|  | x | 1 | x | x | x | x | x | x | Committee T1 enhanced SHDSL |
|  | x | 0 | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |
| NOTE 1 – Use of this bit is defined in "Draft Trial-Use Standard For Telecommunication – Interface Between Networks and Customer Installation – Very High Bit-rate Digital Subscriber Line (VDSL) Metallic Interface – Part 3: Technical Specification for Multi-Carrier Modulation (MCM) Transceivers". NOTE 2 – Use of this bit is defined in "Draft Trial-Use Standard For Telecommunication – Interface Between Networks and Customer Installation – Very High Bit-rate Digital Subscriber Line (VDSL) Metallic Interface – Part 2: Technical Specification for Single-Carrier Modulation (SCM) Transceivers". NOTE 3 – Use of this bit is defined in ETSI TS 101270-2. |  |  |  |  |  |  |  |  |  |

Table 11.0.2/G.994.1 – Standard information field – SPar(1) coding – Octet 3

| Bits |  |  |  |  |  |  |  |  | SPar(1)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | G.992.3 – Annexes A/L |
|  | x | x | x | x | x | x | 1 | x | G.992.3 – Annex B |
|  | x | x | x | x | x | 1 | x | x | G.992.3 – Annex I |
|  | x | x | x | x | 1 | x | x | x | G.992.3 – Annex J |
|  | x | x | x | 1 | x | x | x | x | G.992.4 – Annex A |
|  | x | x | 1 | x | x | x | x | x | G.992.4 – Annex I |
|  | x | 1 | x | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | 0 | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.0.3/G.994.1 – Standard information field – SPar(1) coding – Octet 4

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | SPar(1)s – Octet 4 |
|  | x | x | x | x | x | x | x | 1 | G.992.5 – Annex A |
|  | x | x | x | x | x | x | 1 | x | G.992.5 – Annex B |
|  | x | x | x | x | x | 1 | x | x | G.992.5 – Annex I |
|  | x | x | x | x | 1 | x | x | x | G.992.3 – Annex M |
|  | x | x | x | 1 | x | x | x | x | G.992.5 Annex J- |
|  | x | x | 1 | x | x | x | x | x | IEEE 802.3ah 2BASE-TL |
|  | x | 1 | x | x | x | x | x | x | IEEE 802.3ah 10PASS-TS |
|  | x | 0 | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.0.4/G.994.1 – Standard information field – SPar(1) coding – Octet 5

| Bits |  |  |  |  |  |  |  |  | SPar(1)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | G.992.5 – Annex M |
|  | x | x | x | x | x | x | 1 | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | 1 | x | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | 0 | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.15/G.994.1 – Standard information field – G.991.2 Annex A –
NPar(2) coding– Octet 1

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex A NPar(2)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Training mode (Note) (see also Table 11.14) |
|  | x | x | x | x | x | x | 1 | x | PMMS mode (Note) (see also Table 11.14) |
|  | x | x | x | x | x | 1 | x | x | Regenerator silent period (Note) |
|  | x | x | x | x | 1 | x | x | x | 4-Wire |
|  | x | x | x | 1 | x | x | x | x | SRU |
|  | x | x | 1 | x | x | x | x | x | Diagnostic mode |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |
| NOTE – Only one of these bits shall be set at any given time. |  |  |  |  |  |  |  |  |  |

Table 11.15.1/G.994.1 – Standard information field – G.991.2 Annex A –
NPar(2) coding – Octet 2

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex A NPar(2)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Warm-start enable |
|  | x | x | x | x | x | x | 1 | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.0.1/G.994.1 – Standard information field – G.991.2 Annex A –
SPar(2) coding – Octet 2

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex A SPar(2)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream framing parameters |
|  | x | x | x | x | x | x | 1 | x | Dual-Mode TPS-TC parameters |
|  | x | x | x | x | x | 1 | x | x | Multiple-Pair Operation parameters |
|  | x | x | x | x | 1 | x | x | x | Downstream extended training rates – 16-TCPAM symmetric (Annex F) |
|  | x | x | x | 1 | x | x | x | x | Downstream extended training rates – 32-TCPAM symmetric (Annex F) |
|  | x | x | 1 | x | x | x | x | x | Upstream extended training rates – 16-TCPAM symmetric (Annex F) |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.0.2/G.994.1 – Standard information field – G.991.2 Annex A –
SPar(2) coding – Octet 3

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex A SPar(2)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Upstream extended training rates – 32-TCPAM symmetric (Annex F) |
|  | x | x | x | x | x | x | 1 | x | Downstream extended PMMS rates (Annex F) |
|  | x | x | x | x | x | 1 | x | x | Upstream extended PMMS rates (Annex F) |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.3.8/G.994.1 – Standard information field – G.991.2 Annex A
Downstream PMMS parameters – NPar(3) coding – Octet 9

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex A downstream PMMS NPar(3)s – Octet 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 1 | Fixed value during PMMS (all other values reserved for allocation by the ITU-T) |

Table 11.16.3.9/G.994.1 – Standard information field – G.991.2 Annex A
Downstream PMMS parameters – NPar(3) coding – Octet 10

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex A downstream PMMS NPar(3)s – Octet 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Fixed value during PMMS (all other values reserved for allocation by the ITU-T) |

Table 11.16.3.11/G.994.1 – Standard information field – G.991.2 Annex A
Downstream PMMS parameters – NPar(3) coding – Octet 12

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex A downstream PMMS NPar(3)s – Octet 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | x | x | x | Downstream PMMS scrambler polynomial Index (i2, i1, i0) |

Table 11.16.3.12/G.994.1 – Standard information field – G.991.2 Annex A
Downstream PMMS parameters – NPar(3) coding – Octet 13

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex A downstream PMMS NPar(3)s – Octet 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 1 | x | x | x | x | x | Worst-case PMMS target margin (dB) (bits 5-1  1.0 dB – 10 dB) |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Worst-case PMMS target margin unspecified by terminal (values of bits 6-1 from 1 to 31 reserved for allocation by the ITU-T) |

Table 11.16.3.13/G.994.1 – Standard information field – G.991.2 Annex A
Downstream PMMS parameters – NPar(3) coding – Octet 14

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex A downstream PMMS NPar(3)s – Octet 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 1 | x | x | x | x | x | Current-condition PMMS target margin (dB) (bits 5-1  1.0 dB – 10 dB) |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Current-condition PMMS target margin unspecified by terminal (values of bits 6-1 from 1 to 31 reserved for allocation by the ITU-T) |

Table 11.16.4.8/G.994.1 – Standard information field – G.991.2 Annex A
Upstream PMMS parameters – NPar(3) coding – Octet 9

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex A upstream PMMS NPar(3)s – Octet 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 1 | Fixed value during PMMS (all other values reserved for allocation by the ITU-T) |

Table 11.16.4.9/G.994.1 – Standard information field – G.991.2 Annex A
Upstream PMMS parameters – NPar(3) coding – Octet 10

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex A upstream PMMS NPar(3)s – Octet 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Fixed value during PMMS (all other values reserved for allocation by the ITU-T) |

Table 11.16.4.11/G.994.1 – Standard information field – G.991.2 Annex A
Upstream PMMS parameters – NPar(3) coding – Octet 12

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex A upstream PMMS NPar(3)s – Octet 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | x | x | x | Upstream PMMS scrambler polynomial Index (i2, i1, i0) |

Table 11.16.4.12/G.994.1 – Standard information field – G.991.2 Annex A
Upstream PMMS parameters – NPar(3) coding – Octet 13

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex A upstream PMMS NPar(3)s – Octet 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 1 | x | x | x | x | x | Worst-case PMMS target margin (dB) (bits 5-1  1.0 dB – 10 dB) |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Worst-case PMMS target margin unspecified by terminal (values of bits 6-1 from 1 to 31 reserved for allocation by the ITU-T) |

Table 11.16.4.13/G.994.1 – Standard information field – G.991.2 Annex A
Upstream PMMS parameters – NPar(3) coding – Octet 14

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex A upstream PMMS NPar(3)s – Octet 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 1 | x | x | x | x | x | Current-condition PMMS target margin (dB) (bits 5-1  1.0 dB – 10 dB) |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Current-condition PMMS target margin unspecified by terminal (values of bits 6-1 from 1 to 31 reserved for allocation by the ITU-T) |

Table 11.16.5.2/G.994.1 – Standard information field – G.991.2 Annex A
TPS-TC parameters – NPar(3) coding – Octet 3

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex A TPS-TC parameter NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  | x | x | x | Number of ISDN BRA (0 to 6) |
|  | x | x | x | x | x |  |  |  | Z-bits used for ISDN BRA Signalling (0 to 7) |

Table 11.16.5.3/G.994.1 – Standard information field – G.991.2 Annex A
TPS-TC parameters – NPar(3) coding – Octet 4

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex A TPS-TC parameter NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | PTM |
|  | x | x | x | x | x | x | 1 | x | STM with DSC |
|  | x | x | x | x | x | 1 | x | x | LAPV5 Enveloped POTS or ISDN |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.8/G.994.1 – Standard information field – G.991.2 Annex A
Dual Mode TPS-TC parameters – NPar(3) coding – Octet 1

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex A Dual Mode TPS-TC parameter Npar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | TPS-TCa data rate – n  64 kbit/s (1 to 36) |
|  | x | x | 1 | 1 | 1 | 1 | 1 | 1 | TPS-TCa data rate unspecified by terminal |

Table 11.16.8.1/G.994.1 – Standard information field – G.991.2 Annex A
Dual Mode TPS-TC parameters – NPar(3) coding – Octet 2

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex A Dual Mode TPS-TC parameter Npar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | x | x | x | TPS-TCa sub-data rate – i  8 kbit/s (0 to 7) |
|  | x | x | 1 | 1 | 1 | 1 | 1 | 1 | TPS-TCa sub-data rate unspecified by terminal (values of bits 6-1 from 8 to 62 reserved for allocation by the ITU-T) |

Table 11.16.8.2/G.994.1 – Standard information field – G.991.2 Annex A
Dual Mode TPS-TC parameters – NPar(3) coding – Octet 3

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex A Dual Mode TPS-TC parameter Npar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Type 1 – TPS-TCb: Clear Channel |
|  | x | x | x | x | x | x | 1 | x | Type 1 – TPS-TCb: Clear Channel Byte-Oriented |
|  | x | x | x | x | x | 1 | x | x | Type 1 – TPS-TCb: Unaligned DS1 |
|  | x | x | x | x | 1 | x | x | x | Type 1 – TPS-TCb: Aligned DS1/Fractional DS1 |
|  | x | x | x | 1 | x | x | x | x | Type 1 – TPS-TCb: ATM |
|  | x | x | 1 | x | x | x | x | x | Type 1 – TPS-TCb: PTM |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.8.4/G.994.1 – Standard information field – G.991.2 Annex A
Dual Mode TPS-TC parameters – NPar(3) coding – Octet 5

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex A Dual Mode TPS-TC parameter Npar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Type 2 – TPS-TCa: Unaligned DS1 |
|  | x | x | x | x | x | x | 1 | x | Type 2 – TPS-TCa: Aligned DS1/Fractional DS1 |
|  | x | x | x | x | x | 1 | x | x | Type 2 – TPS-TCb: ATM |
|  | x | x | x | x | 1 | x | x | x | Type 2 – TPS-TCb: PTM |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.8.6/G.994.1 – Standard information field – G.991.2 Annex A
Dual Mode TPS-TC parameters – NPar(3) coding – Octet 7

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex A Dual Mode TPS-TC parameter Npar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Type 1 – TPS-TCa: ISDN BRA |
|  | x | x | x | x | x | x | 1 | x | Type 1 – TPS-TCa: STM with DSC |
|  | x | x | x | x | x | 1 | x | x | Type 1 – TPS-TCa: LAPV5 Enveloped POTS or ISDN |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.16.8.7/G.994.1 – Standard information field – G.991.2 Annex A
Dual Mode TPS-TC parameters – NPar(3) coding – Octet 8

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex A Dual Mode TPS-TC parameter Npar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | DRR Support: 01 DRR not supported 10 DRR supported, STU-C DRR master 11 DRR supported, STU-R DRR master |
|  | x | x | x | x | x | x |  |  | Lead Time (in SHDSL frames) count = bits 6-2 (supported values = 1 to 15) |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | DRR Support and Lead Time unspecified by terminal |

Table 11.16.9/G.994.1 – Standard information field – G.991.2 Annex A
Multiple-Pair Operation parameters – NPar(3) coding – Octet 1

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex A Multiple-Pair Operation parameters NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | M-pair count (count = bits 2-1 + 1) |
|  | x | x | x | x | x | x |  |  | Reserved for allocation by the ITU-T |

Table 11.16.10/G.994.1 – Standard information field – G.991.2 Annex F
Downstream extended training rates – 16-TCPAM symmetric – NPar(3) coding – Octet 1

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F downstream extended training rate – 16-TCPAM symmetric NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  |  | x | Downstream base data rate – Minimum 1 (bit 7) , 16-TCPAM symmetric PSD |
|  | x | x |  |  |  |  | x |  | Downstream base data rate – Maximum 1 (bit 7), 16-TCPAM symmetric PSD |
|  | x | x |  |  |  | x |  |  | Downstream base data rate – Step 1 (bit 7), 16-TCPAM symmetric PSD |
|  | x | x | x | x | x |  |  |  | Reserved for allocation by ITU-T |

Table 11.16.10.1/G.994.1 – Standard information field – G.991.2 Annex F
Downstream extended training rates – 16-TCPAM symmetric – NPar(3) coding – Octet 2

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F downstream extended training rate – 16-TCPAM symmetric NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Downstream base data rate – Minimum 1 (bits 6-1), 16-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.16.10.2/G.994.1 – Standard information field – G.991.2 Annex F
Downstream extended training rates – 16-TCPAM symmetric – NPar(3) coding – Octet 3

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F downstream extended training rate – 16-TCPAM symmetric NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Downstream base data rate – Maximum 1 (bits 6-1), 16-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.16.10.3/G.994.1 – Standard information field – G.991.2 Annex F
Downstream extended training rates – 16-TCPAM symmetric – NPar(3) coding – Octet 4

|  | Bits |  |  |  |  |  |  |  | G.991.2 Annex F downstream extended training rate – 16-TCPAM symmetric NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Downstream base data rate – Step 1 (bits 6-1), 16-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

.

.

.

Table 11.16.10.j1*4-4/G.994.1 – Standard information field – G.991.2 Annex F
Downstream extended training rates – 16-TCPAM symmetric – NPar(3) coding – Octet j1*4-3

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F downstream extended training rate – 16-TCPAM symmetric NPar(3)s – Octet j1*4-3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  |  | x | Downstream base data rate extension – Minimum j1 (bit 7), 16-TCPAM symmetric PSD |
|  | x | x |  |  |  |  | x |  | Downstream base data rate extension – Maximum j1 (bit 7), 16-TCPAM symmetric PSD |
|  | x | x |  |  |  | x |  |  | Downstream base data rate extension – Step j1 (bit 7), 16-TCPAM symmetric PSD |
|  | x | x | x | x | x |  |  |  | Reserved for allocation by ITU-T |

Table 11.16.10.j1*4-3/G.994.1 – Standard information field – G.991.2 Annex F
Downstream extended training rates – 16-TCPAM symmetric – NPar(3) coding – Octet j1*4-2

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F downstream extended training rate – 16-TCPAM symmetric NPar(3)s – Octet j1*4-2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Downstream base data rate extension – Minimum j1 (bits 6-1), 16-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.16.10.j1*4-2/G.994.1 – Standard information field – G.991.2 Annex F
Downstream extended training rates – 16-TCPAM symmetric – NPar(3) coding – Octet j1*4-1

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F downstream extended training rate – 16-TCPAM symmetric NPar(3)s – Octet j1*4-1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Downstream base data rate extension – Maximum j1 (bits 6-1), 16-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.16.10.j1*4-1/G.994.1 – Standard information field – G.991.2 Annex F
Downstream extended training rates – 16-TCPAM symmetric – NPar(3) coding – Octet j1*4

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F downstream extended training rate – 16-TCPAM symmetric NPar(3)s – Octet j1*4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Downstream base data rate extension – Step j1 (bits 6-1), 16-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.16.11/G.994.1 – Standard information field – G.991.2 Annex F
Downstream extended training rates – 32-TCPAM symmetric – NPar(3) coding – Octet 1

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F downstream extended training rate – 32-TCPAM symmetric NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  |  | x | Downstream base data rate – Minimum 1 (bit 7) , 32-TCPAM symmetric PSD |
|  | x | x |  |  |  |  | x |  | Downstream base data rate – Maximum 1 (bit 7), 32-TCPAM symmetric PSD |
|  | x | x |  |  |  | x |  |  | Downstream base data rate – Step 1 (bit 7), 32-TCPAM symmetric PSD |
|  | x | x | x | x | x |  |  |  | Reserved for allocation by ITU-T |

Table 11.16.11.1/G.994.1 – Standard information field – G.991.2 Annex F
Downstream extended training rates – 32-TCPAM symmetric – NPar(3) coding – Octet 2

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F downstream extended training rate 32-TCPAM symmetric NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Downstream base data rate – Minimum 1 (bits 6-1), 32-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.16.11.2/G.994.1 – Standard information field – G.991.2 Annex F
Downstream extended training rates – 32-TCPAM symmetric – NPar(3) coding – Octet 3

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F downstream extended training rate – 32-TCPAM symmetric NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Downstream base data rate – Maximum 1 (bits 6-1), 32-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.16.11.3/G.994.1 – Standard information field – G.991.2 Annex F
Downstream extended training rates – 32-TCPAM symmetric – NPar(3) coding – Octet 4

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F downstream extended training rate – 32-TCPAM symmetric NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Downstream base data rate – Step 1 (bits 6-1), 32-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

.

.

.

Table 11.16.11.j2*4-4/G.994.1 – Standard information field – G.991.2 Annex F
Downstream extended training rates – 32-TCPAM symmetric – NPar(3) coding – Octet j2*4-3

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F downstream extended training rate – 32-TCPAM symmetric NPar(3)s – Octet j2*4-3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  |  | x | Downstream base data rate extension – Minimum j2 (bit 7), 32-TCPAM symmetric PSD |
|  | x | x |  |  |  |  | x |  | Downstream base data rate extension – Maximum j2 (bit 7), 32-TCPAM symmetric PSD |
|  | x | x |  |  |  | x |  |  | Downstream base data rate extension – Step j2 (bit 7), 32-TCPAM symmetric PSD |
|  | x | x | x | x | x |  |  |  | Reserved for allocation by ITU-T |

Table 11.16.11.j2*4-3/G.994.1 – Standard information field – G.991.2 Annex F
Downstream extended training rates – 32-TCPAM symmetric – NPar(3) coding – Octet j2*4-2

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F downstream extended training rate – 32-TCPAM symmetric NPar(3)s – Octet j2*4-2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Downstream base data rate extension – Minimum j2 (bits 6-1), 32-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.16.11.j2*4-2/G.994.1 – Standard information field – G.991.2 Annex F
Downstream extended training rates – 32-TCPAM symmetric – NPar(3) coding – Octet j2*4-1

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F downstream extended training rate – 32-TCPAM symmetric NPar(3)s – Octet j2*4-1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Downstream base data rate extension – Maximum j2 (bits 6-1), 32-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.16.11.j2*4-1/G.994.1 – Standard information field – G.991.2 Annex F
Downstream extended training rates – 32-TCPAM symmetric – NPar(3) coding – Octet j2*4

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F downstream extended training rate – 32-TCPAM symmetric NPar(3)s – Octet j2*4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Downstream base data rate extension – Step j2 (bits 6-1), 32-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.16.12/G.994.1 – Standard information field – G.991.2 Annex F
Upstream extended training rates – 16-TCPAM symmetric – NPar(3) coding – Octet 1

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F upstream extended training rate – 16-TCPAM symmetric NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  |  | x | Upstream base data rate – Minimum 1 (bit 7) , 16-TCPAM symmetric PSD |
|  | x | x |  |  |  |  | x |  | Upstream base data rate – Maximum 1 (bit 7), 16-TCPAM symmetric PSD |
|  | x | x |  |  |  | x |  |  | Upstream base data rate – Step 1 (bit 7), 16-TCPAM symmetric PSD |
|  | x | x | x | x | x |  |  |  | Reserved for allocation by ITU-T |

Table 11.16.12.1/G.994.1 – Standard information field – G.991.2 Annex F
Upstream extended training rates – 16-TCPAM symmetric – NPar(3) coding – Octet 2

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F upstream extended training rate – 16-TCPAM symmetric NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Upstream base data rate – Minimum 1 (bits 6-1), 16-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.16.12.2/G.994.1 – Standard information field – G.991.2 Annex F
Upstream extended training rates – 16-TCPAM symmetric – NPar(3) coding – Octet 3

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F upstream extended training rate – 16-TCPAM symmetric NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Upstream base data rate – Maximum 1 (bits 6-1), 16-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.16.12.3/G.994.1 – Standard information field – G.991.2 Annex F
Upstream extended training rates – 16-TCPAM symmetric – NPar(3) coding – Octet 4

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F upstream extended training rate – 16-TCPAM symmetric NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Upstream base data rate – Step 1 (bits 6-1), 16-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

.

.

.

Table 11.16.12.j3*4-4/G.994.1 – Standard information field – G.991.2 Annex F
Upstream extended training rates – 16-TCPAM symmetric – NPar(3) coding – Octet j3*4-3

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F upstream extended training rate – 16-TCPAM symmetric NPar(3)s – Octet j3*4-3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  |  | x | Upstream base data rate extension – Minimum j3 (bit 7), 16-TCPAM symmetric PSD |
|  | x | x |  |  |  |  | x |  | Upstream base data rate extension – Maximum j3 (bit 7), 16-TCPAM symmetric PSD |
|  | x | x |  |  |  | x |  |  | Upstream base data rate extension – Step j3 (bit 7), 16-TCPAM symmetric PSD |
|  | x | x | x | x | x |  |  |  | Reserved for allocation by ITU-T |

Table 11.16.12.j3*4-3/G.994.1 – Standard information field – G.991.2 Annex F
Upstream extended training rates – 16-TCPAM symmetric – NPar(3) coding – Octet j3*4-2

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F upstream extended training rate – 16-TCPAM symmetric NPar(3)s – Octet j3*4-2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Upstream base data rate extension – Minimum j3 (bits 6-1), 16-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.16.12.j3*4-2/G.994.1 – Standard information field – G.991.2 Annex F
Upstream extended training rates – 16-TCPAM symmetric – NPar(3) coding – Octet j3*4-1

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F upstream extended training rate – 16-TCPAM symmetric NPar(3)s – Octet j3*4-1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Upstream base data rate extension – Maximum j3 (bits 6-1), 16-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.16.12.j3*4-1/G.994.1 – Standard information field – G.991.2 Annex F
Upstream extended training rates – 16-TCPAM symmetric – NPar(3) coding – Octet j3*4

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F upstream extended training rate – 16-TCPAM symmetric NPar(3)s – Octet j3*4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Upstream base data rate extension – Step j3 (bits 6-1), 16-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.16.13/G.994.1 – Standard information field – G.991.2 Annex F
Upstream extended training rates – 32-TCPAM symmetric – NPar(3) coding – Octet 1

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F upstream extended training rate – 32-TCPAM symmetric NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  |  | x | Upstream base data rate – Minimum 1 (bit 7) , 32-TCPAM symmetric PSD |
|  | x | x |  |  |  |  | x |  | Upstream base data rate – Maximum 1 (bit 7), 32-TCPAM symmetric PSD |
|  | x | x |  |  |  | x |  |  | Upstream base data rate – Step 1 (bit 7), 32-TCPAM symmetric PSD |
|  | x | x | x | x | x |  |  |  | Reserved for allocation by ITU-T |

Table 11.16.13.1/G.994.1 – Standard information field – G.991.2 Annex F
Upstream extended training rates – 32-TCPAM symmetric – NPar(3) coding – Octet 2

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F upstream extended training rate – 32-TCPAM symmetric NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Upstream base data rate – Minimum 1 (bits 6-1), 32-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.16.13.2/G.994.1 – Standard information field – G.991.2 Annex F
Upstream extended training rates – 32-TCPAM symmetric – NPar(3) coding – Octet 3

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F upstream extended training rate – 32-TCPAM symmetric NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Upstream base data rate – Maximum 1 (bits 6-1), 32-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.16.13.3/G.994.1 – Standard information field – G.991.2 Annex F
Upstream extended training rates – 32-TCPAM symmetric – NPar(3) coding – Octet 4

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F upstream extended training rate – 32-TCPAM symmetric NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Upstream base data rate – Step 1 (bits 6-1), 32-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

.

.

.

Table 11.16.13.j4*4-4/G.994.1 – Standard information field – G.991.2 Annex F
Upstream extended training rates – 32-TCPAM symmetric – NPar(3) coding – Octet j4*4-3

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F upstream extended training rate – 32-TCPAM symmetric NPar(3)s – Octet j4*4-3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  |  | x | Upstream base data rate extension – Minimum j4 (bit 7), 32-TCPAM symmetric PSD |
|  | x | x |  |  |  |  | x |  | Upstream base data rate extension – Maximum j4 (bit 7), 32-TCPAM symmetric PSD |
|  | x | x |  |  |  | x |  |  | Upstream base data rate extension – Step j4 (bit 7), 32-TCPAM symmetric PSD |
|  | x | x | x | x | x |  |  |  | Reserved for allocation by ITU-T |

Table 11.16.13.j4*4-3/G.994.1 – Standard information field – G.991.2 Annex F
Upstream extended training rates – 32-TCPAM symmetric – NPar(3) coding – Octet j4*4-2

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F upstream extended training rate – 32-TCPAM symmetric NPar(3)s – Octet j4*4-2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Upstream base data rate extension – Minimum j4 (bits 6-1), 32-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.16.13.j4*4-2/G.994.1 – Standard information field – G.991.2 Annex F
Upstream extended training rates – 32-TCPAM symmetric – NPar(3) coding – Octet j4*4-1

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F upstream extended training rate – 32-TCPAM symmetric NPar(3)s – Octet j4*4-1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Upstream base data rate extension – Maximum j4 (bits 6-1), 32-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.16.13.j4*4-1/G.994.1 – Standard information field – G.991.2 Annex F
Upstream extended training rates – 32-TCPAM symmetric – NPar(3) coding – Octet j4*4

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F upstream extended training rate – 32-TCPAM symmetric NPar(3)s – Octet j4*4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Upstream base data rate extension – Step j4 (bits 6-1), 32-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.16.14/G.994.1 – Standard information field – G.991.2 Annex F
Downstream Extended PMMS rates – NPar(3) coding – Octet 1

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F downstream extended PMMS rate NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  |  | x | Downstream base data rate – Minimum 1 (bit 7) , 32-TCPAM symmetric PSD |
|  | x | x |  |  |  |  | x |  | Downstream base data rate – Maximum 1 (bit 7), 32-TCPAM symmetric PSD |
|  | x | x |  |  |  | x |  |  | Downstream base data rate – Step 1 (bit 7), 32-TCPAM symmetric PSD |
|  | x | x | x | x | x |  |  |  | Reserved for use by ITU-T |

Table 11.16.14.1/G.994.1 – Standard information field – G.991.2 Annex F
Downstream Extended PMMS rates – NPar(3) coding – Octet 2

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F downstream extended PMMS rate NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Downstream base data rate – Minimum 1 (bits 6-1), 32-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.16.14.2/G.994.1 – Standard information field – G.991.2 Annex F
Downstream Extended PMMS rates – NPar(3) coding – Octet 3

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F downstream extended PMMS rate NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Downstream base data rate – Maximum 1 (bits 6-1), 32-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.16.14.3/G.994.1 – Standard information field – G.991.2 Annex F
Downstream Extended PMMS rates – NPar(3) coding – Octet 4

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F downstream extended PMMS rate NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Downstream base data rate – Step 1 (bits 6-1), 32-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

.

.

.

Table 11.16.14.j5*4-4/G.994.1 – Standard information field – G.991.2 Annex F
Downstream Extended PMMS rates – NPar(3) coding – Octet j5*4-3

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F downstream extended PMMS rate NPar(3)s – Octet j5*4-3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  |  | x | Downstream base data rate extension – Minimum j5 (bit 7) 32-TCPAM symmetric PSD |
|  | x | x |  |  |  |  | x |  | Downstream base data rate extension – Maximum j5 (bit 7) 32-TCPAM symmetric PSD |
|  | x | x |  |  |  | x |  |  | Downstream base data rate extension – Step j5 (bit 7) 32-TCPAM symmetric PSD |
|  | x | x | x | x | x |  |  |  | Reserved for allocation by ITU-T |

Table 11.16.14.j5*4-3/G.994.1 – Standard information field – G.991.2 Annex F
Downstream Extended PMMS rates – NPar(3) coding – Octet j5*4-2

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F downstream extended PMMS rate NPar(3)s – Octet j5*4-2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Downstream base data rate extension –Minimum j5 (bits 6-1) 32-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.16.14.j5*4-2/G.994.1 – Standard information field – G.991.2 Annex F
Downstream Extended PMMS rates – NPar(3) coding – Octet j5*4-1

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F downstream extended PMMS rate NPar(3)s – Octet j5*4-1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Downstream base data rate extension –Maximum j5 (bits 6-1) 32-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.16.14.j5*4-1/G.994.1 – Standard information field – G.991.2 Annex F
Downstream Extended PMMS rates – NPar(3) coding – Octet j5*4

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F downstream extended PMMS rate NPar(3)s – Octet j5*4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Downstream base data rate extension – Step j5 (bits 6-1) 32-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.16.15/G.994.1 – Standard information field – G.991.2 Annex F
Upstream Extended PMMS rates – NPar(3) coding – Octet 1

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F upstream extended PMMS rate NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  |  | x | Upstream base data rate – Minimum 1 (bit 7), 32-TCPAM symmetric PSD |
|  | x | x |  |  |  |  | x |  | Upstream base data rate – Maximum 1 (bit 7), 32-TCPAM symmetric PSD |
|  | x | x |  |  |  | x |  |  | Upstream base data rate – Step 1 (bit 7), 32-TCPAM symmetric PSD |
|  | x | x | x | x | x |  |  |  | Reserved for use by ITU-T |

Table 11.16.15.1/G.994.1 – Standard information field – G.991.2 Annex F
Upstream Extended PMMS rates – NPar(3) coding – Octet 2

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F upstream extended PMMS rate NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Upstream base data rate – Minimum 1 (bits 6-1), 32-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.16.15.2/G.994.1 – Standard information field – G.991.2 Annex F
Upstream Extended PMMS rates – NPar(3) coding – Octet 3

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F upstream extended PMMS rate NPar(3)s – Octet 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Upstream base data rate – Maximum 1 (bits 6-1), 32-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.16.15.3/G.994.1 – Standard information field – G.991.2 Annex F
Upstream Extended PMMS rates – NPar(3) coding – Octet 4

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F upstream extended PMMS rate NPar(3)s – Octet 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Upstream base data rate – Step 1 (bits 6-1), 32-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

.

.

.

Table 11.16.15.j6*4-4/G.994.1 – Standard information field – G.991.2 Annex F
Upstream Extended PMMS rates – NPar(3) coding – Octet j6*4-3

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F upstream extended PMMS rate NPar(3)s – Octet j6*4-3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  |  | x | Upstream base data rate extension – Minimum j6 (bit 7) 32-TCPAM symmetric PSD |
|  | x | x |  |  |  |  | x |  | Upstream base data rate extension – Maximum j6 (bit 7) 32-TCPAM symmetric PSD |
|  | x | x |  |  |  | x |  |  | Upstream base data rate extension – Step j6 (bit 7) 32-TCPAM symmetric PSD |
|  | x | x | x | x | x |  |  |  | Reserved for allocation by ITU-T |

Table 11.16.15.j6*4-3/G.994.1 – Standard information field – G.991.2 Annex F
Upstream Extended PMMS rates – NPar(3) coding – Octet j6*4-2

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F upstream extended PMMS rate NPar(3)s – Octet j6*4-2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Upstream base data rate extension – Minimum j6 (bits 6-1) 32-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.16.15.j6*4-2/G.994.1 – Standard information field – G.991.2 Annex F
Upstream Extended PMMS rates – NPar(3) coding – Octet j6*4-1

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F upstream extended PMMS rate NPar(3)s – Octet j6*4-1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Upstream base data rate extension – Maximum j6 (bits 6-1) 32-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.16.15.j6*4-1/G.994.1 – Standard information field – G.991.2 Annex F
Upstream Extended PMMS rates – NPar(3) coding – Octet j6*4

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex F upstream extended PMMS rate NPar(3)s – Octet j6*4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Upstream base data rate extension – Step j6 (bits 6-1) 32-TCPAM symmetric PSD (Note) |
|  | NOTE – The rates are determined by combining (bit 7) and the 6 bits in this octet to create a 7-bit number. |  |  |  |  |  |  |  |  |

Table 11.17/G.994.1 – Standard information field – G.991.2 Annex B -
NPar(2) coding – Octet 1

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.991.2 Annex B NPar(2)s – Octet 1 |
|  | x | x | x | x | x | x | x | 1 | Training mode (Note) (see also Table 11.16) |
|  | x | x | x | x | x | x | 1 | x | PMMS mode (Note) (see also Table 11.16) |
|  | x | x | x | x | x | 1 | x | x | Regenerator silent period (Note) |
|  | x | x | x | x | 1 | x | x | x | 4-Wire |
|  | x | x | x | 1 | x | x | x | x | SRU |
|  | x | x | 1 | x | x | x | x | x | Diagnostic Mode |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |
| NOTE – Only one of these bits shall be set at any given time. |  |  |  |  |  |  |  |  |  |

Table 11.17.1/G.994.1 – Standard information field – G.991.2 Annex B -
NPar(2) coding – Octet 2

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.991.2 Annex B NPar(2)s – Octet 2 |
|  | x | x | x | x | x | x | x | 1 | Warm-start enable |
|  | x | x | x | x | x | x | 1 | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |
| – |  |  |  |  |  |  |  |  |  |

Table 11.18.0.1/G.994.1 – Standard information field – G.991.2 Annex B –
SPar(2) coding – Octet 2

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.991.2 Annex B SPar(2)s – Octet 2 |
|  | x | x | x | x | x | x | x | 1 | Upstream framing parameters |
|  | x | x | x | x | x | x | 1 | x | Dual-Mode TPS-TC Parameters |
|  | x | x | x | x | x | 1 | x | x | Multiple-Pair Operation parameters |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.18.3.8/G.994.1 – Standard information field – G.991.2 Annex B
Downstream PMMS parameters – NPar(3) coding – Octet 9

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.991.2 Annex B downstream PMMS NPar(3)s – Octet 9 |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 1 | Fixed value during PMMS (all other values reserved for allocation by the ITU-T) |

Table 11.18.3.9/G.994.1 – Standard information field – G.991.2 Annex B
Downstream PMMS parameters – NPar(3) coding – Octet 10

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.991.2 Annex B downstream PMMS NPar(3)s – Octet 10 |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Fixed value during PMMS (all other values reserved for allocation by the ITU-T) |

Table 11.18.3.11/G.994.1 – Standard information field – G.991.2 Annex B
Downstream PMMS parameters – NPar(3) coding – Octet 12

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.991.2 Annex B downstream PMMS NPar(3)s – Octet 12 |
|  | x | x | 0 | 0 | 0 | x | x | x | Downstream PMMS scrambler polynomial Index (i2, i1, i0) |

Table 11.18.3.12/G.994.1 – Standard information field – G.991.2 Annex B
Downstream PMMS parameters – NPar(3) coding – Octet 13

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.991.2 Annex B downstream PMMS NPar(3)s – Octet 13 |
|  | x | x | 1 | x | x | x | x | x | Worst-case PMMS target margin (dB) (bits 5-1  1.0 dB – 10 dB) |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Worst-case PMMS target margin unspecified by terminal (values of bits 6-1 from 1 to 31 reserved for allocation by the ITU-T) |

Table 11.18.3.13/G.994.1 – Standard information field – G.991.2 Annex B
Downstream PMMS parameters – NPar(3) coding – Octet 14

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.991.2 Annex B downstream PMMS NPar(3)s – Octet 14 |
|  | x | x | 1 | x | x | x | x | x | Current-condition PMMS target margin (dB) (bits 5-1  1.0 dB – 10 dB) |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Current-condition PMMS target margin unspecified by terminal (values of bits 6-1 from 1 to 31 reserved for allocation by the ITU-T) |

Table 11.18.4.8/G.994.1 – Standard information field – G.991.2 Annex B
Upstream PMMS parameters – NPar(3) coding – Octet 9

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.991.2 Annex B upstream PMMS NPar(3)s – Octet 9 |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 1 | Fixed value during PMMS (all other values reserved for allocation by the ITU-T) |

Table 11.18.4.9/G.994.1 – Standard information field – G.991.2 Annex B
Upstream PMMS parameters – NPar(3) coding – Octet 10

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.991.2 Annex B upstream PMMS NPar(3)s – Octet 10 |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Fixed value during PMMS (all other values reserved for allocation by the ITU-T) |

Table 11.18.4.11/G.994.1 – Standard information field – G.991.2 Annex B
Upstream PMMS parameters – NPar(3) coding – Octet 12

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.991.2 Annex B upstream PMMS NPar(3)s – Octet 12 |
|  | x | x | 0 | 0 | 0 | x | x | x | Upstream PMMS scrambler polynomial Index (i2, i1, i0) |

Table 11.18.4.12/G.994.1 – Standard information field – G.991.2 Annex B
Upstream PMMS parameters – NPar(3) coding – Octet 13

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex B upstream PMMS NPar(3)s – Octet 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 1 | x | x | x | x | x | Worst-case PMMS target margin (dB) (bits 5-1  1.0 dB – 10 dB) |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Worst-case PMMS target margin unspecified by terminal (values of bits 6-1 from 1 to 31 reserved for allocation by the ITU-T) |

Table 11.18.4.13/G.994.1 – Standard information field – G.991.2 Annex B
Upstream PMMS parameters – NPar(3) coding – Octet 14

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex B upstream PMMS NPar(3)s – Octet 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 1 | x | x | x | x | x | Current-condition PMMS target margin (dB) (bits 5-1  1.0 dB – 10 dB) |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | Current-condition PMMS target margin unspecified by terminal (values of bits 6-1 from 1 to 31 reserved for allocation by the ITU-T) |

Table 11.18.5.2/G.994.1 – Standard information field – G.991.2 Annex B
TPS-TC parameters – NPar(3) coding – Octet 3

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.991.2 Annex B TPS-TC parameter NPar(3)s – Octet 3 |
|  | x | x | x | x | x | x | x | 1 | Synchronous ISDN-BRA |
|  | x | x | x | x | x | x | 1 | x | PTM |
|  | x | x | x | x | x | 1 | x | x | STM with DSC |
|  | x | x | x | x | 1 | x | x | x | LAPV5 Enveloped POTS or ISDN |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |
| – |  |  |  |  |  |  |  |  |  |

Table 11.18.8/G.994.1 – Standard information field – G.991.2 Annex B
Dual Mode TPS-TC parameters – NPar(3) coding – Octet 1

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex B Dual Mode TPS-TC parameter Npar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | TPS-TCa data rate – n  64 kbit/s (1 to 36) |
|  | x | x | 1 | 1 | 1 | 1 | 1 | 1 | TPS-TCa data rate unspecified by terminal |

Table 11.18.8.1/G.994.1 – Standard information field – G.991.2 Annex B
Dual Mode TPS-TC parameters – NPar(3) coding – Octet 2

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex B Dual Mode TPS-TC parameter Npar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | 0 | 0 | 0 | x | x | x | TPS-TCa sub-data rate – i  8 kbit/s (0 to 7) |
|  | x | x | 1 | 1 | 1 | 1 | 1 | 1 | TPS-TCa sub-data rate unspecified by terminal (values of bits 6-1 from 8 to 62 reserved for allocation by the ITU-T) |

Table 11.18.8.4/G.994.1 – Standard information field – G.991.2 Annex B
Dual Mode TPS-TC parameters – NPar(3) coding – Octet 5

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex B Dual Mode TPS-TC parameter NPar(3)s – Octet 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Type 2 – TPS-TCa: Unaligned D2048U |
|  | x | x | x | x | x | x | 1 | x | Type 2 – TPS-TCa: Unaligned D2048S |
|  | x | x | x | x | x | 1 | x | x | Type 2 – TPS-TCa: Aligned D2048S/Fractional D2048S |
|  | x | x | x | x | 1 | x | x | x | Type 2 – TPS-TCa: Synchronous ISDN BRA |
|  | x | x | x | 1 | x | x | x | x | Type 2 – TPS-TCb: ATM |
|  | x | x | 1 | x | x | x | x | x | Type 2 – TPS-TCb: PTM |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |
| – |  |  |  |  |  |  |  |  |  |

Table 11.18.8.6/G.994.1 – Standard information field – G.991.2 Annex B
Dual Mode TPS-TC parameters – NPar(3) coding – Octet 7

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex B Dual Mode TPS-TC parameter NPar(3)s – Octet 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Type 1 – TPS-TCb: PTM |
|  | x | x | x | x | x | x | 1 | x | Type 1 – TPS-TCa: STM with DSC |
|  | x | x | x | x | x | 1 | x | x | Type 1 – TPS-TCa: LAPV5 Enveloped POTS or ISDN |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |
| – |  |  |  |  |  |  |  |  |  |

Table 11.18.8.7/G.994.1 – Standard information field – G.991.2 Annex B
Dual Mode TPS-TC parameters – NPar(3) coding – Octet 8

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex B Dual Mode TPS-TC parameter NPar(3)s – Octet 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | DRR Support: 01 DRR not supported 10 DRR supported, STU-C DRR master 11 DRR supported, STU-R DRR master |
|  | x | x | x | x | x | x |  |  | Lead Time (in frames) count = bits 6-2 (supported values = 1 to 15) |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | DRR Support and Lead Time unspecified by terminal |

Table 11.18.9/G.994.1 – Standard information field – G.991.2 Annex B
Multiple-Pair Operation parameters – NPar(3) coding – Octet 1

| Bits |  |  |  |  |  |  |  |  | G.991.2 Annex B Multiple-Pair Operation parameters NPar(3)s – Octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x |  |  |  |  | x | x | M-pair count (count = bits 2-1 + 1) |
|  | x | x | x | x | x | x |  |  | Reserved for allocation by the ITU-T |

Table 11.30/G.994.1 – Standard information field – G.992.3 Annex A SPar(2) coding – Octet 1

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.992.3 Annex A SPar(2)s – Octet 1 |
|  | x | x | x | x | x | x | x | 1 | Spectrum bounds upstream |
|  | x | x | x | x | x | x | 1 | x | Spectrum shaping upstream |
|  | x | x | x | x | x | 1 | x | x | Spectrum bounds downstream |
|  | x | x | x | x | 1 | x | x | x | Spectrum shaping downstream |
|  | x | x | x | 1 | x | x | x | x | Transmit signal images above the Nyquist frequency |
|  | x | x | 1 | x | x | x | x | x | Annex L Reach Extended PSD Masks |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.30.6/G.994.1 – Standard information field – G.992.3 Annex L
Reach Extended PSD Masks NPar(3) coding  – Octet 1

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.992.3 Annex L Reach Extended PSD Masks NPar(3)s – Octet 1 |
|  | x | x | x | x | x | x | x | 1 | Upstream Mask 1 supported |
|  | x | x | x | x | x | x | 1 | x | Upstream Mask 2 supported |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.30.6.1/G.994.1 – Standard information field – G.992.3 Annex L
Reach Extended PSD Masks NPar(3) coding  – Octet 2

| Bits |  |  |  |  |  |  |  |  | G.992.3 Annex L Reach Extended PSD Masks NPar(3)s – Octet 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | 1 | Downstream Non-Overlapped Mask supported |
|  | x | x | x | x | x | x | 1 | x | Downstream Overlapped Mask supported |
|  | x | x | x | x | x | 1 | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.36/G.994.1 – Standard information field – G.992.3 Annex J SPar(2) coding – Octet 1

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.992.3 Annex J SPar(2)s – Octet 1 |
|  | x | x | x | x | x | x | x | 1 | Spectrum bounds upstream |
|  | x | x | x | x | x | x | 1 | x | Spectrum shaping upstream |
|  | x | x | x | x | x | 1 | x | x | Spectrum bounds downstream |
|  | x | x | x | x | 1 | x | x | x | Spectrum shaping downstream |
|  | x | x | x | 1 | x | x | x | x | Transmit signal images above the Nyquist frequency |
|  | x | x | 1 | x | x | x | x | x | Submode PSD masks |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.36.6/G.994.1/G.994.1 – Standard information field – G.992.3 Annex J
Submode PSD Mask NPar(3) coding  – Octet 1

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.992.3 Annex J NPar(3)s – Octet 1 |
|  | x | x | x | x | x | x | x | 1 | ADLU-32 supported |
|  | x | x | x | x | x | x | 1 | x | ADLU-36 supported |
|  | x | x | x | x | x | 1 | x | x | ADLU-40 supported |
|  | x | x | x | x | 1 | x | x | x | ADLU-44 supported |
|  | x | x | x | 1 | x | x | x | x | ADLU-48 supported |
|  | x | x | 1 | x | x | x | x | x | ADLU-52 supported |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.36.6.1/G.994.1 – Standard information field – G.992.3 Annex J
Submode PSD mask NPar(3) coding – Octet 2

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.992.3 Annex J NPar(3)s – Octet 2 |
|  | x | x | x | x | x | x | x | 1 | ADLU-56 supported |
|  | x | x | x | x | x | x | 1 | x | ADLU-60 supported |
|  | x | x | x | x | x | 1 | x | x | ADLU-64 supported |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

The other SPar(2) and NPar(3) parameters for G.992.3 Annex J are identical to those of G.992.3 Annex A, except for the Annex L Reach Extended PSD Masks SPar(2) parameter in Table 11.30/G.994.1 and its underlying NPar(3) parameters in Tables 11.30.6 and 11.30.6.1/G.994.1, which are not valid for G.992.3 Annex M. Other than these parameters. Tables 11.30.0.1 through 11.30.44.3/G.994.1 shall be used for G.992.3 Annex J. Transmission of SPar(2) and NPar(3) octets for G.992.3 Annex J shall follow transmission of Table 11.35/G.994.1, and precede transmission of Table 11.37/G.994.1. Effectively, Tables 11.30.0.1 through 11.30.44.3/G.994.1 become renumbered to Tables 11.36.0.1 through 11.36.44.3 for G.992.3 Annex J.

Table 11.49/G.994.1 – Standard information field – G.992.3 Annex M NPar(2) coding

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.992.3 Annex M NPar(2)s |
|  | x | x | x | x | x | x | x | 1 | NTR |
|  | x | x | x | x | x | x | 1 | x | Short initialization |
|  | x | x | x | x | x | 1 | x | x | Diagnostics mode |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.50/G.994.1 – Standard information field – G.992.3 Annex M SPar(2) coding – Octet 1

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.992.3 Annex M SPar(2)s – Octet 1 |
|  | x | x | x | x | x | x | x | 1 | Spectrum bounds upstream |
|  | x | x | x | x | x | x | 1 | x | Spectrum shaping upstream |
|  | x | x | x | x | x | 1 | x | x | Spectrum bounds downstream |
|  | x | x | x | x | 1 | x | x | x | Spectrum shaping downstream |
|  | x | x | x | 1 | x | x | x | x | Transmit signal images above the Nyquist frequency |
|  | x | x | 1 | x | x | x | x | x | Submode PSD masks |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.50.6/G.994.1 – Standard information field – G.992.3 Annex M
NPar(3) Submode PSD Mask – Octet 1

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.992.3 Annex M NPar(3)s – Octet 1 |
|  | x | x | x | x | x | x | x | 1 | EU-32 supported |
|  | x | x | x | x | x | x | 1 | x | EU-36 supported |
|  | x | x | x | x | x | 1 | x | x | EU-40 supported |
|  | x | x | x | x | 1 | x | x | x | EU-44 supported |
|  | x | x | x | 1 | x | x | x | x | EU-48 supported |
|  | x | x | 1 | x | x | x | x | x | EU-52 supported |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.50.6.1/G.994.1 – Standard information field – G.992.3 Annex M
NPar(3) Submode PSD mask – Octet 2

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.992.3 Annex M NPar(3)s – Octet 2 |
|  | x | x | x | x | x | x | x | 1 | EU-56 supported |
|  | x | x | x | x | x | x | 1 | x | EU-60 supported |
|  | x | x | x | x | x | 1 | x | x | EU-64 supported |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

The other SPar(2) and NPar(3) parameters for G.992.3 Annex M are identical to those of G.992.3 Annex A, except for the Annex L Reach Extended PSD Masks SPar(2) parameter in Table 11.30/G.994.1 and its underlying NPar(3) parameters in Tables 11.30.6 and 11.30.6.1/G.994.1, which are not valid for G.992.3 Annex M. Other than these parameters, Tables 11.30.0.1 through 11.30.44.3/G.994.1 shall be used for G.992.3 Annex M. Transmission of SPar(2) and NPar(3) octets for G.992.3 Annex M shall follow transmission of Table 11.49/G.994.1, and precede transmission of Table 11.51/G.994.1. Effectively, Tables 11.30.0.1 through 11.30.44.3/G.994.1 become renumbered to Tables 11.50.0.1 through 11.50.44.3 for G.992.3 Annex M.

Table 11.51/G.994.1 – Standard information field – G.992.5 Annex J NPar(2) coding

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.992.5 Annex J NPar(2)s |
|  | x | x | x | x | x | x | x | 1 | NTR |
|  | x | x | x | x | x | x | 1 | x | Short initialization |
|  | x | x | x | x | x | 1 | x | x | Diagnostics mode |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.52/G.994.1 – Standard information field – G.992.5 Annex J SPar(2) coding – Octet 1

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.992.5 Annex J SPar(2)s – Octet 1 |
|  | x | x | x | x | x | x | x | 1 | Spectrum bounds upstream |
|  | x | x | x | x | x | x | 1 | x | Spectrum shaping upstream |
|  | x | x | x | x | x | 1 | x | x | Spectrum bounds downstream |
|  | x | x | x | x | 1 | x | x | x | Spectrum shaping downstream |
|  | x | x | x | 1 | x | x | x | x | Transmit signal images above the Nyquist frequency |
|  | x | x | 1 | x | x | x | x | x | Submode PSD masks |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.52.6/G.994.1 – Standard information field – G.992.5 Annex J
Submode PSD Mask NPar(3) coding  – Octet 1

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.992.5 Annex J NPar(3)s – Octet 1 |
|  | x | x | x | x | x | x | x | 1 | ADLU-32 supported |
|  | x | x | x | x | x | x | 1 | x | ADLU-36 supported |
|  | x | x | x | x | x | 1 | x | x | ADLU-40 supported |
|  | x | x | x | x | 1 | x | x | x | ADLU-44 supported |
|  | x | x | x | 1 | x | x | x | x | ADLU-48 supported |
|  | x | x | 1 | x | x | x | x | x | ADLU-52 supported |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.52.6.1/G.994.1 – Standard information field – G.992.5 Annex J
Submode PSD mask NPar(3) coding – Octet 2

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.992.5 Annex J NPar(3)s – Octet 2 |
|  | x | x | x | x | x | x | x | 1 | ADLU-56 supported |
|  | x | x | x | x | x | x | 1 | x | ADLU-60 supported |
|  | x | x | x | x | x | 1 | x | x | ADLU-64 supported |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

The other SPar(2) and NPar(3) parameters for G.992.5 Annex J are identical to those of G.992.3 Annex A, except for the Annex L Reach Extended PSD Masks SPar(2) parameter in Table 11.30/G.994.1 and its underlying NPar(3) parameters in Tables 11.30.6 and 11.30.6.1/G.994.1, which are not valid for G.992.5 Annex J. Other than these parameters, Tables 11.30.0.1 through 11.30.44.3/G.994.1 shall be used for G.992.5 Annex J. Transmission of SPar(2) and NPar(3) octets for G.992.5 Annex J shall follow transmission of Table 11.51/G.994.1, and precede transmission of Table 11.53/G.994.1. Effectively, Tables 11.30.0.1 through 11.30.44.3/G.994.1 become renumbered to Tables 11.52.0.1 through 11.52.44.3 for G.992.5 Annex J.

Table 11.57/G.994.1 – Standard information field – G.992.5 Annex M
NPar(2) coding

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.992.5 Annex M NPar(2)s |
|  | x | x | x | x | x | x | x | 1 | NTR |
|  | x | x | x | x | x | x | 1 | x | Short initialization |
|  | x | x | x | x | x | 1 | x | x | Diagnostics mode |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.58/G.994.1 – Standard information field – G.992.5 Annex M
SPar(2) coding – Octet 1

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.992.5 Annex M SPar(2)s – Octet 1 |
|  | x | x | x | x | x | x | x | 1 | Spectrum bounds upstream |
|  | x | x | x | x | x | x | 1 | x | Spectrum shaping upstream |
|  | x | x | x | x | x | 1 | x | x | Spectrum bounds downstream |
|  | x | x | x | x | 1 | x | x | x | Spectrum shaping downstream |
|  | x | x | x | 1 | x | x | x | x | Transmit signal images above the Nyquist frequency |
|  | x | x | 1 | x | x | x | x | x | Submode PSD masks |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.58.6/G.994.1 – Standard information field – G.992.5 Annex M
NPar(3) Submode PSD Mask – Octet 1

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.992.5 Annex M NPar(3)s – Octet 1 |
|  | x | x | x | x | x | x | x | 1 | EU-32 supported |
|  | x | x | x | x | x | x | 1 | x | EU-36 supported |
|  | x | x | x | x | x | 1 | x | x | EU-40 supported |
|  | x | x | x | x | 1 | x | x | x | EU-44 supported |
|  | x | x | x | 1 | x | x | x | x | EU-48 supported |
|  | x | x | 1 | x | x | x | x | x | EU-52 supported |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

Table 11.58.6.1/G.994.1 – Standard information field – G.992.5 Annex M
NPar(3) Submode PSD mask – Octet 2

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | G.992.5 Annex M NPar(3)s – Octet 1 |
|  | x | x | x | x | x | x | x | 1 | EU-56 supported |
|  | x | x | x | x | x | x | 1 | x | EU-60 supported |
|  | x | x | x | x | x | 1 | x | x | EU-64 supported |
|  | x | x | x | x | 1 | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |

The other SPar(2) and NPar(3) parameters for G.992.5 Annex M are identical to those of G.992.3 Annex A, except for the Annex L Reach Extended PSD Masks SPar(2) parameter in Table 11.30/G.994.1 and its underlying NPar(3) parameters in Tables 11.30.6 and 11.30.6.1/G.994.1, which are not valid for G.992.5 Annex M. Other than these parameters, Tables 11.30.0.1 through 11.30.44.3/G.994.1 shall be used for G.992.5 Annex M. Transmission of SPar(2) and NPar(3) octets for G.992.5 Annex M shall follow transmission of Table 11.57/G.994.1. Effectively, Tables 11.30.0.1 through 11.30.44.3/G.994.1 become renumbered to Tables 11.58.0.1 through 11.58.44.3 for G.992.5 Annex M.

# 3)	Table 11.35

Delete the text immediately following Table 11.35.

# 4)	Table 11.47

Revise the text immediately following Table 11.47 as follows:

The SPar(2) and NPar(3) parameters for G.992.5 Annex I are identical to those of G.992.3 Annex A, except for the Annex L Reach Extended PSD Masks SPar(2) parameter in Table 11.30/G.994.1 and its underlying NPar(3) parameters in Tables 11.30.6 and 11.30.6.1/G.994.1, which are not valid for G.992.5 Annex I. Other than these parameters, Tables 11.30 through 11.30.44.3/G.994.1 shall be used for G.992.5 Annex I. Transmission of SPar(2) and NPar(3) octets for G.992.5 Annex I shall follow transmission of Table 11.47/G.994.1, and precede transmission of Table 11.49/G.994.1. Effectively, Tables 11.30 through 11.30.44.3/G.994.1 become renumbered to Tables 11.48 through 11.48.44.3 for G.992.5 Annex I.

# 5)	Clause 3, Definitions

Add the following definition to clause 3 and renumber all subsequent definitions:

3.1	capabilities: The "enabled" (= supported) set of functions, not necessarily the complete set of functions implemented in the unit.

# 6)	New clause 13

Add new clause 13 as follows:

# 13	Management Information Base (MIB) elements

This clause specifies the physical layer management for HSTU systems. It specifies Network Management elements content for configuration management.

## 13.1	Configuration parameters

### 13.1.1	GHS A43 Toneset Maximum PSD level in downstream (GHS_A43_MAXPSDds)

The parameter1 GHS_A43_MAXPSDds is defined as the maximum transmit PSD level for each individual G.hs tone of the A43 toneset in the downstream direction. The PSD level (in dBm/Hz) is calculated as the tone power averaged over a 4.3125 kHz bandwidth. The mandatory range to be supported by the HSTU-C is from –71.5 to –40 dBm/Hz, with 0.5 dB steps. If the value is set to the value –99, then the HSTU-C shall not transmit this toneset.

The value of the Attenuation in G.994.1 Transmit Power per Carrier for carrier set A43 as conveyed in the NPar(2) in Table 9.17/G.994.1 shall comply with the following constraint:

−3.65−Attenuation−36.35≤GHSA43MAXPSDds

### 13.1.2	GHS A43c Toneset Maximum PSD level in downstream (GHS_A43c_MAXPSDds)

The parameter1 GHS_A43c_MAXPSDds is defined as the maximum transmit PSD level for each individual G.hs tone of the A43c toneset in the downstream direction. The PSD level (in dBm/Hz) is calculated as the tone power averaged over a 4.3125 kHz bandwidth. The mandatory range to be supported by the HSTU-C is from –71.5 to –40 dBm/Hz, with 0.5 dB steps. If the value is set to the value –99, then the HSTU-C shall not transmit this toneset.

The value of the Attenuation in G.994.1 Transmit Power per Carrier for carrier set A43c as conveyed in the NPar(2) in Table 9.17/G.994.1 shall comply with the following constraint:

−3.65−Attenuation−36.35≤GHSA43cMAXPSDds

# 7)	Table 11.31

Revise the text immediately following Table 11.31 as follows:

The SPar(2) and NPar(3) parameters for G.992.3 Annex B are identical to those of G.992.3 Annex A, except for the Annex L Reach Extended PSD Masks SPar(2) parameter in Table 11.30/G.994.1 and its underlying NPar(3) parameters in Tables 11.30.6 and 11.30.6.1/G.994.1, which are not valid for G.992.3 Annex B. Other than these parameters, Tables 11.30 through 11.30.44.3/G.994.1 shall be used for G.992.3 Annex B. Transmission of SPar(2) and NPar(3) octets for G.992.3 Annex B shall follow transmission of Table 11.31/G.994.1, and precede transmission of Table 11.33/G.994.1. Effectively, Tables 11.30 through 11.30.44.3/G.994.1 become renumbered to Tables 11.32 through 11.32.44.3 for G.992.3 Annex B.

# 8)	Table 11.33

Revise the text immediately following Table 11.33 as follows:

The SPar(2) and NPar(3) parameters for G.992.3 Annex I are identical to those of G.992.3 Annex A, except for the Annex L Reach Extended PSD Masks SPar(2) parameter in Table 11.30/G.994.1 and its underlying NPar(3) parameters in Tables 11.30.6 and 11.30.6.1/G.994.1, which are not valid for G.992.3 Annex I. Other than these parameters, Tables 11.30 through 11.30.44.3/G.994.1 shall be used for G.992.3 Annex I. Transmission of SPar(2) and NPar(3) octets for G.992.3 Annex I shall follow transmission of Table 11.33/G.994.1, and precede transmission of Table 11.35/G.994.1. Effectively, Tables 11.30 through 11.30.44.3/G.994.1 become renumbered to Tables 11.34 through 11.34.44.3 for G.992.3 Annex I.

# 9)	Table 11.35

Revise the text immediately following Table 11.35 as follows:

The SPar(2) and NPar(3) parameters for G.992.3 Annex J are identical to those of G.992.3 Annex A, except for the Annex L Reach Extended PSD Masks SPar(2) parameter in Table 11.30/G.994.1 and its underlying NPar(3) parameters in Tables 11.30.6 and 11.30.6.1/G.994.1, which are not valid for G.992.3 Annex J. Other than these parameters, Tables 11.30 through 11.30.44.3/G.994.1 shall be used for G.992.3 Annex J. Transmission of SPar(2) and NPar(3) octets for G.992.3 Annex J shall follow transmission of Table 11.35/G.994.1, and precede transmission of Table 11.37/G.994.1. Effectively, Tables 11.30 through 11.30.44.3/G.994.1 become renumbered to Tables 11.36 through 11.36.44.3 for G.992.3 Annex J.

# 10)	Table 11.37

Revise the text immediately following Table 11.37 as follows:

The SPar(2) and NPar(3) parameters for G.992.4 Annex A are identical to those of G.992.3 Annex A, except for the Annex L Reach Extended PSD Masks SPar(2) parameter in Table 11.30/G.994.1 and its underlying NPar(3) parameters in Tables 11.30.6 and 11.30.6.1/G.994.1, which are not valid for G.992.4 Annex A. Other than these parameters, Tables 11.30 through 11.30.44.3/G.994.1 shall be used for G.992.4 Annex A. Transmission of SPar(2) and NPar(3) octets for G.992.4 Annex A shall follow transmission of Table 11.37/G.994.1, and precede transmission of Table 11.39/G.994.1. Effectively, Tables 11.30 through 11.30.44.3/G.994.1 become renumbered to Tables 11.38 through 11.38.44.3 for G.992.4 Annex A.

# 11)	Table 11.39

Revise the text immediately following Table 11.39 as follows:

The SPar(2) and NPar(3) parameters for G.992.4 Annex I are identical to those of G.992.3 Annex A, except for the Annex L Reach Extended PSD Masks SPar(2) parameter in Table 11.30/G.994.1 and its underlying NPar(3) parameters in Tables 11.30.6 and 11.30.6.1/G.994.1, which are not valid for G.992.4 Annex I. Other than these parameters, Tables 11.30 through 11.30.44.3/G.994.1 shall be used for G.992.4 Annex I. Transmission of SPar(2) and NPar(3) octets for G.992.4 Annex I shall follow transmission of Table 11.39/G.994.1. Effectively, Tables 11.30 through 11.30.44.3/G.994.1 become renumbered to Tables 11.40 through 11.40.44.3 for G.992.4 Annex I.

# 12)	Table 11.43

Revise the text immediately following Table 11.43 as follows:

The SPar(2) and NPar(3) parameters for G.992.5 Annex A are identical to those of G.992.3 Annex A, except for the Annex L Reach Extended PSD Masks SPar(2) parameter in Table 11.30/G.994.1 and its underlying NPar(3) parameters in Tables 11.30.6 and 11.30.6.1/G.994.1, which are not valid for G.992.5 Annex A. Other than these parameters, Tables 11.30 through 11.30.44.3/G.994.1 shall be used for G.992.5 Annex A. Transmission of SPar(2) and NPar(3) octets for G.992.5 Annex A shall follow transmission of Table 11.43/G.994.1, and precede transmission of Table 11.45/G.994.1. Effectively, Tables 11.30 through 11.30.44.3/G.994.1 become renumbered to Tables 11.44 through 11.44.44.3/G.994.1 for G.992.5 Annex A.

# 13)	Table 11.45

Revise the text immediately following Table 11.45 as follows:

The SPar(2) and NPar(3) parameters for G.992.5 Annex B are identical to those of G.992.3 Annex A, except for the Annex L Reach Extended PSD Masks SPar(2) parameter in Table 11.30/G.994.1 and its underlying NPar(3) parameters in Tables 11.30.6 and 11.30.6.1/G.994.1, which are not valid for G.992.5 Annex B. Other than these parameters, Tables 11.30 through 11.30.44.3/G.994.1 shall be used for G.992.5 Annex B. Transmission of SPar(2) and NPar(3) octets for G.992.5 Annex B shall follow transmission of Table 11.45/G.994.1, and precede transmission of Table 11.47/G.994.1. Effectively, Tables 11.30 through 11.30.44.3/G.994.1 become renumbered to Tables 11.46 through 11.46.44.3 for G.992.5 Annex B.

# 14)	Table 9.xx

Revise existing and add new Tables 9.x.x as follows:

Table 9.0.2/G.994.1 – Identification field – SPar(1) coding – Octet 3

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | SPar(1)s – Octet 3 |
|  | x | x | x | x | x | x | x | 1 | Relative power level/carrier for upstream carrier set A4 (Note) |
|  | x | x | x | x | x | x | 1 | x | Relative power level/carrier for downstream carrier set A4 (Note) |
|  | x | x | x | x | x | 1 | x | x | Relative power level/carrier for upstream carrier set A43c (Note) |
|  | x | x | x | x | 1 | x | x | x | Relative power level/carrier for downstream carrier set A43c (Note) |
|  | x | x | x | 1 | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | x | 1 | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | 1 | x | x | x | x | x | x | Reserved for allocation by the ITU-T |
|  | x | 0 | 0 | 0 | 0 | 0 | 0 | 0 | No parameters in this octet |
| NOTE – The relative power level/carrier reported in a CLR, CL, MP, or MS message indicates the level used during the current G.994.1 session, including the start-up and cleardown procedures. It does not imply any requirements on the transmit power in this or future sessions. |  |  |  |  |  |  |  |  |  |

Table 9.33/G.994.1 – Identification field – Relative power level/carrier for
upstream carrier set A43c – NPar(2) coding

| Bits |  |  |  |  |  |  |  |  | Relative power level/carrier for upstream carrier set A43c Npar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Attenuation in G.994.1 Transmit Power per Carrier Relative to Maximum Power (bits 6-1 × 0.5 dB) for upstream carrier set A43c (Note). |
| NOTE – All carriers in the carrier set shall be transmitted at the same power level. |  |  |  |  |  |  |  |  |  |

Table 9.35/G.994.1 – Identification field – Relative power level/carrier for
downstream carrier set A43c – NPar(2) coding

| Bits |  |  |  |  |  |  |  |  | Relative power level/carrier for downstream carrier set A43c Npar(2)s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | x | x | x | x | x | x | x | x | Attenuation in G.994.1 Transmit Power per Carrier Relative to Maximum Power (bits 6-1 × 0.5 dB) for downstream carrier set A43c (Note). |
| NOTE – All carriers in the carrier set shall be transmitted at the same power level. |  |  |  |  |  |  |  |  |  |

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure, Internet protocol aspects and Next Generation Networks |
| Series Z | Languages and general software aspects for telecommunication systems |

*	T1 standards are maintained since November 2003 by ATIS.

1	It is expected that HSTU-Cs that are colocated will use the same parameter setting.