|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.996.1 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (02/2001) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital sections and digital line system – Access networks |  |  |  |
|  | Test procedures for digital subscriber line (DSL) transceivers |  |  |  |
|  | ITU-T Recommendation G.996.1 (Formerly CCITT Recommendation) |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| General | G.900–G.909 |
| Parameters for optical fibre cable systems | G.910–G.919 |
| Digital sections at hierarchical bit rates based on a bit rate of 2048 kbit/s | G.920–G.929 |
| Digital line transmission systems on cable at non-hierarchical bit rates | G.930–G.939 |
| Digital line systems provided by FDM transmission bearers | G.940–G.949 |
| Digital line systems | G.950–G.959 |
| Digital section and digital transmission systems for customer access to ISDN | G.960–G.969 |
| Optical fibre submarine cable systems | G.970–G.979 |
| Optical line systems for local and access networks | G.980–G.989 |
| Access networks | G.990–G.999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.996.1 Test procedures for digital subscriber line (DSL) transceivers

- Summary This Recommendation describes the testing procedures for ITU-T Digital Subscriber Line (DSL) Recommendations. The testing procedures described herein include methods for testing DSL transceivers in the presence of crosstalk from other services, Radio Frequency ingress, impulse noise and POTS signalling. Test loops and in-home wiring models are specified for different regions of the world for use during DSL performance testing. Other DSL Recommendations reference this Recommendation for testing procedures and configurations. This Recommendation does not specify performance requirements for these other Recommendations; it only specifies the procedures for measuring the performance requirements for a particular Recommendation.

- Source ITU-T Recommendation G.996.1 was revised by ITU-T Study Group 15 (2001-2004) and approved under the WTSA Resolution 1 procedure on 9 February 2001.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2002

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Definitions		1

4	Abbreviations		1

5	Test procedures and laboratory setup		2

5.1	DSL operation with splitters		3

5.1.1	Laboratory setup		3

5.1.2	Crosstalk and RFI test		5

5.1.3	Impulse noise test		8

5.1.4	POTS interference test		8

5.2	DSL operation without a splitter at the ATU-R		9

5.2.1	Laboratory setup		9

5.2.2	Crosstalk and RFI test		11

5.2.3	Impulse noise test		11

5.2.4	POTS interference test		11

5.3	POTS QoS testing		12

5.3.1	Telephone circuitry		12

5.3.2	Test conditions		13

5.3.3	Summary of test procedure		14

6	Test loops		14

6.1	Local loop configurations		14

6.1.1	North American test loops		14

6.1.2	European local test loops		17

6.1.3	Test loops in an environment coexisting with TCM-ISDN DSL		23

6.2	In-home wiring models		31

6.2.1	In-home wiring model #1		31

6.2.2	In-home wiring model #2: Daisy Chain 4-conductor, 24 AWG, non-paired station wire		32

6.2.3	In-home wiring model #3: Star – 152.4 m (500 ft), 4-pair Category 3 wire		33

6.2.4	In-home wiring model for TCM-ISDN DSL environment		34

7	Power spectral density of crosstalk disturbers		36

7.1	Simulated DSL PSD and induced NEXT		36

7.2	Simulated HDSL PSD and induced NEXT		37

7.3	Simulated T1 line PSD and induced NEXT		38

Page

7.4	Simulated G.992.1 downstream PSD and induced NEXT and FEXT		40

7.4.1	FEXT		40

7.4.2	NEXT		41

7.5	Simulated G.992.1 upstream PSD and induced NEXT and FEXT		42

7.5.1	FEXT		43

7.5.2	NEXT		43

7.6	Simulated G.992.2 downstream NEXT and FEXT		44

7.6.1	FEXT		44

7.6.2	NEXT		45

7.7	Simulated G.992.2 upstream NEXT and FEXT		46

7.7.1	FEXT		46

7.7.2	NEXT		47

7.8	ETSI A, ETSI B and Euro-K crosstalk		48

7.9	Power spectral density of crosstalk disturbers in an environment coexisting with TCM-ISDN DSL		50

7.9.1	Crosstalk disturber type		50

7.9.2	Crosstalk test parameters		51

7.9.3	Simulated TCM-ISDN DSL PSD and induced NEXT and FEXT		52

7.9.4	Simulated HDSL PSD and induced NEXT		55

7.9.5	Simulated ADSL downstream PSD and induced NEXT and FEXT		57

7.9.6	Simulated ADSL upstream PSD and induced NEXT and FEXT		60

8	Characteristics of impulse noise waveforms		62

9	Phone model structures for testing DSL systems without a splitter at the ATU-R		70

9.1	Off-hook phone model #1		70

9.2	Off-hook phone model #2		71

9.3	On-hook phone model		72

Annex A – Primary constants of polyethylene-insulated cable (PIC)		75

ITU-T Recommendation G.996.1

Test procedures for digital subscriber line (DSL) transceivers

# 1	Scope

This Recommendation describes the testing procedures for ITU-T DSL Recommendations. The testing procedures described in this Recommendation include methods for testing DSL transceivers in the presence of crosstalk from other services, Radio Frequency ingress, impulse noise and POTS signalling. Test loops and in-home wiring models are specified for different regions of the world for use during DSL performance testing. Other DSL Recommendations reference this Recommendation for testing procedures and configurations. This Recommendation does not specify performance requirements for these other Recommendations. This Recommendation only specifies the procedures for measuring the performance requirements for a particular Recommendation.

An overview of Digital Subscriber Line (DSL) transceivers can be found in ITU-T G.995.1.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

–	ITU-T G.992.1 (1999), Asymmetric digital subscriber line (ADSL) transceivers.

–	ITU-T G.992.2 (1999), Splitterless asymmetric digital subscriber line (ADSL) transceivers.

# 3	Definitions

This Recommendation defines the following terms:

3.1	downstream: ATU-C to ATU-R direction.

3.2	upstream: ATU-R to ATU-C direction.

3.3	splitter: A filter that separates the high frequency signals (DSL) from the voiceband signals.

3.4	voiceband: The frequency band from 0 to 4 kHz.

3.5	voiceband services: POTS and all data services that use the voiceband.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

ADC	Analogue-to-Digital Converter

ADSL	Asymmetric Digital Subscriber Line

ATU-C	ADSL Transceiver Unit, Central office end

ATU-R	ADSL Transceiver Unit, Remote terminal end

BER	Bit Error Rate

CO	Central Office

DAC	Digital-to-Analogue Converter

DMT	Discrete multitone

DSL	Digital Subscriber Line

ES	Errored Seconds

FEXT	Far-End crosstalk

kbit/s	kilobits per second

NEXT	Near-End crosstalk

POTS	Plain Old Telephone Service

PSD	Power Spectral Density

QoS	Quality of Service

RF	Radio Frequency

RFI	Radio Frequency Ingress

RMS	Root-Mean-Square

RT	Remote Terminal

exclusive-or; modulo-2 addition

xDSL	Any of the various types of Digital Subscriber Lines (DSL)

XT	Crosstalk

xTU-C	xDSL Transceiver Unit – Central office end

xTU-R	xDSL Transceiver Unit – Remote Terminal end

# 5	Test procedures and laboratory setup

The methods in this Recommendation test DSL system transmission performance. These laboratory methods evaluate a system's ability to minimize digital bit errors caused by interference from:

–	crosstalk coupling from other systems;

–	differential and common mode Radio Frequency (RF) ingress

–	background noise;

–	impulse noise;

–	POTS signalling.

These potential sources of impairment are simulated in a laboratory setup that includes test loops, test sets, and interference injection equipment, as well as the test system itself.

The crosstalk and impulse noise interfering signals are simulations that are derived from a consideration of real loop conditions and measurements. The test procedure is to inject the interference into the test loops and measure the effect on system performance by a bit error test simultaneously run on the system information channels.

For crosstalk an initial, or reference, power level for the interference represents the expected worst case. If the interference power can be increased without exceeding a specified error threshold, the system has a positive performance margin. Performance margin, expressed in dB, is the difference between the interference level at which the error threshold is reached, and the reference (or 0 dB) level.

In the case of impulse noise, an increasing interference level is similarly applied up to the error threshold, and the estimated performance is computed from this information. Because the impulse noise characteristics of the loop plant are not completely understood, the estimation method is based on measured data from several sites. The estimated number of error-causing impulses is compared to a specified errored-seconds (ES) criterion. The test procedure makes separate determinations of crosstalk margins and impulse error thresholds, although a background crosstalk interference is applied during impulse tests.

The POTS measurement uses a number of signalling and alerting activities done with real phones and CO lines, and also has crosstalk interference present. The BER test sets check for ES or a BER threshold.

## 5.1	DSL operation with splitters

### 5.1.1	Laboratory setup

Figure 1 shows the test setup for measuring performance margins for the downstream channel in DSL systems with splitters. Figure 2 shows the test setup for measuring performance margins for the upstream channel in DSL systems with splitters. The test setup permits differential and common mode signals to be superimposed on a working link, to simulate ingress situations. The test system consists of a central office transceiver (ATU-C), a remote end transceiver (ATU-R), and associated splitters. The two transceivers are connected together by the test loop. Crosstalk, impulses and Radio Frequency ingress (RFI) are injected at the ATU-R for downstream tests, and at the ATU-C for upstream channel tests.

For downstream testing, as shown in Figure 1, pseudo-random binary data from the transmitter of the bit error rate (BER) test set is presented at the downstream channel input of the ATU-C. The downstream channel output from the ATU-R is connected to the receiver of the same or a similar BER test set. The test set measures BER or errored seconds (ES), as needed. Similar error testing is done in the upstream direction at the rate needed for the particular system under test (see Figure 2).

A telephone set is connected to the telephone jack of the splitter at the ATU-R, and a working telephone line circuit is connected to the telephone jack on the splitter at the ATU-C.

Figure 1/G.996.1 – Laboratory test setup for measuring performance margins 
for the downstream channel with splitters

Figure 2/G.996.1 – Laboratory test setup for measuring performance margins 
for the upstream channel with splitters

The test set is essentially transparent in both directions between the transformers T1 and T2, so a normal working connection can be set up in the presence or absence of ingress.

T1 is a precision balun transformer. This minimizes the effects of any ingress that may be present. Note, however, that it will not compensate for pickup due to a poorly balanced facility connected to this port. The 50-ohm unbalanced signal from T1 passes via T2 to the receiving modem. Crosstalk and voice signals and the differential portion of the ingress signal are applied via hi-Z attenuators to the 50-ohm link between the transformers. T2 is a precision balun transformer, so it does not contribute significant imbalance at the receiving modem. Any imbalance at this port will be due to the modem itself.

The common mode ingress signal is applied via linear power amplifier A1 and resistor Rcm to the centre tap of T2. Amplifier A1 must be capable of producing the required signal level while keeping any resulting differential intermodulation products that fall in-band 113 dBm in any channel (this is equivalent to –150 dBm/Hz across 4.3125 kHz). Assuming that the modem under test has a differential balance of 35 dB, to make the resulting differential component from the 0 dBm common mode signal be 113 dBm in any channel requires the common mode intermodulation products at the centre tap of T2 to be –78 dBm. Filtering at the amplifier output may be used to achieve this.

Rcm is included so that the differential longitudinal of the facility is properly modelled. This is necessary because this impedance, together with the common-mode input impedance of the modem under test, can affect the test result. A value of 200 ohms is recommended for Rcm. It is assumed here that the amplifier has negligible output impedance; if it does not, the circuit can be modified accordingly. Similarly, any filter inserted at the amplifier output should be sufficiently transparent within the pass band that this impedance is maintained.

No phase alignment between the common mode and differential signals is required. It is possible that in some modems that a specific alignment may be beneficial. By testing in two alignments differing by 90 degrees this advantage could be removed. However, this would double the test time. By not specifying the alignment it is intended that modems will by designed for all possible alignments, as in the field no specific alignment can be assumed.

Required attributes of test equipment

•	The Thevenin impedance of all differential noise-coupling circuits connected to the test loop shall be greater than 4000 ohms referred to a 100-ohm impedance point.

•	Common Mode Source Impedance = 200 ohms.

•	The attenuation of the combining circuit must be correctly compensated for in the signal and interference paths. There is an advantage to a minimized loss in the signal path, as this flat loss would need to be removed from the line model loss.

•	The test setup should have a minimum balance of 60 dB. This is 20 dB better than the specified ratio of common mode to differential of 40 dB.

The test circuit must not create excessive spurious products that fall within the band of the test signal. The specification of –113 dBm in a 4.3125 kHz window corresponds to –150 dBm/Hz, and should be sufficient not to bias the tests as it is 10 dB below the white noise floor.

### 5.1.2	Crosstalk and RFI test

Crosstalk and RFI testing is performed to evaluate the performance of DSL systems in the presence of crosstalk from other services in the loop and RF ingress.

#### 5.1.2.1	Crosstalk and RFI injection

One method for injecting impairment is shown in Figure 3 using the crosstalk impairment as an example.

Figure 3/G.996.1 – High impedance crosstalk injection circuit

The method for calibrating the impairment levels is shown in Figure 3a. Both modems and the test loops have been removed and 100-ohm loads put in their place. The differential impairments are adjusted until their value is achieved across the 100-ohm load that replaces the receiver. The common-mode levels refer to the reading on a 50-ohm instrument connected at VcmOC (note that when this technique is used the definition of 0 dBm is 223.6 mVrms). This does not constitute a double termination as the 100-ohm impedance across T2 only creates a differential load. If this measurement is made with a high impedance instrument the levels would be 12 dB higher.

It should be noted that the levels that appear during the actual test may vary from the calibration levels as the modem differential and common mode impedances may vary widely from the 100 ohms and 50 ohms used for calibration.

The actual level of the differential and common mode impairments with the modems present will be a function of the common mode and differential impedances of the modems and test loops.

Figure 3a/G.996.1 – Calibration circuit for impairments

Crosstalk testing

The simulated XT should ideally have the power and spectral density defined by the equations for NEXT and FEXT in clause 7, "Power spectral density of crosstalk disturbers." It is acknowledged, however, that if the method of generating the simulated XT is similar to that shown in Figure 3, then its accuracy will depend on the design of the filter used to shape the white noise. Therefore, a calculated XT PSD may be defined for which a tolerance on f0 of 2% is allowed at each null. Then the accuracy of the simulated XT shall be within 1 dB of the calculated XT for all frequencies at which the calculated value is less than 45 dB below the peak value. The total power of the simulated XT shall be within 0.5 dB of the specified value using the same calibration termination.

The simulated XT PSD shall be verified using the calibration termination shown in Figure 3 and a selective voltmeter or true RMS metre with a bandwidth of approximately 3 kHz. For DSL and HDSL crosstalk the circuit shall be calibrated for 1.3 dB less crosstalk than specified in clause 7, in order to compensate for the use of 100-ohm terminations instead of 135 ohms.

Care should be taken when specifying the white noise generator in Figure 3; consideration should be given to the following factors:

–	The probability distribution of the peak amplitude: The noise shall be Gaussian within all frequency bands.

–	Crest factor: The crest factor is an indication of the number of standard deviations to which the noise follows a Gaussian distribution; the required minimum crest factor is 5.

–	The frequency spectrum: If the noise is generated using digital methods the sequence repetition rate will affect the correlation of the samples, and hence the frequency spectrum.

–	The noise BW shall be at least from 12 kHz to 2.208 MHz.

##### 5.1.2.1.1	Crosstalk injection for TCM-ISDN environment

For testing transmission performance of an ADSL system conforming to Annex C/G.992.1 and Annex C/G.992.2 (ADSL operating in an environment coexisting with TCM-ISDN DSL), the calibrated simulated TCM-ISDN DSL NEXT PSD and TCM-ISDN DSL FEXT PSD defined in clause 7 shall be alternately injected in burst mode as specified in Figure 4, which is the same as the TCM-ISDN DSL signal transceiver scheme. The value of x in the figure may be fixed during testing transmission performance.

The specified error threshold and minimum performance margin shall be met for an arbitrary value of x in the range of 6  x  40.

Figure 4/G.996.1 – TCM-ISDN DSL crosstalk injection method

#### 5.1.2.2	Crosstalk test and RFI method

Before testing, the DSL units are trained with the required crosstalk and RFI interference present. The simulated crosstalk power and RFI is injected at the appropriate reference level. This power level is considered the 0 dB margin level for that type and number of disturbers. For example, the 0 dB margin level for 24-disturber DSL NEXT is 52.6 dBm. Margin measurements are made by changing, in whole dB steps, the power level of the crosstalk and RFI injected at the transceiver and monitoring the BER over the test loops. A tested system has positive margin for a given type of crosstalk and RFI on a given loop if the system was able to operate at the specified BER with injected crosstalk power greater than the 0 dB margin level. If the simulated crosstalk and RFI are summed with background noise to obtain the simulated interference, the power level of the background noise is increased with the power level of the crosstalk and RFI interference during margin testing.

Note that for RFI testing, Amateur Radio interference is not injected during training. Amateur radio interference is only injected at full power level during margin testing.

The criteria for margin level determination shall include a check that the DSL unit can train at the margin level.

The minimum testing times to determine BERs with 95% confidence are shown in Table 1.

Table 1/G.996.1 – Minimum test time for crosstalk

| Bit rate | Minimum test time |
| --- | --- |
| Above 6 Mbit/s | 100 s |
| 1.544 Mbit/s to 6 Mbit/s | 500 s |
| Less than 1.544 Mbit/s | 20 min |

### 5.1.3	Impulse noise test

Impulse noise testing is performed to evaluate the performance of DSL systems in the presence of impulse noise.

#### 5.1.3.1	Impulse noise injection

The same coupling circuit as in 5.1.2.1 is used for impulse noise injection. The amplitude level of the impulses may be measured with an oscilloscope.

#### 5.1.3.2	Impulse noise test method

Before testing, the DSL units are trained with the required crosstalk interference. The test procedure consists of injecting the selected impulse waveform at varying amplitude levels and random phase. At each level the impulse is applied 15 times with a spacing of at least one second while an error measurement is made on the DSL channels. The amplitude (ue) in millivolts at which half the impulses cause an error is determined for each wave form.

Using the above amplitude determinations, the following equation gives the estimated probability that a second will be errored:

E  0.0037 P(u  ue1)  0.0208 P(u  ue2)

where:

P(u>ue)=25ue2, for 5 mV=ue=40 mV

P(u> ue)=0.625ue , for ue>40 mV

ue1	refers to waveform 1

ue2	refers to waveform 2

The resulting probability of errored seconds (ES) value shall be less than 0.14%.

### 5.1.4	POTS interference test

POTS interference test is performed to evaluate the performance of DSL systems in the presence of POTS signalling.

#### 5.1.4.1	POTS interference injection

The interference due to POTS service on the same line is generated by use of actual telephones and central office circuits connected in the normal way to the system under test. The following POTS signalling and alerting activities shall be performed:

–	call phone at ATU-R and allow to ring 25 times;

–	pick up ringing phone at ATU-R, 25 times;

–	perform off-hook and on-hook activity on phone at ATU-R, 25 times;

–	perform pulse and tone dialling.

#### 5.1.4.2	POTS interference test method

Before testing, the DSL units are trained with the required crosstalk interference. Signalling disturbances are created through use of the CO line connected to the splitter at the ATU-C, and the telephone set connected to the telephone jack of the splitter at the ATU-R. During these activities the DSL channels shall be monitored while noting any test conditions that cause errored seconds.

## 5.2	DSL operation without a splitter at the ATU-R

### 5.2.1	Laboratory setup

Figure 5 shows the test setup for measuring performance margins for the downstream channel in DSL systems without a splitter at the ATU-R. Figure 6 shows the test setup for measuring performance margins for the upstream channel in DSL systems without a splitter at the ATU-R. The test setup permits differential and common mode signals to be superimposed on a working link, to simulate ingress situations. The test system consists of a central office transceiver (ATU-C), a remote end transceiver (ATU-R), and splitter at the ATU-C. The two transceivers are connected together by the test loop and the in-home wiring model. Crosstalk, impulses and RFI are injected at the ATU-R for downstream tests, and at the ATU-C for upstream channel tests.

For downstream testing, pseudo-random binary data from the transmitter of the bit error rate (BER) test set is presented at the downstream channel input of the ATU-C. The downstream channel output from the ATU-R is connected to the receiver of the same or a similar BER test set. The test set measures BER or errored seconds (ES), as needed. Similar error testing is done in the upstream direction both at the rates needed for the particular system under test.

A working telephone line circuit is connected to the telephone jack on the splitter at the ATU-C. At the ATU-R, telephones may be added places in the in-home wiring model.

Figure 5/G.996.1 – Laboratory test setup for measuring performance margins 
for the downstream channel without a splitter at the ATU-R

Figure 6/G.996.1 – Laboratory test setup for measuring performance margins 
for the upstream channel without a splitter at the ATU-R

The test set is essentially transparent in both directions between the transformers T1 and T2, so a normal working connection can be set up in the presence or absence of ingress.

T1 is a precision balun transformer. This minimizes the effects of any ingress that may be present. Note, however, that it will not compensate for pickup due to a poorly balanced facility connected to this port. The 50-ohm unbalanced signal from T1 passes via T2 to the receiving modem. Crosstalk and voice signals and the differential portion of the ingress signal are applied via hi-Z attenuators to the 50-ohm link between the transformers. T2 is a precision balun transformer, so it does not contribute significant imbalance at the receiving modem. Any imbalance at this port will be due to the modem itself.

The common mode ingress signal is applied via linear power amplifier A1 and resistor Rcm to the centre tap of T2. Amplifier A1 must be capable of producing the required signal level while keeping any resulting differential intermodulation products that fall in-band 113 dBm in any channel (this is equivalent to –150 dBm/Hz across 4.3125 kHz). Assuming that the modem under test has a differential balance of 35 dB, to make the resulting differential component from the 0 dBm common mode signal be 113 dBm in any channel requires the common mode intermodulation products at the centre tap of T2 to be –78 dBm. Filtering at the amplifier output may be used to achieve this.

Rcm is included so that the longitudinal impedance of the facility is properly modelled. This is necessary because this impedance, together with the common-mode input impedance of the modem under test, can affect the test result. A value of 200 ohms is suggested for Rcm. It is assumed here that the amplifier has negligible output impedance; if it does not, the circuit can be modified accordingly. Similarly, any filter inserted at the amplifier output should be sufficiently transparent within the pass band that this impedance is maintained.

No phase alignment between the common mode and differential signals is proposed. It is possible that in some modems a specific alignment may be beneficial. By testing in two alignments differing by 90 degrees this advantage could be removed. However, this would double the test time. By not specifying the alignment it is intended that modems will by designed for all possible alignments, as in the field no specific alignment can be assumed.

Required attributes of test equipment

•	The Thevenin impedance of all differential noise-coupling circuits connected to the test loop shall be greater than 4000 ohms referred to a 100-ohm impedance point.

•	Common Mode Source Impedance = 200 ohms.

•	The attenuation of the combining circuit must be correctly compensated for in the signal and interference paths. There is an advantage to a minimized loss in the signal path, as this flat loss would need to be removed from the line model loss.

•	The test setup should have a minimum balance of 60 dB. This is 20 dB better than the specified ratio of common mode to differential of 40 dB.

The test circuit must not create excessive spurious products that fall within the band of the test signal. The specification of –113 dBm in a 4.3125 kHz window corresponds to –150 dBm/Hz, and should be sufficient not to bias the tests as it is 10 dB below the white noise floor.

### 5.2.2	Crosstalk and RFI test

Crosstalk and RFI testing is performed to evaluate the performance of DSL systems in the presence of crosstalk from other services in the loop and RF ingress.

#### 5.2.2.1	Crosstalk and RFI injection

Simulated crosstalk, XT (NEXT and/or FEXT) and RFI is introduced into the test loop as described in 5.1.2.1.

##### 5.2.2.1.1	Crosstalk injection for TCM-ISDN environment

For testing transmission performance of an ADSL system conforming to Annex C/G.992.2 (ADSL operating in an environment coexisting with TCM-ISDN DSL), the calibrated simulated TCM-ISDN DSL NEXT PSD and TCM-ISDN DSL FEXT PSD defined in clause 7 shall be injected into the test loop as described in 5.1.2.1.1.

#### 5.2.2.2	Crosstalk test method

Crosstalk testing shall be performed as described in 5.1.2.2.

### 5.2.3	Impulse noise test

Impulse noise testing is performed to evaluate the performance of DSL systems in the presence of impulse noise.

#### 5.2.3.1	Impulse noise injection

The same coupling circuit as in 5.1.2.1 is used for impulse noise injection. The amplitude level of the impulses may be measured with an oscilloscope.

#### 5.2.3.2	Impulse noise test method

Impulse noise testing shall be performed as described in 5.1.3.2.

### 5.2.4	POTS interference test

POTS interference test is performed to evaluate the performance of DSL systems in the presence of POTS signalling.

#### 5.2.4.1	POTS interference injection

The interference due to POTS service on the same line is generated by use of a telephone model structure and central office circuits connected in the normal way to the system under test. The telephone structure is described in clause 9 and should be connected to the appropriate points in the in-home wiring model (see 6.2). The following POTS signalling and alerting activities shall be performed:

–	call ATU-R phones and allow to ring 25 times;

–	pick up ringing phone "A" (see Figure 12) and hold in off-hook position;

–	return phone "A" to on-hook position.

Repeat above steps 25 times.

#### 5.2.4.2	POTS testing without a splitter at the ATU-R

Before testing, the DSL units are trained with the required crosstalk interference. Signalling disturbances are created through use of the CO line connected to the splitter at the ATU-C, and the telephone model structure connected to telephone jacks in the in-home wiring model (see 6.2). During the signalling activities described in 5.1.4.1, the system is tested by measuring:

–	the time required to achieve errorless transmission after a signalling event has occurred (measure fast retrain time);

–	the data rates in the on-hook and off-hook states.

Note that in the first on-hook to off-hook transition the retrain time will be much longer since a full initialization is required in order to generate the off-hook profile that is matched to phone "A". Subsequent on-hook to off-hook transitions should use a fast retrain and the stored profile for phone "A".

## 5.3	POTS QoS testing

### 5.3.1	Telephone circuitry

Figure 7 shows a simplified block diagram of a typical telephone, with signal flow direction indicated by arrows in the connections between blocks. Signal flow on the left side of the 2W/4W hybrid is generally bidirectional, and non-linear distortion generated in this area can result in reflected harmonic and intermodulation components appearing across tip and ring. Signal flow on the right side of the hybrid is generally unidirectional. Non-linear distortion of DSL signals in the speaker driver can result in audible energy at the speaker, but will generally not result in energy seen on the local loop. The microphone interface, of course, is not exposed to DSL energy in the first place, so non-linear distortion in that area is not an issue.

Within the bidirectional line interface, most components are on the side of the hook switch away from tip and ring. This includes the vast majority of sources of non-linear distortion, which are isolated from the loop when the telephone is on-hook.

Measurement criteria

The following criteria must be incorporated when evaluating interference between DSL and POTS channels for a splitterless DSL technology:

–	The primary configuration for measurement of channel interference shall be with one or more telephone devices in an off-hook condition. Most of the circuitry in a typical telephone (including, in nearly all cases, any components which cause non-linear distortion of a DSL signal) is located on the telephone side of the hook switch, and is isolated from DSL signals when the phone is on-hook (see Figure 7). While measurement of energy in the POTS band during on-hook conditions is an appropriate parameter for both splitterless and conventional DSL systems, it is not sufficient by itself for evaluation of splitterless systems – in fact, it is considerably less meaningful than measurements taken with phones off-hook.

–	Interference measurements must include the sound pressure output at the telephone speaker. It is not sufficient to measure the non-linear and intermodulation components generated across tip and ring, as some or all of the distortion can take place on the four-wire side of the telephone hybrid, causing audible noise at the speaker even in the absence of audio band energy on the line. Direct measurement of electrical power at the speaker terminals may not be sufficient due to varying speaker conversion characteristics. For POTS modem and facsimile devices, other measurements related to performance degradation would of course replace a sound pressure measurement.

–	Interference measurements should also include components generated across tip and ring. Excessive harmonic content on the line may result in reduced performance or even failure to connect in the downstream direction over longer loops.

–	There is a broad range of telephone devices which must be accommodated by any splitterless DSL solution. Test results based on a narrow sampling of devices will not accurately reflect performance in the field. Test results presented should include the types and numbers of devices used, to provide some indication of how representative the database may be.

Figure 7/G.996.1 – Typical telephone signal flow

### 5.3.2	Test conditions

The configuration used for testing DSL transceivers for interference with off-hook telephones is shown in Figure 8. A control telephone (with microphone disconnected) is routed through a POTS network simulator to provide switching and signalling functions for the POTS connection. The POTS network simulator shall be capable of introducing GSTN-band electrical noise as is commonly available in simulators designed to test dial modems per ITU-T V.56 bis. The other side of the POTS network simulator is routed through a central office POTS splitter to isolate the DSL channel from the POTS network simulator. The DSL/POTS combined port of the CO POTS splitter is routed through a wideband local loop simulator to the local telephone and the local (ATU-R) transceiver. The central office transceiver (ATU-C) is connected to the appropriate port on the CO POTS splitter. A condenser microphone is acoustically coupled to the telephone handset by positioning it approximately 1 mm away from, and aligned with, the centre axis of the handset speaker. The space between the microphone and the speaker is left unsealed. The microphone and handset are placed in an isolated environment to minimize ambient noise. Sound levels from the handset speaker are measured using a sound level meter, using B weighting per IEC 651 and slow response time. The zero dB sound power reference is 20 Pa.

A signal analyser is used to view the signal characteristics directly across tip and ring using a differential high impedance probe. This analyser is used to set and verify signal levels from the DSL transmitter and to verify test conditions.

For each test case, a POTS connection is established and the sound output of the telephone speaker is recorded without a DSL signal present to establish a control level. Some Recommendations specify or allow the use of a filter in series with the local telephone (e.g. customer premise POTS splitter, in-line filter, etc). The position of this optional filter is shown in Figure 8 with a dashed box.

Figure 8/G.996.1 – Test configuration

### 5.3.3	Summary of test procedure

1)	ATU-C/ATU-R transmitters are off. GSTN simulator noise is off. Remote telephone is quiet. Loop length is zero.

2)	A GSTN connection is established between remote telephone and local telephone.

3)	The acoustic energy of the local telephone speaker is measured for reference: SP1 (this establishes the acoustic noise floor as measured at the local telephone).

4)	The GSTN noise is turned on at the acceptable GSTN service dBrn level of the GSTN service requirement.

5)	The acoustic energy of the local telephone speaker is measured: SP2 (SP2 is thus the acoustic equivalent of the acceptable GSTN service noise on this particular telephone).

6)	GSTN simulator noise is turned off. ATU-C/ATU-R transmitters are turned on at their prescribed transmit levels.

7)	The acoustic energy of the local telephone speaker is measured: SP3. The test result is the difference between SP2 and SP3 in dB. If the test result is positive, i.e. SP3  SP2, then the DSL equipment does not introduce audio noise in excess of that allowed by the GSTN service for the local telephone used.

Test variations

The GSTN requirement may have two dBrn thresholds that require testing. For example, 20 dBrnC may represent preferred service and 30 dBrnC may be unacceptable service.

# 6	Test loops

## 6.1	Local loop configurations

### 6.1.1	North American test loops

See Figure 9.

Figure 9/G.996.1 – North American test loop

Resistance and insertion loss

Tables 2, 3 and 4 provide the calculated resistance and insertion loss between 100-ohm terminations of the loops shown in Figure 9.

Table 2/G.996.1 – Resistance and insertion loss in dB for test loops at 70 F (21.1 C)

| Loop # | Resistance (ohms) | Frequency (kHz) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 | 40 | 100 | 200 | 260 | 300 | 400 | 500 | 600 | 780 | 1100 |  |
| T # 7 | 1127 | 29.8 | 36.7 | 45.2 | 52.8 | 57.3 | 60.2 | 67.7 | 74.8 | 81.7 | 93.0 | 110 | Insertion loss (dB) |
| T # 9 | 877 | 27.6 | 36.4 | 52.5 | 47.5 | 55.7 | 62.0 | 60.3 | 71.5 | 72.2 | 82.7 | 96.2 |  |
| T # 13 | 909 | 26.6 | 34.1 | 47.9 | 48.3 | 55.7 | 61.3 | 62.2 | 71.4 | 74.1 | 85.3 | 100 |  |
| C # 4 | 634 | 17.6 | 22.0 | 29.6 | 39.6 | 40.1 | 42.5 | 49.2 | 50.2 | 53.8 | 55.7 | 70.7 |  |
| C # 6 | 751 | 20.0 | 24.4 | 30.1 | 35.2 | 38.2 | 40.2 | 45.1 | 49.9 | 54.4 | 62.0 | 73.6 |  |
| C # 7 | 562 | 17.3 | 20.9 | 26.8 | 33.6 | 37.8 | 38.6 | 43.1 | 49.9 | 57.9 | 60.2 | 72.7 |  |
| C # 8 | 630 | 19.2 | 22.8 | 27.7 | 34.4 | 38.3 | 40.8 | 46.9 | 52.4 | 57.4 | 65.4 | 77.8 |  |
| Mid-C | 501 | 13.3 | 16.2 | 20.0 | 23.4 | 25.4 | 26.8 | 30.1 | 33.2 | 36.3 | 41.3 | 49.1 |  |

Table 3/G.996.1 – Resistance and insertion loss in dB for test loops at 90 F (32.2 C)

| Loop # | Resistance (ohms) | Frequency (kHz) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 | 40 | 100 | 200 | 260 | 300 | 400 | 500 | 600 | 780 | 1100 |  |
| T # 7 | 1176 | 30.6 | 37.9 | 46.9 | 54.6 | 59.1 | 62.1 | 69.6 | 76.6 | 83.4 | 95.0 | 113 | Insertion loss (dB) |
| T # 9 | 915 | 28.4 | 37.5 | 53.4 | 49.1 | 57.2 | 63.1 | 61.9 | 72.8 | 73.6 | 84.2 | 98.1 |  |
| T # 13 | 948 | 27.4 | 35.2 | 49.0 | 49.9 | 57.2 | 62.5 | 63.7 | 72.8 | 75.6 | 87.0 | 102 |  |
| C # 4 | 658 | 18.0 | 22.6 | 30.4 | 40.3 | 41.0 | 43.5 | 50.0 | 50.9 | 54.3 | 56.6 | 71.6 |  |
| C # 6 | 784 | 20.5 | 25.2 | 31.2 | 36.4 | 39.4 | 41.4 | 46.4 | 51.1 | 55.6 | 63.3 | 75.2 |  |
| C # 7 | 586 | 17.9 | 21.6 | 27.7 | 34.0 | 38.7 | 39.5 | 44.1 | 50.9 | 58.8 | 61.4 | 74.0 |  |
| C # 8 | 657 | 19.8 | 23.6 | 28.7 | 35.4 | 39.3 | 41.8 | 47.9 | 53.5 | 58.6 | 66.8 | 79.4 |  |
| Mid-C | 523 | 13.8 | 16.7 | 20.7 | 24.2 | 26.2 | 27.6 | 30.9 | 34.0 | 37.1 | 42.2 | 50.1 |  |

Table 4/G.996.1 – Resistance and insertion loss in dB for test loops at 120 F (48.9 C)

| Loop # | Resistance (ohms) | Frequency (kHz) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 | 40 | 100 | 200 | 260 | 300 | 400 | 500 | 600 | 780 | 1100 |  |
| T # 7 | 1250 | 31.9 | 39.6 | 49.4 | 57.4 | 61.8 | 64.8 | 72.3 | 79.3 | 86.1 | 97.9 | 116 | Insertion loss (dB) |
| T # 9 | 972 | 29.5 | 39.1 | 54.7 | 51.5 | 59.5 | 65.0 | 64.1 | 74.4 | 75.7 | 86.4 | 101 |  |
| T # 13 | 1008 | 28.5 | 36.8 | 50.7 | 52.3 | 59.5 | 64.5 | 66.0 | 74.9 | 77.9 | 89.4 | 105 |  |
| C # 4 | 704 | 18.9 | 23.8 | 32.2 | 41.9 | 42.8 | 45.2 | 51.5 | 52.8 | 56.0 | 58.7 | 74.1 |  |
| C # 6 | 833 | 21.4 | 26.3 | 32.8 | 38.2 | 41.2 | 43.2 | 48.2 | 52.9 | 57.4 | 65.3 | 77.5 |  |
| C # 7 | 623 | 18.7 | 22.6 | 29.1 | 36.2 | 40.0 | 43.2 | 45.5 | 52.5 | 60.2 | 63.2 | 76.0 |  |
| C #8 | 699 | 20.7 | 24.8 | 30.2 | 36.9 | 40.8 | 43.3 | 49.4 | 55.1 | 60.4 | 68.8 | 81.7 |  |
| Mid-C | 555 | 14.4 | 17.5 | 21.8 | 25.5 | 27.5 | 28.8 | 32.1 | 35.1 | 38.3 | 43.5 | 51.6 |  |

Primary constants

The primary constants R, L, C, and G of polyethylene-insulated cable (PIC), at 0° F, 70° F, and 120° F, are tabulated in Annex A.

The variation of R and L with frequency can be accurately modelled as follows:

R=roc4+ac×f214

and

L=L0+L8×xb1+xb

where: xb=ffmb

The six coefficients for 24 and 26 AWG PIC cable per km at 70° F are given in Table 5.

Table 5/G.996.1 – Coefficients for calculation of R and L
[24 and 26 AWG PIC at 70 F (21.1 C)]

|  | roc | ac | L0 | L8 | fm | b |
| --- | --- | --- | --- | --- | --- | --- |
| 24 AWG | 174.559 | 0.05307 | 617.295 | 478.971 | 553.76 | 1.15298 |
| 26 AWG | 286.176 | 0.14769620 | 675.369 | 488.952 | 806.339 | 0.929 |

For PIC cable, the C values are constant with frequency 51.56 nF/km, and the G values are negligibly small.

### 6.1.2	European local test loops

The variation of the primary line constants (R, L and C) with frequency for the different reference cable types are given in Tables 7 and 8. Note that G is assumed to be zero. The RLC values are quoted per km at a temperature of 20° C and are measured values that have been smoothed.

The variation of R and L with frequency can be accurately modelled as follows:

R=roc4+ac×f214

and

L=L0+L8×xb1+xb

where: xb=ffmb

The six coefficients for the cable types referenced in Figure 10 are given in Table 9. Note also that there are adjustable sections (marked "X") in Table 10. The nominal lengths of these sections are shown in Table 6. The lengths of the sections are based on the reference RLC values for each cable type shown in Tables 7 and 8. For repeatability of measurement results, however, the lengths of these section lengths shall be adjusted to give the overall insertion loss given in Table 6. Insertion loss is measured at 300 kHz with 100  (balanced resistive) source and termination impedances.

Figure 10/G.996.1 – ETSI test loops

Table 6/G.996.1 – Examples of loop set insertion loss, nominal lengths for European testing

| Loop # | Loop insertion loss 36 dB at 300 kHz | Loop insertion loss 36 dB at 300 kHz | Loop insertion loss 51 dB at 300 kHz | Loop insertion loss 61 dB at 300 kHz |
| --- | --- | --- | --- | --- |
|  | Nominal value of adjustable length "X" (km) | Nominal value of adjustable length "X" (km) | Nominal value of adjustable length "X" (km) | Nominal value of adjustable length "X" (km) |
| 1 | 2.55 | 2.55 | 3.60 | 4.30 |
| 2 | 3.40 | 3.40 | 4.80 | 5.70 |
| 3 | 1.40 | 1.40 | 2.45 | 3.15 |
| 4 | 0.90 | 0.90 | 1.90 | 2.65 |
| 5 | 1.45 | 1.45 | 2.50 | 3.20 |
| 6 | 1.30 | 1.30 | 2.35 | 3.05 |
| 7 | 0.60 | 0.60 | 1.60 | 2.20 |
| 8 | 0.75 | 0.75 | 1.80 | 2.50 |

Table 7/G.996.1 – RLC values for 0.32, 0.4, and 0.5 mm PE cables

|  | 0.32 mm C  40 nF/km |  | 0.4 mm C  50 nF/km |  | 0.5 mm C  50 nF/km |  |
| --- | --- | --- | --- | --- | --- | --- |
| Freq. (kHz) | R (ohm/km) | L (mH/km) | R (ohm/km) | L (mH/km) | R (ohm/km) | L (mH/km) |
| 0 | 409.000 | 607.639 | 280.000 | 587.132 | 179.000 | 673.574 |
| 2.5 | 409.009 | 607.639 | 280.007 | 587.075 | 179.015 | 673.466 |
| 10 | 409.140 | 607.639 | 280.110 | 586.738 | 179.244 | 672.923 |
| 20 | 409.557 | 607.639 | 280.440 | 586.099 | 179.970 | 671.980 |
| 30 | 410.251 | 607.639 | 280.988 | 585.322 | 181.161 | 670.896 |
| 40 | 411.216 | 607.639 | 281.748 | 584.443 | 182.790 | 669.716 |
| 50 | 412.447 | 607.639 | 282.718 | 583.483 | 184.822 | 668.468 |
| 100 | 422.302 | 607.631 | 290.433 | 577.878 | 199.608 | 661.677 |
| 150 | 437.337 | 607.570 | 302.070 | 571.525 | 218.721 | 654.622 |
| 200 | 456.086 | 607.327 | 316.393 | 564.889 | 239.132 | 647.735 |
| 250 | 477.229 | 606.639 | 332.348 | 558.233 | 259.461 | 641.208 |
| 300 | 499.757 | 605.074 | 349.167 | 551.714 | 279.173 | 635.119 |
| 350 | 522.967 | 602.046 | 366.345 | 545.431 | 298.103 | 629.489 |
| 400 | 546.395 | 596.934 | 383.562 | 539.437 | 316.230 | 624.309 |
| 450 | 569.748 | 589.337 | 400.626 | 533.759 | 333.591 | 619.557 |
| 500 | 592.843 | 579.376 | 417.427 | 528.409 | 350.243 | 615.202 |
| 550 | 615.576 | 567.822 | 433.904 | 523.385 | 366.246 | 611.211 |
| 600 | 637.885 | 555.867 | 450.027 | 518.677 | 381.657 | 607.552 |
| 650 | 659.743 | 544.657 | 465.785 | 514.272 | 396.528 | 604.192 |
| 700 | 681.138 | 534.942 | 481.180 | 510.153 | 410.907 | 601.104 |
| 750 | 702.072 | 526.991 | 496.218 | 506.304 | 424.835 | 598.261 |
| 800 | 722.556 | 520.732 | 510.912 | 502.707 | 438.348 | 595.639 |
| 850 | 742.601 | 515.919 | 525.274 | 499.343 | 451.480 | 593.217 |
| 900 | 762.224 | 512.264 | 539.320 | 496.197 | 464.258 | 590.975 |
| 950 | 781.442 | 509.503 | 553.064 | 493.252 | 476.71 | 588.896 |
| 1000 | 800.272 | 507.415 | 566.521 | 490.494 | 488.857 | 586.966 |
| 1050 | 818.731 | 505.831 | 579.705 | 487.908 | 500.720 | 585.169 |
| 1100 | 836.837 | 504.623 | 592.628 | 485.481 | 512.317 | 583.495 |
| NOTE – G  0 at all frequencies. |  |  |  |  |  |  |

Table 8/G.996.1 – RLC values for 0.63 and 0.9 mm PE cables

|  | 0.63 mm C  45 nF/km |  | 0.9 mm C  40 nF/km |  |
| --- | --- | --- | --- | --- |
| Freq. (kHz) | R (ohm/km) | L (mH/km) | R (ohm/km) | L (mH/km) |
| 0 | 113.000 | 699.258 | 55.000 | 750.796 |
| 2.5 | 113.028 | 697.943 | 55.088 | 745.504 |
| 10 | 113.442 | 693.361 | 56.361 | 731.961 |
| 20 | 114.737 | 687.008 | 59.941 | 716.775 |
| 30 | 116.803 | 680.714 | 64.777 | 703.875 |
| 40 | 119.523 | 674.593 | 70.127 | 692.707 |
| 50 | 122.768 | 668.690 | 75.586 | 682.914 |
| 100 | 143.115 | 642.718 | 100.769 | 647.496 |
| 150 | 164.938 | 622.050 | 121.866 | 625.140 |
| 200 | 185.689 | 605.496 | 140.075 | 609.652 |
| 250 | 204.996 | 592.048 | 156.273 | 598.256 |
| 300 | 222.961 | 580.960 | 170.987 | 589.504 |
| 350 | 239.764 | 571.691 | 184.556 | 582.563 |
| 400 | 255.575 | 563.845 | 197.208 | 576.919 |
| 450 | 270.533 | 557.129 | 209.104 | 572.237 |
| 500 | 284.753 | 551.323 | 220.365 | 568.287 |
| 550 | 298.330 | 546.260 | 231.081 | 564.910 |
| 600 | 311.339 | 541.809 | 241.326 | 561.988 |
| 650 | 323.844 | 537.868 | 251.155 | 559.435 |
| 700 | 335.897 | 534.358 | 260.615 | 557.183 |
| 750 | 347.542 | 531.212 | 269.745 | 555.183 |
| 800 | 358.819 | 528.378 | 278.577 | 553.394 |
| 850 | 369.758 | 525.813 | 287.138 | 551.784 |
| 900 | 380.388 | 523.480 | 295.452 | 550.327 |
| 950 | 390.734 | 521.352 | 303.538 | 549.002 |
| 1000 | 400.816 | 519.402 | 311.416 | 547.793 |
| 1050 | 410.654 | 517.609 | 319.099 | 546.683 |
| 1100 | 420.264 | 515.956 | 326.602 | 545.663 |
| NOTE – G  0 at all frequencies. |  |  |  |  |

Table 9/G.996.1 – Coefficients for calculation of R and L

|  | roc | ac | L0 | L8 | fm | b |
| --- | --- | --- | --- | --- | --- | --- |
| 0.32 mm | 0.4090 | 0.3822 | 0.6075 | 0.5 | 0.6090 | 5.269 |
| 0.4 mm | 0.2800 | 0.0969 | 0.5873 | 0.4260 | 0.7459 | 1.385 |
| 0.5 mm | 0.1792 | 0.0561 | 0.6746 | 0.5327 | 0.6647 | 1.195 |
| 0.63 mm | 0.113 | 0.0257 | 0.6994 | 0.4772 | 0.2658 | 1.0956 |
| 0.9 mm | 0.0551 | 0.0094 | 0.7509 | 0.5205 | 0.1238 | 0.9604 |

### 6.1.3	Test loops in an environment coexisting with TCM-ISDN DSL

To test the performance of the G.992.1 and G.992.2 transceivers, the test loops specified in Figure 11 shall be used. Note that there are adjustable sections (marked "X") in the figure. The nominal lengths of these sections corresponding to each loop coverage are shown in Table 10. The lengths of the sections are based on the reference RLC values for each cable type shown below. The calculated overall loop insertion loss at 300 kHz and d.c. loop resistance are also provided in Table 10.

Figure 11/G.996.1 – Test loop in an environment coexisting with TCM-ISDN DSL

Table 10/G.996.1 – Loop-set insertion loss and nominal lengths

| Loop insertion loss (160 kHz) | Loop TCM # | Nominal value of "X" | Total loop length | Loop insertion loss (300 kHz) | Group delay (s) (160 kHz) | d.c. loop resistance |
| --- | --- | --- | --- | --- | --- | --- |
| 26.0 dB | 1 | 0.42 km | 5.42 km | 36.1 dB | 28.8 | 314 ohms |
| (60% coverage) | 2 | 1.32 km | 2.82 km | 34.6 dB | 15.5 | 518 ohms |
|  | 3 | 0.78 km | 1.78 km | 31.9 dB | 9.8 | 642 ohms |
|  | 4 | 3.62 km | 3.62 km | 37.0 dB | 19.3 | 376 ohms |
|  | 5 | 2.07 km | 2.07 km | 34.0 dB | 11.4 | 567 ohms |
| 37.0 dB | 1 | 1.95 km | 6.95 km | 51.8 dB | 37.0 | 474 ohms |
| (90% coverage) | 2 | 2.19 km | 3.69 km | 49.0 dB | 20.2 | 758 ohms |
|  | 3 | 1.75 km | 2.75 km | 45.6 dB | 15.4 | 910 ohms |
|  | 4 | 5.15 km | 5.15 km | 52.7 dB | 27.5 | 536 ohms |
|  | 5 | 2.94 km | 2.94 km | 48.3 dB | 16.1 | 807 ohms |
| 50.0 dB | 1 | 3.76 km | 8.76 km | 70.3 dB | 46.6 | 662 ohms |
| (99% coverage) | 2 | 3.23 km | 4.73 km | 66.0 dB | 25.9 | 1041 ohms |
|  | 3 | 2.91 km | 3.91 km | 61.7 dB | 22.0 | 1227 ohms |
|  | 4 | 6.97 km | 6.97 km | 71.2 dB | 37.2 | 724 ohms |
|  | 5 | 3.97 km | 3.97 km | 65.3 dB | 21.8 | 1090 ohms |
| 65.0 dB | 1 | 5.85 km | 10.85 km | 91.7 dB | 57.8 | 879 ohms |
| (99.9% coverage) | 2 | 4.42 km | 5.92 km | 85.6 dB | 32.5 | 1368 ohms |
| (for G.992.2 only) | 3 |  |  |  |  |  |
|  | 4 | 9.06 km | 9.06 km | 92.6 dB | 48.3 | 941 ohms |
|  | 5 | 5.16 km | 5.16 km | 84.9 dB | 28.3 | 1417 ohms |
| NOTE 1 – The values in the table for Loop TCM #2 are given by considering only the section terminated by Customer Premises and Central Office, on condition of detaching the bridge taps (BTs). The effect on the overall insertion loss characteristics caused by the BTs is not taken into account. NOTE 2 – Test loops of the 60-99% coverage are for use with the G.992.1 performance testing. Test loops of the 60-99.9% coverage are for use with the G.992.2 performance testing. It is not necessary to comply with NEXT/FEXT PSL values (NPSLn/FPSLn) defined in 7.9 for test loops of the 99.9% coverage. The resultant net data rates may be given as a function of NPSLn/FPSLn values for this case. NOTE 3 – TCM #3 for 99.9% coverage is removed from the list because the d.c. loop resistance is greater than 1500 ohms. |  |  |  |  |  |  |

Primary line constants

The primary line constants are R, L, C, and G. The variation of R, L, and G with frequency can be accurately calculated by the equations shown below. The coefficients for the calculation of R, L, and G are given in Table 11. The equations below give the value of R in ohm/m, L in H/m, G in mho/m, C in F/m, and f in Hz by using the coefficient values shown in the table. Note that the capacitance C is assumed constant with frequency (C  50 pF/m).

The primary line constants (R, L, G, and C) calculated by the equations below for the cable types referenced in Figure 11 are shown in Tables 13 to 17, as a function of frequency, where the values are indicated R in ohm/km, L in µH/km, G in µmho/km, C in nF/km, f in kHz, and at a temperature of 20º C.

R=2(Ri+Rn+Rns)[ohm/m]L=2(La+Li+Ln+Lns)[H/m]G=ϖCtanδ[mho/m]C=50×10−12[F/m]

Ri=1πr2σfi(R)=1πr2σRe1+j2δrJ01+jδrJ11+jδr

Rn=1πσd2fn(R)=1πσd2Re−1+jδrJ11+jδrJ01+jδr

Rns=1πσd24fn(R)=4πσd2Re−1+jδrJ11+jδrJ01+jδr

La=μ02π1ndr

Li=μrμ02πfi(L)=μ2πRe−δ1+jrJ01+jδrJ11+jδr

Ln=−μ02πrd2fn(L)=−μ02πrd2Re−J21+jδrJ01+jδr

Lns=−μ02πrd24fn(L)=−μ02πrd24Re−J21+jδrJ01+jδr

where:

 	2f	angular frequency

r	the radius of a conductor

	conductivity of copper (conductor)

δ = 2ωσμrμ0	skin depth

d 	22(r+c0)	the distance between the conductor centres of a pair

c0	the insulator thickness of a wire

r	relative permeability of copper (conductor)

0	permeability of vacuum

Table 11/G.996.1 – Coefficients for calculation of R and L

| Coefficient | Paper |  |  |  | Polyethylene |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0.4 mm | 0.5 mm | 0.65 mm | 0.9 mm | 0.32 mm | 0.4 mm | 0.5 mm | 0.65 mm | 0.9 mm |
| r(m) | 0.2  103 | 0.25  103 | 0.325  103 | 0.45  103 | 0.16  103 | 0.2  103 | 0.25  103 | 0.325  103 | 0.45  103 |
| c0 (m) | 0.09  103 | 0.11  103 | 0.17  103 | 0.24  103 | 0.05  103 | 0.13  103 | 0.15  103 | 0.20  103 | 0.27  103 |
| r |  |  |  | 1 |  |  |  |  |  |
| 0(H/m) |  |  |  | 4  107 |  |  |  |  |  |
|  (mho/m) |  |  |  | 5.8  107 |  |  |  |  |  |
| tan  |  | 2.5  102 |  |  | 4.0  104 |  | 5.0  104 |  |  |

The variation of R and L with frequency can be accurately modelled as follows. The five coefficients for the cable types referenced in Figure 11 are given in Table 12. The approximate equations below give the values of R in ohm/m, L in H/m, G in mho/m, C in F/m, and f in Hz by using the coefficient values shown in the table.

R=roc4+ac×f214 [ohm/m]L=xa+xb×f12+xc×f13[H/m]G=ωCtanδ[mho/m]C=50×10−12 [F/m]

Table 12/G.996.1 – Coefficients calculation of R and L (at 20° C)

| Cable type |  | roc | ac | xa | xb | xc |
| --- | --- | --- | --- | --- | --- | --- |
| Paper | 0.4 mm | 2.688  101 | 2.267  1013 | 6.834  107 | 2.094  1010 | 7.205  1010 |
|  | 0.5 mm | 1.724  101 | 9.374  1014 | 7.351  107 | 1.930  1011 | 2.330  109 |
|  | 0.65 mm | 1.041  101 | 2.787  1014 | 8.006  107 | 2.696  1010 | 5.340  109 |
|  | 0.9 mm | 5.589  102 | 7.180  1015 | 8.304  107 | 5.111  1010 | 8.161  109 |
| Polyethylene | 0.32 mm | 4.175  101 | 6.998  1013 | 6.003  107 | 3.919  1010 | 3.081  109 |
|  | 0.4 mm | 2.714  101 | 1.705  1013 | 7.257  107 | 2.059  1010 | 9.678  1010 |
|  | 0.5 mm | 1.742  101 | 7.346  1014 | 7.618  107 | 1.547  1011 | 1.656  109 |
|  | 0.65 mm | 1.048  101 | 2.436  1014 | 8.139  107 | 2.354  1010 | 4.801  109 |
|  | 0.9 mm | 5.630  102 | 6.486  1015 | 8.407  107 | 4.816  1010 | 7.721  109 |

Table 13/G.996.1 – RLC values for 0.4 and 0.5 mm paper-insulated cables

| Frequency (kHz) | 0.4 mm | C  50 nF/km | 0.5 mm | C  50nF/km |
| --- | --- | --- | --- | --- |
|  | R (ohm/km) | L (H/km) | R (ohm/km) | L (H/km) |
| 0.00 | 274.41 | 664.51 | 175.62 | 661.75 |
| 2.50 | 274.42 | 664.51 | 175.64 | 661.73 |
| 10.00 | 274.62 | 664.42 | 175.97 | 661.52 |
| 20.00 | 275.28 | 664.15 | 176.99 | 660.85 |
| 30.00 | 276.36 | 663.69 | 178.68 | 659.74 |
| 40.00 | 277.87 | 663.07 | 180.99 | 658.24 |
| 50.00 | 279.78 | 662.27 | 183.88 | 656.37 |
| 100.00 | 294.75 | 656.09 | 204.86 | 643.13 |
| 150.00 | 316.56 | 647.33 | 231.49 | 627.35 |
| 200.00 | 342.22 | 637.42 | 258.98 | 612.46 |
| 250.00 | 369.40 | 627.46 | 285.41 | 599.57 |
| 300.00 | 396.69 | 618.03 | 310.31 | 588.68 |
| 350.00 | 423.38 | 609.41 | 333.73 | 579.47 |
| 400.00 | 449.19 | 601.65 | 355.79 | 571.61 |
| 450.00 | 474.02 | 594.68 | 376.61 | 564.83 |
| 500.00 | 497.90 | 588.42 | 396.34 | 558.92 |
| 550.00 | 520.87 | 582.79 | 415.08 | 553.73 |
| 600.00 | 542.99 | 577.69 | 432.93 | 549.13 |
| 650.00 | 564.30 | 573.06 | 449.98 | 545.03 |
| 700.00 | 584.86 | 568.84 | 466.34 | 541.34 |
| 750.00 | 604.72 | 564.98 | 482.06 | 538.02 |
| 800.00 | 623.92 | 561.42 | 497.22 | 535.00 |
| 850.00 | 642.50 | 558.15 | 511.88 | 532.24 |
| 900.00 | 660.52 | 555.11 | 526.09 | 529.71 |
| 950.00 | 678.00 | 552.30 | 539.88 | 527.38 |
| 1000.00 | 694.99 | 549.69 | 553.30 | 525.23 |
| 1050.00 | 711.52 | 547.25 | 566.38 | 523.23 |
| 1100.00 | 727.62 | 544.96 | 579.14 | 521.37 |

Table 14/G.996.1 – RLC values for 0.65 and 0.9 mm paper-insulated cables

| Frequency (kHz) | 0.65 mm | C  50nF/km | 0.9 mm | C  50nF/km |
| --- | --- | --- | --- | --- |
|  | R (ohm/km) | L (H/km) | R (ohm/km) | L (H/km) |
| 0.00 | 103.92 | 684.18 | 54.20 | 686.87 |
| 2.50 | 103.95 | 684.15 | 54.27 | 686.73 |
| 10.00 | 104.45 | 683.60 | 55.20 | 684.79 |
| 20.00 | 106.02 | 681.89 | 57.98 | 679.07 |
| 30.00 | 108.52 | 679.19 | 62.04 | 670.97 |
| 40.00 | 111.83 | 675.66 | 66.81 | 661.83 |
| 50.00 | 115.78 | 671.51 | 71.88 | 652.64 |
| 100.00 | 140.25 | 647.61 | 96.11 | 616.45 |
| 150.00 | 165.42 | 626.69 | 116.50 | 594.37 |
| 200.00 | 188.55 | 610.67 | 133.77 | 579.83 |
| 250.00 | 209.58 | 598.33 | 148.81 | 569.58 |
| 300.00 | 228.77 | 588.57 | 162.27 | 561.93 |
| 350.00 | 246.38 | 580.67 | 174.61 | 555.98 |
| 400.00 | 262.68 | 574.16 | 186.08 | 551.17 |
| 450.00 | 277.89 | 568.69 | 196.86 | 547.19 |
| 500.00 | 292.19 | 564.04 | 207.06 | 543.81 |
| 550.00 | 305.73 | 560.02 | 216.77 | 540.90 |
| 600.00 | 318.64 | 556.51 | 226.06 | 538.36 |
| 650.00 | 331.01 | 553.41 | 234.97 | 536.11 |
| 700.00 | 342.90 | 550.65 | 243.55 | 534.10 |
| 750.00 | 354.37 | 548.17 | 251.83 | 532.30 |
| 800.00 | 365.47 | 545.92 | 259.84 | 530.67 |
| 850.00 | 376.23 | 543.88 | 267.60 | 529.18 |
| 900.00 | 386.68 | 542.00 | 275.14 | 527.82 |
| 950.00 | 396.85 | 540.27 | 282.47 | 526.57 |
| 1000.00 | 406.77 | 538.67 | 289.61 | 525.41 |
| 1050.00 | 416.44 | 537.19 | 296.58 | 524.33 |
| 1100.00 | 425.88 | 535.81 | 303.38 | 523.33 |

Table 15/G.996.1 – RLC values for 0.32, 0.4 and 0.5 mm polyethylene-insulated cables

| Frequency (kHz) | 0.32 mm C  50nF/km |  | 0.4 mm C  50nF/km |  | 0.5 mm C  50nF/km |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | R (ohm/km) | L (H/km) | R (ohm/km) | L (H/km) | R (ohm/km) | L (H/km) |
| 0.00 | 428.76 | 624.66 | 274.41 | 716.20 | 175.62 | 703.89 |
| 2.50 | 428.77 | 624.66 | 274.42 | 716.19 | 175.64 | 703.88 |
| 10.00 | 428.92 | 624.62 | 274.59 | 716.13 | 175.91 | 703.70 |
| 20.00 | 429.41 | 624.48 | 275.12 | 715.91 | 176.79 | 703.14 |
| 30.00 | 430.23 | 624.26 | 276.02 | 715.55 | 178.23 | 702.23 |
| 40.00 | 431.37 | 623.94 | 277.25 | 715.05 | 180.20 | 700.98 |
| 50.00 | 432.83 | 623.54 | 278.83 | 714.42 | 182.66 | 699.43 |
| 100.00 | 444.67 | 620.31 | 291.19 | 709.50 | 200.68 | 688.42 |
| 150.00 | 463.20 | 615.30 | 309.35 | 702.50 | 223.85 | 675.19 |
| 200.00 | 486.98 | 608.97 | 330.92 | 694.53 | 248.16 | 662.55 |
| 250.00 | 514.44 | 601.81 | 354.05 | 686.44 | 271.91 | 651.46 |
| 300.00 | 544.17 | 594.24 | 377.58 | 678.73 | 294.59 | 641.96 |
| 350.00 | 575.05 | 586.61 | 400.91 | 671.59 | 316.14 | 633.82 |
| 400.00 | 606.26 | 579.16 | 423.72 | 665.08 | 336.56 | 626.79 |
| 450.00 | 637.24 | 572.03 | 445.91 | 659.18 | 355.93 | 620.66 |
| 500.00 | 667.67 | 565.31 | 467.42 | 653.82 | 374.32 | 615.28 |
| 550.00 | 697.37 | 559.03 | 488.25 | 648.94 | 391.79 | 610.53 |
| 600.00 | 726.25 | 553.18 | 508.41 | 644.49 | 408.45 | 606.30 |
| 650.00 | 754.29 | 547.75 | 527.90 | 640.42 | 424.36 | 602.52 |
| 700.00 | 781.52 | 542.72 | 546.76 | 636.68 | 439.60 | 599.12 |
| 750.00 | 807.95 | 538.05 | 565.01 | 633.23 | 454.24 | 596.04 |
| 800.00 | 833.64 | 533.70 | 582.67 | 630.05 | 468.35 | 593.25 |
| 850.00 | 858.61 | 529.65 | 599.77 | 627.10 | 481.98 | 590.69 |
| 900.00 | 882.92 | 525.88 | 616.36 | 624.36 | 495.18 | 588.35 |
| 950.00 | 906.60 | 522.34 | 632.44 | 621.82 | 508.00 | 586.20 |
| 1000.00 | 929.68 | 519.03 | 648.07 | 619.45 | 520.46 | 584.20 |
| 1050.00 | 952.21 | 515.92 | 663.27 | 617.23 | 532.60 | 582.35 |
| 1100.00 | 974.20 | 513.00 | 678.07 | 615.15 | 544.44 | 580.62 |

Table 16/G.996.1 – RLC values for 0.65 mm and 0.9 mm polyethylene-insulated cables

| Frequency (kHz) | 0.65 mm | C  50nF/km | 0.9 mm | C  50nF/km |
| --- | --- | --- | --- | --- |
|  | R (ohm/km) | L (H/km) | R (ohm/km) | L (H/km) |
| 0.00 | 103.92 | 707.72 | 54.20 | 703.89 |
| 2.50 | 103.95 | 707.69 | 54.26 | 703.77 |
| 10.00 | 104.41 | 707.19 | 55.14 | 701.96 |
| 20.00 | 105.84 | 705.66 | 57.75 | 696.66 |
| 30.00 | 108.14 | 703.22 | 61.57 | 689.13 |
| 40.00 | 111.17 | 700.03 | 66.08 | 680.63 |
| 50.00 | 114.81 | 696.28 | 70.89 | 672.04 |
| 100.00 | 137.50 | 674.59 | 94.16 | 637.90 |
| 150.00 | 161.20 | 655.37 | 113.97 | 616.72 |
| 200.00 | 183.24 | 640.46 | 130.78 | 602.67 |
| 250.00 | 203.43 | 628.83 | 145.41 | 592.72 |
| 300.00 | 221.91 | 619.57 | 158.49 | 585.30 |
| 350.00 | 238.88 | 612.03 | 170.48 | 579.52 |
| 400.00 | 254.58 | 605.79 | 181.62 | 574.85 |
| 450.00 | 269.22 | 600.55 | 192.09 | 570.98 |
| 500.00 | 282.98 | 596.08 | 202.00 | 567.70 |
| 550.00 | 296.00 | 592.23 | 211.44 | 564.88 |
| 600.00 | 308.41 | 588.86 | 220.46 | 562.41 |
| 650.00 | 320.29 | 585.88 | 229.12 | 560.22 |
| 700.00 | 331.71 | 583.23 | 237.45 | 558.28 |
| 750.00 | 342.73 | 580.85 | 245.50 | 556.53 |
| 800.00 | 353.40 | 578.69 | 253.28 | 554.94 |
| 850.00 | 363.73 | 576.73 | 260.82 | 553.50 |
| 900.00 | 373.78 | 574.93 | 268.14 | 552.18 |
| 950.00 | 383.55 | 573.27 | 275.27 | 550.96 |
| 1000.00 | 393.07 | 571.73 | 282.20 | 549.83 |
| 1050.00 | 402.36 | 570.31 | 288.97 | 548.79 |
| 1100.00 | 411.44 | 568.98 | 295.58 | 547.81 |

Table 17/G.996.1 – G values

| Frequency (kHz) | 0.4, 0.5, 0.65, 0.9 mm paper (mho/km) | 0.32 mm polyethylene (mho/km) | 0.4, 0.5, 0.65, 0.9 mm polyethylene (mho/km) |
| --- | --- | --- | --- |
| 0.00 | 0.00 | 0.00 | 0.00 |
| 2.50 | 19.64 | 0.31 | 0.39 |
| 10.00 | 78.54 | 1.26 | 1.57 |
| 20.00 | 157.08 | 2.51 | 3.14 |
| 30.00 | 235.62 | 3.77 | 4.71 |
| 40.00 | 314.16 | 5.03 | 6.28 |
| 50.00 | 392.70 | 6.28 | 7.85 |
| 100.00 | 785.40 | 12.57 | 15.71 |
| 150.00 | 1178.10 | 18.85 | 23.56 |
| 200.00 | 1570.80 | 25.13 | 31.42 |
| 250.00 | 1963.50 | 31.42 | 39.27 |
| 300.00 | 2356.19 | 37.70 | 47.12 |
| 350.00 | 2748.89 | 43.98 | 54.98 |
| 400.00 | 3141.59 | 50.27 | 62.83 |
| 450.00 | 3534.29 | 56.55 | 70.69 |
| 500.00 | 3926.99 | 62.83 | 78.54 |
| 550.00 | 4319.69 | 69.12 | 86.39 |
| 600.00 | 4712.39 | 75.40 | 94.25 |
| 650.00 | 5105.09 | 81.68 | 102.10 |
| 700.00 | 5497.79 | 87.96 | 109.96 |
| 750.00 | 5890.49 | 94.25 | 117.81 |
| 800.00 | 6283.19 | 100.53 | 125.66 |
| 850.00 | 6675.88 | 106.81 | 133.52 |
| 900.00 | 7068.58 | 113.10 | 141.37 |
| 950.00 | 7461.28 | 119.38 | 149.23 |
| 1000.00 | 7853.98 | 125.66 | 157.08 |
| 1050.00 | 8246.68 | 131.95 | 164.93 |
| 1100.00 | 8639.38 | 138.23 | 172.79 |

## 6.2	In-home wiring models

### 6.2.1	In-home wiring model #1

In the model in Figure 12, note that phone "A" will be used for off-hook testing. All wire shown is Category 3 wire.

Category 3 wire has attenuation (a) in dB/100 m (328 ft) that is calculated based on the equation:

Category 3 Attenuation (a)  2.320 f+0.238f,     f in MHz.

Figure 12/G.996.1 – Customer premise in-home wiring model #1

### 6.2.2	In-home wiring model #2: Daisy Chain 4-conductor, 24 AWG, non-paired station wire

See Figure 13.

Figure 13/G.996.1 – Customer premise in-home wiring model #2

Electrical performance – Station wire

| Property | Performance |
| --- | --- |
| Conductor Resistance – Maximum |  |
| 22 AWG | 59.1 ohms/km (18.1 ohms/kft) |
| 24 AWG | 93.8 ohms/km (28.6 ohms/kft) |
| Spark testing |  |
| Voltage – Minimum | 1750 V a.c. or 2500 V d.c. |
| Fault rate – Maximum | 1 per 915 m (3000 ft) |
| Coaxial capacitance – Maximum (at 1000 Hz and 20C) | 279 nF/km (85 nF/kft) |
| Pair-to-pair capacitance unbalance – Maximum(at 1000 Hz) | 410 pF/500 m (125 pF/500 ft) |
| High voltage |  |
| Withstand voltage | 2500 V d.c. |
| Time – Minimum | 2 s |
| Dielectric strength rating – Jacket |  |
| ac voltage potential – Minimum | 1500 V rms |
| Current flow, peak during 90 s test – Maximum | 10 mA |
| Insulation resistivity: Each conductor – Minimum | 152 Mohm  km (500 Mohm  kft) |
| OPENS: Each length CROSSES: Each length SHORTS: Each length | None allowed None allowed None allowed |
| NOTE – 22 AWG  0.65 mm; 24 AWG  0.5 mm. |  |

### 6.2.3	In-home wiring model #3: Star – 152.4 m (500 ft), 4-pair Category 3 wire

In Figure 14, Category 3 wire has attenuation (a) in dB/100 m (328 ft) that is calculated based on the equation:

Category 3 Attenuation (a)  2.320 f+0.238f,     f in MHz

Figure 14/G.996.1 – Customer premise in-home wiring model #3

### 6.2.4	In-home wiring model for TCM-ISDN DSL environment

See Figure 15.

Figure 15/G.996.1 – In-home wiring models in an environment
coexisting with TCM-ISDN DSL

# 7	Power spectral density of crosstalk disturbers

Crosstalk margin measurements are performed with several types of disturbers, DSLs, HDSLs, T1s and ADSL lines. DSL, HDSL, and ADSL crosstalk is from pairs in the same binder group; T1 crosstalk is from pairs in an adjacent binder group.

## 7.1	Simulated DSL PSD and induced NEXT

The power spectral density (PSD) of basic access DSL disturbers is expressed as:

PSDDSL−Disturber=KDSL×2f0×sinπff02πff02×11+ff3dB4,     f3dB = 80 kHz, 0  f  

where: f0=80 kHz,KDSL=59×Vp2R, Vp  2.50 volts and R  135 ohms

This equation gives the single-sided PSD; that is, the integral of PSD, with respect to f, from 0 to infinity, gives the power in watts. PSDDSL-Disturber is the PSD of an 80 ksymbols/s 2B1Q signal with random equiprobable levels, with full-band square-topped pulses and with 2nd order Butterworth filtering (f3dB  80 kHz). Figure 16 shows the PSD of 24-disturber DSL-NEXT.

The PSD of the DSL-NEXT can be expressed as:

PSDDSL−NEXT=PSDDSL-Disturber×xn×f32     for 0  f  , n < 50

where: xn  8.818  1014  (n/49)0.6 or equivalently, xn  0.8536  1014  n0.6

The integration of PSDDSL-Disturber and PSDDSL-NEXT over various frequency ranges of interest is presented in Table 18.

Table 18/G.996.1 – DSL transmit and DSL-induced NEXT power

| Frequency range | Transmit power (dBm) | NEXT power 24-disturber (dBm) |
| --- | --- | --- |
| 0-0.16 MHz | 13.60 | –52.62 |
| 0-0.32 MHz | 13.60 | –52.62 |
| 0-1.544 MHz | 13.60 | –52.62 |
| 0-3 MHz | 13.60 | –52.62 |
| 0-10 MHz | 13.60 | –52.62 |

Figure 16/G.996.1 – 24-disturber DSL-NEXT

## 7.2	Simulated HDSL PSD and induced NEXT

The PSD of HDSL disturbers is expressed as:

PSDHDSL-Disturber=KHDSL×2f0×sinπff02πff02×11+ff3dB8,     f3dB = 196 kHz, 0  f  

where: f0=392 kHz,KDSL=59×Vp2R,     Vp  2.70 volts, and R  135 ohms

This equation gives the single-sided PSD; that is, the integral of PSD, with respect to f, from 0 to infinity, gives the power in watts. PSDHDSL-Disturber is the PSD of a 392 ksymbols/s 2B1Q signal with random equiprobable levels, with full-band square-topped pulses and with 4th order Butterworth filtering (f3dB  196 kHz ). Figure 17 shows the PSD of 10-disturber HDSL-NEXT.

Figure 17/G.996.1 – 10-disturber HDSL NEXT

The PSD of the HDSL-NEXT is expressed as:

PSDHDSL-NEXT=PSDHDSL-Disturber×xn×f32     for 0  f  , n < 50

where: f0  8.818  (n/49)0.6 or equivalently, xn  0.8536  1014  n0.6

The integration of PSDHDSL-Disturber over various frequency ranges of interest is presented in Table 19 along with the induced NEXT power.

Table 19/G.996.1 – HDSL transmit and induced NEXT power

| Frequency range | Transmit power (dBm) | NEXT power 10-disturbers (dBm) |
| --- | --- | --- |
| 0-0.196 MHz | 13.44 | –46.9 |
| 0-0.392 MHz | 13.60 | –46.3 |
| 0-0.784 MHz | 13.60 | –46.3 |
| 0-1.544 MHz | 13.60 | –46.3 |
| 0-1.568 MHz | 13.60 | –46.3 |
| 0-3 MHz | 13.60 | –46.3 |

## 7.3	Simulated T1 line PSD and induced NEXT

The PSD of the T1 line disturber is assumed to be the 50% duty-cycle random Alternate Mark Inversion (AMI) code at 1.544 Mbit/s. The single-sided PSD has the following expression.

PSDT1-Disturber=VP2RL2f0sinπff0πff02sin2πf2f011+ff3dB6f2f2+f3dB2,     (0  f  )

The total power of the transmit T1 signal is computed by:

PT1−total=14VP2RL

It is assumed that the transmitted pulse passes through a low-pass shaping filter. The shaping filter is chosen as a third order low-pass Butterworth filter with 3 dB point at 3.0 MHz. The filter magnitude squared transfer function is:

Hshaping(f)2=11+ff3dB6

In addition, the coupling transformer is modelled as a high-pass filter with 3 dB point at 40 kHz as:

HTransformer(f)2=f2f2+f3dB2

Furthermore, it is assumed that Vp  3.6 volts, RL  100 ohms, and f0  1.544 MHz.

Figure 18 shows the PSD of 4- and 24-disturber T1 NEXT.

Figure 18/G.996.1 – 4- and 24-disturber T1 NEXT

The PSD of the T1 NEXT is expressed as:

PSDT1­NEXT=PSDT1­Disturberxnf32     for 0  f  , n < 50

where: xn  8.818  1014  (n/49)0.6 or equivalently, xn  0.8536  1014  n0.6

The T1 transmit and T1-induced NEXT power using n-crosstalk models (xn) is presented in Table 20.

Table 20/G.996.1 – T1 transmit and induced NEXT power with shaping 
and coupling transformer

| Frequency range | Transmit power (dBm) | NEXT power 4-disturbers (dBm) | NEXT power 24-disturbers (dBm) |
| --- | --- | --- | --- |
| 0-1.544 MHz | 14.1 | –34.7 | –30.0 |
| 0-3 MHz | 14.57 | –32.8 | –28.1 |

For testing, the T1 interferer power and the PSD curve are adjusted downward by a total of 15.5 dB to allow for the adjacent binder effect and an averaging factor which accounts for non-collocation of the T1 and ADSL terminals.

## 7.4	Simulated G.992.1 downstream PSD and induced NEXT and FEXT

The PSD of G.992.1 disturbers is expressed as:

PSDG.992.1­Disturber=KG.992.1×2f0×sinπff02πff02×LPF(f)2×HPF(f)2,    (0  f  )

where: f0  2.208  106 Hz, KG.992.1  0.1104 watts.

This equation gives the single-sided PSD, where KG.992.1 is the total transmitted power in watts for the downstream G.992.1 transmitter before shaping filters, and is set such that the PSD will not exceed the maximum allowed PSD. f0 is the sampling frequency in Hz and

LPF(f)2=fhαfα+fhα,fh=1.104×106Hz,α=3610log(2)=11.96

is a low-pass filter with a 3 dB point at 1104 kHz and 36 dB/octave rolloff, and

HPF(f)2=fα+flαfα+fhα,fl=4000 Hz,fh=25875 Hz,α=57.510logfhfl=7.09

is a high-pass filter with 3 dB points at 4 and 25.875 kHz and 57.5 dB attenuation in the voiceband, separating G.992.1 from POTS. With this set of parameters, the PSDG.992.1 is the PSD of a downstream transmitter that uses all the sub-carriers.

### 7.4.1	FEXT

The FEXT loss model is:

HFEXT(f)2=Hchannel(f)2×k×l×f2×p

where:

Hchannel (f )	is the channel transfer function

k	is the coupling constant and is 8  1020  (n/49)0.6 for n  50 or 3.083  1020 for 10, 1% worst-case disturbers

l	is the coupling path length in metres and equals 2744 m for C #6

f	is in Hz

p	is a metres-to-feet conversion constant and is 3.28 ft/m

The FEXT noise PSD is therefore:

PSDG.992.1­FEXT=PSDG.992.1­Disturber×HFEXT(f)2

The integration of PSDG.992.1 and PSDG.992.1-FEXT over the various frequency ranges is presented in Table 21.

Table 21/G.996.1 – G.992.1 and G.992.1-FEXT power with shaping 
and coupling transformer

| Frequency range | Transmit power (dBm) | FEXT power 10-disturbers (dBm) |
| --- | --- | --- |
| 0-1.104 MHz | 18.99 | –69.61 |
| 0-2.204 MHz | 19.15 | –69.61 |
| 0-4.416 MHz | 19.15 | –69.61 |

Figure 19 shows the theoretical PSD of 10-disturber downstream G.992.1 FEXT on C loop #6.

Figure 19/G.996.1 – 10-disturber downstream G.992.1 FEXT on C loop #6

### 7.4.2	NEXT

The PSD of the G.992.1 NEXT into the upstream is defined as:

PSDG.992.1­NEXT=PSDG.992.1­Disturber×xn×f32     for 0  f  , n < 50

where: xn  8.818  1014  (n/49)0.6 or equivalently, xn  0.8536  1014  n0.6

The integration of the induced NEXT over the band from 0 to 1.104 MHz for n  49 is 25.4 dBm.

Figure 20 shows the theoretical PSD of 10-disturber downstream G.992.1 NEXT.

Figure 20/G.996.1 – 10-disturber downstream G.992.1 NEXT into the upstream

## 7.5	Simulated G.992.1 upstream PSD and induced NEXT and FEXT

The PSD of G.992.1 disturbers is expressed as:

PSDG.992.1­Disturber=KADSL×2f0×sinπff0πff02×LPF(f)2×HPF(f)2,     (0  f  )

where: f0  276  103 Hz, KG.992.1  0.02185 watts.

This equation gives the single-sided PSD, where KG.992.1 is the total transmitted power in watts for the upstream G.992.1 transmitter before shaping filters, and is set such that the PSD will not exceed the maximum allowed PSD. f0 is the sampling frequency in Hz, and

LPF(f)2=fhαfα+fhα,fh=138×103Hz,α=2410log181.125138=20.32

is a low-pass filter with a 3 dB point at 138 kHz and 24 dB attenuation at 181.125 kHz, and

HPF(f)2=fα+flαfα+fhα,fl=4000 Hz,fh=25875 Hz, α=59.510logfhfl=7.34

is a high-pass filter with 3 dB points at 4 and 25.875 kHz and 59.5 dB attenuation in the voiceband, separating G.992.1 from POTS. With this set of parameters the PSDG.992.1 is the PSD of an upstream transmitter that uses all the sub-carriers.

### 7.5.1	FEXT

The FEXT loss model is:

HFEXT(f)2=Hchannel(f)2×k×l×f2×p

where: Hchannel (f ) is the channel transfer function, k is the coupling constant and is

8  1020  (n/49)0.6

for n  50 or 3.083  1020 for 10, 1% worst-case disturbers, l is the coupling path length in metres, f is in Hz, p is a metres-to-feet conversion constant and is 3.28 ft/m.

The FEXT noise PSD is therefore:

PSDG.992.1=PSDG.992.1×HFEXT(f)2

Figure 21 shows the theoretical PSD of 10-disturber upstream G.992.1 FEXT on C loop #6.

Figure 21/G.996.1 – Theoretical 10-disturber upstream G.992.1 FEXT on C loop #6

### 7.5.2	NEXT

The upstream G.992.1 signal nominally occupies the band from 25 to 138 kHz, but the upper sidelobes of the passband signal beyond 138 kHz may also contribute to the NEXT into the downstream signal. Their effect will depend on the method of anti-aliasing used in the remote transmitter. The PSD of the upstream G.992.1 NEXT is expressed as:

PSDG.992.1=PSDG.992.1­Disturber×xn×f32     for 0  f  , n < 50

where: xn  8.818  1014  (n/49)0.6 or equivalently, xn  0.8536  1014  n0.6

Figure 22 shows the theoretical PSD of 10-disturber upstream G.992.1 NEXT into the downstream.

Figure 22/G.996.1 – 10-disturber upstream G.992.1 NEXT into the downstream

## 7.6	Simulated G.992.2 downstream NEXT and FEXT

The single-sided PSD of G.992.2 disturbers is expressed as:

PSDG.992.2­Disturber=PSDG.992.2­AnnexA–3.5 dB

### 7.6.1	FEXT

The FEXT loss model is:

HFEXT(f)2=Hchannel(f)2×k×l×f2×p

where: Hchannel (f ) is the channel transfer function, k is the coupling constant and is

8  1020  (n/49)0.6

for n  50 or 3.083  10–20 for 10, 1% worst-case disturbers, l is the coupling path length in metres, f is in Hz, p is a metres-to-feet conversion constant and is 3.28 ft/m.

The FEXT noise PSD is therefore:

PSDG.992.2­FEXT=PSDG.992.2­Disturber×HFEXT(f)2

The integration of the induced FEXT over the band from 0 to 1.104 MHz and using T loop #7 is 83.9 dBm for n  49, and is 85.8 for n  24.

Figure 23 shows the theoretical PSD of 49-disturber downstream G.992.2 FEXT on T loop #7.

Figure 23/G.996.1 – 49-disturber downstream G.992.2 FEXT on T loop #7

### 7.6.2	NEXT

The PSD of the downstream G.992.2 NEXT into the upstream is defined as:

PSDG.992.2­NEXT=PSDG.992.2­Disturber×xn×f23     for 0  f  , n < 50

where: xn  8.818  1014  (n/49)0.6 or equivalently, xn  0.8536  1014  n0.6

The integration of the induced NEXT over the band from 0 to 1.104 MHz for n  49 is –23.3 dBm, and –25.2 for n  24. Figure 24 shows the theoretical PSD of 49-disturber downstream G.992.2 NEXT into the upstream.

Figure 24/G.996.1 – 49-disturber downstream G.992.2 NEXT into the upstream

## 7.7	Simulated G.992.2 upstream NEXT and FEXT

The PSD of G.992.2 upstream disturbers is expressed as:

PSDG.992.2­Disturber=PSDG.992.2­AnnexA−3.5 dB

### 7.7.1	FEXT

The FEXT loss model is:

HFEXT(f)2=Hchannel(f)2×k×l×f2×p

where: Hchannel (f ) is the channel transfer function, k is the coupling constant and is:

8  1020  (n/49)0.6

for n  50 or 3.083  1020 for 10, 1% worst-case disturbers, l is the coupling path length in meters, f is in Hz , p is a metres-to-feet conversion constant and is 3.28 ft/m.

The FEXT noise PSD is therefore:

PSDG.992.2­FEXT=PSDG.992.2­Disturber×HFEXT(f)2

The integration of the induced FEXT over the band from 0 to 1.104 MHz for n  49 and using T loop #7 is –81.8 dBm, and –83.7 for n  24.

Figure 25 shows the theoretical PSD of 49-disturber upstream G.992.2 FEXT on T loop #7.

Figure 25/G.996.1 – Theoretical 49-disturber upstream G.992.2 FEXT on T loop #7

### 7.7.2	NEXT

The upstream G.992.2 signal nominally occupies the band from 25 to 138 kHz, but the upper sidelobes of the passband signal beyond 138 kHz may also contribute to the NEXT into the downstream signal. Their effect will depend on the method of anti-aliasing used in the remote transmitter. The PSD of the upstream G.992.2 NEXT is expressed as:

PSDG.992.2­NEXT=PSDG.992.2­Disturber×xn×f32     for 0  f < , n < 50

where: xn  8.818  1014  (n/49)0.6 or equivalently, xn  0.8536  1014  n0.6

The integration of the induced NEXT over the band from 0 to 1.104 MHz for n  49 is 43.2 dBm, and –45.1 for n  24.

Figure 26 shows the theoretical PSD of 49-disturber upstream G.992.2 NEXT into the downstream.

Figure 26/G.996.1 – Theoretical 49-disturber upstream G.992.2 NEXT into the downstream

## 7.8	ETSI A, ETSI B and Euro-K crosstalk

The power spectral density of the crosstalk noise sources used for performance testing is given in Figure 27 for ETSI A, and in Figure 28 for ETSI B. ETSI A includes discrete tones, which represent radio frequency interference that is commonly observed, especially on wire pairs routed above ground. Further details of the specification of these noise models are shown in Tables 22, 23 and 24.

The resulting wideband noise power over the frequency range 1 kHz to 1.5 MHz for model A is  0.5 dBm and for model B is –43.0 0.5 dBm.

The noise probability density function should be approximately Gaussian with a crest factor  5.

The accuracy of the power spectral density shall be within 1 dB over the frequency range 1 kHz to 1.5 MHz, when measured with a resolution bandwidth of 1 kHz.

Figure 27/G.996.1 – Single-sided noise power spectral density into 100  for ETSI A

Table 22/G.996.1 – Coordinates for noise ETSI A

| Frequency (kHz) | PSD (dBm/Hz) |
| --- | --- |
| 1 | –100 |
| 79.5 | –100 |
| 795 | –140 |
| 1500 | –140 |

Table 23/G.996.1 – Tone frequencies and powers for noise ETSI A

| Frequency (kHz) | Power (dBm) |
| --- | --- |
| 99 | –70 |
| 207 | –70 |
| 333 | –70 |
| 387 | –70 |
| 531 | –70 |
| 603 | –70 |
| 711 | –70 |
| 801 | –70 |
| 909 | –70 |
| 981 | –70 |

Figure 28/G.996.1 – Single-sided noise power spectral density into 100  for ETSI B

Table 24/G.996.1 – Coordinates for noise ETSI B

| Frequency (kHz) | PSD (dBm/Hz) | PSD (V/Hz) |
| --- | --- | --- |
| 1 | –80 | 31.62 |
| 10 | –100 | 3.16 |
| 300 | –100 | 3.16 |
| 711 | –115 | 0.56 |
| 1500 | –115 | 0.56 |

The Euro-K model has a floor at –140 dBm/Hz, its maximum at –94 dBm/Hz between 135 kHz and 260 kHz. Below 135 kHz it rolls off towards the floor at 7 dB/decade. Above 260 kHz it rolls off towards the floor at –75 dB/decade.

Note that Euro-K is based on a recalculation of ETSI A crosstalk interference at the ATU-C end. It basically removes the secondary HDSL NEXT effects which are exploited at the ATU-R end by noise ETSI A but which do not apply at the ATU-C end.

## 7.9	Power spectral density of crosstalk disturbers in an environment coexisting with TCM-ISDN DSL

### 7.9.1	Crosstalk disturber type

This clause specifies the crosstalk disturbers in an environment coexisting with TCM-ISDN DSL, in terms of the power spectral density (PSD) and crosstalk characteristics. Crosstalk margin measurements are made for the following types of disturbers from pairs in the same binder group.

1)	TCM-ISDN DSL NEXT and FEXT:

–	continuous NEXT;

–	continuous FEXT;

–	alternate NEXT and FEXT (for Annex C/G.992.2).

2)	HDSL PSD NEXT:

–	continuous NEXT.

3)	ADSL downstream NEXT and FEXT:

–	continuous NEXT;

–	continuous FEXT;

–	alternate NEXT and FEXT (for FEXT bitmap mode of Annex C/G.992.2).

4)	ADSL upstream NEXT and FEXT:

–	continuous NEXT;

–	continuous FEXT;

–	alternate NEXT and FEXT (for FEXT bitmap mode of Annex C/G.992.2).

NOTE 1 – The term "ADSL" is used to indicate both G.992.1 and G.992.2, unless otherwise stated.

NOTE 2 – TCM-ISDN DSL alternate NEXT and FEXT injection timing is defined in 7.9.3.4.

### 7.9.2	Crosstalk test parameters

The crosstalk loss as the result of multiple interfering pairs is known as the Power Sum Loss (PSL), and 99% cumulative (1% worst case) values are adopted for ADSL performance test. PSL values vary depending on the number of disturber pairs, the insulation material of the pair conductors (paper, polyethylene), and line conditioning states (pair selection conditions) for DSL installation into a cable pair in the same binder.

NEXT Power Sum Loss in dB for the disturber pair number of "n" is abbreviated to "NPSLn". FEXT Power Sum Loss in dB for the disturber pair number of "n" is abbreviated to "FPSLn". The value of "n  24" is adopted for all crosstalk tests. The values of NPSL24 and FPSL24 are summarized in Table 25 for each insulation material of the pair conductors (paper, polyethylene) and for each line conditioning states (pair selection conditions).

Table 25/G.996.1 – Power sum loss values

| NEXT/FEXT power sum loss | Line conditioning state | Interfering and interfered system attribute | Insulation material of pair conductor |  |
| --- | --- | --- | --- | --- |
|  |  |  | Paper-insulated | Polyethylene-insulated |
| NPSL24 [dB] | Intra-quad | Inter/intra-system | 47.0 dB | 49.5 dB |
| at fNXT  160  103 [Hz] | Inter-quad | Inter-system (Note 1) | 52.5 dB | 53.5 dB |
|  |  | Intra-system (Note 2) | 53.5 dB | 55.5 dB |
| FPSL24 [dB] | Intra-quad | Inter/intra-system | 45.0 dB | 51.0 dB |
| at fFXT  160  103 [Hz] | Inter-quad | Inter-system (Note 1) | 45.0 dB | 51.0 dB |
| at dFXT  1.0  103 [m] |  | Intra-system (Note 2) | 45.5 dB | 53.0 dB |
| NOTE 1 – Mutual interference between different transmission systems. NOTE 2 – Mutual interference between identical transmission systems. |  |  |  |  |

The test loops defined in 6.1.3 contain combined loops that consist of two segments: a paper-insulated cable segment and a polyethylene-insulated cable segment. In cases where such test loops consist of mixed cable type connection, the following NEXT/FEXT simulated noises should be injected.

a)	The NEXT noise should be simulated by using NEXT PSL (NPSL) values corresponding to the insulation material of the cable connected at the near end of a NEXT injection point.

b)	The FEXT noise should be simulated by a power sum of each FEXT coupling path by using the equation below, and should be injected at a far end.

PSDFEXT=PSDDisturber×kHchannel[k]f,dk2×                 ×k10−FPSLn[k]10×d[k]×dFXT−1f2fFXT−2

where:

Hchannel[k](f,d[k]):	channel transfer function of each cable segment [k] consisting test loop

d[k]:	distance of each cable segment [k] consisting test loop

FPSLn[k]:	FEXT PSL value of each cable segment [k] consisting test loop

The circuit for crosstalk injection shall be calibrated for 0.4 dB less crosstalk than specified in this clause in order to compensate for the use of 100-ohm terminations of ADSL instead of 110 ohms of TCM-ISDN DSL for TCM-ISDN DSL crosstalk injection testing. The circuit shall also be calibrated for 1.3 dB less crosstalk than specified in this clause in order to compensate for the use of 100-ohm terminations of ADSL instead of 135 ohms of HDSL for HDSL crosstalk injection testing.

The above calibration can be described in the equation below.

PSDinjectionNEXT(or)FEXT=PSDNEXT(or)FEXT×Z0interferedZ0interfering

where:

PSD[injection]NEXT(or)FEXT:	PSD for injection in the receiver

PSDNEXT(or)FEXT:	simulated PSD given in 7.9

Z0interfered:	termination impedance of interfered system

 100-ohm terminations of ADSL

Z0interfering:	termination impedance of interfering system

 100-ohm terminations of TCM-ISDN DSL

 135-ohm terminations of HDSL

### 7.9.3	Simulated TCM-ISDN DSL PSD and induced NEXT and FEXT

#### 7.9.3.1	TCM-ISDN DSL PSD

The PSD of Basic Rate Access (BRA) DSL disturbers using a Time Compression Multiplex (TCM) method is expressed as:

PSDDSL­Disturber=KDSL×2f0×sinπff02×sinπf2f02πf2f02×11+ff3dB4,     (0  f  )

where: f in Hz, f0  320  103 Hz,  f3dB  2  f0, KDSL=V0p24R, V0p  6.00 volts and R  110 ohms

The equation PSDDSL-Disturber gives the single-sided PSD of a 320 kBaud Alternate Mark Inversion (AMI) signal with random continuous symbol sequence, with 50% duty-cycle pulses and with 2nd order low-pass Butterworth filtering (f3dB  640 kHz).

#### 7.9.3.2	TCM-ISDN DSL NEXT

The PSD of the DSL NEXT can be expressed as:

PSDDSL­NEXT=PSDDSL­Disturber×10−NPSLn10×fNXT      −32f32,     (0  f  )

where: f in Hz,  fNXT  160  103 Hz

The integration of PSDDSL-Disturber and PSDDSL-NEXT over various frequency ranges is presented in Table 26 for the case of the paper-insulated cable with intra-quad line conditioning state.

Table 26/G.996.1 – TCM-ISDN DSL transmit and induced NEXT power

| Frequency range | Transmit power | NEXT power (NPSL24  47.0 dB) |
| --- | --- | --- |
| 0 ~ 640 kHz | 18.6 dBm | –27.3 dBm |
| 0 ~ 1280 kHz | 18.6 dBm | –26.8 dBm |
| 0 ~ 1920 kHz | 18.6 dBm | –26.7 dBm |
| 0 ~ 2560 kHz | 18.6 dBm | –26.7 dBm |
| 0 ~ 3200 kHz | 18.6 dBm | –26.7 dBm |

Figure 29/G.996.1 – 24-disturber TCM-ISDN DSL NEXT

#### 7.9.3.3	TCM-ISDN DSL FEXT

The PSD of the DSL FEXT can be expressed as:

PSDDSL­FEXT=PSDDSL­Disturber×Hchannel(f,d)2×                     ×10−FPSLn10×dFXT−1fFXT−2d×f2,     (0  f  )

where: f in Hz, d in metre, fFXT  160  103 Hz, dFXT  1.0  103 metres

Hchannel (f,d) is the channel transfer function; "d" is the coupling path distance in metres, and depends on test loops defined in 6.1.3. The power sum of PSDDSL-FEXT is required for the test loops consisting of mixed cable type connection, as described in 7.9.2.

The integration of PSDDSL-Disturber and PSDDSL-FEXT over various frequency ranges is presented in Table 27 for the case of the 0.4 mm gauge paper insulated cable with intra-quad line conditioning state and with d  2.07  103, 2.94  103, 3.97  103, and 5.16  103 metres.

Table 27/G.996.1 – TCM-ISDN DSL transmit and induced FEXT power

| Frequency range | Transmit power | FEXT power (FPSL24  45.0 dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | d  2.07 km | d  2.94 km | d  3.97 km | d  5.16 km |
| 0 ~ 640 kHz | 18.6 dBm | –50.3 dBm | –59.5 dBm | –70.4 dBm | –82.7 dBm |
| 0 ~ 1280 kHz | 18.6 dBm | –50.3 dBm | –59.5 dBm | –70.4 dBm | –82.7 dBm |
| 0 ~ 1920 kHz | 18.6 dBm | –50.3 dBm | –59.5 dBm | –70.4 dBm | –82.7 dBm |
| 0 ~ 2560 kHz | 18.6 dBm | –50.3 dBm | –59.5 dBm | –70.4 dBm | –82.7 dBm |
| 0 ~ 3200 kHz | 18.6 dBm | –50.3 dBm | –59.5 dBm | –70.4 dBm | –82.7 dBm |

Figure 30/G.996.1 – 24-Disturber TCM-ISDN DSL FEXT

#### 7.9.3.4	TCM-ISDN DSL alternate NEXT and FEXT injection timing

For testing transmission performance of an ADSL system conforming to G.992.1/Annex C/G.992.2 (ADSL operating in an environment coexisting with TCM-ISDN DSL), the calibrated simulated TCM-ISDN DSL NEXT/FEXT PSD defined above shall be alternately injected in burst mode as specified in Figure 31, which is the worst-case simplified model shown in relation to TTR (TCM-ISDN Timing Reference described in G.992.1/Annex C/G.992.2), and is derived from the TCM-ISDN DSL signal transceiver scheme.

Figure 31/G.996.1 – TCM-ISDN DSL alternate NEXT and FEXT injection timing

NOTE 1 – For testing transmission performance of an ADSL system conforming to G.992.1/G.992.2 main body, it is permitted to inject TCM-ISDN DSL NEXT/FEXT PSD in continuous mode. In this case, the specified error threshold for ADSL with TCM-ISDN DSL crosstalk interference is a BER of 2.12  objective-BER. This is because the TCM-ISDN DSL transmits signals in burst mode. The ratio of burst signal transmitting period to the burst repetition period of 2.5 ms is 377-pulse-periods/800-pulse-periods ( 1/2.12), and the transmit signal in burst mode only causes NEXT/FEXT noise.

NOTE 2 – The FEXT injection timing in Figure 31 is also applicable for testing intra-system crosstalk interference of ADSL system conforming G.992.1/Annex C/G.992.2 FBM. In this test, TCM-ISDN DSL FEXT in ATU-C side should be replaced by ADSL upstream FEXT defined in 7.9.6.3. Also, TCM-ISDN DSL FEXT in ATU-R side should be replaced by ADSL downstream FEXT defined in 7.9.5.3. TCM-ISDN DSL NEXT in ATU-C and ATU-R sides should be replaced by no noise.

### 7.9.4	Simulated HDSL PSD and induced NEXT

#### 7.9.4.1	HDSL PSD

The PSD of HDSL disturbers is expressed as:

PSDHDSL­Disturber=KHDSL×2f0×sinπff02πff02×11+ff3dB8,     (0  f  )

where:

f in Hz,  f0  392  103 Hz,  f3dB  f02, KHDSL=59×V0p2R, V0p  2.70 volts and R  135 ohms

The equation PSDHDSL-Disturber gives the single-sided PSD of a 392 kBaud 2B1Q signal with random continuous equiprobable level symbol sequence, with 100% duty-cycle pulses and with 4th order low-pass Butterworth filtering (f3dB  196 kHz).

#### 7.9.4.2	HDSL NEXT

The PSD of HDSL NEXT can be expressed as:

PSDHDSL­NEXT=PSDHDSL­Disturber×10−NPSLn10×fNXT      −32f32     (0  f  )

where: f in Hz,  fNXT  160 103 Hz

The integration of PSDHDSL-Disturber and PSDHDSL-NEXT over various frequency ranges is presented in Table 28 for the case of the paper-insulated cable with intra-quad line conditioning state.

Table 28/G.996.1 – HDSL transmit and induced NEXT power

| Frequency range | Transmit power | NEXT power (NPSL24  47.0 dB) |
| --- | --- | --- |
| 0 ~ 196 kHz | 13.4 dBm | –37.4 dBm |
| 0 ~ 392 kHz | 13.6 dBm | –36.8 dBm |
| 0 ~ 784 kHz | 13.6 dBm | –36.8 dBm |
| 0 ~ 1568 kHz | 13.6 dBm | –36.8 dBm |
| 0 ~ 3136 kHz | 13.6 dBm | –36.8 dBm |

Figure 32/G.996.1 – 24-disturber HDSL NEXT

### 7.9.5	Simulated ADSL downstream PSD and induced NEXT and FEXT

#### 7.9.5.1	ADSL downstream PSD

The PSD of the G.992.1/G.992.2 disturbers is expressed as:

PSDADSL,ds­Disturber=KADSL,ds×2f0×sinπff02πff02×11+ffLP3dB12×11+fHP3dBfN,

(0  f  )

where:

f in Hz, f0 	2.208  106 Hz, fLP3dB  f02 (for G.992.1),  fLP3dB  f04 (for G.992.2)

fHP3dB 	25.875  103 Hz, N  8 (for EC)

fHP3dB 	138  103 Hz, N  16 (for FDM)

KADSL,ds 	0.1104 watts (corresponding to the maximum passband PSD of 40 dBm/Hz)

The equation PSDADSL,ds-Disturber gives the single-sided PSD with 6th order low-pass Butterworth filtering [fLP3dB  1104 kHz (G.992.1) or 552 kHz (G.992.2)], rejecting stop-band PSD and with Nth order high-pass filtering [fHP3dB  25.875 kHz, N  8 (EC) or 138 kHz, N  16 (FDM)], separating ADSL signal from POTS signal for EC, or separating ADSL downstream signal from ADSL upstream signal for FDM.

#### 7.9.5.2	ADSL downstream NEXT into the upstream

The PSD of the G.992.1/G.992.2 downstream NEXT into the upstream can be expressed as:

PSDADSL,ds­NEXT=PSDADSL,ds­Disturber×10−NPSLn10×fNXT      −32f32,     (0  f  )

where: f in Hz, fNXT  160  103 Hz

The integration of PSDADSL,ds-NEXT over various frequency ranges is presented in Table 29 (G.992.1, FDM) and in Table 30 (G.992.2, FDM) for the case of the paper-insulated cable with intra-quad line conditioning state.

Table 29/G.996.1 – G.992.1 FDM downstream transmit and induced NEXT power

| Frequency range | Transmit power | NEXT power (NPSL24  47.0 dB) |
| --- | --- | --- |
| 0 ~ 1104 kHz | 18.4 dBm | –20.4 dBm |
| 0 ~ 2208 kHz | 18.5 dBm | –20.0 dBm |
| 0 ~ 4416 kHz | 18.5 dBm | –20.0 dBm |

Table 30/G.996.1 – G.992.2 FDM downstream transmit and induced NEXT power

| Frequency range | Transmit power | NEXT power (NPSL24  47.0 dB) |
| --- | --- | --- |
| 0 ~ 552 kHz | 15.5 dBm | –26.7 dBm |
| 0 ~ 1104 kHz | 15.8 dBm | –25.9 dBm |
| 0 ~ 2208 kHz | 15.8 dBm | –25.9 dBm |

Figure 33/G.996.1 – 24-disturber G.992.1 (FDM) downstream NEXT

Figure 34/G.996.1 – 24-disturber G.992.2 (FDM) downstream NEXT

#### 7.9.5.3	ADSL downstream FEXT into the downstream

The PSD of the G.992.1/G.992.2 downstream FEXT into the downstream can be expressed as:

PSDADSL,ds­FEXT=PSDADSL,ds­Disturber×Hchannel(f,d)2× ×10−FPSLn10×dFXT−1fFXT−2d×f2,     (0  f  )

where: f in Hz, d in metre, fFXT  160  103 Hz, dFXT  1.0  103 metres

Hchannel (f, d) is the channel transfer function, "d" is the coupling path distance in metres. The power sum of PSDADSL,ds-FEXT for a combined test loop is described in 7.9.2.

The integration of PSDADSL,ds-Disturber and PSDADSL,ds-FEXT over various frequency ranges is presented in Table 31 (G.992.1, FDM) and in Table 32 (G.992.2, FDM) for the case of the 0.4 mm gauge paper-insulated cable with intra-quad line conditioning state and with d  2.07  103, 2.94  103, 3.97  103, and 5.16  103 metres.

Table 31/G.996.1 – G.992.1 FDM downstream transmit and induced FEXT power

| Frequency range | Transmit power | FEXT power (FPSL24  45.0 dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | d  2.07 km | d  2.94 km | d  3.97 km | d  5.16 km |
| 0 ~ 1104 kHz | 18.4 dBm | –54.9 dBm | –66.5 dBm | –79.5 dBm | –94.2 dBm |
| 0 ~ 2208 kHz | 18.5 dBm | –54.9 dBm | –66.5 dBm | –79.5 dBm | –94.2 dBm |
| 0 ~ 4416 kHz | 18.5 dBm | –54.9 dBm | –66.5 dBm | –79.5 dBm | –94.2 dBm |

Table 32/G.996.1 – G.992.2 FDM downstream transmit and induced FEXT power

| Frequency range | Transmit power | FEXT power (FPSL24  45.0 dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | d  2.07 km | d  2.94 km | d  3.97 km | d  5.16 km |
| 0 ~ 552 kHz | 15.5 dBm | –55.2 dBm | –66.5 dBm | –79.5 dBm | –94.2 dBm |
| 0 ~ 1105 kHz | 15.8 dBm | –55.1 dBm | –66.5 dBm | –79.5 dBm | –94.2 dBm |
| 0 ~ 2208 kHz | 15.8 dBm | –55.1 dBm | –66.5 dBm | –79.5 dBm | –94.2 dBm |

Figure 35/G.996.1 – 24-disturber G.992.1 (FDM) downstream FEXT

Figure 36/G.996.1 – 24-disturber G.992.2 (FDM) downstream FEXT

### 7.9.6	Simulated ADSL upstream PSD and induced NEXT and FEXT

#### 7.9.6.1	ADSL upstream PSD

The single-sided PSD of the G.992.1/G.992.2 upstream disturbers can be assumed as:

PSDADSL,us­Disturber=KADSL,us×2f0×sinπff02πff02×11+ffLP3dB16×11+fHP3dBf8,

(0  f  )

where: f in Hz, f0  276  103 Hz, fLP3dB  138  103, fHP3dB  25.875  103 Hz,

KADSL,us  0.02187 watts (corresponding to the maximum pass band PSD of 38 dBm/Hz)

#### 7.9.6.2	ADSL upstream NEXT into the downstream

The PSD of the G.992.1/G.992.2 upstream NEXT into the downstream can be expressed as:

PSDADSL,us­NEXT=PSDADSL,us­Disturber×10−NPSLa10×fNXT       −32f32,     (0  f  )

where: f in Hz, fNXT  160  103 Hz

The integration of PSDADSL,us-Disturber and PSDADSL,us-NEXT over various frequency ranges is presented in Table 33 for the case of a paper-insulated cable with intra-quad line conditioning state.

Table 33/G.996.1 – G.992.1/G.992.2 upstream transmit and induced NEXT power

| Frequency range | Transmit power | NEXT power (NPSL24  47.0 dB) |
| --- | --- | --- |
| 0 ~ 138.000 kHz | 10.9 dBm | –40.9 dBm |
| 0 ~ 181.125 kHz | 11.0 dBm | –40.6 dBm |
| 0 ~ 224.250 kHz | 11.0 dBm | –40.6 dBm |

Figure 37/G.996.1 – 24-disturber G.992.1/G.992.2 upstream NEXT

#### 7.9.6.3	ADSL upstream FEXT into the upstream

The PSD of the G.992.1/G.992.2 upstream FEXT into the upstream can be expressed as:

PSDADSL,us­FEXT=PSDADSL,us­Disturber×Hchannel(f,d)2× ×10−FPSLn10×dFXT−1fFXT−2d×f2,     (0  f  )

where: f in Hz, d in metre, fFXT  160  103 Hz, dFXT  1.0  103 metres

Hchannel (f, d) is the channel transfer function, "d" is the coupling path distance in metres. The power sum of PSDADSL,us-FEXT for a combined test loop is described in 7.9.2.

The integration of PSDADSL,us-Disturber and PSDADSL,us-FEXT over various frequency ranges is presented in Table 34 for the case of the 0.4 mm gauge paper-insulated cable with intra-quad line conditioning state and with d  2.07  103, 2.94  103, 3.97  103, and 5.16  103 metres.

Table 34/G.996.1 – G.992.1/G.992.2 upstream transmit and induced FEXT power

| Frequency range | Transmit power | FEXT power (FPSL24  45.0 dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | d  2.07 km | d  2.94 km | d  3.97 km | d  5.16 km |
| 0 ~ 138.000 kHz | 10.9 dBm | –59.1 dBm | –66.5 dBm | –75.5 dBm | –86.0 dBm |
| 0 ~ 181.125 kHz | 11.0 dBm | –58.9 dBm | –66.4 dBm | –75.4 dBm | –85.9 dBm |
| 0 ~ 224.250 kHz | 11.0 dBm | –58.9 dBm | –66.4 dBm | –75.4 dBm | –85.9 dBm |

Figure 38/G.996.1 – 24-disturber G.992.1/G.992.2 upstream FEXT

# 8	Characteristics of impulse noise waveforms

The two test impulse waveforms are shown in Figures 39 and 40. Tables 35 and 36 contain the impulse wave amplitude given in millivolts at 160 nanosecond time intervals.

Figure 39/G.996.1 – Test impulse 1

Figure 40/G.996.1 – Test impulse 2

| Table 35/G.996.1 – Impulse number 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Interval # | Amplitude mV | Interval # | Amplitude mV | Interval # | Amplitude mV |
| 1 | 0.0000 | 51 | 6.3934 | 101 | 0.1598 |
| 2 | 0.0000 | 52 | 1.7582 | 102 | 1.7582 |
| 3 | 0.0000 | 53 | 2.2377 | 103 | 0.1598 |
| 4 | 0.0000 | 54 | 4.9549 | 104 | 0.4795 |
| 5 | 0.0000 | 55 | 2.2377 | 105 | 1.2787 |
| 6 | 0.0000 | 56 | 1.7582 | 106 | 0.7992 |
| 7 | 0.0000 | 57 | 5.5943 | 107 | 1.2787 |
| 8 | 0.0000 | 58 | 1.4385 | 108 | 0.7992 |
| 9 | 0.0000 | 59 | 2.3975 | 109 | 0.0000 |
| 10 | 0.9590 | 60 | 3.6762 | 110 | 0.3197 |
| 11 | 0.4795 | 61 | 1.4385 | 111 | 2.2377 |
| 12 | 1.2787 | 62 | 0.4795 | 112 | 1.1188 |
| 13 | 1.1188 | 63 | 5.7541 | 113 | 0.7992 |
| 14 | 1.4385 | 64 | 0.4795 | 114 | 1.5984 |
| 15 | 1.5984 | 65 | 0.3197 | 115 | 0.1598 |
| 16 | 2.2377 | 66 | 3.3566 | 116 | 0.4795 |
| 17 | 1.4385 | 67 | 2.3975 | 117 | 0.9590 |
| 18 | 7.6721 | 68 | 2.3975 | 118 | 0.0000 |
| 19 | 6.7131 | 69 | 3.1967 | 119 | 0.3197 |
| 20 | 16.6229 | 70 | 0.7992 | 120 | 1.5984 |
| 21 | 12.9467 | 71 | 0.6393 | 121 | 0.0000 |
| 22 | 18.7008 | 72 | 3.5164 | 122 | 0.4795 |
| 23 | 9.5902 | 73 | 1.1188 | 123 | 0.7992 |
| 24 | 13.5861 | 74 | 1.7582 | 124 | 0.4795 |
| 25 | 5.2746 | 75 | 2.3975 | 125 | 0.7992 |
| 26 | 6.3934 | 76 | 1.2787 | 126 | 0.9590 |
| 27 | 1.9180 | 77 | 0.9590 | 127 | 0.9590 |
| 28 | 23.0164 | 78 | 3.3566 | 128 | 0.4795 |
| 29 | 3.9959 | 79 | 0.0000 | 129 | 0.6393 |
| 30 | 23.4959 | 80 | 0.1598 | 130 | 0.4795 |
| 31 | 3.1967 | 81 | 3.0369 | 131 | 1.1188 |
| 32 | 4.3156 | 82 | 1.1188 | 132 | 0.0000 |
| 33 | 3.0369 | 83 | 1.5984 | 133 | 0.0000 |
| 34 | 10.7090 | 84 | 2.0779 | 134 | 0.0000 |
| 35 | 2.2377 | 85 | 0.1598 | 135 | 0.0000 |
| 36 | 12.9467 | 86 | 0.3197 | 136 | 0.0000 |
| 37 | 3.1967 | 87 | 2.5574 | 137 | 0.0000 |
| 38 | 1.9180 | 88 | 0.1598 | 138 | 0.0000 |
| 39 | 9.9098 | 89 | 0.1598 | 139 | 0.0000 |
| 40 | 5.5943 | 90 | 2.0779 | 140 | 0.0000 |
| 41 | 5.9139 | 91 | 0.6393 |  |  |
| 42 | 6.7131 | 92 | 0.9590 |  |  |
| 43 | 2.3975 | 93 | 1.7582 |  |  |
| 44 | 1.2787 | 94 | 0.1598 |  |  |
| 45 | 8.4713 | 95 | 0.6393 |  |  |
| 46 | 2.5574 | 96 | 3.0369 |  |  |
| 47 | 2.8771 | 97 | 0.3197 |  |  |
| 48 | 6.0738 | 98 | 0.4795 |  |  |
| 49 | 2.2377 | 99 | 1.4385 |  |  |
| 50 | 1.7582 | 100 | 0.4795 |  |  |

| Table 36/G.996.1 – Impulse number 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Interval # | Amplitude mV | Interval # | Amplitude mV | Interval # | Amplitude mV |
| 1 | 0.0000 | 51 | 0.6404 | 101 | 0.6404 |
| 2 | 0.0000 | 52 | 15.5295 | 102 | 0.6404 |
| 3 | 0.0000 | 53 | 18.8916 | 103 | 0.4803 |
| 4 | 0.0000 | 54 | 3.8424 | 104 | 0.3202 |
| 5 | 0.0000 | 55 | 3.0419 | 105 | 0.9606 |
| 6 | 0.0000 | 56 | 11.6872 | 106 | 2.8818 |
| 7 | 0.0000 | 57 | 0.3202 | 107 | 2.5616 |
| 8 | 0.0000 | 58 | 7.5246 | 108 | 0.8005 |
| 9 | 0.0000 | 59 | 13.4483 | 109 | 0.4803 |
| 10 | 0.6404 | 60 | 18.4113 | 110 | 0.8005 |
| 11 | 0.9606 | 61 | 0.4803 | 111 | 0.4803 |
| 12 | 0.1601 | 62 | 3.0419 | 112 | 0.9606 |
| 13 | 5.4433 | 63 | 9.7660 | 113 | 1.1207 |
| 14 | 12.3276 | 64 | 11.2069 | 114 | 0.6404 |
| 15 | 12.1675 | 65 | 4.0025 | 115 | 0.4803 |
| 16 | 0.0000 | 66 | 0.6404 | 116 | 0.9606 |
| 17 | 5.2832 | 67 | 0.6404 | 117 | 1.4409 |
| 18 | 0.1601 | 68 | 1.7611 | 118 | 1.6010 |
| 19 | 20.8128 | 69 | 3.3621 | 119 | 1.2808 |
| 20 | 45.3078 | 70 | 5.6034 | 120 | 0.9606 |
| 21 | 46.7487 | 71 | 7.8448 | 121 | 0.9606 |
| 22 | 28.9778 | 72 | 2.5616 | 122 | 1.2808 |
| 23 | 13.4483 | 73 | 4.6428 | 123 | 1.1207 |
| 24 | 0.6404 | 74 | 0.6404 | 124 | 1.1207 |
| 25 | 0.9606 | 75 | 10.7266 | 125 | 1.4409 |
| 26 | 14.4089 | 76 | 8.3251 | 126 | 1.4409 |
| 27 | 13.7685 | 77 | 1.9212 | 127 | 1.4409 |
| 28 | 9.4458 | 78 | 3.6823 | 128 | 2.0813 |
| 29 | 17.4507 | 79 | 4.3227 | 129 | 2.4015 |
| 30 | 2.5616 | 80 | 0.3202 | 130 | 1.9212 |
| 31 | 26.5763 | 81 | 2.7217 | 131 | 1.4409 |
| 32 | 16.1699 | 82 | 7.2044 | 132 | 1.1207 |
| 33 | 17.7709 | 83 | 3.2020 | 133 | 1.2808 |
| 34 | 17.1305 | 84 | 2.7217 | 134 | 1.9212 |
| 35 | 13.6084 | 85 | 1.4409 | 135 | 2.2414 |
| 36 | 27.0566 | 86 | 1.2808 | 136 | 2.2414 |
| 37 | 18.0911 | 87 | 1.4409 | 137 | 2.5616 |
| 38 | 14.2488 | 88 | 0.8005 | 138 | 3.0419 |
| 39 | 5.6034 | 89 | 0.1601 | 139 | 3.0419 |
| 40 | 8.1650 | 90 | 0.0000 | 140 | 2.5616 |
| 41 | 12.4877 | 91 | 1.1207 | 141 | 1.2808 |
| 42 | 37.3029 | 92 | 1.1207 | 142 | 0.1601 |
| 43 | 9.6059 | 93 | 0.6404 | 143 | 0.6404 |
| 44 | 18.8916 | 94 | 1.1207 | 144 | 2.5616 |
| 45 | 5.1231 | 95 | 0.6404 | 145 | 3.2020 |
| 46 | 22.2537 | 96 | 1.1207 | 146 | 3.0419 |
| 47 | 1.1207 | 97 | 0.8005 | 147 | 2.5616 |
| 48 | 0.9606 | 98 | 0.1601 | 148 | 2.0813 |
| 49 | 20.4926 | 99 | 1.2808 | 149 | 1.4409 |
| 50 | 14.2488 | 100 | 1.4409 | 150 | 1.6010 |
| 151 | 1.9212 | 201 | 0.8005 | 251 | 1.2808 |
| 152 | 1.9212 | 202 | 0.9606 | 252 | 1.6010 |
| 153 | 2.0813 | 203 | 1.6010 | 253 | 1.6010 |
| 154 | 2.4015 | 204 | 2.4015 | 254 | 1.4409 |
| 155 | 2.5616 | 205 | 2.5616 | 255 | 0.4803 |
| 156 | 2.5616 | 206 | 2.8818 | 256 | 0.4803 |
| 157 | 1.9212 | 207 | 2.7217 | 257 | 0.4803 |
| 158 | 1.6010 | 208 | 1.9212 | 258 | 0.4803 |
| 159 | 1.6010 | 209 | 1.1207 | 259 | 0.9606 |
| 160 | 1.9212 | 210 | 0.9606 | 260 | 1.1207 |
| 161 | 1.9212 | 211 | 1.1207 | 261 | 1.4409 |
| 162 | 2.0813 | 212 | 1.4409 | 262 | 1.2808 |
| 163 | 2.2414 | 213 | 1.7611 | 263 | 0.1601 |
| 164 | 2.5616 | 214 | 2.4015 | 264 | 0.3202 |
| 165 | 2.7217 | 215 | 2.5616 | 265 | 0.0000 |
| 166 | 2.2414 | 216 | 2.2414 | 266 | 0.4803 |
| 167 | 1.2808 | 217 | 1.7611 | 267 | 0.4803 |
| 168 | 1.2808 | 218 | 1.7611 | 268 | 0.4803 |
| 169 | 2.2414 | 219 | 1.4409 | 269 | 0.6404 |
| 170 | 3.0419 | 220 | 0.9606 | 270 | 0.4803 |
| 171 | 2.8818 | 221 | 0.8005 | 271 | 0.1601 |
| 172 | 2.5616 | 222 | 0.9606 | 272 | 0.0000 |
| 173 | 2.2414 | 223 | 1.6010 | 273 | 0.0000 |
| 174 | 1.9212 | 224 | 2.2414 | 274 | 0.1601 |
| 175 | 1.9212 | 225 | 2.4015 | 275 | 0.1601 |
| 176 | 2.2414 | 226 | 2.2414 | 276 | 0.4803 |
| 177 | 2.5616 | 227 | 1.9212 | 277 | 0.6404 |
| 178 | 2.7217 | 228 | 1.4409 | 278 | 0.3202 |
| 179 | 2.5616 | 229 | 0.4803 | 279 | 0.1601 |
| 180 | 2.4015 | 230 | 0.0000 | 280 | 0.4803 |
| 181 | 2.2414 | 231 | 0.6404 | 281 | 0.3202 |
| 182 | 2.0813 | 232 | 1.6010 | 282 | 0.1601 |
| 183 | 1.7611 | 233 | 1.7611 | 283 | 0.3202 |
| 184 | 1.6010 | 234 | 1.6010 | 284 | 0.4803 |
| 185 | 1.7611 | 235 | 1.9212 | 285 | 0.6404 |
| 186 | 2.2414 | 236 | 1.9212 | 286 | 0.4803 |
| 187 | 3.0419 | 237 | 1.4409 | 287 | 0.1601 |
| 188 | 3.2020 | 238 | 0.4803 | 288 | 0.6404 |
| 189 | 2.7217 | 239 | 0.0000 | 289 | 0.6404 |
| 190 | 1.9212 | 240 | 0.0000 | 290 | 0.4803 |
| 191 | 1.2808 | 241 | 0.6404 | 291 | 0.0000 |
| 192 | 0.9606 | 242 | 1.6010 | 292 | 0.6404 |
| 193 | 1.1207 | 243 | 2.4015 | 293 | 0.6404 |
| 194 | 2.0813 | 244 | 1.9212 | 294 | 0.4803 |
| 195 | 2.8818 | 245 | 0.9606 | 295 | 0.1601 |
| 196 | 3.0419 | 246 | 0.4803 | 296 | 0.4803 |
| 197 | 2.7217 | 247 | 0.1601 | 297 | 0.6404 |
| 198 | 2.7217 | 248 | 0.1601 | 298 | 0.4803 |
| 199 | 2.0813 | 249 | 0.0000 | 299 | 0.6404 |
| 200 | 1.4409 | 250 | 0.8005 | 300 | 0.4803 |
| 301 | 0.1601 | 351 | 0.8005 | 401 | 0.9606 |
| 302 | 0.9606 | 352 | 1.4409 | 402 | 0.6404 |
| 303 | 0.9606 | 353 | 1.6010 | 403 | 0.4803 |
| 304 | 0.1601 | 354 | 1.2808 | 404 | 0.6404 |
| 305 | 0.6404 | 355 | 0.6404 | 405 | 0.6404 |
| 306 | 0.8005 | 356 | 0.0000 | 406 | 0.4803 |
| 307 | 0.8005 | 357 | 0.4803 | 407 | 0.3202 |
| 308 | 0.4803 | 358 | 0.6404 | 408 | 0.1601 |
| 309 | 0.1601 | 359 | 0.0000 | 409 | 0.3202 |
| 310 | 0.1601 | 360 | 0.8005 | 410 | 0.4803 |
| 311 | 0.3202 | 361 | 1.4409 | 411 | 0.9606 |
| 312 | 0.1601 | 362 | 1.6010 | 412 | 1.2808 |
| 313 | 0.0000 | 363 | 1.2808 | 413 | 0.9606 |
| 314 | 0.1601 | 364 | 0.6404 | 414 | 0.1601 |
| 315 | 0.6404 | 365 | 0.0000 | 415 | 0.1601 |
| 316 | 0.8005 | 366 | 0.4803 | 416 | 0.0000 |
| 317 | 0.6404 | 367 | 0.1601 | 417 | 0.4803 |
| 318 | 0.4803 | 368 | 0.1601 | 418 | 0.8005 |
| 319 | 0.0000 | 369 | 0.9606 | 419 | 0.6404 |
| 320 | 0.4803 | 370 | 1.4409 | 420 | 0.4803 |
| 321 | 0.4803 | 371 | 1.6010 | 421 | 0.8005 |
| 322 | 0.1601 | 372 | 1.1207 | 422 | 0.8005 |
| 323 | 0.8005 | 373 | 0.3202 | 423 | 0.4803 |
| 324 | 0.8005 | 374 | 0.4803 | 424 | 0.1601 |
| 325 | 0.6404 | 375 | 0.4803 | 425 | 0.0000 |
| 326 | 0.1601 | 376 | 0.1601 | 426 | 0.0000 |
| 327 | 0.4803 | 377 | 0.8005 | 427 | 0.1601 |
| 328 | 0.4803 | 378 | 1.1207 | 428 | 0.3202 |
| 329 | 0.3202 | 379 | 1.1207 | 429 | 0.6404 |
| 330 | 0.3202 | 380 | 0.9606 | 430 | 0.9606 |
| 331 | 0.4803 | 381 | 0.6404 | 431 | 0.8005 |
| 332 | 0.0000 | 382 | 0.1601 | 432 | 0.3202 |
| 333 | 0.6404 | 383 | 0.0000 | 433 | 0.1601 |
| 334 | 1.1207 | 384 | 0.1601 | 434 | 0.0000 |
| 335 | 1.2808 | 385 | 0.6404 | 435 | 0.1601 |
| 336 | 0.6404 | 386 | 1.1207 | 436 | 0.1601 |
| 337 | 0.1601 | 387 | 0.9606 | 437 | 0.1601 |
| 338 | 0.1601 | 388 | 0.6404 | 438 | 0.1601 |
| 339 | 0.0000 | 389 | 0.6404 | 439 | 0.6404 |
| 340 | 0.0000 | 390 | 0.6404 | 440 | 1.1207 |
| 341 | 0.1601 | 391 | 0.3202 | 441 | 0.9606 |
| 342 | 0.3202 | 392 | 0.0000 | 442 | 0.4803 |
| 343 | 0.8005 | 393 | 0.4803 | 443 | 0.0000 |
| 344 | 1.2808 | 394 | 1.1207 | 444 | 0.3202 |
| 345 | 1.2808 | 395 | 1.1207 | 445 | 0.3202 |
| 346 | 0.9606 | 396 | 0.6404 | 446 | 0.0000 |
| 347 | 0.1601 | 397 | 0.1601 | 447 | 0.1601 |
| 348 | 0.8005 | 398 | 0.0000 | 448 | 0.6404 |
| 349 | 0.9606 | 399 | 0.1601 | 449 | 0.9606 |
| 350 | 0.1601 | 400 | 0.8005 | 450 | 0.8005 |
| 451 | 0.6404 | 461 | 0.0000 | 471 | 0.0000 |
| 452 | 0.0000 | 462 | 0.9606 | 472 | 0.0000 |
| 453 | 0.8005 | 463 | 1.1207 | 473 | 0.0000 |
| 454 | 0.8005 | 464 | 0.4803 | 474 | 0.0000 |
| 455 | 0.0000 | 465 | 0.4803 | 475 | 0.0000 |
| 456 | 0.4803 | 466 | 1.1207 | 476 | 0.0000 |
| 457 | 0.6404 | 467 | 1.1207 | 477 | 0.0000 |
| 458 | 0.6404 | 468 | 0.6404 | 478 | 0.0000 |
| 459 | 0.8005 | 469 | 0.0000 | 479 | 0.0000 |
| 460 | 0.6404 | 470 | 0.0000 | 480 | 0.0000 |

# 9	Phone model structures for testing DSL systems without a splitter at the ATU-R

## 9.1	Off-hook phone model #1

Off-hook phone model #1 was generated for testing in the TCM-ISDN noise environment. It could be used for testing in other noise environments as well.

Figures 41, 42 and 43 give the measured off-hook impedance for off-hook phone 1, the measured and modelled off-hook impedance for off-hook phone 1, and the off-hook impedance for off-hook phone model 1.

Figure 41/G.996.1 – Measured off-hook impedance for off-hook phone #1

Figure 42/G.996.1 – Measured and modelled off-hook impedance for off-hook phone #1

Figure 43/G.996.1 – Off-hook impedance for off-hook phone model #1

## 9.2	Off-hook phone model #2

Off-hook phone model #2 (see Figures 44 and 45) was generated for testing in the splitterless North American noise environment. It could be used for testing in other noise environments as well.

Figure 44/G.996.1 – Simple model of an off-hook telephone

Figure 45/G.996.1 – Off-hook impedance of measured telephone compared with model

## 9.3	On-hook phone model

This on-hook phone model (see Figures 46 to 48 and Table 37) was generated for testing in the TCM-ISDN noise environment. It could be used for testing in other noise environments as well.

Figure 46/G.996.1 – On-hook impedance model

Figure 47/G.996.1 – Measured and modelled on-hook impedance

Table 37/G.996.1 – Coefficients for calculation of primary line constants 
for in-home wiring

| Coefficient | 0.5 mm untwisted single-pair wire |
| --- | --- |
| r(m) | 0.25  10–3 |
| co(m) | 0.75  10–3 |
| r | 1 |
| 0 (H/m) | 4  10–7 |
| (mho/m) | 5.8  107 |
| tan  | 5.0  10–4 |
| C(F/m) | 55  10–12 |
| NOTE – d  2(r  co) |  |

Figure 48/G.996.1 – Insertion loss characteristics of 3 m connecting cord

RFI Models

Two RFI models are provided to provide for different distribution of ingressors. The first model, shown in Figure 49, is distinguished by a large ingressor at 540 kHz, which is inside the G.992.2 band. This model is used to stress the G.992.2 performance in the presence of an in-band ingressor. The second model, shown in Figure 50,  has all ingressors above 620 kHz.

Levels of the ingress signals are specified for 80 percentile coverage. The frequencies of the highest level tones have been selected to keep their in-band 2nd and 3rd order products either below 440 kHz or at the same frequency as an intentional ingress tone. Intermod tones below 440 kHz can be practically filtered and intermod products above 540 kHz can be masked by intentional tones with any necessary level adjustments made at calibration. Thus the linearity requirement of the test amplifiers can be reduced.

A minimum spacing of 30 kHz between AM ingressors was maintained as it is unlikely that two significant ingressors will occur at closer spacing given the regulations on AM broadcasters.

Figure 49/G.996.1 – Model 1 RFI Ingress

Table 38/G.996.1 – RFI Model 1

| Ingress Type | Center Frequency (kHz) | Common Mode (dBm) | Differential (dBm) | Comment |
| --- | --- | --- | --- | --- |
| AM1 | 540 | 0 | –40 | Inside G.992.2 band |
| AM2 | 650 | 0 | –40 | Outside G.992.2 inside G.992.1 |
| AM3 | 680 | –30 | –70 | Outside G.992.2 inside G.992.1 |
| AM4 | 760 | –30 | –70 | Outside G.992.2 inside G.992.1 |
| AM5 | 790 | –30 | –70 | Outside G.992.2 inside G.992.1 |
| AM6 | 840 | –30 | –70 | Outside G.992.2 inside G.992.1 |
| AM7 | 1080 | –30 | –70 | Outside G.992.2 inside G.992.1 |
| AM8 | 1330 | 0 | –40 | Outside G.992.2 and G.992.1 possible alias susceptibility |
| Amateur radio | 3500 | 10 | –30 | Outside G.992.2 and G.992.1 possible alias susceptibility Absent during training, present while measuring margin |

Figure 50/G.996.1 – Model 2 RFI Ingress

Table 39/G.996.1 – RFI Model 2

| Ingress Type | Center Frequency (kHz) | Common Mode (dBm) | Differential (dBm) | Comment |
| --- | --- | --- | --- | --- |
| AM1 | 620 | –30 | –70 | Outside G.992.2 inside G.992.1 |
| AM2 | 740 | –30 | –70 | Outside G.992.2 inside G.992.1 |
| AM3 | 800 | 0 | –40 | Outside G.992.2 inside G.992.1 |
| AM4 | 980 | 0 | –40 | Outside G.992.2 inside G.992.1 |
| AM5 | 1100 | –30 | –70 | Outside G.992.2 inside G.992.1 |
| AM6 | 1160 | 0 | –40 | Outside G.992.2 inside G.992.1 |
| Amateur radio | 1900 | 10 | –30 | Outside G.992.2 and G.992.1 possible alias susceptibility Absent during training, present while measuring margin |

Interference Models

AM Model: The AM radio signal and crosstalk interference are present during initialization.

An unmodulated sinusoid is used for simplicity of test equipment and to make the management of cross products easier. The actual AM signal does occupy a 10 kHz band. However, the peak of the modulating signal will usually be around 500 Hz, so most of the energy will fall within 500-1000 Hz of the carrier (usually well within one DMT bin).

It is possible that the dynamic behaviour of AM ingress is a significant factor in the bit swapping and retrain behavior of G.992.1 modems. If further study shows this to be the case, a new model for AM ingress may need to be used for performance testing.

Amateur Radio Model: A model where the amateur radio signal is not present during training, but is present during operation, is used for performance testing. Thus, the amateur radio signal will be applied at full strength at the beginning of margin testing. With the amateur signal present, the level of cross-talkers will be raised until the specified BER is reached. The margin is determined by the amount the cross-talkers were raised. As in the AM radio case, an unmodulated sinusoid is used to model the amateur radio signal.

ANNEX A

Primary constants of polyethylene-insulated cable (PIC)

NOTE – For the conversion of the primary constants values, in Tables A.1 to A.3, for miles (mi) to kilometres (km) divide by 1.61.

Table A.1/G.996.1 – 26 AWG PIC cable primary constants 1 Hz and 5 MHz

| Frequency (Hz) | at 120 F |  |  |  | at 70 F |  |  |  | at 0 F |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | R (ohms/mi) | L (mH/mi) | G (mho/mi) | C (F/mi) | R (ohms/mi) | L (mH/mi) | G (mho/mi) | C (F/mi) | R (ohms/mi) | L (mH/mi) | G (mho/mi) | C (F/mi) |
| 1 | 488.83 | 0.9935 | 0.000 | 0.08300 | 440.75 | 0.9861 | 0.000 | 0.08300 | 373.45 | 0.9758 | 0.000 | 0.08300 |
| 5 | 488.83 | 0.9935 | 0.001 | 0.08300 | 440.75 | 0.9861 | 0.001 | 0.08300 | 373.45 | 0.9758 | 0.001 | 0.08300 |
| 10 | 488.83 | 0.9935 | 0.002 | 0.08300 | 440.75 | 0.9861 | 0.002 | 0.08300 | 373.45 | 0.9758 | 0.002 | 0.08300 |
| 15 | 488.83 | 0.9935 | 0.003 | 0.08300 | 440.76 | 0.9861 | 0.003 | 0.08300 | 373.45 | 0.9758 | 0.003 | 0.08300 |
| 20 | 488.83 | 0.9935 | 0.004 | 0.08300 | 440.76 | 0.9861 | 0.004 | 0.08300 | 373.45 | 0.9758 | 0.004 | 0.08300 |
| 30 | 488.83 | 0.9935 | 0.005 | 0.08300 | 440.76 | 0.9861 | 0.005 | 0.08300 | 373.45 | 0.9758 | 0.005 | 0.08300 |
| 50 | 488.83 | 0.9935 | 0.008 | 0.08300 | 440.76 | 0.9861 | 0.008 | 0.08300 | 373.45 | 0.9758 | 0.008 | 0.08300 |
| 70 | 488.83 | 0.9935 | 0.011 | 0.08300 | 440.76 | 0.9361 | 0.011 | 0.08300 | 373.45 | 0.9757 | 0.011 | 0.08300 |
| 100 | 488.83 | 0.9935 | 0.016 | 0.08300 | 440.76 | 0.9861 | 0.016 | 0.08300 | 373.45 | 0.9757 | 0.016 | 0.08300 |
| 150 | 488.83 | 0.9935 | 0.022 | 0.08300 | 440.76 | 0.9861 | 0.022 | 0.08300 | 373.45 | 0.9757 | 0.022 | 0.08300 |
| 200 | 488.83 | 0.9934 | 0.028 | 0.08300 | 440.76 | 0.9860 | 0.028 | 0.08300 | 373.46 | 0.9757 | 0.028 | 0.08300 |
| 300 | 488.84 | 0.9934 | 0.040 | 0.08300 | 440.76 | 0.9660 | 0.040 | 0.08300 | 373.46 | 0.9757 | 0.040 | 0.08300 |
| 500 | 488.84 | 0.9933 | 0.063 | 0.08300 | 440.77 | 0.9859 | 0.063 | 0.08300 | 373.46 | 0.9756 | 0.063 | 0.08300 |
| 700 | 488.85 | 0.9933 | 0.084 | 0.08300 | 440.78 | 0.9859 | 0.084 | 0.08300 | 373.47 | 0.9755 | 0.084 | 0.08300 |
| 1 000 | 488.86 | 0.9932 | 0.115 | 0.08300 | 440.79 | 0.9858 | 0.115 | 0.08300 | 373.48 | 0.9754 | 0.115 | 0.08300 |
| 1 500 | 488.89 | 0.9930 | 0.164 | 0.08300 | 440.81 | 0.9856 | 0.164 | 0.08300 | 373.50 | 0.9752 | 0.164 | 0.08300 |
| 2 000 | 488.91 | 0.9928 | 0.210 | 0.08300 | 440.83 | 0.9854 | 0.210 | 0.08300 | 373.52 | 0.9751 | 0.210 | 0.08300 |
| 3 000 | 488.97 | 0.9924 | 0.299 | 0.08300 | 440.88 | 0.9850 | 0.299 | 0.08300 | 373.56 | 0.9747 | 0.299 | 0.08300 |
| 5 000 | 489.11 | 0.9917 | 0.466 | 0.08300 | 441.01 | 0.9843 | 0.466 | 0.08300 | 373.67 | 0.9740 | 0.466 | 0.08300 |
| 7 000 | 489.26 | 0.9910 | 0.625 | 0.08300 | 441.15 | 0.9836 | 0.625 | 0.08300 | 373.78 | 0.9733 | 0.625 | 0.08300 |
| 10 000 | 489.53 | 0.9899 | 0.853 | 0.08300 | 441.39 | 0.9825 | 0.853 | 0.08300 | 373.99 | 0.9722 | 0.853 | 0.08300 |
| 15 000 | 490.07 | 0.9881 | 1.213 | 0.08300 | 441.37 | 0.9807 | 1.213 | 0.08300 | 374.40 | 0.9704 | 1.213 | 0.08300 |
| 20 000 | 490.71 | 0.9863 | 1.558 | 0.08300 | 442.83 | 0.9789 | 1.558 | 0.08300 | 376.89 | 0.9687 | 1.558 | 0.08300 |
| 30 000 | 492.30 | 0.9826 | 2.213 | 0.08300 | 443.88 | 0.9758 | 2.217 | 0.08300 | 376.10 | 0.9651 | 2.217 | 0.08300 |
| 50 000 | 496.65 | 0.9733 | 3.458 | 0.08300 | 447.81 | 0.9660 | 3.458 | 0.08300 | 379.43 | 0.9559 | 3.458 | 0.08300 |
| 70 000 | 502.51 | 0.9617 | 4.634 | 0.08300 | 453.09 | 0.9546 | 4.634 | 0.08300 | 383.91 | 0.9446 | 4.634 | 0.08300 |
| 100 000 | 513.93 | 0.9502 | 6.320 | 0.08300 | 463.39 | 0.9432 | 6.320 | 0.08300 | 392.63 | 0.9333 | 6.320 | 0.08300 |
| 150 000 | 536.26 | 0.9375 | 8.993 | 0.08300 | 485.80 | 0.9306 | 8.993 | 0.08300 | 415.15 | 0.9208 | 8.993 | 0.08300 |
| 200 000 | 561.79 | 0.9281 | 11.550 | 0.08300 | 513.04 | 0.9212 | 11.550 | 0.08300 | 444.79 | 0.9115 | 11.550 | 0.08300 |
| 300 000 | 622.63 | 0.9139 | 16.436 | 0.08300 | 575.17 | 0.9062 | 16.436 | 0.08300 | 508.72 | 0.8955 | 16.436 | 0.08300 |
| 500 000 | 746.31 | 0.8910 | 25.633 | 0.08300 | 699.61 | 0.8816 | 25.633 | 0.08300 | 634.23 | 0.8655 | 25.633 | 0.08300 |
| 700 000 | 862.21 | 0.8717 | 34.351 | 0.08300 | 812.95 | 0.8614 | 34.351 | 0.08300 | 743.98 | 0.8468 | 34.351 | 0.08300 |
| 1 000 000 | 1013.99 | 0.8495 | 46.849 | 0.08300 | 956.65 | 0.8331 | 46.849 | 0.08300 | 876.38 | 0.8222 | 46.849 | 0.08300 |
| 1 500 000 | 1398.54 | 0.8271 | 66.665 | 0.08300 | 1154.38 | 0.8146 | 66.665 | 0.08300 | 1058.74 | 0.7972 | 66.665 | 0.08300 |
| 2 000 000 | 1693.35 | 0.8133 | 85.624 | 0.08300 | 1321.07 | 0.8001 | 85.624 | 0.08300 | 1212.60 | 0.7816 | 85.624 | 0.08300 |
| 3 000 000 | 1693.35 | 0.7965 | 121.841 | 0.08300 | 1600.68 | 0.7823 | 121.841 | 0.08300 | 1470.94 | 0.7624 | 121.841 | 0.08300 |
| 5 000 000 | 2160.47 | 0.7794 | 190.021 | 0.08300 | 2044.07 | 0.7638 | 190.021 | 0.08300 | 1881.11 | 0.7420 | 190.021 | 0.08300 |

Table A.2/G.996.1 – 24 AWG PIC cable primary constants 1 Hz and 5 MHz

| Frequency (Hz) | at 120 F |  |  |  | at 70 F |  |  |  | at 0 F |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | R (ohms/mi) | L (mH/mi) | G (mho/mi) | C (F/mi) | R (ohms/mi) | L (mH/mi) | G (mho/mi) | C (F/mi) | R (ohms/mi) | L (mH/mi) | G (mho/mi) | C (F/mi) |
| 1 | 307.43 | 0.9935 | 0.000 | 0.08300 | 277.19 | 0.9861 | 0.000 | 0.08300 | 234.87 | 0.9758 | 0.000 | 0.08300 |
| 5 | 307.43 | 0.9935 | 0.001 | 0.08300 | 277.19 | 0.9861 | 0.001 | 0.08300 | 234.87 | 0.9758 | 0.001 | 0.08300 |
| 10 | 307.43 | 0.9935 | 0.002 | 0.08300 | 277.19 | 0.9861 | 0.002 | 0.08300 | 234.87 | 0.9758 | 0.002 | 0.08300 |
| 15 | 307.43 | 0.9935 | 0.003 | 0.08300 | 277.19 | 0.9861 | 0.003 | 0.08300 | 234.87 | 0.9758 | 0.003 | 0.08300 |
| 20 | 307.43 | 0.9935 | 0.004 | 0.08300 | 277.19 | 0.9861 | 0.004 | 0.08300 | 234.87 | 0.9758 | 0.004 | 0.08300 |
| 30 | 307.43 | 0.9935 | 0.005 | 0.08300 | 277.19 | 0.9861 | 0.005 | 0.08300 | 234.87 | 0.9758 | 0.005 | 0.08300 |
| 50 | 307.43 | 0.9935 | 0.008 | 0.08300 | 277.19 | 0.9861 | 0.008 | 0.08300 | 234.87 | 0.9757 | 0.008 | 0.08300 |
| 70 | 307.43 | 0.9935 | 0.011 | 0.08300 | 277.19 | 0.9861 | 0.011 | 0.08300 | 234.87 | 0.9757 | 0.011 | 0.08300 |
| 100 | 307.43 | 0.9935 | 0.016 | 0.08300 | 277.19 | 0.9861 | 0.016 | 0.08300 | 234.87 | 0.9757 | 0.016 | 0.08300 |
| 150 | 307.43 | 0.9934 | 0.022 | 0.08300 | 277.20 | 0.9860 | 0.022 | 0.08300 | 234.87 | 0.9757 | 0.022 | 0.08300 |
| 200 | 307.43 | 0.9934 | 0.028 | 0.08300 | 277.20 | 0.9860 | 0.028 | 0.08300 | 234.87 | 0.9757 | 0.028 | 0.08300 |
| 300 | 307.43 | 0.9934 | 0.040 | 0.08300 | 277.20 | 0.9860 | 0.040 | 0.08300 | 234.87 | 0.9756 | 0.040 | 0.08300 |
| 500 | 307.44 | 0.9933 | 0.063 | 0.08300 | 277.21 | 0.9859 | 0.063 | 0.08300 | 234.88 | 0.9755 | 0.063 | 0.08300 |
| 700 | 307.45 | 0.9932 | 0.084 | 0.08300 | 277.22 | 0.9858 | 0.084 | 0.08300 | 234.89 | 0.9755 | 0.084 | 0.08300 |
| 1000 | 307.47 | 0.9931 | 0.115 | 0.08300 | 277.23 | 0.9857 | 0.115 | 0.08300 | 234.90 | 0.9753 | 0.115 | 0.08300 |
| 1500 | 307.49 | 0.9928 | 0.164 | 0.08300 | 277.25 | 0.9854 | 0.164 | 0.08300 | 234.92 | 0.9751 | 0.164 | 0.08300 |
| 2000 | 307.52 | 0.9926 | 0.210 | 0.08300 | 277.28 | 0.9852 | 0.210 | 0.08300 | 234.94 | 0.9749 | 0.210 | 0.08300 |
| 3000 | 307.59 | 0.9921 | 0.299 | 0.08300 | 277.34 | 0.9848 | 0.299 | 0.08300 | 234.99 | 0.9744 | 0.299 | 0.08300 |
| 5000 | 307.75 | 0.9912 | 0.466 | 0.08300 | 277.48 | 0.9839 | 0.466 | 0.08300 | 235.11 | 0.9735 | 0.466 | 0.08300 |
| 7000 | 307.94 | 0.9903 | 0.625 | 0.08300 | 277.66 | 0.9829 | 0.625 | 0.08300 | 235.26 | 0.9726 | 0.625 | 0.08300 |
| 10 000 | 308.27 | 0.9889 | 0.853 | 0.08300 | 277.96 | 0.9816 | 0.853 | 0.08300 | 235.51 | 0.9713 | 0.853 | 0.08300 |
| 15 000 | 308.97 | 0.9866 | 1.213 | 0.08300 | 278.58 | 0.9793 | 1.213 | 0.08300 | 236.04 | 0.9690 | 1.213 | 0.08300 |
| 20 000 | 309.82 | 0.9843 | 1.558 | 0.08300 | 279.35 | 0.9770 | 1.558 | 0.08300 | 236.69 | 0.9667 | 1.558 | 0.08300 |
| 30 000 | 311.98 | 0.9796 | 2.217 | 0.08300 | 281.30 | 0.9723 | 2.217 | 0.08300 | 238.35 | 0.9621 | 2.217 | 0.08300 |
| 50 000 | 318.10 | 0.9649 | 3.458 | 0.08300 | 286.82 | 0.9577 | 3.458 | 0.08300 | 243.02 | 0.9476 | 3.458 | 0.08300 |
| 70 000 | 326.39 | 0.9535 | 4.634 | 0.08300 | 294.29 | 0.9464 | 4.634 | 0.08300 | 249.35 | 0.9365 | 4.634 | 0.08300 |
| 100 000 | 339.90 | 0.9417 | 6.320 | 0.08300 | 308.41 | 0.9347 | 6.320 | 0.08300 | 264.34 | 0.9249 | 6.320 | 0.08300 |
| 150 000 | 367.43 | 0.9273 | 8.993 | 0.08300 | 337.22 | 0.9204 | 8.993 | 0.08300 | 294.92 | 0.9107 | 8.993 | 0.08300 |
| 200 000 | 398.81 | 0.9166 | 11.550 | 0.08300 | 369.03 | 0.9087 | 11.550 | 0.08300 | 327.32 | 0.8976 | 11.550 | 0.08300 |
| 300 000 | 460.98 | 0.8978 | 16.436 | 0.08300 | 431.55 | 0.8885 | 16.436 | 0.08300 | 390.34 | 0.8756 | 16.436 | 0.08300 |
| 500 000 | 574.39 | 0.8678 | 25.633 | 0.08300 | 541.69 | 0.8570 | 25.633 | 0.08300 | 495.92 | 0.8419 | 25.633 | 0.08300 |
| 700 000 | 669.84 | 0.8467 | 34.351 | 0.08300 | 632.08 | 0.8350 | 34.351 | 0.08300 | 579.22 | 0.8186 | 34.351 | 0.08300 |
| 1 000 000 | 790.12 | 0.8273 | 46.849 | 0.08300 | 746.04 | 0.8146 | 46.849 | 0.08300 | 684.34 | 0.7969 | 46.849 | 0.08300 |
| 1 500 000 | 955.50 | 0.8084 | 66.665 | 0.08300 | 902.84 | 0.7947 | 66.665 | 0.08300 | 829.13 | 0.7756 | 66.665 | 0.08300 |
| 2 000 000 | 1094.84 | 0.7970 | 85.624 | 0.08300 | 1035.03 | 0.7825 | 85.624 | 0.08300 | 951.29 | 0.7623 | 85.624 | 0.08300 |
| 3 000 000 | 1328.44 | 0.7831 | 121.841 | 0.08300 | 1256.77 | 0.7676 | 121.841 | 0.08300 | 1156.42 | 0.7459 | 121.841 | 0.08300 |
| 5 000 000 | 1698.58 | 0.7729 | 190.021 | 0.08300 | 1608.38 | 0.7523 | 190.021 | 0.08300 | 1482.09 | 0.7235 | 190.021 | 0.08300 |

Table A.3/G.996.1 – 22 AWG PIC cable primary constants 1 Hz and 5 MHz

| Frequency (Hz) | at 120 F |  |  |  | at 70 F |  |  |  | at 0 F |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | R (ohms/mi) | L (mH/mi) | G (mho/mi) | C (F/mi) | R (ohms/mi) | L (mH/mi) | G (ohm/mi) | C (F/mi) | R ohms/mi | L mH/mi | G ohm/mi | C F/mi |
| 1 | 193.28 | 0.9935 | 0.000 | 0.08300 | 174.27 | 0.9861 | 0.000 | 0.08300 | 147.66 | 0.9758 | 0.000 | 0.08300 |
| 5 | 193.28 | 0.9935 | 0.001 | 0.08300 | 174.27 | 0.9861 | 0.001 | 0.08300 | 147.66 | 0.9758 | 0.001 | 0.08300 |
| 10 | 193.28 | 0.9935 | 0.001 | 0.08300 | 174.27 | 0.9861 | 0.001 | 0.08300 | 147.66 | 0.9758 | 0.001 | 0.08300 |
| 15 | 193.28 | 0.9935 | 0.001 | 0.08300 | 174.27 | 0.9861 | 0.001 | 0.08300 | 147.66 | 0.9758 | 0.001 | 0.08300 |
| 20 | 193.28 | 0.9935 | 0.002 | 0.08300 | 174.27 | 0.9861 | 0.002 | 0.08300 | 147.66 | 0.9758 | 0.002 | 0.08300 |
| 30 | 193.28 | 0.9935 | 0.003 | 0.08300 | 174.27 | 0.9861 | 0.003 | 0.08300 | 147.66 | 0.9758 | 0.003 | 0.08300 |
| 50 | 193.28 | 0.9935 | 0.005 | 0.08300 | 174.27 | 0.9861 | 0.005 | 0.08300 | 147.66 | 0.9758 | 0.005 | 0.08300 |
| 70 | 193.28 | 0.9935 | 0.006 | 0.08300 | 174.27 | 0.9861 | 0.006 | 0.08300 | 147.66 | 0.9757 | 0.006 | 0.08300 |
| 100 | 193.28 | 0.9935 | 0.009 | 0.08300 | 174.27 | 0.9861 | 0.009 | 0.08300 | 147.66 | 0.9757 | 0.009 | 0.08300 |
| 150 | 193.28 | 0.9934 | 0.013 | 0.08300 | 174.27 | 0.9860 | 0.013 | 0.08300 | 147.66 | 0.9757 | 0.013 | 0.08300 |
| 200 | 193.28 | 0.9934 | 0.017 | 0.08300 | 174.27 | 0.9860 | 0.017 | 0.08300 | 147.66 | 0.9757 | 0.017 | 0.08300 |
| 300 | 193.29 | 0.9934 | 0.024 | 0.08300 | 174.28 | 0.9860 | 0.024 | 0.08300 | 147.67 | 0.9756 | 0.024 | 0.08300 |
| 500 | 193.29 | 0.9932 | 0.040 | 0.08300 | 174.29 | 0.9858 | 0.040 | 0.08300 | 147.67 | 0.9755 | 0.040 | 0.08300 |
| 700 | 193.30 | 0.9931 | 0.054 | 0.08300 | 174.29 | 0.9857 | 0.054 | 0.08300 | 147.68 | 0.9754 | 0.054 | 0.08300 |
| 1000 | 193.32 | 0.9930 | 0.076 | 0.08300 | 174.31 | 0.9856 | 0.076 | 0.08300 | 147.69 | 0.9752 | 0.076 | 0.08300 |
| 1500 | 193.35 | 0.9927 | 0.110 | 0.08300 | 174.34 | 0.9853 | 0.110 | 0.08300 | 147.72 | 0.9749 | 0.110 | 0.08300 |
| 2000 | 193.39 | 0.9924 | 0.145 | 0.08300 | 174.37 | 0.9850 | 0.145 | 0.08300 | 147.74 | 0.9747 | 0.145 | 0.08300 |
| 3000 | 193.47 | 0.9918 | 0.211 | 0.08300 | 174.44 | 0.9844 | 0.211 | 0.08300 | 147.80 | 0.9741 | 0.211 | 0.08300 |
| 5000 | 193.66 | 0.9906 | 0.341 | 0.08300 | 174.62 | 0.9833 | 0.341 | 0.08300 | 147.95 | 0.9729 | 0.341 | 0.08300 |
| 7000 | 193.90 | 0.9895 | 0.467 | 0.08300 | 174.83 | 0.9821 | 0.467 | 0.08300 | 148.13 | 0.9718 | 0.467 | 0.08300 |
| 10 000 | 194.33 | 0.9877 | 0.652 | 0.08300 | 175.22 | 0.9804 | 0.652 | 0.08300 | 148.47 | 0.9701 | 0.652 | 0.08300 |
| 15 000 | 195.26 | 0.9847 | 0.954 | 0.08300 | 176.06 | 0.9778 | 0.954 | 0.08300 | 149.17 | 0.9671 | 0.954 | 0.08300 |
| 20 000 | 196.43 | 0.9817 | 1.248 | 0.08300 | 177.11 | 0.9744 | 1.248 | 0.08300 | 150.07 | 0.9642 | 1.248 | 0.08300 |
| 30 000 | 199.48 | 0.9744 | 1.824 | 0.08300 | 179.86 | 0.9672 | 1.824 | 0.08300 | 152.39 | 0.9570 | 1.824 | 0.08300 |
| 50 000 | 208.10 | 0.9562 | 2.943 | 0.08300 | 187.64 | 0.9491 | 2.943 | 0.08300 | 158.98 | 0.9391 | 2.943 | 0.08300 |
| 70 000 | 217.24 | 0.9443 | 4.032 | 0.08300 | 197.71 | 0.9372 | 4.032 | 0.08300 | 170.37 | 0.9274 | 4.032 | 0.08300 |
| 100 000 | 234.48 | 0.9309 | 5.630 | 0.08300 | 215.55 | 0.9237 | 5.630 | 0.08300 | 189.06 | 0.9137 | 5.630 | 0.08300 |
| 150 000 | 266.20 | 0.9141 | 8.229 | 0.08300 | 247.57 | 0.9055 | 8.229 | 0.08300 | 221.40 | 0.8935 | 8.229 | 0.08300 |
| 200 000 | 296.40 | 0.8993 | 10.772 | 0.08300 | 277.95 | 0.8898 | 10.772 | 0.08300 | 252.11 | 0.8765 | 10.772 | 0.08300 |
| 300 000 | 353.55 | 0.8749 | 15.744 | 0.08300 | 333.39 | 0.8642 | 15.744 | 0.08300 | 305.18 | 0.8492 | 15.744 | 0.08300 |
| 500 000 | 446.65 | 0.8430 | 25.396 | 0.08300 | 421.57 | 0.8309 | 25.396 | 0.08300 | 386.45 | 0.8140 | 25.396 | 0.08300 |
| 700 000 | 522.27 | 0.8252 | 34.796 | 0.08300 | 493.24 | 0.8123 | 34.796 | 0.08300 | 452.58 | 0.7941 | 34.796 | 0.08300 |
| 1 000 000 | 617.56 | 0.8090 | 48.587 | 0.08300 | 583.59 | 0.7950 | 48.587 | 0.08300 | 536.03 | 0.7756 | 48.587 | 0.08300 |
| 1 500 000 | 748.59 | 0.7933 | 71.014 | 0.08300 | 707.91 | 0.7783 | 71.014 | 0.08300 | 650.97 | 0.7574 | 71.014 | 0.08300 |
| 2 000 000 | 858.98 | 0.7838 | 92.958 | 0.08300 | 812.72 | 0.7681 | 92.958 | 0.08300 | 747.95 | 0.7460 | 92.958 | 0.08300 |
| 3 000 000 | 1044.05 | 0.7759 | 135.865 | 0.08300 | 988.53 | 0.7557 | 135.865 | 0.08300 | 910.79 | 0.7275 | 135.865 | 0.08300 |
| 5 000 000 | 1337.29 | 0.7685 | 219.158 | 0.08300 | 1267.31 | 0.7429 | 219.158 | 0.08300 | 1169.33 | 0.7071 | 219.158 | 0.08300 |

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |