|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.996.1 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | Amendment 1 (03/2003) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital sections and digital line system – Access networks |  |  |  |
|  | Test procedures for digital subscriber line (DSL) transceivers Amendment 1: New Annex B |  |  |  |
|  | ITU-T Recommendation G.996.1 (2001) – Amendment 1 |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| General | G.900–G.909 |
| Parameters for optical fibre cable systems | G.910–G.919 |
| Digital sections at hierarchical bit rates based on a bit rate of 2048 kbit/s | G.920–G.929 |
| Digital line transmission systems on cable at non-hierarchical bit rates | G.930–G.939 |
| Digital line systems provided by FDM transmission bearers | G.940–G.949 |
| Digital line systems | G.950–G.959 |
| Digital section and digital transmission systems for customer access to ISDN | G.960–G.969 |
| Optical fibre submarine cable systems | G.970–G.979 |
| Optical line systems for local and access networks | G.980–G.989 |
| Access networks | G.990–G.999 |
| QUALITY OF SERVICE AND PERFORMANCE | G.1000–G.1999 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.6000–G.6999 |
| DIGITAL TERMINAL EQUIPMENTS | G.7000–G.7999 |
| DIGITAL NETWORKS | G.8000–G.8999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.996.1 Test procedures for digital subscriber line (DSL) transceivers Amendment 1 New Annex B

Summary The amendment adds a new Annex B to ITU-T Rec. G.996.1 that contains primary constants and disturbers for test loops for use in ADSL+ in an environment coexisting with TCM-ISDN DSL.

Source Amendment 1 to ITU-T Recommendation G.996.1 (2001) was prepared by ITU-T Study Group 15 (2001-2004) and approved under the WTSA Resolution 1 procedure on 16 March 2003.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2003

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

ITU-T Recommendation G.996.1

Test procedures for digital subscriber line (DSL) transceivers

Amendment 1

New Annex B

# 1)	New Annex B

Add the following Annex B to ITU-T Rec. G.996.1:

Annex B

Test loops and crosstalk disturbers of ADSL transceivers with extended bandwidth for environment coexisting with TCM-ISDN DSL 
as defined in ITU-T Rec. G.961 Appendix III

## B.1	Test loops

### B.1.1	Loop configurations

To test the performance of ADSL transceivers with extended bandwidth, the test loops in Figure 11 and the associate loop lengths in Table 10 shall be used.

### B.1.2	Primary line constants

The primary line constants are R, L, C, and G. The equations below give the values of R in ohm/m, L in H/m, G in mho/m, C in F/m, and f (frequency) in Hz. The equation of G only is modified to that defined in 6.1.3. The coefficient values are shown in Table B.1. The values of the exponent "ge" contained in the equation of G are added in Table B.1, and the other coefficient values are the same as those shown in Table 11.

R=2Ri+Rn+Rns	[ohm/m]

L=2La+Li+Ln+Lns	[H/m]

C=Ci	[F/m]

G=2πfgeCtanδ	[mho/m]

Ri=1πri2σiReλ2J0λJ1λ;	skin effect

Rn=1πdi2σiRe−λJ1λJ0λ;	intra-pair eddy current effect

Rns=1πdi2σi4Re−λJ1λJ0λ;	intra-quad eddy current effect

La=μ02πlndiri;	external inductance

Li=μi2πRe−1λJ0λJ1λ;	skin effect

Ln=−μ02πridi2Re−J2λJ0λ;	intra-pair eddy current effect

Lns=−μ02πridi24Re−J2λJ0λ;	intra-quad eddy current effect

where:

J0,J1,J2	: zero-, first-, and second-order Bessel functions

Re	: real part in [ ]

λ≡1+jriδi

ri	: radius of conductor [m]

δi=2ωσiμi	: skin depth [m]

σi	: conductivity of copper (conductor) [mho/m]

μ0	: permeability of vacuum [H/m]

μi	: permeability of copper (conductor) [H/m]; =μrμ0

μr	: relative permeability of copper (conductor)

ω	: angular frequency [rad/m]

di	: distance between wire (conductor) centers of a pair [m]

: di=22ri+COi

COi	: thickness of insulator for wire (conductor) [m]

Table B.1/G.996.1 – Coefficient values

| Coefficient | Paper |  |  |  | Polyethylene |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0.4 mm | 0.5 mm | 0.65 mm | 0.9 mm | 0.32 mm | 0.4 mm | 0.5 mm | 0.65 mm | 0.9 mm |
| ri [m] | 0.2 × 103 | 0.25 × 103 | 0.325 × 103 | 0.45 × 103 | 0.16 × 103 | 0.2 × 103 | 0.25 × 103 | 0.325 × 103 | 0.45 × 103 |
| COi [m] | 0.09 × 103 | 0.11 × 103 | 0.17 × 103 | 0.24 × 103 | 0.05 × 103 | 0.13  103 | 0.15 × 103 | 0.20 × 103 | 0.27 × 103 |
| ge | 0.996 | 0.993 | 0.998 | 0.998 | 1.21 | 1.16 | 1.05 | 1.02 | 1.02 |
| tanδ | 2.5 × 102 |  |  |  | 4.0 × 104 | 5.0 × 10–4 |  |  |  |
| Ci [F/m] | 50 × 10–12 |  |  |  |  |  |  |  |  |
| σi [mho/m] | 5.8 × 107 |  |  |  |  |  |  |  |  |
| μ0 [H/m] | 4  107 |  |  |  |  |  |  |  |  |
| μr | 1 |  |  |  |  |  |  |  |  |

### B.1.3	Line transfer function and test loop characteristics

The line transfer function (of voltage) based on the propagation constant is given below. The transfer function assumes no impedance mismatch and perfect terminations by characteristic impedances at both ends, and is a simplified approximation.

Hf=e–γd

γ=(R+jωL)(G+jωC)

where:

d	: loop length [m]

γ	: propagation constant

The test loop characteristics for reference are presented in Tables B.2, B.3 and B.4 as results of calculation using the above line transfer function and the coefficient values.

NOTE – The insertion loss with a source impedance of R = 100 Ω and a terminal impedance of R = 100 Ω should be calculated by using the loop ABCD parameters, and the result is loop length and composition dependent.

Table B.2/G.996.1 – Test loop image attenuation in dB for reference

| Cable type [Loop length 1 km] |  | Frequency [kHz] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 | 40 | 100 | 160 | 260 | 550 | 1100 | 2195 | 3750 |
| Paper | 0.4 mm | 7.07 dB | 8.82 | 11.0 | 12.6 | 15.3 | 22.7 | 33.9 | 50.4 | 69.0 |
|  | 0.5 mm | 5.26 dB | 6.31 | 7.99 | 9.63 | 12.3 | 18.8 | 28.0 | 41.9 | 57.6 |
|  | 0.65 mm | 3.55 dB | 4.15 | 5.66 | 7.16 | 9.39 | 14.4 | 21.6 | 32.8 | 45.8 |
|  | 0.9 mm | 2.14 dB | 2.63 | 4.10 | 5.36 | 7.08 | 10.9 | 16.6 | 25.6 | 36.4 |
| Polyethylene | 0.32 mm | 9.23 dB | 12.0 | 15.5 | 17.4 | 20.2 | 28.7 | 42.5 | 63.0 | 85.2 |
|  | 0.4 mm | 6.89 dB | 8.47 | 10.2 | 11.3 | 13.3 | 18.9 | 27.2 | 39.2 | 52.3 |
|  | 0.5 mm | 5.09 dB | 6.00 | 7.26 | 8.47 | 10.5 | 15.4 | 22.1 | 31.4 | 41.3 |
|  | 0.65 mm | 3.42 dB | 3.91 | 5.09 | 6.27 | 8.02 | 11.8 | 16.8 | 23.9 | 31.4 |
|  | 0.9 mm | 2.03 dB | 2.42 | 3.61 | 4.60 | 5.92 | 8.68 | 12.4 | 17.6 | 23.1 |

Table B.3/G.996.1 – Test loop group delay in μs for reference

| Cable type [Loop length 1 km] |  | Frequency [kHz] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 | 40 | 100 | 160 | 260 | 550 | 1100 | 2195 | 3750 |
| Paper | 0.4 mm | 5.53 μs | 5.31 | 5.49 | 5.48 | 5.36 | 5.13 | 4.99 | 4.91 | 4.86 |
|  | 0.5 mm | 5.32 μs | 5.38 | 5.47 | 5.35 | 5.19 | 5.01 | 4.91 | 4.85 | 4.81 |
|  | 0.65 mm | 5.44 μs | 5.59 | 5.47 | 5.33 | 5.21 | 5.10 | 5.03 | 4.99 | 4.96 |
|  | 0.9 mm | 5.60 μs | 5.56 | 5.30 | 5.20 | 5.13 | 5.06 | 5.01 | 4.98 | 4.96 |
| Polyethylene | 0.32 mm | 5.97 μs | 5.27 | 5.23 | 5.30 | 5.26 | 4.98 | 4.76 | 4.63 | 4.57 |
|  | 0.4 mm | 5.70 μs | 5.52 | 5.74 | 5.75 | 5.66 | 5.47 | 5.35 | 5.28 | 5.24 |
|  | 0.5 mm | 5.48 μs | 5.57 | 5.69 | 5.60 | 5.47 | 5.30 | 5.22 | 5.16 | 5.13 |
|  | 0.65 mm | 5.55 μs | 5.71 | 5.61 | 5.49 | 5.37 | 5.26 | 5.20 | 5.16 | 5.14 |
|  | 0.9 mm | 5.69 μs | 5.65 | 5.42 | 5.31 | 5.25 | 5.18 | 5.13 | 5.10 | 5.09 |

Table B.4/G.996.1 – Test loop characteristic impedance in Ω for reference

| Cable type |  | Frequency [kHz] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 | 40 | 100 | 160 | 260 | 550 | 1100 | 2195 | 3750 |
| Paper | 0.4 mm | 214 Ω | 161 | 127 | 120 | 115 | 110 | 105 | 102 | 100 |
|  | 0.5 mm | 176 Ω | 140 | 120 | 116 | 112 | 106 | 103 | 100 | 98.7 |
|  | 0.65 mm | 147 Ω | 127 | 117 | 114 | 110 | 106 | 104 | 102 | 101 |
|  | 0.9 mm | 128 Ω | 119 | 113 | 110 | 107 | 104 | 102 | 101 | 100 |
| Polyethylene | 0.32 mm | 264 Ω | 191 | 137 | 124 | 117 | 109 | 103 | 98.5 | 95.8 |
|  | 0.4 mm | 215 Ω | 162 | 130 | 124 | 120 | 115 | 112 | 109 | 107 |
|  | 0.5 mm | 177 Ω | 142 | 123 | 119 | 116 | 111 | 108 | 106 | 105 |
|  | 0.65 mm | 148 Ω | 129 | 119 | 116 | 113 | 109 | 107 | 105 | 104 |
|  | 0.9 mm | 129 Ω | 121 | 114 | 112 | 109 | 107 | 105 | 104 | 103 |

## B.2	Crosstalk disturbers

### B.2.1	Disturber types

Crosstalk margin measurements are performed with several types of disturbers, extended-bandwidth-ADSL self, TCM-ISDN DSL (Appendix III/G.961) and VDSL (ITU-T Rec. G.993.1).

Two kinds of noise models are defined as follows. Noise A and Noise Bj (j = 1-3). Noise A only or Noise A + each Noise Bj (j = 1 or 2) shall be injected at each central office or customer premises side end. Noise A + Noise B3 shall be injected at the only customer premises side end. The test should be performed several (3-4) times. A combination Noise Bj and Bk (j<>k) is not used for performance test. Also, simultaneous injection at both central office and customer premises side ends is not used.

Noise A = –140 dBm/Hz AWGN (Additive White Gaussian Noise)

Noise B1 = 24 extended-bandwidth-ADSL self NEXT and FEXT

Noise B2 = 24 TCM-ISDN DSL (Appendix III/G.961) alternate NEXT and FEXT

(see ITU-T Rec. G.996.1 as for the disturber PSD)

Noise B3 = 9 VDSL (ITU-T Rec. G.993.1) NEXT and in-equal level coupled FEXT

(see Annex F/G.993.1 as for the disturber PSD)

TCM-ISDN DSL NEXT and FEXT appear alternately in the same frequency band, and are cyclostationary. Annex B adopts cyclostationary crosstalk injection of NEXT and FEXT as defined in 7.9.3.

The loop length of VDSL disturbers shall be fixed at 150 m and shall not be adjustable, drawing upward to a central office side from a customer premises side end. Namely, an in-equal level coupling of FEXT from VDSL to extended-bandwidth-ADSL is considered, and the only 0.4 mm polyethylene-insulated cable is considered as the coupling path. The disturber loop length of 150 m is determined as a typical length of the most significant interference.

NOTE – Extended-bandwidth-ADSL NEXT and VDSL NEXT injection is for testing input signal dynamic range of an extended-bandwidth-ADSL receiver.

### B.2.2	Power sum loss of crosstalk

NEXT and FEXT PSL values for 24 disturbers with 1% worst case are shown in Table 25, which are used for generating the simulated Noise B1 and B2. NEXT and FEXT PSL values for 9 disturbers with 1% worst case are shown in Table B.5, which are used for generating the simulated Noise B3, where only a polyethylene-insulated cable is considered because an in-equal level FEXT coupling from VDSL to ADSL arises mostly within intra- user-building cables and all of them are polyethylene-insulated.

Table B.5/G.996.1 – Power sum loss values for 9 disturbers with 1% worst case

| NEXT/FEXT power sum loss | Insulation material of pair conductor | Condition |
| --- | --- | --- |
|  | Polyethylene-insulated |  |
| NPSL9 [dB] | 49.5 dB | At fNEXT  160  103 [Hz] |
| FPSL9 [dB] | 51.5 dB | At fFEXT  160  103 [Hz] At dFEXT  1  103 [m] |

### B.2.3	Power spectral density of disturbers

The single-sided power spectral density (PSD) functions in watts/Hz are abbreviated by PSDxDSL-DS(f) and PSDxDSL-US(f) in watts/Hz below for downstream and upstream, respectively.

PSDxDSL-DS(f) and PSDxDSL-US(f) for TCM-ISDN DSL disturber are defined in 7.9.3.1, and those for VDSL are defined in F.3.2.2.1/G.993.1. As for extended-bandwidth-ADSL, they are specified in the related ITU-T Recommendations.

### B.2.4	Power spectral density of crosstalk

XT (crosstalk) PSD for each xDSL disturber is given by multiplying the xDSL disturber PSD and the XT power coupling function.

#### B.2.4.1	Crosstalk power coupling function

XT (crosstalk) power coupling functions XT(f) are given below for cases of NEXT and FEXT.

NEXT power coupling function for use of Noise B1, B2 and B3 is as follows.

XTNEXTf=ZdisturbedZdisturber10−NPSLn10ffNEXT32

FEXT power coupling function for use of Noise B1 and B2 is as follows. The bridged taps (BTs) shall not be incorporated in the simulated FEXT disturber PSD as a part of the FEXT coupling path.

XTFEXTf=ZdisturbedZdisturberΠk e−2γkdk k10−FPSL24k10dkdFEXTffFEXT2

FEXT power coupling function for use of Noise B3 is as follows. The bridged taps (BTs) shall not be incorporated in the simulated FEXT disturber PSD as a part of the FEXT coupling path.

XTFEXTf=ZdisturbedZdisturber  e−2γxdx10−FPSL910ffFEXT2dxdFEXT

where:

f	: frequency in Hz

NPSLn	: NEXT PSL value with n disturbers

FPSL24[k]	: FEXT PSL value with 24 disturbers for each segment [k] consisting test loop

γk	: the propagation constant of each segment [k] consisting test loop

dk	: length of each segment [k] consisting test loop in m

fFEXT = 160 × 103 Hz

dFEXT = 1 × 103 m

Zdisturbed	: termination impedance of disturbed extended-bandwidth-ADSL (= 100 Ω)

Zdisturber	: termination impedance of disturbing xDSL

100 Ω	: for extended-bandwidth-ADSL and VDSL

110 Ω	: for TCM-ISDN

γx	: the propagation constant of the 0.4 mm polyethylene-insulated cable

dx = 150 m

FPSL9	: FEXT PSL value with 9 disturbers for the polyethylene-insulated cable

NOTE 1 – The NEXT power coupling function of XTNEXT(f) is a function of a coupling path length, to be exact, as expressed below. However, Annex B does not adopt the equation below so as to reduce test parameters.

XTNEXTf=ZdisturbedZdisturber10−NPSLn10ffNEXT231− e−4γd

NOTE 2 – FEXT of Noise B1 and B2 assume equal level coupling, i.e., the loop length of a disturbed extended-bandwidth-ADSL is the same as that of a disturbing xDSL so as to reduce test parameters.

NOTE 3 – FEXT of Noise B3 assumes in-equal level coupling, and the FEXT coupling path length dx of 150 m only is tested so as to reduce test parameters.

#### B.2.4.2	Disturber crosstalk PSD

The single-sided XTPSDs (crosstalk PSD) for each xDSL disturber are given by the equations below for the cases NEXT/FEXT and downstream/upstream.

XTPSDxDSL-DS-NEXT(f) = PSDxDSL-DS(f) XTNEXT(f)	watts/Hz

XTPSDxDSL-DS-FEXT(f) = PSDxDSL-DS(f) XTFEXT(f)	watts/Hz

XTPSDxDSL-US-NEXT(f) = PSDxDSL-US(f) XTNEXT(f)	watts/Hz

XTPSDxDSL-US-FEXT(f) = PSDxDSL-US(f) XTFEXT(f)	watts/Hz

XTPSDs injected at each central office or customer premises side end are as follows.

XTPSDxDSL-CO(f) = XTPSDxDSL-DS-NEXT(f) + XTPSDxDSL-US-FEXT(f)	watts/Hz

XTPSDxDSL-CPE(f) = XTPSDxDSL-US-NEXT(f) + XTPSDxDSL-DS-FEXT(f)	watts/Hz

As for TCM-ISDN DSL NEXT and FEXT, cyclostationary crosstalk injection of NEXT and FEXT as defined in 7.9.3 is adopted. Cyclostationary NEXT and FEXT injection timing is shown in Figure B.1, which is reproduced from Figure 31.

Figure B.1/G.996.1 – TCM-ISDN DSL alternate NEXT and FEXT injection timing

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |