|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

| ITU-T | J.113 |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU | (03/98) |

SERIES J: TRANSMISSION OF TELEVISION, SOUND PROGRAMME AND OTHER MULTIMEDIA SIGNALS

Interactive systems for digital television distribution

Digital video broadcasting interaction 
channel through the PSTN/ISDN

ITU-T  Recommendation  J.113

(Previously  CCITT  Recommendation)

ITU-T  J-SERIES  RECOMMENDATIONS

TRANSMISSION OF TELEVISION, SOUND PROGRAMME AND OTHER MULTIMEDIA SIGNALS

| For further details, please refer to ITU-T List of Recommendations. |  |
| --- | --- |
| General Recommendations | J.1–J.9 |
| General specifications for analogue sound-programme transmission | J.10–J.19 |
| Performance characteristics of analogue sound-programme circuits | J.20–J.29 |
| Equipment and lines used for analogue sound-programme circuits | J.30–J.39 |
| Digital encoders for analogue sound-programme signals | J.40–J.49 |
| Digital transmission of sound-programme signals | J.50–J.59 |
| Circuits for analogue television transmission | J.60–J.69 |
| Analogue television transmission over metallic lines and interconnection with radio-relay links | J.70–J.79 |
| Digital transmission of television signals | J.80–J.89 |
| Ancillary digital services for television transmission | J.90–J.99 |
| Operational requirements and methods for television transmission | J.100–J.109 |
| Interactive systems for digital television distribution | J.110–J.129 |
| Transport of MPEG-2 signals on packetised networks | J.130–J.139 |
| Measurement of the quality of service | J.140–J.149 |
| Digital television distribution through local subscriber networks | J.150–J.159 |

ITU-T  RECOMMENDATION  J.113

DIGITAL  VIDEO  BROADCASTING  INTERACTION  
CHANNEL  THROUGH  THE  PSTN/ISDN

Summary

This Recommendation describes the provision of return channel interaction protocols over PSTN and ISDN to, and independently of, digital TV broadcast systems.

Source

ITU-T Recommendation J.113 was prepared by ITU-T Study Group 9 (1997-2000) and was approved under the WTSC Resolution No. 1 procedure on the 18th of March 1998.

Keywords

Digital TV services, interaction channel, ISDN, PSTN.

FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL  PROPERTY  RIGHTS

The ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. The ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, the ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  1998

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Definitions		1

4	Abbreviations		1

5	Protocol stack and system models		2

5.1	Protocol stack model		2

5.2	System model/overview		2

6	Interaction channel for PSTN		3

6.1	PSTN requirement		3

6.2	PSTN interfaces requirement		3

7	Interaction channel for ISDN		3

7.1	ISDN requirement		3

7.2	ISDN interfaces requirement		3

Introduction

This Recommendation is one of a series describing interactive digital TV services. This Recommendation describes the provision of return channel interaction protocols over PSTN and ISDN to, and independently of, digital TV broadcast systems. Other relevant Recommendations are J.110 which describes the basic principles and J.111 which describes protocols independent of the underlying physical and transport protocols of such services.

Recommendation J.113

Recommendation J.113     (03/98)

DIGITAL  VIDEO  BROADCASTING  INTERACTION  
CHANNEL  THROUGH  THE  PSTN/ISDN

(Geneva, 1998)

# 1	Scope

This Recommendation describes the provision of return channel interaction protocols over PSTN and ISDN to, and independently of, digital TV broadcast systems. Other relevant Recommendations are J.110 [1] which describes the basic principles and J.111 [2] which describes the network independent return channel protocols.

# 2	References

The following ITU-T Recommendations, and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[1]	ITU-T Recommendation J.110 (1997), Basic principles for a worldwide common family of systems for the provision of interactive television services.

[2]	ITU-T Recommendation J.111 (1998), Network independent protocols for interactive systems.

[3]	ITU-T Recommendation I.3xx, ISDN – Network aspects.

[4]	ITU-T Recommendations I.430, I.431, ISDN – physical layer.

[5]	ITU-T Recommendation Q.921/I.441 (1997), ISDN user-network interface – Data link layer specification.

[6]	ITU-T Recommendation Q.931/I.451 (1998), ISDN user-network interface Layer 3 specification for basic call control.

[7]	ITU-T Recommendation G.960 (1993), Access digital section for ISDN basic rate access.

[8] 	ITU-T Recommendation G.961 (1993), Digital transmission system on metallic local lines for ISDN basic rate access.

[9]	ITU-T Recommendation I.414 (1997), Overview of Recommendations of Layer 1 for ISDN and B-ISDN customer accesses.

[10]	ITU-T Recommendation I.112 (1993), Vocabulary of terms for ISDN.

# 3	Definitions

The definitions in Recommendation J.110 [1] apply.

# 4	Abbreviations

The abbreviations in Recommendation J.110 [1] apply.

# 5	Protocol stack and system models

## 5.1	Protocol stack model

The protocol stack model used for the purposes of this Recommendation consists of the following layers:

Physical layer: where all the physical (electrical) transmission parameters are defined – network dependent.

Transport layer: defines all the relevant data structures and communication protocols like data containers, etc. – network dependent.

Application layer: is the interactive application software and runtime environments – network independent.

See Figure 1.

FIGURE 1/M.3020...[D01] = 3 CM

This Recommendation describes the network dependent protocols within the transport and physical layers for the PSTN/ISDN. The network independent protocols are described in Recommendation J.111 [2] (to layer 4 of the OSI reference model typically).

## 5.2	System model/overview

Refer to Recommendation J.110 [1].

# 6	Interaction channel for PSTN

## 6.1	PSTN requirement

The PSTN shall provide a bidirectional communication path between interfaces Ax (PSTN/Interactive network adaptor interface) and Ab (PSTN/Interactive interface module interface) of the system model of Figure 2/J.110 [1].

## 6.2	PSTN interfaces requirement

Both the interactive network adaptor at interface Ax and the interactive interface module at interface Ab shall comply with the physical and transport layer functionality requirements relevant to the national PSTN service provider concerned.

# 7	Interaction channel for ISDN

## 7.1	ISDN requirement

The ISDN shall provide a bidirectional communication path between interfaces Ax (ISDN/Interactive network adaptor interface) and Ab (ISDN/Interactive interface module interface) of the system model of Figure 2/J.110 [1]. For the purposes of this Recommendation, it is assumed that the ISDN complies with Recommendation I.3xx [3].

## 7.2	ISDN interfaces requirement

The physical and transport layer functions of the interactive network adaptor at interface Ax and the interactive interface module at interface Ab shall comply with Recommendations I.430, I.431 [4], Q.921/I.441 [5], Q.931/I.451 [6], G.960 [7], G.961 [8], I.414 [9] and I.112 [10].

| ITU-T RECOMMENDATIONS SERIES |  |
| --- | --- |
| Series A | Organization of the work of the ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communication |
| Series Y | Global information infrastructure |
| Series Z | Programming languages |