|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

| ITU-T | J.115 |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU | (09/99) |

SERIES J: TRANSMISSION OF TELEVISION, SOUND PROGRAMME AND OTHER MULTIMEDIA SIGNALS

Interactive systems for digital television distribution

Interaction channel using the global system for mobile communications

ITU-T  Recommendation  J.115

(Previously  CCITT  Recommendation)

ITU-T  J-SERIES  RECOMMENDATIONS

TRANSMISSION OF TELEVISION, SOUND PROGRAMME AND OTHER MULTIMEDIA SIGNALS

| For further details, please refer to ITU-T List of Recommendations. |  |
| --- | --- |
| General Recommendations | J.1–J.9 |
| General specifications for analogue sound-programme transmission | J.10–J.19 |
| Performance characteristics of analogue sound-programme circuits | J.20–J.29 |
| Equipment and lines used for analogue sound-programme circuits | J.30–J.39 |
| Digital encoders for analogue sound-programme signals | J.40–J.49 |
| Digital transmission of sound-programme signals | J.50–J.59 |
| Circuits for analogue television transmission | J.60–J.69 |
| Analogue television transmission over metallic lines and interconnection with radio-relay links | J.70–J.79 |
| Digital transmission of television signals | J.80–J.89 |
| Ancillary digital services for television transmission | J.90–J.99 |
| Operational requirements and methods for television transmission | J.100–J.109 |
| Interactive systems for digital television distribution | J.110–J.129 |
| Transport of MPEG-2 signals on packetised networks | J.130–J.139 |
| Measurement of the quality of service | J.140–J.149 |
| Digital television distribution through local subscriber networks | J.150–J.159 |

ITU-T  RECOMMENDATION  J.115

INTERACTION  CHANNEL  USING  THE  GLOBAL  SYSTEM
FOR  MOBILE  COMMUNICATIONS

Summary

This Recommendation is the baseline specification for the provision of an interaction channel using the Global System for Mobile Communications (GSM) in conjunction with a digital broadcasting delivery medium. It is suitable for use with any broadcast transmission medium, such as cable, satellite, terrestrial, etc., thus ensuring the maximum degree of interoperability and economies of scale.

Source

ITU-T Recommendation J.115 was prepared by ITU-T Study Group 9 (1997-2000) and was approved under the WTSC Resolution No. 1 procedure on the 16 September 1999.

FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL  PROPERTY  RIGHTS

The ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. The ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, the ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2000

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Abbreviations		2

4	Reference model		3

4.1	Protocol stack model		3

4.2	System model		4

5	DVB interaction channel specification for GSM		4

5.1	Physical interfaces		5

5.1.1	External MS		5

5.1.2	Integrated MS		5

5.2	Calling procedures		5

5.3	Forced disconnection		5

Appendix I – Basic characteristics of GSM		6

I.1	GSM, general		6

I.2	Future GSM data services		6

Apendix II – Interworking types		7

II.1	Interworking to PSTN		7

II.2	Interworking to ISDN		8

Introduction

This Recommendation, for the use of the GSM system to provide an interaction channel for a digital broadcasting service, is not transmission medium specific. It can be used with any of the broadcast transmission media currently standardized by the ITU. It is consistent with the generic reference model described in Recommendation J.110, and the network independent protocols in Recommendation J.111.

Recommendation J.115

Recommendation J.115     (09/99)

INTERACTION  CHANNEL  USING  THE  GLOBAL  SYSTEM
FOR  MOBILE  COMMUNICATIONS

(Geneva, 1999)

# 1	Scope

This Recommendation is the baseline specification for the provision of an interaction channel based on Global System for Mobile communications (GSM) to Digital Video Broadcasting (DVB) systems.

This Recommendation does not intend to specify an interaction channel solution associated to each broadcast system because the interoperability of different delivery media to transport the interaction channel is desirable. Therefore the GSM solution for the interaction channel apply to satellite, cable, MATV, SMATV, terrestrial, microwave or any future DVB broadcasting or distribution system.

The solutions provided in this Recommendation for an interaction channel through the GSM are a part of a wider set of alternatives to implement interactive services for DVB systems.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[1]	ITU-T Recommendation J.110 (1997), Basic principles for a worldwide common family of systems for the provision of interactive television services.

[2]	ITU-T Recommendation J.111 (1998), Network independent protocols for interactive systems.

[3]	EN 50201: 1998, Interfaces for DVB-IRD.

[4]	ETSI ETS 300 505, Digital cellular telecommunications system (Phase 2); Mobile Station (MS) features (GSM 02.07 version 4.8.2).

[5]	ETSI ETS 300 556, European digital cellular telecommunications system (Phase 2); Mobile radio interface signalling layer 3; General aspects (GSM 04.07).

[6]	ETSI ETS 300 557, Digital cellular telecommunications system (Phase 2); Mobile radio interface; Layer 3 specification (GSM 04.08 version 4.22.0).

[7]	ETSI ETS 300 600, European digital cellular telecommunications system (Phase 2); Signalling requirements on interworking between the Integrated Services Digital Network (ISDN) or Public Switched Telephone Network (PSTN) and the Public Land Mobile Network (PLMN) (GSM 09.03).

[8]	ETSI ETS 300 604, Digital cellular telecommunications system (Phase 2); General requirements on interworking between the Public Land Mobile Network (PLMN) and the Integrated Services Digital Network (ISDN) or Public Switched Telephone Network (PSTN) (GSM 09.07).

[9]	ETSI ETS 300 582, Digital cellular telecommunications system (Phase 2); General on Terminal Adaptation Functions (TAF) for Mobile Stations (MS) (GSM 07.01).

[10]	ETSI ETS 300 583, European digital cellular telecommunications system (Phase 2); Terminal Adaptation Functions (TAF) for services using asynchronous bearer capabilities (GSM 07.02).

[11]	ETSI ETS 300 500, Digital cellular telecommunications system (Phase 2); Principles of telecommunication services supported by a GSM Public Land Mobile Network (PLMN) (GSM 02.01).

[12]	ETSI ETS 300 501, European digital cellular telecommunications system (Phase 2); Bearer Services (BS) supported by a GSM Public Land Mobile Network (PLMN) (GSM 02.02).

[13]	ETSI ETS 300 522, Digital cellular telecommunications system (Phase 2); Network architecture (GSM 03.02).

[14]	ETSI ETS 300 528, European digital cellular telecommunications system (Phase 2); GSM Public Land Mobile Network (PLMN) connection types (GSM 03.10).

[15]	ETSI ETS 300 550, European digital cellular telecommunications system (Phase 2); Mobile Station – Base Station System (MS – BSS) interface; General aspects and principles (GSM 04.01).

[16]	ETSI ETS 300 551, European digital cellular telecommunications system (Phase 2); GSM Public Land Mobile Network (PLMN) access reference configuration (GSM 04.02).

[17]	ETSI ETS 300 552, European digital cellular telecommunications system (Phase 2); Mobile Station – Base Station System (MS – BSS) interface; Channel structures and access capabilities (GSM 04.03).

[18]	ETSI ETS 300 554, European digital cellular telecommunications system (Phase 2); Data Link (DL) layer; General aspects (GSM 04.05).

[19]	ETSI ETS 300 555, European digital cellular telecommunications system (Phase 2); Mobile Station – Base Station System (MS – BSS) interface; Data Link (DL) layer specification (GSM 04.06).

[20]	ETSI ETS 300 562, European digital cellular telecommunications system (Phase 2); Rate adaption on the Mobile Station – Base Station System (MS – BSS) interface (GSM 04.21).

[21]	ETSI ETS 300 586, European digital cellular telecommunications system (Phase 2); Use of the V series Data Terminal Equipment – Data Circuit terminating Equipment (DTE – DCE) interface at the Mobile Station (MS) for Mobile Termination (MT) configuration (GSM 07.06).

# 3	Abbreviations

This Recommendation uses the following abbreviations:

BC	Broadcast Channel

BIM	Broadcast Interface Module

BSC	Base Station Controller

BTS	Base Transceiver Station

DCE	Data Circuit-terminating Equipment

DTE	Data Terminal Equipment

DVB	Digital Video Broadcasting

EN	European Norm

ETS	European Telecommunication Standard

GMSK	Gaussian Minimum Shift Keying

GSM	Global System for Mobile Communications

IC	Interactive Channel

IIM	Interactive Interface Module

INA	Interactive Network Adapter

ISDN	Integrated Switched Digital Network

MS	Mobile Station

MSC	Mobile Switching Centre

MT	Mobile Termination

NIU	Network Interface Unit

OSI	Open Systems Interconnection

PSTN	Public Switched Telephone Network

RA	Radio Adaptation

SMS	Short Message Service

STB	Set-Top Box

STU	Set-Top Unit

TDMA	Time Division Multiple Access

TE	Terminating Equipment

# 4	Reference model

A reference model for the system architecture of narrow-band interaction channels in a broadband scenario (asymmetric interactive services) is presented in this clause. [1]

## 4.1	Protocol stack model

Within the DVB requirements for asymmetric interactive services supporting broadcast to the home with narrow-band return channel, a simple communications model has been used to identify the necessity and importance of each requirement consisting of the following layers (the layers do not coincide exactly with the OSI layers):

Physical layer:	Where all the physical (electrical) transmission parameters are defined.

Transport layer:	Defines all the relevant data structures and communication protocols like data containers, etc.

Application layer:	Is the interactive application software and runtime environment (e.g. home shopping application, script interpreter, etc.).

This Recommendation addresses the lower two layers (the physical and transport layers), leaving the application layer open to competitive market forces. A simplified model of the OSI layers was adopted to facilitate the production of specifications for these nodes. Figure 1 points out the lower layers of the simplified model and identifies some of the key parameters. Following the user requirements for interactive services, no attempt will be made to consider higher layers in this Recommendation.

FIGURE 1/J.115...[D01]

This Recommendation addresses the GSM specific aspects only. The network-independent protocols are specified separately in Recommendation J.111 [2].

## 4.2	System model

Figure 2 shows the system model which is to be used within DVB for interactive services. In the system model, two channels are established between the service provider and the user:

–	Broadcast Channel (BC): A unidirectional broadband BC including video, audio and data. BC is established from the service provider to the users. It may include the forward interaction path.

–	Interaction Channel (IC): A bidirectional IC is established between the service provider and the user for interaction purposes. It is formed by:

–	Return interaction path (return channel): From the user to the service provider. It is used to make requests to the service provider or to answer questions. It is a narrow-band channel. Also commonly known as return channel.

–	Forward interaction path: From the service provider to the user. It is used to provide some sort of information by the service provider to the user and any other required communication for the interactive service provision. It may be embedded into the Broadcast Channel. It is possible that this channel is not required in some simple implementations which make use of the Broadcast Channel for the carriage of data to the user.

FIGURE 2/J.115...[D02]

The user terminal, here named Set-Top Box (STB), is formed by the Network Interface Unit (NIU) [consisting of the Broadcast Interface Module (BIM) and the Interactive Interface Module (IIM)] and the Set-Top Unit (STU). The user terminal provides interfaces for both broadcast and interaction channels. The interface between the user terminal and the interaction network is via the Interactive Interface Module (IIM).

# 5	DVB interaction channel specification for GSM

A GSM infrastructure can support the implementation of the interaction channel for DVB broadcasting systems by providing a wireless bidirectional communication path between the user terminal and an infrastructure connecting to the service provider (see Figure 3).

GSM is a wireless access technology which constitutes the whole or a part of the Interaction Network. The GSM Network may be complemented with another network to reach the service provider (commonly PSTN/ISDN).

In order to allow the access to the GSM Network, the user terminal shall be provided with a GSM Interactive Interface Module (IIM) referred to as a Mobile Station (MS). The interface between the MS and the GSM Network shall be compliant with the standard requirements on General Terminal Adaption Functions (TAF) for Mobile Stations (MS) as in GSM 07.01 [9] and Terminal Adaption Functions for services using asynchronous bearer capabilities GSM 07.02 [10].

The interface between the GSM Network and the external network to provide the whole Interaction Channel shall be compliant with the general and signalling requirements on interworking between the GSM Network and the ISDN or PSTN as in GSM 09.07 [8] and GSM 09.03 [7].

Depending on the network linking (if used) the GSM Network to the service provider, the Mobile Station should be configured to support the right bearer capabilities. In Appendix II the interworking functions for PSTN and ISDN are described for informative purposes. When possible, it is preferential to implement GSM-ISDN interworking, which provides an end-to-end digital link between the IIM (Set-Top Box) and the INA (Service provider) with lower connection setup times.

The basic characteristics of GSM are described in Appendix I.

FIGURE 3/J.115...[D03]

## 5.1	Physical interfaces

The physical interfaces relate to how the GSM MS is connected to the STU. It applies to user terminals with internal or external MS.

### 5.1.1	External MS

The external MS shall support the interfaces requirements between the User Terminal (DTE) and the MS (DCE) according to GSM 07.06 [21] and in the same way as described in EN 50201 [3] for "modem interface" (subclause 4.3.1.1).

### 5.1.2	Integrated MS

The internal MS shall meet the same requirements as the external MS with the exception of providing the 9-pin interface connector.

## 5.2	Calling procedures

The connection to the GSM Network shall be established according to GSM 04 series. The signalling protocols are described in "Mobile Radio Interface; Layer 3 Specification" (GSM 04.08) [6]. If the service provider is not reachable for any reason (line is busy, coverage issues, channel availability, etc.), the upper layers will be responsible for repeating the calling attempts and/or informing the user.

## 5.3	Forced disconnection

Disconnection for emergency calls can be implemented by the upper layer protocols using the signalling channel.

Appendix I

Basic characteristics of GSM

## I.1	GSM, general

The GSM (Global System for Mobile Communications) standards can be operated in several frequency bands, e.g. 900 MHz, 1800 MHz, and has reached worldwide success. The GSM network is realized as a network of contiguous cells, providing a complete coverage of the service area. Each cell has a Base Transceiver Station (BTS) operating on a dedicated set of radio channels. BTSs are logically grouped together and controlled by a Base Station Controller. A group of BSCs is served by a Mobile services Switching Centre (MSC). The GSM system uses TDMA and GMSK modulation on 200 kHz wide carriers which results in a gross data rate of 270 kbit/s. This capacity is split in 8 full- or 16 half-rate channels and some signalling channels.

GSM is a digital cellular radio network containing standardized services as speech, fax and data. Some of the GSM data services that can be used in the DVB context is presented below. A GSM network does not have to offer all the specified data services, so described data services may not be implemented in all GSM networks.

GSM teleservices offer an end-to-end communication capability, including terminal equipment functions. GSM bearer services operate at OSI layers 1 to 3 between access points, serving as a base for teleservices (not the teleservice SMS) or user-specific communication protocols. GSM offers secure data transfer within the GSM network, due to subscriber authentication and data enciphering.

GSM specifies a teleservice called SMS (Short Message Service). The short message MT/PP (Mobile Terminated/ Point-to-Point) and short message MO/PP (Mobile Originated/Point-to-Point) services can be used to transmit text messages with a maximum length of 160 characters (SMS messages longer than 160 characters are specified and is achieved by a concatenation procedure) between a mobile station and an SMS Service Centre (SMS-SC). An SMS-SC operates as a store-and-forward relay for messages and can be accessed via different sources.

GSM specifies several bearer services. Bearer services are circuit switched and data rates up to 9.6 kbit/s are at present supported. A non-transparent bearer service uses ARQ (automatic retransmission request) and has, therefore an error rate of effectively zero, but a variable delay.

A GSM Network can interwork with other GSM Networks, PSTN (Public Switched Telephone Network), ISDN (Integrated Services Digital Network) and PSPDN (Packet Switched Public Data Network).

## I.2	Future GSM data services

The GSM phase 1 and phase 2 data services, as those presented in I.1, are introduced in most networks. GSM is still evolving, where ETSI is standardizing GSM phase 2+ services.

One of the phase 2+ services are packet data in GSM with speeds up to around 100 kbit/s. The GPRS (General Packet Radio Service) standardization is in its final stages.

A High-Speed Circuit-Switched Data (HSCSD) service formed by combining several traffic channels has also been standardized.

Apendix II

Interworking types

When two dissimilar networks are required to interwork in order to support a communication between two subscribers, one on each network, a number of Interworking Functions (MSC/IWFs) are required to support the communication.

Service interworking is required when the Teleservices at the calling and called terminals are different. No service interworking has been identified as a requirement of GSM system for the purpose of this Recommendation. However, network interworking is required whenever a GSM network and a non-GSM network together are involved to provide an end-to-end connection.

The concept of Bearer Services was developed for the ISDN and has been extended to the GSM. A Bearer Service is described as a type of telecommunication service that provides the capability for the transmission of signals between user-network interfaces.

Bearer Services are described by a number of attributes, where an attribute is defined as a specified characteristic of an object or element whose values distinguish that object or element from others. Refer to GSM 02.02 [12] for a complete list of bearer services, and GSM 04.08 [6] for coding of Bearer Capabilities.

| Bearer Service category in GSM network | Bearer Service in GSM | Bearer Service in ISDN | Service in PSTN |
| --- | --- | --- | --- |
| Circuit mode unstructured with unrestricted digital capability. | Asynchronous Data 300 bit/s | Circuit mode structured, 64 kbit/s unrestricted | Not applicable |
|  | Asynchronous Data 1.2 kbit/s |  |  |
|  | Asynchronous Data 1200/75 bit/s |  |  |
| Transparent and Non-transparent | Asynchronous Data 2.4 kbit/s |  |  |
|  | Asynchronous Data 4.8 kbit/s |  |  |
|  | Asynchronous Data 9.6 kbit/s |  |  |
| 3.1 kHz Audio Ex PLMN | Asynchronous Data 300 bit/s | Circuit mode 3.1 kHz Audio | Circuit mode 3.1 kHz Audio |
|  | Asynchronous Data 1.2 kbit/s |  |  |
| Transparent and Non-transparent | Asynchronous Data 1200/75 bit/s |  |  |
|  | Asynchronous Data 2.4 kbit/s |  |  |
|  | Asynchronous Data 4.8 kbit/s |  |  |
|  | Asynchronous Data 9.6 kbit/s |  |  |

It is necessary to consider separately each type of interworking (i.e. GSM-ISDN and GSM-PSTN) since, in the worst case, "PSTN" could refer to an essentially analogue network without common-channel signalling.

## II.1	Interworking to PSTN

For interworking of data calls between GSM and a PSTN, a modem will be utilized to provide the interworking function (see Figure II.1).

FIGURE I.1/J.115...[D04]

The interworking function will need to negotiate with the user to establish the appropriate modem selection e.g. data rate, modulation scheme, etc. In addition, it will also be required to convert the signalling format, from a combination of out-of-band and in-band, to that suitable for controlling the modem and the autocalling line procedure function where applicable. In the following modem selection procedures it is assumed that the interworking function and modems will be associated with each MSC.

For a data call originated by a circuit mode data terminal on the GSM network, the modem selection is done by using the element "modem type" in the call set-up message (bearer capability).

In addition, other elements of the call setup will indicate the user rate, etc. to be used via that modem. The use of this information, however, means that the network is only able to select a modem from the modem pool which conforms to the speed which the terminal is utilizing at the DTE/DCE interface at the mobile station (e.g. V.22 for 1200 bit/s). The exception to this is where the user has selected the non-transparent service in which case either an auto-bauding or multi-self-selecting-speed modem (e.g. V.32) may be used.

## II.2	Interworking to ISDN

Low layer compatibility checking of the mobile-originated call is carried out by the MSC/IWF to determine the appropriate bearer service selection in the ISDN. This will entail the MSC/IWF in mapping appropriately the GSM BC-IE (Bearer Capability Information Element) to the ISDN BC-IE. If it is not possible for the MSC/IWF to provide a bearer service match, then the MSC/IWF should fail the call and indicate the reason to the user.

The MS should provide further compatibility information (LLC/HLC-IEs) if required for defining end-to-end compatibility.

Where the bearer capability information indicates that the call is a circuit-switched unrestricted digital call, then the MSC/IWF should select the appropriate rate adapted ISDN bearer service.

The selection of the MSC/IWF will be by means of the bearer capability information within the call setup message. The mobile subscriber should be able to select the unrestricted digital capability, which the MSC/IWF will map to the same capability in the ISDN call setup message. If an interworking point is encountered within the ISDN which does not support this service request, then a cause failure message indicating network unable to support service requested will be returned to the GSM network which will then pass this to the mobile subscriber. This will be used at the MS to clear the call. It will then be possible for the mobile subscriber to initiate a new call request this time indicating the transfer capability "3.1 kHz Ex GSM" plus other attributes such as user rate, modem type, etc.

| ITU-T RECOMMENDATIONS SERIES |  |
| --- | --- |
| Series A | Organization of the work of the ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure |
| Series Z | Languages and general software aspects for telecommunication systems |