### 6.3.1	Digital TV Video Processing

Recommendation J.117     (09/99)

The application running on the A/V Source shall control the output of data from the MPEG Transport Stream Source, as shown in Figure 14. The data out of the A/V Source must reach the decoder in the DTV Display consistent with its originator’s requirements. The link between the A/V Source and the DTV is accomplished using an isochronous channel on the IEEE 1394 bus. This provides the guaranteed bit rate required for the transmission of MPEG data.

The data passes to the DTV MPEG Decoder, where the Transport Stream is parsed into video, audio and data streams. The video stream is decoded under the control of the DTV’s local application. The Decoder selects the video and audio to be decoded based on 6.8.2.

The baseband video is then sent on to the Compositor/Format Conversion block where the OSD is mixed with the video and the video may be reformatted to accommodate the limitations of the display. The order of compositor and format conversion processes is a matter of implementation and is not specified in this Recommendation.

### 6.3.2	On-Screen Display

#### 6.3.2.1	Analogue OSD

Support for analogue systems is optional. The application running on the A/V Source may use its NTSC analogue video output as a display method for GUIs. It may overlay its own OSD onto the NTSC analogue signal before delivery to the display.

#### 6.3.2.2	Digital OSD

The application in the A/V Source will generate a bitmap to be transmitted to the DTV. The use of bitmaps means that the look and feel of the OSD is under the control of the A/V Source. The DTV is required to support one graphics plane. Support of bitmaps over an analogue input is optional.

##### 6.3.2.2.1 	OSD Delivery

AN OSD protocol is defined in this subclause that provides the following features:

	4 and 8 bit Colour Look-Up Tables (CLUTs).

	Various defined pixel formats.

	Source can specify colourimetry preference.

	Global writes to load the OSD with a constant value.

	Global clear.

	A data block write to any arbitrary rectangular region of the OSD.

##### 6.3.2.2.2	OSD Transmission

The OSD is transmitted to the DTV via an "asynchronous push" connection on the IEEE 1394 link. Protocols used to set up this connection are defined in Annex A.

##### 6.3.2.2.3	DTV OSD Processing

The DTV shall assemble the OSD bitmap using information transmitted over the IEEE 1394 link and shall mix the bitmap with the digital video, and optionally the analogue video.

### 6.3.3	Discovery of capability

IEEE 1394 asynchronous transactions shall be used by the A/V Source to obtain information on the capabilities of the DTV. This discovery process is defined in 6.9.

### 6.3.4	User-machine control

The user controls the DTV or A/V Source using the Remote Control or front panel controls for that particular unit. Feedback to the user can be provided via the bitmap graphics transmitted over the IEEE 1394 link to be rendered by the DTV.

An illustration of the user-machine control is shown in Figure 15.

### 6.3.5	Optional Analogue Audio and Video Processing

Support for an NTSC analogue interface is recommended but optional in this Recommendation.2

When supported, the analogue signal originates from the analogue A/V Source that is selected by the DTV. The signal is then processed in the DTV in preparation to being presented on the display. This processing may include:

	Format conversion to the native mode of the display.

	Image resizing.

## 6.4	Miscellaneous Requirements

### 6.4.1	Digital Interface Physical layer

The DTV shall support IEEE 1394 at s200 clock rate or higher. At least one IEEE 1394 connector shall be present on the DTV. The connectors may be 4- or 6-pin. All 4-pin connectors shall conform to the connector specification given in IEEE P1394a [3] All 6-pin connectors shall conform with the connector specification given in IEEE 1394-1995 [2]. Support for two or more IEEE 1394 connectors on each device is recommended. The provision of multiple connectors facilitates bus interconnections in the A/V cluster and supports home networking.

### 6.4.2	Digital Interface Link layer

Link layer implementations conforming to this Recommendation shall conform to IEEE 1394-1995 [2], Chapter 6.

### 6.4.3	Digital Interface Transaction layer

Transaction layer implementations conforming to this Recommendation shall conform to IEEE 1394-1995 [2], Chapter 7.

### 6.4.4	Serial bus management

Serial bus management implementations conforming to this Recommendation shall conform to IEEE 1394-1995 [2], Chapter 8.

### 6.4.5	Command and status registers

#### 6.4.5.1	CSR core registers

CSR core registers shall conform to IEEE 1394-1995 [2]. The STATE_CLEAR.cmstr bit shall be implemented according to IEC 61883-1 [5].

#### 6.4.5.2	Serial bus node registers

Serial bus node registers shall be implemented in conformance with IEC 61883-1 [5].

#### 6.4.5.3	Configuration ROM requirements

The DTV shall implement the ROM format as defined in IEC 61883-1 [5]. Implementation requirements for Bus_info_block, Root_directory, and 61883_Unit_Directory shall conform to IEC 61883-1 [5]. A more detailed description of the configuration ROM is discussed in 6.9, which deals with the discovery process.

##### 6.4.5.3.1	IEC 61883 Unit directory

Implementation requirements for Unit_Directory in this Recommendation shall conform to IEC 61883-1 [5]. The Unit_SW_Version shall indicate support for CTS (the least significant bit of the first byte of Unit_SW_Version shall be set to 1) and show AV/C compliance (the least significant bit of the third byte of Unit_SW_Version shall be set to 1). This subject is also covered in 6.9.1.2.

##### 6.4.5.3.2	EIA Unit directory

The EIA Unit Directory is defined in 6.9.1.3.

### 6.4.6	Real time transmission of MPEG-2 Transport Streams

Transmission of MPEG-2 Transport Stream packets over the IEEE 1394 serial bus shall be in accordance with IEC 61883-4 [6].

### 6.4.7	Isochronous data flow management

Isochronous data connections shall be managed via the plug control register (PCR) mechanism defined in IEC 61883-1 [5].

### 6.4.8	Connection management procedures (CMP)

Establishment of connections between input and output plugs on the 1394 bus, breaking of connections on the 1394 bus, and management of connections after a bus reset shall be handled in accordance with IEC 61883-1 [5].

The DTV shall supply a minimum of one input PCR for delivery of digital Transport Streams.

In general, destination devices should set up isochronous channels to source devices. The isochronous channel shall accommodate a 40 Mbit/s bitstream.

#### 6.4.8.1	Isochronous Connection Procedures

Note that the location of oMPR and oPCRs on a source node are determined by Section 7.11 (Figure 14) of IEC 61883. Figure 14 of IEC 61883 specifies the location of Plug Control Registers and is shown in Table 2.

Table 2/J.117 – Plug Control Register Location

| FFFF F000 090016 | oMPR |
| --- | --- |
| FFFF F000 090416 | oPCR[0] |
| ... | ... |
| FFFF F000 097C16 | oPCR[30] |

The number of implemented oPCRs are contained in the #oPCR field (bits 0-4) of the oMPR. #oPCR can be a number from 0-31 (Section 7.2 of IEC 61883). oPCR[0] through oPCR[#oPCR-1] are the implemented oPCRs (Section 7.2 of IEC 61883).

To make a connection from a Video Source to a Video Destination with the Video Destination Device as the controller, the following procedures can be followed:

1)	The Destination Device uses the AV/C OPEN DESCRIPTOR and READ DESCRIPTOR commands to read the Source Device’s Unit Descriptor and determine the output plug.

2)	The Destination Device reads the oPCR associated with the proper output plug to determine the proper bandwidth. Bus Bandwidth Units (BWU) can be calculated from three parameters (data rate, overhead ID, and payload) using the procedure described in Section 7.7 of IEC 61883.

3)	The Destination Device uses the procedure described in Section 8.2 IEC 61883 to establish the connection.

### 6.4.9	Asynchronous connection management

The following rules shall apply to setting up asynchronous connections:

1)	An OSD connection shall only be broken by the node that established it. Asynchronous connections are analogous to isochronous connections in this way.

2)	The OSD consumer (the DTV) shall play the "controller" role (see Annex A.10) in connecting and disconnecting asynchronous plugs.

### 6.4.10	Function Control Protocol (FCP)

Commands carried on the 1394 bus in conformance with this Recommendation shall be carried within the Function Control Protocol defined in IEC 61883-1 [5].

### 6.4.11	Analogue/Digital source selection

The specifications in this subclause apply to DTVs which support the optional analogue/digital source selection under the control of an audio/video source device.

The DTV shall support the CONNECT command for the purpose of allowing an external device that has both analogue and digital outputs to signal which of the two it wishes the DTV to process for display.

If the DTV has selected a particular A/V source, it shall accept a CONNECT command from that source indicating:

source_subunit_type = 1F16, indicating AV unit,

source_subunit_ID = 7, used when subunit type is AV unit,

source_plug indicating either a) the serial bus iPCR plug or b) the external default analogue input plug (both these plug values are provided in the Unit Identifier Descriptor),

destination_subunit_type = 0, indicating video monitor,

destination_subunit_ID indicating the ID of the Monitor Subunit (discovered through the SUBUNIT INFO command),

destination_plug = 0, indicating the Monitor Subunit’s main display input.

When these conditions are met, both the CONTROL and STATUS forms of the command shall be supported.

If the DTV receives a CONNECT command as above, but not from the currently selected A/V source device, it may disconnect from the currently connected source and select the new device and accept and process the command. Alternatively, it may respond with a REJECTED response.

The producer may offer a user option to selectively withhold audio, video, or OSD from the DTV. If such an option is not present, no output (including analogue video) shall be withheld.

### 6.4.12	Content Advisory Information

The DTV shall process any content advisory information that it may receive.

Source devices shall not remove content advisory information that is already present prior to passing the program onto the DTV.

## 6.5	Delivery of OSD Data

The DTV shall support the delivery of OSD data via the asynchronous connections method and protocol defined in Annex A. In an asynchronous connection, data flows between producer and consumer nodes. For this application, the DTV acts as the consumer and any device on the 1394 bus that is a source of OSD data can act as producer.

Data transfer involves a sequence of asynchronous write transactions from producer to consumer. The consumer provides to the producer the address space of a segment buffer. The producer writes to the segment buffer and updates a register indicating to the consumer the availability of data. The consumer processes data coming into the segment buffer and updates a register in the producer that indicates available buffer space.

Flow control is achieved by this count-update protocol, and effectively limits the rate of data flow to the slower of either the producer or consumer nodes.

The OSD data is grouped into variable length sequences of data bytes called frames. A frame contains a number of subframes. In this protocol, a number of different subframe types are defined to deliver the OSD data. Frame boundaries act as synchronization points for these subframes. OSD subframes for OSD data are defined in 6.7.

Annex A defines the AV/C commands that are used to set up and take down Asynchronous Connections, and defines the procedures that shall be used to re-establish Asynchronous connections following a bus reset.

## 6.6	AV/C Command set support

### 6.6.1	AV/C General Commands

The DTV shall comply with the AV/C Digital Interface Command Set General Specification [4]. Support for those commands listed below in Table 3 shall be required for this Recommendation. All of these commands are defined in with the AV/C Digital Interface Command Set General Specification [4], except for ASYNCHRONOUS CONNECTION command which is defined in Annex A.

Table 3/J.117 – Supported Commands

| AV/C Command | Support level (by ctype) |  |  | Comment |
| --- | --- | --- | --- | --- |
|  | C | S | N |  |
| UNIT INFO | – | M | – |  |
| SUBUNIT INFO | – | M | – |  |
| OPEN DESCRIPTOR | M | O | – |  |
| READ DESCRIPTOR | M |  |  |  |
| CONNECT | O/M | O | R | Required for support of source-driven analogue/digital selection, an optional feature |
| ASYNCHRONOUS CONNECTION | M | O | O | As required in Annex A |
| For ctype: C = Control, S = Status and N = Notify. The support level is M = Mandatory, O = Optional, R = Recommended, and – = Not Applicable. |  |  |  |  |

### 6.6.2	AV/C Asynchronous Connection Subfunctions

The AV/C asynchronous command subfunctions required for this Recommendation are summarized in Table 4.

Table 4/J.117 – Subfunction field definitions

| 6.7 OSD Data Subfunction | Value | Meaning |
| --- | --- | --- |
| ALLOCATE | 0116 | Allocate the consumer port resource |
| ATTACH | 0216 | Connect the consumer port to the producer port |
| ALLOCATE_ATTACH | 0316 | Allocate the producer port resource and connect it to the consumer port |
| RELEASE | 0516 | Release the port resource |
| DETACH | 0616 | Disconnect the consumer port |
| DETACH_RELEASE | 0716 | Disconnect and release the producer port resource |
| SUSPEND_PORT | 1016 | Suspend the consumer port |
| RESUME_PORT | 2016 | Resume the consumer port |
| RESTORE_PORT | 4016 | Restore the port resource after a bus reset |

In this subclause the source of the OSD signal is called the OSD Producer and the DTV Display device is called the OSD Consumer.

### 6.7.1	Format of OSD data

#### 6.7.1.1	Subframe types

The defined subframe types are:

Set_OSD_pixel_format: Establishes the format of the basic 16-bit pixels that make up the data definition to follow, and the size and colour depth of the OSD grid. For OSD grid formats with either 4- or 8-bit colour depths, the subframe shall contain either a 4- or 8-bit Colour Look-Up Table (CLUT).

4_bit_OSD_data: Defines 4-bit pixels in a rectangular region. Each 4-bit pixel represents a colour/alpha blend value derived by indirection through the 4-bit CLUT.

8_bit_OSD_data: Defines 8-bit pixels in a rectangular region. Each 8-bit pixel represents a colour/alpha blend value derived by indirection through the 8-bit CLUT.

Uncompressed_16_bit_data: Defines raw uncompressed 16-bit OSD data in a rectangular region.

Fill_region_with_constant: Defines a rectangular region to fill with a 16-bit constant with a format defined by pixel_format.

Clear_OSD: Shall load the complete OSD with a transparent value.

#### 6.7.1.2	Subframe typeCode

The type of each subframe is identified by a typeCode field, as defined in Table 5. All subframes in this protocol, as well as ones defined in future extensions, are formatted with the 8-bit typeCode and 24-bit dataLength in the first quadlet. OSD Consumer equipment that encounters a subframe with an unknown typeCode shall use the dataLength to skip that subframe.

Table 5/J.117 – typeCode coding

| typeCode | Meaning |
| --- | --- |
| 0 | Reserved |
| 1 | Set_OSD_pixel_format |
| 2 | 4_bit_OSD_data |
| 3 | 8_bit_OSD_data |
| 4 | Uncompressed_16_bit_data |
| 5 | Fill_region_with_constant |
| 6 | Clear_OSD |
| 7-255 | Reserved for future use |

#### 6.7.1.3	Subframe processing

The DTV shall process subframes in the order received. A DTV may not have buffer space sufficient to hold a complete frame of OSD data; if that is the case, processing of subframes as they arrive will be necessary. In any case, the DTV should process each subframe as it arrives.

#### 6.7.1.4	Subframe syntax and definition

##### 6.7.1.4.1	Set OSD pixel format subframe

The OSD Producer device shall use the Set_OSD_pixel_format subframe to set the pixel format, colour depth, and CLUT (if applicable) for subsequent delivery of OSD data.

The Set_OSD_pixel_format subframe shall be formatted as shown in Figure 16 when OSD_layout specifies a 16-bit colour depth.

The Set_OSD_pixel_format subframe shall be formatted as shown in Figure 17 when OSD_layout specifies a 4-bit colour depth.

The Set_OSD_pixel_format subframe shall be formatted as shown in Figure 18 when OSD_layout specifies an 8-bit colour depth.

The source shall send the Set_OSD_pixel_format subframe prior to the initial delivery of OSD data to establish the format for the data to follow. The subframe shall also be sent prior to delivery of OSD data in a different pixel format. Note that it is not possible to define one OSD image as a mixture of different pixel formats (pixel_format 0 and 1, for example).

The Set_OSD_pixel_format subframe also selects one of possibly several OSD buffer formats offered by the OSD Consumer.

typeCode for the Set_OSD_pixel_format subframe is 0116.

dataLength shall reflect the number of bytes in the subframe following the dataLength field itself, either 8, 40, or 520, depending upon the colour depth defined in OSD_layout.

OSD_layout specifies the OSD frame buffer dimensions and colour depth and shall be as specified in Table 6.

Table 6/J.117 – OSD layout coding

| OSD_layout | Meaning |
| --- | --- |
| 0 | 640  480  4 |
| 1 | 640  480  8 |
| 2 | 640  480  16 |
| 3-255 | Reserved for future use |

overlay_format specifies the way the OSD Consumer should overlay the selected grid format over decoded video. Table 7 defines coding for overlay_format. Support for formats 1 and 2 is optional. All devices shall support format 0.

Table 7/J.117 – Overlay format coding

| overlay_format | Meaning |
| --- | --- |
| 0 | no stretch requested |
| 1 | Stretch horizontally to 14:9 |
| 2 | Stretch horizontally to 16:9 |
| 3-255 | Reserved for future use |

pixel_format shall be as specified in Table 8. The format for the 16 bit pixel for each pixel format is as shown in Figure 19.

Table 8/J.117 – Pixel format coding

| pixel_format | Meaning |
| --- | --- |
| 0 | Y:Cb:Cr = 6:5:5 |
| 1 | :Y:Cb:Cr = 2:6:4:4 |
| 2 | :Y:Cb:Cr = 4:6:3:3 |
| 3-255 | Reserved for future use |

A zero value for Y indicates lowest luminance (most black). A maximum value for Y (all ones) indicates highest luminance level. Y is related to primary colour signals (R, G, B) in accordance with the colourimetry sections of ITU-R Recommendation BT.709-2 [8], the standard for DTV, or ITU-R Recommendation BT.601-4 [9], the standard for NTSC. An OSD consumer may only support one of the standards; as the complexity of requiring support for both was deemed excessive for the improvement in quality that would result.

Cr and Cb are chrominance vectors related to primary colour signals (R, G, B) in accordance with ITU-R Recommendation BT.709-2 [8] or ITU-R Recommendation BT.601-4 [9].

For pixel_format 0 only, any pixel with a Y value of zero shall be transparent. Pixels with non-zero values of Y shall be opaque.

 is an alpha level associated with pixels in pixel_format 1 or 2.  values for pixel_format 1 are defined in Table 9. For pixel_format 2, values in between 0 and all-ones indicate the mix weight between the OSD pixel and decoded video for each pixel. A zero value for  indicates transparent. The all-ones value indicates opaque. The interpolation between zero and all ones should be approximately linear.

The mix parameter in the Set_OSD_pixel_format subframe is used with pixel_format 1 to specify the alpha-blend value to use for pixels that are neither transparent nor opaque. The following table shall define the interpretation of the 2-bit alpha field in pixels formatted as pixel_format 1. The mix parameter shall be ignored in an OSD Consumer for pixel formats 0 and 2.

Table 9/J.117 –  field interpretation for pixel_format 1

| pixel  | Meaning |
| --- | --- |
| 0 | Opaque |
| 1 | Mix with video using mix parameter in the Set_OSD_pixel_format subframe |
| 2 | Transparent |
| 3 | Reserved for future use |

cs defines the chrominance standard used in the OSD Producer, and acts to notify the OSD Consumer that it should interpret Y-Cb-Cr data in accordance with the referenced standard, if possible. cs is defined in Table 10.

Table 10/J.117 – Colourimetry Standards

| cs | Colour Standard |
| --- | --- |
| 0 | ITU-R BT.709-2 |
| 1 | ITU-R BT.601-4 |

buf indicates whether data is placed into the buffer currently being used for output (buf = 0) or into an a buffer that is not currently being used for output (buf = 1). The buf and sw bits together specify how the data update will take place as shown in Table 11.

When double buffering is not supported in the OSD consumer, buf = 1 has no meaning. In this case the OSD consumer may ignore subframes with buf = 1.

Table 11/J.117 – buf/sw coding

| buf | sw | Rule |
| --- | --- | --- |
| 0 | 0 | Shall put data into the active buffer immediately |
| 0 | 1 | Should start to put data into the active buffer synchronized with the start of the next vertical retrace |
| 1 | 0 | Shall put data into an off-screen buffer |
| 1 | 1 | Shall put data into an off-screen buffer and then swap with the active buffer, synchronized with the start of the next vertical retrace |

sw indicates when the data update will take place.

fill_value is formatted according to OSD_Layout as shown in Table 12.

Table 12/J.117 – fill_value coding

| OSD_layout | format fill_value |
| --- | --- |
| 0 | 4 bits (right justified) |
| 1 | 8 bits (right justified) |
| 2 | as defined by pixel_format Coding |
| 3-255 | Reserved |

Processing within the OSD Consumer of the Set_OSD_pixel_format subframe shall conditionally cause the OSD frame buffers to be initialized to the fill_value value provided:

1)	Processing the first Set_OSD_pixel_format subframe after establishing the OSD connection shall have the effect of setting the OSD frame buffers to the fill pixel value provided in the command.

2)	Processing a subsequent Set_OSD_pixel_format subframe that changes the pixel_format or OSD_layout, the DTV shall initialize the OSD buffers to the fill pixel value provided.

Processing a subsequent Set_OSD_pixel_format subframe that changes only the mix field or CLUT data, previously defined pixel data values do not change.

CLUT[N] is a Colour Look-Up Table Entry. CLUT entries are 16-bits, formatted according to the pixel format defined in pixel_format.

##### 6.7.1.4.2	4-bit OSD data subframe

Figure 20 defines the format of the 4_bit_OSD_data subframe, used to deliver 4-bit pixels from A/V Source to display.

typeCode shall be set to value 0216, indicating the 4_bit_OSD_data subframe format.

The 24-bit dataLength field shall be set to indicate the number of data bytes in the remainder of the subframe. The subframe shall be padded so that the next subframe address is quadlet aligned. Based on the height and width parameters (if both are odd numbers), the least significant 4 bits in the last data byte may be unused.

X_loc is the 12-bit X-coordinate (column number) within the image buffer in the buffer indicated by buf. The coordinate system is defined with 0,0 in the upper left corner.

Y_loc is the 12-bit Y-coordinate (row number) within the image buffer in the buffer indicated by buf.

buf and sw are the same as defined in 6.7.1.4.1, Set_OSD_pixel_format subframe.

width is a 16-bit unsigned integer number representing the width, in pixels, of the OSD region being defined. Value zero is undefined.

height is a 16-bit unsigned integer number representing the height, in pixels, of the OSD region being defined. Value zero is undefined.

pixel[0] through pixel[N] are 4-bit pixel values. The pixels shall be listed in a scan order that is left to right and top to bottom. An illustration of the order is shown in Figure 21. In this particular example width has a value of 6 and height has a value of 4. The display values for each shall be expanded through the 4-bit to 16-bit CLUT defined in the most recently received Set_OSD_pixel_format subframe.

##### 6.7.1.4.3	8-bit OSD data subframe

Figure 22 defines the format of the 8_bit_OSD_data subframe, used to deliver 8-bit pixels from an OSD Producer to an OSD Consumer.

typeCode shall be set to value 0316, indicating the 8_bit_OSD_data subframe format.

The definitions for dataLength, X_loc, Y_loc, width and height are the same as for the 4_bit_OSD_data subframe.

The definitions for buf and sw are the same as defined in 6.7.1.4.1, Set_OSD_pixel_format subframe.

pixel[0] through pixel[N] are 8-bit pixel values. The display values for each shall be derived by indirection through the 8-bit CLUT defined in the last received Set_OSD_pixel_format subframe. The pixels shall be listed in a scan order as shown in Figure 21.

##### 6.7.1.4.4	Uncompressed 16-bit Data Subframe

Figure 23 defines the format of the Uncompressed_16_bit_data subframe, used to deliver uncompressed 16-bit pixels from A/V Source to display.

typeCode shall be set to value 0416, indicating the Uncompressed_16_bit_data subframe format.

The definitions for dataLength, X_loc, Y_loc, width and height are the same as for the 4_bit_OSD_data subframe.

The definitions for buf and sw are the same as defined in 6.7.1.4.1, Set_OSD_pixel_format subframe.

pixel[0] through pixel[N] are 16-bit pixel values. The format of each pixel (in terms of luminance, chrominance, and optional alpha level) are as defined by the Set_OSD_pixel_format subframe. The pixels shall be listed in a scan order as shown in Figure 21.

##### 6.7.1.4.5	Fill region with constant subframe

Figure 24 defines the format of the Fill_region_with_constant subframe, used to direct the display to fill a rectangular area in the display image buffer with a constant value.

typeCode shall be set to value 0516, indicating the Fill_region_with_constant subframe format.

The 24-bit dataLength field shall be set to 12 for this subframe type.

The definitions for dataLength, X_loc, Y_loc, width and height are the same as for the 4_bit_OSD_data subframe.

The definitions for buf and sw are the same as defined in 6.7.1.4.1, Set_OSD_pixel_format subframe.

Fill_Value indicates the constant value to be filled. The format of fill_value shall be as defined by Table 12.

Some implementations of DTVs may provide a feature that allows the colour of the area surrounding the defined OSD grid to be defined. The OSD Producer may specify the fill colour for the surrounding area by setting the width and height fields to FF16.

##### 6.7.1.4.6	Clear OSD subframe

Figure 25 defines the format of the Clear_OSD subframe, used to direct the display to fill the display image buffer with a zero (Transparent) value.

typeCode shall be set to value 0616, indicating the Clear_OSD subframe format.

The 24-bit dataLength field shall be set to 0 for this subframe type.

### 6.7.2	Alignment of OSD with video

A square pixel aspect ratio is desirable. The Monitor Subunit function in the display shall align the 640  480 display grid with video according to the following constraints:

1)	The 640  480 grid shall be centered horizontally within the visible display area.

2)	The 640  480 grid shall be centered vertically within the visible display area, or cover it completely.

An OSD Producer may learn how a DTV will align OSD with various digital video formats by processing data delivered in the Unit Identifier Descriptor, defined in 6.9.3.

### 6.7.3	Behaviour of system during Attach

When an OSD Producer is attached to an OSD Consumer then the OSD Producer may assume that the OSD in the consumer is transparent.

### 6.7.4	Behaviour of system during Suspend/Resume

The DTV may choose whether or not to clear the OSD buffer to all-zeroes (transparent) entering the OSD SUSPEND state defined in A.12.8. It shall be the responsibility of an OSD Producer to define every pixel in the OSD grid following RESUME. Therefore, if the DTV were to take control of OSD for a short time (for example, to display a volume control bar), it could choose not to clear the screen during its use of the screen because it could rely on the full screen being re-initialized when it resumes its connection to the OSD Producer.

## 6.8	Isochronous Data

IEEE 1394 isochronous data channels are used to deliver audio and video data.

### 6.8.1	Service Selection

When an external device supplies a digital transport stream to the DTV, two cases are possible:

a)	The source device includes a navigation application that allows the user to select one service from many that may be available on a given Transport Stream. This device provides a "look and feel" for the user experience and wishes to use the DTV only as a decoder/monitor. If the DTV were to spontaneously take control of OSD and (say) put up a channel banner (channel name and number, program name, etc.), this would interfere with (and possibly conflict with) navigation being handled by the external box.

b)	The source device is something very simple like a stream recorder, and relies on the DTV for service selection.

For case a), the DTV shall use the procedure defined below in 6.8.2 to select audio and video components for decoding and output/display.

The DTV shall recognize the presence of a Single Program Transport Stream (SPTS) as an indication of case a), where the external device is responsible for navigation. If a Transport Stream includes more than one MPEG program (as indicated by the Program Association Table in PID 0), the DTV shall accept responsibility for navigation within that stream.

For case b), the DTV is responsible for service selection. When ATSC A/65 [11] PSIP data is present, it may be used to support this navigation. In this context, however, PSIP shall be processed differently than it would in an off-air digital Transport Stream because:

1)	the Transport Stream must be handled in isolation in the DTV; and

2)	it may have been time-shifted from the point in time that it was originally broadcast.

These considerations lead to the following requirements:

When streams are received through an isochronous link, the channel mapping via the Virtual Channel Table (VCT) and its parameters may change completely at any given time. For example, these changes will occur when a recorded segment in a digital VCR follows another segment with different channels and programs. Accordingly, the Transport Stream ID of the monitored transport stream may also change with different recording segments. In normal DTV operation, the TSID for a given transport stream does not change, therefore DTV receivers should be engineered to operate under variable conditions for this operating mode.

In the VCT, some fields are no longer relevant for the operation of a DTV receiver displaying a program received via a 1394 Isochronous Channel. The fields that may be ignored include, but are not limited to the following: the carrier frequency, the modulation mode, the path-select field, and the out-of-band field.

The major and minor numbers, as well as the short and extended channel names, may be used to provide navigation for services carried within the monitored transport stream. In the case of digital VCRs, major and minor channel numbers also give reference to the original channels from which programs were recorded.

All time of day clock references explicitly or implicitly3 defined in PSIP tables obtained from monitored transport streams should be considered relative to decoded System Time Tables (STT) rather than the local clock of the DTV receiver.

The set of tables identified as EIT-0 give program guide information about services found in the monitored transport stream. Future Event Information Tables (EIT) (EIT-1 to EIT-127), although still valid and applicable, should be considered mostly in an informative sense since there is no guarantee of the constancy of the Transport Stream.

When navigating the Transport Stream, PSIP may identify programs that are not in the current Transport Stream. Accessing such channels is beyond the scope of this Recommendation. The transport_stream_ID (TSID) can be used to make this distinction: Virtual channels with channel_TSID values different than that indicated in the transport_stream_ID field of the Terrestrial Virtual Channel Table (TVCT) or the Cable Virtual Channel Table (CVCT) may be ignored.

### 6.8.2	MPEG-2 Transport Streams

MPEG-2 Transport Stream shall be carried in accordance with IEC 61883-4 [6].

In the case that the source device requests control of the selection of services for decoding in the DTV, the source device shall create a Single Program Transport Stream (SPTS). An SPTS is a valid MPEG-2 Transport Stream, but it contains just one MPEG-2 program.

In the case of a Single Program Transport Stream, the DTV shall use the following process to determine the proper audio/video/data transport packets for decoding:

The MPEG-2 Program Association Table (PAT) in PID 0 shall be examined. The program_map_PID shall be noted.

Transport Stream packets matching the program_map_PID shall be collected to yield the TS_program_map_section for the program.

Parsing the TS_program_map_section will yield the elementary_PID values corresponding to one stream of type 0216 (MPEG-2 video) and one or more streams of type 8116 (AC-3 audio). Other elementary streams (such as data services) may or may not be present. In the case that an audio stream type is not found, audio shall be muted. In the case that a video stream type is not found, video shall be muted.

In the case that multiple audio streams are found, the DTV should choose one based on its language, and based on the user’s default language preference. The DTV should not produce a user dialog at the time of selection, but should instead use the default setting established at unit setup. In the case that no default setting is available, the DTV should select English audio.

The DTV shall de-multiplex the Transport Stream, passing transport packets for these audio and video PID values to the appropriate decoders. If data service components are present and the DTV is capable of processing them, these may be processed.

The DTV shall monitor the PAT and referenced TS_program_map_section continuously for changes. If any change is seen, the PID derivation process shall be repeated.

#### 6.8.2.1	Source alteration of MPEG programs

When processing MPEG programs all elementary stream references shall be maintained unless a user option has deselected an elementary stream.

An example of this is a program may be delivered with a number of audio streams, providing multilingual services. The user can elect to select one audio stream by removing the linkages to other streams in the PAT.

### 6.8.3	DV Streams

The DTV may optionally support decoding of DV coded video. High Definition DV Coded video shall be transported in accordance with IEC 61883-3 [12]. Standard Definition DV Coded video shall be transported in accordance with IEC 61883-2 [13] or IEC 61883-5 [14].

## 6.9	Discovery Process

This subclause defines a discovery process that shall be implemented by every device supporting this Recommendation. The goal of the discovery process is to gather the minimal information necessary to create a functional A/V entertainment cluster. The process relies on the configuration ROM structure as defined in clause 8 of ISO/IEC 13213 [10] and on the OPEN DESCRIPTOR and READ DESCRIPTOR commands of AV/C.

The contents of the configuration ROM, applicable AV/C descriptors, and the discovery process are described below.

### 6.9.1	IEEE 1212 Configuration ROM

This subclause describes briefly the content of the general EIA-775 ROM structure. Most of this structure is a general structure specified in IEEE P1212r [19], IEEE 1212 [10] and IEC 61883. The ROM structure is a hierarchy of information blocks; the blocks higher in the hierarchy point to the blocks beneath them. The location of the initial blocks is fixed while other entries are vendor dependent, but shall be specified by entries within the higher blocks.

#### 6.9.1.1	Bus_Info_block and Root_Directory

Table 13 shows the Bus_Info_Block and Root_Directory of the configuration ROM. The first byte of each entry is known as a key and identifies the type of entry. The following shall be implemented in the configuration ROM of all devices making use of the EIA-775 specification. This includes display devices such as DTVs and source devices such as DVCRs, STBs, etc.

Note that there may be several other structures required based on other protocols to which the device conforms. This diagram contains information for a device which also complies with the IEC 61883-1 [5] protocol. Note that the Root_directory contains pointers to a Model_Directory and two Unit_Directory entries (IEC 61883 and EIA).

#### 6.9.1.2	IEC 61883 Unit Directory

The IEC_61883 unit directory is shown in Table 14. In the Unit_SW_Version field, the least significant bit specifies AV/C (0) as specified in IEC 61883-1 [5]. This directory is pointed at by the Unit_Directory offset, which appears in entry 042816 of the Root Directory (i.e. Table 13).

#### 6.9.1.3	EIA Unit Directory

The EIA Unit Directory is shown in Table 15. The following EIA-775 specific information should appear in the EIA Unit Directory.

Table 13/J.117 – Configuration ROM

Offset      (Base address FFFF F000 0000)

Bus_info_block

| Root_directory Offset |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 04 0016 | 04 | crc_length |  | rom_crc_value |  |  |
| 04 0416 | "1394" |  |  |  |  |  |
| 04 0816 | flags | reserved | cyc_clk_acc |  | max_rec | reserved |
| 04 0C16 | node_vendor_id |  |  |  | chip_id_hi |  |
| 04 1016 | chip_id_lo |  |  |  |  |  |

| Offset |  |  |  |
| --- | --- | --- | --- |
| 04 1416 | root_length |  | CRC |
| 04 1816 | 0316 | model_vendor_id |  |
| 04 1C16 | 8116 | vendor_name_textual_descriptor offset |  |
| 04 2016 | 0C16 | node_capabilities |  |
| 04 2416 | 8D16 | node_unique_id offset |  |
| 04 2816 | D116 | unit_Directory offset (CEI 61883) |  |
|  | D116 | unit_Directory offset (EIA) |  |
|  | optional |  |  |
| xx xx16 | C316 | model_Directory offset |  |

Table 14/J.117 – IEC_61883 unit directory

Unit_Directory (IEC_61883)

| directory length |  | CRC |
| --- | --- | --- |
| 1216 | Unit_Spec_ID (1394TA = 00 A0 2D16) |  |
| 1316 | Unit_SW_Version (first pass key = 0116) |  |
| …. | <<possibly other fields>> |  |
| .... | .... |  |

Table 15/J.117 – EIA unit directory

Unit_Directory (EIA)

| directory length |  | CRC |
| --- | --- | --- |
| 1216 | Unit_Spec_ID (EIA = 00506816) |  |
| 1316 | Unit_SW_Version = 01010016 |  |
| …. | <<possibly other fields>> |  |
| .... | .... |  |

	Unit_Spec_ID = EIA (00506816)

	Unit_SW_Version = 01010016

The Unit_SW_Version designates that the device makes use of this Recommendation. The general format is shown in Table 16.

Table 16/J.117 – Unit_SW_Version coding

| EIA_protocol_specifier indicates the EIA protocol whose version is given in the second and third octets. Value zero is reserved. Value 0116 shall indicate this Recommendation. Other values are reserved for future use by EIA to describe other standards and protocols. First octet | EIA_protocol_specifier (0116 indicates EIA-755) |
| --- | --- |
| Second octet | Major Version Number (currently 0116) |
| Third octet | Minor Version Number (currently 0016) |

#### 6.9.1.4	Model Directory

In addition to the requirement that the Bus_Info_Block, Root_Directory, and Unit Directories be present, it is also required that the Model Directory be present. The following fields (defined in IEEE P1212r [19]) shall be required of all nodes supporting the EIA-775 specification:

	Model_ID;

	Textual descriptor for Model_ID.

The Model_Directory portion of the ROM is referenced by the Model_Directory offset field in the Root Directory. An illustration of the Model Directory is shown in Table 17.

Table 17/J.117 – Model directory

Model_Directory

| directory length |  | CRC |
| --- | --- | --- |
| 1716 | Model_ID |  |
| 8116 | device_name_textual_descriptor offset |  |
| .... | <<possibly other fields>> |  |
| .... | .... |  |

### 6.9.2	Configuration ROM Discovery

The first part of the discovery process allows devices to discover other devices in the network. This process is activated by a bus reset and serves to search and discover existing devices in the cluster. A bus reset may be caused by connecting/disconnecting a device, software initiated reset, etc. The information acquisition should be done using asynchronous read after the Self ID packets during the bus reset process.

The discovery process shall collect all the required information from the configuration ROM from each device. In the above subclause, the general format of the Configuration ROM is shown. For each device, two types of information are identified, general purpose information, and model specific information as follows.

#### 6.9.2.1	General Purpose Information

The following general purpose information is needed.

	EUI: two quadlets at offset 040C16, and 041016.

Every unit that is shipped will have its own, non volatile, 64-bit Extended Unique Identifier (EUI). The EUI is used by each device in the cluster to correlate node address with a unique device. This may be needed to re-establish connections and resume action after a bus reset since the actual node addresses may change during a bus reset. The information acquisition should be done using asynchronous reads to each node’s configuration ROM.

#### 6.9.2.2	Model Information

The model specific information is stored in the Model_Directory as shown above. It includes textual information. This textual information can be used to build the source selection screen on the DTV.

#### 6.9.2.3	EIA Unit Directory information

The configuration ROM also includes an indication on which EIA specifications are supported by this device. A unit spec ID applicable to EIA appears in the EIA Unit Directory. The Unit_SW_Version, defined within the scope of the EIA, designates the node as supporting EIA-775.

Further information on the device can be obtained from the Unit Descriptors (described in the next subclause). This information includes isochronous output plugs, Isochronous input plugs, asynchronous connection output plugs, and asynchronous connection input plugs. A device that is only a video source and OSD producer would only contain the output plugs associated with those functions. This allows for the DTV to know what devices on the cluster are possible source devices (inputs) to the DTV. It is then possible for the DTV to present to the user a screen which allows them to choose the source device for OSD and Video in a way similar to what is done today. This is important for future devices whose type is not known yet, but may take advantage of the DTV’s OSD and video display capability sometime in the future.

### 6.9.3	Unit Identifier Descriptor

The Unit Identifier Descriptor is a discovery mechanism that provides data relevant to both the DTV and compatible source devices.

The AV/C Digital Interface Command Set General Specification [4] defines a descriptor mechanism in 6.8. The data structures defined are applicable to A/V units as well as subunits. If subunit_type value 1F16 is specified in the AV/C OPEN DESCRIPTOR and READ DESCRIPTOR commands, the whole A/V unit (not a subunit) is targeted, and descriptor_type value 0016 returns the Unit Identifier Descriptor (rather than a subunit identifier descriptor).

In general, Unit Identifier Descriptors provide information at the unit level, such as data related to unit capabilities or external hardware or serial bus plugs. The general format of the Unit Identifier Descriptor provides a portion that may be defined by the 1394 Trade Association, and a portion that contains an arbitrary number of information blocks. One type of info block is the "vendor specific" type, in which the entity that has defined it is identified by its 24-bit IEEE-assigned specifier code (OUI)

For this Recommendation, two EIA-defined info blocks are specified. The first one, called EIA_775_plug_info provides plug IDs for OSD, digital and analogue connections, and shall be supplied by any DTV or source device that is compatible with this Recommendation.

The EIA_775_plug_info block reports:

	The plug ID values to be used for attachment of OSD data streams, for both OSD producers and consumers.

	The plug ID values for analogue audio/video inputs and outputs.

	The plug ID values for MPEG-2 digital video, for both source and destination devices.

The second EIA-defined info block is only defined for the DTV. The EIA_775_DTV_info provides capability information for the benefit of source devices. Any DTV compliant with EIA-775 shall supply both the EIA_775_plug_info and the EIA_775_DTV_info in response to unit-directed OPEN DESCRIPTOR and READ DESCRIPTOR commands.

The EIA_775_DTV_info reports capabilities and parameters including:

	OSD formats supported (grid sizes, pixel formats and colour depths).

	Whether double buffering is supported for various video formats.

	OSD overlay characteristics for the ATSC video modes.

	Flags indicating support for various other features.

The syntax of the response to the READ DESCRIPTOR command is given in the following subclauses.

Figure 26 diagrams the relationship between the general subunit identifier descriptor (refer to Section 8.1 in the AV/C Digital Interface Command Set General Specification), the subunit-dependent portion, and the information blocks containing EIA-defined data.

Note that the reference to "subunit" in the General Subunit Identifier Descriptor means "unit" when the descriptor command is unit-addressed.

Figure 26 shows the example case for the DTV, in which both types of EIA info blocks are returned. For A/V source devices, only the EIA-775 plug info block would appear.

An information block is a general-purpose data structure comprised of a 16-bit type code, a 16-bit data length field, and an amount of data indicated by the length field. This Recommendation defines two information block types whose format is formally identified as being specified by EIA. Information blocks with a type code indicating "vendor specific" include a 3-byte Specifier_ID which indicates the company or entity that has defined the vendor specific data contained within. The Specifier_ID for the EIA-defined info blocks shall use the IEEE-assigned identifier code for EIA/CEMA, 00506816.

As of this writing, the 1394 Trade Association has not defined fields within any Unit Identifier Descriptors. DTV devices compliant with this Recommendation shall respond to the OPEN DESCRIPTOR and READ DESCRIPTOR commands with a descriptor that may exclude data fields defined by the TA. Stated another way, the well_defined_fields_length shown in Figure 26 may be zero. Devices processing the Unit Identifier Descriptor in accordance with the present Recommendation shall rely on the EIA vendor-specific information blocks to discover plug numbers and capabilities of the monitor.

Likewise, audio/video source devices compliant with this Recommendation shall respond to the OPEN DESCRIPTOR and READ DESCRIPTOR commands with a descriptor that may exclude data fields that may be defined by the TA. DTVs processing a source device’s Unit Identifier Descriptor in accordance with the present Recommendation shall rely on the EIA vendor-specific information blocks to discover plug numbers of the source device.

Note also that the well_defined_fields_length may also be non-zero – the DTV may return information compliant with the AV/C standard for as defined by the 1394 Trade Association. The TA-defined fields may be processed at the option of the external device.

The following subclauses define all the fields in the data structures given in Figure 26 above.

#### 6.9.3.1	General subunit identifier descriptor

The general format of the subunit identifier descriptor is given in Section II.10.1 of the AV/C General Specification. All subunit identifier descriptors fit this general format. For purposes of the present Recommendation, a DTV or source device shall return the following values for the fields of the Unit Identifier Descriptor:

descriptor_length shall indicate the number of bytes to follow in the structure.

generation_ID shall be 0.

size_of_list_ID, size_of_object_ID, size_of_object_position, number_of_root_object_lists, shall each be 0, or in the case the DTV or source device implements a protocol defined by the 1394 Trade Association for the Unit, the value appropriate to that Specification.

Root object lists may optionally be present, if the DTV or source device conforms to a 1394 TA protocol that defines them. If present, the number_of_root_object_lists shall be set as appropriate.

subunit_dependent_length shall be set to the number of bytes to follow in the subunit_dependent_information structure.

The subunit_dependent_information shall be included, formatted as specified in the following subclause.

#### 6.9.3.2	Subunit dependent information

Figure 27 defines the structure of subunit_dependent_information. Again, note that even though the name is "subunit", in this case it represents unit information because it is returned from a unit-directed AV/C command.

Figure 27 is an example that shows two EIA-defined info blocks following the fields defined by the 1394 TA. The order of info blocks is arbitrary. Any device processing subunit_dependent_information shall be able to handle info blocks appearing in an arbitrary order.

well_defined_fields_length may be 0, or may reflect the length of data fields defined by a 1394 Trade Association protocol for the Unit.

NOTE – any device processing the Unit Identifier Descriptor must process well_defined_fields_length and be able to handle a non-zero value in order to skip the fields between it and the start of the info blocks.

The general structure of EIA-defined info blocks is defined in the following subclauses.

#### 6.9.3.3	EIA Monitor Subunit Info Block

Figure 28 defines the structure of the EIA-defined info blocks. This data structure conforms to the general format of an info block. Since the info_block_type field identifies it as vendor-specific info block, the field following the fields_length identifies the specific vendor (in this case, EIA).

total_length shall be set to reflect the number of bytes to follow to the end of the data structure.

info_block_type is a 16-bit field that indicates the type of this info block. info_block_type shall be set to 0016 to indicate vendor-specific.

fields_length shall also be set to reflect the number of bytes to follow in the data structure. For this info block, fields_length shall equal total_length minus 4.

Specifier_ID is a 24-bit field indicating the specific vendor that has defined this info block. Specifier_ID shall be set to 00506816 (EIA’s IEEE-assigned specifier code).

EIA_info_block_typeCode is a 16-bit field that indicates the format of data to follow in this EIA-defined data structure. Two EIA-defined info block types are defined here. Table 18 defines the coding of the EIA_info_block_typeCode.

Table 18/J.117 – EIA-defined info block typeCode definition

| EIA_info_block_ typeCode | Meaning |
| --- | --- |
| 000016 | Reserved |
| 000116 | EIA_775_plug_info |
| 000216 | EIA_775_DTV_info |
| 000316-FFFF16 | Reserved for future use by EIA |

The EIA_775_plug_info and EIA_775_DTV_info info blocks are defined in the following subclauses.

#### 6.9.3.4	EIA-775 plug info

EIA_775_plug_info is defined by this EIA specification to provide information relating to plug number information and capability of EIA-755 compatible DTVs and A/V source devices.

The DTV and any source device that is capable of supporting digital or analogue audio/video or OSD compliant with this Recommendation shall return the EIA_775_plug_info data structure in response to a READ DESCRIPTOR AV/C command directed at that unit. Figure 29 defines the syntax.

| Figure 29/J.117 – EIA-775 Plug Info EIA-775 Plug Info |  |
| --- | --- |
| address offset | contents |
| 00 0016 | EIA_775_support_level |
| 00 0116 | OSD_input_plug_ID |
| 00 0216 | OSD_output_plug_ID |
| 00 0316 | analogue_video_input_plug_ID |
| 00 0416 | analogue_video_output_plug_ID |
| 00 0516 | digital_transport_stream_input_plug_ID |
| 00 0616 | digital_transport_stream_output_plug_ID |
| 00 0716 |  |
| 00 0816 | transport_stream_input_format |
| 00 0916 |  |
| 00 0A16 |  |
| 00 0B16 |  |
| 00 0C16 | transport_stream_output_format |
| 00 0D16 |  |
| 00 0E16 |  |
| 00 0F16 |  |
| ... | reserved for future definition |
| ... |  |
| 00 1E16 |  |

EIA_775_support_level is an 8-bit unsigned integer that indicates compliance to this version of EIA-775. DTV and source devices compliant with this Recommendation shall return 0016 in the EIA_775_support_level byte. Other values are reserved for future EIA definition.

OSD_input_plug_ID is an 8-bit unsigned integer field that identifies the Asynchronous Connections input plug number upon which an OSD source device may provide EIA-775 compliant OSD data. If a device is not capable of accepting EIA-775 compliant OSD data, OSD_source_plug_ID shall be set to FE16. Otherwise the value of this field shall be in the range A016 through BE16.

OSD_input_plug_ID is an 8-bit unsigned integer field that identifies the Asynchronous Connections output plug number upon which an OSD source device shall provide EIA-775 compliant OSD data. If a device is not capable of generating EIA-775 compliant OSD data, OSD_source_plug_ID shall be set to FE16. Otherwise the value of this field shall be in the range A016 through BE16.

analogue_video_input_plug_ID is an 8-bit unsigned integer field that identifies the plug number upon which the device shall accept analogue video for pass-through. If a device does not support analogue video pass-through, the value of this field shall be set to FE16. Otherwise the value of this field shall be in the range 8016 through 9E16.

analogue_video_output_plug_ID is an 8-bit unsigned integer field that identifies the plug number upon which the device shall output analogue video. If a device does not support analogue output, the value of this field shall be set to FE16. Otherwise the value of this field shall be in the range 8016 through 9E16.

digital_transport_stream_input_plug_ID is an 8-bit unsigned integer field that identifies the plug number upon which a device shall accept digital Transport Stream data. If a device does not support digital transport stream input, the value of this field shall be set to FE16. Otherwise the value of this field shall be in the range 0016 through 1E16.

digital_transport_stream_output_plug_ID is an 8-bit unsigned integer field that identifies the plug number upon which a device shall output digital Transport Stream data. If a device does not support digital transport stream output, the value of this field shall be set to FE16. Otherwise the value of this field shall be in the range 0016 through 1E16.

transport_stream_input_format is a 32-bit field that indicates, in each bit position, support for a particular format for digital data stream on the indicated digital_transport_stream_input_plug_ID. It is coded according to Table 19. A value of 1 in the specified bit position indicates that the indicated video format is supported. A value of 0 indicates that the format is not supported.

Table 19/J.117 – Transport Stream Format

| transport_stream_input_format or transport_stream_output_format bit number | Meaning |
| --- | --- |
| 0 (lsb) | MPEG-2 per ISO/IEC 13818-1 |
| 1 | Digital Video per IEC 61883-2, 61883-3 or 61883-5 |
| 2 | DirecTv transport |
| 3-31 (msb) | Reserved for future definition |

transport_stream_output_format is a 32-bit field that indicates, in each bit position, support for a particular format for digital data stream on the indicated digital_transport_stream_output_plug_ID. It is coded according to Table 19.

digital_transport_stream_output_plug_ID is also coded according to Table 19. A value of 1 in the specified bit position indicates that the indicated video format is supported. A value of 0 indicates that the format is not supported.

#### 6.9.3.5	EIA-775 DTV Info

EIA_775_DTV_info is defined by this Recommendation to provide information relating to monitor capabilities and OSD modes supported in the DTV. Figure 30 defines the syntax.

| Figure 30/J.117 – EIA-775 DTV Info EIA-775 DTV Info |  |
| --- | --- |
| address offset | contents |
| 00 0016 | EIA_DTV_profile_level |
| 00 0116 |  |
| 00 0216 | OSD_formats_supported |
| 00 0316 |  |
| 00 0416 |  |
| 00 0516 |  |
| 00 0616 | double_buffering_supported |
| 00 0716 |  |
| 00 0816 |  |
| 00 0916 |  |
| 00 0A16 | miscellaneous_features |
| 00 0B16 |  |
| 00 0C16 | default_video_format |
| 00 0D16 | analogue_video_conversion_format |
| 00 0E16 |  |
| … | align_1920_1080i |
| 00 1316 |  |
| 00 1416 |  |
| … | align_1920_1080p |
| 00 1916 |  |
| 00 1A16 |  |
| … | align_1280_720p |
| 00 1F16 |  |
| 00 2016 |  |
| … | align_704_480i_4x3 |
| 00 2516 |  |
| 00 2616 |  |
| … | align_704_480p_4x3 |
| 00 2B16 |  |
| 00 2C16 |  |
| … | align_704_480i_16x9 |
| 00 3116 |  |
| 00 3216 |  |
| … | align_704_480p_16x9 |
| 00 3716 |  |
| 00 3816 |  |
| … | align_640_480i |
| 00 3D16 |  |
| 00 3E16 |  |
| … | align_640_480p |
| 00 4316 |  |
| 00 4416 |  |
| … | Reserved for future definition |
| … |  |
| 00 5316 |  |

EIA_DTV_profile_level indicates a minimum level of functionality supported by the DTV according to the following table.

2 	See HDND 1394 Interface Specification, Cable Television Laboratories, for details.

3	An explicit time reference in PSIP is, for example, the start time field for events in the EIT. An implicit time reference is the definition of the time segment for which an EIT is applicable.