Table 20/J.117 – EIA_DTV_profile_level format coding

| EIA_DTV_profile_level | Meaning |
| --- | --- |
| 0 | EIA Profile "A" |
| 1 | EIA Profile "B" |
| 2-255 | Reserved for future use |

Recommendation J.117     (09/99)

EIA Profiles A and B are defined in Tables 21 and 22. Checkmarks indicate that the capability is mandatory for the given profile. "Opt." Indicates that the capability is optional for that profile.

Table 21/J.117 – Capability profiles – general features

| Capability | EIA Profile A | EIA Profile B |
| --- | --- | --- |
| 1394, s200 bus clock (or higher) | ü | ü |
| 4-pin connector | ü | ü |
| IEC 61883-1 digital interface standard | ü | ü |
| IEC 61883-4 for transport of MPEG TS | ü | ü |
| AV/C general command set | ü | ü |
| Decodes all ATSC video formats | ü | ü |
| OSD delivery format supports region-based updating | ü | ü |
| 640  480 OSD grid stretchable to 14:9 | Opt. | Opt. |
| 640  480 OSD grid stretchable to 16:9 | Opt. | Opt. |
| OSD fill surround supported | Opt. | Opt. |
| Source driven digital/analogue selection supported | Opt. | Opt. |
| OSD over analogue video supported | Opt. | Opt. |
| Double buffering supported | Opt. | Opt. |

Table 22/J.117 – Capability profiles – OSD related features

| Capability | EIA Profile A | EIA Profile B |
| --- | --- | --- |
| OSD_layout 0 (640  480  4) pixel format 1 (a:Y:Cb:Cr  2:6:4:4) | ü | ü |
| OSD_layout 0 (640  480  4) pixel format 2 (a:Y:Cb:Cr  4:6:3:3) | Opt. | ü |
| OSD_layout 1 (640  480  8) pixel format 1 (a:Y:Cb:Cr  2:6:4:4) | Opt. | ü |
| OSD_layout 1 (640  480  8) pixel format 2 (a:Y:Cb:Cr  4:6:3:3) | Opt. | ü |
| OSD_layout 1 (640  480  8) pixel format 0 (Y:Cb:Cr  6:5:5) | Opt. | ü |
| OSD_layout 2 (640  480  16) pixel format 1 (a:Y:Cb:Cr  2:6:4:4) | Opt. | ü |
| OSD_layout 2 (640  480  16) pixel format 2 (a:Y:Cb:Cr  4:6:3:3) | Opt. | ü |
| OSD_layout 2 (640  480  16) pixel format 0 (Y:Cb:Cr  6:5:5) | Opt. | ü |

OSD_formats_supported is a 32-bit field that indicates, in each bit position, support for a particular OSD format. It is coded according to Table 23. A value of 1 in the specified bit position indicates that the indicated OSD mode is supported. A value of 0 indicates that the mode is not supported. OSD_layout and pixel_format are as defined in 6.7.1.

Table 23/J.117 – OSD formats supported field coding

| OSD_formats_supported bit number | Meaning |
| --- | --- |
| 0 (lsb) | OSD_layout 0 (640  480  4) pixel format 1 (a:Y:Cb:Cr  2:6:4:4) |
| 1 | OSD_layout 0 (640  480  4) pixel format 2 (a:Y:Cb:Cr  4:6:3:3) |
| 2 | OSD_layout 1 (640  480  8) pixel format 1 (a:Y:Cb:Cr  2:6:4:4) |
| 3 | OSD_layout 1 (640  480  8) pixel format 2 (a:Y:Cb:Cr  4:6:3:3) |
| 4 | OSD_layout 1 (640  480  8) pixel format 0 (Y:Cb:Cr  6:5:5) |
| 5 | OSD_layout 2 (640  480  16) pixel format 1 (a:Y:Cb:Cr  2:6:4:4) |
| 6 | OSD_layout 2 (640  480  16) pixel format 2 (a:Y:Cb:Cr  4:6:3:3) |
| 7 | OSD_layout 2 (640  480  16) pixel format 0 (Y:Cb:Cr  6:5:5) |
| 8-31 (msb) | Reserved for future definition by EIA |

double_buffering_supported is a 32-bit field that indicates, for each of the OSD formats defined in OSD_formats_supported, whether the DTV supports double buffering in that format. A value of 1 in the corresponding bit position indicates that double buffering is supported for the associated OSD format. A value of 0 indicates that it is not.

miscellaneous_features is a 24-bit field that indicates whether or not various miscellaneous features are supported in the monitor. A value of 1 in the specified bit position indicates that the indicated feature is supported. A value of 0 indicates that the feature is not supported. miscellaneous_features is coded according to Table 24.

Table 24/J.117 – Miscellaneous features field coding

| miscellaneous_features bit number | Meaning | Ref. |
| --- | --- | --- |
| 0 (lsb) | Reserved |  |
| 1 | Stretching of 640  480 OSD grid to 14:9 | See 6.3.2.2.1 |
| 2 | Stretching of 640  480 OSD grid to 16:9 | See 6.3.2.2.1 |
| 3 | OSD fill surround | See 6.7.1.4.5 |
| 4 | Source driven digital / analogue selection | See 6.4.11 |
| 5 | OSD over analogue video | See 6.3.2.1 |
| 6-23 (msb) | Reserved for future definition by EIA |  |

default_video_format is an 8-bit field that indicates the digital video mode that will be used in the "default" case, which is the no-video case that occurs when an isochronous channel is connected but is off-line (the input plug is in the suspended state). It is coded according to Table 25. Value 0 may be used only if none of the defined values correctly describe the default.

Table 25/J.117 – Default video format coding

| default_video_format | Meaning |
| --- | --- |
| 0 | Unknown |
| 1 | 1920  1080 interlaced |
| 2 | 1920  1080 progressive |
| 3 | 1280  720 progressive |
| 4 | 704  480 (4  3) interlaced |
| 5 | 704  480 (4  3) progressive |
| 6 | 704  480 (16  9) interlaced |
| 7 | 704  480 (16  9) progressive |
| 8 | 640  480 interlaced |
| 9 | 640  480 progressive |
| 10-255 | Reserved |

The default_video_format is helpful in that it describes OSD display characteristics in the absence of a defined video format upon which to overlay the OSD. The OSD overlay characteristics for the no-video case are the same as when input video in the indicated format is present and OSD is overlaid on that.

analogue_video_conversion_format indicates the video format resulting from DTV processing of analogue standard-definition video input. This field is coded the same as default_video_format (see Table 25). If the DTV does not support OSD over analogue video, value 0 ("Unknown") shall be specified. The overlay characteristics for OSD overlaid on analogue video are the same as when input video in the indicated format is present and OSD is overlaid on that.

The following variables describe display scan conversion and OSD overlay characteristics for various ATSC input digital video formats.

align_1920_1080i defines display and overlay characteristics applicable when OSD is overlaid onto 1920 by 1080 interlaced MPEG video format.

align_1920_1080p defines display and overlay characteristics applicable when OSD is overlaid onto 1920 by 1080 progressive MPEG video format.

align_1280_720p defines display and overlay characteristics applicable when OSD is overlaid onto 1280 by 720 MPEG video format.

align_704_480i_4x3 defines display and overlay characteristics applicable when OSD is overlaid onto 704 by 480 interlaced MPEG video format when the display aspect ratio is 4:3.

align_704_480p_4x3 defines display and overlay characteristics applicable when OSD is overlaid onto 704 by 480 progressive MPEG video format when the display aspect ratio is 4:3.

align_704_480i_16x9 defines display and overlay characteristics applicable when OSD is overlaid onto 704 by 480 interlaced MPEG video format when the display aspect ratio is 16:9.

align_704_480p_16x9 defines display and overlay characteristics applicable when OSD is overlaid onto 704 by 480 progressive MPEG video format when the display aspect ratio is 16:9.

align_640_480i defines display and overlay characteristics applicable when OSD is overlaid onto 640 by 480 interlaced MPEG video format.

align_640_480p defines display and overlay characteristics applicable when OSD is overlaid onto 640 by 480 progressive MPEG video format.

These nine fields are formatted according to Figure 31.

Figure 31/J.117 [D19]

s14 indicates whether the DTV will or will not support stretching the 4:3 grid to 14:9 when overlaying OSD on this video format. Value 0 indicates that 14:9 stretching is not supported. Value 1 indicates that it is.

s16 indicates whether the DTV will or will not support stretching the 4:3 grid to 16:9 when overlaying OSD on this video format. Value 0 indicates that 16:9 stretching is not supported. Value 1 indicates that it is.

ilc indicates whether the DTV will use an progressive (value 0) or interlace (value 1) scan format to display the associated video format.

video_scan_lines is a 12-bit unsigned integer number that indicates the number of output video scan lines that will be used to display the associated video format.

OSD_scan_lines is a 12-bit unsigned integer number that indicates number of output video scan lines that will be used to represent the OSD grid in the vertical dimension when OSD is overlaid on video in the associated format.

pixel_aspect_ratio defines the width-to-height ratio of displayed OSD pixels when overlaid on video in the associated format. It is a dimensionless number coded as an 8-bit unsigned integer in units of 1/128. A value of 128 (8016) indicates square pixels (aspect ratio  1.0). pixel_aspect_ratio may also be considered to be an 8-bit fixed point binary number with a 7-bit fractional part. To compute a pixel_aspect_ratio for a given width-to-height ratio, 1/256 shall be added to the width-to-height ratio and the binary number representing that sum shall be truncated to 7 bits in the fractional part.

Table 26 shows an example of how the DTV might report OSD overlay processing.

Table 26/J.117 – Example scan conversion and OSD alignment

| Video format | video scan lines | OSD scan lines | interlace | width to height ratio | pixel_aspect _ratio |
| --- | --- | --- | --- | --- | --- |
| 1920  1080 interlaced | 1080 | 960 | Yes | 1:1 | 8016 |
| 1920  1080 progressive | 1080 | 960 | Yes | 1:1 | 8016 |
| 1280  720 progressive | 1080 | 960 | Yes | 1:1 | 8016 |
| 704  480 (4  3) interlaced | 960 | 960 | Yes | 8:9 | 7216 |
| 704  480 (4  3) progressive | 960 | 960 | Yes | 8:9 | 7216 |
| 704  480 (16  9) interlaced | 960 | 960 | Yes | 32:27 | 9816 |
| 704  480 (16  9) progressive | 960 | 960 | Yes | 32:27 | 9816 |
| 640  480 interlaced | 960 | 960 | Yes | 1:1 | 8016 |
| 640  480 progressive | 960 | 960 | Yes | 1:1 | 8016 |

#### 6.9.3.6	Minimum descriptor definition for DTV

Based on the data structures given in the preceding subclauses, taking the option not to include data defined in the 1394 Trade Association, one may construct the Unit Identifier Descriptor as shown in Figure 32. The example descriptor is for the DTV, so it contains both the EIA_775_plug_info and EIA_775_DTV_info info blocks.

This view starts with the General Subunit Identifier Descriptor structure given in Section 8.1 of AV/C Digital Interface Command Set General Specification [4].

| Figure 32/J.117 – Minimum Descriptor Unit Identifier Descriptor |  |  |
| --- | --- | --- |
| address offset | contents | value |
| 00 0016 | descriptor_length | 0016 |
| 00 0116 |  | 9316 |
| 00 0216 | generation_ID | 0016 |
| 00 0316 | size_of_list_ID | 0016 |
| 00 0416 | size_of_object_ID | 0016 |
| 00 0516 | size_of_object_position | 0016 |
| 00 0616 | number_of_root_object_lists | 0016 |
| 00 0716 |  | 0016 |
| 00 0816 | subunit_dependent_length | 0016 |
| 00 0916 |  | 8B16 |
| 00 0A16 | total_length | 0016 |
| 00 0B16 |  | 1A16 |
| 00 0C16 | info_block_type = vendor_specific | 0016 |
| 00 0D16 |  | 0016 |
| 00 0E16 | fields_length | 0016 |
| 00 0F16 |  | 1616 |
| 00 1016 |  | 0016 |
| 00 1116 | Specifier_ID = EIA | 5016 |
| 00 1216 |  | 6816 |
| 00 1316 | EIA_info_block_typeCode = 0116 | 0016 |
| 00 1416 | (EIA_775_DTV_info) | 0116 |
| 00 1516 | EIA_775_support_level | var |
| 00 1616 | OSD_input_plug_ID | var |
| 00 1716 | OSD_output_plug_ID | var |
| 00 1816 | analogue_video_input_plug_ID | var |
| 00 1916 | analogue_video_output_plug_ID | var |
| 00 1A16 | digital_transport_stream_input_plug_ID | var |
| 00 1B16 | digital_transport_stream_output_plug_ID | var |
| 00 1C16 |  | var |
| 00 1D16 | transport_stream_input_format | var |
| 00 1E16 |  | var |
| 00 1F16 |  | var |
| 00 2016 |  | var |
| 00 2116 | transport_stream_output_format | var |
| 00 2216 |  | var |
| 00 2316 |  | var |
| 00 2416 |  | 0016 |
| … | [Reserved] | … |
| 00 3316 |  | 0016 |
| 00 3416 | total_length | 0016 |
| 00 3516 |  | 5F16 |
| 00 3616 | info_block_type = vendor_specific | 0016 |
| 00 3716 |  | 0016 |
| 00 3816 | fields_length | 0016 |
| 00 3916 |  | 5B16 |
| 00 3A16 |  | 0016 |
| 00 3B16 | Specifier_ID = EIA | 5016 |
| 00 3C16 |  | 6816 |
| 00 3D16 | EIA_info_block_typeCode = 0216 | 0016 |
| 00 3E16 | (EIA_775_DTV_info) | 0216 |

| Figure 33/J.117 – Minimum Descriptor Example Unit Identifier Descriptor (cont.) |  |  |
| --- | --- | --- |
| address offset | contents | value |
| 00 3F16 | EIA_DTV_profile_level | var |
| 00 4016 |  |  |
| 00 4116 | OSD_formats_supported | var |
| 00 4216 |  |  |
| 00 4316 |  |  |
| 00 4416 |  |  |
| 00 4516 | double_buffering_supported | var |
| 00 4616 |  |  |
| 00 4716 |  |  |
| 00 4816 |  |  |
| 00 4916 | miscellaneous_features | var |
| 00 4A16 |  |  |
| 00 4B16 | default_video_format | var |
| 00 4C16 | analogue_video_conversion_format | var |
| 00 4D16 |  |  |
| … | align_1920_1080i | var |
| 00 5216 |  |  |
| 00 5316 |  |  |
| … | align_1920_1080p | var |
| 00 5816 |  |  |
| 00 5916 |  |  |
| … | align_1280_720p | var |
| 00 5E16 |  |  |
| 00 5F16 |  |  |
| … | align_704_480i_4x3 | var |
| 00 6416 |  |  |
| 00 6516 |  |  |
| … | align_704_480p_4x3 | var |
| 00 6A16 |  |  |
| 00 6B16 |  |  |
| … | align_704_480i_16x9 | var |
| 00 7016 |  |  |
| 00 7116 |  |  |
| … | align_704_480p_16x9 | var |
| 00 7616 |  |  |
| 00 7716 |  |  |
| … | align_640_480i | var |
| 00 7C16 |  |  |
| 00 7D16 |  |  |
| … | align_640_480p | var |
| 00 8216 |  |  |
| 00 8316 |  | 0016 |
| … | [Reserved] | … |
| 00 9216 |  | 0016 |
| 00 9316 | Manufacturer_dependent_length | 0016 |
| 00 9416 |  | 0016 |
| NOTE – manufacturer_dependent_length is a field that is required by the general format of the subunit identifier descriptor, even if it is specified to be zero. |  |  |

Annex  A

OSD Transport and Connection Management (Normative)

## A.1	Introduction

Asynchronous connections were designed to be simple and provide an asynchronous equivalent of isochronous data-transfer services. Specific design objectives included the following:

1)	Lightweight: simple asynchronous unidirectional data-transfer protocols should allow large data frames to be efficiently transferred between producer and consumer nodes.

2)	Robust: the protocols should allow recovery from any number of bus resets and data transmission errors.

3)	Leveraged: existing AV/C resources should be used for connection management purposes.

### A.1.1	Reserved fields

Some fields defined in this subclause are designated as 'reserved' or 'r'. These fields have been set aside for further expansion of this Recommendation and shall be set to zero.

## A.2	Data Transfers

Asynchronous connections are optimized for copying data from a producer node to one or more consumer nodes. With the "push" data transfer model, the producer writes into the consumer’s data buffer.

The consumer starts the cycle by indicating how much data the producer can safely write into the buffer by writing a limit-count value into one of the producers control registers. Data can then be written into the consumers segment buffer until the end of the segment buffer has been reached. The data consumption is triggered by the producer signalling (through a control register update) when the segment write has completed. After processing this data, the consumer re-enables writing to the segment buffer by indicating how much data the producer can safely write.

A count register in the producer and a count register in the consumer provide bi-directional flow control, allowing the data-transfer rate to be paced by the slower of the producer and consumer nodes. The count registers are updated using CompareSwap4 (a 4-byte lock transaction, called compare-and-swap) transaction, which provides a consistency check.

A standard method for grouping data into (potentially large) frames is specified; the count-register update at the end of each frame provides distinctive end-of-frame indications. As an option, data within each frame can be further decomposed into a sequence of subframes, where the length of each subframe is specified by its header.

## A.3	Asynchronous plugs

On asynchronous connections, data transfers involve writes into the address space of another node. This address space and affiliated internal state are called an asynchronous plug. The plug address is minimally 64-byte aligned, i.e. the plug address shall be a multiple of 64.

The controller is responsible for establishing an asynchronous connection, by connecting producer and consumer components. The plug1 and plug2 addresses, located on producer and consumer nodes, need not be related. After the connection process completes, each plug has parameters that specify its counterpart plug location and capabilities, as illustrated in Figure A.1.

Establishing an asynchronous connection affiliates one node (the producer node) with another node (the consumer node). In its simplest form, the producer node produces frames and the consumer node consumes frames.

Figure A.1/J.117 {D20]

## A.4	Asynchronous plug components

### A.4.1	Asynchronous plug spaces

The 64-bit address space is partitioned into 216 (64k) nodes and an asynchronous plug occupies a portion of its node’s address space, as illustrated by Figure A.2.

Figure A.2/J.117 [D21]

An asynchronous plug contains register information (shaded black) and segment-buffer addresses (shaded gray), as illustrated by Figure A.3. The plug’s registers, that correspond to distinct port resources, are located at the lowest plug addresses (starting at plugAddress) and the segment buffer is located at a fixed 64-byte offset from the start of the plug.

The segment buffer is located in the initial portion of its address space. The size of this address space is called segment limit. The segment limit should be a power of two and no less than 64 bytes. The segment limit is expressed as 2n in Figure A.3, while n is an integer greater or equal to 6. The starting address of the address space should be multiple of the segment limit and expressed as m  2n in Figure A.3, while m is an integer greater than 0. The size of the segment buffer shall be multiple of the maxload specified size and no less than 64 bytes and no more than the segment limit.

Figure A.3/J.117 [D22]

For simplicity, the plug-offset addresses of ports never change, although their presence or absence depends on the type of connection that is opened. In this Recommendation, a producer has one oAPR register, and a consumer has one iAPR register, as illustrated in Figure A.4.

Figure A.4/J.117 [D23]

The plug’s resources are specified by their offset addresses from the plug’s address, as listed in Table A.1.

Table A.1/J.117 – Plug resource addresses

| Address offset | Description |
| --- | --- |
| 0 | iAPR |
| 4 | oAPR |
| 8, 12,…60 | Reserved |
| 64 | Segment buffer |

### A.4.2	Asynchronous plug components

A plug resource is directly mapped to a contiguous range of Serial Bus addresses. The plug’s address is the lowest numerical address within this range of visible Serial Bus addresses.

The 32-bit iAPR register (that is written by the producer) is provided for flow-control purposes.

The 32-bit oAPR register (that is written by the consumer) is provided for flow-control purposes.

### A.4.3	Consumer-port address

A consumer-port address is a 64-byte aligned 64-bit Serial Bus address. It provides access to the port’s externally visible flow control register (iAPR).

### A.4.4	Producer-port address

A producer-port address, which provides access to the port’s externally visible flow-control register (oAPR), is minimally 4-byte aligned.

### A.4.5	Segment-buffer address

An asynchronous segment-buffer address is minimally 64-byte aligned.

For simplicity, the size of the segment buffer shall be less than or equal to the alignment size of its base address. The size of the segment buffer shall be a multiple of 64 and its address shall be aligned to its size, although the consumer’s oAPR.countHi value may restrict the producer to use less than the full segment-buffer size. The intent is to allow the producer to simply generate Serial Bus addresses for data writes, by OR’ing the offset address with the segment-buffer address.

The segment buffer shall be a multiple of the maxLoad specified size and is minimally 64 bytes in size. The address space allocated to the segment buffer may be larger than the actual segment-buffer size. For example, a 192-kbyte physical buffer could be mapped into the initial portion of 256-kbyte-aligned segment buffer.

## A.5	Flow control registers

### A.5.1	Register properties

#### A.5.1.1	Command-reset values

All fields shall be unaffected by a command reset.

#### A.5.1.2	Unimplemented registers

On a consumer plug, the oAPR register is not implemented. On a producer plug, the iAPR register is not implemented. When these unimplemented registers are accessed, their behaviour are listed below.

1)	CompareAndSwap. A 4-byte CompareAndSwap lock transaction shall complete with a type_error status.

2)	Other transactions. Other transactions are not supported and should return a type_error. For compatibility with existing hardware interfaces, the writes may update the addressed location and reads may return previously written data.

### A.5.2	Implemented registers

When implemented registers are accessed, their behaviours are listed below. This definition is intended to be compatible with existing hardware interfaces:

1)	CompareAndSwap. A 4-byte CompareAndSwap lock transaction shall complete with an ack_pending and resp_complete status when no errors occur. If an error occurs, the rcode will reflect it.

2)	Other transactions. Other transactions are not supported and should return a type_error. For compatibility with existing hardware interfaces, the writes may update the addressed data and reads may return previously written data.

The requester is responsible for checking whether the CompareAndSwap operation has been effected, by comparing the returned response-data value with the previously sent request-packet argument value. Distinct completion codes or data-bit values are not provided for this purpose.

### A.5.3	iAPR register

#### A.5.3.1	iAPR format

The iAPR register is written by the producer and is located on the consumer. This register contains multiple control bits and a flow-controlling count value, as illustrated Figure A.5.

The reserved bits shall be zero.

The 3-bit mode field provides frame-completion and disconnection-state information, as specified in Table A.2.

The sc (segment count) bit is normally set to the oAPR.sc value after each segment has been transferred, to indicate when that segment has been sent. In some implementations it may be desirable to process incoming information before the producer indicates that the transfer is complete. One reason for doing this would be to process subframes as they arrive. One way to accomplish this would be for the consumer to continuously monitor incoming data to the segment buffer to parse the subframes. If processor cycles are not available to do this then special hardware should be considered which could inform the consumer when complete subframes arrive.

The 24-bit count identifies how many of this segment’s data bytes have been written by the producer, and is used for flow-control purposes.

#### A.5.3.2	iAPR.mode values

The 3-bit mode field provides frame-completion and disconnection-state information, as specified in Table A.2.

Table A.2/J.117 – iAPR.mode values

| mode | Name | Description |
| --- | --- | --- |
| 0 | FREE | Initial State/Disconnect |
| 1 | MORE | The frame has not yet ended |
| 2 | SUSPENDED | Suspend confirmation; suspended frame transfers |
| 3 | – | Reserved |
| 4 | LAST | A successful frame transfer, next frame not grouped |
| 5 | LESS | A truncated length frame was transferred |
| 6 | – | Reserved |
| 7 | – | Reserved |

The FREE value is a distinct initial value. A FREE indication is a request from the producer to initiate a disconnect.

The MORE value labels the leading (not end-of-frame) segments within a frame.

The SUSPENDED value indicates that segments are being discarded (rather than sent) and is asserted after observing a oAPR.modeSUSPEND indication.

The LAST value labels the final segment within a successfully transferred frame, when this is the last frame within a frame group.

The LESS value labels the last segment in an abnormal frame, when the abnormal frame contains valid data but the frame was truncated early.

### A.5.4	oAPR register

#### A.5.4.1	oAPR format

The producer-resident oAPR register is written by the consumer. Multiple control bits and a flow-controlling count value are provided, as illustrated in Figure A.6 The initial value of this register (after a power reset or when the port enters the free state) shall be zero.

The r and reserved bits shall be zero.

The 3-bit mode field provides segment-buffer status information.

The sc (segment count) bit is toggled after each segment has been transferred, to distinctively label the sequential segments.

The 18-bit countHi value shall be concatenated with a 6-bit zero value to generate the count value. The count value indicates how many bytes the producer can safely write, and is used to limit the range of the producer’s data-write transactions. This value shall be a multiple of the maxLoad-specified write-transaction payload size (a minimum payload size is 64 bytes).

The zero-valued run bit inhibits the producer-port from generating segment-buffer writes and control register updates. A one-valued run bit enables the generation of these writes and control register updates. The initial value of the run bit shall be zero and the run bit shall be cleared by a bus reset.

The run bit is intended to delay the producer’s generation of write and update transactions until the consumer’s state has been properly initialized. At this time, the consumer is expected to update the producer’s oAPR register, setting oAPR.run to one.

The 4-bit maxLoad field specifies the data-payload size limitations for segment-buffer writes, as specified in Equation A-5.1. In other words, the amount of data in the write request cannot exceed the payloadSizeInBytes value. The maxLoad value shall be equal-to or larger-than 5 (which corresponds to 64-byte writes) and shall not exceed the size of the node’s ROM-specified max_rec value, as defined in IEEE 1394-1995 [2].

payloadSizeInBytes  2(maxLoad  1) 	(A-5.1)

The connection’s maxLoad field may differ from the node’s max_rec field, because the size of receive-FIFO queues may depend on where the incoming request is routed.

An rcode of resp_conflict_error (as defined in IEEE 1394-1995 [2] as meaning the responder does not have enough resources to handle the requested transaction) is generated when a one valued oAPR.run bit would allow a transaction to be generated and maxLoad has an illegal value.

#### A.5.4.2	oAPR.mode values

The 3-bit mode field provides segment-buffer status information, as specified in Table A.3.

Table A.3/J.117 – oAPR.mode values

| mode | Name | Description |
| --- | --- | --- |
| 0 | FREE | Initial State/Disconnect |
| 1 | – | Reserved |
| 2 | SUSPEND | Suspend frame transfers |
| 3 | – | Reserved |
| 4 | RESUME | Resume frame transfers |
| 5 | SEND | Segment buffer is available |
| 6 | – | Reserved |
| 7 | – | Reserved |

The FREE value is a distinct initial value. A FREE indication is a request from the producer to initiate a disconnect.

The SUSPEND value indicates that segments should be discarded (rather than sent) and is asserted when consumer-local connections to the plug have been detached.

The RESUME value indicates that segments may be sent and is asserted when consumer-local connections to the plug have been re-attached.

The SEND value indicates that segment-buffer space is available, so the next segment can be safely sent.

## A.6	Data-access constraints

### A.6.1	Segment buffer accesses

The segment buffer shall only respond to read and write, either quadlet or block, requests as follows:

1)	Block write checks. The block write transactions may be checked in several ways, as listed below:

a)	Robust. A write with an expected source_ID value (as specified the ATTACH management command) shall be accepted; others shall be terminated with a type_error.

b)	Minimal. All writes shall be accepted, without source_ID checking.

2)	Block write sizes. The size of the normal block write transaction shall be one fixed size, which is the largest power of two, subject to the following constraints:

a)	Speed. The size of the write request shall not exceed the size supported by the effective (S200 is the minimum, s400 is possible) data transfer rate.

b)	Consumer limitations. The size of the write request shall not exceed the capabilities of the consumer, as indicated by its oAPR.maxLoad field.

c)	Producer limitations. This is one of the producer’s supported write request sizes.

d)	The size of the final block within each frame does not obey these rules, but is truncated to generate a write transaction with the number of data bytes being sent. A quadlet write transaction shall be used if 4 aligned data bytes are being sent.

3)	Block or quadlet reads. A block read transaction may have one of the following two behaviours, listed in order of preference:

a)	A type_error status is returned.

b)	Previously written data is returned.

In some implementations, the segment-buffer writes can be serviced by hardware, without microprocessor intervention, in a monotonically increasing order, so that hardware can stream incoming data into affiliated segment-buffer storage.

### A.6.2	Control register accesses

The iAPR and oAPR registers only support aligned quadlet accesses. When such implemented registers are accessed, their behaviours are listed below:

1)	CompareAndSwap. A 4-byte CompareAndSwap lock transaction shall be supported.

2)	Other locks. Lock transactions other than CompareAndSwap shall return a type_error status.

3)	Quadlet read. For consistency with the CompareAndSwap functionality, the quadlet read transaction shall be supported. Note that this transaction is not normally used.

4)	Other read/write. Other read and write transactions are not supported and may have either of the following behaviours, listed in order of preference:

a)	A type_error status is returned.

b)	Reads return unspecified data; writes have unspecified effects.

Although the CompareSwap4 transaction can be serviced by hardware, a microprocessor or higher level sequencer shall be signalled when the transaction completes. This signal allows the affiliated segment data and status information to be processed.

## A.7	Asynchronous communications

### A.7.1	Frame transfer sequences

The flow control for the asynchronous connection can be phrased in terms of the producer and consumer behaviour constraints. Frame transfers are decomposed into a sequence of segment transfers, which are normally performed as follows:

1)	Space indication. The consumer updates the oAPR register, to indicate the segment buffer size.

2)	Segment writes. The producer writes into the segment buffer, until the end of the consumer’s segment buffer or the end of the producer’s frame (whichever comes first) is reached. (The protocols do not rely on these writes being detected by the consumer’s flow-control sequencer.)

3)	Segment indication. The producer updates the iAPR register, to inform the consumer when the segment-buffer writes have completed. The iAPR.mode field communicates the data-transfer status, as follows:

a)	iAPR.mode  MORE. One segment of the frame has been transferred; more segments are expected.

b)	iAPR.mode  LAST. The final segment of the frame has been transferred successfully.

c)	iAPR.mode  LESS. A portion of the frame has been transferred successfully, but the frame was truncated early.

d)	For these segment indications, the asserted iAPR.sc value shall equal the previously observed oAPR.sc value.

4)	Space indication. The consumer shall update the oAPR register, to inform the producer of the updated segment-buffer status. The oAPR.sc bit shall be toggled and the oAPR.mode field communicates the segment-buffer status, as follows:

–	oAPR.mode  SEND. The segment buffer has been emptied; the next segment may be sent.

5)	Next transfer. The next segment transfer depends on the current oAPR.mode value, as follows:

–	If the oAPR.mode value written in step 4)  SEND, then the next segment is written by continuing from step 2).

### A.7.2	Segment transfer constraints

Segment transfers are further restricted as follows:

1)	Register updates. Updates of the oAPR and iAPR registers shall be performed using the 4-byte compare-and-swap transaction.

2)	Segment writes. Segment-buffer writes shall be power-of-two in size, their addresses shall be an integer multiple of their size, and the write shall be no larger than the value specified by the producer-port’s oAPR.maxLoad field. The last write within each segment is an exception; its length corresponds to the number of remaining data bytes.

3)	Segment sizes. The consumer supplied oAPR.count value shall be an integer multiple of the transfer size specified by the oAPR.maxLoad field.

## A.8	Flow control example

As a simple example, consider the transfer of two (34-kbyte and 3-kbyte) frames into a 32-kbyte size consumer buffer. This would typically force additional segment-buffer restarts, as illustrated in Figure A.7. In this illustration, the write (arg1, arg2, arg3) label corresponds to a segment-buffer write: the arg1 value corresponds to the remote segment buffer address; the arg2 value corresponds to the producer’s data buffer address, and the arg3 value corresponds to length.

Each segment-transfer sequence consists of a oAPR update, segment-data writes, and a iAPR update. For clarity, the next oAPR update, that is logically associated with the following frame transfer, is shown at the end of this transfer sequence.

Figure A.7/J.117 [D26]

## A.9	Serial bus transaction errors

The asynchronous connection protocols were designed to allow safe retries of failed Serial Bus transactions. A Serial Bus transaction may fail due to a bus reset (queued requests and responses are discarded) or due to a data-transmission failure. The following design principles are assumed by this Recommendation for the purpose of supporting safe retries:

1)	Control registers. Successive updates of control register values always provide distinctive values, due to mandated changes in these registers’ sc bit or mode-field values.

2)	Segment buffer addresses. The segment-buffer writes are defined to have no effect other than the update of the addressed data value. Processing of duplicate retries can thus be handled as follows:

–	If sequential segment buffer transactions are streamed into memory, by a DMA engine that processes write requests, the addresses of these transactions can be checked. Duplicate transactions have the same segment-buffer address and no intervening control register updates; these duplicates can be safely discarded.

3)	Connection management commands. When desired, the connection-management commands can be designed to be fault tolerant, in the following ways:

a)	Duplicate detection. The success of an initial command is determined by the controller before attempting to retry the failed command.

b)	Resource recovery. The connection context is discarded unless the connection is quickly re-established after a bus reset.

Thus, a reliable connection can be maintained because transactions can be safely retried after busy-retry or response-missing timeouts are detected. Of course, the retries are subject to P1394a-specified tLabel use constraints (defined in IEEE P1394a [3]) and should use the most current (and possibly changed, due to a bus reset) nodeID values.

## A.10	AV/C managed connection sequences

In the general case the controller shown in the following subclauses can be in the producer or the consumer, or can be a separate entity. For this Recommendation the controller shall be located in the consumer. Bus transactions shown from controller to consumer actually occur within the consumer.

### A.10.1	Establishing an asynchronous connection

An asynchronous connection is established by a controller, which sends an ALLOCATE command to the consumer node, as illustrated by Figure A.8. The consumer plug is effectively locked from other changes, including connection overlays, between the processing of the initial ALLOCATE and final ATTACH command.

Figure A.8/J.117 [D27]

The ALLOCATE command (1a, 1b) allocates the plug resources and returns the address of the consumer port to the controller. The following ALLOCATE_ATTACH command is responsible for allocating and connecting the producer plug resources; the final ATTACH command is responsible for connecting the consumer plug.

The plug remains inactive until the consumer updates the producer-resident oAPR register, thereby activating the producer port.

### A.10.2	Breaking an asynchronous connection

The disconnection of asynchronous plugs is initiated by a controller, which sends a DETACH command to the consumer-plug node, as illustrated by Figure A.9.

The DETACH command (1a, 1b) leaves the consumer-plug resource in a passive state. While passive, the consumer-plug resource accepts register updates and segment-buffer writes from the producer, while inhibiting the generation of producer-port updates and segment-buffer writes. That is, the consumer no longer has access to the producer plug.

The following DETACH_RELEASE command (2a, 2b) detaches the producer-port, and releases the producer-plug resources.

The final RELEASE command (3a, 3b) disconnects the consumer-port and releases the plug resources.

### A.10.3	Failure of establishing an asynchronous connection

The controller’s connection sequence may fail if the affiliated producer-plug connection attempt is rejected. In this case, the controller is responsible for disconnecting the reserved consumer-port resource, as illustrated in Figure A.10.

Figure A.9/J.117 [D28]

Figure A.10/J.117 [D29]

### A.10.4	AV/C reconnect timing

Asynchronous connections are affected by bus resets, since the nodeID portions of their connected plugs may have changed. Rather than maintaining these connections across the bus reset, the controller is responsible for resuming the connections (providing each plug with a revised nodeID of the other) shortly after the bus reset. The timing requirements for resumption of asynchronous connections are illustrated in Figure A.11.

Figure A.11/J.117 [D30]

The reconnection of isochronous plugs is mandated to occur within the first second after the bus reset. Since this may consume resources of the controllers and the interconnect, the resumption of asynchronous connections is given more time and shall complete within 10 seconds.

A previously active producer or consumer plug rejects all connection and disconnection commands during these first two seconds, accepting only its expected resumption commands. After the two-second delay, unexpected resumption commands are rejected; the connection and disconnection commands are accepted in the normal fashion.

A previously inactive producer or consumer plug accepts connection and disconnection commands, with no distinction between the commands that are received in the first two seconds and those commands that follow.

#### A.10.4.1	Restoring Asynchronous Connections Communications

After a bus reset, the controllers are responsible for reconnecting previously connected plugs. The reconnection sequence is similar to the normal connection sequences, with the exception that a RESTORE_PORT rather than ALLOCATE command is used to re-establish the connection. The use of a distinct reconnection command allows reconnections and new connections to proceed concurrently, while providing the consumer-plug with sufficient information to distinguish between the two operations, as illustrated by Figure A.12.

A new asynchronous connection in the first two seconds is also allowed, but involves a normal asynchronous connection establishment sequence described in II.A.10.1.

Figure A.12/J.117 [D31]

## A.11	AV/C connection commands

### A.11.1	AV/C connection management commands

In general, AV/C connection-management commands allocate target-resident resources for use by data-connection communications, and perform unit-dependent security checks (if applicable) when the producer is selected.

Within this Recommendation, AV/C commands are presented in quadlet format, so the natural alignment of the defined data structures can be clearly and compactly presented, as previously illustrated. The quadlet presentation style also helps to uncover the presence of unintended unaligned data structures. However, this has no effect on the definition of other AV/C commands.

#### A.11.1.1	Reserved command frame fields

Target shall return a NOT_IMPLEMENTED response if any of these fields are not zero. See the AV/C Digital Interface Command Set General Specification for further details.

### A.11.2	Command Frames

The common frame format for ASYNCHRONOUS CONNECTION command (for ctype of both CONTROL and STATUS) and response, which provides the Asynchronous Connection and Asynchronous Plug management functionality, is illustrated by Figure A.13 below.

| Figure A.13/J.117 – ASYNCHRONOUS CONNECTION common frame format | msb |  |  |  |  |  |  |  | lsb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| opcode | ASYNCHRONOUS CONNECTION (2616) |  |  |  |  |  |  |  |  |
| operand[0] | subfunction |  |  |  |  |  |  |  |  |
| operand[1] | status |  |  |  |  |  |  |  |  |
| operand[2] | plug id |  |  |  |  |  |  |  |  |
| operand[3] | (msb) plug offset |  |  |  |  |  |  |  |  |
| operand[4] |  |  |  |  |  |  |  |  |  |
| operand[5] |  |  |  |  |  |  |  |  |  |
| operand[6] |  |  |  |  |  |  |  |  |  |
| operand[7] |  |  |  |  |  |  |  |  |  |
| operand[8] | (lsb) |  |  | port id |  |  |  | port bits |  |
| operand[9] | connected node ID |  |  |  |  |  |  |  |  |
| operand[10] |  |  |  |  |  |  |  |  |  |
| operand[11] | (msb) connected plug offset |  |  |  |  |  |  |  |  |
| operand[12] |  |  |  |  |  |  |  |  |  |
| operand[13] |  |  |  |  |  |  |  |  |  |
| operand[14] |  |  |  |  |  |  |  |  |  |
| operand[15] |  |  |  |  |  |  |  |  |  |
| operand[16] | (lsb) |  |  | connected port ID |  |  |  | connected portbits |  |
| operand[17] | connected plug ID |  |  |  |  |  |  |  |  |
| operand[18] | ex | r |  | connection count |  |  |  |  |  |
| operand[19] | write interval |  |  |  |  | retry count |  |  |  |
| operand[20] | reserved |  |  |  |  |  |  |  |  |

### A.11.3	Command frame values

The subfunction field indicates the action to be taken by the target for the command frame, which ctype field is CONTROL as shown in Table A.4.

The status field indicates the status of the asynchronous connection or the result of command execution. The status of the asynchronous plug is defined in A.13.2.

In case of the CONTROL command, this field indicates the result of command execution or the status of asynchronous plug. Table A.5 shows the status field values of the response frame in case of CONTROL command.

Table A.4/J.117 – subfunction field definitions

| subfunction | Value | Meaning |
| --- | --- | --- |
| ALLOCATE | 0116 | Allocate the consumer port resource |
| ATTACH | 0216 | Connect the consumer port to the producer port |
| ALLOCATE_ATTACH | 0316 | Allocate the producer port resource and connect it to the consumer port |
| RELEASE | 0516 | Release the port resource |
| DETACH | 0616 | Disconnect the consumer port |
| DETACH_RELEASE | 0716 | Disconnect and release the producer port resource |
| SUSPEND_PORT | 1016 | Suspend the consumer port |
| RESUME_PORT | 2016 | Resume the consumer port |
| RESTORE_PORT | 4016 | Restore the port resource after a bus reset |

Table A.5/J.117 – status values

| Value | Symbol | Response code | Meaning |
| --- | --- | --- | --- |
| 0116 | FREE | ACCEPTED | The specified port is in a FREE state |
| 0216 | FIXED | ACCEPTED | The specified port is in a FIXED state |
| 0316 | ACTIVE | ACCEPTED | The specified port is in an ACTIVE state |
| 0416 | PASSIVE | ACCEPTED | The specified port is in a PASSIVE state |
| 0516 | WAIT | ACCEPTED | The specified port is in a WAIT state |
| 0616 | SUSPENDED | ACCEPTED | The specified port is in a DETACHED |
| 8016 | NO_PLUG | REJECTED | No plug is available now |
| 8116 | NO_PORT | REJECTED | No port of the specified plug is available now |
| 8316 | PLUG_BUSY | REJECTED | The specified plug is not available |
| 8216 | PORT_BUSY | REJECTED | The specified port is not available |
| 8416 | INVALID_OFFSET | REJECTED | The invalid offset address value passed |
| 8816 | NO_CONNECTION | REJECTED | No connection to break, No connection to overlay |
| 9016 | CONNECTED_NODE_ERROR | REJECTED | Connected port not responding |
| 8816 | BROKEN | REJECTED | Disconnected by the connected port |
| 8916 | MAX_OVERLAY | REJECTED | Connection count is already at maximum value |
| FE16 | ANY_OTHER_ERR | REJECTED | Other internal errors |

If the msb of the returned status value is set to 1, this field indicates the error code.

Note that in the case of command frame (both CONTROL and STATUS), this field should be set to FF16, and status value shall be returned in a response frame from the target.

The 8-bit plugId field specifies which of this node’s plugs is being accessed.

The 8-bit plugId value identifies the target-resident plug that is to be connected. For AV/C units, the plugId assignments are specified by Table A.6. The unspecified plugId value normally indicates that the controller has no preference and the port number may be assigned by the target.

Table A.6/J.117 – plugId values

| Value | Description |
| --- | --- |
| 00-9F16 | Not Used (illegal) |
| AO16 | Asynchronous plug[0] |
| A1-BD16 | Asynchronous plug[1] to plug[29] |
| BE16 | Asynchronous plug[30] |
| BF16 | Any available plug |
| C0-FF16 | Not Used (illegal) |

The plug offset field has 42-bit length and this field indicates the base offset address of the plug of the target.

The 4-bit portId field specifies which port is selected, as specified by Table A.7. The unspecified portId value normally indicates that the controller has no preference and the port number may be assigned by the target.

Table A.7/J.117 – portId values

| Value | Description |
| --- | --- |
| 0 | Consumer Port |
| 1 | Producer plug |

The port bits field indicates the constraints and capabilities of specified port. The meaning of this field depends on the type of the port. If the port is the consumer port, this field indicates constraints it has. If the port is the producer port, this field indicates the capability of it.

The connected node ID field indicates the node identifier with which the target is (going to be) connected with.

The connected plug offset field has 42-bit length and this field indicates the base offset address of the plug to which the target plug is (going to be) connected.

The connected port ID field indicates the port identifier or the port to which the target plug is (going to be) connected, and this field has values defined in Table A.7.

The connected port bits field indicates constraints and capabilities of specified port. The meaning of this field depends on the type of the port. If the port is the consumer port, this field indicates constraints it has. If the port is the producer port, this field indicates the capability of it. For the purposes of this Recommendation, these bits shall be zero.

The connected plug ID field indicates the plug identifier of the target, and has the values defined by Table A.6.

The connection count field indicates the number of overlaid connections the consumer node holds. This means how many controllers concern for this connection. If the consumer plug is not connected to any other producer node, this field is 0. After the consumer plug is connected to the producer plug, this field shall be 0116. Note that the value of 3F16 shall be used for the command frame and the response frame may include the current value. As the producer port does not hold this information, this field shall be always 3F16.

The ex bit specifies whether the other controllers are expected to be excluded from this connection or not. In this Recommendation, only one controller is allowed so this bit shall always be 1.

The write interval field shall always be F16.

The retry count field shall always be F16.

The r bit shall be zero.

The 16-bit reserved field shall be zero.