## A.12	AV/C Connection Command Format

Recommendation J.117     (09/99)

### A.12.1	ALLOCATE command

ALLOCATE subfunction is used for the consumer to retrieve the offset address of the specified or available consumer plug from the target. The controller can specify the plug ID of the target, or allow the consumer to allocate the available plug with using "any available plug" value.

A controller may specify the plug ID or it may allow the target to allocate the available plug when the controller has no preference and the target may assign the plug number.

When this subfunction is used with specified plug ID, the specified plug shall not be disconnected and shall not be allocated by the other controller. Otherwise, the issued ALLOCATE command shall be rejected with REJECTED response returned.

In the case that this subfunction is used with "any available plug" value for the plug ID field, if the consumer has an available plug, its plug ID and its offset address should be returned in the ACCEPTED response frame.

Table A.8 illustrates the value of each field in the CONTROL command frame and ACCEPTED response frame of ALLOCATE subfunction.

Table A.8/J.117 – Command and ACCEPTED response frame of ALLOCATE

| field | CONTROL command frame | ACCEPTED response frame |
| --- | --- | --- |
| subfunction | ALLOCATE (0116) |  |
| status | not used (FF16) | FIXED (0116) |
| plug ID | specified plug ID or any available plug (BF16) | specified plug ID or allocated plug ID |
| plug Offset | not used (3 FF FF FF FF FF16) | Offset address of the plug |
| port ID | consumer port (016) |  |
| port bits | not used (112) | supported value |
| connected node ID | not used (FF FF16) |  |
| connected plug offset | not used (3 FF FF FF FF FF16) |  |
| connected port ID | not used (F16) |  |
| connected port bits | not used (112) |  |
| connected plug ID | not used (FF16) |  |
| ex | exclusive (12) |  |
| connection count | not used (3F16) | 0016 (current value) |
| write interval | not used (F16) | required write interval value |
| retry count | not used (F16) | required retry count value |
| NOTE 1 – "" means "same as the previous frame". NOTE 2 – ALLOCATE command can be used for only the consumer node, so the port ID field shall be always 0. |  |  |

If the fields which are used to specify the parameters (other fields which are not described "not used"), include invalid value, or the target cannot allocate the requested plug resource, the target returns the REJECTED response.

Table A.9 illustrates the value of each field in the CONTROL command frame and REJECTED response frame of ALLOCATE subfunction.

Table A.9/J.117 – Command and REJECTED response frame of ALLOCATE

| field | CONTROL command frame | REJECTED response frame |
| --- | --- | --- |
| subfunction | ALLOCATE (0116) |  |
| status | not used (FF16) | error code |
| plug ID | specified plug ID or any available plug (BF16) |  |
| plug Offset | not used (3 FF FF FF FF FF16) |  |
| port ID | consumer port (016) |  |
| port bits | not used (112) |  |
| connected node ID | not used (FF FF16) |  |
| connected plug offset | not used (3 FF FF FF FF FF16) |  |
| connected port ID | not used (F16) |  |
| connected port bits | not used (112) |  |
| connected plug ID | not used (FF16) |  |
| ex | exclusive (12) |  |
| connection count | not used (3F16) |  |
| write interval | not used (F16) |  |
| retry count | not used (F16) |  |
| NOTE 1 – "" means "same as the command frame". NOTE 2 – In the REJECTED response frame, the status field includes the error code as defined in Table A.5. |  |  |

### A.12.2	ALLOCATE_ATTACH command

ALLOCATE_ATTACH subfunction is used for the producer to retrieve the offset address of the specified or available producer plug from the target, and to inform the producer node about the consumer plug resource related information for establishing a connection.

A controller may specify the plug ID, or it may allow the target to allocate the available plug when the controller has no preference and the plug number may be assigned by the producer. And the controller shall specify the to-be-connected plug information that had been retrieved from the consumer node by the ALLOCATE response frame.

In the case that this subfunction is used with "any available plug" value for the plug ID field, if the consumer has an available plug, its plug ID and its offset address would be returned in the ACCEPTED response frame.

Table A.10 illustrates the value of each field in the CONTROL command frame and ACCEPTED response frame for the ALLOCATE_ATTACH subfunction.

Table A.11 illustrates the value of each field in the CONTROL command and the REJECTED response frame for ALLOCATE_ATTACH subfunction.

### A.12.3	ATTACH command

ATTACH subfunction is used for the consumer, to inform the consumer node about the producer plug resource related information to establish a connection.

The controller specifies the consumer plug resource related information (plug ID, offset address, and so on) which had been previously retrieved from the consumer, and the controller also specifies the to-be-connected producer port related information which had been retrieved from the producer node.

Table A.12 illustrates the value of each field in the CONTROL command frame and ACCEPTED response frame for the ATTACH subfunction.

Table A.10/J.117 – Command and ACCEPTED response frame of ALLOCATE_ATTACH

| field | CONTROL command frame | ACCEPTED response frame |
| --- | --- | --- |
| subfunction | ALLOCATE_ATTACH (0316) |  |
| status | not used (FF16) | ACTIVE (0316) |
| plug ID | specified plug ID or any available plug (BF16) | specified plug ID or allocated plug ID |
| plug Offset | not used (3 FF FF FF FF FF16) | Offset address of the plug |
| port ID | specified port ID or any available port (F16) | specified port ID or allocated port ID |
| port bits | not used (112) | supported value |
| connected node ID | specified node ID to be connected (consumer) |  |
| connected plug offset | offset address of the specified plug to be connected (consumer) |  |
| connected port ID | consumer port (016) |  |
| connected port bits | supported value from consumer plug |  |
| connected plug ID | plug ID of the consumer |  |
| ex | exclusive (12) |  |
| connection count | not used (3F16) |  |
| write interval | required value from the consumer |  |
| retry count | required value from the consumer |  |
| NOTE – "" means "same as the command frame". |  |  |

Table A.11/J.117 – Command and REJECTED response for ALLOCATE_ATTACH

| field | CONTROL command frame | REJECTED response frame |
| --- | --- | --- |
| subfunction | ALLOCATE_ATTACH (0316) |  |
| status | not used (FF16) | error code |
| plug ID | specified plug ID or any available plug (BF16) |  |
| plug Offset | not used (3 FF FF FF FF FF16) |  |
| port ID | specified port ID or any available port (F16) |  |
| port bits | not used (112) |  |
| connected node ID | specified node ID to be connected (consumer) |  |
| connected plug offset | offset address of the specified plug to be connected (consumer) |  |
| connected port ID | consumer port (016) |  |
| connected port bits | supported value from consumer plug |  |
| connected plug ID | plug ID of the consumer |  |
| ex | exclusive (12) |  |
| connection count | not used (3F16) |  |
| write interval | required value from the consumer |  |
| retry count | required value from the consumer |  |
| NOTE 1 – "" means "same as the command frame". NOTE 2 – In the REJECTED response frame, the status field includes the error status value as defined in Table A.5. |  |  |

Table A.12/J.117 – Command and ACCEPTED response frame of ATTACH

| field | CONTROL command frame | ACCEPTED response frame |
| --- | --- | --- |
| subfunction | ATTACH (0216) |  |
| status | not used (FF16) | ACTIVE (0316) |
| plug ID | allocated plug ID |  |
| plug Offset | Offset address of the plug |  |
| port ID | consumer port (016) |  |
| port bits | supported value |  |
| connected node ID | specified node ID to be connected (producer) |  |
| connected plug offset | offset address of the specified plug to be connected (producer) |  |
| connected port ID | port ID of the producer port |  |
| connected port bits | supported value from producer plug |  |
| connected plug ID | plug ID of the producer |  |
| ex | exclusive (12) |  |
| connection count | not used (3F16) | 0116 |
| write interval | not used (F16) |  |
| retry count | not used (F16) |  |
| NOTE – "" means "same as the command frame". |  |  |

Table A.13 illustrates the CONTROL command frame and REJECTED response frame for ATTACH subfunction.

Table A.13/J.117 – Command and REJECTED response for ATTACH

| field | CONTROL command frame | REJECTED response frame |
| --- | --- | --- |
| subfunction | ATTACH (0216) |  |
| status | not used (FF16) | error code |
| plug ID | allocated plug ID |  |
| plug Offset | Offset address of the plug |  |
| port ID | consumer port (016) |  |
| port bits | supported value |  |
| connected node ID | specified node ID to be connected (producer) |  |
| connected plug offset | offset address of the specified plug to be connected (producer) |  |
| connected port ID | port ID of the producer port |  |
| connected port bits | supported value from producer plug |  |
| connected plug ID | plug ID of the producer |  |
| ex | exclusive (12) |  |
| connection count | not used (3F16) |  |
| write interval | not used (F16) |  |
| retry count | not used (F16) |  |
| NOTE 1 – "" means "same as the previous frame". NOTE 2 – In the REJECTED response frame, the status field includes the error code as defined at Table A.5. |  |  |

### A.12.4	DETACH command

DETACH subfunction is used for the consumer to break the connection it holds. The controller shall specify the plug ID and port ID values of the target.

If the controller detects the connection count is set to 0 in the ACCEPTED response frame for DETACH subfunction, the controller is responsible for breaking this connection, by issuing the following DETACH_RELEASE and RELEASE commands.

Table A.14 illustrates the value of each field in the CONTROL command frame and ACCEPTED response frame for the DETACH subfunction.

Table A.14/J.117 – Command and ACCEPTED response frame of DETACH

| field | CONTROL command frame | ACCEPTED response frame |
| --- | --- | --- |
| subfunction | DETACH (0616) |  |
| status | not used (FF16) | ACTIVE (0316) or SUSPENDED (0616) or PASSIVE (0416) |
| plug ID | specified plug ID |  |
| plug Offset | not used (3 FF FF FF FF FF16) |  |
| port ID | consumer port (016) |  |
| port bits | not used (112) |  |
| connected node ID | not used (FF FF16) |  |
| connected plug offset | not used (3 FF FF FF FF FF16) |  |
| connected port ID | not used (F16) |  |
| connected port bits | not used (112) |  |
| connected plug ID | not used (FF16) |  |
| ex | exclusive (12) |  |
| connection count | not used (3F16) | decremented connection count |
| write interval | not used (F16) |  |
| retry count | not used (F16) |  |
| NOTE 1 – "" means "same as the previous frame". NOTE 2 – DETACH command can be used for the consumer node only, so the port ID field value shall be always 0. |  |  |

If the passed parameters are invalid, or the target cannot allocate the requested plug resource, the target returns the REJECTED response.

Table A.15 illustrates the value of each field in the CONTROL command frame and REJECTED response frame.

### A.12.5	DETACH_RELEASE Command

DETACH_RELEASE subfunction is used for the producer, to break the connection it holds and to release the port resource.

The controller shall specify the target plug ID and port ID values. Otherwise, the issued RELEASE request may be rejected with REJECTED response returned.

Table A.16 illustrates the value of each field in the CONTROL command frame and ACCEPTED response frame for the DETACH_RELEASE subfunction.

Table A.15/J.117 – Command and REJECTED response for DETACH

| field | CONTROL command frame | REJECTED response frame |
| --- | --- | --- |
| subfunction | DETACH (0616) |  |
| status | not used (FF16) | error code |
| plug ID | specified plug ID |  |
| plug Offset | not used (3 FF FF FF FF FF16) |  |
| port ID | consumer port (016) |  |
| port bits | not used (112) |  |
| connected node ID | not used (FF FF16) |  |
| connected plug offset | not used (3 FF FF FF FF FF16) |  |
| connected port ID | not used (F16) |  |
| connected port bits | not used (112) |  |
| connected plug ID | not used (FF16) |  |
| ex | exclusive (12) |  |
| connection count | not used (3F16) |  |
| write interval | not used (F16) |  |
| retry count | not used (F16) |  |
| NOTE 1 – "" means "same as the previous frame". NOTE 2 – In the REJECTED response frame, the status field includes the error code as defined in Table A.5. |  |  |

Table A.16/J.117 – Command and ACCEPTED response frame of DETACH_RELEASE

| field | CONTROL command frame | ACCEPTED response frame |
| --- | --- | --- |
| subfunction | DETACH _RELEASE (0716) |  |
| status | not used (FF16) | FREE (0116) |
| plug ID | specified plug ID |  |
| plug Offset | not used (3 FF FF FF FF FF16) |  |
| port ID | specified port ID |  |
| port bits | not used (112) |  |
| connected node ID | not used (FF FF16) |  |
| connected plug offset | not used (3 FF FF FF FF FF16) |  |
| connected port ID | not used (F16) |  |
| connected port bits | not used (112) |  |
| connected plug ID | not used (FF16) |  |
| ex | exclusive (12) |  |
| connection count | not used (3F16) |  |
| write interval | not used (F16) |  |
| retry count | not used (F16) |  |
| NOTE 1 – "" means "same as the command frame". NOTE 2 – DETACH_RELEASE command can be used for the producer node only, so the port ID field value shall be specified and shall not be 0. |  |  |

If the passed parameters are invalid, the target returns the REJECTED response.

Table A.17 illustrates the value of each field in the CONTROL command frame and REJECTED response frame for DETACH_RELEASE subfunction.

Table A.17/J.117 – Command and REJECTED response frame of DETACH_RELEASE

| field | CONTROL command frame | REJECTED response frame |
| --- | --- | --- |
| subfunction | DETACH _RELEASE (0716) |  |
| status | not used (FF16) | status value |
| plug ID | specified plug ID |  |
| plug Offset | not used (3 FF FF FF FF FF16) |  |
| port ID | specified port ID |  |
| port bits | not used (112) |  |
| connected node ID | not used (FF FF16) |  |
| connected plug offset | not used (3 FF FF FF FF FF16) |  |
| connected port ID | not used (F16) |  |
| connected port bits | not used (112) |  |
| connected plug ID | not used (FF16) |  |
| ex | exclusive (12) |  |
| connection count | not used (3F16) |  |
| write interval | not used (F16) |  |
| retry count | not used (F16) |  |
| NOTE 1 – "" means "same as the command frame". NOTE 2 – In the REJECTED response frame, the status field includes the error code as defined in Table A.5. |  |  |

### A.12.6	RELEASE command

RELEASE subfunction is used for the consumer to release the plug resource.

Table A.18 illustrates the value of each field in the CONTROL command frame and the ACCEPTED response frame of RELEASE subfunction.

Table A.18/J.117 – Command and ACCEPTED response frame of RELEASE

| field | CONTROL command frame | ACCEPTED response frame |
| --- | --- | --- |
| subfunction | RELEASE (0516) |  |
| status | not used (FF16) | FREE (0116) |
| plug ID | specified plug ID |  |
| plug Offset | not used (3 FF FF FF FF FF16) |  |
| port ID | consumer port (016) |  |
| port bits | not used (112) |  |
| connected node ID | not used (FF FF16) |  |
| connected plug offset | not used (3 FF FF FF FF FF16) |  |
| connected port ID | not used (F16) |  |
| connected port bits | not used (112) |  |
| connected plug ID | not used (FF16) |  |
| ex | exclusive (12) |  |
| connection count | not used (3F16) |  |
| write interval | not used (F16) |  |
| retry count | not used (F16) |  |
| NOTE 1 – "" means "same as the previous frame". NOTE 2 – RELEASE command can be used for the consumer node only, so the port ID field value shall be always 0. |  |  |

If the passed parameters are invalid, or the target cannot allocate the requested plug resource, the target returns the REJECTED response.

Table A.19 illustrates the value of each field in the CONTROL command frame and REJECTED response frame for RELEASE subfunction.

Table A.19/J.117 – Command and REJECTED response frame of RELEASE

| field | CONTROL command frame | REJECTED response frame |
| --- | --- | --- |
| subfunction | RELEASE (0516) |  |
| status | not used (FF16) | error code |
| plug ID | specified plug ID |  |
| plug Offset | not used (3 FF FF FF FF FF16) |  |
| port ID | consumer port (016) |  |
| port bits | not used (112) |  |
| connected node ID | not used (FF FF16) |  |
| connected plug offset | not used (3 FF FF FF FF FF16) |  |
| connected port ID | not used (F16) |  |
| connected port bits | not used (112) |  |
| connected plug ID | not used (FF16) |  |
| ex | exclusive (12) |  |
| connection count | not used (3F16) |  |
| write interval | not used (F16) |  |
| retry count | not used (F16) |  |
| NOTE 1 – "" means "same as the previous frame". NOTE 2 – In the REJECTED response frame, the status field includes the error code as defined in Table A.5. |  |  |

### A.12.7	RESTORE_PORT command

RESTORE_PORT subfunction is used for the producer and the consumer to re-establish the connection after a bus reset.

And also, RESTORE_PORT subfunction is used for the producer to re-establish the multicast connection with specifying the port ID value that had been connected.

The controller shall specify the plug ID and port ID and connected node ID and ex fields. The connected node ID field shall be updated with the new node ID if it changes after a bus reset.

If these values are not consistent with the previous values before a bus reset, the target shall return the REJECTED response.

Table A.20 illustrates the value of each field in the CONTROL command frame and the ACCEPTED response frame of RESTORE_PORT for the producer port.

Table A.21 illustrates the value of each field in the CONTROL command frame and the REJECTED response frame of RESTORE_PORT for the producer port.

Table A.22 illustrates the value of each field in the CONTROL command frame and the ACCEPTED response frame of RESTORE_PORT for the consumer port.

Table A.23 illustrates the value of each field in the CONTROL command frame and the REJECTED response frame of RESTORE_PORT for the consumer port.

Table A.20/J.117 – Command and ACCEPTED response frame of RESTORE_PORT for the producer port

| field | CONTROL command frame | ACCEPTED response frame |
| --- | --- | --- |
| subfunction | RESTORE_PORT (4016) |  |
| status | not used (FF16) | ACTIVE (0316) |
| plug ID | specified plug ID |  |
| plug Offset | not used (3 FF FF FF FF FF16) | offset address of the plug |
| port ID | port ID of the producer port |  |
| port bits | not used (1102) | supported value |
| connected node ID | node ID of consumer |  |
| connected plug offset | not used (3 FF FF FF FF FF16) | offset address of connected plug |
| connected port ID | consumer port (0016) |  |
| connected port bits | not used (1102) | supported value from the consumer port |
| connected plug ID | not used (FF16) | connected plug ID of the consumer |
| ex | exclusive (12) |  |
| connection count | not used (3F16) |  |
| write interval | not used (F16) |  |
| retry count | not used (F16) |  |
| NOTE – "" means "same as the command frame". |  |  |

Table A.21/J.117 – Command and REJECTED response frame of RESTORE_PORT for the producer port

| field | CONTROL command frame | REJECTED response frame |
| --- | --- | --- |
| subfunction | RESTORE_PORT (4016) |  |
| status | not used (FF16) | error code |
| plug ID | specified plug ID |  |
| plug Offset | not used (3 FF FF FF FF FF16) |  |
| port ID | port ID of the producer port |  |
| port bits | not used (1102) |  |
| connected node ID | node ID of consumer |  |
| connected plug offset | not used (3 FF FF FF FF FF16) |  |
| connected port ID | consumer port (0016) |  |
| connected port bits | not used (1102) |  |
| connected plug ID | not used (FF16) |  |
| ex | exclusive (12) |  |
| connection count | not used (3F16) |  |
| write interval | not used (F16) |  |
| retry count | not used (F16) |  |
| NOTE 1 – "" means "same as the command frame". NOTE 2 – In the REJECTED response frame, the status field includes the error code as defined in Table A.5. |  |  |

Table A.22/J.117 – Command and ACCEPTED response frame of RESTORE_PORT for the consumer port

| field | CONTROL command frame | ACCEPTED response frame |
| --- | --- | --- |
| subfunction | RESTORE_PORT (4016) |  |
| status | not used (FF16) | ACTIVE (0316) or SUSPENDED (0616) |
| plug ID | specified plug ID |  |
| plug Offset | not used (3 FF FF FF FF FF16) | offset address of the plug |
| port ID | consumer port (0016) |  |
| port bits | not used (1102) | supported value |
| connected node ID | node ID of producer |  |
| connected plug offset | not used (3 FF FF FF FF FF16) | offset address of connected plug |
| connected port ID | port ID of the producer port |  |
| connected port bits | not used (1102) | supported value from the producer port |
| connected plug ID | not used (FF16) | connected plug ID of the producer |
| ex | exclusive (12) |  |
| connection count | not used (3F16) | 0116 |
| write interval | not used (F16) |  |
| retry count | not used (F16) |  |
| NOTE – "" means "same as the command frame". |  |  |

Table A.23/J.117 – Command and REJECTED response frame of RESTORE_PORT for the consumer port

| field | CONTROL command frame | REJECTED response frame |
| --- | --- | --- |
| subfunction | RESTORE_PORT (4016) |  |
| status | not used (FF16) | error code |
| plug ID | specified plug ID |  |
| plug Offset | not used (3 FF FF FF FF FF16) |  |
| port ID | consumer port (00 16) |  |
| port bits | not used (1102) |  |
| connected node ID | node ID of producer |  |
| connected plug offset | not used (3 FF FF FF FF FF16) |  |
| connected port ID | port ID of the producer port |  |
| connected port bits | not used (1102) |  |
| connected plug ID | not used (FF16) |  |
| ex | exclusive (12) |  |
| connection count | not used (3F16) |  |
| write interval | not used (F16) |  |
| retry count | not used (F16) |  |
| NOTE 1 – "" means "same as the command frame". NOTE 2 – In the REJECTED response frame, the status field includes the error code as defined in Table A.5. |  |  |

### A.12.8	SUSPEND_PORT command

SUSPEND_PORT subfunction is used for the consumer port to suspend the connection. When a connection is suspended, the remaining of the frame data is discarded by the producer, and no data transmission occurred until the RESUME_PORT subfunction is issued.

Table A.24 illustrates the value of each field in the CONTROL command frame and the ACCEPTED response frame of SUSPEND_PORT subfunction.

Table A.24/J.117 – Command and ACCEPTED response frame of SUSPEND_PORT

| field | CONTROL command frame | ACCEPTED response frame |
| --- | --- | --- |
| subfunction | SUSPEND_PORT (1016) |  |
| status | not used (FF16) | SUSPENDED (0616) |
| plug ID | specified plug ID |  |
| plug Offset | not used (3 FF FF FF FF FF16) |  |
| port ID | consumer port (016) |  |
| port bits | not used (112) |  |
| connected node ID | not used (FF FF16) |  |
| connected plug offset | not used (3 FF FF FF FF FF16) |  |
| connected port ID | not used (F16) |  |
| connected port bits | not used (112) |  |
| connected plug ID | not used (FF16) |  |
| ex | exclusive (12) |  |
| connection count | not used (3F16) |  |
| write interval | not used (F16) |  |
| retry count | not used (F16) |  |
| NOTE 1 – "" means "same as the previous frame". NOTE 2 – SUSPEND_PORT command can be used for the consumer node only, so the port ID field value shall be 0. |  |  |

If the passed parameters are invalid, or the consumer port is already in a SUSPENDED state, the target returns the REJECTED response.

Table A.25 illustrates the value of each field in the CONTROL command frame and REJECTED response frame for SUSPEND_PORT subfunction.

### A.12.9	RESUME_PORT command

RESUME_PORT subfunction is used for the suspended-consumer port to resume the frame transmission.

The resumed asynchronous connection layer restarts the frame transmission from the beginning of the frame, and during the SUSPENDED state, frames may be lost.

Table A.26 illustrates the value of each field in the CONTROL command frame and the ACCEPTED response frame of RESUME_PORT subfunction.

If the passed parameters are invalid, or the consumer port is not in a SUSPENDED state, the target returns the REJECTED response.

Table A.27 illustrates the value of each field in the CONTROL command frame and REJECTED response frame for RELEASE subfunction.

Table A.25/J.117 – Command and REJECTED response frame of SUSPEND_PORT

| field | CONTROL command frame | REJECTED response frame |
| --- | --- | --- |
| subfunction | SUSPEND_PORT (1016) |  |
| status | not used (FF16) | error code |
| plug ID | specified plug ID |  |
| plug Offset | not used (3 FF FF FF FF FF16) |  |
| port ID | consumer port (016) |  |
| port bits | not used (112) |  |
| connected node ID | not used (FF FF16) |  |
| connected plug offset | not used (3 FF FF FF FF FF16) |  |
| connected port ID | not used (F16) |  |
| connected port bits | not used (112) |  |
| connected plug ID | not used (FF16) |  |
| ex | exclusive (12) |  |
| connection count | not used (3F16) |  |
| write interval | not used (F16) |  |
| retry count | not used (F16) |  |
| NOTE 1 – "" means "same as the previous frame". NOTE 2 – In the REJECTED response frame, the status field includes the error code as defined in Table A.5. |  |  |

Table A.26/J.117 – Command and ACCEPTED response frame of RESUME_PORT

| field | CONTROL command frame | ACCEPTED response frame |
| --- | --- | --- |
| subfunction | RESUME_PORT (2016) |  |
| status | not used (FF16) | ACTIVE (0316) |
| plug ID | specified plug ID |  |
| plug Offset | not used (3 FF FF FF FF FF16) |  |
| port ID | consumer port (016) |  |
| port bits | not used (112) |  |
| connected node ID | not used (FF FF16) |  |
| connected plug offset | not used (3 FF FF FF FF FF16) |  |
| connected port ID | not used (F16) |  |
| connected port bits | not used (112) |  |
| connected plug ID | not used (FF16) |  |
| ex | exclusive (12) |  |
| connection count | not used (3F16) |  |
| write interval | not used (F16) |  |
| retry count | not used (F16) |  |
| NOTE 1 – "" means "same as the previous frame". NOTE 2 – RESUME_PORT command can be used for the consumer node only, so the port ID field value shall be always 0. |  |  |

Table A.27/J.117 – Command and REJECTED response frame of RESUME_PORT

| field | CONTROL command frame | REJECTED response frame |
| --- | --- | --- |
| subfunction | RESUME_PORT (2016) |  |
| status | not used (FF16) | error code |
| plug ID | specified plug ID |  |
| plug Offset | not used (3 FF FF FF FF FF16) |  |
| port ID | consumer port (016) |  |
| port bits | not used (112) |  |
| connected node ID | not used (FF FF16) |  |
| connected plug offset | not used (3 FF FF FF FF FF16) |  |
| connected port ID | not used (F16) |  |
| connected port bits | not used (112) |  |
| connected plug ID | not used (FF16) |  |
| ex | exclusive (12) |  |
| connection count | not used (3F16) |  |
| write interval | not used (F16) |  |
| retry count | not used (F16) |  |
| NOTE 1 – "" means "same as the previous frame". NOTE 2 – In the REJECTED response frame, the status field includes the error code as defined in Table A.5. |  |  |

## A.13	Asynchronous Connection plug states

### A.13.1	Code definitions

All of the procedures in this subclause use the syntax specified in Table A.28.

### A.13.2	Consumer port states

#### A.13.2.1	Consumer port state machine

The behaviour of a consumer plug is specified by its state machine definition, as illustrated by Figure A.14.

Table A.28/J.117 – State machine code definitions

typedef struct { Byte subfnc; // sous-fonction Byte plugID; // target plug identifier Byte status; // plug status or result of command execution Octlet plugAddr; // offset address of plug, including portID and portBits Doublet connectNodeID; // node identifier of (to-be) connected node Octlet connectPlugOffset; // offset address of (to-be) connected plug, // including connectPortID and connectPortBits Byte connectPlugID; // plug identifier of (to-be) connected port boolean ex; // exclusive request bit Byte connectionCount; // connection count which the (consumer) port holds Byte writeInterval; // write interval value requested from the consumer Byte retryCount; // retry count value requested from the consumer } portInfo; portInfo *cInfo; // portInfo stored in the consumer port of the target portInfo *pInfo; // portInfo stored in the producer port of the target portInfo *req; // portInfo stored in the command frame requested by the controller portInfo *hold; // portInfo stored for deferred response Doublet ctrlID; // controller nodeID Doublet ctrlID_bak; // backuped controller nodeID boolean dr; // flag for disconnect request boolean suspendflg; // flag for suspended state before bus reset Byte avc_cmd ( Doublet *ctrlID, portInfo *req ); // AV/C command frame reception event, returns subfunction&0x7F, // or NULL when no event arrived Byte avc_rsp ( Doublet ctrlID, Byte rsp, portInfo *); // generate AV/C response frame to controller, // specifying rsp as response code void invalidate ( portInfo * ); // invalidate portInfo, filling up with 0xFF void setupInfo ( portInfo *dst, portInfo *src); // copy the valid field from src to dst, making up the dst portInfo Byte ACEvent ( BYTE *mode, BYTE *status); // event indication from AsyncConnections layer, // mode returns register mode value status returns the execution // results from AsyncConnections layer void AttachEvent ( portInfo * ); // ATTACH event request to AsyncConnections layer void DetachEvent ( portInfo * ); // DETACH event request to AsyncConnections layer void RestoreEvent ( portInfo * ); // RESTORE event request to AsyncConnections layer void PassiveEvent ( portInfo * ); // PASSIVE event request to AsyncConnections layer void BreakEvent ( portInfo * ); // BREAK event request to AsyncConnections layer void SuspendEvent ( portInfo * ); // SUSPEND event request to AsyncConnections layer void ResumeEvent ( portInfo * ); // RESUME event request to AsyncConnections layer

#### A.13.2.2	Consumer port state machine notes

State C0:FREE. The FREE state is the initial state of the port, with no committed resources. While in the C0:FREE state, the purpose of the connection management commands is to transition the node to the C2:ACTIVE (connected) state, by passing through the intermediate C1:FIXED state. These state transitions are listed below.

Transition C0a. When ALLOCATE subfunction command is requested, the node generates a port information and return the AV/C response frame to the controller, then transitions to the C1:FIXED state. If the ex bit in a command frame had been set to 1, the target stores the nodeID of the controller to reject the following requests from other controllers.

Transition C0b. When STATUS command is requested, the node reports the current port information by generating an AV/C STABLE response frame.

Transition C0c. All other connection management subfunctions are rejected, generating an AV/C REJECTED response frame.

Transition C0d. Bus resets while in the C0:FREE state leave the port in the same state.

State C1:FIXED. In the FIXED state the ports resources have been allocated, but the node is not yet prepared to accept incoming port requests and cannot send transactions to the remote port (since this port has not yet been identified). While in the C1:FIXED state, the purpose of the connection management commands is to transition the node to the C2:ACTIVE (connected) state. These state transitions are listed below.

Transition C1a. The ATTACH or ATTACH_FRAME subfunction command transitions the node to the fully-connected C2:ACTIVE state. ATTACH or ATTACH_FRAME subfunction command frame includes the connection information such as producer port address. The node requests to an asynchronous connections layer to start by AttachEvent(). Although not illustrated, the oAPR register on the affiliated producer port is also updated, to activate asynchronous connection communications.

Then according to the msb of subfunction value which is stored as dr, the node returns an AV/C response frame (ACCEPTED or INTERIM) to the controller.

If the dr had been set to 1, the target stores the requested port information.

Transition C1b. The RELEASE subfunction command returns the node to the initial C0:FREE state. (This is normally the final step in a connection-abort sequence, which releases the consumer after a producer-port connection is discovered.) The node invalidates the port information and returns an AV/C ACCEPTED response frame to the controller.

Transition C1c. When STATUS command with subfunction set to STATUS is requested, the node reports the current port information by generating an AV/C STABLE response frame.

Transition C1d. After 10 seconds connection timeout, the consumer port transitions to the C0:FREE state. The node invalidates the port information.

Transition C1e. A bus reset returns the port to its initial C0:FREE state. The node invalidates the port information.

Transition C1d. Other connection-management commands are rejected, because they have no meaning while in this transient state or the following stable state. The node returns an AV/C REJECTED response to the controller.

State C2:ACTIVE. In the ACTIVE state the port is operational. While in the C2:ACTIVE state, the purpose of the connection management commands are to allow overlays (which leave the node in the C2:ACTIVE state, but increment the connection count) and to allow disconnections by transitioning the node to the C3:PASSIVE state. These state transitions are listed below.

Transition C2a. The DETACH subfunction command with no overlays (connection count == 1) state transitions the node to a half disconnected C3:PASSIVE state. The node shall request to an asynchronous connections layer to stop requesting the data by PassiveEvent(), setting connection count value to 0, then return an AV/C ACCEPTED response to the controller.

Transition C2b. The DETACH subfunction command decrements the overlaid count if the connection count was more than 1, leaving the port connected in its C2:ACTIVE state. The node returns an AV/C ACCEPTED response frame on success.

Transition C2c. The SUSPEND_PORT subfunction command transitions to C5:SUSPENDED state. The node requests to an asynchronous connections layer to suspend by SuspendEvent(). The node returns an AV/C ACCEPTED response frame to the controller on success.

Transition C2d. When the previous issued command had included the dr bit set to 1 and EndOfFrame was indicated from an asynchronous connections layer, the node requests the asynchronous connections layer to break the connection by issuing BreakEvent(), then invalidates the port information, then returns an AV/C response frame to the controller, indicating the previous issued CONTROL command execution had been finished. If the mode value retrieved from the asynchronous connections layer had been LAST, the AV/C response frame is to be ACCEPTED response, otherwise REJECTED response.

Transition C2e. A bus reset transitions the port to the C4:WAIT state, in preparation of accepting the expected RESTORE_PORT subfunction command (this command restores the nodeID address of its connected plug). The node sets its connect node ID value to FFFF16 (invalid node ID), and connection count to 0. The node sets timer value to 0.

Transition C2f. When STATUS command is requested, the node reports the current port information by generating an AV/C STABLE response frame.

Transition C2g. Other connection-management commands are rejected. The node returns an AV/C REJECTED response to the controller.

State C3:PASSIVE. In the PASSIVE state the port is half disabled; incoming requests are accepted (because the connected port may still be active) but transactions are not generated by this node. While in the C3:PASSIVE state, the purpose of the connection management commands is to allow disconnections by transitioning the node to the C0:FREE state. These state transitions are listed below.

Transition C3a. The RELEASE subfunction command releases the node's allocated resources and returns the node to the C0:FREE state. The node requests to an asynchronous connections layer to release the port by DisconnectEvent(), then invalidates the port information, then returns an AV/C ACCEPTED response to the controller.

Transition C3b. After 10 seconds disconnection timeout, the consumer port transitions to the C0:FREE state. The node invalidates the port information.

Transition C3c. A bus reset returns the port to its initial C0:FREE state. The node invalidates the port information.

Transition C3d. When STATUS command with subfunction set to STATUS is requested, the node reports the current port information by generating an AV/C STABLE response frame.

Transition C3e. Other connection-management commands are rejected. The node returns an AV/C REJECTED response to the controller.

State C4:WAIT. In the WAIT (post reset) state the port is disabled (asynchronous connection requests are not accepted or generated). Only the connection management CONTROL command with subfunction set to RESTORE_PORT or RESTORE_PORT_FRAME, which re-establishes the nodeID value, is accepted. The intent is to temporarily maintain knowledge of pre-existing connections, so that the re-connections can be given precedence over new connection command sequences.

Transition C4a. The RESTORE_PORT or RESTORE_PORT_FRAME command transitions the node to the fully-connected C2:ACTIVE state, setting the connection count to one when this is done. The node stores the connect node ID values to the port information, then sets its connection count value to 1. Then the node requests an asynchronous connections layer to restart by RestoreEvent(). Although not illustrated, the oAPR register on the affiliated producer port is also updated, to re-activate asynchronous connection communications. Then, according to the msb of subfunction value which is stored as dr, the node returns an AV/C response frame (ACCEPTED or INTERIM) to the controller.

If the dr had been set to 1, the target stores the requested port information.

Transition C4b. After 10 seconds reconnection timeout, the consumer port transitions to the C0:FREE state. The node invalidates the port information.

Transition C4c. The RESUME_PORT command transitions the node to the C5:SUSPENDED state, if the suspendflg had been set to TRUE. The node stores the connect node ID values to the port information, then sets its connection count value to 1. Then the node requests an asynchronous connections layer to restart by ResumeEvent(). Although not illustrated, the oAPR register on the affiliated producer port is also updated, to re-activate asynchronous connection communications. The node returns an AV/C response frame (ACCEPTED) to the controller on success.

Transition C4d. When STATUS command with subfunction set to STATUS is requested, the node reports the current port information by generating an AV/C STABLE response frame.

Transition C4e. Other connection-management commands are rejected. The node returns an AV/C REJECTED response to the controller.

Transition C4f. A bus reset leaves the port in an unchanged state, but resets the post-reset timer values to 0.

State C5:SUSPENDED. In the SUSPENDED state the port is suspended (asynchronous connection is suspended to skip the frame transmissions). Only the connection management CONTROL command with subfunction set to RESUME_PORT, which re-enables the frame transmission, is accepted.

Transition C5a. The RESUME_PORT subfunction command sets the asynchronous connections layer to resume from the SUSPENDED state to ACTIVE state. The node requests to an asynchronous connections layer to resume by ResumeEvent(). The node returns an AV/C ACCEPTED response frame to the controller on success.

Transition C5b. When STATUS command is requested, the node reports the current port information by generating an AV/C STABLE response frame.

Transition C5c. Other connection-management commands are rejected. The node returns an AV/C REJECTED response to the controller.

Transition C5d. A bus reset transitions the port to the C4:WAIT state, in preparation of accepting the expected RESTORE_PORT subfunction command (this command restores the nodeID address of its connected plug). The node sets the flag that the state had been a SUSPENDED state, then sets its connect node ID value to FFFF16 (invalid node ID), and connection count to 0. The node sets timer value to 0.

#### A.13.3	Producer port states

##### A.13.3.1	Producer port state machines

The behaviour of a producer port is specified by its state machine definition, illustrated by the Figure A.15.

#### A.13.3.2	Producer port state machine notes

State P0:FREE. The FREE state is the initial state of the port, with no committed resources. While in the P0:FREE state, the purpose of the connection management commands is to transition the node to the P1:ACTIVE (connected) state. These state transitions are listed below.

Transition P0a. The ALLOCATE_ATTACH or ALLOCATE_ATTACH_FRAME subfunction command transitions the node to the connected to be P1:ACTIVE state. ALLOCATE_ATTACH or ALLOCATE_ATTACH _FRAME subfunction command frame includes the connection information such as consumer port address. The node requests to an asynchronous connections layer to start by AttachEvent(). Although not illustrated, the oAPR register on the affiliated producer port is also updated by the consumer, to activate asynchronous connection communications.

If the ex bit in a command frame had been set to 1, the target stores the nodeID of the controller to reject the following requests from other controllers.

Then according to the msb of subfunction value (stored as dr), the node returns an AV/C response frame (ACCEPTED or INTERIM) to the controller.

If the dr had been set to 1, the target stores the requested port information.

Transition P0b. When STATUS command is requested, the node reports the current port information by generating an AV/C STABLE response frame.

Transition P0c. All other connection management subfunctions are rejected, generating an AV/C REJECTED response frame.

Transition P0d. Bus resets while in the C0:FREE state leave the port in the same state.

State P1:ACTIVE. In the ACTIVE state the ports resources have been allocated and connected, in that the node can accept requests from its connected port. However, until this port's oAPR.run bit has been set by an updated from its connected port, this port remains disabled.

Transition P1a. The DETACH_RELEASE subfunction command, when the msb of subfunction value previously requested had been set to 0, transitions the node to a initial P0:FREE state. The node requests to an asynchronous connections layer to stop sending the data by DetachEvent(). The node invalidates the port information and return an AV/C ACCEPTED response frame to the controller.

Transition P1b. When the previous requested dr bit set to 0, the BREAK_ind indication (which indicates the unexpected disconnection) from the asynchronous connections layer transitions the node to a initial P0:FREE state. The node invalidates the port information.

Transition P1c. When the previous requested dr bit set to 1, the BREAK_ind indication (which indicates the unexpected disconnection) from the asynchronous connections layer transitions the node to a initial P0:FREE state. The node invalidates the port information and returns an AV/C REJECTED response frame to the controller.

Transition P1d. When the previous requested dr bit set to 1, the EndOfFrame_ind indication (which indicates the end of frame had been indicated from the producer port) from the asynchronous connections layer transitions the node to a initial P0:FREE state. The node requests to an asynchronous connections layer to break by BreakEvent().

Then the node invalidates the port information, and according to the mode value indicated from the asynchronous connections layer, returns an AV/C response frame to the controller. If mode value had been LAST, which means the normal end of frame, the AV/C response frame indicates ACCEPTED response, otherwise REJECTED response.

Transition P1e. When STATUS command is requested, the node reports the current port information by generating an AV/C STABLE response frame.

Transition P1f. All other connection management subfunctions are rejected, generating an AV/C REJECTED response frame.

Transition P1g. A bus reset transitions the port to the P2:WAIT state, in preparation of accepting the expected RESTORE_PORT or RESTORE_PORT_FRAME command (this command restores the nodeID address of its connected plug). The node sets its connect node ID value to FFFF16 (invalid node ID). The node sets timer value to 0.

State P2:WAIT. In the WAIT (post reset) state the port is disabled (asynchronous connection requests are not accepted or generated). Only the connection management CONTROL command with subfunction set to RESUME_PORT, which recovers the nodeID value of the consumer node, is accepted. The intent is to temporarily maintain knowledge of pre-existing connections, so that the re-connections can be given precedence over new connection command sequences.

Transition P2a. The RESUME_PORT or RESUME_PORT_FRAME command transitions the node to the connected P1:ACTIVE state. The node stores the connect node ID values to the port information. Then the node requests an asynchronous connections layer to restart by RestoreEvent(). Although not illustrated, the oAPR register on the affiliated producer port is also generated, to re-activate asynchronous connection communications. Then, according to the msb of subfunction value (stored as dr), the node returns an AV/C response frame (ACCEPTED or INTERIM) to the controller.

If the dr had been set to 1, the target saves the requested port information.

Transition P2c. When STATUS command with subfunction set to STATUS is requested, the node reports the current port information by generating an AV/C STABLE response frame.

Transition P2c. After 10 seconds reconnection timeout, the consumer port transitions to the P0:FREE state. The node invalidates the port information.

Transition P2d. Other connection-management command are rejected. The node returns an AV/C REJECTED response to the controller.

Transition P2e. A bus reset leaves the port in an unchanged state, but resets the post-reset timer values to 0.

Appendix I

Operational Scenarios (Informative)

The following subclauses provide possible scenarios for control operations. This Recommendation supports other methods.

## I.1	User-machine Control

The user-machine control system illustrated in Figure 15 can produce a number of user scenarios. Three simple scenarios are shown below as examples:

a)	DTV Control:

1)	The user presses a key on the remote control for the DTV.

2)	The application in the DTV responds to the key press by performing an action and possibly creating or updating on-screen graphical display data.

3)	The DTV mixes the bitmap with the displayed video.

4)	The user sees the result of his action on the display, confirming the action of the key press.

b)	STB OSD Control:

1)	The user selects the STB as the source device for the DTV using the DTV remote control and a source selection screen on the DTV.

2)	The user presses a key on the remote control for the STB.

3)	The application in the STB responds to the key press by performing an action and possibly creating or updating on-screen graphical display data.

4)	The bitmap is transmitted over the IEEE 1394 link to the DTV.

5)	The DTV mixes the bitmap with the displayed video.

6)	The user sees the result of his action on the display, confirming the action of the key press.

c)	STB Operational Control:

1)	The user selects the STB as the source device for the DTV using the DTV remote control and a source selection screen on the DTV.

2)	The user presses a key on the remote control for the STB.

3)	The application in the STB responds to the key press by transmitting compressed video over the IEEE 1394 link to the DTV.

4)	The DTV decodes the compressed video from the STB and presents it to the display.

5)	The user sees the new video, confirming the action of the key press.

## I.2	Program a Timed Recording Event on a DVCR using an internal tuner

1)	The user selects the DVCR as the video source for the DTV using the source selection screen of the DTV and the DTV's remote control.

2)	The user selects the menu system of the DVCR using the remote control of the DVCR. The DVCR's OSD is now displayed on the DTV.

3)	The user navigates through the DVCR's menu system to program a delayed recording event the same as is done today.

4)	DONE.

## I.3	Program a Timed Recording Event on a DVCR using an external tuner

1)	The user selects the DVCR as the video source for the DTV using the source selection screen of the DTV and the DTV's remote control.

2)	The user selects the menu system of the DVCR using the remote control of the DVCR. The DVCR's OSD is now displayed on the DTV.

	The user selects the source selection screen of the DVCR using the DVCR Remote Control.

	The user selects an external tuner (e.g. DTV, DBS, CSTB. Or ….) as the source for the DVCR.

	The user sets up a delayed recording event on the DVCR to record the stream from the 1394 input.

3)	The user selects the external tuner as the video source for the DTV using the source selection screen of the DTV and the DTV's remote control.

4)	The user chooses the program on the external tuner using the EPG and remote control associated with that tuner.

5)	Done.

### I.3.1	Recording a Program Received on a STB while being displayed on the DTV

1)	The user selects the DVCR as the video source for the DTV using the source selection screen of the DTV and the DTV's remote control.

2)	The user selects the menu system of the DVCR using the remote control of the DVCR. The DVCR's OSD is now displayed on the DTV.

3)	The user selects the source selection screen of the DVCR using the DVCR Remote Control.

4)	The user selects an external tuner (e.g. DTV, DBS, CSTB, or ....) as source for the VCR.

5)	The user selects the external tuner as the video source for the DTV using the source selection screen of the DTV and the DTV's remote control.

6)	The user chooses the program on the external tuner using the EPG and remote control associated with that tuner.

7)	The user sends a "Record" command to the DVCR using the DVCR remote control.

8)	DONE.

### I.3.2	Recording a Program Received from a second DVCR while being displayed on the DTV

1)	The user selects the DVCR1 as the video source for the DTV using the source selection screen of the DTV and the DTV's remote control.

2)	The user selects the menu system of the DVCR1 using the remote control of DVCR1. DVCR1's OSD is now displayed on the DTV.

3)	The user selects the source selection screen of DVCR1 using the DVCR1 Remote Control.

4)	The user selects DVCR2 as source for the DVCR1.

5)	The user selects DVCR2 as the video source for the DTV using the source selection screen of the DTV and the DTV's remote control.

6)	The user sends "Record" command to DVCR1 using the remote control of DVCR1.

7)	The user sends "Play" command to DVCR2 using the remote control of DVCR2.

8)	DONE.