|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

| ITU-T | J.117 |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU | (09/99) |

SERIES J: TRANSMISSION OF TELEVISION, SOUND PROGRAMME AND OTHER MULTIMEDIA SIGNALS

Interactive systems for digital television distribution

Home digital network interface specification

ITU-T  Recommendation  J.117

(Previously  CCITT  Recommendation)

ITU-T  J-SERIES  RECOMMENDATIONS

TRANSMISSION OF TELEVISION, SOUND PROGRAMME AND OTHER MULTIMEDIA SIGNALS

| For further details, please refer to ITU-T List of Recommendations. |  |
| --- | --- |
| General Recommendations | J.1–J.9 |
| General specifications for analogue sound-programme transmission | J.10–J.19 |
| Performance characteristics of analogue sound-programme circuits | J.20–J.29 |
| Equipment and lines used for analogue sound-programme circuits | J.30–J.39 |
| Digital encoders for analogue sound-programme signals | J.40–J.49 |
| Digital transmission of sound-programme signals | J.50–J.59 |
| Circuits for analogue television transmission | J.60–J.69 |
| Analogue television transmission over metallic lines and interconnection with radio-relay links | J.70–J.79 |
| Digital transmission of television signals | J.80–J.89 |
| Ancillary digital services for television transmission | J.90–J.99 |
| Operational requirements and methods for television transmission | J.100–J.109 |
| Interactive systems for digital television distribution | J.110–J.129 |
| Transport of MPEG-2 signals on packetised networks | J.130–J.139 |
| Measurement of the quality of service | J.140–J.149 |
| Digital television distribution through local subscriber networks | J.150–J.159 |

| ITU-T RECOMMENDATIONS SERIES |  |
| --- | --- |
| Series A | Organization of the work of the ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure |
| Series Z | Languages and general software aspects for telecommunication systems |



Printed in Switzerland

Geneva, 2000

ITU-T  RECOMMENDATION  J.117

HOME  DIGITAL  NETWORK  INTERFACE  SPECIFICATION

# Summary

The need to support cable services for High Definition TV (HDTV) sets, which are starting to emerge in the retail market, is converging with a general movement to interconnect multiple audio/visual (A/V) devices on a common bus or network. The IEEE 1394 interface has emerged as the preferred tool to accomplish this goal. Although the applications and their standards are still emerging, this Recommendation defines requirements and options for a 1394 digital interface between a Home Digital Network Device (HDND), which is a type of Set-Top Box (STB), and a Digital Television (DTV) receiver, with the Recommendation later extended to include a full set of networked devices in the home.  Clause 5 covers the interface from the HDND point of view.  Clause 6 covers the interface from the DTV point of view.  In the event of apparent conflict, the requirements in Clause 5 take precedence over Clause 6.

# Source

ITU-T Recommendation J.117 was prepared by ITU-T Study Group 9 (1997-2000) and was approved under the WTSC Resolution No. 1 procedure on 16 September 1999.

# FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL  PROPERTY  RIGHTS

The ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. The ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, the ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2000

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

# CONTENTS

Page

1	Scope		1

2	References		1

2.1	Normative reference list		1

2.2	Normative reference acquisition		2

2.3	Informative References		2

2.4	Informative document list		3

2.5	Informative document acquisition		3

3	Terms and definitions		3

4	Symbols and abbreviations		4

5	Home Digital Network Device Serial Bus Interface		7

5.1	Overview		7

5.2	Interface Specifications		8

5.2.1	Initialization and Configuration		8

5.2.2	AV/C Discovery Process		8

5.2.3	High-speed Serial Interface		9

5.3	Protocol Stack – Detailed Descriptions		15

5.3.1	Initialization		15

5.3.2	AV Protocols		17

5.3.3	Bitmap OSD Protocols		21

5.3.4	Internet Protocol (IP)		22

6	Home Digital Television Serial Bus Interface		22

Foreword		22

6.1	Introduction		22

6.2	General		25

6.2.1	Scope		25

6.2.2	Compliance Notation		25

6.3	System Overview		25

6.3.1	Digital TV Video Processing		25

6.3.2	On-Screen Display		25

6.3.3	Discovery of capability		25

6.3.4	User-machine control		25

6.3.5	Optional Analogue Audio and Video Processing		26

6.4	Miscellaneous Requirements		26

6.4.1	Digital Interface Physical layer		26

6.4.2	Digital Interface Link layer		26

6.4.3	Digital Interface Transaction layer		26

6.4.4	Serial bus management		26

6.4.5	Command and status registers		27

6.4.6	Real time transmission of MPEG-2 Transport Streams		27

6.4.7	Isochronous data flow management		27

6.4.8	Connection management procedures (CMP)		27

6.4.9	Asynchronous connection management		28

6.4.10	Function Control Protocol (FCP)		28

6.4.11	Analogue/Digital source selection		28

6.4.12	Content Advisory Information		29

6.5	Delivery of OSD Data		29

6.6	AV/C Command set support		29

6.6.1	AV/C General Commands		29

6.6.2	AV/C Asynchronous Connection Subfunctions		30

Page

6.7	OSD Data		30

6.7.1	Format of OSD data		30

6.7.2	Alignment of OSD with video		38

6.7.3	Behaviour of system during Attach		38

6.7.4	Behaviour of system during Suspend/Resume		39

6.8	Isochronous Data		39

6.8.1	Service Selection		39

6.8.2	MPEG-2 Transport Streams		40

6.8.3	DV Streams		41

6.9	Discovery Process		41

6.9.1	IEEE 1212 Configuration ROM		41

6.9.2	Configuration ROM Discovery		43

6.9.3	Unit Identifier Descriptor		44

Annex A – OSD Transport and Connection Management (Normative)		57

A.1	Introduction		57

A.1.1	Reserved fields		57

A.2	Data Transfers		57

A.3	Asynchronous plugs		57

A.4	Asynchronous plug components		58

A.4.1	Asynchronous plug spaces		58

A.4.2	Asynchronous plug components		59

A.4.3	Consumer-port address		60

A.4.4	Producer-port address		60

A.4.5	Segment-buffer address		60

A.5	Flow control registers		60

A.5.1	Register properties		60

A.5.2	Implemented registers		60

A.5.3	iAPR register		61

A.5.4	oAPR register		62

A.6	Data-access constraints		64

A.6.1	Segment buffer accesses		64

A.6.2	Control register accesses		64

A.7	Asynchronous communications		65

A.7.1	Frame transfer sequences		65

A.7.2	Segment transfer constraints		65

A.8	Flow control example		65

A.9	Serial bus transaction errors		66

A.10	AV/C managed connection sequences		67

A.10.1	Establishing an asynchronous connection		67

A.10.2	Breaking an asynchronous connection		67

A.10.3	Failure of establishing an asynchronous connection		67

A.10.4	AV/C reconnect timing		68

A.11	AV/C connection commands		69

A.11.1	AV/C connection management commands		69

A.11.2	Command Frames		70

A.11.3	Command frame values		70

A.12	AV/C Connection Command Format		73

A.12.1	ALLOCATE command		73

A.12.2	ALLOCATE_ATTACH command		74

A.12.3	ATTACH command		74

A.12.4	DETACH command		77

A.12.5	DETACH_RELEASE Command		77

A.12.6	RELEASE command		79

A.12.7	RESTORE_PORT command		80

A.12.8	SUSPEND_PORT command		83

A.12.9	RESUME_PORT command		83

Page

A.13	Asynchronous Connection plug states		85

A.13.1	Code definitions		85

A.13.2	Consumer port states		85

A.13.3	Producer port states		90

Appendix I – Operational Scenarios (Informative)		93

I.1	User-machine Control		93

I.2	Program a Timed Recording Event on a DVCR using an internal tuner		93

I.3	Program a Timed Recording Event on a DVCR using an external tuner		94

I.3.1	Recording a Program Received on a STB while being displayed on the DTV		94

I.3.2	Recording a Program Received from a second DVCR while being displayed on the DTV		94

# Introduction

This Recommendation describes a Home Digital Network Interface (HDNI) specification using the IEEE 1394 High Performance Serial Bus for networking between a Home Digital Network Device (HDND) such as a set-top box, and a Digital Television (DTV) receiver. Protection of program content is also provided against unauthorized copying during transmission across this interface. The IEEE 1394 High Performance Serial Bus offers bi-directional communication capability with a high data rate in excess of 200 Mbit/s on an interactive network of devices connected within the home.

One of the key advantages of the HDNI architecture is that all of the cable-specific protocols and interfaces are in the HDND. This permits the cable operator to support currently implemented systems, as well as promote special cable-based applications without changes to the consumer’s basic A/V equipment. In particular, the conditional access, network management, cable channel lineups, transport modulation, Internet transport technology, and other cable-specific technology are transparent to the delivery of the cable services using the set-top HDND connected to a DTV receiver via this interface.

Recommendation J.117

Recommendation J.117     (09/99)

HOME  DIGITAL  NETWORK  INTERFACE  SPECIFICATION

(Geneva, 1999)

# 1	Scope

The need to support cable services for High Definition TV (HDTV) sets is converging with a general movement to interconnect multiple audio/visual (A/V) devices on a common bus or network. The IEEE 1394 interface has emerged as the preferred tool to accomplish this goal. This Recommendation defines requirements and options for a serial baseband Home Digital Network Interface (HDNI). This interface can connect a Home Digital Network Device (HDND), which is a type of Set-Top Box (STB), with a Digital Television (DTV), that can decode and display an HDTV program. Protection of program content against unauthorized copying at this interface is addressed separately in Recommendation J.95. This Recommendation will later be extended to include a full set of networked devices in the home.

With this in mind, a set of profiles will be defined to extend the functionality of the On-Screen Display (OSD) incrementally for the Electronic Program Guide (EPG) and other application software running in the HDND. This version of the Recommendation will focus on Profile 0 for a directly connected DTV.

It is expected that one of the key advantages of an HDNI architecture is that all of the cable-specific protocols and interfaces are in the HDND. This permits the cable operator to support proprietary legacy systems, as well as promote special cable-based applications without changes to the consumer’s basic A/V equipment. In particular, the conditional access, network management, cable channel lineups, transport modulation, Internet transport technology, and other cable-specific technology are transparent to the delivery of the cable services using the HDNI. The cable operator’s electronic program guide as an integrated and up-to-date navigational environment presents the customer’s package of services.

This functional partitioning moves much of the processing currently done in the set-top box into the consumer’s device. For example, a DTV system can decode the compressed video and overlay the HDND graphical user interface onto the TV display, where both compressed video and graphics are received from the HDND over the connecting serial bus.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

## 2.1	Normative reference list

[1]	ATSC A/53, ATSC Digital Television Standard, 9/16/95.

[2]	IEEE 1394-1995, Standard for a High Performance Serial Bus.

[3]	IEEE P1394a, Draft Standard for a High Performance Serial Bus (Supplement), Draft 2.0, March 15, 1998.

[4]	AV/C Digital Interface Command Set General Specification, Version 3.0, April 15, 1998, 1394 Trade Association.

[5]	IEC 61883-1, Digital interface for consumer audio/video equipment – Part 1: General.

[6]	IEC 61883-4, Digital interface for consumer audio/video equipment – Part 4: MPEG-2 Transport Stream Data Transmission.

[7]	ITU-T Recommendation H.222.0 (1995) | ISO/IEC 13818-1:1996, Information Technology – Generic coding of moving pictures and associated audio information: Systems.

[8]	ITU-R Recommendation BT.709-2, Basic Parameter Values for the HDTV standard for the studio and for International Programme Exchange.

[9]	ITU-R Recommendation BT.601-4, Studio Encoding parameters of digital television for standard 4:3 and wide-screen 16:9 aspect ratios.

[10]	ISO/IEC 13213:1994, ANSI/IEEE Std 1212, Information technology – Microprocessor systems – Control and Status Registers (CSR) Architecture for microcomputer buses.

[11]	ATSC A/65, Program and System Information Protocol for Terrestrial Broadcast and Cable.

[12]	IEC 61883-3, Digital interface for consumer audio/video equipment – Part 3: HD-DVCR Data Transmission.

[13]	IEC 61883-2, Digital interface for consumer audio/video equipment – Part 2: SD-DVCR Data Transmission.

[14]	IEC 61883-5, Consumer audio/video equipment – digital interface – Part 5: SDL-DVCR Data Transmission.

[15]	ITU-T Recommendation J.95 (1999), Copy protection of intellectual property for content delivered on cable television systems.

## 2.2	Normative reference acquisition

ANSI/EIA Standards:

	Global Engineering Documents, World Headquarters, 15 Inverness Way East, Englewood, CO USA 80112-5776; Phone 800-854-7179; Fax 303-397-2740; Internet http://global.ihs.com; Email global@ihs.com

ANSI/EIA Standards: United States of America

	American National Standards Institute, Customer Service, 11 West 42nd Street, New York NY 10036; Phone 212-642 4900; Fax 212-302-1286; E-mail: sales@ansi.org; URL: <http://www.ansi.org>

ATSC Standards:

	Advanced Television Systems Committee (ATSC), 1750 K Street N.W., Suite 1200, Washington, DC 20006; Phone 202-828-3130; Fax 202-828-3131; Internet http://www.atsc.org

IEC Standards:

	Global Engineering Documents, World Headquarters, 15 Inverness Way East, Englewood, CO. USA 80112-5776; Phone 800-854-7179; Fax 303-397-2740; Internet http://global.ihs.com; Email global@ihs.com

SMPTE Standards:

	Society of Motion Picture & Television Engineers (SMPTE), 595 West Hartsdale Avenue, White Plains, NY 10607; Phone 914-761-1100; Fax 914-761-3115; Internet http://www.smpte.org; Email smpte@smpte.org

ITU Standards:

	ITU Sales and Marketing Service, International Telecommunication Union, Place des Nations CH-1211, Geneva 20, Switzerland; Phone +41 22 730 6141; Fax +41 22 730 5194; Internet http://www.itu.org; Email sales@itu.int

1394 Trade Association Documents:

	Contact the 1394 Trade Association, Regency Plaza, Suite 350, 2350 Mission College Blvd. Santa Clara, CA 95054; Phone 408-982-8289; Fax 408-982-8288; internet http://www.1394ta.org

## 2.3	Informative References

The following documents contain information that is useful for understanding this Recommendation. Some of these documents are drafts of standards that may become normative references in a future release of this Recommendation.

## 2.4	Informative document list

[16]	AV/C Compatible Serial Bus Connections, Version 1.0 FC1, November 6, 1998, 1394 Trade Association.

[17]	AV/C Commands for the Management of Asynchronous Serial bus Connections, Version 1.0 FC1, 10 November, 1998, 1394 Trade Association.

[18]	HDND 1394 Interface Specification, Cable Television Laboratories.

[19]	IEEE P1212r, Control and Status Registers (CSR) Architectures for microcomputer buses.

[20]	IPv4 over IEEE 1394 Internet-Draft Specification.

## 2.5	Informative document acquisition

OpenCable Specifications:

	Visit the OpenCable web-site at http://www.opencable.com.

1394 Trade Association Documents

	Contact the 1394 Trade Association, Regency Plaza, Suite 350, 2350 Mission College Blvd. Santa Clara, CA 95054; Phone 408-982-8289; Fax 408-982-8288; internet http://www.1394ta.org

IEEE P1212r:

	Downloadable from: http://www.zayante.com/p1212r.

IPv4 over IEEE 1394 Internet-Draft Specification:

	Downloadable from: http://www.ietf.org/internet-drafts/draft-ietf-ip1394-ipv4-16.txt.

# 3	Terms and definitions

This Recommendation defines the following terms:

3.1	allocate: The process of acquiring the resources, the address and other parameters of a plug for the purpose of establishing an asynchronous connection data transfer capability.

3.2	asynchronous connection: A point-to-point communication path established between a producer node and a consumer node, that supports robust high-bandwidth flow-controlled transfers of one or more data frames.

3.3	asynchronous push: A method of data delivery in which the node producing the data uses 1394 write transactions to deposit data into the address space of a consumer node.

3.4	attach: The process of communicating the address and other parameters of a plug to another plug for the purpose of establishing data transfer capability.

3.5	A/V: Audio and Video.

3.6	byte: 8 bits of data.

3.7	compareSwap4: A bus transaction that stores, at the specified address, a provided data value when the contents of the specified address is equal to a provided argument value. This operation is performed indivisibly on the addressed quadlet.

3.8	consumer: A device that accepts OSD data.

3.9	consumer port: A port that is the sink of data frames and is flow controlled by updates of its externally visible iAPR control register.

3.10	data frame (frame): A contiguous group of data bytes sent between producer and consumer nodes.

3.11	data segment (segment): A largest portion of a data frame that can be written into the segment buffer before updating the consumer’s iAPR register.

3.12	detach: The process of removing access to an asynchronous connection plug’s address space.

3.13	Digital Television (DTV): A device that receives, decodes and presents audio and video material that has been transmitted in a compressed form. The device can be a single unit or it can be constructed from a number of individual components (e.g. a digital terrestrial set top box and an analogue television).

3.14	iAPR: A register affiliated with an asynchronous connection, that indicates how much of data has been produced. This register also has other bits that are used for demarcation of variable-length frames, and to support the connection disconnection sequence.

3.15	oAPR: A producer-resident register affiliated with a segment buffer that is updated by the consumer to indicate how much data has been consumed. This register also has other bits that are used for demarcation of variable-length frames, and to support the connection disconnection sequence.

3.16	oAPR.count: An internal consumer-local register affiliated with a segment buffer, that indicates how much data has been consumed.

3.17	OSD Producer: A device that is the source of an OSD bitmap.

3.18	OSD Consumer: A device that receives an OSD bitmap for the purpose of presenting the information on a display device or storing the information for future use.

3.19	passive: The consumer plug is in this state when it accepts transactions directed at the plug’s address space but does not respond with updates to the producer’s registers.

3.20	plug: A collection of externally visible components (called ports) that can be connected to a sub-unit for the purposes of sending sequences of variable-length frames. There are three types of plugs, those associated with asynchronous connections, those associated with AV/C Isochronous Channels, and those associated with IEC 61883 Isochronous Channels.

3.21	port: A sub-component of an asynchronous connection plug that supports unidirectional asynchronous connection data transfers.

3.22	program: In MPEG-2 terminology, a collection of related elementary stream components making up a television service.

3.23	quadlet: Four bytes of data.

3.24	segment buffer: An externally visible address space on a consumer into which data is written by the connected producer.

3.25	source: A device that produces or passes on OSD data.

3.26	sub-unit: A uniquely identifiable and addressable entity contained within a unit.

3.27	Unit: The instantiation of an AV/C device. A unit is addressable in a specific way using AV/C commands. A unit may contain zero or more sub-units.

# 4	Symbols and abbreviations

This Recommendation uses the following abbreviations:

A/V	Audio/Video

ANSI	American National Standards Institute

API	Application Programming Interface

ATSC	Advanced Television Systems Committee

AV/C	Audio/Video Control

CLUT	Colour Look-Up Table

CIP	Common Isochronous Packet

CMP	Connection Management Procedures

CPTWG	Copy Protection Technical Working Group

CPU	Central Processing Unit

CSR	Control Status Register

CVCT	Cable Virtual Channel Table

DBS	Direct Broadcast Satellite

DTV	Digital Television

DVB	Digital Video Broadcasting

DVD	Digital Versatile Disk

EIA	Electronic Industries Alliance

EPG	Electronic Programme Guide

EUI	Extended Unique Identifier

GUI	Graphical User Interface

HAVi	Home Audio Video Interoperability

HDND	Home Digital Network Device

HTML	Hyper Text Markup Language

IEC	International Electrotechnical Commission

IEEE	Institute of Electrical and Electronics Engineers

ISO	International Organization for Standardization

LAN	Local Area Network

lsb	least significant bit

mbps	Megabits per second

MHP	Multimedia Home Platform

MPEG	Moving Picture Experts Group

msb	most significant bit

OSD	On-Screen Display

OUI	Organization Unique Identifier

PAT	Program Association Table

PCR	Plug Control Register

PID	Program Identifier

PIP	Picture-In-Picture

PMT	Program Map Table

PSIP	Program and System Information Protocol

SI	Service Information

SMPTE	Society for Motion Picture & Television Engineers

SPTS	Single Program Transport Stream

STB	Set-Top Box

TSID	Transport Stream ID

TVCT	Terrestrial Virtual Channel Table

# 5	Home Digital Network Device Serial Bus Interface

## 5.1	Overview

This interface represents a mid-term solution to supporting High Definition TV (HDTV) on cable. In the more immediate term, analogue interfaces and other approaches will be necessary to interface with the displays that have been designed or produced.

With this in mind, a set of profiles will be defined to extend the functionality of the On-Screen Display (OSD) incrementally for the Electronic Program Guide (EPG) and other application software running in the Home Digital Network Device (HDND). This version of the Recommendation will focus on Profile 0 for directly-connected HDTVs. A schematic of this architecture is shown in Figure 1 below.

It is expected that one of the key advantages of an HDND architecture is that all of the cable-specific protocols and interfaces are in the HDND. This permits the cable operator to support proprietary legacy systems, as well as promote special cable-based applications without changes to the consumer’s basic A/V equipment. In particular, the conditional access, network management, cable channel lineups, transport modulation, Internet transport technology, and other cable-specific technology are transparent to the delivery of the cable services using the HDND. The cable operator’s electronic program guide as an integrated and up-to-date navigational environment presents the customer’s package of services. This functional partitioning moves much of the processing currently done in the set-top box into the consumer’s device. This has the potential to produce a HDND that has a lower cost than the equivalent digital set-top for an NTSC TV.

The additional services, such as DVD- or PC-type service, are still accessible via the subscriber’s primary device or through the embedded functionality of the HDTV. It is anticipated that this will provide a rich environment for digital TV manufacturers and others to add functionality and provide product differentiation through their product lines. Most important, it will support a marketplace in which the consumer can pick and choose the level of performance in each of the A/V modules the consumer purchases.

NOTE 1 – A compatible HDTV shall support one or more analogue audio/video inputs, and allow an external device such as a set-top box the ability to select either a digital or an analogue audio/video source for display as covered in clause 6.

NOTE 2 – An external device may optionally supply user interface screens in analogue video format instead of bitmap OSD. Support for this approach is covered in clause 6.

## 5.2	Interface Specifications

This is a general specification for an HDND connected to an HDTV by a 1394 interface. Clause 6 defines a format and method of delivery for on-screen display (OSD) data. The HDND specification in 2.2.4 references clause 6 for the method of delivery of OSD data and the syntax and semantics of the subframes that define the OSD data. Also, clause 6 defines a data structure, the EIA Unit Identifier Descriptor, that allows the HDND to discover the capabilities of the HDTV (see 5.2.1).

On power-up and bus reset, the HDND shall query all other devices on the 1394 bus to collect the information in the Configuration ROM (as defined in the IEC 61883-1 and IEEE 1394 specifications) of each device. The HDND will build a device information table that will correlate the node ID and WWUID of every device on the bus. The following standard data structures will be supported by 1394 nodes:

### 5.2.1	Initialization and Configuration

#### 5.2.1.1	CSR core registers

CSR core registers shall conform to IEEE 1394-1995. The STATE_CLEAR.cmstr bit shall be implemented according to IEC 61883-1.

#### 5.2.1.2	Serial bus node registers

Serial bus node registers shall be implemented in conformance with IEC 61883-1.

#### 5.2.1.3	Configuration ROM requirements

The HDND and DTV shall implement the ROM format as defined in IEC 61883-1. Implementation requirements for Bus_info_block, Root_directory, and Unit_directory shall conform to IEC 61883-1.

##### 5.2.1.3.1	Bus_info_block entry

Implementation requirements for Bus_info_block in this Recommendation shall conform to IEC 61883.

##### 5.2.1.3.2	Root directory

Implementation requirements for Root_directory in this Recommendation shall conform to IEC 61883.

##### 5.2.1.3.3	Unit directory

Implementation requirements for Unit_directory in this Recommendation shall conform to IEC 61883. The Unit_sw_version shall indicate support for AV/C at minimum (the least significant bit of the third byte of Unit_sw_version shall be set to 1).

The EIA Unit_directory specified in clause 6 shall also be present, as required by EIA-775 for source audio/video devices compliant to that standard. The Unit_SW_Version shall be set to indicate the version of EIA-775 supported by the HDND (currently 1.0).

### 5.2.2	AV/C Discovery Process

The HDND shall query the DTV on power-up or bus reset to discover it’s OSD-related capabilities. The DTV will respond to the AV/C SUBUNIT INFO command and indicate (at minimum) that it has a Monitor Subunit, and it will indicate its ID (used for later reference).

The HDND shall use the AV/C OPEN DESCRIPTOR and READ DESCRIPTOR commands to retrieve the Unit Identifier Descriptor, described in 5.2.3.3.7.1.

### 5.2.3	High-speed Serial Interface

All devices conforming with this Recommendation will have at least one, and should have at least two 1394 ports.

The user should be made aware when connecting equipment that the best performance will be possible if a direct connection is made between DTV and HDND, or no devices supporting less than 200 Mbit/s bus speeds are present on the serial bus between HDND and display.

#### 5.2.3.1	Protocol Stack Overview

| Figure 2/J.117 – Protocol Stack for the HDNI serial bus Isochronous |  |  |  |  | Asynchronous |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Application Data Channel |  |  |  |  | OSD |  | Applications, AV/C, etc. | HTML, JavaScript, etc. |
| DVCR (61883) | MPEG (61883) | DSS | Other (61883) |  |  |  |  |  |
| 61883 – CIP |  |  |  |  |  | 61883 – FCP |  | UDP or TCP |
|  |  |  |  |  |  |  |  | IP |
|  |  |  |  |  | 1394 Transaction |  |  |  |
| 1394 Link |  |  |  |  |  |  |  |  |
| 1394 Physical |  |  |  |  |  |  |  |  |

Figure 2 shows the protocol stack defined for the HDNI serial bus. This figure shows the layers in the protocol stack and references the existing standards for 1394 data transport.

IEEE 1394-1995 "Standard for a High Performance Serial Bus" and IEEE P1394a specify the physical and link layer for this interface, and define the physical, link, and transaction layers and the bus management protocol for a high performance serial bus. At the time of this writing, it can be obtained from Global Documents.

IEEE P1394a defines compatible extensions to and clarifications of the IEEE 1394-1995 standard. These are normative and required for devices complying with this Recommendation. The physical layer, link layer, and transaction layer shall be as defined in the IEEE 1394-1995. In addition, devices that conform to this Recommendation must support speed code S200 (196 608 Mbit/s) or greater.

#### 5.2.3.2	A/V Communications Protocols

IEC 61883-1 defines a command/response protocol for delivering commands from a controller to another device over 1394 and connection management protocols for (AV/C) consumer devices plus some other features.

IEC 61883-4 defines the digital data transmission format for transmitting MPEG-2 over an isochronous channel on 1394. IEC 61883-1 and -4 are normative and required for devices complying with this Recommendation. The IEC 61883 Common Isochronous Packet (CIP) shall be used for isochronous data transfer, and IEC 61883 Function Control Protocol (FCP) shall be used for command transfers. The IEC 61883 connection management protocols shall be used.

IEC 61883-2, -3 and -5 define digital data transmission formats for transmitting DVCR format AV data over an isochronous channel on 1394. IEC 61883-2, -3 and -5 are normative but optional for devices complying with this Recommendation.

IEC 61883 is an International Standard and can be obtained from the IEC National Committees (e.g. ANSI in the United States) and other sales outlets.

#### 5.2.3.3	A/V Service Protocols

##### 5.2.3.3.1	MPEG TS

When a digital video service is selected, the HDND shall tune the appropriate MPEG multiplex, then select and (for access-controlled services) descramble the appropriate packets corresponding to the video, audio, private data, and control information needed by the display. These shall be transported over the 1394 interface as an isochronous channel using the IEC 61883-4 standard. This output of the HDND shall be an MPEG Single Program Transport Stream (SPTS) and as such does not require any generalized System/Service Information like ATSC A/65 (PSIP) or A/56 (SI). However, the SPTS does require Program Specific Information (PSI), particularly the Program Association Table (PAT) and Program Map Table (PMT). These two tables are specified in the MPEG Systems Layer.

In the general case, an MPEG-2 Transport Stream may contain multiple services. For the purposes of delivering the MPEG-2 TS to the DTV, the source device shall create a Single Program Transport Stream (SPTS). An SPTS is a valid MPEG-2 Transport Stream, but it contains just one MPEG-2 program. Clause 6 specifies the behaviour of the DTV when presented with an SPTS, and defines the algorithm it uses to select audio and video transport packets for decoding and display.

When processing MPEG programs, all elementary stream references shall be maintained unless a user option has de-selected an elementary stream. This provision ensures that downstream devices that may be present are afforded the option to process any parts of a digital service that has not been processed in the HDND. Note that selection of an audio language track in the HDND by the user may be considered to be an implicit de-selection of all other languages, hence they may be removed from the output SPTS. At minimum, "removal" means removal of elementary stream references from the PMT. Deletion of transport packets for the un-referenced streams may also be done.

The HDND may also support selection of an RF channel and sending the entire transport stream from that channel onto the 1394 interface. See clause 6 for additional information.

##### 5.2.3.3.2	Content Advisory (V-chip) Data

It is understood that the HDND will handle V-chip processing. However, the HDND shall not remove content advisory information that may be present in tuned programs when constructing an output SPTS.

##### 5.2.3.3.3	Digital Content Copy Protection Specification

The level of copy protection shall be determined based on the setting in the Copyright Content Information (CCI) field in the MPEG header. No requirement shall be inferred from this Recommendation that any particular copy protection system is implemented in the DTV.

NOTE – For information, in North America, Content Copy Protection shall be provided for MPEG TS based on the 5C Digital Transmission Content Specification which is described in Appendix III/J.95.

##### 5.2.3.3.4	Digital Interface Command Specification

The primary command interface with the HDTV (or other primary target display) connected to the HDND shall be through the OSD and set-top remote control. Use of the TV remote commands or TV messaging over the 1394 interface for OSD navigation and cable service access is not supported in Profile 0, but is anticipated to be included in later profiles.

The HDND shall support the 1394 Trade Association AV/C Digital Interface Command Set for communicating with other 1394 devices on the network.

The target display shall comply with the AV/C Digital Interface Command Set General Specification [4]. Support for those commands and command forms indicated as "mandatory" in the AV/C Specification shall be required for this Recommendation. Specifically, the HDND shall respond to the AV/C UNIT INFO and SUBUNIT INFO status commands.

The HDND shall report available subunits, although it shall not be required to support control by external devices of any subunit (control commands to subunits may be rejected by the HDND).

##### 5.2.3.3.5	Unit Info command

The target display shall respond to the AV/C UNIT INFO command, indicating that it is a DTV.

##### 5.2.3.3.6	Subunit Info command

The target display shall respond to the AV/C SUBUNIT INFO command with, at minimum, indication that a Monitor Subunit is present, and shall return its ID.

##### 5.2.3.3.7	Open and Read Descriptor commands

The HDND shall use the AV/C OPEN DESCRIPTOR and READ DESCRIPTOR control commands to the DTV, in accordance with clause 6 to learn its capabilities.

The target display shall support the AV/C OPEN DESCRIPTOR and READ DESCRIPTOR commands directed at the DTV unit. The DTV shall return a Unit Identifier Descriptor as described in the following subclause.

###### 5.2.3.3.7.1	HDND Unit Identifier Descriptor

The HDND shall respond to the unit-directed AV/C OPEN DESCRIPTOR and READ DESCRIPTOR status command with the Unit Identifier Descriptor specified in clause 6. The descriptor allows an external device (such as a DTV) to discover the plug IDs of the HDND’s digital, analogue, and OSD outputs.

###### 5.2.3.3.7.2	DTV Unit Identifier Descriptor

The HDND shall query the characteristics of the target display’s on-screen display (OSD) using the A/V SUBUNIT INFO and OPEN DESCRIPTOR and READ DESCRIPTOR commands. The HDND shall process the Unit Identifier Descriptor according to clause 6 to discover capabilities of the display and the plug ID values used for delivery of digital transport streams and OSD data.

This descriptor indicates the capabilities of the target display with regard to OSD and video handling. The Unit Identifier Descriptor reports capabilities including:

	OSD grid formats supported.

	OSD colour depths supported.

	Whether double buffering is supported.

	Video format conversion performed by the DTV for source video formats including 1920  1080, 1280  720, 704  480 (for both 4:3 and 16:9 display aspect ratios) and 640  480.

	The OSD vertical on-screen size and pixel aspect ratio for each of these source video formats.

##### 5.2.3.3.8	External Jack Selection

The HDND or other devices on the 1394 bus may request display of either a digital or analogue input to the target display. The HDND shall use the AV/C CONNECT command as specified in clause 6 to control analogue/digital input selection in the DTV. In the case that it has multiple baseband analogue audio/video inputs, the DTV is responsible for determining the appropriate analogue input when asked to select "analogue" by the HDND.

#### 5.2.3.4	Connection Management

The HDND shall conform to clause 6 with regard to rules governing the establishment and disconnection of isochronous channels and the asynchronous connections used for delivery of OSD data.

#### 5.2.3.5	On-Screen Display

A set of profiles for display-device OSD capabilities is defined to help set baseline functionality and define future directions. All future profile implementations must be backward compatible, supporting all prior profiles.

##### 5.2.3.5.1	Profile 0 (Normative)

Profile 0 is targeted to provide an equivalent level of OSD performance for HDTVs and digital devices when compared to set-tops that are connected to conventional analogue TVs. To accomplish this, bit maps will be transmitted over the 1394 interface to the target device to be composited on the decoded MPEG or NTSC video image.

NOTE – For information, Profile 0 is required in North America for all "cable ready" displays.

Some DTVs compliant with clause 6 will not support composition of OSD onto analogue video. The HDND should be able to accommodate such DTVs by performing this composition itself.

The OSD bitmap transmission format defined in the following subclauses supports various OSD modes, including:

	640  480  4 OSD grid, with a 4-bit to 16-bit Colour Look-Up Table (CLUT)

	640  480  8 OSD grid, with an 8-bit to 16-bit CLUT

	640  480  16 OSD grid (component colour, no CLUT compression)

	Pixel component coding including:

–	Y-Cb-Cr-alpha 6:4:4:2, where each pixel is transparent, opaque, or an alpha blend value defined per screen;

–	Y-Cb-Cr-alpha 6:3:3:4, where each pixel is transparent, opaque, or the alpha value given by the pixel’s 4-bit alpha value;

–	Y-Cb-Cr 6:5:5, where all pixels are opaque;

	profile 0 is subdivided into two sub-profiles, 0a and 0b, according to Pixel component coding including:

–	Y-Cb-Cr-alpha 6:4:4:2, where each pixel is transparent, opaque, or an alpha blend value defined per screen;

–	Y-Cb-Cr-alpha 6:3:3:4, where each pixel is transparent, opaque, or the alpha value given by the pixel’s 4-bit alpha value;

–	Y-Cb-Cr 6:5:5, where all pixels are opaque.

In Table 1 the HDND may discover whether a given display supports Profile 0a or 0b upon processing the Unit Identifier Descriptor.

Note that some devices supporting Profile 0a may be able to offer one or more (but not all) advanced features from the Profile 0b list. The HDND will be able to discover in detail via the Unit Identifier Descriptor what graphics modes and colour depths are available, and may be able to take advantage of any offered.

The video scaling and positioning feature is described in 5.2.3.5.4.

Table 1/J.117 – Capability Profiles

| Capability | Profile 0a | Profile 0b |
| --- | --- | --- |
| 640  480  4 OSD grid, 4-bit to 16-bit CLUT format: |  |  |
| Y-Cb-Cr-alpha 6:4:4:2, transparent, opaque or per-screen alpha value | ü | ü |
| Y-Cb-Cr-alpha 6:3:3:4, transparent, opaque or alpha value per pixel |  | ü |
| 640  480  8 OSD grid, 8-bit to 16-bit CLUT format: |  |  |
| Y-Cb-Cr-alpha 6:4:4:2, transparent, opaque or per-screen alpha value |  | ü |
| Y-Cb-Cr-alpha 6:3:3:4, transparent, opaque or alpha value per pixel |  | ü |
| Y-Cb-Cr 6:5:5 |  | ü |
| 640  480  16 OSD grid, pixel format: |  |  |
| Y-Cb-Cr-alpha 6:4:4:2, transparent, opaque or per-screen alpha value |  | ü |
| Y-Cb-Cr-alpha 6:3:3:4, transparent, opaque or alpha value per pixel |  | ü |
| Y-Cb-Cr 6:5:5 |  | ü |
| Video scaling/positioning |  | ü |

###### 5.2.3.5.1.1	OSD Bitmap Pixel Format

The frame buffer shall support OSD bit maps of 640  480 pixels. Each pixel will be represented by 16 bits of component colour (luminance and chrominance) and alpha overlay transparency information. The sampling density and location of each of these pixel components are identical and coincident. Many displays may also be capable of 14:9 or 16:9 resizing of NTSC video stretched to full screen. In a similar manner the display may be capable of scaling the 640  480 bit map into an appropriate 14:9 or 16:9 full screen overlay.

The baseline format for 16 bit pixels shall be 6:4:4:2 (Y-Cb-Cr-alpha). Optional formats for 16 bit pixels shall be 6:5:5 and 6:3:3:4.

The OSD will be vertically and horizontally centered on the target display.

###### 5.2.3.5.1.2	OSD Bitmap Pixel Transport

OSD data shall be delivered via the "asynchronous push" method, with connection management and flow control in accordance with methods defined in clause 6. Data to be transferred is organized into frames and subframes. For this application, a number of different subframe types are defined, each functioning to establish OSD format or encoding, or to deliver actual OSD data.

For increased transmission efficiency, a 16- or 256-entry Colour Look Up Table (CLUT) that contains 16-bit colour pixel data shall be implemented, thereby enabling each pixel in an OSD bit map to be encoded and transmitted by a 4- or 8-bit index operand.

CLUT entries may be transferred from the HDND to the target display at any time. The most recently loaded CLUT will remain active until it is updated by the HDND. An initial CLUT must be loaded from the HDND to the target display before any index operands can be transferred to the target display for OSD image conversion and display.

###### 5.2.3.5.1.3	OSD Pixel Syntax And Semantics

The HDND shall send OSD data conforming to clause 6. This Recommendation defines the following subframe types:

Set_OSD_pixel_format: Establishes the display mode and format of the basic 16 bit pixels that make up the data definition to follow, and the size and colour depth of the OSD grid. When the pixel format is initially defined, or any time it changes, the display initializes the display buffers of the target display and fills them with a constant pixel value defined in the subframe. For OSD grid formats with 4- or 8-bit colour depths, the subframe contains a 4- or 8-bit Colour Look-Up Table (CLUT).

4_bit_OSD_data: Defines 4-bit pixels in a rectangular region. Each 4-bit pixel represents a colour/alpha blend value derived by indirection through the 4-bit CLUT.

8_bit_OSD_data: Defines 8-bit pixels in a rectangular region. Each 8-bit pixel represents a colour/alpha blend value derived by indirection through the 8-bit CLUT.

Uncompressed_16_bit_data: Defines raw uncompressed 16-bit OSD data in a rectangular region.

Fill_region_with_constant: Defines a rectangular region to fill with a 16 bit constant.

Clear_OSD: Causes the display device to clear the OSD screen.

###### 5.2.3.5.1.4	Video Frame Buffer

The maximum theoretical data transfer rate for the bit maps is approximately 50 Mbit/s for 200 Mbit/s interfaces. 640  480  16  60 Hz would require 295 Mbit/s bandwidth if every frame changed completely. At 50 Mbit/s, we can change 1/6 of the screen every frame and maintain a 60 Hz display. While this is not perfect, we believe this is the best tradeoff between screen resolution, pixel depth, picture complexity, and frame rate.

The HDND and display shall support rectangular region based updating.

The DTV may offer double buffering of OSD. If double buffering is available, OSD data may be directed to the "off screen" buffer by a control bit in the subframe. Another control bit may be used by the HDND to direct the DTV to swap the off screen with the on-screen buffer following processing the data in the subframe, and time the buffer swap to coincide with vertical retrace on the output scan.

##### 5.2.3.5.2	Profile 1 (Informative)

This profile is for discussion purposes only and will be specified in detail in a later version. The current thinking would be to support a higher resolution bit mapped OSD with 960  640  16 (6:3:3:4) and compression. We anticipate doing an RFI soon to determine what compression scheme would be appropriate (e.g. simple, lossless, hardware accelerated) for use with HDTVs.

Arbitrary video resizing with high quality filtering would be desirable for use in value-added consumer displays. Video resizing and positioning for EPG support would also be incorporated.

HDND reception and processing of baseline HDTV remote control commands, mandatory double buffering for OSDs, PCM audio for OSDs, and alternate video (PIP) would all be required in this profile.

##### 5.2.3.5.3	Profile 2 (Informative)

This profile is for discussion purposes only, and will be specified in detail in a later version. This profile could support a presentation engine or browser in the display rather than use bit maps. Examples of this currently under development include DVB-MHP and ATSC broadcast APIs, and the HAVi Level 2 GUI. This would reduce the data traffic on the 1394 interface at the expense of the potential loss of pixel-by-pixel control of the display. Some of the likely core software elements might be HTML 4.0, ECMA Script (JavaScript) and Personal Java. The software stack would need to pass a yet-to-be-defined conformance test and support browser authentication by the HDND.

##### 5.2.3.5.4	Video Scaling and Positioning

The OSD generator potentially will want to place the video inside its graphics rendered content. Examples of this include shrinking the video display into the corner of the display and wrapping the EPG grid around it, or placing video in a web page. The scaling information is derivable from the size. Some devices will only support limited resizing, and shall do closest larger fit.

According to AV/C design methodology, a DTV unit includes a functional block called the Monitor Subunit. The general Monitor Subunit supports multiple analogue or digital audio/video inputs. The Monitor Subunit specification models video processing functions including scaling and positioning, and defines the AV/C commands used to control processing and display functions in the DTV.

At the time of this writing, the Monitor Subunit specification is under development within the 1394 Trade Association. The A/V Working Group plans to finalize the AV/C Monitor Subunit Model and Command Set specification in January 1999.

Support for the scaling and positioning feature also involves a discovery process. The Monitor Subunit specification describes the Monitor Subunit Identifier Descriptor, which allows an HDND to discover DTV capabilities and limitations related to the scaling and positioning feature.

When the AV/C specification for the Monitor Subunit is finalized by the 1394 Trade Association, this HDND specification may be revised to reference it.

##### 5.2.3.5.5	User Input Devices

###### 5.2.3.5.5.1	Wireless Remote Interface

In Profile 0, the HDND shall have a universal remote control. The OSD and application software as appropriate will use the remote commands programmed for the HDND. Remote commands programmed for the TV will be ignored by the HDND.

It is anticipated that use of the TV remote commands and/or device messaging over the 1394 interface will be supported in profile 1.

###### 5.2.3.5.5.2	Other I/O devices (Keyboards, mice, …)

These are still under development and are not defined in this Recommendation.

###### 5.2.3.5.5.3	DVCR, Digital Audio Component Support

These are still under development and are not defined in this Recommendation.

#### 5.2.3.6	IP Over 1394

Defined in Ref. [20].

##### 5.2.3.6.1	DOCSIS Support

These are still under development and are not defined in this Recommendation.

##### 5.2.3.6.2	PacketCable Support

These are still under development and are not defined in this Recommendation.

## 5.3	Protocol Stack – Detailed Descriptions

This subclause describes protocol stacks for the interaction between an AV source device (e.g. Digital VCRs, Cable STBs, etc.) and a DTV (Digital TV) over IEEE 1394. These protocol stacks are categorized into the following:

	Initialization;

	AV Protocols;

	Bitmap OSD Protocols;

	Internet Protocol (IP).

### 5.3.1	Initialization

#### 5.3.1.1	Initialization Protocol Stack

The protocol stack for initialization is defined in Figure 3.

The initialization process consists of two applications:

1)	1394 node discovery application; and

2)	Subunit Identifier descriptor discovery application.

The 1394 Node discovery application should be invoked at IEEE 1394 bus reset, including at device power-up. The 1394 Node discovery application will query all other devices on the 1394 bus to collect the information in the Configuration ROM of each device, as defined in 6.9.2. This application will build a device information table that will correlate the node ID and WWUID of every device on the bus.

The Subunit identifier descriptor discovery application should be invoked at the device power-up. It may be invoked at IEEE 1394 bus reset. The Subunit identifier descriptor discovery application will query the current configuration of the target display’s capabilities, including target display’s OSD (On-Screen-Display), using the AV/C READ DESCRIPTOR Command. In future profiles, Subunit identifier descriptor discovery application can query the HDND’s capabilities using the AV/C READ DESCRIPTOR Command.

| Figure 3/J.117 – Protocol stack for initialization 1394 Node Discovery Application |  | Subunit Identifier Descriptor Discovery Application |
| --- | --- | --- |
|  |  | Command and Control (AV/C read descriptor Command) |
|  |  | Function Control Protocol – FCP (IEC 61883-1) |
| 1394 Serial Bus Management (Configuration ROM, CSR) | 1394 Transaction Layer |  |
|  | 1394 Link Layer |  |
|  | 1394 Physical Layer |  |

#### 5.3.1.2	Description of Specific Protocols

This section describes the specific protocols used in the protocol stacks identified in the section above.

##### 5.3.1.2.1	1394 Physical Layer

Defined in IEEE 1394-1995 and IEEE P1394a.

The devices conforming with this Recommendation shall support speeds of S200 (196 608 Mbit/s) or greater. The choice of using a four (4) pin connector or six (6) pin connector is not specified. Devices conforming with this Recommendation will have least one, and should have at least two 1394 ports implemented.

##### 5.3.1.2.2	1394 Link Layer

Defined in IEEE 1394-1995 and IEEE P1394a.

The devices conforming with this Recommendation shall support both Asynchronous packet transmission and Isochronous packet transmission. Devices capable of sourcing isochronous data shall be Cycle Master Capable.

##### 5.3.1.2.3	1394 Transaction Layer

Defined in IEEE 1394-1995 and IEEE P1394a.

The devices conforming with this Recommendation shall be Asynchronous transaction capable.

##### 5.3.1.2.4	1394 Serial Bus Management

Defined in IEEE 1394-1995 and IEEE P1394a.

Regarding Command and Status Registers (CSRs), the implementation of both CSR Architecture core registers and Serial-Bus-Dependent registers shall conform with IEC 61883-1.

Regarding Configuration ROM, the implementation of Bus_info_Block, root_directory and unit_directories shall conform with IEC 61883-1.

The support of Isochronous resource manager capability is recommended. The support of bus manager capability is optional.

##### 5.3.1.2.5	Function Control Protocol (FCP)

Defined in IEC 61883-1.

The devices conforming with this Recommendation shall implement a Command Register and a Response Register as the target address space of command frame and response frame, respectively. The register address, frame structure and CTS (Command/Transaction Set) value shall conform with IEC 61883-1.

##### 5.3.1.2.6	Command and Control (AV/C READ DESCRIPTOR Command)

Defined in AV/C Digital Interface Command Set General Specification, Version 3.0.

The devices conforming with this Recommendation should support the AV/C READ DESCRIPTOR command to issue or respond to queries using the Subunit identifier descriptor. The Unit Identifier Descriptor for the display device is defined in clause 6 and referenced in 5.2.3.3.7.2.

### 5.3.2	AV Protocols

#### 5.3.2.1	AV Protocol Stacks

This subclause describes the protocol stacks for AV streams.

##### 5.3.2.1.1	MPEG TS Content Flow

The protocol stack for MPEG TS Content Flow is defined in Figure 4.

MPEG TS content flow contains the real time MPEG audio and video Elementary Stream, and MPEG-2 Program Specific Information. Single Program TS (SPTS) is used.

| Figure 4/J.117 – Protocol Stack for MPEG TS Content Flow Real Time MPEG Audio and Video Elementary Streams |  |
| --- | --- |
| MPEG-2 Packetized Elementary Stream (PES) | MPEG-2 Program Specific Information |
| MPEG-2 Transport Stream (Single Program TS) |  |
| Real Time Data Transmission Protocol (IEC 61883-1 and 4) |  |
| 1394 Link Layer |  |
| 1394 Physical Layer |  |

##### 5.3.2.1.2	AV Stream Control

The protocol stack for AV stream control is defined in Figure 5.

AV Stream Control consists of one application, the Isochronous data flow management application.

Isochronous data flow management application establishes/releases the logical connection called Isochronous Connection between the source device and the destination device of an AV stream.

Figure 5/J.117 – Protocol Stack for AV Stream Control Isochronous Data Flow Management Application

Connection Management Procedures – CMP (IEC 61883-1)

1394 Transaction Layer

1394 Link Layer

1394 Physical Layer

##### 5.3.2.1.3	Channel Change Control

The protocol stack for channel change control is defined in Figure 6.

Channel change control to select channel is optional for future profiles. The assumed situation is that the IR (infra-red) remote is directed at the DTV or other device, and is not directed at the HDND. This protocol forwards the selected channel information to OC-STU Subunit from the device with IR remote.

| Figure 6/J.117 – Protocol stack for Channel Change Control (Optional for future profiles) Channel Change Control Application |  |
| --- | --- |
| Command and Control (AV/C Tuner Command) | Universal Remote Command |
| Function Control Protocol – FCP (IEC 61883-1) |  |
| 1394 Transaction Layer |  |
| 1394 Link Layer |  |
| 1394 Physical Layer |  |

##### 5.3.2.1.4	Inter OC-STU Subunit Communication

The protocol stack for inter OC-STU Subunit communication is defined in Figure 7.

Inter OC-STU Subunit communication is optional for future profiles. It is to exchange information among two or more OC-STU Subunits. The assumed information content is the current subscribed program. This is utilized when OC-STU Subunits negotiate to avoid duplicated isochronous streams for a program when multiple DTVs ask to subscribe the same program.

Figure 7/J.117 – Protocol stack for Inter OC-STU Subunit Communication (Optional for future profiles) Inter OC-STU Communication Application

Command and Control (AV/C OC-STU Command; TBD)

Function Control Protocol – FCP (IEC 61883-1)

1394 Transaction Layer

1394 Link Layer

1394 Physical Layer

##### 5.3.2.1.5	OC-STU Subunit and the other device Communication

The protocol stack for OC-STU Subunit and other device communication is defined in Figure 8.

OC-STU Subunit and other device communication is optional for future profiles. It is to exchange information between an OC-STU Subunit and other devices on the 1394 bus.This protocol stack is employed when some control entity located in a device on the 1394 bus wishes to control the OC-STU Subunit. This entity can be located in a DTV or other device, like a PC.

Figure 8/J.117 – Protocol stack for OC-STU Subunit and the other device Communication (Optional for future profiles) OC-STU and the other device Communication Application

Command and Control (AV/C OC-STU Command; TBD)

Function Control Protocol – FCP (IEC 61883-1)

1394 Transaction Layer

1394 Link Layer

1394 Physical Layer

#### 5.3.2.2	Description of Specific Protocols

This subclause describes the specific protocols used in the protocol stacks identified in the subclause above.

##### 5.3.2.2.1	Real-time Data Transmission Protocol

Defined in IEC 61883-1 and -4.

This protocol defines the method to transmit MPEG TS over 1394 using an Isochronous channel.

MPEG TS packets are transmitted within the Common Isochronous Packet (CIP) structure defined in IEC 61883-1.

At the source side, each MPEG TS transport packet is encapsulated in one or more CIPs. The process is as follows:

	Source Packet assembly – A source packet is formed from an MPEG TS transport packet by adding a 4-byte time-stamp.

	Data Block assembly – A data block is formed by segmenting a the 192-byte source packet into 8 data blocks, each with a length of 6 quadlets.

	CIP assembly – A CIP payload is formed from one or more data blocks, taking account of both the MPEG TS encoding data rate and the bandwidth of 1394. A CIP header is added to the payload.

At the destination side, an MPEG TS is extracted from the received CIP(s).

The Packet format and the header information setting shall conform with IEC 61883-1 and -4.

##### 5.3.2.2.2	MPEG-2 Transport Stream

Defined in ISO/IEC 13818 MPEG-2 Part 1.

Single Program TS (SPTS) shall be supported. Multiple program TS may be used.

##### 5.3.2.2.3	MPEG-2 Packetized Elementary Stream (PES)

Defined in ISO/IEC 13818 MPEG-2 Part 1.

##### 5.3.2.2.4	MPEG-2 Program Specific Information (PSI)

Defined in ISO/IEC 13818 MPEG-2 Part 1. PSI will include, at a minimum, the Program Association Table (PAT) and Program Map Table (PMT).

##### 5.3.2.2.5	Real-time MPEG Audio and Video Elementary Streams

Defined in ISO/IEC 11172 MPEG-1 Parts 2 and 3, and ISO/IEC 13818 MPEG-2 Parts 2 and 3.

##### 5.3.2.2.6	Connection Management Procedures (CMP)

Defined in IEC 61883-1.

CMP are used to establish, overlay and break an isochronous connection that is transmitting an AV stream. The connection is from the source device to the destination device. CMP are accomplished by handling the Plug Control Registers located in a source device and a destination device of the connection.

##### 5.3.2.2.7	Command and Control (AV/C CONNECT Command)

Defined in AV/C Digital Interface Command Set General Specification, Version 3.0.

AV/C CONNECT command is used to establish connections within a device.

##### 5.3.2.2.8	Command and Control (AV/C TUNER Command)

Implemented in future profiles.

Defined in AV/C Tuner Model and Command Set Specification, Version 1.0.

AV/C TUNER command is used to change the channel. The specific usage is still under development and is not defined in this Recommendation.

##### 5.3.2.2.9	Universal Remote Command

Implemented in future profiles.

These are still under development and are not defined in this Recommendation.

Universal remote command is used to change the channel.

##### 5.3.2.2.10	Command and Control (AV/C OC-STU Subunit Command)

Implemented in future profiles.

These are still under development and are not defined in this Recommendation.

AV/C OC-STU subunit command set should be defined according to the AV/C General Specification.

The definition of the OC-STU subunit and its command set are still under development and are not defined in this Recommendation.

#### 5.3.2.3	Higher Layer Protocols

Implemented in future profiles.

These are still under development and are not defined in this Recommendation.

For higher layers, additional protocols will be defined supporting API’s implementing script based presentation engines such as HTML, Javascript, and personal Java to facilitate GUIs such as HAVi Level 2, and DVB-MHP and ATSC broadcast APIs.

### 5.3.3	Bitmap OSD Protocols

#### 5.3.3.1	OSD Data Transmission

The protocol stack for OSD data transmission is defined in Figure 9.

The OSD data transmission realizes the generation and the drawing of graphics data in distributed fashion. The source side feeds bitmapped region data and the destination side draws the received data into its graphics memory.

The format of OSD frames and subframes is defined in clause 6.

Figure 9/J.117 – Protocol Stack for OSD Data Transmission Graphics Application

OSD Frames

OSD Subframes

1394 Transaction Layer

1394 Link Layer

1394 Physical Layer

#### 5.3.3.2	OSD Flow Control

The flow of OSD data is managed in accordance with the mechanisms defined in clause 6.

#### 5.3.3.3	OSD Connection Management

The protocol stack for the management of the OSD connection for subframe transmission is defined in Figure 10.

OSD connection management realizes the establishment/release of the logical connection for OSD subframe transmission, and the start/stop of OSD subframe transmission. This is realized with the cooperation of the Graphics application. This application is invoked by the IR remote of the HDND, or the IR remote of the DTV or other devices on the 1394 bus.

The AV/C commands used to establish and disconnect OSD are defined in clause 6.

Figure 10/J.117 – Protocol Stack for OSD Connection Setup OSD Application

ASYNCHRONOUS CONNECTION command subfunctions

Function Control Protocol – FCP (IEC 61883-1)

1394 Transaction Layer

1394 Link Layer

1394 Physical Layer

### 5.3.4	Internet Protocol (IP)

#### 5.3.4.1	IP Protocol Stack

The protocol stack for Internet Protocol (IP) is defined in Figure 11.

This protocol stack realizes the terminal or router function for Internet related protocols.

Figure 11/J.117 – Protocol stack for Internet Protocol High Layer Protocols (for Terminal, Router)

TCP, UDP

IP

IP over IEEE 1394 (IETF Internet Draft)

1394 Transaction Layer

1394 Link Layer

1394 Physical Layer

#### 5.3.4.2	Description of Specific Protocols

This subclause describes the specific protocols used in the protocol stacks identified in the subclause above.

##### 5.3.4.2.1	IP over IEEE 1394

Defined by IETF in [20].

##### 5.3.4.2.2	IP, TCP, UDP, and High Layer Protocols

Defined by IETF in [20].

# 6	Home Digital Television Serial Bus Interface

## Foreword

Users of this Recommendation should be aware that ongoing standardization work in the 1394 Trade Association1 may have a future impact on this Recommendation. The EIA has stated its intention to harmonize its standard with those developed within the 1394 Trade Association, and likewise the TA has indicated its willingness to coordinate standards development with EIA.

Users of this Recommendation should also note that, at some future point, copy protection parameters, methods and/or standards are expected to be established with which copy-protected content traversing the DTV 1394 interface will be required to comply. The EIA has stated its intention to harmonize its standard with those developed for this interface.

## 6.1	Introduction

The currently implemented analogue audio/video home entertainment cluster consists of various signal sources and various display devices. Possible audio/video sources in this system include a video cassette recorder, a DVD player, and a DBS or cable set top box. In the analogue system the audio/video source can overlay its graphical user interface on its output video as shown in Figure 12. This allows the user to control the source based on information shown on the TV display.

A Digital Television (DTV) system using a similar model is not practical. Figure 13 shows an example of what would be required. The process of decoding the original bitstream to include the GUI overlay and then re-encoding for transmission to the DTV adds prohibitive cost and degrades picture quality.

This Recommendation defines a specification for a baseband digital interface to a DTV that provides a level of functionality that is similar to the analogue system. A representation of a typical system is shown in Figure 14. This diagram shows an A/V Source that is capable of producing analogue audio and video and also an MPEG Transport Stream. The analogue signal is transmitted over a standard coaxial cable and the MPEG data is passed over an IEEE 1394 bus. Bitmaps of the source OSD are sent separately over the same IEEE 1394 bus and are mixed with the decoded MPEG video in the DTV prior to being presented on the display. This process removes the need for the A/V Source to perform the additional MPEG video decodes and re-encodes. The IEEE 1394 bus is also used by the source and display to exchange control and status messages.

The digital interface is based on the IEEE 1394 Standard for a High Performance Serial Bus [2], the IEC 61883-1 digital interface standard [5], and on the AV/C Digital Interface Command Set General Specification [4]. The IEEE 1394 Standard originated in Apple Computer as a multimedia interconnect. The standard provides:

	Data transmission at various data rates. This Recommendation requires support for the s200 or higher level of service defined in IEEE 1394-1995 (s200 has a bit rate of 196 608 Mbit/s).

	Inexpensive and physically small interconnection.

	Hot pluggable connection.

	Daisy chained or branched interconnection.

	Guaranteed bandwidth for isochronous data.

This Recommendation is designed to enable interoperability between a DTV compliant with this Recommendation and various types of consumer digital audio/video sources including digital Set-Top Boxes (STBs) and analogue/digital hard disk or Videocassette Recorders (VCRs). This Recommendation defines a level of functionality that will allow compliant systems to:

	Discover and adapt to OSD functionality supported in the display device.

	Pass an MPEG-2 Transport Stream from the bitstream source to the display device.

	Transmit OSD information from the OSD Producer to the display device or OSD Consumer.

	Control the selection of different sources to the display device.

## 6.2	General

### 6.2.1	Scope

This Recommendation includes mechanisms to allow a source of MPEG service, such as a cable or terrestrial set-top box, digital VCR, or DTV to utilize the MPEG decoding and display capabilities in a DTV. A method is included to allow the OSD Producer to supply bitmap graphic overlays for blending and composition in the DTV over decoded video.

This Recommendation supports an optional baseband analogue audio/video connection between an audio/video source device and the DTV. Mechanisms are provided to allow the source device to control the selection of the audio/video source for display in the DTV between an MPEG service decoded in the DTV and incoming analogue audio/video supplied to it via an external input.

Nothing in this Recommendation is intended to constrain the use of other 1394-based protocols.

### 6.2.2	Compliance Notation

As used in this Recommendation, "shall" and "must" denote a mandatory provisions of the Recommendation. "Should" denotes a provision that is recommended but not mandatory. "May" denotes a feature whose presence does not preclude compliance, that may or may not be present at the option of the implementor. "Optional" denotes items that may or may not be present in a compliant DTV.

## 6.3	System Overview

A simple system implementation has already been shown in Figure 14. In this simple system there is one source of A/V signals and one DTV display. This Recommendation supports more complicated systems having multiple sources and multiple DTV displays.

In the simple implementation there are four separate conduits for the exchange of data between the source and display.

1)	Analogue Audio and Video.

2)	Digital Audio and Video.

3)	Control and Status.

4)	OSD.

Each one of these can initially be considered separately but will affect each other in the display.

1 	EIA-775, DTV 1394 Interface Specification, December, 1998.