|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | J.123 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (07/2002) |
|  | SERIES J: CABLE NETWORKS AND TRANSMISSION OF TELEVISION, SOUND PROGRAMME AND OTHER MULTIMEDIA SIGNALS Interactive systems for digital television distribution |  |  |  |
|  | Multiplexing format for webcasting on the TCP/IP network |  |  |  |
|  | ITU-T Recommendation J.123 |  |  |  |

ITU-T J-SERIES  RECOMMENDATIONS

CABLE NETWORKS AND TRANSMISSION OF TELEVISION, SOUND PROGRAMME AND OTHER MULTIMEDIA SIGNALS

| General Recommendations | J.1–J.9 |
| --- | --- |
| General specifications for analogue sound-programme transmission | J.10–J.19 |
| Performance characteristics of analogue sound-programme circuits | J.20–J.29 |
| Equipment and lines used for analogue sound-programme circuits | J.30–J.39 |
| Digital encoders for analogue sound-programme signals | J.40–J.49 |
| Digital transmission of sound-programme signals | J.50–J.59 |
| Circuits for analogue television transmission | J.60–J.69 |
| Analogue television transmission over metallic lines and interconnection with radio-relay links | J.70–J.79 |
| Digital transmission of television signals | J.80–J.89 |
| Ancillary digital services for television transmission | J.90–J.99 |
| Operational requirements and methods for television transmission | J.100–J.109 |
| Interactive systems for digital television distribution | J.110–J.129 |
| Transport of MPEG-2 signals on packetised networks | J.130–J.139 |
| Measurement of the quality of service | J.140–J.149 |
| Digital television distribution through local subscriber networks | J.150–J.159 |
| IPCablecom | J.160–J.179 |
| Miscellaneous | J.180–J.199 |
| Application for Interactive Digital Television | J.200–J.209 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation J.123 Multiplexing format for webcasting on the TCP/IP network

Summary This Recommendation provides a multiplexing format appropriate for audio and video transmission by download-based protocol over TCP/IP without any session control protocols between server and client. However, when session control protocols are not used, some necessary information should be added to the media data. Therefore, this Recommendation defines a multiplexing format particular to webcasting on the TCP/IP. This format carries metadata, digital rights management (DRM) information and formatted text as well as audio and video bitstreams.

Source ITU-T Recommendation J.123 was prepared by ITU-T Study Group 9 (2001-2004) and approved under the WTSA Resolution 1 procedure on 29 July 2002.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2002

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS

Page

1	Scope		1

2	References		1

2.1	Normative reference		1

2.2	Informative references		1

3	Terms and definitions		1

4	Abbreviations		2

5	Reference architecture		2

6	File format		3

6.1	Basic structure		3

6.2	Object Structure		3

7	Box definitions		5

7.1	File Type box		5

7.1.1	Definition		5

7.1.2	Syntax		5

7.1.3	Semantics		5

7.2	Other boxes		5

8	Extension Data		6

8.1	Digital Rights Management		6

8.1.1	Syntax		6

8.1.2	Semantics		6

8.2	Formatted text		7

8.2.1	Basic structure		7

8.2.2	Text describing and controlling element		7

8.2.3	<tsml> element		8

8.2.4	<head> element		8

8.2.5	<layout> element		8

8.2.6	<region> element		9

8.2.7	<font> element in <head> element		9

8.2.8	<body> element		9

8.2.9	<telop> element		10

8.2.10	<font> element in <telop> element		10

8.2.11	<br> element		11

8.2.12	<u> element		11

8.2.13	<rev> element		11

8.2.14	<a> element		12

Page

8.2.15	Limitations for nesting of elements		12

Appendix I  Sample parameters		12

ITU-T Recommendation J.123

Multiplexing format for webcasting on the TCP/IP network

# 1	Scope

This Recommendation defines a multiplexing format appropriate for audio and video transmission by download-based protocol over TCP/IP. By this format, webcasting on TCP/IP network is realized without any session control protocols between server and client.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

## 2.1	Normative reference

[1]	ISO/IEC 14496-1:2001, Information technology – Coding of audio-visual objects – Part 1: Systems.

## 2.2	Informative references

[2]	ITU-T Recommendation J.120 (2000), Distribution of sound and television programs over the IP network.

[3]	ISO/IEC 14496-2:2001, Information technology – Coding of audio-visual objects – Part 2: Visual.

[4]	ISO/IEC 13818-3:1998, Information technology – Generic coding of moving pictures and associated audio information – Part 3: Audio.

[5]	IETF RFC 2068 (1997), Hypertext Transfer Protocol – HTTP/1.1.

# 3	Terms and definitions

This Recommendation defines the following terms:

3.1	box: An object-oriented building block defined by a unique type identifier and length (called 'atom' in the reference [1]).

3.2	chunk: A contiguous set of samples for one track.

3.3	container box: A box whose sole purpose is to contain and group a set of related boxes.

3.4	movie box: A container box whose sub-boxes define the metadata for a presentation ('moov').

3.5	media data box: A container box which can hold the actual media data for a presentation ('mdat').

3.6	presentation: One or more motion sequences, possibly combined with audio.

3.7	sample: An individual frame of video, or a time-contiguous compressed section of audio.

3.8	sample description: A structure which defines and describes the format of some number of samples in a track.

3.9	sample table: A packed directory for the timing and physical layout of the samples in a track.

3.10	track: A collection of related samples, which corresponds to a sequence of images or sampled audio.

3.11	webcasting: Webcasting is defined in ITU-T Rec. J.120: Distribution of sound and television programs over the IP network.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

DRM		Digital Rights Management

HTTP		Hypertext Transport Protocol

IP		Internet Protocol

TCP		Transmission Control Protocol

UUID		Universal Unique Identifier

# 5	Reference architecture

This Recommendation assumes that download-based protocol (e.g. HTTP) should be used for webcasting because it does not require any complex server-client protocols.

The reference architecture for webcasting on TCP/IP is shown in Figure 1.

Figure 1/J.123 – Architecture of webcasting on TCP/IP

# 6	File format

## 6.1	Basic structure

The file format consists of extension data, contents header and media data. Basic structure of the file is shown in Figure 2.

Figure 2/J.123 – Basic structure of a file format

## 6.2	Object Structure

The file is structured as a sequence of objects called "Box"; some of these objects may contain other objects. The sequence of objects in the file shall contain exactly one presentation metadata wrapper (the Movie Box). It shall be located in the beginning of the file. The other objects found at this level may be UUID Boxes, or Media Data Boxes.

The fields in the objects are stored with the most significant byte first, commonly known as network byte order or big-endian format.

All boxes defined in this Recommendation are listed in Table 1, which are marked by a grey shade.

Table 1/J.123 – Box types and structure

| ftyp |  |  |  |  |  |  |  |  | File type and compatibility |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| uuid |  |  |  |  |  |  |  |  | Extension data |
| moov |  |  |  |  |  |  |  |  | Container for all the information |
|  | mvhd |  |  |  |  |  |  |  | Movie header, overall declarations |
|  | trak |  |  |  |  |  |  |  | Container for an individual track or stream |
|  |  | tkhd |  |  |  |  |  |  | Track header, overall information about the track |
|  |  | tref |  |  |  |  |  |  | Track reference container |
|  |  | edts |  |  |  |  |  |  | Edit list container |
|  |  |  |  |  | elst |  |  |  | An edit list |
|  |  | mdia |  |  |  |  |  |  | Container for the media information in a track |
|  |  |  |  |  | mdhd |  |  |  | Media header, overall information about the media |
|  |  |  |  |  | hdlr |  |  |  | Handler, declares the media (handler) type |
|  |  |  |  |  | minf |  |  |  | Media information container |
|  |  |  |  |  |  | vmhd |  |  | Video media header, overall information |
|  |  |  |  |  |  | smhd |  |  | Sound media header, overall information |
|  |  |  |  |  |  | hmhd |  |  | Hint media header, overall information |
|  |  |  |  |  |  | dinf |  |  | Data information box, container |
|  |  |  |  |  |  |  |  | dref | Data reference box, declares source(s) of media data in track |
|  |  |  |  |  |  | stbl |  |  | Sample table box, container for the time/space map |
|  |  |  |  |  |  |  |  | stsd | Sample descriptions (codec types, initialization etc.) |
|  |  |  |  |  |  |  |  | stts | (Decoding) time-to-sample |
|  |  |  |  |  |  |  |  | ctts | (Composition) time to sample |
|  |  |  |  |  |  |  |  | stsc | Sample-to-chunk, partial data-offset information |
|  |  |  |  |  |  |  |  | stsz | Sample sizes (framing) |
|  |  |  |  |  |  |  |  | stz2 | Compact sample sizes (framing) |
|  |  |  |  |  |  |  |  | stco | Chunk offset, partial data-offset information |
|  |  |  |  |  |  |  |  | stss | Sync sample table (random access points) |
|  |  |  |  |  |  |  |  | stsh | Shadow sync sample table |
|  |  |  |  |  |  |  |  | padb | Sample padding bits |
|  |  |  |  |  |  |  |  | stdp | Sample degradation priority |
|  | mvex |  |  |  |  |  |  |  | Movie extends box |
|  |  |  | trex |  |  |  |  |  | Track extends defaults |
| moof |  |  |  |  |  |  |  |  | Movie fragment |
|  | mfhd |  |  |  |  |  |  |  | Movie fragment header |
|  | traf |  |  |  |  |  |  |  | Track fragment |
|  |  | tfhd |  |  |  |  |  |  | Track fragment header |
|  |  | trun |  |  |  |  |  |  | Track fragment run |
| mdat |  |  |  |  |  |  |  |  | Media data container |
| free |  |  |  |  |  |  |  |  | Free space |
| skip |  |  |  |  |  |  |  |  | Free space |
|  | udta |  |  |  |  |  |  |  | User-data |

# 7	Box definitions

## 7.1	File Type box

### 7.1.1	Definition

Box Type: 'ftyp'

Container: File

Mandatory: Yes

Quantity: Exactly one

A media-file structured to this part of this specification may be compatible with more than one detailed specification, and it is therefore not always possible to speak of a single 'type' or 'brand' for the file. This means that the utility of the file name extension and mime type are somewhat reduced.

This box must be placed as early as possible in the file (e.g. after any obligatory signature, but before any significant variable-size boxes such as the UUID Box, Movie Box or Media Data Box). It identifies which specification is the 'best use' of the file, and a minor version of that specification; and also a set of other specifications to which the file complies. Readers implementing this format should attempt to read files which are marked as compatible with any of the specifications which the reader implements. Any incompatible change in a specification should therefore register a new 'brand' identifier to identify files conformant to the new specification.

The type 'isom' is defined in this clause, as identifying files which conform to the format in this Recommendation. More specific identifiers can be used to identify precise versions of specifications providing more detail.

Files would normally be externally identified (e.g. with a file extension or mime type) that identifies the 'best use' (major brand), or the brand that the author believes will provide the greatest compatibility.

### 7.1.2	Syntax

aligned(8) class FileTypeBox

extends Box('ftyp') {

unsigned int(32)	major-brand;

unsigned int(32)	minor-version;

unsigned int(32)	compatible-brands[];	// to end of the box

}

### 7.1.3	Semantics

This box identifies the specifications to which this file complies.

Each brand is a printable four-character code that identifies a precise specification. Only one brand is defined here: 'isom', identifies files structurally conformant to this media-independent part of this specification.

major-brand – is a brand identifier

minor-version – is an informative integer for the minor version of the major brand

compatible-brands – is a list, to the end of the box, of brands

## 7.2	Other boxes

Definitions of all other boxes are found in the reference [1].

# 8	Extension Data

Extension Data is formatted in "uuid" box. Two functions for Extension Data are described as follows:

•	Digital Rights Management

	Copy prohibition

	Expiration date

	Validation period after downloading

	Number of times play

•	Formatted Text (Closed Caption)

	Text decoration: Foreground/background colour, Bold, Italic, Fonts, etc.

	Effects: Scroll, Wipe, Blink, etc.

	Synchronization with the media data

	Hyper link to the Internet

## 8.1	Digital Rights Management

Rights management information controls play and/or re-transmission of the downloaded MP4 file. It is contained in uuid Box of the file format.

### 8.1.1	Syntax

aligned(8) class CopyGuardBox extends FullAtom ('uuid', version = 0, flags){

bit(32)					copy-guard;

unsigned int(32)	limit-date;

unsigned int(32)	limit-period;

unsigned int(32)	limit-count;

}

### 8.1.2	Semantics

| Field | Type | Description | Parameters |
| --- | --- | --- | --- |
| type | uint32 | Type of Box | 'uuid' is set |
| usertype | uint8[16] | ID | "cpgd"-A88C-11d4-8197-09027087703 |
| version | uint8 | Version | 0 is set |
| flags | bit24 | Rights management flags | 0: No limitation 1: Limitation by expiration date 2: Limitation by validated period 4: Limitation by playing number of times Unless the case of No limitation, the following "never copy" flag shall be set to '1' |
| copy-guard | bit32 | "never copy" flag | 0: copy permitted 1: copy prohibited |
| limit-date | uint32 | Expiration date | Specify the expiration date in seconds from 1904/1/1 0:00GMT |
| limit-period | uint32 | Validated period | Specify the validated period in days after the file is downloaded |
| limit-count | uint32 | Playing number of times | '1' means that the file can be played only once |

## 8.2	Formatted text

The formatted text is a closed caption synchronizing to a video content. It is contained in uuid Box of the file format.

### 8.2.1	Basic structure

The formatted text is written by XML-based syntax. The following is an overview of the elements:

| tsml |  |  |  | Root |
| --- | --- | --- | --- | --- |
|  | head |  |  | Header |
|  |  | layout |  | Layout |
|  |  |  | region | Region attribute for displaying telop |
|  |  |  | font | Default font |
|  | body |  |  | Telop body |
|  |  | telop |  | Telop letters |
|  |  |  | font | Font for letters |
|  |  |  | br | Line break |
|  |  |  | u | Underline |
|  |  |  | rev | Reversal of font and background color |
|  |  |  | a | Link |

Syntax of the formatted text is shown as follows.

<tsml>

<head>

:

</head>

<body>

<telop> ... </telop>

<telop> ... </telop>

:

</body>

</tsml>

In this Recommendation, XML declaration is omitted.

### 8.2.2	Text describing and controlling element

Each element is described using the following parameters:

•	attribute

It is an inherent attribute that is described in the beginning tag.

•	child

It shows elements that can be included between the beginning tag and the end tag.

### 8.2.3	<tsml> element

<tsml> element is a root of the formatted text.

| Tag |  |  |
| --- | --- | --- |
| <tsml> </tsml> |  |  |
| attribute |  |  |
| Name | Value (default) | Description |
| child |  |  |
| head | header of telop information |  |
| body | body part of telop information |  |

NOTE – <tsml> element appears only once in the formatted text information.

### 8.2.4	<head> element

<head> element specifies common settings throughout the formatted text information.

| Tag |  |  |
| --- | --- | --- |
| <head> </head> |  |  |
| attribute |  |  |
| Name | Value (default) | Description |
| child |  |  |
| layout | Layout for displaying telop |  |

### 8.2.5	<layout> element

<layout> element specifies the attribute of the displaying area.

| Tag |  |  |
| --- | --- | --- |
| <layout> </layout> |  |  |
| attribute |  |  |
| Name | Value (default) | Description |
| child |  |  |
| region | attribute of telop region |  |
| font | default font |  |

### 8.2.6	<region> element

<region> element specifies the attribute of the formatted text region.

| Tag |  |  |
| --- | --- | --- |
| <region> </region> |  |  |
| attribute |  |  |
| Name | Value (default) | Description |
| background-color | color value #rrggbb | Background color of the region |
| child |  |  |

### 8.2.7	<font> element in <head> element

<font> element specifies the attribute of the font such as a default font color.

| Tag |  |  |
| --- | --- | --- |
| <font> </font> |  |  |
| attribute |  |  |
| Name | Value (default) | Description |
| color | color value #rrggbb | font color |
| child |  |  |

### 8.2.8	<body> element

<body> element includes a body part of the formatted text.

| Tag |  |  |
| --- | --- | --- |
| <body> </body> |  |  |
| attribute |  |  |
| Name | Value (default) | Description |
| child |  |  |
| telop | telop letters |  |

### 8.2.9	<telop> element

<telop> element specifies appearance and letters of the formatted text.

| Tag |  |  |
| --- | --- | --- |
| <telop> </telop> | Texts are included between these tags |  |
| attribute |  |  |
| Name | Value (default) | Description |
| begin | "X" (0) | begin time of the element in ms |
| end | "X" (The end time of the content in the MP4 file) | end time of the element in ms |
| wrap | true ¦ false (false) | automatic word wrap |
| child |  |  |
| font | Fonts for letters |  |
| br | Line break |  |
| u | Underline |  |
| rev | Reversal of colours for fonts and background |  |
| a | Links |  |

NOTE 1 – The beginning time of the contents is taken as 0 ms.

NOTE 2 – If the next <telop> element has earlier beginning time than the end time of the previous <telop>, the next <telop> starts at the beginning time as specified. (The beginning time is given priority.)

The formatted text is displayed at the time of 'begin', and disappeared at the time of 'end'.

Following elements are included in the <telop> element as a child element.

### 8.2.10	<font> element in <telop> element

| Tag |  |  |
| --- | --- | --- |
| <font> </font> |  |  |
| attribute |  |  |
| Name | Value (default) | Description |
| color | COLOR value #rrggbb | Font color |
| child |  |  |
| br | Line break |  |
| u | Underline |  |
| rev | Reversal of font color and background color |  |
| a | Link |  |

### 8.2.11	<br> element

| Tag |  |  |
| --- | --- | --- |
| <br/> |  |  |
| attribute |  |  |
| Name | Value (default) | Description |
| child |  |  |

This element inserts a line break at the arbitrary letter.

### 8.2.12	<u> element

| Tag |  |  |
| --- | --- | --- |
| <u></u> |  |  |
| attribute |  |  |
| Name | Value (default) | Description |
| child |  |  |
| font | Font color |  |
| br | Line break |  |
| rev | Reversal of font color and background color |  |
| a | Link |  |

Enclosed letters by this element are given an underline.

### 8.2.13	<rev> element

| Tag |  |  |
| --- | --- | --- |
| <rev></rev> |  |  |
| attribute |  |  |
| Name | Value (default) | Description |
| child |  |  |
| font | Font color |  |
| br | Line break |  |
| u | Underline |  |
| a | Link |  |

Reversal of font color and background color.

### 8.2.14	<a> element

| Tag |  |  |
| --- | --- | --- |
| <a></a> |  |  |
| attribute |  |  |
| Name | Value (default) | Description |
| href | Link destination |  |
| child |  |  |
| font | Font color |  |
| br | Line break |  |
| u | Underline |  |
| rev | Reversal of font color and background color |  |

Link destination can include telephone number (tel:), e-mail address (mailto:), and WWW (http:).

### 8.2.15	Limitations for nesting of elements

<font>, <u> and <rev> element are permitted only one nest.

ex.

<font><u> ... </u></font>		OK

<font><u><font> ... </font></u></font>	NG

<font><rev><u> ... </u></rev></font>	NG

Appendix I

Sample parameters

Video encoding type: ISO/IEC MPEG-4 Visual Simple Profile Level 1

Audio encoding type: ISO/IEC MPEG-2 Audio Layer 3 (22 050 Hz)

Content length: 					30 seconds

Video frame rate:				10 frames/second

Interleave interval (chunk length): 	  1 second

Multiplexing by these parameters results in the following format:

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |