|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | J.168 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (03/2001) |
|  | SERIES J: CABLE NETWORKS AND TRANSMISSION OF TELEVISION, SOUND PROGRAMME AND OTHER MULTIMEDIA SIGNALS IPCablecom |  |  |  |
|  | IPCablecom media terminal adapter (MTA) MIB requirements |  |  |  |
|  | ITU-T Recommendation J.168 (Formerly CCITT Recommendation) |  |  |  |

ITU-T J-SERIES  RECOMMENDATIONS

CABLE NETWORKS AND TRANSMISSION OF TELEVISION, SOUND PROGRAMME AND OTHER MULTIMEDIA SIGNALS

| General Recommendations | J.1–J.9 |
| --- | --- |
| General specifications for analogue sound-programme transmission | J.10–J.19 |
| Performance characteristics of analogue sound-programme circuits | J.20–J.29 |
| Equipment and lines used for analogue sound-programme circuits | J.30–J.39 |
| Digital encoders for analogue sound-programme signals | J.40–J.49 |
| Digital transmission of sound-programme signals | J.50–J.59 |
| Circuits for analogue television transmission | J.60–J.69 |
| Analogue television transmission over metallic lines and interconnection with radio-relay links | J.70–J.79 |
| Digital transmission of television signals | J.80–J.89 |
| Ancillary digital services for television transmission | J.90–J.99 |
| Operational requirements and methods for television transmission | J.100–J.109 |
| Interactive systems for digital television distribution | J.110–J.129 |
| Transport of MPEG-2 signals on packetised networks | J.130–J.139 |
| Measurement of the quality of service | J.140–J.149 |
| Digital television distribution through local subscriber networks | J.150–J.159 |
| IPCablecom | J.160–J.179 |
| Miscellaneous | J.180–J.199 |
| Application for Interactive Digital Television | J.200–J.209 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation J.168 IPCablecom media terminal adapter (MTA) MIB requirements

- Summary This Recommendation describes IPCablecom's Media Terminal Adapter (MTA) MIB requirements.

- Source ITU-T Recommendation J.168 was prepared by ITU-T Study Group 9 (2001-2004) and approved under the WTSA Resolution 1 procedure on 9 March 2001.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2002

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from ITU.

CONTENTS

Page

1	Scope		1

2	References		1

2.1	Normative references		1

2.2	informative references		1

3	Terms and definitions		2

4	Abbreviations		2

5	Requirements		2

ITU-T Recommendation J.168

IPCablecom media terminal adapter (MTA) MIB requirements

# 1	Scope

This Recommendation describes IPCablecom MTA MIB requirement.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revisions; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

## 2.1	Normative references

–	ITU-T J.162 (2001), Network call signalling protocol for the delivery of time-critical services over cable television networks using cable modems.

–	ITU-T J.166 (2001), IPCablecom management information base (MIB) framework.

–	ITU-T J.167 (2001), Media terminal adapter (MTA) device provisioning requirements for the delivery of real-time services over cable television networks using cable modems.

–	ITU-T J.170 (Draft), IPCablecom security specification.

–	ITU-T X.680 (1997), Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation.

–	ITU-T X.681 (1997), Information technology – Abstract Syntax Notation One (ASN.1): Information object specification.

–	ITU-T X.682 (1997), Information technology – Abstract Syntax Notation One (ASN.1): Constraint specification.

–	ITU-T X.683 (1997), Information technology – Abstract Syntax Notation One (ASN.1): Parameterization of ASN.1 specifications.

–	ITU-T X.690 (1997), Information technology – ASN.1 encoding rules: Specification of Basic Encoding Rules (BER), Canonical Encoding Rules (CER) and Distinguished Encoding Rules (DER).

## 2.2	informative references

–	ITU-T J.160 (Draft), Architectural framework for the delivery of time-critical services over cable television networks using cable modems.

–	IETF RFC 2571 (1999), An Architecture for Describing SNMP Management Frameworks.

–	IETF RFC 2572 (1999), Message Processing and Dispatching for the Simple Network Management Protocol.

–	IETF RFC 2573 (1999), SNMP Applications.

–	IETF RFC 2574 (1999), User-based Security Model (USM) for version 3 of the Simple Network ManagementProtocol (SNMPv3).

# 3	Terms and definitions

This Recommendation defines the following terms:

3.1	cable modem: A cable modem is a layer-two termination device that terminates the customer end of the J.112 connection.

3.2	IPCablecom: An ITU-T project that includes an architecture and a series of Recommendations that enable the delivery of real time services (such as telephony) over the cable television networks using Cable Modems.

3.3	Management Information Base (MIB): the specification of information in a manner that allows standard access through a network management protocol.

3.4	MUST:  The term "MUST" or "MUST NOT" is used as a convention in the present Recommendation to denote an absolutely mandatory aspect of this Recommendation.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

MIB	Management Information Base

MTA	Media Terminal Adaptor

# 5	Requirements

This clause defines the mandatory syntax of the IPCablecom MTA MIB. It follows the IETF Simple Network Management Protocol (SNMP) for defining the managed objects. The MIB is organized as follows:

•	MTA device objects;

•	MTA security objects;

•	objects used for initial provisioning and bootstrapping;

•	objects used for event reporting to syslog, trap, local local

The syntax is given below.

PKTC-MTA-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY,

OBJECT-TYPE,

Integer32,

Unsigned32,

Counter32,

NOTIFICATION-TYPE

FROM SNMPv2-SMI

TruthValue, DisplayString, DateAndTime, RowStatus

FROM SNMPv2-TC

OBJECT-GROUP,

MODULE-COMPLIANCE

FROM SNMPv2-CONF

clabProjIPCablecom

FROM CLAB-DEF-MIB

ifIndex

FROM IF-MIB

docsDevSwCurrentVers

FROM DOCS-CABLE-DEVICE-MIB;     -- version 8

pktcMtaMib MODULE-IDENTITY

LAST-UPDATED    "9912010000Z" -- December 1, 1999

ORGANIZATION    "IPCablecom OSS Group"

CONTACT-INFO

"Roy Spitzer

Postal: Telogy Networks, Inc.

20250 Century Blvd.

Germantown, MD 20855

U.S.A.

Phone:  +1 301-515-6531

Fax:    +1 301-515-7954

E-mail: rspitzer@telogy.com"

DESCRIPTION

"This MIB module supplies the basic management objects

for the MTA Device."

::=  { clabProjPacketCable 1 }

-- Textual conventions

X509Certificate ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"An X509 digital certificate encoded as an ASN.1 DER
object."

SYNTAX OCTET STRING (SIZE (0..4096))

--

--   IPCablecom supports embedded MTA only

--   IPCablecom requires SNMPv3

--

pktcMtaMibObjects       OBJECT IDENTIFIER ::= { pktcMtaMib 1 }

pktcMtaDevBase          OBJECT IDENTIFIER ::= { pktcMtaMibObjects 1 }

pktcMtaDevServer        OBJECT IDENTIFIER ::= { pktcMtaMibObjects 2 }

pktcMtaDevSecurity      OBJECT IDENTIFIER ::= { pktcMtaMibObjects 3 }

pktcMtaDevEvent         OBJECT IDENTIFIER ::= { pktcMtaMibObjects 4 }

--

-- The following group describes the base objects in the MTA

--

pktcMtaDevResetNow OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Setting this object to true(1) causes the device to reset.

Reading this object always returns false(2)."

::= { pktcMtaDevBase 1 }

pktcMtaDevSerialNumber OBJECT-TYPE

SYNTAX      DisplayString (SIZE (0..128))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The manufacturer's serial number for this MTA."

::= { pktcMtaDevBase 2 }

pktcMtaDevHardwareVersion OBJECT-TYPE

SYNTAX      DisplayString (SIZE (0..48))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The manufacturer's hardware version for this MTA."

::= { pktcMtaDevBase 3 }

pktcMtaDevMacAddress  OBJECT-TYPE

SYNTAX      OCTET STRING

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The telephony MAC address for this device."

::= { pktcMtaDevBase 4 }

pktcMtaDevFQDN      OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The Fully Qualified Domain Name for this MTA."

::= { pktcMtaDevBase 5 }

pktcMtaDevEndPntCount    OBJECT-TYPE

SYNTAX      INTEGER (1..255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The physical end points for this MTA."

::= { pktcMtaDevBase 6 }

pktcMtaDevEnabled    OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The MTA Admin Status of this device, where True(1) means

the voice feature is enabled and false(2) indicates that

it is disabled."

::= { pktcMtaDevBase 7 }

pktcMtaDevTypeIdentifier    OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This is a copy of the device type identifier used in the

DHCP option 60 exchanged between the MTA and the DHCP server."

::= { pktcMtaDevBase 8 }

pktcMtaDevProvisioningState    OBJECT-TYPE

SYNTAX      INTEGER

{

pass(1),

inProgress(2),

fail(3)

}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates the completion state of the

provisioning process. Pass or Fail states occur after processing

of the config file is completed. InProgress occurs from boot

time until config file processing is complete. Fail state requires

manual intervention."

::= { pktcMtaDevBase 9 }

pktcMtaDevHttpAccess    OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This indicates whether HTTP file access is supported for

MTA configuration file transfer."

::= { pktcMtaDevBase 10 }

--

-- The following group describes the security objects in the MTA

--

pktcMtaDevManufacturerCertificate OBJECT-TYPE

SYNTAX       X509Certificate

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"ASN.1 DER encoding of the MTA Manufacturer’s X.509 public-key

certificate, called MTA Manufacturer Certificate. It is issued to each

MTA manufacturer and is installed into each MTA either in the factory

or with a code download. The provisioning server cannot update this

certificate."

::= { pktcMtaDevSecurity 1 }

pktcMtaDevCertificate OBJECT-TYPE

SYNTAX       X509Certificate

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"ASN.1 DER encoding of the MTA’s X.509 public-key certificate

issued by the manufacturer and installed into the embedded-MTA in

the factory. This certificate, called MTA Device Certificate, contains

the MTA’s MAC address. It cannot be updated by the provisioning

server."

::= { pktcMtaDevSecurity 2 }

pktcMtaDevSignature OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE (0..256))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"A unique signature created by the MTA for each SNMP Inform

or SNMP Trap or SNMP GetResponse message exchanged prior to

enabling SNMPv3 security ASN.1 encoded Digital signature in

the Cryptographic message syntax (includes nonce)."

::= { pktcMtaDevSecurity 3 }

pktcMtaDevCorrelationId OBJECT-TYPE

SYNTAX      Integer32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Random value generated by the MTA for use in registration

authorization.  It is for use only in the MTA initialization

messages and for MTA configuration file download."

::= { pktcMtaDevSecurity 4 }

pktcMtaDevSecurityTable OBJECT-TYPE

SYNTAX      SEQUENCE OF PktcMtaDevSecurityEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Contains per endpoint security information."

::= {  pktcMtaDevSecurity 5 }

pktcMtaDevSecurityEntry OBJECT-TYPE

SYNTAX      PktcMtaDevSecurityEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"List of security attributes for a single IPCablecom

endpoint interface associated with ifType(104)."

INDEX { ifIndex }

::= { pktcMtaDevSecurityTable 1 }

PktcMtaDevSecurityEntry ::= SEQUENCE {

pktcMtaDevServProviderCertificate OCTET STRING,

pktcMtaDevTelephonyCertificate OCTET STRING,

pktcMtaDevKerberosRealm OCTET STRING,

pktcMtaDevKerbPrincipalName DisplayString,

pktcMtaDevServGracePeriod Integer32,

pktcMtaDevLocalSystemCertificate OCTET STRING,

pktcMtaDevKeyMgmtTimeout1 Integer32,

pktcMtaDevKeyMgmtTimeout2 Integer32

}

pktcMtaDevServProviderCertificate OBJECT-TYPE

SYNTAX       X509Certificate

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"ASN.1 DER encoding of the Telephony Service Provider’s X.509

public-key certificate, called Service Provider Certificate. It serves

as the root of the intra-domain trust hierarchy. Each MTA is

configured with this certificate so that it can authenticate KDCs

owned by the same service provider. The provisioning server needs

the ability to update this certificate in the MTAs via both SNMP and

configuration files."

::= { pktcMtaDevSecurityEntry 1 }

pktcMtaDevTelephonyCertificate OBJECT-TYPE

SYNTAX       X509Certificate

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"ASN.1 DER encoding of the MTA’s X.509 public-key certificate

issued by the Service Provider with either the Service Provider CA or

a Local System CA. This certificate, called MTA Telephony

Certificate, contains the same public key as the MTA Device

Certificate issued by the manufacturer. It is used to authenticate the

identity of the MTA to the TGS (during PKINIT exchanges). The

provisioning server needs the ability to update this certificate in the

MTAs via both SNMP and configuration files."

::= { pktcMtaDevSecurityEntry 2 }

pktcMtaDevKerberosRealm OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE (0..1280))

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Specifies a Kerberos realm (i.e. administrative domain), required

for IPCablecom key management]."

::= { pktcMtaDevSecurityEntry 3 }

pktcMtaDevKerbPrincipalName OBJECT-TYPE

SYNTAX      DisplayString (SIZE(0..40))

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Kerberos principal name for the Call Agent.  This information

is required in order for the MTA to obtain Call Agent Kerberos

tickets.  This principal name does not include the realm, which

is specified as a separate field in this configuration file.  A

Single Kerberos principal name MAY be shared among several

Call Agents."

::= { pktcMtaDevSecurityEntry 4 }

pktcMtaDevServGracePeriod OBJECT-TYPE

SYNTAX      Integer32 (15..600)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The MTA MUST obtain a new Kerberos ticket (with a PKINIT

exchange) this many minutes before the old ticket expires.  The

minimum allowable value is 15 mins.  The default is 30 mins."

DEFVAL { 30 }

::= { pktcMtaDevSecurityEntry 5 }

pktcMtaDevLocalSystemCertificate OBJECT-TYPE

SYNTAX 		X509Certificate

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Telephony Service Provider CA may delegate the issuance of certificates

to a regional Certification Authority called Local System CA (with the

corresponding Local System Certificate).  This parameter is the ASN.1

DER encoding of the Local System Certificate. It MUST have a non-empty

value when the MTA Telephony certificate is signed by a Local System CA.

Otherwise, the value MUST be of length 0."

::= { PktcMtaDevSecurityEntry 6 }

pktcMtaDevKeyMgmtTimeout1 OBJECT-TYPE

SYNTAX      Integer32 (15..600)

UNITS		"seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This timeout applies only when the MTA initiated key management.  It is

the period during which the MTA will save a nonce (inside the sequence

number field) from the sent out AP Request and wait for the matching AP

Reply from the CMS."

::= { pktcMtaDevSecurityEntry 7 }

pktcMtaDevKeyMgmtTimeout2 OBJECT-TYPE

SYNTAX      Integer32 (15..600)

UNITS		"seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This timeout applies only when the CMS initiated key management (with a

Wake Up or Rekey message).  It is the period during which the MTA will

save a nonce (inside the sequence number field) from the sent out AP

Request and wait for the matching AP Reply from the CMS."

::= { pktcMtaDevSecurityEntry 8}

pktcMtaDevTgsTable OBJECT-TYPE

SYNTAX      SEQUENCE OF PktcMtaDevTgsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Contains per endpoint Ticket Granting Server information."

::= {  pktcMtaDevSecurity 8 }

pktcMtaDevTgsEntry OBJECT-TYPE

SYNTAX      PktcMtaDevTgsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"List of Tgs attributes for a single IPCablecom

endpoint interface associated with ifType(104)."

INDEX { ifIndex, pktcMtaDevTgsIndex }

::= { pktcMtaDevTgsTable 1 }

PktcMtaDevTgsEntry ::= SEQUENCE {

pktcMtaDevTgsIndex Integer32,

pktcMtaDevTgsLocation DisplayString,

pktcMtaDevTgsStatus RowStatus

}

pktcMtaDevTgsIndex OBJECT-TYPE

SYNTAX      Integer32

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Index into the TGS table for TGS locations.

IfType specifies the endpoint, TgsIndex specifies a TGS."

::= { pktcMtaDevTgsEntry 1 }

pktcMtaDevTgsLocation OBJECT-TYPE

SYNTAX      DisplayString (SIZE (0..255))

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"Name of the TGS – Ticket Granting Server, which is the Kerberos

Server. This parameter is a FQDN or Ipv4 address.  There may be

multiple entries of this type.  The order in which these entries

are listed is the priority order in which the MTA will attempt to

contact them for this endpoint."

::= { pktcMtaDevTgsEntry 2 }

pktcMtaDevTgsStatus    OBJECT-TYPE

SYNTAX      RowStatus

MAX-ACCESS    read-create

STATUS    current

DESCRIPTION

"This object contains the Row Status associated with

the pktcMtaDevTgsTable."

::= { pktcMtaDevTgsEntry 3 }

--

-- The following group describes server access and parameters used for

-- initial provisioning and bootstrapping.

--

pktcMtaDevServerBootState OBJECT-TYPE

SYNTAX INTEGER {

operational(1),

disabled(2),

waitingForDhcpOffer(3),

waitingForDhcpResponse(4),

waitingForConfig(5),

refusedByCmts(6),

other(7),

unknown(8)

}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"If operational(1), the device has completed loading and

processing of configuration parameters and the Access Node has

completed the Registration exchange.

If disabled(2) then the device was administratively

disabled, possibly by being refused network access in the

configuration file.

If waitingForDhcpOffer(3) then a DHCP Discover has been

transmitted and no offer has yet been received.

If waitingForDhcpResponse(4) then a DHCP Request has been

transmitted and no response has yet been received.

If waitingForConfig(5) then a request to the config parameter

server has been made and no response received.

If refusedByCmts(6) then the Registration Request/Response

exchange with the Access Node failed."

::= { pktcMtaDevServer 1 }

pktcMtaDevServerDhcp OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The IP address or FQDN of the DHCP server that assigned an IP

address to this device. Returns 0.0.0.0 if DHCP was not

used for IP address assignment."

::= { pktcMtaDevServer 2 }

pktcMtaDevServerDns1 OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The IP address or FQDN of the primary DNS server that resolved

an IP address for this device."

::= { pktcMtaDevServer 3 }

pktcMtaDevServerDns2 OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The IP address or FQDN of the secondary DNS server that resolved an IP

address for this device."

::= { pktcMtaDevServer 4 }

pktcMtaDevConfigFile OBJECT-TYPE    SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The URL of the TFTP/HTTP file for downloading provisioning

and configuration parameters to this device. Returns NULL if the

server address is unknown. Supports both TFTP and HTTP"

::= { pktcMtaDevServer 5 }

pktcMtaDevSnmpEntity OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The IP address or FQDN of the SNMP entity for provisioning trap

handling that assigned an IP address to this device. Returns

0.0.0.0 if DHCP was not used for IP address assignment."

::= { pktcMtaDevServer 6 }

--

--  Event Reporting

--

pktcMtaDevEvControl OBJECT-TYPE

SYNTAX INTEGER {

resetLog(1),

useDefaultReporting(2)

}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Setting this object to resetLog(1) empties the event log.

All data is deleted. Setting it to useDefaultReporting(2)

returns all event priorities to their factory-default

reporting. Reading this object always returns

useDefaultReporting(2)."

::= { pktcMtaDevEvent 1 }

pktcMtaDevEvSyslog OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The IP address or FQDN of the Syslog server. If 0.0.0.0,

syslog transmission is inhibited."

::= { pktcMtaDevEvent 2 }

pktcMtaDevEvThrottleAdminStatus OBJECT-TYPE

SYNTAX INTEGER {

unconstrained(1),

maintainBelowThreshold(2),

stopAtThreshold(3),

inhibited(4)

}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Controls the transmission of traps and syslog messages

with respect to the trap pacing threshold.

unconstrained(1) causes traps and syslog messages to be

transmitted without regard to the threshold settings.

maintainBelowThreshold(2) causes trap transmission and

syslog messages to be suppressed if the number of traps

would otherwise exceed the threshold.

stopAtThreshold(3) causes trap transmission to cease

at the threshold, and not resume until directed to do so.

inhibited(4) causes all trap transmission and syslog

messages to be suppressed.

A single event is always treated as a single event for

threshold counting. That is, an event causing both a trap

and a syslog message is still treated as a single event.

Writing to this object resets the thresholding state.

At initial startup, this object has a default value of

unconstrained(1)."

::= { pktcMtaDevEvent 3 }

pktcMtaDevEvThrottleInhibited OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"If true(1), trap and syslog transmission is currently

inhibited due to thresholds and/or the current setting of

pktcMtaDevEvThrottleAdminStatus. In addition, this is set to

true(1) if transmission is inhibited due to no

syslog (pktcMtaDevEvSyslog) or trap (pktcMtaDevNmAccessEntry)

destinations having been set."

::= { pktcMtaDevEvent 4 }

pktcMtaDevEvThrottleThreshold OBJECT-TYPE

SYNTAX      Unsigned32

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of trap/syslog events per pktcMtaDevEvThrottleInterval

to be transmitted before throttling.

A single event is always treated as a single event for

threshold counting. That is, an event causing both a trap

and a syslog message is still treated as a single event.

At initial startup, this object returns 0."

::= { pktcMtaDevEvent 5 }

pktcMtaDevEvThrottleInterval OBJECT-TYPE

SYNTAX      Integer32 (1..2147483647)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The interval over which the trap threshold applies.

At initial startup, this object has a value of 1."

::= { pktcMtaDevEvent 6 }

--

-- The following table controls the reporting of the various classes of

-- events.

--

pktcMtaDevEvControlTable OBJECT-TYPE

SYNTAX      SEQUENCE OF PktcMtaDevEvControlEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table allows control of the reporting of event classes.

For each event priority, a combination of logging and

reporting mechanisms may be chosen. The mapping of event types

to priorities is vendor-dependent. Vendors may also choose to

allow the user to control that mapping through proprietary means."

::= {  pktcMtaDevEvent 7 }

pktcMtaDevEvControlEntry OBJECT-TYPE

SYNTAX      PktcMtaDevEvControlEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Allows configuration of the reporting mechanisms for a

particular event priority."

INDEX { pktcMtaDevEvPriority }

::= { pktcMtaDevEvControlTable 1 }

pktcMtaDevEvControlEntry ::= SEQUENCE {

pktcMtaDevEvPriority        INTEGER,

pktcMtaDevEvReporting       BITS

}

pktcMtaDevEvPriority OBJECT-TYPE

SYNTAX INTEGER {

emergency(1),

alert(2),

critical(3),

error(4),

warning(5),

notice(6),

information(7),

debug(8)

}

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The priority level that is controlled by this

entry. These are ordered from most (emergency) to least (debug)

critical.  Each event with a CM or Access Node has a particular

priority level associated with it (as defined by the

vendor). During normal operation no event more critical than

notice(6) should be generated. Events between warning and

emergency should be generated at appropriate levels of

problems (e.g. emergency when the box is about to

crash)."

::= { pktcMtaDevEvControlEntry 1 }

pktcMtaDevEvReporting OBJECT-TYPE

SYNTAX BITS {

local(0),

traps(1),

syslog(2)

}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Defines the action to be taken on occurrence of this

event class. Implementations may not necessarily support

all options for all event classes, but at minimum must

allow traps and syslogging to be disabled. If the

local(0) bit is set, then log to the internal log, if the

traps(1) bit is set, then generate a trap, if the

syslog(2) bit is set, then send a syslog message

(assuming the syslog address is set)."

::= { pktcMtaDevEvControlEntry 2 }

pktcMtaDevEventTable OBJECT-TYPE

SYNTAX      SEQUENCE OF PktcMtaDevEventEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Contains a log of network and device events that may be

of interest in fault isolation and troubleshooting."

::= {  pktcMtaDevEvent 8 }

pktcMtaDevEventEntry OBJECT-TYPE

SYNTAX      PktcMtaDevEventEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Describes a network or device event that may be of

interest in fault isolation and troubleshooting. Multiple

sequential identical events are represented by

incrementing pktcMtaDevEvCounts and setting

pktcMtaDevEvLastTime to the current time rather than creating

multiple rows.

Entries are created with the first occurrance of an event.

pktcMtaDevEvControl can be used to clear the table.

Individual events can not be deleted."

INDEX { pktcMtaDevEvIndex }

::= { pktcMtaDevEventTable 1 }

PktcMtaDevEventEntry ::= SEQUENCE {

pktcMtaDevEvIndex           INTEGER,

pktcMtaDevEvFirstTime       DateAndTime,

pktcMtaDevEvLastTime        DateAndTime,

pktcMtaDevEvCounts          Counter32,

pktcMtaDevEvLevel           INTEGER,

pktcMtaDevEvId              Unsigned32,

pktcMtaDevEvText            DisplayString

}

pktcMtaDevEvIndex OBJECT-TYPE

SYNTAX      INTEGER (1..2147483647)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Provides relative ordering of the objects in the event

log. This object will always increase except when

(a) the log is reset via pktcMtaDevEvControl,

(b) the device reboots and does not implement non-volatile

storage for this log, or (c) it reaches the value 2^31.

The next entry for all the above cases is 1."

::= { pktcMtaDevEventEntry 1 }

pktcMtaDevEvFirstTime OBJECT-TYPE

SYNTAX      DateAndTime

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The time that this entry was created."

::= { pktcMtaDevEventEntry 2 }

pktcMtaDevEvLastTime OBJECT-TYPE

SYNTAX      DateAndTime

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"If multiple events are reported via the same entry, the

time that the last event for this entry occurred,

otherwise this should have the same value as

pktcMtaDevEvFirstTime."

::= { pktcMtaDevEventEntry 3 }

pktcMtaDevEvCounts OBJECT-TYPE

SYNTAX      Counter32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of consecutive event instances reported by

this entry.  This starts at 1 with the creation of this

row and increments by 1 for each subsequent duplicate event."

::= { pktcMtaDevEventEntry 4 }

pktcMtaDevEvLevel OBJECT-TYPE

SYNTAX INTEGER {

critical(1),

major(2),

minor(3),

warning(4),

information(5),

notice(6),

debug(7)

}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The priority level of this event as defined by the

vendor. These are ordered from most serious (critical)

to least serious (debug)."

::= { pktcMtaDevEventEntry 5 }

--

-- Vendors will provide their own enumerations for the following.

-- The interpretation of the enumeration is unambiguous for a

-- particular value of the vendor's enterprise number in sysObjectID.

--

pktcMtaDevEvId OBJECT-TYPE

SYNTAX      Unsigned32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"For this product, uniquely identifies the type of event

that is reported by this entry."

::= { pktcMtaDevEventEntry 6 }

pktcMtaDevEvText OBJECT-TYPE

SYNTAX      DisplayString

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Provides a human-readable description of the event,

including all relevant context (interface numbers,

etc.)."

::= { pktcMtaDevEventEntry 7 }

--

-- notification group is for future extension.

--

pktcMtaNotification OBJECT IDENTIFIER ::= { pktcMtaMib 2 0 }

pktcMtaConformance  OBJECT IDENTIFIER ::= { pktcMtaMib 3 }

pktcMtaCompliances  OBJECT IDENTIFIER ::= { pktcMtaConformance 1 }

pktcMtaGroups       OBJECT IDENTIFIER ::= { pktcMtaConformance 2 }

--

--    Notification Group

--

pktcMtaProvisioningEnrollment  NOTIFICATION-TYPE

OBJECTS {

pktcMtaDevHardwareVersion,

docsDevSwCurrentVers,

pktcMtaDevTypeIdentifier,

pktcMtaDevMacAddress,

pktcMtaDevCorrelationId,

pktcMtaDevSignature

}

STATUS      current

DESCRIPTION

"This inform is issued to initiate the IPCablecom

process provisioning."

REFERENCE

"Inform as defined in RFC 1902"

::= { pktcMtaNotification 1 }

pktcMtaProvisioningStatus  NOTIFICATION-TYPE

OBJECTS {

pktcMtaDevMacAddress,

pktcMtaDevCorrelationId,

pktcMtaDevSignature,

pktcMtaDevProvisioningState

}

STATUS      current

DESCRIPTION

"This inform is issued to confirm completion of the IPCablecom

provisioning process, and indicate the completion state."

REFERENCE

"Inform as defined in RFC 1902"

::= { pktcMtaNotification 2 }

-- compliance statements

pktcMtaBasicCompliance MODULE-COMPLIANCE

STATUS     current

DESCRIPTION

"The compliance statement for devices that implement

MTA feature."

MODULE   --pktcMtaMib

-- unconditionally mandatory groups

MANDATORY-GROUPS {

pktcMtaGroup

}

-- units of conformance

OBJECT pktcMtaDevCorrelationId

MIN-ACCESS read-only

DESCRIPTION

"not-accessible when request is SNMP get

read-only when used in informs during provisioning."

OBJECT pktcMtaDevCertificate

MIN-ACCESS  read-only

DESCRIPTION

"Read-write when request is through configuration file `

download, otherwise it's a read-only object."

OBJECT pktcMtaDevTelephonyCertificate

MIN-ACCESS  read-only

DESCRIPTION

"Read-write when request is through configuration file

download, otherwise it's a read-only object."

OBJECT pktcMtaDevServProviderCertificate

MIN-ACCESS  read-only

DESCRIPTION

"Read-write when request is through configuration file

download, otherwise it's a read-only object."

OBJECT pktcMtaDevLocalSystemCertificate

MIN-ACCESS  read-only

DESCRIPTION

"Read-write when request is through configuration file

download, otherwise it's a read-only object."

OBJECT pktcMtaDevKerberosRealm

MIN-ACCESS  read-only

DESCRIPTION

"Read-write when request is through configuration file

download, otherwise it's a read-only object."

OBJECT pktcMtaDevTgsLocation

MIN-ACCESS  read-only

DESCRIPTION

"Read-write when request is through configuration file

download, otherwise it's a read-only object."

OBJECT pktcMtaDevKerbPrincipalName

MIN-ACCESS  read-only

DESCRIPTION

"Read-write when request is through configuration file

download, otherwise it's a read-only object."

OBJECT pktcMtaDevServGracePeriod

MIN-ACCESS  read-only

DESCRIPTION

"Read-write when request is through configuration file

download, otherwise it's a read-only object."

::= { pktcMtaCompliances 3 }

pktcMtaGroup OBJECT-GROUP

OBJECTS {

pktcMtaDevResetNow,

pktcMtaDevSerialNumber,

pktcMtaDevHardwareVersion,

pktcMtaDevMacAddress,

pktcMtaDevFQDN,

pktcMtaDevEndPntCount,

pktcMtaDevEnabled,

pktcMtaDevTypeIdentifier,

pktcMtaDevProvisioningState,

pktcMtaDevCertificate,

pktcMtaDevSignature,

pktcMtaDevCorrelationId,

pktcMtaDevManufacturerCertificate,

pktcMtaDevTelephonyCertificate,

pktcMtaDevServProviderCertificate,

pktcMtaDevLocalSystemCertificate,

pktcMtaDevKerberosRealm,

pktcMtaDevTgsLocation,

pktcMtaDevKerbPrincipalName,

pktcMtaDevServGracePeriod,

pktcMtaDevKeyMgmtTimeout1,

pktcMtaDevKeyMgmtTimeout2,

pktcMtaDevServerBootState,

pktcMtaDevServerDhcp,

pktcMtaDevSnmpEntity,

pktcMtaDevEvControl,

pktcMtaDevEvSyslog,

pktcMtaDevEvThrottleAdminStatus,

pktcMtaDevEvThrottleInhibited,

pktcMtaDevEvThrottleThreshold,

pktcMtaDevEvThrottleInterval,

pktcMtaDevEvReporting,

pktcMtaDevEvFirstTime,

pktcMtaDevEvLastTime,

pktcMtaDevEvCounts,

pktcMtaDevEvLevel,

pktcMtaDevEvId,

pktcMtaDevEvText

}

STATUS    current

DESCRIPTION

"Group of objects for IPCablecom MTA MIB."

::= { pktcMtaGroups 1 }

END

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |