|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | J.169 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (03/2001) |
|  | SERIES J: CABLE NETWORKS AND TRANSMISSION OF TELEVISION, SOUND PROGRAMME AND OTHER MULTIMEDIA SIGNALS IPCablecom |  |  |  |
|  | IPCablecom network call signalling (NCS) MIB requirements |  |  |  |
|  | ITU-T Recommendation J.169 (Formerly CCITT Recommendation) |  |  |  |

ITU-T J-SERIES  RECOMMENDATIONS

CABLE NETWORKS AND TRANSMISSION OF TELEVISION, SOUND PROGRAMME AND OTHER MULTIMEDIA SIGNALS

| General Recommendations | J.1–J.9 |
| --- | --- |
| General specifications for analogue sound-programme transmission | J.10–J.19 |
| Performance characteristics of analogue sound-programme circuits | J.20–J.29 |
| Equipment and lines used for analogue sound-programme circuits | J.30–J.39 |
| Digital encoders for analogue sound-programme signals | J.40–J.49 |
| Digital transmission of sound-programme signals | J.50–J.59 |
| Circuits for analogue television transmission | J.60–J.69 |
| Analogue television transmission over metallic lines and interconnection with radio-relay links | J.70–J.79 |
| Digital transmission of television signals | J.80–J.89 |
| Ancillary digital services for television transmission | J.90–J.99 |
| Operational requirements and methods for television transmission | J.100–J.109 |
| Interactive systems for digital television distribution | J.110–J.129 |
| Transport of MPEG-2 signals on packetised networks | J.130–J.139 |
| Measurement of the quality of service | J.140–J.149 |
| Digital television distribution through local subscriber networks | J.150–J.159 |
| IPCablecom | J.160–J.179 |
| Miscellaneous | J.180–J.199 |
| Application for Interactive Digital Television | J.200–J.209 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation J.169 IPCablecom network call signalling (NCS) MIB requirements

- Summary This Recommendation describes the IPCablecom Network Call Signalling (NCS) MIB requirements.

- Source ITU-T Recommendation J.169 was prepared by ITU-T Study Group 9 (2001-2004) and approved under the WTSA Resolution 1 procedure on 9 March 2001.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2002

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from ITU.

CONTENTS

Page

1	Scope		1

2	References		1

2.1	Normative references		1

2.2	Informative reference		1

3	Terms and definitions		1

4	Abbreviations		1

5	Requirements		2

ITU-T Recommendation J.169

IPCablecom network call signalling (NCS) MIB requirements

# 1	Scope

This Recommendation describes the IPCablecom network call signalling (NCS) MIB requirements.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revisions; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

## 2.1	Normative references

–	ITU-T J.166 (2001), IPCablecom management information base (MIB) framework.

–	ITU-T J.162 (2001), Network call signalling protocol for the delivery of time-critical services over cable television networks using cable modems.

–	ITU-T J.167 (2001), Media terminal adapter (MTA) device provisioning requirements for the delivery of real-time services over cable television networks using cable modems.

## 2.2	Informative reference

–	ITU-T J.160 (Draft), Architectural framework for the delivery of time-critical services over cable television networks using cable modems.

# 3	Terms and definitions

This Recommendation defines the following term:

3.1	IPCablecom: An ITU-T project that includes an architecture and a series of Recommendations that enable the delivery of real-time services (such as telephony) over the cable television networks using cable modems.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

MIB	Management Information Base

MTA	Media Terminal Adapter

NCS	Network Call Signalling

SNMP	Simple Network Management Protocol

# 5	Requirements

This clause defines the mandatory syntax of the IPCablecom MTA MIB. It follows the IETF Simple Network Management Protocol (SNMP) for defining the managed objects. The MIB is organized as follows:

•	objects used for codecs;

•	objects used for general signalling related definitions;

•	objects used for endpoint-specific signalling information.

The syntax is given hereafter.

PKTC-SIG-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY,

OBJECT-TYPE,

Integer32,

BITS

FROM SNMPv2-SMI

TEXTUAL-CONVENTION,

RowStatus,

DisplayString,

TruthValue

FROM SNMPv2-TC

OBJECT-GROUP,

MODULE-COMPLIANCE

FROM SNMPv2-CONF

SnmpAdminString

FROM SNMP-FRAMEWORK-MIB

clabProjIPCablecom

FROM CLAB-DEF-MIB

ifIndex

FROM IF-MIB;

pktcSigMib MODULE-IDENTITY

LAST-UPDATED "200006260000Z" -- June 26, 2000

ORGANIZATION "IPCablecom OSS Group"

CONTACT-INFO

DESCRIPTION

"This MIB module supplies the basic management

object for the PacketCable Signalling

protocols. This version of the MIB includes

common signalling and Network Call Signalling

(NCS) related signalling objects."

::= { clabProjPacketCable 2 }

-- Textual Conventions

PktcCodecType ::= TEXTUAL-CONVENTION	STATUS	current

DESCRIPTION "These are the various types of codecs that

May be supported."

SYNTAX	INTEGER {

other		(1),

unknown 		(2),

g729			(3),

g729a		(4),

g729e		(5),

g711mu 		(6),

g726			(7),

g728 		(8),

g711a		(9)

}

PktcRingCadence ::= TEXTUAL-CONVENTION

STATUS 	current

DESCRIPTION

"These are the ring cadence durations that are

supported. 200 ms for each interval. Each interval is

represented by one bit. 0 is no tone, 1 is tone."

SYNTAX BITS {

interval1 (0),

interval2 (1),

interval3 (2),

interval4 (3),

interval5 (4),

interval6 (5),

interval7 (6),

interval8 (7),

interval9 (8),

interval10 (9),

interval11 (10),

interval12 (11),

interval13 (12),

interval14 (13),

interval15 (14),

interval16 (15),

interval17 (16),

interval18 (17),

interval19 (18),

interval20 (19),

interval21 (20),

interval22 (21),

interval23 (22),

interval24 (23),

interval25 (24),

interval26 (25),

interval27 (26),

interval28 (27),

interval29 (28),

interval30 (29)

}

PktcSigType ::= TEXTUAL-CONVENTION

STATUS	current

DESCRIPTION "These are the various types of signalling that

may be supported.

ncs - network call signalling a derivation of

MGCP (Media Gateway Control Protocol)

version 1.0

dcs - distributed call signalling a derivation

of SIP (Session Initiation Protocol)

RFC 2543"

SYNTAX	INTEGER {

other(1),

unknown(2),

ncs(3),

dcs(4)

}

pktcSigMibObjects 				OBJECT IDENTIFIER ::= { pktcSigMib 1 }

pktcSigDevConfigObjects 		OBJECT IDENTIFIER ::= { pktcSigMibObjects 1 }

pktcNcsEndPntConfigObjects 		OBJECT IDENTIFIER ::= { pktcSigMibObjects 2 }

pktcSigEndPntConfigObjects 		OBJECT IDENTIFIER ::= { pktcSigMibObjects 3 }

pktcDcsEndPntConfigObjects 		OBJECT IDENTIFIER ::= { pktcSigMibObjects 4 }

--

-- 				The pktcSigDevCodecTable defines the codecs supported by this

-- 				Media Terminal Adapter (MTA). There is one entry for each

-- 				codecs supported.

--

pktcSigDevCodecTable 	OBJECT-TYPE

SYNTAX 			SEQUENCE OF PktcSigDevCodecEntry

MAX-ACCESS 		not-accessible

STATUS 			current

DESCRIPTION

"This table describes the MTA supported codec types."

::= { pktcSigDevConfigObjects 1 }

pktcSigDevCodecEntry OBJECT-TYPE

SYNTAX 			PktcSigDevCodecEntry

MAX-ACCESS 		not-accessible

STATUS 			current

DESCRIPTION

"List of supported codecs types for the MTA."

INDEX { pktcSigDevCodecIndex }

::= { pktcSigDevCodecTable 1 }

PktcSigDevCodecEntry ::= SEQUENCE {

pktcSigDevCodecIndex INTEGER,

pktcSigDevCodecType 	PktcCodecType

}

pktcSigDevCodecIndex OBJECT-TYPE

SYNTAX	INTEGER 			(1..16383)

MAX-ACCESS 		not-accessible

STATUS 				current

DESCRIPTION

"The index value which uniquely identifies an entry

in the pktcSigDevCodecTable."

::= { pktcSigDevCodecEntry 1 }

pktcSigDevCodecType OBJECT-TYPE

SYNTAX 		PktcCodecType

MAX-ACCESS 	read-only

STATUS 		current

DESCRIPTION

"A codec type supported by this MTA."

::= { pktcSigDevCodecEntry 2 }

--

-- 	These are the common signalling related definitions that affect the

-- 	entire MTA device.

--

pktcSigDevEchoCancellation 	OBJECT-TYPE

SYNTAX 			TruthValue

MAX-ACCESS 		read-only

STATUS 			current

DESCRIPTION

"This object specifies if the device is capable

of echo cancellation."

::= { pktcSigDevConfigObjects 2 }

pktcSigDevSilenceSuppression 	OBJECT-TYPE

SYNTAX 			TruthValue

MAX-ACCESS 		read-only

STATUS 			current

DESCRIPTION

"This object specifies if the device is capable of

silence suppression (Voice Activity Detection)."

::= { pktcSigDevConfigObjects 3 }

pktcSigDevConnectionMode 	OBJECT-TYPE

SYNTAX BITS {

voice(0),

fax(1),

modem(2)

}

MAX-ACCESS 	read-only

STATUS 		current

DESCRIPTION

"This object specifies the connection modes that the

MTA device can support."

::= { pktcSigDevConfigObjects 4 }

--

-- 		In the United States Ring Cadences 0, 6, and 7 are custom

-- 		ring cadences definable by the user. The following three

-- 		objects are used for these definitions.

--

pktcSigDevR0Cadence 	OBJECT-TYPE

SYNTAX 		PktcRingCadence

MAX-ACCESS 	read-write

STATUS 		current

DESCRIPTION

"This object specifies ring cadence 0 (a user-defined

field) where each bit (least significant bit)

represents a duration of 200 milliseconds (6 seconds

total)."

::= { pktcSigDevConfigObjects 5 }

pktcSigDevR6Cadence OBJECT-TYPE

SYNTAX 		PktcRingCadence

MAX-ACCESS 	read-write

STATUS 		current

DESCRIPTION

"This object specifies ring cadence 6 (a user-defined

field) where each bit (least significant bit)

represents a duration of 200 milliseconds (6 seconds

total)."

::= { pktcSigDevConfigObjects 6 }

pktcSigDevR7Cadence 	OBJECT-TYPE

SYNTAX 		PktcRingCadence

MAX-ACCESS 	read-write

STATUS 		current

DESCRIPTION

"This object specifies ring cadence 7 (a user-defined

field) where each bit (least significant bit)

represents a duration of 200 milliseconds (6 seconds

total)."

::= { pktcSigDevConfigObjects 7 }

pktcSigDefCallSigTos 	OBJECT-TYPE

SYNTAX 		Integer32 (0..63)

MAX-ACCESS 	read-write

STATUS 		current

DESCRIPTION

"The default value used in the IP header for setting the

Type of Service (TOS) value for call signalling."

REFERENCE

"Refer to 6.4.2 of ITU-T Recommendation J.162"

DEFVAL	{ 0 }

::= { pktcSigDevConfigObjects 8 }

pktcSigDefMediaStreamTos OBJECT-TYPE

SYNTAX 		Integer32 (0..63)

MAX-ACCESS 	read-write

STATUS 		current

DESCRIPTION

"The default value used in the IP header for setting

the Type of Service (TOS) value for media stream packets."

REFERENCE

"Refer to 6.4.2 of ITU-T Recommendation J.162"

DEFVAL	{ 0 }

::= { pktcSigDevConfigObjects 9 }

pktcSigTosFormatSelector OBJECT-TYPE

SYNTAX	INTEGER {

ipv4TOSOctet(1),

dscpCodepoint(2)

}

MAX-ACCESS 	read-write

STATUS 		current

DESCRIPTION

"The format of the default signalling and media

Type of Service (TOS) values."

::= { pktcSigDevConfigObjects 10 }

--

-- 		pktcSigCapabilityTable - This table defines the valid signalling

-- 		types supported by this MTA.

--

pktcSigCapabilityTable 	OBJECT-TYPE

SYNTAX 		SEQUENCE OF PktcSigCapabilityEntry

MAX-ACCESS 	not-accessible

STATUS 		current

DESCRIPTION

"This table describes the signalling types

by this MTA."

::= { pktcSigDevConfigObjects 11 }

pktcSigCapabilityEntry 	OBJECT-TYPE

SYNTAX 		PktcSigCapabilityEntry

MAX-ACCESS 	not-accessible

STATUS 		current

DESCRIPTION

"Entries in pktcMtaDevSigCapabilityTable -

List of supported signalling types, versions

and vendor extensions for this MTA. Each

entry in the list provides for one signalling

type and version combination. If the device

supports multiple versions of the same

signalling type - it will require mutiple

entries."

INDEX { pktcSignallingIndex }

::= { pktcSigCapabilityTable 1 }

PktcSigCapabilityEntry ::= SEQUENCE {

pktcSignallingIndex 	INTEGER,

pktcSignallingType 	PktcSigType,

pktcSignallingVersion 	SnmpAdminString,

pktcSignallingVendorExtension SnmpAdminString

}

pktcSignallingIndex 	OBJECT-TYPE

SYNTAX		INTEGER (1..16383)

MAX-ACCESS 	not-accessible

STATUS 		current

DESCRIPTION

"The index value which uniquely identifies

an entry in the pktcSigCapabilityTable."

::= { pktcSigCapabilityEntry 1 }

pktcSignallingType 	OBJECT-TYPE

SYNTAX 		PktcSigType

MAX-ACCESS 	read-only

STATUS 		current

DESCRIPTION

"The Type indentifies the type of signalling

used, this can be NCS, DCS, etc. This value

has to be associated with a single signalling

version - reference pktcMtaDevSignallingVersion."

::= { pktcSigCapabilityEntry 2 }

pktcSignallingVersion 	OBJECT-TYPE

SYNTAX 		SnmpAdminString

MAX-ACCESS 	read-only

STATUS 		current

DESCRIPTION

"Provides the version of the signalling type -

reference pktcSignallingType. Exmaples

would be 1.0 or 2.33 etc."

::= { pktcSigCapabilityEntry 3 }

pktcSignallingVendorExtension 	OBJECT-TYPE

SYNTAX 		SnmpAdminString

MAX-ACCESS 	read-only

STATUS 		current

DESCRIPTION

"The vendor extension allows vendors to

provide a list of additional capabilities,

vendors can decide how to encode these

Extensions, although space separated text is

suggested."

::= { pktcSigCapabilityEntry 4 }

--

-- The following table will provide endpoint configuration

-- information that is common to all signalling Protocols.

-- Currently only the signalling index is present in an effort

-- not to deprecate any MIB objects.

--

pktcSigEndPntConfigTable 	OBJECT-TYPE

SYNTAX 		SEQUENCE OF PktcSigEndPntConfigEntry

MAX-ACCESS 	not-accessible

STATUS 		current

DESCRIPTION

"This table describes the packet cable

EndPoint selected signalling type. The number of

entries in this table represents the

number of provisioned end points.

For each conceptual row of pktcSigEndPntConfigTable

defined, an associated row MUST be defined in one

on the specific signalling tables such as

pktcNcsEndPntConfigTable."

::= { pktcSigEndPntConfigObjects 1 }

pktcSigEndPntConfigEntry OBJECT-TYPE

SYNTAX 		PktcSigEndPntConfigEntry

MAX-ACCESS 	not-accessible

STATUS 		current

DESCRIPTION

"Entries in pktcSigEndPntConfigTable -

Each entry describes what signalling type

a particular endpoint uses."

INDEX { ifIndex }

::= { pktcSigEndPntConfigTable 1 }

pktcSigEndPntConfigEntry ::= SEQUENCE {

pktcSigEndPntCapabilityIndex 	INTEGER

}

pktcSigEndPntCapabilityIndex 	OBJECT-TYPE

SYNTAX 		INTEGER (1..16383)

MAX-ACCESS 	read-create

STATUS 		current

DESCRIPTION

"The associated index value in

the pktcSigCapablityTable."

::= { pktcSigEndPntConfigEntry 1 }

--

-- The NCS End Point Config Table is used to define attributes that

-- are specific to connection EndPoints.

--

--

pktcNcsEndPntConfigTable OBJECT-TYPE

SYNTAX 		SEQUENCE OF PktcNcsEndPntConfigEntry

MAX-ACCESS 	not-accessible

STATUS 		current

DESCRIPTION

"This table describes the packet cable NCS EndPoint

configuration. The number of entries in this table

represents the number of provisioned NCS endpoints."

::= { pktcNcsEndPntConfigObjects 1}

pktcNcsEndPntConfigEntry 	OBJECT-TYPE

SYNTAX 		PktcNcsEndPntConfigEntry

MAX-ACCESS 	not-accessible

STATUS 		current

DESCRIPTION

"List of attributes for a single packet cable endpoint

interface."

INDEX { ifIndex }

::= { pktcNcsEndPntConfigTable 1 }

PktcNcsEndPntConfigEntry ::= SEQUENCE {

pktcNcsEndPntConfigCallAgentId 			DisplayString,

pktcNcsEndPntConfigCallAgentUdpPort 		INTEGER,

pktcNcsEndPntConfigPartialDialTO 			INTEGER,	pktcNcsEndPntConfigCriticalDialTO			INTEGER,

pktcNcsEndPntConfigBusyToneTO			INTEGER,

pktcNcsEndPntConfigDialToneTO			INTEGER,

pktcNcsEndPntConfigMessageWaitingTO		INTEGER,

pktcNcsEndPntConfigOffHookWarnToneTO		INTEGER,

pktcNcsEndPntConfigRingingTO				INTEGER,

pktcNcsEndPntConfigRingBackTO			INTEGER,

pktcNcsEndPntConfigReorderToneTO			INTEGER,

pktcNcsEndPntConfigStutterDialToneTO		INTEGER,

pktcNcsEndPntConfigTSMax				INTEGER,

pktcNcsEndPntConfigMax1					INTEGER,

pktcNcsEndPntConfigMax2					INTEGER,

pktcNcsEndPntConfigMax1QEnable 			TruthValue,

pktcNcsEndPntConfigMax2QEnable 			TruthValue,

pktcNcsEndPntConfigMWD					INTEGER,

pktcNcsEndPntConfigTdinit				INTEGER,

pktcNcsEndPntConfigTdmin				INTEGER,

pktcNcsEndPntConfigTdmax				INTEGER,

pktcNcsEndPntConfigRtoMax				INTEGER,

pktcNcsEndPntConfigRtoInit				INTEGER,

pktcNcsEndPntConfigLongDurationKeepAlive	INTEGER,

pktcNcsEndPntConfigThist				INTEGER,

pktcNcsEndPntConfigStatus 			RowStatus

}

pktcNcsEndPntConfigCallAgentId 	OBJECT-TYPE

SYNTAX		DisplayString(SIZE (0..255))

MAX-ACCESS	read-create

STATUS		current

DESCRIPTION

"This object contains a string indicating the call agent name.

The call agent name can be a fully qualified domain name or

an IP address. Refer to RFC 821 for details."

::= { pktcNcsEndPntConfigEntry 1 }

pktcNcsEndPntConfigCallAgentUdpPort 	OBJECT-TYPE

SYNTAX 		INTEGER (1025..65535)

MAX-ACCESS	read-create

STATUS		current

DESCRIPTION

"This object contains the call agent User Datagram Protocol

(UDP) port that is being used for this instance of call

signalling."

REFERENCE

"Refer to 7.2.1.3 of ITU-T Recommendation J.162"

DEFVAL	{ 2427 }

::= { pktcNcsEndPntConfigEntry 2 }

pktcNcsEndPntConfigPartialDialTO OBJECT-TYPE

SYNTAX		INTEGER

UNITS		"seconds"

MAX-ACCESS	read-create

STATUS		current

DESCRIPTION

"This object contains maximum value of the partial

dial time out."

REFERENCE

"Refer to Annex A of ITU-T Recommendation J.162"

DEFVAL	{ 16 }

::= { pktcNcsEndPntConfigEntry 3 }

pktcNcsEndPntConfigCriticalDialTO 	OBJECT-TYPE

SYNTAX 		INTEGER

UNITS 		"seconds"

MAX-ACCESS 	read-create

STATUS 		current

DESCRIPTION

"This object contains the maximum value of the critical

dial time out."

REFERENCE

"Refer to Annex A of ITU-T Recommendation J.162"

DEFVAL 	{ 4 }

::= { pktcNcsEndPntConfigEntry 4 }

pktcNcsEndPntConfigBusyToneTO OBJECT-TYPE

SYNTAX 		INTEGER

UNITS 		"seconds"

MAX-ACCESS	read-create

STATUS 		current

DESCRIPTION

"This object contains the time-out value for busy tone."

REFERENCE

"Refer to Annex A of ITU-T Recommendation J.162"

DEFVAL	{ 30 }

::= { pktcNcsEndPntConfigEntry 5 }

pktcNcsEndPntConfigDialToneTO OBJECT-TYPE

SYNTAX 		INTEGER

UNITS		"seconds"

MAX-ACCESS	read-create

STATUS 		current

DESCRIPTION

"This object contains the time-out value for dial tone."

REFERENCE

"Refer to Annex A of ITU-T Recommendation J.162"

DEFVAL	{ 16 }

::= { pktcNcsEndPntConfigEntry 6 }

pktcNcsEndPntConfigMessageWaitingTO 	OBJECT-TYPE

SYNTAX 		INTEGER

UNITS		"seconds"

MAX-ACCESS 	read-create

STATUS 		current

DESCRIPTION

"This object contains the time-out value for message

waiting indicator."

REFERENCE

"Refer to Annex A of ITU-T Recommendation J.162"

DEFVAL	{ 16 }

::= { pktcNcsEndPntConfigEntry 7 }

pktcNcsEndPntConfigOffHookWarnToneTO 	OBJECT-TYPE

SYNTAX 		INTEGER

UNITS 		"seconds"

MAX-ACCESS 	read-create

STATUS 		current

DESCRIPTION

"This object contains the time-out value for

the off hook Warning tone."

REFERENCE

"Refer to Annex A of ITU-T Recommendation J.162"

DEFVAL 	{ 0 }

::= { pktcNcsEndPntConfigEntry 8 }

pktcNcsEndPntConfigRingingTO OBJECT-TYPE

SYNTAX 		INTEGER

UNITS 		"seconds"

MAX-ACCESS 	read-create

STATUS 		current

DESCRIPTION

"This object contains the time-out value for ringing."

REFERENCE

"Refer to Annex A of ITU-T Recommendation J.162"

DEFVAL 	{ 180 }

::= { pktcNcsEndPntConfigEntry 9 }

pktcNcsEndPntConfigRingBackTO 	OBJECT-TYPE

SYNTAX 		INTEGER

UNITS 		"seconds"

MAX-ACCESS 	read-create

STATUS 		current

DESCRIPTION

"This object contains the time-out value for ring back."

REFERENCE

"Refer to Annex A of ITU-T Recommendation J.162"

DEFVAL 	{ 180 }

::= { pktcNcsEndPntConfigEntry 10 }

pktcNcsEndPntConfigReorderToneTO OBJECT-TYPE

SYNTAX 		INTEGER

UNITS 		"seconds"

MAX-ACCESS 	read-create

STATUS 		current

DESCRIPTION

"This object contains the time-out value for reorder tone."

REFERENCE

"Refer to Annex A of ITU-T Recommendation J.162"

DEFVAL 	{ 30 }

::= { pktcNcsEndPntConfigEntry 11 }

pktcNcsEndPntConfigStutterDialToneTO OBJECT-TYPE

SYNTAX 		INTEGER

UNITS 		"seconds"

MAX-ACCESS 	read-create

STATUS 		current

DESCRIPTION

"This object contains the timeout value for stutter dial tone."

REFERENCE

"Refer to Annex A of ITU-T Recommendation J.162"

DEFVAL	{ 16 }

::= { pktcNcsEndPntConfigEntry 12 }

pktcNcsEndPntConfigTSMax OBJECT-TYPE

SYNTAX		INTEGER

MAX-ACCESS	read-create

STATUS		current

DESCRIPTION

"This object contains the max time in seconds since the

sending of the initial datagram."

REFERENCE

"Refer to 6.4.2 of ITU-T Recommendation J.162"

DEFVAL	{ 20 }

::= { pktcNcsEndPntConfigEntry 13 }

pktcNcsEndPntConfigMax1 OBJECT-TYPE

SYNTAX		INTEGER

MAX-ACCESS	read-create

STATUS		current

DESCRIPTION

"This object contains the suspicious error threshold

for signalling messages."

REFERENCE

"Refer to 6.4.2 of ITU-T Recommendation J.162"

DEFVAL	{ 5 }

::= { pktcNcsEndPntConfigEntry 14 }

pktcNcsEndPntConfigMax2 OBJECT-TYPE

SYNTAX		INTEGER

MAX-ACCESS	read-create

STATUS		current

DESCRIPTION

"This object contains the disconnect error

threshold for signalling messages."

REFERENCE

"Refer to 6.4.2 of ITU-T Recommendation J.162"

DEFVAL	{ 7 }

::= { pktcNcsEndPntConfigEntry 15 }

pktcNcsEndPntConfigMax1QEnable 	OBJECT-TYPE

SYNTAX 		TruthValue

MAX-ACCESS	read-create

STATUS		current

DESCRIPTION

"This object enables/disables the Max1 Domain Name

Server (DNS) query operation when Max1 expires."

DEFVAL	{ true }

::= { pktcNcsEndPntConfigEntry 16 }

pktcNcsEndPntConfigMax2QEnable OBJECT-TYPE

SYNTAX 		TruthValue

MAX-ACCESS	read-create

STATUS		current

DESCRIPTION

"This object enables/disables the Max2 DNS query

operation when Max2 expires."

DEFVAL	{ true }

::= { pktcNcsEndPntConfigEntry 17 }

pktcNcsEndPntConfigMWD OBJECT-TYPE

SYNTAX		INTEGER

UNITS 		"seconds"

MAX-ACCESS	read-create

STATUS		current

DESCRIPTION

"Maximum Waiting Delay (MWD) contains the maximum

number of seconds a MTA waits after a restart."

REFERENCE

"Refer to 6.4.3.5 of ITU-T Recommendation J.162"

DEFVAL	{ 600 }

::= { pktcNcsEndPntConfigEntry 18 }

pktcNcsEndPntConfigTdinit OBJECT-TYPE

SYNTAX		INTEGER

UNITS 		"seconds"

MAX-ACCESS	read-create

STATUS		current

DESCRIPTION

"This object contains the initial number of seconds

a MTA waits after a disconnect."

REFERENCE

"Refer to 6.4.3.6 of ITU-T Recommendation J.162"

DEFVAL	{ 15 }

::= { pktcNcsEndPntConfigEntry 19 }

pktcNcsEndPntConfigTdmin OBJECT-TYPE

SYNTAX		INTEGER

UNITS 		"seconds"

MAX-ACCESS	read-create

STATUS		current

DESCRIPTION

"This object contains the minimum number of seconds a

MTA waits after a disconnect."

REFERENCE

"Refer to 6.4.3.6 of ITU-T Recommendation J.162"

DEFVAL	{ 15 }

::= { pktcNcsEndPntConfigEntry 20 }

pktcNcsEndPntConfigTdmax OBJECT-TYPE

SYNTAX	INTEGER

UNITS 	"seconds"

STATUS	current

DESCRIPTION

"This object contains the maximum number of seconds

a MTA waits after a disconnect."

REFERENCE

"Refer to 6.4.3.6 of ITU-T Recommendation J.162"

DEFVAL	{ 600 }

::= { pktcNcsEndPntConfigEntry 21 }

pktcNcsEndPntConfigRtoMax OBJECT-TYPE

SYNTAX		INTEGER

UNITS		"seconds"

MAX-ACCESS	read-create

STATUS	current

DESCRIPTION

"This object contains the maximum number of seconds

for the retansmission timer."

REFERENCE

"Refer to 7.5.2 of ITU-T Recommendation J.162"

DEFVAL	{ 4 }

::= { pktcNcsEndPntConfigEntry 22 }

pktcNcsEndPntConfigRtoInit OBJECT-TYPE

SYNTAX		INTEGER

UNITS		"milliseconds"

MAX-ACCESS	read-create

STATUS		current

DESCRIPTION

"This object contains the initial number of seconds

for the retransmission timer."

REFERENCE

"Refer to 7.5.2 of ITU-T Recommendation J.162"

DEFVAL	{ 200 }

::= { pktcNcsEndPntConfigEntry 23 }

pktcNcsEndPntConfigLongDurationKeepAlive OBJECT-TYPE

SYNTAX		INTEGER

UNITS 		"minutes"

MAX-ACCESS	read-create

STATUS		current

DESCRIPTION

"Specifies a timeout value in minutes for sending

long duration call notification message."

REFERENCE

"Refer to Annex A of ITU-T Recommendation J.162"

DEFVAL	{ 60 }

::= { pktcNcsEndPntConfigEntry 24 }

pktcNcsEndPntConfigThist 	OBJECT-TYPE

SYNTAX		INTEGER

UNITS 		"seconds"

MAX-ACCESS	read-create

STATUS		current

DESCRIPTION

"Timeout period in seconds before no response is declared."

REFERENCE

"Refer to 6.4.2 of ITU-T Recommendation J.162"

DEFVAL	{ 30 }

::= { pktcNcsEndPntConfigEntry 25

pktcNcsEndPntConfigStatus OBJECT-TYPE

SYNTAX 		RowStatus

MAX-ACCESS	read-create

STATUS		current

DESCRIPTION

"This object contains the Row Status associated with

the pktsNcsEndPntTable."

::= { pktcNcsEndPntConfigEntry 26 }

--

-- notification group is for future extension.

--

pktcSigNotification OBJECT IDENTIFIER 	::= { pktcSigMib 2 0 }

pktcSigConformance OBJECT IDENTIFIER 		::= { pktcSigMib 3 }

pktcSigCompliances OBJECT IDENTIFIER 		::= { pktcSigConformance 1 }

pktcSigGroups OBJECT IDENTIFIER 			::= { pktcSigConformance 2 }

-- compliance statements

pktcSigBasicCompliance MODULE-COMPLIANCE

STATUS	current

DESCRIPTION

"The compliance statement for devices that implement Signalling

on the MTA."

MODULE – pktcSigMib

-- unconditionally mandatory groups

MANDATORY-GROUPS {

pktcSigGroup

}

GROUP pktcNcsGroup

DESCRIPTION

"This group is mandatory for any MTA implementing NCS signalling"

::={ pktcSigCompliances 1 }

-- units of conformance

pktcSigGroup OBJECT-GROUP

OBJECTS {

pktcSigDevCodecType,

pktcSigDevEchoCancellation,

pktcSigDevSilenceSuppression,

pktcSigDevConnectionMode,

pktcSigDevR0Cadence,

pktcSigDevR6Cadence,

pktcSigDevR7Cadence,

pktcSigDefCallSigTos,

pktcSigDefMediaStreamTos,

pktcSigTosFormatSelector,

pktcSignallingType,

pktcSignallingVersion,

pktcSignallingVendorExtension,

pktcSigEndPntCapabilityIndex

}

STATUS	current

DESCRIPTION

"Group of objects for the common portion of the

IPCablecom Signalling MIB."

::= { pktcSigGroups 1 }

pktcNcsGroup OBJECT-GROUP

OBJECTS {

pktcNcsEndPntConfigCallAgentId,

pktcNcsEndPntConfigCallAgentUdpPort,

pktcNcsEndPntConfigPartialDialTO,

pktcNcsEndPntConfigCriticalDialTO,

pktcNcsEndPntConfigBusyToneTO,

pktcNcsEndPntConfigDialToneTO,

pktcNcsEndPntConfigMessageWaitingTO,

pktcNcsEndPntConfigOffHookWarnToneTO,

pktcNcsEndPntConfigRingingTO,

pktcNcsEndPntConfigRingBackTO,

pktcNcsEndPntConfigReorderToneTO,

pktcNcsEndPntConfigStutterDialToneTO,

pktcNcsEndPntConfigTSMax,

pktcNcsEndPntConfigMax1,

pktcNcsEndPntConfigMax2,

pktcNcsEndPntConfigMax1QEnable,

pktcNcsEndPntConfigMax2QEnable,

pktcNcsEndPntConfigMWD,

pktcNcsEndPntConfigTdinit,

pktcNcsEndPntConfigTdmin,

pktcNcsEndPntConfigTdmax,

pktcNcsEndPntConfigRtoMax,

pktcNcsEndPntConfigRtoInit,

pktcNcsEndPntConfigLongDurationKeepAlive,

pktcNcsEndPntConfigThist,

pktcNcsEndPntConfigStatus

}

STATUS	current

DESCRIPTION

"Group of objects for the NCS portion of the

IPCablecom Signalling MIB. This is mandatory for

NCS signalling."

::= { pktcSigGroups 2 }

END

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |