|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | J.176 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (07/2002) |
|  | SERIES J: CABLE NETWORKS AND TRANSMISSION OF TELEVISION, SOUND PROGRAMME AND OTHER MULTIMEDIA SIGNALS IPCablecom |  |  |  |
|  | IPCablecom management event mechanism MIB |  |  |  |
|  | ITU-T Recommendation J.176 |  |  |  |

ITU-T J-SERIES  RECOMMENDATIONS

CABLE NETWORKS AND TRANSMISSION OF TELEVISION, SOUND PROGRAMME AND OTHER MULTIMEDIA SIGNALS

| General Recommendations | J.1–J.9 |
| --- | --- |
| General specifications for analogue sound-programme transmission | J.10–J.19 |
| Performance characteristics of analogue sound-programme circuits | J.20–J.29 |
| Equipment and lines used for analogue sound-programme circuits | J.30–J.39 |
| Digital encoders for analogue sound-programme signals | J.40–J.49 |
| Digital transmission of sound-programme signals | J.50–J.59 |
| Circuits for analogue television transmission | J.60–J.69 |
| Analogue television transmission over metallic lines and interconnection with radio-relay links | J.70–J.79 |
| Digital transmission of television signals | J.80–J.89 |
| Ancillary digital services for television transmission | J.90–J.99 |
| Operational requirements and methods for television transmission | J.100–J.109 |
| Interactive systems for digital television distribution | J.110–J.129 |
| Transport of MPEG-2 signals on packetised networks | J.130–J.139 |
| Measurement of the quality of service | J.140–J.149 |
| Digital television distribution through local subscriber networks | J.150–J.159 |
| IPCablecom | J.160–J.179 |
| Miscellaneous | J.180–J.199 |
| Application for Interactive Digital Television | J.200–J.209 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation J.176 IPCablecom management event mechanism MIB

Summary This Recommendation defines the MIB for Management Event Mechanism that IPCablecom elements can use to report to management systems and/or local logs asynchronous events that indicate malfunction situations and notification about important non-fault situation.

Source ITU-T Recommendation J.176 was prepared by ITU-T Study Group 9 (2001-2004) and approved under the WTSA Resolution 1 procedure on 29 July 2002.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2002

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS

Page

1	Scope		1

2	References		1

2.1	Normative references		1

2.2	Informative		1

3	Terms and definitions		2

4	Abbreviations and acronyms		2

5	Conventions		2

6	IPCablecom Management Event MIB		3

ITU-T Recommendation J.176

IPCablecom management event mechanism MIB

# 1	Scope

The Management Event MIB provides a common data and format definition for events (informative, alarm, etc). It also specifies by what means events are transmitted. Use of a common event mechanism facilitates management of the MTA in a multi-vendor environment and provides a standard means to implement IPCablecom specified events. This Recommendation describes an SNMP MIB to support the management event mechanism as described in ITU-T Rec. J.172. It is intended to be implemented in the MTA, and management devices.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

## 2.1	Normative references

	ITU-T Recommendation J.172 (2002), IPCablecom management event mechanism.

## 2.2	Informative

	ITU-T Recommendation J.160 (2002), Architectural framework for the delivery of time-critical services over cable television networks using cable modems.

	ITU-T Recommendation J.162 (2001), Network call signalling protocol for the delivery of time-critical services over cable television networks using cable modems.

	ITU-T Recommendation J.167 (2001), Media terminal adapter (MTA) device provisioning requirements for the delivery of real-time services over cable television networks using cable modems.

	ITU-T Recommendation J.168 (2001), IPCablecom Media Terminal Adapter (MTA) MIB requirements.

	ITU-T Recommendation J.169 (2001), IPCablecom network call signalling (NCS) MIB requirements.

	ITU-T Recommendation J.170 (2002), IPCablecom security specification.

	IETF RFC 1906 (1996), Transport Mappings for Version 2 of the Simple Network Management Protocol (SNMPv2).

	IETF RFC 2570 (1999), Introduction to Version 3 of the Internet-standard Network Management Framework.

	IETF RFC 2571 (1999), An Architecture for Describing SNMP Management Frameworks.

	IETF RFC 2572 (1999), Message Processing and Dispatching for the Simple Network Management Protocol (SNMP).

	IETF RFC 2573 (1999), SNMP Applications.

	IETF STD0058 (RFC 2579) (1999), Textual Conventions for SMIv2.

# 3	Terms and definitions

This Recommendation defines the following term:

3.1	endpoint: A Terminal, Gateway or MCU.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations:

FQDN		Fully Qualified Domain Name

IANA		Internet Assigned Numbers Authority

IP		Internet Protocol

MAC		Media Access Control

MIB		Management Information Base

MTA		Media Terminal Adapter

SNMP		Simple Network Management Protocol

# 5	Conventions

If this Recommendation is implemented, the keywords "MUST" and "SHALL" as well as "REQUIRED" are to be interpreted as indicating a mandatory aspect of this Recommendation. The keywords indicating a certain level of significance of particular requirements that are used throughout this Recommendation are summarized below.

| "MUST" | This word or the adjective "REQUIRED" means that the item is an absolute requirement of this Recommendation. |
| --- | --- |
| "MUST NOT" | This phrase means that the item is an absolute prohibition of this Recommendation. |
| "SHOULD" | This word or the adjective "RECOMMENDED" means that there may exist valid reasons in particular circumstances to ignore this item, but the full implications should be understood and the case carefully weighed before choosing a different course. |
| "SHOULD NOT" | This phrase means that there may exist valid reasons in particular circumstances when the listed behavior is acceptable or even useful, but the full implications should be understood and the case carefully weighed before implementing any behavior described with this label. |
| "MAY" | This word or the adjective "OPTIONAL" means that this item is truly optional. One vendor may choose to include the item because a particular marketplace requires it or because it enhances the product, for example; another vendor may omit the same item. |

# 6	IPCablecom Management Event MIB

The IPCablecom Management Event MIB MUST be implemented as defined below.

PKTC-EVENT-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY,

OBJECT-TYPE,

Integer32,

Unsigned32,

NOTIFICATION-TYPE,

BITS

FROM SNMPv2-SMI

TruthValue, DisplayString, DateAndTime

FROM SNMPv2-TC

OBJECT-GROUP,

MODULE-COMPLIANCE,

InetAddressType, InetAddress

FROM INET-ADDRESS-MIB

NOTIFICATION-GROUP

FROM SNMPv2-CONF

clabProjPacketCable

FROM CLAB-DEF-MIB;

pktcEventMib MODULE-IDENTITY

LAST-UPDATED		"0203080000Z" -- 03/08/02

ORGANIZATION		"Packet Cable Provisioning/OSS Group"

CONTACT-INFO

"Matt Osman

Postal:	Cable Television Laboratories, Inc.

400 Centennial Parkway

Louisville, Colorado 80027-1266

U.S.A.

Phone:	+1 303-661-9100

Fax:		+1 303-661-9199

E-mail:	m.osman@cablelabs.com"

DESCRIPTION

"This MIB module supplies the basic management objects

for event reporting

Acknowledgements:

Rick Vetter		-	Motorola

Eugene Nechamkin	-	Broadcom"

::=  { clabProjPacketCable 3 }

--

--

pktcDevEventControl		OBJECT IDENTIFIER ::= { pktcEventMib 1 }

pktcDevEventConfig			OBJECT IDENTIFIER ::= { pktcEventMib 2 }

pktcDevEventThrottle		OBJECT IDENTIFIER ::= { pktcEventMib 3 }

pktcDevEventLocal			OBJECT IDENTIFIER ::= { pktcEventMib 4 }

pktcDevEventNotify			OBJECT IDENTIFIER ::= { pktcEventMib 5 }

pktcDevEvNotification 		OBJECT IDENTIFIER ::= { pktcEventMib 6 0 }

--

--  Event Reporting

--

--

--	Event reporting control

--

pktcDevEvControl OBJECT-TYPE

SYNTAX INTEGER {

resetLog(1),

setDefaults(2),

useConfigured(3)

}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines actions related to the event log configuration. Setting this object to resetLog(1) empties the event log.

All event log data is deleted. Setting it to setDefault(2)

restores all event priorities to their factory-default

reporting parameters. Setting it to useConfigured(3) reloads

previously configured parameters."

::= { pktcDevEventControl 1 }

pktcDevEvControlState OBJECT-TYPE

SYNTAX INTEGER {

logReset(1),

defaultsSet(2),

userConfigured(3),

processing(4)

}

MAX-ACCESS	read-only

STATUS			current

DESCRIPTION

"This object reflects the state of the device as modified in

pktcDevEvControl. Processing indicates that a state change

is underway. This object reflects the state of the device."

::= { pktcDevEventControl 2 }

pktcDevEvSyslogAddressType OBJECT-TYPE

SYNTAX      InetAddressType

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The type of Internet address of the Syslog server.

Not all address types may be supported."

::= { pktcDevEventControl 3 }

pktcDevEvSyslogAddress OBJECT-TYPE

SYNTAX			InetAddress

MAX-ACCESS	read-write

STATUS			current

DESCRIPTION

"The IP address of the Syslog server. If 0.0.0.0,

syslog transmission is inhibited. The use of FQDNs is

syntactically allowed but it is discouraged for syslog

servers since not resolving them in a timely manner

may leave the device without access to the Syslog

daemon during critical network events."

::= { pktcDevEventControl 3 }

pktcDevEvSyslogUdpPort OBJECT-TYPE

SYNTAX			Unsigned32

MAX-ACCESS	read-write

STATUS			current

DESCRIPTION

"The UDP port number the syslog device is using to send

requests to the syslog server."

DEFVAL  {514}

::= { pktcDevEventControl 4 }

--

-- Event throttling control

--

pktcDevEvThrottleAdminStatus OBJECT-TYPE

SYNTAX INTEGER {

throttlingInhibited(1),

dynamicThresholding(2),

manualThresholding(3),

eventsInhibited(4)

}

MAX-ACCESS	read-write

STATUS			current

DESCRIPTION

"Controls the transmission of traps and syslog messages

with respect to the trap pacing threshold.

throttlingInhibited(1) causes traps and syslog messages to be

transmitted without regard to the threshold settings.

dynamicThresholding(2) causes trap transmission and

syslog messages to be suppressed if the number of traps

would otherwise exceed the threshold.

manualThresholding(3) causes trap transmission to cease

at the threshold, and not resume until directed to do so.

eventsInhibited(4) causes all trap transmission and syslog

messages to be suppressed.

A single event is always treated as a single event for

threshold counting. That is, an event causing both a trap

and a syslog message is still treated as a single event.

Writing to this object resets the thresholding state.

At initial startup, this object has a default value of

throttlingInhibited(1)."

DEFVAL { throttlingInhibited }

::= { pktcDevEventThrottle 1 }

pktcDevEvThrottleInhibited OBJECT-TYPE

SYNTAX			TruthValue

MAX-ACCESS	read-only

STATUS			current

DESCRIPTION

"If true(1), trap/inform and syslog transmission is currently

inhibited due to thresholds and/or the current setting of

pktcDevEvThrottleAdminStatus. In addition, this is set to

true(1) if transmission is inhibited due to no

syslog (pktcDevEvSyslogAddress) or trap/inform

(pktcMtaDevSnmpEntity) destinations having been set."

::= { pktcDevEventThrottle 2 }

pktcDevEvThrottleThreshold OBJECT-TYPE

SYNTAX			Unsigned32

MAX-ACCESS	read-write

STATUS			current

DESCRIPTION

"Number of trap/syslog events per pktcDevEvThrottleInterval

to be transmitted before throttling.

A single event is always treated as a single event for

Threshold counting. That is, an event causing both a

trap/inform and a syslog message is still treated as a

single event.

At initial startup, this object returns 2."

DEFVAL { 2 }

::= { pktcDevEventThrottle 3 }

pktcDevEvThrottleInterval OBJECT-TYPE

SYNTAX			Integer32 (1..2147483647)

UNITS				"seconds"

MAX-ACCESS	read-write

STATUS			current

DESCRIPTION

"The interval over which the throttle threshold applies.

At initial startup, this object has a value of 1."

DEFVAL { 1 }

::= { pktcDevEventThrottle 4 }

--

-- Event configuration

--

--

-- The following table configures the reporting of the various programmable

-- events.

--

pktcDevEvProgrammableTable OBJECT-TYPE

SYNTAX			SEQUENCE OF PktcDevEvProgrammableEntry

MAX-ACCESS	not-accessible

STATUS			current

DESCRIPTION

"This table allows control of the reporting of event classes.

For each event priority, a combination of logging and

reporting mechanisms may be chosen. The mapping of event types

to priorities is vendor-dependent. Vendors may also choose to

allow the user to control that mapping through proprietary means."

::= {  pktcDevEventConfig 1 }

pktcDevEvProgrammableEntry OBJECT-TYPE

SYNTAX			PktcDevEvProgrammableEntry

MAX-ACCESS	not-accessible

STATUS			current

DESCRIPTION

"Allows configuration of the reporting mechanisms for a

programmable event, including level, report type, and text."

INDEX { pktcDevEvProgrammableId, pktcDevEvProgrammableEnterprise }

::= { pktcDevEvProgrammableTable 1 }

PktcDevEvProgrammableEntry ::= SEQUENCE {

pktcDevEvProgrammableId				Integer32,

pktcDevEvProgrammableEnterprise		Integer32,

pktcDevEvProgrammableLevel			INTEGER,

pktcDevEvProgrammableReporting		BITS,

pktcDevEvProgrammableText 			DisplayString

}

pktcDevEvProgrammableId OBJECT-TYPE

SYNTAX Integer32 (1..2147483647)

MAX-ACCESS	not-accessible

STATUS			current

DESCRIPTION

"ID for a specific programmable event to which the priority and display string are matched. These Event Ids are vendor specific or in the case of IPCablecom events defined in ITU-T Rec. J.172."

::= { pktcDevEvProgrammableEntry 1 }

pktcDevEvProgrammableEnterprise	OBJECT-TYPE

SYNTAX			Integer32  (1..2147483647)

MAX-ACCESS	read-only

STATUS			current

DESCRIPTION

"Provides the IANA enterprise number of the device manufacturer for proprietary events, and the CableLabs IANA enterprise number for PacketCable specified events."

::= { pktcDevEvProgrammableEntry 2 }

pktcDevEvProgrammableLevel OBJECT-TYPE

SYNTAX INTEGER {

critical(1),

major(2),

minor(3),

warning(4),

information(5)

}

MAX-ACCESS	read-write

STATUS			current

DESCRIPTION

"The priority level that is controlled by this entry. These are ordered from most (critical) to least (information) critical.  Each event has a particular priority level associated with it (as defined by the vendor). The levels are described as:

critical(1) – A service-affecting condition that requires immediate corrective action.

major(2) – A service-affecting condition that requires urgent corrective action.

minor(3) – A non-service-affecting fault condition which warrants corrective action in order to avoid a more serious fault.

warning(4) – A potential or impending condition which can lead to a fault;  diagnostic action is suggested.

information(5) – Normal event meant to convey information."

::= { pktcDevEvProgrammableEntry 3 }

pktcDevEvProgrammableReporting OBJECT-TYPE

SYNTAX BITS {

local(0),

traps(1),

syslog(2),

inform(3),

none(4)

}

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Defines the action to be taken on occurrence of this

event class. Implementations may not necessarily support

all options for all event classes, but at minimum must

allow traps and syslogging to be disabled. If the

local(0) bit is set, then log to the internal log, if the

traps(1) bit is set, then generate a trap, if the

syslog(2) bit is set, then send a syslog message

(assuming the syslog address is set) inform(3) bit is set,

then generate an inform, if the none(4) bit is set, then this

event is not generated."

--		DEFVAL { local }

::= { pktcDevEvProgrammableEntry 4 }

pktcDevEvProgrammableText OBJECT-TYPE

SYNTAX DisplayString(SIZE (127))

MAX-ACCESS	read-write

STATUS			current

DESCRIPTION

"Programmable event display string providing a human-readable

description of the event."

::= { pktcDevEvProgrammableEntry 5 }

-- The following table configures the reporting of the various fixed

-- events.

--

pktcDevEvFixedTable OBJECT-TYPE

SYNTAX			SEQUENCE OF PktcDevEvFixedEntry

MAX-ACCESS	not-accessible

STATUS			current

DESCRIPTION

"This table allows control of the reporting of event classes.

For each event priority, a combination of logging and

reporting mechanisms may be chosen. The mapping of event types

to priorities is vendor-dependent. Vendors may also choose to

allow the user to control that mapping through proprietary means."

::= {  pktcDevEventConfig 2 }

pktcDevEvFixedEntry OBJECT-TYPE

SYNTAX			PktcDevEvFixedEntry

MAX-ACCESS	not-accessible

STATUS 			current

DESCRIPTION

"Allows configuration of the reporting mechanisms for a

fixed event, including level, and report type."

INDEX { pktcDevEvFixedId, pktcDevEvFixedEnterprise }

::= { pktcDevEvFixedTable 1 }

PktcDevEvFixedEntry ::= SEQUENCE {

pktcDevEvFixedId			Integer32,

pktcDevEvFixedEnterprise	Integer32,

pktcDevEvFixedLevel		INTEGER,

pktcDevEvFixedReporting		BITS,

pktcDevEvFixedText 		DisplayString

}

pktcDevEvFixedId OBJECT-TYPE

SYNTAX Integer32	(1..2147483647)

MAX-ACCESS	not-accessible

STATUS			current

DESCRIPTION

"ID for a specific fixed event to which the priority and display string are matched. These Event Ids are vendor specific or in the case of IPCablecom events defined in ITU-T Rec. J.172."

::= { pktcDevEvFixedEntry 1 }

pktcDevEvFixedEnterprise	OBJECT-TYPE

SYNTAX			Integer32  (1..2147483647)

MAX-ACCESS	read-only

STATUS			current

DESCRIPTION

"Provides the IANA enterprise number of the device manufacturer for

proprietary events, and the CableLabs IANA enterprise number for

PacketCable specified events."

::= { pktcDevEvFixedEntry 2 }

pktcDevEvFixedLevel OBJECT-TYPE

SYNTAX INTEGER {

critical(1),

major(2),

minor(3),

warning(4),

information(5)

}

MAX-ACCESS	read-write

STATUS			current

DESCRIPTION

"The priority level that is controlled by this entry. These are ordered from most (critical) to least (information) critical.  Each event has a particular priority level associated with it (as defined by the vendor). The levels are described as:

critical(1) – A service-affecting condition that requires immediate corrective action.

major(2) – A service-affecting condition that requires urgent corrective action.

minor(3) – A non-service-affecting fault condition which warrants corrective action in order to avoid a more serious fault.

warning(4) – A potential or impending condition which can lead to a fault;  diagnostic action is suggested.

information(5) – Normal event meant to convey information."

::= { pktcDevEvFixedEntry 3 }

pktcDevEvFixedReporting OBJECT-TYPE

SYNTAX BITS {

local(0),

traps(1),

syslog(2),

inform(3),

none(4)

}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Defines the action to be taken on occurrence of this event class. Implementations may not necessarily support all options for all event classes, but at minimum must allow traps and syslogging to be disabled. If the local(0) bit is set, then log to the internal log, if the traps(1) bit is set, then generate a trap, if the syslog(2) bit is set, then send a syslog message (assuming the syslog address is set) inform(3) bit is set, then generate an inform, if the none(4) bit is set, then this event is not generated."

--			DEFVAL { local }

::= { pktcDevEvFixedEntry 4 }

pktcDevEvFixedText OBJECT-TYPE

SYNTAX DisplayString(SIZE (127))

MAX-ACCESS	read-only

STATUS			current

DESCRIPTION

"Fixed event display string providing a human-readable

description of the event."

::= { pktcDevEvFixedEntry 5 }

--

-- Local event table - for retrieval of events via SNMP

--

pktcDevEventTable OBJECT-TYPE

SYNTAX			SEQUENCE OF PktcDevEventEntry

MAX-ACCESS	not-accessible

STATUS			current

DESCRIPTION

"Contains a log of network and device events that may be

of interest in fault isolation and troubleshooting."

::= {  pktcDevEventLocal 1 }

pktcDevEventEntry OBJECT-TYPE

SYNTAX			PktcDevEventEntry

MAX-ACCESS	not-accessible

STATUS			current

DESCRIPTION

"Describes a network or device event that may be of

interest in fault isolation and troubleshooting.

Entries are created with the first occurrence of an event.

pktcDevEvControl can be used to clear the table.

Individual events can not be deleted."

INDEX { pktcDevEvIndex }

::= { pktcDevEventTable 1 }

PktcDevEventEntry ::= SEQUENCE {

pktcDevEvIndex			INTEGER,

pktcDevEvTime			DateAndTime,

pktcDevEvLevel			INTEGER,

pktcDevEvEnterprise		Integer32,

pktcDevEvId			Unsigned32,

pktcDevEvText			DisplayString,

pktcDevEvMacAddress		OCTET STRING,

pktcDevEvEndpointName	DisplayString

}

pktcDevEvIndex OBJECT-TYPE

SYNTAX			INTEGER (1..2147483647)

MAX-ACCESS	read-only

STATUS			current

DESCRIPTION

"Provides relative ordering of the objects in the event

log. This object will always increase except when

a) the log is reset via pktcDevEvControl,

b) the device reboots and does not implement non-volatile

storage for this log; or

c) it reaches the value 2^31.

The next entry for all the above cases is 1. This also serves as

a indicator of event sequence."

::= { pktcDevEventEntry 1 }

pktcDevEvTime		OBJECT-TYPE

SYNTAX      	DateAndTime

MAX-ACCESS  	read-only

STATUS      	current

DESCRIPTION

"Provides a human-readable description of the

time at which the event occurred."

::= { pktcDevEventEntry 2 }

pktcDevEvLevel OBJECT-TYPE

SYNTAX INTEGER {

critical(1),

major(2),

minor(3),

warning(4),

information(5)

}

MAX-ACCESS	read-only

STATUS			current

DESCRIPTION

"The priority level of this event as defined by the

vendor.  These are ordered from most serious (critical)

to least serious (debug)."

::= { pktcDevEventEntry 3 }

pktcDevEvEnterprise	OBJECT-TYPE

SYNTAX			Integer32

MAX-ACCESS	read-only

STATUS			current

DESCRIPTION

"Provides the IANA enterprise number of the device manufacturer for

proprietary events, and the CableLabs IANA enterprise number for

PacketCable specified events."

::= { pktcDevEventEntry 4 }

pktcDevEvId OBJECT-TYPE

SYNTAX			Unsigned32

MAX-ACCESS	read-only

STATUS			current

DESCRIPTION

"ID for a specific event to which the priority and display string are

matched. These Event Ids are vendor specific or in the case of

IPCablecom events defined in ITU-T Rec. J.172."

::= { pktcDevEventEntry 5 }

pktcDevEvText OBJECT-TYPE

SYNTAX			DisplayString

MAX-ACCESS	read-only

STATUS			current

DESCRIPTION

"Provides a human-readable description of the event,

including all relevant context (interface numbers,

etc.)."

::= { pktcDevEventEntry 6 }

pktcDevEvMacAddress		OBJECT-TYPE

SYNTAX						OCTET STRING

MAX-ACCESS				read-only

STATUS						current

DESCRIPTION

"Provides the MAC address of the device generating the event."

::= { pktcDevEventEntry 7 }

pktcDevEvEndpointName	 OBJECT-TYPE

SYNTAX				DisplayString

MAX-ACCESS		read-only

STATUS				current

DESCRIPTION

"This is the endpoint identifier followed by the FQDN/IP Address

of the device. This is in the form - AALN/X:FQDN/IP Address.

If the event is not specific to an endpoint, then the contents

is just the FQDN/IP address."

::= { pktcDevEventEntry 8 }

--

--	Event Data for Traps - Informs

--

pktcDevEvReportIndex			OBJECT-TYPE

SYNTAX			Integer32  (1..2147483647)

MAX-ACCESS	read-only

STATUS			current

DESCRIPTION

"Integer value starting at one that increases by one

for each event generated. Used for sequencing of events."

::= { pktcDevEventNotify 1 }

pktcDevEvReportTime		OBJECT-TYPE

SYNTAX			DateAndTime

MAX-ACCESS	read-only

STATUS			current

DESCRIPTION

"Provides a human-readable description of the time at

which the event occurred."

::= { pktcDevEventNotify 2 }

pktcDevEvReportLevel			OBJECT-TYPE

SYNTAX			INTEGER {

critical(1),

major(2),

minor(3),

warning(4),

information(5)

}

MAX-ACCESS	read-only

STATUS			current

DESCRIPTION

"The priority level of this event as defined by the

vendor.  These are ordered from most serious (critical)

to least serious (debug)."

::= { pktcDevEventNotify 3 }

pktcDevEvReportEnterprise	OBJECT-TYPE

SYNTAX			Integer32

MAX-ACCESS	read-only

STATUS			current

DESCRIPTION

"Provides the IANA enterprise number of the device

manufacturer for proprietary events, and the CableLabs

IANA enterprise number for PacketCable specified events."

::= { pktcDevEventNotify 4 }

pktcDevEvReportId	OBJECT-TYPE

SYNTAX			Unsigned32

MAX-ACCESS	read-only

STATUS			current

DESCRIPTION

"ID for a specific event to which the priority and display string are matched. These Event Ids are vendor specific or in the case of IPCablecom events defined in ITU-T Rec. J.172."

::= { pktcDevEventNotify 5 }

pktcDevEvReportText			OBJECT-TYPE

SYNTAX			DisplayString

MAX-ACCESS	read-only

STATUS			current

DESCRIPTION

"Provides a human-readable description of the event,

including all relevant context (interface numbers, etc.)."

::= { pktcDevEventNotify 6 }

pktcDevEvReportMacAddress	OBJECT-TYPE

SYNTAX			OCTET STRING

MAX-ACCESS	read-only

STATUS			current

DESCRIPTION

"Provides the MAC address of the device generating the event."

::= { pktcDevEventNotify 7 }

pktcDevEvReportEndpointName	OBJECT-TYPE

SYNTAX			DisplayString

MAX-ACCESS	read-only

STATUS			current

DESCRIPTION

"This is the endpoint identifier followed by the FQDN/IP Address of the device. This identifier is in the form - AALN/X:FQDN/IP Address. If the event is not specific to an endpoint, then the contents is just the FQDN/IP address."

::= { pktcDevEventNotify 8 }

pktcDevEvInform NOTIFICATION-TYPE

OBJECTS { pktcDevEvReportIndex, pktcDevEvReportTime,pktcDevEvReportLevel,

pktcDevEvReportEnterprise,pktcDevEvReportId,pktcDevEvReportText,

pktcDevEvReportMacAddress,pktcDevEvReportEndpointName }

STATUS			current

DESCRIPTION

"Inform for event reporting"

::= { pktcDevEvNotification 1 }

pktcDevEvTrap NOTIFICATION-TYPE

OBJECTS { pktcDevEvReportIndex, pktcDevEvReportTime,pktcDevEvReportLevel,

pktcDevEvReportEnterprise,pktcDevEvReportId,pktcDevEvReportText,

pktcDevEvReportMacAddress,pktcDevEvReportEndpointName }

STATUS	current

DESCRIPTION

"Trap for event reporting"

::= { pktcDevEvNotification 2 }

pktcEventConformance	OBJECT IDENTIFIER ::= { pktcEventMib 7 }

pktcEventCompliances 	OBJECT IDENTIFIER ::= { pktcEventConformance 1 }

pktcEventGroups			OBJECT IDENTIFIER ::= { pktcEventConformance 2 }

-- compliance statements

pktcEventBasicCompliance MODULE-COMPLIANCE

STATUS			current

DESCRIPTION

"The compliance statement for devices that implement

Event reporting feature."

MODULE	-- pktcEventMib

-- unconditionally mandatory groups

MANDATORY-GROUPS {

pktcEventGroup

}

-- units of conformance

::= { pktcEventCompliances 3 }

pktcEventGroup OBJECT-GROUP

OBJECTS {

pktcDevEvControl,

pktcDevEvControlState,

pktcDevEvSyslogAddressType,

pktcDevEvSyslogAddress,

pktcDevEvSyslogUdpPort,

pktcDevEvThrottleAdminStatus,

pktcDevEvThrottleInhibited,

pktcDevEvThrottleThreshold,

pktcDevEvThrottleInterval,

pktcDevEvProgrammableEnterprise,

pktcDevEvProgrammableLevel,

pktcDevEvProgrammableReporting,

pktcDevEvProgrammableText,

pktcDevEvFixedEnterprise,

pktcDevEvFixedLevel,

pktcDevEvFixedReporting,

pktcDevEvFixedText,

pktcDevEvIndex,

pktcDevEvTime,

pktcDevEvLevel,

pktcDevEvEnterprise,

pktcDevEvId,

pktcDevEvText,

pktcDevEvMacAddress,

pktcDevEvEndpointName,

pktcDevEvReportIndex,

pktcDevEvReportTime,

pktcDevEvReportLevel,

pktcDevEvReportEnterprise,

pktcDevEvReportId,

pktcDevEvReportText,

pktcDevEvReportMacAddress,

pktcDevEvReportEndpointName

}

STATUS	current

DESCRIPTION

"Group of objects for PacketCable Event MIB."

::= { pktcEventGroups 1 }

-- Notification Group Added

pktcEventNotificationGroup		NOTIFICATION-GROUP

NOTIFICATIONS { pktcDevEvInform, pktcDevEvTrap }

STATUS	current

DESCRIPTION

"These notifications deal with change in status of

MTA Device."

::= { pktcEventGroups 2 }

END

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |