|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

| ITU-T | J.81 |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU | Amendment 2 (03/98) |

SERIES J: TRANSMISSION OF TELEVISION, SOUND PROGRAMME AND OTHER MULTIMEDIA SIGNALS

Digital transmission of television signals

Transmission of component-codec digital
television signals for contribution-quality applications at the third hierarchical level
of ITU-T Recommendation G.702

Amendment 2:  Appendix IV to Annex A – Results of 34 Mbit/s codec interworking tests (February 1996)

ITU-T  Recommendation  J.81  –  Amendment 2

(Previously  CCITT  Recommendation)

ITU-T  J-SERIES  RECOMMENDATIONS

TRANSMISSION OF TELEVISION, SOUND PROGRAMME AND OTHER MULTIMEDIA SIGNALS

| For further details, please refer to ITU-T List of Recommendations. |  |
| --- | --- |
| General Recommendations | J.1–J.9 |
| General specifications for analogue sound-programme transmission | J.10–J.19 |
| Performance characteristics of analogue sound-programme circuits | J.20–J.29 |
| Equipment and lines used for analogue sound-programme circuits | J.30–J.39 |
| Digital encoders for analogue sound-programme signals | J.40–J.49 |
| Digital transmission of sound-programme signals | J.50–J.59 |
| Circuits for analogue television transmission | J.60–J.69 |
| Analogue television transmission over metallic lines and interconnection with radio-relay links | J.70–J.79 |
| Digital transmission of television signals | J.80–J.89 |
| Ancillary digital services for television transmission | J.90–J.99 |
| Operational requirements and methods for television transmission | J.100–J.109 |
| Interactive systems for digital television distribution | J.110–J.129 |
| Transport of MPEG-2 signals on packetised networks | J.130–J.139 |
| Measurement of the quality of service | J.140–J.149 |
| Digital television distribution through local subscriber networks | J.150–J.159 |

ITU-T  RECOMMENDATION  J.81

TRANSMISSION  OF  COMPONENT-CODED  DIGITAL  TELEVISION  SIGNALS
FOR  CONTRIBUTION-QUALITY  APPLICATIONS  AT  THE  THIRD
HIERARCHICAL  LEVEL  OF  ITU-T  RECOMMENDATION  G.702

AMENDMENT  2

APPENDIX  IV
(to Annex A of Recommendation J.81)

Results of 34 Mbit/s codec interworking tests (February 1996)

Source

Amendment 2, Appendix IV to Annex A of ITU-T Recommendation J.81, was prepared by ITU-T Study Group 9 (1997-2000) and was approved under the WTSC Resolution No. 1 procedure on the 18th of March 1998.

FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL  PROPERTY  RIGHTS

The ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. The ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, the ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  1998

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Introduction

The following new Appendix IV to Annex A is added to Recommendation J.81. It is recalled that Amendment 1 to Recommendation J.81 added a new Appendix II to Annex A which required renumbering the existing Appendix II as Appendix III at that time.

Recommendation J.81

Recommendation ITU-T J.81/amd.2     (03/98)

TRANSMISSION  OF  COMPONENT-CODED  DIGITAL  TELEVISION  SIGNALS
FOR  CONTRIBUTION-QUALITY  APPLICATIONS  AT  THE  THIRD
HIERARCHICAL  LEVEL  OF  ITU-T  RECOMMENDATION  G.702

AMENDMENT  2

appendiX  iv

(to Annex A of Recommendation J.81)

Results of 34 Mbit/s codec interworking tests (February 1996)

(Geneva, 1998)

Correct interworking of four manufacturers’ codecs was achieved in February 1996, after nearly three years of cooperation between the manufacturers and the EBU. This included correct interworking of the composite analogue interfaces, as specified by the EBU, (see Appendix II above).

The test procedure used for the interworking verification consisted of subjective quality evaluation by expert viewers on a selection of EBU/ITU test sequences, and of objective measurements on analogue audio and video.

Tables A.IV.1 and A.IV.2 below summarize the interworking test results for video and audio respectively. A more comprehensive description of the interworking tests is available in the EBU Technical Review No. 269 [1].

At the time of the tests, one manufacturer had not yet implemented SECAM interfaces or ITS and IDS transmission, but in all other respects correct interworking was achieved. The combination "encoder (No. 2) – decoder (No. 1)" gave an alarm signal inversion for the absence of the 34 Mbit/s signal at the input to the decoder. This was the only remaining anomaly in February 1996 and this minor problem was subsequently corrected.

Recovery times, after an interruption of the encoder video input signal, and after an interruption of the decoder 34 Mbit/s input signal, were satisfactory for all combinations of encoder-decoder, namely <  0.5 s and <  2 s respectively.

Audio/video delay differential was also satisfactory for all combinations of encoder-decoder, namely <  2 ms.

All encoders utilize comb-filters to decode PAL and NTSC respectively, thereby obtaining clean component signals, virtually free from luminance/chrominance crosstalk.

Subjective evaluation, carried out in accordance with ITU-R Recommendation BT.500-7, of one generation and three generations of 34 Mbit/s (Recommendation J.81) coding, using ITU-R BT.601-5 interconnections between codecs, gave scores of 4.5 and 4 respectively, on the 1 to 5 scale for picture quality. Audio transmission, using ITU-T J.57 coding, was transparent.

Table  A.IV.1/J.81 – 34 Mbit/s encoder/decoder interworking video test results – February 1996

| Decoder Encoder | Decoder No. 1 | Decoder No. 2 | Decoder No. 3 | Decoder No. 4 |
| --- | --- | --- | --- | --- |
|  | Ext. IDS/ITS: OK Int. IDS/ITS: OK | Ext. IDS/ITS: OK Int. IDS/ITS: OK | Ext. IDS/ITS: OK Int. IDS/ITS: OK | Ext. IDS/ITS: OK Int. IDS/ITS: OK No Int./Ext. IDS |
| Encoder No. 1 | Scrambling (mode 1): OK | Scrambling (mode 1): OK | Scrambling (mode 1): OK | Scrambling (mode 1): OK |
|  | Video: OK | Video: OK | Video: OK | Video: OK but no SECAM coder available in No. 4 decoder |
|  | Audio: OK | Audio: OK | Audio: OK | Audio: OK |
|  | Ext. IDS/ITS: OK Int. IDS/ITS: OK | Ext. IDS/ITS: OK Int. IDS/ITS: OK | Ext. IDS/ITS: OK Int. IDS/ITS: OK | Ext. ITS: OK No Int. ITS No Int./Ext IDS |
| Encoder No. 2 | Scrambling (mode 1): OK | Scrambling (mode 1): OK | Scrambling (mode 1): OK | Scrambling (mode 1): OK |
|  | Video: OK but "34 M absent" alarm | Video: OK | Video: OK | Video: OK but no SECAM coder available in No. 4 decoder |
|  | Audio: OK | Audio: OK | Audio: OK | Audio: OK |
|  | Ext. IDS/ITS: OK Int. IDS/ITS: OK | Ext. IDS/ITS: OK Int. IDS/ITS: OK | Ext. IDS/ITS: OK Int. IDS/ITS: OK | Ext. ITS: intermittent in regular sequence No Int. ITS |
| Encoder No. 3 | Scrambling (mode 1): OK | Scrambling (mode 1): OK | Scrambling (mode 1): OK | Scrambling (mode 1): OK |
|  | Video: OK | Video: OK | Video: OK | Video: OK but no SECAM coder available in No. 4 decoder |
|  | Audio: OK | Audio: OK | Audio: OK | Audio: OK |
|  | Ext. ITS: OK No Int. ITS No Int./Ext. IDS | Ext. ITS: OK No Int. ITS No Int./Ext. IDS | Ext.ITS: Intermittent No Int. ITS No Int./Ext. IDS | Ext.ITS: OK No Int. ITS No Int./Ext. IDS |
| Encoder No. 4 | Scrambling (mode 1): OK | Scrambling (mode 1): OK | Scrambling (mode 1): OK | Scrambling (mode 1): OK |
|  | Video: No SECAM decoder available in No. 4 encoder | Video: No SECAM decoder available in No. 4 encoder | Video: No SECAM decoder available in No. 4 encoder | Video: OK but no SECAM available in No. 4 encoder or decoder |
|  | Audio: OK | Audio: OK | Audio: OK | Audio: OK |
| NOTE – Encoders and decoders No. 4 were manufactured by a combination of two companies who now supply the equipment as two separate manufacturers. Both manufacturers have subsequently implemented ITS and IDS transmission. |  |  |  |  |

Table  A.IV.2/J.81 – 34 Mbit/s encoder/decoder interworking video test results – February 1996

| Decoder Encoder | Decoder No. 1 | Decoder No. 2 | Decoder No. 3 | Decoder No. 4 |
| --- | --- | --- | --- | --- |
|  | Frequency/attenuation 40 Hz-15 kHz:  0.1 dB | Frequency/attenuation 40 Hz-15 kHz:  0.1 dB | Frequency/attenuation 40 Hz-15 kHz:  0.1 dB | Frequency/attenuation 40 Hz-15 kHz:  0.1 dB |
|  | 1020 Hz/+9 dBm THD  0.1% | 1020 Hz/+9 dBm THD  0.1% | 1020 Hz/+9 dBm THD  0.1% | 1020 Hz/+9 dBm THD  0.1% |
| Encoder No. 1 | 60 Hz/+9 dBM THD  0.1% | 60 Hz/+9 dBM THD  0.1% | 60 Hz/+9 dBM THD  0.1% | 60 Hz/+9 dBM THD  0.1% |
|  | X-talk: 79.8 dB | X-talk: 76.8 dB | X-talk: 77.6 dB | X-talk: 83.0 dB |
|  | S/N (wtd): 73.3 dB | S/N (wtd): 75.8 dB | S/N (wtd): 75.4 dB | S/N (wtd): 74.3 dB |
|  | Frequency/attenuation 40 Hz-15 kHz:  0.1 dB | Frequency/attenuation 40 Hz-15 kHz:  0.1 dB | Frequency/attenuation 40 Hz-15 kHz:  0.1 dB | Frequency/attenuation 40 Hz-15 kHz:  0.1 dB |
|  | 1020 Hz/+9 dBm THD  0.1% | 1020 Hz/+9 dBm THD  0.1% | 1020 Hz/+9 dBm THD  0.1% | 1020 Hz/+9 dBm THD  0.1% |
| Encoder No. 2 | 60 Hz/+9 dBm THD  0.1% | 60 Hz/+9 dBm THD  0.1% | 60 Hz/+9 dBm THD  0.1% | 60 Hz/+9 dBm THD  0.1% |
|  | X-talk: 78.5 dB | X-talk: 82.9 dB | X-talk: 79.6 dB | X-talk: 77.1 dB |
|  | S/N (wtd): 73.6 dB | S/N (wtd): 74.6 dB | S/N (wtd): 78.3 dB | S/N (wtd): 71.2 dB |
|  | Frequency/attenuation 40 Hz-15 kHz:  0.1 dB | Frequency/attenuation 40 Hz-15 kHz:  0.1 dB | Frequency/attenuation 40 Hz-15 kHz:  0.1 dB | Frequency/attenuation 40 Hz-15 kHz:  0.1 dB |
|  | 1020 Hz/+9 dBm THD < 0.1% | 1020 Hz/+9 dBm THD  0.1% | 1020 Hz/+9 dBm THD  0.1% | 1020 Hz/+9 dBm THD  0.1% |
| Encoder No. 3 | 60 Hz/+9 dBm THD  0.1% | 60 Hz/+9 dBm THD  0.1% | 60 Hz/+9 dBm THD  0.1% | 60 Hz/+9 dBm THD  0.1% |
|  | X-talk: 81.5 dB | X-talk: 85.8 dB | X-talk: 84.5 dB | X-talk: 84.3 dB |
|  | S/N (wtd): 76.6 dB | S/N (wtd): 81.8 dB | S/N (wtd): 82.5 dB | S/N (wtd): 79.9 dB |
|  | Frequency/attenuation 40 Hz-15 kHz:  0.1 dB | Frequency/attenuation 40 Hz-15 kHz:  0.1 dB | Frequency/attenuation 40 Hz-15 kHz:  0.1 dB | Frequency/attenuation 40 Hz-15 kHz:  0.1 dB |
|  | 1020 Hz/+9 dBm THD  0.1% | 1020 Hz/+9 dBm THD  0.1% | 1020 Hz/+9 dBm THD  0.1% | 1020 Hz/+9 dBm THD  0.1% |
| Encoder No. 4 | 60 Hz/+9 dBm THD  0.1% | 60 Hz/+9 dBm THD  0.1% | 60 Hz/+9 dBm THD  0.1% | 60 Hz/+9 dBm THD  0.1% |
|  | X-talk: 78.6 dB | X-talk: 84.5 dB | X-talk: 81.6 dB | X-talk: 83.0 dB |
|  | S/N (wtd): 68.1 dB | S/N (wtd): 70.5 dB | S/N (wtd): 73.1 dB | S/N (wtd): 77.9 dB |

Bibliography

[1]	FLOWERS (B.G.): Interworking tests on 34 Mbit/s encoders-decoders, EBU Technical Review, No. 269, Autumn 1996.

| ITU-T RECOMMENDATIONS SERIES |  |
| --- | --- |
| Series A | Organization of the work of the ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communication |
| Series Y | Global information infrastructure |
| Series Z | Programming languages |