|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

| ITU-T | J.89 |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU | (09/99) |

SERIES J: TRANSMISSION OF TELEVISION, SOUND PROGRAMME AND OTHER MULTIMEDIA SIGNALS

Digital transmission of television signals

Transport mechanism for component-coded
digital television signals using MPEG-2 4:2:2 P@ML including all service elements 
for contribution and primary distribution

ITU-T  Recommendation  J.89

(Previously  CCITT  Recommendation)

ITU-T  J-SERIES  RECOMMENDATIONS

TRANSMISSION OF TELEVISION, SOUND PROGRAMME AND OTHER MULTIMEDIA SIGNALS

| For further details, please refer to ITU-T List of Recommendations. |  |
| --- | --- |
| General Recommendations | J.1–J.9 |
| General specifications for analogue sound-programme transmission | J.10–J.19 |
| Performance characteristics of analogue sound-programme circuits | J.20–J.29 |
| Equipment and lines used for analogue sound-programme circuits | J.30–J.39 |
| Digital encoders for analogue sound-programme signals | J.40–J.49 |
| Digital transmission of sound-programme signals | J.50–J.59 |
| Circuits for analogue television transmission | J.60–J.69 |
| Analogue television transmission over metallic lines and interconnection with radio-relay links | J.70–J.79 |
| Digital transmission of television signals | J.80–J.89 |
| Ancillary digital services for television transmission | J.90–J.99 |
| Operational requirements and methods for television transmission | J.100–J.109 |
| Interactive systems for digital television distribution | J.110–J.129 |
| Transport of MPEG-2 signals on packetised networks | J.130–J.139 |
| Measurement of the quality of service | J.140–J.149 |
| Digital television distribution through local subscriber networks | J.150–J.159 |

ITU-T  RECOMMENDATION  J.89

TRANSPORT  MECHANISM  FOR  COMPONENT-CODED  DIGITAL  TELEVISION
SIGNALS  USING  MPEG-2  4:2:2  P@ML  INCLUDING  ALL  SERVICE
ELEMENTS  FOR  CONTRIBUTION  AND  PRIMARY  DISTRIBUTION

Summary

This Recommendation specifies the general transport mechanism for conveying all service elements needed for contribution and primary distribution applications of TV programs using MPEG-2 4:2:2 profile at Main level compression. The service elements provided to MPEG-2 coding systems are assumed to be (4:2:2) component video signals, studio quality audio signals and various data signals e.g. teletext and time code.

Source

ITU-T Recommendation J.89 was prepared by ITU-T Study Group 9 (1997-2000) and was approved under the WTSC Resolution No. 1 procedure on the 16 September 1999.

FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL  PROPERTY  RIGHTS

The ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. The ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, the ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2000

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Terms and definitions		2

4	Abbreviations		3

5	Transport mechanism		3

5.1	Systems layer		3

5.2	Video		3

5.3	Compressed audio		4

5.4	Uncompressed audio		4

5.5	Ancillary data		5

5.6	Data		6

5.7	Data lines		6

5.8	Time code		9

5.9	Composite testlines		10

5.10	Encoder information		12

6	Channel adaptations		13

6.1	Transport in PDH/SDH networks		13

6.2	Transport in B-ISDN		13

7	Bibliography		13

Recommendation J.89

Recommendation J.89     (09/99)

Transport  Mechanism  for  COMPONENT-CODED  DIGITAL  TELEVISION
SIGNALS  USING  MPEG-2  4:2:2  P@ML  including  all  service
elements  FOR  CONTRIBUTION  AND  PRIMARY  DISTRIBUTION

(Geneva, 1999)

# 1	Scope

This Recommendation specifies the general transport mechanism for conveying all service elements needed for contribution and primary distribution applications of TV programs using MPEG-2 4:2:2 profile at Main level compression. The service elements provided to MPEG-2 coding systems are assumed to be (4:2:2) component video signals, studio quality audio signals and various data signals e.g. teletext and time code. This Recommendation ensures the compatibility on level of bit-stream into a decoder. It is based on, and is in conformance with, the MPEG-2 standard ITU-T Rec. H.222.0ISO/IEC 13818-1 [1].

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[1]	ITU-T Recommendation H.222.0 (1995) | ISO/IEC 13818-1:1996, Information technology – Generic coding of moving pictures and associated audio information: Systems.

[2]	ITU-T Recommendation H.262 (1995) | ISO/IEC 13818-2:1996, Information technology – Generic coding of moving pictures and associated audio information: Video.

[3]	ISO/IEC 11172-3:1993, Information technology – Coding of moving pictures and associated audio for digital storage media at up to about 1.5 Mbit/s – Part 3: Audio.

[4]	ISO/IEC 13818-7:1997, Information technology – Generic coding of moving pictures and associated audio information – Part 7: Advanced Audio Coding (AAC).

[5]	SMPTE 302-1998, Television – Mapping of AES3 Data into MPEG-2 Transport Stream.

[6]	Recommendation ITU-R BT.1364 (1998), Format of ancillary data signals carried in digital component studio interfaces.

[7]	Recommendation ITU-R BT.1304 (1997), Checksum for error detection and status information in interfaces conforming with Recommendations ITU-R BT.656 and ITU-R BT.799.

[8]	Recommendation ITU-R BT.1366 (1998), Transmission of time code and control code in the ancillary data space of a digital television stream according to ITU-R BT.656, ITU-R BT.799 and ITU-R BT.1120.

[9]	Recommendation ITU-R BT.1305 (1997), Digital audio and auxiliary data as ancillary data signals in interfaces conforming to Recommendations ITU-R BT.656 and ITU-R BT.799.

[10]	Recommendation ITU-R BT.653-3 (1998), Teletext systems.

[11]	Recommendation ITU-R BT.601-5 (1995), Studio encoding parameters of digital television for standard 4:3 and wide-screen 16:9 aspect ratios.

[12]	ITU-T Recommendation J.131 (1998), Transport of MPEG-2 signals in PDH networks.

[13]	ITU-T Recommendation J.132 (1998), Transport of MPEG-2 signals in SDH networks.

[14]	ITU-T Recommendation J.83 (1997), Digital multi-programme systems for television, sound and data services for cable distribution.

[15]	ITU-T Recommendation J.82 (1996), Transport of MPEG-2 constant bit rate television signals in B-ISDN.

# 3	Terms and definitions

This Recommendation defines the following terms:

3.1	access unit: A coded representation of a presentation unit. In the case of audio, an access unit is the coded representation of an audio frame. In the case of video, an access unit includes all the coded data for a picture, and any stuffing that follows it, up to, but not including, the start of the next access unit. If a picture is not proceeded by a group_start_code or a sequence_header_code, the access unit begins with the picture_start_code. If a picture is preceded by a group_start_code and/or a sequence_header_code, the access unit begins with the first byte of the first of these start codes. If it is the last picture preceding a sequence_end_code in the bitstream, all bytes between the last byte of the coded picture and the sequence_end_code (including the sequence_end_code) belong to the access unit.

3.2	bit rate: The rate at which the compressed bit stream is delivered from the channel to the input of a decoder.

3.3	decoding time-stamp: A field that may be present in a PES packet header that indicates the time that an access unit is decoded in the system target decoder.

3.4	packet identifier: A unique integer value used to identify elementary streams of a program in a single or multi-program transport stream as described in 2.4.3 of ITU-T Rec. H.222.0  ISO/IEC 13818-1 [1].

3.5	payload: Payload refers to the bytes which follow the header bytes in a packet. For example, the payload of some transport stream packets includes a PES_packet_header and its PES_packet_data_bytes, or pointer_field and PSI sections, or private data: but a PES_packet_payload consists of only PES_packet_data_bytes. The transport stream packet header and adaptation fields are not payload.

3.6	PES packet: The data structure used to carry elementary stream data. A PES packet consists of a PES packet header followed by a number of contiguous bytes from an elementary data stream. It is a layer in the system coding syntax described in 2.4.3.6 of ITU-T Rec. H.222.0 | ISO/IEC 13818-1 [1].

3.7	PES packet header: The leading fields in a PES packet up to and not including the PES_packet_data_byte fields, where the stream is not a padding stream. In the case of a padding stream the PES packet header is similarly defined as the leading fields in a PES packet up to and not including padding_byte fields.

3.8	PES Stream: A PES stream consists of PES packets, all of whose payloads consist of data from a single elementary stream, and all of which have the same stream_id. Specific semantic constraints apply. Refer to Intro. 4 of ITU-T Rec. H.222.0 | ISO/IEC 13818-1 [1].

3.9	presentation time-stamp: A field that may be present in a PES packet header that indicates the time that a presentation unit is presented in the system target decoder.

3.10	presentation unit: A decoded audio access unit or a decoded picture.

3.11	program clock reference: A time stamp in the transport stream from which decoder timing is derived.

3.12	program specific information: PSI consists of normative data which is necessary for the demultiplexing of transport streams and the successful regeneration of programs and is described in 2.4.4 of ITU-T Rec. H.222.0 | ISO/IEC 13818-1 [1]. An example of privately defined PSI data is the non-mandatory network information table.

3.13	reserved: The term "reserved", when used in the clauses defining the coded bit stream, indicates that the value may be used in the future for ISO defined extensions. Unless otherwise specified within ITU-T Rec. H.222.0 | ISO/IEC 13818-1 [1], all reserved bits shall be set to "1".

3.14	time-stamp: A term that indicates the time of a specific action such as the arrival of a byte or the presentation of a presentation unit.

3.15	transport stream packet header: The leading fields in a transport stream packet, up to and including the continuity_counter field.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

AAC	Advanced Audio Coding

ANC	Ancillary

DTS	Decoding Time Stamp

EDH	Error Detection and Handling

GOP	Group of Pictures

LTC	Longitudinal Time Code

MPEG	Moving Picture Experts Group

PCR	Program Clock Reference

PDH	Plesiochronous Digital Hierarchy

PES	Packetized Elementary Stream

PSI	Program Specific Information

PTS	Presentation Time Stamp

SDH	Synchronous Digital Hierarchy

SEQ	Sequences

TS	Transport Stream

VBI	Vertical Blanking Interval

VITC	Vertical Interval Time Code

VITS	Vertical Interval Test Signals

# 5	Transport mechanism

This Recommendation follows the method of describing bit stream syntax and uses symbols given in clause 2 of ITU-T Rec. H.222.0 | ISO/IEC 13818-1 [1].

## 5.1	Systems layer

PCR repetition rate:	The PCR shall occur typically every field interval (16.6 ms for 525/60 TV system and 20 ms for 625/50 TV system) and at least every 100 ms.

PCR position:	PCRs (and adaptation fields) shall be inserted either in the video stream or in a separate PCR stream.

## 5.2	Video

### 5.2.1	Video PES format

Stream_Id:	"1110 xxxx" (video stream number xxxx).

PES_packet_length: 	"0x0000" (neither specified nor bounded and allowed only in PES packets whose payload consists of bytes from a video elementary stream contained in transport stream packets).

data_alignment_indication:	"1" (the PES packet header is immediately followed by the video start code).
Alignment type 0x03 (GOP or SEQ).

PTS_DTS_flags:	"10" (PTS fields shall be present in the PES packet header).

"11"(Both the PTS fields and DTS fields shall be present in the PES packet header).

### 5.2.2	Video layers

Profile and Level:	The 4:2:2 profile at Main level [2] shall be implemented.

Sequence header repetition rate:	Minimum every second.

## 5.3	Compressed audio

### 5.3.1	PES format

Stream_Id:	"110xxxxx" (audio stream number xxxxx).

data_alignment_indicator:	"1" (the PES packet header is immediately followed by the audio sync word).

Alignment_type 0x01 (Sync word).

PTS_DTS_flags:	"10" (PTS fields shall be present in the PES packet header).

### 5.3.2	Audio layer

MPEG-1 Layer II [3] or MPEG-2 AAC [4] shall be implemented.

#### 5.3.2.1	Coding for stereo

Coding:		MPEG-1 Layer II

Bit rate:		384 kbit/s other bit rates optional

Sampling frequency:	48 kHz

Emphasis:	no emphasis

#### 5.3.2.2	Coding for multichannel

Coding:		MPEG-2 AAC

Number of channels:	up to 6

Bit rate per channel:	up to 80 kbit/s

Sampling frequency:	48 kHz

Emphasis:	no emphasis

## 5.4	Uncompressed audio

The preferred method for transport of uncompressed audio, including SDI embedded audio (Recommendation ITU-R BT.1305 [9]) is according to SMPTE 302 [5].

## 5.5	Ancillary data

Ancillary data is defined by Recommendation ITU-R BT.1364 [6], and includes checksum, timecode and uncompressed digital audio defined in Recommendation ITU-R BT.1304 [7], Recommendation ITU-R BT.1366 [8], and Recommendation ITU-R BT.1305 [9], respectively. The following applies for transport of the ancillary data. This method may optionally be applied to audio stream according to Recommendation ITU-R BT.1305 [9].

### 5.5.1	PES packet format

Stream_Id:	"1011 1101" (private_stream_1).

data_alignment_indicator:	"1" (the PES packet header is immediately followed by the sync word).
Alignment_type "0x02" (Video Access Unit).

PTS_DTS_flags:	"10" (PTS fields shall be present in the PES packet header).

PES_packet_data_byte:	These bytes are coded in accordance with the ANC_data( ) syntax as defined below.

Table 1/J.89 – ANC data field

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| ANC_data( ) { |  |  |
| for (i=0; i<N; i++) { |  |  |
| ANC_data_field( ) |  |  |
| while (!bytealigned) |  |  |
| zero_bit | 1 | "1" |
| } |  |  |
| for (i=0; i<N1; i++){ |  |  |
| stuffing_byte | 8 | "1111 1111" |
| } |  |  |
| } |  |  |
| ANC_data_field ( ) { |  |  |
| "0x000" | 10 | bslbf |
| line_number | 10 | bslbf |
| horizontal_offset | 10 | uimsbf |
| data_ID | 10 | bslbf |
| DBN_SDID | 10 | bslbf |
| data_count | 10 | bslbf |
| for (i=0, i<=data_count; i++){ |  |  |
| user_data_word | 10 | bslbf |
| } |  |  |
| checksum_word | 10 | bslbf |
| } |  |  |
| NOTE – The ANC_data_field ( ) consists of a header ("0x000", line_number, horizontal_offset) followed by the ancillary data packet content (as defined in Recommendation ITU-R BT.1364 [6]) starting after the ancillary data flag. line_number: This 10-bit word contains the line number (1 to 625 and 1 to 525 respectively). horizontal_offset: This 10-bit word contains the horizontal address (0 to 857 and 0 to 863 respectively) in a line indicated by the line_number. |  |  |

## 5.6	Data

The following PES/TS scheme allows optional user data channels (excluding ancillary data) to be transferred at any data rate.

The data format at the access-interface is signalled by user definition in PSI.

If time recovery is requested (e.g. synchronous data channels) it is supported by PTS.

### 5.6.1	PES format

Stream_Id:	"10111101" (private_stream_1).

PES_packet_length:	N * 184 – 6, where N is integer

data_alignment_indicator:	"0" (no alignment) or
"1" (the PES packet header is immediately followed by the sync word).

Alignment_type 0x01 (Sync word).

PTS_DTS_flags:	"10" (PTS fields shall be present in the PES packet header).

PES_header_data_length:	6.

PES_packet_data_bytes:	Filled with the bits of the data channel with removed channel coding.

## 5.7	Data lines

The content of the data lines (for example Teletext according to Recommendation ITU-R BT.653-2 [10] and EBU data line according to EBU Tech 3217) are carried by packets defined with the syntax defined below. The data lines of one video frame form one or more access units.

### 5.7.1	PES packet format

The PES packet syntax and semantics are followed noting the following constraints:

Stream_Id:	"1011 1101" (private_stream_1).

PES_packet_length:	N * 184 – 6, where N is an integer.

data_alignment_indicator:	"1" (the PES packet header is immediately followed by the sync word).

Alignment_type 0x02 (Video Access Unit).

PES_header_data_length:	Set to "0x24".

stuffing_byte:	The PES header is followed by as many stuffing bytes as are required to make up the header data length, so that the entire PES header is 45 bytes long.

PES_packet_data_byte:	These bytes are coded in accordance with the PES data field syntax specified in 5.7.2.

PTS and other optional fields may be present in the PES header, but the header length is always fixed.

### 5.7.2	Syntax for PES data field

See Table 2.

Table 2/J.89 – PES data field

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| PES_data_field ( ) { |  |  |
| data_identifier | 8 | uimsbf |
| for (i=0;I++){ |  |  |
| data_unit_id | 8 | uimsbf |
| data_unit_length | 8 | uimsbf |
| if (data_unit_id = = stuffing_unit){ |  |  |
| stuffing_field | 43*8 | bslbf |
| } |  |  |
| else{ |  |  |
| data_field ( ) |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

The syntax for data field is given in Table 3.

Table 3/J.89 – Teletext data field

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| data_field ( ) { |  |  |
| reserved | 2 | bslbf |
| field_parity | 1 | bslbf |
| line_offset | 5 | uimsbf |
| framing_code | 8 | bslbf |
| magazine_and_packet_address | 16 | bslbf |
| data_block | 320 | bslbf |
| } |  |  |

### 5.7.3	Semantics for PES data field

data identifier: This 8-bit field identifies the type of data carried in the PES packet. It is coded as indicated in Table 4.

Table 4/J.89 – Data identifier

| Data identifier | Value |
| --- | --- |
| 0x00-0x0F | reserved |
| 0x10-0x1F | TXT data |
| 0x20-0x7F | reserved |
| 0x80 | TC data |
| 0x81-0x9E | reserved |
| 0x9F | testline |
| 0xA0 | encoder information |
| 0xA1-0xFF | reserved |

The data identifier shall be set to the same value for each PES packet conveying data in the same Teletext data stream.

data unit id: This 8-bit field identifies the type of data unit. It is coded as indicated in Table 5.

Table 5/J.89 – Data unit id

| Data unit id | Value |
| --- | --- |
| 0x00 | reserved |
| 0x01 | EBU data line |
| 0x02 | teletext system B, 625-line system, non-subtitle data |
| 0x03 | teletext system B, 625-line system, subtitle data |
| 0x04 | teletext system A, 625-line system |
| 0x05 | reserved |
| 0x06 | teletext system C, 625-line system |
| 0x07-0x10 | reserved |
| 0x11 | teletext system A, 525-line system |
| 0x12 | reserved |
| 0x13 | teletext system B, 525-line system |
| 0x14 | reserved |
| 0x15 | teletext system C, 525-line system |
| 0x16 | reserved |
| 0x17 | teletext system D, 525-line system |
| 0x18-0x80 | reserved |
| 0x81 | VITC and LTC |
| 0x82 | VITC |
| 0x83-0xA0 | reserved |
| 0xA1 | encoder status |
| 0xA2 | video coding parameters |
| 0xA3-0xFE | reserved |
| 0xFF | stuffing unit |

data_unit_length: This 8-bit field indicates the number of bytes in the data unit following the length field. For data units carrying Teletext data, this field shall always be set to "0x2C".

stuffing_field: This 43-bytes field is used if needed to fill up the PES to the PES packet length defined in 4.7.1. The field is set to all ones.

reserved: Set to "1".

field_parity: This 1-bit flag specifies the field for which the data is intended; the value "1" indicates the first field of a frame, the value "0" indicates the second field of a frame.

line_offset: This 5-bit field specifies the line number on which the Teletext data packet is intended to be presented if it is transcoded into the VBI.

Within a field, the line-offset numbering shall follow a progressive incremental order except for the undefined line offset value "0". The toggling of the field parity flag indicates a new field. The line_offset is coded as in Table 6.

Table 6/J.89 – Line offset

| line_offset | line number |  |
| --- | --- | --- |
|  | 625-line system | 525-line system |
|  | field_parity = 1 field_parity = 0 | field_parity = 1 field_parity = 0 |
| 0x00 | undefined undefined | undefined undefined |
| 0x01-0x06 | reserved reserved | reserved reserved |
| 0x07 | 7 320 | reserved reserved |
| 0x08 | 8 321 | reserved reserved |
| 0x09 | 9 322 | reserved reserved |
| 0xA0 | 10 323 | 10 273 |
| --- | --- --- | --- --- |
| 0x15 | 21 334 | 21 284 |
| 0x16 | 22 335 | reserved reserved |
| 0x17-0x1F | reserved reserved | reserved reserved |

framing_code, magazine_and_packet_address, data_block: These field correspond to the 43 bytes following the clock-run-in sequence of a Teletext data packet as defined in Recommendation ITU-R BT.653-2 [10], system B, 625/50 television systems. Data packets are inserted in the same order as they are intended to arrive at the Teletext decoder or to be transcoded into the VBI.

For other Teletext systems or the EBU data line the same scheme has to be applied. For lines containing less data bits the remaining bits of the data block are set to 1.

## 5.8	Time code

In the case where the time code is delivered as LTC or VITC the following applies:

### 5.8.1	PES packet format

As defined in 5.7.1.

### 5.8.2	Syntax for PES data field

The syntax for the PES data field is defined in Table 2.

The syntax for data field is given in Table 7.

Table 7/J.89 – Timecode data field

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| data_field ( ){ |  |  |
| reserved | 2 | bslbf |
| field_parity | 1 | bslbf |
| line_offset | 5 | uimsbf |
| VITC_block | 90 | bslbf |
| reserved | 38 | bslbf |
| LTC_block | 80 | bslbf |
| reserved | 17*8 | bslbf |
| } |  |  |

### 5.8.3	Semantics for PES data field

data_identifier: This 8-bit field identifies the type of data carried in the PES packet. It is coded as indicated in Table 4.

data_unit_id: This 8-bit field identifies the type of data unit. It is coded as given in Table 5.

data_unit_length: This 8-bit field indicates the number of bytes in the data unit following the length field. For data units carrying time code only this field shall always be set to "0x2C".

reserved: Set to "1".

field_parity: This 1-bit flag specifies the field for which the data is intended; the value "1" indicates the first field of a frame, the value "0" indicates the second field of a frame.

line_offset: This 5-bit field specifies the line number on which the time code is intended to be presented if it is transcoded into the VBI.

The line_offset is coded as in Table 6.

VITC_block: This field corresponds to the 90 VITC data bits as defined in SMPTE 12M (2), starting with bit number 0. An unused block is filled with ones.

LTC_block: This field corresponds to the 80 LTC data bits defined in SMPTE 12M (2), starting with bit number 0. An unused block is filled with ones.

## 5.9	Composite testlines

The testlines are conveyed uncompressed in PES packets defined below.

### 5.9.1	PES packet format

Stream_Id:	Set to "1011 1101" (private_stream_1).

PES_packet_length:	914 (= 5 * 184 – 6).

PES_scrambling_control:	"00".

data_alignment_indicator:	Set to "1" (aligned).

PES_header_data_length:	9.

stuffing_byte:	The required number to make up the header data length.

PES_packet_data_byte:	These bytes are coded in accordance with the VITS_data_field syntax below.

### 5.9.2	Syntax for PES data field

Table 8/J.89 – VITS data field

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| VITS_data_field ( ){ |  |  |
| data_identifier | 8 | uimsbf |
| field_sequence | 3 | uimsbf |
| line_offset | 5 | uimsbf |
| for (i=0;i<N;i++){ |  |  |
| VITS_data_word | 10 | bslbf |
| } |  |  |
| } |  |  |

### 5.9.3	Semantics for PES data field

data_identifier: This 8-bit field identifies the type of data carried in the PES packet. It is set to "0x9F".

field_sequence: A 3-bit integer which defines the number of the field in the eight field sequence used in PAL systems or the four field_sequence used in NTSC systems as defined in the Table 9.

Table 9/J.89 – field_sequence

| field_sequence | frame | field |
| --- | --- | --- |
| 000 | 1 | 1 |
| 001 | 1 | 2 |
| 010 | 2 | 3 |
| 011 | 2 | 4 |
| 100 | 3 | 5 |
| 101 | 3 | 6 |
| 110 | 4 | 7 |
| 111 | 4 | 8 |

line_offset: This 5-bit field specifies the line number on which the testline is intended to be presented. It is coded according to Table 6.

VITS_data_word: The N VITS data bytes carries the 10-bit samples of the active part of one testline. That are the 720 samples of the digital active line.

The sampling for the testline is in accordance with Recommendation ITU-R BT.601-5 [11] luminance sampling with the following differences:

–	10-bit scale used, with range 0 ... 1023;

–	black corresponds to 288 (= 32 + 256);

–	100% white corresponds to 726.

Temporal sub-sampling respecting the PAL and NTSC sequence respectively may be applied.

## 5.10	Encoder information

Real time information from the encoder to the decoder may be conveyed by packets with the following syntax:

### 5.10.1	PES packet format

As defined in 5.7.1.

### 5.10.2	Syntax for PES data field

The syntax for the PES data field is defined in Table 2.

The syntax for data field is given in Table 10 below.

Table 10/J.89 – Data field for encoder information

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| data_field ( ){ |  |  |
| if (data_unit_id = =encoder_status){ |  |  |
| /*video*/ |  |  |
| video_loss | 1 | bslbf |
| EDH_flags_1 | 15 | bslbf |
| EDH_flags_2 | 15 | bslbf |
| reserved | 1 | bslbf |
| /*audio*/ |  |  |
| audio_loss_ch1 | 1 | bslbf |
| audio_loss_ch2 | 1 | bslbf |
| audio_loss_ch3 | 1 | bslbf |
| audio_loss_ch4 | 1 | bslbf |
| reserved | 4 | bslbf |
| for (k=5; k<data_unit_length; k++) |  |  |
| stuffing_byte | 8 | bslbf |
| } |  |  |
| } |  |  |

### 5.10.3	Semantics for PES data field

data_identifier: This 8-bit field identifies the type of data carried in the PES packet. It is coded as indicated in Table 4 ("0xA0").

data_unit_id: This 8-bit field identifies the type of data unit. It is coded as given in Table 5.

data_unit_length: This 8-bit field indicates the number of bytes in the data unit following the length field and is set to "0x2C".

video_loss: This bit indicates a loss of the video signal at the encoder input if set to "1".

EDH_flags_1: This 15-bit field contains the EDH error flags of the video field 1 as defined in Table 11. The meaning of the flags is in accordance with Recommendation ITU-R BT.1304 [7].

EDH_flags_2: This 15-bit field contains the EDH error flags of the video field 2 (see Table 11).

Table 11/J.89 – EDH_error_flags

| Bit number | Flag |
| --- | --- |
| 1(first) | ancillary data error flag edh |
| 2 | ancillary data error flag eda |
| 3 | ancillary data error flag idh |
| 4 | ancillary data error flag ida |
| 5 | ancillary data error flag ues |
| 6 | active picture error flag edh |
| 7 | active picture error flag eda |
| 8 | active picture error flag idh |
| 9 | active picture error flag ida |
| 10 | active picture error flag ues |
| 11 | full-field error flag edh |
| 12 | full-field error flag eda |
| 13 | full-field error flag idh |
| 14 | full-field error flag ida |
| 15(last) | full-field error flag ues |

audio_loss: The bit number in this field corresponds to the number of the stereo audio channel, and the flag indicates a loss of the audio channel if set to "1"

reserved: Set to "1".

stuffing_byte: This byte is used for stuffing. All bits are set to "1".

# 6	Channel adaptations

## 6.1	Transport in PDH/SDH networks

### 6.1.1	General approach

For PDH and SDH networks, the channel adaptations are according to Recommendations J.131 [12] and J.132 [13], respectively.

With this approach the clocks of the transport stream and of the network are independent and the two bit rates are only related by the network transport capacity limit.

### 6.1.2	Fixed bit rate approach for PDH network

If the transport stream bit rate is equal to the one of the network, a direct mapping could be applied. With this approach the transport stream clock and interface must meet the requirements of the network.

An error protection and scrambling mechanism is to be applied according to A.5/J.83 [14].

## 6.2	Transport in B-ISDN

For B-ISDN network, the channel adaptation is according to Recommendation J.82 [15].

# 7	Bibliography

1)	EBU Tech. 3217 – Specification of insertion data signal equipment for international transmissions, (3rd edition, 1977, re-issued 1986).

2)	SMPTE 12M – Television, Audio and Film – Time and Control Code, 1999.

| ITU-T RECOMMENDATIONS SERIES |  |
| --- | --- |
| Series A | Organization of the work of the ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure |
| Series Z | Languages and general software aspects for telecommunication systems |