|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | J.94 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | Amendment 1 (10/2000) |
|  | SERIES J: TRANSMISSION OF TELEVISION, SOUND PROGRAMME AND OTHER MULTIMEDIA SIGNALS Ancillary digital services for television transmission |  |  |  |
|  | Service information for digital broadcasting in cable television systems Amendment 1: Annex B  Service information delivered out of band in cable television systems |  |  |  |
|  | ITU-T Recommendation J.94 – Amendment 1 (Formerly CCITT Recommendation) |  |  |  |

ITU-T J-SERIES  RECOMMENDATIONS

TRANSMISSION OF TELEVISION, SOUND PROGRAMME AND OTHER MULTIMEDIA SIGNALS

| General Recommendations | J.1–J.9 |
| --- | --- |
| General specifications for analogue sound-programme transmission | J.10–J.19 |
| Performance characteristics of analogue sound-programme circuits | J.20–J.29 |
| Equipment and lines used for analogue sound-programme circuits | J.30–J.39 |
| Digital encoders for analogue sound-programme signals | J.40–J.49 |
| Digital transmission of sound-programme signals | J.50–J.59 |
| Circuits for analogue television transmission | J.60–J.69 |
| Analogue television transmission over metallic lines and interconnection with radio-relay links | J.70–J.79 |
| Digital transmission of television signals | J.80–J.89 |
| Ancillary digital services for television transmission | J.90–J.99 |
| Operational requirements and methods for television transmission | J.100–J.109 |
| Interactive systems for digital television distribution | J.110–J.129 |
| Transport of MPEG-2 signals on packetised networks | J.130–J.139 |
| Measurement of the quality of service | J.140–J.149 |
| Digital television distribution through local subscriber networks | J.150–J.159 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation J.94 Service information for digital broadcasting in cable television systems Amendment 1 annex b Service information delivered out of band for digital cable television systems

- Summary This amendment provides the contents of Annex B that define a standard for Service Information (SI) on cable compatible with digital multiplex bit streams constructed in accordance with ITU-T H.222.0 ¦ ISO/IEC 13818-1 (MPEG-2). It is designed to support "navigation devices" on cable. It defines the syntax and semantics for a standard set of tables providing the data necessary for such a device to discover and access digital and analogue services offered on cable. This SI protocol is nominally delivered in a separate physical channel (i.e. out-of-band) outside the actual transport multiplexes containing the program content.

- Source Amendment 1 to ITU-T Recommendation J.94 was prepared by ITU-T Study Group 9 (1997-2000) and approved by the World Telecommunication Standardization Assembly (Montreal, 27 September  6 October, 2000).

- Keywords Cable television, service information.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2001

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from ITU.

CONTENTS

Page

Annex B – Service information delivered out of band for digital cable television systems		1

B.1	Purpose, scope and organization		1

B.1.1	Purpose		1

B.1.2	Scope		1

B.1.3	Organization		2

B.2	References		3

B.3	Definitions		4

B.3.1	Compliance notation		4

B.3.2	Definition of terms		4

B.3.3	Section and data structure syntax notation		5

B.4	Acronyms and abbreviations		6

B.5	Table structure		7

B.5.1	Table ID ranges and values		7

B.5.2	Extensibility		8

B.5.3	Reserved fields		9

B.5.4	Private table section syntax		9

B.6	Table section formats		10

B.6.1	Network Information Table		10

B.6.2	Network Text Table		15

B.6.3	Short-form Virtual Channel Table Section		17

B.6.4	System Timetable Section		24

B.6.5	Master Guide Table (MGT)		25

B.6.6	Long-form Virtual Channel Table		29

B.6.7	Rating Region Table (RRT)		34

B.6.8	Aggregate Event Information Tables (AEIT)		37

B.6.9	Aggregate Extended Text Tables (AETT)		39

B.7	Descriptors		41

B.7.1	Descriptor usage		41

B.7.2	Stuffing descriptor		42

B.7.3	AC-3 audio descriptor		42

B.7.4	Caption service descriptor		42

B.7.5	Content advisory descriptor		44

B.7.6	Revision detection descriptor		45

B.7.7	Two-Part channel number descriptor		46

B.7.8	Channel properties descriptor		46

B.7.9	Extended channel name descriptor		48

B.7.10	Time-shifted service descriptor		48

Page

B.7.11	Component name descriptor		49

B.7.12	Daylight savings time descriptor		49

B.7.13	User private descriptors		50

B.8	Text string coding		50

B.8.1	Multilingual Text String (MTS) Format		51

B.8.2	Multiple String Structure (MSS)		55

Annex B.A – Operational profiles for cable service information delivery		58

B.A.1	Operational profiles		58

B.A.2	Profile Definition Tables		58

B.A.3	Operational considerations for the use of profiles (Informative)		60

Annex B.B – Packet rates		61

B.B.1	Maximum cycle times		61

B.B.2	Maximum transmission rates		61

B.B.3	Minimum transmission rates		61

Annex B.C – Standard Huffman tables for text compression		61

B.C.1	Character set definition		62

B.C.2	Standard compression Type 1 Encode/Decode Tables		64

B.C.3	Standard compression Type 2 Huffman Encode/Decode tables		84

Appendix B.I – Implementation recommendations		102

B.I.1	Implications for retail digital cable-ready devices		102

B.I.2	Channel number handling		102

B.I.3	Processing of dynamic changes to service information		102

B.I.4	AEITs may include event information for inaccessible channels		102

B.I.5	Splice flag processing		102

Appendix B.II – Service Information overview and guide		103

B.II.1	Table hierarchy		103

B.II.2	SI_base PID		108

B.II.3	Representation of Time		116

Appendix B.III – Daylight Savings Time control		118

ITU-T Recommendation J.94

Service information for digital broadcasting in cable television systems

Amendment 1

ANNEX B

Service information delivered out of band for digital cable television systems

## B.1	Purpose, scope and organization

### B.1.1	Purpose

This annex defines a standard for Service Information (SI) delivered out of band on cable. This annex is designed to support "navigation devices" on cable. The current specification defines the syntax and semantics for a standard set of tables providing the data necessary for such a device to discover and access digital and analogue services offered on cable.

### B.1.2	Scope

This annex defines SI tables delivered via an out-of-band path to support service selection and navigation by digital cable set-top boxes and other "digital cable-ready" devices. The SI tables defined in this annex are formatted in accordance with the Program Specific Information (PSI) data structures defined in MPEG-2 Systems, ITU-T H.222.0 | ISO/IEC 13818-1.

The formal definition of "digital cable-ready" has a scope broader than that of the current standard. The formal definition includes requirements related to navigation and service selection, demodulation and decoding, video format decoding, Emergency Alert handling, and other aspects. The current specification supports, primarily, the navigation and service selection function for services delivered in the clear, as well as those subject to conditional access.

This annex does not address the Electronic Program Guide application itself or any user interface which might deal with the presentation and application of the Service Information.

A digital cable-ready device can take the form of a cable set-top box, a computer, a television, or a convergence of these. Devices such as digital video recorders may also be cable-ready. A digital cable-ready device capable of processing access controlled digital services supports an interface to a conditional access module. As used here, the term "Host" refers to the capability to support an interface to a standard Point Of Deployment (POD) security module.

SI data delivered out of band is transported in accordance with the Extended Channel interface defined in SCTE DVS 131r7 (1998) and SCTE DVS 216r4 (2000). To have access to the Extended Channel interface, the cable-ready device must act as a Host to a POD security module. The Extended Channel interface presents the needed SI data to the Host. This data can be used by the Host for channel navigation, construction of electronic program guides and other associated functions.

Figure B.1 is a high-level block diagram illustrating the POD module to Host interface via the Extended Channel interface. The Host is responsible for providing a standard receiver/QPSK demodulator function for the POD module. The choice of transport format of bits coming across from the receiver/QPSK demodulator to the POD module is by mutual agreement between the POD and the cable head-end equipment. The transport format of data travelling between the Host and

POD module on the Extended Channel interface conforms to standards defined in SCTE DVS 131r7 (1998) and SCTE DVS 216r4 (2000).

Figure B.1/J.94 – A framework for the extended channel service information stream

The POD module may perform various transport, filtering, and error checking/correction functions on the out-of-band data stream as depicted by the box labelled "Transport Processing, Filtering, and Routing." As described in SCTE DVS 216r4 (2000), the Host may request from the POD module to open one or several "flows" in which to receive PSI sections taken from the cable out-of-band data stream. Each flow is associated with a PID value, in accordance with MPEG-2 Transport Stream concepts.

Data flowing to the Host from the POD module that is associated with Service_type=MPEG_section is required to be in the form of MPEG PSI data structures. However, data delivered into the POD from cable out-of-band may or may not be organized in a Transport Stream compliant with ITU-T H.222.0 | ISO/IEC 13818-1. In other words, PID values associated with MPEG-2 tables on the Extended Channel interface may or may not correspond to MPEG-2 Transport Stream packet header PID values from the cable out of band.

Independent of the fact that out-of-band data may reach the POD module via a proprietary method, the data structures delivered across the Extended Channel shall be formatted as MPEG-2 table sections. Like table sections carried in an MPEG-2 Transport Stream, each is associated with a PID value.

### B.1.3	Organization

This annex is organized as follows:

•	Clause B.1  Provides this general introduction.

•	Clause B.2  Lists applicable references.

•	Clause B.3  Provides a list of definitions used in this annex.

•	Clause B.4  Provides a list of acronyms and abbreviations used in this annex.

•	Clause B.5  Describes the basic structure of sections.

•	Clause B.6  Describes formats of sections carried in the Base PID.

•	Clause B.7  Explains descriptors applicable to the tables defined in this annex.

•	Clause B.8  Describes multilingual character string coding.

•	Annex B.A  Defines profiles of choice for cable operator compliance with this annex.

•	Annex B.B  Specifies packet rates for delivery of SI data.

•	Annex B.C  Defines the standard Huffman tables used for text compression.

•	Appendix B.I  Discusses recommendations for receiver implementations.

•	Appendix B.II  Provides an overview of tables defined in this Service Information Annex B.

•	Appendix B.III  Defines the daylight savings time control fields in the System Timetable.

## B.2	References

Normative references

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

	ITU-T H.222.0 (2000) | ISO/IEC 13818-1:2000, Information technology  Generic coding of moving pictures and associated audio information: Systems.

	ITU-T H.262 (2000) | ISO/IEC 13818-2: 2000, Information technology  Generic coding of moving pictures and associated audio information: Video.

	ISO 639:1988, Code for the representation of names of languages.

	ISO 639-2:1998, Codes for the representation of names of languages – Part 2: Alpha-3 code.

	ISO/IEC 8859-1 to 10, Information technology  8-bit single-byte coded graphic character sets.

	ISO/IEC 10646-1:2000, Information technology  Universal Multiple-Octet Coded Character Set (UCS)  Part 1: Architecture and Basic Multilingual Plane.

	ISO/IEC 13818-3:1998, Information technology  Generic coding of moving pictures and associated audio  Part 3: Audio.

Informative references

	ITU-T J.83 (1997), Digital multi-programme systems for television, sound and data services for cable distribution.

	SCTE DVS 031, Digital Video Transmission Standard for Cable Television, Rev.2, 29 May 1997.

	SCTE DVS 097 (1997), Program and System Information Protocol for Terrestrial Broadcast and Cable.

	SCTE DVS 131r7 (1998), Point of Deployment (POD) Module Interface.

	SCTE DVS 208r6 (1999), Cable Emergency Alert Message (EIA-814).

	SCTE DVS 216r4 (2000), POD Extended Channel Specification.

Bibliography

	ATSC Standard A/52 (1995), Digital Audio Compression (AC-3).

	ATSC Standard A/53 (1995), ATSC Digital Television Standard.

	EIA-708, Specification for Advanced Television Closed Captioning (ATVCC), Electronic Industry Association.

	EIA-752, Transport of Transmission Signal Identifier (TSID) Using Extended Data Service (XDS).

	EIA 766, U.S. Rating Region Table (RRT) and Content Advisory Descriptor for Transport of Content Advisory Information Using ATSC A/65 Program and System Information Protocol (PSIP).

## B.3	Definitions

### B.3.1	Compliance notation

As used in this annex, "shall" denotes a mandatory provision of the recommendation. "Should" denotes a provision that is recommended but not mandatory. "May" denotes a feature whose presence does not preclude compliance, that may or may not be present as optional for the implementers.

### B.3.2	Definition of terms

The following terms are used throughout this annex:

B.3.2.1	conditional access: The control and security of subscriber access to cable or broadcast services and events in the form of video, data and voice communications.

B.3.2.2	host: A device capable of supporting a POD module by implementing the interface protocol defined in SCTE DVS 131r7 (1998) and SCTE DVS 216r4 (2000). These protocols define the Extended Channel data path through which the SI tables defined in this annex are passed.

B.3.2.3	navigation: The process of selection and movement among analogue and digital services offered on the cable network. The service information tables defined in this protocol assist in the navigation process by providing physical service locations, channel names and numbers for user reference. Those tables supporting electronic program guides also assist the navigation process.

B.3.2.4	program element: A generic term for one of the elementary streams or other data streams that may be included in a program.

B.3.2.5	program: A collection of program elements. Program elements may be elementary streams. Program elements need not have any defined time base. Those that do have a common time base are intended for synchronized presentation. The term program is also used in the context of a "television program" such as a scheduled daily news broadcast. The distinction between the two usages should be understood by context.

B.3.2.6	region: As used in this annex, a region is a geographical area consisting of one or more countries.

B.3.2.7	section or table section: A data structure comprising a portion of an ITU-T H.222.0 | ISO/IEC 13818-1-defined table, such as the Program Association Table (PAT), Conditional Access Table (CAT), or Program Map Table (PMT). The term conforms to MPEG terminology. All sections begin with the table_ID and end with the CRC_32 field. Sections are carried in Transport Stream packets in which the starting point within a packet payload is indicated by the pointer_field mechanism defined in the ITU-T H.222.0 | ISO/IEC 13818-1 Systems document. The Network Information Table, for example, defines portions of several types of tables.

B.3.2.8	service: ITU-T H.222.0 | ISO/IEC 13818-1 uses the term program to refer to a collection of program elements with no regard to time. In this Service Information annex, the term service is used in this same context to denote a collection of elementary components. Usage of the term service clarifies certain discussions that also involve the notion of the term program in its traditional meaning  for example, in the statement, "A video service carries a series of programs." In a broader sense, service is also intended for multimedia services of video, voice and data, as these services become prevalent.

B.3.2.9	stream: An ordered series of bytes. The usual context for the term stream involves specification of a particular PID (such as the "Program Map PID stream"), in which case the term indicates a series of bytes extracted from the packet multiplex from packets with the indicated PID value.

### B.3.3	Section and data structure syntax notation

This annex contains symbolic references to syntactic elements. These references are typographically distinguished by the use of a different font (e.g. restricted), may contain the underscore character (e.g. sequence_end_code) and may consist of character strings that are not English words (e.g. dynrng).

The formats of sections and data structures in this annex are described using a C-like notational method employed in ITU-T H.222.0 | ISO/IEC 13818-1. Extensions to this method are described in the following clauses.

#### B.3.3.1	Field sizes

Each data structure is described in a table format wherein the size in bits of each variable within that section is listed in a column labelled "Bits." The column adjacent to the Bits column is labelled "Bytes" and indicates the size of the item in bytes. For convenience, several bits within a particular byte or multi-byte variable may be aggregated for the count. Table B.1 is an example:

Table B.1/J.94 – Field sizes example

|  | Bits | Bytes | Format |
| --- | --- | --- | --- |
| foo_section(){ |  |  |  |
| section_syntax_indicator | 1 | 1 |  |
| ... |  |  |  |
| if (section_syntax_indicator) { |  |  |  |
| table_extension | 16 | (2) | uimsbf |
| Reserved | 2 | (1) | bslbf |
| version_number | 5 |  | uimsbf |
| current_next_indicator | 1 |  | bslbf {next, current} |
| ... |  |  |  |
| } |  |  |  |
| ... |  |  |  |

In the byte count column, items that are conditional (because they are within a loop or conditional statement) are in parentheses. Nested parentheses are used if the loops or conditions are nested.

## B.4	Acronyms and abbreviations

The following acronyms and abbreviations are used within this annex:

AEIT		Aggregate Event Information Table

AETT		Aggregate Extended Text Table

ATSC		Advanced Television Standards Committee

BMP		Basic Multilingual Plane

bslbf		bit serial, leftmost bit first

CAT		Conditional Access Table

CC		Closed Caption

CDS		Carrier Definition Subtable

CRC		Cyclic Redundancy Check

DCM		Defined Channels Map

DTV		Digital Television

ECM		Entitlement Control Message

EMM		Entitlement Management Message

ETSI		European Telecommunications Standards Institute

GPS		Global Positioning System

ICM		Inverse Channel Map

ITU		International Telecommunication Union

LSB		Least Significant Bit

L-VCT		Long-form Virtual Channel Table

MGT		Master Guide Table

MMS		Modulation Mode Subtable

MPAA		Motion Picture Association of America

MPEG		Moving Picture Experts Group

MSB		Most Significant Bit

MSS		Multiple String Structure

MTS		Multi-lingual Text String

NTSC		National Television System Committee

NVOD		Near Video On Demand

OOB		Out-of-band

PAT		Program Association Table

PCR		Program Clock Reference

PES		Packetized Elementary Stream

PID		Packet Identifier

PMT		Program Map Table

POD		Point of Deployment

PSIP		Program and System Information Protocol

PTC		Physical Transmission Channel

PTS		Presentation Time Stamp

rpchof		remainder polynomial coefficients, highest order first

RRT		Rating Region Table

SCTE		Society of Cable Telecommunications Engineers

SI		Service Information

SNS		Source Name Subtable

S-VCT		Short-form Virtual Channel Table

TS		Transport Stream

uimsbf		unsigned integer, most significant bit first

UTC		Coordinated Universal Time

VCM		Virtual Channel Map

## B.5	Table structure

This clause describes details of the structure of MPEG-2 tables defined in this annex.

Tables and table sections defined in this Service Information annex are structured in the same manner used for carrying ITU-T H.222.0 | ISO/IEC 13818-1-defined PSI tables. The MPEG-defined 32-bit CRC is required.

### B.5.1	Table ID ranges and values

Table B.2 defines table_ID ranges and values for tables defined in MPEG and in this annex.

Table B.2/J.94 – Table ID ranges and values for out-of-band transport

| Table ID Value (hex) | Tables | PID | Reference |
| --- | --- | --- | --- |
|  | ITU-T H.222.0 ¦ ISO/IEC 13818-1 Sections: |  |  |
| 0x00 | Program Association Table (PAT) | 0 | ITU-T H.222.0 |
| 0x01 | Conditional Access Table (CAT) | 1 | ITU-T H.222.0 |
| 0x02 | TS Program Map Table (PMT) | per PAT | ITU-T H.222.0 |
| 0x03-0x3F | [ISO Reserved] |  |  |
|  | User Private Sections: |  |  |
| 0x40-0x7F | [User Private for other systems] |  |  |
| 0x80-0xBF | [SCTE User Private] |  |  |
|  | Other Standards: |  |  |
| 0xC0-0xC1 | [Used in other standards] |  |  |
|  | Service Information Tables: |  |  |
| 0xC2 | Network Information Table (NIT) | 0x1FFC | B.6.1 |
| 0xC3 | Network Text Table (NTT) | 0x1FFC | B.6.2 |
| 0xC4 | Short-form Virtual Channel Table (S-VCT) | 0x1FFC | B.6.3 |
| 0xC5 | System Timetable (STT) | 0x1FFC | B.6.4 |
| 0xC6 | [Used in other standards] |  |  |
| 0xC7 | Master Guide Table (MGT) | 0x1FFC | B.6.5 |

Table B.2/J.94 – Table ID ranges and values for out-of-band transport (concluded)

| Table ID Value (hex) | Tables | PID | Reference |
| --- | --- | --- | --- |
| 0xC8 | Reserved |  |  |
| 0xC9 | Long-form Virtual Channel Table (L-VCT) | 0x1FFC | B.6.6 |
| 0xCA | Rating Region Table (RRT) | 0x1FFC | B.6.7 |
| 0xCB-0xD5 | [Used in ATSC] |  |  |
| 0xD6 | Aggregate Event Information Table (AEIT) | per MGT | B.6.8 |
| 0xD7 | Aggregate Extended Text Table (AETT) | per MGT | B.6.9 |
| 0xD8 | Cable Emergency Alert Message | 0x1FFC | SCTE DVS 208r6 (1999) |
| 0xD9-0xFE | [Reserved for future use] |  |  |

Table sections defined in this Service Information annex, and any created as user extensions to it are considered "private" with respect to ITU-T H.222.0 | ISO/IEC 13818-1. Table section types 0x80 through 0xBF are user-defined (outside the scope of this Service Information annex).

The maximum total length of any table section defined in this annex is 1024 bytes, except for the MGT, L-VCT, AEIT and AETT, each of which has a maximum total length of 4096 bytes. This total includes table_ID, CRC, and all fields contained within the specific table section.

### B.5.2	Extensibility

This Service Information annex defines a number of tables and table sections. The Service Information annex is designed to be extensible via the following mechanisms:

1)	Reserved Fields: Fields in this Service Information annex marked reserved are reserved for use either when revising this annex, or when another Recommendation is issued that builds upon this one. See B.5.4.

2)	Standard Table Types: As indicated in Table B.2, table_ID values in the range 0xCE through 0xFE are reserved for use either when revising this Service Information annex, or when another Recommendation is issued that builds upon this one.

3)	User Private Table Types: As indicated in Table B.2, table_ID values in the range 0x80 through 0xBF are reserved for "user private" use. The format of user private tables carried in the Network PID shall conform to the syntax described in Table B.3.

4)	User Private Descriptors: Privately defined descriptors may be placed at designated locations throughout the table sections described in this Service Information annex. Ownership of one or more user private descriptors is indicated by the presence of an MPEG registration_descriptor() preceding the descriptor(s).

Table B.3/J.94  Network private table section format

|  | Bits | Bytes | Format |
| --- | --- | --- | --- |
| Network_private_table section(){ |  |  |  |
| private_table_ID | 8 | 1 | uimsbf (0x80 <= table_ID <= 0xBF) |
| section_syntax_indicator | 1 | 2 | bslbf |
| Zero | 1 |  | bslbf |
| Reserved | 2 |  | bslbf |
| section_length | 12 |  | uimsbf |
| if (section_syntax_indicator==1) { |  |  |  |
| table_extension | 16 | (2) | uimsbf |
| Reserved | 2 | (1) | bslbf |
| version_number | 5 |  | uimsbf |
| current_next_indicator | 1 |  | bslbf {next, current} |
| section_number | 8 | (1) | uimsbf |
| last_section_number | 8 | (1) | uimsbf |
| } |  |  |  |
| Zero | 3 | 1 | bslbf |
| protocol_version | 5 |  | See B.5.4.1. |
| format_identifier | 32 | 4 | uimsbf |
| private_message_body() | N*8 | N |  |
| CRC_32 | 32 | 4 | rpchof |
| } |  |  |  |

### B.5.3	Reserved fields

reserved: Fields in this Service Information annex marked reserved shall not be assigned by the user, but shall be available for future use. Hosts are expected to disregard reserved fields for which no definition exists that is known to that unit. Fields marked reserved shall be set to "1" until such time as they are defined and supported.

zero: Indicates the bit or bit field shall be "0".

### B.5.4	Private table section syntax

Table B.3 defines the syntax for user private table sections. The MPEG-defined CRC is required. Refer to ITU-T H.222.0 | ISO/IEC 13818-1 for definition of MPEG-standard fields.

private_table_ID: The value of table_ID in private table sections shall be in the range 0x80 through 0xBF.

#### B.5.4.1	Protocol version

protocol_version: A 5-bit unsigned integer field whose function is to allow, in the future, any defined table type to carry parameters that may be structured fundamentally differently from those defined in the current protocol. At present, all defined table section types in this protocol are defined for protocol_version zero only. Nonzero values of protocol_version may only be processed by Receivers designed to accommodate the later versions as they become standardized.

#### B.5.4.2	Format identifier

format_identifier: A 32-bit unsigned integer value which unambiguously identifies the entity defining this network_private_table_section() syntax. Values for format_identifiers shall be obtained from SCTE.

#### B.5.4.3	Private Message Body

private_message_body(): A data structure defined by the private entity identified by format_identifier.

#### B.5.4.4	CRC

CRC_32: The 32-bit CRC value defined in ITU-T H.222.0 | ISO/IEC 13818-1 for PSI sections. The MPEG-2 CRC shall be checked in the POD, and only messages that pass the CRC check shall be forwarded to the Host. The Host shall not check the CRC.

## B.6	Table section formats

The following clauses define the formats of table sections as they are delivered across the Extended Channel interface from POD module to Host.

### B.6.1	Network Information Table

Sections of the Network Information Table shall be associated on the POD-Host interface with PID value 0x1FFC, the SI_base PID. This table delivers sections of non-textual tables applicable system-wide. The table types included are the Carrier Definition Subtable (CDS) and the Modulation Mode Subtable (MMS).

Table B.4 shows the format of the Network Information Table section.

| Table B.4/J.94 – Network Information Table section format |  |  |  |
| --- | --- | --- | --- |
|  | Bits | Bytes | Format |
| network_info_table_section(){ |  |  |  |
| table_ID | 8 | 1 | uimsbf value 0xC2 |
| Zero | 2 | 2 | bslbf |
| Reserved | 2 |  | bslbf |
| section_length | 12 |  | uimsbf |
| Zero | 3 | 1 | bslbf |
| protocol_version | 5 |  | See B.5.4.1. |
| first_index | 8 | 1 | uimsbf range 1-255 |
| number_of_records | 8 | 1 | uimsbf |
| transmission_medium | 4 | 1 | uimsbf |
| table_subtype | 4 |  | uimsbf (See Table B.5.) |
| for (i=0; i<number_of_records; i++) { |  |  |  |
| if (table_subtype==CDS) { |  |  |  |
| CDS_record() |  | ((5)) |  |
| } |  |  |  |
| if (table_subtype==MMS) { |  |  |  |
| MMS_record() |  | ((6)) |  |
| } |  |  |  |
| Descriptors_count | 8 | (1) | uimsbf range 0-255 |
| for (i=0; i<descriptors_count; i++) { |  |  |  |
| descriptor() | * | ((*)) | Optional |
| } |  |  |  |
| } |  |  |  |
| for (i=0; i<N; i++) { |  |  |  |
| descriptor() | * | (*) | Optional |
| } |  |  |  |
| CRC_32 | 32 | 4 | rpchof |
| } |  |  |  |

table_ID: The table_ID of the Network Information Table section shall be 0xC2.

first_index: An 8-bit unsigned integer number in the range 1 to 255 that indicates the index of the first record to be defined in this table section. If more than one record is provided, the additional records define successive table entries following first_index. The value zero is illegal and shall not be specified.

number_of_records: An 8-bit unsigned integer number that specifies the number of records being defined in this table section. The maximum is limited by the maximum allowed length of the table section.

transmission_medium: This 4-bit field shall be set to zero (0x0).

table_subtype: A 4-bit value that defines the type of table delivered in the table section. One instance of a Network Information Table section can define entries within at most one type of table. The table_subtype parameter is defined in Table B.5.

Table B.5/J.94 – Network Information Table Subtype

| table_subtype | Meaning |
| --- | --- |
| 0 | invalid |
| 1 | CDS  Carrier Definition Subtable |
| 2 | MMS  Modulation Mode Subtable |
| 3-15 | Reserved |

The receiver shall discard a Network Information Table section with table_subtype indicating an unknown or unsupported table_subtype.

#### B.6.1.1	Carrier Definition Subtable (CDS)

Table B.6 defines the structure of the CDS_record(). Each CDS defines a set of carrier frequencies. A full frequency plan table shall be constructed from one or more CDS_record() structures, each defining a starting frequency, a number of carriers, and a frequency spacing for carriers in this group.

The specified carrier represents the nominal centre of the spectral band for all modulation methods, including analogue. Carrier frequencies in the table thus represent the data carrier frequency for digital transmissions modulated using QAM or PSK.

Each CDS_record represents a definition of N carriers. The first_index parameter reflects the index in a flat space between 1 and 255, representing the first carrier in the CDS_record. Starting from the first CDS_record defining carriers C1, C2, C3, …, CN, where N = number_of_carriers, the carrier index for CI is equal to first_index + I – 1. If the table section includes more than one CDS_record(), the carrier index of the second CDS_record would be first_index plus the number of carriers defined in the first CDS_record(), namely, first_index + number_of_carriers. References to the Carrier Definition Subtable, such as the CDS_reference in the virtual_channel() of Table B.20, are to the carrier index (a carrier defined within a CDS_record()), between 1 and N, where N is normally much smaller than 255. These references are not to the index of a CDS_record() itself, which is sequenced from first_index and is not reset to 1 until it exceeds 255.

Note that the carriers, as defined by one or more CDS_record()s, may or may not end up sorted in the order of increasing carrier frequency. Certain frequency plans may be specified by overlapping two or more CDS_record()s, each of which defines equally-spaced carriers.

Note also that carriers may be defined that are currently not in use. To facilitate the compressed delivery format, defined carriers may not reflect reality. An example: carriers at 1, 2, 4, 5, 7, 8 MHz could be defined as eight carriers at 1 MHz spacing (3 MHz and 6 MHz do not really exist, or are not currently in use).

Table B.6/J.94 – CDS record format

|  | Bits | Bytes | Format |
| --- | --- | --- | --- |
| CDS_record(){ |  |  |  |
| number_of_carriers | 8 | 1 | uimsbf |
| spacing_unit | 1 | 2 | bslbf (See Table B.7.) |
| Zero | 1 |  | bslbf |
| Frequency_spacing | 14 |  | uimsbf range 1-16 383 units of 10 or 125 kHz |
| Frequency_unit | 1 | 2 | bslbf (See Table B.8.) |
| first_carrier_frequency | 15 |  | uimsbf range 0-32 767 units of 10 or 125 kHz |
| } |  |  |  |

number_of_carriers: An unsigned integer in the range 1 to 255 that represents the number of carriers whose frequency is being defined by this CDS_record().

spacing_unit: A 1-bit field identifying the units for the frequency_spacing field. Table B.7 defines the coding for spacing_unit.

Table B.7/J.94 – Spacing Unit

| spacing_unit | Meaning |
| --- | --- |
| 0 | 10 kHz spacing |
| 1 | 125 kHz spacing |

frequency_spacing: A 14-bit unsigned integer number in the range 1 to 16 383 that defines the frequency spacing in units of either 10 kHz or 125 kHz, depending upon the value of the spacing_unit parameter. If spacing_unit is zero, indicating 10 kHz, then a value of 1 indicates 10 kHz spacing; 2 indicates 20 kHz, and so on. If the number_of_carriers field is one, the frequency_spacing field is ignored. The maximum frequency spacing that can be represented is (214  1) * 125 kHz = 2047.875 MHz. The minimum frequency spacing is 10 kHz.

frequency_unit: A 1-bit field identifying the units for the first_carrier_frequency field. Table B.8 defines the coding for frequency_unit.

Table B.8/J.94 – Frequency Unit

| Frequency_unit | Meaning |
| --- | --- |
| 0 | 10 kHz units |
| 1 | 125 kHz units |

first_carrier_frequency: A 15-bit unsigned integer number in the range 0 to 32 767 that defines the starting carrier frequency for the carriers defined in this group, in units of either 10 kHz or 125 kHz, depending on the value of frequency_unit. If only one carrier is defined for the group, the first_carrier_frequency represents its frequency. When the frequency_unit indicates 125 kHz, the first_carrier_frequency can be interpreted as a fractional frequency (1/8 MHz) in the least-significant 3 bits, and an integer number of megahertz in the upper 12 bits. The range of frequencies that can be represented is 0 to (215  1) * 125 kHz = 4095.875 MHz.

#### B.6.1.2	Modulation Mode Subtable (MMS)

Table B.9 defines the structure of the MMS_record().

Table B.9/J.94 – MMS record format

|  | Bits | Bytes | Format |
| --- | --- | --- | --- |
| MMS_record(){ |  |  |  |
| transmission_system | 4 | 1 | uimsbf (See Table B.10.) |
| inner_coding_mode | 4 |  | uimsbf (See Table B.11.) |
| split_bitstream_mode | 1 | 1 | bslbf {no, yes} |
| Zero | 2 |  | bslbf |
| modulation_format | 5 |  | uimsbf (See Table B.12.) |
| Zero | 4 | 4 | bslbf |
| symbol_rate | 28 |  | uimsbf units: symbols per second |
| } |  |  |  |

transmission_system: A 4-bit field that identifies the transmission standard employed for the waveform defined by this MMS record. Table B.10 defines the coding for transmission_system.

Table B.10/J.94 – Transmission System

| transmission_system | Meaning |
| --- | --- |
| 0 | unknown  The transmission system is not known. |
| 1 | Reserved (ETSI) |
| 2 | ITU-T J.83 Annex B  The transmission system conforms to the ITU North American standard specified in Annex B/J.83. |
| 3 | Defined for use in other systems |
| 4 | ATSC  The transmission system conforms to the ATSC Digital Television Standard. |
| 5-15 | Reserved (satellite) |

inner_coding_mode: A 4-bit field that indicates the coding mode for the inner code associated with the waveform described in this MMS record. The following values are currently defined: 5/11, 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, and 7/8. Coding of the inner_coding_mode field is shown in Table B.11.

Table B.11/J.94 – Inner Coding Mode

| inner_coding_mode | Meaning |
| --- | --- |
| 0 | rate 5/11 coding |
| 1 | rate 1/2 coding |
| 2 | Reserved |
| 3 | rate 3/5 coding |
| 4 | Reserved |
| 5 | rate 2/3 coding |
| 6 | Reserved |
| 7 | rate 3/4 coding |
| 8 | rate 4/5 coding |
| 9 | rate 5/6 coding |
| 10 | Reserved |
| 11 | rate 7/8 coding |
| 12-14 | Reserved |
| 15 | None – indicates that the waveform does not use concatenated coding |

modulation_format: A 5-bit field that defines the basic modulation format for the carrier. Table B.12 defines the parameter.

| Table B.12/J.94 – Modulation Format |  |
| --- | --- |
| modulation_format | Meaning |
| 0 | unknown  The modulation format is unknown. |
| 1 | QPSK  The modulation format is QPSK (Quadrature Phase Shift Keying). |
| 2 | BPSK  The modulation format is BPSK (Binary Phase Shift Keying). |
| 3 | OQPSK  The modulation format is offset QPSK. |
| 4 | VSB 8  The modulation format is 8-level VSB (Vestigial Sideband). |
| 5 | VSB 16  The modulation format is 16-level VSB. |
| 6 | QAM 16  Modulation format 16-level Quadrature Amplitude Modulation (QAM). |
| 7 | QAM 32  32-level QAM |
| 8 | QAM 64  64-level QAM |
| 9 | QAM 80  80-level QAM |
| 10 | QAM 96  96-level QAM |
| 11 | QAM 112  112-level QAM |
| 12 | QAM 128  128-level QAM |
| 13 | QAM 160  160-level QAM |
| 14 | QAM 192  192-level QAM |
| 15 | QAM 224  224-level QAM |
| 16 | QAM 256  256-level QAM |
| 17 | QAM 320  320-level QAM |
| 18 | QAM 384  384-level QAM |
| 19 | QAM 448  448-level QAM |

Table B.12/J.94 – Modulation Format (concluded)

| modulation_format | Meaning |
| --- | --- |
| 20 | QAM 512  512-level QAM |
| 21 | QAM 640  640-level QAM |
| 22 | QAM 768  768-level QAM |
| 23 | QAM 896  896-level QAM |
| 24 | QAM 1024  1024-level QAM |
| 25-31 | Reserved |

symbol_rate: A 28-bit unsigned integer field that indicates the symbol rate in symbols per second associated with the waveform described in this MMS record.

#### B.6.1.3	Descriptors Count

descriptors_count: An 8-bit unsigned integer value in the range 0 to 255 representing the number of descriptor blocks to follow.

descriptor(): The table section may include at its end one or more structures of the form tag, length, data. The number of descriptors present is determined indirectly by processing the section_length field. Descriptors are defined in B.7.

### B.6.2	Network Text Table

The Network Text Table shall be associated on the POD-Host interface with PID value 0x1FFC, the SI_base PID. This table delivers sections of textual tables applicable system-wide. Each instance of Network Text Table is associated with a language, as such the textual information may be provided multi-lingually. The Network Text Table delivers the Source Name Subtable (SNS).

Table B.13 shows the format of the Network Text Table.

| Table B.13/J.94 – Network Text Table section format |  |  |  |
| --- | --- | --- | --- |
|  | Bits | Bytes | Format |
| network_text_table_section(){ |  |  |  |
| table_ID | 8 | 1 | uimsbf value 0xC3 |
| Zero | 2 | 2 | bslbf |
| Reserved | 2 |  | bslbf |
| section_length | 12 |  | uimsbf |
| Zero | 3 | 1 |  |
| protocol_version | 5 |  | See B.5.4.1. |
| ISO_639_language_code | 24 | 3 | Per ISO 639-2/B |
| transmission_medium | 4 | 1 | uimsbf |
| table_subtype | 4 |  | uimsbf (See Table B.14.) |
| if (table_subtype==SNS) { |  |  |  |
| source_name_subtable() | * | (*) |  |
| } |  |  |  |
| for (i=0; i<N; i++) { |  |  |  |
| descriptor() | * | (*) | Optional |
| } |  |  |  |
| CRC_32 | 32 | 4 | rpchof |
| } |  |  |  |

The Network Text Table carries Multilingual Text Strings, formatted as defined in B.8.2. Text Strings included in the Network Text Table shall not include format effectors (defined in B.8.1.2). If format effectors are present in a text block, the Host is expected to disregard them.

table_ID: The table_ID of the Network Text Table section shall be 0xC3.

ISO_639_language_code: A 3-byte language code per ISO 639-2/B defining the language associated with the text carried in this Network Text Table. The ISO_639_language_code field contains a three-character code as specified by ISO 639-2/B. Each character is coded into 8 bits according to ISO 8859-1 (ISO Latin-1) and inserted, in order, into the 24-bit field. The value 0xFFFFFF shall be used in case the text is available in one language only. The value 0xFFFFFF shall represent a "wild card" match when filtering by language.

transmission_medium: This 4-bit field shall be set to zero (0x0).

table_subtype: A 4-bit value that defines the type of table delivered in the table section. One instance of a Network Text Table section can define entries within at most one type of table. The table_subtype parameter is defined in Table B.14.

Table B.14/J.94 – Network Text Table Subtype

| table_subtype | Meaning |
| --- | --- |
| 0 | Invalid |
| 1-5 | Reserved |
| 6 | SNS  Source Name Subtable |
| 7-15 | Reserved |

A Host shall discard a Network Text Table section with table_subtype indicating an unknown or unsupported value.

The SNS can provide a textual name associated with each service defined in the Short-form Virtual Channel Table, by reference to its source_ID. The format of the source_name_subtable() is given in Table B.15.

| Table B.15/J.94 – Source Name Subtable format |  |  |  |
| --- | --- | --- | --- |
|  | Bits | Bytes | Format |
| source_name_subtable(){ |  |  |  |
| number_of_SNS_records | 8 | 1 | uimsbf range 1-255 |
| for (i=0; i<number_of_SNS_records; i++) { |  |  |  |
| application_type | 1 | (1) | bslbf {false, true} |
| Zero | 7 |  | bslbf |
| if (application_type) { |  |  |  |
| Application_ID | 16 | ((2)) | uimsbf |
| } else { |  |  |  |
| source_ID | 16 | ((2)) | uimsbf |
| } |  |  |  |
| name_length | 8 | (1) | Size of source_name() (L) |
| source_name() | L*8 | (L) | Multilingual text |
| SNS_descriptors_count | 8 | (1) | uimsbf range 0-255 |
| for (i=0; i<SNS_descriptors_count; i++) { |  |  |  |
| descriptor() | * | ((*)) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

- number_of_SNS_records: An unsigned 8-bit integer number in the range 1 to 255 that specifies the number of records being defined in this table section.

- application_type: A Boolean flag, when set, indicates that the name string being defined is for an application of the given application_ID. When the flag is clear, the name string being defined is for a source of the given source_ID. Support for application-type virtual channels is optional. Hosts not supporting application-type virtual channels may disregard name strings associated with these VC. Support for application-type virtual channels is beyond the scope of this annex.

- application_ID: A 16-bit unsigned integer value identifying the application associated with the name string that follows. This field may be disregarded by Hosts not supporting application-type virtual channels.

- source_ID: A 16-bit unsigned integer value identifying the programming source associated with the source name to follow.

- name_length: An unsigned 8-bit integer number in the range 1 to 255 that defines the number of bytes in the source_name() that follows.

- source_name(): A Multilingual Text String defining the name of the source or application, formatted according to the rules defined in B.8.1.

SNS_descriptors_count: An unsigned 8-bit integer number, in the range 0 to 255, that defines the number of descriptors to follow.

descriptor(): The table section may include, at its end, one or more structures of the form tag, length, data. The number of descriptors present is determined indirectly by processing the section_length field. Descriptors are defined in B.7.

### B.6.3	Short-form Virtual Channel Table Section

The Short-form Virtual Channel Table section delivers portions of the Virtual Channel Map (VCM), the Defined Channels Map (DCM) and the Inverse Channel Map (ICM). Sections of the Short-form Virtual Channel Table shall be associated on the POD-Host interface with PID value 0x1FFC, the SI_base PID.

Table B.16 shows the syntax of the Short-form Virtual Channel Table section.

Table B.16/J.94 – Short-form Virtual Channel Table section format

|  | Bits | Bytes | Format |
| --- | --- | --- | --- |
| shortform_virtual_channel_table_section(){ |  |  |  |
| table_ID | 8 | 1 | uimsbf value 0xC4 |
| Zero | 2 | 2 | bslbf |
| Reserved | 2 |  | bslbf |
| section_length | 12 |  | uimsbf |
| Zero | 3 | 1 | bslbf |
| protocol_version | 5 |  | See B.5.4.1. |
| transmission_medium | 4 | 1 | uimsbf |
| table_subtype | 4 |  | uimsbf (See Table B.17.) |
| VCT_ID | 16 | 2 | uimsbf |
| if (table_subtype==DCM) { |  |  |  |
| DCM_structure() | * | (*) |  |
| } |  |  |  |
| if (table_subtype== VCM) { |  |  |  |
| VCM_structure() | * | (*) |  |
| } |  |  |  |

Table B.16/J.94 – Short-form Virtual Channel Table section format (concluded)

|  | Bits | Bytes | Format |
| --- | --- | --- | --- |
| if (table_subtype== ICM) { |  |  |  |
| ICM_structure() | * | (*) |  |
| } |  |  |  |
| for (i=0; i<N; i++) { |  |  |  |
| descriptor() | * | (*) | Optional |
| } |  |  |  |
| CRC_32 | 32 | 4 | rpchof |
| } |  |  |  |

table_ID: The table_ID of the Short-form Virtual Channel Table shall be 0xC4.

transmission_medium: This 4-bit field shall be set to zero (0x0).

table_subtype: A 4-bit field that indicates the map type being delivered in this S-VCT section. Three map types are currently defined: the Virtual Channel Map (VCM), the Defined Channels Map (DCM), and the Inverse Channel Map (ICM). Table B.17 defines table_subtype.

Table B.17/J.94 – S-VCT Table Subtypes

| table_subtype | Meaning |
| --- | --- |
| 0 | VCM  Virtual Channel Map |
| 1 | DCM  Defined Channels Map |
| 2 | ICM  Inverse Channel Map |
| 3-15 | Reserved |

An S-VCT section received with table_subtype indicating an unknown or unsupported map type shall be discarded.

VCT_ID: A 16-bit unsigned integer value, in the range 0x0000 to 0xFFFF, indicating the VCT to which the channel definitions in this table section apply. This 16-bit field may be used by the POD module for filtering purposes. The Host is expected to ignore VCT_ID. Only one version of the S-VCT, corresponding to one value of VCT_ID, shall be delivered to the Host across the Extended Channel interface at a given time.

#### B.6.3.1	Defined Channels Map

Table B.18 shows the format of the DCM_structure().

| Table B.18/J.94 – DCM structure format |  |  |  |
| --- | --- | --- | --- |
|  | Bits | Bytes | Format |
| DCM_structure(){ |  |  |  |
| Zero | 4 | 2 | bslbf |
| first_virtual_channel | 12 |  | uimsbf range 0-4095 |
| zero | 1 | 1 | bslbf |
| DCM_data_length | 7 |  | uimsbf range 1-127 |
| for (i=0; i<DCM_data_length; i++) { |  |  |  |
| range_defined | 1 | (1) | bslbf {no, yes} |
| channels_count | 7 |  | uimsbf range 1-127 |
| } |  |  |  |
| } |  |  |  |

first_virtual_channel: An unsigned 12-bit integer reflecting the first virtual channel whose existence is being provided by this table section, for the map identified by the VCT_ID field. The range is 0 to 4095.

DCM_data_length: A 7-bit unsigned integer number, in the range 1 to 127, that defines the number of DCM data fields to follow in the table section.

The DCM data bytes taken as a whole define which virtual channels, starting at the channel number defined by first_virtual_channel, are defined and which are not. Each DCM_data_field defines two pieces of data: a flag indicating whether this block of channels is defined or not, and the number of channels in the block. The bytes are interpreted in an accumulative way, with a pointer into the Short-form Virtual Channel Table which is initialized to first_virtual_channel. As each byte is processed, the pointer is incremented by the number of channels indicated by the channels_count field.

For example, if channels 2-90, 200-210, 400-410, 600-610, 800-810, and 999 were defined, and first_virtual_channel was zero, the DCM data sequence (in decimal) would be the following, where underlined numbers have the range_defined bit set: 2, 89, 109, 11, 127, 62, 11, 127, 62, 11, 127, 62, 11, 127, 61, 1.

range_defined: A Boolean flag that indicates, when true, that the number of channels given by channels_count is defined in the VCT, starting at the current pointer value. When the flag is clear, the number of channels equal to channels_count are currently not defined starting at the current pointer value.

channels_count: An unsigned 7-bit integer number, in the range 1 to 127, that indicates the number of defined (or undefined) channels in a group.

#### B.6.3.2	Virtual Channel Map

Table B.19 shows the format of the VCM_structure().

Table B.19/J.94 – VCM structure format

|  | Bits | Bytes | Format |
| --- | --- | --- | --- |
| VCM_structure(){ |  |  |  |
| zero | 2 | 1 | bslbf |
| descriptors_included | 1 |  | bslbf {no, yes} |
| Zero | 5 |  | bslbf |
| Splice | 1 | 1 | bslbf {no, yes} |
| Zero | 7 |  | bslbf |
| activation_time | 32 | 4 | uimsbf |
| number_of_VC_records | 8 | 1 |  |
| for (i=0; i<number_of_VC_records; i++) { |  |  |  |
| virtual_channel() | * | (*) |  |
| } |  |  |  |
| } |  |  |  |

descriptors_included: A Boolean flag that indicates, when set, that one or more record-level descriptors are present in the table section. Record-level descriptors are those defined in Table B.20 following the "if (descriptors_included)" statement. When the flag is clear, the record-level descriptor block is absent. The descriptors_included flag is not applicable to the section level descriptors shown at the bottom of Table B.16.

The activation time indicates the time at which the data delivered in the table section will be valid.

splice: A Boolean flag that indicates, when set, that the Host should arm video processing hardware to execute the application of the data delivered in the VCM_structure() at the next MPEG-2 video splice point if the virtual channel changes described in the table section apply to a currently acquired channel, and the activation_time is reached. If the activation is immediate or specified as a time that has since passed, the data should be applied immediately. When the splice flag is clear, the virtual channel change is made directly, without arming video hardware for a splice.

activation_time: A 32-bit unsigned integer field providing the absolute second the virtual channel data carried in the table section will be valid, defined as the number of seconds since 0000 Hours UTC, January 6th, 1980. If the GPS_UTC_offset delivered in the System Timetable is zero, activation_time includes the correction for leap seconds. Otherwise, activation_time can be converted to UTC by subtracting the GPS_UTC_offset. If the activation_time is in the past, the data in the table section shall be considered valid immediately. An activation_time value of zero shall be used to indicate immediate activation.

A Host may enter a virtual channel record whose activation times are in the future into a queue. Such a queue may be called a pending virtual channel queue. Hosts are not required to implement a pending virtual channel queue, and may choose to discard any data that is not currently applicable.

number_of_VC_records: An 8-bit unsigned integer number, in the range 1 to 255, that identifies the number of virtual_channel() records to follow in the table section. The number of records included is further limited by the allowed maximum table section length.

virtual_channel(): Table B.20 defines the virtual_channel() record structure.

Table B.20/J.94 – Virtual channel record format

|  | Bits | Bytes | Format |
| --- | --- | --- | --- |
| virtual_channel(){ |  |  |  |
| Zero | 4 | 2 | bslbf |
| virtual_channel_number | 12 |  | uimsbf range 0-4095 |
| application_virtual_channel | 1 | 1 | bslbf {no, yes} |
| Zero | 1 |  | bslbf |
| path_select | 1 |  | bslbf (See Table B.21.) |
| transport_type | 1 |  | bslbf (See Table B.22.) |
| channel_type | 4 |  | uimsbf (See Table B.23.) |
| if (application_virtual_channel) { |  |  |  |
| application_ID | 16 | (2) |  |
| } else { |  |  |  |
| source_ID | 16 | (2) |  |
| } |  |  |  |
| if (transport_type==MPEG_2) { |  |  |  |
| CDS_reference | 8 | ((1)) | uimsbf range 1-255 |
| program_number | 16 | ((2)) |  |
| MMS_reference | 8 | ((1)) | uimsbf range 1-255 |
| } else { /* non-MPEG-2 */ |  |  |  |
| CDS_reference | 8 | ((1)) | uimsbf range 0-255 |
| Scrambled | 1 | ((1)) | bslbf {no, yes} |
| Zero | 3 |  | bslbf |
| video_standard | 4 |  | uimsbf (See Table B.24.) |
| Zero | 16 | ((2)) | bslbf |
| } |  |  |  |
| if (descriptors_included) { |  |  |  |

Table B.20/J.94 – Virtual channel record format (concluded)

|  | Bits | Bytes | Format |
| --- | --- | --- | --- |
| descriptors_count | 8 | (1) | uimsbf |
| for (i=0; i<descriptors_count; i++) { |  |  |  |
| descriptor() | * | ((*)) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

virtual_channel_number: An unsigned 12-bit integer, in the range 0 to 4095, reflecting the virtual channel whose definition is being provided by this virtual channel record, for the map identified by the VCT_ID field.

application_virtual_channel: A binary flag that, when set, indicates this virtual channel defines an access point represented by the application_ID. When the flag is clear, the channel is not an application access point, and this virtual channel defines an access point represented by the source_ID. Support for application-type virtual channels is optional. Hosts not supporting application-type virtual channels may disregard all data associated with them. Support for application-type virtual channels is beyond the scope of this annex.

path_select: A 1-bit field that associates the virtual channel with a transmission path. For the cable transmission medium, path_select identifies which physical cable carries the Transport Stream associated with this virtual channel. Table B.21 defines path_select.

Table B.21/J.94 – Path Select

| path_select | meaning |
| --- | --- |
| 0 | path 1 |
| 1 | path 2 |

transport_type: A 1-bit field identifying the type of transport carried on this carrier as either being an MPEG-2 transport (value zero), or not (value one). Table B.22 defines the coding.

Table B.22/J.94 – Transport Type

| transport_type | Meaning |
| --- | --- |
| 0 | MPEG-2 transport |
| 1 | non-MPEG-2 transport |

channel_type: A 4-bit field defining the channel type. Table B.23 defines channel_type.

Table B.23/J.94 – Channel Type

| channel_type | Meaning |
| --- | --- |
| 0 | normal  Indicates that the record is a regular virtual channel record. For non-MPEG-2 channels, the waveform_type shall be defined as "normal." |
| 1 | hidden  Indicates that the record identifies a virtual channel that may not be accessed by the user by direct entry of the channel number (hidden). Hidden channels are skipped when the user is channel surfing, and appear as if undefined if accessed by direct channel entry. Programs constructed for use by specific applications (such as NVOD theaters) utilize hidden virtual channels. If a channel_properties_descriptor() is present and the hide_guide bit is 0, the channel may be considered to be inactive. Inactive channels may appear in EPG displays. |
| 2-15 | reserved  Hosts are expected to treat virtual channel records of unknown channel_type the same as non-existent (undefined) channels. |

application_ID: A 16-bit unsigned integer number, in the range 0x0001 to 0xFFFF, that identifies the application associated with the virtual channel, on a system-wide basis. One particular program guide application, for example, may look for a program carrying data in its native transmission format by searching through the Short-form Virtual Channel Table for a match on its assigned application_ID. In some cases, one application may be able to process streams associated with more than one application ID. The application ID may be used to distinguish content as well as format, for the benefit of processing within the application. The value zero for application_ID shall not be assigned; if specified in a Virtual Channel record, the value zero indicates "unknown" or "inapplicable" for the application_ID/source_ID field.

Support for application-type virtual channels is optional. Hosts not supporting application-type virtual channels may disregard all data associated with them. Support for application-type virtual channels is beyond the scope of this annex.

source_ID: A 16-bit unsigned integer number, in the range 0x0000 to 0xFFFF, that identifies the programming source associated with the virtual channel, on a system-wide basis. In this context, a source is one specific source of video, text, data, or audio programming. For the purposes of referencing virtual channels to the program guide database, each such program source is associated with a unique value of source_ID. The source_ID itself may appear in an EPG database, where it tags entries to specific services. The value zero for source_ID, if used, shall indicate the channel is not associated with a source ID.

program_number: A 16-bit unsigned integer number that associates the virtual channel number being defined with services defined in the Program Association and TS Program Map Table sections. Access to elementary streams defined in each virtual channel record involves first acquiring the Transport Stream on the carrier associated with the virtual channel, then referencing the Program Association section in PID 0 to find the PID associated with the TS Program Map Table section for this program_number. PIDs for each elementary stream are then found by acquisition of the TS Program Map Table section.

A program_number with value 0x0000 (invalid as a regular program number) is reserved to indicate that the Host is expected to discard the corresponding virtual channel record from the queue of pending virtual channel changes. Records are identified in the pending queue by their activation_time, VCT_ID, and virtual_channel_number. If no pending virtual channel change is found in the Host's queue, no action should be taken for this virtual channel (i.e. the record is expected to be discarded).

For inactive channels (those not currently present in the Transport Stream), program_number shall be set to zero. This number shall not be interpreted as pointing to a Program Map Table entry.

descriptors_count: An 8-bit unsigned integer value, in the range 0 to 255, that defines the number of descriptors to follow.

CDS_reference: An unsigned 8-bit integer number, in the range 0 to 255, that identifies the frequency associated with this virtual channel. Values 1 to 255 of CDS_reference are used as indices into the Carrier Definition Subtable to find a frequency to tune to acquire the virtual channel. The value zero is reserved to indicate that the referenced service is carried on all digital multiplexes in this VCM. The CDS_reference field shall be disregarded for inactive channels.

MMS_reference: An 8-bit unsigned integer value, in the range 0 to 255, that references an entry in the Modulation Mode Subtable (MMS). The value zero is illegal and shall not be specified. For digital waveforms, the MMS_reference associates the carrier with a digital modulation mode. For Host implementations that support only one set of modulation parameters, in systems in which one modulation method is used for all carriers, storage and processing of the MMS_reference is unnecessary. The MMS_reference field shall be disregarded for inactive channels.

video_standard: A 4-bit field that indicates the video standard associated with this non-Standard virtual channel. Table B.24 defines video_standard.

Table B.24/J.94 – Video Standard

| video_standard | Meaning |
| --- | --- |
| 0 | NTSC  The video standard is NTSC. |
| 1 | PAL 625  The video standard is 625-line PAL. |
| 2 | PAL 525  The video standard is 525-line PAL. |
| 3 | SECAM  The video standard is SECAM. |
| 4 | MAC  The video standard is MAC. |
| 5-15 | Reserved |

descriptor(): The table section may include, at its end, one or more structures of the form tag, length, data. The number of descriptors present is determined indirectly by processing the section_length field. Descriptors are defined in B.7.

#### B.6.3.3	Inverse Channel Map

The Inverse Channel Map, once reconstructed in the Host from a sequence of Virtual Channel records that belong to the ICM, consists of a list of source_ID/virtual_channel_number pairs, ordered by source_ID. The Host may use this table to quickly find the virtual channel carrying the program given by a particular value of source_ID (by binary search), if such a virtual channel exists. One Inverse Channel Map can be defined per Virtual Channel Map. The ICM may be constructed from the VCM, or linear searches may be done to resolve source_ID references. Transmission of the ICM is therefore optional.

Virtual channels that provide access points for applications (i.e. with the application_virtual_channel flag set to "yes") are not included in the ICM.

Table B.25 describes the format of the ICM_structure().

Table B.25/J.94 – ICM structure format

|  | Bits | Bytes | Format |
| --- | --- | --- | --- |
| ICM_structure(){ |  |  |  |
| Zero | 4 | 2 | bslbf |
| first_map_index | 12 |  | uimsbf range 0-4095 |
| zero | 1 | 1 | bslbf |
| record_count | 7 |  | uimsbf range 1-127 |
| for (i=0; i<record_count; i++) { |  |  |  |
| source_ID | 16 | (2) | uimsbf |
| zero | 4 | (2) | bslbf |
| virtual_channel_number | 12 |  | uimsbf range 0-4095 |
| } |  |  |  |
| } |  |  |  |

first_map_index: A 12-bit unsigned integer, in the range 0 to 4095, that represents the index into the Inverse Channel Map where data carried in this ICM_structure() should be stored.

record_count: A 7-bit unsigned integer value, in the range 1 to 127, that represents the total number of source_ID/virtual_channel pairs defined in this table section.

source_ID: A 16-bit unsigned integer number, in the range 0x0000 to 0xFFFF, that identifies the source associated with the virtual channel, on a system-wide basis. In this context, a "source" is one specific source of video, text, data, or audio programming. For the purposes of referencing virtual channels to the program guide database, each such source is associated with a unique value of source_ID.

virtual_channel_number: A 12-bit unsigned integer value, in the range 0 to 4095, that represents the virtual channel, in the Short-form Virtual Channel Table section (see Table B.16) given by VCT_ID, associated with the given source_ID through the virtual_channel() record (see Table B.20). A virtual_channel_number of zero indicates that the program given by source_ID is currently not carried in this Short-form Virtual Channel Table. Such placeholders are useful in the case where the existence of a certain program within a VCM may come and go.

### B.6.4	System Timetable Section

The System Timetable is used to synchronize Hosts with accurate calendar time. The System Timetable shall be associated on the POD-Host interface with PID value 0x1FFC, the SI_base PID. Rate of transmission is typically once per minute, at second 00 of each minute.

The processing of the System Timetable in the Host is time-critical. Delays between reception and processing of the table section increase the inaccuracy of timed events. Processing delays should be kept below 200 milliseconds.

Table B.26 shows the format of the System Timetable section.

Table B.26/J.94 – System Timetable section format

|  | Bits | Bytes | Format |
| --- | --- | --- | --- |
| system_time_table_section(){ |  |  |  |
| table_ID | 8 | 1 | uimsbf value 0xC5 |
| Zero | 2 | 2 | bslbf |
| Reserved | 2 |  | bslbf |
| section_length | 12 |  | uimsbf |
| Zero | 3 | 1 |  |
| protocol_version | 5 |  | See B.5.4.1. |
| Zero | 8 | 1 | bslbf |
| system_time | 32 | 4 | uimsbf |
| GPS_UTC_offset | 8 | 1 | uimsbf seconds |
| for (i=0; i<N; i++) { |  |  |  |
| descriptor() | * | (*) | Optional |
| } |  |  |  |
| CRC_32 | 32 | 4 | rpchof |
| } |  |  |  |

table_ID: The table_ID of the System Timetable shall be 0xC5.

system_time: A 32-bit unsigned integer quantity representing the current system time, as the number of GPS seconds since 0000 Hours UTC, January 6th, 1980. The system_time value may or may not include the correction factor for leap seconds, depending upon the value of GPS_UTC_offset, as described below.

GPS_UTC_offset: An 8-bit value that serves dual roles. When set to zero, the field indicates that the system_time field carries UTC time directly. When GPS_UTC_offset is not equal to zero, it is interpreted as an 8-bit unsigned integer that defines the current offset in whole seconds between GPS and UTC time standards. To convert GPS time to UTC, the GPS_UTC_offset is subtracted from GPS time. Whenever the International Bureau of Weights and Measures decides that the current offset is too far in error, an additional leap second may be added (or subtracted), and the GPS_UTC_offset will reflect the change.

descriptor(): The table section may include at its end one or more structures of the form tag, length, data. The number of descriptors present is determined indirectly by processing the section_length field. Descriptors are defined in B.7.

### B.6.5	Master Guide Table (MGT)

The Master Guide Table is used to indicate the location, size, and version of tables it references. The MGT shall be associated on the POD-Host interface with PID value 0x1FFC, the SI_base PID. The MGT syntax is shown in Table B.27. Syntax and semantics are identical to SCTE DVS 097, ATSC Standard A/65 (1997), except that additional table types are added to refer to all tables defined in this protocol.

Table B.27/J.94 – Master Guide Table section format

|  | Bits | Bytes | Format |
| --- | --- | --- | --- |
| master_guide_table_section () { |  |  |  |
| table_ID | 8 | 1 | 0xC7 |
| Section_syntax_indicator | 1 | 2 | '1' |
| private_indicator | 1 |  | '1' |
| Reserved | 2 |  | '11' |
| Section_length | 12 |  | uimsbf |
| map_ID | 16 | 2 | uimsbf |
| Reserved | 2 | 1 | '11' |
| version_number | 5 |  | uimsbf |
| current_next_indicator | 1 |  | '1' |
| section_number | 8 | 1 | 0x00 |
| last_section_number | 8 | 1 | 0x00 |
| protocol_version | 8 | 1 | uimsbf |
| Tables_defined | 16 | 2 | uimsbf |
| for (i=0;i<tables_defined;i++) { |  |  |  |
| table_type | 16 | 2 | uimsbf |
| Reserved | 3 | 2 | '111' |
| table_type_PID | 13 |  | uimsbf |
| Reserved | 3 | 1 | '111' |
| table_type_version_number | 5 |  | uimsbf |
| number_bytes | 32 | 4 | uimsbf |
| Reserved | 4 | 2 | '1111' |
| table_type_descriptors_length | 12 |  | uimsbf |
| for (k=0;k<N;k++) |  |  |  |
| descriptor() | var |  |  |
| } |  |  |  |
| Reserved | 4 | 2 | '1111' |
| descriptors_length | 12 |  | uimsbf |
| for (I = 0;I< N;I++) |  |  |  |
| descriptor() | var |  |  |
| CRC_32 | 32 | 4 | rpchof |
| } |  |  |  |

table_ID: The table_ID of the Master Guide Table section shall be 0xC7.

section_syntax_indicator: This 1-bit field shall be set to '1'. It denotes that the section follows the generic section syntax beyond the section length field.

private_indicator: This 1-bit field shall be set to '1'.

section_length: 12-bit field specifying the number of remaining bytes in this section immediately following the section_length field up to the end of the section. The value of the section_length shall be no larger than 4093.

map_ID: This 16-bit field may be used by the POD module for filtering purposes. The Host is expected to ignore map_ID. Only one version of the MGT, corresponding to one value of map_ID shall be delivered to the Host across the Extended Channel interface at a given time. Consequently, the Host can disregard map_ID and may process the MGT version_number field as an indication that the MGT version has changed.

NOTE  The map_ID may be considered to be an identifier for this instance of the Master Guide Table. In some applications, the POD module may receive multiple Master Guide Table sections corresponding to distinct channel maps. In this case, the POD module is responsible for accepting one MGT and discard the others. It may use the map_ID to filter them, using information provided outside the scope of this annex.

In every case, the Host will receive just one MGT across the POD to Host interface, and the map_ID parameter may be ignored.

version_number: This 5-bit field is the version number of MGT. The version number shall be incremented by 1 modulo 32 when any field in the table_types defined in the loop below or the MGT itself changes.

current_next_indicator: This 1-bit indicator is always set to '1' for the MGT section; the MGT sent is always currently applicable.

section_number: The value of this 8-bit field shall always be 0x00 (this table is only one section long).

last_section_number: The value of this 8-bit field shall always be 0x00.

protocol_version: An 8-bit unsigned integer field whose function shall be to allow, in the future, this table type to carry parameters that may be structured differently than those defined in the current protocol. At present, the only valid value for protocol_version is zero. Non-zero values of protocol_version may only be processed by Hosts designed to accommodate the later versions as they become standardized.

tables_defined: This 16-bit unsigned integer in the range 0 to 65 535 represents the number of tables in the following loop.

table_type: This 16-bit unsigned integer specifies the type of table, based on Table B.28.

Table B.28/J.94 – MGT Table Types

| table_type | Meaning |
| --- | --- |
| 0x0000-0x0001 | [Assigned by ATSC] |
| 0x0002 | Long-form Virtual Channel Table with current_next_indicator=1 |
| 0x0003 | Long-form Virtual Channel Table with current_next_indicator=0 |
| 0x0004 | [Assigned by ATSC] |
| 0x0005-0x000F | [Reserved] |
| 0x0010 | Short-form Virtual Channel Table-VCM Subtype |
| 0x0011 | Short-form Virtual Channel Table-DCM Subtype |
| 0x0012 | Short-form Virtual Channel Table-ICM Subtype |
| 0x0013-0x01F | [Reserved] |
| 0x0020 | Network Information Table-CDS Table Subtype |
| 0x0021 | Network Information Table-MMS Table Subtype |
| 0x0021-0x02F | [Reserved] |
| 0x0030 | Network Text Table-SNS Subtype |
| 0x0031-0x00FF | [Reserved] |
| 0x0100-0x017F | [Assigned by ATSC] |
| 0x0180-0x01FF | [Reserved] |
| 0x0200-0x027F | [Assigned by ATSC] |
| 0x028F-0x0300 | [Reserved] |
| 0x0301-0x03FF | Rating Region Table with rating_region 1-255 |
| 0x0400-0x0FFF | [User private] |
| 0x1000-0x10FF | Aggregate Event Information Table with MGT_tag 0 to 255 |
| 0x1100-0x11FF | Aggregate Extended Text Table with MGT_tag 0 to 255 |
| 0x1200-0xFFFF | [Reserved] |

For table types formatted with the MPEG short-form syntax, the revision_detection_descriptor() shall be used to indicate the section number and version. For example, table_type 0x0020 indicates the Network Information Table, CDS table subtype. One MGT reference to CDS would cover all sections of the delivered CDS.

MGT table types 0x1000 through 0x10FF reference AEIT instances with MGT_tag values 0x00 through 0xFF, respectively. Table types 0x1100 through 0x11FF reference AETT instances with MGT_tag values 0x00 through 0xFF, respectively. A table_type value of 0x1023 in the MGT, for example, refers to the instance of the AEIT with MGT_tag value 0x23.

Note that the choice of value of the MGT_tag is independent of the timeslot number. For example, the MGT_tag value used to deliver AEIT-0 may be zero or any other value up to 255.

table_type_PID: This 13-bit field specifies the PID for the table_type described in the loop.

table_type_version_number: This 5-bit field reflects the version number of the table_type described in the loop. The value of this field shall be the same as the version_number entered in the corresponding fields of tables and table instances. The version number for the next L-VCT (current_next_indicator = 0) shall be one unit more (modulo 32) than the version number for the current L-VCT (current_next_indicator = 1).

number_bytes: This 32-bit unsigned integer field indicates the total number of bytes used for the table_type described in the loop. There may be more than one instance of the indicated table_type.

table_type_descriptors_length: Total length of the descriptors for the table_type described in the loop (in bytes).

descriptors_length: Total length of the MGT descriptor list that follows (in bytes).

descriptor(): The table section may include, at its end, one or more structures of the form tag, length, data. Descriptors are defined in B.7.

CRC_32: This is a 32-bit field that contains the CRC value to ensure a zero output from the registers in the decoder defined in Annex A of ITU-T H.222.0 | ISO/IEC 13818-1 "MPEG-2 Systems" after processing the entire Master Guide Table section.

#### B.6.5.1	Restrictions on PID values

Certain restrictions apply to the PID values specified in the MGT. These restrictions are necessary to ensure the Host can collect EPG data using a minimum number of concurrent flows on the Extended Channel.

•	All AEIT and AETT table sections with common MGT_tag values shall share a common PID.

•	AEIT-0, AETT-0, AEIT-1 and AETT-1 instances shall share a common PID value.

•	AEIT-2, AETT-2, AEIT-3 and AETT-3 instances shall be associated with a second separate PID value.

•	EPG data describing events farther into the future may be associated with one or more PID values; the second PID value may be used for all or some of the AEIT/AETT-4 through AEIT/AETT-N instances (N < 256).

#### B.6.5.2	Restrictions on order of occurrence of table references

For all table references except AEIT and AETT, the order of appearance in the MGT of various table references is not specified or restricted. For AEIT and AETT references, the following restriction applies:

•	The order of appearance of AEIT/AETT references in the MGT shall correspond to increasing time slot assignments.

NOTE  This rule allows a Host to know, before processing the AEIT/AETT data, which table instances correspond to near-term data and which correspond to data farther into the future. This information is useful if the Host has insufficient RAM to hold all data transmitted.

### B.6.6	Long-form Virtual Channel Table

The Long-form Virtual Channel Table is carried in MPEG-2 table sections with table ID 0xC9, and conforms to the syntax and semantics of the MPEG-2 Private Section as described in 2.4.4.10 and 2.4.4.11 of ITU-T H.222.0 | ISO/IEC 13818-1. The Long-form Virtual Channel Table shall be associated on the POD-Host interface with PID value 0x1FFC, the SI_base PID.

The bit stream syntax for the Long-form Virtual Channel Table is shown in Table B.29.

| Table B.29/J.94 – Long-form Virtual Channel Table section format |  |  |  |
| --- | --- | --- | --- |
| Syntax | Bits | Bytes | Format |
| longform_virtual_channel_table_section () { |  |  |  |
| table_id | 8 | 1 | 0xC9 |
| section_syntax_indicator | 1 | 2 | '1' |
| private_indicator | 1 |  | '1' |
| Reserved | 2 |  | '11' |
| section_length | 12 |  | uimsbf |
| map_ID | 16 | 2 | uimsbf |
| Reserved | 2 | 1 | '11' |
| version_number | 5 |  | uimsbf |
| current_next_indicator | 1 |  | bslbf |
| section_number | 8 | 1 | uimsbf |
| last_section_number | 8 | 1 | uimsbf |
| protocol_version | 8 | 1 | uimsbf |
| num_channels_in_section | 8 | 1 | uimsbf |
| For(i=0; i<num_channels_in_section;i++) { |  |  |  |
| short_name | 7*16 | (14) | unicode™BMP |
| reserved | 4 | (3) | '1111' |
| major_channel_number | 10 |  | uimsbf |
| minor_channel_number | 10 |  | uimsbf |
| modulation mode | 8 | (1) | uimsbf |
| carrier_frequency | 32 | (4) | uimsbf |
| channel_TSID | 16 | (2) | uimsbf |
| program_number | 16 | (2) | uimsbf |
| reserved | 2 | (2) | '11' |
| access_controlled | 1 |  | bslbf |
| hidden | 1 |  | bslbf |
| path_select | 1 |  | bslbf |
| out_of_band | 1 |  | bslbf |
| hide_guide | 1 |  | bslbf |
| reserved | 3 |  | '111' |
| service_type | 6 |  | uimsbf |
| source_id | 16 | (2) | uimsbf |
| reserved | 6 | (2) | '111111' |
| descriptors_length | 10 |  | uimsbf |
| for (i=0;i<N;i++) { |  |  |  |
| descriptors() |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table B.29/J.94 – Long-form Virtual Channel Table section format (concluded)

| Syntax | Bits | Bytes | Format |
| --- | --- | --- | --- |
| reserved | 6 | 2 | '111111' |
| additional_descriptors_length | 10 |  | uimsbf |
| For(j=0; j<N;j++) { |  |  |  |
| additional_descriptors() |  | var |  |
| } |  |  |  |
| CRC_32 | 32 | 4 | rpchof |
| } |  |  |  |

table_id: An 8-bit unsigned integer number that indicates the type of table section being defined here. For the longform_virtual_channel_table_section, the table_ID shall be 0xC9.

section_syntax_indicator: The section_syntax_indicator is a one-bit field which shall be set to '1' for the longform_virtual_channel_table_section().

private_indicator: This 1-bit field shall be set to '1'.

section_length: This is a twelve-bit field that specifies the number of bytes of the section, starting immediately following the section_length field, and including the CRC. The value in this field shall not exceed 4093.

map_ID: A 16-bit identifier for this Long-form Virtual Channel Table. In some applications, the POD module may receive multiple Long-form Virtual Channel Table sections corresponding to distinct channel maps. In this case, the POD may use the map_ID to distinguish them, using information provided outside the scope of this annex. In every case, the Host will receive just one L-VCT across the POD to Host interface, and the map_ID parameter may be ignored.

version_number: This 5-bit field is the version number of the Long-form Virtual Channel Table. For the current L-VCT (current_next_indicator = 1), the version number shall be incremented by 1 whenever the value of the current L-VCT changes. Upon reaching the value 31, it wraps around to 0. For the next L-VCT (current_next_indicator = 0), the version number shall be one unit more than that of the current L-VCT (also in modulo 32 arithmetic). In any case, the value of the version_number shall be identical to that of the corresponding entries in the MGT.

current_next_indicator: A 1-bit indicator, which when set to '1' indicates that the Long-form Virtual Channel Table sent is currently applicable. When the bit is set to '0', it indicates that the table sent is not yet applicable and shall be the next table to become valid.

section_number: This 8-bit field gives the number of this section. The section_number of the first section in the Long-form Virtual Channel Table shall be 0x00. It shall be incremented by one with each additional section in the Long-form Virtual Channel Table.

last_section_number: This 8-bit field specifies the number of the last section (that is, the section with the highest section_number) of the complete Long-form Virtual Channel Table.

protocol_version: An 8-bit unsigned integer field whose function is to allow, in the future, this table type to carry parameters that may be structured differently than those defined in the current protocol. At present, the only valid value for protocol_version is zero. Non-zero values of protocol_version may only be processed by Hosts designed to accommodate the later versions as they become standardized.

num_channels_in_section: This 8-bit field specifies the number of virtual channels in the L-VCT section. The number is limited by the section length.

short_name: The name of the virtual channel, represented as a sequence of one to seven 16-bit character codes coded in accordance with the Basic Multilingual Plane (BMP) of Unicode™, as specified in ISO/IEC 10646-1. If the name of the virtual channel is shorter than seven Unicode™

characters, one or more instances of the null character value 0x0000 shall be used to pad the string to its fixed 14-byte length.

major_channel_number, minor_channel_number: These two 10-bit fields represent either a two-part or a one-part virtual channel number associated with the virtual channel being defined in this iteration of the "for" loop. One-part numbers range from 0 to 16 383. Two-part numbers consist of a major and a minor number part; the range of each is 0 to 999. The one- or two-part number acts as the user's reference number for the virtual channel. Some channels may be represented with a one-part number while others in the VCT are represented with two-part numbers.

The six MSBs of the major_channel_number field, when all 1, indicate that a one-part number is being specified. The value of the one-part number is given, in C syntax, by:

one_part_number = (major_channel_number & 0x00F) << 10 + minor_channel_number

When the six MSBs of the major_channel_number field are not all 1, and the 10-bit major_channel_number field is less than 1000, two fields specify a two-part channel number. The value of the two-part number is given by major_channel_number and minor_channel_number.

Table B.30 summarizes the coding of the major_channel_number and minor_channel_number fields.

Table B.30/J.94 – Major and minor channel number field coding

|  | 20-bit major/minor field (10-bit major + 10-bit minor) |  | User channel number |
| --- | --- | --- | --- |
| Two-part channel numbers (1000 major numbers, each with 1000 minor numbers) | Major Number (10 bits) | Minor Number (10 bits) | Two-part user channel number |
|  | 000d | 000d | 0-0 |
|  | 000d | 001d | 0-1 |
|  | … | … | … |
|  | 000d | 999d | 0-999 |
|  | 001d | 000d | 1-0 |
|  | … | … | … |
|  | 999d | 999d | 999-999 |
| [Reserved] | 000d to 999d | 1000d-1023d | N/A |
|  | 1000-1007d | All values | N/A |
| One-part channel numbers (16 383 linear space numbers) | 6-bit flag (set = 111111b) | One-Part Number (14 bits) | One-part user channel number |
|  | Set | 0d | 0 |
|  | Set | 1d | 1 |
|  | Set | … | … |
|  | Set | 16383d | 16383 |

modulation_mode: An 8-bit unsigned integer number that indicates the modulation mode for the transmitted carrier associated with this virtual channel. Values of modulation_mode are defined by this annex in Table B.31. For digital signals, the standard values for modulation mode (values below 0x80) indicate transport framing structure, channel coding, interleaving, channel modulation, forward error correction, symbol rate, and other transmission-related parameters, by means of a reference to an appropriate standard. Values of modulation_mode 0x80 and above are outside the scope of SCTE. These may be used to specify non-standard modulation modes in private systems. A value of 0x80 for modulation_mode indicates that modulation parameters are specified in a private descriptor. The modulation_mode field shall be disregarded for inactive channels.

Table B.31/J.94 – Modulation modes

| Modulation_mode | Meaning |
| --- | --- |
| 0x00 | [Reserved] |
| 0x01 | analogue  The virtual channel is modulated using standard analogue methods for analogue television. |
| 0x02 | SCTE_mode_1  The virtual channel has a symbol rate of 5.057 Msymb/s, transmitted in accordance with Digital Transmission Standard for Cable Television, Ref. SCTE DVS 031 (Mode 1). Typically, mode 1 will be used for 64-QAM. |
| 0x03 | SCTE_mode_2  The virtual channel has a symbol rate of 5.361 Msymb/s, transmitted in accordance with Digital Transmission Standard for Cable Television, Ref. SCTE DVS 031 (Mode 2). Typically, mode 2 will be used for 256-QAM. |
| 0x04 | ATSC (8 VSB)  The virtual channel uses the 8-VSB modulation method conforming to the ATSC Digital Television Standard, ATSC Standard A/53 (1995). |
| 0x05 | ATSC (16 VSB)  The virtual channel uses the 16-VSB modulation method conforming to the ATSC Digital Television Standard, ATSC Standard A/53 (1995). |
| 0x06-0x7F | [Reserved for future use] |
| 0x80 | Modulation parameters are defined by a private descriptor |
| 0x81-0xFF | [User Private] |

carrier_frequency: A 32-bit unsigned integer that represents the carrier frequency associated with the analogue or digital transmission associated with this virtual channel, in Hz. For QAM-modulated signals, the given carrier_frequency represents the location of the digitally modulated carrier; for VSB-modulated signals, the given carrier_frequency represents the location of the pilot tone; for analogue signals, it represents the frequency of the picture carrier. The carrier_frequency field shall be disregarded for inactive channels.

channel_TSID: A 16-bit unsigned integer field, in the range 0x0000 to 0xFFFF, that represents the MPEG-2 Transport Stream ID associated with the Transport Stream carrying the MPEG-2 program referenced by this virtual channel. For inactive channels, channel_TSID represents the ID of the Transport Stream that will carry the service when it becomes active. The Host may use the channel_TSID to verify that a TS acquired at the referenced carrier frequency is actually the desired multiplex. Analogue signals may have a TSID provided that it is different from any DTV Transport Stream identifier; that is, it shall be truly unique if present. A value of 0xFFFF for channel_TSID shall be specified for analogue channels that do not have a valid TSID.

program_number: A 16-bit unsigned integer number that associates the virtual channel being defined here with the MPEG-2 Program Association and TS Program Map tables. For virtual channels representing analogue services, a value of 0xFFFF shall be specified for program_number. For inactive channels (those not currently present in the Transport Stream), program_number shall be set to zero. This number shall not be interpreted as pointing to a Program Map Table entry.

access_controlled: A 1-bit Boolean flag, when set, indicates that events associated with this virtual channel may be access controlled. When the flag is set to 0, event access is not restricted.

hidden: A 1-bit Boolean flag that indicates, when set, that the virtual channel is not accessed by the user by direct entry of the virtual channel number. Hidden virtual channels are skipped when the user is channel surfing, and appear as if undefined, if accessed by direct channel entry. Typical applications for hidden channels are test signals and NVOD services. Whether a hidden channel and its event may appear in EPG displays depends on the state of the hide_guide bit.

path_select: A 1-bit field that associates the virtual channel with a transmission path. Two paths are available as defined in Table B.32. For the cable transmission medium, path_select identifies which of two physical input cables carries the Transport Stream associated with this virtual channel.

Table B.32/J.94 – Path Select

| path_select | Meaning |
| --- | --- |
| 0 | path 1 |
| 1 | path 2 |

out_of_band: A Boolean flag that indicates, when set, that the virtual channel defined in this iteration of the "for" loop is carried on the cable on the Extended Channel interface carrying the tables defined in this protocol. When clear, the virtual channel is carried within a standard tuned multiplex at that frequency.

NOTE  A virtual channel carried on the out-of-band channel may be acquired by opening a flow between Host and POD to capture the PAT on PID 0. Processing the PAT will determine the PID associated with that service's PMT. Then, a flow can be opened to capture and process the PMT to determine the PIDs associated with elementary stream components of the service. Finally, a flow associated with the service's PID can be opened to capture service-related data.

hide_guide: A Boolean flag that indicates, when set to 0 for a hidden channel, that the virtual channel and its events may appear in EPG displays. This bit shall be ignored for channels which do not have the hidden bit set, so that non-hidden channels and their events may always be included in EPG displays regardless of the state of the hide_guide bit. Typical applications for hidden channels with the hide_guide bit set to 1 are test signals and services accessible through application-level pointers.

An inactive channel is defined as a channel that has program guide data available, but the channel is not currently on the air. Inactive channels are represented as hidden channels with the hide_guide bit set to 0. The Transport Stream shall not carry a Program Map Table representing an inactive channel.

service_type: A 6-bit enumerated type field that identifies the type of service carried in this virtual channel, based on Table B.33.

Table B.33/J.94 – Service Types

| service_type | Meaning |
| --- | --- |
| 0x00 | [Reserved] |
| 0x01 | analogue_television  The virtual channel carries analogue television programming |
| 0x02 | ATSC_digital_television  The virtual channel carries television programming (audio, video and data) conforming to the ATSC Digital Television Standard |
| 0x03 | ATSC_audio_only  The virtual channel conforms to the ATSC Digital Television Standard, and has one or more standard audio and data components but no video. |
| 0x04 | ATSC_data_broadcast_service  Conforming to the ATSC data broadcast standard under development by T3/S13. |
| 0x05-0x3F | [Reserved for future ATSC use] |

source_id: A 16-bit unsigned integer number that identifies the programming source associated with the virtual channel. In this context, a source is one specific source of video, text, data, or audio programming. Source ID value zero is reserved to indicate that the programming source is not identified. Source ID values in the range 0x0001 to 0x0FFF shall be unique within the Transport Stream that carries the VCT, while values 0x1000 to 0xFFFF shall be unique at the regional level. Values for source_IDs 0x1000 and above shall be issued and administered by a Registration Authority designated by the ATSC.

descriptors_length: Total length (in bytes) of the descriptors for this virtual channel that follows.

additional_descriptors_length: Total length (in bytes) of the VCT descriptor list that follows.

CRC_32: This is a 32-bit field that contains the CRC value that ensures a zero output from the registers in the decoder defined in Annex A of ITU-T H.222.0 | ISO/IEC 13818-1 "MPEG-2 Systems" after processing the entire Long-form Virtual Channel Table section.

For inactive channels, the short_name, major_channel_number, and minor_channel_number fields reflect the name and channel number of the inactive channel, and may be used in construction of the program guide. The source_ID for inactive channels is used, as it is for active channels, to link the virtual channel to the program guide data. The service_type field and attribute flags reflect the characteristics of the channel that will be valid when it is active.

### B.6.7	Rating Region Table (RRT)

The Rating Region Table carries rating information for multiple geographical regions. The RRT shall be associated on the POD-Host interface with PID value 0x1FFC, the SI_base PID.

Transmission of the RRT is required whenever any Transport Stream carries a service that includes a content_advisory_descriptor() in one of its Program Map Tables, or if a content_advisory_descriptor() appears in any transmitted AEIT. An instance of the RRT for each region referenced in any content_advisory_descriptor() shall be transmitted.

Each RRT instance, identified by rating_region (the eight least significant bits of table_id_extension), conveys the rating system information for one specific region. The size of each RRT instance shall not be more than 1024 bytes (including section header and trailer), and it shall be carried by only one MPEG-2 private section.

Table B.34 describes the Rating Region Table.

| Table B.34/J.94 – Rating Region Table section format |  |  |  |
| --- | --- | --- | --- |
|  | Bits | Bytes | Format |
| rating_region_table_section () { |  |  |  |
| table_ID | 8 | 1 | 0xCA |
| section_syntax_indicator | 1 | 2 | '1' |
| private_indicator | 1 |  | '1' |
| Reserved | 2 |  | '11' |
| section_length | 12 |  | uimsbf |
| table_ID_extension{ |  |  |  |
| Reserved | 8 | 1 | 0xFF |
| rating_region | 8 | 1 | uimsbf |
| } |  |  |  |
| Reserved | 2 | 1 | '11' |
| version_number | 5 |  | uimsbf |
| current_next_indicator | 1 |  | '1' |
| section_number | 8 | 1 | uimsbf |
| last_section_number | 8 | 1 | uimsbf |

Table B.34/J.94 – Rating Region Table section format (concluded)

|  | Bits | Bytes | Format |
| --- | --- | --- | --- |
| protocol_version | 8 | 1 | uimsbf |
| rating_region_name_length | 8 | 1 | uimsbf |
| rating_region_name_text() | var |  |  |
| dimensions_defined | 8 | 1 | uimsbf |
| for(i=0; i<dimensions_defined;i++) { |  |  |  |
| dimension_name_length | 8 | 1 | uimsbf |
| dimension_name_text() | var |  |  |
| Reserved | 3 | 1 | '111' |
| graduated_scale | 1 |  | bslbf |
| values_defined | 4 |  | uimsbf |
| for (j=0;j<values_defined;j ++) { |  |  |  |
| abbrev_rating_value_length | 8 | 1 | uimsbf |
| abbrev_rating_value_ text() | var |  |  |
| rating_value_length | 8 | 1 | uimsbf |
| rating_value_ text() | var |  |  |
| } |  |  |  |
| } |  |  |  |
| Reserved | 6 | 2 | '111111' |
| descriptors_length | 10 |  | uimsbf |
| for (i=0;i<N;i++) { |  |  |  |
| descriptors() | var |  |  |
| } |  |  |  |
| CRC_32 | 32 | 4 | rpchof |
| } |  |  |  |

table_ID: The table_ID of the Rating Region Table (RRT) shall be 0xCA.

section_syntax_indicator: This 1-bit field shall be set to '1'. It denotes that the section follows the generic section syntax beyond the section length field.

private_indicator: This 1-bit field shall be set to '1'.

section_length: 12-bit field specifying the number of remaining bytes in this section immediately following the section_length field up to the end of the section. The value of the section_length shall be no larger than 1021.

rating_region: An 8-bit unsigned integer number that defines the rating region to be associated with the text in this rating_region_table_section(). The value of this field is the identifier of this rating region, and thus this field may be used by the other tables (e.g. MGT) for referring to a specific rating region table. Values of rating_region are defined in Table B.35.

Table B.35/J.94 – Rating Regions

| rating_region | Rating Region Name |
| --- | --- |
| 0x00 | Forbidden |
| 0x01 | US (50 states + possessions) |
| 0x02-0xFF | [Reserved] |

version_number: This 5-bit field is the version number of the Rating Region Table identified by combination of the fields table_ID and table_ID_extension. The version number shall be incremented by 1 modulo 32 when any field in this instance of the Rating Region Table changes. The value of this field shall be the same as that of the corresponding entry in MGT.

current_next_indicator: This 1-bit indicator is always set to '1'.

section_number: The value of this 8-bit field shall always be 0x00.

last_section_number: The value of this 8-bit field shall always be 0x00.

protocol_version: The value of this 8-bit field shall always be 0x00.

rating_region_name_length: An 8-bit unsigned integer number that defines the total length (in bytes) of the rating_region_name_text() field to follow.

rating_region_name_text(): A data structure containing a Multiple String Structure which represents the rating region name, e.g. "U.S. (50 states + possessions)", associated with the value given by rating_region. The rating_region_name_text() shall be formatted according to the Multiple String Structure (see B.8.2). The display string for the rating region name shall be limited to 32 characters or less.

dimensions_defined: This 8-bit field (1-255) specifies the number of dimensions defined in this rating_region_table_section().

dimension_name_length: An 8-bit unsigned integer number that defines the total length in bytes of the dimension_name_text() field to follow.

dimension_name_text(): A data structure containing a Multiple String Structure which represents the dimension name being described in the loop. One dimension in the U.S. rating region, for example, is used to describe the MPAA list. The dimension name for such a case may be defined as "MPAA". The dimension_name_text() shall be formatted according to the Multiple String Structure (see B.8.2). The dimension name display string shall be limited to 20 characters or less.

graduated_scale: This 1-bit flag indicates whether or not the rating values in this dimension represent a graduated scale, i.e. higher rating values represent increasing levels of rated content within the dimension. Value 1 means yes, while value 0 means no.

values_defined: This 4-bit field (1-15) specifies the number of values defined for this particular dimension.

abbrev_rating_value_length: An 8-bit unsigned integer number that defines the total length (in bytes) of the abbrev_rating_value_text() field to follow.

abbrev_rating_value_text(): A data structure containing a Multiple String Structure which represents the abbreviated name for one particular rating value. The abbreviated name for rating value 0 shall be set to a null string, i.e. "". The abbrev_rating_value_text() shall be formatted according to the Multiple String Structure (see B.8.2). The abbreviated value display string shall be limited to 8 characters or less.

rating_value_length: An 8-bit unsigned integer number that defines the total length (in bytes) of the rating_value_text() field to follow.

rating_value_text(): A data structure containing a Multiple String Structure which represents the full name for one particular rating value. The full name for rating value 0 shall be set to a null string, i.e. "". The rating_value_text() shall be formatted according to the Multiple String Structure (see B.8.2). The rating value display string shall be limited to 150 characters or less.

descriptors_length: Length (in bytes) of all of the descriptors that follow this field.

CRC_32: This is a 32-bit field that contains the CRC value that ensures a zero output from the registers in the decoder defined in Annex A of ITU-T H.222.0 | ISO/IEC 13818-1 "MPEG-2 Systems" after processing the entire Rating Region Table section.

### B.6.8	Aggregate Event Information Tables (AEIT)

The Aggregate Event Information Table delivers event title and schedule information that may be used to support an Electronic Program Guide application. The transmission format allows instances of table sections for different time periods to be associated with common PID values. For use on the Extended Channel (out-of-band), reduction of the total number of PID values in use for SI data is important, because the POD module can typically support only a small number of concurrent data flows (each associated with one PID value).

Each AEIT instance describes event data for one three-hour time period. The start time for any AEIT is constrained to be one of the following eight UTC times: 00:00 (midnight), 03:00, 06:00, 09:00, 12:00 (noon), 15:00, 18:00, and 21:00.

The notation AEIT-n refers to the AEIT corresponding to timeslot n. Value 0 for n indicates the current timeslot, value 1 the next timeslot, etc. The same notational methods apply to AETT.

Except for AEIT-0, each AEIT instance shall include event data only for those events actually starting within the covered time period. AEIT-0 shall also include event data for all events starting in a prior timeslot but continuing into the current timeslot. In addition, if the VCT entry for a particular source ID includes a time_shifted_service_descriptor(), AEIT-0 shall describe event data for active events on any channels referenced through the time_shifted_service_descriptor().

ETMs for events described in AEIT-0 shall be provided in AETT-0 on the PID associated with AEIT-0 until they are no longer referenced by AEIT-0.

Table B.36 defines the syntax of the Aggregate Event Information Table.

| Table B.36/J.94 – Aggregate Event Information Table format |  |  |  |
| --- | --- | --- | --- |
| Syntax | Bits | Bytes | Format |
| aggregate_event_information_table_section () { |  |  |  |
| table_ID | 8 | 1 | 0xD6 |
| section_syntax_indicator | 1 | 2 | '1' |
| private_indicator | 1 |  | '1' |
| Reserved | 2 |  | '11' |
| section_length | 12 |  | uimsbf |
| AEIT_subtype | 8 | 1 | uimsbf |
| MGT_tag | 8 | 1 | uimsbf |
| Reserved | 2 |  | '11' |
| version_number | 5 |  | uimsbf |
| current_next_indicator | 1 |  | '1' |
| section_number | 8 | 1 | uimsbf |
| last_section_number | 8 | 1 | uimsbf |
| if (AEIT_subtype == 0) { |  |  |  |
| num_sources_in_section | 8 | 1 | uimsbf |
| for (j = 0; j< num_sources_in_section;j++) { |  |  |  |
| source_ID | 16 | (2) | uimsbf |
| Num_events | 8 | (1) | uimsbf |
| for (j = 0; j< num_events;j++) { |  |  |  |
| reserved | 2 | ((2)) | '11' |
| event_ID | 14 |  | uimsbf |
| start_time | 32 | ((4)) | uimsbf |

Table B.36/J.94 – Aggregate Event Information Table format (concluded)

| Syntax | Bits | Bytes | Format |
| --- | --- | --- | --- |
| reserved | 2 | ((3)) | '11' |
| ETM_present | 2 |  | bslbf |
| duration | 20 |  | uimsbf |
| title_length | 8 | ((1)) | uimsbf |
| title_text() | var |  |  |
| reserved | 4 | ((2)) | '1111' |
| descriptors_length | 12 |  |  |
| for (i=0;i<N;i++) { |  |  |  |
| descriptor() |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| else |  |  |  |
| reserved | n*8 | n |  |
| CRC_32 | 32 | 4 | rpchof |
| } |  |  |  |

table_ID: The table_ID of the Aggregate Event Information Table shall be 0xD6.

section_syntax_indicator: This 1-bit field shall be set to '1'. It denotes that the section follows the generic section syntax beyond the section length field.

private_indicator: This 1-bit field shall be set to '1'.

section_length: 12-bit field specifying the number of remaining bytes in this section immediately following the section_length field up to the end of the section, including the CRC_32 field. The value of this field shall not exceed 4093.

AEIT_subtype: This 8-bit field identifies the subtype of the AEIT. In the current protocol, only table subtype value 0x00 is defined. Host devices shall discard instances of the aggregate_event_information_table_section() in which an unknown AEIT_subtype is specified (currently, any value other than zero).

MGT_tag: An 8-bit field that ties this AEIT instance to the corresponding table_type in the MGT and to an AETT instance with the same value. The MGT_tag value for an AEIT instance for a given timeslot shall be one higher (modulo 256) than the instance for the preceding time period.

version_number: This 5-bit field is the version number of the AEIT instance. An instance is identified by the MGT_tag. The version number shall be incremented by 1 modulo 32 when any field in the AEIT instance changes. The value of this field shall be identical to that of the corresponding entry in the MGT.

current_next_indicator: This 1-bit indicator is always set to '1' for AEIT sections; the AEIT sent is always currently applicable.

section_number: This 8-bit field gives the number of this section.

last_section_number: This 8-bit field specifies the number of the last section.

num_sources_in_section: This 8-bit field gives the number of iterations of the "for" loop describing program schedule data.

source_ID: This 16-bit field specifies the source_ID of the virtual channel carrying the events described in this section.

num_events: Indicates the number of events to follow associated with the program source identified by source_ID. Value 0 indicates no events are defined for this source for the time period covered by the AEIT instance.

event_ID: This 14-bit field specifies the identification number of the event described. This number serves as a part of the event ETM_ID (identifier for event Extended Text Message). An assigned event_ID shall be unique at least within the scope of the instance of the AEIT in which it appears. Accordingly, as an example, the event associated with event_ID 0x0123 in AEIT-m shall be considered to be an event distinct from event_ID 0x0123 in AEIT-n, when m is not equal to n.

start_time: A 32-bit unsigned integer quantity representing the start time of this event as the number of seconds since 0000 Hours UTC, January 6th, 1980. If the GPS_UTC_offset delivered in the System Timetable is zero, start_time includes the correction for leap seconds. Otherwise, start_time can be converted to UTC by subtracting the GPS_UTC_offset.

ETM_present: This 2-bit field indicates the existence of an Extended Text Message (ETM) based on Table B.37.

Table B.37/J.94 – ETM_present

| ETM_present | Meaning |
| --- | --- |
| 0x00 | No ETM |
| 0x01 | ETM present on this out-of-band Extended Channel |
| 0x02-0x03 | [Reserved for future use] |

duration: Duration of this event in seconds.

title_length: This field specifies the length (in bytes) of the title_text(). Value 0 means that no title exists for this event.

title_text(): The event title in the format of a Multiple String Structure. title_text() shall be formatted according to the Multiple String Structure (see B.8.2).

descriptors_length: Total length (in bytes) of the event descriptor list that follows.

CRC_32: This is a 32-bit field that contains the CRC value that ensures a zero output from the registers in the decoder defined in Annex A of ITU-T H.222.0 | ISO/IEC 13818-1 "MPEG-2 Systems" after processing the entire Aggregate Event Information Table section.

### B.6.9	Aggregate Extended Text Tables (AETT)

The Aggregate Extended Text Table contains Extended Text Messages (ETM), which are used to provide detailed descriptions of events. An ETM is a multiple string data structure. Thus, it may represent a description in several different languages (each string corresponding to one language). If necessary, the description may be truncated to fit the allocated display space.

The transmission format of the AETT and its affiliated AEIT allows instances of AEIT/AETT table sections for different time slots to be associated with common PID values.

AETT-n shall be associated with the same PID value as AEIT-n for a given value of n.

The Aggregate Extended Text Table is carried in an MPEG-2 private section with table_ID 0xD7. An instance of the AETT includes one or more ETMs. Each description is distinguished by its unique 32-bit ETM_ID.

Table B.38 defines the syntax of the Aggregate Extended Text Table.

Table B.38/J.94 – Aggregate Extended Text Table format

| Syntax | Bits | Bytes | Format |
| --- | --- | --- | --- |
| aggregate_extended_text_table_section () { |  |  |  |
| table_ID | 8 | 1 | 0xD7 |
| section_syntax_indicator | 1 | 2 | '1' |
| private_indicator | 1 |  | '1' |
| Reserved | 2 |  | '11' |
| section_length | 12 |  | uimsbf |
| AETT_subtype | 8 | 1 | uimsbf |
| MGT_tag | 8 | 1 | uimsbf |
| Reserved | 2 | 1 | '11' |
| version_number | 5 |  | uimsbf |
| current_next_indicator | 1 |  | '1' |
| section_number | 8 | 1 | uimsbf |
| last_section_number | 8 | 1 | uimsbf |
| if (AETT_subtype == 0) { |  |  |  |
| num_blocks_in_section | 8 | 1 | uimsbf |
| for (j = 0; j< num_blocks_in_section;j++) { |  |  |  |
| ETM_ID | 32 | (4) | uimsbf |
| reserved | 4 | (2) | '1111' |
| extended_text_length | 12 |  | uimsbf |
| extended_text_message() | var |  |  |
| } |  |  |  |
| } |  |  |  |
| Else |  |  |  |
| reserved | n*8 | n |  |
| CRC_32 | 32 | 4 | rpchof |
| } |  |  |  |

table_ID: The table_ID of the Aggregate Extended Text Table shall be 0xD7.

section_syntax_indicator: This 1-bit field shall be set to '1'. It denotes that the section follows the generic section syntax beyond the section length field.

private_indicator: This 1-bit field shall be set to '1'.

section_length: 12-bit field specifying the number of remaining bytes in the section immediately following the section_length field up to the end of the section. The value of the section_length shall be no larger than 4093.

AETT_subtype: This 8-bit field identifies the subtype of the AETT. In the current protocol, only table subtype value 0x00 is defined. Host devices shall discard instances of the aggregate_extended_text_table_section() in which an unknown AETT_subtype is specified (currently, any value other than zero).

MGT_tag: An 8-bit field that ties this AETT instance to the corresponding table_type in the MGT and to an AEIT instance with the same value. The MGT_tag value for an AETT instance for a given time period shall be one higher (modulo 256) than the instance for the preceding time period.

version_number: This 5-bit field is the version number of the AETT instance. An instance is uniquely identified by its MGT_tag. The version number shall be incremented by 1 modulo 32 when any field in the AETT instance changes. The value of this field shall be identical to that of the corresponding entry in the MGT.

current_next_indicator: This 1-bit indicator is always set to '1' for AETT sections; the AETT sent is always currently applicable.

section_number: This 8-bit field gives the number of this section.

last_section_number: This 8-bit field specifies the number of the last section.

num_blocks_in_section: This 8-bit field gives the number of iterations of the "for" loop describing ETM data.

ETM_ID: Unique 32-bit identifier of this Extended Text Message. This identifier is assigned by the rule shown in Table B.39.

Table B.39/J.94 – ETM ID

|  | MSB |  | LSB |
| --- | --- | --- | --- |
| Bit | 31 16 | 15 2 | 1 0 |
| event ETM_ID | source_ID | event_ID | 1 0 |

extended_text_length: A 12-bit unsigned integer number that represents the length, in bytes, of the extended_text_message() field directly following.

extended_text_message(): The extended text message in the format of a Multiple String Structure (see B.8.2).

CRC_32: This is a 32-bit field that contains the CRC value that ensures a zero output from the registers in the decoder defined in Annex A of ITU-T H.222.0 | ISO/IEC 13818-1 "MPEG-2 Systems" after processing the entire Transport Stream AETT section.

## B.7	Descriptors

This clause defines descriptors applicable for use with various table sections defined in this annex.

### B.7.1	Descriptor usage

Table B.40 lists all descriptors, their tag numbers and associated table sections applicable to out-of-band SI transport. Asterisks mark the tables where the descriptors may appear. The range of descriptor tags defined or reserved by MPEG-2 includes those with tag values 0x3F or below, plus 0xFF.

| Table B.40/J.94 – Descriptor usage |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Descriptor name | Tag | Table section |  |  |  |  |  |  |  |  |
|  |  | PMT | NIT | NTT | S-VCT | STT | MGT | L-VCT | RRT | AEIT |
| Stuffing descriptor | 0x80 | * | * | * | * | * | * | * | * | * |
| AC-3 audio descriptor | 0x81 | * |  |  |  |  |  |  |  | * |
| Caption service descriptor | 0x86 | * |  |  |  |  |  |  |  | * |
| Content advisory descriptor | 0x87 | * |  |  |  |  |  |  |  | * |
| Revision detection descriptor | 0x93 |  | * | * | * |  |  |  |  |  |
| Two part channel no. descriptor | 0x94 |  |  |  | * |  |  |  |  |  |
| Channel properties descriptor | 0x95 |  |  |  | * |  |  |  |  |  |

Table B.40/J.94 – Descriptor usage (concluded)

| Descriptor name | Tag | Table section |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PMT | NIT | NTT | S-VCT | STT | MGT | L-VCT | RRT | AEIT |
| Daylight savings time descriptor | 0x96 |  |  |  |  | * |  |  |  |  |
| Extended channel name descriptor | 0xA0 |  |  |  |  |  |  | * |  |  |
| Time shifted service descriptor | 0xA2 |  |  |  |  |  |  | * |  |  |
| Component name descriptor | 0xA3 | * |  |  |  |  |  |  |  |  |
| User private descriptors | 0xC0-0xFF |  | * | * | * | * | * | * | * | * |

### B.7.2	Stuffing descriptor

For certain applications it is necessary to define a block of N bytes as a placeholder. The N bytes themselves are not to be processed or interpreted. The stuffing_descriptor() is specified for this purpose. The stuffing_descriptor() is simply a descriptor type for which the contents, as indicated by the descriptor_length field, are to be disregarded. The tag type for the stuffing descriptor is 0x80. The stuffing_descriptor() may appear where descriptors are allowed in any table defined in this annex.

### B.7.3	AC-3 audio descriptor

The AC-3 audio descriptor, as defined in ATSC Standard A/52 (1995), and constrained in Annex B of ATSC Standard A/53 (1995), may be used in the PMT and/or in AEITs.

### B.7.4	Caption service descriptor

The caption service descriptor provides closed captioning information, such as closed captioning type and language code for events with closed captioning service. This descriptor shall not appear on events with no closed captioning service.

The bit stream syntax for the Caption Service Descriptor is shown in Table B.41.

| Table B.41/J.94 – Caption Service Descriptor format |  |  |  |
| --- | --- | --- | --- |
| Syntax | Bits | Bytes | Format |
| caption_service_descriptor() { |  |  |  |
| descriptor_tag | 8 | 1 | 0x86 |
| descriptor_length | 8 | 1 | uimsbf |
| Reserved | 3 | 1 | '111' |
| number_of_services | 5 |  | uimsbf |
| for (i=0;i<number_of_services;i++) { |  |  |  |
| Language | 8*3 | (3) | uimsbf |
| cc_type | 1 | (1) | bslbf |
| Reserved | 1 |  | '1' |
| if (cc_type==line21) { |  |  |  |
| reserved | 5 |  | '11111' |
| line21_field | 1 |  | bslbf |
| } |  |  |  |

Table B.41/J.94 – Caption Service Descriptor format (concluded)

| Syntax | Bits | Bytes | Format |
| --- | --- | --- | --- |
| Else |  |  |  |
| caption_service_number | 6 |  | uimsbf |
| easy_reader | 1 | (2) | bslbf |
| wide_aspect_ratio | 1 |  | bslbf |
| Reserved | 14 |  | '11111111111111' |
| } |  |  |  |
| } |  |  |  |

descriptor_tag: An 8-bit field that identifies the type of descriptor. For the caption_service_descriptor() the value is 0x86.

descriptor_length: An 8-bit count of the number of bytes following the descriptor_length itself.

number_of_services: An unsigned 5-bit integer in the range 1 to 16 that indicates the number of closed caption services present in the associated video service. Note that if the video service does not carry television closed captioning, the caption_service_descriptor() shall not be present either in the Program Map Table or in the Aggregate Event Information Table.

Each iteration of the "for" loop defines one closed caption service present as a sub-stream within the 9600 bit/s closed captioning stream. Each iteration provides the sub-stream's language, attributes, and (for advanced captions) the associated Service Number reference. Refer to EIA-708 Specification for Advanced Television Closed Captioning (ATVCC), for a description of the use of the Service Number field within the syntax of the closed caption stream.

language: A 3-byte language code per ISO 639-2/B defining the language associated with one closed caption service. The ISO_639_language_code field contains a three-character code as specified by ISO 639-2/B. Each character is coded into 8 bits according to ISO 8859-1 (ISO Latin-1) and inserted in order into the 24-bit field.

cc_type: A flag that indicates, when set, that an advanced television closed caption service is present in accordance with EIA-708 Specification for Advanced Television Closed Captioning (ATVCC). When the flag is clear, a line-21 closed caption service is present. For line 21 closed captions, the line21_field indicates whether the service is carried in the even or odd field.

line21_field: A flag that indicates, when set, that the line 21 closed caption service is associated with the field 2 of the NTSC waveform. When the flag is clear, the line-21 closed caption service is associated with field 1 of the NTSC waveform. The line21_field flag is defined only if the cc_type flag indicates line-21 closed caption service.

caption_service_number: A 6-bit unsigned integer value in the range zero to 63 that identifies the Service Number within the closed captioning stream that is associated with the language and attributes defined in this iteration of the "for" loop. See EIA-708 Specification for Advanced Television Closed Captioning (ATVCC) for a description of the use of the Service Number. The caption_service_number field is defined only if the cc_type flag indicates closed captioning in accordance with EIA-708 Specification for Advanced Television Closed Captioning (ATVCC).

easy_reader: A Boolean flag which indicates, when set, that the closed caption service contains text tailored to the needs of beginning readers. Refer to EIA-708 Specification for Advanced Television Closed Captioning (ATVCC), for a description of "easy reader" television closed captioning services. When the flag is clear, the closed caption service is not so tailored.

wide_aspect_ratio: A Boolean flag which indicates, when set, that the closed caption service is formatted for displays with 16:9 aspect ratio. When the flag is clear, the closed caption service is formatted for 4:3 display, but may be optionally displayed centered within a 16:9 display.

### B.7.5	Content advisory descriptor

The content_advisory_descriptor() is used to indicate, for a given event, ratings for any or all of the rating dimensions defined in the RRT (Rating Region Table). Ratings may be given for any or all of the defined regions, up to a maximum of 8 regions per event. An event without a content_advisory_descriptor() indicates that the rating value for any rating dimension defined in any rating region is zero. The absence of ratings for a specific dimension is completely equivalent to having a zero-valued rating for such a dimension. The absence of ratings for a specific region implies the absence of ratings for all of the dimensions in the region. The absence of a content_advisory_descriptor() for a specific event implies the absence of ratings for all of the regions for the event. The bit stream syntax for the content_advisory_descriptor() is shown in Table B.42.

Table B.42/J.94 – Content Advisory Descriptor format

| Syntax | Bits | Bytes | Format |
| --- | --- | --- | --- |
| content_advisory_descriptor() { |  |  |  |
| descriptor_tag | 8 | 1 | 0x87 |
| descriptor_length | 8 | 1 | uimsbf |
| Reserved | 2 | 1 | '11' |
| rating_region_count | 6 |  |  |
| for (i=0; i<rating_region_count; i++) { |  |  |  |
| rating_region | 8 | 1 | uimsbf |
| rated_dimensions | 8 | 1 | uimsbf |
| for (j=0;j<rated_dimensions;j++) { |  |  |  |
| rating_dimension_j | 8 | 1 | uimsbf |
| reserved | 4 | 1 | '1111' |
| rating_value | 4 |  | uimsbf |
| } |  |  |  |
| rating_description_length | 8 | 1 | uimsbf |
| rating_description_text() | var |  |  |
| } |  |  |  |
| } |  |  |  |

descriptor_tag: This 8-bit unsigned integer shall have the value 0x87, identifying this descriptor as content_advisory_descriptor.

descriptor_length: This 8-bit unsigned integer specifies the length (in bytes) immediately following this field up to the end of this descriptor.

rating_region_count: A 6-bit unsigned integer value in the range 1 to 8 that indicates the number of rating region specifications to follow.

rating_region: An unsigned 8-bit integer that specifies the rating region for which the data in the bytes to follow is defined. The rating_region associates ratings data given here with data defined in a Ratings Region Table tagged with the corresponding rating region.

rated_dimensions: An 8-bit unsigned integer field that specifies the number of rating dimensions for which content advisories are specified for this event. The value of this field shall not be greater than the value specified by the field dimensions_defined in the corresponding RRT section.

rating_dimension_j: An 8-bit unsigned integer field specifies the dimension index into the RRT instance for the region specified by the field rating_region. These dimension indices shall be listed in numerical order, i.e. the value of rating_dimension_j+1 shall be greater than that of rating_dimension_j.

rating_value: A 4-bit field represents the rating value of the dimension specified by the field rating_dimension_j for the region given by rating_region.

rating_description_length: An 8-bit unsigned integer value in the range 0 to 80 that represents the length of the rating_description_text() field to follow.

rating_description_text(): The rating description in the format of a Multiple String Structure (see B.8.2). The rating_description display string shall be limited to 16 characters or less. The rating description text shall represent the program's rating in an abbreviated form suitable for on-screen display. The rating description text collects multidimensional text information into a single small text string. If "xxx" and "yyy" are abbreviated forms for rating values in two dimensions, then "xxx-yyy" and "xxx (yyy)" are examples of possible strings represented in rating_description_text().

The program source provider shall be the responsible party for insertion of correct content_advisory_descriptors in the Program Map Table (PMT). Also, the content_advisory_descriptors may be included in Aggregate Event Information Tables. If content_ advisory_descriptors are available both in AEIT and PMT, the PMT should be used first, then the AEITs.

### B.7.6	Revision detection descriptor

The revision_detection_descriptor() is used to indicate whether new information is contained in the table section in which it appears.

Table B.43 describes the revision_detection_descriptor. This descriptor should be the first descriptor in the list to limit processing overhead.

Table B.43/J.94 – Revision Detection Descriptor format

|  | Bits | Bytes | Format |
| --- | --- | --- | --- |
| revision_detection_descriptor(){ |  |  |  |
| descriptor_tag | 8 | 1 | uimsbf value 0x93 |
| descriptor_length | 8 | 1 | uimsbf |
| reserved | 3 | 1 | bslbf |
| table_version_number | 5 |  | uimsbf range 0-31 |
| section_number | 8 | 1 | uimsbf range 0-255 |
| last_section_number | 8 | 1 | uimsbf range 0-255 |
| } |  |  |  |

descriptor_tag: An 8-bit unsigned integer number that identifies the descriptor as a revision_detection_descriptor(). The tag shall have the value 0x93.

descriptor_length: An 8-bit unsigned integer number that indicates the number of bytes to follow in the descriptor. At present, just three bytes are defined, but the length field shall be processed to allow new data to be added to the descriptor in the future.

table_version_number: This 5-bit unsigned integer in the range 0 to 31 identifies the version of the current table. This integer applies only to the table (or the section of it) currently transmitted. Other types of tables may have different version numbers. To indicate a change in a specific table, this integer is incremented by 1 modulo 32.

section_number: An 8-bit unsigned integer in the range 0 to 255 that identifies the current table section. Version numbers for all sections of a table must be the same. Note that section_number = 0 indicates the first section of a table.

last_section_number: An 8-bit unsigned integer in the range 0 to 255 that identifies the number of sections in a table. Note that if the last_section_number = 0, then there is only one section in this table.

### B.7.7	Two-Part channel number descriptor

Table B.44 describes the two_part_channel_number_descriptor(). This descriptor may appear in the virtual_channel() record, contained in the VCM_structure; within the Short-form Virtual Channel Table section. The descriptor may be used by compatible Hosts to associate a two-part user channel number with any virtual channel. Some channels may have a two_part_channel_number_descriptor() while others do not.

NOTE  For the L-VCT, the 10-bit major/minor number fields can be coded to represent a one-part channel number. The one-part representation is not needed for the major/minor number fields in the two_part_channel_number_descriptor() in the S-VCT, because there is already a 12-bit one-part number on each channel in S-VCT. It would cause confusion to allow a second one-part number to be associated with a channel defined in S-VCT.

Table B.44/J.94 – Two-part Channel Number Descriptor format

|  | Bits | Bytes | Format |
| --- | --- | --- | --- |
| two_part_channel_number_descriptor(){ |  |  |  |
| descriptor_tag | 8 | 1 | uimsbf value 0x94 |
| descriptor_length | 8 | 1 | uimsbf |
| Reserved | 6 | 2 | bslbf |
| major_channel_number | 10 |  | uimsbf range 0-999 |
| Reserved | 6 | 2 | bslbf |
| minor_channel_number | 10 |  | uimsbf range 0-999 |
| } |  |  |  |

descriptor_tag: An 8-bit unsigned integer number that identifies the descriptor as a two_part_channel_number_descriptor(). The tag shall have the value 0x94.

descriptor_length: An 8-bit unsigned integer number that indicates the number of bytes to follow in the descriptor. At present, just four bytes are defined, but the length field shall be processed to allow new data to be added to the descriptor in the future.

major_channel_number: A 10-bit unsigned integer in the range 0 to 999 that identifies the "major" channel number to be associated with the virtual channel.

minor_channel_number: A 10-bit unsigned integer in the range 0 to 999 that identifies the "minor" channel number to be associated with the virtual channel.

Hosts that support two-part channel numbering must support this descriptor. It is only mandatory for this descriptor to be sent in the instance where system support of two-part channel numbering is required. This means for virtual_channel() records where the Host does not receive the two-part channel number descriptor, that the Host is expected to use the virtual_channel_number described in the virtual_channel() record in B.6.3.2.

### B.7.8	Channel properties descriptor

The channel_properties_descriptor() is defined to allow both forms of VCTs (S-VCT and L-VCT) carrying the same properties. Table B.45 describes the syntax for this descriptor. The descriptor may appear within a virtual_channel() record in the Short-form Virtual Channel Table.

Table B.45/J.94 – Channel Properties Descriptor format

|  | Bits | Bytes | Format |
| --- | --- | --- | --- |
| channel_properties_descriptor(){ |  |  |  |
| descriptor_tag | 8 | 1 | uimsbf value 0x95 |
| descriptor_length | 8 | 1 | uimsbf |
| channel_TSID | 16 | 2 | uimsbf |
| reserved | 6 | 1 | '111111' |
| out_of_band_channel | 1 |  | uimsbf |
| access_controlled | 1 |  | uimbsf |
| hide_guide | 1 | 1 | bslbf |
| reserved | 1 |  | '1' |
| service_type | 6 |  | uimsbf |
| } |  |  |  |

descriptor_tag: An 8-bit unsigned integer number that identifies the descriptor as a channel_properties_descriptor(). The tag shall have the value 0x95.

descriptor_length: An 8-bit unsigned integer number that indicates the number of bytes to follow in the descriptor. At present, just four bytes are defined, but the length field shall be processed to allow new data to be added to the descriptor in the future.

channel_TSID: A 16-bit unsigned integer field in the range 0x0000 to 0xFFFF that represents the MPEG-2 Transport Stream ID associated with the Transport Stream carrying the MPEG-2 program referenced by this virtual channel. For inactive channels, channel_TSID represents the ID of the Transport Stream that will carry the service when it becomes active. The Host may use the channel_TSID to verify that a TS acquired at the referenced carrier frequency is actually the desired multiplex. Analogue signals may have a TSID that is different from any MPEG-2 Transport Stream identifier, that is, it shall be truly unique if present. A value of 0xFFFF for channel_TSID shall be specified for situations where a valid TSID is not known (reserved as a wildcard capability).

out_of_band: A Boolean flag that indicates, when set, that the virtual channel associated with this descriptor is carried on the cable on the Extended Channel interface carrying the tables defined in this protocol. When clear, the virtual channel is carried within a standard tuned multiplex at that frequency.

access_controlled: A Boolean flag that indicates, when set, that events associated with this virtual channel may be access controlled. When the flag is zero, event access is not restricted.

hide_guide: A Boolean flag that indicates, when set to 0 for a channel of channel_type hidden, that the virtual channel and its events may appear in EPG displays. This bit shall be ignored for channels which are not the hidden type, so that non-hidden channels and their events may always be included in EPG displays regardless of the state of the hide_guide bit. Typical applications for hidden channels with the hide_guide bit set to 1 are test signals and services accessible through application-level pointers.

service_type: A 6-bit enumerated type field that identifies the type of service carried in this virtual channel. Service type is coded according to Table B.33.

Hosts may use this descriptor to become aware of aspects of the channel. In the case where this descriptor is not received, the Host must tune the channel and self-discover these aspects of the channel. For example, if this descriptor is not sent, and the channel is access controlled, the Host must determine when it can obtain access permission (the same as if that bit in the descriptor were set). Similar rules can be applied for service type and channel_TSID.

### B.7.9	Extended channel name descriptor

The extended channel name descriptor provides the long channel name for the virtual channel containing this descriptor.

The bit stream syntax for the extended channel name descriptor is shown in Table B.46.

Table B.46/J.94 – Extended Channel Name Descriptor format

| Syntax | Bits | Bytes | Format |
| --- | --- | --- | --- |
| extended_channel_name_descriptor() { |  |  |  |
| descriptor_tag | 8 | 1 | 0xA0 |
| descriptor_length | 8 | 1 | uimsbf |
| long_channel_name_text() | Var |  |  |
| } |  |  |  |

descriptor_tag: This 8-bit unsigned integer shall have the value 0xA0, identifying this descriptor as extended_channel_name_descriptor().

descriptor_length: This 8-bit unsigned integer specifies the length (in bytes) immediately following this field up to the end of this descriptor.

long_channel_name_text(): The long channel name in the format of a Multiple String Structure (see B.8.2).

### B.7.10	Time-shifted service descriptor

This descriptor links one virtual channel with one or more virtual channels that carry the same programming on a time-shifted basis. The typical application is for Near Video On Demand (NVOD) services.

NOTE – For the L-VCT, the 10-bit major/minor number fields can be coded to represent a one-part channel number. The one-part representation is not applicable for the major/minor number fields in the time_shifted_services_descriptor() because this descriptor is not applicable to S-VCT (see Table B.A.2). The major/minor number fields in the time_shifted_services_descriptor() are only used to match against fields in the L-VCT.

The bit stream syntax for the time_shifted_service_descriptor() is shown in Table B.47.

Table B.47/J.94 – Time-Shifted Service Descriptor format

| Syntax | Bits | Bytes | Format |
| --- | --- | --- | --- |
| time_shifted_service_descriptor() { |  |  |  |
| descriptor_tag | 8 | 1 | 0xA2 |
| descriptor_length | 8 | 1 | uimsbf |
| reserved | 3 | 1 | '111' |
| number_of_services | 5 |  | uimsbf |
| for (i=0;i<number_of_services;i++) { |  |  |  |
| reserved | 6 | 1 | '111111' |
| time_shift | 10 | 1 | uimsbf |
| reserved | 4 | 2 | '1111' |
| major_channel_number | 10 |  | uimsbf |
| minor_channel_number | 10 | 2 | uimsbf |
| } |  |  |  |
| } |  |  |  |

descriptor_tag: This 8-bit unsigned integer shall have the value 0xA2, identifying this descriptor as time_shifted_service_descriptor().

descriptor_length: This 8-bit unsigned integer specifies the length (in bytes) immediately following this field up to the end of this descriptor.

number_of_services: A 5-bit number in the range 1 to 20 that indicates the number of time-shifted services being defined here.

time_shift: A 10-bit number in the range 1 to 720 that represents the number of minutes the time-shifted service indicated by major_channel_number and minor_channel_number is time-shifted from the virtual channel associated with this descriptor.

major_channel_number: A 10-bit number in the range 1 to 999 that represents the "major" channel number associated with a time-shifted service.

minor_channel_number: A 10-bit number in the range 0 to 999 that, when non-zero, represents the "minor" or "sub-" channel number of the virtual channel that carries a time-shifted service.

### B.7.11	Component name descriptor

Table B.48 defines the component_name_descriptor(), which serves to define an optional textual name tag for any component of the service.

Table B.48/J.94 – Component Name Descriptor format

| Syntax | Bits | Bytes | Format |
| --- | --- | --- | --- |
| component_name_descriptor() { |  |  |  |
| descriptor_tag | 8 | 1 | 0xA3 |
| descriptor_length | 8 | 1 | uimsbf |
| component_name_string() | var |  |  |
| } |  |  |  |

descriptor_tag: This 8-bit unsigned integer shall have the value 0xA3, identifying this descriptor as component_name_descriptor.

descriptor_length: This 8-bit unsigned integer specifies the length (in bytes) immediately following this field up to the end of this descriptor.

component_name_string(): The name string in the format of a Multiple String Structure (see B.8.2).

### B.7.12	Daylight savings time descriptor

This descriptor is defined for optional carriage in the System Timetable section (and in no other type of table). Hosts may use the data in the descriptor if present. If not present, no indication is being provided as to whether daylight savings time is in effect or not. In other words, the Host shall not infer that the lack of a descriptor means that daylight savings time is not currently in effect.

A description of the use of the daylight_savings_time_descriptor() is provided in Appendix B.III. The syntax is shown in Table B.49.

Table B.49/J.94 – Daylight Savings Time Descriptor format

| Syntax | Bits | Bytes | Format |
| --- | --- | --- | --- |
| daylight_savings_time_descriptor() { |  |  |  |
| descriptor_tag | 8 | 1 | uimsbf value 0x96 |
| descriptor_length | 8 | 1 | uimsbf |
| DS_status | 1 | 1 | bslbf |
| reserved | 2 |  | '11' |
| DS_day_of_month | 5 |  | uimsbf |
| DS_hour | 8 | 8 | uimsbf |
| } |  |  |  |

descriptor_tag: This 8-bit unsigned integer shall have the value 0x96, identifying this descriptor as daylight_savings_time_descriptor.

descriptor_length: This 8-bit unsigned integer specifies the length (in bytes) immediately following this field up to the end of this descriptor.

DS_status: This bit indicates the status of daylight savings.

DS_status = '0': Not in daylight savings time.

DS_status = '1': In daylight savings time.

DS_day_of_month: This 5-bit unsigned integer field indicates the local day of the month on which the transition into or out of daylight savings time is to occur (1-31).

DS_hour: This 8-bit unsigned integer field indicates the local hour at which the transition into or out of daylight savings time is to occur (0-18). This usually occurs at 2 a.m. in the United States.

### B.7.13	User private descriptors

Privately defined descriptors are those with descriptor_tag in the range 0xC0 through 0xFF. They may be placed at any location where descriptors may be included within the table sections described in this Service Information annex. Ownership of one or more user private descriptors is indicated by the presence of an MPEG registration_descriptor() preceding the descriptor(s).

## B.8	Text string coding

This clause describes the format of text strings in this Service Information annex. Two different formats are used in this annex. Text strings in the Network Text Table uses a format called Multilingual Text String (MTS), consisting of one or more mode-length-segment blocks. The MTS format is described in B.8.1. All other tables and descriptors use a data structure called Multiple String Structure, described in B.8.2. Tables B.50 and B.51 summarize these rules.

Table B.50/J.94 – Text String Coding Format in Tables

| Table ID Value (hex) | Table | Coding | Reference |
| --- | --- | --- | --- |
| 0xC3 | Network Text Table (NTT) | MTS | B.8.1 |
| 0xCA | Rating Region Table (RRT) | MSS | B.8.2 |
| 0xD6 | Aggregate Event Information Table (AEIT) | MSS | B.8.2 |
| 0xD7 | Aggregate Extended Text Table (AETT) | MSS | B.8.2 |

Table B.51/J.94  Text String Coding Format in Descriptors

| Descriptor tag value (hex) | Descriptor | Coding | Reference |
| --- | --- | --- | --- |
| 0x87 | Content advisory descriptor | MSS | B.8.2 |
| 0xA0 | Extended channel name descriptor | MSS | B.8.2 |
| 0xA3 | Component name descriptor | MSS | B.8.2 |

### B.8.1	Multilingual Text String (MTS) Format

The format of Multilingual Text Strings adheres to the following structure. Items in square brackets may be repeated one or more times:

<mode><length><segment> [ <mode><length><segment> ]

A string_length field always precedes the one or more instances of mode, length, segment. This field is described in each instance where multilingual text is used, and may be either 8- or 16-bits in length, as appropriate. The value of string_length represents the sum total of all mode, length, segment blocks comprising the multilingual text string to follow, and serves to indicate the end of the text string structure.

The multilingual text data structure is designed to accommodate the need to represent a text string composed of characters from a variety of alphabets, as well as ideographic characters. Whereas characters could be represented using 16- or 32-bit character codes (as does Unicode [ISO/IEC 10646-1]), that form is inefficient and wasteful of transmission bandwidth for strings composed primarily of alphabetic rather than ideographic characters. To accommodate the need to handle Chinese, Japanese, and Korean, modes are defined that allow 16-bit (double byte) character representations in standard formats.

References below to ISO/IEC 10646-1 (Unicode) shall be to the Basic Multilingual Plane (BMP) within that standard.

mode: An 8-bit value representing the text mode to be used to interpret characters in the segment to follow. See Table B.52 for definition. Mode bytes in the range zero through 0x3E select Unicode character code pages. Mode byte value 0x3F selects 16-bit Unicode character coding. Mode bytes in the range 0x40 through 0xFF represent selection of a format effector function such as underline ON or new line. If mode is in the range 0x40 to 0x9F, then the length/segment portion is omitted. Format effector codes in the range 0x40 through 0x9F involve no associated parametric data; hence the omission of the length/segment portion. Format effector codes in the range 0xA0 through 0xFF include one or more parameters specific to the particular format effector function.

| Table B.52/J.94 – Mode Byte Encoding |  |  |
| --- | --- | --- |
| Mode Byte | Meaning | Language(s) or script |
| 0x00 | Select ISO/IEC 10646-1 Page 0x00 | ASCII, ISO Latin-1 (Roman) |
| 0x01 | Select ISO/IEC 10646-1 Page 0x01 | European Latin (many)a) |
| 0x02 | Select ISO/IEC 10646-1 Page 0x02 | Standard Phonetic |
| 0x03 | Select ISO/IEC 10646-1 Page 0x03 | Greek |
| 0x04 | Select ISO/IEC 10646-1 Page 0x04 | Russian, Slavic |
| 0x05 | Select ISO/IEC 10646-1 Page 0x05 | Armenian, Hebrew |
| 0x06 | Select ISO/IEC 10646-1 Page 0x06 | Arabicb) |
| 0x07-0x08 | Reserved |  |
| 0x09 | Select ISO/IEC 10646-1 Page 0x09 | Devanagaric), Bengali |

Table B.52/J.94 – Mode Byte Encoding (concluded)

| Mode Byte | Meaning | Language(s) or script |
| --- | --- | --- |
| 0x0A | Select ISO/IEC 10646-1 Page 0x0A | Punjabi, Gujarti |
| 0x0B | Select ISO/IEC 10646-1 Page 0x0B | Oriya, Tamil |
| 0x0C | Select ISO/IEC 10646-1 Page 0x0C | Telugu, Kannada |
| 0x0D | Select ISO/IEC 10646-1 Page 0x0D | Malayalam |
| 0x0E | Select ISO/IEC 10646-1 Page 0x0E | Thai, Lao |
| 0x0F | Select ISO/IEC 10646-1 Page 0x0F | Tibetan |
| 0x10 | Select ISO/IEC 10646-1 Page 0x10 | Georgian |
| 0x11-0x1F | Reserved |  |
| 0x20 | Select ISO/IEC 10646-1 Page 0x20 | Miscellaneousd) |
| 0x21 | Select ISO/IEC 10646-1 Page 0x21 | Misc. symbols, arrows |
| 0x22 | Select ISO/IEC 10646-1 Page 0x22 | Mathematical operators |
| 0x23 | Select ISO/IEC 10646-1 Page 0x23 | Misc. technical |
| 0x24 | Select ISO/IEC 10646-1 Page 0x24 | OCR, enclosed alpha-num. |
| 0x25 | Select ISO/IEC 10646-1 Page 0x25 | Form and chart components |
| 0x26 | Select ISO/IEC 10646-1 Page 0x26 | Miscellaneous dingbats |
| 0x27 | Select ISO/IEC 10646-1 Page 0x27 | Zapf dingbats |
| 0x28-0x2F | Reserved |  |
| 0x30 | Select ISO/IEC 10646-1 Page 0x30 | Hiragana, Katakana |
| 0x31 | Select ISO/IEC 10646-1 Page 0x31 | Bopomopho, Hangul elem. |
| 0x32 | Select ISO/IEC 10646-1 Page 0x32 | Enclosed CJK Letters, ideo. |
| 0x33 | Select ISO/IEC 10646-1 Page 0x33 | Enclosed CJK Letters, ideo. |
| 0x34-0x3E | Reserved |  |
| 0x3F | Select 16-bit ISO/IEC 10646-1 mode | All |
| 0x40-0x9F | Format effector (single byte) | See Table B.41. |
| 0xA0-0xFF | Format effector (with parameter[s]) |  |
| a) When combined with page zero (ASCII and ISO Latin-1), covers Afrikaans, Breton, Basque, Catalan, Croatian, Czech, Danish, Dutch, Esperanto, Estonian, Faroese, Finnish, Flemish, Firsian, Greenlandic, Hungarian, Icelandic, Italian, Latin, Latvian, Lithuanian, Malay, Maltese, Norwegian, Polish, Portuguese, Provencal, Ghaeto-Romanic, Romanian, Romany, Slovak, Slovenian, Serbian, Spanish, Swedish, Turkish, and Welsh. b) Also Persian, Urdu, Pashto, Sindhi, and Kurdish. c) Devanagari script is used for writing Sanskrit and Hindi, as well as other languages of northern India (such as Marathi) and of Nepal (Nepali). In addition, at least two dozen other Indian languages use Devanagari script. d) General punctuation, superscripts and subscripts, currency symbols, and other diacritics. |  |  |

Table B.53 describes the format of the multilingual_text_string().

Table B.53/J.94 – Multilingual text string format

|  | Bits | Bytes | Format |
| --- | --- | --- | --- |
| multilingual_text_string(){ |  |  |  |
| For (i=0; i<N; i++) { |  |  |  |
| Mode | 8 | (1) | uimsbf |
| if (mode < 0x3F) { |  |  |  |
| eightbit_string_length | 8 | ((1)) | uimsbf |
| for (i=0; i<eightbit_string_length; I++) { |  |  |  |
| eightbit_char | 8 | (((1))) | uimsbf |
| } |  |  |  |
| } else if (mode==0x3F) { |  |  |  |
| sixteenbit_string_length | 8 | ((1)) | uimsbf (even) |
| for (i=0; i<(sixteenbit_string_length); i+=2) { |  |  |  |
| sixteenbit_char | 16 | (((2))) | uimsbf |
| } |  |  |  |
| } else if (mode >= 0xA0) { |  |  |  |
| format_effector_param_length | 8 | ((1)) | uimsbf |
| for (i=0; i<(format_effector_param_length); i++) { |  |  |  |
| format_effector_data | 8 | (((1))) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

length: An 8-bit unsigned integer number representing the number of bytes in the segment to follow in this block.

segment: An array of bytes representing a character string formatted according to the mode byte.

#### B.8.1.1	Mode byte definition

The mode byte is used either to select an ISO/IEC 10646-1 code page from the BMP (exact mapping, or in the case of page zero, an extended mapping as defined herein), or to indicate that the text segment is coded in one of a number of standard double-byte formats. Table B.52 shows the encoding of the mode byte. Values in the zero to 0x33 range select ISO/IEC 10646-1 code pages.

Value 0x3F selects double-byte forms used with non-alphabetic script systems, where the segment consists of a sequence of 16-bit character codes according to the ISO/IEC 10646-1 standard. Byte ordering is high-order byte first (Motorola 680xx style), also known as big-endian.

#### B.8.1.2	Format effectors

Mode bytes in the 0x40 to 0xFF range are defined as format effectors. Table B.54 defines the encoding for currently defined single-byte values. Format effectors in the range 0x40 through 0x9F are self-contained, and do not have a length or data field following them. Format effectors in the range 0xA0 through 0xFF include a multi-byte parameter field. No multi-byte format effectors are currently defined.

Table B.54/J.94 – Format Effector Function Codes

| Mode byte | Meaning |
| --- | --- |
| 0x40-0x7F | Reserved |
| 0x80 | new line, left justify |
| 0x81 | new line, right justify |
| 0x82 | new line, center |
| 0x83 | italics ON |
| 0x84 | italics OFF |
| 0x85 | underline ON |
| 0x86 | underline OFF |
| 0x87 | bold ON |
| 0x88 | bold OFF |
| 0x89-0x9F | Reserved |

Line justification

Values 0x80, 0x81, and 0x82 signify the end of a line of displayed text. Value 0x80 indicates that the text is displayed left justified within an enclosing rectangular region (defined outside the scope of the text string). Value 0x81 indicates that the text is displayed right justified. Value 0x82 indicates that the text is centered on the line. The dimensions and location on the screen of the box into which text is placed is defined outside the scope of the text string itself.

Italics, underline, bold attributes

These format effectors toggle italics, underline, and bold display attributes. The italics, underline, and bold format effectors indicate the start or end of the associated formatting within a text string. Formatting extends through new lines. For example, to display three lines of bold text, only one instance of the bold ON format effector is required.

Processing of unknown or unsupported format effectors

Hosts must discard format effectors that are unknown, or known not to be supported within a specific Host model. If a parameter value carries an undefined value, that format effector is expected to be discarded.

#### B.8.1.3	Default attributes

Upon entry to a multilingual text string, all mode toggles (bold, underline, italics) shall be assumed "OFF".

#### B.8.1.4	Mode Zero

ISO/IEC 10646-1 page zero (U+0000 through U+00FF) includes ASCII in the lower half (U+0000 through U+007F), and Latin characters from ISO 8859-1, Latin-1, in U+0090 through U+00FF. This set of characters covers Danish, Dutch, Faroese, Finnish, French, German, Icelandic, Irish, Italian, Norwegian, Portuguese, Spanish and Swedish. Many other languages can be written with this set of letters, including Hawaiian, Indonesian/Malay, and Swahili.

Table B.55 shows encodings of page zero characters in the range 0x80 through 0x9F (these are undefined within ISO/IEC 10646-1).

Table B.55/J.94 – Encodings of columns 8 and 9
of mode zero latin character set

|  | 8 | 9 |
| --- | --- | --- |
| 0 | <RESERVED> | <RESERVED> |
| 1 | <RESERVED> | <RESERVED> |
| 2 | <RESERVED> | <RESERVED> |
| 3 | <RESERVED> | <RESERVED> |
| 4 | <RESERVED> | <RESERVED> |
| 5 | <RESERVED> | <RESERVED> |
| 6 | <RESERVED> | <RESERVED> |
| 7 | <RESERVED> | <RESERVED> |
| 8 | <RESERVED> | U+2030  <PER MILLE> |
| 9 | <RESERVED> | <RESERVED> |
| A | <RESERVED> | U+266A  <MUSICAL NOTE> |
| B | <RESERVED> | <RESERVED> |
| C | <RESERVED> | U+2190  <LEFT ARROW> |
| D | <RESERVED> | U+2191  <UP ARROW> |
| E | <RESERVED> | U+2192  <RIGHT ARROW> |
| F | <RESERVED> | U+2193  <DOWN ARROW> |

#### B.8.1.5	Supported characters

Support for specific characters and languages depends upon the specific model of Standard-compatible Host. Not all Hosts support all defined character sets or character codes. Use of multilingual text must be predicated on the knowledge of limitations in character rendering inherent in different Host models for which text is available.

### B.8.2	Multiple String Structure (MSS)

The Multiple String Structure is a general data structure used specifically for text strings. Text strings appear as event titles, long channel names, the ETT messages, and RRT text items. The bit stream syntax for the Multiple String Structure is shown in Table B.56.

| Table B.56/J.94 – Multiple String Structure |  |  |
| --- | --- | --- |
| Syntax | Bits | Format |
| multiple_string_structure () { |  |  |
| number_strings | 8 | uimsbf |
| for (i= 0;i< number_strings;i++) { |  |  |
| ISO_639_language_code | 8*3 | uimsbf |
| number_segments | 8 | uimsbf |
| for (j=0;j<number_segments;j++) { |  |  |
| compression_type | 8 | uimsbf |
| Mode | 8 | uimsbf |
| number_bytes | 8 | uimsbf |
| for (k= 0;k<number_bytes;k++) |  |  |
| compressed_string_byte [k] | 8 | bslbf |
| } |  |  |
| } |  |  |
| } |  |  |

number_strings: This 8-bit unsigned integer field identifies the number of strings in the following data.

ISO_639_language_code: This 3-byte (24 bits) field, in conformance with ISO 639-2/B, specifies the language used for the ith string.

number_segments: This 8-bit unsigned integer field identifies the number of segments in the following data. A specific mode is assigned for each segment.

compression_type: This 8-bit field identifies the compression type for the jth segment. Allowed values for this field are shown in Table B.57.

Table B.57/J.94 – Compression types

| compression_type | Compression method |
| --- | --- |
| 0x00 | No compression |
| 0x01 | Huffman coding using standard encode/decode tables defined in Table C.4 and C.5 in Annex C of SCTE DVS 097, ATSC Standard A/65 (1997). |
| 0x02 | Huffman coding using standard encode/decode tables defined in Table C.6 and C.7 in Annex C of SCTE DVS 097, ATSC Standard A/65 (1997). |
| 0x03 to 0xAF | Reserved |
| 0xB0 to 0xFF | User private |

mode: An 8-bit value representing the text mode to be used to interpret characters in the segment to follow. See Table B.58 for definition. Mode values in the range zero through 0x3E select 8-bit Unicode™ character code pages. Mode value 0x3F selects 16-bit Unicode™ character coding. Mode values 0x40 through 0xDF are reserved for future use by ATSC. Mode values 0xE0 through 0xFE are user private. Mode value 0xFF indicates the text mode is not applicable. Hosts shall ignore string bytes associated with unknown or unsupported mode values.

| Table B.58/J.94 – Modes |  |  |
| --- | --- | --- |
| Mode | Meaning | Language(s) or script |
| 0x00 | Select ISO/IEC 10646-1 Page 0x00 | ASCII, ISO Latin-1 (Roman)a) |
| 0x01 | Select ISO/IEC 10646-1 Page 0x01 | European Latin (many)b) |
| 0x02 | Select ISO/IEC 10646-1 Page 0x02 | Standard Phonetic |
| 0x03 | Select ISO/IEC 10646-1 Page 0x03 | Greek |
| 0x04 | Select ISO/IEC 10646-1 Page 0x04 | Russian, Slavic |
| 0x05 | Select ISO/IEC 10646-1 Page 0x05 | Armenian, Hebrew |
| 0x06 | Select ISO/IEC 10646-1 Page 0x06 | Arabicc) |
| 0x07-0x08 | Reserved |  |
| 0x09 | Select ISO/IEC 10646-1 Page 0x09 | Devanagarid), Bengali |
| 0x0A | Select ISO/IEC 10646-1 Page 0x0A | Punjabi, Gujarati |
| 0x0B | Select ISO/IEC 10646-1 Page 0x0B | Oriya, Tamil |
| 0x0C | Select ISO/IEC 10646-1 Page 0x0C | Telugu, Kannada |
| 0x0D | Select ISO/IEC 10646-1 Page 0x0D | Malayalam |
| 0x0E | Select ISO/IEC 10646-1 Page 0x0E | Thai, Lao |
| 0x0F | Select ISO/IEC 10646-1 Page 0x0F | Tibetan |

Table B.58/J.94 – Modes (concluded)

| Mode | Meaning | Language(s) or script |
| --- | --- | --- |
| 0x10 | Select ISO/IEC 10646-1 Page 0x10 | Georgian |
| 0x11-0x1F | Reserved |  |
| 0x20 | Select ISO/IEC 10646-1 Page 0x20 | Miscellaneous |
| 0x21 | Select ISO/IEC 10646-1 Page 0x21 | Misc. symbols, arrows |
| 0x22 | Select ISO/IEC 10646-1 Page 0x22 | Mathematical operators |
| 0x23 | Select ISO/IEC 10646-1 Page 0x23 | Misc. technical |
| 0x24 | Select ISO/IEC 10646-1 Page 0x24 | OCR, enclosed alpha-num. |
| 0x25 | Select ISO/IEC 10646-1 Page 0x25 | Form and chart components |
| 0x26 | Select ISO/IEC 10646-1 Page 0x26 | Miscellaneous dingbats |
| 0x27 | Select ISO/IEC 10646-1 Page 0x27 | Zapf dingbats |
| 0x28-0x2F | Reserved |  |
| 0x30 | Select ISO/IEC 10646-1 Page 0x30 | Hiragana, Katakana |
| 0x31 | Select ISO/IEC 10646-1 Page 0x31 | Bopomopho, Hangul elem. |
| 0x32 | Select ISO/IEC 10646-1 Page 0x32 | Enclosed CJK Letters, ideo. |
| 0x33 | Select ISO/IEC 10646-1 Page 0x33 | Enclosed CJK Letters, ideo. |
| 0x34-0x3E | Reserved |  |
| 0x3F | Select 16-bit ISO/IEC 10646-1 mode | All |
| 0x40-0xDF | Reserved |  |
| 0xE0-0xFE | User private |  |
| 0xFF | Not applicable |  |
| a) The languages supported by ASCII plus the Latin-1 supplement include Danish, Dutch, English, Faroese, Finnish, Flemish, German, Icelandic, Irish, Italian, Norwegian, Portuguese, Spanish and Swedish. Many other languages can be written with this set of characters, including Hawaiian, Indonesian, and Swahili. b) When combined with page zero (ASCII and ISO Latin-1), covers Afrikaans, Breton, Basque, Catalan, Croatian, Czech, Esperanto, Estonian, French, Frisian, Greenlandic, Hungarian, Latin, Latvian, Lithuanian, Maltese, Polish, Provencal, Rhaeto-Romanic, Romanian, Romany, Sami, Slovak, Slovenian, Sorbian, Turkish, Welsh, and many others. c) Also Persian, Urdu, Pashto, Sindhi, and Kurdish. d) Devanagari script is used for writing Sanskrit and Hindi, as well as other languages of northern India (such as Marathi) and of Nepal (Nepali). In addition, at least two dozen other Indian languages use Devanagari script. |  |  |

number_bytes: This 8-bit unsigned integer field identifies the number of bytes that follows.

compressed_string_byte[k]: The kth byte of the jth segment.

Annex B.A

Operational profiles for cable service information delivery

### B.A.1	Operational profiles

This Annex B.A specifies Service Information tables that are required for delivery via an out-of-band channel on cable. Six profiles are described with required and optional data specified for out-of-band transport via cable. Adherence to these profile specifications is necessary for compliance with SCTE standard transport streams.

#### B.A.1.1	Profile 1 – Baseline

This Baseline Profile reflects a practice in cable where the Short-Form Virtual Channel Table, the Modulation Mode Subtable and the Carrier Definition Subtable are used for channel navigation.

#### B.A.1.2	Profile 2 – Revision detection

Profile 2 uses the same channel navigation mechanism as Profile 1 while adding a detection mechanism that facilitates revision handling of tables. The revision detection mechanism is applicable to the Network Information Table, Network Text Table, and S-VCT that are also used in Profile 1.

#### B.A.1.3	Profile 3 – Parental advisory

Profile 3 uses Profile 2 as the base and adds support for the Rating Region Table in order to be compliant with the FCC-mandated V-chip content advisory scheme. Since for the U.S. and its possessions, EIA-766 defines the contents of version 0 RRT, use of RRT is more applicable to outside of North America. The channel navigation mechanism is the same as in Profile 1.

#### B.A.1.4	Profile 4 – Standard electronic program guide data

Profile 4 uses Profile 3 as the base and further defines a standard format for delivery of Electronic Program Guide data by using the Aggregate Event Information Table and the Aggregate Extended Text Table. The Master Guide Table shall be supported to manage the AEITs, AETTs and other applicable tables from Profile 3. The same mechanism as in Profile 1 is used for channel navigation.

#### B.A.1.5	Profile 5 – Combination

Support for channel navigation based on L-VCT and MGT is added. Backward compatibility with systems operating within profiles 1 to 4 is maintained. Using Profile 5, a cable operator could have a mixture of devices requiring the S-VCT, NIT and NTT tables as well as ones requiring the long-form tables: i.e. L-VCT, MGT.

When using Profile 5, both the S-VCT and the L-VCT shall be present, and each shall describe all available services.

#### B.A.1.6	Profile 6 – PSIP Only

Profile 6 is based solely on long-form tables and is an extension of the terrestrial broadcasting mechanism. Channel navigation is based on the Long-form Virtual Channel Table. The AEIT and the optional AETT streams are used to provide EPG data.

### B.A.2	Profile Definition Tables

In order to conform to this Service Information Annex B.A, a cable operator shall send a collection of tables that corresponds to one or more of the defined operational profiles defined in Table B.A.1 and Table B.A.2.

Table B.A.1/J.94 – Usage of Table Sections in Various Profiles

|  |  | Profile 1 | Profile 2 | Profile 3 | Profile 4 | Profile 5 | Profile 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Table Section | Table ID | Baseline | Revision Detection | Parental Advisory | Standard EPG Data | Combi- nation | PSIP only (Note 1) |
| Network Information Table | 0xC2 |  |  |  |  |  |  |
| Carrier Definition Subtable |  | M | M | M | M | M |  |
| Modulation Mode Subtable |  | M | M | M | M | M |  |
| Network Text Table | 0xC3 |  |  |  |  |  |  |
| Source Name Subtable |  | O | O | O | M | M |  |
| Short-form Virtual Channel Table | 0xC4 |  |  |  |  |  |  |
| Virtual Channel Map |  | M | M | M | M | M |  |
| Defined Channels Map |  | M | M | M | M | M |  |
| Inverse Channel Map |  | O | O | O | O | O |  |
| System Timetable | 0xC5 | M | M | M | M | M | M |
| Master Guide Table | 0xC7 |  |  | (Note 2) | M | M | M |
| Rating Region Table | 0xCA |  |  | (Note 3) | (Note 3) | (Note 3) | (Note 3) |
| Long-form Virtual Channel Table | 0xC9 |  |  |  |  | M | M |
| Aggregate Event Information Table | 0xD6 |  |  |  | M | M | M |
| Aggregate Extended Text Table | 0xD7 |  |  |  | O | O | O |
| M Mandatory (shall be present) O Optional (may or may not be present)  Not applicable (shall not be present) NOTE 1  Exception: System Timetable (table ID 0xC5 is used here instead of table ID 0xCD defined in PSIP) and other modifications. NOTE 2  Mandatory for outside of North America to describe any transmitted RRT. For region 0x01 (US and possessions), delivery of an RRT is optional, because this table is standardized in EIA-766. NOTE 3  Exception: delivery of the RRT corresponding to region 0x01 (US and possessions) is optional, because this table is standardized in EIA-766. |  |  |  |  |  |  |  |

Table B.A.2/J.94 – Usage of Descriptors in Various Profiles

|  |  | Profile 1 | Profile 2 | Profile 3 | Profile 4 | Profile 5 | Profile 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Descriptor (and associated table) | Tag | Baseline | Revision Detection | Parental Advisory | Standard EPG Data | Combi- nation | PSIP only (Note 1) |
| AC-3 audio (PMT, AEIT) | 0x81 |  |  |  | O | O | O |
| Caption service (PMT, AEIT) | 0x86 |  |  |  | O | O | O |
| Content advisory (PMT, AEIT) | 0x87 |  |  | (Note 2) | (Note 2) | (Note 2) | (Note 2) |
| Revision detection (NIT,NTT, S-VCT) | 0x93 |  | M | M | M | M |  |
| Two-part channel number (S-VCT) | 0x94 |  |  |  | O | O |  |
| Channel properties (S-CT) | 0x95 |  |  |  | O | O |  |
| Daylight savings time (STT) | 0x96 |  |  | O | M | M | M |
| Extended channel name (L-VCT) | 0xA0 |  |  |  |  | O | O |
| Time-shifted service (L-CT) | 0xA2 |  |  |  |  | O | O |
| Component name (PMT) | 0xA3 |  |  |  | O | O | O |
| M Mandatory (shall be present) O Optional (may or may not be present)  Not applicable (shall not be present) NOTE 1 – Exception: System Timetable (table ID 0xC5 is used here instead of table ID 0xCD defined in PSIP) and other modifications. NOTE 2 – The content_advisory_descriptor() shall be present in the AEIT and PMT for a given program when Content Advisory data is available for that program. It is not required for programs for which Content Advisory data is not available. |  |  |  |  |  |  |  |

### B.A.3	Operational considerations for the use of profiles (Informative)

1)	If devices deployed in a particular cable system require the S-VCT in Profiles 1-5 for navigation, cable operator's use of P6 will cause operational problems.

2)	If devices in use require L-VCT for navigation, cable operator's use of Profiles 1-4 will cause operational problems.

3)	To provide EPG data, cable-ready devices operating on a cable system conforming to Profiles 1, 2 or 3 must use alternative protocols and methods which are beyond the scope of this Annex B.A.

Annex B.B

Packet rates

### B.B.1	Maximum cycle times

Table B.B.1 lists the maximum cycle time for Service Information table sections for out-of-band cable operation, when the indicated table is present.

Table B.B.1/J.94 – Maximum cycle time for the STT, MGT, S-VCT, L-VCT and RRT

| Table Section | STT | MGT | S-VCT | L-VCT | RRT |
| --- | --- | --- | --- | --- | --- |
| Cycle time | 1 min | 500 msec | 2 min | 2 min | 1 min |

### B.B.2	Maximum transmission rates

Table B.B.2 lists the maximum transmission rate for SI packet streams.

Table B.B.2/J.94 – Maximum rate for each packet stream

| PID | SI_base PID | Any AEIT/AETT PID |
| --- | --- | --- |
| Rate (bit/s) | 150 000 | 150 000 |

### B.B.3	Minimum transmission rates

Table B.B.3 lists the minimum transmission rate for SI packet streams. Minimum per-PID bit rates are required to ensure efficiency of recovery of EPG data covering the current time period (3 hours minimum) across the POD to Host interface, given the small number of PID values that can be used concurrently.

Table B.B.3/J.94 – Minimum rate for each packet stream

| PID | AEIT-0,1/AETT-0,1 PID |
| --- | --- |
| Rate (bit/s) | 10 000 |

Annex B.C

Standard Huffman tables for text compression

This Annex B.C describes the compression method adopted for the transmission of English-language text strings in PSIP. The method distinguishes two types of text strings: titles and program descriptions. For each of these types, Huffman tables are defined based on 1st-order conditional probabilities. Clause B.C.2 defines standard Huffman encode and decode tables optimized for English-language text such as that typically found in program titles. Clause B.C.3 defines Huffman encode and decode tables optimized for English-language text such as that typically found in program descriptions. Hosts supporting the English language are expected to support decoding of text using either of these two standard Huffman compression tables.

The encode tables provide necessary and sufficient information to build the Huffman trees that need to be implemented for decoding. The decode tables described in Tables B.C.5 and B.C.7 are a particular mapping of those trees into a numerical array suitable for storage. This array can be easily implemented and used with the decoding algorithm. However, the user is free to design its own decoding tables as long as they follow the Huffman trees and rules defined in this annex.

### B.C.1	Character set definition

This compression method supports the full ISO/IEC 8859-1 (Latin-1) character set, although only characters in the ASCII range (character codes 1 to 127) can be compressed. The following characters in Table B.C.1 have special definitions:

Table B.C.1/J.94 – Characters with special definitions

| Character | Value (Decimal) | Meaning |
| --- | --- | --- |
| String Terminate (ASCII Null) | 0 | The Terminate character is used to terminate strings. The Terminate character is appended to the string in either compressed or uncompressed form. The first encoded character in a compressed string is encoded/decoded from the Terminate sub-tree. In other words, when encoding or decoding the first character in a compressed string, assume that the previous character was a Terminate character. |
| Order-1 Escape (ASCII ESC) | 27 | Used to escape from first-order context to uncompressed context. The character which follows the Escape character is uncompressed. |

#### B.C.1.1	First Order Escape

The order-1 Huffman trees are partial, that is, codes are not defined for every possible character sequence. For example, the standard decode tables do not contain codes for the character sequence qp. When uncompressed text contains a character sequence which is not defined in the decode table, the order-1 escape character is used to escape back to the uncompressed context. Uncompressed symbols are coded as 8-bit ASCII (Latin-1). For example, the character sequence qpa would be coded with compressed q, compressed ESC, uncompressed p, compressed a.

First-order escape rules for compressed strings:

•	Any character which follows a first-order escape character is an uncompressed (8-bit) character. (Any character which follows an uncompressed escape character is compressed).

•	Characters (128 … 255) cannot be compressed.

•	Any character which follows a character from the set (128 … 255) is uncompressed.

#### B.C.1.2	Decode table data structures

Decode tables have two sections:

•	Tree Root Offset List: Provides the table offsets, in bytes from the start of the decode table, for the roots of the 128 first-order decode trees. The list is contained in bytes (0 … 255) of the decode table, and is defined by the first "for" loop in Table B.C.1.

•	Order-1 Decode Trees: Each and every character in the range (0 … 127) has a corresponding first-order decode tree. For example, if the previous character was "s", then the decoder would use the "s" first-order decode tree (decode tree #115) to decode the next character (ASCII "s" equals 115 decimal). These 128 decode trees are delimited by the second "for" loop in Table B.C.2.

Decode tables have the following format:

Table B.C.2/J.94 – Decode Table Format

| Syntax |  | Bits |  | Format |
| --- | --- | --- | --- | --- |
| decode_table() { |  |  |  |  |
| for (i==0; i<128; i++) { |  |  |  |  |
| byte_offset_of_char_i_tree_root | 16 |  | uimsbf |  |
| } |  |  |  |  |
| for (i==0; i<128; i++) { |  |  |  |  |
| character_i_order_1_tree() | 8*M |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Note that even though the ISO Latin-1 character set supports up to 256 characters, only the first 128 characters may be represented in compressed form.

##### B.C.1.2.1	Tree root byte offsets

byte_offset_of_character_i_tree_root: A 16-bit unsigned integer specifying the location, in bytes from the beginning of the decode table, of the root for the ith character's order-1 tree.

##### B.C.1.2.2	Order-1 decode trees

Order-1 decode trees are binary trees. The roots of the decode trees are located at the table offsets specified in the tree root offset list. The left and right children of a given node are specified as word offsets from the root of the tree (a word is equivalent to two bytes).

Decode trees have the format as shown in Table B.C.3:

Table B.C.3/J.94 – Decode tree format

| Syntax | Bits | Format |
| --- | --- | --- |
| character_i_order_1_tree() { |  |  |
| for (j==0; j<N; j++) { |  |  |
| left_child_word_offset_or_char_leaf | 8 | uimsbf |
| right_child_word_offset_or_char_leaf | 8 | uimsbf |
| } |  |  |
| } |  |  |

left_child_word_offset_or_character_leaf: An 8-bit unsigned integer number with the following interpretation: If the highest bit is cleared (i.e. bit 7 is zero), the number specifies the offset, in words, of the left child from the root of the order-1 decode tree; if the highest bit is set (bit 7 is one), the lower 7 bits give the code (e.g. in ASCII) for a leaf character.

right_child_word_offset_or_character_leaf: An 8-bit unsigned integer number with the following interpretation: If the highest bit is cleared (i.e. bit 7 is zero), the number specifies the offset, in words, of the right child from the root of the order-1 decode tree; if the highest bit is set (bit 7 is one), the lower 7 bits give the code (e.g. in ASCII) for a leaf character.

Each node (corresponding to one iteration of the for-loop) has a byte for the left child or character, and a byte for the right child or character.

Characters are leaves of the order-1 decode trees, and are differentiated from intermediate nodes by the byte's most significant bit. When the most significant bit is set, the byte is a character leaf. When the most significant bit is not set, the byte contains the tabular word offset of the child node.

### B.C.2	Standard compression Type 1 Encode/Decode Tables

The following encode/decode tables (Tables B.C.4 and B.C.5) are optimized for English-language program title text. These tables correspond to multiple_string_structure() with compression_type value 0x01, and a mode equal to 0xFF.

Table B.C.4/J.94 – English-language Program Title Encode Table

Prior Symbol:  0 Symbol: 27  Code: 11001011

Prior Symbol:  0 Symbol: '$'  Code: 1100101011

Prior Symbol:  0 Symbol: '2'  Code: 011010010

Prior Symbol:  0 Symbol: '4'  Code: 1100101010

Prior Symbol:  0 Symbol: '7'  Code: 011010011

Prior Symbol:  0 Symbol: 'A'  Code: 0111

Prior Symbol:  0 Symbol: 'B'  Code: 1001

Prior Symbol:  0 Symbol: 'C'  Code: 1011

Prior Symbol:  0 Symbol: 'D'  Code: 11011

Prior Symbol:  0 Symbol: 'E'  Code: 10001

Prior Symbol:  0 Symbol: 'F'  Code: 11000

Prior Symbol:  0 Symbol: 'G'  Code: 11100

Prior Symbol:  0 Symbol: 'H'  Code: 11111

Prior Symbol:  0 Symbol: 'I'  Code: 10000

Prior Symbol:  0 Symbol: 'J'  Code: 01100

Prior Symbol:  0 Symbol: 'K'  Code: 1100110

Prior Symbol:  0 Symbol: 'L'  Code: 11101

Prior Symbol:  0 Symbol: 'M'  Code: 1010

Prior Symbol:  0 Symbol: 'N'  Code: 0011

Prior Symbol:  0 Symbol: 'O'  Code: 011011

Prior Symbol:  0 Symbol: 'P'  Code: 11110

Prior Symbol:  0 Symbol: 'Q'  Code: 01101000

Prior Symbol:  0 Symbol: 'R'  Code: 11010

Prior Symbol:  0 Symbol: 'S'  Code: 000

Prior Symbol:  0 Symbol: 'T'  Code: 010

Prior Symbol:  0 Symbol: 'U'  Code: 0110101

Prior Symbol:  0 Symbol: 'V'  Code: 1100111

Prior Symbol:  0 Symbol: 'W'  Code: 0010

Prior Symbol:  0 Symbol: 'Y'  Code: 1100100

Prior Symbol:  0 Symbol: 'Z'  Code: 110010100

Prior Symbol:  1 Symbol: 27  Code: 1

Prior Symbol:  2 Symbol: 27  Code: 1

Prior Symbol:  3 Symbol: 27  Code: 1

Prior Symbol:  4 Symbol: 27  Code: 1

Prior Symbol:  5 Symbol: 27  Code: 1

Prior Symbol:  6 Symbol: 27  Code: 1

Prior Symbol:  7 Symbol: 27  Code: 1

Prior Symbol:  8 Symbol: 27  Code: 1

Prior Symbol:  9 Symbol: 27  Code: 1

Prior Symbol: 10 Symbol: 27  Code: 1

Prior Symbol: 11 Symbol: 27  Code: 1

Prior Symbol: 12 Symbol: 27  Code: 1

Prior Symbol: 13 Symbol: 27  Code: 1

Prior Symbol: 14 Symbol: 27  Code: 1

Prior Symbol: 15 Symbol: 27  Code: 1

Prior Symbol: 16 Symbol: 27  Code: 1

Prior Symbol: 17 Symbol: 27  Code: 1

Prior Symbol: 18 Symbol: 27  Code: 1

Prior Symbol: 19 Symbol: 27  Code: 1

Prior Symbol: 20 Symbol: 27  Code: 1

Prior Symbol: 21 Symbol: 27  Code: 1

Prior Symbol: 22 Symbol: 27  Code: 1

Prior Symbol: 23 Symbol: 27  Code: 1

Prior Symbol: 24 Symbol: 27  Code: 1

Prior Symbol: 25 Symbol: 27  Code: 1

Prior Symbol: 26 Symbol: 27  Code: 1

Prior Symbol: 27 Symbol: 27  Code: 1

Prior Symbol: 28 Symbol: 27  Code: 1

Prior Symbol: 29 Symbol: 27  Code: 1

Prior Symbol: 30 Symbol: 27  Code: 1

Prior Symbol: 31 Symbol: 27  Code: 1

Prior Symbol: ' ' Symbol: 27  Code: 10010100

Prior Symbol: ' ' Symbol: '&'  Code: 010001

Prior Symbol: ' ' Symbol: '''  Code: 010000100

Prior Symbol: ' ' Symbol: '-'  Code: 00000001

Prior Symbol: ' ' Symbol: '1'  Code: 010000101

Prior Symbol: ' ' Symbol: '2'  Code: 00000010

Prior Symbol: ' ' Symbol: '3'  Code: 01000001

Prior Symbol: ' ' Symbol: '9'  Code: 000000000

Prior Symbol: ' ' Symbol: 'A'  Code: 10111

Prior Symbol: ' ' Symbol: 'B'  Code: 0010

Prior Symbol: ' ' Symbol: 'C'  Code: 1100

Prior Symbol: ' ' Symbol: 'D'  Code: 11100

Prior Symbol: ' ' Symbol: 'E'  Code: 011010

Prior Symbol: ' ' Symbol: 'F'  Code: 10011

Prior Symbol: ' ' Symbol: 'G'  Code: 00001

Prior Symbol: ' ' Symbol: 'H'  Code: 10101

Prior Symbol: ' ' Symbol: 'I'  Code: 111111

Prior Symbol: ' ' Symbol: 'J'  Code: 111110

Prior Symbol: ' ' Symbol: 'K'  Code: 010011

Prior Symbol: ' ' Symbol: 'L'  Code: 11110

Prior Symbol: ' ' Symbol: 'M'  Code: 0101

Prior Symbol: ' ' Symbol: 'N'  Code: 10110

Prior Symbol: ' ' Symbol: 'O'  Code: 011011

Prior Symbol: ' ' Symbol: 'P'  Code: 11101

Prior Symbol: ' ' Symbol: 'Q'  Code: 100100011

Prior Symbol: ' ' Symbol: 'R'  Code: 10100

Prior Symbol: ' ' Symbol: 'S'  Code: 1101

Prior Symbol: ' ' Symbol: 'T'  Code: 1000

Prior Symbol: ' ' Symbol: 'U'  Code: 1001001

Prior Symbol: ' ' Symbol: 'V'  Code: 1001011

Prior Symbol: ' ' Symbol: 'W'  Code: 0011

Prior Symbol: ' ' Symbol: 'X'  Code: 0000000010

Prior Symbol: ' ' Symbol: 'Y'  Code: 000001

Prior Symbol: ' ' Symbol: 'Z'  Code: 00000011

Prior Symbol: ' ' Symbol: 'a'  Code: 01100

Prior Symbol: ' ' Symbol: 'b'  Code: 10010101

Prior Symbol: ' ' Symbol: 'c'  Code: 01000000

Prior Symbol: ' ' Symbol: 'd'  Code: 01000011

Prior Symbol: ' ' Symbol: 'e'  Code: 0000000011

Prior Symbol: ' ' Symbol: 'f'  Code: 10010000

Prior Symbol: ' ' Symbol: 'i'  Code: 010010

Prior Symbol: ' ' Symbol: 'l'  Code: 100100010

Prior Symbol: ' ' Symbol: 'o'  Code: 0001

Prior Symbol: ' ' Symbol: 't'  Code: 0111

Prior Symbol: '!' Symbol:  0  Code: 1

Prior Symbol: '!' Symbol: 27  Code: 01

Prior Symbol: '!' Symbol: ' '  Code: 00

Prior Symbol: '"' Symbol: 27  Code: 1

Prior Symbol: '#' Symbol: 27  Code: 1

Prior Symbol: '$' Symbol: 27  Code: 1

Prior Symbol: '$' Symbol: '1'  Code: 0

Prior Symbol: '%' Symbol: 27  Code: 1

Prior Symbol: '&' Symbol: 27  Code: 0

Prior Symbol: '&' Symbol: ' '  Code: 1

Prior Symbol: ''' Symbol: 27  Code: 011

Prior Symbol: ''' Symbol: ' '  Code: 010

Prior Symbol: ''' Symbol: '9'  Code: 0001

Prior Symbol: ''' Symbol: 'd'  Code: 0000

Prior Symbol: ''' Symbol: 's'  Code: 1

Prior Symbol: ''' Symbol: 't'  Code: 001

Prior Symbol: '(' Symbol: 27  Code: 1

Prior Symbol: ')' Symbol: 27  Code: 1

Prior Symbol: '*' Symbol: 27  Code: 00

Prior Symbol: '*' Symbol: 'A'  Code: 01

Prior Symbol: '*' Symbol: 'H'  Code: 10

Prior Symbol: '*' Symbol: 'S'  Code: 11

Prior Symbol: '+' Symbol: 27  Code: 1

Prior Symbol: ',' Symbol: 27  Code: 0

Prior Symbol: ',' Symbol: ' '  Code: 1

Prior Symbol: '-' Symbol: 27  Code: 01

Prior Symbol: '-' Symbol: ' '  Code: 111

Prior Symbol: '-' Symbol: '-'  Code: 1101

Prior Symbol: '-' Symbol: '1'  Code: 1000

Prior Symbol: '-' Symbol: 'A'  Code: 001

Prior Symbol: '-' Symbol: 'M'  Code: 000

Prior Symbol: '-' Symbol: 'R'  Code: 1001

Prior Symbol: '-' Symbol: 'S'  Code: 1010

Prior Symbol: '-' Symbol: 'T'  Code: 1011

Prior Symbol: '-' Symbol: 'U'  Code: 1100

Prior Symbol: '.' Symbol:  0  Code: 111

Prior Symbol: '.' Symbol: 27  Code: 101

Prior Symbol: '.' Symbol: ' '  Code: 0

Prior Symbol: '.' Symbol: '.'  Code: 110

Prior Symbol: '.' Symbol: 'I'  Code: 10010

Prior Symbol: '.' Symbol: 'S'  Code: 1000

Prior Symbol: '.' Symbol: 'W'  Code: 10011

Prior Symbol: '/' Symbol: 27  Code: 1

Prior Symbol: '0' Symbol:  0  Code: 01

Prior Symbol: '0' Symbol: 27  Code: 001

Prior Symbol: '0' Symbol: ' '  Code: 10

Prior Symbol: '0' Symbol: '-'  Code: 000

Prior Symbol: '0' Symbol: '0'  Code: 11

Prior Symbol: '1' Symbol:  0  Code: 010

Prior Symbol: '1' Symbol: 27  Code: 011

Prior Symbol: '1' Symbol: ' '  Code: 110

Prior Symbol: '1' Symbol: '0'  Code: 111

Prior Symbol: '1' Symbol: '1'  Code: 100

Prior Symbol: '1' Symbol: '2'  Code: 101

Prior Symbol: '1' Symbol: '9'  Code: 00

Prior Symbol: '2' Symbol:  0  Code: 11

Prior Symbol: '2' Symbol: 27  Code: 10

Prior Symbol: '2' Symbol: '0'  Code: 01

Prior Symbol: '2' Symbol: '1'  Code: 000

Prior Symbol: '2' Symbol: ':'  Code: 001

Prior Symbol: '3' Symbol:  0  Code: 0

Prior Symbol: '3' Symbol: 27  Code: 11

Prior Symbol: '3' Symbol: '0'  Code: 10

Prior Symbol: '4' Symbol: 27  Code: 0

Prior Symbol: '4' Symbol: '8'  Code: 1

Prior Symbol: '5' Symbol: 27  Code: 1

Prior Symbol: '6' Symbol: 27  Code: 1

Prior Symbol: '7' Symbol: 27  Code: 0

Prior Symbol: '7' Symbol: '0'  Code: 1

Prior Symbol: '8' Symbol: 27  Code: 0

Prior Symbol: '8' Symbol: ' '  Code: 1

Prior Symbol: '9' Symbol: 27  Code: 11

Prior Symbol: '9' Symbol: '0'  Code: 01

Prior Symbol: '9' Symbol: '1'  Code: 100

Prior Symbol: '9' Symbol: '3'  Code: 101

Prior Symbol: '9' Symbol: '9'  Code: 00

Prior Symbol: ':' Symbol: 27  Code: 0

Prior Symbol: ':' Symbol: ' '  Code: 1

Prior Symbol: ';' Symbol: 27  Code: 1

Prior Symbol: '<' Symbol: 27  Code: 1

Prior Symbol: '=' Symbol: 27  Code: 1

Prior Symbol: '>' Symbol: 27  Code: 1

Prior Symbol: '?' Symbol:  0  Code: 1

Prior Symbol: '?' Symbol: 27  Code: 0

Prior Symbol: '@' Symbol: 27  Code: 1

Prior Symbol: 'A' Symbol: 27  Code: 00010

Prior Symbol: 'A' Symbol: ' '  Code: 010

Prior Symbol: 'A' Symbol: '*'  Code: 1101000

Prior Symbol: 'A' Symbol: '-'  Code: 1101001

Prior Symbol: 'A' Symbol: '.'  Code: 1101010

Prior Symbol: 'A' Symbol: 'B'  Code: 110110

Prior Symbol: 'A' Symbol: 'b'  Code: 110010

Prior Symbol: 'A' Symbol: 'c'  Code: 01100

Prior Symbol: 'A' Symbol: 'd'  Code: 001

Prior Symbol: 'A' Symbol: 'f'  Code: 01101

Prior Symbol: 'A' Symbol: 'g'  Code: 011110

Prior Symbol: 'A' Symbol: 'i'  Code: 110011

Prior Symbol: 'A' Symbol: 'l'  Code: 100

Prior Symbol: 'A' Symbol: 'm'  Code: 111

Prior Symbol: 'A' Symbol: 'n'  Code: 101

Prior Symbol: 'A' Symbol: 'p'  Code: 110111

Prior Symbol: 'A' Symbol: 'r'  Code: 0000

Prior Symbol: 'A' Symbol: 's'  Code: 00011

Prior Symbol: 'A' Symbol: 't'  Code: 011111

Prior Symbol: 'A' Symbol: 'u'  Code: 11000

Prior Symbol: 'A' Symbol: 'v'  Code: 1101011

Prior Symbol: 'A' Symbol: 'w'  Code: 01110

Prior Symbol: 'B' Symbol: 27  Code: 00010

Prior Symbol: 'B' Symbol: 'A'  Code: 000110

Prior Symbol: 'B' Symbol: 'C'  Code: 0000

Prior Symbol: 'B' Symbol: 'S'  Code: 000111

Prior Symbol: 'B' Symbol: 'a'  Code: 111

Prior Symbol: 'B' Symbol: 'e'  Code: 01

Prior Symbol: 'B' Symbol: 'i'  Code: 1010

Prior Symbol: 'B' Symbol: 'l'  Code: 1011

Prior Symbol: 'B' Symbol: 'o'  Code: 110

Prior Symbol: 'B' Symbol: 'r'  Code: 001

Prior Symbol: 'B' Symbol: 'u'  Code: 100

Prior Symbol: 'C' Symbol: 27  Code: 00101

Prior Symbol: 'C' Symbol: ' '  Code: 10110

Prior Symbol: 'C' Symbol: 'A'  Code: 0011100

Prior Symbol: 'C' Symbol: 'B'  Code: 001111

Prior Symbol: 'C' Symbol: 'O'  Code: 101110

Prior Symbol: 'C' Symbol: 'a'  Code: 100

Prior Symbol: 'C' Symbol: 'e'  Code: 101111

Prior Symbol: 'C' Symbol: 'h'  Code: 01

Prior Symbol: 'C' Symbol: 'i'  Code: 00110

Prior Symbol: 'C' Symbol: 'l'  Code: 000

Prior Symbol: 'C' Symbol: 'o'  Code: 11

Prior Symbol: 'C' Symbol: 'r'  Code: 1010

Prior Symbol: 'C' Symbol: 'u'  Code: 00100

Prior Symbol: 'C' Symbol: 'y'  Code: 0011101

Prior Symbol: 'D' Symbol: 27  Code: 01001

Prior Symbol: 'D' Symbol: 'a'  Code: 10

Prior Symbol: 'D' Symbol: 'e'  Code: 111

Prior Symbol: 'D' Symbol: 'i'  Code: 110

Prior Symbol: 'D' Symbol: 'o'  Code: 00

Prior Symbol: 'D' Symbol: 'r'  Code: 011

Prior Symbol: 'D' Symbol: 'u'  Code: 0101

Prior Symbol: 'D' Symbol: 'y'  Code: 01000

Prior Symbol: 'E' Symbol: 27  Code: 011

Prior Symbol: 'E' Symbol: 'C'  Code: 1010

Prior Symbol: 'E' Symbol: 'a'  Code: 111

Prior Symbol: 'E' Symbol: 'd'  Code: 000

Prior Symbol: 'E' Symbol: 'l'  Code: 1100

Prior Symbol: 'E' Symbol: 'm'  Code: 0100

Prior Symbol: 'E' Symbol: 'n'  Code: 1101

Prior Symbol: 'E' Symbol: 'q'  Code: 101110

Prior Symbol: 'E' Symbol: 's'  Code: 10110

Prior Symbol: 'E' Symbol: 'u'  Code: 101111

Prior Symbol: 'E' Symbol: 'v'  Code: 100

Prior Symbol: 'E' Symbol: 'x'  Code: 001

Prior Symbol: 'E' Symbol: 'y'  Code: 0101

Prior Symbol: 'F' Symbol: 27  Code: 011111

Prior Symbol: 'F' Symbol: ' '  Code: 011110

Prior Symbol: 'F' Symbol: 'L'  Code: 01110

Prior Symbol: 'F' Symbol: 'a'  Code: 10

Prior Symbol: 'F' Symbol: 'e'  Code: 0110

Prior Symbol: 'F' Symbol: 'i'  Code: 110

Prior Symbol: 'F' Symbol: 'l'  Code: 000

Prior Symbol: 'F' Symbol: 'o'  Code: 010

Prior Symbol: 'F' Symbol: 'r'  Code: 111

Prior Symbol: 'F' Symbol: 'u'  Code: 001

Prior Symbol: 'G' Symbol: 27  Code: 10110

Prior Symbol: 'G' Symbol: '.'  Code: 101010

Prior Symbol: 'G' Symbol: 'A'  Code: 101111

Prior Symbol: 'G' Symbol: 'a'  Code: 1110

Prior Symbol: 'G' Symbol: 'e'  Code: 110

Prior Symbol: 'G' Symbol: 'h'  Code: 10100

Prior Symbol: 'G' Symbol: 'i'  Code: 100

Prior Symbol: 'G' Symbol: 'l'  Code: 101011

Prior Symbol: 'G' Symbol: 'o'  Code: 01

Prior Symbol: 'G' Symbol: 'r'  Code: 00

Prior Symbol: 'G' Symbol: 'u'  Code: 1111

Prior Symbol: 'G' Symbol: 'y'  Code: 101110

Prior Symbol: 'H' Symbol:  0  Code: 111010

Prior Symbol: 'H' Symbol: 27  Code: 111011

Prior Symbol: 'H' Symbol: 'a'  Code: 110

Prior Symbol: 'H' Symbol: 'e'  Code: 10

Prior Symbol: 'H' Symbol: 'i'  Code: 1111

Prior Symbol: 'H' Symbol: 'o'  Code: 0

Prior Symbol: 'H' Symbol: 'u'  Code: 11100

Prior Symbol: 'I' Symbol:  0  Code: 1000

Prior Symbol: 'I' Symbol: 27  Code: 1001

Prior Symbol: 'I' Symbol: ' '  Code: 11110

Prior Symbol: 'I' Symbol: '.'  Code: 111110

Prior Symbol: 'I' Symbol: ':'  Code: 101110

Prior Symbol: 'I' Symbol: 'I'  Code: 1100

Prior Symbol: 'I' Symbol: 'T'  Code: 101111

Prior Symbol: 'I' Symbol: 'c'  Code: 10110

Prior Symbol: 'I' Symbol: 'm'  Code: 1010

Prior Symbol: 'I' Symbol: 'n'  Code: 0

Prior Symbol: 'I' Symbol: 'r'  Code: 111111

Prior Symbol: 'I' Symbol: 's'  Code: 1101

Prior Symbol: 'I' Symbol: 't'  Code: 1110

Prior Symbol: 'J' Symbol: 27  Code: 000

Prior Symbol: 'J' Symbol: 'a'  Code: 01

Prior Symbol: 'J' Symbol: 'e'  Code: 11

Prior Symbol: 'J' Symbol: 'o'  Code: 10

Prior Symbol: 'J' Symbol: 'u'  Code: 001

Prior Symbol: 'K' Symbol: 27  Code: 000

Prior Symbol: 'K' Symbol: 'a'  Code: 0100

Prior Symbol: 'K' Symbol: 'e'  Code: 001

Prior Symbol: 'K' Symbol: 'i'  Code: 1

Prior Symbol: 'K' Symbol: 'n'  Code: 0111

Prior Symbol: 'K' Symbol: 'o'  Code: 0101

Prior Symbol: 'K' Symbol: 'u'  Code: 0110

Prior Symbol: 'L' Symbol: 27  Code: 01001

Prior Symbol: 'L' Symbol: ' '  Code: 01000

Prior Symbol: 'L' Symbol: 'a'  Code: 10

Prior Symbol: 'L' Symbol: 'e'  Code: 011

Prior Symbol: 'L' Symbol: 'i'  Code: 11

Prior Symbol: 'L' Symbol: 'o'  Code: 00

Prior Symbol: 'L' Symbol: 'u'  Code: 0101

Prior Symbol: 'M' Symbol: 27  Code: 1011111

Prior Symbol: 'M' Symbol: '*'  Code: 10111100

Prior Symbol: 'M' Symbol: 'T'  Code: 10111101

Prior Symbol: 'M' Symbol: 'a'  Code: 11

Prior Symbol: 'M' Symbol: 'c'  Code: 101110

Prior Symbol: 'M' Symbol: 'e'  Code: 1010

Prior Symbol: 'M' Symbol: 'i'  Code: 100

Prior Symbol: 'M' Symbol: 'o'  Code: 00

Prior Symbol: 'M' Symbol: 'r'  Code: 10110

Prior Symbol: 'M' Symbol: 'u'  Code: 010

Prior Symbol: 'M' Symbol: 'y'  Code: 011

Prior Symbol: 'N' Symbol: 27  Code: 1000

Prior Symbol: 'N' Symbol: ' '  Code: 110001

Prior Symbol: 'N' Symbol: 'B'  Code: 1001

Prior Symbol: 'N' Symbol: 'F'  Code: 110010

Prior Symbol: 'N' Symbol: 'N'  Code: 110000

Prior Symbol: 'N' Symbol: 'a'  Code: 1101

Prior Symbol: 'N' Symbol: 'e'  Code: 0

Prior Symbol: 'N' Symbol: 'i'  Code: 111

Prior Symbol: 'N' Symbol: 'o'  Code: 101

Prior Symbol: 'N' Symbol: 'u'  Code: 110011

Prior Symbol: 'O' Symbol: 27  Code: 010

Prior Symbol: 'O' Symbol: ' '  Code: 001

Prior Symbol: 'O' Symbol: 'd'  Code: 01110

Prior Symbol: 'O' Symbol: 'f'  Code: 11010

Prior Symbol: 'O' Symbol: 'l'  Code: 1100

Prior Symbol: 'O' Symbol: 'n'  Code: 10

Prior Symbol: 'O' Symbol: 'p'  Code: 0001

Prior Symbol: 'O' Symbol: 'r'  Code: 0110

Prior Symbol: 'O' Symbol: 's'  Code: 01111

Prior Symbol: 'O' Symbol: 'u'  Code: 111

Prior Symbol: 'O' Symbol: 'v'  Code: 11011

Prior Symbol: 'O' Symbol: 'w'  Code: 0000

Prior Symbol: 'P' Symbol: 27  Code: 111111

Prior Symbol: 'P' Symbol: ' '  Code: 1111100

Prior Symbol: 'P' Symbol: '.'  Code: 011001

Prior Symbol: 'P' Symbol: 'G'  Code: 111101

Prior Symbol: 'P' Symbol: 'R'  Code: 111100

Prior Symbol: 'P' Symbol: 'a'  Code: 00

Prior Symbol: 'P' Symbol: 'e'  Code: 010

Prior Symbol: 'P' Symbol: 'i'  Code: 0111

Prior Symbol: 'P' Symbol: 'l'  Code: 1110

Prior Symbol: 'P' Symbol: 'o'  Code: 110

Prior Symbol: 'P' Symbol: 'r'  Code: 10

Prior Symbol: 'P' Symbol: 's'  Code: 1111101

Prior Symbol: 'P' Symbol: 'u'  Code: 01101

Prior Symbol: 'P' Symbol: 'y'  Code: 011000

Prior Symbol: 'Q' Symbol: 27  Code: 00

Prior Symbol: 'Q' Symbol: 'V'  Code: 01

Prior Symbol: 'Q' Symbol: 'u'  Code: 1

Prior Symbol: 'R' Symbol: 27  Code: 10001

Prior Symbol: 'R' Symbol: 'a'  Code: 101

Prior Symbol: 'R' Symbol: 'e'  Code: 11

Prior Symbol: 'R' Symbol: 'h'  Code: 10000

Prior Symbol: 'R' Symbol: 'i'  Code: 00

Prior Symbol: 'R' Symbol: 'o'  Code: 01

Prior Symbol: 'R' Symbol: 'u'  Code: 1001

Prior Symbol: 'S' Symbol: 27  Code: 101110

Prior Symbol: 'S' Symbol: ' '  Code: 1110100

Prior Symbol: 'S' Symbol: '*'  Code: 1011000

Prior Symbol: 'S' Symbol: '.'  Code: 1011011

Prior Symbol: 'S' Symbol: 'a'  Code: 1111

Prior Symbol: 'S' Symbol: 'c'  Code: 11100

Prior Symbol: 'S' Symbol: 'e'  Code: 000

Prior Symbol: 'S' Symbol: 'h'  Code: 100

Prior Symbol: 'S' Symbol: 'i'  Code: 1100

Prior Symbol: 'S' Symbol: 'k'  Code: 101111

Prior Symbol: 'S' Symbol: 'l'  Code: 1011001

Prior Symbol: 'S' Symbol: 'm'  Code: 1110110

Prior Symbol: 'S' Symbol: 'n'  Code: 1110111

Prior Symbol: 'S' Symbol: 'o'  Code: 1010

Prior Symbol: 'S' Symbol: 'p'  Code: 001

Prior Symbol: 'S' Symbol: 'q'  Code: 1011010

Prior Symbol: 'S' Symbol: 't'  Code: 01

Prior Symbol: 'S' Symbol: 'u'  Code: 1101

Prior Symbol: 'S' Symbol: 'w'  Code: 1110101

Prior Symbol: 'T' Symbol: 27  Code: 1111010

Prior Symbol: 'T' Symbol: '-'  Code: 11110110

Prior Symbol: 'T' Symbol: 'N'  Code: 11110111

Prior Symbol: 'T' Symbol: 'V'  Code: 111100

Prior Symbol: 'T' Symbol: 'a'  Code: 1010

Prior Symbol: 'T' Symbol: 'e'  Code: 1011

Prior Symbol: 'T' Symbol: 'h'  Code: 0

Prior Symbol: 'T' Symbol: 'i'  Code: 1110

Prior Symbol: 'T' Symbol: 'o'  Code: 110

Prior Symbol: 'T' Symbol: 'r'  Code: 100

Prior Symbol: 'T' Symbol: 'u'  Code: 111110

Prior Symbol: 'T' Symbol: 'w'  Code: 111111

Prior Symbol: 'U' Symbol: 27  Code: 101

Prior Symbol: 'U' Symbol: '.'  Code: 1001

Prior Symbol: 'U' Symbol: 'l'  Code: 1000

Prior Symbol: 'U' Symbol: 'n'  Code: 0

Prior Symbol: 'U' Symbol: 'p'  Code: 11

Prior Symbol: 'V' Symbol:  0  Code: 000

Prior Symbol: 'V' Symbol: 27  Code: 0011

Prior Symbol: 'V' Symbol: ' '  Code: 01010

Prior Symbol: 'V' Symbol: 'C'  Code: 01011

Prior Symbol: 'V' Symbol: 'a'  Code: 011

Prior Symbol: 'V' Symbol: 'e'  Code: 0100

Prior Symbol: 'V' Symbol: 'i'  Code: 1

Prior Symbol: 'V' Symbol: 'o'  Code: 0010

Prior Symbol: 'W' Symbol: 27  Code: 00011

Prior Symbol: 'W' Symbol: 'F'  Code: 000100

Prior Symbol: 'W' Symbol: 'W'  Code: 000101

Prior Symbol: 'W' Symbol: 'a'  Code: 111

Prior Symbol: 'W' Symbol: 'e'  Code: 110

Prior Symbol: 'W' Symbol: 'h'  Code: 001

Prior Symbol: 'W' Symbol: 'i'  Code: 01

Prior Symbol: 'W' Symbol: 'o'  Code: 10

Prior Symbol: 'W' Symbol: 'r'  Code: 0000

Prior Symbol: 'X' Symbol: 27  Code: 1

Prior Symbol: 'Y' Symbol: 27  Code: 001

Prior Symbol: 'Y' Symbol: 'a'  Code: 000

Prior Symbol: 'Y' Symbol: 'e'  Code: 01

Prior Symbol: 'Y' Symbol: 'o'  Code: 1

Prior Symbol: 'Z' Symbol: 27  Code: 00

Prior Symbol: 'Z' Symbol: 'a'  Code: 01

Prior Symbol: 'Z' Symbol: 'o'  Code: 1

Prior Symbol: '[' Symbol: 27  Code: 1

Prior Symbol: '\' Symbol: 27  Code: 1

Prior Symbol: ']' Symbol: 27  Code: 1

Prior Symbol: '^' Symbol: 27  Code: 1

Prior Symbol: '_' Symbol: 27  Code: 1

Prior Symbol: '`' Symbol: 27  Code: 1

Prior Symbol: 'a' Symbol:  0  Code: 00010

Prior Symbol: 'a' Symbol: 27  Code: 1111010110

Prior Symbol: 'a' Symbol: ' '  Code: 10110

Prior Symbol: 'a' Symbol: '''  Code: 11110100

Prior Symbol: 'a' Symbol: ':'  Code: 1111010111

Prior Symbol: 'a' Symbol: 'b'  Code: 010010

Prior Symbol: 'a' Symbol: 'c'  Code: 11111

Prior Symbol: 'a' Symbol: 'd'  Code: 10100

Prior Symbol: 'a' Symbol: 'e'  Code: 101011000

Prior Symbol: 'a' Symbol: 'f'  Code: 10101101

Prior Symbol: 'a' Symbol: 'g'  Code: 01000

Prior Symbol: 'a' Symbol: 'h'  Code: 0100111

Prior Symbol: 'a' Symbol: 'i'  Code: 10111

Prior Symbol: 'a' Symbol: 'j'  Code: 101011001

Prior Symbol: 'a' Symbol: 'k'  Code: 101010

Prior Symbol: 'a' Symbol: 'l'  Code: 001

Prior Symbol: 'a' Symbol: 'm'  Code: 0101

Prior Symbol: 'a' Symbol: 'n'  Code: 110

Prior Symbol: 'a' Symbol: 'p'  Code: 111100

Prior Symbol: 'a' Symbol: 'r'  Code: 100

Prior Symbol: 'a' Symbol: 's'  Code: 1110

Prior Symbol: 'a' Symbol: 't'  Code: 011

Prior Symbol: 'a' Symbol: 'u'  Code: 1111011

Prior Symbol: 'a' Symbol: 'v'  Code: 00011

Prior Symbol: 'a' Symbol: 'w'  Code: 1010111

Prior Symbol: 'a' Symbol: 'x'  Code: 111101010

Prior Symbol: 'a' Symbol: 'y'  Code: 0000

Prior Symbol: 'a' Symbol: 'z'  Code: 0100110

Prior Symbol: 'b' Symbol:  0  Code: 11111

Prior Symbol: 'b' Symbol: 27  Code: 111101

Prior Symbol: 'b' Symbol: ' '  Code: 0110

Prior Symbol: 'b' Symbol: 'a'  Code: 00

Prior Symbol: 'b' Symbol: 'b'  Code: 01111

Prior Symbol: 'b' Symbol: 'e'  Code: 1010

Prior Symbol: 'b' Symbol: 'i'  Code: 1110

Prior Symbol: 'b' Symbol: 'l'  Code: 010

Prior Symbol: 'b' Symbol: 'o'  Code: 110

Prior Symbol: 'b' Symbol: 'r'  Code: 1011

Prior Symbol: 'b' Symbol: 's'  Code: 111100

Prior Symbol: 'b' Symbol: 'u'  Code: 01110

Prior Symbol: 'b' Symbol: 'y'  Code: 100

Prior Symbol: 'c' Symbol:  0  Code: 010110

Prior Symbol: 'c' Symbol: 27  Code: 1000011

Prior Symbol: 'c' Symbol: ' '  Code: 0100

Prior Symbol: 'c' Symbol: 'C'  Code: 0010110

Prior Symbol: 'c' Symbol: 'G'  Code: 1000010

Prior Symbol: 'c' Symbol: 'L'  Code: 0010111

Prior Symbol: 'c' Symbol: 'a'  Code: 011

Prior Symbol: 'c' Symbol: 'c'  Code: 001010

Prior Symbol: 'c' Symbol: 'e'  Code: 111

Prior Symbol: 'c' Symbol: 'h'  Code: 101

Prior Symbol: 'c' Symbol: 'i'  Code: 0011

Prior Symbol: 'c' Symbol: 'k'  Code: 110

Prior Symbol: 'c' Symbol: 'l'  Code: 010111

Prior Symbol: 'c' Symbol: 'o'  Code: 1001

Prior Symbol: 'c' Symbol: 'r'  Code: 10001

Prior Symbol: 'c' Symbol: 's'  Code: 00100

Prior Symbol: 'c' Symbol: 't'  Code: 000

Prior Symbol: 'c' Symbol: 'u'  Code: 01010

Prior Symbol: 'c' Symbol: 'y'  Code: 100000

Prior Symbol: 'd' Symbol:  0  Code: 011

Prior Symbol: 'd' Symbol: 27  Code: 101110

Prior Symbol: 'd' Symbol: ' '  Code: 11

Prior Symbol: 'd' Symbol: '.'  Code: 101101110

Prior Symbol: 'd' Symbol: 'a'  Code: 1010

Prior Symbol: 'd' Symbol: 'd'  Code: 100000

Prior Symbol: 'd' Symbol: 'e'  Code: 00

Prior Symbol: 'd' Symbol: 'g'  Code: 100001

Prior Symbol: 'd' Symbol: 'i'  Code: 1001

Prior Symbol: 'd' Symbol: 'l'  Code: 1011010

Prior Symbol: 'd' Symbol: 'o'  Code: 101111

Prior Symbol: 'd' Symbol: 'r'  Code: 101100

Prior Symbol: 'd' Symbol: 's'  Code: 0101

Prior Symbol: 'd' Symbol: 'u'  Code: 101101111

Prior Symbol: 'd' Symbol: 'v'  Code: 10001

Prior Symbol: 'd' Symbol: 'w'  Code: 10110110

Prior Symbol: 'd' Symbol: 'y'  Code: 0100

Prior Symbol: 'e' Symbol:  0  Code: 001

Prior Symbol: 'e' Symbol: 27  Code: 1010111100

Prior Symbol: 'e' Symbol: ' '  Code: 01

Prior Symbol: 'e' Symbol: '!'  Code: 1010111101

Prior Symbol: 'e' Symbol: '''  Code: 10101100

Prior Symbol: 'e' Symbol: '-'  Code: 1010111110

Prior Symbol: 'e' Symbol: ':'  Code: 00010010

Prior Symbol: 'e' Symbol: 'a'  Code: 1000

Prior Symbol: 'e' Symbol: 'b'  Code: 10101101

Prior Symbol: 'e' Symbol: 'c'  Code: 100111

Prior Symbol: 'e' Symbol: 'd'  Code: 00011

Prior Symbol: 'e' Symbol: 'e'  Code: 10100

Prior Symbol: 'e' Symbol: 'f'  Code: 1001100

Prior Symbol: 'e' Symbol: 'g'  Code: 1010100

Prior Symbol: 'e' Symbol: 'h'  Code: 1010111111

Prior Symbol: 'e' Symbol: 'i'  Code: 10101110

Prior Symbol: 'e' Symbol: 'j'  Code: 000100000

Prior Symbol: 'e' Symbol: 'k'  Code: 1010101

Prior Symbol: 'e' Symbol: 'l'  Code: 10010

Prior Symbol: 'e' Symbol: 'm'  Code: 1001101

Prior Symbol: 'e' Symbol: 'n'  Code: 1110

Prior Symbol: 'e' Symbol: 'o'  Code: 000101

Prior Symbol: 'e' Symbol: 'p'  Code: 000001

Prior Symbol: 'e' Symbol: 'q'  Code: 000100001

Prior Symbol: 'e' Symbol: 'r'  Code: 110

Prior Symbol: 'e' Symbol: 's'  Code: 1111

Prior Symbol: 'e' Symbol: 't'  Code: 10110

Prior Symbol: 'e' Symbol: 'u'  Code: 000100010

Prior Symbol: 'e' Symbol: 'v'  Code: 000000

Prior Symbol: 'e' Symbol: 'w'  Code: 10111

Prior Symbol: 'e' Symbol: 'x'  Code: 00010011

Prior Symbol: 'e' Symbol: 'y'  Code: 00001

Prior Symbol: 'e' Symbol: 'z'  Code: 000100011

Prior Symbol: 'f' Symbol:  0  Code: 11100

Prior Symbol: 'f' Symbol: 27  Code: 1111001

Prior Symbol: 'f' Symbol: ' '  Code: 0

Prior Symbol: 'f' Symbol: 'a'  Code: 11101

Prior Symbol: 'f' Symbol: 'e'  Code: 110

Prior Symbol: 'f' Symbol: 'f'  Code: 1011

Prior Symbol: 'f' Symbol: 'i'  Code: 1001

Prior Symbol: 'f' Symbol: 'l'  Code: 111101

Prior Symbol: 'f' Symbol: 'o'  Code: 1010

Prior Symbol: 'f' Symbol: 'r'  Code: 111111

Prior Symbol: 'f' Symbol: 's'  Code: 111110

Prior Symbol: 'f' Symbol: 't'  Code: 1000

Prior Symbol: 'f' Symbol: 'u'  Code: 1111000

Prior Symbol: 'g' Symbol:  0  Code: 110

Prior Symbol: 'g' Symbol: 27  Code: 1110000

Prior Symbol: 'g' Symbol: ' '  Code: 01

Prior Symbol: 'g' Symbol: '''  Code: 1001100

Prior Symbol: 'g' Symbol: ':'  Code: 11100010

Prior Symbol: 'g' Symbol: 'a'  Code: 1000

Prior Symbol: 'g' Symbol: 'e'  Code: 101

Prior Symbol: 'g' Symbol: 'g'  Code: 1111010

Prior Symbol: 'g' Symbol: 'h'  Code: 00

Prior Symbol: 'g' Symbol: 'i'  Code: 11101

Prior Symbol: 'g' Symbol: 'l'  Code: 1111011

Prior Symbol: 'g' Symbol: 'n'  Code: 100111

Prior Symbol: 'g' Symbol: 'o'  Code: 111001

Prior Symbol: 'g' Symbol: 'r'  Code: 10010

Prior Symbol: 'g' Symbol: 's'  Code: 11111

Prior Symbol: 'g' Symbol: 't'  Code: 1001101

Prior Symbol: 'g' Symbol: 'u'  Code: 111100

Prior Symbol: 'g' Symbol: 'y'  Code: 11100011

Prior Symbol: 'h' Symbol:  0  Code: 11101

Prior Symbol: 'h' Symbol: 27  Code: 1110001

Prior Symbol: 'h' Symbol: ' '  Code: 1011

Prior Symbol: 'h' Symbol: 'a'  Code: 1100

Prior Symbol: 'h' Symbol: 'b'  Code: 11100110

Prior Symbol: 'h' Symbol: 'e'  Code: 0

Prior Symbol: 'h' Symbol: 'i'  Code: 100

Prior Symbol: 'h' Symbol: 'l'  Code: 1110010

Prior Symbol: 'h' Symbol: 'n'  Code: 101001

Prior Symbol: 'h' Symbol: 'o'  Code: 1101

Prior Symbol: 'h' Symbol: 'r'  Code: 10101

Prior Symbol: 'h' Symbol: 't'  Code: 1111

Prior Symbol: 'h' Symbol: 'u'  Code: 11100111

Prior Symbol: 'h' Symbol: 'w'  Code: 1110000

Prior Symbol: 'h' Symbol: 'y'  Code: 101000

Prior Symbol: 'i' Symbol:  0  Code: 00110101

Prior Symbol: 'i' Symbol: 27  Code: 00110110

Prior Symbol: 'i' Symbol: ' '  Code: 000100

Prior Symbol: 'i' Symbol: '!'  Code: 001101000

Prior Symbol: 'i' Symbol: 'a'  Code: 00011

Prior Symbol: 'i' Symbol: 'b'  Code: 0011000

Prior Symbol: 'i' Symbol: 'c'  Code: 1111

Prior Symbol: 'i' Symbol: 'd'  Code: 0010

Prior Symbol: 'i' Symbol: 'e'  Code: 1101

Prior Symbol: 'i' Symbol: 'f'  Code: 00111

Prior Symbol: 'i' Symbol: 'g'  Code: 1100

Prior Symbol: 'i' Symbol: 'i'  Code: 00110010

Prior Symbol: 'i' Symbol: 'k'  Code: 00110011

Prior Symbol: 'i' Symbol: 'l'  Code: 0110

Prior Symbol: 'i' Symbol: 'm'  Code: 11101

Prior Symbol: 'i' Symbol: 'n'  Code: 10

Prior Symbol: 'i' Symbol: 'o'  Code: 0100

Prior Symbol: 'i' Symbol: 'p'  Code: 000101

Prior Symbol: 'i' Symbol: 'r'  Code: 11100

Prior Symbol: 'i' Symbol: 's'  Code: 0111

Prior Symbol: 'i' Symbol: 't'  Code: 0101

Prior Symbol: 'i' Symbol: 'v'  Code: 0000

Prior Symbol: 'i' Symbol: 'x'  Code: 001101001

Prior Symbol: 'i' Symbol: 'z'  Code: 00110111

Prior Symbol: 'j' Symbol: 27  Code: 10

Prior Symbol: 'j' Symbol: 'a'  Code: 11

Prior Symbol: 'j' Symbol: 'o'  Code: 0

Prior Symbol: 'k' Symbol:  0  Code: 01

Prior Symbol: 'k' Symbol: 27  Code: 00011

Prior Symbol: 'k' Symbol: ' '  Code: 111

Prior Symbol: 'k' Symbol: ':'  Code: 00001

Prior Symbol: 'k' Symbol: 'T'  Code: 000000

Prior Symbol: 'k' Symbol: 'a'  Code: 001111

Prior Symbol: 'k' Symbol: 'e'  Code: 10

Prior Symbol: 'k' Symbol: 'f'  Code: 000100

Prior Symbol: 'k' Symbol: 'i'  Code: 110

Prior Symbol: 'k' Symbol: 'l'  Code: 000101

Prior Symbol: 'k' Symbol: 'o'  Code: 000001

Prior Symbol: 'k' Symbol: 's'  Code: 0010

Prior Symbol: 'k' Symbol: 'w'  Code: 001110

Prior Symbol: 'k' Symbol: 'y'  Code: 00110

Prior Symbol: 'l' Symbol:  0  Code: 1000

Prior Symbol: 'l' Symbol: 27  Code: 0111001

Prior Symbol: 'l' Symbol: ' '  Code: 010

Prior Symbol: 'l' Symbol: '''  Code: 01100010

Prior Symbol: 'l' Symbol: '-'  Code: 11110011

Prior Symbol: 'l' Symbol: ':'  Code: 01100011

Prior Symbol: 'l' Symbol: 'a'  Code: 1110

Prior Symbol: 'l' Symbol: 'b'  Code: 0110000

Prior Symbol: 'l' Symbol: 'c'  Code: 01110000

Prior Symbol: 'l' Symbol: 'd'  Code: 000

Prior Symbol: 'l' Symbol: 'e'  Code: 110

Prior Symbol: 'l' Symbol: 'f'  Code: 1111000

Prior Symbol: 'l' Symbol: 'i'  Code: 001

Prior Symbol: 'l' Symbol: 'k'  Code: 011001

Prior Symbol: 'l' Symbol: 'l'  Code: 101

Prior Symbol: 'l' Symbol: 'm'  Code: 1111010

Prior Symbol: 'l' Symbol: 'o'  Code: 11111

Prior Symbol: 'l' Symbol: 'r'  Code: 11110010

Prior Symbol: 'l' Symbol: 's'  Code: 01101

Prior Symbol: 'l' Symbol: 't'  Code: 011101

Prior Symbol: 'l' Symbol: 'u'  Code: 01111

Prior Symbol: 'l' Symbol: 'v'  Code: 1111011

Prior Symbol: 'l' Symbol: 'w'  Code: 01110001

Prior Symbol: 'l' Symbol: 'y'  Code: 1001

Prior Symbol: 'm' Symbol:  0  Code: 0100

Prior Symbol: 'm' Symbol: 27  Code: 010101

Prior Symbol: 'm' Symbol: ' '  Code: 001

Prior Symbol: 'm' Symbol: 'a'  Code: 101

Prior Symbol: 'm' Symbol: 'b'  Code: 0000

Prior Symbol: 'm' Symbol: 'e'  Code: 11

Prior Symbol: 'm' Symbol: 'i'  Code: 011

Prior Symbol: 'm' Symbol: 'm'  Code: 0001

Prior Symbol: 'm' Symbol: 'o'  Code: 1001

Prior Symbol: 'm' Symbol: 'p'  Code: 1000

Prior Symbol: 'm' Symbol: 's'  Code: 010111

Prior Symbol: 'm' Symbol: 'u'  Code: 010110

Prior Symbol: 'm' Symbol: 'y'  Code: 010100

Prior Symbol: 'n' Symbol:  0  Code: 000

Prior Symbol: 'n' Symbol: 27  Code: 01110011

Prior Symbol: 'n' Symbol: ' '  Code: 110

Prior Symbol: 'n' Symbol: '''  Code: 011101

Prior Symbol: 'n' Symbol: ':'  Code: 1001010

Prior Symbol: 'n' Symbol: 'a'  Code: 11100

Prior Symbol: 'n' Symbol: 'b'  Code: 111010000

Prior Symbol: 'n' Symbol: 'c'  Code: 01111

Prior Symbol: 'n' Symbol: 'd'  Code: 001

Prior Symbol: 'n' Symbol: 'e'  Code: 010

Prior Symbol: 'n' Symbol: 'f'  Code: 1001011

Prior Symbol: 'n' Symbol: 'g'  Code: 101

Prior Symbol: 'n' Symbol: 'h'  Code: 111010101

Prior Symbol: 'n' Symbol: 'i'  Code: 1000

Prior Symbol: 'n' Symbol: 'j'  Code: 111010001

Prior Symbol: 'n' Symbol: 'k'  Code: 1110110

Prior Symbol: 'n' Symbol: 'l'  Code: 111010110

Prior Symbol: 'n' Symbol: 'm'  Code: 111010111

Prior Symbol: 'n' Symbol: 'n'  Code: 10011

Prior Symbol: 'n' Symbol: 'o'  Code: 1110111

Prior Symbol: 'n' Symbol: 'r'  Code: 111010100

Prior Symbol: 'n' Symbol: 's'  Code: 0110

Prior Symbol: 'n' Symbol: 't'  Code: 1111

Prior Symbol: 'n' Symbol: 'u'  Code: 11101001

Prior Symbol: 'n' Symbol: 'v'  Code: 0111000

Prior Symbol: 'n' Symbol: 'y'  Code: 100100

Prior Symbol: 'n' Symbol: 'z'  Code: 01110010

Prior Symbol: 'o' Symbol:  0  Code: 00101

Prior Symbol: 'o' Symbol: 27  Code: 01110001

Prior Symbol: 'o' Symbol: ' '  Code: 0101

Prior Symbol: 'o' Symbol: '''  Code: 01110000

Prior Symbol: 'o' Symbol: '.'  Code: 0111011010

Prior Symbol: 'o' Symbol: '?'  Code: 011101100

Prior Symbol: 'o' Symbol: 'a'  Code: 1100010

Prior Symbol: 'o' Symbol: 'b'  Code: 001001

Prior Symbol: 'o' Symbol: 'c'  Code: 110000

Prior Symbol: 'o' Symbol: 'd'  Code: 01111

Prior Symbol: 'o' Symbol: 'e'  Code: 0111001

Prior Symbol: 'o' Symbol: 'f'  Code: 1001

Prior Symbol: 'o' Symbol: 'g'  Code: 00010

Prior Symbol: 'o' Symbol: 'h'  Code: 0111010

Prior Symbol: 'o' Symbol: 'i'  Code: 01110111

Prior Symbol: 'o' Symbol: 'k'  Code: 1100011

Prior Symbol: 'o' Symbol: 'l'  Code: 0100

Prior Symbol: 'o' Symbol: 'm'  Code: 1000

Prior Symbol: 'o' Symbol: 'n'  Code: 111

Prior Symbol: 'o' Symbol: 'o'  Code: 0011

Prior Symbol: 'o' Symbol: 'p'  Code: 01101

Prior Symbol: 'o' Symbol: 'r'  Code: 101

Prior Symbol: 'o' Symbol: 's'  Code: 11001

Prior Symbol: 'o' Symbol: 't'  Code: 00011

Prior Symbol: 'o' Symbol: 'u'  Code: 1101

Prior Symbol: 'o' Symbol: 'v'  Code: 01100

Prior Symbol: 'o' Symbol: 'w'  Code: 0000

Prior Symbol: 'o' Symbol: 'x'  Code: 0010000

Prior Symbol: 'o' Symbol: 'y'  Code: 0010001

Prior Symbol: 'o' Symbol: 'z'  Code: 0111011011

Prior Symbol: 'p' Symbol:  0  Code: 1101

Prior Symbol: 'p' Symbol: 27  Code: 101110

Prior Symbol: 'p' Symbol: ' '  Code: 010

Prior Symbol: 'p' Symbol: '''  Code: 1100101

Prior Symbol: 'p' Symbol: 'a'  Code: 1001

Prior Symbol: 'p' Symbol: 'd'  Code: 101111

Prior Symbol: 'p' Symbol: 'e'  Code: 111

Prior Symbol: 'p' Symbol: 'h'  Code: 11000

Prior Symbol: 'p' Symbol: 'i'  Code: 1010

Prior Symbol: 'p' Symbol: 'l'  Code: 0110

Prior Symbol: 'p' Symbol: 'm'  Code: 1100100

Prior Symbol: 'p' Symbol: 'o'  Code: 00

Prior Symbol: 'p' Symbol: 'p'  Code: 0111

Prior Symbol: 'p' Symbol: 'r'  Code: 10001

Prior Symbol: 'p' Symbol: 's'  Code: 10000

Prior Symbol: 'p' Symbol: 't'  Code: 10110

Prior Symbol: 'p' Symbol: 'y'  Code: 110011

Prior Symbol: 'q' Symbol: 27  Code: 0

Prior Symbol: 'q' Symbol: 'u'  Code: 1

Prior Symbol: 'r' Symbol:  0  Code: 1001

Prior Symbol: 'r' Symbol: 27  Code: 01100101

Prior Symbol: 'r' Symbol: ' '  Code: 1111

Prior Symbol: 'r' Symbol: '''  Code: 0110011

Prior Symbol: 'r' Symbol: ','  Code: 110011101

Prior Symbol: 'r' Symbol: '.'  Code: 0111100

Prior Symbol: 'r' Symbol: ':'  Code: 110011100

Prior Symbol: 'r' Symbol: 'a'  Code: 000

Prior Symbol: 'r' Symbol: 'b'  Code: 01111101

Prior Symbol: 'r' Symbol: 'c'  Code: 0111111

Prior Symbol: 'r' Symbol: 'd'  Code: 11000

Prior Symbol: 'r' Symbol: 'e'  Code: 101

Prior Symbol: 'r' Symbol: 'f'  Code: 11001111

Prior Symbol: 'r' Symbol: 'g'  Code: 0111101

Prior Symbol: 'r' Symbol: 'i'  Code: 010

Prior Symbol: 'r' Symbol: 'k'  Code: 110010

Prior Symbol: 'r' Symbol: 'l'  Code: 0011

Prior Symbol: 'r' Symbol: 'm'  Code: 011000

Prior Symbol: 'r' Symbol: 'n'  Code: 01101

Prior Symbol: 'r' Symbol: 'o'  Code: 1101

Prior Symbol: 'r' Symbol: 'p'  Code: 01111100

Prior Symbol: 'r' Symbol: 'r'  Code: 01110

Prior Symbol: 'r' Symbol: 's'  Code: 1110

Prior Symbol: 'r' Symbol: 't'  Code: 1000

Prior Symbol: 'r' Symbol: 'u'  Code: 1100110

Prior Symbol: 'r' Symbol: 'v'  Code: 01100100

Prior Symbol: 'r' Symbol: 'y'  Code: 0010

Prior Symbol: 's' Symbol:  0  Code: 11

Prior Symbol: 's' Symbol: 27  Code: 0010011

Prior Symbol: 's' Symbol: ' '  Code: 01

Prior Symbol: 's' Symbol: '''  Code: 001011010

Prior Symbol: 's' Symbol: ','  Code: 001011011

Prior Symbol: 's' Symbol: '.'  Code: 00100101

Prior Symbol: 's' Symbol: ':'  Code: 0000001

Prior Symbol: 's' Symbol: '?'  Code: 001011100

Prior Symbol: 's' Symbol: 'C'  Code: 001011101

Prior Symbol: 's' Symbol: 'H'  Code: 001011110

Prior Symbol: 's' Symbol: 'a'  Code: 101010

Prior Symbol: 's' Symbol: 'c'  Code: 101011

Prior Symbol: 's' Symbol: 'd'  Code: 001011111

Prior Symbol: 's' Symbol: 'e'  Code: 1011

Prior Symbol: 's' Symbol: 'f'  Code: 00000000

Prior Symbol: 's' Symbol: 'h'  Code: 00001

Prior Symbol: 's' Symbol: 'i'  Code: 0011

Prior Symbol: 's' Symbol: 'k'  Code: 000001

Prior Symbol: 's' Symbol: 'l'  Code: 00101010

Prior Symbol: 's' Symbol: 'm'  Code: 00000001

Prior Symbol: 's' Symbol: 'n'  Code: 00101011

Prior Symbol: 's' Symbol: 'o'  Code: 10100

Prior Symbol: 's' Symbol: 'p'  Code: 001000

Prior Symbol: 's' Symbol: 'r'  Code: 00100100

Prior Symbol: 's' Symbol: 's'  Code: 0001

Prior Symbol: 's' Symbol: 't'  Code: 100

Prior Symbol: 's' Symbol: 'u'  Code: 0010100

Prior Symbol: 's' Symbol: 'y'  Code: 00101100

Prior Symbol: 't' Symbol:  0  Code: 010

Prior Symbol: 't' Symbol: 27  Code: 11000010

Prior Symbol: 't' Symbol: ' '  Code: 101

Prior Symbol: 't' Symbol: '''  Code: 11000011

Prior Symbol: 't' Symbol: ':'  Code: 110110000

Prior Symbol: 't' Symbol: '?'  Code: 110110001

Prior Symbol: 't' Symbol: 'a'  Code: 0000

Prior Symbol: 't' Symbol: 'b'  Code: 100000

Prior Symbol: 't' Symbol: 'c'  Code: 1101101

Prior Symbol: 't' Symbol: 'd'  Code: 11000000

Prior Symbol: 't' Symbol: 'e'  Code: 011

Prior Symbol: 't' Symbol: 'h'  Code: 111

Prior Symbol: 't' Symbol: 'i'  Code: 001

Prior Symbol: 't' Symbol: 'l'  Code: 10001

Prior Symbol: 't' Symbol: 'm'  Code: 100001

Prior Symbol: 't' Symbol: 'n'  Code: 11011001

Prior Symbol: 't' Symbol: 'o'  Code: 1001

Prior Symbol: 't' Symbol: 'r'  Code: 11010

Prior Symbol: 't' Symbol: 's'  Code: 0001

Prior Symbol: 't' Symbol: 't'  Code: 110111

Prior Symbol: 't' Symbol: 'u'  Code: 11001

Prior Symbol: 't' Symbol: 'w'  Code: 11000001

Prior Symbol: 't' Symbol: 'y'  Code: 110001

Prior Symbol: 'u' Symbol:  0  Code: 0011110

Prior Symbol: 'u' Symbol: 27  Code: 000100

Prior Symbol: 'u' Symbol: ' '  Code: 001110

Prior Symbol: 'u' Symbol: 'a'  Code: 00110

Prior Symbol: 'u' Symbol: 'b'  Code: 10011

Prior Symbol: 'u' Symbol: 'c'  Code: 11100

Prior Symbol: 'u' Symbol: 'd'  Code: 10000

Prior Symbol: 'u' Symbol: 'e'  Code: 0010

Prior Symbol: 'u' Symbol: 'f'  Code: 0011111

Prior Symbol: 'u' Symbol: 'g'  Code: 11101

Prior Symbol: 'u' Symbol: 'i'  Code: 00011

Prior Symbol: 'u' Symbol: 'k'  Code: 0001010

Prior Symbol: 'u' Symbol: 'l'  Code: 0000

Prior Symbol: 'u' Symbol: 'm'  Code: 10010

Prior Symbol: 'u' Symbol: 'n'  Code: 110

Prior Symbol: 'u' Symbol: 'p'  Code: 10001

Prior Symbol: 'u' Symbol: 'r'  Code: 01

Prior Symbol: 'u' Symbol: 's'  Code: 101

Prior Symbol: 'u' Symbol: 't'  Code: 1111

Prior Symbol: 'u' Symbol: 'z'  Code: 0001011

Prior Symbol: 'v' Symbol: 27  Code: 0010

Prior Symbol: 'v' Symbol: 'a'  Code: 000

Prior Symbol: 'v' Symbol: 'e'  Code: 1

Prior Symbol: 'v' Symbol: 'i'  Code: 01

Prior Symbol: 'v' Symbol: 'o'  Code: 00111

Prior Symbol: 'v' Symbol: 's'  Code: 00110

Prior Symbol: 'w' Symbol:  0  Code: 001

Prior Symbol: 'w' Symbol: 27  Code: 01010

Prior Symbol: 'w' Symbol: ' '  Code: 011

Prior Symbol: 'w' Symbol: '''  Code: 010010

Prior Symbol: 'w' Symbol: 'a'  Code: 000

Prior Symbol: 'w' Symbol: 'b'  Code: 010011

Prior Symbol: 'w' Symbol: 'c'  Code: 010111

Prior Symbol: 'w' Symbol: 'e'  Code: 1111

Prior Symbol: 'w' Symbol: 'i'  Code: 1100

Prior Symbol: 'w' Symbol: 'l'  Code: 010110

Prior Symbol: 'w' Symbol: 'n'  Code: 1110

Prior Symbol: 'w' Symbol: 'o'  Code: 1101

Prior Symbol: 'w' Symbol: 'r'  Code: 01000

Prior Symbol: 'w' Symbol: 's'  Code: 10

Prior Symbol: 'x' Symbol:  0  Code: 110

Prior Symbol: 'x' Symbol: 27  Code: 1010

Prior Symbol: 'x' Symbol: ' '  Code: 1011

Prior Symbol: 'x' Symbol: 'a'  Code: 000

Prior Symbol: 'x' Symbol: 'e'  Code: 001

Prior Symbol: 'x' Symbol: 'i'  Code: 100

Prior Symbol: 'x' Symbol: 'p'  Code: 111

Prior Symbol: 'x' Symbol: 't'  Code: 01

Prior Symbol: 'y' Symbol:  0  Code: 10

Prior Symbol: 'y' Symbol: 27  Code: 111110

Prior Symbol: 'y' Symbol: ' '  Code: 0

Prior Symbol: 'y' Symbol: '!'  Code: 1101101

Prior Symbol: 'y' Symbol: '''  Code: 110101

Prior Symbol: 'y' Symbol: '-'  Code: 11110101

Prior Symbol: 'y' Symbol: 'a'  Code: 1101110

Prior Symbol: 'y' Symbol: 'b'  Code: 1111011

Prior Symbol: 'y' Symbol: 'c'  Code: 11110100

Prior Symbol: 'y' Symbol: 'd'  Code: 1100000

Prior Symbol: 'y' Symbol: 'e'  Code: 11001

Prior Symbol: 'y' Symbol: 'i'  Code: 1100001

Prior Symbol: 'y' Symbol: 'l'  Code: 111111

Prior Symbol: 'y' Symbol: 'm'  Code: 1101111

Prior Symbol: 'y' Symbol: 'n'  Code: 1100010

Prior Symbol: 'y' Symbol: 'o'  Code: 1100011

Prior Symbol: 'y' Symbol: 'p'  Code: 1101000

Prior Symbol: 'y' Symbol: 's'  Code: 1110

Prior Symbol: 'y' Symbol: 't'  Code: 1101001

Prior Symbol: 'y' Symbol: 'v'  Code: 1101100

Prior Symbol: 'y' Symbol: 'w'  Code: 111100

Prior Symbol: 'z' Symbol:  0  Code: 110

Prior Symbol: 'z' Symbol: 27  Code: 100

Prior Symbol: 'z' Symbol: ' '  Code: 000

Prior Symbol: 'z' Symbol: 'a'  Code: 01

Prior Symbol: 'z' Symbol: 'e'  Code: 1010

Prior Symbol: 'z' Symbol: 'i'  Code: 111

Prior Symbol: 'z' Symbol: 'y'  Code: 001

Prior Symbol: 'z' Symbol: 'z'  Code: 1011

Prior Symbol: '{' Symbol: 27  Code: 1

Prior Symbol: '|' Symbol: 27  Code: 1

Prior Symbol: '}' Symbol: 27  Code: 1

Prior Symbol: '~' Symbol: 27  Code: 1

Prior Symbol: 127 Symbol: 27  Code: 1

Table B.C.5/J.94 – English-language Program Title Decode Table

0  1

1  0

2  1

3  58

4  1

5  60

6  1

7  62

8  1

9  64

10  1

11  66

12  1

13  68

14  1

15  70

16  1

17  72

18  1

19  74

20  1

21  76

22  1

23  78

24  1

25  80

26  1

27  82

28  1

29  84

30  1

31  86

32  1

33  88

34  1

35  90

36  1

37  92

38  1

39  94

40  1

41  96

42  1

43  98

44  1

45  100

46  1

47  102

48  1

49  104

50  1

51  106

52  1

53  108

54  1

55  110

56  1

57  112

58  1

59  114

60  1

61  116

62  1

63  118

64  1

65  120

66  1

67  206

68  1

69  210

70  1

71  212

72  1

73  214

74  1

75  216

76  1

77  218

78  1

79  220

80  1

81  230

82  1

83  232

84  1

85  234

86  1

87  240

88  1

89  242

90  1

91  244

92  2

93  6

94  2

95  18

96  2

97  20

98  2

99  28

100  2

101  40

102  2

103  48

104  2

105  52

106  2

107  54

108  2

109  56

110  2

111  58

112  2

113  60

114  2

115  62

116  2

117  70

118  2

119  72

120  2

121  74

122  2

123  76

124  2

125  78

126  2

127  80

128  2

129  82

130  2

131  84

132  2

133  126

134  2

135  146

136  2

137  172

138  2

139  186

140  2

141  210

142  2

143  228

144  2

145  250

146  3

147  6

148  3

149  30

150  3

151  38

152  3

153  50

154  3

155  62

156  3

157  82

158  3

159  100

160  3

161  122

162  3

163  148

164  3

165  152

166  3

167  164

168  3

169  200

170  3

171  222

172  3

173  230

174  3

175  244

176  4

177  4

178  4

179  6

180  4

181  12

182  4

183  16

184  4

185  18

186  4

187  20

188  4

189  22

190  4

191  24

192  4

193  26

194  4

195  28

196  4

197  82

198  4

199  106

200  4

201  142

202  4

203  174

204  4

205  238

206  5

207  6

208  5

209  40

210  5

211  68

212  5

213  114

214  5

215  118

216  5

217  144

218  5

219  190

220  5

221  214

222  6

223  10

224  6

225  68

226  6

227  100

228  6

229  102

230  6

231  154

232  6

233  208

234  6

235  252

236  7

237  34

238  7

239  44

240  7

241  70

242  7

243  84

244  7

245  124

246  7

247  138

248  7

249  140

250  7

251  142

252  7

253  144

254  7

255  146

256  27

257  28

258  180

259  164

260  178

261  183

262  218

263  1

264  209

265  2

266  3

267  155

268  4

269  213

270  217

271  5

272  203

273  214

274  6

275  207

276  7

277  8

278  202

279  9

280  201

281  197

282  198

283  10

284  210

285  196

286  199

287  204

288  208

289  200

290  215

291  206

292  11

293  193

294  12

295  194

296  205

297  195

298  13

299  14

300  15

301  16

302  211

303  17

304  212

305  18

306  19

307  20

308  21

309  22

310  23

311  24

312  25

313  26

314  155

315  155

316  155

317  155

318  155

319  155

320  155

321  155

322  155

323  155

324  155

325  155

326  155

327  155

328  155

329  155

330  155

331  155

332  155

333  155

334  155

335  155

336  155

337  155

338  155

339  155

340  155

341  155

342  155

343  155

344  155

345  155

346  155

347  155

348  155

349  155

350  155

351  155

352  155

353  155

354  155

355  155

356  155

357  155

358  155

359  155

360  155

361  155

362  155

363  155

364  155

365  155

366  155

367  155

368  155

369  155

370  155

371  155

372  155

373  155

374  155

375  155

376  41

377  42

378  216

379  229

380  185

381  1

382  167

383  177

384  236

385  209

386  2

387  173

388  178

389  218

390  227

391  179

392  3

393  228

394  230

395  4

396  155

397  226

398  5

399  6

400  7

401  8

402  9

403  213

404  10

405  214

406  11

407  217

408  12

409  166

410  233

411  203

412  197

413  207

414  13

415  14

416  202

417  201

418  15

419  199

420  16

421  17

422  225

423  18

424  19

425  198

426  210

427  200

428  206

429  193

430  196

431  208

432  204

433  20

434  21

435  239

436  194

437  215

438  22

439  205

440  23

441  244

442  212

443  24

444  25

445  26

446  195

447  211

448  27

449  28

450  29

451  30

452  31

453  32

454  33

455  34

456  35

457  36

458  37

459  38

460  39

461  40

462  1

463  128

464  160

465  155

466  155

467  155

468  155

469  155

470  177

471  155

472  155

473  155

474  155

475  160

476  4

477  243

478  228

479  185

480  1

481  244

482  160

483  155

484  2

485  3

486  155

487  155

488  155

489  155

490  1

491  2

492  155

493  193

494  200

495  211

496  155

497  155

498  155

499  160

500  7

501  8

502  177

503  210

504  211

505  212

506  213

507  173

508  205

509  193

510  1

511  2

512  3

513  160

514  4

515  155

516  5

517  6

518  160

519  5

520  201

521  215

522  211

523  1

524  2

525  155

526  174

527  128

528  3

529  4

530  155

531  155

532  2

533  3

534  173

535  155

536  1

537  128

538  160

539  176

540  4

541  5

542  128

543  155

544  177

545  178

546  160

547  176

548  185

549  1

550  2

551  3

552  2

553  3

554  177

555  186

556  1

557  176

558  155

559  128

560  128

561  1

562  176

563  155

564  155

565  184

566  155

567  155

568  155

569  155

570  155

571  176

572  155

573  160

574  2

575  3

576  177

577  179

578  185

579  176

580  1

581  155

582  155

583  160

584  155

585  155

586  155

587  155

588  155

589  155

590  155

591  155

592  155

593  128

594  155

595  155

596  19

597  20

598  170

599  173

600  174

601  246

602  231

603  244

604  226

605  233

606  1

607  2

608  194

609  240

610  155

611  243

612  227

613  230

614  247

615  3

616  245

617  4

618  5

619  6

620  242

621  7

622  8

623  9

624  10

625  11

626  12

627  228

628  160

629  13

630  236

631  238

632  14

633  237

634  15

635  16

636  17

637  18

638  8

639  9

640  193

641  211

642  155

643  1

644  195

645  2

646  233

647  236

648  3

649  242

650  245

651  4

652  239

653  225

654  5

655  229

656  6

657  7

658  11

659  12

660  193

661  249

662  1

663  194

664  207

665  229

666  245

667  155

668  233

669  2

670  160

671  3

672  4

673  5

674  242

675  6

676  236

677  7

678  225

679  8

680  9

681  232

682  10

683  239

684  5

685  6

686  249

687  155

688  1

689  245

690  2

691  242

692  233

693  229

694  239

695  3

696  225

697  4

698  10

699  11

700  241

701  245

702  243

703  1

704  237

705  249

706  195

707  2

708  236

709  238

710  228

711  248

712  3

713  155

714  246

715  4

716  5

717  225

718  6

719  7

720  8

721  9

722  7

723  8

724  160

725  155

726  204

727  1

728  229

729  2

730  236

731  245

732  239

733  3

734  233

735  242

736  4

737  5

738  225

739  6

740  9

741  10

742  174

743  236

744  249

745  193

746  232

747  1

748  155

749  2

750  3

751  4

752  225

753  245

754  233

755  5

756  229

757  6

758  242

759  239

760  7

761  8

762  239

763  5

764  128

765  155

766  245

767  1

768  2

769  233

770  225

771  3

772  229

773  4

774  238

775  11

776  186

777  212

778  174

779  242

780  227

781  1

782  160

783  2

784  128

785  155

786  237

787  3

788  201

789  243

790  244

791  4

792  5

793  6

794  7

795  8

796  9

797  10

798  2

799  3

800  155

801  245

802  1

803  225

804  239

805  229

806  5

807  233

808  225

809  239

810  245

811  238

812  155

813  229

814  1

815  2

816  3

817  4

818  4

819  5

820  160

821  155

822  1

823  245

824  2

825  229

826  239

827  3

828  225

829  233

830  8

831  9

832  170

833  212

834  1

835  155

836  227

837  2

838  242

839  3

840  229

841  4

842  245

843  249

844  233

845  5

846  239

847  6

848  7

849  225

850  229

851  8

852  206

853  160

854  198

855  245

856  1

857  2

858  155

859  194

860  3

861  225

862  4

863  239

864  5

865  233

866  6

867  7

868  9

869  10

870  228

871  243

872  230

873  246

874  247

875  240

876  242

877  1

878  236

879  2

880  3

881  160

882  155

883  4

884  5

885  245

886  6

887  7

888  238

889  8

890  11

891  12

892  160

893  243

894  249

895  174

896  210

897  199

898  1

899  155

900  2

901  245

902  3

903  4

904  5

905  233

906  236

907  6

908  229

909  7

910  239

911  8

912  225

913  9

914  242

915  10

916  1

917  245

918  155

919  214

920  4

921  5

922  232

923  155

924  1

925  245

926  2

927  225

928  233

929  239

930  3

931  229

932  16

933  17

934  170

935  236

936  241

937  174

938  160

939  247

940  237

941  238

942  1

943  2

944  155

945  235

946  3

947  4

948  5

949  6

950  227

951  7

952  239

953  8

954  233

955  245

956  9

957  225

958  229

959  240

960  232

961  10

962  11

963  12

964  13

965  244

966  14

967  15

968  232

969  10

970  173

971  206

972  155

973  1

974  214

975  2

976  245

977  247

978  3

979  4

980  225

981  229

982  233

983  5

984  242

985  6

986  239

987  7

988  8

989  9

990  238

991  3

992  236

993  174

994  1

995  155

996  2

997  240

998  6

999  233

1000  160

1001  195

1002  239

1003  155

1004  229

1005  1

1006  128

1007  2

1008  3

1009  225

1010  4

1011  5

1012  6

1013  7

1014  198

1015  215

1016  1

1017  155

1018  242

1019  2

1020  3

1021  232

1022  229

1023  225

1024  4

1025  233

1026  239

1027  5

1028  155

1029  155

1030  2

1031  239

1032  225

1033  155

1034  1

1035  229

1036  1

1037  239

1038  155

1039  225

1040  155

1041  155

1042  155

1043  155

1044  155

1045  155

1046  155

1047  155

1048  155

1049  155

1050  155

1051  155

1052  25

1053  26

1054  155

1055  186

1056  229

1057  234

1058  248

1059  1

1060  2

1061  230

1062  167

1063  3

1064  250

1065  232

1066  4

1067  247

1068  5

1069  245

1070  226

1071  6

1072  235

1073  7

1074  240

1075  8

1076  128

1077  246

1078  231

1079  9

1080  228

1081  10

1082  160

1083  233

1084  11

1085  227

1086  249

1087  12

1088  13

1089  237

1090  14

1091  15

1092  243

1093  16

1094  17

1095  236

1096  18

1097  244

1098  242

1099  19

1100  238

1101  20

1102  21

1103  22

1104  23

1105  24

1106  10

1107  11

1108  243

1109  155

1110  245

1111  226

1112  1

1113  128

1114  160

1115  2

1116  229

1117  242

1118  233

1119  3

1120  236

1121  4

1122  249

1123  5

1124  239

1125  6

1126  225

1127  7

1128  8

1129  9

1130  16

1131  17

1132  195

1133  204

1134  199

1135  155

1136  227

1137  1

1138  128

1139  236

1140  249

1141  2

1142  243

1143  3

1144  245

1145  4

1146  5

1147  242

1148  6

1149  233

1150  160

1151  7

1152  8

1153  239

1154  244

1155  9

1156  10

1157  225

1158  11

1159  232

1160  235

1161  229

1162  12

1163  13

1164  14

1165  15

1166  14

1167  15

1168  174

1169  245

1170  247

1171  1

1172  236

1173  2

1174  228

1175  231

1176  242

1177  3

1178  155

1179  239

1180  4

1181  246

1182  5

1183  6

1184  249

1185  243

1186  7

1187  233

1188  225

1189  8

1190  9

1191  128

1192  10

1193  11

1194  229

1195  12

1196  13

1197  160

1198  30

1199  31

1200  155

1201  161

1202  173

1203  232

1204  234

1205  241

1206  245

1207  250

1208  1

1209  2

1210  3

1211  4

1212  186

1213  248

1214  167

1215  226

1216  233

1217  5

1218  6

1219  7

1220  230

1221  237

1222  231

1223  235

1224  8

1225  9

1226  246

1227  240

1228  10

1229  239

1230  11

1231  227

1232  12

1233  13

1234  14

1235  249

1236  15

1237  228

1238  236

1239  16

1240  229

1241  17

1242  244

1243  247

1244  18

1245  19

1246  225

1247  20

1248  21

1249  22

1250  238

1251  243

1252  23

1253  128

1254  24

1255  25

1256  242

1257  26

1258  27

1259  160

1260  28

1261  29

1262  160

1263  11

1264  245

1265  155

1266  1

1267  236

1268  243

1269  242

1270  128

1271  225

1272  2

1273  3

1274  244

1275  233

1276  239

1277  230

1278  4

1279  5

1280  6

1281  7

1282  229

1283  8

1284  9

1285  10

1286  15

1287  16

1288  186

1289  249

1290  167

1291  244

1292  155

1293  1

1294  231

1295  236

1296  2

1297  238

1298  3

1299  239

1300  245

1301  4

1302  242

1303  5

1304  6

1305  233

1306  7

1307  243

1308  225

1309  8

1310  9

1311  10

1312  11

1313  229

1314  128

1315  12

1316  232

1317  160

1318  13

1319  14

1320  229

1321  13

1322  226

1323  245

1324  247

1325  155

1326  236

1327  1

1328  249

1329  238

1330  2

1331  3

1332  4

1333  242

1334  5

1335  128

1336  6

1337  160

1338  225

1339  239

1340  7

1341  244

1342  233

1343  8

1344  9

1345  10

1346  11

1347  12

1348  21

1349  22

1350  161

1351  248

1352  233

1353  235

1354  1

1355  128

1356  155

1357  250

1358  226

1359  2

1360  3

1361  4

1362  160

1363  240

1364  5

1365  6

1366  7

1367  225

1368  8

1369  230

1370  242

1371  237

1372  246

1373  9

1374  228

1375  10

1376  239

1377  244

1378  236

1379  243

1380  231

1381  229

1382  11

1383  227

1384  12

1385  13

1386  14

1387  15

1388  16

1389  17

1390  18

1391  19

1392  238

1393  20

1394  239

1395  1

1396  155

1397  225

1398  11

1399  12

1400  212

1401  239

1402  230

1403  236

1404  247

1405  225

1406  1

1407  186

1408  2

1409  155

1410  249

1411  3

1412  4

1413  5

1414  243

1415  6

1416  7

1417  8

1418  233

1419  160

1420  9

1421  128

1422  229

1423  10

1424  21

1425  22

1426  167

1427  186

1428  227

1429  247

1430  242

1431  173

1432  226

1433  1

1434  2

1435  155

1436  230

1437  3

1438  237

1439  246

1440  4

1441  235

1442  5

1443  244

1444  6

1445  7

1446  8

1447  243

1448  9

1449  245

1450  10

1451  239

1452  11

1453  12

1454  128

1455  249

1456  225

1457  13

1458  228

1459  233

1460  160

1461  14

1462  15

1463  236

1464  229

1465  16

1466  17

1467  18

1468  19

1469  20

1470  10

1471  11

1472  249

1473  155

1474  245

1475  243

1476  1

1477  2

1478  226

1479  237

1480  128

1481  3

1482  240

1483  239

1484  4

1485  160

1486  5

1487  233

1488  6

1489  225

1490  7

1491  8

1492  9

1493  229

1494  24

1495  25

1496  226

1497  234

1498  242

1499  232

1500  236

1501  237

1502  250

1503  155

1504  1

1505  245

1506  2

1507  3

1508  246

1509  4

1510  186

1511  230

1512  5

1513  6

1514  235

1515  239

1516  7

1517  167

1518  249

1519  8

1520  9

1521  10

1522  11

1523  227

1524  12

1525  238

1526  225

1527  13

1528  243

1529  14

1530  233

1531  15

1532  16

1533  244

1534  128

1535  228

1536  229

1537  17

1538  18

1539  231

1540  160

1541  19

1542  20

1543  21

1544  22

1545  23

1546  27

1547  28

1548  174

1549  250

1550  191

1551  1

1552  167

1553  155

1554  2

1555  233

1556  248

1557  249

1558  3

1559  229

1560  232

1561  4

1562  225

1563  235

1564  5

1565  226

1566  6

1567  7

1568  227

1569  8

1570  231

1571  244

1572  9

1573  128

1574  246

1575  240

1576  10

1577  228

1578  11

1579  243

1580  247

1581  12

1582  13

1583  239

1584  236

1585  160

1586  14

1587  15

1588  237

1589  230

1590  16

1591  245

1592  17

1593  18

1594  19

1595  20

1596  21

1597  242

1598  22

1599  238

1600  23

1601  24

1602  25

1603  26

1604  14

1605  15

1606  237

1607  167

1608  155

1609  228

1610  1

1611  249

1612  243

1613  242

1614  244

1615  2

1616  232

1617  3

1618  236

1619  240

1620  4

1621  225

1622  233

1623  5

1624  6

1625  128

1626  160

1627  7

1628  8

1629  9

1630  10

1631  229

1632  239

1633  11

1634  12

1635  13

1636  155

1637  245

1638  24

1639  25

1640  186

1641  172

1642  246

1643  155

1644  240

1645  226

1646  1

1647  230

1648  2

1649  167

1650  174

1651  231

1652  3

1653  227

1654  245

1655  4

1656  237

1657  5

1658  6

1659  7

1660  235

1661  8

1662  9

1663  238

1664  242

1665  10

1666  228

1667  11

1668  249

1669  236

1670  12

1671  13

1672  244

1673  128

1674  14

1675  239

1676  243

1677  160

1678  225

1679  15

1680  233

1681  16

1682  17

1683  229

1684  18

1685  19

1686  20

1687  21

1688  22

1689  23

1690  25

1691  26

1692  167

1693  172

1694  191

1695  195

1696  200

1697  228

1698  230

1699  237

1700  242

1701  174

1702  236

1703  238

1704  249

1705  1

1706  2

1707  3

1708  4

1709  186

1710  5

1711  155

1712  245

1713  6

1714  7

1715  8

1716  9

1717  235

1718  240

1719  10

1720  11

1721  12

1722  225

1723  227

1724  13

1725  232

1726  14

1727  15

1728  239

1729  16

1730  17

1731  243

1732  18

1733  233

1734  19

1735  229

1736  20

1737  21

1738  244

1739  22

1740  23

1741  160

1742  24

1743  128

1744  20

1745  21

1746  186

1747  191

1748  228

1749  247

1750  155

1751  167

1752  1

1753  238

1754  2

1755  3

1756  4

1757  227

1758  226

1759  237

1760  5

1761  249

1762  6

1763  244

1764  7

1765  236

1766  8

1767  245

1768  242

1769  9

1770  225

1771  243

1772  10

1773  239

1774  11

1775  12

1776  13

1777  233

1778  128

1779  229

1780  14

1781  160

1782  15

1783  232

1784  16

1785  17

1786  18

1787  19

1788  17

1789  18

1790  235

1791  250

1792  128

1793  230

1794  155

1795  1

1796  160

1797  2

1798  3

1799  233

1800  225

1801  4

1802  228

1803  240

1804  237

1805  226

1806  227

1807  231

1808  236

1809  5

1810  229

1811  6

1812  7

1813  8

1814  9

1815  244

1816  10

1817  11

1818  12

1819  243

1820  238

1821  13

1822  14

1823  242

1824  15

1825  16

1826  4

1827  229

1828  243

1829  239

1830  155

1831  1

1832  225

1833  2

1834  3

1835  233

1836  11

1837  12

1838  167

1839  226

1840  236

1841  227

1842  242

1843  1

1844  155

1845  2

1846  3

1847  4

1848  233

1849  239

1850  238

1851  229

1852  225

1853  128

1854  5

1855  160

1856  6

1857  7

1858  8

1859  9

1860  243

1861  10

1862  5

1863  6

1864  155

1865  160

1866  225

1867  229

1868  233

1869  1

1870  128

1871  240

1872  2

1873  244

1874  3

1875  4

1876  160

1877  19

1878  227

1879  173

1880  228

1881  233

1882  238

1883  239

1884  240

1885  244

1886  246

1887  161

1888  225

1889  237

1890  1

1891  226

1892  2

1893  3

1894  4

1895  167

1896  5

1897  6

1898  247

1899  7

1900  155

1901  236

1902  8

1903  229

1904  9

1905  10

1906  11

1907  12

1908  13

1909  14

1910  243

1911  15

1912  16

1913  17

1914  128

1915  18

1916  5

1917  6

1918  229

1919  250

1920  160

1921  249

1922  155

1923  1

1924  128

1925  233

1926  2

1927  225

1928  3

1929  4

1930  155

1931  155

1932  155

1933  155

1934  155

1935  155

1936  155

1937  155

1938  155

1939  155

### B.C.3	Standard compression Type 2 Huffman Encode/Decode tables

The following encode/decode tables (Tables B.C.6 and B.C.7) are optimized for English-language program description text. These tables correspond to multiple_string_structure() with compression_type value 0x02, and mode equal to 0xFF.

Table B.C.6/J.94 – English-language Program Description Encode Table

Prior Symbol:  0 Symbol: 27  Code: 1110000

Prior Symbol:  0 Symbol: '"'  Code: 111001

Prior Symbol:  0 Symbol: 'A'  Code: 010

Prior Symbol:  0 Symbol: 'B'  Code: 0011

Prior Symbol:  0 Symbol: 'C'  Code: 0111

Prior Symbol:  0 Symbol: 'D'  Code: 11101

Prior Symbol:  0 Symbol: 'E'  Code: 10010

Prior Symbol:  0 Symbol: 'F'  Code: 10110

Prior Symbol:  0 Symbol: 'G'  Code: 011011

Prior Symbol:  0 Symbol: 'H'  Code: 10111

Prior Symbol:  0 Symbol: 'I'  Code: 011000

Prior Symbol:  0 Symbol: 'J'  Code: 1100

Prior Symbol:  0 Symbol: 'K'  Code: 00101

Prior Symbol:  0 Symbol: 'L'  Code: 10011

Prior Symbol:  0 Symbol: 'M'  Code: 1111

Prior Symbol:  0 Symbol: 'N'  Code: 00100

Prior Symbol:  0 Symbol: 'O'  Code: 011001

Prior Symbol:  0 Symbol: 'P'  Code: 000

Prior Symbol:  0 Symbol: 'R'  Code: 1000

Prior Symbol:  0 Symbol: 'S'  Code: 1010

Prior Symbol:  0 Symbol: 'T'  Code: 1101

Prior Symbol:  0 Symbol: 'V'  Code: 1110001

Prior Symbol:  0 Symbol: 'W'  Code: 011010

Prior Symbol:  1 Symbol: 27  Code: 1

Prior Symbol:  2 Symbol: 27  Code: 1

Prior Symbol:  3 Symbol: 27  Code: 1

Prior Symbol:  4 Symbol: 27  Code: 1

Prior Symbol:  5 Symbol: 27  Code: 1

Prior Symbol:  6 Symbol: 27  Code: 1

Prior Symbol:  7 Symbol: 27  Code: 1

Prior Symbol:  8 Symbol: 27  Code: 1

Prior Symbol:  9 Symbol: 27  Code: 1

Prior Symbol: 10 Symbol: 27  Code: 1

Prior Symbol: 11 Symbol: 27  Code: 1

Prior Symbol: 12 Symbol: 27  Code: 1

Prior Symbol: 13 Symbol: 27  Code: 1

Prior Symbol: 14 Symbol: 27  Code: 1

Prior Symbol: 15 Symbol: 27  Code: 1

Prior Symbol: 16 Symbol: 27  Code: 1

Prior Symbol: 17 Symbol: 27  Code: 1

Prior Symbol: 18 Symbol: 27  Code: 1

Prior Symbol: 19 Symbol: 27  Code: 1

Prior Symbol: 20 Symbol: 27  Code: 1

Prior Symbol: 21 Symbol: 27  Code: 1

Prior Symbol: 22 Symbol: 27  Code: 1

Prior Symbol: 23 Symbol: 27  Code: 1

Prior Symbol: 24 Symbol: 27  Code: 1

Prior Symbol: 25 Symbol: 27  Code: 1

Prior Symbol: 26 Symbol: 27  Code: 1

Prior Symbol: 27 Symbol: 27  Code: 1

Prior Symbol: 28 Symbol: 27  Code: 1

Prior Symbol: 29 Symbol: 27  Code: 1

Prior Symbol: 30 Symbol: 27  Code: 1

Prior Symbol: 31 Symbol: 27  Code: 1

Prior Symbol: ' ' Symbol: 27  Code: 101000001

Prior Symbol: ' ' Symbol: '"'  Code: 111111010

Prior Symbol: ' ' Symbol: '('  Code: 1111111100

Prior Symbol: ' ' Symbol: '-'  Code: 11111111110

Prior Symbol: ' ' Symbol: '/'  Code: 11111111111

Prior Symbol: ' ' Symbol: '1'  Code: 0101011

Prior Symbol: ' ' Symbol: '2'  Code: 0100010

Prior Symbol: ' ' Symbol: '3'  Code: 1111111101

Prior Symbol: ' ' Symbol: '4'  Code: 110010100

Prior Symbol: ' ' Symbol: '5'  Code: 1111111110

Prior Symbol: ' ' Symbol: '7'  Code: 1010000000

Prior Symbol: ' ' Symbol: 'A'  Code: 10010

Prior Symbol: ' ' Symbol: 'B'  Code: 010100

Prior Symbol: ' ' Symbol: 'C'  Code: 111100

Prior Symbol: ' ' Symbol: 'D'  Code: 1111010

Prior Symbol: ' ' Symbol: 'E'  Code: 0100011

Prior Symbol: ' ' Symbol: 'F'  Code: 0101010

Prior Symbol: ' ' Symbol: 'G'  Code: 000010

Prior Symbol: ' ' Symbol: 'H'  Code: 1111011

Prior Symbol: ' ' Symbol: 'I'  Code: 11001011

Prior Symbol: ' ' Symbol: 'J'  Code: 000011

Prior Symbol: ' ' Symbol: 'K'  Code: 1100100

Prior Symbol: ' ' Symbol: 'L'  Code: 010110

Prior Symbol: ' ' Symbol: 'M'  Code: 101001

Prior Symbol: ' ' Symbol: 'N'  Code: 001100

Prior Symbol: ' ' Symbol: 'O'  Code: 10100001

Prior Symbol: ' ' Symbol: 'P'  Code: 001101

Prior Symbol: ' ' Symbol: 'R'  Code: 1111100

Prior Symbol: ' ' Symbol: 'S'  Code: 01001

Prior Symbol: ' ' Symbol: 'T'  Code: 1100110

Prior Symbol: ' ' Symbol: 'U'  Code: 111111011

Prior Symbol: ' ' Symbol: 'V'  Code: 111111100

Prior Symbol: ' ' Symbol: 'W'  Code: 010000

Prior Symbol: ' ' Symbol: 'Y'  Code: 111111101

Prior Symbol: ' ' Symbol: 'Z'  Code: 1010000001

Prior Symbol: ' ' Symbol: 'a'  Code: 011

Prior Symbol: ' ' Symbol: 'b'  Code: 10111

Prior Symbol: ' ' Symbol: 'c'  Code: 10011

Prior Symbol: ' ' Symbol: 'd'  Code: 10000

Prior Symbol: ' ' Symbol: 'e'  Code: 100010

Prior Symbol: ' ' Symbol: 'f'  Code: 11101

Prior Symbol: ' ' Symbol: 'g'  Code: 100011

Prior Symbol: ' ' Symbol: 'h'  Code: 0001

Prior Symbol: ' ' Symbol: 'i'  Code: 10101

Prior Symbol: ' ' Symbol: 'j'  Code: 11001111

Prior Symbol: ' ' Symbol: 'k'  Code: 11111010

Prior Symbol: ' ' Symbol: 'l'  Code: 010111

Prior Symbol: ' ' Symbol: 'm'  Code: 00000

Prior Symbol: ' ' Symbol: 'n'  Code: 1010001

Prior Symbol: ' ' Symbol: 'o'  Code: 0010

Prior Symbol: ' ' Symbol: 'p'  Code: 10110

Prior Symbol: ' ' Symbol: 'q'  Code: 110010101

Prior Symbol: ' ' Symbol: 'r'  Code: 00111

Prior Symbol: ' ' Symbol: 's'  Code: 11100

Prior Symbol: ' ' Symbol: 't'  Code: 1101

Prior Symbol: ' ' Symbol: 'u'  Code: 11111011

Prior Symbol: ' ' Symbol: 'v'  Code: 11111100

Prior Symbol: ' ' Symbol: 'w'  Code: 11000

Prior Symbol: ' ' Symbol: 'y'  Code: 11001110

Prior Symbol: '!' Symbol: 27  Code: 1

Prior Symbol: '"' Symbol:  0  Code: 000

Prior Symbol: '"' Symbol: 27  Code: 10

Prior Symbol: '"' Symbol: ' '  Code: 11

Prior Symbol: '"' Symbol: '.'  Code: 001

Prior Symbol: '"' Symbol: 'H'  Code: 010

Prior Symbol: '"' Symbol: 'T'  Code: 011

Prior Symbol: '#' Symbol: 27  Code: 1

Prior Symbol: '$' Symbol: 27  Code: 1

Prior Symbol: '%' Symbol: 27  Code: 1

Prior Symbol: '&' Symbol: 27  Code: 1

Prior Symbol: ''' Symbol: 27  Code: 00

Prior Symbol: ''' Symbol: ' '  Code: 010

Prior Symbol: ''' Symbol: 's'  Code: 1

Prior Symbol: ''' Symbol: 't'  Code: 011

Prior Symbol: '(' Symbol: 27  Code: 1

Prior Symbol: ')' Symbol: 27  Code: 1

Prior Symbol: ')' Symbol: ','  Code: 0

Prior Symbol: '*' Symbol: 27  Code: 1

Prior Symbol: '+' Symbol: 27  Code: 1

Prior Symbol: ',' Symbol: 27  Code: 00

Prior Symbol: ',' Symbol: ' '  Code: 1

Prior Symbol: ',' Symbol: '"'  Code: 01

Prior Symbol: '-' Symbol: 27  Code: 10

Prior Symbol: '-' Symbol: ' '  Code: 1110

Prior Symbol: '-' Symbol: 'a'  Code: 000

Prior Symbol: '-' Symbol: 'b'  Code: 0010

Prior Symbol: '-' Symbol: 'c'  Code: 110

Prior Symbol: '-' Symbol: 'd'  Code: 0011

Prior Symbol: '-' Symbol: 'e'  Code: 0100

Prior Symbol: '-' Symbol: 'f'  Code: 0101

Prior Symbol: '-' Symbol: 'r'  Code: 1111

Prior Symbol: '-' Symbol: 's'  Code: 011

Prior Symbol: '.' Symbol:  0  Code: 1

Prior Symbol: '.' Symbol: 27  Code: 000

Prior Symbol: '.' Symbol: ' '  Code: 01

Prior Symbol: '.' Symbol: '"'  Code: 0010

Prior Symbol: '.' Symbol: 'J'  Code: 00110

Prior Symbol: '.' Symbol: 'S'  Code: 00111

Prior Symbol: '/' Symbol: 27  Code: 0

Prior Symbol: '/' Symbol: ' '  Code: 1

Prior Symbol: '0' Symbol: 27  Code: 100

Prior Symbol: '0' Symbol: ' '  Code: 111

Prior Symbol: '0' Symbol: '0'  Code: 00

Prior Symbol: '0' Symbol: '7'  Code: 101

Prior Symbol: '0' Symbol: 's'  Code: 01

Prior Symbol: '0' Symbol: 't'  Code: 110

Prior Symbol: '1' Symbol: 27  Code: 111

Prior Symbol: '1' Symbol: ' '  Code: 10

Prior Symbol: '1' Symbol: '8'  Code: 110

Prior Symbol: '1' Symbol: '9'  Code: 0

Prior Symbol: '2' Symbol: 27  Code: 101

Prior Symbol: '2' Symbol: ' '  Code: 11

Prior Symbol: '2' Symbol: '.'  Code: 0

Prior Symbol: '2' Symbol: '6'  Code: 100

Prior Symbol: '3' Symbol: 27  Code: 10

Prior Symbol: '3' Symbol: ' '  Code: 0

Prior Symbol: '3' Symbol: '0'  Code: 11

Prior Symbol: '4' Symbol: 27  Code: 10

Prior Symbol: '4' Symbol: ' '  Code: 11

Prior Symbol: '4' Symbol: '.'  Code: 0

Prior Symbol: '5' Symbol: 27  Code: 11

Prior Symbol: '5' Symbol: ' '  Code: 10

Prior Symbol: '5' Symbol: '.'  Code: 0

Prior Symbol: '6' Symbol: 27  Code: 1

Prior Symbol: '7' Symbol: 27  Code: 0

Prior Symbol: '7' Symbol: ','  Code: 10

Prior Symbol: '7' Symbol: '.'  Code: 11

Prior Symbol: '8' Symbol: 27  Code: 1

Prior Symbol: '9' Symbol: 27  Code: 110

Prior Symbol: '9' Symbol: ' '  Code: 111

Prior Symbol: '9' Symbol: '5'  Code: 00

Prior Symbol: '9' Symbol: '6'  Code: 01

Prior Symbol: '9' Symbol: '8'  Code: 10

Prior Symbol: ':' Symbol: 27  Code: 0

Prior Symbol: ':' Symbol: ' '  Code: 1

Prior Symbol: ';' Symbol: 27  Code: 0

Prior Symbol: ';' Symbol: ' '  Code: 1

Prior Symbol: '<' Symbol: 27  Code: 1

Prior Symbol: '=' Symbol: 27  Code: 1

Prior Symbol: '>' Symbol: 27  Code: 1

Prior Symbol: '?' Symbol: 27  Code: 0

Prior Symbol: '?' Symbol: ' '  Code: 1

Prior Symbol: '@' Symbol: 27  Code: 1

Prior Symbol: 'A' Symbol: 27  Code: 10010

Prior Symbol: 'A' Symbol: ' '  Code: 11

Prior Symbol: 'A' Symbol: 'd'  Code: 10011

Prior Symbol: 'A' Symbol: 'f'  Code: 101000

Prior Symbol: 'A' Symbol: 'l'  Code: 00

Prior Symbol: 'A' Symbol: 'm'  Code: 10101

Prior Symbol: 'A' Symbol: 'n'  Code: 01

Prior Symbol: 'A' Symbol: 'r'  Code: 1011

Prior Symbol: 'A' Symbol: 's'  Code: 10000

Prior Symbol: 'A' Symbol: 't'  Code: 10001

Prior Symbol: 'A' Symbol: 'u'  Code: 101001

Prior Symbol: 'B' Symbol: 27  Code: 10010

Prior Symbol: 'B' Symbol: 'a'  Code: 101

Prior Symbol: 'B' Symbol: 'e'  Code: 111

Prior Symbol: 'B' Symbol: 'i'  Code: 00

Prior Symbol: 'B' Symbol: 'l'  Code: 10011

Prior Symbol: 'B' Symbol: 'o'  Code: 110

Prior Symbol: 'B' Symbol: 'r'  Code: 01

Prior Symbol: 'B' Symbol: 'u'  Code: 1000

Prior Symbol: 'C' Symbol: 27  Code: 01110

Prior Symbol: 'C' Symbol: 'a'  Code: 00

Prior Symbol: 'C' Symbol: 'h'  Code: 10

Prior Symbol: 'C' Symbol: 'i'  Code: 01111

Prior Symbol: 'C' Symbol: 'l'  Code: 110

Prior Symbol: 'C' Symbol: 'o'  Code: 111

Prior Symbol: 'C' Symbol: 'r'  Code: 0101

Prior Symbol: 'C' Symbol: 'u'  Code: 0110

Prior Symbol: 'C' Symbol: 'y'  Code: 0100

Prior Symbol: 'D' Symbol: 27  Code: 1111

Prior Symbol: 'D' Symbol: 'a'  Code: 01

Prior Symbol: 'D' Symbol: 'e'  Code: 100

Prior Symbol: 'D' Symbol: 'i'  Code: 00

Prior Symbol: 'D' Symbol: 'o'  Code: 101

Prior Symbol: 'D' Symbol: 'r'  Code: 1101

Prior Symbol: 'D' Symbol: 'u'  Code: 1110

Prior Symbol: 'D' Symbol: 'y'  Code: 1100

Prior Symbol: 'E' Symbol: 27  Code: 10

Prior Symbol: 'E' Symbol: 'a'  Code: 0110

Prior Symbol: 'E' Symbol: 'd'  Code: 000

Prior Symbol: 'E' Symbol: 'i'  Code: 0111

Prior Symbol: 'E' Symbol: 'l'  Code: 001

Prior Symbol: 'E' Symbol: 'n'  Code: 1100

Prior Symbol: 'E' Symbol: 'r'  Code: 111

Prior Symbol: 'E' Symbol: 's'  Code: 010

Prior Symbol: 'E' Symbol: 'v'  Code: 1101

Prior Symbol: 'F' Symbol: 27  Code: 00

Prior Symbol: 'F' Symbol: 'e'  Code: 100

Prior Symbol: 'F' Symbol: 'l'  Code: 101

Prior Symbol: 'F' Symbol: 'o'  Code: 01

Prior Symbol: 'F' Symbol: 'r'  Code: 11

Prior Symbol: 'G' Symbol: 27  Code: 000

Prior Symbol: 'G' Symbol: 'a'  Code: 110

Prior Symbol: 'G' Symbol: 'e'  Code: 01

Prior Symbol: 'G' Symbol: 'i'  Code: 100

Prior Symbol: 'G' Symbol: 'l'  Code: 001

Prior Symbol: 'G' Symbol: 'o'  Code: 1011

Prior Symbol: 'G' Symbol: 'r'  Code: 111

Prior Symbol: 'G' Symbol: 'u'  Code: 1010

Prior Symbol: 'H' Symbol: 27  Code: 010

Prior Symbol: 'H' Symbol: 'a'  Code: 00

Prior Symbol: 'H' Symbol: 'e'  Code: 011

Prior Symbol: 'H' Symbol: 'i'  Code: 110

Prior Symbol: 'H' Symbol: 'o'  Code: 10

Prior Symbol: 'H' Symbol: 'u'  Code: 111

Prior Symbol: 'I' Symbol: 27  Code: 011

Prior Symbol: 'I' Symbol: ' '  Code: 000

Prior Symbol: 'I' Symbol: '.'  Code: 100

Prior Symbol: 'I' Symbol: 'I'  Code: 001

Prior Symbol: 'I' Symbol: 'n'  Code: 11

Prior Symbol: 'I' Symbol: 'r'  Code: 101

Prior Symbol: 'I' Symbol: 's'  Code: 010

Prior Symbol: 'J' Symbol: 27  Code: 1000

Prior Symbol: 'J' Symbol: '.'  Code: 1001

Prior Symbol: 'J' Symbol: 'a'  Code: 111

Prior Symbol: 'J' Symbol: 'e'  Code: 1101

Prior Symbol: 'J' Symbol: 'i'  Code: 1100

Prior Symbol: 'J' Symbol: 'o'  Code: 0

Prior Symbol: 'J' Symbol: 'u'  Code: 101

Prior Symbol: 'K' Symbol: 27  Code: 111

Prior Symbol: 'K' Symbol: 'a'  Code: 100

Prior Symbol: 'K' Symbol: 'e'  Code: 0

Prior Symbol: 'K' Symbol: 'i'  Code: 101

Prior Symbol: 'K' Symbol: 'r'  Code: 110

Prior Symbol: 'L' Symbol: 27  Code: 0110

Prior Symbol: 'L' Symbol: 'a'  Code: 11

Prior Symbol: 'L' Symbol: 'e'  Code: 00

Prior Symbol: 'L' Symbol: 'i'  Code: 0111

Prior Symbol: 'L' Symbol: 'o'  Code: 10

Prior Symbol: 'L' Symbol: 'u'  Code: 010

Prior Symbol: 'M' Symbol: 27  Code: 11010

Prior Symbol: 'M' Symbol: 'a'  Code: 0

Prior Symbol: 'M' Symbol: 'c'  Code: 11011

Prior Symbol: 'M' Symbol: 'e'  Code: 1111

Prior Symbol: 'M' Symbol: 'i'  Code: 10

Prior Symbol: 'M' Symbol: 'o'  Code: 1100

Prior Symbol: 'M' Symbol: 'u'  Code: 1110

Prior Symbol: 'N' Symbol: 27  Code: 1100

Prior Symbol: 'N' Symbol: 'a'  Code: 111

Prior Symbol: 'N' Symbol: 'e'  Code: 0

Prior Symbol: 'N' Symbol: 'i'  Code: 1101

Prior Symbol: 'N' Symbol: 'o'  Code: 10

Prior Symbol: 'O' Symbol: 27  Code: 10

Prior Symbol: 'O' Symbol: '''  Code: 010

Prior Symbol: 'O' Symbol: 'l'  Code: 110

Prior Symbol: 'O' Symbol: 'n'  Code: 011

Prior Symbol: 'O' Symbol: 'r'  Code: 111

Prior Symbol: 'O' Symbol: 's'  Code: 00

Prior Symbol: 'P' Symbol: 27  Code: 10010

Prior Symbol: 'P' Symbol: 'a'  Code: 0

Prior Symbol: 'P' Symbol: 'e'  Code: 111

Prior Symbol: 'P' Symbol: 'h'  Code: 10011

Prior Symbol: 'P' Symbol: 'i'  Code: 1000

Prior Symbol: 'P' Symbol: 'l'  Code: 1101

Prior Symbol: 'P' Symbol: 'o'  Code: 101

Prior Symbol: 'P' Symbol: 'r'  Code: 1100

Prior Symbol: 'Q' Symbol: 27  Code: 1

Prior Symbol: 'R' Symbol: 27  Code: 0000

Prior Symbol: 'R' Symbol: '.'  Code: 0001

Prior Symbol: 'R' Symbol: 'a'  Code: 01

Prior Symbol: 'R' Symbol: 'e'  Code: 10

Prior Symbol: 'R' Symbol: 'i'  Code: 001

Prior Symbol: 'R' Symbol: 'o'  Code: 11

Prior Symbol: 'S' Symbol: 27  Code: 1011

Prior Symbol: 'S' Symbol: '.'  Code: 0001

Prior Symbol: 'S' Symbol: 'a'  Code: 100

Prior Symbol: 'S' Symbol: 'c'  Code: 0010

Prior Symbol: 'S' Symbol: 'e'  Code: 1110

Prior Symbol: 'S' Symbol: 'h'  Code: 110

Prior Symbol: 'S' Symbol: 'i'  Code: 0011

Prior Symbol: 'S' Symbol: 'o'  Code: 1111

Prior Symbol: 'S' Symbol: 't'  Code: 01

Prior Symbol: 'S' Symbol: 'u'  Code: 1010

Prior Symbol: 'S' Symbol: 'v'  Code: 00000

Prior Symbol: 'S' Symbol: 'y'  Code: 00001

Prior Symbol: 'T' Symbol: 27  Code: 1010

Prior Symbol: 'T' Symbol: 'V'  Code: 1000

Prior Symbol: 'T' Symbol: 'a'  Code: 1001

Prior Symbol: 'T' Symbol: 'e'  Code: 11010

Prior Symbol: 'T' Symbol: 'h'  Code: 0

Prior Symbol: 'T' Symbol: 'i'  Code: 1011

Prior Symbol: 'T' Symbol: 'o'  Code: 111

Prior Symbol: 'T' Symbol: 'r'  Code: 1100

Prior Symbol: 'T' Symbol: 'w'  Code: 11011

Prior Symbol: 'U' Symbol: 27  Code: 10

Prior Symbol: 'U' Symbol: '.'  Code: 0

Prior Symbol: 'U' Symbol: 'n'  Code: 11

Prior Symbol: 'V' Symbol: 27  Code: 111

Prior Symbol: 'V' Symbol: ' '  Code: 10

Prior Symbol: 'V' Symbol: 'e'  Code: 110

Prior Symbol: 'V' Symbol: 'i'  Code: 0

Prior Symbol: 'W' Symbol: 27  Code: 010

Prior Symbol: 'W' Symbol: 'a'  Code: 111

Prior Symbol: 'W' Symbol: 'e'  Code: 110

Prior Symbol: 'W' Symbol: 'h'  Code: 011

Prior Symbol: 'W' Symbol: 'i'  Code: 10

Prior Symbol: 'W' Symbol: 'o'  Code: 00

Prior Symbol: 'X' Symbol: 27  Code: 1

Prior Symbol: 'Y' Symbol: 27  Code: 0

Prior Symbol: 'Y' Symbol: 'o'  Code: 1

Prior Symbol: 'Z' Symbol: 27  Code: 1

Prior Symbol: '[' Symbol: 27  Code: 1

Prior Symbol: '\' Symbol: 27  Code: 1

Prior Symbol: ']' Symbol: 27  Code: 1

Prior Symbol: '^' Symbol: 27  Code: 1

Prior Symbol: '_' Symbol: 27  Code: 1

Prior Symbol: '`' Symbol: 27  Code: 1

Prior Symbol: 'a' Symbol: 27  Code: 111001101

Prior Symbol: 'a' Symbol: ' '  Code: 101

Prior Symbol: 'a' Symbol: '''  Code: 111001110

Prior Symbol: 'a' Symbol: '.'  Code: 1110010

Prior Symbol: 'a' Symbol: 'b'  Code: 001011

Prior Symbol: 'a' Symbol: 'c'  Code: 11001

Prior Symbol: 'a' Symbol: 'd'  Code: 00111

Prior Symbol: 'a' Symbol: 'e'  Code: 0011001

Prior Symbol: 'a' Symbol: 'f'  Code: 001010

Prior Symbol: 'a' Symbol: 'g'  Code: 00100

Prior Symbol: 'a' Symbol: 'h'  Code: 001100010

Prior Symbol: 'a' Symbol: 'i'  Code: 111000

Prior Symbol: 'a' Symbol: 'k'  Code: 110000

Prior Symbol: 'a' Symbol: 'l'  Code: 1101

Prior Symbol: 'a' Symbol: 'm'  Code: 11101

Prior Symbol: 'a' Symbol: 'n'  Code: 01

Prior Symbol: 'a' Symbol: 'o'  Code: 001100011

Prior Symbol: 'a' Symbol: 'p'  Code: 00000

Prior Symbol: 'a' Symbol: 'r'  Code: 100

Prior Symbol: 'a' Symbol: 's'  Code: 0001

Prior Symbol: 'a' Symbol: 't'  Code: 1111

Prior Symbol: 'a' Symbol: 'u'  Code: 110001

Prior Symbol: 'a' Symbol: 'v'  Code: 001101

Prior Symbol: 'a' Symbol: 'w'  Code: 111001111

Prior Symbol: 'a' Symbol: 'x'  Code: 111001100

Prior Symbol: 'a' Symbol: 'y'  Code: 00001

Prior Symbol: 'a' Symbol: 'z'  Code: 00110000

Prior Symbol: 'b' Symbol: 27  Code: 101000

Prior Symbol: 'b' Symbol: ' '  Code: 0101

Prior Symbol: 'b' Symbol: '.'  Code: 101001

Prior Symbol: 'b' Symbol: 'a'  Code: 100

Prior Symbol: 'b' Symbol: 'b'  Code: 101010

Prior Symbol: 'b' Symbol: 'd'  Code: 1010110

Prior Symbol: 'b' Symbol: 'e'  Code: 00

Prior Symbol: 'b' Symbol: 'i'  Code: 1011

Prior Symbol: 'b' Symbol: 'l'  Code: 0100

Prior Symbol: 'b' Symbol: 'o'  Code: 110

Prior Symbol: 'b' Symbol: 'r'  Code: 1110

Prior Symbol: 'b' Symbol: 's'  Code: 1010111

Prior Symbol: 'b' Symbol: 'u'  Code: 1111

Prior Symbol: 'b' Symbol: 'y'  Code: 011

Prior Symbol: 'c' Symbol: 27  Code: 00010

Prior Symbol: 'c' Symbol: ' '  Code: 10000

Prior Symbol: 'c' Symbol: ','  Code: 010000

Prior Symbol: 'c' Symbol: '.'  Code: 0100011

Prior Symbol: 'c' Symbol: 'D'  Code: 0100110

Prior Symbol: 'c' Symbol: 'a'  Code: 110

Prior Symbol: 'c' Symbol: 'c'  Code: 010010

Prior Symbol: 'c' Symbol: 'e'  Code: 011

Prior Symbol: 'c' Symbol: 'h'  Code: 111

Prior Symbol: 'c' Symbol: 'i'  Code: 0101

Prior Symbol: 'c' Symbol: 'k'  Code: 1001

Prior Symbol: 'c' Symbol: 'l'  Code: 10001

Prior Symbol: 'c' Symbol: 'o'  Code: 101

Prior Symbol: 'c' Symbol: 'q'  Code: 0100010

Prior Symbol: 'c' Symbol: 'r'  Code: 00011

Prior Symbol: 'c' Symbol: 't'  Code: 001

Prior Symbol: 'c' Symbol: 'u'  Code: 0000

Prior Symbol: 'c' Symbol: 'y'  Code: 0100111

Prior Symbol: 'd' Symbol: 27  Code: 1010001

Prior Symbol: 'd' Symbol: ' '  Code: 11

Prior Symbol: 'd' Symbol: '''  Code: 01111010

Prior Symbol: 'd' Symbol: ','  Code: 101011

Prior Symbol: 'd' Symbol: '.'  Code: 0100

Prior Symbol: 'd' Symbol: ';'  Code: 01111011

Prior Symbol: 'd' Symbol: 'a'  Code: 1000

Prior Symbol: 'd' Symbol: 'd'  Code: 01010

Prior Symbol: 'd' Symbol: 'e'  Code: 00

Prior Symbol: 'd' Symbol: 'f'  Code: 10100000

Prior Symbol: 'd' Symbol: 'g'  Code: 10101011

Prior Symbol: 'd' Symbol: 'i'  Code: 1011

Prior Symbol: 'd' Symbol: 'l'  Code: 011111

Prior Symbol: 'd' Symbol: 'm'  Code: 10100001

Prior Symbol: 'd' Symbol: 'n'  Code: 1010100

Prior Symbol: 'd' Symbol: 'o'  Code: 0110

Prior Symbol: 'd' Symbol: 'r'  Code: 01110

Prior Symbol: 'd' Symbol: 's'  Code: 1001

Prior Symbol: 'd' Symbol: 'u'  Code: 101001

Prior Symbol: 'd' Symbol: 'v'  Code: 0111100

Prior Symbol: 'd' Symbol: 'w'  Code: 10101010

Prior Symbol: 'd' Symbol: 'y'  Code: 01011

Prior Symbol: 'e' Symbol: 27  Code: 101110011

Prior Symbol: 'e' Symbol: ' '  Code: 111

Prior Symbol: 'e' Symbol: '''  Code: 10111010

Prior Symbol: 'e' Symbol: ')'  Code: 100110000

Prior Symbol: 'e' Symbol: ','  Code: 000111

Prior Symbol: 'e' Symbol: '-'  Code: 10011001

Prior Symbol: 'e' Symbol: '.'  Code: 00110

Prior Symbol: 'e' Symbol: ';'  Code: 10011010

Prior Symbol: 'e' Symbol: 'a'  Code: 1000

Prior Symbol: 'e' Symbol: 'b'  Code: 0001100

Prior Symbol: 'e' Symbol: 'c'  Code: 10010

Prior Symbol: 'e' Symbol: 'd'  Code: 0000

Prior Symbol: 'e' Symbol: 'e'  Code: 10100

Prior Symbol: 'e' Symbol: 'f'  Code: 10111011

Prior Symbol: 'e' Symbol: 'g'  Code: 0001101

Prior Symbol: 'e' Symbol: 'h'  Code: 100110001

Prior Symbol: 'e' Symbol: 'i'  Code: 000100

Prior Symbol: 'e' Symbol: 'k'  Code: 10011011

Prior Symbol: 'e' Symbol: 'l'  Code: 0010

Prior Symbol: 'e' Symbol: 'm'  Code: 100111

Prior Symbol: 'e' Symbol: 'n'  Code: 010

Prior Symbol: 'e' Symbol: 'o'  Code: 001110

Prior Symbol: 'e' Symbol: 'p'  Code: 001111

Prior Symbol: 'e' Symbol: 'r'  Code: 110

Prior Symbol: 'e' Symbol: 's'  Code: 011

Prior Symbol: 'e' Symbol: 't'  Code: 10101

Prior Symbol: 'e' Symbol: 'u'  Code: 101110010

Prior Symbol: 'e' Symbol: 'v'  Code: 101100

Prior Symbol: 'e' Symbol: 'w'  Code: 101111

Prior Symbol: 'e' Symbol: 'x'  Code: 000101

Prior Symbol: 'e' Symbol: 'y'  Code: 101101

Prior Symbol: 'e' Symbol: 'z'  Code: 10111000

Prior Symbol: 'f' Symbol: 27  Code: 1110111

Prior Symbol: 'f' Symbol: ' '  Code: 10

Prior Symbol: 'f' Symbol: '.'  Code: 1110110

Prior Symbol: 'f' Symbol: 'a'  Code: 1111

Prior Symbol: 'f' Symbol: 'e'  Code: 000

Prior Symbol: 'f' Symbol: 'f'  Code: 0101

Prior Symbol: 'f' Symbol: 'i'  Code: 001

Prior Symbol: 'f' Symbol: 'l'  Code: 111010

Prior Symbol: 'f' Symbol: 'o'  Code: 110

Prior Symbol: 'f' Symbol: 'r'  Code: 011

Prior Symbol: 'f' Symbol: 't'  Code: 0100

Prior Symbol: 'f' Symbol: 'u'  Code: 11100

Prior Symbol: 'g' Symbol: 27  Code: 1111010

Prior Symbol: 'g' Symbol: ' '  Code: 10

Prior Symbol: 'g' Symbol: '''  Code: 1111011

Prior Symbol: 'g' Symbol: ','  Code: 111110

Prior Symbol: 'g' Symbol: '-'  Code: 0101010

Prior Symbol: 'g' Symbol: '.'  Code: 01011

Prior Symbol: 'g' Symbol: 'a'  Code: 1110

Prior Symbol: 'g' Symbol: 'e'  Code: 00

Prior Symbol: 'g' Symbol: 'g'  Code: 0101011

Prior Symbol: 'g' Symbol: 'h'  Code: 011

Prior Symbol: 'g' Symbol: 'i'  Code: 1101

Prior Symbol: 'g' Symbol: 'l'  Code: 111100

Prior Symbol: 'g' Symbol: 'o'  Code: 0100

Prior Symbol: 'g' Symbol: 'r'  Code: 111111

Prior Symbol: 'g' Symbol: 's'  Code: 11000

Prior Symbol: 'g' Symbol: 'u'  Code: 11001

Prior Symbol: 'g' Symbol: 'y'  Code: 010100

Prior Symbol: 'h' Symbol: 27  Code: 1011100

Prior Symbol: 'h' Symbol: ' '  Code: 100

Prior Symbol: 'h' Symbol: '''  Code: 10101000

Prior Symbol: 'h' Symbol: ','  Code: 10101001

Prior Symbol: 'h' Symbol: '-'  Code: 10101011

Prior Symbol: 'h' Symbol: '.'  Code: 101001

Prior Symbol: 'h' Symbol: 'a'  Code: 011

Prior Symbol: 'h' Symbol: 'e'  Code: 11

Prior Symbol: 'h' Symbol: 'i'  Code: 00

Prior Symbol: 'h' Symbol: 'n'  Code: 101011

Prior Symbol: 'h' Symbol: 'o'  Code: 010

Prior Symbol: 'h' Symbol: 'r'  Code: 101111

Prior Symbol: 'h' Symbol: 's'  Code: 10101010

Prior Symbol: 'h' Symbol: 't'  Code: 10110

Prior Symbol: 'h' Symbol: 'u'  Code: 101000

Prior Symbol: 'h' Symbol: 'y'  Code: 1011101

Prior Symbol: 'i' Symbol: 27  Code: 00011101

Prior Symbol: 'i' Symbol: ' '  Code: 0001111

Prior Symbol: 'i' Symbol: ','  Code: 100110100

Prior Symbol: 'i' Symbol: '.'  Code: 10011000

Prior Symbol: 'i' Symbol: 'a'  Code: 11010

Prior Symbol: 'i' Symbol: 'b'  Code: 100110101

Prior Symbol: 'i' Symbol: 'c'  Code: 1111

Prior Symbol: 'i' Symbol: 'd'  Code: 10000

Prior Symbol: 'i' Symbol: 'e'  Code: 1110

Prior Symbol: 'i' Symbol: 'f'  Code: 100111

Prior Symbol: 'i' Symbol: 'g'  Code: 10010

Prior Symbol: 'i' Symbol: 'k'  Code: 10011011

Prior Symbol: 'i' Symbol: 'l'  Code: 1100

Prior Symbol: 'i' Symbol: 'm'  Code: 10001

Prior Symbol: 'i' Symbol: 'n'  Code: 01

Prior Symbol: 'i' Symbol: 'o'  Code: 11011

Prior Symbol: 'i' Symbol: 'p'  Code: 000110

Prior Symbol: 'i' Symbol: 'r'  Code: 0000

Prior Symbol: 'i' Symbol: 's'  Code: 101

Prior Symbol: 'i' Symbol: 't'  Code: 001

Prior Symbol: 'i' Symbol: 'v'  Code: 00010

Prior Symbol: 'i' Symbol: 'x'  Code: 00011100

Prior Symbol: 'i' Symbol: 'z'  Code: 10011001

Prior Symbol: 'j' Symbol: 27  Code: 000

Prior Symbol: 'j' Symbol: 'a'  Code: 001

Prior Symbol: 'j' Symbol: 'e'  Code: 010

Prior Symbol: 'j' Symbol: 'o'  Code: 1

Prior Symbol: 'j' Symbol: 'u'  Code: 011

Prior Symbol: 'k' Symbol: 27  Code: 0000

Prior Symbol: 'k' Symbol: ' '  Code: 01

Prior Symbol: 'k' Symbol: '''  Code: 10000

Prior Symbol: 'k' Symbol: ','  Code: 10011

Prior Symbol: 'k' Symbol: '.'  Code: 0001

Prior Symbol: 'k' Symbol: 'e'  Code: 11

Prior Symbol: 'k' Symbol: 'i'  Code: 101

Prior Symbol: 'k' Symbol: 'l'  Code: 100100

Prior Symbol: 'k' Symbol: 'n'  Code: 10001

Prior Symbol: 'k' Symbol: 's'  Code: 001

Prior Symbol: 'k' Symbol: 'y'  Code: 100101

Prior Symbol: 'l' Symbol: 27  Code: 0011100

Prior Symbol: 'l' Symbol: ' '  Code: 110

Prior Symbol: 'l' Symbol: '''  Code: 00111100

Prior Symbol: 'l' Symbol: ','  Code: 001101

Prior Symbol: 'l' Symbol: '-'  Code: 00111101

Prior Symbol: 'l' Symbol: '.'  Code: 00100

Prior Symbol: 'l' Symbol: 'a'  Code: 000

Prior Symbol: 'l' Symbol: 'b'  Code: 0011101

Prior Symbol: 'l' Symbol: 'c'  Code: 00111111

Prior Symbol: 'l' Symbol: 'd'  Code: 10111

Prior Symbol: 'l' Symbol: 'e'  Code: 111

Prior Symbol: 'l' Symbol: 'f'  Code: 010110

Prior Symbol: 'l' Symbol: 'i'  Code: 011

Prior Symbol: 'l' Symbol: 'k'  Code: 10110110

Prior Symbol: 'l' Symbol: 'l'  Code: 100

Prior Symbol: 'l' Symbol: 'm'  Code: 010111

Prior Symbol: 'l' Symbol: 'n'  Code: 00111110

Prior Symbol: 'l' Symbol: 'o'  Code: 1010

Prior Symbol: 'l' Symbol: 'p'  Code: 00101

Prior Symbol: 'l' Symbol: 'r'  Code: 10110111

Prior Symbol: 'l' Symbol: 's'  Code: 01010

Prior Symbol: 'l' Symbol: 't'  Code: 001100

Prior Symbol: 'l' Symbol: 'u'  Code: 1011010

Prior Symbol: 'l' Symbol: 'v'  Code: 101100

Prior Symbol: 'l' Symbol: 'y'  Code: 0100

Prior Symbol: 'm' Symbol: 27  Code: 101010

Prior Symbol: 'm' Symbol: ' '  Code: 111

Prior Symbol: 'm' Symbol: '''  Code: 1010110

Prior Symbol: 'm' Symbol: '.'  Code: 110101

Prior Symbol: 'm' Symbol: ';'  Code: 1010111

Prior Symbol: 'm' Symbol: 'a'  Code: 00

Prior Symbol: 'm' Symbol: 'b'  Code: 10100

Prior Symbol: 'm' Symbol: 'e'  Code: 01

Prior Symbol: 'm' Symbol: 'i'  Code: 1100

Prior Symbol: 'm' Symbol: 'm'  Code: 10110

Prior Symbol: 'm' Symbol: 'o'  Code: 1000

Prior Symbol: 'm' Symbol: 'p'  Code: 1001

Prior Symbol: 'm' Symbol: 's'  Code: 10111

Prior Symbol: 'm' Symbol: 'u'  Code: 11011

Prior Symbol: 'm' Symbol: 'y'  Code: 110100

Prior Symbol: 'n' Symbol: 27  Code: 0100000

Prior Symbol: 'n' Symbol: ' '  Code: 10

Prior Symbol: 'n' Symbol: '''  Code: 0100011

Prior Symbol: 'n' Symbol: ','  Code: 111100

Prior Symbol: 'n' Symbol: '-'  Code: 011011010

Prior Symbol: 'n' Symbol: '.'  Code: 01100

Prior Symbol: 'n' Symbol: ';'  Code: 011011011

Prior Symbol: 'n' Symbol: 'a'  Code: 11111

Prior Symbol: 'n' Symbol: 'b'  Code: 011011100

Prior Symbol: 'n' Symbol: 'c'  Code: 01001

Prior Symbol: 'n' Symbol: 'd'  Code: 110

Prior Symbol: 'n' Symbol: 'e'  Code: 001

Prior Symbol: 'n' Symbol: 'f'  Code: 01000101

Prior Symbol: 'n' Symbol: 'g'  Code: 000

Prior Symbol: 'n' Symbol: 'i'  Code: 01111

Prior Symbol: 'n' Symbol: 'j'  Code: 011011101

Prior Symbol: 'n' Symbol: 'k'  Code: 1111010

Prior Symbol: 'n' Symbol: 'l'  Code: 01101100

Prior Symbol: 'n' Symbol: 'm'  Code: 011011110

Prior Symbol: 'n' Symbol: 'n'  Code: 01110

Prior Symbol: 'n' Symbol: 'o'  Code: 1111011

Prior Symbol: 'n' Symbol: 'r'  Code: 011011111

Prior Symbol: 'n' Symbol: 's'  Code: 0101

Prior Symbol: 'n' Symbol: 't'  Code: 1110

Prior Symbol: 'n' Symbol: 'u'  Code: 0100001

Prior Symbol: 'n' Symbol: 'v'  Code: 0110100

Prior Symbol: 'n' Symbol: 'y'  Code: 0110101

Prior Symbol: 'n' Symbol: 'z'  Code: 01000100

Prior Symbol: 'o' Symbol: 27  Code: 101010011

Prior Symbol: 'o' Symbol: ' '  Code: 001

Prior Symbol: 'o' Symbol: ','  Code: 01001111

Prior Symbol: 'o' Symbol: '-'  Code: 01001110

Prior Symbol: 'o' Symbol: '.'  Code: 0100110

Prior Symbol: 'o' Symbol: 'B'  Code: 101010010

Prior Symbol: 'o' Symbol: 'a'  Code: 100001

Prior Symbol: 'o' Symbol: 'b'  Code: 110111

Prior Symbol: 'o' Symbol: 'c'  Code: 100000

Prior Symbol: 'o' Symbol: 'd'  Code: 110101

Prior Symbol: 'o' Symbol: 'e'  Code: 1010101

Prior Symbol: 'o' Symbol: 'f'  Code: 000

Prior Symbol: 'o' Symbol: 'g'  Code: 1101000

Prior Symbol: 'o' Symbol: 'h'  Code: 1101001

Prior Symbol: 'o' Symbol: 'i'  Code: 1101101

Prior Symbol: 'o' Symbol: 'k'  Code: 010010

Prior Symbol: 'o' Symbol: 'l'  Code: 0101

Prior Symbol: 'o' Symbol: 'm'  Code: 1100

Prior Symbol: 'o' Symbol: 'n'  Code: 111

Prior Symbol: 'o' Symbol: 'o'  Code: 10100

Prior Symbol: 'o' Symbol: 'p'  Code: 01000

Prior Symbol: 'o' Symbol: 'r'  Code: 011

Prior Symbol: 'o' Symbol: 's'  Code: 10001

Prior Symbol: 'o' Symbol: 't'  Code: 10010

Prior Symbol: 'o' Symbol: 'u'  Code: 1011

Prior Symbol: 'o' Symbol: 'v'  Code: 101011

Prior Symbol: 'o' Symbol: 'w'  Code: 10011

Prior Symbol: 'o' Symbol: 'x'  Code: 10101000

Prior Symbol: 'o' Symbol: 'y'  Code: 1101100

Prior Symbol: 'p' Symbol: 27  Code: 011011

Prior Symbol: 'p' Symbol: ' '  Code: 000

Prior Symbol: 'p' Symbol: '-'  Code: 1010010

Prior Symbol: 'p' Symbol: '.'  Code: 101000

Prior Symbol: 'p' Symbol: 'a'  Code: 001

Prior Symbol: 'p' Symbol: 'e'  Code: 110

Prior Symbol: 'p' Symbol: 'h'  Code: 1111

Prior Symbol: 'p' Symbol: 'i'  Code: 1011

Prior Symbol: 'p' Symbol: 'l'  Code: 010

Prior Symbol: 'p' Symbol: 'm'  Code: 1010011

Prior Symbol: 'p' Symbol: 'o'  Code: 0111

Prior Symbol: 'p' Symbol: 'p'  Code: 11101

Prior Symbol: 'p' Symbol: 'r'  Code: 100

Prior Symbol: 'p' Symbol: 's'  Code: 01100

Prior Symbol: 'p' Symbol: 't'  Code: 11100

Prior Symbol: 'p' Symbol: 'u'  Code: 10101

Prior Symbol: 'p' Symbol: 'y'  Code: 011010

Prior Symbol: 'q' Symbol: 27  Code: 0

Prior Symbol: 'q' Symbol: 'u'  Code: 1

Prior Symbol: 'r' Symbol: 27  Code: 10011111

Prior Symbol: 'r' Symbol: ' '  Code: 111

Prior Symbol: 'r' Symbol: '''  Code: 1001110

Prior Symbol: 'r' Symbol: ')'  Code: 100111100

Prior Symbol: 'r' Symbol: ','  Code: 100100

Prior Symbol: 'r' Symbol: '-'  Code: 11001100

Prior Symbol: 'r' Symbol: '.'  Code: 10001

Prior Symbol: 'r' Symbol: ';'  Code: 100111101

Prior Symbol: 'r' Symbol: 'a'  Code: 1101

Prior Symbol: 'r' Symbol: 'b'  Code: 11001101

Prior Symbol: 'r' Symbol: 'c'  Code: 100001

Prior Symbol: 'r' Symbol: 'd'  Code: 11000

Prior Symbol: 'r' Symbol: 'e'  Code: 101

Prior Symbol: 'r' Symbol: 'f'  Code: 110011111

Prior Symbol: 'r' Symbol: 'g'  Code: 100101

Prior Symbol: 'r' Symbol: 'i'  Code: 010

Prior Symbol: 'r' Symbol: 'k'  Code: 110010

Prior Symbol: 'r' Symbol: 'l'  Code: 00100

Prior Symbol: 'r' Symbol: 'm'  Code: 00101

Prior Symbol: 'r' Symbol: 'n'  Code: 01100

Prior Symbol: 'r' Symbol: 'o'  Code: 000

Prior Symbol: 'r' Symbol: 'p'  Code: 11001110

Prior Symbol: 'r' Symbol: 'r'  Code: 100110

Prior Symbol: 'r' Symbol: 's'  Code: 0111

Prior Symbol: 'r' Symbol: 't'  Code: 0011

Prior Symbol: 'r' Symbol: 'u'  Code: 100000

Prior Symbol: 'r' Symbol: 'v'  Code: 110011110

Prior Symbol: 'r' Symbol: 'y'  Code: 01101

Prior Symbol: 's' Symbol: 27  Code: 10011100

Prior Symbol: 's' Symbol: ' '  Code: 0

Prior Symbol: 's' Symbol: '"'  Code: 100111100

Prior Symbol: 's' Symbol: '''  Code: 100111101

Prior Symbol: 's' Symbol: ','  Code: 111011

Prior Symbol: 's' Symbol: '.'  Code: 1000

Prior Symbol: 's' Symbol: ';'  Code: 11101011

Prior Symbol: 's' Symbol: 'a'  Code: 110011

Prior Symbol: 's' Symbol: 'b'  Code: 100111110

Prior Symbol: 's' Symbol: 'c'  Code: 10010

Prior Symbol: 's' Symbol: 'e'  Code: 1101

Prior Symbol: 's' Symbol: 'h'  Code: 11000

Prior Symbol: 's' Symbol: 'i'  Code: 11100

Prior Symbol: 's' Symbol: 'k'  Code: 100111111

Prior Symbol: 's' Symbol: 'l'  Code: 1110100

Prior Symbol: 's' Symbol: 'm'  Code: 111010100

Prior Symbol: 's' Symbol: 'n'  Code: 111010101

Prior Symbol: 's' Symbol: 'o'  Code: 11110

Prior Symbol: 's' Symbol: 'p'  Code: 1001101

Prior Symbol: 's' Symbol: 's'  Code: 11111

Prior Symbol: 's' Symbol: 't'  Code: 101

Prior Symbol: 's' Symbol: 'u'  Code: 110010

Prior Symbol: 's' Symbol: 'w'  Code: 10011101

Prior Symbol: 's' Symbol: 'y'  Code: 1001100

Prior Symbol: 't' Symbol: 27  Code: 11000011

Prior Symbol: 't' Symbol: ' '  Code: 111

Prior Symbol: 't' Symbol: '''  Code: 11000100

Prior Symbol: 't' Symbol: ','  Code: 0111100

Prior Symbol: 't' Symbol: '-'  Code: 01111110

Prior Symbol: 't' Symbol: '.'  Code: 01101

Prior Symbol: 't' Symbol: ';'  Code: 110000100

Prior Symbol: 't' Symbol: 'a'  Code: 0100

Prior Symbol: 't' Symbol: 'b'  Code: 110000101

Prior Symbol: 't' Symbol: 'c'  Code: 11000101

Prior Symbol: 't' Symbol: 'e'  Code: 101

Prior Symbol: 't' Symbol: 'h'  Code: 00

Prior Symbol: 't' Symbol: 'i'  Code: 1101

Prior Symbol: 't' Symbol: 'l'  Code: 0111101

Prior Symbol: 't' Symbol: 'm'  Code: 01111111

Prior Symbol: 't' Symbol: 'n'  Code: 0111110

Prior Symbol: 't' Symbol: 'o'  Code: 100

Prior Symbol: 't' Symbol: 'r'  Code: 11001

Prior Symbol: 't' Symbol: 's'  Code: 0101

Prior Symbol: 't' Symbol: 't'  Code: 01100

Prior Symbol: 't' Symbol: 'u'  Code: 01110

Prior Symbol: 't' Symbol: 'w'  Code: 1100000

Prior Symbol: 't' Symbol: 'y'  Code: 1100011

Prior Symbol: 'u' Symbol: 27  Code: 1001100

Prior Symbol: 'u' Symbol: ' '  Code: 100000

Prior Symbol: 'u' Symbol: 'a'  Code: 100111

Prior Symbol: 'u' Symbol: 'b'  Code: 100001

Prior Symbol: 'u' Symbol: 'c'  Code: 10001

Prior Symbol: 'u' Symbol: 'd'  Code: 11100

Prior Symbol: 'u' Symbol: 'e'  Code: 11101

Prior Symbol: 'u' Symbol: 'g'  Code: 11110

Prior Symbol: 'u' Symbol: 'i'  Code: 10010

Prior Symbol: 'u' Symbol: 'k'  Code: 1001101

Prior Symbol: 'u' Symbol: 'l'  Code: 0100

Prior Symbol: 'u' Symbol: 'm'  Code: 111111

Prior Symbol: 'u' Symbol: 'n'  Code: 110

Prior Symbol: 'u' Symbol: 'o'  Code: 11111010

Prior Symbol: 'u' Symbol: 'p'  Code: 0101

Prior Symbol: 'u' Symbol: 'r'  Code: 00

Prior Symbol: 'u' Symbol: 's'  Code: 011

Prior Symbol: 'u' Symbol: 't'  Code: 101

Prior Symbol: 'u' Symbol: 'v'  Code: 11111011

Prior Symbol: 'u' Symbol: 'y'  Code: 1111100

Prior Symbol: 'v' Symbol: 27  Code: 00010

Prior Symbol: 'v' Symbol: 'a'  Code: 001

Prior Symbol: 'v' Symbol: 'e'  Code: 1

Prior Symbol: 'v' Symbol: 'i'  Code: 01

Prior Symbol: 'v' Symbol: 'o'  Code: 0000

Prior Symbol: 'v' Symbol: 's'  Code: 000110

Prior Symbol: 'v' Symbol: 'y'  Code: 000111

Prior Symbol: 'w' Symbol: 27  Code: 011101

Prior Symbol: 'w' Symbol: ' '  Code: 001

Prior Symbol: 'w' Symbol: '.'  Code: 011100

Prior Symbol: 'w' Symbol: 'a'  Code: 010

Prior Symbol: 'w' Symbol: 'e'  Code: 1110

Prior Symbol: 'w' Symbol: 'h'  Code: 000

Prior Symbol: 'w' Symbol: 'i'  Code: 10

Prior Symbol: 'w' Symbol: 'l'  Code: 011110

Prior Symbol: 'w' Symbol: 'm'  Code: 011111

Prior Symbol: 'w' Symbol: 'n'  Code: 11111

Prior Symbol: 'w' Symbol: 'o'  Code: 110

Prior Symbol: 'w' Symbol: 'r'  Code: 0110

Prior Symbol: 'w' Symbol: 's'  Code: 11110

Prior Symbol: 'x' Symbol: 27  Code: 10

Prior Symbol: 'x' Symbol: ' '  Code: 0110

Prior Symbol: 'x' Symbol: ','  Code: 0111

Prior Symbol: 'x' Symbol: '-'  Code: 1100

Prior Symbol: 'x' Symbol: 'a'  Code: 111

Prior Symbol: 'x' Symbol: 'e'  Code: 00

Prior Symbol: 'x' Symbol: 'i'  Code: 010

Prior Symbol: 'x' Symbol: 't'  Code: 1101

Prior Symbol: 'y' Symbol: 27  Code: 01010

Prior Symbol: 'y' Symbol: ' '  Code: 1

Prior Symbol: 'y' Symbol: '''  Code: 010010

Prior Symbol: 'y' Symbol: ','  Code: 0001

Prior Symbol: 'y' Symbol: '.'  Code: 0111

Prior Symbol: 'y' Symbol: ';'  Code: 011001

Prior Symbol: 'y' Symbol: '?'  Code: 0100110

Prior Symbol: 'y' Symbol: 'a'  Code: 0100111

Prior Symbol: 'y' Symbol: 'b'  Code: 0110000

Prior Symbol: 'y' Symbol: 'd'  Code: 000001

Prior Symbol: 'y' Symbol: 'e'  Code: 0010

Prior Symbol: 'y' Symbol: 'f'  Code: 0110001

Prior Symbol: 'y' Symbol: 'i'  Code: 000010

Prior Symbol: 'y' Symbol: 'l'  Code: 01000

Prior Symbol: 'y' Symbol: 'm'  Code: 000000

Prior Symbol: 'y' Symbol: 'n'  Code: 01011

Prior Symbol: 'y' Symbol: 'o'  Code: 01101

Prior Symbol: 'y' Symbol: 's'  Code: 0011

Prior Symbol: 'y' Symbol: 'w'  Code: 000011

Prior Symbol: 'z' Symbol: 27  Code: 100

Prior Symbol: 'z' Symbol: ' '  Code: 1110

Prior Symbol: 'z' Symbol: '.'  Code: 1111

Prior Symbol: 'z' Symbol: 'a'  Code: 000

Prior Symbol: 'z' Symbol: 'e'  Code: 001

Prior Symbol: 'z' Symbol: 'i'  Code: 110

Prior Symbol: 'z' Symbol: 'l'  Code: 010

Prior Symbol: 'z' Symbol: 'o'  Code: 101

Prior Symbol: 'z' Symbol: 'z'  Code: 011

Prior Symbol: '{' Symbol: 27  Code: 1

Prior Symbol: '|' Symbol: 27  Code: 1

Prior Symbol: '}' Symbol: 27  Code: 1

Prior Symbol: '~' Symbol: 27  Code: 1

Prior Symbol: 127 Symbol: 27  Code: 1

Table B.C.7/J.94 – English-language Program Description Decode Table

0  1

1  0

2  1

3  44

4  1

5  46

6  1

7  48

8  1

9  50

10  1

11  52

12  1

13  54

14  1

15  56

16  1

17  58

18  1

19  60

20  1

21  62

22  1

23  64

24  1

25  66

26  1

27  68

28  1

29  70

30  1

31  72

32  1

33  74

34  1

35  76

36  1

37  78

38  1

39  80

40  1

41  82

42  1

43  84

44  1

45  86

46  1

47  88

48  1

49  90

50  1

51  92

52  1

53  94

54  1

55  96

56  1

57  98

58  1

59  100

60  1

61  102

62  1

63  104

64  1

65  106

66  1

67  222

68  1

69  224

70  1

71  234

72  1

73  236

74  1

75  238

76  1

77  240

78  1

79  242

80  1

81  248

82  1

83  250

84  1

85  252

86  1

87  254

88  2

89  0

90  2

91  4

92  2

93  22

94  2

95  32

96  2

97  34

98  2

99  44

100  2

101  50

102  2

103  56

104  2

105  60

106  2

107  64

108  2

109  68

110  2

111  70

112  2

113  74

114  2

115  76

116  2

117  84

118  2

119  86

120  2

121  88

122  2

123  90

124  2

125  92

126  2

127  94

128  2

129  96

130  2

131  98

132  2

133  118

134  2

135  132

136  2

137  148

138  2

139  162

140  2

141  178

142  2

143  186

144  2

145  200

146  2

147  210

148  2

149  222

150  2

151  234

152  2

153  242

154  2

155  252

156  3

157  8

158  3

159  16

160  3

161  26

162  3

163  40

164  3

165  42

166  3

167  52

168  3

169  74

170  3

171  90

172  3

173  94

174  3

175  100

176  3

177  110

178  3

179  112

180  3

181  114

182  3

183  116

184  3

185  118

186  3

187  120

188  3

189  122

190  3

191  124

192  3

193  126

194  3

195  128

196  3

197  180

198  3

199  206

200  3

201  240

202  4

203  26

204  4

205  88

206  4

207  110

208  4

209  142

210  4

211  172

212  4

213  216

214  4

215  224

216  4

217  244

218  5

219  36

220  5

221  64

222  5

223  118

224  5

225  174

226  5

227  206

228  5

229  208

230  6

231  6

232  6

233  52

234  6

235  96

236  6

237  134

238  6

239  146

240  6

241  170

242  6

243  184

244  6

245  220

246  6

247  236 248  6

249  238

250  6

251  240

252  6

253  242

254  6

255  244

256  20

257  21

258  155

259  214

260  201

261  207

262  215

263  199

264  1

265  162

266  206

267  203

268  2

269  3

270  197

271  204

272  198

273  200

274  4

275  196

276  5

277  194

278  6

279  195

280  210

281  7

282  211

283  8

284  202

285  212

286  9

287  205

288  208

289  10

290  193

291  11

292  12

293  13

294  14

295  15

296  16

297  17

298  18

299  19

300  155

301  155

302  155

303  155

304  155

305  155

306  155

307  155

308  155

309  155

310  155

311  155

312  155

313  155

314  155

315  155

316  155

317  155

318  155

319  155

320  155

321  155

322  155

323  155

324  155

325  155

326  155

327  155

328  155

329  155

330  155

331  155

332  155

333  155

334  155

335  155

336  155

337  155

338  155

339  155

340  155

341  155

342  155

343  155

344  155

345  155

346  155

347  155

348  155

349  155

350  155

351  155

352  155

353  155

354  155

355  155

356  155

357  155

358  155

359  155

360  155

361  155

362  56

363  57

364  173

365  175

366  183

367  218

368  168

369  179

370  181

371  1

372  2

373  155

374  180

375  241

376  162

377  213

378  214

379  217

380  3

381  4

382  5

383  207

384  6

385  201

386  249

387  234

388  235

389  245

390  246

391  7

392  8

393  9

394  178

395  197

396  198

397  177

398  10

399  238

400  203

401  11

402  212

403  12

404  196

405  200

406  210

407  13

408  14

409  15

410  199

411  202

412  206

413  208

414  215

415  16

416  194

417  17

418  204

419  236

420  229

421  231

422  18

423  205

424  19

425  20

426  195

427  21

428  22

429  23

430  237

431  24

432  25

433  242

434  26

435  211

436  27

437  28

438  228

439  29

440  193

441  227

442  30

443  233

444  240

445  226

446  247

447  31

448  243

449  230

450  32

451  33

452  34

453  232

454  239

455  35

456  36

457  37

458  38

459  39

460  40

461  41

462  42

463  244

464  43

465  44

466  45

467  46

468  47

469  225

470  48

471  49

472  50

473  51

474  52

475  53

476  54

477  55

478  155

479  155

480  3

481  4

482  128

483  174

484  200

485  212

486  1

487  2

488  155

489  160

490  155

491  155

492  155

493  155

494  155

495  155

496  155

497  155

498  2

499  243

500  160

501  244

502  155

503  1

504  155

505  155

506  172

507  155

508  155

509  155

510  155

511  155

512  1

513  160

514  155

515  162

516  7

517  8

518  226

519  228

520  229

521  230

522  160

523  242

524  225

525  1

526  2

527  243

528  227

529  3

530  4

531  5

532  155

533  6

534  4

535  128

536  202

537  211

538  162

539  1

540  155

541  2

542  3

543  160

544  155

545  160

546  3

547  4

548  155

549  183

550  244

551  160

552  176

553  243

554  1

555  2

556  185

557  2

558  184

559  155

560  160

561  1

562  174

563  2

564  182

565  155

566  1

567  160

568  160

569  1

570  155

571  176

572  174

573  1

574  155

575  160

576  174

577  1

578  160

579  155

580  155

581  155

582  155

583  1

584  172

585  174

586  155

587  155

588  2

589  3

590  155

591  160

592  181

593  182

594  184

595  1

596  155

597  160

598  155

599  160

600  155

601  155

602  155

603  155

604  155

605  155

606  155

607  160

608  155

609  155

610  8

611  9

612  230

613  245

614  243

615  244

616  155

617  228

618  1

619  237

620  2

621  3

622  4

623  242

624  5

625  6

626  236

627  238

628  7

629  160

630  5

631  6

632  155

633  236

634  245

635  1

636  2

637  225

638  239

639  229

640  233

641  242

642  3

643  4

644  6

645  7

646  155

647  233

648  249

649  242

650  245

651  1

652  2

653  3

654  236

655  239

656  225

657  4

658  232

659  5

660  5

661  6

662  249

663  242

664  245

665  155

666  229

667  239

668  1

669  2

670  233

671  225

672  3

673  4

674  6

675  7

676  225

677  233

678  238

679  246

680  228

681  236

682  243

683  1

684  2

685  242

686  3

687  4

688  155

689  5

690  2

691  3

692  229

693  236

694  155

695  239

696  1

697  242

698  5

699  6

700  245

701  239

702  155

703  236

704  233

705  1

706  225

707  242

708  2

709  229

710  3

711  4

712  3

713  4

714  155

715  229

716  233

717  245

718  225

719  1

720  239

721  2

722  4

723  5

724  160

725  201

726  243

727  155

728  174

729  242

730  1

731  2

732  3

733  238

734  239

735  5

736  155

737  174

738  233

739  229

740  1

741  245

742  2

743  225

744  3

745  4

746  229

747  3

748  225

749  233

750  242

751  155

752  1

753  2

754  3

755  4

756  155

757  233

758  245

759  1

760  229

761  2

762  239

763  225

764  225

765  5

766  155

767  227

768  239

769  1

770  245

771  229

772  2

773  3

774  233

775  4

776  229

777  3

778  155

779  233

780  1

781  225

782  239

783  2

784  3

785  4

786  167

787  238

788  236

789  242

790  243

791  1

792  155

793  2

794  225

795  6

796  155

797  232

798  233

799  1

800  242

801  236

802  2

803  239

804  3

805  229

806  4

807  5

808  155

809  155

810  3

811  4

812  155

813  174

814  1

815  233

816  2

817  225

818  229

819  239

820  9

821  10

822  246

823  249

824  1

825  174

826  227

827  233

828  245

829  155

830  229

831  239

832  2

833  3

834  225

835  4

836  232

837  5

838  6

839  244

840  7

841  8

842  232

843  7

844  229

845  247

846  214

847  225

848  155

849  233

850  242

851  1

852  2

853  3

854  4

855  239

856  5

857  6

858  174

859  1

860  155

861  238

862  233

863  2

864  229

865  155

866  160

867  1

868  3

869  4

870  155

871  232

872  229

873  225

874  239

875  1

876  233

877  2

878  155

879  155

880  155

881  239

882  155

883  155

884  155

885  155

886  155

887  155

888  155

889  155

890  155

891  155

892  155

893  155

894  155

895  155

896  24

897  25

898  232

899  239

900  248

901  155

902  167

903  247

904  250

905  1

906  2

907  3

908  4

909  229

910  174

911  5

912  230

913  226

914  6

915  246

916  235

917  245

918  233

919  7

920  240

921  249

922  231

923  8

924  9

925  228

926  10

927  227

928  11

929  237

930  12

931  243

932  13

933  14

934  15

935  236

936  16

937  244

938  17

939  18

940  242

941  160

942  19

943  20

944  21

945  238

946  22

947  23

948  11

949  12

950  228

951  243

952  155

953  174

954  226

955  1

956  2

957  3

958  236

959  160

960  4

961  233

962  242

963  245

964  5

965  249

966  225

967  6

968  239

969  7

970  229

971  8

972  9

973  10

974  15

975  16

976  241

977  174

978  196

979  249

980  172

981  1

982  227

983  2

984  155

985  242

986  3

987  4

988  160

989  236

990  245

991  5

992  6

993  233

994  7

995  235

996  8

997  244

998  9

999  229

1000  10

1001  239

1002  225

1003  232

1004  11

1005  12

1006  13

1007  14

1008  19

1009  20

1010  167

1011  187

1012  230

1013  237

1014  247

1015  231

1016  246

1017  1

1018  2

1019  155

1020  238

1021  3

1022  4

1023  236

1024  5

1025  245

1026  6

1027  172

1028  228

1029  249

1030  242

1031  7

1032  8

1033  9

1034  174

1035  10

1036  239

1037  11

1038  225

1039  243

1040  12

1041  233

1042  13

1043  14

1044  15

1045  16

1046  229

1047  17

1048  18

1049  160

1050  29

1051  30

1052  169

1053  232

1054  245

1055  155

1056  1

1057  173

1058  187

1059  235

1060  250

1061  2

1062  167

1063  230

1064  226

1065  231

1066  3

1067  4

1068  5

1069  6

1070  233

1071  248

1072  7

1073  172

1074  239

1075  240

1076  8

1077  237

1078  246

1079  249

1080  9

1081  247

1082  10

1083  11

1084  174

1085  12

1086  227

1087  13

1088  229

1089  244

1090  14

1091  15

1092  228

1093  16

1094  236

1095  17

1096  225

1097  18

1098  19

1099  20

1100  21

1101  22

1102  238

1103  243

1104  23

1105  24

1106  242

1107  160

1108  25

1109  26

1110  27

1111  28

1112  9

1113  10

1114  174

1115  155

1116  236

1117  1

1118  245

1119  2

1120  244

1121  230

1122  3

1123  225

1124  229

1125  233

1126  4

1127  242

1128  239

1129  5

1130  6

1131  7

1132  160

1133  8

1134  14

1135  15

1136  173

1137  231

1138  155

1139  167

1140  249

1141  1

1142  236

1143  2

1144  172

1145  242

1146  3

1147  174

1148  243

1149  245

1150  4

1151  5

1152  239

1153  6

1154  7

1155  233

1156  225

1157  8

1158  9

1159  232

1160  10

1161  11

1162  229

1163  12

1164  160

1165  13

1166  13

1167  14

1168  167

1169  172

1170  243

1171  173

1172  1

1173  2

1174  155

1175  249

1176  245

1177  174

1178  3

1179  238

1180  4

1181  242

1182  5

1183  6

1184  244

1185  7

1186  8

1187  9

1188  239

1189  225

1190  160

1191  10

1192  233

1193  11

1194  12

1195  229

1196  20

1197  21

1198  172

1199  226

1200  248

1201  155

1202  174

1203  250

1204  1

1205  235

1206  2

1207  160

1208  3

1209  4

1210  240

1211  5

1212  6

1213  230

1214  246

1215  7

1216  228

1217  237

1218  231

1219  8

1220  225

1221  239

1222  242

1223  9

1224  10

1225  11

1226  236

1227  12

1228  229

1229  227

1230  13

1231  244

1232  14

1233  243

1234  15

1235  16

1236  17

1237  238

1238  18

1239  19

1240  3

1241  239

1242  155

1243  225

1244  229

1245  245

1246  1

1247  2

1248  8

1249  9

1250  236

1251  249

1252  167

1253  238

1254  1

1255  172

1256  155

1257  174

1258  2

1259  3

1260  4

1261  243

1262  5

1263  233

1264  6

1265  160

1266  7

1267  229

1268  22

1269  23

1270  167

1271  173

1272  238

1273  227

1274  235

1275  242

1276  155

1277  226

1278  1

1279  2

1280  245

1281  3

1282  244

1283  172

1284  4

1285  5

1286  230

1287  237

1288  246

1289  6

1290  174

1291  240

1292  7

1293  8

1294  243

1295  9

1296  10

1297  228

1298  11

1299  12

1300  249

1301  13

1302  239

1303  14

1304  225

1305  15

1306  16

1307  233

1308  236

1309  17

1310  160

1311  229

1312  18

1313  19

1314  20

1315  21

1316  12

1317  13

1318  167

1319  187

1320  155

1321  1

1322  249

1323  174

1324  226

1325  2

1326  237

1327  243

1328  3

1329  245

1330  239

1331  240

1332  4

1333  5

1334  233

1335  6

1336  7

1337  8

1338  9

1339  160

1340  225

1341  229

1342  10

1343  11

1344  25

1345  26

1346  173

1347  187

1348  226

1349  234

1350  237

1351  242

1352  250

1353  230

1354  236

1355  1

1356  2

1357  3

1358  155

1359  245

1360  4

1361  167

1362  246

1363  249

1364  5

1365  6

1366  235

1367  239

1368  7

1369  8

1370  9

1371  10

1372  172

1373  11

1374  12

1375  227

1376  174

1377  13

1378  238

1379  233

1380  14

1381  225

1382  15

1383  243

1384  16

1385  17

1386  244

1387  18

1388  231

1389  229

1390  19

1391  20

1392  228

1393  21

1394  22

1395  23

1396  160

1397  24

1398  26

1399  27

1400  194

1401  155

1402  173

1403  172

1404  248

1405  1

1406  174

1407  2

1408  3

1409  229

1410  231

1411  232

1412  249

1413  233

1414  235

1415  4

1416  227

1417  225

1418  5

1419  246

1420  6

1421  228

1422  7

1423  226

1424  240

1425  8

1426  9

1427  243

1428  244

1429  247

1430  239

1431  10

1432  11

1433  12

1434  13

1435  236

1436  14

1437  15

1438  16

1439  245

1440  237

1441  17

1442  230

1443  160

1444  18

1445  242

1446  19

1447  20

1448  21

1449  238

1450  22

1451  23

1452  24

1453  25

1454  14

1455  15

1456  173

1457  237

1458  249

1459  155

1460  174

1461  1

1462  243

1463  2

1464  3

1465  245

1466  244

1467  240

1468  4

1469  239

1470  5

1471  233

1472  6

1473  232

1474  160

1475  225

1476  236

1477  7

1478  242

1479  8

1480  229

1481  9

1482  10

1483  11

1484  12

1485  13

1486  155

1487  245

1488  25

1489  26

1490  169

1491  187

1492  246

1493  230

1494  1

1495  155

1496  173

1497  226

1498  240

1499  2

1500  167

1501  3

1502  4

1503  5

1504  245

1505  227

1506  172

1507  231

1508  242

1509  6

1510  235

1511  7

1512  236

1513  237

1514  238

1515  249

1516  8

1517  174

1518  9

1519  10

1520  228

1521  11

1522  12

1523  244

1524  13

1525  243

1526  14

1527  15

1528  16

1529  225

1530  239

1531  17

1532  233

1533  18

1534  19

1535  229

1536  20

1537  160

1538  21

1539  22

1540  23

1541  24

1542  160

1543  22

1544  162

1545  167

1546  226

1547  235

1548  237

1549  238

1550  155

1551  247

1552  1

1553  2

1554  3

1555  187

1556  249

1557  240

1558  4

1559  5

1560  236

1561  6

1562  7

1563  8

1564  245

1565  225

1566  9

1567  172

1568  227

1569  10

1570  232

1571  11

1572  233

1573  12

1574  239

1575  243

1576  174

1577  13

1578  14

1579  229

1580  15

1581  16

1582  17

1583  244

1584  18

1585  19

1586  20

1587  21

1588  20

1589  21

1590  187

1591  226

1592  173

1593  237

1594  1

1595  155

1596  167

1597  227

1598  172

1599  236

1600  238

1601  2

1602  247

1603  3

1604  4

1605  249

1606  5

1607  6

1608  7

1609  8

1610  244

1611  174

1612  245

1613  9

1614  10

1615  242

1616  225

1617  243

1618  11

1619  12

1620  13

1621  233

1622  14

1623  15

1624  239

1625  229

1626  16

1627  160

1628  232

1629  17

1630  18

1631  19

1632  17

1633  18

1634  239

1635  246

1636  155

1637  235

1638  249

1639  1

1640  160

1641  226

1642  2

1643  225

1644  3

1645  237

1646  4

1647  227

1648  233

1649  5

1650  228

1651  229

1652  231

1653  6

1654  236

1655  240

1656  7

1657  8

1658  9

1659  10

1660  11

1661  243

1662  12

1663  244

1664  238

1665  13

1666  242

1667  14

1668  15

1669  16

1670  5

1671  229

1672  243

1673  249

1674  155

1675  1

1676  239

1677  2

1678  3

1679  225

1680  4

1681  233

1682  10

1683  11

1684  174

1685  155

1686  236

1687  237

1688  1

1689  2

1690  243

1691  238

1692  242

1693  3

1694  229

1695  4

1696  232

1697  160

1698  225

1699  5

1700  239

1701  6

1702  7

1703  8

1704  233

1705  9

1706  5

1707  6

1708  160

1709  172

1710  173

1711  244

1712  233

1713  1

1714  2

1715  225

1716  229

1717  3

1718  155

1719  4

1720  17

1721  160

1722  191

1723  225

1724  226

1725  230

1726  237

1727  228

1728  233

1729  247

1730  167

1731  1

1732  2

1733  187

1734  3

1735  4

1736  236

1737  5

1738  155

1739  238

1740  6

1741  239

1742  7

1743  172

1744  229

1745  243

1746  8

1747  9

1748  10

1749  174

1750  11

1751  12

1752  13

1753  14

1754  15

1755  16

1756  6

1757  7

1758  160

1759  174

1760  225

1761  229

1762  236

1763  250

1764  155

1765  239

1766  233

1767  1

1768  2

1769  3

1770  4

1771  5

1772  155

1773  155

1774  155

1775  155

1776  155

1777  155

1778  155

1779  155

1780  155

1781  155

Appendix B.I

Implementation recommendations

### B.I.1	Implications for retail digital cable-ready devices

Given that a cable operator could choose to deliver SI tables according to any of the profiles defined in Annex B.A on any given hub, digital cable-ready devices offered for retail sale should be able to accept a Short-form Virtual Channel Table for basic navigation if the Long-form Virtual Channel is not provided. It should also accept the Long-form Virtual Channel Table if the Short-form table is not provided.

### B.I.2	Channel number handling

Host devices are expected to support navigation based on virtual channel records associated with two-part channel numbers. If an S-VCT virtual channel record includes a two_part_channel_number_descriptor(), the Host is expected to use it, and to disregard the 12-bit virtual_channel_number field in the same virtual_channel() record.

If a two_part_channel_number_descriptor() is not present in the record-level descriptors loop of a particular S-VCT virtual channel record, the Host is expected to use the virtual_channel_number field in the virtual_channel() record, (see Table B.20) as the channel number reference.

Both numbering schemes may co-exist in a channel map, but each individual channel must be considered labelled with either a one-part or a two-part number.

### B.I.3	Processing of dynamic changes to service information

The Host is expected to monitor SI data on a continuous basis, and react to changes dynamically. For example, an update to an S-VCT or L-VCT may indicate that the definition of the currently acquired virtual channel has changed. The change could involve, for example, association of the channel with a different MPEG-2 program_number within a Transport Stream on a different carrier frequency. In response to such a change, the Host is expected to tune to and acquire the service as redefined.

For some types of changes, the Host is not expected to respond in a visible way. For example, the name of the current event may change, but the new name would be visible as the response to a regular user action to show the event name on-screen or in a program guide display.

### B.I.4	AEITs may include event information for inaccessible channels

In the out-of-band system, depending on the data delivery methods employed by the cable headend and POD module, there may be occasions where AEITs are broadcast for which some set-top boxes do not have corresponding virtual channel assignments. In these cases, the Host is expected to discard portions of the AEITs corresponding to source_ID values not present in the Virtual Channel Table (short- or long-form).

For example, the AEIT may include data describing the program schedule for a service identified with source_ID value 0x0123. Let's say the Virtual Channel Table does not include a channel associated with source_ID 0x0123. When constructing a program guide display, the channel name, number and physical location associated with events tied to source_ID 0x0123 will not be available. Therefore, the events described in the AEIT data for this channel are inaccessible, and the AEIT records for this source_ID should be discarded.

### B.I.5	Splice flag processing

The S-VCT includes a flag called splice. Hosts supporting application of virtual channel changes tied to video splice point timing are expected to execute the change after two seconds following the activation_time, in the absence of a video splice point prior to that time.

Support of the splice timing function is optional in Hosts. A Host not supporting the splice timing feature is expected to apply the data delivered in the VCM_structure() at the indicated activation time (i.e. the splice flag may be simply disregarded).

Appendix B.II

Service Information overview and guide

### B.II.1	Table hierarchy

Figures B.II.1 through B.II.5 describe the relationships between SI tables for Profiles 1 through 6 in a simplified form. A mandatory table is shown in solid box. An optional table is shown in dotted box. An italicized name indicates a sub-table or a map carried within the table.

Figure B.II.1/J.94 – Hierarchy of Table Sections – Profiles 1 and 2

Figure B.II.2/J.94 – Hierarchy of Table Sections – Profile 3

Figure B.II.3/J.94  Hierarchy of Table Sections  Profile 4

Figure B.II.4/J.94 – Hierarchy of Table Sections – Profile 5

Figure B.II.5/J.94 – Hierarchy of Table Sections – Profile 6

The Short-form Virtual Channel Table section (table_ID 0xC4) or the Long-form Virtual Channel Table (table_ID 0xC9) provide navigation data on the out-of-band path. If MGT is provided, it references all tables present in Service Information (except the System Timetable).

The Master Guide Table provides general information about all of the other tables including the S-VCT, L-VCT, RRT, AEIT, and AETT. It defines table sizes necessary for memory allocation during decoding; it defines version numbers to identify those tables that need to be updated; and it gives the packet identifier (PID) values associated with instances of AEITs and AETTs.

In Profile 3 and higher, the Rating Region Table must be included, with one exception, to describe rating regions in use. The exception is that delivery of version 0 of the RRT for region 0x01 (US and possessions), need not be sent because this table is standardized in EIA-766. Furthermore, for Profile , the MGT need not be sent if no RRT is sent.

Aggregate Event Information Tables are included in the out-of-band data in Profiles 4-6. Each AEIT instance describes the events or TV programs associated with a particular three-hour time slot. In the AEIT table structure, program schedule and title data for all virtual channels is aggregated together.

Each AEIT instance is valid for a time interval of three hours. As shown in Figure B.II.3, at minimum, AEIT-0 through AEIT-3 must be sent. Therefore, when Profiles 4-6 are used, current program information and information covering nine to twelve hours of future programming will be available to the Host.

Up to 256 AEITs may be transmitted; over 30 days of future programming may therefore be described. For the fourth timeslot and beyond (AEIT-4 through AEIT-N), the tables may be associated with the same or different PID values.

The start time for any AEIT is constrained to be one of the following UTC times: 00:00 (midnight), 03:00, 06:00, 09:00, 12:00 (noon), 15:00, 18:00, and 21:00. Imposing constraints on the start times as well as the interval duration simplifies re-multiplexing. During re-multiplexing, AEIT tables coming from several distinct Transport Streams may end up grouped together or vice versa. If no constraints were imposed, re-multiplexing equipment would have to parse AEIT by content in real time, which is a difficult task.

However, it is also possible to regenerate one or several AEIT at any time for correcting and/or updating the content (e.g. in cases where "to be assigned" events become known). Regeneration of an AEIT may be flagged by updating version fields in the MGT.  A new AEIT may also be associated with a PID value not in current use. The MGT may be updated to show this new PID value association.

In Profiles 4-6, there can be several Aggregate Extended Text Tables, each of them having its associated PID defined in the MGT. As its name indicates, the purpose of an Aggregate Extended Text Table is to carry textual data. For example, for an event such as a movie listed in the AEIT, the typical data is a short paragraph that describes the movie itself. Each Aggregate Event Information Table can have one associated AETT. Each AETT instance includes all the text associated with events starting within a particular timeslot. Aggregate Extended Text Tables are optional in Profiles 4-6.

### B.II.2	SI_base PID

Data associated with the SI_base PID defines information of system-wide applicability such as frequency plans, channel maps, and channel names. The SI_base PID value is 0x1FFC. The types of table sections that may be included in the Network Stream include:

	Network Information Table, carrying the:

•	Carrier Definition Subtable,

•	Modulation Mode Subtable;

	Network Text Table, carrying the Source Name Subtable;

	Short-form Virtual Channel Table, carrying the:

•	Virtual Channel Map,

•	Defined Channels Map,

•	Inverse Channels Map;

	Long-form Virtual Channel Table;

	Master Guide Table;

	Rating Region Table;

	System Timetable.

Carrier Definition Subtable

The Carrier Definition Subtable provides a foundation for the definition of frequency plans by defining a set of carrier frequencies appropriate to a particular transmission medium. The CDS is stored in the Host as an array of as many as 255 CDS records, each consisting of:

•	Carrier frequency, 15 bits, in units of 10 or 125 kHz.

Modulation Mode Subtable

The Modulation Mode Subtable provides a foundation for quick acquisition of digitally modulated waveforms. A separate MMS shall be transmitted in Network data for each transmission medium supported by that network. An MMS is stored in the Host as an array of up to 255 MMS records, each consisting of:

•	Modulation format: analogue NTSC or QAM;

•	Transmission system: ITU-T (North America) or ATSC;

•	Symbol rate, in units of 1 Hz;

•	Inner coding mode, expressed as either "none" or an integer ratio such as 1/2 or 3/4;

•	For QAM modulation, the number of levels.

Each MMS contains entries for each modulation mode currently in use by any digital waveform, plus entries for any modes anticipated to be used. As with the CDS, changes to the table are rare.

Parameters defined within the MMS are not specifically manipulated by Hosts compliant with the SI protocol, but are referenced by the Host when attempting to acquire a digitally encoded and modulated waveform.

Short-form Virtual Channel Table and Virtual Channel Record

The Short-form Virtual Channel Table is a hierarchical data structure that may carry within it the Virtual Channel Map and Virtual Channel record, for support of up to 4096 channel definition records. Each virtual channel is associated with a 16-bit reference ID number called the source_ID. Each record in the VCM consists of:

•	The MPEG program number, associating the virtual channel record with a program defined in the Program Association Table and TS Program Map Table.

•	For virtual channels associated with programs carried in a program guide, the source_ID, a number that may be used to link the virtual channel to entries in the Electronic Program Guide (EPG) database.

•	For virtual channels used as access paths to application code or data (such as EPG), the application ID.

Source ID

Source ID is a 16-bit number associated with each program source, defined in such a way that every programming source offered anywhere in the system described in this Service Information annex is uniquely identified. For example, HBO/W has a different assigned source ID than HBO/E, and both are different from HBO-2 or HBO-3. Uniqueness is necessary to maintain correct linkages between an EPG database and virtual channel tables. See below for a discussion of the relationship between source_ID, virtual channels, and an EPG database.

Source Names and Source Name Subtable

The Source Name is a variable length multilingual text string associating a source ID with a textual name. The Source Name Subtable is delivered within the Network Text Table section.

Source name information is delivered in a table format separate from the table containing other information comprising the virtual channel table. Name information is not strictly necessary for channel acquisition, and (depending on the memory management scheme employed in the Host) may not always be available from memory at acquisition time. Source name information may be refreshed often, and can be available within several seconds of acquisition.

An EPG database may define textual reference names associated with given program sources (referenced by source ID). Such a database may be used to derive virtual channel names in some applications, though in an EPG database the name is generally abbreviated due to display considerations.

Name data is, unlike the regular VCT data, language tagged, so that multilingual source names may be defined. Transmission format for multilingual text is defined to include references to multiple phonetic and ideographic character sets.

Defined Channels Map and Inverse Channels Map

For a given Standard-compliant channel, DCM data consist of a series of bytes that, taken as a whole, specify which channels in the map are defined, and which are not.

Each Virtual Channel Table has associated with it a table listing source_IDs and their associated virtual channel numbers. The source_ID values are sorted by value from the lowest to the highest in the table, to facilitate (using a binary search) lookup of a virtual channel given a source ID.

Master Guide Table

Use of the MGT is optional in certain profiles. Table B.II.1 shows a typical Master Guide Table indicating, in this case, the existence in the Transport Stream of a Long-form Virtual Channel Table, the Rating Region Table, four Aggregate Event Information Tables, and two Aggregate Extended Text Tables describing the first six hours' events.

Table B.II.1/J.94 – Example Master Guide Table content

| table_type | PID | version_number | table size (bytes) |
| --- | --- | --- | --- |
| LVCT | 0x1FFC | 4 | 5 922 |
| RRT – region 6 | 0x1FFC | 0 | 1 020 |
| AEIT-0 – MGT_tag = 56 | 0x1DD2 | 6 | 29 250 |
| AEIT-1 – MGT_tag = 57 | 0x1DD2 | 4 | 28 440 |
| AEIT-2 – MGT_tag = 58 | 0x1DD3 | 10 | 25 704 |
| AEIT-3 – MGT_tag = 59 | 0x1DD3 | 2 | 27 606 |
| AETT-0 – MGT_tag = 56 | 0x1DD2 | 2 | 24 004 |
| AETT-1 – MGT_tag = 57 | 0x1DD2 | 7 | 25 922 |
| AETT-2 – MGT_tag = 58 | 0x1DD3 | 8 | 27 711 |
| AETT-3 – MGT_tag = 59 | 0x1DD3 | 0 | 19 945 |

The first entry of the MGT describes the version number and size of the Long-form Virtual Channel Table. The second entry corresponds to an instance of the Rating Region Table for region 6. If some region's policy makers decided to use more than one instance of an RRT, the MGT would list each PID, version number, and size.

The next entries in the MGT correspond to the four AEITs that must be supplied in the Transport Stream for profiles 4-6. After the AEITs, the MGT references four Aggregate Extended Text Tables. The PID values for AEIT-0 and AEIT-1 are both 0x1DD2. MGT_tag values 56 and 57 are used for these. For AEIT-2 AEIT-3, PID 0x1DD3 is used. The last four references are to Aggregate ETTs.

Note that AETT-n shares a common PID value with AEIT-n for every value of n. AEIT-0 and AETT-0 are associated with PID 0x1DD2, as are AEIT-1 and AETT-1. AEIT-2 and AETT-2 are associated with PID 0x1DD3, etc.

Descriptors can be added for each entry as well as for the entire MGT. By using descriptors, future improvements can be incorporated without modifying the basic structure of the MGT. The MGT is like a flag table that continuously informs the Host about the status of all the other tables (except the System Time which has an independent function). The MGT is continuously monitored at the Host to prepare and anticipate changes in the channel/event structure. When tables are changed at the broadcast side and the PID association is unchanged, their version numbers are incremented and the new numbers are listed in the MGT. Another method that can be used to change tables is to associate the updated tables with different PID values, and then update the MGT to reference the new PID values. Based on the MGT version or PID updates and on the memory requirements, the Host can reload the newly defined tables for proper operation.

Table B.II.2 is an example MGT that may be sent after the instance in Table B.II.2 has expired due to the passage of time. In this example, three hours have passed, and the time slot covered in the old AEIT-0 is in the past. The AEIT with MGT_tag = 57 moves now to become AEIT-0. The AEIT with MGT_tag = 58, the new AEIT-1, moves to PID 0x1DD2. A new AEIT is added to the mix, the AEIT with MGT_tag = 60.

Table B.II.2/J.94 – Example Revised Master Guide Table content

| table_type | PID | version_number | table size (bytes) |
| --- | --- | --- | --- |
| LVCT | 0x1FFC | 4 | 5 922 |
| RRT – region 6 | 0x1FFC | 0 | 1 020 |
| AEIT-0 – MGT_tag = 57 | 0x1DD2 | 4 | 28 440 |
| AEIT-1 – MGT_tag = 58 | 0x1DD2 | 10 | 25 704 |
| AEIT-2 – MGT_tag = 59 | 0x1DD3 | 2 | 27 606 |
| AEIT-3 – MGT_tag = 60 | 0x1DD3 | 0 | 30 055 |
| AETT-0 – MGT_tag = 57 | 0x1DD2 | 7 | 25 922 |
| AETT-1 – MGT_tag = 58 | 0x1DD2 | 8 | 27 711 |
| AETT-2 – MGT_tag = 59 | 0x1DD3 | 0 | 19 945 |
| AETT-3 – MGT_tag = 60 | 0x1DD3 | 0 | 22 522 |

L-VCT

The L-VCT combines all the data pertinent to the description of a virtual channel into a single table. Use of the L-VCT instead of the S-VCT eliminates the need to send CDS, MMS, SNS, DCM, or ICM. The L-VCT follows the standard MPEG-2 long-form section syntax (section_syntax_indicator = 1).

Rating Region Table

The Rating Region Table is a fixed data structure in the sense that its content remains mostly unchanged. It defines the rating standard that is applicable for each region and/or country. The concept of table instance introduced in the previous clause is also used for the RRT. Several instances of the RRT can be constructed and carried in the Transport Stream simultaneously. Each instance is identified by a different table_id_extension value (which becomes the rating_region in the RRT syntax) and corresponds to one and only one particular region. Each instance has a different version number which is also carried in the MGT. This feature allows updating each instance separately.

Figure B.II.3 shows an example of one instance of an RRT, defined for rating region 99 and carrying an example rating system. Each event listed in any of the EITs may carry a content advisory descriptor. This descriptor is an index or pointer to one or more instances of the RRT.

Figure B.II.3/J.94 – An instance of a Rating Region Table

Aggregate Event Information Tables and Aggregate Extended Text Tables

The purpose of an AEIT is to list all events for those channels that appear in the VCT for a given time window. As mentioned before, AEIT-0 describes the events for the first 3 hours and AEIT-1 for the second 3 hours. AEIT-0 and AEIT-1 share a common associated PID value as defined in the MGT. In MPEG, tables can have a multitude of instances. When different instances of a table share the same table_id value and PID, they are distinguished by differences in the 16-bit table_id_extension field.

In this SI appendix for out-of-band use, each instance of AEIT-k contains a list of events for a each virtual channel. Linkage to each channel in the VCT is made via the source_ID. For the AEIT, the table_id_extension field appears as MGT_tag.

Figure B.II.4 shows, for example, a program provider's instance for AEIT-0.

Figure B.II.4/J.94 – Example AEIT-0

AEIT-0 is unique in that it must list all events starting within the three-hour time period it covers, as well as any events that started earlier but extend into the covered period. For all other AEITs, only those events actually starting within the three hour time period are included. The Host is expected to collect AEITs in order of their time coverage. If AEIT-4 is available to the Host but AEIT-3 is not, for example, information for events that started in the time period covered by AEIT-3 but extending into AEIT-4 will not be available for display.

Figure B.II.4 shows an example of a small AEIT-0, including event data for two sources, a channel called "TSPN" (source_ID 22) and one called "MOOV" (source_ID 80). For the three-hour period covered by AEIT-0, 9 a.m. to noon, three events are listed for TSPN and two for MOOV. The field event_id is a number used to identify each event. The event_id is used to link events with associated text delivered in the AETT. The assignment of an event_ID value must be unique within a source ID and a 3-hour interval defined by one AEIT instance. The event_id is followed by the start_time and then the length_in_seconds. Notice that for AEIT-0 only, events can have start times before the activation time of the table. ETMs are simply long textual descriptions. The collection of ETMs constitutes an Aggregate Extended Text Table (ETT).

An example of an ETM for the Car Racing event may be:

"Live coverage from Indianapolis. This car race has become the largest single-day sporting event in the world. Two hundred laps of full action and speed."

Several descriptors can be associated with each event. The most important is the content advisory descriptor which assigns a rating value according to one or more systems. Recall that the actual rating system definitions are tabulated within the RRT.

Figure B.II.5 diagrams the AEIT data structure. As shown, the AEIT includes event data for all sources listed in the VCT. In the figure, the hatched box represents one or more "event data" blocks, each comprised of the data items shown in the upper left.

Figure B.II.5/J.94 – AEIT data structure

Figure B.II.6 diagrams the AETT data structure. The AETT aggregates text for a given timeslot into one sectioned MPEG table.

Figure B.II.6/J.94 – Structure of AETT

An AETT-n instance for a given value of n (timeslot) is associated with the same PID value as AEIT-n. This means that they can be collected using a single Extended Channel data flow between Host and POD.

Inactive Channels

Any channels in the L-VCT which are not currently active shall have the hidden attribute set to 1 and the hide_guide attribute set to 0. Inactive channels in the S-VCT shall have the hidden attribute in channel_type, and the hide_guide flag in the channel_properties_descriptor() set to 0.

Table B.II.3 shows expected DTV behavior for the various combinations of the hidden and hide_guide attributes. In the table the "x" entry indicates "don't care." A check in the "surf" column indicates the channel is available by channel surfing and via direct channel number entry. A check in the "guide" column indicates that the channel may appear in the program guide listing.

Table B.II.3/J.94 – Receiver Behavior with hidden and hide_guide attributes

| hidden | Hide_guide |  | Receiver Behavior |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Surf |  | Guide |  |  |  |
| 0 |  | x |  |  |  |  |  | Normal channel |
| 1 |  | 1 |  |  |  |  |  | Special access only |
| 1 |  | 0 |  |  |  |  |  | Inactive channel |

### B.II.3	Representation of Time

The System Timetable provides time of day information to Hosts. In this Service Information appendix, time of day is represented as the number of seconds that have elapsed since the beginning of "GPS time," 0000 Hours UTC, January 6th, 1980. GPS time is referenced to the Master Clock at the US Naval Observatory and steered to Coordinated Universal Time (UTC). UTC is the current time of day at the time zone local to Greenwich, England, and is the time source we use to set our clocks.

The cycle of the seasons, technically known as the tropical year, is approximately 365.2422 days. Using the Gregorian calendar we adjust for the fractional day by occasionally adding an extra day to the year. Every fourth year is a leap year, except that three leap years in every 400 are skipped (the centennial years not divisible by 400). With this scheme there are 97 leap years in each 400 year span, yielding an average year that is 365.2425 days long.

UTC is occasionally adjusted by one-second increments to ensure that the difference between a uniform time scale defined by atomic clocks does not differ from the Earth's rotational time by more than 0.9 seconds. The timing of occurrence of these "leap seconds" is determined by careful observations of the Earth's rotation; each is announced months in advance. On the days it is scheduled to occur, the leap second is inserted just following 12:59:59 p.m. UTC.

UTC can be directly computed from the count of GPS seconds since January 6th, 1980 by subtracting from it the count of leap seconds that have occurred since the beginning of GPS time. In the months just following January 1st, 1999, this offset was 13 seconds.

This protocol defines various time-related events and activities, including starting times for programs, text display, changes to VCTs, and others. Two methods of time distribution are used in headend systems. One method derives time in the form of GPS seconds from GPS Hosts. These Hosts also provide current GPS/UTC offset data. The second method of time distribution relies on the Internet Standard Network Time Protocol (NTP). NTP servers provide output in the form of UTC time, and do not provide GPS/UTC offset data. The Standard-compliant Host is synchronized to system time by the System Timetable, which provides time either in the form of GPS seconds since week zero of GPS time, January 6th, 1980, or directly in UTC time. The interpretation depends on the value of the GPS/UTC offset field. The special value of zero is used to indicate that the system is being driven by a UTC time source directly, and that GPS/UTC offset data is not available.

System Time

GPS satellites typically output GPS time in a format consisting of a week count (Tw) and a seconds within the week count (Ts), where week zero is defined as starting January 6th, 1980. For purposes of building the System Timetable, the following formula may be used:

T = (Tw * 604 800) + Ts

There are 604 800 seconds per week.

When converting between GPS seconds and current local time in hours/minutes/seconds, the following factors must be taken into account:

•	GPS to UTC offset  Given a time represented as GPS seconds, the Host first subtracts the GPS/UTC offset to convert to UTC.

•	1980  The first year of GPS time started on January 6th, yielding 361 days in the first year (1980 was also a leap year).

•	Leap years  The number of leap years that occurred between the current GPS second and 1980 must be accounted for. A leap year is a year whose number is evenly divisible by four, or, in the case of century years, by 400.

NOTE  According to this rule, the year 2000 is a leap year even though it is a century year, because it is also divisible by 400.

•	Time zones  Time zones are signed integer values in the range –12 to +13 hours, where positive numbers represent zones east of the Greenwich meridian and negative numbers west of it. Pacific Standard Time (PST) is 8 hours behind standard time, and Eastern Standard Time (EST) is 5 hours behind. The system defined by this Service Information standard accommodates time zones that are not an integral number of hours offset from Greenwich by defining time zone as an 11-bit signed integer number in units of minutes. To convert to local time, the time zone is added to Greenwich time using signed integer arithmetic.

•	Daylight savings time  If applicable, daylight savings time must be taken into account. On a unit by unit basis, each Host may be given a definition for when daylight savings time is entered into in Spring, and when it is exited in Fall. Entry/exit points are given as absolute times (GPS seconds), and hence are given in one second resolution.

Transmission Format for Event Times

In this messaging protocol, the absolute time of action is specified for most events in terms of an unsigned 32-bit integer number, the count of GPS seconds since January 6th, 1980. This count does not wrap until after the year 2116.

Handling of Leap Second Events

In this Service Information protocol, times of future events (such as event start times in the EIT) are specified the same as time of day, as the count of seconds since January 6th, 1980. Converting an event start time to UTC and local time involves the same calculation as the conversion of system time to local time. In both cases, the leap seconds count is subtracted from the count of GPS seconds to derive UTC.

GPS time is used to represent future times because it allows the Host to compute the time interval to the future event without regard for the possible leap second that may occur in the meantime. Also, if UTC were to be used instead, it wouldn't be possible to specify an event time that occurred right at the point in time where a leap second was added. UTC is discontinuous at those points.

Around the time a leap second event occurs, program start times represented in local time (UTC adjusted by local time zone and [as needed] daylight savings time) may appear to be off by plus or minus one second. Generating equipment may use one of two methods to handle leap seconds.

In method A, generating equipment does not anticipate the future occurrence of a leap second. In this case, prior to the leap second, program start times will appear correct. An event starting at exactly 10 a.m. will be computed as starting at 10:00:00. But just following the leap second, that same event time will be computed as 9:59:59. The generating equipment should re-compute the start times in all the EITs and introduce the leap second correction. Once that happens, and Hosts have updated their EIT data, the computed time will again show as 10:00:00. In this way the disruption can be limited to a matter of seconds.

In method B, generating equipment does anticipate the occurrence of a leap second, and adjusts program start times for events happening after the new leap second is added. If the leap second event is to occur at midnight tonight, an event starting at 10 a.m. tomorrow will be computed by receiving equipment as starting at 10:00:01.

For certain types of events, the precision of method B is necessary. By specifying events using a time system that involves no discontinuities, difficulties involving leap seconds are avoided. Events such as program start times do not require that level of precision. Therefore, method A works well.

Handling of Leap Second Events

Consider the following example. Times are given relative to UTC, and would be corrected to local time zone and daylight savings time as necessary.

•	Time of day (UTC):		1:00 p.m., December 30th, 1998

•	Event start time (UTC):	2:00 p.m., January 2nd, 1999

•	A leap second event will occur just after 12:59:59 p.m. on December 31st , 1998.

•	Leap seconds count on December 30th is 12.

The data in the System Timetable is:

•	GPS seconds = 599 058 012 = 0x23B4E65C

•	GPS to UTC offset = 12

Using method A (upcoming leap second event is not accounted for):

•	Event start time in EIT: 	599 320 812 = 0x23B8E8EC

•	Converted to UTC:	2:00:00 p.m., January 2nd, 1999

•	Number of seconds to event:	262 800 = 73 hours, 0 minutes, 0 seconds

Using method B (upcoming leap second event is anticipated):

•	Event start time in EIT: 	599 320 813 = 0x23B8E8ED

•	Converted to UTC:	2:00:01 p.m., January 2nd, 1999

•	Number of seconds to event:	262 801 = 73 hours, 0 minutes, 1 second

Note that using method B, the number of seconds to event is correct, and does not need to be recomputed when the leap seconds count moves from 12 to 13 at year-end.

Appendix B.III

Daylight Savings Time control

In order to convert GPS into local time, the Host needs to store a time offset (from GPS to local time) in local memory and an indicator as to whether daylight savings is observed. These two quantities can be obtained from the user interface (indicating time zone and daylight savings observance) or from the conditional access system, if present, and stored in non-volatile Host memory.

Since there is a common time (GPS) transmitted in SI, a mechanism to indicate when the Host should switch into (or out of) daylight savings time at the appropriate local time can be very useful. Once all the Hosts have transitioned at their local times, the entire system can be shifted into daylight savings time. This is accomplished by appropriate setting of the daylight_savings in the daylight_savings_time_descriptor() the STT. The basic use of daylight savings fields through the year is shown in Table B.III.1.

Table B.III.1/J.94 – Basic use of daylight savings fields through the year

| Conditions | DS status | DS_day of_month | DS_hour |
| --- | --- | --- | --- |
| At the beginning of the year (January) daylight savings is off. This is the status of the fields until: | 0 | 0 | 0 |
|  When the transition into daylight savings time is within less than one month, the DS_day_of_month field takes the value day_in, and the DS_hour field takes the value hour_in. The DS_status bit is 0 indicating it is not yet daylight savings time. (The transition is to occur on the day_in day of the month at hour=hour_in; for example, if the transition were on April 15 at 2 a.m., then day_in=15 and hour_in=2.) | 0 | day_in | hour_in |
|  After all time zone daylight transitions (within the span of the network) have occurred, the DS_status bit takes the value 1, indicating that daylight savings time is on. The DS_day_of_month field and the DS_hour field take the value 0. (In the U.S., this transition has to occur no later than 7 p.m. Pacific Time on the day day_in.) This is the status of the fields until: | 1 | 0 | 0 |
| When the transition out of daylight savings time is within less than one month, the DS_day_of_month field takes the value day_out, and the DS_hour field takes the value hour_out. The DS_status bit is 1 indicating it is still daylight savings time. (The transition is to occur on the day_out day of the month at hour=hour_out; for example, if the transition were on October 27 at 2 a.m., then day_out=27 and hour_out=2.) | 1 | day_out | hour_out |
|  After all time zones (within the span of the network) have shifted out of daylight savings time, the DS_status bit takes the value 0, indicating that daylight savings time is off. The DS_day_of_month field and the DS_hour field take the value 0. (In the U.S., this transition has to occur no later than 7 p.m. Pacific Time on the day day_out.) This finishes the cycle. | 0 | 0 | 0 |

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |

The Base PID is the PID associated with the "base" Service Information tables. In this protocol, the base_PID is fixed at 0x1FFC. Refer to Table B.2.

NOTE  Assignment of table_ID values in the 0xCE to 0xFE range requires coordination between ATSC and SCTE.

Note that transmission systems using VSB modulation transmit spectra are not symmetrical about the carrier or pilot tone. Acquisition of a VSB-modulated signal involves computation of the pilot tone (or in analogue VSB, the picture carrier) location relative to the centre of the band. For example, for the ATSC Digital Television Standard (ASTC A/53), where the channel bandwidth is 6 MHz, the pilot tone is located 310 kHz above the lower edge of the channel, or 2.690 MHz below the specified centre of the band. Similarly, for analogue NTSC, the picture carrier is 1.25 MHz above the lower edge of the channel, or 1.75 MHz below the specified centre of the band.

Please refer to B.6.8 for definition of the AEIT-n and AETT-n notation convention used in this annex.

A method to include such a unique 16-bit "Transmission Signal ID" in the NTSC VBI is specified in the EIA-752 specification.

Although AEIT is similar in structure to the EIT in ATSC A/65, its properties differ from EIT in this regard.

Source ID and application ID need never be defined in the same virtual channel record, therefore they share a common 16-bit field in the stored map. Channels are defined as for "application access" or not; if they are application access, the field defines the application ID, if not, it defines the source ID.

Prior to that time, all initial Receivers will surely be out of service, and new ones can be designed to handle the wrap condition.