|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | J.94 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | Amendment 2 (03/2001) |
|  | SERIES J: CABLE NETWORKS AND TRANSMISSION OF TELEVISION, SOUND PROGRAMME AND OTHER MULTIMEDIA SIGNALS Ancillary digital services for television transmission |  |  |  |
|  | Service information for digital broadcasting in cable television systems Amendment 2: Revised Annex C – Service information for digital multi-programme System C |  |  |  |
|  | ITU-T Recommendation J.94 – Amendment 2 (Formerly CCITT Recommendation) |  |  |  |

ITU-T J-SERIES  RECOMMENDATIONS

CABLE NETWORKS AND TRANSMISSION OF TELEVISION, SOUND PROGRAMME AND OTHER MULTIMEDIA SIGNALS

| General Recommendations | J.1–J.9 |
| --- | --- |
| General specifications for analogue sound-programme transmission | J.10–J.19 |
| Performance characteristics of analogue sound-programme circuits | J.20–J.29 |
| Equipment and lines used for analogue sound-programme circuits | J.30–J.39 |
| Digital encoders for analogue sound-programme signals | J.40–J.49 |
| Digital transmission of sound-programme signals | J.50–J.59 |
| Circuits for analogue television transmission | J.60–J.69 |
| Analogue television transmission over metallic lines and interconnection with radio-relay links | J.70–J.79 |
| Digital transmission of television signals | J.80–J.89 |
| Ancillary digital services for television transmission | J.90–J.99 |
| Operational requirements and methods for television transmission | J.100–J.109 |
| Interactive systems for digital television distribution | J.110–J.129 |
| Transport of MPEG-2 signals on packetised networks | J.130–J.139 |
| Measurement of the quality of service | J.140–J.149 |
| Digital television distribution through local subscriber networks | J.150–J.159 |
| IPCablecom | J.160–J.179 |
| Miscellaneous | J.180–J.199 |
| Application for Interactive Digital Television | J.200–J.209 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation J.94 Service information for digital broadcasting in cable television systems Amendment 2 Revised Annex C – Service information for digital multi-programme System C

- Summary This annex describes the service information for digital broadcasting by cable television of Annex C/J.83. The specifications basically constitute a subset of the ones in Annex A. However, there are some specifications which are different from those of Annex A and also there are some specifications which are yet to be established.

- Source Amendment 2 to ITU-T Recommendation J.94 was prepared by ITU-T Study Group 9 (2001-2004) and approved under the WTSA Resolution 1 procedure on 9 March 2001.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2001

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from ITU.

ITU-T Recommendation J.94

Service information for digital broadcasting in cable television systems

Amendment 2

Revised Annex C – Service information for digital multi-programme System C

Replace old Annex C by the following:

Annex C

Service information for digital multi-programme System C

## C.1	SI tables

The specifications for SI tables are fully aligned with those in Annex A both in table names and in their function. See Table C.1.

Table C.1/J.94 – SI tables and their function

| Table | Function |
| --- | --- |
| Program Association Table (PAT) | For each service in the multiplex, the PAT indicates the location (the PID values of the Transport Stream packets) of the corresponding Program Map Table (PMT). It also gives the location of the Network Information Table (NIT). |
| Conditional Access Table (CAT) | The CAT provides information on the Conditional Access (CA) systems used in the multiplex; the information is private (not defined within ITU-T H.222.0 ¦ ISO/IEC 13818-1) and dependent on the CA system, but includes the location of the EMM stream, when applicable. |
| Program Map Table (PMT) | The PMT identifies and indicates the locations of the streams that make up each service, and the location of the Program Clock Reference fields for a service. |
| Network Information Table (NIT) | The location of the NIT is defined in ITU-T H.222.0 ¦ ISO/IEC 13818-1, but the data format is outside the scope of ITU-T H.222.0 ¦ ISO/IEC 13818-1. It is intended to provide information about the physical network. The syntax and semantics of the NIT are defined in this Recommendation. |
| Bouquet Association Table (BAT) | The BAT provides information regarding bouquets. As well as giving the name of the bouquet, it provides a list of services for each bouquet. |
| Service Description Table (SDT) | The SDT contains data describing the services in the system, e.g. names of services, the service provider, etc. |

Table C.1/J.94 – SI tables and their function (concluded)

| Table | Function |
| --- | --- |
| Event Information Table (EIT) | The EIT contains data concerning events or programs such as event name, start time, duration, etc.; the use of different descriptors allows the transmission of different kinds of event information, e.g. for different service types. |
| Running Status Table (RST) | The RST gives the status of an event (running/not running). The RST updates this information and allows timely automatic switching to events. |
| Time and Date Table (TDT) | The TDT gives information relating to present time and date. This information is given in a separate table due to the frequent updating of the time information. |
| Stuffing Table (ST) | The ST is used to invalidate existing sections, for example at delivery system boundaries. |

The PID allocation for SI and the allocation of table_id values are as shown in Tables C.2 and C.3, which are the same as those in Tables A.1 and A.2.

Table C.2/J.94 – PID allocation for SI

| Table | PID value |
| --- | --- |
| PAT | 0x0000 |
| CAT | 0x0001 |
| NIT, ST | 0x0010 |
| SDT, BAT, ST | 0x0011 |
| EIT, ST | 0x0012 |
| RST, ST | 0x0013 |
| TDT | 0x0014 |
| NULL | 0x1FFF |

Table C.3/J.94 – Allocation of table_id values

| Value | Table and description |
| --- | --- |
| 0x00 | PAT |
| 0x01 | CAT |
| 0x02 | PMT |
| 0x40 | NIT, network_information_section-actual_network |
| 0x41 | NIT, network_information_section-other_network |
| 0x42 | SDT, service_description_section-actual_transport_stream |
| 0x46 | SDT, service_description_section-other_transport_stream |
| 0x4A | BAT |
| 0x4E | EIT, event_information_section-actual_transport_stream, present/following |
| 0x4F | EIT, event_information_section-other_transport_stream, present/following |

Table C.3/J.94 – Allocation of table_id values (concluded)

| Value | Table and description |
| --- | --- |
| 0x50 to 0x5F | EIT, event_information_section-actual_transport_stream, before 8th day EIT, event_information_section-actual_transport_stream, on or after 8th day |
| 0x60 to 0x6F | EIT, event_information_section-other_transport_stream, before 8th day EIT, event_information_section-other_transport_stream, on or after 8th day |
| 0x70 | TDT, time_date_section |
| 0x71 | RST, running_status_section |
| 0x72 | ST, stuffing_section |
| 0x82 to 0x85 | Reserved for conditional access system |
| 0x90 to 0xBF | Selectable as operator setting table_id |

## C.2	Descriptor

### C.2.1	Location and tag value

The location and tag value of each descriptor are as shown in Table C.4. The description, data structure, and syntax of each descriptor are the same as those in Table A.12. However, the coding of the data field of each descriptor is not specified.

Table C.4/J.94 – Possible locations of descriptors

| Descriptor | Tag value | NIT | BAT | SDT | EIT | PMT | CAT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_descriptor | 0x09 |  |  |  |  | * | * |
| network_name_descriptor | 0x40 | * |  |  |  |  |  |
| stuffing_descriptor | 0x42 | * | * | * | * |  |  |
| cable_delivery_system_descriptor | 0x44 | * |  |  |  |  |  |
| bouquet_name_descriptor | 0x47 |  | * | * |  |  |  |
| service_descriptor | 0x48 |  |  | * |  |  |  |
| linkage_descriptor | 0x4A | * | * | * | * |  |  |
| NVOD_reference_descriptor | 0x4B |  |  | * |  |  |  |
| time_shifted_service_descriptor | 0x4C |  |  | * |  |  |  |
| short_event_descriptor | 0x4D |  |  |  | * |  |  |
| extended_event_descriptor | 0x4E |  |  |  | * |  |  |
| time_shifted_event_descriptor | 0x4F |  |  |  | * |  |  |
| component_descriptor | 0x50 |  |  |  | * |  |  |
| mosaic_descriptor | 0x51 |  |  | * |  | * |  |
| stream_identifier_descriptor | 0x52 |  |  |  |  | * |  |
| content_descriptor | 0x54 |  |  |  | * |  |  |
| parental_rating_descriptor | 0x55 |  |  |  | * |  |  |
| User-defined | 0x80 to 0xBF |  |  |  |  |  |  |
| Forbidden | 0xFF |  |  |  |  |  |  |

Table C.4/J.94 – Possible locations of descriptors (concluded)

| Descriptor | Tag value | NIT | BAT | SDT | EIT | PMT | CAT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| area_specified_service_descriptor | 0x96 |  | * | * |  |  |  |
| data_coding_method_descriptor | 0xFD |  |  |  |  | * |  |
| * Possible location |  |  |  |  |  |  |  |

Descriptors which are used in Japan but not specified in Annex A are detailed in the following clauses.

### C.2.2	CA descriptor

The CA descriptor which is described in CAT and PMT identifies the type of conditional access and also identifies the PID in TS packet that carries the information related to conditional access. Conditional access is only available when this descriptor is used. See Table C.5.

Table C.5/J.94 – CA descriptor

| Syntax | Bits | Identifier | Note |
| --- | --- | --- | --- |
| CA_descriptor(){ |  |  |  |
| descriptor_tag | 8 | uimsbf |  |
| descriptor_length | 8 | uimsbf |  |
| CA_system_id | 16 | uimsbf |  |
| reserved | 3 | bslbf | "111" |
| CA_PID | 13 | uimsbf |  |
| for (i = 0; i < N; i++) { |  |  |  |
| private_data } | 8xN | bslbf |  |
| } |  |  |  |

### C.2.3	Area specified service descriptor

This descriptor is used to render the services to the specified part within a given service area by transmitting either the area list of the service reception area or the one beyond the service reception area (see Table C.6). Area specified service is only available when this descriptor is used.

Table C.6/J.94 – Area specified service descriptor

| Syntax | Bits | Identifier | Note |
| --- | --- | --- | --- |
| area_specified_service _descriptor(){ |  |  |  |
| descriptor_tag | 8 | uimsbf |  |
| descriptor_length | 8 | uimsbf |  |
| descriptor_flag | 1 | bslbf | (1: available, 0: not available) |
| reserved | 7 | bslbf |  |
| for (i = 0; i < N; i++) { area_code | 24 | bslbf | alphanumeric 3 characters |
| } |  |  |  |
| } |  |  |  |

### C.2.4	Data coding method descriptor

The data coding method descriptor which is described in PMT identifies the data coding method for data broadcasting services. See Table C.7.

Table C.7/J.94 – Data coding method descriptor

| Syntax | Bits | Identifier | Note |
| --- | --- | --- | --- |
| data_coding_method_descriptor(){ |  |  |  |
| descriptor_tag | 8 | uimsbf |  |
| descriptor_length | 8 | uimsbf |  |
| data_component_id | 16 | uimsbf |  |
| for (i = 0; i < N; i++) { |  |  |  |
| additional_identification_information | 8xN | bslbf |  |
| } |  |  |  |
| } |  |  |  |

### C.2.5	Cable delivery system descriptor

This descriptor which is described in NIT identifies the physical conditions of the cable channel. See Table C.8.

Table C.8/J.94 – Cable delivery system descriptor

| Syntax | No. of bits | Identifier |
| --- | --- | --- |
| cable_delivery_system_descriptor(){ |  |  |
| descriptor_tag | 8 | uimsbf |
| descriptor_length | 8 | uimsbf |
| Frequency | 32 | bslbf |
| reserved_future_use | 8 | bslbf |
| frame_type | 4 | bslbf |
| FEC_outer | 4 | bslbf |
| Modulation | 8 | bslbf |
| symbol_rate | 28 | bslbf |
| FEC_inner | 4 | bslbf |
| } |  |  |

Semantics for cable delivery system descriptor

frequency: The frequency is a 32-bit field giving the 4-bit BCD values specifying 8 characters of the frequency value. For the cable_delivery_system_descriptor, the frequency is coded in MHz, where the decimal occurs after the fourth character (e.g. 0312.0000 MHz).

frame_type: The frame_type is a 4-bit field specifying the frame type according to Table C.9. The frame type indicates the number of slots in the TSMF, N, and the maximum number of TSs transmitted simultaneously, M if the TSMF is used. The values of N and M should be identical to those in the [draft new Recommendation D.145].

Table C.9/J.94 – Frame type

| frame_type bit 3210 | Description |
| --- | --- |
| 0000 | Reserved for future use |
| 0001 | (N, M) = (53, 15)a) |
| 0010 to 1110 | Reserved for future use |
| 1111 | None – indicates that the waveform does not use TSMF |
| a) The frame type (N, M) is (53,15) for Annex C. It might be determined for other transmission systems. |  |

FEC_outer: The FEC_outer is a 4-bit field specifying the outer Forward Error Correction (FEC) scheme used according to Table C.10.

Table C.10/J.94 – Outer FEC scheme

| FEC_outer bit 3210 | Description |
| --- | --- |
| 0000 | Not defined |
| 0001 | No outer FEC coding |
| 0010 | RS(204/188) |
| 0011 to 1111 | Reserved for future use |

modulation: This is an 8-bit field. It specifies the modulation scheme used on a cable delivery system according to Table C.11.

Table C.11/J.94 – Modulation scheme for cable

| Modulation (hex) | Description |
| --- | --- |
| 0x00 | Not defined |
| 0x01 | 16-QAM |
| 0x02 | 32-QAM |
| 0x03 | 64-QAM |
| 0x04 | 128-QAM |
| 0x05 | 256-QAM |
| 0x06 to 0xFF | Reserved for future use |

symbol_rate: The symbol_rate is a 28-bit field giving the 4-bit BCD values specifying 7 characters of the symbol_rate in Msymbol/s where the decimal point occurs after the third character (e.g. 027.4500).

FEC_inner: The FEC_inner is a 4-bit field specifying the inner FEC scheme used according to Table C.12.

Table C.12/J.94 – Inner FEC scheme

| FEC_inner bit 3210 | Description |
| --- | --- |
| 0000 | Not defined |
| 0001 | 1/2 conv. code rate |
| 0010 | 2/3 conv. code rate |
| 0011 | 3/4 conv. code rate |
| 0100 | 5/6 conv. code rate |
| 0101 | 7/8 conv. code rate |
| 1111 | No conv. Coding |
| 0110 to 1110 | Reserved for future use |

## C.3	Character code tables

The tables corresponding to Annex A are under study now.

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |