|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | L.46 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (10/2000) |
|  | SERIES L: CONSTRUCTION, INSTALLATION AND PROTECTION OF CABLES AND OTHER ELEMENTS OF OUTSIDE PLANT |  |  |  |
|  | Protection of telecommunication cables and plant from biological attack |  |  |  |
|  | ITU-T Recommendation L.46 (Formerly CCITT Recommendation) |  |  |  |

ITU-T Recommendation L.46 Protection of telecommunication cables and plant from biological attack

- Summary This Recommendation describes biological attacks and countermeasures for protection of telecommunication cables. It deals with the kinds of biological attack, weakness of cables, features of damage, and considers alternative ways of protecting the plant including dependence on cable position.

- Source ITU-T Recommendation L.46 was prepared by ITU-T Study Group 6 (1997-2000) and approved by the World Telecommunication Standardization Assembly (Montreal, 27 September  6 October 2000).

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2001

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from ITU.

CONTENTS

Page

1	Scope		1

2	The kinds of biological attacks		1

3	Types of damage experienced		1

3.1	Mammals (e.g. squirrels, rats, mice and gophers)		1

3.2	Insects		1

3.2.1	Termites and ants		1

3.2.2	Beetles, larvae and other insects		1

3.3	Birds		2

3.4	Micro-organisms		2

4	Type of protection		2

4.1	Positioning of plant in certain locations		2

4.2	Elimination of the pest		2

4.3	Protection of telecom plant		3

4.4	Cable construction		3

Appendix I		4

Appendix II		5

ITU-T Recommendation L.46

Protection of telecommunication cables and plant from biological attack

# 1	Scope

There is evidence that the outside of the plant can be damaged from biological attack. Some attacks are localized to particular environments which nurture certain types of infestation. Sometimes movement of the plant away from these areas is enough to avoid damage. More typically, the plant is shielded against attack by preventing penetration of pests some of which are listed in clause 2.

# 2	The kinds of biological attacks

It is known that cable damage may be caused by:

–	mammals, squirrels, mice, rats, moles, gophers and other rodents;

–	birds like woodpeckers and cockatoos;

–	insects such as termites, ants, beetles, wasps and caterpillars;

–	micro-organisms like bacteria, fungus and/or moulds.

# 3	Types of damage experienced

From an investigation of the damage sustained it is possible to determine the type of pest involved. Typical features of damage are given below.

## 3.1	Mammals (e.g. squirrels, rats, mice and gophers)

Vertical chiselling of incisors on the edge of irregular holes is typical of the damage caused by rodents.

Spherical or cylindrical holes with smooth walls are characteristic for the attack of moles. The animals are often found in the hole.

## 3.2	Insects

### 3.2.1	Termites and ants

Irregular holes that may extend over the whole circumference, looking as though the sheath has been peeled off, are typical of termites and ants.

A mat appearance, to a surface that has been chewed over, is typical of termite gnawing, because they dig and lever upward with their short squat mandibles. Scratch marks and a transverse assault on the cable are characteristic of the ant, which has long, sharp mandibles and adopts a cutting action. In the early stages, the attack is characterized by local gatherings of numerous, small, individual holes.

Ants and termites can give off an acidic secretion that is corrosive to lead.

### 3.2.2	Beetles, larvae and other insects

Horizontal traces of mandibles of different shapes on the edges of the holes, are indications of attacks by insects and larvae. Circular bores are typical of larvae and beetles. Depending on the angle at which they penetrate the sheath, they may look circular or elongated with a width equal to the diameter of the bore.

## 3.3	Birds

Irregular nibbles by beaks on the edge of and around irregular holes, are signs of attacks by birds.

NOTE  Earwigs, spiders, pupas and other animals, often invade junction boxes and other open spaces. If the ventilation holes are big enough, even bird-nests, squirrel-nests and mice-nests can be found in the boxes. The debris these animals (in 3.1 to 3.3) produce from their excrements, also contribute to the breakdown of the insulation of connectors and terminal blocks.

## 3.4	Micro-organisms

Deterioration of the mechanical properties of cellulose fibres and elasticized polymers, is characteristic of attacks by fungi and bacteria. These attacks depend on the material and its manufacturing process.

Local aeration corrosion cells can be produced by the attachment of barnacles, which are tight enough to exclude oxygen. These cells can be the result of bacterial metabolism in a polluted environment.

# 4	Type of protection

## 4.1	Positioning of plant in certain locations

Attacks on telecom plants may be confined to certain areas where the conditions allow the pests to thrive. If it is possible to re-route the plant away from these areas, instances of attack and plant damage can be avoided.

Examples of particular environments and the type of biological attacker found in them are shown in Table 1.

Table 1/L.46  Typical environment for pests

| Environment | Pest |
| --- | --- |
| Breeding places in rubber estates | Insects |
| Willow, cascara, scrub oak areas and old trees | Olive beetles |
| Telecom plant in contact with plants (vegetable) – cable drums-poles-wood debris-bamboo-roots | Insects and larvae |
| Dry rotting tree roots and wood debris | Termites |
| Above ground Up to 1.5 m in hard soil Deeper in soft soil | Ants |
| Polluted soil and water creating anaerobic conditions | Bacterial metabolism |
| Overhead | Squirrels and birds |

## 4.2	Elimination of the pest

Where it is not possible to re-site the plant one possible action could be to eliminate the pests such as insects and rodents using chemicals or poisons.

It should be noted that some chemicals and poisons are useful as a countermeasure for biological attacks, but they sometimes can also be a danger to humans and cause environmental pollution. Therefore, very careful handling is required when using any chemical method to prevent biological attacks. Also, materials' safety data and other specifications should be referred to before they are

used. Poisons and chemicals should be stored safely in accordance with the supplier's recommendations.

Some chemicals are useful as a deterrent. These chemicals do not work as poisons but they repel the attacker. For example, some chemicals are sprayed on the cable surface and leave a special smell as a deterrent.

## 4.3	Protection of telecom plant

Another option for protection of the plant is to shield it by the use of protective materials around the plant in order to prevent access. Examples of the way to prevent access of the pests are given in Table 2.

Table 2/L.46  External measures for protection against pests

| Action | Pest |
| --- | --- |
| Application of screens over ventilation opening | Squirrels, rodents and birds |
| Steel net over cable | Squirrels |
| Tightening of fittings in junction boxes, blocking with epoxy resin casting in small boxes | Ants, insects and beetles |
| Compact into hard soil around cables and joints | Ants and insects |
| Cables in small diameter duct to prevent entry | Rats and mice |
| Galvanized iron guards around the cables as they emerge from the ground | Rodents, ants and insects |
| Chemical repellents sprayed on telecom plant or spread around it | Insects, spiders and wasps |
| Covering/repair of damaged duct | Rats |

## 4.4	Cable construction

Cables with sheaths of lead, polyethylene, polyvinylchloride, neoprene and other polymers are all susceptible to attacks. Examples of cable constructions that offer protections are given in Table 3.

Table 3/L.46  Cable construction for protection against pests

| Protection | Pest |
| --- | --- |
| Steel wire, fibreglass or aramid armouring | Squirrels and rodents |
| Steel sheath or steel tape laminate | Birds |
| Polyamide sheath | Ants and termites |
| Helical steel or brass tape | Birds (woodpeckers) and termites |
| Helical steel (0.2 mm) lapping with bituminous compound flooding | Contact with plants induce beetle and larvae attack |

Aluminium tape offers limited protection, especially thin foil (e.g. 0.1 mm) that is easy to penetrate. Therefore, the thickness of aluminium tape should be considered carefully.

Cables with steel armouring and/or dielectrical armouring such as fibreglass do not suffer any damage of this type as long as the armour is intact. Discontinuity in the armor leads to damage from beetles and larvae. Openings in the steel armour may have their origin in imperfect splicing and careless repairs, wrinkles and mechanical damage or by corrosion.

Coatings of hessian, jute yarn or paper fibres impregnated with tar or bituminous compounds, give no protection.

Appendix I

Table I.1 shows the main biological attackers and countermeasures.

Table I.1/L.46  The main biological attackers and countermeasures

| Caused by | Attackers | Position | Troubles | Countermeasures |
| --- | --- | --- | --- | --- |
| Mammals | Rats and Mice | Underground cables Duct cables Indoor cables | Bite, Food, Cut | Armouring net OLMA Sealing of duct Chemicals |
| Mammals | Moles | Underground cables | Cut | Armouring net OLMA |
| Mammals | Squirrels | Aerial | Bite, Cut | OLMA |
| Birds | Woodpeckers Other Birds | Aerial | Peck, Make hole Make nest | Steel tape OLMA |
| Insects | Ants and Termites | Underground Aerial | Bite Bite | Polyamide jacket Steel tape and Brass tape Air tightness of closures |
| Insects | Cicada | Aerial | Lay Eggs, Make hole | Steel tape |
| Micro organism | Bacteria | ----------- | ---------- | --------- |
| OLMA Over Lay Metallic Armouring |  |  |  |  |

Appendix II

Experience for biological attacks is shown in this Appendix from Table II.1 to II.7. The information is gathered from Argentina, Italy, Japan, Spain, Ukraine and Brazil. The places of attacks are shown in Figure II.1.

Figure II.1/L.46  Illustration for places attacked by plants and animals

Table II.1/L.46  Argentine experience

| Place (Figure II.1) | Season | Caused by | Kind of cause | Example of trouble | When occurred | Countermeasures |
| --- | --- | --- | --- | --- | --- | --- |
| B | All season | Mammals | Rats | Bite |  | None |
| G | All season | Bacteria | Others | Peck |  | None |
| F | All season | Bacteria | Others | Make Nest |  | None |

Table II.2/L.46  Italian experience

| Place (Figure II.1) | Season | Caused by | Kind of cause | Example of trouble | When occurred | Countermeasures |
| --- | --- | --- | --- | --- | --- | --- |
| D | All season | Mammals | Rats | Bite | 1993 to 1995 | None |

Table II.3/L.46  Indonesian experience

| Place (Figure II.1) | Season | Caused by | Kind of cause | Example of trouble | When occurred | Countermeasures |
| --- | --- | --- | --- | --- | --- | --- |
| Jakarta | All season | Mammals | Rats | Bite, cut | 1998 | Making sealing of the cable's ducts |
| Jakarta | All season | Insect | Ants | Bite, make nest | 1997 | Chemicals, inserting wax |
| Ujung Pandang | All season | Insect | Ants | Bite, short circuit | 1997 | None |
| Many places | All season | Mammals | Mice | Bite, cut, short circuit, make nest | 1996-1998 | None |
| Ambon | All season | Birds | Cockatoos | Bite | 1997-1998 | None |

Table II.4/L.46  Japanese experience

| Place (Figure II.1) | Season | Caused by | Kind of cause | Example of trouble | When occurred | Countermeasures |
| --- | --- | --- | --- | --- | --- | --- |
| G | All season | Mammals, insects | Rats, squirrels, mice, caterpillars | Bite (Figures II.2 and II.3) | 1996-1999 | OLMA (Steel outer layer) (Figure II.4) Replacement with HS cable (Figure II.5) |
| F | Autumn | insects | Ants | Bite | 1998 | Improvement of airtightness |
| H | Summer | insects | Cicada | Lay eggs | 1998 | OLMA (Steel outer layer) Replacement with HS metallic wire |

Figure II.2/L.46  Example of trouble by rats

Figure II.3/L.46  Example of trouble by caterpillars

Figure II.4/L.46  OLMA

Figure II.5/L.46  Configuration of high strength (HS) cable

Table II.5/L.46  Spanish experience

| Place (Figure II.1) | Season | Caused by | Kind of cause | Example of trouble | When occurred | Countermeasures |
| --- | --- | --- | --- | --- | --- | --- |
| D | All season | Mammals | Rats | Cut | 1996 | None |
| M | All season | Mammals | Rats | Cut | 1996 | None |
| M | All season | Mammals | Rats | Cut | 1997 | None |
| M | All season | Mammals | Rats | Cut | 1997 | Sealing |

Table II.6/L.46  Ukrainian experience

| Place (Figure II.1) | Season | Caused by | Kind of cause | Examplof trouble | When occurred | Countermeasures |
| --- | --- | --- | --- | --- | --- | --- |
| J (3 events) | All season | Mammals | Rats | Bite | 1998 | Over lay metallic fray |
| J (3 events) | All season | Mammals | Rats | Food, Bite | 1998 | OLMA |
| D, J | All season | Mammals | Rats | Bite, Cut, Scratch | 1998 | Armouring Net, chemicals |
| D | All season | Mammals | Rats | Bite | 1996 | Over lay |
| D (20 event) | All season | Mammals | Rats | Bite | 1997 | Over lay |
| D | All season | Mammals | Rats | Bite | 1998 | Making sealing of duct |
| D (2 event) | All season | Mammals | Rats | Cut, Food, Bite | 1998 | Over lay |
| D | All season | Mammals | Rats | Food | 1998 | OLMA |

Table II.6/L.46  Ukrainian experience (continued)

| Place (Figure II.1) | Season | Caused by | Kind of cause | Examplof trouble | When occurred | Countermeasures |
| --- | --- | --- | --- | --- | --- | --- |
| D (manhole) | All season | Mammals | Rats | Bite | 1998 | OLMA |
| E | All season | Mammals | Rats | Scratch (gnawed sheath) | 1997 | Armouring Net |
| E (2 event) | All season | Mammals | Rats | Food (gnawed sheath) | 1998 |  |
| D | Spring Summer | Mammals | Rats | Bite (gnawed sheath) | 1998 |  |
| D (10 event) | Spring Summer | Mammals | Rats | Food (gnawed sheath) | 1998 | Chemicals |
| D (manhole 4 event) | Spring Summer | Mammals | Rats | Food (gnawed sheath) | 1998 | Chemicals |
| D (manhole) | Summer Autumn | Mammals | Rats | Bite (gnawed sheath | 1998 |  |
| J | Spring | Mammals | Rats | Bite | 1997 |  |
| J (4 event) | Spring | Mammals | Rats | Bite | 1998 |  |
| J | Spring | Mammals | Rats | Bite | 1998 | Over lay |
| J | Spring | Mammals | Rats | Bite | 1998 | Chemicals |
| D (2 event) | Spring | Mammals | Rats | Bite (FO damaged) | 1996 |  |
| D (2 event) | Spring | Mammals | Rats | Bite | 1998 | Chemicals |
| D (2 event) | Spring | Mammals | Rats | Food, Bite | 1998 | Over lay |
| D (7 event) | Spring | Mammals | Rats | Bite | 1998 |  |
| D (manhole 4 event) | Spring | Mammals | Rats | Bite | 1996 | Chemicals |
| D (manhole) | Spring | Mammals | Rats | Bite (damaged cables; 10 mm) | 1998 | Making embedding cable with OLMA |
| B | Spring | Mammals | Rats | Bite | 1998 | Armouring Net |
| E (2 event) | Spring | Mammals | Rats | Food | 1998 | OLMA |
| E | Spring | Mammals | Rats | Bite | 1998 | Armouring Net |
| E | Spring | Mammals | Rats | Bite | 1998 | Chemicals |

Table II.6/L.46  Ukrainian experience (continued)

| Place (Figure II.1) | Season | Caused by | Kind of cause | Examplof trouble | When occurred | Countermeasures |
| --- | --- | --- | --- | --- | --- | --- |
| J (3 event) | Summer | Mammals | Rats | Bite | 1998 | Over lay |
| D | Summer | Mammals | Rats | Bite (FO damaged) | 1996 |  |
| D | Summer | Mammals | Rats | Bite (Damaged cables; 3 m) | 1996 | Over lay |
| D | Summer | Mammals | Rats | Food (consumed leaden sheath) | 1996 | Over lay |
| D (2 event) | Summer | Mammals | Rats | Bite (damaged cable; 0.5-3 m) | 1997 | Over lay |
| D | Summer | Mammals | Rats | Food | 1998 | Over lay |
| D | Summer | Mammals | Rats | Food | 1998 | Over lay |
| D (4 events) | Summer | Mammals | Rats | Bite | 1998 |  |
| D (manhole 4 events) | Summer | Mammals | Rats | Bite | 1998 | Chemicals |
| D (manhole 20 events) | Summer | Mammals | Rats | Bite | 1998 |  |
| E | Summer | Mammals | Rats | Food | 1998 | OLMA |
| E | Summer | Mammals | Rats | Bite | 1998 | Armouring Net |
| E | Summer | Mammals | Rats | Cut | 1998 |  |
| E | Summer | Mammals | Rats | Bite | 1998 | Chemicals |
| D | Autumn | Mammals | Rats | Bite (FO damaged) | 1998 |  |
| D | Autumn | Mammals | Rats | Food | 1998 | Over lay |
| D | Autumn | Mammals | Rats | Bite | 1998 | Chemicals |
| D (50 events) | Autumn | Mammals | Rats | Cut | 1998 |  |
| D (20 events) | Autumn | Mammals | Rats | Cut | 1998 | Chemicals |
| D (8 events) | Autumn | Mammals | Rats | Scratch, Food, Bite | 1998 | Over lay |
| D (manhole) | Autumn | Mammals | Rats | Bite (FO damaged) | 1997 |  |
| D (manhole) | Autumn | Mammals | Rats | Bite | 1998 |  |

Table II.6/L.46  Ukrainian experience (continued)

| Place (Figure II.1) | Season | Caused by | Kind of cause | Examplof trouble | When occurred | Countermeasures |
| --- | --- | --- | --- | --- | --- | --- |
| D (manhole 4 events) | Autumn | Mammals | Rats | Bite | 1998 | Chemicals |
| E (2 events) | Autumn | Mammals | Rats | Cut | 1998 | Chemicals |
| J | Winter | Mammals | Rats | Bite | 1998 | Chemicals |
| J | Winter | Mammals | Rats | Cut | 1998 |  |
| D | Winter | Mammals | Rats | Cut | 1997 | OLMA |
| D (2 events) | Winter | Mammals | Rats | Bite | 1998 |  |
| D | Winter | Mammals | Rats | Bite | 1998 | Over lay |
| D (manhole) | Winter | Mammals | Rats | Bite | 1998 |  |
| D | Autumn | Mammals | Mice | Bite, Cut, Scratch (gnawed sheath: 40×100 mm) | 1998 | Armouring Net, chemicals |
| E | Autumn | Mammals | Mice | Scratch (gnawed sheath) | 1996 |  |
| E | Spring – Autumn | Mammals | Mice | Bite, Food, Nest (D=0.5-10 mm) | 1997 1998 | Chemicals |
| E | Spring Summer | Mammals | Mice | Cut, Food | 1998 |  |
| D | Spring Summer | Mammals | Mice | Bite (gnawed sheath) | 1998 |  |
| E (2 events) | Spring | Mammals | Mice | Food, Cut, Make Nest, Scratch | 1998 | OLMA |
| E | Spring | Mammals | Mice | Bite (circle d=10 mm) | 1997 | OLMA |
| E | Spring | Mammals | Mice | Food (damaged cables) | 1998 |  |
| D (2 events) | Spring | Mammals | Mice | Cut, Food (damaged cables) | 1998 |  |
| D (2 events) | Spring | Mammals | Mice | Others | 1998 | Over lay |

Table II.6/L.46  Ukrainian experience (continued)

| Place (Figure II.1) | Season | Caused by | Kind of cause | Examplof trouble | When occurred | Countermeasures |
| --- | --- | --- | --- | --- | --- | --- |
| E | Summer | Mammals | Mice | Bite (damaged cables: 4 m, 2 m) | 1997 | Over lay |
| E | Summer | Mammals | Mice | Make Holes | 1997-1998 | Chemicals |
| E (2 events) | Summer | Mammals | Mice | Bite | 1998 | Armouring Net |
| E (2 events) | Summer | Mammals | Mice | Cut, Make Nest, Scratch | 1998 | OLMA |
| E | Summer | Mammals | Mice | Food | 1998 |  |
| E (2 events) | Summer | Mammals | Mice | Cut, Bite (damaged cables) | 1998 |  |
| E | Summer | Mammals | Mice | Food (damaged cables) | 1998 |  |
| D | Summer | Mammals | Mice | Food (damaged cables) | 1998 |  |
| D (2 events) | Summer | Mammals | Mice | Cut, Food (damaged cables) | 1998 |  |
| D | Summer | Mammals | Mice | Food (damaged cables) | 1998 |  |
| D (manhole) | Summer | Mammals | Mice | Others | 1998 | Chemicals |
| E (events) | Autumn | Mammals | Mice | Bite (damaged cables: 2~6 m, 10 m) | 1991-1992, 1996 | Over lay |
| E | Autumn | Mammals | Mice | Others | 1997 | Over lay |
| E (events) | Autumn | Mammals | Mice | Cut, Food (damaged cables) | 1998 |  |
| E | Autumn | Mammals | Mice | Food | 1998 | OLMA |
| D | Autumn | Mammals | Mice | Others | 1997 | Over lay |
| D | Autumn | Mammals | Mice | Food (damaged cables) | 1998 |  |

Table II.6/L.46  Ukrainian experience (continued)

| Place (Figure II.1) | Season | Caused by | Kind of cause | Examplof trouble | When occurred | Countermeasures |
| --- | --- | --- | --- | --- | --- | --- |
| J (2 events) | Autumn | Mammals | Rats, Mice | Bite, Make Nest (gnawed sheath: 2 m~10 m) | 1997-1998 | Chemicals Armouring Net |
| D (2 events) | Autumn | Mammals | Rats, Mice | Bite, Make Nest (gnawed sheath: 2 m~10 m) | 1997-1998 | Chemicals Armouring Net |
| B (2 events) | Autumn Summer | Mammals | Rats, Mice | Cut, Bite | 1998 | Over lay |
| E (12 events) | Winter | Mammals | Rats, Mice | Cut, Bite | 1998 | Over lay |
| D (manhole) | Autumn | Mammals | Rats, Mice | Bite, Cut (L=30 mm) | 1998 | Over lay |
| E (37 events) | Autumn | Mammals | Rats, Mice | Bite (gnawed sheath: up to 1 m) | 1998 |  |
| E | Spring Summer Autumn | Mammals | Rats, Moles | Bite, Food, Make Nest, Cut (gnawed sheath: 10 mm~ 3 m) | 1997-1998 | Chemicals Armouring Net |
| D (39 events) | Autumn | Mammals | Rats, Mice, Gophers | Cut, Bite, Food, Scratch | 1997-1998 | Over lay Chemicals |
| E | Autumn | Mammals | Gophers | Food | 1998 | OLMA Chemicals |
| E (3 events) | Spring | Mammals | Gophers | Bite (damaged cables: 15 m, 10 m) | 1998 | Over lay |
| E | Spring | Mammals | Gophers | Bite | 1998 | Armouring Net |
| E | Spring | Mammals | Gophers | Cut | 1997 |  |
| E | Summer | Mammals | Gophers | Bite (damaged cables: 2 m) | 1996 | Over lay |
| E | Summer | Mammals | Gophers | Bite (damaged cables: 2 m) | 1997 | Over lay |
| E | Summer | Mammals | Gophers | Cut | 1998 | Armouring Net |

Table II.6/L.46  Ukrainian experience (continued)

| Place (Figure II.1) | Season | Caused by | Kind of cause | Examplof trouble | When occurred | Countermeasures |
| --- | --- | --- | --- | --- | --- | --- |
| E | Summer | Mammals | Gophers | Food | 1996 | OLMA Chemicals |
| E (3 events) | Autumn | Mammals | Gophers | Bite, Make Nest (damaged cables: 0.1 m, 0.5 m) | 1997 | Over lay |
| E (3 events) | Autumn | Mammals | Gophers | Bite (damaged cables: 0.2 m,0.5 m) | 1995 1997 | Over lay |
| E (2 events) | Autumn | Mammals | Gophers | Cut | 1998 | Armouring Net |
| E (2 events) | Autumn | Mammals | Gophers | Food | 1995 1997 | OLMA Chemicals |
| E | Autumn | Mammals | Gophers | Cut, Make Nest, Scratch | 1998 | OLMA |
| E | Winter | Mammals | Gophers | Bite (damaged cables: 50 mm) | 1998 | Over lay |
| E (6 events) | Winter | Mammals | Gophers | Bite (damaged cables: 0.15~0.3 m, 20 m) | 1996 1998 | Over lay |
| E | Winter | Mammals | Gophers | Cut, Make Nest, Scratch | 1998 | OLMA |
| E (4 events) | Spring | Mammals | Moles | Cut, Food (damaged cables) | 1998 |  |
| E (2 events) | Summer | Mammals | Moles | Bite (gnawed sheath), Food (damaged cables) | 1998 |  |
| E | Summer | Mammals | Moles | Make Holes | 1998 | Armouring Net |
| E | Autumn | Mammals | Moles | Bite (gnawed sheath) | 1998 |  |
| E | Autumn | Mammals | Moles | Food | 1998 | OLMA |

Table II.6/L.46  Ukrainian experience (continued)

| Place (Figure II.1) | Season | Caused by | Kind of cause | Examplof trouble | When occurred | Countermeasures |
| --- | --- | --- | --- | --- | --- | --- |
| E | Summer | Mammals | Hamsters | Food (gnawed sheath) | 1998 | OLMA |
| E (13 events) | Spring Autumn | Mammals | Hamsters | Bite | 1998 |  |
| E (3 events) | Winter | Mammals | Hamsters | Bite (hole d:20 mm) | 1996- 1998 | OLMA |
| E | Summer | Mammals | Hamsters | Bite | 1998 | Armouring Net |
| E (2 events) | Autumn | Mammals | Hamsters | Bite | 1998 | Armouring Net |
| E (2 events) | Summer | Mammals | Beavers | Cut | 1995- 1996 | Armouring Net |
| E (3 events) | Autumn | Mammals | Beavers | Cut | 1998 | Armouring Net |
| E | Summer | Mammals | Ondatra | Bite (damaged cables: 0.3 m) | 1995 | Over lay |
| D (manhole) | Winter | Mammals | Dogs | Bite (FO damaged) | 1994 |  |
| G (4 events) | Summer | Birds | Woodpeckers | Peck (peck and holes in poles) | 1998 |  |
| F (2 events) | Spring | Birds | Woodpeckers | Peck (holes d=25 mm, 30 mm) | 1998 |  |
| F (10 events) | Autumn | Birds | Woodpeckers | Peck (nest) | 1998 |  |
| F (8 events) | Spring Summer | Birds | Storks | Make Nest | 1998 | Over lay |
| F (3 events) | Autumn Summer | Insects | Beetles (barkbeetles) | Lay Eggs Food | 1997- 1998 | Over lay |
| H (2 events) | Autumn | Insects | Wasps | Cut, Make Nest | 1998 | Over lay |
| I | Summer Autumn | Insects | Wasps | Make Nest | 1998 | Over lay |
| E | Spring | Insects | Ants | Lay eggs, Degradation of insulation | 1998 | Over lay |
| D | Autumn | Bacteria | Unknown | Culture media | 1998 |  |
| E | Autumn | Bacteria | Unknown | Culture media | 1998 |  |

Table II.6/L.46  Ukrainian experience (concluded)

| Place (Figure II.1) | Season | Caused by | Kind of cause | Examplof trouble | When occurred | Countermeasures |
| --- | --- | --- | --- | --- | --- | --- |
| D | Autumn | Bacteria | Unknown | Culture media | 1998 |  |
| D | Spring | Fungus | Fungus | Corrosion | 1998 | Chemicals |
| E | Summer | Plants | Unknown | Cut | 1998 |  |

Table II.7/L.46  Brazilian experience

| Place (Figure II.1) | Season | Caused by | Kind of cause | Example of trouble | When occurred | Countermeasures |
| --- | --- | --- | --- | --- | --- | --- |
| B (Buried Optical Cable) | All season | Mammals | Rodents | Bite, Make Holes | 1994 | OLMA, dielectric armour |
| B (Induct Optical cable) | All season | Mammals | Rats | Bite, Make Holes, Scratch, Cut | 1996 | OLMA (Figure II.6) |
| F (Aerial Closure-metallic) | All season | Insects | Ants | Make Pressure, Corrosion | 1994 | – |
| E (Metallic Cable) | All season | Insects | Termites | Make Holes | 1982 | OLMA (Figure II.7) |
| G (Aerial Optical Cable) | All season | Birds | Others | Bite, Make Holes | 1993 | – |
| B (Buried Optical Cable) | All season | Mammals | Rodents | Bite, Make Holes, Scratch | 1995 | OLMA, dielectric armour (Figure II.8) |
| G (Aerial Optical Cable) | All season | Mammals | Rodents | Bite, Make Holes | 1999 | (Figure II.9) |
| OLMA Over Lay Metallic Armouring |  |  |  |  |  |  |

Figure II.6/L.46  Induct optical cable

Figure II.7/L.46  Metallic cable damaged by insect

Figure II.8/L.46  Buried cable

Figure II.9/L.46  Aerial cable

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |