|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | L.62 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (09/2004) |
|  | SERIES L: CONSTRUCTION, INSTALLATION AND PROTECTION OF CABLES AND OTHER ELEMENTS OF OUTSIDE PLANT |  |  |  |
|  | Practical aspects of unbundling services by multiple operators in copper access networks |  |  |  |
|  | ITU-T Recommendation L.62 |  |  |  |

ITU-T Recommendation L.62 Practical aspects of unbundling services by multiple operators in copper access networks

Summary In many countries, Other Licensed Operators (OLOs) are allowed to compete with the incumbent operator. This creates an environment where a company has to install, operate and maintain its network bearing in mind that other networks exist right beside it, or even at the same location. In a number of countries, it is also determined that the operators should share some parts of the network with OLOs, in a transparent process to the users. This is called Unbundling of Network Elements or, in short, unbundling, and is a very complex task. Some new issues have to be taken into account to allow the accommodation of those operators sharing the same location to do so without problems. To guarantee an environment where operators interact but do not affect the Quality of Service provided by other operators, legal, regulatory and administrative statements must be followed by the correct technical solutions, which assure the network integrity, easy use of equipment and access to security. This Recommendation is intended to provide the guidelines to achieve these targets for the local loop in copper networks.

Source ITU-T Recommendation L.62 was approved on 6 September 2004 by ITU-T Study Group 6 (2001-2004) under the ITU-T Recommendation A.8 procedure.

Keywords Copper cable, ISDN, local loop, network element, shared network, unbundling, xDSL.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g. interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met.  The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2005

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Definitions		2

4	Abbreviations		3

5	Topologies		4

5.1	Indoor		4

5.2	Outdoor		5

6	Issues related to unbundling		5

6.1	Interfacing operators on the MDF		5

6.2	Interfacing operators on cabinets or closures		6

6.3	Interfacing operators on aerial installations		6

6.4	Interfacing operators on buried installations		6

6.5	Interfacing operators in the manhole		6

6.6	Interfacing operators on the customer premises		6

6.7	Cable frequency spectrum management plan		7

6.8	Installation and maintenance		7

7	Issues related to co-location		7

7.1	Typical co-location inside MDF		7

7.2	Virtual co-location inside MDF		8

7.3	Co-location outside MDF		8

8	Shared sub-loops		9

Introduction

Network elements can be unbundled in a number of ways which are described in this Recommendation. Co-location is also described, as it is intrinsically related to unbundling.

Crucial aspects must be foreseen, e.g., the evolution of the optical access network, which reduces the copper network, getting closer to the customer premises. It may be difficult to change the network topology maintaining services already provided by OLOs in the same network element.

It must be noted that when multiple services are provided over the same network element, each service must not affect the Quality of Service provided by other operators and shall comply with the Recommendations applicable for each service when provided alone. Also, a service must not interfere with other services in use, or which are planned to be used, in a network element.

ITU-T Recommendation L.62

Practical aspects of unbundling services by multiple
operators in copper access networks

# 1	Scope

This Recommendation relates to the technical aspects of copper local loop unbundling which, it should be noted are subject to national regulations. Although a number of network elements can be defined, this Recommendation focuses specifically on the copper local loop because it is the most prevalent worldwide.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

–	ITU-T Recommendation K.58 (2003), EMC, resistibility and safety requirements and procedures for co-located telecommunication installations.

–	ITU-T Recommendation K.59 (2003), EMC, resistibility and safety requirements and procedures for connection to unbundled cables.

–	ITU-T Recommendation L.19 (2003), Multi-pair copper network cable supporting shared multiple services such as POTS, ISDN and xDSL.

# 3	Definitions

This Recommendation defines the following terms.

For better understanding of the terms, the main types of unbundling of the local loop are outlined in Figure 1 and described below. See abbreviations on the next item.

Figure 1/L.62 – Typical types of unbundled network elements

3.1	full unbundling: OLO has direct access to the twisted copper pair and can use the pair to provide any service allowed (xDSL, voice), without interfering with other services provided in the same cable, usually associated with co-location of OLO's equipment.

3.2	line sharing: Voice service (POTS, PSTN) and data transmission services (xDSL, ISDN) are provided using the same copper pair. Voice is provided in the lower frequency band and data transmission in the higher frequency band. Both operators must physically access the pair, and the OLO's equipment is usually housed in a co-location area inside the incumbent's central office building. Splitters have to be installed, both in the customer's premises and in the central office, to guide the signals suitably to the computer and to the telephone terminal, and to their correspondent networks.

3.3	bitstream: Actually, this is not unbundling, but a higher layer service, with less physical contact between the incumbent and the OLO. In this Recommendation, this term includes the wholesale services provided by the incumbent to other operators that, otherwise, would not have access to their customers. Typically, the incumbent operator installs and operates the network from the customer premises to the DSLAM, providing the OLO with a bundle of channels (usually STM-1) usually associated with a transportation service (back haul) to the nearest OLO's PoP.

3.4	co-location: An OLO may want to (or may be obliged to) rent space in the incumbent's central office to install its own equipment. Figure 1 shows a typical co-location area inside the CO, with the OLO's DSLAM installed on it. Due to an eventual lack of suitable space, there are some different kinds of co-location arrangements. When some OLOs want to (or may be obliged to) use the same room (or cabinet), there is a "shared co-location". A "virtual co-location" takes place when the DSLAM belongs to the OLO, but is installed and operated by the incumbent. Also, equipment may be housed outside the incumbent's building (remote co-location), in another building or in containers located in parking lots, for example. Another special situation, described in 3.5, occurs when an outdoor remote DSLAM is needed.

3.5	outdoor remote DSLAM: This is the toughest situation, requiring critical technical solutions. When a network has remote concentration (or switching) elements in cabinets along the copper loops, it may be necessary to connect those elements to the OLO's network. Usually there is no space for co-location and the elements are in an aggressive environment. This connection can be a very challenging task, as the equipments of both operators may have to be able to accept external tie cables and/or have space inside to install the OLO's DSLAM. Other solutions may include connection of the incumbent's cabinet to the OLO's cabinet, compatible printed circuit cards inserted in the incumbent's remote equipment, and the transport of the signals by the incumbent to the OLO's nearest PoP. The physical access is an important issue that has to have a convenient solution, because of security matters. Special attention is needed when the insertion of signals is allowed at some point along the cable between the central office and the customer's premises. Such signals may interfere in the other services already provided on the same cable since they can be stronger than the others at the insertion point.

3.6	sub-loop: There are special circumstances, usually in long copper loops, where parts of the local loop are rented separately from each other. The individual sub-loop elements usually available are feeders, distribution and concentration elements. Problems and solutions are similar to those listed for the outdoor remote DSLAM.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

ADSL	Asymmetric Digital Subscriber Line

CO	Central Office

DSLAM	Digital Subscriber Line Access Multiplexer

EMC	Electromagnetic Compatibility

EMI	Electromagnetic Interference

HDF	Handover Distribution Frame

HDSL	High bit rate Digital Subscriber Line

ISDN	Integrated Services Digital Network

LLU	Local Loop Unbundling also called Unbundling of the Local Loop (ULL)

MDF	Main Distribution Frame

NE	Network Element

NTP	Network Termination Point

OLO	Other Licensed Operator

PoP	Point of Presence

POTS	Plain Old Telephone Service

PSTN	Public Switched Telephone Network

SDSL	Symmetric Digital Subscriber Line

SHDSL	Single pair High speed Digital Subscriber Line

SSDSL	Synchronized Symmetrical Digital Subscriber Line

STM	Synchronous Transport Module

UADSL	Universal Asymmetric Digital Subscriber Line

UNE	Unbundling of Network Elements

VDSL	Very high-speed Digital Subscriber Line

xDSL	Any of the various types of Digital Subscriber Lines

# Topologies

The connection between incumbent and OLO equipments can be done at points that depend on the topology of the network. The following items describe some recommended configurations.

## 5.1	Indoor

Figure 2/L.62 – Typical DSL network and indoor connection points

## 5.2	Outdoor

Figure 3/L.62 – Typical DSL network and outdoor connection points

# Issues related to unbundling

Specific rules must be agreed upon with all operators that share a network element. There should be security rules related to physical access, fire safety, electrical compatibility, EMC/EMI, etc.

Information about that part of the network should be available, to avoid problems like interruptions, interferences, etc.

## 6.1	Interfacing operators on the MDF

Figure 4/L.62 – Typical MDF connection points

## 6.2	Interfacing operators on cabinets or closures

Figure 5/L.62 – Typical cabinets and closures connection points

## 6.3	Interfacing operators on aerial installations

This infrequent configuration needs further study.

## 6.4	Interfacing operators on buried installations

This infrequent configuration needs further study.

## 6.5	Interfacing operators in the manhole

Figure 6/L.62 – Typical manhole connection points

## 6.6	Interfacing operators on the customer premises

This subject is for further study.

## 6.7	Cable frequency spectrum management plan

The occupation of the frequency spectrum must be determined before the shared services start, to avoid interferences. A spectrum management policy must be implemented and respected by all operators involved. Some services, e.g., voice, may have precedence over others.

Factors that must be taken into account include: cable characteristics related to data transmission; connector characteristics; copper pair pre-qualification and conditioning; services allowed simultaneously and their protocols; maximum number of pairs used for each service in a cable; selection of pairs as a function of different services, etc.

## 6.8	Installation and maintenance

The involved operators must agree upon the installation and maintenance procedures, including action flowchart; time-limits and responsibilities; network integrity assurance; implementation of new technologies and topologies; proceedings to solve conflicts, etc.

# 7	Issues related to co-location

Specific rules must be agreed upon with all operators that co-locate equipments. There should be security rules related to physical access, fire safety, electrical compatibility, EMC/EMI, etc.

## 7.1	Typical co-location inside MDF

Figure 7/L.62 – Typical co-location inside MDF

## 7.2	Virtual co-location inside MDF

Figure 8/L.62 – Typical virtual co-location inside MDF

## 7.3	Co-location outside MDF

Figure 9/L.62 – Typical co-location outside MDF

# 8	Shared sub-loops

Specific rules must be agreed upon with all operators that share sub-loops. There should be security rules related to physical access, fire safety, electrical compatibility, EMC/EMI, etc.

Special attention must be given to external tie cables and the related hardware to be used at the operators interface point, e.g., splices, connectors, etc.

Figure 10/L.62 – Typical shared sub-loop

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure, Internet protocol aspects and Next Generation Networks |
| Series Z | Languages and general software aspects for telecommunication systems |

Extracted from ITU's abbreviations database (http://www.itu.int/sancho).