|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | L.63 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (10/2004) |
|  | SERIES L: CONSTRUCTION, INSTALLATION AND PROTECTION OF CABLES AND OTHER ELEMENTS OF OUTSIDE PLANT |  |  |  |
|  | Safety procedures for outdoor installations |  |  |  |
|  | CAUTION ! PREPUBLISHED RECOMMENDATION This prepublication is an unedited version of a recently approved Recommendation. It will be replaced by the published version after editing. Therefore, there will be differences between this prepublication and the published version. |  |  |  |

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g. interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met.  The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU [had/had not] received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2004

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

ITU-T Recommendation L.63

Safety Procedures for Outdoor Installations

(Geneva, 2004)

Summary

This Recommendation has the objective to establish the procedures of safety for personnel and operation and protection against fire in outdoor telecommunications installations and in shared infrastructures.

# CONTENTS

1	Scope

2	References

3	Infrastructure

3.1	Duct systems

3.2	Cable tunnels

3.3	Manholes

3.4	Sewage infrastructure installation

4	Outside plant network

4.1	Aerial network

4.2	Buried network

5	Outside plant equipments

6	Shared infrastructure

# 1	Scope

This recommendation seeks to guide the several administrations in safety procedures for personnel and fire protection for outdoors Telecommunications installations, such as: duct systems, manholes, tunnels, aerial, underground and buried networks, subscribers, equipments for outside plant, and networks in sewage infrastructures.

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

ITU-T Recommendation L.20- Creation of a Fire Security Code for Telecommunications Facilities

ITU-T Recommendation L.21 - Fire Detection and Alarm Systems, Detector and Sounder Devices

ITU-Recommendation L.22- Fire Protection

ITU-T Recommendation K.11- Principles of Protection Against Over Voltages and Over Currents

ITU-T Recommendation K.13- Induced Voltages in Cables with Plastics Insulated Cables

ITU-T Recommendation K.25 - Protection of Optical Fiber Cables

ITU-T Recommendation K.26 - Protection of Telecommunication Lines Against Harmful Effects From Electric Power and Electrified railway Lines

ITU-T Recommendation K.51 - Safety Criteria for Telecommunication Equipment

ITU- T Recommendation K.58 - EMC, resistibility and safety requirements and procedures for co-located telecommunication installations

ITU- T Recommendation K.59 - EMC, resistibility and safety requirements and procedure for connection to unbundled cables

ITU- T Recommendation K.64 - Safe working practices for outside equipment installed in particular environments

# 3	Infrastructure

For the purpose of this Recommendation infrastructure will be considered as the following outdoor installations.

## 3.1	Duct Systems

Select permanent locations for underground structures considering future road developments and plans of others utilities.

Locate manholes away from road intersections considering safety of worker and the public.

Use bottom ducts first, use outside ducts first.

In the construction or maintenance of duct lines, the projects must first be analyzed observing rigorously all the procedures of security demanded by the administration for the workmanship and a total knowledge that this installed in the route to be constructed or recovered.

To have to hand the complete project of all the existing installations in the place, type of land, adequate material of security, signalling, models of typical shoring and mechanicals equipments.

In the case of the existence of water canalization, fuel, gas, energy the required procedures of security must be observed for each in case that, mainly observing the depths demanded for each administration.

The use of ducts and spacers constructed of Poly(vinyl)chloride (PVC) or polyethylene (PE) and, if possible, concrete is recommended.

For each type of installation the security procedures are restricted, depth of ditch, type of land, and control of signalling of the performance of the worker when of its in the intervention.

## 3.2	Cable Tunnels

Normally cable tunnels are constructed inside the building of the telephone central office. The systems of security are normally smoke and fire and are controlled by detectors (visible and/or audible), fire extinguishers, and gas detection that are installed within the tunnel and the galleries, and supervising through proper systems of detection.

It is recommended that the projected detection systems to be electrically activated use low voltage to prevent the risk of explosion, mainly for the possibility of the existence of gases in the tunnels.

All security devices must be constantly monitored. In case of the tunnels where there exists shared infrastructure it is recommended that each company makes the identification and that the installations are kept of the distances appropriated.

In the case of tunnels, cables to be next to installations of gas or the gasoline or sharing infrastructure, it is recommended alarm devices are used for the presence of smoke, gas or gasoline.

In the tunnels of cables the same ones must be filled with earth using itself of the same point of grounded of the central office. In all the underground splices must be filled with bonding with the grounded of the chambers.

## 3.3	Manholes

When opening manholes the following considerations are recommended:

- when opening the manhole, observe the presence of water, gasoline, fuel, etc., inside;

- to carry thorough basic tests of gas detection;

- to wait a minimum of 10 minutes at least before entering;

- to keep gas detectors operational, if possible, while working inside;

- to keep activated an audible alarm system;

- to only use, if possible, equipment with low voltage and low current;

- not to remain for long periods in inside of the chambers;

- to always keep a means of fast exit from the chambers (stairs);

- to always work with two or more workers;

- to always maintain a system of external signalling;

- during rain, to prevent the water entrance to the inside of chambers;

- in the case of the use of commercial power to install a device of protection against excessive current and voltage;

- in all entrances of manholes, points of control for pressurization of cables must exist

## 3.4	Sewage Infrastructure Installations

In sewage installations, cables and components must be manufactured observing all the protection of health for the worker against possible contamination.

The rules established by the health and environmental administrations regarding this situation must be observed.

All workers must be oriented by specialists in health and environment.

Due to the aggressiveness of these installations, this subject will have be the object of further study.

# 4	Outside Plant Network

## 4.1	Aerial Network

For security against fire in networks’ telecommunication cables, the following considerations are recommended:

- To use in cables sheaths manufactured with components not flame propagating, with low smoke and zero halogen emission environments of great agglomeration of people, such us subway or train stations;

- When projecting the installation of the cables or wires observing the distances of separation of energy networks and the maximum arrow of the cable or wire in relation to the ground.

- The ground points must be independent of the networks of electric energy.

- Verify a level of tension between phases.

- It is NOT recommended to plan or install equipment that needs grounding in poles where exists energy grounded points.

## 4.2	Buried Network

The minimum depth of the buried cables is typically 1,20 m.

Buried cables may be protected against keraunic activity by the installation of a copper conductor or steel wire of 6 mm², installed approximately 600 mm above of the cable. During installation, the cables must be placed with an indication band 200 mm above the cable.

Along of the installation may be indicated through landmarks the existence of buried cables. For protection of them splices must be located in places of easy access.

This type of installation requires a system of efficient supervision in the direction to signal the more correctly possible localization of damages.

# 5	Outside Plant Equipments

Some equipment is installed in outside plant such us subscriber multiplex, DSLAMs, amplifiers for CATV, Subscriber Remote Units, HDSL Modems, Optical Distribution Frame, Main Distribution Frame, and SDH MUX.  Which safety and fire control must be remote and efficiently made on the office building.

# 6	Shared Infrastructure

It is recommended that for shared infrastructure, protection against fire and safety are made of form not to overlap one of the others.

The types of protections can be made of differentiated form and thus each worker can work and if to protect inside of limit of performance.

A minimum distance between infrastructures must be observed.

Appendix I

Ukraine Experience

Fire safety in the Duct System - Ukraine Experiences

# Introduction

This Appendix contains some information about the duct system in Ukraine as well as accepted fire safety measures in the duct system. The answers to the Questionnaire of fire safety in the duct system are attached.

# I.1	Duct system of Ukraine

The communication duct system consists of buried ducts and different types of inspection pits (e.g. manholes, collectors, tunnels and cable chambers).

The buried ducts consist of packets of separate pipes or from multihole blocks. The maximum capacity of duct system is 48 or 60 channels. The overall distance between some inspection pits is 150 m.

The duct materials are concrete, asbestos, polyethylene, steel and ceramic (Figure I.1).

Figure I.1. Different types of a duct (concrete, asbestos, polyethylene, steel)

The oldest constructions are the concrete blocks. The rectangular blocks by length of 1 m have one, two or three holes (diameter -100 mm). The separate blocks are end to end combined and joined by means of cement solution.

The most widespread constructions are asbestos ducts. The separate ducts by length of 3-6 m have inside diameter 100 mm. The ducts are joined by means of cement solution or by ring joint.

However, the using of asbestos ducts is dangerous for health of constructing personal and maintenance personnel. Therefore it is advised not to use this kind of materials when the new ducts system are constructed.

Recently polyethylene ducts are applied in big cities. The separate HDPE ducts by length of 8-12 meter have outside diameter 110 mm. The ducts are joined by fusing or by ring joint.

The short steel tubes are used for the entering ducts into the buildings. And steel tubes are applied in free-flowing grounds. The separate ducts by length of 4-12 m have inside diameter 90-96 mm and joined among themselves by fusing. The steel ducts are joined with asbestos ducts by means of cement solution.

The ceramic ducts are used very rarely in the acid grounds only. The separate ducts by length of 0.9 m have inside diameter 100 mm.

The main type of inspection pit is a manhole. The reinforced concrete (entire or composite) manholes and brick manholes are used. Three types of manholes are applied: feed-through, angular, splitter (Figure I.2).

Figure I.2. Building blocks of a communication duct

Composite reinforced concrete manholes are most widespread. They consist of lower wall ring with the bottom and upper wall ring with overlap.

Brick manholes are less reliable and more permeable for gases and water. However it is easy to construct the manhole of any shape and dimensions. This is important where the duct system is under construction near some utilities or buildings.

The pig-iron hatches are built in manholes for input (Figure I.3). The inlet diameter is 600 mm. The hatch consists of pig-iron frame, pig-iron upper cover and steel lower cover. The lower cover has fitting for hanging of lock. Both covers have apertures for gas control without unclosing of covers. The mass of hatch is 138 kg (heavy type) or 82 kg (light type).

Figure I.3. The pig-iron hatch

There are cantilever hooks arrange in the manholes for layout of cables. There are cantilevers and cantilever hooks in the manholes for arrangement of cables (figure I.4).

Figure I.4. Pig- iron cantilever hooks

The collector (figure I.5) is underground room for a share locality of elements of different services (communication cables, electric power cables, hot-water pipes, steam conduits, air flow guides, drains pipelines and low pressure gas pipelines).

The depth of presence (finding) of collectors is 1 - 1.2 m from a ground surface. The in-house overall dimensions are: width - 1.7-2.7 m, height - 1.8-3.0 m.

Figure I.5 - Underground collector

In some cities the communication optical fibre cables are installed in tunnels of subway. They dispose on cantilever hooks of a special construction. In the passenger stations these cables are laid under platform and put into tunnel through vertical well holes. In the well holes the cables are attached to wall by special metal clamps.

# I.2	Fire safety measures in manholes, collectors and tunnels

Before the starting of operations in manhole and collectors the check of presence of gas should be executed by gas-analyser. When the gas is presented electrical ventilator should execute the compulsory ventilation. When the gas is absent the natural ventilation should be executed through open ducts.

The metal instruments covered by copper should be applied for opening the manhole cover. In winter when the cover freezes to frame of hatch the hot sand should be applied for opening the manhole cover.

For operating in the manhole the crew of not less than two men should be used. One man should stay outside of manhole always.

The illumination of manholes and collectors should be produced by low voltage (12V). The low voltage accumulators or portable batteries may also be used.

It is forbidden to use an open flame in manholes and collectors.

In collectors and tunnels it is forbidden to touch with power cables and to lay instruments and materials on these cables.

In tunnels the fire retardant cables should be applied only. These cables may use the PVC, LSOH or other non-inflammable sheaths. In subway tunnels the hot sensor units are installed. The hot sensors and the smoke sensors are installed in each station. The fire hydrants and hand-operated carbon fire extinguishers are also install in each station.

Appendix II

Brazilian Experience

Safety of Personnel for Installation in Infrastructures of Power Lines Distribution

The following points were excerpts from the Brazilian regulation on this issue, and considered relevant.

- Is not recommended the installation of telephones cables in infrastructure of electric power with voltage higher than 75 kV.

- It is not recommended the project of telephones networks in lines parallel with electric power networks with nominal voltage higher than 35 kV.

- The ground resistance must be the maximum of 13 .

- The followings distances between networks are recommended:

| Voltage (V) | Minimum Distance (m) |
| --- | --- |
| ### Up 600 | 0,60 |
| Of 600 a 15000 | 1,30 |
| of 15000 a 35000 | 1,80 |
| of 35000 a 70000 | 2,20 |

- In the crossings of telephone networks and electrical power networks of up to 70 kV, must be observed a 90º±15° angle.

- In the crossings of telephone networks and electrical power networks with voltage between 35 kV and 70 kV, the messenger must be grounded in both sides of the electric line, with a maximum ground resistance of 30 .

_________________